|  | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

ITU-T	Q.424

TELECOMMUNICATION

STANDARDIZATION SECTOR

OF  ITU

SPECIFICATIONS  OF  SIGNALLING  SYSTEM  R2

LINE  SIGNALLING,  DIGITAL  VERSION

PROTECTION  AGAINST  THE  EFFECTS  OF  FAULTY  TRANSMISSION

ITU-T  Recommendation  Q.424

(Extract from the Blue Book)

# NOTES

1	ITU-T Recommendation Q.424 was published in Fascicle VI.4 of the Blue Book. This file is an extract from the Blue Book. While the presentation and layout of the text might be slightly different from the Blue Book version, the contents of the file are identical to the Blue Book version and copyright conditions remain unchanged (see below).

2	In this Recommendation, the expression “Administration” is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

ã  ITU  1988, 1993

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

Recommendation Q.424

Fascicle VI.4 - Rec. Q.424

3.3  PROTECTION AGAINST THE EFFECTS OF FAULTY TRANSMISSION

Faulty transmission conditions in PCM systems can lead to degradation of the speech channels and erroneous signalling. In the case of primary PCM multiplex equipment operating at 2048 kbit/s failures due to loss of frame or multi-frame alignment and/or failure of any other important function results in both PCM terminals going into alarm condition in accordance with Recommendations G.732 and G.734.

Thus both PCM terminals apply the state corresponding to state 1 on the PCM line on each "receive" signalling channel at the interfaces with the switching equipment, as indicated in Table 4 of Recommendation G.732. In this way, the incoming switching equipment receives the equivalent of af = 1, bf = 1 on the PCM line and the outgoing switching equipment receives the equivalent of ab = 1, bb = 1.

These characteristics are taken into account in the present specifications (see § 3.2.4) so that:

-	at the outgoing end (see Table 3/Q.422) a PCM fault results in a blocked state, seizure acknowledged state or clear-back state. This means that all circuits in the idle state of a faulty PCM multiplex will be blocked against seizure and that seized circuits will go to or remain in the seizure acknowledged or clear-back state;

-	at the incoming end (see Table 4/Q.422) a PCM fault can be identified and appropriate actions can be taken.

When the signalling equipment is a part of a digital exchange, it may receive alarm indications in a form other than both signalling bits in state one. The failure may be detected by the signalling equipment or an indication be received from the PCM terminal according to Recommendation G.734.

When the signalling equipment recognizes a failure it must:

-	block the detection of signalling transitions to avoid recognition of erroneous signalling codes caused by the failure. This action must be taken as soon as possible and at least within 3 ms as specified in Recommendation G.734 for a PCM terminal,

-	react as specified in Tables 3/Q.422 and 4/Q.422 when a signalling code a = 1, b = 1 is detected at the input of signalling equipment situated at the analogue access of a PCM terminal equipment complying with Recommendation G.732.