|  | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

ITU-T	Q.713

TELECOMMUNICATION	(03/93)
STANDARDIZATION  SECTOR
OF  ITU

SPECIFICATIONS  OF  SIGNALLING
SYSTEM  No.  7

SIGNALLING  SYSTEM  No.  7  –
SCCP  FORMATS  AND  CODES

ITU-T  Recommendation  Q.713

(Previously “CCITT  Recommendation”)

# FOREWORD

The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of the International Telecommunication Union. The ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Conference (WTSC), which meets every four years, established the topics for study by the ITU-T Study Groups which, in their turn, produce Recommendations on these topics.

ITU-T Recommendation Q.713 was revised by the ITU-T Study Group XI (1988-1993) and was approved by the WTSC (Helsinki, March 1-12, 1993).

___________________

NOTES

1	As a consequence of a reform process within the International Telecommunication Union (ITU), the CCITT ceased to exist as of 28 February 1993. In its place, the ITU Telecommunication Standardization Sector (ITU-T) was created as of 1 March 1993. Similarly, in this reform process, the CCIR and the IFRB have been replaced by the Radiocommunication Sector.

In order not to delay publication of this Recommendation, no change has been made in the text to references containing the acronyms “CCITT, CCIR or IFRB” or their associated entities such as Plenary Assembly, Secretariat, etc. Future editions of this Recommendation will contain the proper terminology related to the new ITU structure.

2	In this Recommendation, the expression “Administration” is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

ã  ITU  1994

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

CONTENTS

Recommendation Q.713     (03/93)

Page

1	General		1

1.1	Routing label		1

1.2	Message type code		1

1.3	Formatting principles		2

1.4	Mandatory fixed part		2

1.5	Mandatory variable part		3

1.6	Optional part		4

1.7	End of optional parameters octet		4

1.8	Order of transmission		4

1.9	Coding of spare bits		4

1.10	National message types and parameters		4

2	Coding of the general parts		4

2.1	Coding of the message type		4

2.2	Coding of the length indicator		4

2.3	Coding of the pointers		4

3	SCCP parameters		5

3.1	End of optional parameters		5

3.2	Destination local reference		5

3.3	Source local reference		5

3.4	Called party address		6

3.5	Calling party address		12

3.6	Protocol class		12

3.7	Segmenting/reassembling		13

3.8	Receive sequence number		13

3.9	Sequencing/segmenting		14

3.10	Credit			14

3.11	Release cause		14

3.12	Return cause		15

3.13	Reset cause		15

3.14	Error cause		16

3.15	Refusal cause		16

3.16	Data			17

3.17	Segmentation		17

3.18	Hop counter		18

4	SCCP messages and codes		18

4.1	General		18

4.2	Connection request (CR)		18

4.3	Connection confirm (CC)		19

4.4	Connection refused (CREF)		20

4.5	Released (RLSD)		20

4.6	Release complete (RLC)		21

4.7	Data form 1 (DT1)		21

4.8	Data form 2 (DT2)		21

4.9	Data acknowledgement (AK)		22

4.10	Unitdata (UDT)		22

4.11	Unitdata service (UDTS)		23

4.12	Expedited data (ED)		23

4.13	Expedited data acknowledgement (EA)		24

4.14	Reset request (RSR)		24

4.15	Reset confirm (RSC)		25

4.16	Protocol data unit error (ERR)		25

4.17	Inactivity test (IT)		25

4.18	Extended unitdata (XUDT)		26

4.19	Extended unitdata service (XUDTS)		27

5	SCCP Management messages and codes		27

5.1	General		27

5.2	SCMG message parameters		28

5.3	SCMG messages		29

Annex A – Mapping for cause parameter values		30

A.1	Introduction		30

A.2	Connection refusal		30

A.3	Connection release		30

A.4	Connection reset		30

Recommendation Q.713

Recommendation Q.713     (03/93)

Signalling  system  no.  7  –  sccp  formats  and  codesXE "Signalling  system  no.  7  –  sccp  formats  and  codes"§

(Malaga-Torremolinos, 1984; modified at Helsinki, 1993)

# 1	General

The Signalling Connection Control Part (SCCP) messages are carried on the signalling data link by means of Signal Units the format of which is described in 2.2/Q.703.

The Service Information Octet format and coding is described in 14.2/Q.704. The Service Indicator is coded 0011 for the SCCP.

The Signalling Information Field (SIF) of each Message Signal Unit containing an SCCP message consists of an integral number of octets.

A message consists of the following parts (see Figure 1):

–	the routing label;

–	the message type code;

–	the mandatory fixed part;

–	the mandatory variable part;

–	the optional part, which may contain fixed length and variable length fields.

The description of the various parts is contained in the following clauses. SCCP Management messages and codes are provided in clause 5.

## 1.1	Routing labelXE "Routing label"§

The standard routing label specified in 2.2/Q.704 is used. The rules for the generation of the Signalling Link Selection (SLS) code are described in 2.2.1/Q.711.

FIGURE 1/Q.713 Routing label

Message type code

Mandatory fixed part

Mandatory variable part

Optional part

General layout

## 1.2	Message type codeXE "Message type code"§

The message type code consists of a one octet field, and is mandatory for all messages. The message type code uniquely defines the function and format of each SCCP message. The allocation of message type codes, with reference to the appropriate descriptive clause of this Recommendation is summarized in Table 1. Table 1 also contains an indication of the applicability of the various message types to the relevant classes of protocol.

TABLE  1/Q.713

SCCP message types

|  | Classes |  |  |  |  | Reference |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Message type | 0 | 1 | 2 | 3 | (subclause) |  | Code |  |
| CR Connection Request |  |  | X | X | 4.2 |  | 0000 0001 |  |
| CC Connection Confirm |  |  | X | X | 4.3 |  | 0000 0010 |  |
| CREF Connection Refused |  |  | X | X | 4.4 |  | 0000 0011 |  |
| RLSD Released |  |  | X | X | 4.5 |  | 0000 0100 |  |
| RLC Release Complete |  |  | X | X | 4.6 |  | 0000 0101 |  |
| DT1 Data Form 1 |  |  | X |  | 4.7 |  | 0000 0110 |  |
| DT2 Data Form 2 |  |  |  | X | 4.8 |  | 0000 0111 |  |
| AK Data Acknowledgement |  |  |  | X | 4.9 |  | 0000 1000 |  |
| UDT Unitdata | X | X |  |  | 4.10 |  | 0000 1001 |  |
| UDTS Unitdata Service | X | X |  |  | 4.11 |  | 0000 1010 |  |
| ED Expedited Data |  |  |  | X | 4.12 |  | 0000 1011 |  |
| EA Expedited Data Acknowledgement |  |  |  | X | 4.13 |  | 0000 1100 |  |
| RSR Reset Request |  |  |  | X | 4.14 |  | 0000 1101 |  |
| RSC Reset Confirm |  |  |  | X | 4.15 |  | 0000 1110 |  |
| ERR Protocol Data Unit Error |  |  | X | X | 4.16 |  | 0000 1111 |  |
| IT Inactivity Test |  |  | X | X | 4.17 |  | 0001 0000 |  |
| XUDT Extended Unitdata | X | X |  |  | 4.18 |  | 0001 0001 |  |
| XUDTS Extended Unitdata Service | X | X |  |  | 4.19 |  | 0001 0010 |  |
| X Type of message in this protocol class. |  |  |  |  |  |  |  |  |

## 1.3	Formatting principles

Each message consists of a number of parameters listed and described in clause 3. Each parameter has a “name” which is coded as a single octet (see clause 3). The length of a parameter may be fixed or variable, and a “length indicator” of one octet for each parameter may be included as described below.

The detailed format is uniquely defined for each message type as described in clause 4.

A general SCCP message format is shown in Figure 2.

## 1.4	Mandatory fixed part

Those parameters that are mandatory and of fixed length for a particular message type will be contained in the mandatory fixed part. The position, length and order of the parameters is uniquely defined by the message type. Thus the names of the parameters and the length indicators are not included in the message.

FIGURE 2/Q.713...[D01] = 3 CM  (118%)

## 1.5	Mandatory variable part

Mandatory parameters of variable length will be included in the mandatory variable part. The name of each parameter and the order in which the pointers are sent is implicit in the message type. Parameter names are, therefore, not included in the message. A pointer is used to indicate the beginning of each parameter. Because of this, parameters may be sent in an order different from that of the pointers. Each pointer is encoded as a single octet. The details of how pointers are encoded is found in 2.3. The number of parameters, and thus the number of pointers is uniquely defined by the message type.

A pointer is also included to indicate the beginning of the optional part. If the message type indicates that no optional part is allowed, then this pointer will not be present. If the message type indicates that an optional part is possible, but there is no optional part included in this particular message, then a pointer field containing all zeros will be used.

All the pointers are sent consecutively at the beginning of the mandatory variable part. Each parameter contains the parameter length indicator followed by the contents of the parameter.

## 1.6	Optional part

The optional part consists of parameters that may or may not occur in any particular message type. Both fixed length and variable length parameters may be included. Optional parameters may be transmitted in any order1). Each optional parameter will include the parameter name (one octet) and the length indicator (one octet) followed by the parameter contents.

## 1.7	End of optional parameters octet

After all optional parameters have been sent, an end of optional parameters octet containing all zeros will be transmitted. This octet is only included if optional parameters are present in the message.

## 1.8	Order of transmission

Since all the parameters consist of an integral number of octets, the formats are presented as a stack of octets. The first octet transmitted is the one shown at the top of the stack and the last is the one at the bottom (see Figure 2).

Within each octet, the bits are transmitted with the least significant bit first.

## 1.9	Coding of spare bits

According to the general rules defined in Recommendation Q.700, spare bits are coded 0 unless indicated otherwise at the originating nodes. At intermediate nodes, they are passed transparently. At destination nodes, they need not be examined.

## 1.10	National message types and parameters

If message type codes and parameter codes are required for national uses, it is suggested that the codes be selected from the highest code downwards, that is starting at code 11111110. Code 11111111 is reserved for future use.

# 2	Coding of the general parts

## 2.1	Coding of the message typeXE "Coding of the message type"§

The coding of the message is shown in Table 1.

## 2.2	Coding of the length indicatorXE "Coding of the length indicator"§

The length indicator field is binary coded to indicate the number of octets in the parameter content field. The length indicator does not include the parameter name octet or the length indicator octet.

## 2.3	Coding of the pointersXE "Coding of the pointers"§

The pointer value (in binary) gives the number of octets between the pointer itself (included) and the first octet (not included) of the parameter associated with that pointer2).

The pointer value all zeros is used to indicate that, in the case of optional parameters, no optional parameter is present.

# 3	SCCP parametersXE "SCCP parameters"§

The parameter name codes are given in Table 2 with reference to the subclauses in which they are described.

TABLE  2/Q.713

SCCP parameter name codes

| Parameter name | Reference (Subclause) | Parameter name code 8765 4321 |
| --- | --- | --- |
| End of optional parameters | 3.1 | 0000 0000 |
| Destination local reference | 3.2 | 0000 0001 |
| Source local reference | 3.3 | 0000 0010 |
| Called party address | 3.4 | 0000 0011 |
| Calling party address | 3.5 | 0000 0100 |
| Protocol class | 3.6 | 0000 0101 |
| Segmenting/reassembling | 3.7 | 0000 0110 |
| Receive sequence number | 3.8 | 0000 0111 |
| Sequencing/segmenting | 3.9 | 0000 1000 |
| Credit | 3.10 | 0000 1001 |
| Release cause | 3.11 | 0000 1010 |
| Return cause | 3.12 | 0000 1011 |
| Reset cause | 3.13 | 0000 1100 |
| Error cause | 3.14 | 0000 1101 |
| Refusal cause | 3.15 | 0000 1110 |
| Data | 3.16 | 0000 1111 |
| Segmentation | 3.17 | 0001 0000 |
| Hop counter | 3.18 | 0001 0001 |

## 3.1	End of optional parametersXE "End of optional parameters"§

The “end of optional parameters” parameter field consists of a single octet containing all zeros.

## 3.2	Destination local referenceXE "Destination local reference"§

The “destination local reference” parameter field is a three-octet field containing a reference number which, in outgoing messages, has been allocated to the connection section by the remote node.

The coding “all ones” is reserved, its use is for further study.

## 3.3	Source local referenceXE "Source local reference"§

The “source local reference” parameter field is a three-octet field containing a reference number which is generated and used by the local node to identify the connection section.

The coding “all ones” is reserved, its use is for further study.

## 3.4	Called party addressXE "Called party address"§

The “called party address” is a variable length parameter. Its structure is shown in Figure 3.

FIGURE  3/Q.713

|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octet 1 | Address indicator |  |  |  |  |  |  |  |
| Octet 2 |  |  |  |  |  |  |  |  |
| . |  |  |  |  |  |  |  |  |
| . | Address |  |  |  |  |  |  |  |
| . |  |  |  |  |  |  |  |  |
| Octet n |  |  |  |  |  |  |  |  |

Called/Calling party address

### 3.4.1	Address indicatorXE "Address indicator"§

The “address indicator” indicates the type of address information contained in the address field (see Figure 4). The address consists of one or any combination of the following elements:

–	signalling point code;

–	global title (for instance, dialled digits);

–	subsystem number.

FIGURE  4/Q.713

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Reserved for national use | Routing indicator | Global title indicator |  |  |  | SSN indicator | Point code indicator |

Address indicator encoding

A “1” in bit 1 indicates that the address contains a signalling point code.

A “1” in bit 2 indicates that the address contains a subsystem number.

Bits 3 - 6 of the address indicator octet contain the global title indicator, which is encoded as follows:

Bits	6543

0000	No global title included

0001	Global title includes nature of address indicator only

0010	Global title includes translation type only3)

0011	Global title includes translation type, numbering plan and encoding scheme3)

0100	Global title includes translation type, numbering plan, encoding scheme and nature of address indicator

Spare international

Spare national

1111	Reserved for extension

When a global title is used in the called party address, it is suggested that the called party address contain a subsystem number. This serves to simplify message reformatting following global title translation. The subsystem number should be encoded “00000000” when the subsystem number is not known, e.g. before translation.

Bit 7 of the address indicator octet contains routing information identifying which address element should be used for routing.

A “0” in bit 7 indicates that routing should be based on the global title in the address.

A “1” in bit 7 indicates that routing should be based on the destination point code in the MTP routing label and the subsystem number information in the called party address.

Bit 8 of the address indicator octet is designated for national use.

### 3.4.2	AddressXE "Address"§

The various elements, when provided, occur in the order: point code, subsystem number, global title, as shown in Figure 5.

FIGURE  5/Q.713

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Signalling point code |  |  |  |  |  |  |  |
| Subsystem number |  |  |  |  |  |  |  |
| Global title |  |  |  |  |  |  |  |

Ordering of address elements

#### 3.4.2.1	Signalling point codeXE "Signalling point code"§

The signalling point code, when provided, is represented by two octets. Bits 7 and 8 in the second octet are set to zero (see Figure 6).

FIGURE  6/Q.713

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 |  |  |  |  |  |  |

Signalling point code encoding

#### 3.4.2.2	Subsystem numberXE "Subsystem number"§

The subsystem number (SSN) identifies an SCCP user function and, when provided, consists of one octet coded as follows:

Bits	87654321

00000000	SSN not known/not used

00000001	SCCP management

00000010	Reserved for CCITT allocation

00000011	ISDN user part

00000100	OMAP (Operation, Maintenance and Administration Part)

00000101	MAP (Mobile Application Part)

00000110	HLR (Home Location Register)

00000111	VLR (Visitor Location Register)

00001000	MSC (Mobile Switching Centre)

00001001	EIC (Equipment Identifier Centre)

00001010	AUC (Authentication Centre)

Spare

11111111	Reserved for expansion

Network specific subsystem numbers should be assigned in descending order starting with “11111110”.

#### 3.4.2.3	Global titleXE "Global title"§4)

The format of the global title is of variable length. Figures 7, 9, 10 and 11 show four possible formats for global title.

##### 3.4.2.3.1	  Global title indicator = 0001

See Figure 7.

FIGURE  7/Q.713

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| O/E | Nature of address indicator |  |  |  |  |  |  | Octet 1 |
| Address information |  |  |  |  |  |  |  | Octet 2 and further |

Global title format for indicator 0001

Bits 1 to 7 of octet 1 contain the nature of address indicator and are coded as follows:

Bits	7654321

0000000	Spare

0000001	Subscriber number

0000010	Reserved for national use

0000011	National significant number

0000100	International number

Spare

Bit 8 of octet 1 contains the odd/even indicator and is coded as follows:

Bit	8

0	Even number of address signals

1	Odd number of address signals

The octets 2 and further contain a number of address signals and possibly a filler as shown in Figure 8.

FIGURE  8/Q.713

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2nd address signal |  |  |  | 1st address signal |  |  |  | Octet 2 |
| 4th address signal |  |  |  | 3rd address signal |  |  |  | Octet 3 |
| ... |  |  |  |  |  |  |  |  |
| Filler (if necessary) |  |  |  | n th address signal |  |  |  | Octet m |

Address information

Each address signal is coded as follows:

0000	digit 0

0001	digit 1

0010	digit 2

0011	digit 3

0100	digit 4

0101	digit 5

0110	digit 6

0111	digit 7

1000	digit 8

1001	digit 9

1010	Spare

1011	Code 115)

1100	Code 125)

1101	Spare

1110	Spare

1111	ST5)

In case of an odd number of address signals, a filler code 0000 is inserted after the last address signal.

##### 3.4.2.3.2	  Global title indicator = 0010

Figure 9 shows the format of the global title, if the global title indicator equals “0010”.

FIGURE  9/Q.713

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Translation type |  |  |  |  |  |  |  | Octet 1 |
| Address information |  |  |  |  |  |  |  | Octet 2 and further |

Global title format for indicator 0010

The translation type is a one-octet field that is used to direct the message to the appropriate global title translation function6). Thus, it may be possible for the address information to be translated into different values for and different combinations of DPCs, SSNs and GTs.

This octet will be coded “00000000” when not used. Translation types for internetwork services will be assigned in ascending order starting with 00000001”. Translation types for network specific services will be assigned in descending order starting with “11111110”. The code “11111111” is reserved for expansion. However, the exact coding of translation types in the international network is for further study. Additional requirements may be placed on this field as a result of further work on Transaction Capabilities and the ISDN User Part.

In the case of this global title format (0010), the translation type may also imply the encoding scheme, used to encode the address information and the numbering plan.

##### 3.4.2.3.3	  Global title indicator = 0011

See Figure 10.

FIGURE  10/Q.713

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Translation type |  |  |  |  |  |  |  | Octet 1 |
| Numbering plan |  |  |  | Encoding scheme |  |  |  | Octet 2 |
| Address information |  |  |  |  |  |  |  | Octet 3 and further |

Global title format for indicator 0011

The translation type is as described in 3.4.2.3.2.

The numbering plan is encoded as follows7):

Bits	8765

0000	Unknown

0001	ISDN/Telephony Numbering Plan (see Recommendations E.163 and E.164)

0010	Spare

0011	Data Numbering Plan (Recommendation X.121)

0100	Telex Numbering Plan (Recommendation F.69)

0101	Maritime Mobile Numbering Plan (Recommendations E.210 and E.211)

0110	Land Mobile Numbering Plan (Recommendation E.212)

0111	ISDN/Mobile Numbering Plan (Recommendation E.214)

Spare

1111	Reserved

The encoding scheme is encoded as follows:

Bits	4321

0000	Unknown

0001	BCD, odd number of digits

0010	BCD, even number of digits

Spare

1111	Reserved

If the encoding scheme is binary coded decimal, the global title value, starting from octet 3, is encoded as shown in Figure 8.

##### 3.4.2.3.4	  Global title indicator = 0100

See Figure 11.

FIGURE  11/Q.713

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Translation type |  |  |  |  |  |  |  | Octet 1 |
| Numbering plan |  |  |  | Encoding scheme |  |  |  | Octet 2 |
| Spare |  | Nature of address indicator |  |  |  |  |  | Octet 3 |
| Address information |  |  |  |  |  |  |  | Octet 4 and further |

Global title format for indicator 0100

The field “translation type” is as described in 3.4.2.3.2. The fields “numbering plan” and “encoding scheme” are as described in 3.4.2.3.3. The field “nature of address indicator” is as described in 3.4.2.3.1.

If the encoding scheme is binary coded decimal, the global title value, starting from octet 4, is encoded as shown in Figure 8.

## 3.5	Calling party addressXE "Calling party address"§

The “calling party address” is a variable length parameter. Its structure is the same as the “called party address”.

An SCCP should be able to receive and/or transfer a connectionless message in which the calling party address parameter only consists of the address indicator octet, where bits 1 to 7 are coded all zeros.

However, it is recommended that the origination point does not code the calling party address octet where bits 1 to 7 are coded all zeros. It is recommended that further information (GT and/or SSN) should be provided.

## 3.6	Protocol classXE "Protocol class"§

The “protocol class” parameter field is a four bit field containing the protocol class.

Bits 1 - 4 are coded as follows:

4 3 2 1

0 0 0 0	class 0

0 0 0 1	class 1

0 0 1 0	class 2

0 0 1 1	class 3

When bits 1 - 4 are coded to indicate a connection-oriented-protocol class (class 2, class 3), bits 5 - 8 are spare.

When bits 1 - 4 are coded to indicate a connectionless protocol class (class 0, class 1), bits 5 - 8 are used to specify message handling as follows:

Bits	8765

0000	No special options

Spare

1000	Return message on error

Spare

## 3.7	Segmenting/reassemblingXE "Segmenting/reassembling"§

The “segmenting/reassembling” parameter field is a one octet field and is structured as follows:

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Reserve |  |  |  |  |  |  | M |

Bits 8 - 2 are spare.

Bit 1 is used for the More Data indication and is coded as follows:

0	No more data

1	More data

## 3.8	Receive sequence numberXE "Receive sequence number"§

The “receive sequence number” parameter field is a one octet field and is structured as follows:

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| P(R) |  |  |  |  |  |  | / |

Bits 8 - 2 contain the receive sequence number P(R) used to indicate the sequence number of the next expected message. P(R) is binary coded and bit 2 is the LSB.

Bit 1 is spare.

## 3.9	Sequencing/segmentingXE "Sequencing/segmenting"§

The sequencing/segmenting parameter field consists of two octets and is structured as follows:

|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octet 1 | P(S) |  |  |  |  |  |  | / |
| Octet 2 | P(R) |  |  |  |  |  |  | M |

Bits 8 - 2 of octet 1 are used for indicating the send sequence number P(S). P(S) is binary coded and bit 2 is the LSB.

Bit 1 of octet 1 is spare.

Bits 8 - 2 of octet 2 are used for indicating the receive sequence number P(R). P(R) is binary coded and bit 2 is the LSB.

Bit 1 of octet 2 is used for the More Data indication and is coded as follows:

0	No more data

1	More data

The sequencing/segmenting parameter field is used exclusively in protocol class 3.

## 3.10	CreditXE "Credit"§

The “credit” parameter field is a one octet field used in the protocol classes which include flow control functions. It contains the window size value coded in pure binary.

## 3.11	Release causeXE "Release cause"§

The release cause parameter field is a one octet field containing the reason for the release of the connection.

The coding of the release cause field is as follows:

Bits	87654321

00000000	End user originated

00000001	End user congestion

00000010	End user failure

00000011	SCCP user originated

00000100	Remote procedure error

00000101	Inconsistent connection data

00000110	Access failure

00000111	Access congestion

00001000	Subsystem failure

00001001	Subsystem congestion8)

00001010	MTP failure

00001011	Network congestion

00001100	Expiration of reset timer

00001101	Expiration of receive inactivity timer

00001110	Not obtainable

00001111	Unqualified

00010000	SCCP failure

Spare

NOTE – A more comprehensive list of causes covering X.96 call progress information is for further study.

## 3.12	Return causeXE "Return cause"§

In the Unitdata Service or Extended Unitdata Service message, the “return cause” parameter field is a one octet field containing the reason for message return. Bits 1 - 8 are coded as follows:

Bits	87654321

00000000	No translation for an address of such nature

00000001	No translation for this specific address

00000010	Subsystem congestion8)

00000011	Subsystem failure

00000100	Unequipped user

00000101	MTP failure

00000110	Network congestion

00000111	Unqualified

00001000	Error in message transporta)

00001001	Error in local processinga)

00001010	Destination cannot perform reassemblya)

00001011	SCCP failure

Spare

a)	Only applicable to XUDTS message.

## 3.13	Reset causeXE "Reset cause"§

The “reset cause” parameter field is a one octet field containing the reason for the resetting of the connection.

The coding of the reset cause field is as follows:

Bits	87654321

00000000	End user originated

00000001	SCCP user originated

00000010	Message out of order – Incorrect P(S)

00000011	Message out of order – Incorrect P(R)

00000100	Remote procedure error – Message out of window

00000101	Remote procedure error – Incorrect P(S) after (re)initialization

00000110	Remote procedure error – General

00000111	Remote end user operational

00001000	Network operational

00001001	Access operational

00001010	Network congestion

00001011	Not obtainable

00001100	Unqualified

Spare

## 3.14	Error causeXE "Error cause"§

The “error cause” parameter field is a one octet field containing the indication of the exact protocol error.

The coding of the error cause field is as follows:

Bits	87654321

00000000	Local reference number (LRN) mismatch – Unassigned destination LRN

00000001	Local reference number (LRN) mismatch – Inconsistent source LRN

00000010	Point code mismatch9)

00000011	Service class mismatch

00000100	Unqualified

Spare

## 3.15	Refusal causeXE "Refusal cause"§

The refusal cause parameter field is a one octet field containing the reason for the refusal of the connection.

The coding of the refusal cause field is as follows:

Bits	87654321

00000000	End user originated

00000001	End user congestion

00000010	End user failure

00000011	SCCP user originated

00000100	Destination address unknown

00000101	Destination inaccessible

00000110	Network resource – QOS not available/non-transient

00000111	Network resource – QOS not available/transient

00001000	Access failure

00001001	Access congestion

00001010	Subsystem failure

00001011	Subsystem congestion10)

00001100	Expiration of the connection establishment timer

00001101	Incompatible user data

00001110	Not obtainable

00001111	Unqualified

Spare

NOTES

1	The inclusion of the routing failure causes as specified for the “return cause” parameter in 3.12 is for further study.

2	A more comprehensive list of causes covering Recommendation X.96 call progress information is for further study.

## 3.16	DataXE "Data"§

The “data” parameter field is a variable length field containing SCCP-user data to be transferred transparently between the SCCP user functions.

## 3.17	SegmentationXE "Segmentation"§

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| F | C | Spare |  | Remaining segment |  |  |  | Octet 1 |
|  |  |  |  |  |  |  |  | Octet 2 |
| Local reference |  |  |  |  |  |  |  | Octet 3 |
|  |  |  |  |  |  |  |  | Octet 4 |

Bit 8 of octet 1 is used for First segment indication and is coded as follows:

0	In all segments but the first

1	First segment

Bit 7 of octet 1 is used to keep in the message in sequence delivery option required by the SCCP user and is coded as follows:

0	Not in sequence delivery

1	In sequence delivery

Bits 6 and 5 in octet 1 are spare bits.

Bits 4 - 1 of octet 1 are used to indicate the number of remaining segments. The values 0000 to 1111 are possible; the value 0000 indicates the last segment.

## 3.18	Hop counterXE "Hop counter"§

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Hop counter |  |  |  |  |  |  |  |

The hop counter parameter consists of one octet. The value of the hop counter, which is decremented on each global title translation, should be in range 15 to 1.

# 4	SCCP messages and codesXE "SCCP messages and codes"§

## 4.1	General

4.1.1	In the following subclauses, the format and coding of the SCCP messages is specified.

For each message a list of the relevant parameters is given in a tabular form.

4.1.2	For each parameter, the table also includes:

–	a reference to the subclause where the formatting and coding of the parameter content is specified;

–	the type of the parameter – The following types are used in the tables:

F	Mandatory fixed length parameter;

V	Mandatory variable length parameter;

O	Optional parameter of fixed or variable length;

–	the length of the parameter – The value in the table includes:

–	for type F parameters the length, in octets, of the parameter content;

–	for type V parameters the length, in octets, of the length indicator and of the parameter content (the minimum and the maximum length are indicated);

–	for type O parameters the length, in octets, of the parameter name, length indicator and parameter content (for variable length parameters, the minimum and maximum length is indicated).

4.1.3	For each message the number of pointers included is also specified.

4.1.4	For each message type, type F parameters and the pointers for the type V parameters must be sent in the order specified in the following tables.

## 4.2	Connection request (CR)XE "Connection request (CR)"§

The CR message contains:

–	the routing label;

–	2 pointers;

–	the parameters indicated in Table 3.

TABLE  3/Q.713

Message type – Connection request

| Parameter | Reference (subclause) | Type (F V O) | Lenght (octets) |
| --- | --- | --- | --- |
| Message type code | 2.1 | F | 1 |
| Source local reference | 3.3 | F | 3 |
| Protocol class | 3.6 | F | 1 |
| Called party address | 3.4 | V | 3 minimum |
| Credit | 3.10 | O | 3 |
| Calling party address | 3.5 | O | 4 minimum |
| Data | 3.16 | O | 3 - 130 |
| End of optional parameters | 3.1 | O | 1 |

## 4.3	Connection confirm (CC)XE "Connection confirm (CC)"§

The CC message contains:

–	the routing label;

–	1 pointer;

–	the parameters indicated in Table 4.

TABLE  4/Q.713

Message type – Connection confirm

| Parameter | Reference (subclause) | Type (F V O) | Length (octets) |
| --- | --- | --- | --- |
| Message type | 2.1 | F | 1 |
| Destination local reference | 3.2 | F | 3 |
| Source local reference | 3.3 | F | 3 |
| Protocol class | 3.6 | F | 1 |
| Credit | 3.10 | O | 3 |
| Called party address | 3.4 | O | 4 minimum |
| Data | 3.16 | O | 3 - 130 |
| End of optional parameter | 3.1 | O | 1 |

## 4.4	Connection refused (CREF)XE "Connection refused (CREF)"§

The message contains:

–	the routing label;

–	1 pointer;

–	the parameters indicated in Table 5.

TABLE  5/Q.713

Message type – Connection refused

| Parameter | Reference (subclause) | Type (F V O) | Length (octets) |
| --- | --- | --- | --- |
| Message type | 2.1 | F | 1 |
| Destination local reference | 3.2 | F | 3 |
| Refusal cause | 3.15 | F | 1 |
| Called party address | 3.4 | O | 4 minimum |
| Data | 3.16 | O | 3 - 130 |
| End of optional parameter | 3.1 | O | 1 |

## 4.5	Released (RLSD)XE "Released (RLSD)"§

The RLSD message contains:

–	the routing label;

–	1 pointer;

–	the parameters indicated in Table 6.

TABLE  6/Q.713

Message type – Released

| Parameter | Reference (subclause) | Type (F V O) | Length (octets) |
| --- | --- | --- | --- |
| Message type | 2.1 | F | 1 |
| Destination local reference | 3.2 | F | 3 |
| Source local reference | 3.3 | F | 3 |
| Release cause | 3.11 | F | 1 |
| Data | 3.16 | O | 3 - 130 |
| End of optional parameter | 3.1 | O | 1 |

## 4.6	Release complete (RLC)XE "Release complete (RLC)"§

The RLC message contains:

–	the routing label;

–	no pointers;

–	the parameters indicated in Table 7.

TABLE  7/Q.713

Message type – Release complete

| Parameter | Reference (subclause) | Type (F V O) | Length (octets) |
| --- | --- | --- | --- |
| Message type | 2.1 | F | 1 |
| Destination local reference | 3.2 | F | 3 |
| Source local reference | 3.3 | F | 3 |

## 4.7	Data form 1 (DT1)XE "Data form 1 (DT1)"§

The DT1 message contains:

–	the routing label;

–	1 pointer;

–	the parameters indicated in Table 8.

TABLE  8/Q.713

Message type – Data form 1

| Parameter | Reference (subclause) | Type (F V O) | Length (octets) |
| --- | --- | --- | --- |
| Message type | 2.1 | F | 1 |
| Destination local reference | 3.2 | F | 3 |
| Segmenting/reassembling | 3.7 | F | 1 |
| Data | 3.16 | V | 2 - 256 |

## 4.8	Data form 2 (DT2)XE "Data form 2 (DT2)"§

The DT2 message contains:

–	the routing label;

–	1 pointer;

–	the parameters indicated in Table 9.

TABLE  9/Q.713

Message type – Data form 2

| Parameter | Reference (subclause) | Type (F V O) | Length (octets) |
| --- | --- | --- | --- |
| Message type | 2.1 | F | 1 |
| Destination local reference | 3.2 | F | 3 |
| Sequencing/segmenting | 3.9 | F | 2 |
| Data | 3.16 | V | 2 - 256 |

## 4.9	Data acknowledgement (AK)XE "Data acknowledgement (AK)"§

The AK message contains:

–	the routing label;

–	no pointers;

–	the parameters indicated in Table 10.

TABLE  10/Q.713

Message type – Data acknowledgement

| Parameter | Reference (subclause) | Type (F V O) | Length (octets) |
| --- | --- | --- | --- |
| Message type | 2.1 | F | 1 |
| Destination local reference | 3.2 | F | 3 |
| Receive sequence number | 3.8 | F | 1 |
| Credit | 3.10 | F | 1 |

## 4.10	Unitdata (UDT)XE "Unitdata (UDT)"§

The UDT message contains:

–	the routing label;

–	3 pointers;

–	the parameters indicated in Table 11.

TABLE  11/Q.713

Message type – Unitdata

| Parameter | Reference (subclause) | Type (F V O) | Length (octets) |
| --- | --- | --- | --- |
| Message type | 2.1 | F | 1 |
| Protocol class | 3.6 | F | 1 |
| Called party address | 3.4 | V | 3 minimum |
| Calling party address | 3.5 | V | 2 minimum |
| Data | 3.16 | V | 2 - Xa) |
| a) Due to the ongoing studies on the SCCP called and calling party address, the maximum length of this parameter needs further study. It is also noted that the transfer of up to 255 octets of user data is allowed when the SCCP called and calling party address do not include global title. |  |  |  |

## 4.11	Unitdata service (UDTS)XE "Unitdata service (UDTS)"§

The UDTS message contains:

–	the routing label;

–	3 pointers;

–	the parameters indicated in Table 12.

TABLE  12/Q.713

Message type – Unitdata service

| Parameter | Reference (subclause) | Type (F V O) | Length (octets) |
| --- | --- | --- | --- |
| Message type | 2.1 | F | 1 |
| Return cause | 3.12 | F | 1 |
| Called party address | 3.4 | V | 3 minimum |
| Calling party address | 3.5 | V | 2 minimum |
| Data | 3.16 | V | 2 - Xa) |
| a) See a) Table 11. |  |  |  |

## 4.12	Expedited data (ED)XE "Expedited data (ED)"§

The ED message contains:

–	the routing label;

–	1 pointer;

–	the parameters indicated in Table 13.

TABLE  13/Q.713

Message type – Expedited data

| Parameter | Reference (subclause) | Type (F V O) | Length (octets) |
| --- | --- | --- | --- |
| Message type | 2.1 | F | 1 |
| Destination local reference | 3.2 | F | 3 |
| Data | 3.16 | V | 2 - 33 |

## 4.13	Expedited data acknowledgement (EA)XE "Expedited data acknowledgement (EA)"§

The EA message contains:

–	the routing label;

–	no pointers;

–	the parameters indicated in Table 14.

TABLE  14/Q.713

Message type – Expedited data acknowledgement

| Parameter | Reference (subclause) | Type (F V O) | Length (octets) |
| --- | --- | --- | --- |
| Message type | 2.1 | F | 1 |
| Destination local reference | 3.2 | F | 3 |

## 4.14	Reset request (RSR)XE "Reset request (RSR)"§

The RSR message contains:

–	the routing label;

–	1 pointer (this allows for inclusion of optional parameters in the future);

–	the parameters indicated in Table 15.

TABLE  15/Q.713

Message type – Reset request

| Parameter | Reference (subclause) | Type (F V O) | Length (octets) |
| --- | --- | --- | --- |
| Message type | 2.1 | F | 1 |
| Destination local reference | 3.2 | F | 3 |
| Source local reference | 3.3 | F | 3 |
| Reset cause | 3.13 | F | 1 |

## 4.15	Reset confirm (RSC)XE "Reset confirm (RSC)"§

The RSC message contains:

–	the routing label;

–	no pointers;

–	the parameters indicated in Table 16.

TABLE  16/Q.713

Message type – Reset confirmation

| Parameter | Reference (subclause) | Type (F V O) | Length (octets) |
| --- | --- | --- | --- |
| Message type | 2.1 | F | 1 |
| Destination local reference | 3.2 | F | 3 |
| Source local reference | 3.3 | F | 3 |

## 4.16	Protocol data unit error (ERR)XE "Protocol data unit error (ERR)"§

The ERR message contains:

–	the routing label;

–	1 pointer (this allows for inclusion of optional parameters in the future);

–	the parameters indicated in Table 17.

TABLE  17/Q.713

Message type – Protocol data unit error

| Parameter | Reference (subclause) | Type (F V O) | Length (octets) |
| --- | --- | --- | --- |
| Message type | 2.1 | F | 1 |
| Destination local reference | 3.2 | F | 3 |
| Error cause | 3.14 | F | 1 |

## 4.17	Inactivity test (IT)XE "Inactivity test (IT)"§

The IT message contains:

–	the routing label;

–	no pointers;

–	the parameters indicated in Table 18.

TABLE  18/Q.713

Message type – Inactivity test

| Parameter | Reference (subclause) | Type (F V O) | Length (octets) |
| --- | --- | --- | --- |
| Message type | 2.1 | F | 1 |
| Destination local reference | 3.2 | F | 3 |
| Source local reference | 3.3 | F | 3 |
| Protocol class | 3.6 | F | 1 |
| Sequencing/segmentinga) | 3.9 | F | 2 |
| Credita) | 3.10 | F | 1 |
| a) Information in these parameter fields reflect those values sent in the last Data Form 2 or Data acknowledgement message. They are ignored if the protocol class parameter indicates class 2. |  |  |  |

## 4.18	Extended unitdata (XUDT)XE "Extended unitdata (XUDT)"§

The XUDT message contains:

–	the routing label;

–	4 pointers;

–	the parameters indicated in Table 19.

TABLE  19/Q.713

Message type – Extended unitdata

| Parameter | Reference (subclause) | Type (F V O) | Legnth (octets) |
| --- | --- | --- | --- |
| Message type | 2.1 | F | 1 |
| Protocol class | 3.6 | F | 1 |
| Hop counter | 3.18 | F | 1 |
| Called party address | 3.4 | V | 3 minimum |
| Calling party address | 3.5 | V | 2 minimum |
| Data | 3.16 | V | 2 - Ya) |
| Segmentation | 3.17 | O | 6b) |
| End of optional parameters | 3.1 | O | 1 |
| a) The maximum length of this parameter is for further study. b) Should not be present in case of a single XUDT message. |  |  |  |

## 4.19	Extended unitdata service (XUDTS)XE "Extended unitdata service (XUDTS)"§

The XUDTS message contains:

–	the routing label;

–	4 pointers;

–	the parameters indicated in Table 20.

TABLE  20/Q.713

Message type – Extended unitdata service

| Parameter | Reference (subclause) | Type (F V O) | Length (octets) |
| --- | --- | --- | --- |
| Message type | 2.1 | F | 1 |
| Return cause | 3.12 | F | 1 |
| Hop counter | 3.18 | F | 1 |
| Called party address | 3.4 | V | 3 minimum |
| Calling party address | 3.5 | V | 2 minimum |
| Data | 3.16 | V | 2 - Ya) |
| Segmentation | 3.17 | O | 6 |
| End of optional parameters | 3.1 | O | 1 |
| a) The maximum length of this parameter is for further study. |  |  |  |

# 5	SCCP Management messages and codesXE "SCCP Management messages and codes"§

## 5.1	General

SCCP Management (SCMG) messages are carried using the connectionless service of the SCCP. When transferring SCMG messages, class 0 is requested with the “discard message on error” option. SCCP management message parts are provided in the “data” parameter of the Unitdata message.

The Unitdata message contains:

–	the routing label;

–	3 pointers;

–	the parameters indicated in Table 21.

Descriptions of the various parts are contained in the following subclauses.

### 5.1.1	SCMG format identifierXE "SCMG format identifier"§

The SCMG format identifier consists of a one-octet field, which is mandatory for all SCMG messages. The SCMG format identifier uniquely defines the function and format of each SCMG message. The allocation of SCMG format identifiers is shown in Table 22.

TABLE  21/Q.713

SCCP management message format

| Parameter | Reference (subclause) | Type (F V O) | Length (octets) |
| --- | --- | --- | --- |
| Message type (= Unitdata) | 2.1 | F | 1 |
| Protocol class (= Class 0, no return) | 3.6 | F | 1 |
| Called party address (SSN = SCCP management) | 3.4 | V | 3 minimum |
| Calling party address (SSN = SCCP management) | 3.5 | V | 3 minimuma) |
| Data (Data consists of an SCMG message with form as in Table 22) | 3.16 | V | 6 |
| a) SSN is always present. |  |  |  |

TABLE  22/Q.713

SCMG format identifiers

| Message | Code 87654321 |
| --- | --- |
| SSA Subsystem-allowed | 00000001 |
| SSP Subsystem-prohibited | 00000010 |
| SST Subsystem-status-test | 00000011 |
| SOR Subsystem-out-of-service-request | 00000100 |
| SOG Subsystem-out-of-service-grant | 00000101 |

### 5.1.2	Formatting principles

The formatting principles used for SCCP messages, as described in subclauses 1.3, 1.4, 1.5, 1.6, 2.2 and 2.3 apply to SCMG messages.

## 5.2	SCMG message parametersXE "SCMG message parameters"§

SCMG parameter name codes are given in Table 23 with reference to the subclauses in which they are described. Presently, these parameter name codes are not used since all SCMG messages contain mandatory fixed parameters only.

### 5.2.1	End of optional parameters

The “end of optional parameters” parameter field consists of a single octet containing all zeros.

TABLE  23/Q.713

SCMG parameter name codes

| Parameter | Reference (subclause) | Parameter name code 87654321 |
| --- | --- | --- |
| End of optional parameters | 5.2.1 | 00000000 |
| Affected SSN | 5.2.2 | 00000001 |
| Affected PC | 5.2.3 | 00000010 |
| Subsystem multiplicity indicator | 5.2.4 | 00000011 |

### 5.2.2	Affected SSN

The “affected subsystem number (SSN)” parameter field consists of one octet coded as directed for the called party address field (see 3.4.2.2).

### 5.2.3	Affected PC

The “affected signalling point code (PC)” parameter field is represented by two octets which are coded as directed for the called party address field (see 3.4.2.1).

### 5.2.4	Subsystem multiplicity indicator (for further study)

The “subsystem multiplicity indicator” parameter field consists of one octet coded as shown in Figure 12.

FIGURE  12/Q.713

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Spare |  |  |  |  |  | SMI |  |

Subsystem multiplicity indicator format

The coding of the SMI field is as follows:

Bits	2 1

0 0	Affected subsystem multiplicity unknown

0 1	Affected subsystem is solitary

1 0	Affected subsystem is duplicated

1 1	Spare

Bits 3 - 8 are spare.

## 5.3	SCMG messagesXE "SCMG messages"§

Presently, all SCMG messages contain mandatory fixed parameters only. Each SCMG message contains:

–	0 pointers;

–	the parameters indicated in Table 24.

TABLE  24/Q.713

SCMG message

| Parameter | Reference (subclause) | Type (F V O) | Length (octets) |
| --- | --- | --- | --- |
| SCMG format identifier (Message type code) | 5.1.1 | F | 1 |
| Affected SSN | 5.2.2 | F | 1 |
| Affected PC | 5.2.3 | F | 2 |
| Subsystem multiplicity indicator | 5.2.4 | F | 1 |

Annex A

Mapping for cause parameter valuesXE "Mapping for cause parameter values"§

(This annex forms an integral part of this Recommendation)

## A.1	Introduction

During connection refusal/release/reset, the SCCP and its users could take necessary corrective actions, if any, only upon relevant information available to them. Thus, it would be very helpful if those information could be conveyed correctly.

During connection release, the “release cause” parameter in the Released (RLSD) message and the N-DISCONNECT primitive (with parameters “originator” and “reason”) are used together to convey those information on the initiator and the cause of the connection release. In addition, the N-DISCONNECT primitive is also used together with the “refusal cause” parameter in the Connection Refused (CREF) message to convey those information during connection refusal. During connection reset, the “reset cause” parameter in the Reset Request (RSR) message and the N-RESET primitive (with parameters “originator” and “reason”) are used together similarly.

In order to convey those information correctly, this annex provides a guideline for the mapping of values between the cause parameters and the corresponding N-primitive parameters during various scenarios.

## A.2	Connection refusal

Table A.1 describes the mapping of values between the “refusal cause” parameter (see 3.15) and the “originator”, “reason” parameters in the N-DISCONNECT primitive (see 2.1.1.2.4/Q.711).

## A.3	Connection release

Table A.2 describes the mapping of values between the “release cause” parameter (see 3.11) and the “originator”, “reason” parameters in the N-DISCONNECT primitive (see 2.1.1.2.4/Q.711).

## A.4	Connection reset

Table A.3 describes the mapping of values between the “reset cause” parameter (see 3.13) and the “originator”, “reason” parameters in the N-RESET primitive (see 2.1.1.2.3/Q.711).

TABLE  A.1/Q.713

Mapping during connection refusal

| CREF Message |  | N-DISCONNECT primitive |  |
| --- | --- | --- | --- |
| Code | Refusal cause | Reason | Originator |
| 00000000 | End user originated | Connection refusal – End user originated | NSU |
| 00000001 | End user congestion | Connection refusal – End user congestion | NSU |
| 00000010 | End user failure | Connection refusal – End user failure | NSU |
| 00000011 | SCCP user originated | Connection refusal – SCCP user originated | NSU |
| 00000100 | Destination address unknown | Connection refusal – Destination address unknown (non-transient condition) | NSP |
| 00000101 | Destination inaccessible | Connection refusal – Destination inaccessible/transient condition | NSP |
| 00000110 | Network resource – QOS unavailable/non-transient | Connection refusal – QOS unavailable/non-transient condition | NSPa) |
| 00000111 | Network resource – QOS unavailable/transient | Connection refusal – QOS unavailable/transient condition | NSPa) |
| 00001000 | Access failure | Connection refusal – Access failure | NSU |
| 00001001 | Access congestion | Connection refusal – Access congestion | NSU |
| 00001010 | Subsystem failure | Connection refusal – Destination inaccessible/non-transient condition | NSP |
| 00001011 | Subsystem congestion | Connection refusal – Subsystem congestion | NSU |
| 00001100 | Expiration of connection estimated timer | Connection refusal – Reason unspecified/transient | NSPa) |
| 00001101 | Inconsistent user data | Connection refusal – Incompatible information in NSDU | NSU |
| 00001110 | Not obtainable | Connection refusal – Reason unspecified/transient | NSPa) |
| 00001110 | Not obtainable | Connection refusal – Undefined | Undefined |
| 00001111 | Unqualified | Connection refusal – Reason unspecified/transient | NSPa) |
| 00001111 | Unqualified | Connection refusal – Undefined | Undefined |
| NSU Network Service User NSP Network Service Provider a) Only those cases will be applicable if the SCCP originates the refusal procedure in response to REQUEST interface element. |  |  |  |

TABLE  A.2/Q.713

Mapping during connection relesae

| RLSD Message |  | N-DISCONNECT primitive |  |
| --- | --- | --- | --- |
| Code | Release cause | Reason | Originator |
| 00000000 | End user originated | Disconnection – Normal condition | NSU |
| 00000001 | End user congestion | Disconnection – End user congestion | NSU |
| 00000010 | End user failure | Disconnection – End user failure | NSU |
| 00000011 | SCCP user originated | Disconnection – SCCP user originated | NSU |
| 00000100 | Remote procedure error | Disconnection – Abnormal condition of transient nature | NSP |
| 00000101 | Inconsistent connection data | Disconnection – Abnormal condition of transient nature | NSP |
| 00000110 | Access failure | Disconnection – Access failure | NSU |
| 00000111 | Access congestion | Disconnection – Access congestion | NSU |
| 00001000 | Subsystem failure | Disconnection - Abnormal condition of non-transient nature | NSP |
| 00001001 | Subsystem congestion | Disconnection – Subsystem congestion | NSU |
| 00001010 | Network failure | Disconnection – Abnormal condition of non-transient nature | NSP |
| 00001011 | Network congestion | Disconnection – Abnormal condition of transient nature | NSP |
| 00001100 | Expiration of reset timer | Disconnection – Abnormal condition of transient nature | NSP |
| 00001101 | Expiration of receive inactivity timer | Disconnection – Abnormal condition of transient nature | NSP |
| 00001110 | Not obtainablea) | Disconnection – Undefined | NSP |
| 00001110 | Not obtainablea) | Disconnection – Undefined | Undefined |
| 00001111 | Unqualified | Disconnection – Abnormal condition | NSU |
| 00001111 | Unqualified | Disconnection – Undefined | NSP |
| 00001111 | Unqualified | Disconnection – Undefined | Undefined |
| 00010000 | SCCP failure | Disconnection – Abnormal condition of non-transient nature | NSP |
| a) The need for this value is for further study. |  |  |  |

TABLE  A.3/Q.713

Mapping during connection reset

| RSR Message |  | N-RESET primitive |  |
| --- | --- | --- | --- |
| Code | Reset cause | Reason | Originator |
| 00000000 | End user originated | Reset – User synchronization | NSU |
| 00000001 | SCCP user originated | Reset – User synchronization | NSU |
| 00000010 | Message out of order – Incorrect P(S) | Reset – Unspecified | NSP |
| 00000011 | Message out of order – Incorrect P(R) | Reset – Unspecified | NSP |
| 00000100 | Remote procedure error – Message out of window | Reset – Unspecified | NSP |
| 00000101 | Remote procedure error – Incorrect P(S) after initialization | Reset – Unspecified | NSP |
| 00000110 | Remote procedure error – General | Reset – Unspecified | NSP |
| 00000111 | Remote end user operational | Reset – User synchronization | NSU |
| 00001000 | Network operational | Reset – Unspecified | NSP |
| 00001001 | Access operational | Reset – User synchronization | NSU |
| 00001010 | Network congestion | Reset – Network congestion | NSP |
| 00001011 | Not obtainablea) | Reset – Unspecified | NSP |
| 00001011 | Not obtainablea) | Reset – Undefined | Undefined |
| 00001100 | Unqualified | Reset – Unspecified | NSP |
| 00001100 | Unqualified | Reset – Undefined | Undefined |
| a) The need for this value is for further study. |  |  |  |

1)	It is for further study if any constraint in the order of transmission will be introduced.

2	)	For example, a pointer value of “00000001” indicates that the associated parameter begins in the octet immediately following the pointer. A pointer value of “00001010” indicates that nine octets of information exist between the pointer octet and the first octet of the parameter associated with that pointer.

3)	Full E.164 numbering plan address is used in these two cases for Recommendation E.164 based global titles.

4)	Incorporation of NSAP address in the SCCP global title is for further study.

5)	The application of these codes in actual networks is for further study.

6	)	A translation type may for instance imply a specific service to be provided by the SCCP user, such as free phone number translation, or identify the category of service to be provided, for example, dialed number screening, password validation or transmission of digits to telephone network address.

7)	The support of all numbering plans is not mandatory.

8)	Subsystem congestion control procedure is for further study.

9)	National option.

10) Subsystem congestion control procedure is for further study.