|  | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

ITU-T	Q.751.1

TELECOMMUNICATION	(10/95)
STANDARDIZATION  SECTOR
OF  ITU

SPECIFICATIONS  OF  SIGNALLING  SYSTEM  No.  7

NETWORK  ELEMENT  MANAGEMENT
INFORMATION  MODEL  FOR  THE  MESSAGE
TRANSFER  PART  (MTP)

ITU-T  Recommendation  Q.751.1

(Previously “CCITT  Recommendation”)

FOREWORD

The ITU-T (Telecommunication Standardization Sector) is a permanent organ of the International Telecommunication Union (ITU). The ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Conference (WTSC), which meets every four years, establishes the topics for study by the ITU-T Study Groups which, in their turn, produce Recommendations on these topics.

The approval of Recommendations by the Members of the ITU-T is covered by the procedure laid down in WTSC Resolution No. 1 (Helsinki, March 1-12, 1993).

ITU-T Recommendation Q.751.1 was prepared by ITU-T Study Group 11 (1993-1996) and was approved under the WTSC Resolution No. 1 procedure on the 17th of October 1995.

___________________

NOTE

In this Recommendation, the expression “Administration” is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

ã  ITU  1996

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

CONTENTS

Recommendation Q.751.1     (10/95)

Page

1	Scope		1

2	References		1

3	Definitions		2

4	Abbreviations		3

5	Conventions		4

6	Informal description of managed object classes		4

6.1	Diagrams		4

6.2	Managed Object classes		8

6.2.1	Signalling Linkset Timer Profile		8

6.2.2	Managed Switching Element		9

6.2.3	MTP Access Point		10

6.2.4	MTP Level 2 Protocol Profile		11

6.2.5	MTP Level 3		14

6.2.6	MTP Signalling Point		15

6.2.7	Signalling Data Link Termination Point		19

6.2.8	Signalling Linkset Termination Point		20

6.2.9	Signalling Link Termination Point		22

6.2.10	Signalling Link Timer Profile		25

6.2.11	Signalling Point Timers Profile		26

6.2.12	Signalling Route Network Element Part		28

6.2.13	Signalling Routeset Network Element Part		32

6.2.14	Signalling Terminal		34

6.2.15	STP Screening Table		35

6.2.16	stpScreeningTableLine		36

7	Formal specification		37

7.1	Managed object classes definitions		38

7.1.1	Signalling Link Set Timers Profile		38

7.1.2	Managed Switching Element		38

7.1.3	MTP Access Point		38

7.1.4	MTP Level 2 Protocol Profile		39

7.1.5	MTP Level 3		39

7.1.6	MTP Signalling Point		40

7.1.7	Signalling Data Link Termination Point		41

7.1.8	Signalling Link Set Termination Point		42

7.1.9	Signalling Link Termination Point		43

7.1.10	Signalling Link Timers Profile		45

7.1.11	Signalling Point Timers Profile		46

7.1.12	Signalling Route Network Element Part		46

7.1.13	Signalling Route Set Network Element Part		48

7.1.14	Signalling Terminal		49

7.1.15	STP Screening Table		49

7.1.16	STP Screening Table Line		50

7.2	Package definitions		50

7.3	Parameter definitions		55

7.4	Attribute definitions		56

7.5	Action definitions		72

Page

7.6	Notification definitions		73

7.7	Naming binding definitions		73

7.8	Abstract syntax productions		77

Annex A – Overview of OMT notation		85

Annex B – Informal entity-relationship diagrams		86

B.1	Entity-relationship diagram for MTP		87

B.1.2	MTP management hierarchies and managed item line diagrams		92

Annex C – Examples		113

C.1	Example state mapping of signLinkSetTp		113

C.2	Example state mapping of signLinkTp		113

C.3	Example state mapping for signRouteSetNePart		115

C.4	Examples for the signRouteNePart MOC		116

Annex D – Networkmanagement of an SS No. 7 network		118

D.1	Plain text description of object classes		118

D.1.1	CIC-table at networkmanagement level		119

D.1.2	Signalling link		119

D.1.3	Signalling LinkSet		120

D.1.4	Signalling point		120

D.1.5	Signalling Route		121

D.1.6	Signalling RouteSet		122

D.1.7	SS No. 7 network		122

D.1.8	Measurements		122

D.2	Semi-formal specification of the managed object classes		122

Annex E – Formal descriptions of MTP measurements		122

E.1	Relationship with Recommendation Q.752		123

E.2	Diagrams of managed object classes for measurements		131

E.3	Managed Object Class definitions		133

E.3.1	All 30 Minutes Signalling Link Duration Data		133

E.3.2	All 5 And 30 Minutes Signalling Link Duration Data		133

E.3.3	All Signalling Link Utilization Data		133

E.3.4	All Signalling Point Utilization Data		133

E.3.5	Handled MSUs per OPC/DPC/SIO		134

E.3.6	Handled Octets per OPC/DPC/SIO		134

E.3.7	Handled Octets per SIO		134

E.3.8	Obligated Signalling Link Duration Data		134

E.3.9	Obligated Signalling Link Utilization Data		135

E.3.10	Obligated Signalling Point Utilization Data		135

E.3.11	Received Octets per OPC		135

E.3.12	Received Octets per OPC/SIO		135

E.3.13	Route Set Information Data		136

E.3.14	Signalling Link Congestion Data		136

E.3.15	Signalling Link Set Duration Data		136

Page

E.3.16	Signalling Point Data		136

E.3.17	SS No. 7 Current Data		137

E.3.18	SS No. 7 First and Interval Current Data		137

E.3.19	SS No. 7 First and Interval Threshold Data		137

E.3.20	Transmitted Octets per DPC		137

E.3.21	Transmitted Octets per DPC/SIO		137

E.4	Package definitions		138

E.5	Attribute Definitions		138

E.6	Name Binding Definitions		141

E.7	ASN.1 productions		143

Annex F – MTP route verification test		144

F.1	Informal description		144

F.2	Formal specification		145

F.2.1	Managed object classes definitions		145

F.2.2	Package definitions		146

F.2.3	Attribute definitions		146

F.2.4	Action definitions		147

F.2.5	Notification definitions		147

F.2.6	Naming binding definitions		147

F.2.7	Abstract syntax productions		147

SUMMARY

The management functionality described in this Recommendation pertains to the Network Element Management of the Message Transfer Part (MTP). It contains the Network Element Management Information Model for the MTP, that is the definition of network element managed objects.

Annex B contains an informal description of the management view of the MTP resources, the main body of the Recommendation refines this view into a formal description of the network element management information model.

Recommendation Q.751.1

Recommendation Q.751.1     (10/95)

NETWORK  ELEMENT  MANAGEMENT  INFORMATION  MODEL
FOR  THE  MESSAGE  TRANSFER  PART  (MTP)

(Geneva, 1995)

# 1	Scope

The management functionality described in this Recommendation pertains to the Network Element Management of the Message Transfer Part (MTP). It contains the Network Element Management Information Model for the MTP, that is the definition of network element managed objects.

This Recommendation is part of a Series of Recommendations (Q.750) for management of SS No. 7 Network consisting of several signalling points. A signalling point is a node in the Signalling System No. 7 Network.

This Recommendation only pertains to management of one single SS No. 7 network.

# 2	References

The following Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; all users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

[1]	ITU-T Recommendation A.15 (1993), Elaboration and presentation of texts for Recommendations of the ITU Telecommunication Standardization Sector.

[2]	CCITT Recommendation M.3010 (1992), Principles for a telecommunications management network.

[3]	CCITT Recommendation M.3100 (1992), Generic network information model.

[4]	ITU-T Recommendation Q.700 (1993), Introduction to CCITT Signalling System No. 7.

[5]	ITU-T Recommendation Q.701 (1993), Functional description of the Message Transfer Part (MTP) of Signalling System No. 7.

[6]	CCITT Recommendation Q.702 (1988), Signalling data link.

[7]	ITU-T Recommendation Q.703 (1993), Signalling System No. 7 – Signalling link.

[8]	ITU-T Recommendation Q.704 (1993), Signalling System No. 7 – Signalling network functions and messages.

[9]	ITU-T Recommendation Q.705 (1993), Signalling System No. 7 – Signalling network structure.

[10]	CCITT Recommendation Q.707 (1988), Testing and maintenance.

[11]	ITU-T Recommendation Q.750 (1993), Overview of Signalling System No. 7 management.

[12]	ITU-T Recommendation Q.752 (1993), Monitoring and measurements for Signalling System No. 7 networks.

[13]	ITU-T Recommendation Q.753 (1993), Signalling System No. 7 management functions MRVT, SRVT and CVT and definition of OMASE-user.

[14]	ITU-T Recommendation Q.754 (1993), Signalling System No. 7 management Application Service Element (ASE) definitions.

[15]	ITU-T Recommendation Q.764 (1993), Signalling System No. 7 – ISDN user part signalling procedures.

[16]	ITU-T Recommendation Q.822 (1994), Stage 1, stage 2 and stage 3 description for the Q3 interface – Performance management.

[17]	CCITT Recommendation X.208 (1988), Specification of Abstract Syntax Notation One (ASN.1).

[18]	ITU-T Recommendation X.283 (1993), Elements of management information related to the OSI network layer.

[19]	ITU-T Recommendation X.680 (1994), Information technology – Abstract Syntax Notation One (ASN.1): Specification of basic notation.

[20]	CCITT Recommendation X.700 (1992), Management framework for Open Systems Interconnection (OSI) for CCITT applications.

[21]	CCITT Recommendation X.701 (1992), Information technology – Open Systems Interconnection – Systems management overview.

[22]	CCITT Recommendation X.710 (1991), Common management information service definition for CCITT applications.

[23]	CCITT Recommendation X.720 (1992), Information technology – Open Systems Interconnection – Structure of management information: Management information model.

[24]	CCITT Recommendation X.721 (1992), Information technology – Open Systems Interconnection – Structure of management information: Definition of management information.

[25]	CCITT Recommendation X.722 (1992), Information technology – Open Systems Interconnection – Structure of management information: Guidelines for the definition of managed objects.

[26]	ITU-T Recommendation X.723 (1993), Information technology – Open Systems Interconnection – Structure of management information: Generic management information.

[27]	CCITT Recommendation X.731 (1992), Information technology – Open Systems Interconnection – Systems management: State management function.

[28]	CCITT Recommendation X.733 (1992), Information technology – Open Systems Interconnection – Systems management: Alarm reporting function.

# 3	Definitions

For the purposes of this Recommendation, the following definitions apply:

This Recommendation makes use of the following terms defined in Recommendation M.3010:

a)	performance management;

b)	configuration management;

c)	fault management;

d)	Telecommunications Management Network (TMN).

This Recommendation makes use of the following terms defined in Recommendation X.700:

–	object instance.

This Recommendation makes use of the following terms defined in Recommendation X.701:

a)	managed object class;

b)	management information;

c)	notification.

This Recommendation makes use of the following terms defined in Recommendation X.710:

–	attribute.

This Recommendation makes use of the following terms defined in Recommendation X.720:

a)	inheritance;

b)	name binding;

c)	package;

d)	parameter.

This Recommendation makes use of the following terms defined in Recommendation X.722:

–	template.

# 4	Abbreviations

Abbreviations regarding the MTP are listed in Table 1/Q.704. Additionally, the following abbreviations are used throughout this Recommendation:

B-ISUP	Broadband ISUP

CIC	Circuit Identification Code

ERD	Entity Relationship Diagram

GDMO	Guidelines for the Definition of Managed Objects

ISDN	Integrated Services Digital Network

ISUP	ISDN User Part

LS	Signalling Linkset

MO	Managed Object

MOC	Managed Object Class

MRVT	MTP Route Verification Test

MSU	Message Signal Unit

MTP	Message Transfer Part

NE	Network Element

OMAP	Operation, Maintenance and Administration Part

OS	Operations System

OSI	Open Systems Interconnection

PCR	Preventive Cyclic Retransmission

SCCP	Signalling Connection Control Part

SEP	Signalling End Point

SIB	Status Indication Busy

SL	Signalling Link

SLC	Signalling Link Code

SLS	Signalling Link Selection

SP	Signalling Point

SS No. 7	Signalling System No. 7

STEP	Signalling Transfer End Point

STM	Synchronous Transfer Mode

STP	Signalling Transfer Point

TC	Transaction Capability

TMN	Telecommunications Management Network

UP	User Part

# 5	Conventions

Recommendation A.1500 is used for the elaboration and presentation of texts for Recommendations of the ITU-T.

The Guidelines for the Definition of Managed Objects (GDMO), defined in Recommendation X.722, are used. In case of differences between the formal part (clause 7) and the informal parts of this Recommendation the formal part is to be regarded as leading.

Throughout this Recommendation the wording "The managed object class x ......" refers to a particular managed object class while the wording "An x ....." refers to an instance of the managed object class "x".

Modelling of redundancy is avoided (e.g. relationships between managed objects are described in one MOC only). However, for some implementations, it may be useful or necessary to add some additional information to some managed object classes, depending on the user’s needs. This can be done by subclassing. For all MTP protocol timers that are modelled in this Recommendation, it is implementation dependent what value they take when they are not used.

# 6	Informal description of managed object classesXE "Informal description of managed object classes"§

This clause gives an informal description – in alphabetical order – of the managed object classes. Diagrams are presented for inheritance, containment and pointer relationships. For each object-class a brief description is given, including the attributes, notifications and actions. Each MOC is (informally) described using tables. In these tables, (I), (M) and (C) are used with the following meaning:

–	(I)	This element is inherited from a superclass.

–	(M)	This element is mandatory.

–	(C)	This element is optional, preconditions on presence may apply.

Measurements for MTP are described in Recommendation Q.752, Tables 1, 2, 3, 4, 5 and 6. The modelling of these measurements for management can be found in Annex E.

A comprehensive analysis of the entities from the MTP protocol that are of relevance to management can be found in Annex B. Annex B consists of entity relationships diagrams and accompanying descriptive text.

Abstracted from Annex B is the collection of managed object classes that forms the management information model. This is described in this clause.

Formal definitions of the managed object classes are given in clause 7.

## 6.1	Diagrams

A summary on the notation used for the diagrams is given in Annex A. See Figure 1.

Naming, containment and pointer relationships are vizualized in the following two diagrams. Figure 2 shows the case without the use of timer profiles while Figure 3 displays the case with. The diagrams show all managed object classes except for the ones that model the MTP measurements which can be found in Annex E.

An international SS No. 7-gateway exchange contains at least one signalling point for each network which it serves. There is one mtpLevel3 per signalling point, hence an exchange may contain multiple mtpLevel3s.

FIGURE 1/Q.751.1...[D01] =  PAGE PLEINE

XE "Inheritance tree for MTP managed objects"§

FIGURE 2/Q.751.1...[D02] =  PAGE PLEINE

XE "Relationships for MTP managed objects classes (without timer profiles)"§

FIGURE 3/Q.751.1...[D03] =  PAGE PLEINE

XE "Relationships for MTP managed objects classes (with timer profiles)"§

## 6.2	Managed Object classes

### 6.2.1	Signalling Linkset Timer ProfileXE "Signalling Linkset Timer Profile"§

| lsTimersProfile |  |  |
| --- | --- | --- |
| Attributes | Notifications | Actions |
| lsTimersProfileId |  |  |
| q704-t6 |  |  |
| q704-t8 |  |  |
| q704-t10 |  |  |
| "ITU-T Rec. M.3100 (1992)": createDeleteNotificationsPackage (C) |  |  |
|  | objectCreation |  |
|  | objectDeletion |  |
| "ITU-T Rec. M.3100 (1992)": attributeValueChangeNotificationPackage (C) |  |  |
|  | attributeValueChange |  |
| lsTimersProfileNamePackage (C) |  |  |
| lsTimersProfileName |  |  |

An instance of this managed object class represents the Q.704 timers that are associated to a signalling linkset. Instances of this class are contained in the switching element. There can exist multiple instances of this class within one switching element and multiple signalling links set can share a given instance of this class. The use of this managed object class is optional. Alternatively, the timer attributes can all be part of the mtpSignPoint.

If the name package is supported: a create request with a value for the name attribute that is already used by another instance of the same object class will be rejected.

If an attempt is made to delete an lsTimersProfile which is still referenced by a signLinkSetTp the delete request will be rejected.

###### Attributes

1)	lsTimersProfileId: this attribute identifies an instance of this MOC;

2)	q704-t6: Q.704 timer "Delay to avoid message mis-sequencing on controlled rerouting";

3)	q704-t8: Q.704 timer "Transfer prohibited inhibition timer (transient solution)";

4)	q704-t10: Q.704 timer "Waiting to repeat signalling routeset test message";

5)	lsTimersProfileName: (optional) attribute for additional name for lsTimersProfiles.

###### Notifications

1)	objectCreation: (optional) this notification is emitted if an instance is created;

2)	objectDeletion: (optional) this notification is emitted if an instance is deleted;

3)	attributeValueChange: (optional) this notification is emitted if an attribute value changes.

### 6.2.2	Managed Switching ElementXE "Managed Switching Element"§

| managedSwitchingElement |  |  |
| --- | --- | --- |
| Attributes | Notifications | Actions |
| "ITU-T Rec. M.3100 (1992)": managedElement (I) |  |  |
| managedElementId | environmentalAlarm |  |
| systemTitle | equipmentAlarm |  |
| vendorName | communicationsAlarm |  |
| alarmStatus | processingAlarm |  |
| administrativeState |  |  |
| operationalState |  |  |
| usageState |  |  |
| "ITU-T Rec. M.3100 (1992)": stateChangeNotificationPackage (I,M) |  |  |
|  | stateChange |  |
| "ITU-T Rec. M.3100 (1992)": createDeleteNotificationPackage (I,C) |  |  |
|  | objectCreation |  |
|  | objectDeletion |  |
| "ITU-T Rec. M.3100 (1992)": attributeValueChangeNotificationPackage (I,C) |  |  |
|  | attributeValueChange |  |
| managedSwitchingElementNamePackage (C) |  |  |
| managedSwitchingElement Name |  |  |

The ManagedSwitchingElement represents, for management purposes, an exchange, i.e. the aggregate of traffic carrying devices, switching stages, controlling and signalling means at a network node that enables subscriber lines to be interconnected and packets to be forwarded as required by individual users. It is derived from the generic managed object class "managedElement".

###### Attributes

1)	managedElementId: this attribute identifies an instance of this MOC;

2)	systemTitle;

3)	vendorName;

4)	alarmStatus;

5)	OSI-states: administrativeState, operationalState and usageState;

6)	managedSwitchingElementName: (optional) attribute for additional name for managedSwitchingElements.

###### Notifications

1)	environmentalAlarm;

2)	equipmentAlarm;

3)	communicationsAlarm;

4)	processingAlarm:

5)	stateChange: this notification is emitted if the state attributes change;

6)	objectCreation: (optional) this notification is emitted if an instance is created;

7)	objectDeletion: (optional) this notification is emitted if an instance is deleted;

8)	attributeValueChange: (optional) this notification is emitted if an attribute value changes.

### 6.2.3	MTP Access PointXE "MTP Access Point"§

| mtpAccessPoint |  |  |
| --- | --- | --- |
| Attributes | Notifications | Actions |
| operationalState |  |  |
| availabilityStatus |  |  |
| "ITU-T Rec. X.283 (1993)":nSAP (M,I) |  |  |
| sap2Address | objectDeletion |  |
|  | objectCreation |  |
| "ITU-T Rec. X.723 (1992)":sap2 (M,I) |  |  |
| sapId | stateChange |  |
| userEntityNames |  |  |
| providerEntityNames |  |  |
| "ITU-T Rec. M.3100 (1992)": attributeValueChangeNotificationPackage (I, C) |  |  |
|  | attributeValueChange |  |
| "ITU-T Rec. M.3100 (1992)": alarmSeverityAssignmentPointerPackage (C) |  |  |
| alarmSeverityAssignmentPointer |  |  |
| mtpAccessPointNamePackage (C) |  |  |
| mtpAccessPointName |  |  |
| ss7OnOccEventPackage (C) |  |  |
|  | ss7OnOccEvent |  |

This managed object class is derived from the generic managed object class nSAP (network Service Access Point). The sap2Address attribute contains the address of the mtpAccessPoint. It contains the Signalling Point Code (SPC), Service Indicator (SI) and MTP Network Identity (NI). The sapId attribute is used in naming instances of the mtpAccessPoint managed object class.

The userEntityNames attribute contains the distinguishable names of the managed objects that represent the user entities that are using the mtpAccessPoint, e.g. the SCCP instance. The providerEntityNames attribute contains the distinguishable names of the managed objects that represent the provider entities that are supporting the mtpAccessPoint, i.e. the instance of the MTP.

A signalling point can have the MTP status:

–	allowed, the mtpAccessPoint is accessible;

–	congested, this means that the mtpAccessPoint is accessible but the path to it is heavily loaded;

–	prohibited, this means that the mtpAccessPoint is not accessible.

If the name package is supported: a create request with a value for the name attribute that is already used by another instance of the same managed object class will be rejected.

### 6.2.4	MTP Level 2 Protocol ProfileXE "MTP Level 2 Protocol Profile"§

| mtpL2ProtocolProfile |  |  |
| --- | --- | --- |
| Attributes | Notifications | Actions |
| mtpL2ProtocolProfileId |  |  |
| errorCorrectionMethod |  |  |
| transmissionRate |  |  |
| l2TimerT1 |  |  |
| l2TimerT2 |  |  |
| l2TimerT3 |  |  |
| l2TimerT4N |  |  |
| l2TimerT4E |  |  |
| l2TimerT5 |  |  |
| l2TimerT6 |  |  |
| l2TimerT7 |  |  |
| transmissionCongestionThresholdOnsetLevel1 |  |  |
| transmissionCongestionThresholdAbatementLevel1 |  |  |
| "ITU-T Rec. M.3100 (1992)": attributeValueChangeNotificationPackage (C) |  |  |
|  | attributeValueChange |  |
| "ITU-T Rec. M.3100 (1992)": createDeleteNotificationsPackage (C) |  |  |
|  | objectCreation |  |
|  | objectDeletion |  |
| bufferMechanismPackage (C) |  |  |
| numberOfThresholdLevels |  |  |
| congestionHandlingPackage (C) |  |  |
| congestionCounting |  |  |
| congestionReportingBaseObject |  |  |
| loopDelayPackage (C) |  |  |
| loopDelay |  |  |
| mtpL2ProtocolProfileNamePackage (C) |  |  |
| mtpL2ProtocolProfileName |  |  |
| multipleTransmissionCongestionLevelsPackage (C) |  |  |
| transmissionCongestionThresholdDiscardLevel1 |  |  |
| transmissionCongestionThresholdOnsetLevel2 |  |  |
| transmissionCongestionThresholdAbatementLevel2 |  |  |
| transmissionCongestionThresholdDiscardLevel2 |  |  |
| transmissionCongestionThresholdOnsetLevel3 |  |  |
| transmissionCongestionThresholdAbatementLevel3 |  |  |
| transmissionCongestionThresholdDiscardLevel3 |  |  |

| mtpL2ProtocolProfile |  |  |
| --- | --- | --- |
| Attributes | Notifications | Actions |
| multipleTransmissionCongestionStatesPackage (C) |  |  |
| timerTx |  |  |
| timerTy |  |  |
| numberOfCongestionStates |  |  |
| initialLevelIfCongested |  |  |
| pCRPAckage (C) |  |  |
| maxMSURetransN1 |  |  |
| maxOctRetransN2 |  |  |
| receiveCongestionLevelsPackage (C) |  |  |
| receiveCongestionThresholdOnset |  |  |
| receiveCongestionThresholdAbatement |  |  |

In this managed object class, information which is the same for many links is held. Each signLinkTp is associated with one mtpL2ProtocolProfile via a pointer. Several signLinkTp managed objects can use the same instance of mtpL2ProtocolProfile. This allows the operator an easy way to assign parameters when creating new links and to modify many characteristic coherent values of a link at the same time by changing the pointer in signLinkTp to another profile.

The manufacturer can provide object instances of this class with pre-set values.

The operator can create specific object instances as required. The only constraint is that the contained values fit with the offered protocol capabilities in the switch and lie within the limits specified in the standards.

If the name package is supported: a create request with a value for the name attribute that is already used by another instance of the same object class will be rejected.

If an attempt is made to delete an mtpL2ProtocolProfile which is still referenced by a signLinkTp the delete request will be rejected.

Modifying attributes other than the mtpL2ProtocolProfileName is not allowed. Instead a new object instance with the respective attribute values has to be created and referenced by the signLinkTp instance.

###### Attributes

The managed object class mtpL2ProtocolProfile contains the following attributes:

1)	mtpL2ProtocolProfileId: identifies the mtpL2ProtocolProfile instance within the managedSwitchingElement instance.

2)	errorCorrectionMethod: specifies the ECM (PCR or basic) which is used by a signalling link.

3)	transmissionRate: specifies the transmission rate to which the profile fits. (This attribute also implies, if the transmission is analog or digital, i.e. 64 kbit/s and 56 kbit/s are digital, 4.8 kbit is analog). Refer to 12.3/Q.703 for the relation of the timer ranges to the nominal values of the other attributes.

4)	l2TimerT1: level 2 timer "Alignment ready".

5)	l2TimerT2: level 2 timer "Not aligned".

6)	l2TimerT3: level 2 timer "Aligned".

7)	l2TimerT4N: level 2 timer "Proving period timer normal".

8)	l2TimerT4E: level 2 timer "Proving period timer emergency".

9)	l2TimerT5: level 2 timer "Sending SIB".

10)	l2TimerT6: level 2 timer "Remote congestion".

11)	l2TimerT7: level 2 timer "Excessive delay of acknowledgement".

12)	transmissionCongestionThresholdOnsetLevel1: if the number of MSUs in the transmission/retransmission buffer gets bigger than this value, the respective link is congested in congestionLevel 1. CongestionLevel 1 measures start.

13)	transmissionCongestionThresholdAbatementLevel1: if the number of MSUs in the transmission/ retransmission buffer of a link in the congestionLevel 1 gets smaller than this number, the respective link is not any more congested. CongestionLevel 1 measures end.

14)	numberOfThresholdLevels: (optional) this attribute equals the number of threshold levels that are used for congestion control.

15)	congestionCounting: (optional) this attribute indicates whether congestion counting is message based or octet based.

16)	congestionReportingBaseObject: (optional) this attribute indicates on what object congestion counting is based.

17)	loopDelay: optional attribute, represents the loop delay, as defined in 6.4.2/Q.703.

18)	mtpL2ProtocolProfileName: (optional) attribute for additional name for mtpL2ProtocolProfile.

19)	transmissionCongestionThresholdDiscardLevel1 – optional attribute: if the number of MSUs in the transmission buffer of a link in the congestionLevel 2 gets bigger than this number, MSUs are discarded.

20)	transmissionCongestionThresholdOnsetLevel2 – optional attribute: if the number of MSUs in the transmission/retransmission buffer gets bigger than this value, the respective link is congested in congestionLevel 2. CongestionLevel 2 measures start.

21)	transmissionCongestionThresholdAbatementLevel2 – optional attribute: if the number of MSUs in the transmission/retransmission buffer of a link in the congestionLevel 1 gets smaller than this number, the respective link is not any more in congestionLevel 2. CongestionLevel 2 measures end.

22)	transmissionCongestionThresholdDiscardLevel2 – optional attribute: if the number of MSUs in the transmission buffer of a link in the congestionLevel 2 gets bigger than this number, MSUs are discarded.

23)	transmissionCongestionThresholdOnsetLevel3 – optional attribute: if the number of MSUs in the transmission/retransmission buffer gets bigger than this value, the respective link is congested in congestionLevel 3. CongestionLevel 3 measures start.

24)	transmissionCongestionThresholdAbatementLevel3 – optional attribute: if the number of MSUs in the transmission/retransmission buffer of a link in the congestionLevel 1 gets smaller than this number, the respective link is not any more in congestionLevel 3. CongestionLevel 3 measures end.

25)	transmissionCongestionThresholdDiscardLevel3 – optional attribute: if the number of MSUs in the transmission buffer of a link in the congestionLevel 3 gets bigger than this number, MSUs are discarded.

26)	timerTx: (optional) attribute timer threshold, see 3.8.2.3/Q.704.

27)	timerTy: (optional) attribute timer threshold, see 3.8.2.3/Q.704.

28)	numberOfCongestionStates: (optional) this attribute equals the number of congestion states used in the "multiple link congestion states without congestion priority", see 3.8.2.3/Q.704.

29)	initialLevelIfCongested: (optional) this attribute represents the initial state of congestion states used in the "multiple link congestion states without congestion priority", see 3.8.2.3/Q.704.

30)	maxMSURetransN1: (optional) maximum number of MSUs which are available for retransmission when PCR is used (name in Recommendation Q.703: N1).

31)	maxOctRetransN2: (optional) PCR threshold for the number of octets available for retransmission (name in 6.4/Q.703: N2).

32)	receiveCongestionThresholdOnset – optional attribute: if the number of MSUs in the reception buffer gets bigger than this value, Level 2 flow control measures begin. MSUs are still accepted.

33)	receiveCongestionThresholdAbatement – optional attribute: if the number of MSUs in the reception buffer gets smaller than this number, Level 2 flow control measures end.

NOTE – The attributes 12), 13) and 19) to 25) represent the threshold levels for the transmission/retransmission buffer. For the international network, only one threshold level is defined. This level is referred here to as congestionLevel 1. For national networks, up to three threshold levels can be defined. Therefore the thresholds belonging to congestionLevels 2 and 3 are optional, as are all the discard levels since these imply use of congestion methods with priorities. See B.1.2.9.2. The behaviour of a link, if its congestionState or its level of congestion changes are to be described at the managed object class signLinkTp.

For all attributes, apply the operation GET.

###### Notifications

1)	attributeValueChange: (optional) this notification is emitted if an attribute value changes.

2)	objectCreation: (optional) this notification is emitted if an instance is created.

3)	objectDeletion: (optional) this notification is emitted if an instance is deleted.

### 6.2.5	MTP Level 3XE "MTP Level 3"§

| mtpLevel3 |  |  |
| --- | --- | --- |
| Attributes | Notifications | Actions |
| mtpLevel3Id | stateChange |  |
| administrativeState |  |  |
| operationalState |  |  |
| usageState |  |  |
| proceduralStatus |  |  |
| "ITU-T Rec. M.3100 (1992)": alarmSeverityAssignmentPointerPackage (C) |  |  |
| alarmSeverityAssignmentProfilePointer |  |  |
| "ITU-T Rec. M.3100 (1992)": attributeValueChangeNotificationPackage (C) |  |  |
|  | attributeValueChange |  |
| "ITU-T Rec. M.3100 (1992)": createDeleteNotificationsPackage (C) |  |  |
|  | objectCreation |  |
|  | objectDeletion |  |
| communicationsAlarmPackage (C) |  |  |
|  | communicationsAlarm |  |
| mtpLevel3NamePackage (C) |  |  |
| mtpLevel3Name |  |  |
| ss7OnOccEventPackage (C) |  |  |
|  | ss7OnOccEvent |  |

Level 3 of MTP is the signalling network functionality of the SPs. Its purpose is to transfer SS No. 7 messages between SPs.

If the name package is supported: a create request with a value for the name attribute that is already used by another instance of the same object class will be rejected.

###### Attributes

1)	mtpLevel3Id: this attribute is the identification of the instance of this class.

2)	OSI states administrativeState, operationalState, usageState and proceduralStatus: these attributes represent the OSI-management state (as defined in Recommendation X.731) for the MTP level 3.

3)	alarmSeverityAssignmentProfilePointer: (optional).

4)	mtpLevel3Name: (optional) attribute for additional name for mtpLevel3.

###### Notifications

1)	stateChange: in case one of the state-attributes changes this notification is emitted. The notification will contain the old and the new value of the related state-attributes.

2)	attributeValueChange: (optional) this notification is emitted if an attribute value changes.

3)	objectCreation: (optional) this notification is emitted if an instance is created.

4)	objectDeletion: (optional) this notification is emitted if an instance is deleted.

5)	"Recommendation X.721 | ISO/IEC 10165-2: 1992": communicationsAlarm: (optional), the following probable causes can apply: Probable Cause = 501 (AdjacentPCInaccessible), with the inaccessible adjacent pointcode as a parameter, which represents measurement Q.752/5.1 (PerceivedSeverity = Maj/Min/War) and measurement Q.752/5.4 (PerceivedSeverity = Cleared).

6)	ss7OnOccEvent: (optional) event notification for which the following probable causes can apply: Probable Cause = 405 (LinkSetFailure), with the inaccessible adjacent pointcode as a parameter, which represents measurement Q.752/4.5 (PerceivedSeverity = Maj/Min/War) and measurement Q.752/4.6 (PerceivedSeverity = Cleared).

Probable Cause = 413 (LinkSetChange), with the involved linksets as parameter, which represents measurement Q.752/4.13.

### 6.2.6	MTP Signalling PointXE "MTP Signalling Point"§

| mtpSignPoint |  |  |
| --- | --- | --- |
| Attributes | Notifications | Actions |
| mtpSignPointId | stateChange |  |
| pointCode | mtpRestarting |  |
| spType |  |  |
| operationalState |  |  |
| proceduralStatus |  |  |
| availabilityStatus |  |  |
| version |  |  |
| networkIndicator |  |  |
| "ITU-T Rec. M.3100 (1992)": attributeValueChangeNotificationPackage (C) |  |  |
|  | attributeValueChange |  |
| "ITU-T Rec. M.3100 (1992)": createDeleteNotificationsPackage |  |  |
|  | objectCreation |  |
|  | objectDeletion |  |

| mtpSignPoint |  |  |
| --- | --- | --- |
| Attributes | Notifications | Actions |
| mtpSignPointNamePackage (C) |  |  |
| mtpSignPointName |  |  |
| spTimersPackage (C) |  |  |
| q704-t1 |  |  |
| q704-t2 |  |  |
| q704-t3 |  |  |
| q704-t4 |  |  |
| q704-t5 |  |  |
| q704-t6 |  |  |
| q704-t7a) |  |  |
| q704-t8 |  |  |
| q704-t10 |  |  |
| q704-t11a) |  |  |
| q704-t12 |  |  |
| q704-t13 |  |  |
| q704-t14 |  |  |
| q704-t15a) |  |  |
| q704-t16a) |  |  |
| q704-t17 |  |  |
| q704-t18 |  |  |
| q704-t19 |  |  |
| q704-t20 |  |  |
| q704-t21 |  |  |
| q704-t22 |  |  |
| q704-t23 |  |  |
| q704-t24a) |  |  |
| q707-t1 |  |  |
| q707-t2 |  |  |
| spTimersProfilePointerPackage (C) |  |  |
| spTimersProfilePointer |  |  |
| a) These timers are national options. It is implementation dependent what value they take them they are not used. |  |  |

The signalling point is a node in the SS No. 7 network. The MTP permits three types of nodes:

–	SEP: the signalling end point, this type of SP contains MTP and ISUP (and or other MTP Users), but cannot act as an intermediate MTP transfer node in the SS No. 7 network. (It can act as an SCCP Relay Node.)

–	STP: the MTP signalling transfer point, this type of SP acts as an intermediate node between SEPs to transfer messages through the SS No. 7 network, it only contains MTP1).

–	STEP: MTP signalling transfer and end point, the type combines the previous mentioned types, thus acting both as a SEP and as a STP.

The signPoint may be locked, unlocked or restarted for management purposes.

An mtpSignPoint can be created by an operator or automatically. In case the spTimersProfilePackage is used, a create or set request is rejected, if:

i)	the spTimersProfilePointer does not reference a spTimersProfile; or

ii)	the spTimersProfilePointer would reference an instance which does not exist.

If the name package is supported: a create request with a value for the name attribute that is already used by another instance of the same object class will be rejected.

An mtpSignPoint can be deleted if and only if it does not contain any other managed object class instances, except for contained measurements.

NOTE – Management actions carried out at a signalling point are performed on subordinate MOCs.

###### Attributes

1)	mtpSignPointId: this attribute is used to identify an instance of this MOC.

2)	pointCode: this attribute represents the point code which is used within the MTP signalling procedures.

3)	spType: this attribute represents the SP-type: SEP, STP or STEP. This attribute is readable, and optionally writable.

4)	OSI states administrativeState, operationalState, proceduralStatus and availabilityStatus: these attributes represent the OSI-management states.

5)	version: this attribute represents the version (e.g. whether it is Blue Book, Red Book or some other) of the MTP implementation. The attribute is read-only. This attribute is implementation dependent.

6)	networkIndicator: this attribute indicates the network indicator of the network the signalling point belongs to.

7)	mtpSignPointName: (optional) attribute for additional name for mtpSignPoints.

8)	q704-t1: Q.704 timer "Delay to avoid message mis-sequencing on changeover". This attribute is optional. If used, option B applies.

9)	q704-t2: Q.704 timer "Waiting for changeover acknowledgement". This attribute is optional. If used, option B applies.

10)	q704-t3: Q.704 timer "Time controlled diversion-delay to avoid mis-sequencing on change back". This attribute is optional. If used, option B applies.

11)	q704-t4: Q.704 timer "Waiting for change back acknowledgement (first attempt)". This attribute is optional. If used, option B applies.

12)	q704-t5: Q.704 timer "Waiting for change back acknowledgement (second attempt)". This attribute is optional. If used, option B applies.

13)	q704-t6: Q.704 timer "Delay to avoid message mis-sequencing on controlled rerouting". This attribute is optional. If used, option B applies.

14)	q704-t7: Q.704 timer "Waiting for signalling data link connection acknowledgement". This attribute is optional. If used, option B applies.

15)	q704-t8: Q.704 timer "Transfer prohibited inhibition timer (transient solution)". This attribute is optional. If used, option B applies.

16)	q704-t10: Q.704 timer "Waiting to repeat signalling routeset test message". This attribute is optional. If used, option B applies.

17)	q704-t11: Q.704 timer "Transfer restricted timer. (This is one way of implementing the function described in 13.4/Q.704 and mainly intended to simplify STPs.)" This attribute is optional. If used, option B applies.

18)	q704-t12: Q.704 timer "Waiting for uninhibit acknowledgement". This attribute is optional. If used, option B applies.

19)	q704-t13: Q.704 timer "Waiting for force uninhibit". This attribute is optional. If used, option B applies.

20)	q704-t14: Q.704 timer "Waiting for inhibition acknowledgement". This attribute is optional. If used, option B applies.

21)	q704-t15: Q.704 timer "Waiting to start signalling routeset congestion test". This attribute is optional. If used, option B applies.

22)	q704-t16: Q.704 timer "Waiting for routeset congestion status update". This attribute is optional. If used, option B applies.

23)	q704-t17: Q.704 timer "Delay to avoid oscillation of initial alignment failure and link restart". This attribute is optional. If used, option B applies.

24)	q704-t18: Q.704 timer "Timer within a signalling point whose MTP restarts, for supervising link and linkset activation as well as the receipt of routing information". This attribute is optional. If used, option B applies.

25)	q704-t19: Q.704 timer "Supervision timer during MTP restart to avoid possible ping-pong of TFP, TFR2) and TRA messages". This attribute is optional. If used, option B applies.

26)	q704-t20: Q.704 timer "Overall MTP restart timer at the signalling point whose MTP restarts. Overall MTP restart timer at a signalling point adjacent to one whose MTP restarts". This attribute is optional. If used, option B applies.

27)	q704-t21: Q.704 timer "Overall MTP restart timer at a signalling point adjacent to one whose MTP restarts". This attribute is optional. If used, option B applies.

28)	q704-t22: Q.704 timer "Local inhibit test timer". This attribute is optional. If used, option B applies.

29)	q704-t23: Q.704 timer "Remote inhibit test timer". This attribute is optional. If used, option B applies.

30)	q704-t24: Q.704 timer "Stabilizing timer after removal of local processor outage, used in LPO latching to RPO (national option)". This attribute is optional. If used, option B applies.

31)	q707-t1: Q.707 timer T1. This attribute is optional. If used, option B applies.

32)	q707-t2: Q.707 timer T2. This attribute is optional. If used, option B applies.

33)	spTimersProfilePointer: (optional) this attribute indicates the timer profile used for this particular MTP signalling point. If used, option A applies to this instance.

This managed object class can be used in two ways: either option A or option B. The options are mutually exclusive. Option A (see also Figure 3) uses a separate profile for the SP timers from Recommendations Q.704 and Q.707, where option B includes all Q.704 and Q.707 timers attributes in the instances of this MOC itself (see Figure 2).

###### Notifications

1)	stateChange: if the state attributes change, this notification will be emitted.

2)	mtpRestarting: if the MTP is restarting this managed object class will emit this notification (once).

3)	attributeValueChange: (optional) this notification is emitted if an attribute value changes.

4)	objectCreation: (optional) this notification is emitted if an instance is created.

5)	objectDeletion: (optional) this notification is emitted if an instance is deleted.

### 6.2.7	Signalling Data Link Termination PointXE "Signalling Data Link Termination Point"§

| signDataLinkTp |  |  |
| --- | --- | --- |
| Attributes | Notifications | Actions |
| signDataLinkTpId | stateChange |  |
| adjPc |  |  |
| transmissionRate |  |  |
| operationalState |  |  |
| equipmentPointer |  |  |
| "ITU-T Rec. M.3100 (1992)": attributeValueChangeNotificationPackage (C) |  |  |
|  | attributeValueChange |  |
| "ITU-T Rec. M.3100 (1992)": createDeleteNotificationsPackage (C) |  |  |
|  | objectCreation |  |
|  | objectDeletion |  |
| cICPackage (C) |  |  |
| cIC |  |  |
| signDataLinkTpNamePackage (C) |  |  |
| signDataLinkTpName |  |  |
| stmChannelPackage (C) |  |  |
| stmChannel |  |  |

For management purposes only the part of the signalling data link that resides within the SP is being considered. It is called the signalling data link Termination Point (TP), which is in fact a Circuit Termination Point (CTP) especially allocated to SS No. 7. The following management aspects are relevant to the signalling data link TP. The signDataLinkTp shall be in an OSI-management-state (see Recommendation X.731). The signDataLinkTp managed object class will notify the OS in case a failure can be identified as located in the signalling data link.

A create request is rejected if the equipmentPointer would reference equipment that does not exist.

If the name package is supported: a create request with a value for the name attribute that is already used by another instance of the same object class will be rejected.

###### Attributes

1)	signDataLinkTpId: this attribute identifies particular instances of this class.

2)	adjPc: this attribute represents the point code of the adjacent SP, to which the resource represented by this instance is connected. The attribute is read-only.

3)	transmissionRate: this attribute specifies the transmissionRate and implies the transmissionType (i.e. 56 and 64 kbit/s imply digital, 4.8 kbit/s implies analog transmission).

4)	operational state: in normal operation this state is "enabled". In case of failure of the signalling data link (in transmission system or part residing within the managed switching element) this state will be "disabled".

5)	equipmentPointer: this attribute references the physical equipment (port) where the signalling data link is connected to the exchange.

6)	cIC: (optional), this attribute is used to reference the SS No. 7 trunk used by the datalink. Its value has to be unique within the SPs connected by the trunk.

7)	signDataLinkTpName: (optional) attribute for additional name for signDataLinkTps.

8)	stmChannel: (optional) this attribute denotes the STM channel which defines the signalling datalink on the PCM transmission system.

###### Notifications

1)	stateChange: when a state changes this notification will be emitted.

2)	attributeValueChange: (optional) this notification is emitted if an attribute value changes.

3)	objectCreation: (optional) this notification is emitted if an instance is created.

4)	objectDeletion: (optional) this notification is emitted if an instance is deleted.

### 6.2.8	Signalling Linkset Termination PointXE "Signalling Linkset Termination Point"§

| signLinkSetTp |  |  |
| --- | --- | --- |
| Attributes | Notifications | Actions |
| signLinkSetTpId | stateChange |  |
| adjPc |  |  |
| maxCapacityLS |  |  |
| currentCapacityLS |  |  |
| congestionControlMethod |  |  |
| usageState |  |  |
| operationalState |  |  |
| availabilityStatus |  |  |
| periodicLinkTestFlag |  |  |
| administrativeStatePackage (C) |  |  |
| administrativeState |  |  |
| "ITU-T Rec. M.3100 (1992)": alarmSeverityAssignmentPointerPackage (C) |  |  |
| alarmSeverityAssignmentProfilePointer |  |  |
| "ITU-T Rec. M.3100 (1992)": attributeValueChangeNotificationPackage (C) |  |  |
|  | attributeValueChange |  |
| "ITU-T Rec. M.3100 (1992)": createDeleteNotificationsPackage |  |  |
|  | objectCreation |  |
|  | objectDeletion |  |
| communicationsAlarmPackage (C) |  |  |
|  | communicationsAlarm |  |
| inLsLoadShareAlgorithmPackage (C) |  |  |
| inLsLoadShareAlgorithm |  |  |
| lsTimersProfilePointerPackage (C) |  |  |
| lsTimersProfilePointer |  |  |
| noBasicLinkAllocationPackage (C) |  |  |
| numberOfNormallyActiveSignLinkTps |  |  |
| signLinkSetTpNamePackage (C) |  |  |
| signLinkSetTpName |  |  |

Corresponding to the NE-part of a signalling linkset a managed object class signLinkSetTp is defined. An OS can manage a complete signalling linkset (at the network management level) by managing both ends of the corresponding signLinkSetTps. Adding and removing links to and from linksets may ultimately be done via the appropriate network management managed objects.

A creation request is only successful if a signRouteSetNePart exists within the mtpSignPoint for the value of attribute adjPC.

In case the lsTimersProfilePackage is used, a create or set request is rejected, if:

i)	the lsTimersProfilePointer does not reference a lsTimersProfile; or

ii)	the lsTimersProfilePointer would reference an instance which does not exist.

If the name package is supported: a create request with a value for the name attribute that is already used by another instance of the same object class will be rejected.

If it is tried to delete a signLinkSetTp, which is referenced by at least one signRouteNePart, the deletion of the signLinkSetTp is rejected.

###### Attributes

1)	signLinkSetTpId: this attribute identifies a particular instance of this class.

2)	adjPc: this attribute represents the point code of the signalling point to which this linkset is connected. This attribute is read-only.

3)	maxCapacityLS: the maximum capacity is reflected in this attribute. This attribute is read and optionally writable. The maximum capacity of a signalling linkset is the maximum load that should be placed on the linkset, when all links that could be active in the linkset are, and are working in service.

4)	currentCapacityLS: the current capacity for the linkset is reflected in this attribute. It is read-only. The current capacity of signalling linkset is the sum over the current capacities of the signalling links (or over the maximum capacities where the current capacities of the links of the linkset are not defined) which are active, working and in service.

5)	congestionControlMethod: this attribute reflects the congestion control method used within a signalling point. It is set per SP when the first signLinkSetTp is created within the mtpSignPoint.

6)	OSI states operationalState, usageState and availabilityStatus: these attributes represent the OSI-management states. The administrativeState is optional.

7)	periodicLinkTestFlag: this attribute indicates whether the periodic test procedure of Recommendation Q.707 is applied to the links in this signalling linkset.

8)	alarmSeverityAssignmentProfilePointer: (optional).

9)	inLsLoadShareAlgorithm: (optional) the specific algorithm which is used for assignment of the SLS codes is indicated by this attribute. The value of this attribute is vendor-dependent. This optional attribute is optionally writable.

10)	lsTimersProfilePointer: (optional) this attribute indicates the timer profile used for this particular MTP signalling linkset. It is an optional attribute.

11)	numberOfNormallyActiveSignLinkTps: (optional) this attribute represents the number of normally active signalling links in a linkset, as defined in clause 12/Q.704.

12)	signLinkSetTpName: (optional) attribute for additional name for signLinkSetTps.

###### Notifications

1)	stateChange: this notification is emitted when a state change has occurred.

2)	attributeValueChange: (optional) this notification is emitted if an attribute value changes.

3)	objectCreation: (optional) this notification is emitted if an instance is created.

4)	objectDeletion: (optional) this notification is emitted if an instance is deleted.

5)	communicationsAlarm: for communication alarms emitted from this MOC, the following probable cause can apply: Probable Cause = 403 (LinkSetFailure), which represents measurements Q.752/4.3 (PerceivedSeverity = Maj/Min/War) and Q.752/4.4 (PerceivedSeverity = Cleared).

### 6.2.9	Signalling Link Termination PointXE "Signalling Link Termination Point"§

| signLinkTp |  |  |
| --- | --- | --- |
| Attributes | Notifications | Actions |
| slCode | stateChange | localInhibit |
| slsCodeCurrentList |  | localUninhibit |
| maxCapacitySL |  |  |
| currentCapacitySL |  |  |
| linkTpStatus |  |  |
| administrativeState |  |  |
| operationalState |  |  |
| usageState |  |  |
| proceduralStatus |  |  |
| mtpL2ProtocolProfilePointer |  |  |
| signTermPointer |  |  |
| signDataLinkTpPointer |  |  |
| "ITU-T Rec. M.3100 (1992)": alarmSeverityAssignmentPointerPackage (C) |  |  |
| alarmSeverityAssignmentProfilePointer |  |  |
| "ITU-T Rec. M.3100 (1992)": attributeValueChangeNotificationPackage (C) |  |  |
|  | attributeValueChange |  |
| "ITU-T Rec. M.3100 (1992)": createDeleteNotificationsPackage (C) |  |  |
|  | objectCreation |  |
|  | objectDeletion |  |
| communicationsAlarmPackage (C) |  |  |
|  | communicationsAlarm |  |
| linkCongestionLevelPackage (C) |  |  |
| linkCongestionLevel |  |  |
| relatedLinkGroupNumberPackage (C) |  |  |
| relatedLinkGroupNumber |  |  |
| signDataLinkTpListPackage (C) |  |  |
| signDataLinkTpList |  |  |
| signLinkTestPackage (C) |  |  |
|  |  | signLinkTest |

| signLinkTp |  |  |
| --- | --- | --- |
| Attributes | Notifications | Actions |
| signLinkTpNamePackage (C) |  |  |
| signLinkTpName |  |  |
| signTermListPackage (C) |  |  |
| signTermList |  |  |
| slsCodeNormalListPackage (C) |  |  |
| slsCodeNormalList |  |  |
| slTimersProfilePointerPackage (C) |  |  |
| slTimersProfilePointer |  |  |
| ss7OnOccEventPackage (C) |  |  |
|  | ss7OnOccEvent |  |

This managed object class models the part of a signalling link that is residing in the NE.

A create or set request is rejected, if:

i)	a pointer would reference an instance, which does not belong to the appropriate object class; or,

ii)	a pointer would reference an instance which does not exist; or

iii)	a signDataLinkTpPointer would reference a signDataLinkTp which is not in the same mtpSignPoint; or

iv)	a signDataLinkTpPointer would reference a signDataLinkTp which has a different value of adjPc than the superior signLinkSetTp; or

v)	the link would reference a datalink and a mtpL2ProtocolProfile whose transmission rates are not the same; or

vi)	the link would reference a mtpL2ProtocolProfile where the appropriate packages for the congestionControlMethod of the superior signLinkSetTp are not present (see B.1.2.9.2); or

vii)	the spTimersProfilePackage is present in the superior mtpSignPoint AND the link would reference a mtpL2ProtocolProfile whose l2TimerT2 is not bigger than the q704-t17 of the spTimersProfile referenced by the mtpSignPoint; or

viii)	the spTimersPackage is present in the superior mtpSignPoint AND the link would reference a mtpL2ProtocolProfile whose l2TimerT2 is not bigger than the q704-t17 of the mtpSignPoint.

This rejection also takes place if only a single pointer within the signDataLinkList or the signTermList (if present) is wrong.

A delete request is rejected, if the linkTpStatus does not contain the value deactivated.

If the name package is supported: a create request with a value for the name attribute that is already used by another instance of the same object class will be rejected.

###### Attributes

1)	slCode: the Signalling Link Code (SLC) is the same value (between 0 and 15) at each end of the link, and is different from that of any other link between the same two adjacent signalling points.

2)	slsCodeCurrentList: this attribute represents the SLS-Code which is currently used on the signLinkTp. It may be different from the slsCodeNormalList, in case some fault has occurred. This attribute is read-only.

3)	maxCapacitySL: this attribute describes the maximum capacity for the signLinkTp. The attribute is read-only. The maximum capacity of a signalling link is the maximum load that should be placed on the signalling link.

4)	currentCapacitySL: this attribute represents the current capacity. It is read-only. The current capacity of a signalling link (implementation dependent, consequently optional attribute) is the maximum load that should be placed on the signalling link given the present configuration of Signalling Data Links (SDLs) to signalling terminals. Could be used where Signalling Terminal (ST) capacities might be different in an implementation and automatic allocation of SDL to ST can be done, and/or where a signalling terminal could control several signalling links.

5)	linkTpStatus: this is a set-valued attribute. It contains the SS No. 7 functional statuses as described in Recommendation Q.704. Values are local blocked, remote blocked, local inhibited, remote inhibited, failed and deactivated.

6)	OSI states administrativeState, operationalState, usageState and proceduralStatus: these attributes represent the OSI-management states of this MOC.

7)	mtpL2ProtocolProfilePointer: this attribute refers to an instance of the mtpL2ProtocolProfile managed object class used for this signalling link termination point.

8)	signTermPointer: this attribute identifies the signTerm that is allocated to this signLinkTp.

9)	signDataLinkTpPointer: this attribute identifies the signDataLinkTp that is allocated to this signLinkTp.

10)	alarmSeverityAssignmentProfilePointer: (optional).

11)	linkCongestionLevel: (optional) this attribute is used to reflect link congestion, if multiple congestion levels are used. Its value in case of no congestion is "none". Its value can change from system inside.

12)	relatedLinkGroupNumber: this attribute identifies the link group which the signalling link belongs to. It is an optional attribute.

13)	signDataLinkTpList: (optional) this attribute lists the signDataLink instances that can be used for dynamical allocation to the signLinkTp.

14)	signLinkTpName: (optional) attribute for additional name for signLinkTps.

15)	signTermList: (optional) this attribute lists the signTerm instances that can be used for dynamical allocation to the signLinkTp.

16)	slsCodeNormalList: this attribute indicates which SLS-Codes are initially administratively assigned to this signLinkTp for the normal operation. This optional attribute is readable and writable.

17)	slTimersProfilePointer: this attribute indicates the timer profile used for this particular MTP signalling link. It is an optional attribute.

###### Notifications

1)	stateChange: if a state attribute changes, then the OS will be informed by this notification.

2)	attributeValueChange: (optional) this notification is emitted if an attribute value changes.

3)	objectCreation: (optional) this notification is emitted if an instance is created.

4)	objectDeletion: (optional) this notification is emitted if an instance is deleted.

5)	communicationsAlarm: (optional) for communication alarms emitted from this class, the following probable cause can apply: Probable Cause = 102 (slFailure) which represents measurement Q.752/1.2 – measurement Q.752/1.6 (SpecificProblems = 003 abnormalFIBRorBSNR, 004 excessiveAckDelay, 005 excessiveErrorRate, 006 excessiveCongDuration) and Q.752/1.12 (PerceivedSeverity = Cleared).

6)	ss7OnOccEvent: (optional) notification for which the following probable causes can apply:

Probable Cause = 110 (localChangeOver) which represents measurement Q.752/1.10 (PerceivedSeverity = Maj/Min/War) and measurement Q.752/1.11 (PerceivedSeverity = Cleared).

Probable Cause = 210 (remoteProcOutage) which represents measurements Q.752/2.10 (PerceivedSeverity = Maj/Min/War) and Q.752/2.11 (PerceivedSeverity = Cleared).

Probable Cause = 216 (start of local inhibition) which represents measurement Q.752/2.16 (PerceivedSeverity = Maj/Min/War) and Q.752/2.17 (PerceivedSeverity = Cleared).

Probable Cause = 218 (start of remote inhibition) which represents measurement Q.752/2.18 (PerceivedSeverity = Maj/Min/War) and Q.752/2.19 (PerceivedSeverity = Cleared).

###### Actions

1)	localInhibit: this action inhibits the signalling link.

2)	localUninhibit: this action uninhibits the signalling link.

3)	signLinkTest: this optional action initiates the signalling link test defined in Recommendation Q.707.

### 6.2.10	Signalling Link Timer ProfileXE "Signalling Link Timer Profile"§

| slTimersProfile |  |  |
| --- | --- | --- |
| Attributes | Notifications | Actions |
| slTimersProfileId |  |  |
| q704-t1 |  |  |
| q704-t3 |  |  |
| q704-t17 |  |  |
| q704-t24a) |  |  |
| "ITU-T Rec. M.3100 (1992)": attributeValueChangeNotificationPackage (C) |  |  |
|  | attributeValueChange |  |
| "ITU-T Rec. M.3100 (1992)": createDeleteNotificationsPackage (C) |  |  |
|  | objectCreation |  |
|  | objectDeletion |  |
| slTimersProfileNamePackage (C) |  |  |
| slTimersProfileName |  |  |
| a) This timer is a national option. It is implementation dependent what value it takes when it is not used. |  |  |

The slTimersProfile represents the Q.704 timers that are associated to a signalling link. slTimersProfiles are contained in the switching element. There can exist slTimersProfiles within one switching element and multiple signalling links can share a given slTimersProfile. The use of this MOC is optional. Alternatively, the timer attributes can all be part of the mtpSignPoint MOC.

If the name package is supported: a create request with a value for the name attribute that is already used by another instance of the same object class will be rejected.

If an attempt is made to delete an slTimersProfile which is still referenced by a signLinkTp the delete request will be rejected.

###### Attributes

1)	slTimersProfileId.

2)	q704-t1: Q.704 timer "Delay to avoid message mis-sequencing on changeover". Constraint: the same value applies for all links associated with an instance of the procedure.

3)	q704-t3: Q.704 timer "Time controlled diversion-delay to avoid mis-sequencing on change back". The value of this attribute is network implementation dependent.

4)	q704-t17: Q.704 timer "Delay to avoid oscillation of initial alignment failure and link restart".

5)	q704-t24: Q.704 timer "Stabilizing timer after removal of local processor outage, used in LPO latching to RPO (national option)".

6)	slTimersProfileName: (optional) attribute for additional name for slTimerProfile.

###### Notifications

1)	attributeValueChange: (optional) this notification is emitted if an attribute value changes.

2)	objectCreation: (optional) this notification is emitted if an instance is created.

3)	objectDeletion: (optional) this notification is emitted if an instance is deleted.

### 6.2.11	Signalling Point Timers ProfileXE "Signalling Point Timers Profile"§

| spTimersProfile |  |  |
| --- | --- | --- |
| Attributes | Notifications | Actions |
| spTimersProfileId |  |  |
| q704-t2 |  |  |
| q704-t4 |  |  |
| q704-t5 |  |  |
| q704-t7a) |  |  |
| q704-t11a) |  |  |
| q704-t12 |  |  |
| q704-t13 |  |  |
| q704-t14 |  |  |
| q704-t15a) |  |  |
| q704-t16a) |  |  |
| q704-t18 |  |  |
| q704-t19 |  |  |
| q704-t20 |  |  |
| q704-t21 |  |  |
| q704-t22 |  |  |
| q704-t23 |  |  |

| spTimersProfile |  |  |
| --- | --- | --- |
| Attributes | Notifications | Actions |
| q707-t1 |  |  |
| q707-t2 |  |  |
| "ITU-T Rec. M.3100 (1992)": attributeValueChangeNotificationPackage (C) |  |  |
|  | attributeValueChange |  |
| "ITU-T Rec. M.3100 (1992)": createDeleteNotificationsPackage (C) |  |  |
|  | objectCreation |  |
|  | objectDeletion |  |
| spTimersProfileNamePackage (C) |  |  |
| spTimersProfileName |  |  |
| a) These timers are national options. It is implementation dependent what value they take when they are not used. |  |  |

The spTimersProfile represents the Q.704 timers that are associated to a signalling point. spTimersProfiles are contained in the switching element. There can exist multiple spTimersProfiles within one switching element and multiple signalling points can share a given spTimersProfiles. The use of this MOC is optional. Alternatively, the timer attributes can all be part of the mtpSignPoint MOC.

If the name package is supported: a create request with a value for the name attribute that is already used by another instance of the same object class will be rejected.

If an attempt is made to delete an spTimersProfile which is still referenced by a mtpSignPoint the delete request will be rejected.

###### Attributes

1)	spTimersProfileId: this attribute identifies an instance of this MOC.

2)	q704-t2: Q.704 timer "Waiting for changeover acknowledgement".

3)	q704-t4: Q.704 timer "Waiting for change back acknowledgement (first attempt)".

4)	q704-t5: Q.704 timer "Waiting for change back acknowledgement (second attempt)".

5)	q704-t7: Q.704 timer "Waiting for signalling data link connection acknowledgement".

6)	q704-t11: Q.704 timer "Transfer restricted timer. (This is one way of implementing the function described in 13.4/Q.704 and mainly intended to simplify STPs)".

7)	q704-t12: Q.704 timer "Waiting for uninhibit acknowledgement".

8)	q704-t13: Q.704 timer "Waiting for force uninhibit".

9)	q704-t14: Q.704 timer "Waiting for inhibition acknowledgement".

10)	q704-t15: Q.704 timer "Waiting to start signalling routeset congestion test".

11)	q704-t16: Q.704 timer "Waiting for routeset congestion status update".

12)	q704-t18: Q.704 timer "Timer within a signalling point whose MTP restarts, for supervising link and linkset activation as well as the receipt of routing information".

13)	q704-t19: Q.704 timer "Supervision timer during MTP restart to avoid possible ping-pong of TFP, TFR3) and TRA messages".

14)	q704-t20: Q.704 timer "Overall MTP restart timer at the signalling point whose MTP restarts. Overall MTP restart timer at a signalling point adjacent to one whose MTP restarts".

15)	q704-t21: Q.704 timer "Overall MTP restart timer at a signalling point adjacent to one whose MTP restarts".

16)	q704-t22: Q.704 timer "Local inhibit test timer".

17)	q704-t23: Q.704 timer "Remote inhibit test timer".

18)	q707-t1: Q.707 timer T1.

19)	q707-t2: Q.707 timer T2.

20)	spTimersProfileName: (optional) attribute for additional name for spTimerProfile.

Recommendation Q.751.1     (10/95)

###### Notifications

1)	attributeValueChange: (optional) this notification is emitted if an attribute value changes.

2)	objectCreation: (optional) this notification is emitted if an instance is created.

3)	objectDeletion: (optional) this notification is emitted if an instance is deleted.

### 6.2.12	Signalling Route Network Element PartXE "Signalling Route Network Element Part"§

| signRouteNePart |  |  |
| --- | --- | --- |
| Attributes | Notifications | Actions |
| signRouteNePartId | stateChange |  |
| signLinkSetTpPointer |  |  |
| administrativeState |  |  |
| operationalState |  |  |
| availabilityStatus |  |  |
| "ITU-T Rec. M.3100 (1992)":attributeValueChangeNotificationPackage (C) |  |  |
|  | attributeValueChange |  |
| "ITU-T Rec. M.3100 (1992)": createDeleteNotificationsPackage (C) |  |  |
|  | objectCreation |  |
|  | objectDeletion |  |
| clsLoadsharingInformationPackage (C) |  |  |
| inClsLoadsharingAlgorithm |  |  |
| fixedRouteNePartPriorityPackage (C) |  |  |
| fixedPriority |  |  |
| flexibleRouteNePartPriorityPackage (C) |  |  |
| flexiblePriority | attributeValueChange |  |
| priorityMode |  |  |
| loadsharingInformationRouteNePartPackage (C) |  |  |
| loadsharingInformationRouteNePart |  |  |
| mtpLoadSharingObjectForRouteNePartPointerPackage (C) |  |  |
| mtpLoadSharingObjectForRouteNePartPointer |  |  |

| signRouteNePart |  |  |
| --- | --- | --- |
| Attributes | Notifications | Actions |
| signRouteNePartNamePackage (C) |  |  |
| signRouteNePartName |  |  |
| slsListPackage (C) |  |  |
| slsList |  |  |
| usageStatePackage (C) |  |  |
| usageState |  |  |

The multiple instantiable MOC signRouteNePart is contained in the MOC signRouteSetNePart. In the signRouteNePart instances contained in the same signRouteSetNePart information is in existence via which signLinkSetTp instances – as possible first segment of the route in network view – messages are intended to be routed and actually can be routed to the respective destination signalling point. Each single signRouteNePart instance contains the respective information for a specific signLinkSetTp.

If changes of these information on network element level are to take place, coordination on network level is necessary because of the broad effect of these changes within the network.

A create request is rejected, if the signLinkSetTpPointer would reference an instance, which:

i)	does not belong to the signLinkSetTp object class; or

ii)	does not exist; or

iii)	is not contained in the same mtpSignPoint as the signRouteNePart; or

iv)	is already referenced by a signRouteNePart contained within the same signRouteSetNePart.

A delete request is rejected if its administrativeState does not equal locked.

If the name package is supported: a create request with a value for the name attribute that is already used by another instance of the same object class will be rejected.

###### Attributes

1)	signRouteNePartId: this is the naming attribute of signRouteNePart. It identifies a signRouteNePart instance within the signRouteSet. This attribute is defined at creation time and is only readable.

2)	signLinkSetTpPointer: references the signLinkSetTp which is intended to be used as first segment of the succession of linksets, which form the signalling route on the network level. This attribute is only readable.

3)	administrativeState: describes whether it is administratively permitted to route messages towards the respective destination point via the referenced signLinkSetTp. Before the administrative state of a signRouteNePart, which is aiming towards a non-adjacent signalling point, can be set to "unlocked", a direct linkset must be used for a route towards the respective adjacent point.

This attribute is readable and writable.

4)	operationalState: describes whether the respective destination signalling point is accessible – (="enabled") or not (="disabled") via the referenced signLinkSetTp.

For the management system this attribute is only readable. Its value can change from system inside.

The value is "enabled", if the administrativeState = unlocked, the referenced signLinkSetTp is in the operational state "enabled" and no TFP messages concerning the respective destination and the respective signLinkSetTp are valid.

5)	availabilityStatus: this attribute gives additional information about the availability of the signRouteNePart. It is furthermore needed to map some of the MTP functional states (prohibited: availabilityStatus = {dependency}, restricted: availabilityStatus = {degraded}) and is used for the case that the route is locked not by the OS, but, for example, by a local administrator (administrativeState = unlocked and availabilityStatus = {off line}).

6)	inClsLoadsharingAlgorithm: (optional) this attribute represents the over- and in-linkset loadsharing algorithm within a combined linkset.

Attributes for priority: these attributes determine, if the signRouteNePart is used as current route. The signRouteNePart instances with operational state = "enabled" contained in the same signRouteSetNePart are chosen in ascending order as current routes (The lower the value, the higher the priority).

There are different behaviours for the handling of "priority gaps" possible:

i)	priorityGapAllowedBehaviour;

ii)	priorityGapNotAllowedBehaviour.

In the latter case it is not possible to have a priority value, if there is a lower value, which is not used in one of the routes of the routeset. For example, in case of deleting a signRouteNePart instance whose priority value is not equal to the priority of at least one of the remaining signRouteNeParts contained in the same signRouteSetNePart, the priority values of the remaining instances have to be decremented by one (see also example in table below).

What kind of priority gap behaviour is performed by the switch is to be derived from the combination of the values of the attributes vendorName and version of the managedSwitchingElement.

For implementation dependent purposes this attribute can have the same value for different signRouteNePart instances contained in the same signRouteSetNePart.

This attribute is present in two conditional packages, of which only one but exact one is to be used.

In the conditional package "fixedRouteNePartPriorityPackage" this attribute is only readable and is named:

7)	fixedPriority:

NOTE – The expression "fixed" does not pertain to the priority gap behaviour.

In the conditional package "flexibleRouteNePartPriorityPackage" the attribute is called:

8)	flexiblePriority: which is read – and – only together with the attribute priorityMode – writable. In this case an attributeValueChange-Notification for this attribute is mandatory, because the value of this attribute can change due to management actions on another signRouteNePart instance.

9)	priorityMode: this attribute is part of the flexibleRouteNePartPriorityPackage. It describes the influence of the management action concerning one specific signRouteNePart on the priority attributes of other signRouteNePart instances contained in the same signRouteSetNePart in case of creating a signRouteNePart instance or modifying its priority attribute.

This attribute is only writable, and this only together with the writing of attribute priority. This attribute is needed for the following reason: The interdependence of the attribute priority with the same attribute of other instances contained in the same signRouteSetNePart forces specific behaviours for creating a signRouteNePart instance and for modifying the priority of a signRouteNePart object instance.

If a new signRouteNePart instance (route) is to be created with a priority attribute value which exists in one or more of the created routes from the same signRouteSetNePart, there are the following three possibilities to put the new route in the priority order:

i)	either the new route shall join the others with the same priority (priorityMode = EQUAL); or

ii)	it shall displace – if necessary – the other priorities, i.e. for the priorities of all created routes from the same signRouteSetNePart with equal or lower priority the priority value has to be incremented by one (priorityMode = INSERT);

iii)	or for priorityMode = EXCHANGE_SINGLE the new route gets the specified priority and any route which had this priority before gets the lowest priority value which is not used by a route within the routeset.

When modifying the priority attribute of a route from a "starting" value to a "target" value, which already exists in one of the created routes contained in the same signRouteSetNePart, there are four variations possible:

i)	Joining (priorityMode = EQUAL) – see above.

ii)	Displacing (priorityMode = INSERT) – see above.

iii)	Exchanging with priority mode = "EXCHANGE_SINGLE" – see below.

iv)	Exchanging with priority mode = "EXCHANGE_GROUP" – see below.

"EXCHANGE_GROUP" is necessary to allow exchanging of priorities without exceeding the limit for the number of routes with the same priority attribute value.

Concerning iii) and iv) above:

If by modifying the priority with one of the EXCHANGE priority modes to a "target" value which is present in several routes of a signRouteSetNePart, then all the priority values of these routes are exchanged for the last ("starting") priority value of the specified route.

In case of "EXCHANGE_GROUP", the priorities of all routes which have the same "starting" priority attribute value as the specified route are changed to the "target" value.

In case of priorityMode "EXCHANGE_SINGLE" only the priority of the specified route is changed to the "target" value.

If by modifying the priority with "EQUAL" a "priority gap" would occur, then for the priorities of all routes with priorities lower than the "gap priority" the priority value must be decremented by one, if the priorityGapNotAllowedBehaviour is used.

10)	loadsharingInformationRouteNePart: this optional attribute contains specific information for target specific loadsharing via the current routes working on a route basis. This information can be used by, for example, the referenced mtpLoadsharingObjectForRoute to execute the target specific loadsharing.

11)	mtpLoadsharingObjectForRouteNePartPointer: this attribute is optional and is used to reference implementation dependent loadsharing MOC instances needed to model implementation dependent mechanisms for loadsharing over linksets working on a route basis (e.g. such an object can represent a loadsharing collection with specific characteristics to which the route belongs).

12)	signRouteNePartName: (optional) attribute for assigning an additional name to the signRouteNePart.

13)	slsList: this optional attribute has to be used if loadsharing over linksets toward a specific destination is done. It indicates which SLSs are assigned to this signRouteNePart, in case it is a current route (i.e. the messages with these SLSs are sent over the allocated linkset).

It has to be ensured that all SLSs are covered and no SLS exists more than one time within the slsList attributes of the current routes.

The attribute is read-only and can be written from system inside.

14)	usageState: this optional attribute is used to cover restriction of a route. Its value is set to "busy", if the route is restricted ("transfer restricted" is a national option).

The attribute is only readable.

###### Notifications

1)	stateChange: in case one of the state-attributes changes this notification is emitted. The notification will contain the old and the new value of the related state-attributes.

2)	objectCreation: (optional) this notification is emitted if an instance is created.

3)	objectDeletion: (optional) this notification is emitted if an instance is deleted.

4)	attributeValueChange: (optional) this notification is emitted if an attribute value changes.

### 6.2.13	Signalling Routeset Network Element PartXE "Signalling Routeset Network Element Part"§

| signRouteSetNePart |  |  |
| --- | --- | --- |
| Attributes | Notifications | Actions |
| signRouteSetNePartId | stateChange |  |
| administrativeState |  |  |
| operationalState |  |  |
| "ITU-T Rec. M.3100 (1992)":alarmSeverityAssignmentPointerPackage (C) |  |  |
| alarmSeverityAssignmentProfilePointer |  |  |
| "ITU-T Rec. M.3100 (1992)": attributeValueChangeNotificationPackage (C) |  |  |
|  | attributeValueChange |  |
| "ITU-T Rec. M.3100 (1992)":createDeleteNotificationsPackage (C) |  |  |
|  | objectCreation |  |
|  | objectDeletion |  |
| communicationsAlarmPackage (C) |  |  |
|  | communicationsAlarm |  |
| congestedStatePackage (C) |  |  |
| congestedState | attributeValueChange |  |
| congestionLevelPackage (C) |  |  |
| congestionLevel |  |  |
| loadsharingInformationRouteSetNePartPackage (C) |  |  |
| loadsharingInformationRouteSetNePart |  |  |
| mtpLoadSharingObjectForRouteSetNePartPointerPackage (C) |  |  |
| mtpLoadSharingObjectForRouteSetNePartPointer |  |  |
| remoteExchangeLabelPackage |  |  |
| remoteExchangeLabel |  |  |
| signRouteSetNePartNamePackage (C) |  |  |
| signRouteSetNePartName |  |  |
| ss7OnOccEventPackage (C) |  |  |
|  | ss7OnOccEvent |  |

The multiple instantiable MOC signRouteSetNePart is contained in the MOC mtpSignPoint. It groups together information which is available in the own network element A concerning a specific signalling point of the network element B within the same network. This information comprises among others the identification of the destination (the signalling point of the network element B), its accessibility from the own mtpSignPoint of network element A via the SS No. 7 network and if it is allowed to route messages towards this signalling destination.

If changes of these information on network element level are to take place by the management, coordination on network level is necessary because of the broad effect of these changes within the network.

The behaviour in case of inaccessibility or re-accessibility of the respective destination depends, if it is an adjacent point or not. The behaviour comprehends the respective MTP actions (like sending TFP, TFA etc.).

If the name package is supported: a create request with a value for the name attribute that is already used by another instance of the same object class will be rejected.

A delete request is rejected, if the administrativeState of the signRouteSetNePart does not equal locked. A check would be needed before deletion, if a routeset is still in a routing table of a user part.

###### Attributes

1)	signRouteSetNePartId: this is the naming attribute of signRouteSetNePart. It identifies a signRouteSetNePart instance within the mtpSignPoint and equals the signalling point code of the respective destination signalling point. This attribute is defined at creation time and is only readable.

2)	administrativeState: describes whether it is administratively permitted to route SS No. 7 MSUs towards the respective destination point. This attribute is readable and writable.

3)	operationalState: this attribute describes whether the respective destination signalling point is accessible – (="enabled") or not (="disabled"). For the management system this attribute is only readable. Its value can change from system inside.

4)	alarmSeverityAssignmentProfilePointer: (optional).

The following two attributes are optional, but one of them has to be present.

5)	congestedState: this attribute is used to reflect routeset congestion, if no multiple congestion levels are used. Its value is "congested" in case of routeset congestion. For the management system this attribute is only readable. Its value can change from system inside.

6)	congestionLevel: this attribute is used to reflect routeset congestion, if multiple congestion levels are used. Its value in case of no congestion is "none". For the management system this attribute is only readable. Its value can change from system inside.

7)	loadsharingInformationRouteSetNePart: this optional attribute contains specific information for target specific loadsharing via the current routes working on a routeset basis via the current routes. This information can be used by, for example, the referenced mtpLoadsharingObjectForRouteSetNePart to execute the target specific loadsharing. (If this object is for example the representation of a loadsharing algorithm, this information can contain parameters for this algorithm.)

8)	mtpLoadsharingObjectForRouteSetNePartPointer: this attribute is optional and is used to reference implementation dependent loadsharing MOC instances needed to model implementation dependent mechanisms for loadsharing over linksets working on a routeset basis.

9)	remoteExchangeLabel: this optional attribute specifies the remote exchange which contains the mtpSignPoint with the point code equal to the signRouteSetNePartId. The attribute is read- and writable.

10)	signRouteSetNePartName: this optional attribute additionally identifies a signRouteSetNePart instance within the managed object class; its value is unique within the signRouteSetNePart MOC. If at creation time the value of this attribute given for the instance to be created already exists in another signRouteNePart instance within the managedSwitchingElement, then the creation is rejected and not carried out. The attribute is read- and writable.

###### Notifications

1)	stateChange: if a state attribute changes, then this notification is emitted.

2)	attributeValueChange: (optional) this notification is emitted if an attribute value changes.

3)	objectCreation: (optional) this notification is emitted if an instance is created.

4)	objectDeletion: (optional) this notification is emitted if an instance is deleted.

5)	communicationsAlarm: for notifications emitted from this class, the following probable causes can apply:

Probable Cause = 411 (RouteSetUnavailable), which represents measurements Q.752/4.11 (PerceivedSeverity = Maj/Min/War) and Q.751/4.12 (PerceivedSeverity = Cleared).

6)	ss7OnOccEvent: (optional) event notification for which the following probable causes can apply:

Probable Cause = 002 (remoteUserPartUnavailable), with the involved user part as parameter. Possible values for SpecificProblems are: 007 "unknown", 008 "unequipped" and 009 "inaccessibleRemoteUser".

### 6.2.14	Signalling TerminalXE "Signalling Terminal"§

| signTerm |  |  |
| --- | --- | --- |
| Attributes | Notifications | Actions |
| signTermId | stateChange |  |
| administrativeState |  |  |
| operationalState |  |  |
| usageState |  |  |
| proceduralStatus |  |  |
| availabilityStatus |  |  |
| equipmentPointer |  |  |
| "ITU-T Rec. M.3100 (1992)": attributeValueChangeNotificationPackage (C) |  |  |
|  | attributeValueChange |  |
| "ITU-T Rec. M.3100 (1992)":createDeleteNotificationsPackage (C) |  |  |
|  | objectCreation |  |
|  | objectDeletion |  |
| signTermNamePackage (C) |  |  |
| signTermName |  |  |

The signTerm MOC defines the management capabilities for the signalling terminal. The signalling terminal is defined in Recommendation Q.703.

A create request is rejected if the equipmentPointer would reference equipment that does not exist.

If the name package is supported: a create request with a value for the name attribute that is already used by another instance of the same object class will be rejected.

###### Attributes

1)	signTermId: this attribute identifies an instance of this MOC.

2)	OSI states administrativeState, operationalState, usageState, proceduralStatus and availabilityStatus: this attribute identifies the OSI-management state.

3)	equipmentPointer: this attribute references the physical equipment of the signalling terminal.

4)	signTermName: (optional) attribute for assigning an additional name to the signTerm.

###### Notifications

1)	stateChange: if a state change occurs, then this notification will be emitted.

2)	attributeValueChange: (optional) this notification is emitted if an attribute value changes.

3)	objectCreation: (optional) this notification is emitted if an instance is created.

4)	objectDeletion: (optional) this notification is emitted if an instance is deleted.

### 6.2.15	STP Screening TableXE "STP Screening Table"§

| stpScreeningTable |  |  |
| --- | --- | --- |
| Attributes | Notifications | Actions |
| stpScreeningTableId | stateChange |  |
| administrativeState |  |  |
| treatmentOfOutsideRanges |  |  |
| stpScreeningTableListMode |  |  |
| screeningByLinkSetOrByOpc |  |  |
| getScreenedOpcsOrLinkSetsByDpcActionPackage (C) |  |  |
|  |  | getScreenedOpcsOrLinkSetsByDpc |
| "ITU-T Rec. M.3100 (1992)": attributeValueChangeNotificationPackage (C) |  |  |
|  | attributeValueChange |  |
| "ITU-T Rec. M.3100 (1992)":createDeleteNotificationsPackage (C) |  |  |
|  | objectCreation |  |
|  | objectDeletion |  |
| stpScreeningTableNamePackage (C) |  |  |
| stpScreeningTableName |  |  |

The MOC STP Screening Table is introduced to meet the requirements of 8.2/Q.705 and 8.3/Q.705. Clause 8/Q.705 offers an optional functionality in order to identify and to handle unauthorized SS No.7 messages. For this purpose a lot of information is necessary (e.g. three different types of tables which are usable in several ways). For the Operator/Administration it must be possible to establish and to modify these information. This MOC is the base for this function. Together with its subordinate stpScreeningTableLine instances it contains all the information necessary for an STP to identify and to handle unauthorized SS No. 7 messages. Since the prevention of unauthorized use of an STP is an optional function this managed object is optional as well. If no stpScreeningTableInstance does exist, no STP screening takes place within the superior signPoint. The options to identify unauthorized SS No. 7 messages (8.2/Q.705) are: OPC/DPC, ILS/DPC and OLS/DPC policing. The option is chosen by the attribute screeningByLinkSetOrByOpc.

If the name package is supported: a create request with a value for the name attribute that is already used by another instance of the same object class will be rejected.

###### Attributes

The managed object class STP Screening Table contains the following attributes :

1)	stpScreeningTableId: this attribute identifies an instance of this managed object class. It is defined at creation time and only readable.

2)	administrativeState: this attribute represents the OSI administrative state as defined in Recommendation X.731. It has the initial value "locked".

3)	treatmentOfOutsideRanges: this attribute determines the treatment of the messages which are outside the designated ranges. This attribute is single-valued and read-write-able.

4)	stpScreeningTableListMode: this attribute defines how the content of the subordinate stpScreeningTableLine’s has to be used. It indicates whether the STP access is allowed (TRUE) or not (FALSE) by the designated combinations. This attribute is single-valued and read-write-able.

5)	screeningByLinkSetOrByOpc: this attribute defines which of the different options to identify unauthorized SS No. 7 messages (8.2/Q.705) is represented by this list. This attribute is single-valued, defined at creation time and only readable.

6)	stpScreeningTableName: (optional) attribute for assigning an additional name to the stpScreeningTable.

###### Notifications

1)	stateChange: this notification is emitted if a state attribute changes.

2)	attributeValueChange: this notification is emitted if an attribute value changes.

3)	objectCreation: this notification is emitted if an instance is created.

4)	objectDeletion: this notification is emitted if an instance is deleted.

###### Action

–	getScreenedOpcsOrLinkSetsByDpc: this (optional) action gets all OPCs respectively linksets (depending on the value of the screeningByLinkSetOrByOPC attribute) together with their respective message treatments which are screened in combination with a specific DPC within all the stpScreeningTableLineContents of all stpScreeningTableLine instances contained in the stpScreeningTable.

The resulting list in the reply is empty, if no OPC or Linkset within all the stpScreeningTableLine-Contents of all stpScreeningTableLine instances contained in the stpScreeningTable is screened in combination with the DPC – given in the information syntax of the action –, or if no stpScreeningTableLine instance contained in the stpScreeningTable does exist.

### 6.2.16	stpScreeningTableLineXE "stpScreeningTableLine"§

| stpScreeningTableLine |  |  |
| --- | --- | --- |
| Attributes | Notifications | Actions |
| stpScreeningTableLineId |  |  |
| stpScreeningTableLineContent |  |  |
| "ITU-T Rec. M.3100 (1992)": attributeValueChangeNotificationPackage (C) |  |  |
|  | attributeValueChange |  |
| "ITU-T Rec. M.3100 (1992)":createDeleteNotificationsPackage (C) |  |  |
|  | objectCreation |  |
|  | objectDeletion |  |
| stpScreeningTableLineNamePackage (C) |  |  |
| stpScreeningTableLineName |  |  |

The multiple instantiable MOC STP Screening TableLine is introduced to meet the requirements of 8.2/Q.705 and 8.3/Q.705. Clause 8/Q.705 offers an optional functionality in order to identify and to handle unauthorized SS No.7 messages. For the Operator/Administration it must be possible to establish and to modify these information. This MOC together with MOC stpScreeningTable contains all the information necessary for an STP to identify and to handle unauthorized SS No. 7 messages. Since the prevention of unauthorized use of an STP is an optional function this managed object is optional as well.

The total number of screened DPC’s across all stpScreeningTableLine instances contained by a stpScreeningTable must be less or equal to maxNoOfScreenedDpcs. Respectively unallowed requests are rejected.

If the name package is supported: a create request with a value for the name attribute that is already used by another instance of the same object class will be rejected.

###### Attributes

The managed object class STP Screening Table Line contains the following attributes :

1)	stpScreeningTableLineId: this attribute identifies an instance of this managed object class. It is defined at creation time and only readable. Depending on the value of attribute screeningByLinkSetOrByDpc of the superior stpScreeningTable instance it represents the OPC or linkset which is designated to be screened for STP traffic. If one tries to create an instance with an stpScreeningTableLineId which would represent a linkset which is not contained within the same signPoint as the stpScreeningTable, the create request is rejected.

2)	stpScreeningTableLineContent: this attribute represents the content of one line of the stpScreeningTable. The content consists of elements indicating the designated DPCs which are to be screened in combination with the OPC or linkset given by the stpScreeningTableLineId and an indication how a message fulfilling such a combination is to be treated. This attribute is read-write-able, also adding and removing of one or more elements shall be possible.

The total number of screened DPC's across all stpScreeningTableLine instances contained by a stpScreeningTable must be less or equal to maxNoOfScreenedDpcs. Respectively unallowed requests are rejected.

3)	stpScreeningTableLineName: (optional) attribute for assigning an additional name to the stpScreeningTableLine.

###### Notifications

1)	attributeValueChange: this notification is emitted if an attribute value changes.

2)	objectCreation: this notification is emitted if an instance is created.

3)	objectDeletion: this notification is emitted if an instance is deleted.

# 7	Formal specification

In this clause the identified managed object classes are formally described using GDMO [X.721] and ASN.1 [X.208].

Measurements for MTP are described in Recommendation Q.752, Tables 1, 2, 3, 4, 5 and 6. The modelling of these measurements for management can be found in Annex E of this Recommendation.

## 7.1	Managed object classes definitions

### 7.1.1	Signalling Link Set Timers ProfileXE "Signalling Link Set Timers Profile"§

lsTimersProfile MANAGED OBJECT CLASS

DERIVED FROM "ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":top;

CHARACTERIZED BY lsTimersProfilePackage PACKAGE

BEHAVIOUR lsTimersProfileBehaviour BEHAVIOUR DEFINED AS

"A signalling link set timer profile is a set of Q.704 timers which are related to the characteristics of signalling links sets.

The managed object class lsTimersProfile is a support managed object class which can be instantiated in order to define a particular profile. At creation time of a signLinkSetTp one of the lsTimersProfiles is referenced.";;

ATTRIBUTES

lsTimersProfileId	GET SET BY CREATE,

q704-t6		GET SET BY CREATE,

q704-t8		GET SET BY CREATE,

q704-t10		GET SET BY CREATE;;;

CONDITIONAL PACKAGES

"ITU-T Rec. M.3100 (1992)":createDeleteNotificationsPackage PRESENT IF "the objectCreation and objectDeletion notifications defined in 'ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992' are supported by an instance of this class",

"ITU-T Rec. M.3100 (1992)":attributeValueChangeNotificationPackage PRESENT IF "the attributeValueChangeNotification defined in 'ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992' is supported by an instance of this class",

lsTimersProfileNamePackage PRESENT IF "an instance supports it";

REGISTERED AS { mtpObjectClass 1 };

### 7.1.2	Managed Switching ElementXE "Managed Switching Element"§

managedSwitchingElement MANAGED OBJECT CLASS

DERIVED FROM		"ITU-T Rec. M.3100 (1992)":managedElement;

CHARACTERIZED BY	managedSwitchingElementPackage PACKAGE

BEHAVIOUR		managedSwitchingElementBehaviour BEHAVIOUR DEFINED AS

"The Managed Switching Element represents, for management purposes, an exchange, i.e. the aggregate of traffic carrying devices, switching stages, controlling and signalling means at a network node that enables subscriber lines to be interconnected and packets to be forwarded as required by individual users.";;

ATTRIBUTES

"ITU-T Rec. M.3100 (1992)": vendorName	GET SET BY CREATE;;;

CONDITIONAL PACKAGES

managedSwitchingElementNamePackage PRESENT IF "an instance supports it";

REGISTERED AS { mtpObjectClass 2 };

### 7.1.3	MTP Access PointXE "MTP Access Point"§

mtpAccessPoint MANAGED OBJECT CLASS

DERIVED FROM "ITU-T Rec. X.283 (1993) | ISO/IEC 10733 : 1992":nSAP;

CHARACTERIZED BY mtpAccessPointPkg PACKAGE

BEHAVIOUR mtpAccessPointBehaviour BEHAVIOUR DEFINED AS

"The sap2Address attribute contains the address of the mtpAccessPoint. It contains the Signalling Point Code (SPC), Service Indicator (SI) and MTP Network Identity (NI). The sapId attribute is used in naming instances of the mtpAccessPoint managed object class.

The userEntityNames attribute contains the distinguished names of the managed objects that represent the user entities that are using the mtpAccessPoint, e.g. the SCCP instance. The providerEntityNames attribute contains the distinguished names of the managed objects that represent the provider entities that are supporting the mtpAccessPoint, i.e., the instance of the MTP.

A mtpAccessPoint can have the MTP status:

–	allowed, this means that the mtpAccessPoint is accessible and is functioning normally

–	congested, this means that the mtpAccessPoint is accessible but the path to it is heavily loaded

–	prohibited, this means that the mtpAccessPoint not accessible.

If the ss7OnOccEventPackage is present, event notification with the following probable causes can apply: Probable Cause = 001 (localUserPartUnavailable)";;;;

CONDITIONAL PACKAGES

"ITU-T Rec. M.3100 (1992)":alarmSeverityAssignmentPointerPackage PRESENT IF "an instance supports it and ss7OnOccEventPackage is present",

mtpAccessPointNamePackage PRESENT IF "an instance supports it",

ss7OnOccEventPackage PRESENT IF "an instance supports it";

REGISTERED AS { mtpObjectClass 3 };

### 7.1.4	MTP Level 2 Protocol ProfileXE "MTP Level 2 Protocol Profile"§

mtpL2ProtocolProfile MANAGED OBJECT CLASS

DERIVED FROM		"ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":top;

CHARACTERIZED BY	mtpL2ProtocolProfilePkg PACKAGE

BEHAVIOUR		mtpL2ProtocolProfileBehaviour BEHAVIOUR DEFINED AS

"An MTP level 2 protocol profile is a set of Q.703 parameters which defines the layer 2 characteristics of Signalling Links.

The managed object class mtpL2ProtocolProfile is a support managed object class which can be instantiated in order to define a particular profile. At creation time of a signLinkTp one of the mtpL2ProtocolProfiles is referenced.";;

ATTRIBUTES

mtpL2ProtocolProfileId	GET SET BY CREATE,

errorCorrectionMethod	GET SET BY CREATE,

transmissionRate		GET SET BY CREATE,

l2TimerT1		GET SET BY CREATE,

l2TimerT2		GET SET BY CREATE,

l2TimerT3		GET SET BY CREATE,

l2TimerT4N		GET SET BY CREATE,

l2TimerT4E		GET SET BY CREATE,

l2TimerT5		GET SET BY CREATE,

l2TimerT6		GET SET BY CREATE,

l2TimerT7		GET SET BY CREATE,

transmissionCongestionThresholdOnsetLevel1	GET SET BY CREATE,

transmissionCongestionThresholdAbatementLevel1	GET SET BY CREATE;;;

CONDITIONAL PACKAGES

"ITU-T Rec. M.3100 (1992)":attributeValueChangeNotificationPackage PRESENT IF "the attributeValueChangeNotification defined in 'ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992' is supported by an instance of this class",

"ITU-T Rec. M.3100 (1992)":createDeleteNotificationsPackage PRESENT IF "the objectCreation and objectDeletion notifications defined in 'ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992' are supported by an instance of this class",

bufferMechanismPackage PRESENT IF "an instance supports it",

congestionHandlingPackage PRESENT IF "an instance supports it",

loopDelayPackage PRESENT IF "an instance supports it",

mtpL2ProtocolProfileNamePackage PRESENT IF "an instance supports it",

multipleTransmissionCongestionLevelsPackage PRESENT IF "an instance supports it",

multipleTransmissionCongestionStatesPackage PRESENT IF "an instance supports it",

pCRPackage PRESENT IF "an instance supports it",

receiveCongestionLevelsPackage PRESENT IF "an instance supports it";

REGISTERED AS { mtpObjectClass 4 };

### 7.1.5	MTP Level 3XE "MTP Level 3"§

mtpLevel3 MANAGED OBJECT CLASS

DERIVED FROM	"ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":top;

CHARACTERIZED BY	mtpLevel3Package PACKAGE

BEHAVIOUR 	mtpLevel3Behaviour BEHAVIOUR DEFINED AS

"This managed object class models MTP Level 3 functionality.

If the 'ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992':communicationsAlarm notification is used, the following probable causes can apply:

Probable Cause = 501 (AdjacentPCInaccessible), with the inaccessible adjacent pointcode as a parameter, which represents measurement Q.752/5.1 (PerceivedSeverity=Maj/Min/War) and measurement Q.752/5.4 (PerceivedSeverity=Cleared).

If the ss7OnOccEventPackage is included, event notifications with the following probable causes can apply:

Probable Cause = 405 (LinkSetFailure), with the inaccessible adjacent pointcode as a parameter, which represents measurement Q.752/4.5 (PerceivedSeverity=Maj/Min/War) and measurement Q.752/4.6 (PerceivedSeverity=Cleared).

Probable Cause = 413 (LinkSetChange), with the involved link sets as parameter, which represents measurement Q.752/4.13.

";;

ATTRIBUTES

mtpLevel3Id		GET SET BY CREATE,

"ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 :

1992":administrativeState	GET-REPLACE,

"ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 :

1992":operationalState	GET,

"ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 :

1992":usageState		GET,

"ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 :

1992":proceduralStatus	GET;

NOTIFICATIONS

"ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":stateChange;;;

CONDITIONAL PACKAGES

"ITU-T Rec. M.3100 (1992)":alarmSeverityAssignmentPointerPackage PRESENT IF "an instance supports it and the (communicationsAlarmPackage or ss7OnOccEventPackage) is present",

"ITU-T Rec. M.3100 (1992)":attributeValueChangeNotificationPackage PRESENT IF "the attributeValueChangeNotification defined in 'ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992' is supported by an instance of this class",

"ITU-T Rec. M.3100 (1992)":createDeleteNotificationsPackage PRESENT IF "the objectCreation and objectDeletion notifications defined in 'ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992' are supported by an instance of this class",

communicationsAlarmPackage PRESENT IF "an instance supports it",

mtpLevel3NamePackage PRESENT IF "an instance supports it",

ss7OnOccEventPackage PRESENT IF "an instance supports it";

REGISTERED AS { mtpObjectClass 5 };

### 7.1.6	MTP Signalling PointXE "MTP Signalling Point"§

mtpSignPoint MANAGED OBJECT CLASS

DERIVED FROM		"ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":top;

CHARACTERIZED BY	mtpSignPointPackage PACKAGE

BEHAVIOUR 	mtpSignPointBehaviour BEHAVIOUR DEFINED AS

"A signalling point is a node in the signalling network. It is part of a managed switching element (exchange). The MTP permits three types of node:

–	SEP: the signalling end point, this type of SP contains MTP and ISUP (and or other MTP Users), but cannot act as an intermediate MTP transfer node in the SS No.7 network. (It can act as an SCCP Relay Node.)

–	STP: the MTP signalling transfer point, this type of SP acts as an intermediate node between SEPs to transfer messages through the SS No.7-network, it only contains MTP. (In case of an STP, OMAP, and hence also TC and SCCP may be present.)

–	STEP: MTP signalling transfer and end point, the type combines the previous mentioned types, thus acting both as a SEP and as a STP.

A mtpSignPoint can have the MTP status:

–	allowed, this means that the mtpSignPoint is accessible and is functioning normally (operational state = enabled, availability status = Æ)

–	congested, this means that the mtpSignPoint is accessible but is heavily loaded (operational state = enabled, availability status = { degraded })

–	prohibited, this means that the mtpSignPoint not accessible (operational state = disabled).

During MTP restart, the proceduralStatus is {initializing} until the restart is finished.";;

ATTRIBUTES

mtpSignPointId		GET SET BY CREATE,

pointCode		GET SET BY CREATE,

spType			GET SET BY CREATE,

--  In some implementations the spType attribute might additionally be writable.

"ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 :

1992":operationalState	GET,

"ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 :

1992":proceduralStatus	GET,

"ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 :

1992":availabilityStatus	GET,

"ITU-T Rec. M.3100 (1992)":version	GET SET BY CREATE,

networkIndicator		GET SET BY CREATE;

NOTIFICATIONS

"ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":stateChange,

mtpRestarting;;;

CONDITIONAL PACKAGES

"ITU-T Rec. M.3100 (1992)":attributeValueChangeNotificationPackage PRESENT IF "the attributeValueChangeNotification defined in 'ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992' is supported by an instance of this class",

"ITU-T Rec. M.3100 (1992)":createDeleteNotificationsPackage PRESENT IF "the objectCreation and objectDeletion notifications defined in 'ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992' are supported by an instance of this class",

mtpSignPointNamePackage PRESENT IF "an instance supports it",

spTimersPackage PRESENT IF "the instance does not support the spTimersProfilePointerPackage",

spTimersProfilePointerPackage PRESENT IF "the instance does not support the spTimersPackage";

REGISTERED AS { mtpObjectClass 6 };

### 7.1.7	Signalling Data Link Termination PointXE "Signalling Data Link Termination Point"§

signDataLinkTp MANAGED OBJECT CLASS

DERIVED FROM "ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":top;

CHARACTERIZED BY signDataLinkTpPackage PACKAGE

BEHAVIOUR signDataLinkTpBehaviour BEHAVIOUR DEFINED AS

"This managed object represents the termination of the signalling data link within the SP as defined in Q.702.

The operational state is 'enabled' in normal operation. In case of failure of the part of the signalling data link that is within control of the managed switching element, this state will be 'disabled'.";;

-- Note that for a complete view of the status of a signalling data link a network view is required.

ATTRIBUTES

signDataLinkTpId	GET SET BY CREATE,

adjPc	GET SET BY CREATE,

transmissionRate	GET SET BY CREATE,

"ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 :

1992":operationalState	GET,

equipmentPointer 	GET SET BY CREATE;

NOTIFICATIONS

"ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":stateChange;;;

CONDITIONAL PACKAGES

"ITU-T Rec. M.3100 (1992)":attributeValueChangeNotificationPackage PRESENT IF "the attributeValueChangeNotification defined in 'ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992' is supported by an instance of this class",

"ITU-T Rec. M.3100 (1992)":createDeleteNotificationsPackage PRESENT IF "the objectCreation and objectDeletion notifications defined in 'ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992' are supported by an instance of this class",

clCPackage PRESENT IF "an instance supports it",

signDataLinkTpNamePackage PRESENT IF "an instance supports it",

stmChannelPackage PRESENT IF "an instance supports it or the signalling point supports the automatic allocation of signalling datalinks";

REGISTERED AS { mtpObjectClass 7 };

### 7.1.8	Signalling Link Set Termination PointXE "Signalling Link Set Termination Point"§

-- An example of the OSI state mapping for this class can be found in Annex C.

signLinkSetTp MANAGED OBJECT CLASS

DERIVED FROM "ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":top;

CHARACTERIZED BY signLinkSetTpPackage PACKAGE

BEHAVIOUR signLinkSetTpBehaviour BEHAVIOUR DEFINED AS

"This managed object class represents the SS No. 7 signalling link set termination point. Link sets are defined in Recommendation Q.704.

The states are directly related to the set of link termination point's states, except for the optional administrativeState.

General principles on state information:

–	The states of the managed object linkset are only readable, except for the optional administrativeState.

–	Activation and deactivation of a linkset may be done via operations on the links (e.g. by using scoping functions) or the optional administrativeState can be used for this.

Based on these principles the states of a linkset are defined as follows:

–	operational state (read-only for the OS). The operational state is enabled whenever one link of the linkset is enabled, i.e. neither FAILED nor DEACTIVATED nor LOCAL BLOCKED nor REMOTE BLOCKED.

–	usage state (read-only for the OS). The usage state reflects the usage of the belonging links. It is ACTIVE if at least one of the belonging links is ACTIVE and no link is BUSY. It contains BUSY if one of the belonging links is BUSY. It is IDLE if all links are UNAVAILABLE.

–	availability status (read-only for the OS). The availability status indicates unavailability or reduced availability of the belonging links. It contains DEPENDENCY if all the belonging links are unavailable for user traffic. It contains DEGRADED if at least one belonging link is not available for user traffic.

For the different reasons of unavailability (failed, deactivated, local blocked, remote blocked, local inhibited and remote inhibited) see 3.2.1/Q.704.

If the 'ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992':communicationsAlarm notification is used, the following probable causes can apply:

Probable Cause=403 (LinkSetFailure), which represents measurements Q.752/4.3

(PerceivedSeverity=Maj/Min/War) and Q.752/4.4 (PerceivedSeverity=Cleared).

State change notifications are emitted for all state and status changes. ";;

ATTRIBUTES

signLinkSetTpId	GET SET BY CREATE,

adjPc		GET SET BY CREATE,

maxCapacityLS	GET SET BY CREATE,

-- In some implementations, the maxCapacityLS attribute might be additionally writable.

currentCapacityLS	GET,

congestionControlMethod	GET SET BY CREATE,

"ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 :

1992":usage State	GET,

"ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 :

1992":operationalState	GET,

"ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 :

1992":availabilityStatus	GET,

periodicLinkTestFlag	DEFAULT VALUE MTPDefinedTypesModule.

periodicTestFlagDefault

GET-REPLACE;

NOTIFICATIONS

"ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":stateChange;;;

CONDITIONAL PACKAGES

administrativeStatePackage PRESENT IF "an instance supports it",

"ITU-T Rec. M.3100 (1992)":alarmSeverityAssignmentPointerPackage PRESENT IF "an 
instance supports it and the communicationsAlarmPackage is present",

"ITU-T Rec. M.3100 (1992)":attributeValueChangeNotificationPackage PRESENT IF "the attributeValueChangeNotification defined in 'ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992' is supported by an instance of this class",

"ITU-T Rec. M.3100 (1992)":createDeleteNotificationsPackage PRESENT IF "the objectCreation and objectDeletion notifications defined in 'ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992' are supported by an instance of this class",

communicationsAlarmPackage PRESENT IF "an instance supports it",

inLsLoadShareAlgorithmPackage PRESENT IF "an instance supports it",

lsTimersProfilePointerPackage PRESENT IF "the instance of superior class mtpSignPoint does not support the conditional spTimersPackage",

noBasicLinkAllocationPackage PRESENT IF "Basic link allocation is not done",

signLinkSetTpNamePackage PRESENT IF "an instance supports it";

REGISTERED AS { mtpObjectClass 8 };

### 7.1.9	Signalling Link Termination PointXE "Signalling Link Termination Point"§

-- An example of mapping of the MTP functional states to the incorporated OSI management states

-- can be found in Annex C.

signLinkTp MANAGED OBJECT CLASS

DERIVED FROM "ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":top;

CHARACTERIZED BY signLinkTpPackage PACKAGE

BEHAVIOUR signLinkTpBehaviour BEHAVIOUR DEFINED AS

"This managed object represents the termination of the signalling link within the signalling point as defined in Recommendation Q.703. The signLinkTp object class represents that part of a signalling link which is located in one NE.

The following state and status attributes are supported:

administrative state:

–	locked: it is administratively not permitted to transport traffic on the link termination point.

–	unlocked: it is administratively permitted to transport user part traffic or test traffic on the link termination point.

operational state:

–	enabled: the link termination point is operationally able to transport user part traffic or test traffic.

–	disabled: the link termination point is operationally unable to transport user part traffic or test traffic.

The operational state is disabled whenever the link termination point status contains Failed, Deactivated, Local Blocked, Remote Blocked, or the procedural status is Initializing.

usage state:

–	idle: the link termination point carries no user part traffic (test traffic may be present).

–	'active': the link termination point is not congested. Currently the link termination point carries user part traffic.

–	busy: the link termination point is congested due to user part traffic.

The usage state is idle whenever the link termination point status is not empty i.e. the usage state immediately reflects the availability of the signLinkTp for user traffic. For national MTP options applying several congestion levels an additional attribute indicating these levels may be added.

procedural status:

–	The procedural status initializing is valid during initial alignment procedure.

–	The procedural status not initalized applies when the link termination point is not aligned and the initial alignment procedure has not been started.

Link termination point status. This is a set-valued attribute. The following statusses can exist alone or in combinations with each other. The exact definition for each of the following SS7-functional states is to be gained from Q.704

–	local blocked

–	remote blocked

–	local inhibited

–	remote inhibited

–	failed

–	deactivated.

If the 'ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992':communicationsAlarm notification is used, the following probable causes can apply:

Probable Cause = 102 (slFailure) which represents measurement Q.752/1.2 – measurement Q.752/1.6 (SpecificProblems=003 abnormalFIBRorBSNR, 004 excessiveAckDelay, 005 excessiveErrorRate, 006 excessiveCongDuration) and  Q.752/1.12.(PerceivedSeverity=cleared)

If the ss7OnOccEventPackage is present, the event notifications with the following probable causes can apply:

Probable Cause = 110 (localChangeOver) which represents measurement Q.752/1.10

(PerceivedSeverity=Maj/Min/War) and measurement Q.752/1.11 (PerceivedSeverity=Cleared).

Probable Cause = 210 (remoteProcOutage) which represents measurements Q.752/2.10

(PerceivedSeverity=Maj/Min/War) and Q.752/2.11 (PerceivedSeverity=Cleared).

Probable Cause = 216 (start of local inhibition) which represents measurement Q.752/2.16

(PerceivedSeverity=Maj/Min/War) and Q.752/2.17 (PerceivedSeverity=Cleared).

Probable Cause = 218 (start of remote inhibition) which represents measurement Q.752/2.18

(PerceivedSeverity=Maj/Min/War) and Q.752/2.19 (PerceivedSeverity=Cleared).

A create or set request is rejected, if

i)	a pointer would reference an instance, which does not belong to the appropriate object class

OR	ii)	a pointer would reference an instance which is not existing

OR	iii)	a signDataLinkTpPointer would reference a signDataLinkTp which is not in the same mtpSignPoint

OR	iv)	a signDataLinkTpPointer would reference a signDataLinkTp which has a different value of adjPc than the superior signLinkSetTp

OR	v)	the link would reference a datalink and a mtpL2ProtocolProfile whose transmissionRates is not the same

OR	vi)	the link would reference a mtpL2ProtocolProfile where the appropriate packages for the congestionControlMethod of the superior signLinkSetTp are not present (see B.1.2.9.2)

OR	vii)	the spTimersProfilePackage is present in the superior mtpSignPoint AND the link would reference a mtpL2ProtocolProfile whose l2TimerT2 is not bigger than the q704-t17 of the spTimersProfile referenced by the mtpSignPoint

OR	viii)	the spTimersPackage is present in the superior mtpSignPoint AND the link would reference a mtpL2ProtocolProfile whose l2TimerT2 is not bigger than the q704-t17 of the mtpSignPoint.

The rejection also takes place if only a single pointer within the signDataLinkList or the signTermList (if present) is wrong.

A delete request is rejected, if the linkTpStatus does not contain the value deactivated,";;

ATTRIBUTES

slCode			GET SET BY CREATE,

slsCodeCurrentList	GET,

maxCapacitySL	GET,

currentCapacitySL	GET,

linkTpStatus		GET,

"ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 :

1992":administrativeState	GET-REPLACE,

"ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 :

1992":operationalState	GET,

"ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 :

1992":usageState	GET,

"ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 :

1992":proceduralStatus	GET,

mtpL2ProtocolProfilePointer	GET-REPLACE,

signTermPointer	GET SET BY CREATE,

signDataLinkTpPointer	GET-REPLACE;

ACTIONS

localInhibit

localUninhibit

NOTIFICATIONS

"ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":stateChange;;;

CONDITIONAL PACKAGES

"ITU-T Rec. M.3100 (1992)":alarmSeverityAssignmentPointerPackage PRESENT IF "an instance supports it and the (communicationsAlarmPackage or ss7OnOccEventPackage) is present",

"ITU-T Rec. M.3100 (1992)":attributeValueChangeNotificationPackage PRESENT IF "the attributeValueChangeNotification defined in 'ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992' is supported by an instance of this class",

"ITU-T Rec. M.3100 (1992)":createDeleteNotificationsPackage PRESENT IF "the objectCreation and objectDeletion notifications defined in 'ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992' are supported by an instance of this class",

communicationsAlarmPackage PRESENT IF "an instance supports it",

linkCongestionLevelPackage PRESENT IF "the exchange support the operation, management and administration of multiple transmission congestion levels (national option)",

relatedLinkGroupNumberPackage PRESENT IF "an instance supports it",

signDataLinkTpListPackage PRESENT IF "an instance supports it",

signLinkTestPackage PRESENT IF "the on-demand signalling link test as described in Q.707 is supported",

signLinkTpNamePackage PRESENT IF "an instance supports it",

signTermListPackage PRESENT IF "an instance supports it or an instance supports the signDataLinkTpListPackage",

slsCodeNormalListPackage PRESENT IF "an instance supports it",

slTimersProfilePointerPackage PRESENT IF "the instance of superior class mtpSignPoint does not support the conditional spTimersPackage",

ss7OnOccEventPackage PRESENT IF "an instance supports it";

REGISTERED AS [ mtpObjectClass 9 };

### 7.1.10	Signalling Link Timers ProfileXE "Signalling Link Timers Profile"§

slTimersProfile MANAGED OBJECT CLASS

DERIVED FROM	"ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":top;

CHARACTERIZED BY

slTimersProfilePackage PACKAGE

BEHAVIOUR slTimersProfileBehaviour BEHAVIOUR DEFINED AS

"A signalling link timer profile is a set of Q.704 timers which are related to the characteristics of Signalling Links.

The managed object class slTimersProfile is a support managed object class which can be instantiated in order to define a particular profile. At creation time of a signLinkTp one of the slTimersProfiles may be referenced..";;

ATTRIBUTES

slTimersProfileId	GET SET BY CREATE,

q704-t1		GET SET BY CREATE,

q704-t3		GET SET BY CREATE,

q704-t17		GET SET BY CREATE,

q704-t24		GET SET BY CREATE;;;

CONDITIONAL PACKAGES

"ITU-T Rec. M.3100 (1992)":createDeleteNotificationsPackage PRESENT IF "the objectCreation and objectDeletion notifications defined in 'ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992' are supported by an instance of this class",

"ITU-T Rec. M.3100 (1992)":attributeValueChangeNotificationPackage PRESENT IF "the attributeValueChangeNotification defined in 'ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992' is supported by an instance of this class",

slTimersProfileNamePackage PRESENT IF "an instance supports it";

REGISTERED AS { mtpObjectClass 10 };

### 7.1.11	Signalling Point Timers ProfileXE "Signalling Point Timers Profile"§

spTimersProfile MANAGED OBJECT CLASS

DERIVED FROM  "ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":top;

CHARACTERIZED BY

spTimersProfilePackage PACKAGE

BEHAVIOUR spTimersProfileBehaviour BEHAVIOUR DEFINED AS

"A signalling link set timer profile is set of Q.704 and Q.707 timers which are related to the characteristics of signalling points.

The managed object class spTimersProfile is a support managed object class which can be instantiated in order to define a particular profile. At creation time of a signPoint one of the spTimersProfiles is referenced.";;

ATTRIBUTES

spTimersProfileId	GET SET BY CREATE,

q704-t2	GET SET BY CREATE,

q704-t4	GET SET BY CREATE,

q704-t5	GET SET BY CREATE,

q704-t7	GET SET BY CREATE,

q704-t11	GET SET BY CREATE,

q704-t12	GET SET BY CREATE,

q704-t13	GET SET BY CREATE,

q704-t14	GET SET BY CREATE,

q704-t15	GET SET BY CREATE,

q704-t16 	GET SET BY CREATE,

q704-t18	GET SET BY CREATE,

q704-t19	GET SET BY CREATE,

q704-t20	GET SET BY CREATE,

q704-t21	GET SET BY CREATE,

q704-t22	GET SET BY CREATE,

q704-t23	GET SET BY CREATE,

q707-t1	GET SET BY CREATE,

q707-t2	GET SET BY CREATE;;;

CONDITIONAL PACKAGES

"ITU-T Rec. M.3100 (1992)":createDeleteNotificationsPackage PRESENT IF "the objectCreation and objectDeletion notifications defined in 'ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992' are supported by an instance of this class",

"ITU-T Rec. M.3100 (1992)":attributeValueChangeNotificationPackage PRESENT IF "the attributeValueChangeNotification defined in 'ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992' is supported by an instance of this class",

spTimersProfileNamePackage PRESENT IF "an instance supports it";

REGISTERED AS { mtpObjectClass 11 };

### 7.1.12	Signalling Route Network Element PartXE "Signalling Route Network Element Part"§

NOTE – The optional package ClsLoadsharingInformationPackage shows an example of how the loadsharing mechanism may be implemented. However, this option might complicate the operation of SS No. 7 networks. Therefore, other implementation dependent solutions might be used in which the load sharing within the individual link sets of a combined link set is made independent of the destination.

signRouteNePart MANAGED OBJECT CLASS

DERIVED FROM	"ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":top;
	CHARACTERIZED BY	signRouteNePartPackage PACKAGE

BEHAVIOUR	signRouteNePartBehaviour BEHAVIOUR DEFINED AS

"A signalling route (A,C,B)i is defined as an ordered sequence of the near end signalling point A (i.e. the network element), an adjacent SP C and the destination SP B. The adjacent SP C may be identical with the destination B, or it may or may not be directly interconnected to the destination SP.

The adjacent SP C must be directly connected to the near end SP by at least one linkset which is intended to convey MSUs sent from A to B (hence the use of the term 'adjacent').

Two signalling routes (A,C,B)i and (A,C',B)j are distinct if the adjacent SP is different.

The signRouteNePart Managed Object class defines the management capabilities of the resources which define a specific route segment (Link Set to be traversed) and its priority within the routeset as seen from the own SP.

A particular signRouteNePart is related to one or more Signalling Routes on the network level.

The priority in which the network Signalling Routes are used, is defined by means of assigning priorities to all involved route segments. If from a particular SP two or more route segments are used with the same priority, loadsharing between Signalling Routes may occur. A Combined Link Set is a set of Link Sets used with the same priority by message routing.

The signLinkSetTp used for routing is allocated at creation time and cannot be changed during the existence of a particular signRouteNePart.

The administrativeState attribute describes whether it is administratively permitted to route messages towards the respective destination point via the referenced signLinkSetTp.

The operationalState attribute is 'enabled' if both the referenced signLinkSetTp is in the operational state 'enabled' and no TFP message concerning the respective destinations has been received from the adjPC of the respective signLinkSetTp.

The optional usageState attribute is used to cover restriction of a route. Its value is set to 'busy', if the route is restricted ("transfer restricted' is a national option).

The availabilityStatus attribute gives additional information about the availability of the signRouteNePart instance. It is furthermore needed to map some of the MTP functional states (prohibited: availabilityStatus = {dependency}, restricted: availabilityStatus={degraded}) and is used for the case that the route is locked not by the OS, but e.g. by a local administrator (administrativeState=unlocked and availabilityStatus={off line}).";;

ATTRIBUTES

signRouteNePartId		GET SET BY CREATE,

signLinkSetTPPointer	GET SET BY CREATE,

"ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 :

1992":administrativeState 	GET-REPLACE,

"ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 :

1992":operationalState 	GET,

"ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 :

1992":availabilityStatus	GET;

NOTIFICATIONS

"ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":stateChange;;;

CONDITIONAL PACKAGES

"ITU-T Rec. M.3100 (1992)":attributeValueChangeNotificationPackage PRESENT IF "the attributeValueChangeNotification defined in 'ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992' is supported by an instance of this class",

"ITU-T Rec. M.3100 (1992)":createDeleteNotificationsPackage PRESENT IF "the objectCreation and objectDeletion notifications defined in 'ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992' are supported by an instance of this class",

clsLoadsharingInformationPackage PRESENT IF "an instance supports it",

fixedRouteNePartPriorityPackage PRESENT IF "the instance does not use the flexibleRouteNePartPriorityPackage",

flexibleRouteNePartPriorityPackage PRESENT IF "the instance does not use the fixedRouteNePartPriorityPackage",

loadsharingInformationRouteNePartPackage PRESENT IF "the instance supports it",

mtpLoadsharingObjectForRouteNePartPointerPackage PRESENT IF "the instance supports it",

signRouteNePartNamePackage PRESENT IF "an instance supports it",

slsListPackage PRESENT IF "the instance supports it",

usageStatePackage PRESENT IF "the instance supports 'transfer restricted'";

REGISTERED AS { mtpObjectClass 12 }

### 7.1.13	Signalling Route Set Network Element PartXE "Signalling Route Set Network Element Part"§

signRouteSetNePart MANAGED OBJECT CLASS

DERIVED FROM	"ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":top;

CHARACTERIZED BY	signRouteSetNePartPackage PACKAGE

BEHAVIOUR signRouteSetNePartBehaviour BEHAVIOUR DEFINED AS

"A Signalling Route Set is a complete set of routes that can be used to carry traffic toward a specific destination signalling point.

The signRouteSetNePart Managed Object class defines the management capabilities of the resources which represent a specific destination SP and its accessibility as seen from the own SP. This SP may be the originating SP of the MTP messages or an STP.

A particular signRouteSetNePart may be related to one or more Signalling Route Sets on network level.

The administrativeState attribute describes whether it is administratively permitted to route SS No. 7 MSUs towards the respective destination point. Before the administrative state of a signRouteSetNePart can be set to 'unlocked', there must be at least one signRouteNePart contained within the signRouteSetNePart.

The operationalState attribute describes whether the respective destination signalling point is accessible - ('enabled') or not ('disabled'). If the operational states of all signRouteNePart instances contained in the signRouteSetNePart are 'disabled', then the operational state of the signRouteSetNePart is 'disabled', in any other case it is 'enabled'.

The usageState attribute is used to reflect route set congestion. The value is 'busy' in case of route set congestion.

If the 'ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2: 1992':communicationsAlarm notification is present, it indicates at least the following probable causes:

Probable Cause = 411(RouteSetUnavailable), which represents measurements Q.752/4.11

(PerceivedSeverity=Maj/Min/War) and Q.751/4.12 (PerceivedSeverity=Cleared).

If the ss7OnOccEventPackage is present, event notifications with the following probable cause can apply:

Probable Cause = 002 (remoteUserPartUnavailable), with the involved user part as parameter. Possible values for SpecificProblems are: 007 'unknown', 008 'unequipped' and 009 'inaccessibleRemoteUser'.

An activation or deactivation of a signRouteSetNePart must lead to the activation respectively deactivation of all contained signRouteNeParts.";;

ATTRIBUTES

signRouteSetNePartId	GET SET BY CREATE,

"ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 :

1992":administrativeState	GET-REPLACE,

"ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 :

1992":operationalState	GET,

NOTIFICATIONS

"ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 :

1992":stateChange;;;

CONDITIONAL PACKAGES

"ITU-T Rec. M.3100 (1992)":alarmSeverityAssignmentPointerPackage PRESENT IF "an instance supports it and the (communicationsAlarmPackage or ss7OnOccEventPackage) is present",

"ITU-T Rec. M.3100 (1992)":attributeValueChangeNotificationPackage PRESENT IF "the attributeValueChangeNotification defined in 'ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992' is supported by an instance of this class",

ITU-T Rec. M.3100 (1992)":createDeleteNotificationsPackage PRESENT IF "the objectCreation and objectDeletion notifications defined in 'ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992' are supported by an instance of this class",

communicationsAlarmPackage PRESENT IF "an instance supports it",

congestedStatePackage PRESENT IF "the instance does not use the congestionLevelPackage",

congestionLevelPackage PRESENT IF "the instance does not use the congestedStatePackage",

loadsharingInformationRouteSetNePartPackage PRESENT IF "the instance supports it",

mtpLoadsharingObjectForRouteSetNePartPointerPackage PRESENT IF "the instance supports it",

remoteExchangeLabelPackage PRESENT IF "the instance supports it",

signRouteSetNePartNamePackage PRESENT IF "the instance supports it",

ss7OnOccEventPackage PRESENT IF "an instance supports it";

REGISTERED AS { mtpObjectClass 13 };

### 7.1.14	Signalling TerminalXE "Signalling Terminal"§

signTerm MANAGED OBJECT CLASS

DERIVED FROM	"ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":top;

CHARACTERIZED BY	signTermPackage PACKAGE

BEHAVIOUR	signTermBehaviour BEHAVIOUR DEFINED AS

"The signTerm MOC defines the management capabilities for the signalling terminal. The signalling terminal is defined in ITU-T Rec. Q.703.";;

ATTRIBUTES

signTermId		GET SET BY CREATE,

"ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 :

1992":administrativeState	GET-REPLACE,

"ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 :

1992":operationalState	GET,

--  the application of the following state is f.f.s.

"ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 :

1992":usageState		GET,

--  the application of the following state is f.f.s.

"ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 :

1992":proceduralStatus	GET,

--  the application of the following state is f.f.s.

"ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 :

1992":availabilityStatus	GET,

equipmentPointer		GET SET BY CREATE;

NOTIFICATIONS

"ITU-T Rec. X.721 | ISO/IEC 10165-2 : 1992":stateChange;;;
	CONDITIONAL PACKAGES

ITU-T Rec. M.3100 (1992)":createDeleteNotificationsPackage PRESENT IF "the objectCreation and objectDeletion notifications defined in 'ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992' are supported by an instance of this class",

"ITU-T Rec. M.3100 (1992)":attributeValueChangeNotificationPackage PRESENT IF "the attributeValueChangeNotification defined in 'ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992' is supported by an instance of this class",

signTermNamePackage PRESENT IF "an instance supports it";

REGISTERED AS { mtpObjectClass 14 };

### 7.1.15	STP Screening TableXE "STP Screening Table"§

stpScreeningTable MANAGED OBJECT CLASS

DERIVED FROM		"ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":top;

CHARACTERIZED BY	stpScreeningTablePkg PACKAGE
		BEHAVIOUR stpScreeningTableBehaviour BEHAVIOUR DEFINED AS

"The stpScreeningTable object class is the base for STP screening. Together with its subordinate stpScreeningTableLine's it contains all the information which are necessary for an STP to identify and to handle unauthorized SS No. 7 messages as defined in 8.2/Q.705 and 8.3/Q.705.

";;

ATTRIBUTES

stpScreeningTableId		GET SET BY CREATE,

"ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 :

1992":administrativeState

INITIAL VALUE MTPDefinedTypesModule.

administrativeStateLocked	GET-REPLACE,

treatmentOfOutsideRanges	GET-REPLACE,

stpScreeningTableListMode	GET-REPLACE,

screeningByLinkSetOrByDPCOpc	GET SET BY CREATE;

NOTIFICATIONS

"ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":stateChange;;;

CONDITIONAL PACKAGES

"ITU-T Rec. M.3100 (1992)":attributeValueChangeNotificationPackage PRESENT IF "the attributeValueChangeNotification defined in 'ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992' is supported by an instance of this class",

"ITU-T Rec. M.3100 (1992)":createDeleteNotificationsPackage PRESENT IF "the objectCreation and objectDeletion notifications defined in 'ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992' are supported by an instance of this class",

getScreenedOpcsOrLinkSetsByDpcActionPackage PRESENT IF "an instance supports it",

stpScreeningTableNamePackage PRESENT IF "an instance supports it";

REGISTERED AS { mtpObjectClass 15 };

### 7.1.16	STP Screening Table LineXE "STP Screening Table Line"§

stpScreeningTableLine MANAGED OBJECT CLASS

DERIVED FROM	"ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":top,

CHARACTERIZED BY stpScreeningTableLinePkg PACKAGE

BEHAVIOUR stpScreeningTableLineBehaviour BEHAVIOUR DEFINED AS

"The stpScreeningTableLine together with its superior stpScreeningTable contains all the information which are necessary for an STP to identify and to handle unauthorized SS No. 7 messages as defined in 8.2/Q.705 and 8.3/Q.705.

The total number of screened DPC's across all stpScreeningTableLine instances contained by a stpScreeningTable must be less or equal to maxNoOfScreenedDpcs.";;

ATTRIBUTES

stpScreeningTableLineId	GET SET BY CREATE.

stpScreeningTableLineContent	GET-REPLACE ADD-REMOVE;;;

CONDITIONAL PACKAGES

"ITU-T Rec. M.3100 (1992)":attributeValueChangeNotificationPackage PRESENT IF "the attributeValueChangeNotification defined in 'ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992' is supported by an instance of this class",

"ITU-T Rec. M.3100 (1992)":createDeleteNotificationsPackage PRESENT IF "the objectCreation and objectDeletion notifications defined in 'ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992' are supported by an instance of this class";

REGISTERED AS { mtpObjectClass 16 };

## 7.2	Package definitionsXE "Package definitions"§

administrativeStatePackage PACKAGE

BEHAVIOUR administrativeStatePackageBehaviour BEHAVIOUR DEFINED AS

"This package can be used for activiated and deactivated link sets. Possible values of the administrativeState attribute are: locked and unlocked.

If transition takes place from locked to unlocked, this will result in activiation of all links in the set that are unlocked.

If transition takes place from unlocked to locked, this will result in deactiviation of all links in the set. This has no effect on the administrative states of the links themselves";;

ATTRIBUTES

"ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 :

1992":administrativeState		GET-REPLACE;

REGISTERED AS { mtpPackage 1 };

bufferMechanismPackage PACKAGE

ATTRIBUTES

numberOfThresholdLevels	GET SET BY CREATE;

REGISTERED AS { mtpPackage 2 };

clCPackage PACKAGE

ATTRIBUTES

clC	GET SET BY CREATE;

REGISTERED AS { mtpPackage 3 };

clsLoadsharingInformationPackage PACKAGE

ATTRIBUTES

inClsLoadsharingAlgorithm	GET-REPLACE;

REGISTERED AS { mtpPackage 4 };

communicationsAlarmPackage PACKAGE

NOTIFICATIONS

"ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":communicationsAlarm

inaccessibleSp;

REGISTERED AS { mtpPackage 5 };

congestedStatePackage PACKAGE

BEHAVIOUR congestedStatePackageBehaviour BEHAVIOUR DEFINED AS

"This package supplies an attribute for the simple congestion method used e.g. in the international network.";;

ATTRIBUTES

congestedState	GET;

NOTIFICATIONS

"ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":attributeValueChange;

REGISTERED AS { mtpPackage 6 };

congestionHandlingPackage PACKAGE

ATTRIBUTES

congestionCounting	GET SET BY CREATE,

congestionReportingBaseObject	GET SET BY CREATE;

REGISTERED AS { mtpPackage 7 };

congestionLevelPackage PACKAGE

BEHAVIOUR congestionLevelPackageBehaviour BEHAVIOUR DEFINED AS

"This package supplies an attribute to allow for several congestion levels for the signRouteSetNePart.";;

ATTRIBUTES

congestionLevel	GET;

REGISTERED AS { mtpPackage 8 };

getScreenedOpcsOrLinkSetsByDpcActionPackage PACKAGE

ACTIONS

getScreenedOpcsOrLinkSetsByDpc

REGISTERED AS { mtpPackage 9 };

fixedRouteNePartPriorityPackage PACKAGE

ATTRIBUTES

fixedPriority	GET SET BY CREATE;

REGISTERED AS { mtpPackage 10 };

flexibleRouteNePartPriorityPackage PACKAGE

ATTRIBUTES

flexiblePriority	GET-REPLACE,

priorityMode	GET-REPLACE;

NOTIFICATIONS

"ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":attributeValueChange;

REGISTERED AS { mtpPackage 11 };

inLsLoadShareAlgorithmPackage PACKAGE

ATTRIBUTES

inLsLoadShareAlgorithm	GET;

-- in some implementations the inLsLoadShareAlgorithm attribute might be additionally writable

REGISTERED AS { mtpPackage 12 };

linkCongestionLevelPackage PACKAGE

ATTRIBUTES

linkCongestionLevel	GET SET BY CREATE;

REGISTERED AS { mtpPackage 13 };

loadsharingInformationRouteNePartPackage PACKAGE

ATTRIBUTES

loadsharingInformationRouteNePart	GET-REPLACE;

REGISTERED AS { mtpPackage 14 };

loadsharingInformationRouteSetNePartPackage PACKAGE

ATTRIBUTES

loadsharingInformationRouteSetNePart	GET-REPLACE;

REGISTERED AS { mtpPackage 15 };

loopDelayPackage PACKAGE

ATTRIBUTES

loopDelay	GET SET BY CREATE;

REGISTERED AS { mtpPackage 16 };

lsTimersProfileNamePackage PACKAGE

ATTRIBUTES

lsTimersProfileName	GET-REPLACE;

REGISTERED AS { mtpPackage 17 };

lsTimersProfilePointerPackage PACKAGE

ATTRIBUTES

lsTimersProfilePointer	GET-REPLACE;

REGISTERED AS { mtpPackage 18 };

managedSwitchingElementNamePackage PACKAGE

ATTRIBUTES

managedSwitchingElementName	GET-REPLACE;

REGISTERED AS { mtpPackage 19 };

mtpAccessPointNamePackage PACKAGE

ATTRIBUTES

mtpAccessPointName	GET-REPLACE;

REGISTERED AS { mtpPackage 20 };

mtpL2ProtocolProfileNamePackage PACKAGE

ATTRIBUTES

mtpL2ProtocolProfileName	GET-REPLACE;

REGISTERED AS { mtpPackage 21 };

mtpLevel3NamePackage PACKAGE

ATTRIBUTES

mtpLevel3Name	GET-REPLACE;

REGISTERED AS { mtpPackage 22 };

mtpLoadsharingObjectForRouteNePartPointerPackage PACKAGE

ATTRIBUTES

mtpLoadsharingObjectForRouteNePartPointer		GET-REPLACE;

REGISTERED AS { mtpPackage 23 };

mtpLoadsharingObjectForRouteSetNePartPointerPackage PACKAGE

ATTRIBUTES

mtpLoadsharingObjectForRouteSetNePartPointer		GET-REPLACE;

REGISTERED AS { mtpPackage 24 };

mtpSignPointNamePackage PACKAGE

ATTRIBUTES

mtpSignPointName	GET-REPLACE;

REGISTERED AS { mtpPackage 25 };

multipleTransmissionCongestionLevelsPackage PACKAGE

BEHAVIOUR multipleTransmissionCongestionLevelsPackageBehaviour BEHAVIOUR DEFINED AS

"This package contains the second and third levels of transmission congestion. The values of the thresholds belonging to congestion level 2 have to be bigger than the respective ones of congestion level 1. The values of the thresholds belonging to congestion level 3 have to be bigger than the respective ones of congestion level 2.";;

ATTRIBUTES

transmissionCongestionThresholdDiscardLevel1		GET SET BY CREATE,

transmissionCongestionThresholdOnsetLevel2		GET SET BY CREATE,

transmissionCongestionThresholdAbatementLevel2	GET SET BY CREATE,

transmissionCongestionThresholdDiscardLevel2		GET SET BY CREATE,

transmissionCongestionThresholdOnsetLevel3		GET SET BY CREATE,

transmissionCongestionThresholdAbatementLevel3	GET SET BY CREATE,

transmissionCongestionThresholdDiscardLevel3		GET SET BY CREATE,

REGISTERED AS { mtpPackage 26};

multipleTransmissionCongestionStatesPackage PACKAGE

BEHAVIOUR multipleTransmissionCongestionStatesPackageBehaviour BEHAVIOUR DEFINED AS

"This package contains the attributes related to the management of 'multiple link congestion states without congestion priority' method. This method is described in 3.8.2.3/Q.704. See also B.1.2.9.2. The range of initialLevelOfCongestion is [1,numberOfCongestionStates].";;

ATTRIBUTES

timerTx	GET SET BY CREATE,

timerTy	GET SET BY CREATE,

numberOfCongestionStates	GET SET BY CREATE,

initialLevelOfCongestion	GET SET BY CREATE;

REGISTERED AS { mtpPackage 27 };

noBasicLinkAllocationPackage PACKAGE

ATTRIBUTES

numberOfNormallyActiveSignLinksTps	GET-REPLACE;

REGISTERED AS { mtpPackage 28 };

pCRPackage PACKAGE

BEHAVIOUR pCRPackageBehaviour BEHAVIOUR DEFINED AS

"This package contains signLinkTp attributes for the PCR method";;

ATTRIBUTES

maxMSUsRetransN1	GET SET BY CREATE,

maxOctRetransN2		GET SET BY CREATE;

REGISTERED AS { mtpPackage 29 };

receiveCongestionLevelsPackage PACKAGE

BEHAVIOUR receiveCongestionLevelsPackageBehaviour BEHAVIOUR DEFINED AS

"This package contains the levels of receive congestions";;

ATTRIBUTES

receiveCongestionThresholdOnset	GET SET BY CREATE,

receiveCongestionThresholdAbatement	GET SET BY CREATE;

REGISTERED AS { mtpPackage 30 };

relatedLinkGroupNumberPackage PACKAGE

ATTRIBUTES

relatedLinkGroupNumber	GET-REPLACE;

REGISTERED AS { mtpPackage 31 };

remoteExchangeLabelPackage PACKAGE

BEHAVIOUR remoteExchangeLabelPackageBehaviour BEHAVIOUR DEFINED AS

"This package supplies the possibility to have textual information about the remote exchange that contains the destination signalling point.";;

ATTRIBUTES

remoteExchangeLabel	GET-REPLACE;

REGISTERED AS { mtpPackage 32 };

signDataLinkTpListPackage PACKAGE

ATTRIBUTES

signDataLinkTpList	GET;

REGISTERED AS { mtpPackage 33 };

signDataLinkTpNamePackage PACKAGE

ATTRIBUTES

signDataLinkTpName	GET-REPLACE;

REGISTERED AS { mtpPackage 34 };

signLinkSetTpNamePackage PACKAGE

ATTRIBUTES

signLinkSetTpName	GET-REPLACE;

REGISTERED AS { mtpPackage 35 };

signLinkTestPackage PACKAGE

ACTIONS

signLinkTest;

REGISTERED AS { mtpPackage 36 };

signLinkTpNamePackage PACKAGE

ATTRIBUTES

signLinkTpName	GET-REPLACE;

REGISTERED AS { mtpPackage 37 };

signRouteNePartNamePackage PACKAGE

ATTRIBUTES

signRouteNePartName	GET-REPLACE;

REGISTERED AS { mtpPackage 38 };

signRouteSetNePartNamePackage PACKAGE

ATTRIBUTES

signRouteSetNePartName	GET-REPLACE;

REGISTERED AS { mtpPackage 39 };

signTermListPackage PACKAGE

ATTRIBUTES

signTermList	GET;

REGISTERED AS { mtpPackage 40 };

signTermNamePackage PACKAGE

ATTRIBUTES

signTermName	GET-REPLACE;

REGISTERED AS { mtpPackage 41 };

slsCodeNormalListPackage PACKAGE

ATTRIBUTES

slsCodeNormalList	GET SET BY CREATE;

-- in some implementations this attribute might additionally be writable

REGISTERED AS { mtpPackage 42 };

slsListPackage PACKAGE

ATTRIBUTES

slsList	GET SET BY CREATE;

REGISTERED AS { mtpPackage 43 };

slTimersProfileNamePackage PACKAGE

ATTRIBUTES

slTimersProfileName	GET-REPLACE;

REGISTERED AS { mtpPackage 44 };

slTimersProfilePointerPackage PACKAGE

ATTRIBUTES

slTimersProfilePointer	GET-REPLACE;

REGISTERED AS { mtpPackage 45 };

spTimersPackage PACKAGE

BEHAVIOUR spTimersPackageBehaviour BEHAVIOUR DEFINED AS

"This package contains the values of the Q.704 and Q.707 timers.";;

ATTRIBUTES

q704-t1	GET SET BY CREATE,

q704-t2	GET SET BY CREATE,

q704-t3	GET SET BY CREATE,

q704-t4	GET SET BY CREATE,

q704-t5	GET SET BY CREATE,

q704-t6	GET SET BY CREATE,

q704-t7	GET SET BY CREATE,

q704-t8	GET SET BY CREATE,

q704-t10	GET SET BY CREATE,

q704-t11	GET SET BY CREATE,

q704-t12	GET SET BY CREATE,

q704-t13	GET SET BY CREATE,

q704-t14	GET SET BY CREATE,

q704-t15	GET SET BY CREATE,

q704-t16	GET SET BY CREATE,

q704-t17	GET SET BY CREATE,

q704-t18	GET SET BY CREATE,

q704-t19	GET SET BY CREATE,

q704-t20	GET SET BY CREATE,

q704-t21	GET SET BY CREATE,

q704-t22	GET SET BY CREATE,

q704-t23	GET SET BY CREATE,

q704-t24	GET SET BY CREATE,

q707-t1	GET SET BY CREATE,

q707-t2	GET SET BY CREATE;

REGISTERED AS { mtpPackage 46 };

spTimersProfileNamePackage PACKAGE

ATTRIBUTES

spTimersProfileName	GET-REPLACE;

REGISTERED AS { mtpPackage 47 };

spTimersProfilePointerPackage PACKAGE

ATTRIBUTES

spTimersProfilePointer	GET-REPLACE;

REGISTERED AS { mtpPackage 48 };

ss7OnOccEventPackage PACKAGE

NOTIFICATIONS

ss7OnOccEvent

changeInLsToAdjSp

inaccessibleSp

remoteUserPartUnavailable;

REGISTERED AS { mtpPackage 49 };

stmChannelPackage PACKAGE

ATTRIBUTES

stmChannel	GET;

REGISTERED AS { mtpPackage 50 };

stpScreeningTableNamePackage PACKAGE

ATTRIBUTES

stpScreeningTableName	GET-REPLACE;

REGISTERED AS { mtpPackage 51 };

usageStatePackage PACKAGE

BEHAVIOUR usageStatePackageBehaviour BEHAVIOUR DEFINED AS

"This package supplies a possibility to cover 'transfer restricted' for a route.";;

ATTRIBUTES

ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":usageState	GET;

REGISTERED AS { mtpPackage 52 };

## 7.3	Parameter definitionsXE "Parameter definitions"§

changeInLsToAdjSp		PARAMETER

CONTEXT	EVENT-INFO;

WITH SYNTAX		MTPDefinedTypesModule.ChangeInLsToAdjSpInfo;

REGISTERED AS { mtpParameter 1 };

inaccessibleSp		PARAMETER

CONTEXT	EVENT-INFO;

WITH SYNTAX		MTPDefinedTypesModule.InaccessibleSpInfo;

REGISTERED AS { mtpParameter 2 };

remoteUserPartUnavailable		PARAMETER

CONTEXT	EVENT-INFO;

WITH SYNTAX	MTPDefinedTypesModule.UserPart;

REGISTERED AS { mtpParameter 3 };

## 7.4	Attribute definitionsXE "Attribute definitions"§

Recommendation Q.751.1     (10/95)

adjPc ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.PointCode;

MATCHES FOR EQUALITY;

BEHAVIOUR adjPcBehaviour BEHAVIOUR DEFINED AS

"This attribute represents the pointcode of an adjacent SP.";;

REGISTERED AS { mtpAttribute 1 };

cIC ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.Cic;

MATCHES FOR EQUALITY;

BEHAVIOUR cicBehaviour BEHAVIOUR DEFINED AS

"This attribute is used to reference the SS No. 7 trunk used by the datalink. Its value has to be unique within the SP's connected by the trunk.";;

REGISTERED AS { mtpAttribute 2 };

congestedState ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.CongestedState

MATCHES FOR EQUALITY;

BEHAVIOUR congestedStateBehaviour BEHAVIOUR DEFINED AS

"This attribute is used to reflect route set congestion, if no multiple congestion levels are used. Its value is 'congested' in case of route set congestion. Its value can change from system inside.";;

REGISTERED AS { mtpAttribute 3 };

congestionControlMethod ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.CongestionControlMethod;

BEHAVIOUR congestionControlMethodBehaviour BEHAVIOUR DEFINED AS

"This attribute reflects the congestion control method used within a signalling point.";;

REGISTERED AS { mtpAttribute 4 };

congestionCounting ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.CongestionCounting;

BEHAVIOUR congestionCountingBehaviour BEHAVIOUR DEFINED AS

"This attribute indicates whether congestion counting is message based or octet based.";;

REGISTERED AS { mtpAttribute 5 };

congestionLevel ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.CongestionLevel;

MATCHES FOR EQUALITY;

BEHAVIOUR congestionLevelBehaviour BEHAVIOUR DEFINED AS

"This attribute is used to reflect route set congestion, if multiple congestion levels are used. Its value in case of no congestion is 'none'. Its value can change from system inside.";;

REGISTERED AS { mtpAttribute 6 };

congestionReportingBaseObject ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.CongestionReportingBaseObject;

BEHAVIOUR congestionReportingBaseObjectBehaviour BEHAVIOUR DEFINED AS

"This attribute indicates on what object congestion counting is based.";;

REGISTERED AS { mtpAttribute 7 };

currentCapacityLS ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.CapacityLS;

BEHAVIOUR currentCapacityLSBehaviour BEHAVIOUR DEFINED AS

"This attribute represents the current capacity of a signalling link set.";;

REGISTERED AS { mtpAttribute 8 };

currentCapacitySL ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.CapacityLink;

BEHAVIOUR currentCapacitySLBehaviour BEHAVIOUR DEFINED AS

"This attribute represents the current capacity of a signalling link.";;

REGISTERED AS { mtpAttribute 9 };

dpc ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.PointCode;

MATCHES FOR EQUALITY;

BEHAVIOUR dpcBehaviour BEHAVIOUR DEFINED AS

"This attribute represents a destination pointcode.";;

REGISTERED AS { mtpAttribute 10 };

equipmentPointer ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.Pointer;

MATCHES FOR EQUALITY;

BEHAVIOUR equipmentPointerBehaviour BEHAVIOUR DEFINED AS

"This attribute is used to reference physical equipment.";;

REGISTERED AS { mtpAttribute 11 };

errorCorrectionMethod ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.ErrorCorrectionMethod;

MATCHES FOR EQUALITY;

BEHAVIOUR errorCorrectionMethodBehaviour BEHAVIOUR DEFINED AS

"This attribute is used to specify the type of error-correction the signalling link is using.";;

REGISTERED AS { mtpAttribute 12 };

fixedPriority ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.FixedPriority;

BEHAVIOUR fixedPriorityBehaviour BEHAVIOUR DEFINED AS

"This attribute determines, if the signRouteNePart is used as current route. The signRouteNePart instances with operational state = 'enabled' contained in the same signRouteSetNePart are chosen in ascending order as current routes (The lower the value, the higher the priority).

There are different behaviours for the handling of 'priority gaps' possible:

i) priorityGapAllowedBEHAVIOUR ii) priorityGapNotAllowedBehaviour:

In the latter case it is not possible, to have a priority value, if there is a lower value, which is not used in one of the routes of the route set. E.g. in case of deleting a signRouteNePart instance whose priority value is not equal to the priority of at least one of the remaining signRouteNeParts contained in the same signRouteSetNePart, the priority values of the remaining instances have to be decremented by one (see also example in table below).

What kind of priority gap behaviour is performed by the switch is to be derived from the combination of the values of the attributes vendorName and version of the managedSwitchingElement.";;

REGISTERED AS { mtpAttribute 13 };

flexiblePriority ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.FlexiblePriority;

BEHAVIOUR flexiblePriorityBehaviour BEHAVIOUR DEFINED AS

"This attribute determines, if the signRouteNePart is used as current route. The signRouteNePart instances with operational state = 'enabled' contained in the same signRouteSetNePart are chosen in ascending order as current routes (The lower the value, the higher the priority).

There are different behaviours for the handling of 'priority gaps' possible:

i) priorityGapAllowedBEHAVIOUR ii) priorityGapNotAllowedBehaviour:

In the latter case it is not possible, to have a priority value. If there is a lower value, which is not used in one of the routes of the route set. E.g. in case of deleting a signRouteNePart instance whose priority value is not equal to the priority of at least one of the remaining signRouteNeParts contained in the same signRouteSetNePart, the priority values of the remaining instances have to be decremented by one (see also example in table below).

What kind of priority gap behaviour is performed by the switch is to be derived from the combination of the values of the attributes vendorName and version of the managedSwitchingElement.

This attribute is writable only together with the attribute priorityMode.";;

REGISTERED AS { mtpAttribute 14 };

inClsLoadsharingAlgorithm ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.InClsLoadsharingAlgorithm;

BEHAVIOUR inClsLoadsharingAlgorithmBehaviour BEHAVIOUR

DEFINED AS

"This attribute represents the over- and in-link set loadsharing algorithm within a combined link set.";;

REGISTERED AS { mtpAttribute 15 };

initialLevelOfCongestion ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.InitialLevelOfCongestion;

BEHAVIOUR initialLevelOfCongestionBehaviour BEHAVIOUR DEFINED AS

"This attribute represents the initial level of congestion.";;

REGISTERED AS { mtpAttribute 16 };

inLsLoadShareAlgorithm ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.InLsLoadShareAlgorithm;

BEHAVIOUR inLsLoadShareAlgorithmBehaviour BEHAVIOUR DEFINED AS

"The inLsLoadShareAlgorithm specifies a registered, vendor-specific within-linkset loadsharing algorithm.";;

REGISTERED AS { mtpAttribute 17 };

l2TimerT1 ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.L2TimerT1;

BEHAVIOUR	l2TimerT1Behaviour BEHAVIOUR

DEFINED AS

"Q.703: Level 2 timer 'Alignment ready'. The unit for the INTEGER range is milliseconds.";;

REGISTERED AS { mtpAttribute 18 };

l2TimerT2 ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.L2TimerT2;

BEHAVIOUR	l2TimerT2Behaviour BEHAVIOUR

DEFINED AS

"Q.703: Level 2 timer 'Not aligned'. The unit for the INTEGER range is milliseconds.";;

REGISTERED AS { mtpAttribute 19 };

l2TimerT3 ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.L2TimerT3;

BEHAVIOUR	l2TimerT3Behaviour BEHAVIOUR

DEFINED AS

"Q.703: Level 2 timer 'Aligned'. The unit for the INTEGER range is milliseconds.";;

REGISTERED AS { mtpAttribute 20 };

l2TimerT4E ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.L2TimerT4E;

BEHAVIOUR	l2TimerT4eBehaviour BEHAVIOUR

DEFINED AS

"Q.703: Level 2 timer 'Proving period timer emergency'. The unit for the INTEGER range is milliseconds.";;

REGISTERED AS { mtpAttribute 21 };

l2TimerT4N ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.L2TimerT4N;

BEHAVIOUR	l2TimerT4nBehaviour BEHAVIOUR

DEFINED AS

"Q.703: Level 2 timer 'Proving period timer normal'. The unit for the INTEGER range is milliseconds.";;

REGISTERED AS { mtpAttribute 22 };

l2TimerT5 ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.L2TimerT5;

BEHAVIOUR	l2TimerT5Behaviour BEHAVIOUR

DEFINED AS

"Q.703: Level 2 timer 'Sending SIB'. The unit for the INTEGER range is milliseconds.";;

REGISTERED AS { mtpAttribute 23 };

l2TimerT6 ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.L2TimerT6;

BEHAVIOUR	l2TimerT6Behaviour BEHAVIOUR

DEFINED AS

"Q.703: Level 2 timer 'Remote congestion'. The unit for the INTEGER range is milliseconds.";;

REGISTERED AS { mtpAttribute 24 };

l2TimerT7 ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.L2TimerT7;

BEHAVIOUR	l2TimerT7Behaviour BEHAVIOUR

DEFINED AS

"Q.703: Delay to avoid message mis-sequence on changeover. The unit for the INTEGER range is milliseconds.";;

REGISTERED AS { mtpAttribute 25 };

linkCongestionLevel ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.CongestionLevel;

MATCHES FOR EQUALITY;

BEHAVIOUR linkCongestionLevelBehaviour BEHAVIOUR DEFINED AS

"This attribute is used to reflect link congestion, if multiple congestion levels are used. Its value in case of no congestion is 'none'. Its value can change from system inside.";;

REGISTERED AS { mtpAttribute 26 };

linkTPStatus ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.LinkTPStatus;

MATCHES FOR EQUALITY;

BEHAVIOUR linkTpStatusBehaviour BEHAVIOUR DEFINED AS

"The linkTpStatus contains the SS No. 7 functional statuses as described in Recommendation Q.704. Possible states indicated are local blocked, remote blocked, local inhibited, remote inhibited, failed and deactivated.";;

REGISTERED AS { mtpAttribute 27 };

loadsharingInformationRouteNePart ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.LoadsharingInformationRouteNePart;

BEHAVIOUR loadsharingInformationRouteNePartBehaviour BEHAVIOUR DEFINED AS

"This attribute contains specific information for target specific loadsharing via the current routes working on a route basis. This information can be used by e.g. the referenced mtpLoadsharingObjectFor-Route to execute the target specific loadsharing.";;

REGISTERED AS { mtpAttribute 28 };

loadsharingInformationRouteSetNePart ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.LoadsharingInformationRouteSetNePart;

BEHAVIOUR loadsharingInformationRouteSetNePartBehaviour BEHAVIOUR DEFINED AS

"This attribute contains specific information for target specific loadsharing via the current routes working on a routeset basis via the current routes. This information can be used by e.g. the referenced mtpLoadsharingObjectFor-RouteSet to execute the target specific loadsharing.";;

REGISTERED AS { mtpAttribute 29 };

loopDelay ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.LoopDelay;

MATCHES FOR EQUALITY;

BEHAVIOUR loopDelayBehaviour BEHAVIOUR DEFINED AS

"This attribute models loop delay.";;

REGISTERED AS { mtpAttribute 30 };

lsTimersProfileId ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.NameType;

MATCHES FOR EQUALITY;

BEHAVIOUR lsTimersProfileIdBehaviour BEHAVIOUR DEFINED AS

"This attribute is used for naming instances.";;

REGISTERED AS { mtpAttribute 31 };

lsTimersProfileName ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.AdditionalName;

MATCHES FOR EQUALITY;

BEHAVIOUR lsTimersProfileNameBehaviour BEHAVIOUR DEFINED AS

"This attribute is an additional name for instances of the lsTimersProfile managed object class.";;

REGISTERED AS { mtpAttribute 32 };

lsTimersProfilePointer ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.Pointer;

MATCHES FOR EQUALITY;

BEHAVIOUR lsTimersProfilePointerBehaviour BEHAVIOUR DEFINED AS

"This attribute to the instance of the class lsTimersProfile, containing the Q.704 timers for the signalling link set.";;

REGISTERED AS { mtpAttribute 33 };

managedSwitchingElementName ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.AdditionalName;

MATCHES FOR EQUALITY;

BEHAVIOUR managedSwitchingElementNameBehaviour BEHAVIOUR DEFINED AS

"This attribute is an additional name for instances of the managedSwitchingElement managed object class.";;

REGISTERED AS { mtpAttribute 34 };

maxCapacityLS ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.CapacityLS;

REGISTERED AS { mtpAttribute 35 };

maxCapacitySL ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.CapacityLink;

REGISTERED AS { mtpAttribute 36 };

maxMSUsRetransN1 ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.MaxMSUsRetransN1;

BEHAVIOUR maxMSUsRetransN1Behaviour BEHAVIOUR

DEFINED AS

"This attribute specifies the maximum number of MSUs which are available for retransmission (name in Q.703:N1).";;

REGISTERED AS { mtpAttribute 37 };

maxOctRetransN2 ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.MaxOctRetransN2;

BEHAVIOUR maxOctRetransN2Behaviour BEHAVIOUR DEFINED AS

"This attribute specifies the maximum number of octets, which are available for retransmission (Q.703: N2).";;

REGISTERED AS { mtpAttribute 38 };

mtpAccessPointName ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.AdditionalName;

MATCHES FOR EQUALITY;

BEHAVIOUR mtpAccessPointNameBehaviour BEHAVIOUR DEFINED AS

"This attribute is an additional name for instances of the mtpAccessPoint managed object class.";;

REGISTERED AS { mtpAttribute 39 };

mtpL2ProtocolProfileId ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.NameType;

MATCHES FOR EQUALITY;

BEHAVIOUR mtpL2ProtocolProfileIdBehaviour BEHAVIOUR

DEFINED AS

"Identifies a mtpL2ProtocolProfile instance within the managedSwitching-element; this is an attribute type whose distinguished value can be used as an RDN when naming an instance of the mtpL2ProtocolProfile object class.";;

REGISTERED AS { mtpAttribute 40 };

mtpL2ProtocolProfileName ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.AdditionalName;

MATCHES FOR EQUALITY;

BEHAVIOUR mtpL2ProtocolProfileNameBehaviour BEHAVIOUR DEFINED AS

"This attribute is an additional name for instances of the mtpL2ProtocolProfile managed object class.";;

REGISTERED AS { mtpAttribute 41 };

mtpL2ProtocolProfilePointer ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.Pointer;

MATCHES FOR EQUALITY;

BEHAVIOUR mtpL2ProtocolProfilePointerBehaviour BEHAVIOUR

DEFINED AS

"Identifies an mtpL2ProtocolProfile instance.";;

REGISTERED AS { mtpAttribute 42 };

mtpLevel3Id ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.NameType;

MATCHES FOR EQUALITY;

BEHAVIOUR mtpLevel3IdBehaviour BEHAVIOUR DEFINED AS

"This attribute is used for naming instances.";;

REGISTERED AS { mtpAttribute 43 };

mtpLevel3Name ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.AdditionalName;

MATCHES FOR EQUALITY;

BEHAVIOUR mtpLevel3NameBehaviour BEHAVIOUR DEFINED AS

"This attribute is an additional name for instances of the mtpLevel3 managed object class.";;

REGISTERED AS { mtpAttribute 44 };

mtpLoadsharingObjectForRouteNePartPointer ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.Pointer;

MATCHES FOR EQUALITY;

BEHAVIOUR mtpLoadsharingObjectForRouteNePartPointerBehaviour BEHAVIOUR DEFINED AS

"This attribute is used to reference implementation dependent loadsharing object class instances needed to model implementation dependent mechanisms for loadsharing over linksets working on a route basis.";;

REGISTERED AS { mtpAttribute 45 };

mtpLoadsharingObjectForRouteSetNePartPointer ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.Pointer;

MATCHES FOR EQUALITY;

BEHAVIOUR mtpLoadsharingObjectForRouteSetNePartPointerBehaviour BEHAVIOUR DEFINED AS

"This attribute is used to reference implementation dependent loadsharing object class instances needed to model implementation dependent mechanisms for loadsharing over linksets working on a routeset basis.";;

REGISTERED AS { mtpAttribute 46 };

mtpSignPointId ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.NameType;

MATCHES FOR EQUALITY;

BEHAVIOUR mtpSignPointIdBehaviour BEHAVIOUR DEFINED AS

"This attribute is used for naming instances.";;

REGISTERED AS { mtpAttribute 47 };

mtpSignPointName ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.AdditionalName;

MATCHES FOR EQUALITY;

BEHAVIOUR mtpSignPointNameBehaviour BEHAVIOUR DEFINED AS

"This attribute is an additional name for instances of the mtpSignPoint managed object class.";;

REGISTERED AS { mtpAttribute 48 };

networkIndicator ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.NetworkIndicator;

MATCHES FOR EQUALITY;

BEHAVIOUR networkIndicatorBehaviour BEHAVIOUR DEFINED AS

"This attribute models the network indicator of the network the signalling point belongs to.";;

REGISTERED AS { mtpAttribute 49 };

numberOfCongestionStates ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.NumCongStates;

MATCHES FOR EQUALITY;

BEHAVIOUR numberOfCongestionStatesBehaviour BEHAVIOUR DEFINED AS

"This attribute specifies the number of congestion states used in the 'multiple link congestion states without congestion priority' mechanism as described in 3.8.2.3/Q.704.";;

REGISTERED AS { mtpAttribute 50 };

numberOfNormallyActiveSignLinksTps ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.NumberOfNormallyActiveSignLinksTps;

MATCHES FOR EQUALITY;

BEHAVIOUR numberOfNormallyActiveSignLinksTpsBehaviour BEHAVIOUR DEFINED AS

"This attribute represents the number of normally active signalling links in a link set, as defined in clause 12/Q.704.";;

REGISTERED AS { mtpAttribute 51 };

numberOfThresholdLevels ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.NumberOfThresholdLevels;

BEHAVIOUR numberOfThresholdsLevelsBehaviour BEHAVIOUR DEFINED AS

"This attribute equals the number of threshold levels that are used for congestion control.";;

REGISTERED AS { mtpAttribute 52 };

opc ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.PointCode;

MATCHES FOR EQUALITY;

BEHAVIOUR opcBehaviour BEHAVIOUR DEFINED AS

"This attribute represents the pointcode of an SP from which a certain message has been originated.";;

REGISTERED AS { mtpAttribute 53 };

periodicLinkTestFlag ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.PeriodicLinkTestFlag;

BEHAVIOUR periodicLinkTestFlagBehaviour BEHAVIOUR DEFINED AS

"This attribute indicates whether the periodical signalling link test procedure of Recommendation Q.707 is applied to a link set.";;

REGISTERED AS { mtpAttribute 54 };

pointCode ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.PointCode;

MATCHES FOR EQUALITY;

BEHAVIOUR pointCodeBehaviour BEHAVIOUR DEFINED AS

"This attribute represents the pointcode of a certain SP.";;

REGISTERED AS { mtpAttribute 55 };

priorityMode ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.PriorityMode;

BEHAVIOUR priorityModeBehaviour BEHAVIOUR DEFINED AS

"This attribute is necessary in case of creating a signRouteNePart instance or modifiying its priority attribute. It describes the influence of the management action on the priority attributes of other signRouteNePart instances contained in the same signRouteSetNePart.

This attribute is only writable, and this only together with the writing of attribute flexiblePriority.

The meanings of the attribute values are:

EQUAL: If by modifying the priority with priorityMode 'EQUAL' a 'priority gap' would come into existence, then for the priorities of all created signRouteNePart instances from the same signRouteSetNePart with priorities lower than the 'gap priority' the priority value has to be decremented by one.

INSERT: Priority values of all created signRouteNePart instances from the same signRouteSetNePart with equal or lower priorities are incremented by one.

EXCHANGE_SINGLE: The priority values of all created signRouteNePart instances from the same signRouteSetNePart which have the specified 'target' priority value, are exchanged for the last ('starting') priority value of the specified signRouteNePart instance.

EXCHANGE_GROUP: The priority values of all created signRouteNePart instances from the same signRouteSetNePart which have the specified 'target' priority value, are exchanged for the last ('starting') priority value of the specified signRouteNePart instance. Additionally the priority values of all created signRouteNePart instances from the same signRouteSetNePart, which have the same value as the 'starting' priority value of the specified signRouteNePart instance, are changed to the 'target' value.";;

REGISTERED AS { mtpAttribute 56 };

q704-t1  ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.TimerValue;

BEHAVIOUR q704-t1Behaviour BEHAVIOUR DEFINED AS

"This attribute defines the initial timer value of timer T1 'Delay to avoid message mis-sequencing on changeover from this link' defined in Recommendation Q.704. The value range is from 500 to 1200 ms. For routes with long propagation delays a minimum value of 800 ms is used.The same value applies for all links associated with an instance of the procedure.";;

REGISTERED AS { mtpAttribute 57 };

q704-t2  ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.TimerValue;

BEHAVIOUR q704-t2Behaviour BEHAVIOUR DEFINED AS

"This attribute defines the initial timer value of timer T2 'Waiting for changeover acknowledgement' defined in Recommendation Q.704. The value range is from 700 to 2000 ms. For routes with long propagation delays a minimum value of 1400 ms is used.";;

REGISTERED AS { mtpAttribute 58 };

q704-t3  ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.TimerValue;

BEHAVIOUR q704-t3Behaviour BEHAVIOUR DEFINED AS

"This attribute defines the initial timer value of timer T3 'Time controlled diversion-delay to avoid mis-sequencing on changeback from this link' defined in Recommendation Q.704. The value range is from 500 to 1200 ms. For routes with long propagation delays a minimum value of 800 ms is used. The value of this attribute is network implementation dependent.";;

REGISTERED AS { mtpAttribute 59 };

q704-t4  ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.TimerValue;

BEHAVIOUR q704-t4Behaviour BEHAVIOUR DEFINED AS

"This attribute defines the initial timer value of timer T4 'Waiting for changeback acknowledgement (first attempt)' defined in Recommendation Q.704. The value range is from 500 to 1200 ms. For routes with long propagation delays a minimum value of 800 ms is used.";;

REGISTERED AS { mtpAttribute 60 };

q704-t5  ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.TimerValue;

BEHAVIOUR q704-t5Behaviour BEHAVIOUR DEFINED AS

"This attribute defines the initial timer value of timer T5 'Waiting for changeback acknowledgement (second attempt)' defined in Recommendation Q.704. The value range is from 500 to 1200 ms. For routes with long propagation delays a minimum value of 800 ms is used.";;

REGISTERED AS { mtpAttribute 61 };

q704-t6  ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.TimerValue;

BEHAVIOUR q704-t6Behaviour BEHAVIOUR DEFINED AS

"This attribute defines the initial timer value of timer T6 'Delay to avoid message mis-sequencing on controlled rerouting' defined in Recommendation Q.704. The value range is from 500 to 1200 ms. For routes with long propagation delays a minimum value of 800 ms is used.";;

REGISTERED AS { mtpAttribute 62 };

q704-t7  ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.TimerValue;

BEHAVIOUR q704-t7Behaviour BEHAVIOUR DEFINED AS

"This attribute defines the initial timer value of timer T7 'Waiting for signalling data link connection acknowledgement' defined in Recommendation Q.704. The value range is from 1 to 2 seconds.";;

REGISTERED AS { mtpAttribute 63 };

q704-t8  ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.TimerValue;

BEHAVIOUR q704-t8Behaviour BEHAVIOUR DEFINED AS

"This attribute defines the initial timer value of timer T8 'Transfer prohibited inhibition timer (transient solution)' defined in Recommendation Q.704. The value range is from 800 to 1200 ms.";;

REGISTERED AS { mtpAttribute 64 };

q704-t10  ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.TimerValue;

BEHAVIOUR q704-t10Behaviour BEHAVIOUR DEFINED AS

"This attribute defines the initial timer value of timer T10 'Waiting to repeat signalling route set test message' defined in Recommendation Q.704. The value range is from 30 to 60 seconds. The maximum value may be extended at the discretion of the management function in certain situations, e.g. many signalling points being unavailable or signalling points of known long term unavailability.";;

REGISTERED AS { mtpAttribute 65 };

q704-t11  ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.TimerValue;

BEHAVIOUR q704-t11Behaviour BEHAVIOUR DEFINED AS

"This attribute defines the initial timer value of timer T11 'Transfer restricted timer' defined in Recommendation Q.704. The value range is from 30 to 90 seconds. This is one way of implementing the function described in 13.4/Q.704 and mainly intended to simplify STPs.";;

REGISTERED AS { mtpAttribute 66 };

q704-t12  ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.TimerValue;

BEHAVIOUR q704-t12Behaviour BEHAVIOUR DEFINED AS

"This attribute defines the initial timer value of timer T12 'Waiting for uninhibit acknowledgement' defined in Recommendation Q.704. The value range is from 800 to 1500 ms.";;

REGISTERED AS { mtpAttribute 67 };

q704-t13  ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.TimerValue;

BEHAVIOUR q704-t13Behaviour BEHAVIOUR DEFINED AS

"This attribute defines the initial timer value of timer T13 'Waiting for force uninhibit' defined in Recommendation Q.704. The value range is from 800 to 1500 ms.";;

REGISTERED AS { mtpAttribute 68 };

q704-t14  ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.TimerValue;

BEHAVIOUR q704-t14Behaviour BEHAVIOUR DEFINED AS

"This attribute defines the initial timer value of timer T14 'Waiting for inhibition acknowledgement' defined in Recommendation Q.704. The value range is from 2 to 3 seconds.";;

REGISTERED AS { mtpAttribute 69 };

q704-t15  ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.TimerValue;

BEHAVIOUR q704-t15Behaviour BEHAVIOUR DEFINED AS

"This attribute defines the initial timer value of timer T15 'Waiting to start signalling route set congestion test' defined in Recommendation Q.704. The value range is from 2 to 3 seconds.";;

REGISTERED AS { mtpAttribute 70 };

q704-t16  ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.TimerValue;

BEHAVIOUR q704-t16Behaviour BEHAVIOUR DEFINED AS

"This attribute defines the initial timer value of timer T16 'Waiting for route set congestion status update' defined in Recommendation Q.704. The value range is from 1.4 to 2 seconds.";;

REGISTERED AS { mtpAttribute 71 };

q704-t17  ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.TimerValue;

BEHAVIOUR q704-t17Behaviour BEHAVIOUR DEFINED AS

"This attribute defines the initial timer value of timer T17 'Delay to avoid oscillation of initial alignment failure and link restart' defined in Recommendation Q.704. The value range is from 800 to 1500 ms. The value should be less than the corresponding timer 2 in 12.2.1.2/Q.703.";;

REGISTERED AS { mtpAttribute 72 };

q704-t18  ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.TimerValue;

BEHAVIOUR q704-t18Behaviour BEHAVIOUR DEFINED AS

"This attribute defines the initial timer value of timer T18 defined in Recommendation Q.704. T18 is the Timer within a signalling point whose MTP restarts, for supervising link and linkset activation as well as the receipt of routing information. The value is implementation and network dependent. Criteria to choose T18 are given in 9.2/Q.704.";;

REGISTERED AS { mtpAttribute 73 };

q704-t19  ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.TimerValue;

BEHAVIOUR q704-t19Behaviour BEHAVIOUR DEFINED AS

"This attribute defines the initial timer value of timer T19 'Supervision timer during MTP restart to avoid possible ping-pong of TFP, TFR and TRA messages.' defined in Recommendation Q.704. The value range is from 67 to 69 seconds.";;

REGISTERED AS { mtpAttribute 74 };

q704-t20  ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.TimerValue;

BEHAVIOUR q704-t20Behaviour BEHAVIOUR DEFINED AS

"This attribute defines the initial timer value of timer T20 'Overall MTP restart timer at the signalling point whose MTP restarts.' defined in Recommendation Q.704. The value range is from 59 to 61 seconds.";;

REGISTERED AS { mtpAttribute 75 };

q704-t21  ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.TimerValue;

BEHAVIOUR q704-t21Behaviour BEHAVIOUR DEFINED AS

"This attribute defines the initial timer value of timer T21 'Overall MTP restart timer at a signalling point adjacent to one whose MTP restarts.' defined in Recommendation Q.704. The value range is from 63 to 65 seconds.";;

REGISTERED AS { mtpAttribute 76 };

q704-t22  ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.TimerValue;

BEHAVIOUR q704-t22Behaviour BEHAVIOUR DEFINED AS

"This attribute defines the initial timer value of timer T22 'Local inhibit test timer' defined in Recommendation Q.704. The value range is from 3 to 6 minutes (provisional value).";;

REGISTERED AS { mtpAttribute 77 };

q704-t23  ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.TimerValue;

BEHAVIOUR q704-t23Behaviour BEHAVIOUR DEFINED AS

"This attribute defines the initial timer value of timer T23 'Remote inhibit test timer' defined in Recommendation Q.704. The value range is from 3 to 6 minutes (provisional value).";;

REGISTERED AS { mtpAttribute 78 };

q704-t24  ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.TimerValue;

BEHAVIOUR q704-t24Behaviour BEHAVIOUR DEFINED AS

"This attribute defines the initial timer value of timer T24 'Stabilising timer after removal of local processor outage, used in LPO latching to RPO (national option)' defined in Recommendation Q.704. The value is 500 ms (provisional value).";;

REGISTERED AS { mtpAttribute 79 };

q707-t1 ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.TimerValue;

BEHAVIOUR q707-t1Behaviour BEHAVIOUR DEFINED AS

"This attribute defines a timer value of timer T1 which is defined in Recommendation Q.707.";;

REGISTERED AS { mtpAttribute 80 };

q707-t2 ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.TimerValue;

BEHAVIOUR q707-t2Behaviour BEHAVIOUR DEFINED AS

"This attribute defines a timer value of timer T2 which is defined in Recommendation Q.707.";;

REGISTERED AS { mtpAttribute 81 };

receiveCongestionThresholdAbatement ATTRIBUTE

WITH ATTRIBUTE SYNTAX

MTPDefinedTypesModule.ReceiveCongestionThresholdAbatement;

BEHAVIOUR  receiveCongestionThresholdAbatementBehaviour BEHAVIOUR DEFINED AS

"If the number of MSUs in the reception buffer gets smaller than this number, Level 2 flow control measures end. The value has to be smaller than the respective congestion onset threshold.";;

REGISTERED AS { mtpAttribute 82 };

receiveCongestionThresholdOnset ATTRIBUTE

WITH ATTRIBUTE SYNTAX

MTPDefinedTypesModule.ReceiveCongestionThresholdOnset;

BEHAVIOUR  receiveCongestionThresholdOnsetBehaviour BEHAVIOUR

DEFINED AS

"If the number of MSUs in the reception buffer gets bigger than this value, Level 2 flow control measures begin. MSUs are still accepted.";;

REGISTERED AS { mtpAttribute 83 };

relatedLinkGroupNumber ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.RelatedLinkGroupNumber;

BEHAVIOUR relatedLinkGroupNumberBehaviour BEHAVIOUR DEFINED AS

"This attribute identifies the link group which the signalling link belongs to.";;

REGISTERED AS { mtpAttribute 84 };

remoteExchangeLabel ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.RemoteExchangeLabel;

MATCHES FOR EQUALITY;

BEHAVIOUR remoteExchangeLabelBehaviour BEHAVIOUR DEFINED AS

"This attribute specifies the remote exchange which contains the mtpSignPoint with the point code equal to the signRouteSetNePartId.";;

REGISTERED AS { mtpAttribute 85 };

screeningByLinkSetOrByOpc ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.ScreeningByLinkSetOrByOpc;

BEHAVIOUR screeningByLinkSetOrByOpcBehaviour BEHAVIOUR DEFINED AS

"This attribute defines which of the different options of 8.2/Q.705 are used for identifying unauthorized SS No. 7 messages.";;

REGISTERED AS { mtpAttribute 86 };

signDataLinkTpId ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.NameType;

MATCHES FOR EQUALITY;

BEHAVIOUR signDataLinkTpIdBehaviour BEHAVIOUR DEFINED AS

"This attribute is used for naming instances.";;

REGISTERED AS { mtpAttribute 87 };

signDataLinkTpList ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.PointerList;

MATCHES FOR EQUALITY;

BEHAVIOUR signDataLinkTpListBehaviour BEHAVIOUR DEFINED AS

"This attribute lists the signDataLink instances that can be used for dynamical allocation to the signLinkTp.";;

REGISTERED AS { mtpAttribute 88 };

signDataLinkTpName ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.AdditionalName;

MATCHES FOR EQUALITY;

BEHAVIOUR signDataLinkTpNameBehaviour BEHAVIOUR DEFINED AS

"This attribute is an additional name for instances of the signDataLinkTp managed object class.";;

REGISTERED AS { mtpAttribute 89 };

signDataLinkTpPointer ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.Pointer

MATCHES FOR EQUALITY;

BEHAVIOUR signDataLinkTpPointerBehaviour BEHAVIOUR DEFINED AS

"This attribute references a particular signDataLinkTp instance. The following restriction applies to this attribute: modification is only allowed if the administrative state of the instance that contains this attribute is 'locked'.";;

REGISTERED AS { mtpAttribute 90 };

signLinkSetTpId ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.NameType;

MATCHES FOR EQUALITY;

BEHAVIOUR signLinkSetTpIdBehaviour BEHAVIOUR DEFINED AS

"This attribute is used for naming instances.";;

REGISTERED AS { mtpAttribute 91 };

signLinkSetTpName ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.AdditionalName;

MATCHES FOR EQUALITY;

BEHAVIOUR signLinkSetTpNameBehaviour BEHAVIOUR DEFINED AS

"This attribute is an additional name for instances of the signLinkSetTp managed object class.";;

REGISTERED AS { mtpAttribute 92 };

signLinkSetTpPointer ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.Pointer;

MATCHES FOR EQUALITY;

BEHAVIOUR signLinkSetTpPointerBehaviour BEHAVIOUR DEFINED AS

"References the signLinkSetTp which is intended to be used as first segment of the succession of linksets, which form the signalling route on the network level.

If during creation of a signRouteNePart instance the value of this attribute is referencing a signLinkSetTp instance, which is not created or is already referenced by a signRouteNePart of the same signRouteSetNePart, then the creation of the signRouteNePart is rejected and not carried out.

It is not allowed to reference a signLinksetTp, which is not contained in the same mtpSignPoint as the signRouteSetNePart where the signRouteNePart is contained in.";;

REGISTERED AS { mtpAttribute 93 };

signLinkTpName ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.AdditionalName;

MATCHES FOR EQUALITY;

BEHAVIOUR signLinkTpNameBehaviour BEHAVIOUR DEFINED AS

"This attribute is an additional name for instances of the signLinkTp managed object class.";;

REGISTERED AS { mtpAttribute 94 };

signLinkTpPointer ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.Pointer;

MATCHES FOR EQUALITY;

BEHAVIOUR signLinkTpPointerBehaviour BEHAVIOUR DEFINED AS

"This attribute identifies the signalling link termination point that uses this particular signalling data link.";;

REGISTERED AS { mtpAttribute 95 };

signRouteNePartId ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.NameType;

MATCHES FOR EQUALITY;

BEHAVIOUR signRouteNePartIdBehaviour BEHAVIOUR DEFINED AS

"This is the naming attribute of signRouteNePart. It identifies a signRouteNePart instance within the signRouteSetNePart.";;

REGISTERED AS { mtpAttribute 96 };

signRouteNePartName ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.AdditionalName;

MATCHES FOR EQUALITY;

BEHAVIOUR signRouteNePartNameBehaviour BEHAVIOUR DEFINED AS

"This attribute is an additional name for instances of the signRouteNePart managed object class.";;

REGISTERED AS { mtpAttribute 97 };

signRouteSetNePartId ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.NameType;

MATCHES FOR EQUALITY;

BEHAVIOUR signRouteSetNePartIdBehaviour BEHAVIOUR DEFINED AS

"This is the naming attribute of signRouteSetNePart. It identifies a signRouteSetNePart instance within the mtpSignPoint and equals the signalling point code of the respective destination signalling point.";;

REGISTERED AS { mtpAttribute 98 };

signRouteSetNePartName ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.AdditionalName;

MATCHES FOR EQUALITY;

BEHAVIOUR signRouteSetNePartNameBehaviour BEHAVIOUR DEFINED AS

"Identifies a signRouteSetNePart instance within the managed object class; its value is unique within the signRouteSetNePart object class. In this attribute implementation or administration dependent information - like used for naming schemes - can be stored. If at creation time the value of this attribute given for the instance to be created already exists in another signRouteNePart instance, then the creation is rejected and not carried out.";;

REGISTERED AS { mtpAttribute 99 };

signTermId ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.NameType;

MATCHES FOR EQUALITY;

BEHAVIOUR signTermIdBehaviour BEHAVIOUR DEFINED AS

"This attribute is used for naming instances.";;

REGISTERED AS { mtpAttribute 100 };

signTermList ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.PointerList;

BEHAVIOUR signTermListBehaviour BEHAVIOUR DEFINED AS

"This attribute lists the signTerm instances that can be used for dynamical allocation to the signLinkTp.";;

REGISTERED AS { mtpAttribute 101 };

signTermName ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.AdditionalName;

MATCHES FOR EQUALITY;

BEHAVIOUR signTermNameBehaviour BEHAVIOUR DEFINED AS

"This attribute is an additional name for instances of the signTerm managed object class.";;

REGISTERED AS { mtpAttribute 102 };

signTermPointer ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.Pointer;

MATCHES FOR EQUALITY;

BEHAVIOUR signTermPointerBehaviour BEHAVIOUR DEFINED AS

"This attribute identifies a particular signTerm instance.";;

REGISTERED AS { mtpAttribute 103 };

slCode ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.SlCode;

MATCHES FOR EQUALITY;

BEHAVIOUR slCodeBehaviour BEHAVIOUR DEFINED AS

"If one SLC has been assigned, it should be checked that this SLC will not be assigned again.";;

REGISTERED AS { mtpAttribute 104 };

slsCodeCurrentList ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.SLSCodeCurrentList;

MATCHES FOR SET-INTERSECTION;

BEHAVIOUR slsCodeCurrentListBehaviour BEHAVIOUR DEFINED AS

"This attribute indicates, which SLSs are currently assigned to this signLinkTp. It has to be ensured, that all SLSs are covered and no SLS exists more than one time within the slsCodeCurrentList attributes of the signLinkTp instances contained within one signLinkSetTp. The content of this attribute can be different from that of the attribute slsCodeNormalList. This attribute can be changed from system inside.";;

REGISTERED AS { mtpAttribute 105 };

slsCodeNormalList ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.SlsCodeNormalList;

MATCHES FOR SET-INTERSECTION;

BEHAVIOUR slsCodeNormalListBehaviour BEHAVIOUR DEFINED AS

"This attribute indicates, which SLSs are administratively assigned to this signLinkTp for the case of normal operation.";;

REGISTERED AS { mtpAttribute 106 };

slsList ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.SlsCodeList;

BEHAVIOUR slsListBehaviour BEHAVIOUR DEFINED AS

"This attribute has to be used if loadsharing over link sets toward a specific destination is done. It indicates which SLSs are assigned to this signRouteNePart, in case it is a current route (i.e. the messages with these SLSs are sent over the allocated link set).

It has to be ensured that all SLSs are covered and no SLS exists more than one time within the slsList attributes of the current routes.

The attribute can be written from system inside.";;

REGISTERED AS { mtpAttribute 107 };

slTimersProfileId ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.NameType;

MATCHES FOR EQUALITY;

BEHAVIOUR slTimersProfileIdBehaviour BEHAVIOUR DEFINED AS

"This attribute is used for naming instances.";;

REGISTERED AS { mtpAttribute 108 };

slTimersProfileName ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.AdditionalName;

MATCHES FOR EQUALITY;

BEHAVIOUR slTimersProfileNameBehaviour BEHAVIOUR DEFINED AS

"This attribute is an additional name for instances of the slTimersProfile managed object class.";;

REGISTERED AS { mtpAttribute 109 };

slTimersProfilePointer ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.Pointer;

MATCHES FOR EQUALITY;

BEHAVIOUR slTimersProfilePointerBehaviour BEHAVIOUR DEFINED AS

"This attribute points to the instance of the class slTimersProfile, containing the Q.704 timers for the signalling link.";;

REGISTERED AS { mtpAttribute 110 };

spTimersProfileId ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.NameType;

MATCHES FOR EQUALITY;

BEHAVIOUR spTimersProfileIdBehaviour BEHAVIOUR DEFINED AS

"This attribute is used for naming instances.";;

REGISTERED AS { mtpAttribute 111 };

spTimersProfileName ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.AdditionalName;

MATCHES FOR EQUALITY;

BEHAVIOUR spTimersProfileNameBehaviour BEHAVIOUR DEFINED AS

"This attribute is an additional name for instances of the spTimersProfile managed object class.";;

REGISTERED AS { mtpAttribute 112 };

spTimersProfilePointer ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.Pointer;

MATCHES FOR EQUALITY;

BEHAVIOUR spTimersProfilePointerBehaviour BEHAVIOUR DEFINED AS

"This attribute points to the instance of the class spTimersProfile, containing the Q.704 and Q.707 timers for the signalling point.";;

REGISTERED AS { mtpAttribute 113 };

spType ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.SpType;

MATCHES FOR EQUALITY;

BEHAVIOUR spTypeBehaviour BEHAVIOUR DEFINED AS

"This attribute represents the SP-type: SEP, STEP or STP.";;

REGISTERED AS { mtpAttribute 114 };

stmChannel ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.StmChannel;

MATCHES FOR EQUALITY;

BEHAVIOUR stmChannelBehaviour BEHAVIOUR DEFINED AS

"This attribute denotes the STM channel which defines the signalling datalink on the PCM transmission system.";;

REGISTERED AS { mtpAttribute 115 };

stpScreeningTableListMode ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.StpScreeningTableListMode;

BEHAVIOUR stpScreeningTableListModeBehaviour BEHAVIOUR DEFINED AS

"The stpScreeningTableListMode attribute indicates whether the STP access is allowed or not by the designated combinations contained in the subordinate stpScreeningTableLine's (see 8.2/Q.705).

TRUE:	STP access allowed

FALSE:	STP access inhibited.";;

REGISTERED AS { mtpAttribute 116 };

stpScreeningTableId ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.NameType;

MATCHES FOR EQUALITY;

BEHAVIOUR stpScreeningTableIdBehaviour BEHAVIOUR DEFINED AS

"The STP Screening Table Id is an attribute used for naming purposes.";;

REGISTERED AS { mtpAttribute 117 };

stpScreeningTableLineContent ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.StpScreeningTableLineContent;

BEHAVIOUR stpScreeningTableLineContentBehaviour BEHAVIOUR DEFINED AS

"The stpScreeningTableLineContent attribute represents the content of one line of the STP ScreeningTable. The content consists of elements indicating the designated DPCs which are to be screened in combination with the OPC or linkset given by the stpScreeningTableLineId and an indication how a message fulfilling such a combination is to be treated.";;

REGISTERED AS { mtpAttribute 118 };

stpScreeningTableLineId ATTRIBUTE

MATCHES FOR EQUALITY;

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.StpScreeningTableLineId;

BEHAVIOUR stpScreeningTableLineIdBehaviour BEHAVIOUR DEFINED AS

"The stpScreeningTableLineId is an attribute used for naming purposes. Depending on the value of attribute screeningByLinkSetOrByDpc of the superior stpScreeningTable instance it represents the OPC or linkset which is designated to be screened for STP traffic.";;

REGISTERED AS { mtpAttribute 119 };

stpScreeningTableLineName ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.AdditionalName;

MATCHES FOR EQUALITY;

BEHAVIOUR stpScreeningTableLineNameBehaviour BEHAVIOUR DEFINED AS

"This attribute is an additional name for instances of the stpScreeningTableLine managed object class.";;

REGISTERED AS { mtpAttribute 120 };

stpScreeningTableName ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.AdditionalName;

MATCHES FOR EQUALITY;

BEHAVIOUR stpScreeningTableNameBehaviour BEHAVIOUR DEFINED AS

"This attribute is an additional name for instances of the stpScreeningTable managed object class.";;

REGISTERED AS { mtpAttribute 121 };

timerTx ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.TimerValue;

MATCHES FOR EQUALITY;

BEHAVIOUR timerTxBehaviour BEHAVIOUR DEFINED AS

"This attribute identifies the Tx timer value for the 'multiple link congestion states without congestion priority' mechanism as described in 3.8.2.3/Q.704.";;

REGISTERED AS { mtpAttribute 122 };

timerTy ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.TimerValue;

BEHAVIOUR timerTyBehaviour BEHAVIOUR DEFINED AS

"This attribute identifies the Ty timer value for the 'multiple link congestion states without congestion priority' mechanism as described in 3.8.2.3/Q.704.";;

REGISTERED AS { mtpAttribute 123 };

transmissionCongestionThresholdAbatementLevel1 ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.

TransmissionCongestionThresholdAbatementLevel1;

BEHAVIOUR transmissionCongestionThresholdAbatementLevel1Behaviour BEHAVIOUR

DEFINED AS

"If the number of MSUs in the transmission / retransmission buffer gets smaller than this number, Level 2 flow control measures end. The value has to be smaller than the respective congestion onset threshold.";;

REGISTERED AS { mtpAttribute 124 };

transmissionCongestionThresholdAbatementLevel2 ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.

TransmissionCongestionThresholdAbatementLevel2;

BEHAVIOUR transmissionCongestionThresholdAbatementLevel2Behaviour BEHAVIOUR

DEFINED AS

"If the number of MSUs in the transm./retransm. buffer of a link in the congestionLevel 2 gets smaller than this number, the resp. link is not any more in congestionLevel 2. CongestionLevel 2 measures end. The value has to be smaller than the respective congestion onset threshold.";;

REGISTERED AS { mtpAttribute 125 };

transmissionCongestionThresholdAbatementLevel3 ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.

TransmissionCongestionThresholdAbatementLevel3;

BEHAVIOUR transmissionCongestionThresholdAbatementLevel3Behaviour BEHAVIOUR

DEFINED AS

"If the number of MSUs in the transm./retransm. buffer of a link in the congestionLevel 3 gets smaller than this number, the resp. link is not any more in congestionLevel 3. CongestionLevel 3 measures end. The value has to be smaller than the respective congestion onset threshold.";;

REGISTERED AS { mtpAttribute 126 };

transmissionCongestionThresholdDiscardLevel1 ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.

TransmissionCongestionThresholdDiscardLevel1;

BEHAVIOUR transmissionCongestionThresholdDiscardLevel1Behaviour BEHAVIOUR

DEFINED AS

"If the number of MSUs in the transmission / retransmission buffer of a link in the congestion level 1 gets bigger than this number, MSUs are discarded (this threshold equals the respective buffer size). The value has to be bigger than the respective congestion onset threshold.";;

REGISTERED AS { mtpAttribute 127 };

transmissionCongestionThresholdDiscardLevel2 ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.

TransmissionCongestionThresholdDiscardLevel2;

BEHAVIOUR transmissionCongestionThresholdDiscardLevel2Behaviour BEHAVIOUR

DEFINED AS

"If the number of MSUs in the transm./retransm. buffer of a link in the congestion level 2 gets bigger than this number, MSUs are discarded (this threshold equals the respective buffer size). The value has to be bigger than the respective congestion onset threshold.";;

REGISTERED AS { mtpAttribute 128 };

transmissionCongestionThresholdDiscardLevel3 ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.

TransmissionCongestionThresholdDiscardLevel3;

BEHAVIOUR transmissionCongestionThresholdDiscardLevel3Behaviour BEHAVIOUR

DEFINED AS

"If the number of MSUs in the transmission / retransmission buffer of a link in the congestion level 3 gets bigger than this number, MSUs are discarded (this threshold equals the respective buffer size). The value has to be bigger than the respective congestion onset threshold.";;

REGISTERED AS { mtpAttribute 129 };

transmissionCongestionThresholdOnsetLevel1 ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.

TransmissionCongestionThresholdOnsetLevel1;

BEHAVIOUR transmissionCongestionThresholdOnsetLevel1Behaviour BEHAVIOUR

DEFINED AS

"If the number of MSUs in the transmission / retransmission buffer gets bigger than this value, the respective link is congested in congestionLevel 1. CongestionLevel 1 measures start.";;

REGISTERED AS { mtpAttribute 130 };

transmissionCongestionThresholdOnsetLevel2 ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.

TransmissionCongestionThresholdOnsetLevel2;

BEHAVIOUR transmissionCongestionThresholdOnsetLevel2Behaviour BEHAVIOUR

DEFINED AS

"If the number of MSUs in the transmission / retransmission buffer gets bigger than this value, the respective link is congested in congestionLevel 2. CongestionLevel 2 measures start.";;

REGISTERED AS { mtpAttribute 131 };

transmissionCongestionThresholdOnsetLevel3 ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.

TransmissionCongestionThresholdOnsetLevel3;

BEHAVIOUR transmissionCongestionThresholdOnsetLevel3Behaviour BEHAVIOUR

DEFINED AS

"If the number of MSUs in the transmission / retransmission buffer gets bigger than this value, the respective link is congested in congestionLevel 3. CongestionLevel 3 measures start.";;

REGISTERED AS { mtpAttribute 132 };

transmissionRate ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.TransmissionRate;

MATCHES FOR EQUALITY;

BEHAVIOUR transmissionRateBehaviour BEHAVIOUR DEFINED AS

"This attribute represents the transmissionRate and implies the transmissionType (i.e. 56 and 64 kbits imply digital, 4.8 kbits implies analog transmission).";;

REGISTERED AS { mtpAttribute 133 };

treatmentOfOutsideRanges ATTRIBUTE

WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.TreatmentOf;

BEHAVIOUR treatmentOfOutsideRangesBehaviour BEHAVIOUR DEFINED AS

"The treatmentOfOutsideRanges attribute specifies the treatment of the messages outside the designated ranges given the stpScreeningTable managed object (see 8.3/Q.705, part two).

normal : allow all STP messages outside the designated ranges

discard : discard all STP messages outside the designated ranges.";;

REGISTERED AS { mtpAttribute 134 };

## 7.5	Action definitionsXE "Action definitions"§

getScreenedOpcsOrLinkSetsByDpc ACTION

BEHAVIOUR getScreenedOpcsOrLinkSetsByDpcBehaviour BEHAVIOUR DEFINED AS

"This action gets all OPCs or linksets (depending on the value of the screeningByLinkSetOrByOPC attribute) together with their respective messageTreatments which are screened in combination with a specific DPC within all the stpScreeningTableLineContents of all stpScreeningTableLine's contained in the stpScreeningTable.

The resulting list in the reply is empty, if no OPC or Linkset within all the stpScreeningTableLine (Contents of all stpScreeningTableLine's contained in the stpScreeningTable is screened in combination with the DPC - given in the information syntax), or if no stpScreeningTableLine contained in the stpScreeningTable does exist.";;

MODE CONFIRMED;

WITH INFORMATION SYNTAX MTPDefinedTypesModule.SpecificDpc

WITH REPLY SYNTAX MTPDefinedTypesModule.ScreenedOpcsOrLinkSetsList;

REGISTERED AS { mtpAction 1 };

localInhibit ACTION

BEHAVIOUR localInhibitBehaviour BEHAVIOUR DEFINED AS

"This action inhibits the signalling link termination point. If the inhibition procedure completes successfully the linkTpStatus contains localInhibit. This action is not performed if the linkTpStatus contains localInhibit.";;

MODE CONFIRMED;

WITH REPLY SYNTAX MTPDefinedTypesModule.ResultOfAction;

REGISTERED AS { mtpAction 2 };

localUninhibit ACTION

BEHAVIOUR localUninhibitBehaviour BEHAVIOUR DEFINED AS

"This action uninhibits the signalling link termination point. If the uninhibition procedure completes successfully the localInhibit is removed from linkTpStatus. This action is not performed if the linkTpStatus does not contain localInhibit.";;

MODE CONFIRMED;

WITH REPLY SYNTAX MTPDefinedTypesModule.ResultOfAction;

REGISTERED AS { mtpAction 3 };

replaceSignTerm ACTION

BEHAVIOUR replaceSignTermBehaviour BEHAVIOUR DEFINED AS

"This action replaces the signalling terminal in a signalling link termination point. This action is not performed if:

i)	the signTermPointer does not reference a signTerm

or	ii)	the new signTerm does not exist

or	iii)	the signTerm is already referenced by the link

or	iv)	the linkTpStatus does not contain the values 'deactivated' or 'failed'.

If the action has been successful, the signTermPointer has the value of the signTermId given in the information syntax; otherwise the pointer value remains unchanged.";;

MODE CONFIRMED;

WITH INFORMATION SYNTAX	MTPDefinedTypesModule.Pointer;

WITH REPLY SYNTAX MTPDefinedTypesModule.ResultOfAction;

REGISTERED AS { mtpAction 4 };

signLinkTest ACTION

BEHAVIOUR signLinkTestBehaviour BEHAVIOUR DEFINED AS

"This test is performed to analyse the signalling link.";;

WITH REPLY SYNTAX MTPDefinedTypesModule.ResultOfAction;

REGISTERED AS { mtpAction 5 };

## 7.6	Notification definitionsXE "Notification definitions"§

mtpRestarting NOTIFICATION

WITH INFORMATION SYNTAX MTPDefinedTypesModule.MtpRestarting;

REGISTERED AS { mtpNotification 1 };

ss7OnOccEvent NOTIFICATION

BEHAVIOUR ss7OnOccEventBhv BEHAVIOUR DEFINED AS

"This notification is used to report the Q.752 on-occurrence measurement results and other MTP events, which are not reported as a communication alarm.";;

WITH INFORMATION SYNTAX MTPDefinedTypesModule.SS7OnOccEventInfo;

REGISTERED AS { mtpNotification 2 };

## 7.7	Naming binding definitionsXE "Naming binding definitions"§

Naming is specified by structure rules and name bindings. The structure rules define the distinguished names that the managed objects shall have and the ways in which the objects are related to one another in the Management Information Tree (MIT).

Each structure rule defines a sequence of name bindings. The name binding selects the attribute to be used in the relative distinguished name of an object. The structure rule herewith selects all the attributes to be used in the distinguished name of an object.

For the labels that are used for the name binding templates, the convention "superiorClass-subordinateClass" is followed.

managedSwitchingElement-mtpSignPoint NAME BINDING

SUBORDINATE OBJECT CLASS	mtpSignPoint;

NAMED BY

SUPERIOR OBJECT CLASS	managedSwitchingElement;

WITH ATTRIBUTE mtpSignPointId;

BEHAVIOUR managedSwitchingElement-mtpSignPointBehaviour BEHAVIOUR DEFINED AS

"An mtpSignPoint can be created by an operator or automatically.

In case the spTimersProfilePackage is used, a create or set request is rejected, if

i)	the spTimersProfilePointer does not reference an spTimersProfile

OR	ii)	the spTimersProfilePointer would reference an instance which does not exist

If the name package is supported: a create request with a value for the name attribute that is already used by another instance of the same object class will be rejected.

An mtpSignPoint can be deleted if and only if it does not contain any other managed object class instances, except for contained measurements.";;

CREATE;

DELETE ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS { mtpNameBinding 1 };

mtpSignPoint-signRouteSetNePart NAME BINDING

SUBORDINATE OBJECT CLASS	signRouteSetNePart;

NAMED BY

SUPERIOR OBJECT CLASS	mtpSignPoint;

WITH ATTRIBUTE signRouteSetNePartId;

BEHAVIOUR mtpSignPoint-signRouteSetNePartBehaviour BEHAVIOUR DEFINED AS

"If the name package is supported: a create request with a value for the name attribute that is already used by another instance of the same object class will be rejected.

A delete request is rejected, if the administrativeState of the signRouteSetNePart does not equal locked.";;

CREATE;

DELETE ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS { mtpNameBinding 2 };

signRouteSetNePart-signRouteNePart NAME BINDING

SUBORDINATE OBJECT CLASS 	signRouteNePart;

NAMED BY

SUPERIOR OBJECT CLASS	signRouteSetNePart;

WITH ATTRIBUTE signRouteNePartId;

BEHAVIOUR signRouteSetNePart-signRouteNePartBehaviour BEHAVIOUR DEFINED AS

"A create request is rejected, if the signlinkSetTpPointer would reference an instance, which

i)	does not belong to the signLinkSetTp object class

OR	ii)	does not exist

OR	iii)	is not contained in the same mtpSignPoint as the signRouteNePart

OR	iv)	is already referenced by a signRouteNePart contained within the same signRouteSetNePart

A delete request is rejected if its administrativeState does not equal locked.

If the name package is supported: a create request with a value for the name attribute that is already used by another instance of the same object class will be rejected.";;

CREATE;

DELETE;

REGISTERED AS { mtpNameBinding 3 };

signLinkSetTp-signLinkTp NAME BINDING

SUBORDINATE OBJECT CLASS	signLinkTp AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS	signLinkSetTp;

WITH ATTRIBUTE slCode;

BEHAVIOUR signLinkSetTp-signLinkTpBehaviour BEHAVIOUR DEFINED AS

"If the name package is supported: a create request with a value for the name attribute that is already used by another instance of the same object class will be rejected.";;

CREATE;

DELETE;

REGISTERED AS { mtpNameBinding 4 };

mtpSignPoint-mtpAccessPoint NAME BINDING

SUBORDINATE OBJECT CLASS	mtpAccessPoint;

NAMED BY

SUPERIOR OBJECT CLASS	mtpSignPoint;

WITH ATTRIBUTE	"ITU-T Rec. X.723 (1993) | ISO/IEC 10165-5 : 1993":sapId;

BEHAVIOUR mtpSignPoint-mtpAccessPointBehaviour BEHAVIOUR DEFINED AS

"If the name package is supported: a create request with a value for the name attribute that is already used by another instance of the same object class will be rejected.";;

CREATE;

DELETE;

REGISTERED AS { mtpNameBinding 5 };

mtpSignPoint-mtpLevel3 NAME BINDING

SUBORDINATE OBJECT CLASS	mtpLevel3;

NAMED BY

SUPERIOR OBJECT CLASS	mtpSignPoint;

WITH ATTRIBUTE mtpLevel3Id;

BEHAVIOUR mtpSignPoint-mtpLevel3Behaviour BEHAVIOUR DEFINED AS

"If the name package is supported: a create request with a value for the name attribute that is already used by another instance of the same object class will be rejected.";;

CREATE;

DELETE ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS { mtpNameBinding 6 };

mtpSignPoint-signDataLinkTp NAME BINDING

SUBORDINATE OBJECT CLASS	signDataLinkTp;

NAMED BY

SUPERIOR OBJECT CLASS	mtpSignPoint;

WITH ATTRIBUTE signDataLinkTpld;

BEHAVIOUR mtpSignPoint-signDataLinkTpBehaviour BEHAVIOUR DEFINED AS

"A create request is rejected if the equipmentPointer would reference equipment that does not exist.

If the name package is supported: a create request with a value for the name attribute that is already used by another instance of the same object class will be rejected.";;

CREATE;

DELETE;

WITH ATTRIBUTEsignDataLinkId;
REGISTERED AS { mtpNameBinding 7 };

mtpSignPoint-signLinkSetTp NAME BINDING

SUBORDINATE OBJECT CLASS	signLinkSetTp;

NAMED BY

SUPERIOR OBJECT CLASS	mtpSignPoint;

WITH ATTRIBUTE signLinkSetId;
	BEHAVIOUR mtpSignPoint-signLinkSetTpBehaviour BEHAVIOUR DEFINED AS

"A creation request is only successful if a signRouteSetNePart exists within the mtpSignPoint for the value of attribute adjPC

In case the lsTimersProfilePackage is used, create or set request is rejected, if

i)	the lsTimersProfilePointer does not reference an lsTimersProfile

OR	ii)	the lsTimersProfilePointer would reference an instance which does not exist

If the name package is supported: a create request with a value for the name attribute that is already used by another instance of the same object class will be rejected.

If it is tried to delete a signLinkSetTp, which is referenced by at least one signRouteNePart, the deletion of the signLinkSetTp is rejected and not carried out.";;

CREATE;

DELETE ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS { mtpNameBinding 8 };

managedSwitchingElement-signTerm NAME BINDING

SUBORDINATE OBJECT CLASS	signTerm;

NAMED BY

SUPERIOR OBJECT CLASS	managedSwitchingElement;

WITH ATTRIBUTE signTermId;

BEHAVIOUR managedSwitchingElement-signTermBehaviour BEHAVIOUR DEFINED AS

"A create request is rejected if the equipmentPointer would reference equipment that does not exist.

If the name package is supported: a create request with a value for the name attribute that is already used by another instance of the same object class will be rejected.";;

CREATE;

DELETE;

REGISTERED AS { mtpNameBinding 9 };

mtpSignPoint-stpScreeningTable NAME BINDING

SUBORDINATE OBJECT CLASS	stpScreeningTable;

NAMED BY

SUPERIOR OBJECT CLASS	mtpSignPoint;

WITH ATTRIBUTE stpScreeningTableId;

BEHAVIOUR mtpSignPoint-stpScreeningTableBehaviour BEHAVIOUR DEFINED AS

"If the name package is supported: a create request with a value for the name attribute that is already used by another instance of the same object class will be rejected.

It is only allowed to delete an instance of this object class if its administrative state equals 'locked'.";;

CREATE;

DELETE;

REGISTERED AS { mtpNameBinding 10 };

managedSwitchingElement-lsTimersProfile NAME BINDING

SUBORDINATE OBJECT CLASS 	lsTimersProfile;

NAMED BY

SUPERIOR OBJECT CLASS	managedSwitchingElement;

WITH ATTRIBUTE		lsTimersProfileId;

BEHAVIOUR managedSwitchingElement-lsTimersProfileBehaviour BEHAVIOUR DEFINED AS

"If the name package is supported: a create request with a value for the name attribute that is already used by another instance of the same object class will be rejected.

If an attempt is made to delete an lsTimersProfile which is still referenced by a signLinkSetTp the delete request will be rejected.";;

CREATE;

DELETE;

REGISTERED AS { mtpNameBinding 11 };

managedSwitchingElement-mtpL2ProtocolProfile NAME BINDING

SUBORDINATE OBJECT CLASS	mtpL2ProtocolProfile;

NAMED BY

SUPERIOR OBJECT CLASS	managedSwitchingElement;

WITH ATTRIBUTE mtpL2ProtocolProfileId;

BEHAVIOUR managedSwitchingElement-mtpL2ProtocolProfileBehaviour BEHAVIOUR DEFINED AS

"If the name package is supported: a create request with a value for the name attribute that is already used by another instance of the same object class will be rejected.

If an attempt  is made to delete an mtpL2ProtocolProfile which is still referenced by a signLinkTP the delete request will be rejected.";;

CREATE;

DELETE;

REGISTERED AS { mtpNameBinding 12 };

managedSwitchingElement-slTimersProfile NAME BINDING

SUBORDINATE OBJECT CLASS	slTimersProfile;

NAMED BY

SUPERIOR OBJECT CLASS	managedSwitchingElement;

WITH ATTRIBUTE		slTimersProfile Id;

BEHAVIOUR managedSwitchingElement-slTimersProfileBehaviour BEHAVIOUR DEFINED AS

"If the name package is supported: a create request with a value for the name attribute that is 	
already used by another instance of the same object class will be rejected.

If an attempt is made to delete an slTimersProfile which is still referenced by a signLinkTp the delete request will be rejected.";;

CREATE;

DELETE;

REGISTERED AS { mtpNameBinding 13 };

managedSwitchingElement-spTimersProfile NAME BINDING

SUBORDINATE OBJECT CLASS	spTimersProfile;

NAMED BY

SUPERIOR OBJECT CLASS	managedSwitchingElement;

WITH ATTRIBUTE	spTimersProfileId;

BEHAVIOUR managedSwitchingElement-spTimersProfileBehaviour BEHAVIOUR DEFINED AS

"If the name package is supported: a create request with a value for the name attribute that is already used by another instance of the same object class will be rejected.

If an attempt is made to delete an spTimersProfile which is still referenced by an mtpSignPoint the delete request will be rejected.";;

CREATE;

DELETE;

REGISTERED AS { mtpNameBinding 14 };

stpScreeningTable-stpScreeningTableLine NAME BINDING

SUBORDINATE OBJECT CLASS	stpScreeningTableLine;

NAMED BY

SUPERIOR OBJECT CLASS	stpScreeningTable;

WITH ATTRIBUTE stpScreeningTableLineId;

BEHAVIOUR stpScreeningTable-stpScreeningTableLineBehaviour BEHAVIOUR DEFINED AS

"If the name package is supported: a create request with a value for the name attribute that is already used by another instance of the same object class will be rejected. The total number of screened DPC's across all stpScreeningTableLine instances contained by an stpScreeningTable must be less or equal to maxNoOfScreenedDpcs. A respectively unallowed create request is rejected. If one tries to create an instance with an stpScreeningTableLineId which would represent a linkset which is not contained within the same signPoint as the stpScreeningTable, the create request is rejected. Instances of this object class can be deleted automatically if the superior stpScreeningTable is deleted.";;

CREATE;

DELETE;

REGISTERED AS { mtpNameBinding 15 };

## 7.8	Abstract syntax productionsXE "Abstract syntax productions"§

-- The syntax of this ASN.1 is based on Recommendation X.208. However, from Recommendation X.680,

-- the "ellipsis notation" is used.

MTPDefinedTypesModule

{itu-t recommendation q(17) omap(751) mtp(1) informationModel(0) asn1Modules(2)

mtpDefinedTypesModule(0)}

DEFINITIONS IMPLICIT TAGS ::= BEGIN

IMPORTS

AdministrativeState, SimpleNameType, ProbableCause, SpecificProblems, PerceivedSeverity,

NotificationIdentifier, CorrelatedNotifications, AdditionalText, AdditionalInformation FROM Attribute-

ASN1Module

{joint-iso-itu-t ms(9) smi(3) part2(2) asn1Module(2) 1}

NameType, Pointer, ObjectList, PointerOrNull FROM ASN1DefinedTypesModule

{itu-t recommendation m gnm(3100) informationModel(0) asn1Modules(2) asn1DefinedTypesModule(0)}

ObjectInstance FROM CMIP-1

{joint-iso-itu-t ms(9) cmip(1) modules(0) protocol(3)};

mtpInformationModel OBJECT IDENTIFIER ::= {itu-t recommendation q(17) omap(751) mtp(1) informationModel(0)}

mtpObjectClass OBJECT IDENTIFIER ::= {mtpInformationModel managedObjectClass(3)}

mtpPackage OBJECT IDENTIFIER ::= {mtpInformationModel package(4)}

mtpParameter OBJECT IDENTIFIER ::= {mtpInformationModel parameter(5)}

mtpAttribute OBJECT IDENTIFIER ::= {mtpInformationModel attribute(7)}

mtpNameBinding OBJECT IDENTIFIER ::= {mtpInformationModel nameBinding(6)}

mtpAction OBJECT IDENTIFIER ::= {mtpInformationModel action(9)}

mtpNotification OBJECT IDENTIFIER ::= {mtpInformationModel notification(10)}

mtpSpecificExtensions OBJECT IDENTIFIER ::= { mtpInformationModel specificExtensions(0) }

AdditionalName ::= IA5String -- the size of this string is implementation dependent

administrativeStateLocked	AdministrativeState ::= locked

AlternativeSLCodeList ::= SEQUENCE SIZE (0..15) OF SLCode

CapacityLink ::= INTEGER (0..maxOctetsPerSecondLink)

CapacityLS ::= INTEGER (0..maxOctetsPerSecondLS)

ChangeInLsToAdjSpInfo ::= SEQUENCE {

oldSignLinkSetTp   NameType,

newSignLinkSetTp NameType }

Cic ::= INTEGER (0..4095)

CongestedState ::= ENUMERATED {

notCongested	(0),

congested	(1)}

CongestionControlMethod ::= ENUMERATED {

unknown	(0),

ccmQ704International	(1),

ccmQ704NationalWithPrio	(2),

ccmQ704NationalNoPrioBufferMechanism	(3),

ccmQ704NationalNoPrioTimerMechanism	(4)}

CongestionCounting ::= ENUMERATED {

octet	(0),

message	(1)}

CongestionLevel ::= ENUMERATED {

none	(0),

congestionLevel1	(1),

congestionLevel2	(2),

congestionLevel3	(3)}

CongestionReportingBaseObject ::= ENUMERATED {

routeSet	(0),

linkSetOfCongestedRouteSet	(1),

linkOfCongestedRouteSet	(2),

congestedLinkOfCongestedRoute	(3)}

DpcComponent ::= CHOICE {

dpc	[0]	PointCode,

dpcRange	[1]	SEQUENCE {

firstDPC	PointCode,

lastDPC	PointCode }}

ErrorCorrectionMethod ::= ENUMERATED

{basicRetr	(0),

prevCyclRetr	(1) }

EventsCongestionLevel ::= SEQUENCE {

events	INTEGER,

conglevel	CongestionLevel }

EventsUP  ::= SEQUENCE {

events	INTEGER,

userpart	UserPart}

FixedPriority ::= Priority

FlexiblePriority ::= Priority

InaccessibleSpInfo ::= SEQUENCE {

networkIndicator	NetworkIndicator,

pointCode		PointCode }

InClsLoadsharingAlgorithm ::= SET OF SlsLinkPriorityList

InfoRequest ::= BITSTRING {

pointCode	(0),

pointCodeList 	(1),

routePriorityList	(2),

...}

InitialLevelOfCongestion ::= INTEGER (1..3) -- This equals the value s from B.1.2.9.2

InLsLoadShareAlgorithm ::= OBJECT IDENTIFIER

L2TimerT1 ::= INTEGER (13000..600000)

-- time, in milliseconds

L2TimerT2 ::= INTEGER (5000..150000)

-- time, in milliseconds

L2TimerT3 ::= INTEGER (1000..14000)

-- time, in milliseconds

L2TimerT4E ::= INTEGER (400..8000)

-- time, in milliseconds

L2TimerT4N ::= INTEGER (7500..120000)

-- time, in milliseconds

L2TimerT5 ::= INTEGER (80..120)

-- time, in milliseconds

L2TimerT6 ::= INTEGER (3000..12000)

-- time, in milliseconds

L2TimerT7 ::= INTEGER (500..6000)

-- time, in milliseconds

LinkSetPriorityListSntx ::= SEQUENCE OF NameType

LinkTPStatus ::= BIT STRING {

localBlocked	(0),

remoteBlocked	(1),

localInhibited	(2),

remoteInhibited	(3),

failed	(4),

deactivated	(5)}

LoadsharingInformationRouteNePart ::= IA5String

-- the size of this string is implementation dependent

LoadsharingInformationRouteSetNePart ::= IA5String

-- the size of this string is implementation dependent

LocalUpUnavailable ::= UserPart

LoopDelay ::= INTEGER

-- time, in milliseconds

ManagementExtension ::= SEQUENCE {

identifier	OBJECT IDENTIFIER,

significance	[1]	BOOLEAN DEFAULT FALSE,

information	[2]	ANY DEFINED BY identifier }

maxLinksInLs  INTEGER :: = 16

MaxMSUsRetransN1 ::= INTEGER (0..127)

-- lower range is implementation dependent

MaxMSUsRetransN2 ::= INTEGER

maxNoOfScreenedDpcs INTEGER ::= 128

-- This value is suggested by clause 8/Q.705. It is likely that this value is too low for international networks

maxNoOfScreenedOpcs INTEGER ::= 128

-- This value is suggested by clause 8/Q.705. It is likely that this value is too low for international networks

MaxNumOfTests ::= INTEGER

maxOctetsPerSecondLink  INTEGER ::= 8000

maxOctetsPerSecondLS  INTEGER ::= 128000

MaxOctRetransN2 ::= INTEGER

maxSpc INTEGER ::= 16383

MRVTActionResult ::= CHOICE {

resultOfMRTVT	[0]	ResultOfMRVT,

reasonForRejection	[1]	ReasonForRejection}

MRVTFailureString ::= BIT STRING {

detectedLoop	(0),

excessiveLengthRoute	(1),

unknownDestination	(2),

routeInaccessible	(3),

processingFailure	(4),

unknownInitiatingSP	(5),

timerExpired	(6),

spNotAnSTP	(7),

indirectRoute	(8),

maxNrMRVTestsAlready	(9), -- Info from adjacent SP

... }

MtpRestarting ::= PointCode

MtpRouteVerifTestResultSntx ::= SEQUENCE {

testedDestination	PointCode,	-- 2.2.2.3 b)/Q.753

routeTestResult	ResultOfMRVT,	-- 2.2.2.3 c)/Q.753

failureType	[1]	MRVTFailureString OPTIONAL,	-- 2.1.1.3.1/Q.754

routeTrace	[2]	RouteTrace OPTIONAL,	-- 2.2.2.3 d)/Q.753

copyData	[6]	OCTET STRING OPTIONAL	-- 2.2.2.3 e)/Q.753

}

NetworkIndicator ::= ENUMERATED{

internationalNetwork1	(0),

internationalNetwork2	(1),

nationalNetwork 1	(2)

nationalNetwork 2	(3)}

NumCongStates ::= INTEGER (0..3)

NumberOfNormallyActiveSignLinksTps ::= INTEGER (0..16)

NumberOfThresholdLevels ::= INTEGER (2 | 3)

PeriodicLinkTestFlag ::= ENUMERATED {

on	(0),

off	(1)}

periodicTestFlagDefault PeriodicLinkTestFlag ::= off

PointCode ::= INTEGER (0..maxSpc)

PointCodeList ::= SEQUENCE OF PointCode

PointerList ::= SEQUENCE OF Pointer

Priority ::= INTEGER

-- maximum value is implementation dependent

PriorityInfo ::= INTEGER {

unknown 	(0),

firstChoice 	(1),

secondChoice 	(2),

thirdChoice 	(3),

...} (0..255)

PriorityMode ::= ENUMERATED {

insert	(0),

equal	(1),

exchangeSingle	(2),

exchangeGroup	(3),

priorityModeDefault PriorityMode ::= insert

localUserPartUnavailable ProbableCause ::= globalValue: {mtpSpecificExtensions 001}

q752Item1p02 ProbableCause ::= globalValue: {mtpSpecificExtensions 102}

q752Item1p10 ProbableCause ::= globalValue: {mtpSpecificExtensions 110}

q752Item2p10 ProbableCause ::= globalValue: {mtpSpecificExtensions 210}

q752Item2p16 ProbableCause ::= globalValue: {mtpSpecificExtensions 216}

q752Item2p18 ProbableCause ::= globalValue: {mtpSpecificExtensions 218}

q752Item3p06 ProbableCause ::= globalValue: {mtpSpecificExtensions 306}

q752Item3p11 ProbableCause ::= globalValue: {mtpSpecificExtensions 311}

q752Item4p03 ProbableCause ::= globalValue: {mtpSpecificExtensions 403}

q752Item4p05 ProbableCause ::= globalValue: {mtpSpecificExtensions 405}

q752Item4p11 ProbableCause ::= globalValue: {mtpSpecificExtensions 411}

q752Item4p13 ProbableCause ::= globalValue: {mtpSpecificExtensions 413}

q752Item5p01 ProbableCause ::= globalValue: {mtpSpecificExtensions 501}

q752Item5p05 ProbableCause ::= globalValue: {mtpSpecificExtensions 505}

q752Item5p06 ProbableCause ::= globalValue: {mtpSpecificExtensions 506}

q752Item5p07 ProbableCause ::= globalValue: {mtpSpecificExtensions 507}

q752Item5p08 ProbableCause ::= globalValue: {mtpSpecificExtensions 508}

remoteUserPartUnavailable ProbableCause:: = globalValue: {mtpSpecificExtensions 002}

abnormalFIBRorBSNR SpecificProblems ::= {{mtpSpecificExtensions 003}}

excessiveAckDelay SpecificProblems ::= {{mtpSpecificExtensions 004}}

excessiveCongDuration SpecificProblems ::= {{mtpSpecificExtensions 006}}

excessiveErrorRate SpecificProblems ::= {{mtpSpecificExtensions 005}}

remoteUPInaccessible SpecificProblems ::= {{mtpSpecificExtensions 009}}

remoteUPUnequipped SpecificProblems ::= {{mtpSpecificExtensions 008}}

remoteUPUnknown SpecificProblems ::= {{mtpSpecificExtensions 007}}

ReasonForRejection ::= ENUMERATED {

routeTestInstanceLocked	(10),

routeTestInstanceDisabled	(11),

routeTestInstanceBusy	(12),

routeTestAlreadyInProgress	(13)}

ReceiveCongestionThresholdAbatement ::= INTEGER

ReceiveCongestionThresholdOnset ::= INTEGER

RelatedLinkGroupNumber ::= INTEGER (0..15)

RemoteExchangeLabel ::= IA5String

-- the size of this string is implementation dependent

RemoteUpUnavailable ::= SEQUENCE {

pointCode	[0]		PointCode,

userPart	[1]		UserPart }

ResultMtpRouteTest ::= SEQUENCE {

testedDestination	PointCode,

routeTestActionResult	MRVTActionResult,

announcementOfMRVR	[0]	BOOLEAN OPTIONAL,

failureType	[1]	MRVTFailureString OPTIONAL,

routeTrace	[2]	RouteTrace OPTIONAL,

pointCodeInfo	[3]	PointCode OPTIONAL,

pointCodeListInfo	[4]	PointCodeList OPTIONAL,

routePriorityListInfo	[5]	RoutePriorityListInfo OPTIONAL,

copyData	[6]	OCTET STRING OPTIONAL,

...}

ResultOfAction ::= ENUMERATED {

success	(0),

unsuccessful	(1),

notPerformed	(2),

linkAlreadyInhibited	(3),

linkNotInhibited	(4),

wrongClassReferenced	(5),

signTermNonExisting	(6),

signTermAlreadyReferenced	(7),

wrongSignLinkTpStatus	(8),

...}

ResultOfMRVT ::= ENUMERATED {

success 	(0),

partialSuccess	(1), -- 2.1.1.3.2/Q.754, Partial  Success

failure 	(2), -- 2.1.1.3.1/Q.754, Failure

}

ReturnUnknownParams ::= BIT STRING {

tag15	(0),

tag16	(1),

...}

RoutePriorityListInfo ::= SEQUENCE OF PriorityInfo

RouteSetUnavailable ::= OBJECT IDENTIFIER

RouteTrace ::= CHOICE {

success	[0] IMPLICIT PointCodeList,

detectedLoop	[1] IMPLICIT PointCodeList,

excessiveLengthRoute	[2] IMPLICIT PointCodeList,

unknownDestination	[3] IMPLICIT NULL,

routeInaccessible	[4] IMPLICIT PointCodeList,

processingFailure	[5] IMPLICIT NULL,

unknownInitiatingSP	[6] IMPLICIT PointCode,

timerExpired	[7] IMPLICIT PointCodeList,

spNotAnSTP	[8] IMPLICIT PointCodeList,

indirectRoute	[9] IMPLICIT PointCode}

ScreenedOpcsOrLinkSetsList ::= SET SIZE (0..maxNoOfScreenedOpcs) OF

SEQUENCE {

screenedOpcOrLinkset	StpScreeningTableLineId,

messageTreatment	TreatmentOf }

ScreeningByLinkSetOrByOpc ::= ENUMERATED {

byOpc	(0),

byIncomingLinkset	(1),

byOutgoingLinkset	(2)}

Seconds ::= INTEGER

SIOType ::= SEQUENCE {

service ENUMERATED {

signallingNetworkManagement	(0),

signallingNetworkTesting	(1),

sccp	(3),

tup	(4),

isup	(5),

dup	(6),

mt	(8),

b-isup	(9),

siup	(10),

...} ,

subService ENUMERATED {

internationalNetwork	(0),

nationalNetwork	(2),

...}

}

SignRouteSetNePartName ::= IA5String

-- the size of this string is implementation dependent

SlCode ::= INTEGER (0..15)

Sls ::= INTEGER (0..15)

SLSCodeCurrentList ::= SlsCodeList

SlsCodeList ::= SET SIZE (0..16) OF Sls

-- Each Sls value can occur at most once in a given SET

SlsCodeNormalList ::= SlsCodeList

SlsLinkPriorityList ::= SEQUENCE {

slsCode	[0]	Sls,

normalSlcCode	[1]	SlCode,

alternativeSlCodeList	[2]	AlternativeSlCodeList

}

StmChannel ::= INTEGER (1..31)

SpecificDpc ::= PointCode

SpType ::= ENUMERATED

{sep	(0),

step	(1),

stp	(2)}

SS7OnOccEventInfo ::= SEQUENCE{

probableCause	ProbableCause,

specificProblems	[1]	SpecificProblems OPTIONAL,

perceivedSeverity	PerceivedSeverity,

notificationIdentifier	[5]	NotificationIdentifier OPTIONAL,

correlatedNotifications	[6]	CorrelatedNotifications OPTIONAL,

additionalText	AdditionalText OPTIONAL,

additionalInformation	[10]	AdditionalInformation OPTIONAL }

StartMtpRouteTest ::= SEQUENCE {

testDestination	PointCode,

traceRequested	BOOLEAN,

threshold	ThresholdN,

infoRequest	[0]	InfoRequest OPTIONAL,

returnUnknownParams	[1]	ReturnUnknownParams OPTIONAL,

directRouteCheck	[2]	BOOLEAN OPTIONAL}

StpScreeningElement ::= SEQUENCE{

designatedDPC	DpcComponent,

messageTreatment	TreatmentOf }

StpScreeningTableLineContent ::= SET SIZE (1..maxNoOfScreenedDpcs) OF StpScreeningElement

StpScreeningTableLineId ::= CHOICE {

designatedlinkset	[0]	Objectinstance,

designateddopc	[1]	PointCode}

StpScreeningTableListMode ::= BOOLEAN

SuermNSU ::= INTEGER (0..256)

-- lower range is implementation dependent

SuermThresh ::= INTEGER (32 | 64) -- (minSuermThresh or maxSuermThres)

TestResult ::= BIT STRING

{detectedLoop	(0),

excessiveLengthRoute	(1),

unknownDestination	(2),

routeInaccessible	(3),

processingFailure	(4),

unknownInitiatingSP	(5),

timerExpired	(6),

spNotAnSTP	(7),

indirectRoute	(8),

maxNrMRVTestsAlready	(9),

...}

ThresholdN ::= INTEGER

TimeD ::= INTEGER

TimerValue ::= INTEGER (0..60000)

TransmissionCongestionThresholdAbatementLevel1 ::= INTEGER

TransmissionCongestionThresholdAbatementLevel2 ::= INTEGER

TransmissionCongestionThresholdAbatementLevel3 ::= INTEGER

TransmissionCongestionThresholdDiscardLevel1 ::= INTEGER

TransmissionCongestionThresholdDiscardLevel2 ::= INTEGER

TransmissionCongestionThresholdDiscardLevel3 ::= INTEGER

TransmissionCongestionThresholdOnsetLevel1 ::= INTEGER

TransmissionCongestionThresholdOnsetLevel2 ::= INTEGER

TransmissionCongestionThresholdOnsetLevel3 ::= INTEGER

TransmissionRate ::= ENUMERATED {

kbits4point8	(0),

kbits56	(1),

kbits64	(2)}

TreatmentOf ::= ENUMERATED {

normal	(0),

discard	(1) }

UserPart ::= ENUMERATED

{ isup	(5),

sccp	 	(3),

tup		(4),

dup		(6),

mt		(8),

b-isup	(9),

siup		(10),
	...}

END -- end of MTPDefinedTypesModule

Annex A

Overview of OMT notationXE "Overview of OMT notation"§

(This annex forms an integral part of this Recommendation)

Recommendation Q.751.1     (10/95)

FIGURE ...[D04] = 14.09 CM

Annex B

Informal entity-relationship diagrams

(This annex forms an integral part of this Recommendation)

This subclause contains an informal description of the management view of the SS No. 7 resources, which was refined into the formal description of managed objects in the main body of this Recommendation. The entity-relationship diagrams can be read as follows:

FIGURE ...[D05] = 15.02 CM

To read this relation, start with, say, the item "Signalling Point" [which also exists in entity-relation (E-R) diagrams other than this], and read through the relation R14 to "Can Be 1" Signalling End Point. Conversely, a Signalling End Point is just one Signalling Point (the "Is A 1" relation).

For two-way "exclusive or" (xÈ) relations, entry one way is on a line marked without an attached È, and exit is on just one line with an attached È.

Entry the other way is on just one of the lines with an attached È, and exit is on the line without an attached È.

For one way xÈ relations, follow the arrows and apply the appropriate rule.

For two-way "inclusive or" (È) relations, entry one way is on the line without an attached È, and exit is on one or more lines with an attached È. Entry the other way is on one or more lines with attached È s, and exit on the line without an attached È.

For one way È relations, follow the arrows and apply the appropriate rule.

For two-way "and" (Ç) relations, entry one way is from all of the lines with &s attached, and exit is on the line without an &.

Entry the other way is on the line without an &, and exit is on all the lines with attached &s.

For one-way Ç relations, follow the arrows and apply the appropriate rule.

#### The definition of relations

The general properties of the relations between SS No. 7 managed items are described below.

#### Mathematical definition

If X and Y are sets, a relation R from X to Y is a subset R Í X ´ Y with (x,y) Î R ' x ÎX, y ÎY if x and y are R-related.

The Domain of R is defined by: Domain (R) = { x ÎX | (x,y) Î R for some y Î Y } and the Range of R is defined by: Range (R) = { y Î Y | (x,y) Î R for some x Î X }.

#### Relations of the E-R diagrams

The relations on E-R diagrams adopt the ideas from the definition of mathematical relations given above, but usually they are not mathematical relations themselves. In particular, a two-way relation, read in one direction, is usually not the inverse of that read in the other direction (even though the name might be the same!). In addition, ordering is sometimes imposed, and additional constraints might exist, on sets from the E-R diagram.

#### Key to logical symbols used in this annex

{ }		the contents denote a set.

Î		element : x Î Y means x is an element of set Y.

| or '	means "such that". So { x Î X | (x,y) Î R for some y ÎY } means the set of elements x of X such that (x,y) is an element of set R for some element y of set Y.

Ì		logical inclusion. So A Ì B means that set A is included in set B.

Í		improper logical inclusion symbol. C Í D means C equals D or is included in D.

Ç		logical and.

È		logical or.

Æ		the empty symbol. So A º Æ means that A is an empty set.

## B.1	Entity-relationship diagram for MTPXE "Entity-relationship diagram for MTP"§

All relations which are described within the protocol of MTP are illustrated in the entity-relationship diagram below. This diagram is used to derive the management aspects for MTP. It is used as a "shopping list" for the managed object definitions.

It should be noted that Figures B.1 through B.3 show an abstraction of this ERD to the MTP managed objects for naming and containment. There, a different notation is used, which is more appropriate than this one for managed object definitions. The ERD notation used here is suited to problem domain descriptions.

FIGURE B.1/Q.751.1...[D06] = 19.4 CM

#### Off-Page Relations

| Identity | From item | Function | To item |
| --- | --- | --- | --- |
| R3 | Signalling Network Component | Can Be 1 | Signalling Link |
| R7 | Signalling Point | Is The Endpoint Of N | Signalling Link |
| R21 | Signalling Point | Is The Endpoint Of N ³ 0 | Signalling Link Group |
| R6 | Signalling Point | Is The Endpoint Of N | Signalling LinkSet |
| R8 | Signalling Point | Is Traversed By N | Signalling Route |
| R10 | Signalling Point | Is The Endpoint Of N | Signalling Route |
| R22 | Signalling Point | Is Traversed By N | Signalling RouteSet |
| R23 | Signalling Point | Is The Endpoint Of N | Signalling RouteSet |
| R24 | Originating & Destination Points | May Be Associated By N | Signalling Relation |
| R26 | Signalling Point | Is An End Or Intermediate Point Of N | Signalling Route |
| R35 | MTP User | Utilises N | Signalling Relation |
| NOTES 1 Relation R24 is an "and" relation. 2 R15 is not used. 3 R14 is an "inclusive or" relation. |  |  |  |

FIGURE  B.1/Q.751.1 (end)

SS No. 7 MTP managed items entity-relation diagramXE "SS No. 7 MTP managed items entity-relation diagram"§

FIGURE B.2/Q.751.1...[D07] = 17.63 CM

#### Off-Page Relations

| Identity | From item | Function | To item |
| --- | --- | --- | --- |
| R3 | Signalling Link | Is A 1 | Signalling Network Component |
| R7 | Signalling Link | Directly Connects 2 | Signalling Point |
| R21 | Signalling Link Group | Directly Connects 2 | Signalling Point |
| R6 | Signalling LinkSet | Directly Connects 2 | Signalling Point |
| R8 | Signalling Route | Traverses N | Signalling Point |
| R10 | Signalling Route | Indirectly Connects 2 | Signalling Point |
| R22 | Signalling RouteSet | Has M Sets Of N Traversed | Signalling Point |
| R23 | Signalling RouteSet | Indirectly Connects 2 | Signalling Point |
| R24 | Signalling Relation | Relates 1 (twice) | Originating & Destination Points |
| R26 | Signalling Route | Contains An Ordered Sequence Of N | Signalling Point |
| R29 | Signalling Link | Connects 2 & Uses 1 | Signalling Terminal & Signalling Data Link |
| R35 | Signalling Relation | Is Utilized By 1 | MTP User |
| NOTE – Relations R24 and R29 are "and" relations. |  |  |  |

FIGURE  B.2/Q.751.1 (end)

SS No. 7 MTP managed items entity-relation diagram part 2

FIGURE B.3/Q.751.1...[D08] = 9.06 CM

#### Off-Page Relations

| Identity | From item | Function | To item |
| --- | --- | --- | --- |
| R29 | 2 Signalling Terminal & 1 Signalling Data Link | Are Associated With 1 | Signalling Link |

FIGURE  B.3/Q.751.1 (end)

Informal MTP ERD part 3

Relation R29 associates two signalling terminals and one signalling data link with one signalling link. One signalling terminal must be in the SP (SP A) at one end of the signalling link, the other at the other end of the signalling link (SP B).

The rules for selecting the two signalling terminals and one signalling data link are summarised in the following table:

|  | Fixed ST at SP A | Auto allocation of ST at SP A | Fixed ST at SP B | Auto allocation of ST at SP B |
| --- | --- | --- | --- | --- |
| Auto allocation of SdL | No | Yes | No | Yes |
| Fixed allocation of SdL | Yes Yes No No | No No Yes Yes | Yes No Yes No | No Yes No Yes |

### B.1.2	MTP management hierarchies and managed item line diagramsXE "MTP management hierarchies and managed item line diagrams"§

#### B.1.2.1	Management hierarchiesXE "Management hierarchies"§

The most significant hierarchy is the tree from Signalling Relation to Signalling terminal and Signalling Data Link via Signalling Link. If an element of the tree is subjected to management action, consideration is required of the effects upon the preceding and following elements (which might affect their preceding and following elements respectively). This aspect is considered in detail for each tree element.

The line diagrams give an informal view of the network management view of the managed items.

The network management view is considered throughout, with the network element (SP) view given also in some cases. Usually the network element view is just one half of the network management view. This former view should accommodate all known implementations, with those properties that are not common to all implementations defined as "optional" in some way.

In what follows, relations and attributes are shown which might (depending upon network or vendor implementation) be manageable to a greater or lesser extent. For example, some attributes might be human-machine up dateable in some implementations, read-only in some, and not accessible to the management interface in others.

A minimum set of items with their relations, attributes and behaviours needs to be defined, and then supersets of those items should be defined to cover the known managed items and their properties in existing MTP networks.

In the following, it is assumed that the maintenance, operations and administration personnel would take action at the appropriate hierarchical level for the item being managed. Thus, for example, if a linkset were to be moved "out of service" the command would be issued to the linkset. This command would be permitted, unless a routeset would thereby become unavailable (if it were desired to remove the routeset from service, a command would have been issued to the routeset). The action of removing the linkset from service might be realised by automatically issuing a command to remove each of its constituent links from service, in this case the check on the last available link would allow it to be removed.

#### B.1.2.2	Line diagram conventions

The relation names are the same as on the informal MTP managed items E-R diagram (where the relations exist), the conventions adopted are:

a)	the item name is on the left hand side of the drawing, with the Recommendation defining the item listed underneath. The identity of the item is listed under the item name;

b)	the associated relations are listed on the right hand side, a solid leftwards pointing arrow indicates a relation with the referred-to item above this item in the MTP routing rules or management tree;

c)	a solid rightwards pointing arrow indicates a relation with the referred-to item below this item in the MTP routing rules or management tree;

d)	no arrow on a solid line indicates an attribute with no relation currently defined (although it is possible that some of the measurement-type attributes might have objects defined at some stage);

e)	optional relations are represented by dashed arrows;

f)	optional attributes are represented by dashed lines;

g)	dotted lines indicate mathematical or logical relations;

h)	a dotted line with inward pointing arrow indicates an input parameter to a mathematical or logical relation;

i)	an outward pointing dotted arrow indicates an output parameter (or result) of a logical or mathematical relation;

j)	relations are defined in bold type and numbered according to the MTP managed items E-R diagram.

#### B.1.2.3	Signalling End point (SEP)

See Figure B.4.

##### B.1.2.3.1	Behaviour

A Signalling end point (SEP) is the node at which the MTP Users, and end users, can access the SS No. 7 network. It acts simultaneously as an originating point and a destination point for the signalling interactions of a number of users of the SS No. 7 network [and these users have Service Indicators (SIs) different from 0000 and 0001, although the SEP also contains MTP Signalling Network Management and Testing and Maintenance, because it is a signalling point].

Each user possesses a number of sources and sinks of message traffic (e.g. ISUP generates a number of ISUP calls per trunk group per second, each call requires the exchange of several messages).

A signalling end point may be created and destroyed. Before it is destroyed, all reference to it in the network’s MTP routing tables (and in the respective MTP Users’ data) must have been removed.

##### B.1.2.3.2	Notifications

These are the ones defined for a signalling point, viz.:

–	signalling point unavailable; and

–	signalling point available.

When a signalling end point becomes available (i.e. its first link becomes available at MTP level 2), after a time of unavailability of at least the Q.704 T1 value, it is deemed to be restarting as an SEP.

FIGURE B.4/Q.751.1...[D09] = 11.07 CM
XE "Signalling End Point line diagram"§

#### B.1.2.4	Signalling Transfer Point (STP)

See Figure B.5.

FIGURE B.5/Q.751.1...[D10] = 6.52 CM
XE "Signalling transfer point line diagram"§

##### B.1.2.4.1	Behaviour

A signalling Transfer Point (STP) acts as a transit switch for SS No. 7 messages.

There are two main reasons for using it:

i)	to provide alternative routes for messages, and hence increase the robustness of the SS No. 7 network; and

ii)	to reduce the number of signalling links over that required for a full mesh network, and hence reduce network complexity.

An STP, by virtue of being a signalling point, possesses at least two MTP users, viz. MTP Signalling Network Management, and Testing and Maintenance (SI = 0000 and 0001 respectively). See 4-13/Q.704 and Recommendation Q.707. It may also contain network element management OMAP (along with a vestigial SCCP and TC to support OMAP).

An STP may be created and destroyed. Before it is destroyed, all reference to it in the network’s MTP routing tables must have been removed (i.e. all routesets using the STP must have been deleted).

##### B.1.2.4.2	Notifications

These are the ones defined for a signalling point, viz.:

–	signalling point unavailable;

–	signalling point available.

When an STP becomes available (i.e. its first link becomes available at MTP level 2), after a period of at least the Q.704 timer T1 during which it was unavailable, it is deemed to be restarting as an STP.

#### B.1.2.5	Signalling Transfer and End Point (STEP)

This managed item combines the functions of an SEP and an STP. As such, it has the attributes of both. See Figure B.6.

FIGURE B.6/Q.751.1...[D11] = 12.23 CM
XE "STEP line diagram"§

##### B.1.2.5.1	Behaviour

This is as an SEP plus STP, except that on MTP restart the STEP behaves as an STP.

#### B.1.2.6	Signalling Relation

This item is defined as a supporting item for MTP User (Parts) and will not be a managed object in its own right. See Figure B.7.

FIGURE B.7/Q.751.1...[D12] = 12.14 CM
XE "Signalling relation line diagram"§

##### B.1.2.6.1	Behaviour

A signalling relation needs to exist when some service requires communication between origin and destination Signalling Points (SPs) in the SS No. 7 network. Usually a signalling relation is established in each direction. The service will be associated with an MTP User (e.g. SCCP or ISUP, or even the MTP itself), and the signalling relation is identified by the MTP User and the origin, destination pair.

The signalling relation has as an attribute the set of SLS codes that it supports for the MTP User. The MTP User may add or delete SLS code values from this set. Such a change might require the routes comprising the routeset supporting the signalling relation to vary the SLS codes they support.

Signalling relations used by the MTP signalling network management, and MTP testing and maintenance, have as an attribute the set of Signalling Link Codes (SLCs) for the links they use, support and test.

A signalling relation may be added (created), and destroyed (deleted).

The signalling relation is created when the services using it are established by the network administration, and persists and is used for all communication by the MTP User between origin and destination, until the service is discontinued by the administration.

The origin and destination SPs, and the MTP User at both the origin and destination, must exist before the signalling relation can be created.

All signalling relations between origin and destination must be removed before either origin or destination can be destroyed.

A signalling relation becomes unavailable if the MTP User at the destination becomes inaccessible to the origin (note that the signalling relation is one way, but there will usually be another defined for the other direction).

If the MTP User at the origin attempts to communicate with its peer at the destination in the signalling relation, and the remote User is inaccessible, the remote MTP may send a "User Part Unavailable" message to the MTP at the origin SP. Such a UPU message contains a reason, if the sending MTP is post-1992 Recommendation, which indicates that the MTP User is either unequipped (and so the signalling relation is not valid), inaccessible, or its condition is unknown. ("condition unknown" is also that given by pre-1992 but post-1988 Recommendation MTPs).

Before the signalling relation can be used, the routeset upon which it relies must be created and brought into service.

##### B.1.2.6.2	Notifications

User part Unavailable:

–	inaccessible remote User; or

–	unequipped remote User; or

–	unknown.

#### B.1.2.7	RoutesetXE "Routeset"§

See Figure B.8.

FIGURE B.8/Q.751.1...[D13] = 14.21 CM
XE "Signalling routeset line diagram"§

##### B.1.2.7.1	Behaviour

Recommendation Q.704 allows for different MTP Users, or the MTP itself, to use different routings between an origin SP and a destination SP. The collection of all signalling routes for all MTP Users forms the signalling routeset.

It is helpful to qualify the notion of "routeset" to ease the description of route properties. Accordingly, the concepts of complete signalling routeset, normal signalling routeset and current signalling routeset are defined.

1)	The network element view, at the instant of creation or modification of the routing data, of the complete signalling routesetXE "complete signalling routeset"§ O(A,B) is defined as the set of all distinct signalling routes intended to convey MSUs sent from or through the near end SP A to the destination SP B.

A member of this set is called a signalling route.

In the above the network element view of a route is used.

The network management view uses the same wording in the definition, but it uses the network management view of a route, with A just the origin SP.

2)	The network element view, at the instant of creation or modification of the routing data, of the normal signalling routesetXE "normal signalling routeset"§ N(A,B) is defined as the set of all distinct signalling routes that would convey MSUs sent from or through the near end SP A to destination SP B, in the absence of any failure in the complete routeset O(A,B).

The network management view uses the same wording, but with A just the origin SP.

A member of the set is called a normal signalling route.

3)	The network element view of the current signalling routesetXE "current signalling routeset"§ C(A,B) is defined as the set of all distinct routes that convey MSUs sent from or through the near end SP A to the destination SP B, at the moment of observation.

The network management view uses the same wording, but with A just the origin SP.

A member of the set is called a current signalling route.

Note that different observations might show different current signalling routesets.

###### B.1.2.7.1.1	Relationship between definitions

If we write:

O:	complete routeset O(A,B).

N:	normal routeset N(A,B).

C:	current routeset C(A,B).

Then we have:

C Í O, N Í O

###### B.1.2.7.1.2	Operations

A signalling routeset can be created (added), destroyed (deleted), and have signalling relations added to it or deleted from it.

The routeset is created when the first signalling relation between the origin signalling point and the destination is created, and it is destroyed when the last signalling relation between the origin and destination is destroyed.

Each of the signalling points on every route of the routeset must have a signalling relation between it and the destination defined and operational before any signalling relation from origin to destination can become operational.

A signalling routeset can be congested or uncongested. A network option allows up to three levels of congestion. The "congested" marking occurs when a message for the destination served by the routeset is first routed using a signalling link marked as congested. See 3.8.4./Q.704.

##### B.1.2.7.2	Choice of route from the routesetXE "Choice of route from the routeset"§

The Q.704 network element view of route choice is as follows:

Clause 4/Q.704 states that one or more alternative linksets or combined linksets are allocated for each destination which may be reached from a signalling point.

A combined linkset is a load sharing collection of two or more linksets.

The possible linksets (or combined linksets) are arranged in a certain priority order4). To route a message to a destination, the highest priority available linkset (or combined linkset) is chosen.

If a combined linkset is to be used, the linkset from the combined linkset is selected according to an (SP or network) implementation dependent loadsharing algorithm. Such algorithms usually take as an input the SLS code of the message to be transported – at any time each linkset of the combined linkset supports a disjoint set of SLS codes for the destination, the whole combined linkset supporting all values in (0..15). See, for example, Recommendation Q.705.

#### B.1.2.8	Signalling RouteXE "Signalling Route"§

The line diagram for this is as follows (see Figure B.9):

##### B.1.2.8.1	Behaviour

A signalling route is used to convey MTP User messages from origin SP to destination SP (and the MTP itself is an MTP User).

For OMAP purposes the network element view and network management view of a route are defined as follows:

#### Network element view

A signalling route (A,C,B)i is defined as an ordered sequence of the near end signalling point A (i.e. the network element), an adjacent SP C and the destination SP B. The adjacent SP C may be identical with the destination B, or it may or may not be directly interconnected to the destination SP.

The adjacent SP C must be directly connected to the near end SP by at least one linkset which is intended to convey MSUs sent from A to B (hence the use of the term "adjacent").

Two signalling routes (A,C,B)i and (A,C¢,B)j are distinct if the adjacent SP is different.

#### Network management view:

A signalling route (A,B)i is defined as an ordered sequence of the origin SP A, zero or several intermediate SPs (i.e. STPs), and the destination SP B. Each intermediate STP is directly interconnected to its neighbour SPs in the ordered sequence by at least one linkset which is intended to convey MSUs sent from A to B.

The second SP in the sequence is the same as that in the network element view of the route (A,B)i. If this second SP "C" is not the destination, then there is a signalling route (C,B) which is intended to convey messages sent from A to B.

FIGURE B.9/Q.751.1...[D14] = 20.03 CM
XE "Signalling route line diagram"§

Two signalling routes are distinct if one or more SPs are different in the corresponding two ordered sequences.

There is a network implementation dependent value N for the maximum number of signalling points in any route (see also clause 2/Q.753).

Signalling routes can be created and destroyed, activated and deactivated.

A signalling route is created after the signalling routeset to which the route will belong is created, or if the routeset requires expansion for reliability or capacity reasons.

No signalling point may appear more than once in any valid signalling route.

A route becomes deactivated if any of its constituent signalling route components (SPs or connecting linksets) is deactivated, and is active only when all of its signalling route components are active.

A route may only be deactivated if it is not the last remaining activated route in the routeset, or it is permitted to deactivate the routeset.

A route is defined as active when the required network resources are present, when all necessary routing data is present in the signalling points of the network, and the route is ready to carry message traffic.

##### B.1.2.8.2	Attributes

1)	CapacityXE "Capacity"§

Several capacities could be defined, such as maximum, or current. The current capacity is the message carrying capability of the signalling route component of the route which has the lowest available capacity to devote to this route. The capacity of a route needs to be defined to identify the maximum message traffic that the route is allowed to support.

2)	UtilizationXE "Utilization"§

This is the message traffic actually being carried on this route.

3)	OMAP stateXE "OMAP state"§

This is only defined if "Route" is defined as a network management managed object, and is then mapped from the functional state on to the OSI states appropriate to it.

4)	Functional stateXE "Functional state"§

This includes the activated or deactivated status, and is formed by the logical "and" of the functional states of all of the signalling route components in the sequence. The state includes an indication of route congestion (+ congestion level if appropriate), obtained from the maximum value of all of its components, with the component’s value obtained considering all of the routes using it.

The Q.704 statuses also apply, namely available, unavailable or restricted.

5)	EfficiencyXE "Efficiency"§

This has no definition in Recommendation Q.704, for Recommendation Q.751 we state that a route is more efficient than another to the same destination if it has fewer signalling transfer points in it, or the projected message delay is significantly lower.

###### B.1.2.8.2.1	Notifications

These are:

–	signalling route unavailableXE "signalling route unavailable"§;

–	signalling route availableXE "signalling route available"§;

–	signalling route restrictedXE "signalling route restricted"§ (national option).

##### B.1.2.8.3	Supporting definition of "signalling path"XE "Supporting definition of \"signalling path\""§

The definition of a signalling path is provided here, in order to describe the (non-ITU specified) load sharing applicable within and between linksets (in the network element view), and between routes (in the network management view). The concept is also useful in determining, in the MTP Route Verification Test, if a reported loop is a true or pseudo-loop.

#### Network element view

A signalling path (A,B)i is defined as an ordered sequence of the near end signalling point A (i.e. the network element), the link connecting A to an adjacent SP C which is designated to carry from A to B messages with the SLS code associated with (A,B)i, and the destination SP B. The adjacent SP C may be identical with the destination B, or it may or may not be directly interconnected to the destination SP.

Associated with the signalling path is a priority and the SLS code.

Two signalling paths (A,B)i and (A,B)j are distinct if they have different links, or the SLS code associated with one is different from that associated with the other. If two signalling paths (A,B)i and (A,B)j have different links but the same priority, then they must have different SLS codes.

Two signalling paths (A,B)i and (A,B)j cannot have the same link, the same associated SLS code and different priorities.

#### Network management view

A signalling path (A,B)i is defined as an ordered sequence of the origin SP A, a sequence of one or more combinations of signalling link and its far end signalling point, and the destination SP B. Each link is designated to convey MSUs [with the SLS code associated with (A,B)i in the network element view] sent from A to B.

The signalling point at the near end of the first link of the sequence is the origin A. The far end of the first link in the sequence is the adjacent SP C in the network element view of the path (A,B)i. If this SP "C" is not the destination, then there is a signalling path (C,B) with the SLS associated with (A,B)i.

Associated with a signalling path is an SLS code and a compound priority. The compound priority is a sequence of elements arranged in the same order as the signalling links of the signalling path. Each element of the compound priority is the priority associated with the network element view of the signalling path from the corresponding signalling link’s near end to the destination.

Two signalling paths (A,B)i and (A,B)j are distinct if they have different links, or the SLS code associated with one is different from that associated with the other. If two signalling paths (A,B)i and (A,B)j have different links but the same compound priority, then they are associated with different SLS codes.

Two signalling paths (A,B)i and (A,B)j cannot have the same links, the same associated SLS code and different compound priorities.

A valid signalling path is defined as a path which contains any particular link at most once in the ordered sequence, with the sequence having no more than a (network implementation dependent) defined number M of links. In addition, each intermediate signalling point in the sequence must have the MTP transfer function.

We define the "line" of a signalling path as its contained sequence of signalling points.

##### B.1.2.8.4	Supporting definition of "signalling path group"XE "Supporting definition of \"signalling path group\""§

A signalling path group (in the network management view) is the complete collection of signalling paths with the same sequence of signalling points and the same associated compound priority.

Associated with the signalling path group is a disjoint set of SLS codes, one for each signalling path of the group.

##### B.1.2.8.5	Constraints upon signalling pathsXE "Constraints upon signalling paths"§

The set of signalling paths with a particular associated priority from a near end A to a destination B is not complete unless it has a complete set of SLS codes associated with it (i.e. all values from decimal 0 to decimal 15).

##### B.1.2.8.6	Relation between message routing, signalling paths and signalling routeXE "Relation between message routing, signalling paths and signalling route"§

If a message is to be transported between its origin and destination, there must be a signalling path for it. The message will be carried on the highest priority available path for the SLS code of the message.

The current load of a route is the sum of the loads of its constituent current signalling paths.

##### B.1.2.8.7	Relation between signalling path priority and linkset or combined linkset priorityXE "Relation between signalling path priority and linkset or combined linkset priority"§

Let PL(r,l) be the (network element view of the) priority of linkset l in the routeset r as defined in 4.2.1/Q.704 paragraph 3. We define PL(r,s) = ¥ if linkset s is not used in the routeset r. This implies that the "highest" linkset priority has the lowest numerical value. Let n(l) be the number of links in linkset l.

Then the network element view of the path priority of a path of routeset r, using a link of linkset l is:

where D is calculated as follows:

i)	if linkset l does not form part of a combined linkset for routeset r, at priority PL(r,l), D is the relative priority of the link within linkset l for the traffic stream served by the path (as in 4.2.1/Q.704 paragraph 4 first sentence). (This uses the assumption that a linkset is available if any link within it is available); or

ii)	if linkset l does form part of a combined linkset for routeset r, at priority PL(r,l), D is the relative priority of the link within the combined linkset for the traffic stream served by the path. It might be that all other links of the combined linkset are alternatives for this link, at linkset priority PL(r,l), or (as in 4.2.1/Q.704, paragraph 4 first sentence) all other links of the linkset l. This latter case means that, if the links in the other linksets of the combined linkset are to be used as alternatives for "our" link, then it will have to be at (a) different linkset priority (or priorities).

See B.1.2.10.2 for examples of this relation.

#### B.1.2.9	Signalling LinksetXE "Signalling Linkset"§

See Figure B.10.

##### B.1.2.9.1	Behaviour

A Signalling Linkset is a collection of Signalling Links used for connecting adjacent Signalling Points.

A Signalling Linkset may be part of many Routes. It can be part of a number of Combined Linksets, it may contain a number of Signalling Link Groups (a link group is a set of links with the same characteristics, e.g. transmission speed).

A Signalling Linkset is constructed from between 1 and 16 Signalling Links, all of which have the same two end points (Signalling Points) as the Signalling Linkset.

Note that a linkset is a unidirectional concept, but because signalling links are bi-directional, and must have the same identity (signalling link code, SLC) at each end, the complete set of signalling links in the signalling linksets defined at signalling point "A", whose far end is signalling point "B", is the same as the set of signalling links in the signalling linksets defined at "B" whose far end is "A". See B.1.2.10.2 for examples.

A network or vendor defined manageable message load sharing algorithm might be defined for each SP of the linkset.

A Signalling Linkset may be created, destroyed, activated (with normal and emergency restart), and deactivated.

FIGURE B.10/Q.751.1...[D15] = 22.01 CM
XE "Signalling Linkset line diagram"§

###### Constraints:

a)	At least one route must exist (i.e. be identified) to require to use the linkset before it can be created;

b)	all links in the linkset must have been removed before the linkset can be destroyed;

c)	a linkset cannot be destroyed until it has been deactivated, and until the concerned routes have been identified as being destroyable;

d)	a linkset must exist before it can be activated;

e)	a linkset may only be deactivated if its deactivation is permitted for all routes utilizing this linkset as a signalling route component (if the linkset is deactivated, each route or partial route using the linkset would become deactivated. This could lead to some routeset becoming unavailable, and for this to occur the command must have been issued upon the routeset).

##### B.1.2.9.2	Attributes

1)	Load sharing algorithm

This is shown as an optionally manageable attribute. The load sharing algorithm within the linkset is defined in Recommendation Q.704 to rely upon the SLS codes of offered messages, but not their destination, and at one time a particular available link will carry all messages with a particular SLS code.

2)	Timers

Timers might be updateable within the Q.703, Q.704 and Q.707 limits.

i)	Level 2: if any of these are human-machine up dateable, there should be a check that the limits are those of ITU-T (formerly CCITT) Recommendation 12.3/Q.703. In addition, if the network limits the error correction method to be the same for all links of the linkset, and if a satellite is in the transmission path of the linkset, the "excessive delay of acknowledgement" timer T7 should be constrained to be not less than 0.8 seconds.

ii)	Level 3: if any of the timers are human-machine updateable5), then their values should be restricted to those given in ITU-T (formerly CCITT) Recommendation 16.8/Q.704. Certain timers might apply on a linkset, rather than a link, basis, e.g. T1-T6, T10, T12-T14, T17, T21, T23.

3)	Error correction method

It is desirable that this is the same for all links in the linkset. It might be possible to define the method by human-machine interface, or to read the mechanism.

4)	Congestion control method

This might be a network-wide scheme, rather than a per-linkset scheme or a per SP scheme. It is unlikely to be a per routeset scheme.

Congestion of a routeset is first detected when a message with DPC that of the routeset is to be placed into the transmission or retransmission buffer of the link, and a congestion threshold is thereby exceeded, or a congestion threshold of the link has already been passed but this message is the first to the DPC at this congestion level.

One of two main types of congestion control mechanism can be applied to a routeset:

1)	with message priorities; or

2)	without message priorities.

Type 1 uses multiple levels of congestion, and message discard within the MTP, if an offered message for a particular routeset has a priority lower than the current discard level. For every such message received, the origin (local or remote) is told that routeset congestion exists, and its level (which is the congestion level of the link). The MTP at a remote SP receiving such an indication remembers it, and starts a signalling routeset congestion test, employing timers T15 and T16. (See 11.2.4/Q.704, 13.7/Q.704 and 13.9/Q.704).

Type 2 has two subtypes:

i)	single congestion level; or

ii)	multiple congestion levels.

For both of these subtypes, the origin (local or remote) of a message is told when that message is the first to indicate (a particular level of) congestion. If multiple levels of congestion are used, the particular congestion level is also indicated (this is initially set to a value s for the link when congestion starts). Indications are returned for the initial and every 8th message or Nth octet (N provisionally is 279 to 300) subsequently received, to this message’s origin. The counting can be for the congested routeset, or per linkset of the congested routeset, or per link of the congested routeset, or per congested link of the congested routeset. Messages are not discarded by the MTP for this congestion control type, and the signalling routeset congestion test is not used.

For multiple congestion levels, 3.8/Q.704 suggests using a timer mechanism with timers Tx and Ty in an SP containing the congested link, to determine the signalling link congestion status. Here, there is just one congestion onset and one congestion abatement threshold. When the congestion onset threshold is passed, the link’s congestion status is set to s. If the buffer occupancy continues to be above the congestion onset threshold during Tx, then the signalling link’s congestion status is increased by 1. For each additional Tx when the buffer occupancy persists above the onset threshold, the congestion status is increased by 1. If the buffer occupancy falls below the congestion abatement threshold, and persists for a time Ty below the threshold, then the congestion status of the link is decreased by 1. For each additional Ty with the buffer occupancy persisting below the abatement threshold, the congestion status is decreased by 1 (until 0 is reached, with the link uncongested). A similar effect can be achieved to the timer mechanism by using multiple congestion onset and abatement thresholds (as in SPs with type 1 congestion control, but with appropriate values for thresholds).

For both subtypes, the congestion status of the routeset is set to that of the congested link, within the SP containing it.

The congestion control type should be indicated. This might be a read-only value (at best), and is set per SP.

For type 1, timer values might be read-writable, just read-only, or not even readable, per SP. Congestion onset and abatement thresholds, and discard thresholds, might be similarly manageable (but it is possible that these are manageable per SP, per link or per linkset).

For type 2, the subtype should be indicated, this might also be read-only per SP. Also the message counting or octet counting (with value of N) method should be indicated per SP (this might be a read-only indication, but if octet counting is used, N might be read-only, read-write, or neither, per SP).

The method of reporting (per congested routeset, per linkset of the congested routeset, per link of the congested routeset, or per congested link of the congested routeset) might be read-only per SP.

For subtype i), the congestion onset and abatement threshold might be read-only, or read-write, or neither, per SP, or per linkset, or per link.

For subtype ii), if there is one congestion onset and one abatement threshold, this might be read-only, read-write, or neither, again per SP, per linkset, or per link. The timer values Tx and Ty are likely to be manageable (if at all) only per SP. The value of s, if manageable, might be per SP, per link, or per linkset.

For subtype ii) with multiple congestion thresholds, these might be manageable (if at all) per SP, per linkset, or per link.

It is unlikely that the multiple levels of congestion with message priorities method would co-exist in the same network as the multiple levels of congestion without priorities method, but the "international" method (i.e. one level only of congestion, no message priorities) might co-exist with the multiple levels of congestion without priorities method, and then it would be desirable to select the scheme per linkset.

5)	Loop delay is an optional read-only attribute.

6)	Utilization

This is the message traffic carried by the linkset (as measured during the last accumulation period). It is the sum of the traffics carried by its constituent links.

7)	Performance

This is the set of linkset measurements from, for example, Table 4/Q.752.

8)	OMAP state

This is to be mapped from the Q.704 functional state into appropriate OSI states.

9)	Functional state

This is the state selected from those shown in Figure 36/Q.704.

10)	Maximum capacity

This can be expressed as the number of MSUs (and octets) transportable each way per second on the linkset, when all assigned activate-able links are working at their maximum capacity.

11)	Current capacity

This is the calculated maximum message traffic (MSUs and octets) that can be carried with the currently available links.

#### B.1.2.10	Combined linksetXE "Combined linkset"§

See Figure B.11.

FIGURE B.11/Q.751.1...[D16] = 6.61 CM
XE "Combined linkset line diagram"§

##### B.1.2.10.1	Behaviour

A combined linkset is a collection of two or more signalling linksets, whose near ends are the same signalling point.

Associated with the combined linkset’s network element view are one or more destinations, whose message traffic from or through the near end SP is load shared across the constituent signalling linksets, according to an (SP or network implementation dependent) algorithm per destination.

Thus a combined linkset is the network element view, at the near end SP, of the corresponding network management set of equal priority routing choices to the destination(s) served by the combined linkset.

A linkset may form part of several combined linksets.

A combined linkset may be created and destroyed (i.e. have linksets allocated to it or de-allocated from it).

##### B.1.2.10.2	Examples of linkset, parallel and combined linksetsXE "Examples of linkset, parallel and combined linksets"§

See Figure B.12.

FIGURE B.12/Q.751.1...[D17] = 9.43 CM

Here the linkset ls 1 defined from SP A to SP B contains three links, and is also defined as just one linkset from B to A.

A has two parallel linksets, ls 2 and ls 3, from A to C. C defines these as just one linkset ls 4 to A.

A has a combined linkset cls 1, consisting of the two two-link linksets ls 5 and ls 6, to reach D. By definition, at least one of the constituent linksets of a combined linkset must access the destination through an STP (two are shown here, X and Y).

#### Path priority and linkset priority exampleXE "Path priority and linkset priority example"§

Let the normal load sharing rule for destination D on combined linkset cls 1 for messages at A be:

–	messages with SLS codes in the range [0..7] are routed over linkset ls 5;

–	messages with SLS codes in the range [8..15] are routed over linkset ls 6.

Let cls 1 have a (combined) linkset priority of p in the routeset (A, D). Identify the links of ls 5 as 5-1 and 5-2, and those of ls 6 as 6-1 and 6-2.

Suppose the normal SLS code allocations at A for messages to D are 0,2,4,6 on link 5-1; 1,3,5,7 on link 5-2; 8,10,12,14 on link 6-1 and 9,11,13,15 on link 6-2.

Then, according to 4.2.1/Q.704, link 5-2 is an alternative to 5-1. Hence, at (combined) linkset priority p, the path priority relative to linkset ls 5 for link 5-1 to serve the SLS code 2 is 1, and for link 5-2 to serve it is 2.

The absolute path priorities are G + 1 and G + 2 respectively, where G is the sum of the number of all links in linksets used by A to D whose linkset priorities are p - 1 or better.

According to Recommendation Q.704, ls 6 might be an alternative linkset to ls 5, but the linkset priority would be p + 1 at least. Then, if link 6-1 were to be used as an alternative to link 5-1, after link 5-2, the absolute path priority for SLS code 2 on link 6-1 is (G + 2) + 1.

However, if all the other links in the combined linkset are alternatives for a given link, at the same (combined) linkset priority p, then given a cyclic redistribution of SLS codes from link 5-1 on failure to all other links in the combined linkset, the first alternative link for SLS code 2 would be link 6-1, and so the absolute path priority for SLS 2 on link 6-1 is G + 2.

#### B.1.2.11	Signalling linkXE "Signalling link"§

See Figure B.13.

##### B.1.2.11.1	Behaviour

A signalling link is a constituent of the linkset directly connecting a signalling point with an adjacent SP. It is the combination of the signalling data link and signalling terminals.

The status of signalling links is defined in ITU-T (formerly CCITT) Recommendation 3.2/Q.704, and the link management procedures are defined in clause 12/Q.704.

A signalling link can be a member of only one signalling linkset, and its ends must be in the same signalling points as the linkset’s.

A signalling link may be created and destroyed, activated and deactivated, inhibited or uninhibited. It might be tested by, for example, the signalling link test defined in Recommendation Q.707 (when inhibited, and possibly when not, the test could be run manually).

Constraints:

a)	a link can only be created if the linkset to which it will belong exists;

b)	a link must be deactivated before it can be destroyed;

c)	a link cannot be deactivated if by doing so its linkset would be deactivated, or it is permitted to deactivate the linkset;

d)	the constraints upon inhibiting the link are defined in clause 10/Q.704. If any destination were to become inaccessible upon inhibiting the link, the MTP refuses the inhibition request from OMAP. Note that a link may be automatically uninhibited if a destination has become inaccessible and the use of the inhibited link would remedy this.

FIGURE B.13/Q.751.1...[D18] = 24.46 CM
XE "Signalling link line diagram"§

##### B.1.2.11.2	Attributes

1)	The Signalling Link Code (SLC) is the same value (between 0 and 15) at each end of the link, and is different from that of any other link between the same two adjacent signalling points.

2)	The signalling link may be digital 64 kbit/s (the standard digital rate), or analogue with a minimum equivalent bit rate for telephony applications of 4.8 kbit/s (see Recommendation Q.702). It also has a type, which is terrestrial, satellite or unknown.

3)	The error correction method (basic or preventive cyclic retransmission) is an attribute of the link. It might be allocable to the link by human-machine interaction, possibly on a linkset basis. It might be read-only, or not readable. If the method is PCR, the values of N1 and N2 must be allocated at each end, possibly by human-machine interaction.

4)	Transmission (and possibly retransmission) buffer thresholds exist for the determination of onset of congestion. Multiple thresholds might be defined, to allow different levels of response. A timer, or thresholds, might be used to determine the abatement of congestion. If the method of multiple congestion levels with message priorities is used, then message discard thresholds are also defined. All these thresholds might be human-machine up dateable. Any quantitative limits (and checks that the abatement threshold is sufficiently lower then the corresponding onset threshold to avoid oscillation) would need to be enforced.

5)	SLS codes might be readable, or adjustable by human-machine interaction (but this needs coordination at linkset level).

6)	MTP level 2 and level 3 timers might be adjustable, within the limits given by Recommendations Q.703 and Q.704, by human-machine interaction. If periodic signalling link tests (see Recommendation Q.707) are performed, the period might also be adjustable.

7)	The loop delay’s value and tolerance might be readable.

8)	Capacities of the signalling link might be affected by, for example, automatic allocation of signalling terminals or signalling data links. If different types of terminal are possible at one end of the link, the current and maximum capacities might be different. It might be beneficial to read these.

9)	Utilization might be obtained from, for example, Table 3/Q.752 measurements.

10)	Performance figures might be obtained from measurements in Tables 1, 2 and 3 of Recommendation Q.752.

11)	OMAP state: this would be mapped from the functional state, if it is desired to model the SS No. 7 signalling link states in OSI management terms.

12)	Functional state: this is defined by Recommendations Q.704 and Q.703.

13)	Network implementation specific attributes might exist (e.g. Receive buffer congestion onset and abatement thresholds might need to be defined for some implementations; some mechanism is necessary. The method’s parameters might be adjustable by human-machine interaction).

##### B.1.2.11.3	Relationship between network element and network management viewsXE "Relationship between network element and network management views"§

The network element (i.e. SP) view can be constructed from the network management view by considering one end of the signalling link.

#### B.1.2.12	Signalling data linkXE "Signalling data link"§

A signalling data link is the MTP level 2 connection serving a signalling link.

If automatic allocation of signalling data links (and by implication automatic allocation of signalling terminals) is employed between the two connected signalling points, then any pre-designated communication path may be used for the signalling data link, and signalling within the MTP occurs when the signalling data link is to be allocated. Otherwise, the signalling data link is allocated semi-permanently to the signalling link.

#### B.1.2.13	Signalling terminalXE "Signalling terminal"§

A signalling terminal is the termination of a signalling link at an SP. Each end of the signalling link employs a signalling terminal. The signalling terminal can be (semi-) permanently allocated to the signalling link as in the Basic Link Management scheme (see 12.2/Q.704), or it can be connected via a switchblock and allocated automatically (see 12.3/Q.704).

Automatic allocation of signalling terminals may be done even though the signalling data link can be (semi-) permanently allocated to the signalling link.

#### B.1.2.14	MTP routing tableXE "MTP routing table"§

This is a supporting item, which contains details of all the network’s routesets. The network management view enables the individual network element views (i.e. the SPs’ MTP routing tables) to be coordinated. It may be thought of as a list of routesets organized as a matrix of rows and columns, the element in row i column j is the routeset from SP i to destination SP j.

It has an administrative state, to enable changes to be made consistently in the network.

##### B.1.2.14.1	Use of MTP routing tableXE "Use of MTP routing table"§

A signalling point wishing to route a load-shared message to a destination D chooses from the routeset to D the highest priority available linkset or combined linkset.

If the highest priority available entity is a linkset, the link over which to send the message is determined from the linkset’s load sharing algorithm and the message’s SLS code.

If the entity is a combined linkset, the selection of the link used may be viewed as consisting of two stages (although in practice these stages may be concatenated into one): first the linkset of the combined linkset is chosen, using the inter-linkset load sharing algorithm current in the combined linkset for D, and the whole or part of the SLS code of the message; next the link is chosen from the linkset, using the linkset’s load sharing algorithm and the SLS code of the message.

Annex C

Examples

(This annex forms an integral part of this Recommendation)

Recommendation Q.751.1     (10/95)

NOTE – The tables in C.1, C.2, C.3 do not show ALL possible combinations – which is the reason why they are titled "examples" – of MTP states or of OSI/TMN states of the respective managed objects. However, the given cases are not just examples, but show the correct MTP-OSI/TMN-state mapping of the respective managed objects.

## C.1	Example state mapping of signLinkSetTpXE "Example state mapping of signLinkSetTp"§

MTP versus OSI/TMN state of signLinkSetTp

| MTP functional states |  |  | OSI/TMN state for linkset |  |  |
| --- | --- | --- | --- | --- | --- |
| Link 1 | Link 2 | Link 3 | Operational | Usage | Availability |
| Available | Available | Available | Enabled | Active or busy | – |
| Available | Available | Unavailable | Enabled | Active or busy | Degraded |
| Unavailable | Unavailable | Unavailable because inhibition | Enabled | Idle | Degraded |
| Unavailable | Unavailable | Unavailable | Disabled | Idle | Dependency |

## C.2	Example state mapping of signLinkTpXE "Example state mapping of signLinkTp"§

The following three tables give examples for the state mapping:

a)	for available signLinkTps;

b)	for not available signLinkTps due to one unavailability reason;

c)	for not available signLinkTps with a combination of several unavailability reasons.

#### State mapping for available signalling Link termination points

MTP versus OSI/TMN state of signLinkTp

| Functional state | Admin. state | Operat. state | Usage state | Proc. status | Link termination point status |
| --- | --- | --- | --- | --- | --- |
| Unblocked uninhibited (restored) activated not congested | Unlocked | Enabled | Active | – | – |
| Unblocked uninhibited (restored) activated congested | Unlocked | Enabled | Busy | – | – |
| “–” Denotes the empty status. |  |  |  |  |  |

#### State mapping for not available signalling Link termination points

MTP versus OSI/TMN state of signLinkTp

| Functional state | Admin. state | Operat. state | Usage state | Proc. status | Link termination point status |
| --- | --- | --- | --- | --- | --- |
| Unblocked uninhibited (restored) activated | Unlocked | Enabled | Idle | – | – a) |
| Loc.blocked uninhibited (restored) activated | Unlocked | Disabled | Idle | – | Loc–blk |
| Rem. blocked uninhibited (restored) activated | Unlocked | Disabled | Idle | – | Rem–blk |
| Unblocked Loc.inhib. (restored) | Unlocked | Enabled | Idle | – | Loc–inh |
| Unblocked rem.inhib. (restored) activated | Unlocked | Enabled | Idle | – | Rem–inh |
| Unblocked uninhibited failed | Unlocked | Disabled | Idle | Initializing or not initial. | Failed |
| Unblocked uninhibited (restored) deactivated by OS | Locked | Disabled | Idle | Not initial. | Deact |
| Unblocked uninhibited deactivated by MTP L3 | Unlocked | Disabled | Idle | Not initial. or initializing | Deact |
| a) This is the status after initial alignment, during signalling link test. |  |  |  |  |  |

#### Combined state mapping for not available signalling Link termination points

Examples: Combinations of two, three and four failure reasons.

MTP versus OSI/TMN state of signLinkTp

| Functional state | Admin. state | Operat. state | Usage state | Proc. status | Link termination point status |
| --- | --- | --- | --- | --- | --- |
| Loc.blocked uninhibited failed | Unlocked | Disabled | Idle | Not initial. or initializing | Loc-blk. failed |
| Loc.blocked rem.inhib. deactivated by OS | Locked | Disabled | Idle | Not initial. | Loc-blk, rem-inh, deact |
| Loc.blocked loc. inhib. rem.inhib. deactivated by OS | Locked | Disabled | Idle | Not initial. | Loc-blk, Loc-inh, rem-inh, deact |

## C.3	Example state mapping for signRouteSetNePartXE "Example state mapping for signRouteSetNePart"§

The following table shows the possible state combinations and their relation to the MTP functional state of the signRouteSetNePart.

State mapping for signRouteSetNePart

| Functional state | Admin. state | Operat. state | Congested statea) | Description |
| --- | --- | --- | --- | --- |
| Available | Unlocked | Enabled | Not congested | The route set is accessible |
| Congested | Unlocked | Enabled | Congesteda) | The route set is accessible, but congested |
| Unavailable | Unlocked | Disabled | Not congested | The destination SP is inaccessible, but administratively intended to be used |
| Unavailable | Locked | Enabled | Not congested | The route set is accessible, but is administratively locked |
| Unavailable | Locked | Enabled | Congesteda) | The route set is accessible, but is congested and administratively locked |
| Unavailable | Locked | Disabled | Not congested | The destination SP is inaccessible and administratively prohibited to be used |
| a) If the attribute congestionLevel is used, the respective congestion level applies. |  |  |  |  |

## C.4	Examples for the signRouteNePart MOCXE "Examples for the signRouteNePart MOC"§

The following table represents an example to help understand the way the priorities of the routes behave depending on the value of the priorityMode in case of modifying the priority of one route:

| "target" priority for route5 => priority = 2 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | resulting priority in case of priorityMode = |  |  |  |
| signRouteNePart | "starting priority" | EQUAL | INSERT | EXCHANGE SINGLE | EXCHANGE GROUP |
| route1 | 1 | 1 | 1 | 1 | 1 |
| route2 | 2 | 2 | 3 | 4 | 4 |
| route3 | 2 | 2 | 3 | 4 | 4 |
| route4 | 3 | 3 | 4 | 3 | 3 |
| route5 | 4 | 2 | 2 | 2 | 2 |
| route6 | 4 | 4 | 5 | 4 | 2 |

The following table represents an example to help understand the way the priorities of the routes behave depending on the possibility or impossibility of a "priority gap":

|  |  | resulting priority in case of |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | "priority gap" allowed and |  | "priority gap" not allowed and |  |
| signRouteNePart | "starting priority" | deleting route4 | modifying route4’s priority to 2 with priorityMode = EQUAL | deleting route4 | modifying route4’s priority to 2 with priorityMode = EQUAL |
| route1 | 1 | 1 | 1 | 1 | 1 |
| route2 | 2 | 2 | 2 | 2 | 2 |
| route3 | 2 | 2 | 2 | 2 | 2 |
| route4 | 3 | – | 2 | – | 2 |
| route5 | 4 | 4 | 4 | 3 | 3 |
| route6 | 4 | 4 | 4 | 3 | 3 |

The following table shows the possible state combinations and their relation to the MTP functional state of the signRouteNePart:

| Functional state | Administrative state | Operational state | Usage state | Availability status | Description |
| --- | --- | --- | --- | --- | --- |
| Available | Unlocked | Enable | Active | {} | Available and in use |
| Available | Unlocked | Enable | Idle | {} | Available but not used (Note 1) |
| Restricted (Note 2) | Unlocked | Enable | Busy | {Degraded} | A TFR message has been received and the Route is in use |
| Restricted (Note 2) | Unlocked | Enable | Idle | {Degraded} | A TFR message has been received and the Route is not in use |
| Prohibited (Note 3) | Unlocked | Disable | Idle | {Dependency} | A TFP message has been received |
| Unavailable (Note 3) | Unlocked | Disable | Idle | {Dependency} | The Link Set is unavailable |
| Unavailable | Locked | Disable | Idle | {Off Line} | The Route is locked by the manager |
| Unavailable | Unlocked | Disable | Idle | {Off Line} | The Route is unlocked by the OS but locked by a local administrator |
| NOTES 1 Because a route with higher priority is available (see also 6.2.12). 2 National option. 3 These two states can be distinguished by reading additionally the corresponding Link (Set) State. |  |  |  |  |  |

Annex D

Networkmanagement of an SS No. 7 networkXE "Networkmanagement of an SS No. 7 network"§

(This annex forms an integral part of this Recommendation)

The relationship between the objects on the NM-level and the objects on the NE-level ought to be described.

See Clause 3/Q.750 for a comparison of networkelementmanagement with networkmanagement.

## D.1	Plain text description of object classes

This subclause gives a plain text description of the various managed object-classes which are distinguished on the networkmanagement level. For each object-class the SS No. 7 function and the management aspects are clarified. See Figure D.1.

FIGURE D.1/Q.751.1...[D19] = 14.6 CM

NOTE – A signDataLink managed object class might be required on the networkmanagement layer, this is for further study. Other possible managed object classes are: MRVT, CVT, SRVT, etc.

### D.1.1	CIC-table at networkmanagement levelXE "CIC-table at networkmanagement level"§

#### SS No. 7 function

For the identification of telecommunications circuits, ISUP uses the Circuit Identification Code (CIC). It is essential to keep track of the issued CICs in order to certify that the two SEPs at either end of a telecommunications circuit, use the same CIC for this circuit.

#### Management aspects

At the networkmanagement level it must be possible to assign or to modify the CIC-value of a certain telecommunication circuit. An assignment or a modification will cause implicitly a modification of the two corresponding cicTable-objects at networkelementmanagement level. (A direct modification of a cicTable-object at networkelementmanagement level should not be allowed.)

| Object Class | Attributes | Notifications | Actions |
| --- | --- | --- | --- |
| netwCicTable | netwCicTableId |  |  |
|  | cicCircuitComb |  |  |

The attribute cicCircuitComb contains a (OPC-DPC-unique) CIC-identifier for all the telecommunication circuits (of every possible signalling relation in the network).

### D.1.2	Signalling linkXE "Signalling link"§

#### SS No. 7 function

The signalling link on networkmanagement level is modelled by a signLink managed object class. Management-operations on a signLink object will in general have counterpart management-operations on the two corresponding signLinkTp objects.

#### Management aspects

At networkmanagement level it should be possible to reconfigure a signalling link, i.e. allocate different signalling terminals or a different signalling datalink to it. It should be possible to request or to modify the state of a signalling link.

Also it must be possible to modify certain attributes (e.g. congestion thresholds) or collect measurement notifications from the signalling link.

| Object class | Attributes | Notifications | Actions |
| --- | --- | --- | --- |
| signLink | signLinkTpId | stateChange | signLinkTest |
| – (Type - 0) – (Type - 1) – (Type - 3) | state | occQ752-SL | reconfigureLink |
|  | q703-tx |  | modifyTimer |
|  | q704-tx |  |  |
|  | maxCapacitySL |  |  |
|  | currentCapacitySL |  |  |
|  | congThreshTrxLeft |  |  |
|  | congThreshTrxRight |  |  |
|  | congThreshRecLeft |  |  |
|  | congThreshRecRight |  |  |
|  | linkTestPatternLeft |  |  |
|  | linkTestPatternRight |  |  |

For the above-mentioned attributes with a left and right-indication, both ends of the signalling link may have different values.

### D.1.3	Signalling LinkSetXE "Signalling LinkSet"§

#### SS No. 7 function

The object class signLinkSet reflects the characteristics of the signalling link set on networkmanagement level. It is represented by two signLinkSetTps on networkelement level. In defining actions on the signLinkSet, these actions will in general result in actions onto the corresponding signLinkSetTps.

#### Management aspects

For networkmanagement it must be possible to change the state of the signLinkset (as far as the administrative state, the repair-status and the control-status is concerned). Also it must be possible to modify certain attributes or collect measurement notifications from the signalling linkset.

| Object class | Attributes | Notifications | Actions |
| --- | --- | --- | --- |
| signLinkSet | signLinkSetId | stateChange |  |
|  | state | occQ752-SLS |  |
|  | leftSignPoint |  |  |
|  | rightSignPoint |  |  |
|  | errCorrMethod |  |  |
|  | loadShareAlgorithmLeft |  |  |
|  | loadShareAlgorithmRight |  |  |
|  | congControlMethod |  |  |
|  | maxCapacityLSLeft |  |  |
|  | maxCapacityLSRight |  |  |
|  | currentCapacityLSLeft |  |  |
|  | currentCapacityLSRight |  |  |
|  | setOfSignRoutes |  |  |

Whether the set of signalling linkgroups from which a signLinkSet is constructed should be made visible is for further study.

### D.1.4	Signalling pointXE "Signalling point"§

#### SS No. 7 function

The object class netwSignPoint reflects the characteristics of the signalling point as viewed by a network manager.

#### Management aspects

At networkmanagement level it must be possible to start an MRVT and CVT for a specific destination and it must be possible to restart a netwSignPoint. Also it must be possible to modify or request the state of the netwSignPoint. The netwSignPoint object has a read-only attribute called hierarchyLayer. This attribute denotes an identification of the layer in the signalling network hierarchy to which this netwSignPoint belongs. It can be important for testing whether the implemented routing satisfies certain rules.

| Object class | Attributes | Notifications | Actions |
| --- | --- | --- | --- |
| netwSignPoint | netwSignPointId | mrvtTestResult | startMrvt |
|  | state | stateChange | restartSp |
|  | spType | cvtResult | startCvt |
|  | pointCode |  |  |
|  | location |  |  |
|  | hierarchyLayer |  |  |
| location: This represents a geographical location. |  |  |  |

### D.1.5	Signalling RouteXE "Signalling Route"§

NOTE – The managed objects signRoute and signRouteSet are present in the NM-layer because we suppose that in this way an Administration is able to modify in a coordinated manner all individual Routing Tables within the signalling Points.

Note that a particular signRoute may be part of many other signRoute(s), and a particular signRouteSet may be part of many other signRouteSet(s), and any change to the former would cause corresponding changes to the latter. Such changes might best be co-ordinated automatically by an OS.

#### SS No. 7 function

A signalling route is an ordered sequence of signalling points, directly interconnected by linksets.

#### Management aspects

For networkmanagement it must be possible to change the state of the signRoute (as far as the administrative state, the repair-status and the control-status is concerned).

Such a modification will result in an (automatic) modification of mtpRouteTable objects on networkelement-management level. If this signRoute forms a part of another signRoute, then any modification will have a corresponding effect upon the other signRoute.

NOTE – Whether a signalling relation managed object class adds additional information is for further study.

It may be required to add subclasses for various route-types. These are routes with special characteristics.

| Object class | Attributes | Notifications | Actions |
| --- | --- | --- | --- |
| signRoute | signRouteId | stateChange |  |
|  | signPointSequence |  |  |
|  | signLinkSetSequence |  |  |
|  | state |  |  |

The signPointSequence attribute is defined as a sequence of signPointId's. The signLinkSetSequence is defined as a sequence of signLinkSetIds.

### D.1.6	Signalling RouteSetXE "Signalling RouteSet"§

#### SS No. 7 function

A signalling route set is the set of the signalling routes between two Signalling Points (SPs). For each signalling relation in the network a signalling route set (signRouteSet object) is defined.

#### Management aspects

On the networkmanagement level it must be possible to define or to modify an existing signRouteSet. Such an activity on the networkmanagement level will result in automatic and implicit definition or modification of mtpRouteTable objects on the networkelementmanagement level. Also it must be possible to change the state of the signRouteSet (as far as the administrative state, the repair-status and the control-status is concerned).

| Object class | Attributes | Notifications | Actions |
| --- | --- | --- | --- |
| signRouteSet | signRouteSetId | stateChange |  |
|  | opc |  |  |
|  | dpc |  |  |
|  | state |  |  |

### D.1.7	SS No. 7 networkXE "SS No. 7 network"§

### D.1.8	Measurements

#### SS No. 7 function

#### Management aspects

In Recommendation Q.752 a large number of measurements is defined, for instance duration measurements (i.e. duration measurements) which have to be activated on demand. For instance, for a signLink object a measurement objectclass on networkmanagement level (netwMsmt) could be defined. When such a measurement is activated on the networkmanagement level, implicitly and automatically two measurements could be started on the networkelementmanagement level which pertain to the two corresponding signLinkTp-objects.

NOTE – For every measurement defined in Recommendation Q.752 it should be studied how they have to be modelled on networkmanagement level.

## D.2	Semi-formal specification of the managed object classes

NOTE – To be supplied.

Annex E

Formal descriptions of MTP measurementsXE "Formal descriptions of MTP measurements"§

(This annex forms an integral part of this Recommendation)

This annex contains GDMO templates for modelling measurements for the MTP. These measurements are described in  Tables 1 to 6/Q.752. Subclause E.1 contains tables that indicate the relationship with the modelling done in this annex to the descriptions in Recommendation Q.752. Subclause E.2 presents the GDMO modelling. The ASN.1 descriptions for attributes defined in this annex are included in the main body of this Recommendation.

## E.1	Relationship with Recommendation Q.752

| Q.752 meas. | Represented by | With name | With syntax | In Managed Object Class |
| --- | --- | --- | --- | --- |
| 1.1 | Attribute | slInServiceDuration | Seconds | oblSignallingLinkDurationData, granularityPeriod = 30min (obl/perm measurement) |
| 1.2 | Notification | X.721:communicationsAlarm | probableCause = 102 | signLinkTp (obl/perm measurement) |
| 1.3 | Notification | X.721:communicationsAlarm | probableCause = 102 specificProblems = 003 | signLinkTp |
| 1.4 | Notification | X.721:communicationsAlarm | probableCause = 102 specificProblems = 004 | signLinkTp |
| 1.5 | Notification | X.721:communicationsAlarm | probableCause = 102 specificProblems = 005 | signLinkTp |
| 1.6 | Notification | X.721:communicationsAlarm | probableCause = 102 specificProblems = 006 | signLinkTp |
| 1.7 | Attribute | slAlignment | X.721:counter | allSignallingLinkUtilizationData, granularityPeriod = 5,30min |
| 1.8 | Attribute | signUnitsReceived | X.721:counter | oblSignallingLinkUtilizationData, granularityPeriod = 5,30min |
| 1.9 | Attribute | negAckReceived | X.721:counter | allSignallingLinkUtilizationData, granularityPeriod = 5,30min |
| Q.752 meas. | Represented by | With name | With syntax | In Managed Object Class |
| 1.10 | Attribute | localChangeOvers | X.721:counter | allSignallingLinkUtilizationData, granularityPeriod = 30min |
|  | Notification | ss7OnOccEvent | probableCause = 110 PerceivedSeverity = Maj/Min/War | signLinkTp |
| 1.11 | Notification | ss7OnOccEvent | probableCause = 110 PerceivedSeverity = Cleared | signLinkTp |
| 1.12 | Notification | X.721:communicationsAlarm | probableCause = 102 PerceivedSeverity = Cleared | signLinkTp |
| 2.1 | Attribute | slUnavailabilityDuration | Seconds | oblSignallingLinkDurationData, granularityPeriod = 30min (obl/perm measurement) |
| 2.5 | Attribute | slLocalInhibition | Seconds | all30MinSignallingLinkDurationData, granularityPeriod = 30min |
| 2.6 | Attribute | slRemoteInhibition | Seconds | all30MinSignallingLinkDurationData, granularityPeriod = 30min |
| 2.7 | Attribute | slFailed | Seconds | all30MinSignallingLinkDurationData, granularityPeriod = 30min |
| 2.9 | Attribute | slRemoteProcOutage | Seconds | all30MinSignallingLinkDurationData, granularityPeriod = 30min |
| Q.752 meas. | Represented by | With name | With syntax | In Managed Object Class |
| 2.10 | Notification | ss7OnOccEvent | probableCause = 210 PerceivedSeverity = Maj/Min/War | signLinkTp |
| 2.11 | Notification | ss7OnOccEvent | probableCause = 210 PerceivedSeverity = Cleared | signLinkTp |
| 2.13 | Attribute | localMgntInhibit | Seconds | all5And30MinSignallingLinkDurationData, granularityPeriod = 5,30min |
| 2.14 | Attribute | localMgntUninhibit | Seconds | all5And30MinSignallingLinkDurationData, granularityPeriod = 5,30min |
| 2.15 | Attribute | localBusy | X.721:counter | all5And30MinSignallingLinkDurationData, granularityPeriod = 5,30min |
| 2.16 | Notification | ss7OnOccEvent | probableCause = 216 PerceivedSeverity = Maj/Min/War | signLinkTp |
| 2.17 | Notification | ss7OnOccEvent | probableCause = 216 PerceivedSeverity = Cleared | signLinkTp |
| 2.18 | Notification | ss7OnOccEvent | probableCause = 218 PerceivedSeverity = Maj/Min/War | signLinkTp |
| 2.19 | Notification | ss7OnOccEvent | probableCause = 218 PerceivedSeverity = Cleared | signLinkTp |
| Q.752 meas. | Represented by | With name | With syntax | In Managed Object Class |
| 3.1 | Attribute | transmittedOctetsSIFSIO | X.721:counter | oblSignallingLinkUtilizationData granularityPeriod = 5,30min (obl/perm measurement) |
| 3.2 | Attribute | retransmittedOctets | X.721:counter | allSignallingLinkUtilizationData, granularityPeriod = 5,30min |
| 3.3 | Attribute | transmittedMSUs | X.721:counter | allSignallingLinkUtilizationData, granularityPeriod = 5,30min |
| 3.4 | Attribute | receivedOctetsSIFSIO | X.721:counter | oblSignallingLinkUtilizationData, granularityPeriod = 5,30min (obl/act measurement) |
| 3.5 | Attribute | receivedMSUs | X.721:counter | allSignallingLinkUtilizationData, granularityPeriod = 5,30min |
| 3.6 | Attribute | slCongestedStarts | X.721:counter | signallingLinkCongestionData, granularityPeriod = 5,30min |
|  | Notification | X.721:qualityOfServiceAlarm | probableCause = 306 PerceivedSeverity = Maj/Min/War | signLinkTp |
|  | First and interval attribute | firstAndIntervalThresholdValue | Events with value 1 | firstAndIntervalThreshold |
| 3.7 | Attribute | slCongestedDuration | Seconds | signallingLinkCongestionData, granularityPeriod = 30min |
| Q.752 meas. | Represented by | With name | With syntax | In Managed Object Class |
| 3.9 | Attribute | slCongestionStops | X.721:counter | signallingLinkCongestionData |
|  | Notification | X.721:qualityOfServiceAlarm | probableCause = 306 PerceivedSeverity = Cleared | signLinkTp |
|  | First and interval attribute | firstAndIntervalThresholdValue | Events with value 1 | firstAndIntervalThreshold |
| 3.10 | Attribute | discardedMSUs | X.721:counter | oblSignallingLinkUtilizationData, granularityPeriod = 5,30min (obl/perm measurement) |
| 3.11 | Attribute | congestionEventsMSULoss | X.721:counter | signallingLinkCongestionData, granularityPeriod = 5,30min |
|  | Notification | X.721:qualityOfServiceAlarm | probableCause = 311 | signLinkTp |
|  | First and interval attribute | firstAndIntervalThresholdValue | Events with value 1 | firstAndIntervalThreshold |
| 4.2 | Attribute | slsUnavailable | Seconds | SLSDurationData, granularityPeriod = 30min |
| 4.3 | Notification | X.721:communicationsAlarm | probableCause = LinkSetFailure403 PerceivedSeverity = Maj/Min/War | signLinkSetTp |
| 4.4 | Notification | X.721:communicationsAlarm | probableCause = 403 PerceivedSeverity = Cleared | signLinkSetTp |
| Q.752 meas. | represented by | With name | With syntax | In Managed Object Class |
| 4.5 | Notification | ss7OnOccEvent | probableCause = 405 PerceivedSeverity = Maj/Min/War AddInfo = InaccessibleSp | mtpLevel3 |
| 4.6 | Notification | ss7OnOccEvent | probableCause = 405 PerceivedSeverity = Cleared AddInfo = InaccessibleSp | mtpLevel3 |
| 4.9 | Attribute | routeSetUnavailable | X.721:counter | routeSetInformationData, granularityPeriod = 30min |
| 4.10 | Attribute | routeSetUnavailableDuration | Seconds | routeSetInformationData, granularityPeriod = 30min |
| 4.11 | Notification | X.721:communicationsAlarm | probableCause = 411 | routeSetNePart |
| 4.12 | Notification | X.721:communicationsAlarm | probableCause = 411 PerceivedSeverity = Cleared | routeSetNePart |
| 4.13 | Notification | ss7OnOccEvent | probableCause = 413 PerceivedSeverity = Warning AddInfo = ChangeInLsToAdjSp | mtpLevel3 |
| 5.1 | Attribute | adjacentInaccessibleEvents | X.721:counter | signallingPointData, granularityPeriod = 5,30min |
| Q.752 meas. | Represented by | With name | With syntax | In Managed Object Class |
| 5.1 (Cont.) | Notification | X.721:communicationsAlarm | probableCause = 501 PerceivedSeverity = Maj/Min/War AddInfo = InaccessibleSpi | mtpLevel3 |
| 5.2 | Attribute | adjacentInaccessibleDuration | Seconds | oblSignallingPointUtilizationData, granularityPeriod = 5,30min (obl/perm measurement) |
| 5.4 | Notification | X.721:communicationsAlarm | probableCause = 501 PerceivedSeverity = Cleared AddInfo = InaccessibleSpi | mtpLevel3 |
| 5.5 | Attribute | discardedMSUs | X.721:counter | signallingPointData, granularityPeriod = 30min (obl/perm measurement) |
|  | Notification | X.721:qualityOfServiceAlarm | probableCause = 505 | signallingPointData |
|  | First and interval attribute | firstAndIntervalThresholdValue | Events with value 1 | firstAndIntervalThreshold |
| 5.6 | Attribute | transmittedUPUnavailable | EventsUP | signallingPointData |
|  | Notification | X.721:qualityOfServiceAlarm | probableCause = 506 | signallingPointData |
|  | First and interval attribute | firstAndIntervalThresholdValue | Events with value 1 | firstAndIntervalThreshold |
| 5.7 | Attribute | receivedUPUnavailable | EventsUP | signallingPointData |
|  | Notification | X.721:qualityOfServiceAlarm | probableCause = 507 | signallingPointData |
|  | First and interval attribute | firstAndIntervalThresholdValue | Events with value 1 | firstAndIntervalThreshold |
| Q.752 meas. | Represented by | With name | With syntax | In Managed Object Class |
| 5.8 | Attribute | receivedTFC | EventsCongestionLevel | signallingPointData |
|  | Notification | X.721:qualityOfServiceAlarm | probableCause = 508 | signallingPointData |
|  | First and interval attribute | firstAndIntervalThresholdValue | Events with value 1 | firstAndIntervalThreshold |
| 6.1 | Attribute | receivedOctetsOPC | X.721:counter | receivedOctetsOPC, granularityPeriod = 5,30min |
| 6.2 | Attribute | transmittedOctetsDPC | X.721:counter | transmittedOctetsDPC, granularityPeriod = 5,30min |
| 6.3 | Attribute | handledOctetsSIO | X.721:counter | handledOctetsSIO, granularityPeriod = 5,30min |
| 6.4 | Attribute | receivedOctetsOpcSio | X.721:counter | receivedOctetsOpcSio, granularityPeriod = 5,30min |
| 6.5 | Attribute | transmittedOctetsDpcSio | X.721:counter | transmittedOctetsDpcSio, granularityPeriod = 5,30min |
| 6.6 | Attribute | handledOctetsOpcDpcSio | X.721:counter | handledOctetsOpcDpcSio, granularityPeriod = 5,30min |
| 6.7 | Attribute | handledMSUsOpcDpcSio | X.721:counter | handledMSUsOpcDpcSio, granularityPeriod = 5,30min |

## E.2	Diagrams of managed object classes for measurements

The following two diagrams provide an overview of the inheritance and naming relations between the managed object classes defined for the MTP measurements. See Figures E.1 and E.2.

FIGURE E.1/Q.751.1...[D20] = page pleine
XE "Inheritance tree for managed object classes representing MTP measurements"§

FIGURE E.2/Q.751.1...[D21] = page pleine
XE "Naming schema for managed objects representing MTP measurements"§

## E.3	Managed Object Class definitions

The following managed object class definitions given in alphabetical order.

### E.3.1	All 30 Minutes Signalling Link Duration DataXE "All 30 Minutes Signalling Link Duration Data"§

all30MinSignallingLinkDurationData MANAGED OBJECT CLASS
	DERIVED FROM oblSignallingLinkDurationData;
	CHARACTERIZED BY all30MinSignallingLinkDurationDataPackage PACKAGE
		BEHAVIOUR all30MinSignallingLinkDurationDataBehaviour BEHAVIOUR DEFINED AS

"This managed object class contains all signalling link duration data with
			granularityPeriod 30 minutes. The attributes are read-only.";;

ATTRIBUTES

slLocalInhibition	GET SET BY CREATE, -- Measurement Q.752/2.5
			slRemoteInhibition	GET SET BY CREATE, -- Measurement Q.752/2.6
			slFailed	GET SET BY CREATE, -- Measurement Q.752/2.7
			slRemoteProcOutage	GET SET BY CREATE;;; -- Measurement Q.752/2.9
REGISTERED AS { mtpObjectClass 17 };

### E.3.2	All 5 And 30 Minutes Signalling Link Duration DataXE "All 5 And 30 Minutes Signalling Link Duration Data"§XE "All 5 And 30 Minutes Signalling Link Duration Data"§

all5And30MinSignallingLinkDurationData MANAGED OBJECT CLASS
	DERIVED FROM ss7CurrentData;
	CHARACTERIZED BY all5And30MinSignallingLinkDurationDataPackage PACKAGE
		BEHAVIOUR all5And30MinSignallingLinkDurationDataBehaviour BEHAVIOUR DEFINED AS

"This managed object class contains all obligated signalling link duration data. The
			preferred granularityPeriod is 5 and 30 minutes. The attributes are read-only.";;

ATTRIBUTES

localMgntInhibit	GET SET BY CREATE, -- Measurement Q.752/2.13
			localMgntUninhibit	GET SET BY CREATE, -- Measurement Q.752/2.14
			localBusy	GET SET BY CREATE;;; -- Measurement Q.752/2.15
REGISTERED AS { mtpObjectClass 18 };

### E.3.3	All Signalling Link Utilization DataXE "All Signalling Link Utilization Data"§

allSignallingLinkUtilizationData MANAGED OBJECT CLASS
	DERIVED FROM oblSignallingLinkUtilizationData;
	CHARACTERIZED BY allSignallingLinkUtilizationDataPackage PACKAGE
		BEHAVIOUR allSignallingLinkUtilizationDataBehaviour BEHAVIOUR DEFINED AS

"This managed object class contains all signalling link utilization data. The preferred
			granularityPeriods are 5 and 30 minutes. The attributes are read-only.";;

ATTRIBUTES
			slAlignment	GET SET BY CREATE, -- Measurement Q.752/1.7
			negAckReceived	GET SET BY CREATE, -- Measurement Q.752/1.9
			retransmittedOctets	GET SET BY CREATE, -- Measurement Q.752/3.2
			transmittedMSUs	GET SET BY CREATE, -- Measurement Q.752/3.3
			receivedMSUs	GET SET BY CREATE;;; -- Measurement Q.751/3.5
	CONDITIONAL PACKAGES

localChangeOversPackage
	PRESENT IF "the granularity period is 30 minutes";
REGISTERED AS { mtpObjectClass 19 };

### E.3.4	All Signalling Point Utilization DataXE "All Signalling Point Utilization Data"§

allSignallingPointUtilizationData MANAGED OBJECT CLASS
	DERIVED FROM oblSignallingPointUtilizationData;
	CHARACTERIZED BY allSignallingPointUtilizationDataPackage PACKAGE

BEHAVIOUR allSignallingPointUtilizationDataBehaviour BEHAVIOUR DEFINED AS

"This managed object class contains all signalling point utilization data. The preferred
			granularityPeriods are 5 or 30 minutes. The attributes are read-only.";;

ATTRIBUTES
			adjacentInaccessibleEvents	GET SET BY CREATE;;; -- Measurement Q.752/5.1
REGISTERED AS { mtpObjectClass 20 };

### E.3.5	Handled MSUs per OPC/DPC/SIOXE "Handled MSUs per OPC/DPC/SIO"§

handledMSUsOpcDpcSioMeasurement MANAGED OBJECT CLASS
	DERIVED FROM ss7CurrentData;
	CHARACTERIZED BY handledMSUsOpcDpcSioMeasurementPackage PACKAGE
		BEHAVIOUR handledMSUsOpcDpcSioMeasurementBehaviour BEHAVIOUR DEFINED AS

"This managed object class starts a measurement for a specific OPC/DPC/SIO combination.
The preferred granularityPeriods are 5 and 30 minutes. Activation of these measurements
should be limited to a small number of OPC/DPC combinations at a given time. The measurement attribute is read-only.";;

ATTRIBUTES
			opc	GET SET BY CREATE,
			dpc	GET SET BY CREATE,
			sio	GET SET BY CREATE,
			handledMSUsOpcDpcSio	GET SET BY CREATE;;; -- Measurement Q.752/6.7
REGISTERED AS { mtpObjectClass 21 };

### E.3.6	Handled Octets per OPC/DPC/SIOXE "Handled Octets per OPC/DPC/SIO"§

handledOctetsOpcDpcSioMeasurement MANAGED OBJECT CLASS
	DERIVED FROM ss7CurrentData;
	CHARACTERIZED BY handledOctetsOpcDpcSioMeasurementPackage PACKAGE
		BEHAVIOUR handledOctetsOpcDpcSioMeasurementBehaviour BEHAVIOUR DEFINED AS

"This managed object class starts a measurement for a specific OPC/DPC/SIO combination. The preferred granularityPeriods are 5 and 30 minutes. Activation of these measurements should be limited to a small number of OPC/DPC combinations at a given time. The attribute is read-only.";;

ATTRIBUTES
			opc	GET SET BY CREATE,
			dpc	GET SET BY CREATE,
			sio	GET SET BY CREATE,
			handledOctetsOpcDpcSio	GET SET BY CREATE;;; -- Measurement Q.752/6.6
REGISTERED AS { mtpObjectClass 22 };

### E.3.7	Handled Octets per SIOXE "Handled Octets per SIO"§

handledOctetsSIOMeasurement MANAGED OBJECT CLASS
	DERIVED FROM ss7CurrentData;
	CHARACTERIZED BY handledOctetsSIOMeasurementPackage PACKAGE
		BEHAVIOUR handledOctetsSIOMeasurementBehaviour BEHAVIOUR DEFINED AS

"This managed object class starts a measurement for a specific SIO. The preferred
			granularityPeriods are 5 and 30 minutes. The attribute is read-only.";;

ATTRIBUTES
			sio	GET SET BY CREATE,
			handledOctetsSIO	GET SET BY CREATE;;; -- Measurement Q.752/6.3
REGISTERED AS { mtpObjectClass 23 };

### E.3.8	Obligated Signalling Link Duration DataXE "Obligated Signalling Link Duration Data"§

oblSignallingLinkDurationData MANAGED OBJECT CLASS
	DERIVED FROM ss7CurrentData;
	CHARACTERIZED BY oblSignallingLinkDurationDataPackage PACKAGE
		BEHAVIOUR oblSignallingLinkDurationDataBehaviour BEHAVIOUR DEFINED AS

"This managed object class contains the obligated signalling link duration data. The
			preferred granularityPeriod is 30 minutes. The attributes are read-only.";;

ATTRIBUTES
			slUnavailabilityDuration	GET SET BY CREATE, -- Measurement Q.752/2.1
			slInServiceDuration	GET SET BY CREATE;;; -- Measurement Q.752/1.1
REGISTERED AS { mtpObjectClass 24 };

### E.3.9	Obligated Signalling Link Utilization DataXE "Obligated Signalling Link Utilization Data"§

oblSignallingLinkUtilizationData MANAGED OBJECT CLASS
	DERIVED FROM ss7CurrentData;
	CHARACTERIZED BY oblSignallingLinkUtilizationDataPackage PACKAGE
		BEHAVIOUR oblSignallingLinkUtilizationDataBehaviour BEHAVIOUR DEFINED AS

"This managed object class contains all obligated signalling link utilization data. The
			granularityPeriod is 30 minutes. The attributes are read-only.";;

ATTRIBUTES
			signUnitsReceived	GET SET BY CREATE, -- Measurement Q.752/1.8
			transmittedOctetsSIFSIO	GET SET BY CREATE, -- Measurement Q.752/3.1
			receivedOctetsSIFSIO	GET SET BY CREATE, -- Measurement Q.751/3.4
			discardedMSUs	GET SET BY CREATE;;; -- Measurement Q.751/3.10
REGISTERED AS { mtpObjectClass  25 };

### E.3.10	Obligated Signalling Point Utilization DataXE "Obligated Signalling Point Utilization Data"§

oblSignallingPointUtilizationData MANAGED OBJECT CLASS
	DERIVED FROM ss7CurrentData;
	CHARACTERIZED BY oblSignallingPointUtilizationDataPackage PACKAGE
		BEHAVIOUR oblSignallingPointUtilizationDataBehaviour BEHAVIOUR DEFINED AS

"This managed object class contains all obligated signalling point utilization data. The
		preferred granularityPeriods are 5 minutes. The attributes are read-only.";;

ATTRIBUTES
			adjacentInaccessibleDuration	GET SET BY CREATE;;;   -- Measurement Q.752/5.2
REGISTERED AS { mtpObjectClass 26 };

### E.3.11	Received Octets per OPCXE "Received Octets per OPC"§

receivedOctetsOPCMeasurement MANAGED OBJECT CLASS
	DERIVED FROM ss7CurrentData;
	CHARACTERIZED BY receivedOctetsOPCMeasurementPackage PACKAGE

BEHAVIOUR receivedOctetsOPCMeasurementBehaviour BEHAVIOUR DEFINED AS

"This managed object class starts a measurement for a specific OPC. The preferred
			granularityPeriods are 5 and 30 minutes. Activation of these measurements should be
			limited to a small number of signalling point codes at a given time. The attribute is
			read-only.";;

ATTRIBUTES
			opc	GET SET BY CREATE,
			receivedOctetsOPC	GET SET BY CREATE;;; -- Measurement Q.752/6.1
REGISTERED AS { mtpObjectClass 27 };

### E.3.12	Received Octets per OPC/SIOXE "Received Octets per OPC/SIO"§

receivedOctetsOpcSioMeasurement MANAGED OBJECT CLASS
	DERIVED FROM ss7CurrentData;
	CHARACTERIZED BY receivedOctetsOpcSioMeasurementPackage PACKAGE
		BEHAVIOUR receivedOctetsOpcSioMeasurementBehaviour BEHAVIOUR DEFINED AS

"This managed object class starts a measurement for a specific OPC/SIO combination. The	
			preferred granularityPeriods are 5 and 30 minutes. Activation of these measurements should
			be limited to a small number of signalling point codes at a given time. The attribute is
			read-only.";;

ATTRIBUTES
			opc	GET SET BY CREATE,
			sio	GET SET BY CREATE,
			receivedOctetsOpcSio	GET SET BY CREATE;;; -- Measurement Q.752/6.4
REGISTERED AS { mtpObjectClass 28 };

### E.3.13	Route Set Information DataXE "Route Set Information Data"§

routeSetInformationData MANAGED OBJECT CLASS
	DERIVED FROM ss7CurrentData;
	CHARACTERIZED BY routeSetInformationDataPackage PACKAGE
		BEHAVIOUR routeSetInformationDataBehaviour BEHAVIOUR DEFINED AS

"This managed object class contains all permanent signalling point duration data. The
			granularityPeriod is 30 minutes. The attributes are read-only.";;

ATTRIBUTES
			routeSetUnavailable	GET SET BY CREATE,  -- Measurement Q.752/4.9
			routeSetUnavailableDuration	GET SET BY CREATE;;;  -- Measurement  Q.752/4.10
REGISTERED AS { mtpObjectClass 29 };

### E.3.14	Signalling Link Congestion DataXE "Signalling Link Congestion Data"§

signallingLinkCongestionData MANAGED OBJECT CLASS
	DERIVED FROM ss7FirstAndIntervalCurrentData;
	CHARACTERIZED BY signallingLinkCongestionDataPackage PACKAGE
		BEHAVIOUR signallingLinkCongestionDataBehaviour BEHAVIOUR DEFINED AS

"This managed object class contains the optional signalling link congestion data. The preferred granularityPeriods are 5 and 30 minutes. The attributes slCongestedStarts, slCongestionStops and congestionEventsMSULoss can be referenced from an instance of ss7FirstAndIntervalThresholdData when they are used for a first and interval measurement. The attributes are read-only.";;

ATTRIBUTES
			slCongestedStarts	GET SET BY CREATE, -- Measurement Q.752/3.6
			slCongestionStops	GET SET BY CREATE, -- Measurement Q.752/3.9
			congestionEventsMSULoss	GET SET BY CREATE;;; -- Measurement Q.752/3.11

CONDITIONAL PACKAGES
	slCongestedDurationPackage PRESENT IF "the granularity period is 30 minutes";
REGISTERED AS { mtpObjectClass 30 };

### E.3.15	Signalling Link Set Duration DataXE "Signalling Link Set Duration Data"§

signallingLinkSetDurationData MANAGED OBJECT CLASS
	DERIVED FROM ss7CurrentData;
	CHARACTERIZED BY signallingLinkSetDurationDataPackage PACKAGE
		BEHAVIOUR signallingLinkSetDurationDataBehaviour BEHAVIOUR DEFINED AS

"This managed object class contains all signalling link set duration data. The
granularityPeriod is 30 minutes. The attributes are read-only.";;

ATTRIBUTES
			slsUnavailable	GET SET BY CREATE;;; -- Measurement Q.752/4.2
REGISTERED AS { mtpObjectClass 31 };

### E.3.16	Signalling Point DataXE "Signalling Point Data"§

signallingPointData MANAGED OBJECT CLASS
	DERIVED FROM ss7FirstAndIntervalCurrentData;
	CHARACTERIZED BY signallingPointDataPackage PACKAGE
		BEHAVIOUR signallingPointDataBehaviour BEHAVIOUR DEFINED AS

"This managed object class contains information about signalling point. The preferred
granularityPeriods are 5 or  30 minutes. All attributes are read-only and are referenced by an
instance of ss7FirstAndIntervalThresholdData managed object class so does discardedMSUs
attribute when it is used as a first and interval measurement.";;

ATTRIBUTES
			discardedMSUs	GET SET BY CREATE,   -- Measurement Q.752/5.5
			transmittedUPUnavailable	GET SET BY CREATE,   -- Measurement Q.752/5.6
			receivedUPUnavailable	GET SET BY CREATE,   -- Measurement Q.752/5.7
			receivedTFC	GET SET BY CREATE;;;   -- Measurement Q.752/5.8
REGISTERED AS { mtpObjectClass 32 };

### E.3.17	SS No. 7 Current DataXE "SS No. 7 Current Data"§

ss7CurrentData MANAGED OBJECT CLASS
	DERIVED FROM "ITU-T Rec. Q.822 (1994)":currentData;
	CHARACTERIZED BY ss7CurrentDataPackage PACKAGE
		BEHAVIOUR ss7CurrentDataBehaviour BEHAVIOUR DEFINED AS

"This managed object class is used for subtyping of Q.752 duration measurements.
Subtypes will have to contain one or more attributes that can be scanned.";;;;

REGISTERED AS { mtpObjectClass 33 };

### E.3.18	SS No. 7 First and Interval Current DataXE "SS No. 7 First and Interval Current Data"§

ss7FirstAndIntervalCurrentData MANAGED OBJECT CLASS
	DERIVED FROM ss7CurrentData;
	CHARACTERIZED BY ss7FirstAndIntervalCurrentDataPackage PACKAGE
		BEHAVIOUR ss7FirstAndIntervalCurrentDataBehaviour BEHAVIOUR DEFINED AS

"This managed object class is used for subtyping of Q.752 first and interval measurements.
Subtypes will have to contain one or more attributes that can be scanned. The start of a
measurement can possibly generate an X.721:qualityOfServiceAlarm with an indication of the
probable cause in an instance of another class.";;;;

REGISTERED AS { mtpObjectClass 34 };

### E.3.19	SS No. 7 First and Interval Threshold DataXE "SS No. 7 First and Interval Threshold Data"§

ss7FirstAndIntervalThresholdData MANAGED OBJECT CLASS
	DERIVED FROM "ITU-T Rec. Q.822 (1994)": thresholdData;
	CHARACTERIZED BY
		"ITU-T Rec. Q.822 (1994)": counterThresholdListPackage,

ss7FirstAndIntervalThresholdDataPackage PACKAGE
		BEHAVIOUR ss7FirstAndIntervalThresholdDataBehaviour BEHAVIOUR DEFINED AS

"The conditional package counterThresholdListPackage is mandatory for this subclass. The
threshold values of the attributes in the counterThresholdAttributeList will be 1, indicating that
the first event should be notified to the managed using a qualityOfServiceAlarm notification.
The attributes being referenced will be part of instances of ss7FirstAndIntervalCurrentData or
one of its subclasses.";;;;

REGISTERED AS { mtpObjectClass 35 };

### E.3.20	Transmitted Octets per DPCXE "Transmitted Octets per DPC"§

transmittedOctetsDPCMeasurement MANAGED OBJECT CLASS
	DERIVED FROM ss7CurrentData;
	CHARACTERIZED BY transmittedOctetsDPCMeasurementPackage PACKAGE
		BEHAVIOUR transmittedOctetsDPCMeasurementBehaviour BEHAVIOUR DEFINED AS

"This managed object class starts a measurement for a specific DPC. The preferred
granularityPeriods are 5 and 30 minutes. Activation of these measurements should be
limited to a small number of signalling point codes at a given time. The attribute is read-only.";;

ATTRIBUTES
			dpc	GET SET BY CREATE,
			transmittedOctetsDPC	GET SET BY CREATE;;; -- Measurement Q.752/6.2
REGISTERED AS { mtpObjectClass 36 };

### E.3.21	Transmitted Octets per DPC/SIOXE "Transmitted Octets per DPC/SIO"§

transmittedOctetsDpcSioMeasurement MANAGED OBJECT CLASS
	DERIVED FROM ss7CurrentData;
	CHARACTERIZED BY transmittedOctetsDpcSioMeasurementPackage PACKAGE
		BEHAVIOUR transmittedOctetsDpcSioMeasurementBehaviour BEHAVIOUR DEFINED AS

"This managed object class starts a measurement for a specific DPC/SIO combination. The
preferred granularityPeriods are 5 and 30 minutes. Activation of these measurements should
be limited to a small number of signalling point codes at a given time. The attribute is
read-only.";;

ATTRIBUTES
			dpc	GET SET BY CREATE,
			sio	GET SET BY CREATE,
			transmittedOctetsDpcSio	GET SET BY CREATE;;; -- Measurement Q.752/6.5
REGISTERED AS { mtpObjectClass 37 };

## E.4	Package definitionsXE "Package definitions"§

localChangeOversPackage PACKAGE
	ATTRIBUTES
		localChangeOvers	GET SET BY CREATE; -- Measurement Q.752/1.10
REGISTERED AS { mtpPackage 53 };

slCongestedDurationPackage PACKAGE
		ATTRIBUTES
			slCongestedDuration	GET SET BY CREATE; -- Measurement Q.752/3.7
REGISTERED AS { mtpPackage 54 };

## E.5	Attribute DefinitionsXE "Attribute Definitions"§

adjacentInaccessibleDuration ATTRIBUTE
	WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.Seconds;
	MATCHES FOR EQUALITY;
	BEHAVIOUR adjacentInaccessibleDurationBehaviour BEHAVIOUR DEFINED AS

"This attribute represents measurement Q.752/5.2.";;

REGISTERED AS { mtpAttribute 135 };

adjacentInaccessibleEvents ATTRIBUTE
	DERIVED FROM "ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":counter;
	BEHAVIOUR adjacentInaccessibleEventsBehaviour BEHAVIOUR DEFINED AS

"This attribute represents measurement Q.752/5.1.";;

REGISTERED AS { mtpAttribute 136 };

congestionEventsMSULoss ATTRIBUTE
	DERIVED FROM "ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":counter;
	BEHAVIOUR congestionEventsMSULossBehaviour BEHAVIOUR DEFINED AS

"This attribute represents measurement Q.752/3.11.";;

REGISTERED AS { mtpAttribute 137 };

discardedMSUs ATTRIBUTE
	DERIVED FROM "ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":counter;
	BEHAVIOUR discardedMSUsBehaviour BEHAVIOUR DEFINED AS

"This attribute represents measurement Q.751/3.10.";;

REGISTERED AS { mtpAttribute 138 };

handledOctetsOpcDpcSio ATTRIBUTE
	DERIVED FROM "ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":counter;
	BEHAVIOUR handledOctetsOpcDpcSioBehaviour BEHAVIOUR DEFINED AS

"This attribute represents measurement Q.752/6.6.";;

REGISTERED AS { mtpAttribute 139 };

handledOctetsSIO ATTRIBUTE
	DERIVED FROM "ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":counter;
	BEHAVIOUR handledOctetsSIOBehaviour BEHAVIOUR DEFINED AS

"This attribute represents measurement Q.752/6.3.";;

REGISTERED AS { mtpAttribute 140 };

handledMSUsOpcDpcSio ATTRIBUTE
	DERIVED FROM "ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":counter;
	BEHAVIOUR handledMSUsOpcDpcSioBehaviour BEHAVIOUR DEFINED AS

"This attribute represents measurement Q.752/6.7.";;

REGISTERED AS { mtpAttribute 141 };

localBusy  ATTRIBUTE
	DERIVED FROM "ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":counter;
	BEHAVIOUR localBusyBehaviour  BEHAVIOUR DEFINED AS

"This attribute represents measurement Q.752/2.15. See also 9.3/Q.704.";;

REGISTERED AS { mtpAttribute 142 };

localChangeOvers ATTRIBUTE
	DERIVED FROM "ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":counter;
	BEHAVIOUR localChangeOversBehaviour BEHAVIOUR DEFINED AS

"This attribute represents measurement Q.752/1.10. See also 5/Q.704.";;

REGISTERED AS { mtpAttribute 143 };

localMgntInhibit  ATTRIBUTE
	WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.Seconds;
	MATCHES FOR EQUALITY;
	BEHAVIOUR localMgntInhibitBehaviour  BEHAVIOUR DEFINED AS

"This attribute represents measurement Q.752/2.13. See also 10.2/Q.704.";;

REGISTERED AS { mtpAttribute 144 };

localMgntUninhibit ATTRIBUTE
	WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.Seconds;
	MATCHES FOR EQUALITY;
	BEHAVIOUR localMgntUninhibitBehaviour BEHAVIOUR DEFINED AS

"This attribute represents measurement Q.752/2.14. See also 10.3/Q.704.";;

REGISTERED AS { mtpAttribute 145 };

negAckReceived ATTRIBUTE
	DERIVED FROM "ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":counter;
	BEHAVIOUR negAckReceivedBehaviour BEHAVIOUR DEFINED AS

"This attribute represents measurement Q.752/1.9.";;

REGISTERED AS { mtpAttribute 146 };

receivedMSUs ATTRIBUTE
	DERIVED FROM "ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":counter;
	BEHAVIOUR receivedMSUsBehaviour BEHAVIOUR DEFINED AS

"This attribute represents measurement Q.751/3.5.";;

REGISTERED AS { mtpAttribute 147 };

receivedOctetsOPC ATTRIBUTE
	DERIVED FROM "ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":counter;
	BEHAVIOUR receivedOctetsOPCBehaviour BEHAVIOUR DEFINED AS

"This attribute represents measurement Q.752/6.1.";;

REGISTERED AS { mtpAttribute 148 };

receivedOctetsOpcSio ATTRIBUTE
	DERIVED FROM "ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":counter;
	BEHAVIOUR receivedOctetsOpcSioBehaviour BEHAVIOUR DEFINED AS

"This attribute represents measurement Q.752/6.4.";;

REGISTERED AS { mtpAttribute 149 };

receivedOctetsSIFSIO ATTRIBUTE
	DERIVED FROM "ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":counter;
	BEHAVIOUR receivedOctetsSIFSIOBehaviour BEHAVIOUR DEFINED AS

"This attribute represents measurement Q.751/3.4.";;

REGISTERED AS { mtpAttribute 150 };

receivedTFC ATTRIBUTE
	WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.EventsCongestionLevel;
	BEHAVIOUR receivedTFCBehaviour BEHAVIOUR DEFINED AS

"This attribute represents measurement Q.752/5.8.";;

REGISTERED AS { mtpAttribute 151 };

receivedUPUnavailable ATTRIBUTE
	WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.EventsUP;
	BEHAVIOUR receivedUPUnavailableBehaviour BEHAVIOUR DEFINED AS

"This attribute represents measurement Q.752/5.7. See also Recommendation Q.704/11.7.2.";;

REGISTERED AS { mtpAttribute 152 };

retransmittedOctets ATTRIBUTE
	DERIVED FROM "ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":counter;
	BEHAVIOUR retransmittedOctetsBehaviour BEHAVIOUR DEFINED AS

"This attribute represents measurement Q.752/3.2. See also 5/Q.704.";;

REGISTERED AS { mtpAttribute 153 };

routeSetUnavailable ATTRIBUTE
	DERIVED FROM "ITU Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":counter;
	BEHAVIOUR routeSetUnavailableBehaviour BEHAVIOUR DEFINED AS

"This attribute represents measurement Q.752/4.9. See also Recommendation Q.704/11.2.2.";;

REGISTERED AS { mtpAttribute 154 };

routeSetUnavailableDuration ATTRIBUTE
	WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.Seconds;
	MATCHES FOR EQUALITY;
	BEHAVIOUR routeSetUnavailableDurationBehaviour  BEHAVIOUR DEFINED AS

"This attribute represents measurement Q.752/4.10. See also Recommendation Q.704/11.2.1.";;

REGISTERED AS { mtpAttribute 155 };

signUnitsReceived ATTRIBUTE
	DERIVED FROM "ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":counter;
	BEHAVIOUR signUnitsReceivedBehaviour BEHAVIOUR DEFINED AS

"This attribute represents measurement Q.752/1.8. See also 4/Q.704.";;

REGISTERED AS { mtpAttribute 156 };

sio ATTRIBUTE
	WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.SIOType;
	MATCHES FOR EQUALITY;
REGISTERED AS { mtpAttribute 157 };

slAlignment ATTRIBUTE
	DERIVED FROM "ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":counter;
	BEHAVIOUR slAlignmentBehaviour BEHAVIOUR DEFINED AS

"This attribute represents measurement Q.752/1.7.";;

REGISTERED AS { mtpAttribute 158 };

slCongestedDuration  ATTRIBUTE
	WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.Seconds;
	MATCHES FOR EQUALITY;
	BEHAVIOUR slCongestedDurationBehaviour  BEHAVIOUR DEFINED AS

"This attribute represents measurement Q.752/3.7.";;

REGISTERED AS { mtpAttribute 159 };

slCongestedStarts  ATTRIBUTE
	DERIVED FROM "ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":counter;
	BEHAVIOUR slCongestedStartsBehaviour  BEHAVIOUR DEFINED AS

"This attribute represents measurement Q.752/3.6. See also 3.8/Q.704.";;

REGISTERED AS { mtpAttribute 160 };

slCongestionStops  ATTRIBUTE
	DERIVED FROM "ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":counter;
	BEHAVIOUR slCongestionStopsBehaviour  BEHAVIOUR DEFINED AS

"This attribute represents measurement Q.752/3.9. See also 3.8/Q.704.";;

REGISTERED AS { mtpAttribute 161 };

slFailed ATTRIBUTE
	WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.Seconds;
	MATCHES FOR EQUALITY;
	BEHAVIOUR slFailedBehaviour BEHAVIOUR DEFINED AS

"This attribute represents measurement Q.752/2.7. See also 3.2.2/Q.704.";;

REGISTERED AS { mtpAttribute 162 };

slInServiceDuration ATTRIBUTE
	WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.Seconds;
	MATCHES FOR EQUALITY;
	BEHAVIOUR slInServiceDurationBehaviour BEHAVIOUR DEFINED AS

"This attribute represents measurement Q.752/1.1.";;

REGISTERED AS { mtpAttribute 163 };

slLocalInhibition ATTRIBUTE
	WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.Seconds;
	MATCHES FOR EQUALITY;
	BEHAVIOUR slLocalInhibitionBehaviour BEHAVIOUR DEFINED AS

"This attribute represents measurement Q.752/2.5. See also 3.2.8/Q.704.";;

REGISTERED AS { mtpAttribute 164 };

slRemoteInhibition ATTRIBUTE
	WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.Seconds;
	MATCHES FOR EQUALITY;
	BEHAVIOUR slRemoteInhibitionBehaviour BEHAVIOUR DEFINED AS

"This attribute represents measurement Q.752/2.6. See also 3.2.2/Q.704.";;

REGISTERED AS { mtpAttribute 165 };

slRemoteProcOutage ATTRIBUTE
	WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.Seconds;
	MATCHES FOR EQUALITY;
	BEHAVIOUR slRemoteProcOutageBehaviour BEHAVIOUR DEFINED AS

"This attribute represents measurement Q.752/2.9. See also 3.2.6/Q.704.";;

REGISTERED AS { mtpAttribute 166 };

slsUnavailable ATTRIBUTE
	WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.Seconds;
	BEHAVIOUR slsUnavailableBehaviour BEHAVIOUR DEFINED AS

"This attribute represents measurement Q.752/4.2.";;

REGISTERED AS { mtpAttribute 167 };

slUnavailable ATTRIBUTE
	WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.Seconds;
	MATCHES FOR EQUALITY;
	BEHAVIOUR slUnavailableBehaviour BEHAVIOUR DEFINED AS

"This attribute represents measurement Q.752/4.2.";;

REGISTERED AS { mtpAttribute 168 };

slUnavailabilityDuration ATTRIBUTE
	WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.Seconds;
	MATCHES FOR EQUALITY;
	BEHAVIOUR slUnavailabilityDurationBehaviour BEHAVIOUR DEFINED AS

"This attribute represents measurement Q.752/2.1.";;

REGISTERED AS { mtpAttribute 169 };

transmittedMSUs ATTRIBUTE
	DERIVED FROM "ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":counter;
	BEHAVIOUR transmittedMSUsBehaviour BEHAVIOUR DEFINED AS

"This attribute represents measurement Q.752/3.3.";;

REGISTERED AS { mtpAttribute 170 };

transmittedOctetsSIFSIO ATTRIBUTE
	DERIVED FROM "ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":counter;
	BEHAVIOUR transmittedOctetsSIFSIOBehaviour BEHAVIOUR DEFINED AS

"This attribute represents measurement Q.752/3.1. See also 2.3.8/Q.704.";;

REGISTERED AS { mtpAttribute 171 };

transmittedOctetsDPC ATTRIBUTE
	DERIVED FROM "ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":counter;
	BEHAVIOUR transmittedOctetsDPCBehaviour BEHAVIOUR DEFINED AS

"This attribute represents measurement Q.752/6.2.";;

REGISTERED AS { mtpAttribute 172 };

transmittedOctetsDpcSio ATTRIBUTE
	DERIVED FROM "ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":counter;
	BEHAVIOUR transmittedOctetsDpcSioBehaviour BEHAVIOUR DEFINED AS

"This attribute represents measurement Q.752/6.5.";;

REGISTERED AS { mtpAttribute 173 };

transmittedUPUnavailable ATTRIBUTE
	WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.EventsUP;
	BEHAVIOUR transmittedUPUnavailableBehaviour BEHAVIOUR DEFINED AS

"This attribute represents measurement Q.752/5.6. See also Recommendation Q.704/11.7.2.";;

REGISTERED AS { mtpAttribute 174 };

## E.6	Name Binding DefinitionsXE "Name Binding Definitions"§

mtpSignPoint-handledMSUsOpcDpcSioMeasurement NAME BINDING
	SUBORDINATE OBJECT CLASS	handledMSUsOpcDpcSioMeasurement;
	NAMED BY
	SUPERIOR OBJECT CLASS	mtpSignPoint;
	WITH ATTRIBUTE	"ITU-T Rec. X.739 (1993) | ISO/IEC 10164-11 : 1993": scannerId;
	CREATE;
	DELETE ;
REGISTERED AS { mtpNameBinding 16 };

mtpSignPoint-handledOctetsOpcDpcSioMeasurement NAME BINDING
	SUBORDINATE OBJECT CLASS	handledOctetsOpcDpcSioMeasurement;
	NAMED BY
	SUPERIOR OBJECT CLASS	mtpSignPoint;
	WITH ATTRIBUTE	"ITU-T Rec. X.739 (1993) | ISO/IEC 10164-11 : 1993": scannerId;
	CREATE;
	DELETE ;
REGISTERED AS { mtpNameBinding 17 };

mtpSignPoint-handledOctetsSIOMeasurement NAME BINDING
	SUBORDINATE OBJECT CLASS	handledOctetsSIOMeasurement;
	NAMED BY
	SUPERIOR OBJECT CLASS	mtpSignPoint;
	WITH ATTRIBUTE	"ITU-T Rec. X.739 (1993) | ISO/IEC 10164-11 : 1993": scannerId;
	CREATE;
	DELETE ;
REGISTERED AS { mtpNameBinding 18 };

mtpSignPoint-oblSignallingPointUtilisationData NAME BINDING
	SUBORDINATE OBJECT CLASS	oblSignallingPointUtilisationData   AND SUBCLASSES;
	NAMED BY
	SUPERIOR OBJECT CLASS	mtpSignPoint;
	WITH ATTRIBUTE	"ITU-T Rec. X.739 (1993) | ISO/IEC 10164-11 : 1993": scannerId;
	CREATE;
	DELETE ;
REGISTERED AS { mtpNameBinding 19 };

mtpSignPoint-receivedOctetsOPCMeasurement NAME BINDING
	SUBORDINATE OBJECT CLASS	receivedOctetsOPCMeasurement;
	NAMED BY
	SUPERIOR OBJECT CLASS	mtpSignPoint;
	WITH ATTRIBUTE	"ITU-T Rec. X.739 (1993) | ISO/IEC 10164-11 : 1993": scannerId;
	CREATE;
	DELETE ;
REGISTERED AS { mtpNameBinding 20 };

mtpSignPoint-receivedOctetsOpcSioMeasurement NAME BINDING
	SUBORDINATE OBJECT CLASS	receivedOctetsOpcSioMeasurement;
	NAMED BY
	SUPERIOR OBJECT CLASS	mtpSignPoint;
	WITH ATTRIBUTE	"ITU-T Rec. X.739 (1993) | ISO/IEC 10164-11 : 1993": scannerId;
	CREATE;
	DELETE ;
REGISTERED AS { mtpNameBinding 21 };

routeSetNePart-routeSetInformationData NAME BINDING
	SUBORDINATE OBJECT CLASS	routeSetInformationData;
	NAMED BY
	SUPERIOR OBJECT CLASS	routeSetNePart;
	WITH ATTRIBUTE	"ITU-T Rec. X.739 (1993) | ISO/IEC 10164-11 : 1993": scannerId;
	CREATE;
	DELETE ;
REGISTERED AS { mtpNameBinding 22 };

mtpSignPoint-signallingPointData NAME BINDING
	SUBORDINATE OBJECT CLASS	signallingPointData;
	NAMED BY
	SUPERIOR OBJECT CLASS	mtpSignPoint;
	WITH ATTRIBUTE	"ITU-T Rec. X.739 (1993) | ISO/IEC 10164-11 : 1993": scannerId;
	CREATE;
	DELETE ;
REGISTERED AS { mtpNameBinding 23 };

mtpSignPoint-transmittedOctetsDPCMeasurement NAME BINDING
	SUBORDINATE OBJECT CLASS	transmittedOctetsDPCMeasurement;
	NAMED BY
	SUPERIOR OBJECT CLASS	mtpSignPoint;
	WITH ATTRIBUTE	"ITU-T Rec. X.739 (1993) | ISO/IEC 10164-11 : 1993": scannerId;
	CREATE;
	DELETE ;
REGISTERED AS { mtpNameBinding 24 };

mtpSignPoint-transmittedOctetsDpcSioMeasurement NAME BINDING
	SUBORDINATE OBJECT CLASS	transmittedOctetsDpcSioMeasurement;
	NAMED BY
	SUPERIOR OBJECT CLASS	mtpSignPoint;
	WITH ATTRIBUTE	"ITU-T Rec. X.739 (1993) | ISO/IEC 10164-11 : 1993": scannerId;
	CREATE;
	DELETE ;
REGISTERED AS { mtpNameBinding 25 };

signLinkSetTP-signallingLinkSetDurationData NAME BINDING
	SUBORDINATE OBJECT CLASS	signallingLinkSetDurationData;
	NAMED BY
	SUPERIOR OBJECT CLASS	signLinkSetTp;
	WITH ATTRIBUTE	"ITU-T Rec. X.739 (1993) | ISO/IEC 10164-11 : 1993": scannerId;
	CREATE;
	DELETE ;
REGISTERED AS { mtpNameBinding 26 };

signLinkTp-all5And30MinSignallingLinkDurationData NAME BINDING
	SUBORDINATE OBJECT CLASS	all5And30MinSignallingLinkDurationData;
	NAMED BY
	SUPERIOR OBJECT CLASS	signLinkTp;
	WITH ATTRIBUTE	"ITU-T Rec. X.739 (1993) | ISO/IEC 10164-11 : 1993": scannerId;
	CREATE;
	DELETE ;
REGISTERED AS { mtpNameBinding 27 };

signLinkTp-oblSignallingLinkDurationData NAME BINDING
	SUBORDINATE OBJECT CLASS	oblSignallingLinkDurationData AND SUBCLASSES;
	NAMED BY
	SUPERIOR OBJECT CLASS	signLinkTp;
	WITH ATTRIBUTE	"ITU-T Rec. X.739 (1993) | ISO/IEC 10164-11 : 1993": scannerId;
	CREATE;
	DELETE ;
REGISTERED AS { mtpNameBinding 28 };

signLinkTp-oblSignallingLinkUtilisationData NAME BINDING
	SUBORDINATE OBJECT CLASS	oblSignallingLinkUtilisationData AND SUBCLASSES;
	NAMED BY
	SUPERIOR OBJECT CLASS	signLinkTp;
	WITH ATTRIBUTE	"ITU-T Rec. X.739 (1993) | ISO/IEC 10164-11 : 1993": scannerId;
	CREATE;
	DELETE ;
REGISTERED AS { mtpNameBinding 29 };

signLinkTp-signallingLinkCongestionData NAME BINDING
	SUBORDINATE OBJECT CLASS	signallingLinkCongestionData;
	NAMED BY
	SUPERIOR OBJECT CLASS	signLinkTp;
	WITH ATTRIBUTE	"ITU-T Rec. X.739 (1993) | ISO/IEC 10164-11 : 1993": scannerId;
	CREATE;
	DELETE ;
REGISTERED AS { mtpNameBinding 30 };

## E.7	ASN.1 productions

The ASN.1 productions for this annex can be found in 7.8.

Annex F

MTP route verification testXE "MTP route verification test"§

(This annex forms an integral part of this Recommendation)

## F.1	Informal description

| mtpRouteVerifTest |  |  |
| --- | --- | --- |
| Attributes | Notifications | Actions |
| mtpRouteTestId | mtpRouteTestResult | startMtpRouteTest |
| administrativeState | stateChange |  |
| operationalState | objectCreation |  |
| usageState | objectDeletion |  |
| availabilityState |  |  |
| providerObject |  |  |
| version |  |  |
| maxNumOfTests |  |  |
| timeD |  |  |

This managed object class models the MTP Route Verification Test (Recommendation Q.753). By means of instances of this class, an OS can start a route verification test in an SP and it can collect the test result.

#### Attributes

1)	mtpRouteTestId: this attribute represents the instance of this object class.

2)	administrativeState: this attribute represents the OSI administrative state as defined in Recommenda-tion X.731. This attribute is read- and writable.

3)	operationalState: this attribute represents the OSI operational state as defined in Recommendation X.731. This attribute is only readable.

4)	usageState: this attribute represents the OSI usage state as defined in Recommendation X.731. This attribute is only readable.

5)	availabilityState: this attribute represents the OSI availability state as defined in Recommendation X.731. This attribute is only readable.

6)	providerObject: this attribute identifies the managed objects acting in a service provider role for the MRVT object (e.g. OMASE, TCAP, SCCP). The semantics of this attribute type are specified in Recommendation X.732. This attribute is read- and writable.

7)	version: this attribute represents the version of the MRVT implementation. The attribute type is defined in Recommendation M.3100. This attribute is only readable.

8)	maxNumOfTests: this attribute represents the maximum number of simultaneous route tests at one SP. This attribute is only readable.

9)	timeD: this attribute represents the maximum performance time D for an STP to process MRVT and MRVA messages. The value of D should be calculated off-line (e.g. by an OS) in accordance with 2.4.2/Q.753. This attribute is only readable.

#### Notification

1)	mtpRouteTestResult: this notification will be emitted if an MRVR message has been received. It includes the information that is contained in a received MRVR message.

2)	stateChange.

3)	objectCreation.

4)	objectDeletion.

#### Actions

–	startMtpRouteTest: this action starts an MRVT for one test destination. The PC of the test destination and the threshold N of the maximum allowed number of STPs crossed are part of the action syntax. If the MRVT is complete, the result will be returned in the action reply. If the object cannot accept the action due to internal conditions, the reason for the rejection will be returned in the action reply.

## F.2	Formal specification

### F.2.1	Managed object classes definitionsXE "Managed object classes definitions"§

#### MTP Route Verification Test

mtpRouteVerifTest MANAGED OBJECT CLASS
	DERIVED FROM	"ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":top;
	CHARACTERIZED BY	mtpRouteTestPackage PACKAGE
		BEHAVIOUR	mtpRouteTestBehaviour BEHAVIOUR DEFINED AS

"The route verification test is used to determine whether the data of the MTP route tables throughout the network is consistent. The test is started from a certain SP by its OS and this SP will report the test result to its OS. The test checks whether a certain destination can be reached via all possible routes without loops and excessive length routes.

Administrative State:

–	Unlocked: The MRVT is allowed to perform its normal test functions. ACTIONS will be accepted to
start an MRVT for a certain destination.

–	Locked: The MRVT is not allowed to perform its normal functions. No ACTIONS will be accepted.

–	Shutting down: The usage of the MRVT is administratively limited to the running tests. ACTIONS to start a new MRVT will be rejected. If all running tests are finished, the administrative state becomes locked.

Operational State:

–	Enabled: When the MRVT is in the enabled operational state, it is fully operational.

–	Disabled: The MRVT is incapable of performing its normal function. ACTIONS to start an MRVT will be rejected.

Availability Status:

The supported values for this attribute are:

–	Dependency: The MRVT cannot operate because a server resource on which it depends is unavailable (e.g. OMASE, TCAP, SCCP). In this case the operational state is disabled.

–	Empty SET

Usage State:
			The supported values for this attribute are:
			–	Idle: No test is running.
			–	Active: One or more tests are running, and the instance has spare capacity to provide
				additional MRVTs simultaneously.
			–	Busy: The instance cannot provide an additional test, because the maximum number of tests
				that can be run simultaneously has been reached. The number of tests is limited by the value
				of the 'maxNumOfTests' attribute. ACTIONS to start an additional MRVT will be rejected.

			A change in the operational state shall cause a state change notification. A change in
			the administrative state or the usage state shall not emit a state change notification.";;

ATTRIBUTES

mtpRouteTestId			GET SET BY CREATE,
			"ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 :
			1992":administrativeState			GET-REPLACE,
			"ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 :
			1992":operationalState			GET,
			"ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 :
			1992":usageState			GET,
			"ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 :
			1992":availabilityStatus			GET,
			"ITU-T Rec. M.3100 (1992)":version			GET SET BY CREATE,
			"ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 :
			1992":providerObject			GET-REPLACE,
			maxNumOfTests			GET SET BY CREATE,
			timeD			GET SET BY CREATE;
		ACTIONS

startMtpRouteTest;
		NOTIFICATIONS
			mtpRouteTestResult,

"ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":stateChange;;;
	CONDITIONAL PACKAGES

"ITU-T Rec. M.3100 (1992)":createDeleteNotificationsPackage PRESENT IF "the objectCreation
	and objectDeletion notifications defined in 'ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992'
	are supported by an instance of this class",

"ITU-T Rec. M.3100 (1992)":attributeValueChangeNotificationPackage PRESENT IF "the
	attributeValueChangeNotification defined in 'ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992'
	is supported by an instance of this class",

mtpRouteVerifTestNamePackage PRESENT IF "an instance supports it";

REGISTERED AS { mtpObjectClass 1 };

### F.2.2	Package definitionsXE "Package definitions"§

mtpRouteVerifTestNamePackage PACKAGE
	ATTRIBUTES
		mtpRouteVerifTestName		GET-REPLACE;
REGISTERED AS { mtpPackage 1 };

### F.2.3	Attribute definitionsXE "Attribute definitions"§

maxNumOfTests ATTRIBUTE
	WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.MaxNumOfTests;

BEHAVIOUR maxNumOfTestsBehaviour BEHAVIOUR DEFINED AS

"This attribute represents the maximum number of simultaneous route tests at one SP.";;

REGISTERED AS { mtpAttribute 2 };

mtpRouteTestId ATTRIBUTE
	WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.NameType;
	MATCHES FOR EQUALITY, SUBSTRINGS;
REGISTERED AS { mtpAttribute 3 };

mtpRouteVerifTestName ATTRIBUTE
	WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.AdditionalName;
	MATCHES FOR EQUALITY;
	BEHAVIOUR mtpRouteVerifTestNameBehaviour BEHAVIOUR DEFINED AS
		"This attribute is an additional name for instances of the mtpRouteVerifTest managed object
		class.";;
REGISTERED AS { mtpAttribute 4 };

timeD ATTRIBUTE
	WITH ATTRIBUTE SYNTAX MTPDefinedTypesModule.TimeD;
	BEHAVIOUR timeDBehaviour BEHAVIOUR DEFINED AS

"This attribute represents the maximum performance time D for an STP to process MRVT
		and MRVA messages.";;

REGISTERED AS { mtpAttribute 5 };

### F.2.4	Action definitionsXE "Action definitions"§

startMtpRouteTest ACTION
	BEHAVIOUR
	startMtpRouteTestBehaviour BEHAVIOUR DEFINED AS

"This action is used by the Management Process to initiate an MRVT procedure. The MRVT
procedure sends an MRVT message for each signalling route which is contained in the MTP routing
tables to reach the test destination. The destination (DPC) of each of these messages is the adjacent
signalling point within the particular route under test. For each MRVT message a corresponding
MRVA message is expected.
When the MRVT procedure is initiated, a timer T1 is started.
The value of timer T1 depends on the value of the attribute 'timeD' and the value of 'threshold' given
in the action parameters (2.4/Q.753).
When all MRVA messages expected have been received or when T1 expires, the test is complete

and the result will be returned to the Management Process in the action reply.
The Action will be rejected, if
–	the administrative state is locked.
–	the operational state is disabled.
–	the usage state is busy.
–	an MRVT for the test destination given in the action parameters is already in progress.
The reason for the rejection will be returned to the Management Process in the action reply.";;

MODE CONFIRMED;
	WITH INFORMATION SYNTAX	MTPDefinedTypesModule.StartMtpRouteTest;
	WITH REPLY SYNTAX	MTPDefinedTypesModule.ResultMtpRouteTest;
REGISTERED AS { mtpAction 1 };

### F.2.5	Notification definitionsXE "Notification definitions"§

mtpRouteTestResult NOTIFICATION
	BEHAVIOUR	mtpRouteTestResultBehaviour BEHAVIOUR DEFINED AS

"The reception of an MRVR message regardless of whether or not the receiving SP was the initiator
causes this notification type. The notification includes the information contained in the received
MRVR message.";;

WITH INFORMATION SYNTAX MTPDefinedTypesModule.MtpRouteVerifTestResultSntx;

REGISTERED AS { mtpNotification 2 };

### F.2.6	Naming binding definitionsXE "Naming binding definitions"§

mtpSignPoint-mtpRouteVerifTest NAME BINDING
	SUBORDINATE OBJECT CLASS	mtpRouteVerifTest;
	NAMED BY
	SUPERIOR OBJECT CLASS	mtpSignPoint;
	WITH ATTRIBUTE mtpRouteTestId;
	CREATE;
	DELETE;
REGISTERED AS { mtpNameBinding 6 };

### F.2.7	Abstract syntax productions

The ASN.1 productions for this annex can be found in 7.8.

1	) 	In case of an STP, OMAP, and hence also TC and SSCP may be present. If no other user parts than OMAP are present, the SP is still regarded as just an STP.

2) 	National option.

3) 	National option.

4)	In OMAP this is defined to be the linkset priority for the destination (or respectively the combined linkset priority).

5	)	Note that MTP level 3 timers are implicitly MTP-wide, and not just per linkset. But it might be convenient to hold their values per linkset.