|  | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

ITU-T	Q.754

TELECOMMUNICATION	(03/93)
STANDARDIZATION  SECTOR
OF  ITU

SPECIFICATIONS  OF  SIGNALLING  SYSTEM  No.  7

SIGNALLING  SYSTEM  No.  7  MANAGEMENT

SIGNALLING  SYSTEM  NUMBER  7  
MANAGEMENT  APPLICATION  SERVICE  
ELEMENT  (ASE)  DEFINITIONS

ITU-T  Recommendation  Q.754

(Previously “CCITT  Recommendation”)

# FOREWORD

The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of the International Telecommunication Union. The ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Conference (WTSC), which meets every four years, established the topics for study by the ITU-T Study Groups which, in their turn, produce Recommendations on these topics.

ITU-T Recommendation Q.754 was prepared by the ITU-T Study Group XI (1988-1993) and was approved by the WTSC (Helsinki, March 1-12, 1993).

___________________

NOTES

1	As a consequence of a reform process within the International Telecommunication Union (ITU), the CCITT ceased to exist as of 28 February 1993. In its place, the ITU Telecommunication Standardization Sector (ITU-T) was created as of 1 March 1993. Similarly, in this reform process, the CCIR and the IFRB have been replaced by the Radiocommunication Sector.

In order not to delay publication of this Recommendation, no change has been made in the text to references containing the acronyms “CCITT, CCIR or IFRB” or their associated entities such as Plenary Assembly, Secretariat, etc. Future editions of this Recommendation will contain the proper terminology related to the new ITU structure.

2	In this Recommendation, the expression “Administration” is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

ã  ITU  1994

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

CONTENTS

Recommendation Q.754     (03/93)

Page

1	Introduction		1

2	MTP		1

2.1	MTP Routing Verification Test (MRVT)		1

3	SCCP		9

3.1	SCCP Routing Verification Test (SRVT) ASE		9

4	Circuit management		22

4.1	Circuit Validation Test (CVT) ASE		22

5	Transaction capabilities		24

6	General definitions		24

6.1	Objects and operations		24

6.2	Primitives and procedures of the OMASE protocol		25

6.3	Abstract syntax of the OMASE protocol		31

Annex A		36

Recommendation Q.754

Recommendation Q.754     (03/93)

SIGNALLING  SYSTEM  NUMBER  7  MANAGEMENT  APPLICATION  
SERVICE  ELEMENT  (ASE)  DEFINITIONSXE "SIGNALLING  SYSTEM  NUMBER  7  MANAGEMENT  APPLICATION  
SERVICE  ELEMENT  (ASE)  DEFINITIONS"§

(Helsinki, 1993)

# 1	Introduction

Note that in the event of a conflict between Recommendations Q.753 and Q.754, Q.753 will take precedence.

This Recommendation defines the OMAP ASE, OMASE. OMASE provides the services invoked using the OM-EVENT-REPORT and OM-CONFIRMED-ACTION primitives across the OMASE-User to OMASE boundary. (See Recommendation Q.753 for a diagram and mapping between the services invoked in the OMASE-User and those of OMASE.)

The OMASE services are derived from those defined in CMIP1).

The OMASE primitives are defined in clause 6, the formal syntax defined in Figure 3 uses Transaction Capabilities (TC) OPERATION and ERROR macros. The interworking between OMASE and TC is also given in clause 6.

OMASE provides operations allowing the network administration, via the OMAP Management Process and the OMASE-User, to perform MTP and SCCP Routing Verification Tests (MRVT and SRVT), and circuit validation tests (CVT). This Recommendation contains the ASE definition for MRVT, SRVT and CVT.

The SRVT referred-to here is for the specific test in 3.2.2/Q.753.

The arguments used for primitives across the OMAP Management Process to OMASE-User boundary, and for primitives across the OMASE-User to OMASE boundary, and between OMASE and TC contain the same information if they have the same name. Those arguments are defined in this Recommendation.

# 2	MTPXE "MTP"§

## 2.1	MTP Routing Verification Test (MRVT)2)XE "MTP Routing Verification Test (MRVT)2)"§

The MRV Test initiated at the test origin results in an OM-CONFIRMED-ACTION primitive being used from the OMASE-User to OMASE, which includes the testRoute command as a parameter. If a trace of the routes is requested, or a fault exists, the OM-EVENT-REPORT primitive is invoked at the test originator from OMASE, which includes the routeTrace event as a parameter.

testRoute is specified using the CNF-ACTION macro defined in Figure 3, routeTrace is specified using the EVENT macro defined in Figure 3.

For MRVT, the ObjectClass indicates MTP Routing Tables, and the ObjectInstance contains the Point Code of the test destination. The testRoute Action makes use of the BEGIN (MRVT) message with result (MRVA) returning in an END. The routeTrace Event (MRVR) uses a BEGIN message with pre-arranged end.

### 2.1.1	testRoute ActionXE "testRoute Action"§

The testRoute Action is invoked to initiate an MTP routing verification test. At the initiator node, this invocation is requested by the Administration via the MIS-User or a local interface, through the OMAP Management Process and OMASE-User. At subsequent nodes, the Action is requested implicitly by the receipt of a testRoute Action invocation.

A successful reply indicates successful completion of the test at the point it was invoked and, implicitly, at all subsequent points where the test was invoked. A failure indication is returned to indicate that the test failed in this or a subsequent node.

| testRoute CNF-ACTION | Timer = T1 | Class = 1 | Code = 00000001 |
| --- | --- | --- | --- |
| ActionArg |  | Opt/Man | References |
| initiatingSP |  | M | 2.1.1.1.1 |
| traceRequested |  | M | 2.1.1.1.2 |
| threshold |  | M | 2.1.1.1.3 |
| pointCodesTraversed |  | M | 2.1.1.1.4 |
| ActionResult |  |  |  |
| empty |  |  |  |
| Linked Operations |  |  |  |
| N/A |  |  |  |
| Specific Errors |  |  | References |
| failure |  |  | 2.1.1.3.1 |
| partialSuccess |  |  | 2.1.1.3.2 |

| testRoute CNF-ACTION |  |  |
| --- | --- | --- |
| ACTIONINFOARG SEQUENCE { |  |  |
|  | initiatingSP | [0] IMPLICIT PointCode, |
|  | traceRequested | [1] IMPLICIT BOOLEAN, |
|  | threshold | [2] IMPLICIT INTEGER, |
|  | pointCodesTraversed | [3] IMPLICIT PointCodeList |
| } |  |  |
| SPECIFICERRORS | { failure, partialSuccess } |  |
| ::= 1 |  |  |

#### 2.1.1.1	testRoute Action Arguments

##### 2.1.1.1.1	initiatingSP

The initiatingSP identifies the original requestor of the test. It is of type PointCode, defined as an octet string.

| Parameter | Code |
| --- | --- |
| initiatingSP | 10000000 |
| Contents |  |
| Bit 0 contains the first bit of the Point Code, |  |
| Bit 1 contains the second bit of the Point Code, etc. |  |

PointCode ::= OCTET STRING

##### 2.1.1.1.2	traceRequested

traceRequested indicates that a trace of all routes used to reach the destination should be reported to the originator (the routeTrace Event is described in 2.1.2). It is of type BOOLEAN.

| Parameter | Code |
| --- | --- |
| traceRequested | 10000001 |
| Contents | Meaning |
| TRUE (= 1) | trace was requested, return trace information on success and failure |
| FALSE (= 0) | trace not requested, return trace information only on failure |

##### 2.1.1.1.3	threshold

The originator sets a maximum threshold level of Signalling Points (SP) which are allowed to be crossed in the course of the test (including the initiator if it is an STP). This aids in detecting overly long routes. This threshold is an integral number of SPs, thus it is of type INTEGER.

| Parameter | Code |
| --- | --- |
| threshold | 10000010 |
| Contents |  |
| Integer number |  |

##### 2.1.1.1.4	pointCodesTraversed

As each intermediate SP is crossed, it adds its own Point Code to the list of Point Codes traversed. This aids in detecting loops and is also useful information in case of a failure or if a route trace is requested. It is a list of Point Codes, thus of type PointCodeList. This PointCodeList could be empty.

| Parameter | Code |
| --- | --- |
| pointCodesTraversed | 10100011 |
| Contents |  |
| Sequence of Point Codes, tagged as “PointCode” with the contents indicating the exact Point Code |  |

PointCodeList ::= SEQUENCE OF PointCode

#### 2.1.1.2	Action Results

There are no contents in a successful return indication.

#### 2.1.1.3	Action Errors

SpecificErrors are possible errors which can occur during this test, and which are unique to this test. These specific errors are in addition to the errors already identified in the om-Action-Confirmed service and appear as parameters to the Processing Failure Error.

##### 2.1.1.3.1	failure

failure indicates a condition of total failure, where no route worked correctly. Most often this will be used as a failure indication from the point which detects the error and does not invoke any further testRoute Actions. The failure SpecificError has with it a parameter to indicate the error condition causing the failure. This parameter, failureType, is represented as a bit string. In addition, the second parameter is to be used when failureType indicates the error UnknownInitiatingSP. traceSent indicates whether or not a routeTrace Event has been invoked to report trace information. It is necessary to indicate this for this error since the node detecting the error cannot send the routeTrace, thus the previous node must. traceSent is a type of BOOLEAN.

| Specific Error | Code |
| --- | --- |
| failure | 00000001 |
| Parameters | References |
| failureType | 2.1.1.3.1 |
| traceSent | 2.1.1.3.1 |

µ

| Parameter | Code |
| --- | --- |
| failureType | 10000000 |
| Bit | Meaning |
| 0 | detectedLoop |
| 1 | excessiveLengthRoute |
| 2 | unknownObjectInstance |
| 3 | routeInaccessible |
| 4 | processingFailure |
| 5 | unknownInitiatingSP |
| 6 | timerExpired |
| 7 | sPNotAnSTP |

§

| Parameter | Code |
| --- | --- |
| traceSent | 10000001 |
| Contents | Meaning |
| TRUE | the trace information was sent |
| FALSE | the trace information was not sent |

| failure | SPECIFIC-ERROR |  |  |
| --- | --- | --- | --- |
|  | PARAMETER SEQUENCE | { failureType | [0] IMPLICIT FailureString, |
|  |  | traceSent | [ l ] IMPLICIT BOOLEAN } |
|  | ::= 1 |  |  |

| FailureString ::= BIT STRING |  |
| --- | --- |
|  | { detectedLoop (0), |
|  | excessiveLengthRoute (1), |
|  | unknownObjectInstance (2), |
|  | routeInaccessible (3), |
|  | processingFailure (4), |
|  | unknownInitiatingSP (5), |
|  | timerExpired (6), |
|  | sPNotAnSTP (7) } |

##### 2.1.1.3.2	partialSuccess

This indication is given when at least one testRoute Cnf Action invocation failed and at least one succeeded (at least partially). In this case, each type of error that occurred will be noted and sent in the final reply. The format and contents of partial success are the same as failure.

| Specific Error | Code |
| --- | --- |
| partialSuccess | 00000010 |
| Parameters | References |
| failureType | 2.1.1.3.1 |
| traceSent | 2.1.1.3.1 |

| partialSuccess | SPECIFIC-ERROR |  |  |
| --- | --- | --- | --- |
|  | PARAMETER SEQUENCE | { failureType | [0] IMPLICIT FailureString, |
|  |  | traceSent | [1] IMPLICIT BOOLEAN } |
|  | ::= 2 |  |  |

### 2.1.2	routeTrace Event

The routeTrace Event reports trace information. Trace information consists of zero, one or more Point Codes, such as the Point Code detecting an error or the entire list of Point Codes traversed along a route. This event is invoked either at the explicit request of the originating node (indicated by traceRequested, see 2.1.1.1.2) or by failure at any point along the route. This event is not confirmed, therefore no replies to this invocation are expected (no error or success indications are expected).

| routeTrace EVENT | Timer = 0 | Class = 4 | Code = 00000010 |
| --- | --- | --- | --- |
| EventInfo |  | Opt/Man (Note) | References |
| success |  | O | 2.1.2.1.1 |
| detectedLoop |  | O | 2.1.2.1.2 |
| excessiveLengthRoute |  | O | 2.1.2.1.3 |
| unknownObjectInstance |  | O | 2.1.2.1.4 |
| routeInaccessible |  | O | 2.1.2.1.5 |
| processingFailure |  | O | 2.1.2.1.6 |
| unknownInitiatingSP |  | O | 2.1.2.1.7 |
| timerExpired |  | O | 2.1.2.1.8 |
| sPNotAnSTP |  | O | 2.1.2.1.9 |
| NOTE – One and only one of these must be present. |  |  |  |

| routeTrace | EVENT |  |  |
| --- | --- | --- | --- |
|  | EVENTINFO CHOICE { |  |  |
|  |  | success | [0] IMPLICIT PointCodeList, |
|  |  | detectedLoop | [1] IMPLICIT PointCodeList, |
|  |  | excessiveLengthRoute | [2] IMPLICIT PointCodeList, |
|  |  | unknownObjectInstance | [3] IMPLICIT NULL, |
|  |  | routeInaccessible | [4] IMPLICIT PointCode, |
|  |  | processingFailure | [5] IMPLICIT NULL, |
|  |  | unknownInitiatingSP | [6] IMPLICIT PointCode, |
|  |  | timerExpired | [7] IMPLICIT PointCodeList, |
|  |  | sPNotAnSTP | [8] IMPLICIT PointCodeList, |
|  | } |  |  |
|  | ::= 2 |  |  |

#### 2.1.2.1	Event Information

##### 2.1.2.1.1 success

On successful completion, the trace of the Point Codes (one or more) of the crossed SPs are included.

| Parameter | Code |
| --- | --- |
| success | 10100000 |
| Contents | Reference |
| Sequence of Point Codes, tagged as “PointCode”, with contents indicating the exact Point Code | 2.1.1.1.4 |

##### 2.1.2.1.2	detectedLoop

When a loop is detected, the Point Codes (three or more) contained in the loop are included.

| Parameter | Code |
| --- | --- |
| detectedLoop | 10100001 |
| Contents | Reference |
| Sequence of Point Codes, tagged as “PointCode”, with contents indicating the exact Point Code | 2.1.1.1.4 |

##### 2.1.2.1.3	excessiveLengthRoute

When an excessively long route is found (threshold exceeded), the entire route is included.

| Parameter | Code |
| --- | --- |
| excessiveLengthRoute | 10100010 |
| Contents | Reference |
| Sequence of Point Codes, tagged as “PointCode”, with contents indicating the exact Point Code | 2.1.1.1.4 |

##### 2.1.2.1.4	unknownObjectlnstance

If the object instance is unknown, no additional information is required.

| Parameter | Code |
| --- | --- |
| unknownObjectInstance | 10000011 |
| Contents | Reference |
| empty | – |

##### 2.1.2.1.5	routeInaccessible

The Point Code of the node where the route was inaccessible is included.

| Parameter | Code |
| --- | --- |
| routeInaccessible | 10000100 |
| Contents | Reference |
| Bit 0 contains the first bit of the Point Code, | 2.1.1.1.1 |
| Bit 1 contains the second bit of the Point Code, etc. |  |

##### 2.1.2.1.6	processingFailure

If a processing failure occurs, no additional information is required.

| Parameter | Code |
| --- | --- |
| processingFailure | 10000101 |
| Contents | Reference |
| empty | – |

##### 2.1.2.1.7	unknownInitiatingSP

The Point Code of the node detecting the unknown Initiating SP is included.

| Parameter | Code |
| --- | --- |
| unknownInitiatingSP | 10000110 |
| Contents | Reference |
| Bit 0 contains the first bit of the Point Code, | 2.1.1.1.1 |
| Bit 1 contains the second bit of the Point Code, etc. |  |

##### 2.1.2.1.8	timerExpired

The Point Code(s) of the node(s) from where no result for the testRoute Action was received is included.

| Parameter | Code |
| --- | --- |
| timerExpired | 10100111 |
| Contents | Reference |
| Sequence of Point Codes, tagged as “PointCode”, with contents indicating the exact Point Code | 2.1.1.1.4 |

##### 2.1.2.1.9	sPNotAnSTP

If the intermediate SP receiving an MRVT message does not have the MTP transfer function, the list of crossed SPs to reach this SP is included.

| Parameter | Code |
| --- | --- |
| sPNotAnSTP | 10101000 |
| Contents | Reference |
| Sequence of Point Codes, tagged as “PointCode”, with contents indicating the exact Point Code | 2.1.1.1.4 |

# 3	SCCPXE "SCCP"§

## 3.1	SCCP Routing Verification Test (SRVT) ASEXE "SCCP Routing Verification Test (SRVT) ASE"§

This specific SRVT’s functions are defined in 3.2.2/Q.753. The SRV Test initiated at the test origin results in an OM-CONFIRMED-ACTION primitive being used from the OMASE-User to OMASE, which includes the testRoute command as a parameter. If a trace of the routes is requested, or a fault exists, the OM-EVENT-REPORT primitive is invoked at the test originator from OMASE, which includes the routeTrace event as a parameter.

testRoute is specified using the CNF-ACTION macro defined in Figure 3, routeTrace is specified using the EVENT macro defined in Figure 3.

The ObjectClass indicates SCCP Global Title Translation Tables and the ObjectInstance contains the Global Title Indicator and Tested Global Title. The GTI is coded as defined in the SCCP Address Indicator. The testRoute Action (SRVT) makes use of the BEGIN message with the result (SRVA) returning in an END. The routeTrace Event (SRVR) uses a BEGIN message with prearranged end.

### 3.1.1	testRouteAction

The testRoute Action is invoked to initiate an SCCP Routing Verification Test. At the initiator node, this invocation is requested by the Administration via the MIS-User or a local interface, through the OMAP Management Process and OMASE-User. At subsequent nodes, the Action is requested implicitly by the receipt of a testRoute Action invocation. A successful reply indicates successful completion of the test at the point it was invoked and, implicitly, at all subsequent points where the test was invoked. A failure indication is returned to indicate that the test failed in this or a subsequent node.

| testRoute CNF-ACTION | Timer = T2 | Class = 1 | Code = 00000001 |
| --- | --- | --- | --- |
| ActionArg |  | Opt/Man | References |
| initiatingSP |  | M | 3.1.1.1.1 |
| traceRequested |  | M | 3.1.1.1.2 |
| threshold |  | M | 3.1.1.1.3 |
| pointCodesTraversed |  | M | 3.1.1.1.4 |
| formIndicator |  | M | 3.1.1.1.5 |
| mtpBackwardRoutingRequested |  | M | 3.1.1.1.6 |
| testInitiatorGT |  | O | 3.1.1.1.7 |
| destinationPC |  | O | 3.1.1.1.8 |
| destinationSSN |  | O | 3.1.1.1.9 |
| backupDPC |  | O | 3.1.1.1.10 |
| backupSSN |  | O | 3.1.1.1.11 |
| originalGT |  | O | 3.1.1.1.12 |
| ActionResult |  |  |  |
| empty |  |  |  |
| Linked Operations |  |  |  |
| N/A |  |  |  |
| Specific Errors |  |  | References |
| failure |  |  | 3.1.1.3.1 |
| partialSuccess |  |  | 3.1.1.3.2 |

| testRoute CNF-ACTION |  |  |
| --- | --- | --- |
| ACTIONINFOARG SEQUENCE { |  |  |
|  | initiatingSP | [0] IMPLICIT PointCode, |
|  | traceRequested | [1] IMPLICIT BOOLEAN, |
|  | threshold | [2] IMPLICIT INTEGER, |
|  | pointCodesTraversed | [3] IMPLICIT PointCodeList, |
|  | formIndicator | [4] IMPLICIT FormIndicator, |
|  | mtpBackwardRoutingRequested | [5] IMPLICIT BOOLEAN, |
|  | testInitiatorGT | [6] IMPLICIT GlobalTitle OPTIONAL, |
|  | destinationPC | [7] IMPLICIT PointCode OPTIONAL, |
|  | destinationSSN | [8] IMPLICIT SubsystemNumber OPTIONAL, |
|  | backupDPC | [9] IMPLICIT PointCode OPTIONAL, |
|  | backupSSN | [10] IMPLICIT SubsystemNumber OPTIONAL, |
|  | originalGT | [11] IMPLICIT GlobalTitle OPTIONAL |
| } |  |  |
| SPECIFICERRORS |  | { failure, partialSuccess } |
| ::= 1 |  |  |

#### 3.1.1.1	testRoute Action Arguments

##### 3.1.1.1.1	initiatingSP

The initiatingSP identifies the test initiator. It is of type PointCode.

| Parameter | Code |
| --- | --- |
| initiatingSP | 10000000 |
| Contents |  |
| Bit 0 contains the first bit of the Point Code, |  |
| Bit 1 contains the second bit of the Point Code, etc. |  |

PointCode ::= OCTET STRING

##### 3.1.1.1.2	traceRequested

traceRequested indicates that a trace of all routes used to reach the destination should be reported to the originator (the routeTrace Event is described in 3.1.2). It is of type BOOLEAN.

| Parameter | Code |
| --- | --- |
| traceRequested | 10000001 |
| Contents | Meaning |
| TRUE (= 1) | trace was requested, return trace information on success and failure |
| FALSE (= 0) | trace not requested, return trace information only on failure |

##### 3.1.1.1.3	threshold

The originator sets a maximum threshold level of Translation Signalling Points (TSP) which are allowed to be crossed in the course of the test (including the initiator if it is an SCCP Relay Node). This aids in detecting overly long routes. This threshold is an integral number of SP’s, thus it is of type INTEGER.

| Parameter | Code |
| --- | --- |
| threshold | 10000010 |
| Contents |  |
| Integer number |  |

##### 3.1.1.1.4	pointCodesTraversed

As each Translation SP is crossed, it adds its own Point Code to the list of Point Codes traversed. This aids in detecting loops and is also useful information in case of a failure or if a route trace is requested. It is a list of Point Codes, thus of type PointCodeList. This PointCodeList could be empty.

| Parameter | Code |
| --- | --- |
| pointCodesTraversed | 10100011 |
| Contents |  |
| Sequence of Point Codes, tagged as “PointCode” with the contents indicating the exact Point Code |  |

PointCodeList ::= SEQUENCE OF PointCode

##### 3.1.1.1.5	formIndicator

The formIndicator identifies the form of the SRVT message, i.e. either Request, Verify or Compare. lt is of type INTEGER, with the values defined as below.

| Parameter | Code |
| --- | --- |
| formIndicator | 10000100 |
| Contents |  |
| Value 0 = Compare |  |
| Value 1 = No Compare |  |

| FormIndicator ::= | INTEGER |
| --- | --- |
|  | { compare (0), |
|  | noCompare (l) } |

##### 3.1.1.1.6	mtpBackwardRoutingRequested

The mtpBackwardRoutingRequested identifies whether MTP backward routing towards the OPC is required for test success. It is of type BOOLEAN.

| Parameter | Code |
| --- | --- |
| mtpBackwardRoutingRequested | 10000101 |
| Contents |  |
| TRUE (= 1) Routing Requested |  |
| FALSE (= 0) Routing Not Requested |  |

##### 3.1.1.1.7	testInitiatorGT

The testInitiatorGT identifies the Global Title Indicator and the initiator’s Global Title. It is of type OCTET STRING.

| Parameter | Code |
| --- | --- |
| testInitiatorGT | 10000110 |
| Contents |  |
| Octet 1 = GlobalTitle Indicator |  |
| Octet 2, 3, ... = Initiator Global Title |  |

GlobalTitle ::= OCTET STRING

##### 3.1.1.1.8	destinationPC

The dcstinationPC identifies the Destination Point Code (PPC or TPC). It is of type PointCode.

| Parameter | Code |
| --- | --- |
| destinationPC | 10000111 |
| Contents |  |
| Bit 0 contains the first bit of the Point Code, |  |
| Bit 1 contains the second bit of the Point Code, etc. |  |

##### 3.1.1.1.9	destinationSSN

The destinationSSN identifies the destination Subsystem Number. It is of type OCTET STRING.

| Parameter | Code |
| --- | --- |
| destinationSSN | 10001000 |
| Contents |  |
| Bit 0 contains the first bit of the Subsystem Number, |  |
| Bit 1 contains the second bit of the Subsystem Number, etc. |  |

SubsystemNumber ::= OCTET STRING

##### 3.1.1.1.10	backupDPC

The backupDPC identifies the backup Destination Point Code (SPC). It is of type PointCode.

| Parameter | Code |
| --- | --- |
| backupDPC | 10001001 |
| Contents |  |
| Bit 0 contains the first bit of the Point Code, |  |
| Bit 1 contains the second bit of the Point Code, etc. |  |

##### 3.1.1.1.11	backupSSN

The backupSSN identifies the backup destination Subsystem Number. It is of type OCTET STRING.

| Parameter | Code |
| --- | --- |
| backupSSN | 10001010 |
| Contents |  |
| Bit 0 contains the first bit of the Subsystem Number, |  |
| Bit 1 contains the second bit of the Subsystem Number, etc. |  |

##### 3.1.1.1.12	originalGT

The originalGT field is only present in an SRVT message if translation of a GT in the called party address yields or has already yielded a replacement GT.

In this case, the field in an SRVT message to be sent out is as follows:

i)	if the SRVT message to be sent out is not the compare form, and the test is initiated by receipt of an SRVT message containing an originalGT, then the field is copied across; or

ii)	in all other cases, the originalGT sent out is the GT of the called party address for the SRVT message before translation.

The field is used as the GT of the calling party address in any SRVR message sent and, for the compare form of SRVT message, by the mate TSP receiving it to check that its translation yields the GT in the called party address field of the compare SRVT message received.

The type of originalGT is GlobalTitle.

| Parameter | Code |
| --- | --- |
| originalGT | 10001011 |
| Contents |  |
| Octet 1 = Global Title Indicator |  |
| Octet 2, 3,... = Original Global Title |  |

#### 3.1.1.2	Action Results

There are no contents in a successful return indication.

#### 3.1.1.3	Action Errors

SpecificErrors are possible errors which can occur during this test, and which are unique to this test. These specific errors are in addition to the errors already identified in the om-Action-Confirmed service and appear as parameters to the Processing Failure Error.

##### 3.1.1.3.1	failure

failure indicates a condition of failure, where a Translation could not be successfully done, or was incorrect. Most often this will be used as a failure indication from the point which detects the error and does not invoke any further testRoute Actions. The failure SpecificError has with it a parameter to indicate the error condition causing the failure. This parameter, failureType, is represented as a bit string. In addition, the second parameter is to be used when failureType indicates the error Unknown Initiating SP. traceSent indicates whether or not a routeTrace Event has been invoked to report trace information. It is necessary to indicate this for this error since the node detecting the error cannot send the routeTrace, thus the previous node must. traceSent is a type of BOOLEAN and is optional.

| Specific Error | Code |
| --- | --- |
| failure | 00000001 |
| Parameters | References |
| failureType | 3.1.1.3.1 |
| traceSent | 3.1.1.3.1 |

| Parameter | Code |
| --- | --- |
| failureType | 10000000 |
| Bit | Meaning |
| 0 | detectedLoop |
| 1 | excessiveLengthRoute |
| 2 | unknownObjectInstance |
| 3 | routeInaccessible |
| 4 | processingFailure |
| 5 | unknownInitiatingSP |
| 6 | timerExpired |
| 7 | wrongSP |
| 8 | incorrectTranslation-Primary |
| 9 | incorrectTranslation-Secondary |
| 10 | incorrectTranslation-Intermediate |
| 11 | notPrimaryDestination |
| 12 | notSecondaryDestination |
| 13 | notRecognizedPrimary |
| 14 | notRecognizedSecondary |
| 15 | routingProblem |

| Parameter | Code |
| --- | --- |
| traceSent | 10000001 |
| Contents | Meaning |
| TRUE | the trace information was sent |
| FALSE | the trace information was not sent |

| failure | SPECIFIC-ERROR |  |  |
| --- | --- | --- | --- |
|  | PARAMETER SEQUENCE | { failureType | [0] IMPLICIT FailureString, |
|  |  | traceSent | [l ] IMPLICIT BOOLEAN } |
|  | ::= 1 |  |  |

| FailureString ::= BIT STRING |  |
| --- | --- |
|  | { detectedLoop (0), |
|  | excessiveLengthRoute (1), |
|  | unknownObject (2), |
|  | routeInaccessible (3), |
|  | processingFailure (4), |
|  | unknownInitiatingSP (5), |
|  | timerExpired (6), |
|  | wrongSP (7), |
|  | incorrectTranslation-Primary (8), |
|  | incorrectTranslation-Secondary (9), |
|  | incorrectTranslation-Intermediate (10), |
|  | notPrimaryDestination (11), |
|  | notSecondaryDestination (12), |
|  | notRecognizedPrimary (13), |
|  | notRecognizedSecondary (14), |
|  | routingProblem (15) } |

##### 3.1.1.3.2	partialSuccess

This indication is given when at least one testRoute Cnf Action invocation failed and at least one succeeded (at least partially). In this case, each type of error that occurred will be noted and sent in the final reply. The format and contents of partial success are the same as failure.

| Specific Error | Code |
| --- | --- |
| partialSuccess | 00000010 |
| Parameters | References |
| failureType | 3.1.1.3.1 |
| traceSent | 3.1.1.3.1 |

| partialSuccess | SPECIFIC-ERROR |  |  |
| --- | --- | --- | --- |
|  | PARAMETER SEQUENCE | { failureType | [0] IMPLICIT FailureString, |
|  |  | traceSent | [1] IMPLICIT BOOLEAN } |
|  | ::= 2 |  |  |

### 3.1.2	routeTrace Event

The routeTrace Event reports trace information. Trace information consists of one or more Point Codes, such as the entire list of Translation Point Codes traversed along a route. This event is invoked either at the explicit request of the originating node (indicated by traceRequested, see 3.1.1.1.2) or by failure at any point along the route. This event is not confirmed, therefore no replies to this invocation are expected (no error or success indications are expected).

| routeTrace EVENT | Timer = 0 | Class = 4 | Code = 00000010 |
| --- | --- | --- | --- |
| EventInfo |  | Opt/Man (Note) | References |
| success |  | O | 3.1.2.1.1 |
| detectedLoop |  | O | 3.1.2.1.2 |
| excessiveLengthRoute |  | O | 3.1.2.1.3 |
| unknownObjectInstance |  | O | 3.1.2.1.4 |
| routeInaccessible |  | O | 3.1.2.1.5 |
| processingFailure |  | O | 3.1.2.1.6 |
| unknownInitiatingSP |  | O | 3.1.2.1.7 |
| timerExpired |  | O | 3.1.2.1.8 |
| wrongSP |  | O | 3.1.2.1.9 |
| incorrectTranslation-Primary |  | O | 3.1.2.1.10 |
| incorrectTranslation-Secondary |  | O | 3.1.2.1.11 |
| incorrectTranslation-Intermediate |  | O | 3.1.2.1.12 |
| notPrimaryDestination |  | O | 3.1.2.1.13 |
| notSecondaryDestination |  | O | 3.1.2.1.14 |
| notRecognizedPrimary |  | O | 3.1.2.1.15 |
| notRecognizedSecondary |  | O | 3.1.2.1.16 |
| routingProblem |  | O | 3.1.2.1.17 |
| Note – One and only one of these indications must be present. |  |  |  |

| routeTrace | EVENT |  |  |
| --- | --- | --- | --- |
|  | EVENTINFO CHOICE { |  |  |
|  | success |  | [0] IMPLICIT PointCodeList, |
|  | detectedLoop |  | [1] IMPLICIT PointCodeList, |
|  | excessiveLengthRoute |  | [2] IMPLICIT PointCodeList, |
|  | unknownObjectInstance |  | [3] IMPLICIT NULL, |
|  | routeInaccessible |  | [4] IMPLICIT PointCode, |
|  | processingFailure |  | [5] IMPLICIT NULL, |
|  | unknownInitiatingSP |  | [6] IMPLICIT PointCode, |
|  | timerExpired |  | [7] IMPLICIT PointCodeList, |
|  | wrongSP |  | [8] IMPLICIT PointCodeList, |
|  | incorrectTranslation-Primary |  | [9] IMPLICIT PointCodeList, |
|  | incorrectTranslation-Secondary |  | [10] IMPLICIT PointCodeList, |
|  | incorrectTranslation-Intermediate |  | [11] IMPLICIT PointCodeList, |
|  | notPrimaryDestination |  | [12] IMPLICIT PointCodeList, |
|  | notSecondaryDestination |  | [13] IMPLICIT PointCodeList, |
|  | notRecognizedPrimary |  | [14] IMPLICIT PointCodeList, |
|  | notRecognizedSecondary |  | [15] IMPLICIT PointCodeList, |
|  | routingProblem |  | [16] IMPLICIT PointCodeList } |
|  | ::= 2 |  |  |

#### 3.1.2.1	Event Information

##### 3.1.2.1.1	success

On successful completion, the trace of the Point Codes (one or more) of the crossed SCCP Relay Nodes are included.

| Parameter | Code |
| --- | --- |
| success | 10100000 |
| Contents | Reference |
| Sequence of one or more Point Codes, tagged as “PointCode”, with contents indicating the exact Point Code | 3.1.1.1.4 |

##### 3.1.2.1.2	detectedLoop

When a loop is detected, the Point Codes (three or more) contained in the loop are included.

| Parameter | Code |
| --- | --- |
| detectedLoop | 10100001 |
| Contents | Reference |
| Sequence of Point Codes, tagged as “PointCode”, with contents indicating the exact Point Code | 3.1.1.1.4 |

##### 3.1.2.1.3	excessiveLengthRoute

When an excessively long route is found (threshold exceeded), the entire route is included.

| Parameter | Code |
| --- | --- |
| excessiveLengthRoute | 10100010 |
| Contents | Reference |
| Sequence of Point Codes, tagged as “PointCode”, with contents indicating the exact Point Code | 3.1.1.1.4 |

##### 3.1.2.1.4	unknownObjectInstance

If the object is unknown, no additional information is required. For the SRVT, this refers to the case when no translation data exists for the GTI + GT.

| Parameter | Code |
| --- | --- |
| unknownObjectInstance | 10000011 |
| Contents | Reference |
| empty | – |

##### 3.1.2.1.5	routeInaccessible

The Point Code of the node where the route was inaccessible is included.

| Parameter | Code |
| --- | --- |
| routeInaccessible | 10000100 |
| Contents | Reference |
| Bit 0 contains the first bit of the Point Code, | 3.1.1.1.1 |
| Bit 1 contains the second bit of the Point Code, etc. |  |

##### 3.1.2.1.6	processingFailure

If a processing failure occurs, no additional information is required.

| Parameter | Code |
| --- | --- |
| processingFailure | 10000101 |
| Contents | Reference |
| empty | – |

##### 3.1.2.1.7	unknownInitiatingSP

The Point Code of the node detecting the unknown initiating Signalling Point is included.

| Parameter | Code |
| --- | --- |
| unknownInitiatingSP | 10000110 |
| Contents | Reference |
| Bit 0 contains the first bit of the Point Code, | 3.1.1.1.1 |
| Bit 1 contains the second bit of the Point Code, etc. |  |

##### 3.1.2.1.8	timerExpired

The Point Code(s) of the node(s) from where no result for the testRoute Action was received is included.

| Parameter | Code |
| --- | --- |
| timerExpired | 10100111 |
| Contents | Reference |
| Sequence of Point Codes, tagged as “PointCode”, with contents indicating the exact Point Code | 3.1.1.1.4 |

##### 3.1.2.1.9	wrongSP

The complete list of Translation SPs traversed in the route to the invalid SP is included.

| Parameter | Code |
| --- | --- |
| wrongSP | 10101000 |
| Contents | Reference |
| Sequence of Point Codes, tagged as “PointCode”, with contents indicating the exact Point Code | 3.1.1.1.4 |

##### 3.1.2.1.10	incorrectTranslation-Primary

The complete list of Translation SPs traversed in the route to the incorrect primary destination is included.

µ

| Parameter | Code |
| --- | --- |
| incorrectTranslation-Primary | 10101001 |
| Contents | Reference |
| Sequence of Point Codes, tagged as “PointCode”, with contents indicating the exact Point Code | 3.1.1.1.4 |

§

##### 3.1.2.1.11	incorrectTranslation-Secondary

The complete list of Translation SPs traversed in the route to the incorrect secondary destination is included.

| Parameter | Code |
| --- | --- |
| incorrectTranslation-Secondary | 10101010 |
| Contents | Reference |
| Sequence of Point Codes, tagged as “PointCode”, with contents indicating the exact Point Code | 3.1.1.1.4 |

##### 3.1.2.1.12	incorrectTranslation-Intermediate

The complete list of Translation SPs traversed in the route to the incorrect intermediate point is included.

| Parameter | Code |
| --- | --- |
| incorrectTranslation-Intermediate | 10101011 |
| Contents | Reference |
| Sequence of Point Codes, tagged as “PointCode”, with contents indicating the exact Point Code | 3.1.1.1.4 |

##### 3.1.2.1.13	notPrimaryDestination

The complete list of Translation SPs traversed in the route to the invalid primary destination is included.

| Parameter | Code |
| --- | --- |
| notPrimaryDestination | 10101100 |
| Contents | Reference |
| Sequence of Point Codes, tagged as “PointCode”, with contents indicating the exact Point Code | 3.1.1.1.4 |

##### 3.1.2.1.14	notSecondaryDestination

The complete list of Translation SPs traversed in the route to the invalid secondary destination is included.

| Parameter | Code |
| --- | --- |
| notSecondaryDestination | 10101101 |
| Contents | Reference |
| Sequence of Point Codes, tagged as “PointCode”, with contents indicating the exact Point Code | 3.1.1.1.4 |

##### 3.1.2.1.15	notRecognizedPrimary

The complete list of Translation SPs traversed in the route to the secondary destination is included.

| Parameter | Code |
| --- | --- |
| notSecondaryDestination | 10101110 |
| Contents | References |
| Sequence of Point Codes, Tagged as “PointCode”, with contents indicating the exact Point Code | 3.1.1.1.4 |

##### 3.1.2.1.16	notRecognizedSecondary

The complete list of Translation SPs traversed in route to the primary destination is included.

| Parameter | Code |
| --- | --- |
| notRecognizedSecondary | 10101111 |
| Contents | Reference |
| Sequence of Point Codes, tagged as “PointCode”, with contents indicating the exact Point Code | 3.1.1.1.4 |

##### 3.1.2.1.17	routingProblem

The complete list of Translation SPs traversed in the route to the possible routing problem. This occurs when the Point Code from the translation is not recognized.

| Parameter | Code |
| --- | --- |
| routingProblem | 10110000 |
| Contents | Reference |
| Sequence of Point Codes, tagged as “PointCode”, with contents indicating the exact Point Code | 3.1.1.1.4 |

# 4	Circuit managementXE "Circuit management"§

## 4.1	Circuit Validation Test (CVT) ASEXE "Circuit Validation Test (CVT) ASE"§

The Circuit Validation Test ASE provides the services accessed via om-Action-Confirmed as described in Figure 3. lt uses an instance based on a Circuit Management Object Class defined in Recommendation Q.751. The BaseManagedObjectClass indicates cvt-Cic-Tables-1992, and the BaseManagedObjectInstance identifies the CktGrpInfo (circuit group information identifying the pre-defined identifier for the circuit and its group agreed between the exchanges at the circuit group’s ends) and its CIC known in the sending SP.

### 4.1.1	cktValidTest CnfAction

The Circuit Validation Test Request and subsequent return of the Circuit Validation Response is mapped into a confirmed action. The action is the request for the far-end test.

| cktValidTest CNF-ACTION | Timer = Tc | Class = 1 | Code = 00000001 |
| --- | --- | --- | --- |
| ActionArg |  | Opt/Man | References |
| requestingSP |  | M | 4.1.1 |
| timer |  | O | 4.1.1 |
| ActionResult |  |  | Reference |
| success |  |  | 4.1.1 |
| Linked Operations |  |  |  |
| N/A |  |  |  |
| Specific Errors |  |  |  |
| failure |  |  |  |

See 4.2/Q.753 for the possible values of Tc and timer.

| cktValidTest | CNF-ACTION |  |  |
| --- | --- | --- | --- |
| ACTIONARG SEQUENCE { |  |  |  |
|  | requestingSP | RequestingSP, |  |
|  | timer | Timer OPTIONAL } |  |
|  | ACTIONRESULT |  | Success |
|  | SPECIFICERRORS |  | { failure } |
|  | ::= 3 |  |  |

### 4.1.2	Action Arguments

The requesting SP is the Point Code of the signalling point initiating the test procedure. It is of type Octet String as stated below.

RequestingSP ::= OCTET STRING

### 4.1.3	Action Results

The Action Results are returned in a return result component upon success.  The contents of the two parameters are to be defined based on the CVT procedure.

| Success | ::= SEQUENCE { |  |
| --- | --- | --- |
|  | cktGrpInfo | [0] IMPLICIT CktGrpInfo, |
|  | cICName | [1] IMPLICIT OCTET STRING OPTIONAL |
|  | } |  |

Note that CktGrpInfo is defined as OCTET STRING.

### 4.1.4	Specific Error

The specific error indicates the failure and reason for failure.  The contents of the two parameters are to be defined based on the CVT procedure.

| failure | SPECIFIC-ERROR |
| --- | --- |
|  | PARAMETER SEQUENCE { |
|  | cktGrpInfo [0] IMPLICIT CktGrpInfo, |
|  | cICName [1] IMPLICIT OCTET STRING OPTIONAL |
|  | } |

Note that CktGrpInfo is defined as OCTET STRING.

The CVT failure reasons are:

a)	CIC not assigned at near end;

b)	wrong near end data for circuit;

c)	valid tone not received at near end;

d)	overall test timer Tc expired before CVR received;

e)	CVR message received before synchronization achieved in bit pattern test;

f)	Tc expired before synchronization is achieved in bit pattern test;

g)	bit pattern still being received when Tc expires;

h)	bit pattern still being received when CVR message is received;

i)	tone being received when Tc expires;

j)	tone being received when CVR message is received;

k)	near end CIC and far end CIC do not match (near end check on receipt of CVR message);

l)	CVR message received indicating failure:

–	CIC not assigned at far end;

–	wrong far end data for circuit;

–	group characteristics unavailable at far end;

m)	failure – unspecified.

# 5	Transaction capabilities

For further study.

# 6	General definitions

## 6.1	Objects and operations

OMAP runs tests on objects such as the MTP and SCCP routing tables. These objects are described here as “Object Classes” and are identified by an object identifier which specifies this Recommendation and the type of object. This structure is shown below for the OMAP object identifiers mtp-Routing-Tables, sccp-Routing-Tables, and cvt-Cic-Tables.

| oMAP | OBJECT IDENTIFIER ::= { ccitt recommendation q 754 } |
| --- | --- |
| mtp-Routing-Tables-1992 | OBJECT IDENTIFIER ::= { oMAP 0 } |
| sccp-Routing-Tables-1992 | OBJECT IDENTIFIER ::= { oMAP 1 } |
| cvt-Cic-Tables-1992 | OBJECT IDENTIFIER ::= { oMAP 5 } |

The Object Class of MTP Routing Tables is 0011857200 (hexadecimal), for SCCP Routing Tables is 0011857201 (hexadecimal), and for CVT CIC Tables is 0011857205 (hexadecimal). See Annex C/X.208 and 22/X.209.

Tables 1 and 2 show the OM-primitives, Figure 1 shows the OMAP operations derived from CMIP (IS 9596), and Figure 3 contains a formal syntax of OMASE.

The operations derived from CMIP are listed below.

Currently Defined Operations

0 eventReport

7 confirmedAction

## 6.2	Primitives and procedures of the OMASE protocolXE "Primitives andprocedures of the OMASE protocol"§

### 6.2.1	General

The OMASE protocol uses the TC-service as defined in Recommendation Q.771. The Invoke ID and the Dialogue ID correspond with those defined for the TC-service.

OMASE is modelled using a Protocol Machine (referred-to as OMPM below). The abbreviation APDU stands for Application Protocol Data Unit in what follows, it refers to the contents of the primitive(s) passed between OMASE and TC.

Figure A.1 shows the model including TC and SCCP, the OMPM resides in OMASE. Figure A.2 shows an example of particular instances of the primitives in an MRV Test (but without OM-EVENT-REPORT).

### 6.2.2	OM-EVENT-REPORT

#### 6.2.2.1	Service primitive

The OM-EVENT-REPORT primitive used between the OMASE-User and OMASE is defined in Table 1.

The specific event that occurred is interpreted in the context of the object class specified.

TABLE 1/Q.754

OM-EVENT-REPORT Parameters

| Parameter Name | Req/Ind |
| --- | --- |
| CallingPartyAddress | M |
| CalledPartyAddress | M |
| DialogueID | M |
| InvokelD | M |
| ManagedObjectClass | M |
| ManagedObjectInstance | M |
| EventType | M |
| EventTime | O |
| EventInfo | O |

Parameter definitions

CallingPartyAddress: As defined in the calling address of 2.2/Q.711.

CalledPartyAddress: As defined in the called address of 2.2/Q.711. The above addresses serve to identify OMAP at the calling and called SP respectively. For MRVT they can both be in the form of point code plus (OMAP) subsystem number, for SRVT they are in a form suitable for the type of SCCP routing applied in the test.

DialogueID: As defined in Recommendations Q.771-Q.775. It maps to Transaction ID. defined in Recommen-dation Q.772.

InvokeID: As defined in Recommendation Q.772.

ManagedObjectClass: Identifies the class of objects for which this event is defined.

ManagedObjectInstance: Identifies the object instance on which the event is reported.

EventType: Specifies the particular event that is being reported by the object instance.

EventTime: Specifies the time at which the event was generated.

EventInfo: Provides additional event specific information.

#### 6.2.2.2	Event reporting procedure

##### 6.2.2.2.1 Receipt of OM-EVENT-REPORT request

The event reporting procedure is initiated by receipt of the OM-EVENT-REPORT request primitive. When this occurs, the OMPM constructs the APDU requesting the eventReport operation, and transmits the APDU using the TC-INVOKE and TC-BEGIN service.

The TC-INVOKE request primitive contains the following parameters and values:

·	Dialogue ID – Defined by the OMASE-User.

·	Invoke ID – Defined by the OMASE-User.

·	Operation – Set to eventReport.

·	Class – Set to 4.

·	Parameters – Those following the word “PARAMETER” in the definition of eventReport. The value of the parameter eventType specifies which action is to be performed – it should indicate routeTrace for the procedures defined at present.

·	Timeout – Set to 0 for both MRVT and SRVT.

The TC-BEGIN request primitive uses the following parameters and values:

·	Destination address – As received in the OM-EVENT-REPORT request primitive CalledPartyAddress.

·	Originating address – As received in the OM-EVENT-REPORT request primitive CallingPartyAddress.

·	Dialogue ID – As in the TC-INVOKE.

The N-UNITDATA request primitive ultimately issued to the SCCP due to the receipt of these TC request primitives should contain the Sequence control parameter set to indicate “sequence guaranteed”, and the Return option parameter should be set to indicate “discard message on error”. See 2.2.2/Q.711.

After transmission of the APDU, the OMPM terminates the dialogue using the TC-END request primitive with parameters Dialogue ID and Termination, the latter indicating “prearranged end”.

##### 6.2.2.2.2	Receipt of TC-BEGIN with TC-INVOKE indication

On receipt of a well-formed APDU requesting the eventReport operation from the TC-BEGIN and TC-INVOKE indication primitives, the OMPM issues an OM-EVENT-REPORT indication primitive. If the APDU is not well-formed, the OMPM discards it.

The OMPM terminates the dialogue with a TC-END request primitive with parameters Dialogue ID and Termination, the latter indicates “prearranged end”.

##### 6.2.2.2.3	Receipt of TC-BEGIN with TC-L-REJECT indication

In this case, the OMPM issues a TC-END request primitive with parameters Dialogue ID and Termination, the latter indicating “prearranged end”.

##### 6.2.2.2.4	Receipt of TC-P-ABORT indication

In this case, the OMPM ignores the TC-P-ABORT.

### 6.2.3	OM-CONFIRMED-ACTION

#### 6.2.3.1	Service primitive

The OM-CONFIRMED-ACTION service is shown in Table 2. The specific action to be performed is interpreted in the context of the object class specified.  This service is a confirmed service (a report of success or failure is always sent).

TABLE 2/Q.754

OM-CONFIRMED-ACTION Service

| Parameter Name | Req/Ind | Res/Con |
| --- | --- | --- |
| CallingPartyAddress | M | M |
| CalledPartyAddress | M | M |
| DialogueID | M | M |
| InvokeID | M | M |
| AccessControl | O | – |
| BaseManagedObjectClass | M | – |
| BaseManagedObjectInstance | M | – |
| ActionInfo | M | – |
| ActionResult | – | Ma) |
| ActionError | – | Mb) |
| Timer | Mc) | – |
| a) Mandatory in Return Result component (may be empty). b) Mandatory in Return Error component. c) This parameter is only in the request primitive. |  |  |

Parameter definitions

CallingPartyAddress: See Table 1.

CalledPartyAddress: See Table 1.

DialogueID: Mapped by TCAP into transaction ID as defined in Recommendation Q.772.

InvokeID: As defined in Recommendation Q.772.

AccessControl: Information to be used as input to access control functions.

BaseManagedObjectClass: Identifies the class of objects for which this action is defined.

BaseManagedObjectInstance: Identifies the object instance on which the action is to be performed.

ActionInfo: Is a sequence of ActionType and (optional) ActionInfoArg. ActionType is defined by the CNF-ACTION macro, and specifies a particular action that is to be performed on the object instance. ActionInfoArg contains the parameters for the action to be performed.

ActionResult: This field contains the result of the successful action performed, as appropriate.

ActionError: This field indicates error or problem status information if the action did not successfully complete.

Timer: This parameter contains the particular value for the timeout period waiting for the response. It is set to T1 for MRVT, T2 for SRVT, or Tc for CVT.

The value is given in Recommendation Q.753.

#### 6.2.3.2	Procedures for confirmed action

##### 6.2.3.2.1	Receipt of OM-CONFIRMED-ACTION request

The confirmedAction procedure is initiated by the receipt of the OM-CONFIRMED-ACTION request primitive. In this case, the OMPM constructs an APDU requesting the confirmedAction operation, and transmits the APDU using the TC-INVOKE and TC-BEGIN service.

The TC-INVOKE request primitive contains the following parameters and values:

·	Operation – Takes the value of confirmedAction.

·	Class – Value is 1.

·	Parameters – Corresponds with the parameters of confirmedAction as defined after the keyword “PARAMETER” of the operation definition. The value “testRoute” is obtained by derivation from CNF-ACTION of the localForm of ActionTypeID from ActionType of ActionInfo.

·	Timeout – Is copied from the parameter “Timer” in the OM-CONFIRMED-ACTION request.

·	The Invoke ID and Dialogue ID are copied from the OM-CONFIRMED-ACTION request.

The TC-BEGIN request primitive uses the following parameters and values:

·	Dialogue ID – As in the TC-INVOKE.

·	Destination address – The CalledPartyAddress of the OM-CONFIRMED-ACTION request.

·	Originating address – The CallingPartyAddress of the OM-CONFIRMED-ACTION request.

The N-UNITDATA request primitive ultimately issued to the SCCP due to the receipt of these TC request primitives should contain the Sequence control parameter set to indicate “sequence not guaranteed”, and the Return option parameter should be set to indicate “return message on error”. See 2.2.2/Q.711.

##### 6.2.3.2.2	Receipt of TC-BEGIN with TC-INVOKE indication

In this case, if the APDU is well-formed and requests the confirmedAction operation, the OMPM issues an OM-CONFIRMED-ACTION indication primitive to the OMASE-User.

If the APDU is not well formed, the OMPM ignores the TC indications.

An implementation-dependent mechanism allied to that defined in 3.3.4/Q.774 should cater for any local problems.

If the APDU contains extra parameters, they are ignored by the OMPM.

##### 6.2.3.2.3	Receipt of OM-CONFIRMED-ACTION response

The OM-CONFIRMED-ACTION response primitive can contain either the ActionResult parameter, or the ActionError parameter.

The ActionResult parameter indicates that the execution of the operation was successful, and the OMPM issues a TC-RESULT-L request primitive.  If the test was a CVT, the following parameters are included in the TC-RESULT-L:

·	Operation – Has the value of confirmedAction.

·	Parameters – Corresponds to the parameter Success of ACTIONRESULT for the CVT.

The presence of the ActionError parameter indicates that the operation was unsuccessful, and the OMPM issues a TC-U-ERROR request primitive with the following parameters:

·	Error – Takes the appropriate error value from the set defined after the word “ERRORS” of the operation definition.

·	Parameters – Corresponds with the parameters defined after the word “PARAMETER” of the definition of the error.

The result of the operation is transmitted by the OMPM issuing a TC-END request with parameters Dialogue ID and Termination, the latter indicating “basic end”.

The N-UNITDATA request primitive ultimately issued to the SCCP due to the receipt of these TC request primitives should contain the Sequence control parameter set to indicate “sequence guaranteed”, and the Return option parameter should be set to indicate “discard message on error”. See 2.2.2/Q.711.

##### 6.2.3.2.4	Receipt of TC-END with TC-RESULT-L indication

In this case, the OMPM if the APDU is well-formed, issues an OM-CONFIRMED-ACTION confirm primitive with parameter ActionResult to the OMASE-User (including the Dialogue ID).

If the APDU is not well formed, the OMPM ignores the TC primitives3).

##### 6.2.3.2.5	Receipt of TC-END with TC-U-ERROR indication

If the APDU is well-formed, the OMPM issues an OM-CONFIRMED-ACTION confirm primitive with parameter ActionError (and Dialogue ID) to the OMASE-User.

If the APDU is not well formed, the OMPM ignores the TC primitives3).

##### 6.2.3.2.6	Receipt of TC-L-CANCEL indication

This occurs if the invocation timer expires.

In this case, the OMPM issues an OM-CONFIRMED-ACTION confirm primitive, with the specific error “failure” for CNF-ACTION, and if the operation invoked was testRoute, the parameter failureType indicates timerExpired.

The OMPM terminates the dialogue with a TC-END request primitive, with the Termination parameter indicating “prearranged end”.

##### 6.2.3.2.7	Receipt of TC-BEGIN or TC-END with TC-L-REJECT indications

Receipt of TC-BEGIN with TC-L-REJECT indication is illustrated in Figure 2a.

FIGURE 2a/Q.754...[D01] = 3 CM

If the OMPM receives a TC-L-REJECT indication with a TC-BEGIN indication, the OMPM terminates the dialogue by issuing a TC-END request primitive with the Termination parameter indicating “prearranged end”.

If the OMPM receives a TC-L-REJECT indication with a TC-END indication, it issues an OM-CONFIRMED-ACTION confirm primitive with the specific error “failure” of CNF-ACTION, and if testRoute was invoked, the parameter failureType in the confirm primitive indicates “routeInaccessible”. This is illustrated in the Figure 2b.

##### 6.2.3.2.8	Receipt of TC-END with TC-R-REJECT indication

In this case, the OMPM issues an OM-CONFIRMED-ACTION confirm primitive with the specific error “failure” of CNF-ACTION, and if testRoute was invoked, the parameter failureType in the confirm primitive indicates “routeInaccessible”.

##### 6.2.3.2.9	Receipt of TC-P-ABORT indication

This is illustrated by the two diagrams of Figure 2c.

FIGURE 2b/Q.754...[D02] = 3 CM

FIGURE 2c/Q.754...[D03] = 3 CM

In this case, the OMPM issues an OM-CONFIRMED-ACTION confirm primitive with the specific error “failure” of CNF-ACTION, and if testRoute was invoked, the parameter failureType in the confirm primitive indicates “routeInaccessible”.

##### 6.2.3.2.10	Receipt of TC-NOTICE

In this case, the OMPM issues an OM-CONFIRMED-ACTION confirm primitive with the specific error “failure” of CNF-ACTION, and if testRoute was invoked, the parameter failureType in the confirm primitive indicates “routeInaccessible”.

Error definitions

A number of errors have been referred-to in the definition of the two OM-services. These errors are defined in this subclause.

Definitions

noSuchObjectClass: The object class in the Invoke APDU is not recognised by the receiving end.

noSuchObjectInstance: While the object class in the Invoke APDU is recognised, there is no corresponding object instance of that class at the receiving end.

accessDenied: Access to the resource is not allowed.

processingFailure: A failure occurred while processing a specific action or event. The failure indicators are action or event specific.

noSuchAction: The action type is not supported by or known to the receiving end.

noSuchArgument: The argument specified is not known to or supported by the receiving end.

invalidArgumentValue: The argument value is not appropriate for the receiving end.

## 6.3	Abstract syntax of the OMASE protocolXE "Abstract syntax of the OMASE protocol"§

-- OMASE  protocol

OMASE { ccitt recommendation q 754 omase(0) version1(1) }

DEFINITIONS ::=

BEGIN

-- TCAP definitions

EXPORTS EVENT, CNF-ACTION, SPECIFIC-ERROR;

IMPORTS OPERATION, ERROR FROM TCAPMessage { ccitt recommendation q 773 modules(0) messages(1)

version2(2) };

-- OMASE operations

eventReport OPERATION

PARAMETER SEQUENCE {

managedObjectClass	ObjectClass,

managedObjectInstance	Objectlnstance,

eventTime			[5] IMPLICIT GeneralizedTime OPTIONAL,

eventType			[7] IMPLICIT EVENT,

eventlnfo			[8] ANY DEFINED BY eventType OPTIONAL }

::= localValue 0

confirmedAction OPERATION

PARAMETER SEQUENCE {

COMPONENTS OF	BaseManagedObjectld,

accessControl		[5] AccessControl OPTIONAL,

actionlnfo		[12] IMPLICIT ActionInfo }

RESULT	actionResult	ActionResult

ERRORS	{ accessDenied, invalidArgumentValue,

noSuchAction, noSuchArgument,

noSuchObjectClass, noSuchObjectlnstance,

processingFailure }

::= localValue 7

FIGURE 3/Q.754 (sheet 1 of 5)

Formal syntax of OMASE services

-- The om-service error definitions are as follows:

noSuchObjectClass			ERROR

PARAMETER					ObjectClass

::= localValue 0

noSuchObjectlnstance			ERROR

PARAMETER					ObjectInstance

::= localValue 1

accessDenied				ERROR

::= localValue 2

noSuchAction				ERROR

PARAMETER					NoSuchAction

::= localValue 9

processingFailure	ERROR

PARAMETER					ProcessingFailure -- optional

::=  localValue 10

noSuchArgument				ERROR

PARAMETER					NoSuchArgument

::= localValue 14

invalidArgumentValue			ERROR

PARAMETER					InvalidArgumentValue

::= localValue 15

FIGURE 3/Q.754 (sheet 2 of 5)

Formal syntax of OMASE services

-- The following gives the supporting type definitions:

ActionArgument			::= SEQUENCE	{ COMPONENTS OF		BaseManagedObjectld,

accessControl			[5] AccessControl OPTIONAL,

actionlnfo			[12] IMPLICIT ActionInfo }

Actionlnfo				::= SEQUENCE	{ actionType			ActionTypeld,

actionlnfoArg			[4] ANY DEFINED BY actionType

OPTIONAL }

ActionResult				::= SEQUENCE	{ managedObjectClass		ObjectClass

OPTIONAL,

managedObjectInstance		ObjectInstance OPTIONAL,

currentTime			[5] IMPLICIT GeneralizedTime

OPTIONAL,

actionReply			[6] IMPLICIT ActionReply

OPTIONAL }

ActionTypeld				::= CHOICE	{

-- globalForm ...

localForm			[3] IMPLICIT CNF-ACTION }

BaseManagedObjectId		::= SEQUENCE	{ baseManagedObjectClass	ObjectClass,

baseManagedObjectlnstance	Objectlnstance }

EventReportArgument		::= SEQUENCE	{ managedObjectClass		ObjectClass,

managedObjectlnstance		Objectlnstance,

eventTime			[5] IMPLICIT GeneralizedTime

OPTIONAL,

eventType			EventTypeld,

eventInfo			[8] ANY DEFINED BY eventType

OPTIONAL }

EventTypeId				::= CHOICE	{

-- globalForm ...

localForm			[7] IMPLICIT EVENT }

FIGURE 3/Q.754 (sheet 3 of 5)

Formal syntax of OMASE services

InvalidArgumentValue		::= CHOICE	{ actionValue			[0] IMPLICIT ActionInfo,

eventValue			[1] IMPLICIT SEQUENCE {

eventType			EventTypeld,

eventlnfo			[8] ANY DEFINED BY eventType

OPTIONAL }}

NoSuchAction			::= SEQUENCE	{ managedObjectClass		ObjectClass,

actionType			ActionTypeld }

NoSuchArgument			::= CHOICE	{ actionld			0] IMPLICIT SEQUENCE {

managedObjectClass		ObjectClass OPTIONAL,

actionType			ActionTypeId },

eventld				[1] IMPLICIT SEQUENCE {

managedObjectClass		ObjectClass OPTIONAL,

eventType			EventTypeID }}

ObjectClass				::= CHOICE	{ globalForm			[0] IMPLICIT

OBJECT IDENTIFIER,

-- ...

}

Objectlnstance			::= CHOICE	{

-- ...

nonSpecificForm		[3] IMPLICIT OCTET STRING,

-- ...

}

ProcessingFailure			::= SEQUENCE	{ managedObjectClass		ObjectClass

OPTIONAL,

managedObjectlnstance		Objectlnstance OPTIONAL,

specificErrorlnfo		[5] IMPLICIT SpecificErrorlnfo }

SpecificError				::= INTEGER	-- defined by object class

SpecificErrorInfo			::= SEQUENCE	{ errorType			[0] IMPLICIT SpecificError,

errorParm			[1] ANY DEFINED BY errorType

OPTIONAL }

Timer					::= INTEGER	-- seconds

FIGURE 3/Q.754 (sheet 4 of 5)

Formal syntax of OMASE services

-- Specific event reports are categorised by object class. The protocol uses may be described

-- by the EVENT MACRO below.

EVENT MACRO ::=

BEGIN

TYPE NOTATION		::=	Eventlnfo

VALUE NOTATION		::=	value(VALUE INTEGER)

EventInfo			::=	"EVENTINFO" NamedType½empty

NamedType		::=	identifier type½type

END

-- Specific Actions are categorised by object class. The protocol uses may be described

-- by the CNF-ACTION macro below.

CNF-ACTION MACRO ::=

BEGIN

TYPE NOTATION		::=	ActionArg ActionResult SpecificErrors

VALUE NOTATION		::=	value(VALUE INTEGER)

ActionArg			::=	"ACTIONARG" NamedType½empty

ActionResult		::=	"ACTIONRESULT" NamedType½empty

SpecificErrors		::=	"SPECIFICERRORS" "{" SpecificErrorList"}"½empty

NamedType		::=	identifier type½type

SpecificErrorList		::=	SpecificError½SpecificErrorList","SpecificError

SpecificError		::=	value(SPECIFIC-ERROR)

END

-- Errors that are action or event specific are defined using the SPECIFIC-ERROR macro below.

SPECIFIC-ERROR MACRO ::=

BEGIN

TYPE NOTATION		::=	ProcessingErrorParm

VALUE NOTATION		::=	value(VALUE INTEGER)

ProcessingErrorParm		::=	«PARAMETER» NamedType½empty

NamedType			::=	identifier type½type

END

END -- protocole OMASE

FIGURE 3/Q.754 (sheet 5 of 5)

Formal syntax of OMASE services

Annex  A

(This annex forms an integral part of this Recommendation)

FIGURE A.1/Q.754...[D04] = 3 CM

Figure A.2 illustrates the use of the primitives in an MRV Test. The OMASE-User at the origin, on receiving a “sendMRVT” request from the Management Process (MP – see Recommendation Q.753 for the model used), constructs an OM-CONFIRMED-ACTION request. The sequence is then as shown by the primitive and message sequence, up to number 5. At this point, if the node is not the tested destination, the OMASE-User receiving the OM-CONFIRMED-ACTION indication requests OM-CONFIRMED-ACTION of OMASE, to send out MRVT messages on all routes to the tested destination in the routing table. When all MRVA messages are received (seen in the OMASE-User as OM-CONFIRMED-ACTION confirm primitives), the OMASE-User issues the OM-CONFIRMED-ACTION response primitive as at 6.

FIGURE A.2/Q.754...[D05] = 3 CM

FIGURE A.3/Q.754

|  | Field Name |  |  |  |  |  |  |  | Bit Encoding | Reference/Explanation |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Message Type Tag |  |  |  |  |  |  |  | 01100010 | = Begin (Table 8/Q.773) |
|  | Message Length |  |  |  |  |  |  |  | 00110010 | 50 octets following TC part |
|  |  | Transaction ID Tag |  |  |  |  |  |  | 01001000 | = Originating (Table 10/Q.773) |
|  |  | Length |  |  |  |  |  |  | 00000100 | 4 octets |
|  |  | Transaction ID Value |  |  |  |  |  |  | xxxxxxxx | TCAP based on a dialogue |
|  |  |  |  |  |  |  |  |  | xxxxxxxx | at the user level |
|  |  |  |  |  |  |  |  |  | xxxxxxxx |  |
|  |  |  |  |  |  |  |  |  | xxxxxxxx |  |
|  |  | Component Portion Tag |  |  |  |  |  |  | 01101100 | (Table 14/Q.773) |
|  |  | Length |  |  |  |  |  |  | 00101010 | All 42 octets below here |
|  |  |  | Component Type Tag |  |  |  |  |  | 10100001 | = Invoke (Table 19/Q.773) |
|  |  |  | Length |  |  |  |  |  | 00101000 | All 40 octets below here |
|  |  |  |  | Component ID Tag |  |  |  |  | 00000010 | = Invoke ID (Table 20/Q.773) |
|  |  |  |  | Length |  |  |  |  | 00000001 | 1 octet |
|  |  |  |  | Invoke ID Value |  |  |  |  | xxxxxxxx | OMAP PROVIDED |
|  |  |  |  | Operation Code Tag |  |  |  |  | 00000010 | = Local (Table 22/Q.773) |
|  |  |  |  | Length |  |  |  |  | 00000001 | 1 octet |
|  |  |  |  | Operation Code |  |  |  |  | 00000111 | = Confirmed Action (Figure 3/Q.754) |
|  |  |  |  | Parameter Sequence Tag |  |  |  |  | 00110000 | = Sequence Tag (Table 23/Q.773) |
|  |  |  |  | Length |  |  |  |  | 00100000 | All 32 octets below here |
|  |  |  |  |  | Object Class Tag |  |  |  | 10000000 | globalForm X.711 & X.209 |
|  |  |  |  |  | Length |  |  |  | 00000101 | 5 octets |
|  |  |  |  |  | Value-MTP Routing Tables |  |  |  | 00000000 | CCITT, Rec. |
|  |  |  |  |  |  |  |  |  | 00010001 | q |
|  |  |  |  |  |  |  |  |  | 10000101 | 85 => 754 |
|  |  |  |  |  |  |  |  |  | 01110010 | 72 => |
|  |  |  |  |  |  |  |  |  | 00000000 | MTP Routing Tables 1992 |
|  |  |  |  |  | Object Instance Tag |  |  |  | 10000011 | nonSpecificForm X.711 & X.209 |
|  |  |  |  |  | Length |  |  |  | 00000010 | 2 octets |
|  |  |  |  |  | Object Instance Value |  |  |  | xxxxxxxx | (OMAP) Tested destination |
|  |  |  |  |  |  |  |  |  | xxxxxxxx |  |
|  |  |  |  |  | Action Info Tag |  |  |  | 10101100 | X.711 & X.209 |
|  |  |  |  |  | Length |  |  |  | 00010011 | All 19 octets below here |
|  |  |  |  |  |  | Action Type Tag |  |  | 10000011 | localForm X.711 & X.209 |
|  |  |  |  |  |  | Length |  |  | 00000001 | 1 octet |
|  |  |  |  |  |  | CNF-ACTION |  |  | 00000001 | = testRoute (8.1.1/Q.754) |
|  |  |  |  |  |  | Action Info Arg Tag |  |  | 10100100 | X.711 & X.209 |
|  |  |  |  |  |  | Length |  |  | 00001110 | All 14 octets below here |
|  |  |  |  |  |  |  | Parameter Seq. Tag |  | 00110000 | = Sequence Tag (Table 23/Q.773) |
|  |  |  |  |  |  |  | Length |  | 00001100 | All 12 octets below here |
|  |  |  |  |  |  |  |  | Initiating SP Tag | 10000000 | 8.1.1.1.1/Q.754, X.209 |
|  |  |  |  |  |  |  |  | Length | 00000010 | 2 octets |
|  |  |  |  |  |  |  |  | Initiating SP Value | xxxxxxxx | (OMAP) test initiator |
|  |  |  |  |  |  |  |  |  | xxxxxxxx |  |
|  |  |  |  |  |  |  |  | Trace Req. Tag | 10000001 | 8.1.1.1.2/Q.754, X.209 |
|  |  |  |  |  |  |  |  | Length | 00000001 | 1 octet |
|  |  |  |  |  |  |  |  | Value | 00000001 | = TRUE |
|  |  |  |  |  |  |  |  | Threshold Tag | 10000010 | = threshold (8.1.1.1.3/Q.754) |
|  |  |  |  |  |  |  |  | Length | 00000001 | 1 octet |
|  |  |  |  |  |  |  |  | Value of threshold | xxxxxxxx | OMAP PROVIDED |
|  |  |  |  |  |  |  |  | Point Code Trav. Tag | 10100011 | 8.1.1.1.4/Q.754 |
|  |  |  |  |  |  |  |  | Length | 00000000 | empty Point Code list |

Example of an MRVT message delivered to the SCCP

FIGURE A.4/Q.754

|  | Field Name |  |  |  | Bit Encoding | Reference/Explanation |
| --- | --- | --- | --- | --- | --- | --- |
|  | Message Type Tag |  |  |  | 01100100 | = END (Table 8/Q.773) |
|  | Message Length |  |  |  | 00001101 | 13 octets following in TC part |
|  |  | Transaction ID Tag |  |  | 01001001 | = Destination (Table 10/Q.773) |
|  |  | Length |  |  | 00000100 | 4 octets |
|  |  | Transaction ID Value |  |  | xxxxxxxx | Same as in BEGIN (MRVT msg) |
|  |  |  |  |  | xxxxxxxx |  |
|  |  |  |  |  | xxxxxxxx |  |
|  |  |  |  |  | xxxxxxxx |  |
|  |  | Component Portion Tag |  |  | 01101100 | (Table 14/Q.773) |
|  |  | Length |  |  | 00000101 | All 5 octets below here |
|  |  |  | Component Type Tag |  | 10100010 | = Ret. Res.(L) (Table 19/Q.773) |
|  |  |  | Length |  | 00000011 | All 3 octets below here |
|  |  |  |  | Component ID Tag | 00000010 | = Invoke ID (Table 20/Q.773) |
|  |  |  |  | Length | 00000001 | 1 octet |
|  |  |  |  | Invoke ID Value | xxxxxxxx | Same as MRVT message (Correlation) |

Example of an MRVA (success) message delivered to the SCCP

FIGURE A.5/Q.754

|  | Field Name |  |  |  |  |  |  |  | Bit Encoding | Reference/Explanation |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Message Type Tag |  |  |  |  |  |  |  | 01100100 | = END (Table 8/Q.773) |
|  | Message Length |  |  |  |  |  |  |  | 00100000 | 32 octets following in TC part |
|  |  | Transaction ID Tag |  |  |  |  |  |  | 01001001 | = Destination (Table 10/Q.773) |
|  |  | Length |  |  |  |  |  |  | 00000100 | 4 octets |
|  |  | Transaction ID Value |  |  |  |  |  |  | xxxxxxxx | Same as in BEGIN (MRVT msg) |
|  |  |  |  |  |  |  |  |  | xxxxxxxx |  |
|  |  |  |  |  |  |  |  |  | xxxxxxxx |  |
|  |  |  |  |  |  |  |  |  | xxxxxxxx |  |
|  |  | Component Portion Tag |  |  |  |  |  |  | 01101100 | (Table 14/Q.773) |
|  |  | Length |  |  |  |  |  |  | 00011000 | All 24 octets below here |
|  |  |  | Component Type Tag |  |  |  |  |  | 10100011 | = Ret. Error (Table 19/Q.773) |
|  |  |  | Length |  |  |  |  |  | 00010110 | All 22 octets below here |
|  |  |  |  | Component ID Tag |  |  |  |  | 00000010 | = Invoke ID (Table 20/Q.773) |
|  |  |  |  | Length |  |  |  |  | 00000001 | 1 octet |
|  |  |  |  | Invoke ID Value |  |  |  |  | xxxxxxxx | Same as MRVT message (Correlation) |
|  |  |  |  | Error Code Tag |  |  |  |  | 00000010 | Table 24/Q.773 (local) |
|  |  |  |  | Length |  |  |  |  | 00000001 | 1 octet |
|  |  |  |  | Processing Failure |  |  |  |  | 00001010 | Figure 3/Q.754 |
|  |  |  |  | Parameter Sequence Tag |  |  |  |  | 00110000 | = Sequence Tag (Table 23/Q.773) |
|  |  |  |  | Length |  |  |  |  | 00001110 | All 14 octets below here |
|  |  |  |  |  | Specific Error Info Tag |  |  |  | 10100101 | Figure 3/Q.754 |
|  |  |  |  |  | Length |  |  |  | 00001100 | all 12 octets below here |
|  |  |  |  |  |  | Error Type Tag |  |  | 10000000 | Figure 3/Q.754 |
|  |  |  |  |  |  | Length |  |  | 00000001 | 1 octet |
|  |  |  |  |  |  | Failure |  |  | 00000001 | 2.1.1.3.1/Q.754 |
|  |  |  |  |  |  | Error Parameters |  |  | 10100001 | 2.1.1.3.1/Q.754 |
|  |  |  |  |  |  | Length |  |  | 00000111 | All 7 octets below here |
|  |  |  |  |  |  |  |  | Failure Type Tag | 10000000 | 2.1.1.3.1/Q.754 |
|  |  |  |  |  |  |  |  | Length | 00000010 | 2 octets |
|  |  |  |  |  |  |  |  | Unused Bits | 00000000 | no bits |
|  |  |  |  |  |  |  |  | Failure String | xxxxxxxx | Depends on type failure (2.1.1.3.1/Q.754) |
|  |  |  |  |  |  |  |  | Trace Sent Tag | 10000001 | 2.1.1.3.1/Q.754 |
|  |  |  |  |  |  |  |  | Length | 00000001 | 1 octet |
|  |  |  |  |  |  |  |  | Trace Sent Value | 0000000x | True = 1, False = 0 (2.1.1.3.1/Q.754) |

Example of an MRVA (failure) message delivered to the SCCP

FIGURE A.6/Q.754

|  | Field Name |  |  |  |  |  |  |  | Bit Encoding | Reference/Explanation |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Message Type Tag |  |  |  |  |  |  |  | 01100010 | = BEGIN (Table 8/Q.773) |
|  | Message Length |  |  |  |  |  |  |  | 00101100 | 44 octets following TC part |
|  |  | Transaction ID Tag |  |  |  |  |  |  | 01001000 | = Originating (Table 10/Q.773) |
|  |  | Length |  |  |  |  |  |  | 00000100 | 4 octets |
|  |  | Transaction ID Value |  |  |  |  |  |  | xxxxxxxx | TCAP based on a dialogue at |
|  |  |  |  |  |  |  |  |  | xxxxxxxx | the user level |
|  |  |  |  |  |  |  |  |  | xxxxxxxx |  |
|  |  |  |  |  |  |  |  |  | xxxxxxxx |  |
|  |  | Component Portion Tag |  |  |  |  |  |  | 01101100 | (Table 14/Q.773) |
|  |  | Length |  |  |  |  |  |  | 00100100 | All 36 octets below here |
|  |  |  | Component Type Tag |  |  |  |  |  | 10100001 | = Invoke (Table 19/Q.773) |
|  |  |  | Length |  |  |  |  |  | 00100010 | All 34 octets below here |
|  |  |  |  | Component ID Tag |  |  |  |  | 00000010 | = Invoke ID (Table 20/Q.773) |
|  |  |  |  | Length |  |  |  |  | 00000001 | 1 octet |
|  |  |  |  | Invoke ID Value |  |  |  |  | xxxxxxxx | OMAP PROVIDED |
|  |  |  |  | Operation Code Tag |  |  |  |  | 00000010 | = Local (Table 22/Q.773) |
|  |  |  |  | Length |  |  |  |  | 00000001 | 1 octet |
|  |  |  |  | Operation Code |  |  |  |  | 00000000 | = Event Report (Figure 3/Q.754) |
|  |  |  |  | Parameter Sequence Tag |  |  |  |  | 00110000 | = Sequence Tag (Table 23/Q.773) |
|  |  |  |  | Length |  |  |  |  | 00011010 | All 26 octets below here |
|  |  |  |  |  | Object Class Tag |  |  |  | 10000000 | (Figure 3/Q.754) |
|  |  |  |  |  | Length |  |  |  | 00000101 | 5 octets |
|  |  |  |  |  | Value-MTP Routing Tables |  |  |  | 00000000 | CCITT, Rec. |
|  |  |  |  |  |  |  |  |  | 00010001 | q |
|  |  |  |  |  |  |  |  |  | 10000101 | 85 => 754 |
|  |  |  |  |  |  |  |  |  | 01110010 | 72 => |
|  |  |  |  |  |  |  |  |  | 00000000 | MTP Routing Tables 1992 |
|  |  |  |  |  | Object Instance Tag |  |  |  | 10000011 | (Figure 3/Q.754) |
|  |  |  |  |  | Length |  |  |  | 00000010 | 2 octets |
|  |  |  |  |  | Object Instance Value |  |  |  | xxxxxxxx | Terminating PC (OMAP) |
|  |  |  |  |  |  |  |  |  | xxxxxxxx | <Tested Destination> |
|  |  |  |  |  | Event Type Tag |  |  |  | 10000111 | Figure 3/Q.754 |
|  |  |  |  |  | Length |  |  |  | 00000001 | 1 octet |
|  |  |  |  |  | Event Type |  |  |  | 00000010 | = routeTrace (2.1.2/Q.754) |
|  |  |  |  |  | Event Info Type Tag |  |  |  | 10101000 | Figure 3/Q.754 |
|  |  |  |  |  | Length |  |  |  | 00001010 | All 10 octets below here |
|  |  |  |  |  |  | Success Identifier |  |  | 10100000 | 2.1.2.1.1/Q.754 |
|  |  |  |  |  |  | Length |  |  | 00001000 | All 8 octets below here |
|  |  |  |  |  |  |  |  | Point Code Tag | 00000100 | = OCTET STRING |
|  |  |  |  |  |  |  |  | Length | 00000010 | 2 octets |
|  |  |  |  |  |  |  |  | Point Code | xxxxxxxx |  |
|  |  |  |  |  |  |  |  |  | xxxxxxxx |  |
|  |  |  |  |  |  |  |  | Point Code Tag | 00000100 | = OCTET STRING |
|  |  |  |  |  |  |  |  | Length | 00000010 | 2 octets |
|  |  |  |  |  |  |  |  | Point Code | xxxxxxxx |  |
|  |  |  |  |  |  |  |  |  | xxxxxxxx |  |

Example of an MRVR (success) message delivered to the SCCP

1)	CMIP is defined in ISO IS 9596 and in Recommendation X.711.

2) 	See Recommendations X.208 and X.209 for the description of the formal notation.

3	) 	The use of the overall guard timer in the OMASE-User at the test initiator node enables the test to fail gracefully in this circumstance.