|  | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

ITU-T	Q.824.1

TELECOMMUNICATION	(10/95)
STANDARDIZATION  SECTOR
OF  ITU

SPECIFICATIONS  OF  SIGNALLING  SYSTEM  No.  7

STAGES  2  AND  3  DESCRIPTION
FOR  THE  Q3  INTERFACE  –  CUSTOMER
ADMINISTRATION  –  INTEGRATED  SERVICES
DIGITAL  NETWORK  (ISDN)  BASIC
AND  PRIMARY  RATE  ACCESS

ITU-T  Recommendation  Q.824.1

(Previously “CCITT  Recommendation”)

FOREWORD

The ITU-T (Telecommunication Standardization Sector) is a permanent organ of the International Telecommunication Union (ITU). The ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Conference (WTSC), which meets every four years, establishes the topics for study by the ITU-T Study Groups which, in their turn, produce Recommendations on these topics.

The approval of Recommendations by the Members of the ITU-T is covered by the procedure laid down in WTSC Resolution No. 1 (Helsinki, March 1-12, 1993).

ITU-T Recommendation Q.824.1 was prepared by ITU-T Study Group 11 (1993-1996) and was approved under the WTSC Resolution No. 1 procedure on the 17th of October 1995.

___________________

NOTE

In this Recommendation, the expression “Administration” is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

ã  ITU  1996

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

CONTENTS

Recommendation Q.824.1     (10/95)

Page

1	Introduction		1

1.1	Purpose and scope		1

1.2	Cross-reference		1

1.3	Application		1

1.4	General overview		1

1.5	Managed object naming and attribute syntax		6

2	References		6

3	Access Object Classes		7

3.1	Access Channels		7

3.2	Access Ports		7

3.3	Access Port Profiles		8

3.4	Layer Entities		11

4	Catalogued Access Object Classes		13

4.1	Catalogued Access Port ISDN Primary Rate		13

4.2	Catalogued Access Port Profile ISDN		13

4.3	Catalogued Access Port Profile ISDN Basic Rate		14

4.4	Catalogued Layer Entity DSS 1		14

4.5	Catalogued Layer Entity LAPD		14

5	ISDN Terminal Object Classes		15

5.1	Terminal Configuration		15

5.2	Terminal Service Profile		15

6	Bearer Service Object Classes		16

6.1	Bearer Services		16

7	Operations Support Managed Object Classes		20

7.1	Service Manager ISDN		20

7.2	Service Manager Retrieve Service		21

8	Package Templates		21

8.1	Active Terminal List		21

8.2	Assignment Of Timeslots		21

8.3	Automatic Negotiation		21

8.4	Bearer Service For Audio IC		21

8.5	Bearer Service For 384 kbit/s Data Primary IC		22

8.6	Bearer Service For 1536 kbit/s Data Primary IC		22

8.7	Bearer Service For 1920 kbit/s Data Primary IC		22

8.8	Bearer Service For Multiple-Rate Data Primary IC		22

8.9	Bearer Service List		22

8.10	Call Reference		22

8.11	Circuit Speech Primary IC		23

8.12	Circuit Multi-Use Primary IC		23

8.13	Circuit Unrestricted Digital Data Primary IC		23

8.14	Circuit Unrestricted Digital Data Rate Adapted From 56 kbit/s Primary IC		23

8.15	Deactivation Capabilities		23

8.16	Link Setting		23

8.17	Manage ISDN Terminal		23

8.18	Max Combined Thruput B-Channel		23

8.19	Max Combined Thruput D-Channel		24

Page

8.20	Network Provided Tone		24

8.21	nT2ISDN Access Port Profile		24

8.22	Number Of D-Channel Links		24

8.23	Optional Deactivation		25

8.24	Selection Procedures		25

8.25	DCE Timers		25

9	Attribute templates		25

9.1	Access Channel Pointer List		25

9.2	Active Terminal List		25

9.3	Assignment Of Timeslots		25

9.4	Automatic XID Notification		26

9.5	Bearer Service For 384 kbit/s Data Primary IC		26

9.6	Bearer Service For 1536 kbit/s Data Primary IC		26

9.7	Bearer Service For 1920 kbit/s Data Primary IC		26

9.8	Bearer Service For Multiple-Rate Data Primary IC		27

9.9	Bearer Service List		27

9.10	Bit Rate Of Primary Rate Interface		27

9.11	Calling Number Screening Id		27

9.12	Catalogued Layer Entity LAPD Id		27

9.13	Circuit Audio Primary IC		28

9.14	Circuit Multi-Use Primary IC		28

9.15	Circuit Unrestricted Digital Data Rate Adapted From 56 kbit/s Primary IC		28

9.16	Circuit Speech Primary IC		28

9.17	Circuit Unrestricted Digital Data Primary IC		28

9.18	Calling Party Default Directory Number		28

9.19	Calling Party Number Discard Control		29

9.20	Calling Party Number Provision Necessary		29

9.21	Calling Party Valid Directory Number List		29

9.22	Catalogued Access Port ISDN Primary Rate Id		29

9.23	Catalogued Access Port Profile ISDN Id		30

9.24	Catalogued Layer Entity DSS 1 ID		30

9.25	Channel Selection		30

9.26	D-channel Packet Default Directory Number		30

9.27	D-Channel Packet Directory Number List		30

9.28	D-Channel Primary Pointer		31

9.29	D-Channel Secondary Pointer		31

9.30	D-Channel T301		31

9.31	D-Channel T303		31

9.32	D-Channel T304		31

9.33	D-Channel T305		32

9.34	D-Channel T306		32

9.35	D-Channel T307		32

9.36	D-Channel T308		32

9.37	D-Channel T309		32

9.38	D-Channel T310		32

9.39	D-Channel T312		33

9.40	D-Channel T314		33

9.41	D-Channel T316		33

9.42	D-Channel T317		33

9.43	D-Channel T320		33

9.44	D-Channel T321		34

9.45	D-Channel T322		34

Page

9.46	D-Channel T330		34

9.47	Deactivation Capabilities		34

9.48	Directory Number Appearance Identifier List		34

9.49	Directory Number Reference		35

9.50	DTE Compatibility		35

9.51	Early Cut Thru Remote Network Interwork		35

9.52	Early Cut Thru User Provided Audible Ring		35

9.53	Feature Activators All Directory Number		36

9.54	Feature Activators Per Directory Number		36

9.55	Feature Activators Per Hunt Make Busy		36

9.56	Feature Activators Per Stop Hunt		36

9.57	Feature Indicators All Directory Number		37

9.58	Feature Indicators Per Directory Number		37

9.59	Feature Indicators Per Hunt Make Busy		37

9.60	Feature Indicators Per Stop Hunt		37

9.61	Incoming Default Thruput Class		38

9.62	Incoming Max Packet Size		38

9.63	Incoming Window Size		38

9.64	Interface Type		38

9.65	Layer 2 Info Entity Pointer		39

9.66	Layer Entity LAPD Pointer		39

9.67	Layer 3 Info Entity Pointer		39

9.68	Link Level Window Size		39

9.69	Link Option		39

9.70	Max Bits Per Information Frame		40

9.71	Max Combined Thruput Class		40

9.72	Max Number of Call Reference		40

9.73	Max Transmission Attempts		40

9.74	Network Provided Tones		40

9.75	Network User Identification		41

9.76	Network User Id Override		41

9.77	Network User Id Selection		41

9.78	Network UserId Supplement		41

9.79	Network UserId User Validate		42

9.80	Notification Class		42

9.81	Number Of D-Channel Links		42

9.82	Outgoing Default Thruput Class		42

9.83	Outgoing Max Packet Size		43

9.84	Outgoing Window Size		43

9.85	Screen Calling Party Number		43

9.86	Semi-Permanent Access Packet Handler Default Directory Number		43

9.87	Signalling Parameter Negotiation		43

9.88	Terminal Configuration Id		44

9.89	Terminal configuration Pointer		44

9.90	TSP ID		44

9.91	Terminal Limit		44

9.92	Terminal Service Profile		45

10	Parameter Templates		45

11	Name Bindings		45

11.1	Catalogued Access Port ISDN		45

11.2	Calling Number Screening		45

Page

11.3	Catalogued Access Port Profile ISDN Primary Rate		45

11.4	Catalogued Layer Entity LAPD		46

11.5	Catalogued Layer Entity DSS 1		46

11.6	Terminal Configuration		46

11.7	Terminal Service Profile		46

11.8	X.25 Network User Identification		46

11.9	Layer Entity X25PLP Shared		46

12	Service Provisioning Actions		47

12.1	Change Directory Number		47

12.2	Establish ISDN Access		47

12.3	Remove ISDN Access		48

12.4	Establish ISDN Service		48

12.5	Remove ISDN Service		49

12.6	Establish ISDN Terminal		49

12.7	Remove ISDN Terminal		50

12.8	Retrieve Service		50

13	Type Definitions		51

14	Actions		55

14.1	Conventions		55

14.2	Change Directory Number		56

14.3	Establish ISDN Access		57

14.4	Establish ISDN Service		58

14.5	Establish ISDN Terminal		59

14.6	Remove ISDN Access		60

14.7	Remove ISDN Service		61

14.8	Remove ISDN Terminal		62

14.9	Retrieve Customer Service		63

SUMMARY

The purpose of this Recommendation is to provide the Stages 2 and 3 descriptions of the Q3 interface between a local exchange and the Telecommunications Management Network (TMN) for the support of configuration management functions in support of the customer administration of ISDN Basic and Primary access. Customer administration is a management activity that the network operator performs in order to exchange with the customer all the customer related management data and functions required to offer a telecommunications service, and to exchange with the network all the customer related management data and functions necessary for the network to produce that telecommunications service. This Recommendation supports the administration of the customer configuration in the local exchange by the TMN. This Recommendation is part of a series of Recommendations. In this Recommendation the ISDN Basic and Primary Rate technology specific managed objects are defined.

Recommendation Q.824.1

Recommendation Q.824.1     (10/95)

STAGES  2  AND  3  DESCRIPTION  FOR  THE  Q3  INTERFACE  –  CUSTOMER
ADMINISTRATION  –  INTEGRATED  SERVICES  DIGITAL  NETWORK  (ISDN)
BASIC  AND  PRIMARY  RATE  ACCESS

(Geneva, 1995)

# 1	Introduction

## 1.1	Purpose and scope

Customer administration is a management activity that the network operator performs in order to exchange with the customer all the customer related management data and functions required to offer a telecommunications service and to exchange with the network all the customer related management data and functions necessary for the network to produce that telecommunications service.

The purpose of this Recommendation is to provide the ISDN technology specific Stage 2 and 3 description of the Q3 interface between a local exchange and the Telecommunications Management Network (TMN) for the support of configuration management functions.

The Q3 interface is the TMN interface between network elements or Q-adapters which interface to Operations Systems (OSs) without mediation and between OSs and mediation devices as described in Recommendation M.3100.

## 1.2	Cross-reference

This Recommendation is based on the Stage 1 management service description given in the M.3000-Series Recommendations including Recommendation M.3400. This Recommendation provides the Stage 2 and 3 descriptions for handling the Customer AdministrationXE "Customer Administration"§ for the ISDN technology based on the ISDN service capabilities descriptions provided in I.200, I.210, I.220 and I.230-Series Recommendations, and based on the common Stage 2 and 3 descriptions given in Recommendation Q.824.0. The information model provided by this Recommendation may be used for the Customer Administration purposes either over a Q3 interfaceXE "Q3 interface"§ or over the ISDN UNI as described in Recommendation Q.942.

## 1.3	Application

The management information included in this Recommendation may be exchanged by implementations of the Common Management Information Service Element (CMISE). The Transaction-Oriented class of OAM&P applications is supported in this Recommendation by defining object classes, their attributes, and their relationships. The protocol suites are given in Recommendations Q.811 and Q.812. No special requirements are identified.

## 1.4	General overview

### 1.4.1	Information model diagrams

The following information model diagrams have been drawn for the purpose of clarifying the relations between the different object classes of Customer Administration. There are three different types of diagrams:

1)	Entity-Relationship Models showing the relations of the different managed objects.

2)	Inheritance Hierarchy showing how managed objects are derived from each other (i.e. the different paths of inherited characteristics of the different managed objects).

3)	Naming Hierarchy showing the derivation of names for managed objects (i.e. the different naming paths for instances of managed objects).

These three different diagrams are only for clarification. The formal specification in terms of GDMO templates and ASN.1 type definitions are the relevant information for the implementation of this Recommendation. See Figures 1, 2 and 3.

FIGURE 1a/Q.824.1[D01] = 21 CM (page pleine)

FIGURE 1b/Q.824.1[D02] = 12 CM

FIGURE 1c/Q.824.1[D03] = 13 CM

FIGURE 2/Q.824.1[D04] = 24 CM  (PLEINE PAGE)
XE "Inheritance hierarchy"§

FIGURE 3/Q.824.1[D05] = 21 CM(PLEINE PAGE) XE "Naming hierarchy"§

#### Entity-Relationship ModelsXE "Entity-Relationship Models"§

The following Entity-Relationship Model describes particular relationships among the ISDN objects and other managed objects.

The E-R diagrams illustrate the intended way of applying the model. However, the E-R diagrams do not show all possible relationships supported by the model. The E-R diagrams show relationships in which managed objects may participate. Instances of a class or a subclass may not be eligible to participate in the indicated relationship. In case of containment this means that an alternate name binding will exist; in relationships implemented via pointers the pointer value will be null if an instance cannot or does not participate in the relationship.

## 1.5	Managed object naming and attribute syntax

Throughout this Recommendation, all attributes are named according to the following guidelines:

–	The name of an attribute is composed of the name of an object class followed by the string “Ptr” if and only if the attribute value is intended to identify a specific object class.

–	If an attribute value is intended to identify different object classes, a descriptive name is given to that attribute and a description is provided in the attribute behaviour.

–	The name of an attribute is composed of the name of an object class followed by the string “Id” if and only if the attribute value is intended to identify the name of the object class holding that attribute.

# 2	References

The following Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; all users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

–	CCITT Recommendation I.231.1 (1988), 64 kbit/s Unrestricted, 8 kHz structured.

–	CCITT Recommendation I.231.2 (1988), 64 kbit/s, 8 kHz structured, usable for speech information transfer.

–	CCITT Recommendation I.231.3 (1988), 64 kbit/s, 8 kHz structured, usable for 3.1 kHz audio information transfer.

–	ITU-T Recommendation I.231.9 (1993), Circuit Mode 64 kbit/s 8 kHz structured multi-use bearer service category.

–	CCITT Recommendation I.232 (1988), Packet-mode bearer services categories.

–	CCITT Recommendation I.251.3 (1992), Calling Line Identification Presentation.

–	CCITT Recommendation M.3010 (1992), Principles for a telecommunications management network.

–	CCITT Recommendation M.3020 (1992), TMN interface specification methodology.

–	ITU-T Recommendation M.3100 (1995), Generic network information model.

–	CCITT Recommendation M.3400 (1992), TMN management functions.

–	ITU-T Recommendation Q.811 (1993), Lower layer protocol profiles for the Q3 interface.

–	ITU-T Recommendation Q.812 (1993), Upper layer protocol profiles for the Q3 interface.

–	ITU-T Recommendation Q.824.0 (1995), Stages 2 and 3 description for the Q3 interface – Customer administration – Common information.

–	ITU-T Recommendation Q.921 (1993), ISDN user-network interface – Data link layer specification.

–	ITU-T Recommendation Q.931 (1993), Digital Subscriber Signalling System No. 1 (DSS 1) – ISDN user-network interface layer 3 specification for basic call control.

–	ITU-T Recommendation Q.932 (1993), Digital Subscriber Signalling System No. 1 (DSS 1) – Generic procedures for the control of ISDN supplementary services.

–	ITU-T Recommendation X.25 (1993), Interface between Data Terminal Equipment (DTE) and Data Circuit-terminating Equipment (DCE) for terminals operating in the packet mode and connected to public data networks by dedicated circuit.

–	ITU-T Recommendation X.31 (1993), Support of packet mode terminal equipment by an ISDN.

–	ITU-T Recommendation X.281 (1995), Information technology – Elements of management information related to OSI physical layer.

–	ITU-T Recommendation X.282 (1995), Elements of management information related to OSI data link layer.

–	ITU-T Recommendation X.283 (1993), Elements of management information related to the OSI network layer.

–	CCITT Recommendation X.700 (1992), Management framework for Open Systems Interconnection (OSI) for CCITT applications.

–	CCITT Recommendation X.701 (1992), Information technology – Open Systems Interconnection – Systems management overview.

–	CCITT Recommendation X.710 (1991), Common management information service definition for CCITT applications.

–	CCITT Recommendation X.711 (1991), Common management information protocol specification for CCITT applications.

–	CCITT Recommendation X.720 (1992), Information technology – Open Systems Interconnection – Structure of management information: Management information model.

–	CCITT Recommendation X.721 (1992), Information technology – Open Systems Interconnection – Structure of management information: Definition of management information.

–	CCITT Recommendation X.722 (1992), Information technology – Open Systems Interconnection – Structure of management information: Guidelines for the definition of managed objects.

–	ITU-T Recommendation X.723 (1993), Information technology – Open Systems Interconnection – Structure of management information: Generic management information.

# 3	Access Object ClassesXE "Access Object Classes"§

## 3.1	Access ChannelsXE "Access Channels"§

### 3.1.1	Access Channel B-ChannelXE "Access Channel B-Channel"§

bChannel		MANAGED OBJECT CLASS

DERIVED FROM		"ITU-T Rec. Q.824.0":accessChannel;

CHARACTERIZED BY

"CCITT Rec. M.3100":channelNumberPackage,

bChannelPkg			PACKAGE

BEHAVIOUR

bChannelBhv	BEHAVIOUR

DEFINED AS "This object class represents the attributes related to a particular B-channel on an ISDN access port.";;;;

REGISTERED AS	{cAISDNObjectClass 1};

### 3.1.2	Access Channel D-ChannelXE "Access Channel D-Channel"§

dChannel		MANAGED OBJECT CLASS

DERIVED FROM		"ITU-T Rec. Q.824.0":accessChannel;

CHARACTERIZED BY

dChannelPkg			PACKAGE

BEHAVIOUR

dChannelBhv	BEHAVIOUR

DEFINED AS	"This object class represents the attributes and characteristics related to a particular D-channel on an ISDN access port.";;;;

REGISTERED AS	{cAISDNObjectClass 2};

## 3.2	Access PortsXE "Access Ports"§

### 3.2.1	Access Port ISDN Basic RateXE "Access Port ISDN Basic Rate"§

accessPortISDNBasicRate		MANAGED OBJECT CLASS

DERIVED FROM		"ITU-T Rec. Q.824.0":accessPort;

CHARACTERIZED BY

accessPortISDNBasicRatePkg			PACKAGE

BEHAVIOUR

accessPortISDNBasicRateBhv	BEHAVIOUR

DEFINED AS	"This object class represents the access port termination supporting the ISDN Basic Rate service.";;;;

REGISTERED AS	{cAISDNObjectClass 3};

### 3.2.2	Access Port ISDN Primary RateXE "Access Port ISDN Primary Rate"§

accessPortISDNPrimaryRate		MANAGED OBJECT CLASS

DERIVED FROM		"ITU-T Rec. Q.824.0":accessPort;

CHARACTERIZED BY

accessPortISDNPrimaryRatePkg			PACKAGE

BEHAVIOUR

accessPortISDNPrimaryRateBhv	BEHAVIOUR

DEFINED AS	"This object class represents the access port termination supporting the DSS 1 interface identifier in the Primary Rate ISDN service.";;;;

REGISTERED AS	{cAISDNObjectClass 4};

## 3.3	Access Port ProfilesXE "Access Port Profiles"§

### 3.3.1	Access Port Profile ISDNXE "Access Port Profile ISDN"§

accessPortProfileISDN		MANAGED OBJECT CLASS

DERIVED FROM		"ITU-T Rec. Q.824.0":accessPortProfile;

CHARACTERIZED BY

accessPortProfileISDNPkg			PACKAGE

BEHAVIOUR

accessPortProfileISDNBhv	BEHAVIOUR

DEFINED AS	"This subclass of the access port profile adds ISDN specifics to the general access port profile class and is further subclassed into particular interface types (i.e. Basic and Primary). Only characteristics common to basic and primary are placed in this class.

This class provides the association between the B- and D-channels needed to support non-associated signalling. This type of association can be applied to any subclass of this class including Basic and Primary Rate access ports.

The supportedByAccessPortPtrList attribute inherited from the accessPortProfile managed object class may point to multiple accessPorts managed objects in this object and its subclasses.

The numberOfBChannels attribute controls the number of B-channel resources that the Access Port Profile ISDN has simultaneous access to. The value of this attribute must be equal to or less than the number of B-channel Access Channel object instances associated with the Access Port Profile. The default value of –1 for this indicates that all of the B-channel Access Channel object instances associated with the Access Port Profile are available.";;

ATTRIBUTES

accessChannelPtrList										REPLACE-WITH-DEFAULT

DEFAULT VALUE CAISDNAttributeModule.emptySet				GET-REPLACE

ADD-REMOVE,

numberOfBChannels										REPLACE-WITH-DEFAULT
																DEFAULT VALUE

CAISDNAttributeModule.minusONE 							GET-REPLACE

ADD-REMOVE,

layer2LAPDEntityPtr										GET,

dChannelPacketDirectoryNumberList							GET,

dChannelPacketDefaultDirectoryNumber						GET-REPLACE;;;

CONDITIONAL PACKAGES

bearerServiceListPkg				PRESENT IF "an instance supports it.",

callReferencePkg					PRESENT IF "if restricting the max numbers of call reference is enforced.",

selectionProceduresPkg				PRESENT IF "if supported by Administration.",

nT2ISDNaccessPortProfilePkg			PRESENT IF "NT2 type service is supported on the access port and the service is supported by the Administration.";

REGISTERED AS	{cAISDNObjectClass 5};

### 3.3.2	Access Port Profile ISDN Basic RateXE "Access Port Profile ISDN Basic Rate"§

accessPortProfileISDNBasicRate		MANAGED OBJECT CLASS

DERIVED FROM		accessPortProfileISDN;

CHARACTERIZED BY

accessPortProfileISDNBasicRatePkg			PACKAGE

BEHAVIOUR

accessPortProfileISDNBasicRateBhv	BEHAVIOUR

DEFINED AS	"This object associates the Access Port resources with service for ISDN Basic Rate. The Basic Rate access supporting this access port Profile may provide up to two 64 kbit/s B-channel and one 16 kbit/s D-channel service.";;;;

CONDITIONAL PACKAGES

deactivationCapabilitiesPkg			PRESENT IF "deactivationCapabilities procedure is supported by Administration.",

activeTerminalListPkg				PRESENT IF "an Administration supports it.",

numberOfDChannelLinksPkg			PRESENT IF "if controlling the number of D-channel links on a per dChannel basis is supported by the Administration.";

REGISTERED AS	{cAISDNObjectClass 6};

### 3.3.3	Access Port Profile ISDN Primary RateXE "Access Port Profile ISDN Primary Rate"§

accessPortProfileISDNPrimaryRate			MANAGED OBJECT CLASS

DERIVED FROM		accessPortProfileISDN;

CHARACTERIZED BY

accessPortProfileISDNPrimaryRatePkg				PACKAGE

BEHAVIOUR

accessPortProfileISDNPrimaryRateBhv		BEHAVIOUR

DEFINED AS	"This object associates the Access Port resources with services for the ISDN primary rate. The primary rate access supporting this access port profile can range from that of a single primary rate section up to that including many primary rate sections. In addition to the information channels and the D-channel, a secondary D-channel may also be supported in a backup arrangement (see Annex F/Q.931).";;

ATTRIBUTES

dChannelPrimaryPtr										GET-REPLACE,

dChannelSecondaryPtr										GET-REPLACE;;;

CONDITIONAL PACKAGES

REGISTERED AS	{cAISDNObjectClass 7};

### 3.3.4	Calling Number ScreeningXE "Calling Number Screening"§

callingNumberScreening			MANAGED OBJECT CLASS

DERIVED FROM		"CCITT Rec. X.721(1992)":top;

CHARACTERIZED BY

callingNumberScreeningPkg					PACKAGE

BEHAVIOUR

callingNumberScreeningPackageBhv	BEHAVIOUR

DEFINED AS	"This object identifies characteristics of the Access Port Profile ISDN object class to be instantiated for screening as defined in Calling Line Supplementary Service I.251.3.

The attributes included in this object provide an interface with the capability of defining the set of Calling Party Valid Directory Numbers and a Calling Party Default Directory Number for the interface. It also provides a boolean attribute whose value determines whether or not calling party number screening is to be used by call originations.

The following table shows the possible combinations of the attributes for Calling Party Number Provision Necessary (CPNPN), Screening of Calling Party Number (SCPN), Calling Party Number Discard Control (CPNDC), and Calling Party Default Directory Number (CPDDN).

CPNPN indicates whether the calling party number must be provided by the calling user for call originations. If CPNPN is Y, then all call originations from the interface must provide a valid calling party number.

SCPN indicates whether calling party numbers received from the interface for call originations will be screened against the list of valid DNs for the interface.

CPNDC indicates whether failed-screening and not-screened calling party numbers are discarded from the calling information. If CPNDC is Y, then calling party numbers that are either not screened or fail screening are discarded.

CPDDN is the default DN (or DNs/bearer) that are used for call originations from this interface where the user does not provide a valid calling party number.

The scenario represented by the first row requires that the user provide valid calling party numbers (CPNPN = Y) for all call set-ups from the interface. User-provided calling party numbers will be screened for validity (SCPN = Y). If the calling party number fails screening, then it is discarded (in fact, the whole call is discarded so CPNDC really has a different meaning in this case). If the calling party number is not provided by the user then the call is cleared. CPDDN is not applicable in this scenario since the DN for the call must come from the user.

| CPNPN | SCPN | CPNDC | CPDDN |
| --- | --- | --- | --- |
| Y | Y | Y | N/A |
| N | N | N | Required |
| N | N | Y | Required |
| N | Y | N | Required |
| N | Y | Y | Required |

The second row represents a scenario where the user does not have to include the calling party number (CPNPN = N) for call set-ups from the interface. If the calling party number is provided by the user, it is not screened (SCPN = N) and it is not discarded (CPNDC = N). Any user-provided calling party number is passed as part of the call and may be delivered to the called party if the called party supports two-number delivery. CPDDN is required, and in fact, the default DN is used for every call origination from the interface.

The third row represents a scenario where the user does not have to include the calling party number (CPNPN = N) for call set-ups from the interface. If the calling party number is provided, it is not screened (SCPN = N) and discarded (CPNDC = Y). CPDDN is required, and in fact, the default DN is used for every call origination from the interface. Any user-provided calling party numbers are ignored (not screened and discarded).

The fourth row represents a scenario where the user does not have to include the calling party number (CPNPN = N) for call set-ups from the interface. If the calling party number is provided it will be screened for validity (SCPN = Y). If the user-provided calling party number passes screening then that DN is used for the call. CPDDN is required, and the default DN is used for call originations from the interface when the user-provided calling party number fails screening, or if the user does not provide a calling party number. Any user-provided calling party numbers that fail screening are also passed as part of the call (CPNDC = N) and may be delivered to the called party if the called party supports two-number delivery.

The fifth row represents a scenario where the user does not have to include the calling party number (CPNPN = N) for call set-ups from the interface. If the calling party number is provided it will be screened for validity (SCPN = Y). If the user-provided calling party number passes screening then that DN is used for the call. CPDDN is required, and the default DN is used for call originations from the interface when the user-provided calling party number fails screening, or if the user does not provide a calling party number. Any user-provided calling party numbers that fail screening are discarded (CPNDC = Y).";;

ATTRIBUTES

callingNumberScreeningId									GET

SET-BY-CREATE

callingPartyValidDirectoryNumberList							GET,

callingPartyNumberProvisionNecessary							REPLACE-WITH-DEFAULT

DEFAULT VALUE CAISDNAttributeModule.false 				GET-REPLACE,

screenCallingPartyNumber									REPLACE-WITH-DEFAULT

DEFAULT VALUE CAISDNAttributeModule.true					GET-REPLACE,

callingPartyNumberDiscardCtrl								REPLACE-WITH-DEFAULT

DEFAULT VALUE CAISDNAttributeModule.true					GET-REPLACE,

callingPartyDefaultDirectoryNNumber 							GET-REPLACE;

NOTIFICATIONS

"CCITT Rec. X.721 | ISO/IEC 10165-2": attributeValueChange,

"CCITT Rec. X.721 | ISO/IEC 10165-2": objectCreation,

"CCITT Rec. X.721 | ISO/IEC 10165-2": objectDeletion;

;;

REGISTERED AS	{cAISDNObjectClass 8};

### 3.3.5	Network User IdentificationXE "Network User Identification"§

x25NetworkUserIdentification		MANAGED OBJECT CLASS

DERIVED FROM		"ITU-T Rec. Q.824.0":optionalUserFacilities;

CHARACTERIZED BY

x25NetworkUserIdentificationPkg		PACKAGE

BEHAVIOUR

x25NetworkUserIdentificationBhv		BEHAVIOUR

DEFINED AS	"This managed object contains attributes to provide information to the network for purposes of billing, security, network management, or to invoke subscribed facilities.";;

ATTRIBUTES

networkUserIdentificationREPLACE-WITH-DEFAULT

DEFAULT VALUE CAOptionalUserFacilitiesModule.false			GET-REPLACE,

networkUserIdentificationOverride											REPLACE-WITH-DEFAULT

DEFAULT VALUE CAOptionalUserFacilitiesModule.false			GET-REPLACE,

networkUserIdentificationSelection									REPLACE-WITH-DEFAULT

DEFAULT VALUE CAOptionalUserFacilitiesModule.false			GET-REPLACE,

networkUserIdentificationSupplement								REPLACE-WITH-DEFAULT

DEFAULT VALUE CAOptionalUserFacilitiesModule.false			GET-REPLACE,

networkUserIdentificationUserValidate								REPLACE-WITH-DEFAULT

DEFAULT VALUE CAOptionalUserFacilitiesModule.false			GET-REPLACE;

NOTIFICATIONS

"CCITT Rec. X.721 | ISO/IEC 10165-2:1992": attributeValueChange,

"CCITT Rec. X.721 | ISO/IEC 10165-2:1992": objectCreation,

"CCITT Rec. X.721 | ISO/IEC 10165-2:1992": objectDeletion;;;

REGISTERED AS	{cAISDNObjectClass 9};

## 3.4	Layer EntitiesXE "Layer Entities"§

### 3.4.1	Layer Entity LAPBXE "Layer Entity LAPB"§

This class describes the customizable characteristics of LAPB protocol. That is, the characteristics whose values can be selected by the customer for his individual packet bearer service subscriptions. This class is subclassed from Layer Entity.

layerEntityLAPB		MANAGED OBJECT CLASS

DERIVED FROM		"ITU-T Rec. Q.824.0":layerEntity;

CHARACTERIZED BY

layerEntityLAPBPkg			PACKAGE

BEHAVIOUR

layerEntityLAPBBhv	BEHAVIOUR

DEFINED AS	"This object class includes attributes applicable to X.25 layer 2 (LAPB), whose values are assignable on a Directory Number/Packet Data Bearer Service basis. Each instance of this object class represents a particular profile of attribute values and is associated with instances of the bearerServiceForPacketSwitchedDataobject class by pointers to instances of this object class.";;

ATTRIBUTES

"ITU-T Rec. X.282":sequenceModulus							GET-REPLACE;;;

CONDITIONAL PACKAGES

x25DCETimersPkg		PRESENT IF "Timers are specified by subscribers.";

REGISTERED AS	{cAISDNObjectClass 10};

### 3.4.2	Layer Entity LAPDXE "Layer Entity LAPD"§

layerEntityLAPD		MANAGED OBJECT CLASS

DERIVED FROM		"ITU-T Rec. Q.824.0":layerEntity;

CHARACTERIZED BY

layerEntityLAPDPkg			PACKAGE

BEHAVIOUR

layerEntityLAPDBhv	BEHAVIOUR

DEFINED AS	"This class describes the customizable characteristics of the LAPD protocol that are used for user information purpose. These customizable characteristics are the values that can be selected by the customer for his individual bearer service subscriptions. This class is subclassed from Layer Entity. These attributes are not customizable when the LAPD instance is a signalling one.";;;;

CONDITIONAL PACKAGES

automaticNegotiationPkg		PRESENT IF "parameter negotiation is supported.",

linkSettingPkg				PRESENT IF "if a per link setting is supported.",

optionalDeactivationPkg		PRESENT IF "the Administration supports deactivation on a LAPD basis.";

REGISTERED AS	{cAISDNObjectClass 11};

### 3.4.3	Layer Entity X.25 PLPXE "Layer Entity X.25 PLP"§

layerEntityX25PLP		MANAGED OBJECT CLASS

DERIVED FROM		"ITU-T Rec. Q.824.0":layerEntity ;

CHARACTERIZED BY

layerEntityX25PLPPkg				PACKAGE

BEHAVIOUR

layerEntityX25PLPBhv		BEHAVIOUR

DEFINED AS	"This class describes the customizable characteristics of X.25 Layer 3 protocol that are used for signalling purpose and for user information purpose. These customizable characteristics are the values that can be selected by the customer for his individual packet bearer service subscriptions. This object class includes attributes applicable to X.25 layer 3 whose values are assignable on a Directory Number/Packet Data Bearer Service basis. Each instance of this object class represents a particular profile of attribute values.";;

ATTRIBUTES

dtEcompatibility

REPLACE-WITH-DEFAULT

DEFAULT VALUE CAISDNModule.dte84						GET-REPLACE,

incomingMaxPacketSize											REPLACE-WITH-DEFAULT

DEFAULT VALUE CAISDNModule.size128						GET-REPLACE,

incomingWindowSize											REPLACE-WITH-DEFAULT

DEFAULT VALUE CAISDNModule.two  						GET-REPLACE,

incomingDefaultThruputClass										REPLACE-WITH-DEFAULT

DEFAULT VALUE CAISDNModule.baud9600					GET-REPLACE,

outgoingMaxPacketSize											REPLACE-WITH-DEFAULT

DEFAULT VALUE CAISDNModule.size128						GET-REPLACE,

outgoingWindowSize											REPLACE-WITH-DEFAULT

DEFAULT VALUE CAISDNModule.two  						GET-REPLACE,

outgoingDefaultThruputClass										REPLACE-WITH-DEFAULT

DEFAULT VALUE CAISDNModule.baud9600					GET-REPLACE,

"ITU-T Rec. X.283":onlineFacilityRegistration 						GET-REPLACE,

"ITU-T Rec. X.283":extendedPacketSequencing 						GET-REPLACE,

"ITU-T Rec. X.283":dBitModification 								GET-REPLACE,

"ITU-T Rec. X.283":packetRetransmission 							GET-REPLACE,

"ITU-T Rec. X.283":nonStandardDefaultPacketSizes 					GET-REPLACE,

"ITU-T Rec. X.283":nonStandardDefaultWindowSizes					GET-REPLACE,

"ITU-T Rec. X.283":flowControlParameterNegotiation 					REPLACE-WITH-DEFAULT

DEFAULT VALUE CAISDNModule.false						GET-REPLACE,

"ITU-T Rec. X.283":throughputClassNegotiation 						REPLACE-WITH-DEFAULT

DEFAULT VALUE CAISDNModule.false						GET-REPLACE,

"ITU-T Rec. X.283":fastSelectAcceptance 							REPLACE-WITH-DEFAULT

DEFAULT VALUE CAISDNModule.false						GET-REPLACE;;;

CONDITIONAL PACKAGES

maxCombinedThruputBChanPkg		PRESENT IF "the maxCombinedThruputDChanPkg is not present.",

maxCombinedThruputDChanPkg		PRESENT IF "the maxCombinedThruputBChanPkg is not present.";

REGISTERED AS	{cAISDNObjectClass 12};

### 3.4.4	Layer Entity X.25 PLP SharedXE "Layer Entity X.25 PLP Shared"§

layerEntityX25PLPShared		MANAGED OBJECT CLASS

DERIVED FROM				"ITU-T Rec. Q.824.0":layerEntity ;

CHARACTERIZED BY

layerEntityX25PLPSharedPkg				PACKAGE

BEHAVIOUR

layerEntityX25PLPSharedBhv	BEHAVIOUR

DEFINED AS	" This class describes the customizable characteristics of X.25 Layer 3 protocol that are used for signalling purpose and for user information purpose. Instances of this object class are used to define commonly used parameter groupings that may be shared by multiple customers. This object class includes attributes applicable to X.25 layer 3 whose values are assignable on a Directory Number/Packet Data Bearer Service basis. Each instance of this object class represents a particular profile of attribute values .";;

ATTRIBUTES

dTECompatibilty

REPLACE-WITH-DEFAULT

DEFAULT VALUE CAISDNModule.dte84						GET-REPLACE,

incomingMaxPacketSize REPLACE-WITH-DEFAULT

DEFAULT VALUE CAISDNModule.size128						GET-REPLACE,

incomingWindowSize REPLACE-WITH-DEFAULT

DEFAULT VALUE CAISDNModule.two  						GET-REPLACE,

incomingDefaultThruputClass REPLACE-WITH-DEFAULT

DEFAULT VALUE CAISDNModule.baud9600					GET-REPLACE,

outgoingMaxPacketSize REPLACE-WITH-DEFAULT

DEFAULT VALUE CAISDNModule.size128						GET-REPLACE,

outgoingWindowSize REPLACE-WITH-DEFAULT

DEFAULT VALUE CAISDNModule.two  						GET-REPLACE,

outgoingDefaultThruputClass REPLACE-WITH-DEFAULT

DEFAULT VALUE CAISDNModule.baud9600					GET-REPLACE,

"ITU-T Rec. X.283":onlineFacilityRegistration 						GET-REPLACE,

"ITU-T Rec. X.283":extendedPacketSequencing	 					GET-REPLACE,

"ITU-T Rec. X.283":dBitModification								GET-REPLACE,

"ITU-T Rec. X.283":packetRetransmission							GET-REPLACE,

"ITU-T Rec. X.283":nonStandardDefaultPacketSizes					GET-REPLACE,

"ITU-T Rec. X.283":nonStandardDefaultWindowSizes					GET-REPLACE,

"ITU-T Rec. X.283":flowControlParameterNegotiation					REPLACE-WITH-DEFAULT

DEFAULT VALUE CAISDNModule.false						GET-REPLACE,

"ITU-T Rec. X.283":throughputClassNegotiation						REPLACE-WITH-DEFAULT

DEFAULT VALUE CAISDNModule.false						GET-REPLACE,

"ITU-T Rec. X.283":fastSelectAcceptance							REPLACE-WITH-DEFAULT

DEFAULT VALUE CAISDNModule.false						GET-REPLACE;;;

CONDITIONAL PACKAGES

maxCombinedThruputBChanPkg		PRESENT IF "the maxCombinedThruputDChanPkg is not present.",

maxCombinedThruputDChanPkg		PRESENT IF "the maxCombinedThruputBChanPkg is not present.";

REGISTERED AS	{cAISDNObjectClass 13};

# 4	Catalogued Access Object ClassesXE "Catalogued Access Object Classes"§

## 4.1	Catalogued Access Port ISDN Primary RateXE "Catalogued Access Port ISDN Primary Rate"§

cataloguedAccessPortISDNPrimaryRate		MANAGED OBJECT CLASS

DERIVED FROM		"CCITT Rec. X.721(1992)": top;

CHARACTERIZED BY

cataloguedAccessPortISDNPrimaryRatePkg			PACKAGE

BEHAVIOUR

cataloguedAccessPortISDNPrimaryRateBhv	BEHAVIOUR

DEFINED AS	"This object class contains the non-customizable attributes of the access port supporting the DSS 1 interface of the Primary Rate ISDN service.";;

ATTRIBUTES

cataloguedAccessPortISDNPrimaryRateId 						GET SET-BY-CREATE,

bitRateOfPrimaryRateInterface 								GET-REPLACE;

NOTIFICATIONS

"CCITT Rec. X.721": objectCreation,

"CCITT Rec. X.721": objectDeletion,

"CCITT Rec. X.721": attributeValueChange;;;

REGISTERED AS	{cAISDNObjectClass 14};

## 4.2	Catalogued Access Port Profile ISDNXE "Catalogued Access Port Profile ISDN"§

cataloguedAccessPortProfileISDN		MANAGED OBJECT CLASS

DERIVED FROM		"CCITT Rec. X.721(1992)": top;

CHARACTERIZED BY

cataloguedAccessPortProfileISDNPkg			PACKAGE

BEHAVIOUR

cataloguedAccessPortProfileISDNBhv	BEHAVIOUR

DEFINED AS	"This object class defines the access port profile attributes that are common to the ISDN basic and primary rate interfaces that the Administration may manage on a per exchange basis.";;

ATTRIBUTES

cataloguedAccessPortProfileISDNId							GET SET-BY-CREATE,

channelSelection											GET-REPLACE;;;

NOTIFICATIONS

"CCITT Rec. X.721": objectCreation,

"CCITT Rec. X.721": objectDeletion,

"CCITT Rec. X.721": attributeValueChange;;;

REGISTERED AS	{cAISDNObjectClass 15};

## 4.3	Catalogued Access Port Profile ISDN Basic RateXE "Catalogued Access Port Profile ISDN Basic Rate"§

cataloguedAccessPortProfileISDNBasicRate		MANAGED OBJECT CLASS

DERIVED FROM		cataloguedAccessPortProfileISDN;

CHARACTERIZED BY

cataloguedAccessPortProfileISDNBasicRatePkg			PACKAGE

BEHAVIOUR

cataloguedAccessPortProfileISDNBasicRateBhv	BEHAVIOUR

DEFINED AS	"This object class defines the access port profile ISDN basic rate interface characteristics that the Administration may manage on a per exchange basis.";;

ATTRIBUTES

numberOfDChannelLinks									GET-REPLACE;

NOTIFICATIONS

"CCITT Rec. X.721": objectCreation,

"CCITT Rec. X.721": objectDeletion,

"CCITT Rec. X.721": attributeValueChange;;;

REGISTERED AS	{cAISDNObjectClass 16};

## 4.4	Catalogued Layer Entity DSS 1XE "Catalogued Layer Entity DSS 1"§

cataloguedLayerEntityDSS1		MANAGED OBJECT CLASS

DERIVED FROM		"CCITT Rec. X.721": top;

CHARACTERIZED BY

cataloguedLayerEntityDSS1Pkg			PACKAGE

BEHAVIOUR

cataloguedLayerEntityDSS1Bhv	BEHAVIOUR

DEFINED AS	"This object class represents the Layer 3 signalling parameters that are provided for customer service on a switch-wide basis.";;;

ATTRIBUTES

cataloguedLayerEntityDSS1Id									GET SET-BY-CREATE,

dChannelT301												GET-REPLACE,

dChannelT303												GET-REPLACE,

dChannelT304												GET-REPLACE,

dChannelT305												GET-REPLACE,

dChannelT306												GET-REPLACE,

dChannelT307												GET-REPLACE,

dChannelT308												GET-REPLACE,

dChannelT309												GET-REPLACE,

dChannelT310												GET-REPLACE,

dChannelT312												GET-REPLACE,

dChannelT314												GET-REPLACE,

dChannelT316												GET-REPLACE,

dChannelT317												GET-REPLACE,

dChannelT320												GET-REPLACE,

dChannelT321												GET-REPLACE,

dChannelT322												GET-REPLACE,

dChannelT330												GET-REPLACE;

NOTIFICATIONS

"CCITT Rec. X.721": objectCreation,

"CCITT Rec. X.721": objectDeletion,

"CCITT Rec. X.721": attributeValueChange;;;

REGISTERED AS	{cAISDNObjectClass 17};

## 4.5	Catalogued Layer Entity LAPDXE "Catalogued Layer Entity LAPD"§

cataloguedLayerEntityLAPD		MANAGED OBJECT CLASS

DERIVED FROM		"CCITT Rec. X.721(1992)": top;

CHARACTERIZED BY

cataloguedLayerEntityLAPDPkg					PACKAGE

BEHAVIOUR

cataloguedLayerEntityLAPDBhv			BEHAVIOUR

DEFINED AS	"The catalogued LAPD layer entity object class is a class of managed objects that represents characteristics of the LAPD protocol that are applicable to all customers on the switch that have ISDN access. This class also provides the ability for an Administration to set LAPD characteristics for all ISDN customer accesses without the need for individual LAPD entity settings.";;

ATTRIBUTES

cataloguedLayerEntityLAPDId								GET,

interfaceType												GET,

linkLevelWindowSize										GET-REPLACE,

maxBitsPerInformationFrame									GET-REPLACE,

maxTransmissionAttempts									GET-REPLACE;;;

NOTIFICATIONS

"CCITT Rec. X.721": objectCreation,

"CCITT Rec. X.721": objectDeletion,

"CCITT Rec. X.721": attributeValueChange;;;

REGISTERED AS	{cAISDNObjectClass 18};

# 5	ISDN Terminal Object ClassesXE "ISDN Terminal Object Classes"§

## 5.1	Terminal ConfigurationXE "Terminal Configuration"§

terminalConfiguration		MANAGED OBJECT CLASS

DERIVED FROM		"CCITT Rec.  X.721": top;

CHARACTERIZED BY

terminalConfigurationPkg			PACKAGE

BEHAVIOUR

terminalConfiguratonBhv	BEHAVIOUR

DEFINED AS	"The Terminal Configuration managed object class is a class of managed objects that represents instances of ISDN terminal push button and indicator lamp configurations. This managed object class includes the attributes by which the physical Feature Activator (e.g. buttons) and Feature Indicator (e.g. lamps) at the user terminal are functionally assigned. This object class may effectively represent a shared library or unique non-shared object depending on this service.";;

ATTRIBUTES

terminalConfigurationId										GET SET-BY-CREATE,

featureActivatorAllDirectoryNumber							GET-REPLACE,

featureActivatorsPerDirectoryNumber							GET-REPLACE,

featureActivatorsPerStopHunt									GET-REPLACE,

featureActivatorsPerHuntMakeBus								GET-REPLACE,

featureIndicatorsAllDirectoryNumber							GET-REPLACE,

featureIndicatorsPerDirectoryNumber							GET-REPLACE,

featureIndicatorsPerStopHunt									GET-REPLACE,

featureIndicatorsPerHuntMakeBusy							GET-REPLACE;

NOTIFICATIONS

"CCITT Rec. X.721 | ISO/IEC 10165-2 ": attributeValueChange,

"CCITT Rec. X.721 | ISO/IEC 10165-2": objectCreation,

"CCITT Rec. X.721 | ISO/IEC 10165-2": objectDeletion;;;

REGISTERED AS	{cAISDNObjectClass 19};

## 5.2	Terminal Service ProfileXE "Terminal Service Profile"§

terminalServiceProfile	MANAGED OBJECT CLASS

DERIVED FROM		"CCITT Rec. X.721(1992)": top;

CHARACTERIZED BY

terminalServiceProfilePkg			PACKAGE

BEHAVIOUR

terminalServiceProfileBhv	BEHAVIOUR

DEFINED AS	"The Terminal Service Profile (TSP) object class is a class of managed objects that represents instances of Terminal Service Profiles for self-initializing ISDN terminals on a Basic Rate Interface (BRI). The TSP is a grouping of service profile parameters for one or more terminals on a BRI. Each terminal on a BRI must be initialized with a unique identifier called a Service Profile Identifier (SPID). The SPID uniquely identifies the ISDN terminal to the switch and to the supporting Operations System (OS). See Annex A/Q.932 for details.

The numberOfBChannels attribute limits the B-channel resources that the terminalServiceProfile can use at any point in time. The value of the attribute in this object must be equal to or less than the value of the numberOfBChannels attribute in the associated accessPortProfileIsdn object instance. The default value of –1 for this attribute indicates that all of the accessChannel object instances associated with the accessPortProfileIsdn object are available for use.";;

ATTRIBUTES

terminalServiceProfileId										GET SET-BY-CREATE,

"Q.824.0":accessPortProfilePtr								GET-REPLACE,

directoryNumberAppearanceIdentifierList						GET-REPLACE
																ADD-REMOVE,

directoryNumberReference									GET-REPLACE,

numberOfBChannels										REPLACE-WITH-DEFAULT

DEFAULT VALUE CAISDNModule.minusOne					GET-REPLACE,

terminalConfigurationPtr  									GET-REPLACE,

terminalLimit												REPLACE-WITH-DEFAULT

DEFAULT VALUE CAISDNModule.one  						GET-REPLACE,

tspid														GET-REPLACE;

NOTIFICATIONS

"CCITT Rec. X.721 | ISO/IEC 10165-2": attributeValueChange,

"CCITT Rec. X.721 | ISO/IEC 10165-2": objectCreation,

"CCITT Rec. X.721 | ISO/IEC 10165-2": objectDeletion;;;

REGISTERED AS	{cAISDNObjectClass 20};

# 6	Bearer Service Object ClassesXE "Bearer Service Object Classes"§

## 6.1	Bearer ServicesXE "Bearer Services"§

### 6.1.1	Bearer Service For 384 kbit/s DataXE "Bearer Service For 384 kbit/s Data"§

bearerServiceFor384kbpsData		MANAGED OBJECT CLASS

DERIVED FROM				"ITU-T Rec. 824.0":bearerService;

CHARACTERIZED BY

bearerServiceFor384kbpsDataPkg				PACKAGE

BEHAVIOUR

bearerServiceFor384kbpsDataBhv		BEHAVIOUR

DEFINED AS	"This object class represents the characteristics of the Circuit-Mode, 384 kbit/s Unrestricted Digital Transmission, 8000 Hz Structured, Demand, Point-To-Point, and Bidirectional Symmetric bearer service. This bearer service and the associated values for the I.210 Information Transfer and Access attributes, are identified in I.231.6. A call using this bearer service must use channels from single primary rate interfaces as modeled by Access Port.

The numberOfBChannels attribute inherited from the bearerService superclass must indicate either all B-channels are available (–1) or must be a mutiple of 6 to indicate that only some of the underlying B-channels may be used for this bearer service. If all B-channels are indicated by the numberOfBChannels attribute, then on each related Access Port largest mutiple of 6 less than or equal to the total number of B-channels on that AccessPort may be used for this service. For example, if this service is associated with two AccessPort objects, one with 23 B-channels and the other with 24 B-channels, then a –1 in the numberOfBChannels attribute indicates that 18 B-channels may be used from the first AccessPort and that 24 may be used from the second AccessPort.";;

ATTRIBUTES

assignmentOfTimeslots GET-REPLACE;;;

CONDITIONAL PACKAGES

bearerServiceFor384kbpsDataPrimaryICPkg		PRESENT IF "an instance supports it.";

REGISTERED AS	{cAISDNObjectClass 21};

### 6.1.2	Bearer Service For 1536 kbit/s DataXE "Bearer Service For 1536 kbit/s Data"§

bearerServiceFor1536kbpsData	MANAGED OBJECT CLASS

DERIVED FROM				"ITU-T Rec. 824.0":bearerService;

CHARACTERIZED BY

bearerServiceFor1536kbpsDataPkg				PACKAGE

BEHAVIOUR

bearerServiceFor1536kbpsDataBhv		BEHAVIOUR

DEFINED AS	"This object class represents the characteristics of the Circuit-Mode, 1536 kbit/s Unrestricted Digital Transmission, 8000 Hz Structured, Demand, Point-To-Point, and Bidirectional Symmetric bearer service. This bearer service and the associated values for the I.210 Information Transfer and Access attributes, are identified in I.231.7. A call using this bearer service must use channels from a single Access Port.

The numberOfBChannels attribute inherited from the bearerService superclass must indicate either all B-channels are available (–1) or must be a mutiple of 24 to indicate that only some of the underlying B-channels may be used for this bearer service. If all B-channels are indicated by the numberOfBChannels attribute, then on each related Access Port the largest mutiple of 24 less than or equal to the total number of B-channels on each AccessPort may be used for this service. For example, if this service is associated with two AccessPort objects, one with 23 B-channels and the other with 24 B-channels, then a –1 in the numberOfBChannels attribute indicates that 24 B-channels from the second Access Port may be used (and no B-channels may be used from the Access Port with only 23 B-channels).";;;;

CONDITIONAL PACKAGES

bearerServiceFor1536kbpsDataPrimaryICPkg		PRESENT IF "an instance supports it.",

assignmentOfTimeslotsPkg		PRESENT IF "2Mb/sec interfaces are supported and an instance supports it.";

REGISTERED AS	{cAISDNObjectClass 22};

### 6.1.3	Bearer Service For 1920 kbit/s DataXE "Bearer Service For 1920 kbit/s Data"§

bearerServiceFor1920kbpsData	MANAGED OBJECT CLASS

DERIVED FROM				"ITU-T Rec. 824.0":bearerService;

CHARACTERIZED BY

bearerServiceFor1920kbpsDataPkg					PACKAGE

BEHAVIOUR

bearerServiceFor1920kbpsDataBhv			BEHAVIOUR

DEFINED AS	"This object class represents the characteristics of the Circuit-Mode, 1920 kbit/s Unrestricted Digital Transmission, 8000 Hz Structured, Demand, Point-To-Point, and Bidirectional Symmetric bearer service. This bearer service and the associated values for the I.210 Information Transfer and Access attributes, are identified in I.231.8. A call using this bearer service must use channels from a single Access Port.";;;;

CONDITIONAL PACKAGES

bearerServiceFor1920kbpsDataPrimaryICPkg 			PRESENT IF "an instance supports it.";

REGISTERED AS	{cAISDNObjectClass 23};

### 6.1.4	Bearer Service For Multiple-Rate DataXE "Bearer Service For Multiple-Rate Data"§

bearerServiceForMuliple-RateData			MANAGED OBJECT CLASS

DERIVED FROM						"ITU-T Rec. 824.0":bearerService;

CHARACTERIZED BY

bearerServiceForMultipleRateDataPkg				PACKAGE

BEHAVIOUR

bearerServiceForMultipleRateDataBhv		BEHAVIOUR

DEFINED AS	"This object class represents the characteristics of the Circuit-Mode, Multiple-Rate Unrestricted Digital Transmission, 8000 Hz Structured, Demand, Point-To-Point, and Bidirectional Symmetric bearer service. This bearer service and the associated values for the I.210 Information Transfer and Access attributes, are identified in I.231.10. A call using this bearer service must use channels from single physical interfaces as modeled by Access Port.

The numberOfBChannels attribute inherited from the bearerService superclass must indicate either all B-channels are available (–1) or must indicate the maximum number of B-channels that may be used for this service.";;

ATTRIBUTES

assignmentOfTimeslots GET-REPLACE;;;

CONDITIONAL PACKAGES

bearerServiceForMultipleRateDataPrimaryICPkg		PRESENT IF "an instance supports it.";

REGISTERED AS	{cAISDNObjectClass 24};

### 6.1.5	Circuit 3.1 kHzXE "Circuit 3.1 kHz"§

The 3.1 kHz audio bearer service object class  specifies the characteristics of ISDN audio services.

circuit3D1kHz			MANAGED OBJECT CLASS

DERIVED FROM		"ITU-T Rec. Q.824.0":bearerService;

CHARACTERIZED BY

circuit3D1kHzPkg				PACKAGE

BEHAVIOUR

circuit3D1KHzBhv		BEHAVIOUR

DEFINED AS	"This object class and its superclass, bearerService, represent the characteristics of the Circuit-Mode, 64 kbit/s, 8 kHz Structured, Usable for Circuit 3.1 kHz Information Transfer bearer service. This bearer service and the associated values for the I.210 Information Transfer and Access attributes, are identified in I.231.3.";;;;

CONDITIONAL PACKAGES

bearerServiceForAudioICPkg			PRESENT IF "inter-exchange carrier subscription is supported.",

networkProvidedTonePkg			PRESENT IF "Network Provided Tone subscription is supported.";

REGISTERED AS	{cAISDNObjectClass 25};

### 6.1.6	Circuit Combined Switched Digital DataXE "Circuit Combined Switched Digital Data"§

This object class is a subclass derived from the Bearer Service Circuit Unrestricted Digital Data Rate Adapted From 56 kbit/s and Bearer Service Circuit Unrestricted Digital Data. The object class thereby represents the combined characteristics of both of its superclasses. An instance of this object class cannot coexist with an instance of either its superclasses for the same directory number assignment.

circuitCombinedSwitchedDigitalData		MANAGED OBJECT CLASS

DERIVED FROM

circuitUnrestrictedDigitalData,

circuitUnrestrictedDigitalData;

CHARACTERIZED BY

circuitCombinedSwitchedDigitalDataPkg				PACKAGE

BEHAVIOUR

circuitCombinedSwitchedDigitalDataBhv		BEHAVIOUR

DEFINED AS	"This object  class represents a grouping of the following bearer services:

–		Circuit-Mode, 64 kbit/s Unrestricted Digital Data Transmission, 8 kHz Structured bearer service,

–		Circuit-Mode, 64 kbit/s Unrestricted Digital Data Transmission adapted from 56 kbit/s, 8 kHz Structured, Demand, Point-to-Point, and Bidirectional Symmetric bearer service.

The grouping of these circuit-mode bearer services is required when the services choice or resources allocation are the same whether the capability is 64 kbit/s or adapted 56 kbit/s. This object class is a subclass of the bearer Service Restricted Data and bearer Service unrestricted Data object classes and inherits the characteristics and behaviours from both superclasses.";;;;

REGISTERED AS	{cAISDNObjectClass 26};

### 6.1.7	Circuit Combined Voice BandXE "Circuit Combined Voice Band"§

This object class is a subclass derived from the Bearer Service Circuit 3.1 kHz and Bearer Service Speech object classes by multiple inheritance. The object class thereby represents the combined characteristics of both of its superclasses. An instance of this object class cannot coexist with an instance of either its superclasses for the same directory number assignment.

circuitCombinedVoiceBand		MANAGED OBJECT CLASS

DERIVED FROM

circuit3D1kHz,

circuitSpeech;

CHARACTERIZED BY

circuitCombinedVoiceBandPkg				PACKAGE

BEHAVIOUR

circuitCombinedVoiceBandBhv		BEHAVIOUR

DEFINED AS	"This object class represents a grouping of the characteristics of the following bearer services:

–	Circuit-Mode, 64 kbit/s, 8 kHz Structured, Usable for 3.1 kHz Audio Information Transfer bearer services.

–		Circuit-Mode, 64 kbit/s, 8 kHz Structured, Usable for Speech Demand, Point-to-Point, and Bidirectional Symmetric bearer service.

These bearer services are identified in I.231 and I.232. The grouping of these circuit-mode bearer services is required when the services choice or resources allocation are the same whether the capability is 3.1 KHz or speech. This object class is a subclass of thecircuitSpeech and bearer Service3kHzAudio superclasses and inherits the characteristics and behaviours from both superclasses.";;;;

REGISTERED AS	{cAISDNObjectClass 27};

### 6.1.8	Circuit Multi-UseXE "Circuit Multi-Use"§

The multi-use bearer Service object class specifies the characteristics of ISDN audio services.

circuitMultiUse		MANAGED OBJECT CLASS

DERIVED FROM 		"ITU-T Rec. Q.824.0":bearerService;

CHARACTERIZED BY

circuitMultiUsePkg				PACKAGE

BEHAVIOUR

circuitMultiUseBhv		BEHAVIOUR

DEFINED AS	"This object class and its superclass, bearerService, represent the characteristics of the Circuit-Mode, 64 kbit/s, 8 kHz Structured, Usable for MultiUse Information Transfer bearer service. This bearer service and associated values for the I.210 Information Transfer and Access attributes, are identified in I.231.9.";;;;

CONDITIONAL PACKAGES

networkProvidedTonePkg		PRESENT IF "Network Provided Tone Subscription is supported.",

circuitMultiUsePrimaryICPkg		PRESENT IF "inter-exchange carrier subscription is supported.";

REGISTERED AS	{cAISDNObjectClass 28};

### 6.1.9	Bearer Service For 64 kbit/s Data (Unrestricted) Rate Adapted From 56 kbit/sXE "Bearer Service For 64 kbit/s Data (Unrestricted) Rate Adapted From 56 kbit/s"§

circuitUnrestrictedDigitalDataRateAdaptedFrom56 kbit/s		MANAGED OBJECT CLASS

DERIVED FROM		"ITU-T Rec. 824.0":bearerService;

CHARACTERIZED BY

circuitUnrestrictedDigitalDataRateAdaptedFrom56kbpsPkg				PACKAGE

BEHAVIOUR

circuitUnrestrictedDigitalDataRateAdaptedFrom56kbpsBhv		BEHAVIOUR

DEFINED AS	"This object represents the characteristics of the Circuit-Mode, 64 kbit/s Unrestricted bearer service as defined in I.231 and I.232 rate adapted to 56 kbit/s according to V.110.";;;;

CONDITIONAL PACKAGES

circuitUnrestrictedDigitalDataPrimaryICPkg	PRESENT IF "inter-exchange carrier subscription is supported.";

REGISTERED AS	{cAISDNObjectClass 29};

### 6.1.10	Circuit SpeechXE "Circuit Speech"§

The Circuit speech bearer service object class specifies the characteristics of ISDN speech services.

circuitSpeech			MANAGED OBJECT CLASS

DERIVED FROM		"ITU-T Rec. Q.824.0":bearerService;

CHARACTERIZED BY

circuitSpeechPkg					PACKAGE

BEHAVIOUR

circuitSpeechBhv			BEHAVIOUR

DEFINED AS	"This object class represents the characteristics of the Circuit-Mode, 64 kbit/s, 8 kHz Structured, Usable for Speech bearer service. This bearer service, and the associated values for the I.210 Information Transfer and Access attributes, are identified in I.231.2. This bearer service is intended to support network characteristics appropriate for speech, such as 4-wire analog transmission, low bit rate voice encoding, and Time Assignment Speech Interpolation (TASI) techniques.";;;;

CONDITIONAL PACKAGES

networkProvidedTonePkg			PRESENT IF "Network Provided Tone Subscription is supported.",

circuitSpeechPrimaryICPkg			PRESENT IF "inter-exchange carrier subscription is supported.";

REGISTERED AS	{cAISDNObjectClass 30};

### 6.1.11	Circuit Unrestricted Digital DataXE "Circuit Unrestricted Digital Data"§

The unrestricted digital data bearer service object class specifies the characteristics of ISDN circuit mode data calls using unrestricted digital facilities.

circuitUnrestrictedDigitalData			MANAGED OBJECT CLASS

DERIVED FROM		"ITU-T Rec. Q.824.0":bearerService;

CHARACTERIZED BY

circuitUnrestrictedDigitalDataPkg	PACKAGE

BEHAVIOUR

circuitUnrestrictedDigitalDataBhv	BEHAVIOUR

DEFINED AS	"This object class represents the characteristics of the Circuit-Mode, 64 kbit/s Unrestricted Digital Transmission, 8 kHz Structured bearer service. This bearer service and the associated values for the I.210 Transfer and Access attributes, are identified in I.231.1.";;;;

CONDITIONAL PACKAGES

circuitUnrestrictedDigitalDataPrimaryICPkg		PRESENT IF "inter-exchange carrier subscription is supported.";

REGISTERED AS	{cAISDNObjectClass 31};

### 6.1.12	PacketXE "Packet"§

The Packet bearer service object class specifies the characteristics of ISDN Packet services.

packet		MANAGED OBJECT CLASS

DERIVED FROM		"ITU-T Rec. Q.824.0":bearerService;

CHARACTERIZED BY

packetPkg				PACKAGE

BEHAVIOUR

packetBhv		BEHAVIOUR

DEFINED AS	"This object class represents the characteristics of the Packet bearer service. The transmission rate of the packet mode service is defined by the associated B- and D-channel characteristics. This bearer service and the associated values for the I.210 Transfer and Access attributes, are identified in I.232. This class is not instantiated, only its subclasses are instatiated.";;

ATTRIBUTES

notificationClass											GET-REPLACE,

layer2InfoEntityPtr											GET-REPLACE,

layer3InfoEntityPtr											GET-REPLACE;;;

REGISTERED AS	{cAISDNObjectClass 32};

### 6.1.13	Packet B-channelXE "Packet B-channel"§

packetBChannel		MANAGED OBJECT CLASS

DERIVED FROM					packet;

CHARACTERIZED BY

packetBChannelPkg					PACKAGE

BEHAVIOUR

packetPacketBChannelBhv	BEHAVIOUR

DEFINED AS	"This object class represents the characteristics of the Packet bearer service running over a B-channel. This bearer service and the associated values for the I.210 Information Transfer and Access attributes, are identified in I.232.";;

ATTRIBUTES

semiPermAccessPacketHandlerDefaultDN						GET-REPLACE;;;

REGISTERED AS	{cAISDNObjectClass 33};

### 6.1.14	Packet D-ChannelXE "Packet D-Channel"§

packetDChannel		MANAGED OBJECT CLASS

DERIVED FROM				packet;

CHARACTERIZED BY

packetDChannelPkg			PACKAGE

BEHAVIOUR

packetDChannelBhv	BEHAVIOUR

DEFINED AS	"This object class represents the characteristics of the Packet bearer service running over a D-channel. This bearer service and the associated values for the I.210 Information Transfer and Access attributes, are identified in I.232.";;;;

REGISTERED AS	{cAISDNObjectClass 34};

# 7	Operations Support Managed Object ClassesXE "Operations Support Managed Object Classes"§

## 7.1	Service Manager ISDNXE "Service Manager ISDN"§

serviceManagerISDN	MANAGED OBJECT CLASS

DERIVED FROM		"ITU-T Rec. Q.824.0":serviceManager;

CHARACTERIZED BY

serviceManagerISDNPkg				PACKAGE

BEHAVIOUR

serviceManagerISDNBhv		BEHAVIOUR

DEFINED AS "The serviceManagerISDN will perform the actions that establish and remove ISDN accesses, services and terminals. In addition, the serviceManagerISDN will perform actions that will change a directory number.";;

ACTIONS

establishISDNAccess invalidReferenceError,

removeISDNAccess invalidReferenceError,

establishISDNService invalidReferenceError,

removeISDNService invalidReferenceError,

changeDirectoryNumber invalidReferenceError;;;

CONDITIONAL PACKAGES

manageISDNTerminalPackage		PRESENT IF "supported by Administration.";

REGISTERED AS	{cAISDNObjectClass 35};

## 7.2	Service Manager Retrieve ServiceXE "Service Manager Retrieve Service"§

serviceManagerRetrieveService			MANAGED OBJECT CLASS

DERIVED FROM			"ITU-T Rec. Q.824.0":serviceManager;

CHARACTERIZED BY

serviceManagerRetrieveServicePkg				PACKAGE

BEHAVIOUR

serviceManagerRetrieveServiceBhv		BEHAVIOUR

DEFINED AS	"The serviceManagerCustService is service independent managed object whose action will retrieve all service information related to a given customer service.";;

ACTIONS

retrieveService invalidReferenceError;;;

REGISTERED AS	{cAISDNObjectClass 36};

# 8	Package TemplatesXE "Package Templates"§

## 8.1	Active Terminal ListXE "Active Terminal List"§

activeTerminalListPkg		PACKAGE

BEHAVIOUR activeTerminalListPkgBhv	BEHAVIOUR

DEFINED AS	"This package provides information on the terminal currently active on the access port profile. The activeTerminalList attribute is a get only attribute that is set when a terminal initializes on the access port profile. For terminals with fixed TEI, the TEI is recorded when the terminal is in the Mutliframe Established state. For automatic terminals, the TEI is set when the terminals enter the TEI assigned state. For fully initializing terminals (those using the optional procedures of Q.932), the SPID and USID may also be recorded in this attribute when the terminal initializes.";;

ATTRIBUTES

activeTerminalList GET;

REGISTERED AS	{cAISDNPackage 1};

## 8.2	Assignment Of Timeslots

assignmentOfTimeslotsPkg			PACKAGE

BEHAVIOUR

assignmentOfTimeslotsBhv	BEHAVIOUR

DEFINED AS	"This package provides an attribute whose value is used to determine how wideband channel selection is done."

ATTRIBUTES

assignmentOfTimeslots										GET-REPLACE;

REGISTERED AS	{cAISDNPackage 2};

## 8.3	Automatic NegotiationXE "Automatic Negotiation"§

automaticNegotiationPkg		PACKAGE

ATTRIBUTES

automaticXIDnotification										GET-REPLACE,

signallingParameterNegotiation								GET-REPLACE;

REGISTERED AS	{cAISDNPackage 3};

## 8.4	Bearer Service For Audio ICXE "Bearer Service For Audio IC"§

bearerServiceForAudioICPkg		PACKAGE

ATTRIBUTES

circuitAudioPrimaryIC										GET-REPLACE;

REGISTERED AS	{cAISDNPackage 4};

## 8.5	Bearer Service For 384 kbit/s Data Primary ICXE "Bearer Service For 384 kbit/s Data Primary IC"§

bearerServiceFor384kbpsDataPrimaryICPkg					PACKAGE

BEHAVIOUR

bearerServiceFor384kbpsDataPrimaryICPkgBhv		BEHAVIOUR

DEFINED AS	"This package provides an attribute whose value identifies the Inter-Exchange Carrier assigned to the subscriber service this object represents.";;

ATTRIBUTES

bearerServiceFor384kbpsDataPrimary IC						GET-REPLACE;

REGISTERED AS	{cAISDNPackage 5};

## 8.6	Bearer Service For 1536 kbit/s Data Primary ICXE "Bearer Service For 1536 kbit/s Data Primary IC"§

bearerServiceFor1536kbpsDataPrimaryICPkg				PACKAGE

BEHAVIOUR

bearerServiceFor1536kbpsDataPrimaryICPkgBhv		BEHAVIOUR

DEFINED AS	"This package provides an attribute whose value identifies the Inter-Exchange Carrier assigned to the subscriber service this object represents.";;

ATTRIBUTES

bearerServiceFor1536kbpsDataPrimary IC						GET-REPLACE;

REGISTERED AS	{cAISDNPackage 6};

## 8.7	Bearer Service For 1920 kbit/s Data Primary ICXE "Bearer Service For 1920 kbit/s Data Primary IC"§

bearerServiceFor1920kbpsDataPrimaryICPkg				PACKAGE

BEHAVIOUR

bearerServiceFor1920kbpsDataPrimaryICPkgBhv		BEHAVIOUR

DEFINED AS	"This package provides an attribute whose value identifies the Inter-Exchange Carrier assigned to the subscriber service this object represents.";;

ATTRIBUTES

bearerServiceFor1920kbpsDataPrimary IC						GET-REPLACE;

REGISTERED AS	{cAISDNPackage 7};

## 8.8	Bearer Service For Multiple-Rate Data Primary ICXE "Bearer Service For Multiple-Rate Data Primary IC"§

bearerServiceForMultipleRateDataPrimaryICPkg				PACKAGE

BEHAVIOUR

bearerServiceForMultipleRateDataPrimaryICPkgBhv	BEHAVIOUR

DEFINED AS	"This package provides an attribute whose value identifies the Inter-Exchange Carrier assigned to the subscriber service this object represents.";;

ATTRIBUTES

bearerServiceForMultipleRateDataPrimary IC					GET-REPLACE;

REGISTERED AS	{cAISDNPackage 8};

## 8.9	Bearer Service ListXE "Bearer Service List"§

bearerServiceListPkg	PACKAGE

ATTRIBUTES

bearerServiceList 											GET-REPLACE

ADD-REMOVE;

REGISTERED AS	{cAISDNPackage 9};

## 8.10	Call ReferenceXE "Call Reference"§

callReferencePkg		PACKAGE

BEHAVIOUR

callReferencePkgBhv	BEHAVIOUR

DEFINED AS "The maxNumberOfCallReference attribute indicates the maximum number of the simultaneous layer 3 connections for signalling (SAPI = 0 for all TEIs on this Access Port Profile). It must be at least as large as the largest callReferenceBusyLimit attribute (in ISDN Circuit Service Set managed object contained in the Bearer Services).";;

ATTRIBUTES

maxNumberOfCallReference									GET-REPLACE;

REGISTERED AS	{cAISDNPackage 10};

## 8.11	Circuit Speech Primary ICXE "Circuit Speech Primary"§

circuitSpeechPrimaryICPkg		PACKAGE

ATTRIBUTES

circuitSpeechPrimaryIC										GET-REPLACE;

REGISTERED AS	{cAISDNPackage 11};

## 8.12	Circuit Multi-Use Primary ICXE "Circuit Multi-User Primary"§

circuitMultiUsePrimaryICPkg		PACKAGE

ATTRIBUTES

circuitMultiUsePrimaryIC									GET-REPLACE;

REGISTERED AS	{cAISDNPackage 12};

## 8.13	Circuit Unrestricted Digital Data Primary ICXE "Circuit Unrestricted Digital Data Primary IC"§

circuitUnrestrictedDigitalDataPrimaryICPkg		PACKAGE

ATTRIBUTES

circuitUnrestrictedDigitalDataPrimaryIC 						GET-REPLACE;

REGISTERED AS	{cAISDNPackage 13};

## 8.14	Circuit Unrestricted Digital Data Rate Adapted From 56 kbit/s Primary ICXE "Circuit Unrestricted Digital Data Rate Adapted From 56 kbit/s Primary IC"§

circuitUnrestrictedDigitalDataRateAdaptedFrom56kbpsPrimaryICPkg		PACKAGE

ATTRIBUTES

circuitUnrestrictedDigitalDataRateAdaptedFrom56kbpsPrimaryIC	GET-REPLACE;

REGISTERED AS	{cAISDNPackage 14};

## 8.15	Deactivation CapabilitiesXE "Deactivation Capabilities"§

deactivationCapabilitiesPkg		PACKAGE

ATTRIBUTES

deactivationCapabilities 										GET-REPLACE;

REGISTERED AS	{cAISDNPackage 15};

## 8.16	Link SettingXE "Link Setting"§

linkSettingPkg		PACKAGE

ATTRIBUTES

linkLevelWindowSize										GET-REPLACE,

maxBitsPerInformationFrame									GET-REPLACE,

maxTransmissionAttempts									GET-REPLACE;

REGISTERED AS	{cAISDNPackage 16};

## 8.17	Manage ISDN TerminalXE "Manage ISDN Terminal"§

manageISDNTerminalPackage	PACKAGE

ACTIONS

establishISDNTerminal invalidReferenceError,

removeISDNTerminal invalidReferenceError;

REGISTERED AS	{cAISDNPackage 17};

## 8.18	Max Combined Thruput B-ChannelXE "Max Combined Thruput B-Channel"§

maxCombinedThruputBChanPkg				PACKAGE

BEHAVIOUR

maxCombinedThruputBChanPkgBhv 	BEHAVIOUR

DEFINED AS	"This package identifies the maximum combined throughput permitted on the B-Channel.";;

ATTRIBUTES

maxCombinedThruputClass 									REPLACE-WITH-DEFAULT

DEFAULT VALUE CAISDNModule.null 						GET-REPLACE;

REGISTERED AS	{cAISDNPackage 18};

## 8.19	Max Combined Thruput D-ChannelXE "Max Combined Thruput D-Channel"§

maxCombinedThruputDChanPkg				PACKAGE

BEHAVIOUR

maxCombinedThruputDChanPkgBhv	BEHAVIOUR

DEFINED AS	"This package identifies the maximum combined throughput permitted on the D-Channel.";;

ATTRIBUTES

maxCombinedThruputClass									REPLACE-WITH-DEFAULT

DEFAULT VALUE CAISDNModule.null  						GET-REPLACE;

REGISTERED AS	{cAISDNPackage 19};

## 8.20	Network Provided ToneXE "Network Provided Tone"§

networkProvidedTonePkg		PACKAGE

ATTRIBUTES

networkProvidedTones										REPLACE-WITH-DEFAULT

DEFAULT VALUE CAISDNModule.true						GET-REPLACE;

REGISTERED AS	{cAISDNPackage 20};

## 8.21	nT2ISDN Access Port ProfileXE "nT2ISDN Access Port Profile"§

nT2ISDNaccessPortProfilePkg					PACKAGE

BEHAVIOUR

nT2ISDNAccessPortProfilePkgBhv		BEHAVIOUR

DEFINED AS	"This object package represents the characteristics of the ISDN Access Port Profile object class to be instantied for support of a nT2 (class 2) ISDN interface configuration.";;

ATTRIBUTES

earlyCutThruUserProvidedAudibleRing 							GET-REPLACE,

earlyCutThruRemoteNetworkInterwork							GET-REPLACE;

REGISTERED AS	{cAISDNPackage 21};

## 8.22	Number Of D-Channel LinksXE "Number Of D-Channel Links"§

numberOfDChannelLinksPkg					PACKAGE

BEHAVIOUR

numberOfDChannelLinksPkgBhv		BEHAVIOUR

DEFINED AS	"The linkOption attribute is a choice of fixed or an identification of the values for the 4 attributes callControlTEItotal, callControlTEIsw, packTEItotal and packTEIsw. The selection of fixed links (which can be represented by null) would indicate that there is exactly one signalling data link connection at sapi 0, TEI 0; and one packet link at sapi 16, TEI 0. The number of signalling links for the fixed option is 3. This includes one link for the signalling access controller at sapi 63, TEI 127.

The dynamic choice is the default with callControlTEItotal and packTEItotal set to 127 and callControlTEIsw and packTEIsw set to 63. Where:

callControlTEItotal is the total number of TEIs that can be assigned for Call Control,

callControlTEIsw is either null or is the number of TEIs that can be assigned by the switch for Call Control. The null value indicates that the way TEIs are assigned is not controlled.

packTEItotal is the total number of TEIs that can be assigned for packet,

packTEIsw is either null or is the number of TEIs that can be assigned by the switch for packet. The null value indicates that the way TEIs are assigned is not controlled.

The number of signalling links must be less than or equal to the sum of callControlTEItotal and packetTEItotal.";;

ATTRIBUTES

linkOption												REPLACE-WITH-DEFAULT

DEFAULT VALUE CAISDNModule.linkOptionDefault				GET-REPLACE,

numberOfDChannelLinks									GET-REPLACE;

REGISTERED AS	{cAISDNPackage 22};

## 8.23	Optional DeactivationXE "Optional Deactivation"§

optionalDeactivationPkg			PACKAGE

ATTRIBUTES

deactivationCapabilities										GET-REPLACE;

REGISTERED AS	{cAISDNPackage 23};

## 8.24	Selection ProceduresXE "Selection Procedures"§

selectionProceduresPkg	PACKAGE

ATTRIBUTES

channelSelection											GET-REPLACE;

REGISTERED AS	{cAISDNPackage 24};

## 8.25	DCE TimersXE "DCE Timers"§

x25DCETimersPkg		PACKAGE

ATTRIBUTES

"ITU-T Rec. X.282":t1Timer 									GET-REPLACE,

"ITU-T Rec. X.282":t2Timer 									GET-REPLACE,

"ITU-T Rec. X.282":t3Timer 									GET-REPLACE,

"ITU-T Rec. X.282":t4Timer 									GET-REPLACE,

"ITU-T Rec. X.282":n1 										GET-REPLACE,

"ITU-T Rec. X.282":n2 										GET-REPLACE,

"ITU-T Rec. X.282":k 										GET-REPLACE;

REGISTERED AS	{cAISDNPackage 25};

# 9	Attribute templatesXE "Attribute templates"§

This clause contains the ASN.1 definitions for all attributes in the described object classes. These definitions identify the function of the attributes and their valid characteristics, such as their valid values, interdependencies, read/write constraints, etc. The attributes are identified by their ASN.1 descriptors.

## 9.1	Access Channel Pointer ListXE "Access Channel Pointer List"§

accessChannelPtrList		ATTRIBUTE

WITH ATTRIBUTE SYNTAX

CAISDNAttributeModule.AccessChannelPtrList;

MATCHES FOR SET-INTERSECTION, SET-COMPARISON;

DEFINED AS	"This is a set-valued attribute whose value(s) points to one or more instances of the Access Channel object class.";;

REGISTERED AS	{cAISDNAttribute 1};

## 9.2	Active Terminal ListXE "Active Terminal List"§

activeTerminalList		ATTRIBUTE

WITH ATTRIBUTE SYNTAX

CAISDN Module.ActiveTerminalList;

MATCHES FOR EQUALITY;

BEHAVIOUR

activeTerminalListBhv	BEHAVIOUR

DEFINED AS	"This attribute identifies the active terminals on the interface by the spid, tei., and usid. It is used to support maintenance. Spid and usid are allowed to be NULL if spid initialization procedures are not used.";;

REGISTERED AS	{cAISDNAttribute 2};

## 9.3	Assignment Of TimeslotsXE "Assignment Of Timeslots"§

assignmentOfTimeslots		ATTRIBUTE

WITH ATTRIBUTE SYNTAX

CAISDNModule.AssignmentOfTimeslots;

MATCHES FOR EQUALITY;

BEHAVIOUR

AssignmentOfTimeslotsBehaviour		BEHAVIOUR

DEFINED AS	"This attribute identifies how timeslots are assigned for wideband (e.g. 384k/sec) bearer services. The channels must always be with a single physical interface (as modeled by access port). However, within the interface the channels may be assigned to:

–	fixed positions of contiguous channels (such as specified by Annex A/I.431 and Annex B/I.431);

–	loating positions of contiguous channels (the channels must be contiguous but may start at any position within the physical interface that can support that number of contiguous channels;

–	flexibly to any channels available within the physical interface without regard to whether the channels are contiguous or not.

Note that this attribute controls how calls are offered to subscribers. It is a network provider option what channel assignments are accepted from subscribers.";;

REGISTERED AS	{cAISDNAttribute 3};

## 9.4	Automatic XID NotificationXE "Automatic XID Notification"§

automaticXIDnotification			ATTRIBUTE

WITH ATTRIBUTE SYNTAX

CAISDNAttributeModule.Boolean;

MATCHES FOR EQUALITY;

BEHAVIOUR

automaticXIDnotificationBhv		BEHAVIOUR

DEFINED AS	"The XID frames may be exchanged only after the link has been made active. Exchange of XID frames before the link is made active is controlled by "automaticXIDnotification". This Boolean value attribute controls the exchange of XID frames between the switch and the user equipment. A "True" value of the attribute indicates that XID frames can be exchanged between the link controller in the switch and user equipment. A "False" value of the attribute indicates that XID frames cannot be exchanged.";;

REGISTERED AS	{cAISDNAttribute 4};

## 9.5	Bearer Service For 384 kbit/s Data Primary ICXE "Bearer Service For 384 kbit/s Data Primary IC"§

bearerServiceFor384kbpsDataPrimaryIC		ATTRIBUTE

WITH ATTRIBUTE SYNTAX

CAISDNModule.PrimaryIC;

MATCHES FOR EQUALITY;

BEHAVIOUR

bearerServiceFor384kbpsDataPrimaryICBhv		BEHAVIOUR

DEFINED AS	"This attribute identifies the Primary Inter-Exchange Carrier (PIC) selected by the ISDN 384 kbit/s Data Service Subscriber.";;

REGISTERED AS	{cAISDNAttribute 5};

## 9.6	Bearer Service For 1536 kbit/s Data Primary ICXE "Bearer Service For 1536 kbit/s Data Primary IC"§

bearerServiceFor1536kbpsDataPrimaryIC		ATTRIBUTE

WITH ATTRIBUTE SYNTAX

CAISDNModule.PrimaryIC;

MATCHES FOR EQUALITY;

BEHAVIOUR

bearerServiceFor1536kbpsDataPrimaryICBhv	BEHAVIOUR

DEFINED AS	"This attribute identifies the Primary Inter-Exchange Carrier (PIC) selected by the ISDN 1536 kbit/s Data Service Subscriber.";;

REGISTERED AS	{cAISDNAttribute 6};

## 9.7	Bearer Service For 1920 kbit/s Data Primary ICXE "Bearer Service For 1920 kbit/s Data Primary IC"§

bearerServiceFor1920DataPrimaryIC		ATTRIBUTE

WITH ATTRIBUTE SYNTAX

CAISDNModule.PrimaryIC;

MATCHES FOR EQUALITY;

BEHAVIOUR

bearerServiceFor1920kbpsDataPrimaryICBhv	BEHAVIOUR

DEFINED AS	"This attribute identifies the Primary Inter-Exchange Carrier (PIC) selected by the ISDN 1920 kbit/s Data Service Subscriber.";;

REGISTERED AS	{cAISDNAttribute 7};

## 9.8	Bearer Service For Multiple-Rate Data Primary ICXE "Bearer Service For Multiple-Rate Data Primary IC"§

bearerServiceForMultipleRateDataPrimaryIC		ATTRIBUTE

WITH ATTRIBUTE SYNTAX

CAISDNModule.PrimaryIC;

MATCHES FOR EQUALITY;

BEHAVIOUR

bearerServiceForMultipleRateDataPrimaryICBhv		BEHAVIOUR

DEFINED AS	"This attribute identifies the Primary Inter-Exchange Carrier (PIC) selected by the ISDN Multiple-Rate kbit/s Data Service Subscriber.";;

REGISTERED AS	{cAISDNAttribute 8};

## 9.9	Bearer Service ListXE "Bearer Service List"§

bearerServiceList	ATTRIBUTE

WITH ATTRIBUTE SYNTAX

CAISDNAttributeModule.BearerServiceList;

MATCHES FOR EQUALITY, SET-COMPARISON, SET-INTERSECTION;

BEHAVIOUR

bearerServiceListBhv	BEHAVIOUR

DEFINED AS	"The bearerServiceList attribute is a list of one or more bearerService types.";;

REGISTERED AS	{cAISDNAttribute 9};

## 9.10	Bit Rate Of Primary Rate InterfaceXE "Bit Rate Of Primary Rate Interface"§

bitRateOfPrimaryRateInterface	ATTRIBUTE

WITH ATTRIBUTE SYNTAX

CAISDNAttributeModule.BitRateOfPrimaryRateInterface;

MATCHES FOR SET-INTERSECTION, SET-COMPARISON;

BEHAVIOUR

bitRateOfPrimaryRateInterfaceBhv	BEHAVIOUR

DEFINED AS	"This attribute indicates bit rate of Primary rate interface. When the value of this attribute is "rate1544", the bit rate of interface is 1544 kbit/s.When the value of this attribute is "rate2048", the bit rate of interface is 2048 kbit/s.";;

REGISTERED AS	{cAISDNAttribute 10};

## 9.11	Calling Number Screening IdXE "Calling Number Screening Id"§

callingNumberScreeningId		ATTRIBUTE

WITH ATTRIBUTE SYNTAX CAISDNModule.NameType;

MATCHES FOR EQUALITY, ORDERING, SUBSTRINGS;

BEHAVIOUR

callingNumberScreeningIdBhv		BEHAVIOUR

DEFINED AS "This is the naming attribute of the calling number screening managed object. If the string choice for the syntax is used, matching on the substings is permitted. If the number choice for the syntax is used, then matching on the ordering is permitted.";;

REGISTERED AS	{cAISDNAttribute 11};

## 9.12	Catalogued Layer Entity LAPD IdXE "Catalogued Layer Entity LAPD Id"§

cataloguedLayerEntityLAPDId		ATTRIBUTE

WITH ATTRIBUTE SYNTAX

CACommonModule.NameType;

MATCHES FOR EQUALITY, ORDERING, SUBSTRINGS;

BEHAVIOUR

cataloguedLayerEntityLAPDIdBhv			BEHAVIOUR

DEFINED AS	"This is a naming attribute of the Access Channel managed object. If the string choice for the syntax is used, matching on the substrings is permitted. If the number choice for the syntax is used, then matching on ordering is permitted.";;

REGISTERED AS	{cAISDNAttribute 12};

## 9.13	Circuit Audio Primary ICXE "Circuit Audio Primary IC"§

circuitAudioPrimaryIC		ATTRIBUTE

WITH ATTRIBUTE SYNTAX

CAISDNAttributeModule.PrimaryIC;

MATCHES FOR EQUALITY;

BEHAVIOUR

circuitAudioPrimaryICBhv	BEHAVIOUR

DEFINED AS	"This attribute represents the Primary Inter-Exchange Carrier (PIC) selected by the bearerServiceAudio service subscriber.";;

REGISTERED AS	{cAISDNAttribute 13};

## 9.14	Circuit Multi-Use Primary ICXE "Circuit Multi-User Primary IC"§

circuitMultiUsePrimaryIC		ATTRIBUTE

WITH ATTRIBUTE SYNTAX

CAISDNAttributeModule.PrimaryIC;

MATCHES FOR EQUALITY;

BEHAVIOUR

circuitMultiUsePrimaryICBhv		BEHAVIOUR

DEFINED AS	"This attribute represents the Primary Inter-Exchange Carrier (PIC) selected by the circuitMultiUse service subscriber.";;

REGISTERED AS	{cAISDNAttribute 14};

## 9.15	Circuit Unrestricted Digital Data Rate Adapted From 56 kbit/s Primary ICXE "Circuit Unrestricted Digital Data Rate Adapted From 56 kbit/s Primary IC"§

circuitUnrestrictedDigitalDataRateAdaptedFrom56kbpsPrimaryIC		ATTRIBUTE

WITH ATTRIBUTE SYNTAX

CAISDNModule.PrimaryIC;

MATCHES FOR EQUALITY;

BEHAVIOUR

circuitUnrestrictedDigitalDataRateAdaptedFrom56kbpsPrimaryICBhv   BEHAVIOUR

DEFINED AS	"This attribute represents the Primary Inter-Exchange Carrier (PIC) selected by the circuitUnrestrictedDigitalDataRateAdaptedFrom56kbps service subscriber.";;

REGISTERED AS	{cAISDNAttribute 15};

## 9.16	Circuit Speech Primary ICXE "Circuit Speech Primary IC"§

circuitSpeechPrimaryIC			ATTRIBUTE

WITH ATTRIBUTE SYNTAX

CAISDNAttributeModule.PrimaryIC;

MATCHES FOR EQUALITY;

BEHAVIOUR

circuitSpeechPrimaryICBhv		BEHAVIOUR

DEFINED AS	"This attribute represents the Primary Inter-Exchange Carrier (PIC) selected by the CircuitSpeech service subscriber.";;

REGISTERED AS	{cAISDNAttribute 16};

## 9.17	Circuit Unrestricted Digital Data Primary ICXE "Circuit Unrestricted Digital Data Primary IC"§

circuitUnrestrictedDigitalDataPrimaryIC		ATTRIBUTE

WITH ATTRIBUTE SYNTAX

CAISDNAttributeModule.PrimaryIC;

MATCHES FOR EQUALITY;

BEHAVIOUR

circuitUnrestrictedDigitalDataPrimaryICBhv		BEHAVIOUR

DEFINED AS	"This attribute represents the Primary Inter-Exchange Carrier (PIC) selected by the circuitUnrestrictedDigitalData service subscriber.";;

REGISTERED AS	{cAISDNAttribute 17};

## 9.18	Calling Party Default Directory NumberXE "Calling Party Default Directory Number"§

callingPartyDefaultDN			ATTRIBUTE

WITH ATTRIBUTE SYNTAX

CAISDNAttributeModule.CallingPartyDefaultDirectoryNumber;

MATCHES FOR EQUALITY;

BEHAVIOUR

callingPartyDefaultDNBhv		BEHAVIOUR

DEFINED AS	"The attribute is applicable only if the Calling Party Number Provision Necessary attribute is set to the boolean value FALSE. The value(s) of this attribute is the calling party number default Directory Number(s) to be used for billing purposes when the calling party numbers are not provided by the CPE. The attribute value choices may be either one Default Directory Number with the choice ALL specified (indicating that the Default DN applies to all bearer services assigned to the Directory Number), or a list of Directory Numbers per bearer service. The attribute value set may be 1 to 3 sequences. The form of each sequence within the value set is: <Directory Number>,<bearer service>|ALL

The bearer service(s) type must be part of the set of values of the bearer service attribute.";;

REGISTERED AS	{cAISDNAttribute 18};

## 9.19	Calling Party Number Discard ControlXE "Calling Party Number Discard Control"§

callingPartyNumberDiscardCtrl				ATTRIBUTE

WITH ATTRIBUTE SYNTAX

CAISDNAttributeModule.Boolean;

MATCHES FOR EQUALITY;

BEHAVIOUR

callingPartyNumberDiscardCtrlBhv		BEHAVIOUR

DEFINED AS "When the value of this attribute is "TRUE" (default), the switch discards user supplied Directory Numbers, Directory Numbers not screened or that failed screening, and calling party numbers when they occur and uses a single default Directory Number as the calling party number. If the attribute value is "FALSE", the discard feature does not apply. If the Calling Party Number Provision Necessary attribute = "TRUE", then the value of this attribute should be "FALSE". This attribute is only applicable to ISDN services.";;

REGISTERED AS	{cAISDNAttribute 19};

## 9.20	Calling Party Number Provision NecessaryXE "Calling Party Number Provision Necessary"§

callingPartyNumberProvisionNecessary			ATTRIBUTE

WITH ATTRIBUTE SYNTAX

CAISDNAttributeModule.Boolean;

MATCHES FOR EQUALITY;

BEHAVIOUR

callingPartyNumberProvisionNecessaryBhv			BEHAVIOUR

DEFINED AS	"When the value of this attribute is FALSE, the call processing will accept any call originations from this access line without the calling party number information being given. When the value of this attribute is FALSE, the value of the callingPartyDefaultDN attribute is assumed as the originating directory number for billing purposes. The default value of this attribute is FALSE.

The attribute value TRUE means that calling party information is required by call processing and if the information is not provided by the CPE on a call origination, the call processing should reject the call.";;

REGISTERED AS	{cAISDNAttribute 20};

## 9.21	Calling Party Valid Directory Number ListXE "Calling Party Valid Directory Number List"§

callingPartyValidDirectoryNumberList			ATTRIBUTE

WITH ATTRIBUTE SYNTAX

CAISDNAttributeModule.CallingPartyValidDirectoryNumberList;

MATCHES FOR EQUALITY, SET-COMPARISON, SET-INTERSECTION;

BEHAVIOUR

callingPartyValidDirectoryNumberListBhv			BEHAVIOUR

DEFINED AS	"This set-value attribute identifies the individual directory numbers that originate calls from an instance of the ISDN Access Port object class.";;

REGISTERED AS	{cAISDNAttribute 21};

## 9.22	Catalogued Access Port ISDN Primary Rate IdXE "Catalogued Access Port ISDN Primary Rate Id"§

cataloguedAccessPortISDNPrimaryRateId		ATTRIBUTE

WITH ATTRIBUTE SYNTAX

CAISDNAttributeModule.NameType;

MATCHES FOR EQUALITY, ORDERING, SUBSTRINGS;

BEHAVIOUR

cataloguedAccessPortISDNPrimaryRateIdBhv		BEHAVIOUR

DEFINED AS	"This is a naming attribute. If the string choice for the syntax is used, matching on the substrings is permitted. If the number choice for the syntax is used, then matching on ordering is permitted.";;

REGISTERED AS	{cAISDNAttribute 22};

## 9.23	Catalogued Access Port Profile ISDN IdXE "Catalogued Access Port Profile ISDN Id"§

Recommendation Q.824.1     (10/95)

cataloguedAccessPortProfileISDNId			ATTRIBUTE

WITH ATTRIBUTE SYNTAX

CAISDNAttributeModule.NameType;

MATCHES FOR EQUALITY, ORDERING, SUBSTRINGS;

BEHAVIOUR

cataloguedAccessPortProfileISDNIdBhv			BEHAVIOUR

DEFINED AS	"This is a naming attribute. If the string choice for the syntax is used, matching on the substrings is permitted. If the number choice for the syntax is used, then matching on ordering is permitted.";;

REGISTERED AS	{cAISDNAttribute 23};

## 9.24	Catalogued Layer Entity DSS 1 IDXE "Catalogued Layer Entity DSS 1 ID"§

cataloguedLayerEntityDSS1Id			ATTRIBUTE

WITH ATTRIBUTE SYNTAX

CAISDNModule.NameType;

MATCHES FOR EQUALITY, ORDERING, SUBSTRINGS;

BEHAVIOUR

cataloguedLayerEntityDSS1IdBhv		BEHAVIOUR

DEFINED AS	"This is a naming attribute. If the string choice for the syntax is used, matching on the substrings is permitted. If the number choice for the syntax is used, then matching on ordering is permitted.";;

REGISTERED AS	{cAISDNAttribute 24};

## 9.25	Channel SelectionXE "Channel Selection"§

channelSelection		ATTRIBUTE

WITH ATTRIBUTE SYNTAX

CAISDNAttributeModule.ChannelSelection;

MATCHES FOR EQUALITY;

BEHAVIOUR

channelSelectionBhv		BEHAVIOUR

DEFINED AS	"This attribute is used to set the B-channel selection procedure from the network to the user. The selection procedure is shown in 5.2.3.1/Q.931. The value byNetwork(0) means that the channel is indicated by the network which corresponds to items 1) and 2) in 5.2.3.1/Q.931 and by user (1) means that any channel is acceptable which corresponds to item 3) in 5.2.3.1/Q.931.";;

REGISTERED AS	{cAISDNAttribute 25};

## 9.26	D-channel Packet Default Directory NumberXE "D-channel Packet Default Directory Number"§

dChannelPacketDefaultDirectoryNumber		ATTRIBUTE

WITH ATTRIBUTE SYNTAX

CAISDNAttributeModule.DirectoryNumber;

MATCHES FOR EQUALITY;

BEHAVIOUR

dChannelPacketDefaultDirectoryNumberBhv			BEHAVIOUR

DEFINED AS	"The value of this attribute is a Numeric string representing a particular default Directory Number (Directory Number) from the set of Directory Numbers listed in the DChannelDirectoryNumberList attribute. This Directory Number is used when no X.25 Calling Address is provided due to the caller using in-band procedures.";;

REGISTERED AS	{cAISDNAttribute 26};

## 9.27	D-Channel Packet Directory Number ListXE "D-Channel Packet Directory Number List"§

dChannelPacketDirectoryNumberList			ATTRIBUTE

WITH ATTRIBUTE SYNTAX

CAISDNAttributeModule.DirectoryNumberList;

MATCHES FOR EQUALITY;

BEHAVIOUR

dChannelPacketDirectoryNumberListBhv			BEHAVIOUR

DEFINED AS	"The value(s) of this attribute are 1 or more Numeric strings representing the Directory Numbers (Directory Number) that provide a Directory Number screening set for use over a D-Channel packet connection (LAPD SAPI address field set to Packet Control).";;

REGISTERED AS	{cAISDNAttribute 27};

## 9.28	D-Channel Primary PointerXE "D-Channel Primary Pointer"§

dChannelPrimaryPtr			ATTRIBUTE

WITH ATTRIBUTE SYNTAX

CAISDNAttributeModule.ObjectInstance;

MATCHES FOR EQUALITY, SET-COMPARISON, SET-INTERSECTION;

BEHAVIOUR

dChannelPrimaryPtrBhv		BEHAVIOUR

DEFINED AS	"The dChannelPrimaryPtr attribute of the ISDN Access Port Profile Primary Rate object class and identifies the primary D-Channel object.";;

REGISTERED AS	{cAISDNAttribute 28};

## 9.29	D-Channel Secondary PointerXE "D-Channel Secondary Pointer"§

dChannelSecondaryPtr			ATTRIBUTE

WITH ATTRIBUTE SYNTAX

CAISDNAttributeModule.ObjectInstance;

MATCHES FOR EQUALITY, SET-COMPARISON, SET-INTERSECTION;

BEHAVIOUR

dChannelSecondaryPtrBhv	BEHAVIOUR

DEFINED AS	"This is pointer to the optional backup D-Channel primary rate resource object instance for a Primary Rate Access.";;

REGISTERED AS	{cAISDNAttribute 29};

## 9.30	D-Channel T301XE "D-Channel T301"§

dChannelT301				ATTRIBUTE

WITH ATTRIBUTE SYNTAX

CAISDNModule.DChannelT3xx;

MATCHES FOR EQUALITY;

BEHAVIOUR

dChannelT301Bhv		BEHAVIOUR

DEFINED AS	"This attribute provides the value of call control timer T301 defined in Q.931.";;

REGISTERED AS	{cAISDNAttribute 30};

## 9.31	D-Channel T303XE "D-Channel T303"§

dChannelT303				ATTRIBUTE

WITH ATTRIBUTE SYNTAX

CAISDNModule.DChannelT3xx;

MATCHES FOR EQUALITY;

BEHAVIOUR

dChannelT303Bhv		BEHAVIOUR

DEFINED AS	"This attribute provides the value of call control timer T303 defined in Q.931.";;

REGISTERED AS	{cAISDNAttribute 31};

## 9.32	D-Channel T304XE "D-Channel T304"§

dChannelT304				ATTRIBUTE

WITH ATTRIBUTE SYNTAX

CAISDNModule.DChannelT3xx;

MATCHES FOR EQUALITY;

BEHAVIOUR

dChannelT304Bhv		BEHAVIOUR

DEFINED AS	"This attribute provides the value of call control timer T304 defined in Q.931.";;

REGISTERED AS	{cAISDNAttribute 32};

## 9.33	D-Channel T305XE "D-Channel T305"§

dChannelT305				ATTRIBUTE

WITH ATTRIBUTE SYNTAX

CAISDNModule.DChannelT3xx;

MATCHES FOR EQUALITY;

BEHAVIOUR

dChannelT305Bhv		BEHAVIOUR

DEFINED AS	"This attribute provides the value of call control timer T305 defined in Q.931.";;

REGISTERED AS	{cAISDNAttribute 33};

## 9.34	D-Channel T306XE "D-Channel T306"§

dChannelT306				ATTRIBUTE

WITH ATTRIBUTE SYNTAX

CAISDNModule.DChannelT3xx;

MATCHES FOR EQUALITY;

BEHAVIOUR

dChannelT306Bhv		BEHAVIOUR

DEFINED AS	"This attribute provides the value of call control timer T306 defined in Q.931.";;

REGISTERED AS	{cAISDNAttribute 34};

## 9.35	D-Channel T307XE "D-Channel T307"§

dChannelT307				ATTRIBUTE

WITH ATTRIBUTE SYNTAX

CAISDNModule.DChannelT3xx;

MATCHES FOR EQUALITY;

BEHAVIOUR

dChannelT307Bhv		BEHAVIOUR

DEFINED AS	"This attribute provides the value of call control timer T307 defined in Q.931.";;

REGISTERED AS	{cAISDNAttribute 35};

## 9.36	D-Channel T308XE "D-Channel T308"§

dChannelT308				ATTRIBUTE

WITH ATTRIBUTE SYNTAX

CAISDNModule.DChannelT3xx;

MATCHES FOR EQUALITY;

BEHAVIOUR

dChannelT308Bhv		BEHAVIOUR

DEFINED AS	"This attribute provides the value of call control timer T308 defined in Q.931.";;

REGISTERED AS	{cAISDNAttribute 36};

## 9.37	D-Channel T309XE "D-Channel T309"§

dChannelT309				ATTRIBUTE

WITH ATTRIBUTE SYNTAX

CAISDNModule.DChannelT3xx;

MATCHES FOR EQUALITY;

BEHAVIOUR

dChannelT309Bhv		BEHAVIOUR

DEFINED AS	"This attribute provides the value of call control timer T309 defined in Q.931.";;

REGISTERED AS	{cAISDNAttribute 37};

## 9.38	D-Channel T310XE "D-Channel T310"§

dChannelT310 				ATTRIBUTE

WITH ATTRIBUTE SYNTAX

CAISDNModule.DChannelT3xx;

MATCHES FOR EQUALITY;

BEHAVIOUR

dChannelT310Bhv		BEHAVIOUR

DEFINED AS	"This attribute provides the value of call control timer T310 defined in Q.931.";;

REGISTERED AS	{cAISDNAttribute 38};

## 9.39	D-Channel T312XE "D-Channel T312"§

dChannelT312				ATTRIBUTE

WITH ATTRIBUTE SYNTAX

CAISDNModule.DChannelT3xx;

MATCHES FOR EQUALITY;

BEHAVIOUR

dChannelT312Bhv		BEHAVIOUR

DEFINED AS	"This attribute provides the value of call control timer T312 defined in Q.931.";;

REGISTERED AS	{cAISDNAttribute 39};

## 9.40	D-Channel T314XE "D-Channel T314"§

dChannelT314				ATTRIBUTE

WITH ATTRIBUTE SYNTAX

CAISDNModule.DChannelT3xx;

MATCHES FOR EQUALITY;

BEHAVIOUR

dChannelT314Bhv		BEHAVIOUR

DEFINED AS	"This attribute provides the value of call control timer T314 defined in Q.931.";;

REGISTERED AS	{cAISDNAttribute 40};

## 9.41	D-Channel T316XE "D-Channel T316"§

dChannelT316				ATTRIBUTE

WITH ATTRIBUTE SYNTAX

CAISDNModule.DChannelT3xx;

MATCHES FOR EQUALITY;

BEHAVIOUR

dChannelT316Bhv		BEHAVIOUR

DEFINED AS	"This attribute provides the value of call control timer T316 defined in Q.931.";;

REGISTERED AS	{cAISDNAttribute 41};

## 9.42	D-Channel T317XE "D-Channel T317"§

dChannelT317				ATTRIBUTE

WITH ATTRIBUTE SYNTAX

CAISDNModule.DChannelT3xx;

MATCHES FOR EQUALITY;

BEHAVIOUR

dChannelT317Bhv		BEHAVIOUR

DEFINED AS	"This attribute provides the value of call control timer T317 defined in Q.931.";;

REGISTERED AS	{cAISDNAttribute 42};

## 9.43	D-Channel T320XE "D-Channel T320"§

dChannelT320				ATTRIBUTE

WITH ATTRIBUTE SYNTAX

CAISDNModule.DChannelT3xx;

MATCHES FOR EQUALITY;

BEHAVIOUR

dChannelT320Bhv		BEHAVIOUR

DEFINED AS	"This attribute provides the value of call control timer T320 defined in Q.931.";;

REGISTERED AS	{cAISDNAttribute 43};

## 9.44	D-Channel T321XE "D-Channel T321"§

dChannelT321				ATTRIBUTE

WITH ATTRIBUTE SYNTAX

CAISDNModule.DChannelT3xx;

MATCHES FOR EQUALITY;

BEHAVIOUR

dChannelT321Bhv		BEHAVIOUR

DEFINED AS	"This attribute provides the value of call control timer T321 defined in Q.931.";;

REGISTERED AS	{cAISDNAttribute 44};

## 9.45	D-Channel T322XE "D-Channel T322"§

dChannelT322				ATTRIBUTE

WITH ATTRIBUTE SYNTAX

CAISDNModule.DChannelT3xx;

MATCHES FOR EQUALITY;

BEHAVIOUR

dChannelT322Bhv		BEHAVIOUR

DEFINED AS	"This attribute provides the value of call control timer T322 defined in Q.931.";;

REGISTERED AS	{cAISDNAttribute 45};

## 9.46	D-Channel T330XE "D-Channel T330"§

dChannelT330						ATTRIBUTE

WITH ATTRIBUTE SYNTAX

CAISDNModule.DChannelT3xx;

MATCHES FOR EQUALITY;

BEHAVIOUR

dChannelT330Bhv		BEHAVIOUR

DEFINED AS	"This attribute provides the value of call control timer T330 defined in Q.931.";;

REGISTERED AS	{cAISDNAttribute 46};

## 9.47	Deactivation CapabilitiesXE "Deactivation Capabilities"§

deactivationCapabilities				ATTRIBUTE

WITH ATTRIBUTE SYNTAX

CAISDNAttributeModule.Boolean;

MATCHES FOR EQUALITY;

BEHAVIOUR

deactivationCapabilitiesBhv	BEHAVIOUR

DEFINED AS	"This attribute indicates whether the exchange supports deactivation or not. In the APP, this attribute is a boolean with the true value stating that the APP supports deactivation by the user; the false value indicates that the APP supports only permanent deactivation. In the layer 2 entity, this attribute indicates whether signalling or information entity support deactivation by the user or not. The attribute is a boolean; the false value is only possible if the same attribute in the ISDN APP is also false.";;

REGISTERED AS	{cAISDNAttribute 47};

## 9.48	Directory Number Appearance Identifier ListXE "Directory Number Appearance Identifier List"§

directoryNumberAppearanceIdentifierList		ATTRIBUTE

WITH ATTRIBUTE SYNTAX

CAISDNModule. DirectoryNumberAppearanceIdentifierList;

MATCHES FOR EQUALITY, SET-COMPARISON, SET-INTERSECTION;

BEHAVIOUR

directoryNumberAppearanceIdentifierBhv		BEHAVIOUR

DEFINED AS	"This attribute identifies the terminal controlled call appearance identifier information for each DN associated with the Terminal Service Profile. These call appearance identifiers are not used for call set-up or for compatibility checks for incoming calls. This information is simply used for associating a DN with a particular key on a terminal. This attribute also identifies the default bearer service associated with each terminal controlled call appearance. This BS information can be used in originating calls, if not overridden by the user.";;

REGISTERED AS	{cAISDNAttribute 48};

## 9.49	Directory Number ReferenceXE "Directory Number Reference"§

directoryNumberReference			ATTRIBUTE

WITH ATTRIBUTE SYNTAX

CAISDNAttributeModule.DirectoryNumberReference;

MATCHES FOR EQUALITY;

BEHAVIOUR

directoryNumberReferenceBhv		BEHAVIOUR

DEFINED AS	"The function of this set-valued attribute is to assign a logical number (integer) to each Directory Number/Bearer service pair associated with this TSP. This attribute is used in the attribute value structure of the Feature Activators per Directory Number/BS (FADN) and Feature Indicators per Directory Number/BS (FIDN) attributes of the Terminal Configuration object class instance associated with this TSP. The set of value(s) of this attribute are each of the following sequence:

<DirectoryNumberR>,<DirectoryNumber>,<BS>

where:

DirectoryNumberR is an integer in the range from 1 to 128, which is the Directory Number/BS reference number used by attributes of the associated TCGN object associated with this TSP object,

Directory Number is the Directory Number to which the DirectoryNumberR is assigned.

BS is the ISDN bearer service contained in the Directory Number Profile managed object.";;

REGISTERED AS	{cAISDNAttribute 49};

## 9.50	DTE CompatibilityXE "DTE Compatibility"§

dTECompatibilty						ATTRIBUTE

WITH ATTRIBUTE SYNTAX

CAISDNModule.DTECompatibility;

MATCHES FOR EQUALITY;

BEHAVIOUR

dTECompatibiltyBhv 			BEHAVIOUR

DEFINED AS	"The value of this attribute provides compatibility with DTE built to 1980, 	1984, 1988 or 1993 X.25 specifications.";;

REGISTERED AS	{cAISDNAttribute 50};

## 9.51	Early Cut Thru Remote Network InterworkXE "Early Cut Thru Remote Network Interwork"§

earlyCutThruRemoteNetworkInterwork	ATTRIBUTE

WITH ATTRIBUTE SYNTAX

CAISDNAttributeModule.Boolean;

MATCHES FOR EQUALITY;

BEHAVIOUR

earlyCutThruRemoteNetworkInterworkBhv	BEHAVIOUR

DEFINED AS	"This attribute indicates whether the procedures described in Annex K/Q.931are activated or not. It is a boolean value that is associated with the attribute earlyCutThruUserProvidedAudibleRing. If earlyCutThruUserProvidedAudibleRing is true, earlyCutThruRemoteNetworkInterwork must be true to allow the PBX to return the tone back towards the network.";;

REGISTERED AS	{cAISDNAttribute 51};

## 9.52	Early Cut Thru User Provided Audible RingXE "Early Cut Thru User Provided Audible Ring"§

earlyCutThruUserProvidedAudibleRing 		ATTRIBUTE

WITH ATTRIBUTE SYNTAX

CAISDNAttributeModule.Boolean;

MATCHES FOR EQUALITY;

BEHAVIOUR

earlyCutThruUserProvidedAudibleRingBhv	BEHAVIOUR

DEFINED AS	"This attribute indicates whether the exchange provides users with tone or not. It is a boolean, with the true value indicating that the exchange is not providing the tones while early cut through is provided.";;

REGISTERED AS	{cAISDNAttribute 52};

## 9.53	Feature Activators All Directory NumberXE "Feature Activators All Directory Number"§

featureActivatorsAllDirectoryNumber		ATTRIBUTE

WITH ATTRIBUTE SYNTAX

CAISDNAttributeModule.FeatureActivator;

MATCHES FOR EQUALITY;

BEHAVIOUR

featureActivatorsAllDirectoryNumberBhv	BEHAVIOUR

DEFINED AS	"This attribute, (FA), is a set-valued attribute of the Terminal Configuration object class. This attribute identifies Feature Activators for features which have the same value for all Directory Number/BSs on an ISDN terminal. This attribute identifies Feature Activators for TSP features to be applied to all Directory Number/BS pairs identified on an ISDN terminal. At least 64 feature activators should be assignable for the combination of the FA and FADirectoryNumber TCGN features. Each value in the set is a sequence of data items in the following format:

n,	<ObjectID>

where: an integer in the range of 0 to 16383, identifying a Feature Activator value to be sent by an ISDN terminal when the user activates a key, The identifier of a Service Feature Object, in ASN.1 format.";;

REGISTERED AS	{cAISDNAttribute 53};

## 9.54	Feature Activators Per Directory NumberXE "Feature Activators Per Directory Number"§

featureActivatorsPerDirectoryNumber		ATTRIBUTE

WITH ATTRIBUTE SYNTAX

CAISDNAttributeModule.FeatureActivator;

MATCHES FOR EQUALITY;

BEHAVIOUR

featureActivatorsPerDirectoryNumberBhv	BEHAVIOUR

DEFINED AS	"This attribute is a set-valued attribute of the Terminal Configuration object class. This attribute identifies Feature Activators for TSP features requiring different values for each Directory Number/BS pair on an ISDN terminal. This feature is operable only for specific Directory Number/CT pairs identified by the Directory Number Reference attribute in the TSP object associated with this TCGN object. At least 64 feature activators should be assignable for the combination of the FA and FADirectoryNumber TCGN features. Each value in the set is a sequence of data items in the following format: 	n,	m,	<ObjectID>

where: an integer in the range of 0 to 16383, identifying a Feature Activator value to be sent by an ISDN terminal when the user activates a key, an integer in the range of 1 to 128, representing the Directory Number Reference that specifically identifies a Directory Number/BS pair to which the Feature Activator is assigned, The identifier of a Service Feature Object in ASN.1 format.";;

REGISTERED AS	{cAISDNAttribute 54};

## 9.55	Feature Activators Per Hunt Make BusyXE "Feature Activators Per Hunt Make Busy"§

featureActivatorsPerHuntMakeBusy		ATTRIBUTE

WITH ATTRIBUTE SYNTAX

CAISDNAttributeModule.FeatureActivator;

MATCHES FOR EQUALITY;

BEHAVIOUR

featureActivatorsPerHuntMakeBusyBhv	BEHAVIOUR

DEFINED AS	"This attribute identifies Feature Activators used to activate and deactivate the Hunt Make Busy application on an ISDN Terminal.";;

REGISTERED AS	{cAISDNAttribute 55};

## 9.56	Feature Activators Per Stop HuntXE "Feature Activators Per Stop Hunt"§

featureActivatorsPerStopHunt				ATTRIBUTE

WITH ATTRIBUTE SYNTAX

CAISDNAttributeModule.FeatureActivator;

MATCHES FOR EQUALITY;

BEHAVIOUR

featureActivatorsPerStopHuntBhv		BEHAVIOUR

DEFINED AS	"This attribute identifies Feature Activators used to activate and deactivate the Stop Hunt application on an ISDN terminal.";;

REGISTERED AS	{cAISDNAttribute 56};

## 9.57	Feature Indicators All Directory NumberXE "Feature Indicators All Directory Number"§

featureIndicatorsAllDirectoryNumber		ATTRIBUTE

WITH ATTRIBUTE SYNTAX

CAISDNAttributeModule.FeatureIndicator;

MATCHES FOR EQUALITY;

BEHAVIOUR

featureIndicatorsAllDirectoryNumberBhv	BEHAVIOUR

DEFINED AS	"This set-valued attribute identifies Feature Indicators at an ISDN terminal for features which have the same value for all Directory Number/BS. At least 64 feature indicators may be assignable to a given terminal. Each attribute value in the set is a sequence of data items in the following format:

n,<Object ID>

where:

n = an integer in the range of 0 to 16383, identifying a Feature Indicator value to be received by an ISDN terminal to light a particular lamp at the terminal,

Object ID = The identifier a supplementary feature to be which this indicator is assigned.";;

REGISTERED AS	{cAISDNAttribute 57};

## 9.58	Feature Indicators Per Directory NumberXE "Feature Indicators Per Directory Number"§

featureIndicatorsPerDirectoryNumber	ATTRIBUTE

WITH ATTRIBUTE SYNTAX

CAISDNAttributeModule.FeatureIndicatorsPerDirectoryNumber;

MATCHES FOR EQUALITY;

BEHAVIOUR

featureIndicatorsPerDirectoryNumberBhv	BEHAVIOUR

DEFINED AS	"This attribute is a set-valued attribute of the Terminal Configuration (TCGN) object class. This attribute identifies Feature Indicators for TSP features requiring different values for each Directory Number/BS pair on an ISDN terminal. This feature indication applies only for specific Directory Number/CT pairs identified by the Directory Number Reference attribute in the TSP object associated with this TCGN object. At least 64 feature activators should be assignable for the combination of the FI and FIDirectoryNumber TCGN features. Each value in the set is a sequence of data items in the following format:

n,			m,		<Object			ID>

where: An integer in the range of 0 to 16383, identifying a Feature Indicator or value to be returned to an ISDN terminal indicating the status of the assigned feature, an integer in the range of 1 to 128, representing the Directory Number Reference, (DirectoryNumberR), that specifically identifies a DirectoryNumber/BS pair to which the Feature Indicator is assigned, The identifier of a Service Feature Object in ASN.1 format. ";;

REGISTERED AS	{cAISDNAttribute 58};

## 9.59	Feature Indicators Per Hunt Make BusyXE "Feature Indicators Per Hunt Make Busy"§

featureIndicatorsPerHuntMakeBusy		ATTRIBUTE

WITH ATTRIBUTE SYNTAX

CAISDNAttributeModule.FeatureActivatorValue;

MATCHES FOR EQUALITY;

BEHAVIOUR

featureIndicatorsPerHuntMakeBusyBhv	BEHAVIOUR

DEFINED AS	"This attribute identifies the Feature Indicator that displays the status of the Make Busy application on an ISDN terminal.";;

REGISTERED AS	{cAISDNAttribute 59};

## 9.60	Feature Indicators Per Stop HuntXE "Feature Indicators Per Stop Hunt"§

featureIndicatorsPerStopHunt				ATTRIBUTE

WITH ATTRIBUTE SYNTAX

CAISDNAttributeModule.FeatureActivatorValue;

MATCHES FOR EQUALITY;

BEHAVIOUR

featureIndicatorsPerStopHuntBhv		BEHAVIOUR

DEFINED AS	"This attribute identifies the Feature Indicator that displays the status of the Stop Hunt application on an ISDN terminal.";;

REGISTERED AS	{cAISDNAttribute 60};

## 9.61	Incoming Default Thruput ClassXE "Incoming Default Thruput Class"§

incomingDefaultThruputClass				ATTRIBUTE

WITH ATTRIBUTE SYNTAX

CAISDNAttributeModule.ThruputClass;

MATCHES FOR EQUALITY;

BEHAVIOUR

incomingDefaultThruputClassBhv		BEHAVIOUR

DEFINED AS	"The values of this attribute identify selections of the default throughput class for the incoming direction on the B- or D-Channels. ";;

REGISTERED AS	{cAISDNAttribute 61};

## 9.62	Incoming Max Packet SizeXE "Incoming Max Packet Size"§

incomingMaxPacketSize					ATTRIBUTE

WITH ATTRIBUTE SYNTAX

CAISDNAttributeModule.PacketSize;

MATCHES FOR EQUALITY;

BEHAVIOUR

incomingMaxPacketSizeBhv		BEHAVIOUR

DEFINED AS	"The function of this attribute is to allow the subscriber to choose a maximum packet size (different from the default of 128 provided by the network) for the incoming direction on the B- or D-Channels.

The enumerated choice value of this attribute is one of the following, representing the maximum allowable packet size for a logical channel which does not include Private Virtual Circuits (PVC).

·	16 (octets)

·	32 (octets)

·	64 (octets)

·	128 (octets)

·	256 (octets)

·	512 (octets)

·	1024 (octets)

·	2048 (octets)

·	4096 (octets)";;

REGISTERED AS	{cAISDNAttribute 62};

## 9.63	Incoming Window Size

incomingWindowSize				ATTRIBUTE

WITH ATTRIBUTE SYNTAX

CAISDNAttributeModule.WindowSize;

MATCHES FOR EQUALITY;

BEHAVIOUR

incomingWindowSizeBhv			BEHAVIOUR

DEFINED AS	"This attribute is an attribute of the Packet Switched Subscriber Services object class. The value of this attribute allows for the subscription to non-standard default window sizes for the incoming direction. The attribute value is an integer whose range of permissible values depends upon the value of the Packet Level Sequencing (PLSQ) attribute. If modulo 8 sequencing is specified by PLSQ, the permissible range of the IWS integer is 1 to 7. If modulo 128 is specified by PLSQ, the range of permissible values for IWS is 1 to 60 (61 to 127 is desirable).";;

REGISTERED AS	{cAISDNAttribute 63};

## 9.64	Interface Type

interfaceType							ATTRIBUTE

WITH ATTRIBUTE SYNTAX

CAISDNModule.InterfaceType;

MATCHES FOR EQUALITY;

BEHAVIOUR

interfaceTypeBhv 			BEHAVIOUR

DEFINED AS	"This attribute indicates whether channel access is provided via a basic or a primary rate interface.";;

REGISTERED AS	{cAISDNAttribute 64};

## 9.65	Layer 2 Info Entity PointerXE "Layer 2 Info Entity Pointer"§

layer2InfoEntityPtr						ATTRIBUTE

WITH ATTRIBUTE SYNTAX

CAISDNAttributeModule.ObjectInstance;

MATCHES FOR EQUALITY;

BEHAVIOUR

layer2InfoEntityPtrBhv			BEHAVIOUR

DEFINED AS	"This attribute is used as a pointer to an instance of the Layer2InfoEntity managed object class.";;

REGISTERED AS	{cAISDNAttribute 65};

## 9.66	Layer Entity LAPD PointerXE "Layer Entity LAPD Pointer"§

layerEntityLAPDPtr					ATTRIBUTE

WITH ATTRIBUTE SYNTAX

CAISDNAttributeModule.ObjectInstance;

MATCHES FOR EQUALITY;

BEHAVIOUR

layerEntityLAPDPtrBhv			BEHAVIOUR

DEFINED AS	"This attribute is used as a pointer to an instance of the Layer2LAPDEntity managed object class.";;

REGISTERED AS	{cAISDNAttribute 66};

## 9.67	Layer 3 Info Entity PointerXE "Layer 3 Info Entity Pointer"§

layer3InfoEntityPtr						ATTRIBUTE

WITH ATTRIBUTE SYNTAX

CAISDNAttributeModule.ObjectInstance;

MATCHES FOR EQUALITY;

BEHAVIOUR

layer3InfoEntityPtrBhv			BEHAVIOUR

DEFINED AS	"This attribute is used as a pointer to an instance of the Layer3InfoEntity managed object class.";;

REGISTERED AS	{cAISDNAttribute 67};

## 9.68	Link Level Window SizeXE "Link Level Window Size"§

linkLevelWindowSize					ATTRIBUTE

WITH ATTRIBUTE SYNTAX

CAISDNAttributeModule.WindowSize;

MATCHES FOR EQUALITY;

BEHAVIOUR

linkLevelWindowSizeBhv			BEHAVIOUR

DEFINED AS	"This attribute specifies the link layer window size as defined in Q.921. The linkLevelWindowSize attribute value determines the number of packets allowed in a packet window. The permissible values of this attribute are a range of integer values between 1 and 60, with the following conditions:

·	if the linkLevelFrameSequence attribute is 0, (Modulo 8 frame sequence), the value range of the linkLevelWindowSize attribute is 1 to 7,

·	if the linkLevelFrameSequence attribute is 1, (Modulo 128 frame sequence), the value range of the linkLevelWindowSize attribute is 1 to 127.";;

REGISTERED AS	{cAISDNAttribute 68};

## 9.69	Link OptionXE "Link Option"§

linkOption							ATTRIBUTE

WITH ATTRIBUTE SYNTAX

CAISDNAttributeModule.LinkOption;

MATCHES FOR EQUALITY;

BEHAVIOUR

linkOptionBhv					BEHAVIOUR

DEFINED AS	"This attribute defines whether the user has subscribed to dynamic or static tei assignment.";;

REGISTERED AS	{cAISDNAttribute 69};

## 9.70	Max Bits Per Information FrameXE "Max Bits Per Information Frame"§

maxBitsPerInformationFrame				ATTRIBUTE

WITH ATTRIBUTE SYNTAX

CAISDNAttributeModule.MaxBitsPerInformationFrame;

MATCHES FOR EQUALITY;

BEHAVIOUR

maxBitsPerInformationFrameBhv		BEHAVIOUR

DEFINED AS	"This attribute determines the maximum number of bits allowed in the Information frame on LAPB over the B-Channel. The values permissible for this attribute is a single choice from the following enumerated list: 2120, 4168, 8264, 16456, 32840.";;

REGISTERED AS	{cAISDNAttribute 70};

## 9.71	Max Combined Thruput ClassXE "Max Combined Thruput Class"§

maxCombinedThruputClass				ATTRIBUTE

WITH ATTRIBUTE SYNTAX

CAISDNAttributeModule.MaxCombinedThruputClass;

MATCHES FOR EQUALITY;

BEHAVIOUR

maxCombinedThruputClassBhv		BEHAVIOUR

DEFINED AS	"This attribute determines the maximum value for the sum of the throughput classes allowed on the B-channel before the switch classifies that the channel is busy. The attribute value is either an alphanumeric string "NULL" (default) or a discrete numeric rate value.";;

REGISTERED AS	{cAISDNAttribute 71};

## 9.72	Max Number of Call ReferenceXE "Max Number of Call Reference"§

maxNumberOfCallReference				ATTRIBUTE

WITH ATTRIBUTE SYNTAX

CAISDNAttributeModule.MaxNumberOfCallReference;

MATCHES FOR EQUALITY;

BEHAVIOUR

maxNumberOfCallReferenceBhv		BEHAVIOUR

DEFINED AS	"This attribute indicates the maximum number of the simultaneous layer 3 connections for signalling (SAPI = 0 for all TEIs on this Access Port Profile).";;

REGISTERED AS	{cAISDNAttribute 72};

## 9.73	Max Transmission AttemptsXE "Max Transmission Attempts"§

maxTransmissionAttempts			ATTRIBUTE

WITH ATTRIBUTE SYNTAX

CAISDNAttributeModule.MaxTransmissionAttempts;

MATCHES FOR EQUALITY;

BEHAVIOUR

maxTransmissionRequestsBhv		BEHAVIOUR

DEFINED AS	"This attribute specifies the maximum number of attempts allowed on the B-Channel to complete a successful transmission. The value of this attribute is an integer in the range from 2 through 15, directly representing the maximum permissable attempts. The default value of this attribute is 3.";;

REGISTERED AS	{cAISDNAttribute 73};

## 9.74	Network Provided TonesXE "Network Provided Tones"§

networkProvidedTones					ATTRIBUTE

WITH ATTRIBUTE SYNTAX

CAISDNAttributeModule.Boolean;

MATCHES FOR EQUALITY;

BEHAVIOUR

networkProvidedTonesBhv		BEHAVIOUR

DEFINED AS	"This attribute indicates that a tone and/or announcement is to be provided by the network to indicate the progress or otherwise the status of a call. It is a Boolean attribute with a deafult value of true, as defined in I.231, which means that tones are provided by the network.";;

REGISTERED AS	{cAISDNAttribute 74};

## 9.75	Network User IdentificationXE "Network User Identification"§

networkUserIdentification				ATTRIBUTE

WITH ATTRIBUTE SYNTAX

CAISDNAttributeModule.Boolean;

MATCHES FOR EQUALITY;

BEHAVIOUR

networkUserIdentificationBhv		BEHAVIOUR

DEFINED AS	"This is a boolean attribute that determines Network User Identification (NUI) capability for a Packet Mode user. When the value of this attribute is TRUE, the interface is configured to allow NUI selection facility to be provided to the network for billing, security or network management purposes on a per call basis. When FALSE, the NUI capability is not assigned to this access interface.";;

REGISTERED AS	{cAISDNAttribute 75};

## 9.76	Network User Id OverrideXE "Network User Id Override"§

networkUserIdentificationOverride			ATTRIBUTE

WITH ATTRIBUTE SYNTAX

CAISDNAttributeModule.Boolean;

MATCHES FOR EQUALITY;

BEHAVIOUR

networkUserIdentificationOverrideBhv	BEHAVIOUR

DEFINED AS	"This is a boolean attribute that determines Network User Identification (NUI) override capabilty. This attribute value is applicable only if the networkUserId attribute value is TRUE, indicating that the interface is configured to allow NUI selection facility to be provided to the network for billing, security or network management purposes on a per call basis. When this attribute value is TRUE, the interface is configured so that the NUI information may be associated with a user profile that will be used for the duration of the call. When FALSE, the NUI Override capability is not assigned to this access interface.";;

REGISTERED AS	{cAISDNAttribute 76};

## 9.77	Network User Id SelectionXE "Network User Id Selection"§

networkUserIdentificationSelection		ATTRIBUTE

WITH ATTRIBUTE SYNTAX

CAOptionalUserFacilitiesModule.Boolean;

MATCHES FOR EQUALITY;

BEHAVIOUR

networkUserIdentificationSelectionBhv	BEHAVIOUR

DEFINED AS	"This is a boolean attribute that determines Network User Identification (NUI) Selection and Acceptance capability. This attribute value is applicable only if the networkUserIdentification attribute value is TRUE, indicating that the interface is configured to allow NUI selection facility to be provided to the network for billing, security or network management purposes on a per call basis. When the value of this attribute is TRUE, the NUI selection may be signalled in Call Accepted packets. When FALSE, the NUI Selection is not permitted in Call Accepted packets.";;

REGISTERED AS	{cAISDNAttribute 77};

## 9.78	Network UserId SupplementXE "Network UserId Supplement"§

networkUserIdentificationSupplement		ATTRIBUTE

WITH ATTRIBUTE SYNTAX

CAOptionalUserFacilitiesModule.Boolean;

MATCHES FOR EQUALITY;

BEHAVIOUR

networkUserIdentificationSupplementBhv	BEHAVIOUR

DEFINED AS	"This is a boolean attribute that determines Network User Identification (NUI) Supplemental User Identification (SUI) capabilty. This attribute value is applicable only if the networkUserIdentification attribute value is TRUE, indicating that the interface is configured to allow NUI selection facility to be provided to the network for billing, security or network management purposes on a per call basis. When the value of this attribute is TRUE, the Packet Handler Function (PHF) is configured to signal SUI information. When FALSE, the PHF is not configured for SUI.";;

REGISTERED AS	{cAISDNAttribute 78};

## 9.79	Network UserId User ValidateXE "Network UserId User Validate"§

networkUserIdentificationUserValidate		ATTRIBUTE

WITH ATTRIBUTE SYNTAX

CAOptionalUserFacilitiesModule.Boolean;

MATCHES FOR EQUALITY;

BEHAVIOUR

networkUserIdentificationUserValidateBhv	BEHAVIOUR

DEFINED AS	"This is a boolean attribute that determines Network User Identification (NUI) user validation capabilty. This attribute value is applicable only if the networkUserIdentification attribute value is TRUE, indicating that the interface is configured to allow NUI selection facility to be provided to the network for billing, security or network management purposes on a per call basis. When the attribute value is TRUE, validated NUI values may be passed over the interface from the user to the network and unvalidated NUI values may be passed over the interface from the network to the user. The default value is FALSE, indicating that user validation is not allowed.";;

REGISTERED AS	{cAISDNAttribute 79};

## 9.80	Notification ClassXE "Notification Class"§

notificationClass						ATTRIBUTE

WITH ATTRIBUTE SYNTAX

CAISDNAttributeModule.NotificationClass;

MATCHES FOR EQUALITY;

BEHAVIOUR

notificationClassBhv			BEHAVIOUR

DEFINED AS	"The value of this attribute indicates whether the packet bearer service is "without notification", "with conditional notification" or "with systematic notification" as described in X.31 and Q.931.";;

REGISTERED AS	{cAISDNAttribute 80};

## 9.81	Number Of D-Channel LinksXE "Number Of D-Channel Links"§

numberOfDChannelLinks				ATTRIBUTE

WITH ATTRIBUTE SYNTAX

CAISDNAttributeModule.NumberOfDChannelLinks;

MATCHES FOR EQUALITY;

BEHAVIOUR

numberOfDChannelLinksBhv		BEHAVIOUR

DEFINED AS	"This attribute indicates the max number of D-Channel links on the basic rate access. This includes signalling links and links used for other services (e.g. packets).";;

REGISTERED AS	{cAISDNAttribute 81};

## 9.82	Outgoing Default Thruput ClassXE "Outgoing Default Thruput Class"§

outgoingDefaultThruputClass				ATTRIBUTE

WITH ATTRIBUTE SYNTAX

CAISDNAttributeModule.ThruputClass;

MATCHES FOR EQUALITY;

BEHAVIOUR

outgoingDefaultThruputClassBhv		BEHAVIOUR

DEFINED AS	"The values of this attribute identify selections of the default throughput class for the outgoing direction on the B- or D-Channels.";;

REGISTERED AS	{cAISDNAttribute 82};

## 9.83	Outgoing Max Packet SizeXE "Outgoing Max Packet Size"§

outgoingMaxPacketSize					ATTRIBUTE

WITH ATTRIBUTE SYNTAX

CAISDNAttributeModule.PacketSize;

MATCHES FOR EQUALITY;

BEHAVIOUR

outgoingMaxPacketSize Bhv		BEHAVIOUR

DEFINED AS	"The function of this attribute is to allow the subscriber to choose a maximum packet size (different than the default of 128 provided by the network) for the outcoming direction on the B- or D-Channels.

The enumerated choice value of this attribute is one of the following, representing the maximum allowable packet size for a logical channel which does not include Private Virtual Circuits (PVC).: 16, 32, 64, 128, 256, 512, 1024, 2048, or 4096 octets.";;

REGISTERED AS	{cAISDNAttribute 83};

## 9.84	Outgoing Window SizeXE "Outgoing Window Size"§

outgoingWindowSize					ATTRIBUTE

WITH ATTRIBUTE SYNTAX

CAISDNAttributeModule.WindowSize;

MATCHES FOR EQUALITY;

BEHAVIOUR

outgoingWindowSizeBhv			BEHAVIOUR

DEFINED AS	"This attribute specifies the packet window size for outgoing packet transmissions in LAPB. The value of this attribute allows for the subscription to non-standard default window sizes for the outgoing direction. The attribute value is an integer whose range of permissible values depends upon the value of the packetLevelSequence attribute. If modulo 8 sequencing is specified by the packetLevelSequence attribute, the permissible range is an integer between 1 and 7. If modulo 128 is specified by the packetLevelSequence attribute, the range of permissible values is 1 to 60 (61 to 127 is desirable).";;

REGISTERED AS	{cAISDNAttribute 84};

## 9.85	Screen Calling Party NumberXE "Screen Calling Party Number"§

screenCallingPartyNumber			ATTRIBUTE

WITH ATTRIBUTE SYNTAX

CAISDNAttributeModule.Boolean;

MATCHES FOR EQUALITY;

BEHAVIOUR

screenCallingPartyNumberBhv		BEHAVIOUR

DEFINED AS	"This is a boolean attribute. When the value of this attribute is TRUE, the switch will screen calling party number information for validity when provided on a call by the user equipment. The attribute value FALSE is the default and will prohibit the switch from screening calling party numbers. If the attribute value is FALSE, then the Calling Party Number Default DN attribute is required. If the Calling Party Number Provision Necessary attribute value is TRUE, then this attribute must also be TRUE.";;

REGISTERED AS	{cAISDNAttribute 85};

## 9.86	Semi-Permanent Access Packet Handler Default Directory NumberXE "Semi-Permanent Access Packet Handler Default Directory Number"§

semiPermAccessPacketHandlerDefaultDN	ATTRIBUTE

WITH ATTRIBUTE SYNTAX

CAISDNAttributeModule.SemiPermAccessPacketHandlerDefaultDN;

MATCHES FOR EQUALITY;

BEHAVIOUR

semiPermAccessPacketHandlerDefaultDNBhv	BEHAVIOUR

DEFINED AS	" The values of this attribute value identifies the Directory Number assigned to each access channels to be used as the default DN if a DN is not included in the outgoing set-up message to a nailed-up B-channel.";;

REGISTERED AS	{cAISDNAttribute 86};

## 9.87	Signalling Parameter NegotiationXE "Signalling Parameter Negotiation"§

signallingParameterNegotiation			ATTRIBUTE

WITH ATTRIBUTE SYNTAX

CAISDNAttributeModule.Boolean;

MATCHES FOR EQUALITY;

BEHAVIOUR

signallingParameterNegotiationBhv		BEHAVIOUR

DEFINED AS	"This is a boolean attribute. The attribute value TRUE allows the use of signalling parameter negotiation, providing that the switch supports XID frames.";;

REGISTERED AS	{cAISDNAttribute 87};

## 9.88	Terminal Configuration IdXE "Terminal Configuration Id"§

terminalConfigurationId					ATTRIBUTE

WITH ATTRIBUTE SYNTAX

CAISDNAttributeModule.NameType;

MATCHES FOR EQUALITY, ORDERING, SUBSTRINGS;

BEHAVIOUR

terminalConfigurationIdBhv		BEHAVIOUR

DEFINED AS	"This is a naming attribute. If the string choice for the syntax is used, matching on the substrings is permitted. If the number choice for the syntax is used, then matching on ordering is permitted.";;

REGISTERED AS	{cAISDNAttribute 88};

## 9.89	Terminal configuration PointerXE "Terminal configuration Pointer"§

terminalConfigurationPtr				ATTRIBUTE

WITH ATTRIBUTE SYNTAX

CAISDNAttributeModule.ObjectInstance;

MATCHES FOR EQUALITY, SET-COMPARISON, SET-INTERSECTION;

BEHAVIOUR

terminalConfigurationPtrBhv		BEHAVIOUR

DEFINED AS	"This attribute is used as a pointer to an instance of the terminal Configuration managed object class.";;

REGISTERED AS	{cAISDNAttribute 89};

## 9.90	TSP IDXE "TSP ID"§

tspid								ATTRIBUTE

WITH ATTRIBUTE SYNTAX

CAISDNModule.TSPID;

MATCHES FOR EQUALITY, ORDERING, SUBSTRINGS;

BEHAVIOUR

tspidBhv						BEHAVIOUR

DEFINED AS	"The value of this attribute is a numeric string of up to 18 numeric characters that identify the subscriber ISDN Basic Rate CPE profiles from a human user perspective. Prior to CPE initialization, a terminalServiceProfile (TSP) object instance is created including an assignment of a tspid attribute by the service provider's Service Order Process. Prior to using the terminal, a serviceProfileIdentifier (SPID) is entered by the subscriber at the CPE. The SPID, provided to the subscriber at Service Order time by the Administration, includes the TSPID as a component. The other componenet of the SPID is the terminalIdentifier (TID) that identifies at protocol layer 3 the particular CPE terminal being initialized.";;

REGISTERED AS	{cAISDNAttribute 90};

## 9.91	Terminal LimitXE "Terminal Limit"§

terminalLimit							ATTRIBUTE

WITH ATTRIBUTE SYNTAX

CAISDNAttributeModule.TerminalLimit;

MATCHES FOR EQUALITY;

BEHAVIOUR

terminalLimitBhv			BEHAVIOUR

DEFINED AS	"This attribute is an integer in the range from 0 to 62 which identifies the maximum number of Basic Rate ISDN terminals that can share an instance of the TSP object class. A value of 0 stops service for the terminalServiceProfile.";;

REGISTERED AS	{cAISDNAttribute 91};

## 9.92	Terminal Service ProfileXE "Terminal Service Profile"§

terminalServiceProfileId					ATTRIBUTE

WITH ATTRIBUTE SYNTAX

CAISDNModule.NameType;

MATCHES FOR EQUALITY, ORDERING, SUBSTRINGS;

BEHAVIOUR

terminalServiceProfileIdBhv		BEHAVIOUR

DEFINED AS	"This is a naming attribute. If the string choice for the syntax is used, matching on the substrings is 	permitted. If the number choice for the syntax is used, then matching on ordering is permitted.";;

REGISTERED AS	{cAISDNAttribute 92};

# 10	Parameter TemplatesXE "Parameter Templates"§

invalidReferenceError					PARAMETER

CONTEXT SPECIFIC-ERROR;

WITH SYNTAX CAISDNModule.ObjectInstance;

BEHAVIOUR

invalidReferenceErrorBehaviour		BEHAVIOUR

DEFINED AS	"This error is sent to the Managing System when a CMIP request would cause an object to point to an object instance which is either non-existent, not of an appropriate type or not in an appropriate state. The invalid reference pointer value (an object instance name) of the request is returned to the managing system along with the error message.";;

REGISTERED AS	{cAISDNParameter 1};

# 11	Name BindingsXE "Name Bindings"§

## 11.1	Catalogued Access Port ISDNXE "Catalogued Access Port ISDN"§

cataloguedAccessPortISDN-managedElement								NAME BINDING

SUBORDINATE OBJECT CLASS cataloguedAccessPortISDN AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS "CCITT Rec. M.3100":managedElement AND SUBCLASSES;

WITH ATTRIBUTE cataloguedAccessPortISDNId;

CREATE;

DELETE;

REGISTERED AS	{cAISDNNameBinding 3};

## 11.2	Calling Number ScreeningXE "Calling Number Screening"§

callingNumberScreening-accessPortProfileISDN 		NAME BINDING

SUBORDINATE OBJECT CLASS callingNumberScreening AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS accessPortProfileISDN AND SUBCLASSES;

WITH ATTRIBUTE callingNumberScreeningId;

CREATE

WITH-AUTOMATIC-INSTANCE-NAMING,

WITH-REFERENCE-OBJECT;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS	{cAISDNNameBinding 4};

## 11.3	Catalogued Access Port Profile ISDN Primary RateXE "Catalogued Access Port Profile ISDN Primary Rate"§

cataloguedAccessPortProfileISDN-managedElement	NAME BINDING

SUBORDINATE OBJECT CLASS cataloguedAccessPortProfileISDN AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS "CCITT Rec. M.3100(1992)":managedElement AND SUBCLASSES;

WITH ATTRIBUTE cataloguedAccessPortProfileISDNId;

CREATE;

DELETE;

REGISTERED AS	{cAISDNNameBinding 5};

## 11.4	Catalogued Layer Entity LAPDXE "Catalogued Layer Entity LAPD"§

cataloguedLayerEntityLAPD-managedElement						NAME BINDING

SUBORDINATE OBJECT CLASS cataloguedLayerEntityLAPD AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS "CCITT Rec. M.3100(1992)":managedElement AND SUBCLASSES;

WITH ATTRIBUTE cataloguedLayerEntityLAPDId;

CREATE;

DELETE;

REGISTERED AS	{cAISDNNameBinding 6};

## 11.5	Catalogued Layer Entity DSS 1XE "Catalogued Layer Entity DSS 1"§

cataloguedLayerEntityDSS1-managedElement 	NAME BINDING

SUBORDINATE OBJECT CLASS cataloguedLayerEntityDSS1 AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS "CCITT Rec. M.3100":managedElement AND SUBCLASSES;

WITH ATTRIBUTE cataloguedLayerEntityDSS1Id;

CREATE;

DELETE;

REGISTERED AS	{cAISDNNameBinding 7};

## 11.6	Terminal ConfigurationXE "Terminal Configuration"§

terminalConfiguration-managedElement	NAME BINDING

SUBORDINATE OBJECT CLASS terminalConfiguration AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS "CCITT Rec. M.3100(1992)":managedElement AND SUBCLASSES;

WITH ATTRIBUTE terminalConfigurationId;

CREATE;

DELETE;

REGISTERED AS	{cAISDNNameBinding 11};

## 11.7	Terminal Service ProfileXE "Terminal Service Profile"§

terminalServiceProfile-managedElement	NAME BINDING

SUBORDINATE OBJECT CLASS terminalServiceProfile AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS "CCITT Rec. M.3100(1992)":managedElement AND SUBCLASSES;

WITH ATTRIBUTE terminalServiceProfileId;

CREATE;

DELETE;

REGISTERED AS	{cAISDNNameBinding 12};

## 11.8	X.25 Network User IdentificationXE "X.25 Network User Identification"§

x25NetworkUserIdentification-accessPortProfileISDN	NAME BINDING

SUBORDINATE OBJECT CLASS x25NetworkUserIdentification AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS accessPortProfileISDN AND SUBCLASSES;

WITH ATTRIBUTE "ITU-T Rec. Q.824.0":optionalUserFacilitiesId;

CREATE

WITH-AUTOMATIC-INSTANCE-NAMING,

WITH-REFERENCE-OBJECT;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS	{cAISDNNameBinding 13};

## 11.9	Layer Entity X25PLP SharedXE "Layer Entity X25PLP Shared"§

layerEntityX25PLPShared-managedElement	NAME BINDING

SUBORDINATE OBJECT CLASS layerEntityX25PLPShared AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS managedElement AND SUBCLASSES;

WITH ATTRIBUTE layerEntityId;

CREATE;

DELETE;

REGISTERED AS	{cACommonNameBinding 14};

# 12	Service Provisioning ActionsXE "Service Provisioning Actions"§

## 12.1	Change Directory NumberXE "Change Directory Number"§

changeDirectoryNumber 		ACTION

BEHAVIOUR

changeDirectoryNumberBhv 		BEHAVIOUR

DEFINED AS	"This action is used to change the Directory Number for a given customer service. The action request identifies the customer service with the old Directory Number Name. The request also indicates the new Directory Number Name to use and the intercept Treatment Termination to apply to the old Directory Number Name.

The action verifies that the old directoryNumberName is in service, and that the new Directory NumberName and interceptTreatment are valid. The new directoryNumberName is considered valid if it exists and does not have a relationship with a customerProfile or its subclasses (it is not in service). If not, the agent returns an invalid reference error.

The action sets the interceptTreatmentOrigin and interceptTreatmentTerm of the new Directory NumberName based on the values of those attributes of the old DirectoryNumberName and then sets the value of interceptTreatmentTerm of the old DirectoryNumberName to the value provided by the action request information.

Moreover, the relationship of the customerProfile or its subclasses associated with the old DirectoryNumberName is deleted and replaced by a relationship with the old DirectoryNumberName.";;

MODE CONFIRMED;

WITH INFORMATION SYNTAX

SpmAttributeModule.ChangeDirectoryNumberRequest;

REGISTERED AS	{cAISDNAction 1};

## 12.2	Establish ISDN AccessXE "Establish ISDN Access"§

establishISDNAccess		ACTION

BEHAVIOUR

establishISDNAccessBhv			BEHAVIOUR

DEFINED AS	"The action first verifies that the access port trail termination point name identified in the service is valid. If it is not valid the agent returns an invalid reference error. The access port trail termination point name is considered valid if all of the following conditions are met:

·	an instance of accessPortTrailTerminationPoint exists for the name provided in the action;

·	the accessPortTrailTerminationPoint can support ISDN services.

Depending on the actual syntax selected, a different behaviour will apply for the remaining of the action execution:

·	if a servicePackage is provided, the service is instantiated based on the definition provided by a service package and the instantiateISDNAccesslServicePackageBehaviour applies;

·	if a copyCommand is selected, the service is instantiated based on the definition provided by an already existing service and the copyISDNAccessCommandBehaviour applies.

In all cases, if the action is successful, the reply will indicate so and will also contain the list of names of the object instances just created. Otherwise the action leaves the MIB unaffected (unchanged) and returns the specified error message .";,

copyISDNAccessCommandBehaviour	BEHAVIOUR

DEFINED AS	"The action verifies that the source Access Port Profile Name in the service is valid. If it is not valid the agent returns an invalid reference error. The existing Access Port Profile Name is considered valid if it exists and supports ISDN service.

The action creates a duplicate of the accessPortProfile subtree of the service identified by the existing Acces Port Profile Name.";,

instantiateISDNAccessServicePackageBhv	BEHAVIOUR

DEFINED AS	"The action determines the existence of the service package name provided in the action request parameters. If it does not exist, the agent returns an invalid reference error.

The action creates a duplicate of the accessPortProfile containment subtree of the service package for use by the new service.";;

MODE CONFIRMED;

WITH INFORMATION SYNTAX

CAISDNAttributeModule.EstablishISDNAccessRequest;

WITH REPLY SYNTAX CAISDNAttributeModule.CreatedInstancesName;

REGISTERED AS	{cAISDNAction 2};

## 12.3	Remove ISDN AccessXE "Remove ISDN Access"§

removeISDNAccess			ACTION

BEHAVIOUR

removeISDNAccessBhv			BEHAVIOUR

DEFINED AS	"This action removes an ISDN Access that is not associated with any CustomerProfile. In addition, all contained object classes, and all associated supplementary service object classes based on the Access Port Profile Name parameter in the action request.

The action verifies that the access port service profile name exists, and that there are no associated customer profiles.  If not the agent returns an invalid reference error.

The name of all the deleted object instances is sent back as a reply to the managing system.";;

MODE CONFIRMED;

WITH INFORMATION SYNTAX

CAISDNAttributeModule.RemoveISDNAccessRequest;

WITH REPLY SYNTAX CAISDNAttributeModule.DeletedInstancesName;

REGISTERED AS	{cAISDNAction 3};

## 12.4	Establish ISDN ServiceXE "Establish ISDN Service"§

establishISDNService		ACTION

BEHAVIOUR

establishISDNServiceBhv			BEHAVIOUR

DEFINED AS	"The action first verifies that the directory number name(s) and termination point name identified in the service are valid. If either is not valid the agent returns an invalid reference error. The directory number name is considered valid if it exists and does not have a relationship with a customerProfile or its subclasses and its administrative state is unlocked. The termination point name is considered valid if all of the following conditions are met:

·	an instance of accessPortTrailTerminationPoint exists for the termination point name provided in the action;

·	the accessPortTrailTerminationPoint can support ISDN services, either with or without a physical line card change.

Depending on the actual syntax selected, a different behaviour will apply for the remaining of the action execution:

·	if a servicePackage is provided, the service is instantiated based on the definition provided by a service package and the instantiateServicePackageBehaviour applies;

·	if a copyCommand is selected, the service is instantiated based on the definition provided by an already existing service and the copyCommandBehaviour applies.

In all cases, if the action is successful, the reply will indicate so and will also contain the list of names of the object instances just created. Otherwise the action leaves the MIB unaffected (unchanged) and returns the specified error message.";,

copyISDNServiceCommandBehaviour 	BEHAVIOUR

DEFINED AS	"The action verifies that the existing Customer Profile, Directory Number Name(s) and existing Termination Point Name identified in the service are valid. If any are not valid the agent returns an invalid reference error. The existing Directory Number Name(s) is considered valid if it exists and it is in service (has a relationship with the specified customerProfile and an accessPortProfile) on the specified existing Termination Point Name.

If the existing Termination Point Name is not provided, the existing Directory Number Name(s) must be in service on only one office equipment. Otherwise, the agent returns an invalid reference error with a NULL value for the Existing Service Office Equipment Number Name.

The action creates a duplicate of  the customerProfile or its subclasses and accessPortProfile containment subtrees of the service identified by the existing Directory Number Name and existing Termination Point Name (if provided) for use by the new service, and create the following relationships:

·	directoryNumber - customerProfile or its subclasses

·	customerProfile or its subclasses - accessPortProfile

·	accessPortProfile - accessPortTrailTerminationPoint.

If an accessPortProfile already exists in association with the accessPortProfileTrailTerminationPoint, then a new one does not need to be instantiated.";;

MODE CONFIRMED;

WITH INFORMATION SYNTAX

CAISDNAttributeModule.EstablishISDNServiceRequest;

WITH REPLY SYNTAX CAISDNAttributeModule.CreatedInstancesName;

REGISTERED AS	{cAISDNAction 4};

## 12.5	Remove ISDN ServiceXE "Remove ISDN Service"§

removeISDNService			ACTION

BEHAVIOUR

removeISDNServiceBhv 			BEHAVIOUR

DEFINED AS	"This action removes an ISDN Service based on the Directory Number Name and Bearer Service parameters in the action request. Removing an ISDN Service consists of removing the Bearer Service, all contained objects, and all associated supplementary service objects. If the last bearer service is removed, the Customer profile or its subclasses object that contained the removed bearer service is also removed.

The action verifies that the directory number name exists. If not the agent returns an invalid reference error.

The name of all the deleted object instances is sent back as a reply to the managing system.";;

MODE CONFIRMED;

WITH INFORMATION SYNTAX

CAISDNAttributeModule.RemoveISDNServiceRequest;

WITH REPLY SYNTAX CAISDNAttributeModule.DeletedInstancesName;

REGISTERED AS	{cAISDNAction 5};

## 12.6	Establish ISDN TerminalXE "Establish ISDN Terminal"§

establishISDNTerminal		ACTION

BEHAVIOUR

establishISDNTerminalBhv 		BEHAVIOUR

DEFINED AS	"The action first verifies that the TSPID and access port profile name identified in the service are valid. If either is not valid the agent returns an invalid reference error. TSPID is considered valid if it is not assigned in to any existing TSP object instance. The access port profile name is considered valid if all of the following conditions are met:

·	an instance of accessPortProfile exists for the name provided in the action;

·	the accessPortProfile can support ISDN services.

Depending on the actual syntax selected, a different behaviour will apply for the remaining of the action execution:

·	if a ""servicePackage"" is provided, the service is instantiated based on the definition provided by a service package and the instantiateISDNTerminalServicePackageBehaviour applies;

·	if a ""copyCommand"" is selected, the service is instantiated based on the definition provided by an already existing service and the copyISDNTerminalCommandBehaviour applies.

In all cases, if the action is successful, the reply will indicate so and will also contain the list of names of the object instances just created. Otherwise the action leaves the MIB unaffected (unchanged) and returns the specified error message .";,

copyISDNTerminalCommandBhv		BEHAVIOUR

DEFINED AS	"The action verifies that the source Terminal Service Profile Name in the service is valid. If it is not valid the agent returns an invalid reference error. The existing Terminal Service Profile Name is considered valid if it exists.

The action creates a duplicate of  the terminalServiceProfile subtree of the service identified by the existing Terminal Service Profile Name.";,

instantiateISDNTerminalServicePackageBhv	BEHAVIOUR

DEFINED AS	"The action determines the existence of the service package and (optional) terminal configuration names provided in the action request parameters. If either does not exist, the agent returns an invalid reference error.

The action creates a duplicate of  the terminalServiceProfile containment subtree of the service package for use by the new service.";;

MODE CONFIRMED;

WITH INFORMATION SYNTAX

CAISDNAttributeModule.EstablishISDNTerminalRequest;

WITH REPLY SYNTAX CAISDNAttributeModule.CreatedInstancesName;

REGISTERED AS	{cAISDNAction 6};

## 12.7	Remove ISDN TerminalXE "Remove ISDN Terminal"§

removeISDNTerminal		ACTION

BEHAVIOUR

removeISDNTerminalBhv			BEHAVIOUR

DEFINED AS	"This action removes a Terminal Service Profile, all contained objects, and all associated supplementary service objects based on the Terminal Service Profile Name parameter in the action request.

The action verifies that the terminal service profile name exists. If not the agent returns an invalid reference error.

The name of all the deleted object instances is sent back as a reply to the managing system.";;

MODE CONFIRMED;

WITH INFORMATION SYNTAX

CAISDNAttributeModule.RemoveISDNTerminalRequest;

WITH REPLY SYNTAX CAISDNAttributeModule.DeletedInstancesName;

REGISTERED AS	{cAISDNAction 7};

## 12.8	Retrieve ServiceXE "Retrieve Service"§

retrieveService				ACTION

BEHAVIOUR

retrieveCustomerServiceBhv		BEHAVIOUR

DEFINED AS	"This action is used to retrieve a customer service identified by either the Directory Number Name or the Termination Point Name. When both are present, only the part of the service common to both will be retrieved.

If either the Directory Number Name or the Termination Point Name do not exist, no service exists and the agent returns an invalid reference error.   If the Directory Number Name is not in service on the specified Termination Point Name then an invalid Reference error is returned to the managing system.

Otherwise, the action is successful and linked replies contain the objects in the customer service profile according to the following rules:

If the action request contains only the directory number name, then the object instances returned are all of the following:

·	the directoryNumber,

·	the customerProfile or its subclasses related to the directoryNumber, and its contained object instances,

·	all the accessPortProfiles related to the customerProfile or its subclasses, and their contained object instances,

·	the terminationPoints related to all the accessPortProfiles.

If the action request contains only the termination point name, then the object instances returned are all of the following:

·	the terminationPoint with the attribute officeEquipment equal to that of the action request parameter,

·	the accessPortProfile related to the terminationPoint, and its contained object instances,

·	all the customerProfile or its subclassess related to the accessPortProfile, and their contained object instances,

·	the directoryNumbers related to all the customerProfile or its subclassess.

If the action request contains both the directory number name and the termination point name, then the object instances returned are all of the following:

·	the directoryNumber,

·	the customerProfile or its subclasses related to the directoryNumber, and its contained object instances,

·	the terminationPoint with the attribute officeEquipment equal to that of the action request parameter,

·	the accessPortProfile related to the terminationPoint, and its contained object instances.";;

MODE CONFIRMED;

WITH INFORMATION SYNTAX

CAISDNModule. RetrieveCustomerServiceRequest;

WITH REPLY SYNTAX CAISDNModule. RetrieveCustomerServiceReply;

REGISTERED AS	{cAISDNAction 8};

# 13	Type DefinitionsXE "Type Definitions"§

CAISDNModule {itu-t(0) recommendation(0) q(17) ca(824) dot(127) isdn(1) informationModel(0) asn1Modules(2) cAISDNModule(0)}

DEFINITIONS ::= BEGIN

--  EXPORTS Everything;

IMPORTS

InterceptTreatmentTerm,

NumberOfBChannels,

DirectoryNumber,

DirectoryNumberList

FROM CACommonModule {itu-t(0) recommendation(0) q(17) ca(824) dot(127) common(0) informationModel(0) asn1Modules(2) cACommonModule(0)}

UsageState,

OperationalState

FROM Attribute-ASN1Module {joint-iso-ccitt ms(9) smi(3) part2(2) asn1Module(2) 1}

k,

n1,

n2,

sequenceModulus,

t1Timer,

t2Timer,

t3Timer,

t4Timer

FROM DLM {joint-iso-ccitt network-layer(15) management(0) asn1Module(2) 0}

dBitModification,

defaultPacketSize,

defaultThroughputClass,

defaultWindowSize,

extendedPacketSequencing,

fastSelectAcceptance,

flowControlParameterNegotiation,

nonStandardDefaultPacketSizes,

nonStandardDefaultWindowSizes,

onlineFacilityRegistration,

packetRetransmission,

throughputClassNegotiation

FROM NLM {joint-iso-ccitt network-layer(13) management(0) nLM(2) asn1Module(2) 0}

ObjectInstance,

ObjectClass

FROM CMIP-1 {joint-iso-ccitt ms(9) cmip(1) modules(0) protocol(3)}

AlarmStatus,

Boolean,

ChannelNumber,

NameType,

ObjectList,

Pointer,

PointerOrNull

FROM ASN1DefinedTypesModule {ccitt recommendation m(13) gnm(3100) informationModel(0) asn1Modules(2) asn1DefinedTypesModule(0)};

q824-1InformationModel OBJECT IDENTIFIER ::= {itu-t(0) recommendation(0) q(17) ca(824) dot(127) isdn(1) informationModel(0)}

cAISDNObjectClass OBJECT IDENTIFIER ::= {q824-1InformationModel managedObjectClass(3)}

cAISDNPackage OBJECT IDENTIFIER ::= {q824-1InformationModel package(4)}

cAISDNParameter OBJECT IDENTIFIER ::= {q824-1InformationModel parameter(5)}

cAISDNAttribute OBJECT IDENTIFIER ::= {q824-1InformationModel attribute(7)}

cAISDNNameBinding OBJECT IDENTIFIER ::= {q824-1InformationModel nameBinding(6)}

cAISDNAction OBJECT IDENTIFIER ::= {q824-1InformationModel action(9)}

--  default value definitions  --

false Boolean ::= FALSE

true Boolean ::= TRUE

null NULL ::= NULL

minusOne INTEGER ::= –1

emptySet AccessChannelPtrList ::= { }

two INTEGER ::= 2

baud9600 ThruputClass ::= baud9600

size128 PacketSize ::= size128

dte84 DTECompatibility ::= dte84

linkOptionDefault LinkOption ::= dynamic :{callControlTEItotal    	64

callControlTEIsw		63

packTEItotal			64

packTEIsw			63}

--  supporting productions  --

DTECompatibility ::= ENUMERATED {

dte80			(0),

dte84			(1),

dte88			(2),

dte93			(3)

}

AccessChannelPtrList	::= SET OF ObjectInstance

ActiveTerminalList ::= SET OF SEQUENCE {

spid					[0]	ServiceProfileIdentifier,

tei					[1]	TerminalEndPointIdentifier,

usid					[2]	UserServiceId }

AppearanceInfo ::= SEQUENCE {

callAppearanceId		CallAppearanceIdentifier,

defaultBearerService 	BearerService}

AssignmentOfTimeslots 	::= ENUMERATED {

fixed 			(0),

floating 			(1),

flexible 			(2)}

BearerServiceList		::= SET OF BearerService

BearerService ::= 		ENUMERATED {

speech(0),

audio3D1(1),

audio7(2),

audioComb(3),

cmd56(4),

cmd64(5),

cmdComb(6)}

BitRateOfPrimaryRateInterface ::= ENUMERATED{

bitRate1544	(0)

bitRate2048	(1)}

CallAppearanceIdentifier ::= INTEGER

CallingPartyDefaultDirectoryNumber ::= CHOICE {

noDefault				NULL,

directoryNumber		DirectoryNumber,			--  ALL Bearer Services

perBearerService		SET OF SEQUENCE {

directoryNumber 		DirectoryNumber,

bearerService			BearerService}}

CallingPartyValidDirectoryNumberList ::= SET OF DirectoryNumber

ChangeDirectoryNumberRequest ::= SEQUENCE {

oldDirectoryNumberName 	ObjectInstance,

oldDirectoryNumberIntercept	InterceptTreatmentTerm,

newDirectoryNumberName 	ObjectInstance}

ChannelSelection ::= ENUMERATED {byNetwork (0), byUser (1)}

CopyISDNAccessCommandDescription ::= SEQUENCE {

sourceAPPName		ObjectInstance,

tpNameList			SET OF ObjectInstance}

CopyISDNServiceCommandDescription ::= SEQUENCE {

sourceCustomerProfileName				ObjectInstance,

accessPortName 				ObjectInstance,

resourceDescription 		SET OF SEQUENCE {

sourceDirectoryNumber  ObjectInstance,

newDirectoryNumber  ObjectInstance}}

CopyISDNTerminalCommandDescription ::= SEQUENCE {

sourceTerminalName		ObjectInstance,

aPPName					ObjectInstance,

sPId						IA5String (SIZE(1..18))}

CreatedInstancesName ::= SET OF ObjectInstance

DChannelT3xx ::= INTEGER -- number of seconds

DeletedInstancesName ::= SET OF ObjectInstance

DirectoryNumberAppearanceIdentifierList ::= SEQUENCE {

directoryNumber 		DirectoryNumber,

COMPONENTS OF AppearanceInfo}

DirectoryNumberReference ::= SEQUENCE {dnr INTEGER (1..128),

dn DirectoryNumber,

bs BearerService}

Dynamic ::= SEQUENCE {

callControlTEItotal		INTEGER,  --  total number of TEIs that can be assigned to signalling

callControlTEIsw		FlexType,

packTEItotal		INTEGER,  --  total number of TEIs that can be assigned to the packet.

packTEIsw		FlexType}

EstablishISDNAccessRequest ::= CHOICE {

servicePackageISDNAccess		[0] ServicePackageISDNAccessDescription,

copyISDNAccessCommand		[1] CopyISDNAccessCommandDescription }

EstablishISDNServiceRequest ::= CHOICE {

servicePackageISDN				[0] ServicePackageISDNDescription,

copyISDNServiceCommand		[1] CopyISDNServiceCommandDescription }

EstablishISDNTerminalRequest ::= CHOICE {

servicePackageISDNTerminal		[0] ServicePackageISDNTerminalDescription,

copyISDNTerminalCommand		[1] CopyISDNTerminalCommandDescription }

FlexType ::= CHOICE {

switchAssigned  	INTEGER,

uncontrolled		NULL}

MasterFeatureList ::= GraphicString(SIZE(1 .. 7))

FeatureActivatorValue ::= INTEGER(0..16383)

FeatureActivatorsAllDirectoryNumber ::= SET OF SEQUENCE {

featureActivatorValue 		FeatureActivatorValue,

masterFeatureList 			MasterFeatureList}

FeatureActivatorsPerDirectoryNumber ::= SET OF SEQUENCE {

featureActivatorValue 		FeatureActivatorValue,

directoryNumberReference 	INTEGER(1..128),

masterFeatureList 			MasterFeatureList}

FeatureIndicatorsAllDirectoryNumber ::= SET OF SEQUENCE {

featureActivatorValue 		FeatureActivatorValue,

masterFeatureList 			MasterFeatureList}

FeatureIndicatorsPerDirectoryNumber ::= SET OF SEQUENCE {

featureActivatorValue 		FeatureActivatorValue,

directoryNumberReference 	INTEGER(1..128),

masterFeatureList 			MasterFeatureList}

InterfaceType ::= ENUMERATED {

basic			(0),

primary		(1)

}

LinkOption	::= CHOICE {fixed NULL, dynamic Dynamic}

MaxBitsPerInformationFrame	::= INTEGER

MaxCombinedThruputClass	::= CHOICE {

maxCombinedThruput	MaxCombinedThruput,

null					NULL}

MaxCombinedThruput ::= ENUMERATED {

baud16000 					(0),

baud18000 					(1),

baud20000 					(2),

baud22000 					(3),

baud24000 					(4),

baud26000 					(5),

baud28000 					(6),

baud30000 					(7),

baud32000 					(8),

baud64000 					(9),

baud72000 					(10),

baud80000 					(11),

baud88000 					(12),

baud96000 					(13),

baud104000 					(14),

baud112000 					(15),

baud120000 					(16),

baud128000 					(17)

}

MaxNumberOfCallReference ::= INTEGER

MaxTransmissionAttempts	::= INTEGER

NotificationClass ::= ENUMERATED {

noNotificationClass(1),				--  without notification

conditionalNotificationClass(2),		--  with conditional notification

unconditionalNotificationClass(3)}

NumberOfDChannelLinks ::= INTEGER (1..256)

PacketSize ::= ENUMERATED {

size16	 				(0),

size32	 				(1),

size64	 				(2),

size128					(3),

size256					(4),

size512					(5),

size1024					(6),

size2048					(7),

size4096					(8)}

PossibleServicePackages			SERVICE-PACKAGE-SPECIFIC-DATA ::= {...}

PrimaryIC	::= IA5String

RemoveISDNAccessRequest ::= ObjectInstance

RemoveISDNServiceRequest ::= SEQUENCE {

directoryNumberName	ObjectInstance,

bearerServiceName	ObjectInstance}

RemoveISDNTerminalRequest ::= ObjectInstance

RetrieveCustomerServiceRequest ::= SEQUENCE {

tpName 				[0] ObjectInstance OPTIONAL,

directoryNumber 		[1] ObjectInstance OPTIONAL,

partyLineIdentifier 		[2] PartyLineIdentifier OPTIONAL

--	PartyLineIdentifier is a parameter required to identify a particular customer when muliple customers are

--	provided service on a single analog line.

}

RetrieveCustomerServiceReply ::= SET OF ObjectInstance

SemiPermAccessPacketHandlerDefaultDirectoryNumber ::= SET OF

SEQUENCE { defaultDirectoryNumber  DirectoryNumber,

bChannel		ChannelNumber}

ServiceProfileIdentifier ::= CHOICE {

null					NULL,

spid					IA5String(SIZE(3 .. 20))}

ServicePackageISDNAccessDescription ::= SEQUENCE {

servicePackageName	ObjectInstance,

accessPortName 	ObjectInstance}

ServicePackageISDNDescription ::= SEQUENCE {

servicePackageName

SERVICE-PACKAGE-SPECIFIC-DATA. & servicePackageName ({PossibleServicePackages}),

accessPortName 	ObjectInstance,

serviceDescription 	SET OF SEQUENCE {

templateDirectoryNumberName	ObjectInstance,

directoryNumberName 			ObjectInstance,

serviceRequestInfo

SERVICE-PACKAGE-SPECIFIC-DATA. & ServiceRequestInfo ({PossibleServicePackages}

{@service PackageName})	OPTIONAL}}

ServicePackageISDNTerminalDescription ::= SEQUENCE {

servicePackageName	ObjectInstance,

aPPName 		ObjectInstance,

sPId						IA5String (SIZE(1..18)),

terminalConfigurationName	ObjectInstance	OPTIONAL}

TerminalEndPointIdentifier	::= CHOICE { auto 	NULL,

non-Auto	INTEGER(0...126)}

TerminalLimit	::= INTEGER(0..62)

TerminalServiceProfilePtrList	::= SET OF ObjectInstance

ThruputClass ::= ENUMERATED {

baud75 					(0),

baud150 					(1),

baud300 					(2),

baud600 					(3),

baud1200   				(4),

baud2400   				(5),

baud4800   				(6),

baud9600   				(7),

baud19200 				(8),

baud48000 				(9),

baud56000 				(10),

baud64000 				(11)}

TSPID ::= IA5String (SIZE(1..18))

UserServiceId ::= CHOICE {

null				NULL,

uid				INTEGER(0 .. 126)}

WindowSize	::= CHOICE	{lowRange	[0] INTEGER(1..7),

highRange	[1] INTEGER(61..167)}

END  --  Type definitions  --

# 14	Actions

This clause contains the parameter templates for the services defined in the previous clauses.

## 14.1	Conventions

The definition of each service in this Recommendation includes a table that lists the parameters of its primitives. For a given primitive, the presence of each parameter is described by one of the following values:

M		The parameter is mandatory.

(=)		The value of the parameter is equal to the body of the parameter in the column to the left.

U		Use of the parameter is a service-user option – the parameter is not present in the interaction.

C		The parameter is conditionally present – the conditions are defined by the text that describes the parameter.

## 14.2	Change Directory NumberXE "Change Directory Number"§

The Change Directory Number service is used to allow a managing system (OS) to request a change in the directory number for a given customer service by the agent. This action uses the CMIS M-ACTION service. Table 1 gives the parameters for this action.

TABLE  1/Q.824.1

Change Directory Number ParametersXE "Change Directory Number Parameters"§

| Parameter name | Req./Ind. | Rsp./Cnf. |
| --- | --- | --- |
| Invoke Identifier | M | M= |
| Linked Identifier | – | C |
| Mode | M | – |
| Base Object Class | M | – |
| Base Object Instance | M | – |
| Scope | U | – |
| Filter | U | – |
| Managed Object Class | – | C |
| Managed Object Instance | – | C |
| Access Control | U | – |
| Synchronization | U | – |
| Action Type | M | C(=) |
| Action Information | M | – |
| ChangeDirectoryNumberRequest | M | – |
| Old Directory Number Name | M |  |
| Old Directory Number Intercept | M |  |
| New Directory Number Name | M |  |
| Current Time | – | U |
| Errors | – | C |

## 14.3	Establish ISDN AccessXE "Establish ISDN Access"§

The Establish ISDN Access service is used to allow a managing system (OS) to request that an ISDN Access service be created by the agent. This action uses the CMIS M-ACTION service. Table 2 gives the parameters for this action.

TABLE  2/Q.824.1

Establish ISDN Access ParametersXE "Establish ISDN Access Parameters"§

| Parameter name | Req./Ind. | Rsp./Cnf. |
| --- | --- | --- |
| Invoke Identifier | M | M= |
| Linked Identifier | – | C |
| Mode | M | – |
| Base Object Class | M | – |
| Base Object Instance | M | – |
| Scope | U | – |
| Filter | U | – |
| Managed Object Class | – | C |
| Managed Object Instance | – | C |
| Access Control | U | – |
| Synchronization | U | – |
| Action Type | M | C(=) |
| Action Information | M | – |
| Service Package ISDN Access | C1 | – |
| Service Package Name | M | – |
| TP Name | M | – |
| Copy ISDN Access Command | C1 | – |
| Source APP Name | M | – |
| tp Name List | M | – |
| Current Time | – | U |
| Action Result | – | M |
| Created Instances Name | – | M |
| Errors | – | C |
| C1 Either Service Package ISDN Access or Copy ISDN Access Command must be present. |  |  |

## 14.4	Establish ISDN ServiceXE "Establish ISDN Service"§

The Establish ISDN Service service is used to allow a managing system (OS) to request that an ISDN Service be created by the agent. This action uses the CMIS M-ACTION service. Table 3 gives the parameters for this action.

TABLE  3/Q.824.1

Establish ISDN Service ParametersXE "Establish ISDN Service Parameters"§

| Parameter name | Req./Ind. | Rsp./Cnf. |
| --- | --- | --- |
| Invoke Identifier | M | M= |
| Linked Identifier | – | C |
| Mode | M | – |
| Base Object Class | M | – |
| Base Object Instance | M | – |
| Scope | U | – |
| Filter | U | – |
| Managed Object Class | – | C |
| Managed Object Instance | – | C |
| Access Control | U | – |
| Synchronization | U | – |
| Action Type | M | C(=) |
| Action Information | M | – |
| Service PackageISDN | C1 | – |
| Service Package Name | M | – |
| tp Name | M | – |
| Service Description | M | – |
| Copy ISDN Access Command | C1 | – |
| Source Customer Profile Name | M | – |
| tp Name | M | – |
| Resource Description | M | – |
| Current Time | – | U |
| Action Result | – | M |
| Created Instances Name | – | M |
| Errors | – | C |
| C1 Either the Service Package ISDN or the Copy ISDN Service Command must be present. |  |  |

## 14.5	Establish ISDN TerminalXE "Establish ISDN Terminal"§

The Establish ISDN Terminal service is used to allow a managing system (OS) to request that an ISDN Terminal service be created by the agent. This action uses the CMIS M-ACTION service. Table 4 gives the parameters for this action.

TABLE  4/Q.824.1

Establish ISDN Terminal ParametersXE "Establish ISDN Terminal Parameters"§

| Parameter name | Req./Ind. | Rsp./Cnf. |
| --- | --- | --- |
| Invoke Identifier | M | M= |
| Linked Identifier | – | C |
| Mode | M | – |
| Base Object Class | M | – |
| Base Object Instance | M | – |
| Scope | U | – |
| Filter | U | – |
| Managed Object Class | – | C |
| Managed Object Instance | – | C |
| Access Control | U | – |
| Synchronization | U | – |
| Action Type | M | C(=) |
| Action Information | M | – |
| Service Package ISDN Terminal | C1 | – |
| Service Package Name | M | – |
| APP Name | M | – |
| TSPID | M | – |
| Termination Configuration Name | U | – |
| Copy ISDN Terminal Command | C1 | – |
| Source Terminal Name | M | – |
| APP Name | M | – |
| TSPID | M | – |
| Current Time | – | U |
| Action Result | – | M |
| Created Instances Name | – | M |
| Errors | – | C |
| C1 One of Service Package ISDN Terminal or Copy ISDN Terminal Command must be present. |  |  |

## 14.6	Remove ISDN AccessXE "Remove ISDN Access"§

The Remove ISDN Access service is used to allow a managing system (OS) to request that an ISDN Access service be removed by the agent. This action uses the CMIS M-ACTION service. Table 5 gives the parameters for this action.

TABLE  5/Q.824.1

Remove ISDN Access ParametersXE "Remove ISDN Access Parameters"§

| Parameter name | Req./Ind. | Rsp./Cnf. |
| --- | --- | --- |
| Invoke Identifier | M | M= |
| Linked Identifier | – | C |
| Mode | M | – |
| Base Object Class | M | – |
| Base Object Instance | M | – |
| Scope | U | – |
| Filter | U | – |
| Managed Object Class | – | C |
| Managed Object Instance | – | C |
| Access Control | U | – |
| Synchronization | U | – |
| Action Type | M | C(=) |
| Action Information | M | – |
| Remove ISDN Access Request | M | – |
| Current Time | – | U |
| Action Result | – | M |
| Deleted Instances Name | – | M |
| Errors | – | C |

## 14.7	Remove ISDN ServiceXE "Remove ISDN Service"§

The Remove ISDN Service service is used to allow a managing system (OS) to request that an ISDN Service be removed by the agent. This action uses the CMIS M-ACTION service. Table 6 gives the parameters for this action.

TABLE  6/Q.824.1

Remove ISDN Service ParametersXE "Remove ISDN Service Parameters"§

| Parameter name | Req./Ind. | Rsp./Cnf. |
| --- | --- | --- |
| Invoke Identifier | M | M= |
| Linked Identifier | – | C |
| Mode | M | – |
| Base Object Class | M | – |
| Base Object Instance | M | – |
| Scope | U | – |
| Filter | U | – |
| Managed Object Class | – | C |
| Managed Object Instance | – | C |
| Access Control | U | – |
| Synchronization | U | – |
| Action Type | M | C(=) |
| Action Information | M | – |
| Directory Number Name | M | – |
| Bearer Service | M | – |
| Current Time | – | U |
| Action Result | – | M |
| Remove Instances Name | – | M |
| Errors | – | C |

## 14.8	Remove ISDN TerminalXE "Remove ISDN Terminal"§

The Remove ISDN Terminal service is used to allow a managing system (OS) to request that an ISDN Terminal service be removed by the agent. This action uses the CMIS M-ACTION service. Table 7 gives the parameters for this action.

TABLE  7/Q.824.1

Remove ISDN Terminal ParametersXE "Remove ISDN Terminal Parameters"§

| Parameter name | Req./Ind. | Rsp./Cnf. |
| --- | --- | --- |
| Invoke Identifier | M | M= |
| Linked Identifier | – | C |
| Mode | M | – |
| Base Object Class | M | – |
| Base Object Instance | M | – |
| Scope | U | – |
| Filter | U | – |
| Managed Object Class | – | C |
| Managed Object Instance | – | C |
| Access Control | U | – |
| Synchronization | U | – |
| Action Type | M | C(=) |
| Action Information | M | – |
| Remove ISDN Terminal | M | – |
| Current Time | – | U |
| Action Result | – | M |
| Delete Instances Name | – | M |
| Errors | – | C |

## 14.9	Retrieve Customer ServiceXE "Retrieve Customer Service"§

The RetrieveCustomerService service is used to allow a managing system (OS) to request to retrieve a customer service identified by either the Directory Number Name or the Termination Point Name. When both are present, only the common part of the service to both will be retrieved. This M-ACTION service does not modify any objects or attributes. Table 8 provides the parameters for this action.

TABLE  8/Q.824.1

Retrieve Customer Service ParametersXE "Retrieve Customer Service Parameters"§

| Parameter name | Req./Ind. | Rsp./Cnf. |
| --- | --- | --- |
| Invoke Identifier | M | M= |
| Linked Identifier | – | M |
| Mode | M | – |
| Base Object Class | M | – |
| Base Object Instance | M | – |
| Scope | U | – |
| Filter | U | – |
| Managed Object Class | – | C |
| Managed Object Instance | – | C |
| Access Control | U | – |
| Synchronization | U | – |
| Action Type | M | C(=) |
| Action Information | M | – |
| Termination Point Name | U | – |
| Directory Number Name | U | – |
| Party Line Identifier | U | – |
| Current Time | – | U |
| Action Result | – | M |
| RetrieveCustomerServiceReply | – | M |
| Errors | – | C |