|  | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

ITU-T	Q.2110

TELECOMMUNICATION	(07/94)
STANDARDIZATION  SECTOR
OF  ITU

BROADBAND  ISDN

B-ISDN  ATM  ADAPTATION  LAYER  –  SERVICE  SPECIFIC  CONNECTION  
ORIENTED  PROTOCOL  (SSCOP)

ITU-T  Recommendation  Q.2110

(Previously “CCITT  Recommendation”)

# FOREWORD

The ITU-T (Telecommunication Standardization Sector) is a permanent organ of the International Telecommunication Union (ITU). The ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis..

The World Telecommunication Standardization Conference (WTSC), which meets every four years, etablishes the topics for study by the ITU-T Study Groups which, in their turn, produce Recommendations on these topics.

The approval of Recommendations by the Members of the ITU-T is covered by the procedure laid down in WTSC Resolution No. 1 (Helsinki, March 1-12, 1993).

ITU-T Recommendation Q.2110 was prepared by ITU-T Study Group 11 (1993-1996) and was approved under the WTSC Resolution No. 1 procedure on the 29th of July 1994.

___________________

NOTE

In this Recommendation, the expression “Administration” is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

ã  ITU  1995

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

CONTENTS

Recommendation Q.2110     (07/94)

Page

1	Scope		1

2	Normative references		1

3	Abbreviations		1

4	General		3

5	Functions of the SSCOP		4

6	Elements for layer to layer communication		5

6.1	Signals between SSCOP and SSCF, and SSCOP and SSCS layer management		5

6.2	State transition diagram for sequences of signals		7

6.3	Signals between SSCOP and CPCS		7

7	Protocol elements for peer-to-peer communications		9

7.1	SSCOP PDUs		9

7.2	SSCOP PDU formats		11

7.3	States of SSCOP protocol entity		16

7.4	SSCOP state variables		17

7.5	SSCOP PDU parameters		18

7.6	SSCOP timers		19

7.7	SSCOP parameters		20

7.8	SSCOP credit and flow control		21

8	Specification of SSCOP		22

8.1	Overview		22

8.2	SDL diagrams		24

Annex A – Management error indications		78

Annex B – Protocol implementation conformance statement (PICS) proforma to Recommendation Q.2110		79

B.1	General		79

B.2	Abbreviations and special symbols		79

B.3	Instructions for completing the PICS proforma		79

B.4	Global statement of conformance		80

B.5	SSCOP – Q.2110		80

Appendix I – Concepts and terminology		83

Appendix II – Examples of SSCOP operation		85

Appendix III – Summary of buffer and state variable management		93

Appendix IV – Default window size for SSCOP		95

# SUMMARY

The intent of this Recommendation is to provide a new protocol specification that can be used in the B-ISDN ATM Adaptation Layer (AAL). This protocol, called the Service Specific Connection Oriented Protocol (SSCOP), provides assured data delivery between AAL connection endpoints.

The ATM Adaptation layer (AAL) is defined as enhancing the services provided by the ATM layer to support the functions required by the next higher layer. Different AALs support various protocols to suit the different needs of a range of AAL service users. One particular type of AAL service user is the signalling entity wishing to communicate with a peer entity. The signalling AAL (SAAL) comprises AAL functions necessary to support such a signalling entity. The structure of the SAAL is defined in Recommendation Q.2100.

The SSCOP has been defined to provide functions required in the SAAL. The SAAL is a combination of two sublayers: a common part and a service specific part. The common part protocol is defined in Recommendation I.363. The service specific part is also known as the Service Specific Convergence Sublayer (SSCS). In the SAAL, the SSCS itself is functionally divided into the SSCOP and an Service Specific Coordination Function (SSCF) which maps the services provided by the SSCOP to the needs of the user of the SAAL. This structure allows a common connection oriented protocol with error recovery (the SSCOP) to provide a generic reliable data transfer service for different AAL interfaces defined by the SSCF. Two such SSCFs, one for signalling at the User Network Interface (UNI) and one for signalling at the Network to Network Interface (NNI), have been defined in Recommendations Q.2130 and Q.2140, respectively. It is also possible to define additional SSCFs over the common SSCOP to provide different AAL services.

The SSCOP is a peer-to-peer protocol which provides the following functions:

–	transfer of user data with sequence integrity;

–	error correction by selective retransmission;

–	flow control;

–	connection control;

–	error reporting to layer management;

–	connection maintenance in the prolonged absence of data transfer;

–	local data retrieval by the user;

–	error detection of protocol control information; and

–	status reporting.

This Recommendation describes for SSCOP the necessary elements for layer to layer communication, the elements for peer-to-peer communication, a detailed protocol specification, and examples of SSCOP operation.

Keywords

AAL	ATM Adaptation Layer

ATM	Asynchronous Transfer Mode

B-ISDN	Broadband Integrated Services Network

NNI	Network Node Interface

SAAL	Signalling AAL

SSCF	Service Specific Coordination Function

SSCOP	Service Specific Connection Oriented Protocol

SSCS	Service Specific Convergence Sublayer

UNI	User to Network Interface

Recommendation Q.2110

Recommendation Q.2110     (07/94)

B-ISDN  ATM  ADAPTATION  LAYER  –  SERVICE  SPECIFIC
CONNECTION  ORIENTED  PROTOCOL  (SSCOP)

(Geneva, 1994)

# 1	Scope

This Recommendation describes the Service Specific Connection Oriented Protocol (SSCOP). This Recommendation specifies the peer-to-peer protocol for the transfer of information and control between any pair of SSCOP entities, the interactions between the SSCOP and an SSCF, the interactions between the SSCOP and the AAL Common Part, and the interactions between the SSCOP and the AAL management plane. Recommendation Q.2100 [8] describes how this Recommendation is related to the SSCF for signalling at the UNI, Recommendation Q.2130 [9], and to the SSCF for signalling at the NNI, Recommendation Q.2140 [10].

# 2	Normative references

The following Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision: all users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of currently valid ITU-T Recommendation is published.

[1]	CCITT Recommendation X.200, Reference model of open systems interconnection for CCITT applications.

[2]	CCITT Recommendation X.210, OSI layer service conventions.

[3]	ITU-T Recommendation I.150, B-ISDN Asynchronous Transfer Mode Functional Characteristics.

[4]	ITU-T Recommendation I.361, B-ISDN ATM Layer Specification.

[5]	ITU-T Recommendation Q.2931, B-ISDN access signalling system DSS2 (Digital Subscriber Signalling System No. 2).

[6]	ITU-T Recommendation Q.704, Signalling Network Functions and Messages.

[7]	ITU-T Recommendation I.363, B-ISDN ATM Adaptation Layer (AAL) Specification.

[8]	ITU-T Recommendation Q.2100, B-ISDN Signalling ATM Adaptation Layer (SAAL) overview description.

[9]	ITU-T Recommendation Q.2130, B-ISDN Signalling ATM Adaptation Layer – Service Specific Coordination Function for support of signalling at the user-to-network interface (SSCF at UNI).

[10]	ITU-T Recommendation Q.2140, B-ISDN Signalling ATM Adaptation Layer – Service Specific Coordination Function for Signalling at the Network Node Interface (SSCF at NNI).

# 3	Abbreviations

AA	ATM Adaptation

AAL	ATM Adaptation Layer

ATM	Asynchronous Transfer Mode

B-ISDN	Broadband Integrated Services Digital Network

BGAK	Begin Acknowledge (PDU)

BGN	Begin (PDU)

BGREJ	Begin Reject (PDU)

BR	Buffer Release

CPCS	Common Part Convergence Sublayer

END	End (PDU)

ENDAK	End Acknowledge (PDU)

ER	Error Recovery (PDU)

ERAK	Error Recovery Acknowledge (PDU)

ID	Interface Data

LM	Layer Management

MAA	Management ATM Adaptation

MaxCC	Maximum Connection Control (Count)

MaxPD	Maximum Poll Data (Count)

MaxSTAT	Maximum STAT (Count)

MD	Management Data (PDU)

MSB	Most Significant Bit

MTP	Message Transfer Part

MU	Message Unit

NNI	Network Node Interface

OSI	Open Systems Interconnection

PAD	Padding

PCI	Protocol Control Information

PD	POLL Data

PDU	Protocol Data Unit

PICS	Protocol Implementation Conformance Statement

PL	Pad Length

POLL	Poll (PDU)

QOS	Quality of Service

R	Reserved (field)

RN	Retrieval Number

RS	Resynchronization (PDU)

RSAK	Resynchronization Acknowledge (PDU)

Rsvd	Reserved (field)

S	Source (field)

SAAL	Signalling ATM Adaptation Layer

SAP	Service Access Point

SAR	Segmentation And Reassembly

SD	Sequenced Data (PDU)

SDL	Specification and Description Language

SDU	Service Data Unit

SN	Sequence Number

SSCF	Service Specific Coordination Function

SSCOP	Service Specific Connection Oriented Protocol

SSCS	Service Specific Convergence Sublayer

STAT	Solicited Status (PDU)

UD	Unnumbered Data (PDU)

UNI	User Network Interface

USTAT	Unsolicited Status (PDU)

UU	User-to-User

VR	Receiver state Variable

VT	Transmitter state Variable

# 4	General

The Service Specific Connection Oriented Protocol (SSCOP) resides in the Service Specific Convergence Sublayer (SSCS) of the ATM Adaptation Layer (AAL). Figure 1 illustrates the structure of the AAL. SSCOP is used to transfer variable length Service Data Units (SDUs) between users of SSCOP.

Figure  1/Q.2110.....[D01]  =  16 cm

SSCOP provides its service to a Service Specific Coordination Function (SSCF). The SSCF maps the service of SSCOP to the needs of the AAL user. The SSCFs are specified in other Recommendations. SSCOP utilizes the service of the CPCS (Common Part Convergence Sublayer) and SAR protocols which provide an unassured information transfer and a mechanism for detecting corruption of SSCOP Protocol Data Units (PDUs). The CPCS and SAR protocols are specified in Recommendation I.363 [7].

As shown in Figure 1, the AAL is functionally divided into the common part and the Service Specific Convergence Sublayer (SSCS). The SSCS is specific to the needs of the particular service application and in some cases may be functionally null. The SSCOP can operate over different AAL Common Part Protocols and may be used by different SSCFs; the SSCF is specific to the needs of the service application. The Common Part protocols are specified in Recommendation I.363 [7].

One currently defined use of SSCOP is within the signalling AAL (SAAL). The purpose of the SAAL is to convey information between layer 3 entities across the UNI and NNI using Asynchronous Transfer Mode (ATM) virtual channels. Other uses of SSCOP are for further study.

The definition of the SAAL takes into consideration the principles and terminology of Recommendations X.200 [1] and X.210 [2] – The reference model and layer service conventions for Open Systems Interconnection (OSI).

NOTES

1	The physical layer is currently defined in Recommendations I.150 [3] and I.361 [4]. Layer 3 is defined in Recommendations Q.2931 [5] for the UNI and Recommendation Q.704 [6] for the NNI. References should be made to these Recommendations for the complete definition of the protocols and procedures across the B-ISDN UNI and NNI, respectively.

2	The term “layer 3” is used to indicate the layer above the SAAL, the user of the SAAL services.

This Recommendation specifies:

a)	the peer-to-peer protocol for the transfer of information and control between any pair of SSCOP entities;

b)	the interactions between the SSCOP and an SSCF;

c)	the interactions between the SSCOP and the AAL Common Part;

d)	the interactions between the SSCOP and the AAL management plane.

# 5	Functions of the SSCOPXE "Functions of the SSCOP"§

The SSCOP performs the following functions.

a)	Sequence Integrity

This function preserves the order of SSCOP SDUs that were submitted for transfer by SSCOP.

b)	Error Correction by Selective Retransmission

Through a sequencing mechanism, the receiving SSCOP entity can detect missing SSCOP SDUs. This function corrects sequence errors through retransmission.

c)	Flow Control

This function allows an SSCOP receiver to control the rate at which the peer SSCOP transmitter entity may send information.

d)	Error Reporting to Layer Management

This function indicates to layer management errors which have occurred.

e)	Keep Alive

This function verifies that the two peer SSCOP entities participating in a connection are remaining in a link connection established state even in the case of a prolonged absence of data transfer.

f)	Local Data Retrieval

This function allows the local SSCOP user to retrieve in-sequence SDUs which have not yet been released by the SSCOP entity.

g)	Connection Control

This function performs the establishment, release, and resynchronization of an SSCOP connection. It also allows the transmission of variable length user-to-user information without a guarantee of delivery.

h)	Transfer of User-Data

This function is used for the conveyance of user data between users of the SSCOP. SSCOP supports both assured and unassured data transfer.

i)	Protocol Error Detection and Recovery

This function detects and recovers from errors in the operation of the protocol.

j)	Status Reporting

This function allows the transmitter and receiver peer entities to exchange status information.

# 6	Elements for layer to layer communication

This clause defines the signals and state transition diagram for sequences of signals between SSCOP and the SSCF. The term “signal” is used instead of “primitive” in order to reflect the fact that between SSCF and SSCOP there is no service access point defined.

## 6.1	Signals between SSCOP and SSCF, and SSCOP and SSCS layer managementXE "Signals between SSCOP and SSCF, and SSCOP and SSCS layer management"§

The following repertoire of AA-signals between SSCF and SSCOP, and MAA-signals between the SSCS layer management and SSCOP, is defined (see Table 1):

TABLE  1/Q.2110

SSCOP signals and parameters

|  | Type |  |  |  |
| --- | --- | --- | --- | --- |
| Generic Name | Request | Indication | Response | Confirmation |
| AA-ESTABLISH | SSCOP-UU BR | SSCOP-UU | SSCOP-UU BR | SSCOP-UU |
| AA-RELEASE | SSCOP-UU | SSCOP-UU Source | Not Defined | – |
| AA-DATA | MU | MU SN | Not Defined | Not Defined |
| AA-RESYNC | SSCOP-UU | SSCOP-UU | – | – |
| AA-RECOVER | Not Defined | – | – | Not Defined |
| AA-UNITDATA | MU | MU | Not Defined | Not Defined |
| AA-RETRIEVE | RN | MU | Not Defined | Not Defined |
| AA-RETRIEVE COMPLETE | Not Defined | – | Not Defined | Not Defined |
| MAA-ERROR | Not Defined | Code Count | Not Defined | Not Defined |
| MAA-UNITDATA | MU | MU | Not Defined | Not Defined |
| – The signal has no parameters. |  |  |  |  |

### 6.1.1	Signal definition

The definition of these signals is as follows:

a)	The AA-ESTABLISH signals are used to establish a point-to-point connection for assured information transfer between peer user entities.

b)	The AA-RELEASE signals are used to terminate a point-to-point connection for assured information transfer between peer user entities.

c)	The AA-DATA signals are used for the assured point-to-point transfer of SDUs between peer user entities.

d)	The AA-RESYNC signals are used to resynchronize the SSCOP connection.

e)	The AA-RECOVER signals are used during recovery from protocol errors.

f)	The AA-UNITDATA signals are used for the non-assured, broadcast and point-to-point, transfer of SDUs between peer user entities.

g)	AA-RETRIEVE signals are used to retrieve SDUs submitted by the user for transmission but not yet released by the transmitter.

h)	AA-RETRIEVE COMPLETE signal is used to indicate that there are no additional SDUs to be returned to the SSCOP user.

i)	MAA-ERROR signals are used to report SSCOP protocol errors and certain events to layer management.

j)	MAA-UNITDATA signals are used for the non-assured, broadcast and point-to-point, transfer of SDUs between SSCOP and peer layer management entities.

### 6.1.2	Parameter definition

Table 1 lists the parameters associated with each SSCOP signal. The definition of the parameters is as follows:

a)	The message Unit (MU) parameter is used during information transfer to convey a variable-length message. In AA-DATA.request, AA-UNITDATA.request, and MAA-UNITDATA.request signals, this parameter is mapped transparently into the Information field of an SSCOP PDU. For AA-DATA.indication, AA-UNITDATA.indication, and MAA-UNITDATA.indication signals, this parameter contains the contents of the information field of the received SSCOP PDU. In AA-RETRIEVE.indication signals, this parameter contains a message unit returned to the SSCOP user from either the transmitter queue (data not yet sent) or the transmitter buffer. The MU is an integral multiple of one octet.

b)	The SSCOP user-to-user information (SSCOP-UU) parameter is used during connection control to convey a variable-length user-to-user message. The transfer of SSCOP-UU in BGN, BGAK, BGREJ, RS, and END PDUs can not be guaranteed. In request and response signals, this parameter is mapped transparently into the SSCOP-UU (SSCOP User-to-User) field of an SSCOP PDU. For indication and confirm signals, this parameter contains the contents of the SSCOP-UU field of the received SSCOP PDU. The SSCOP-UU is an integral multiple of one octet, if it is present. The SSCOP-UU may be null (no data present).

c)	The sequence number (SN) parameter indicates the value of N(S) in the received SD PDU, and may be used to support the data retrieval operation.

d)	The retrieval number (RN) parameter is used to support data retrieval. The value RN + 1 indicates the value of the N(S) for the first SD PDU to be retrieved. A value of “Unknown” indicates that only the not yet transmitted SD PDUs are to be retrieved. A value of “Total” indicates that all the SD PDUs in both the transmission buffer and transmission queue are to be retrieved.

e)	The buffer release (BR) parameter indicates whether the transmitter may release its buffers upon subsequent release of the connection. This parameter also allows for the release of selectively acknowledged messages from the transmitter buffer. A value of “Yes” indicates that the transmission buffer and transmission queue may be released, and a value of “No” indicates that the transmission buffer and transmission queue may not be released.

f)	The code parameter indicates the type of protocol error that occurred. The Code parameters are defined in Annex A.

g)	The source parameter indicates to the SSCOP user whether the SSCOP layer or the peer SSCOP user originated the connection release. This parameter assumes one of two values: “SSCOP” or “User”. If “SSCOP” is indicated, the user should disregard the SSCOP-UU parameter, if present.

h)	The count parameter indicates the number of SD PDU retransmissions that occurred.

## 6.2	State transition diagram for sequences of signals

This subclause defines the constraints on the sequences in which the signals may occur. The sequences are related to the states at one point-to-point SSCOP endpoint, between SSCOP and an SSCF.

The possible overall sequences of signals at a point-to-point SSCOP endpoint are defined in the state transition diagram, Figure 2. The model illustrates the behaviour of SSCOP as seen by the SSCF. This model assumes that a request or response signal is never issued at the same time as an indication or confirm signal. The model also assumes that the signals are serviced immediately and in zero time. In the diagram:

a)	the signals AA-UNITDATA request and AA-UNITDATA indication are associated with Unacknowledged Data Transfer and thus permitted in any state – they are not shown;

b)	any other signal which is not shown as resulting in a transition (from one state to the same state, or from one state to a different state) is not permitted in that state;

c)	it is assumed that the signals passed between SSCOP and an SSCF are coordinated so that collisions do not occur;

d)	the Idle state (State 1) reflects the absence of a connection. It is the initial and final state of any sequence, and once it has been re-entered, the connection is released.

## 6.3	Signals between SSCOP and CPCSXE "Signals between SSCOP and CPCS"§

This Recommendation assumes the CPCS message mode of operation without the option “corrupted data delivery”. Use of Streaming Mode is for further study.

The SSCOP makes use of two signals between the SSCOP and CPCS: CPCS-UNITDATA.invoke and CPCS-UNITDATA.signal. SSCOP PDUs submitted for transmission to the peer are placed in the Interface Data (ID) parameter of the CPCS-UNITDATA.invoke. Messages extracted from the Interface Data (ID) parameter of the CPCS-UNITDATA.signal are received SSCOP PDUs from the peer.

Additional parameters of the CPCS-UNITDATA are set as follows:

CPCS-LP	For CPCS-UNITDATA.invoke, CPCS-LP = 0. This parameter is ignored in CPCS-UNITDATA.signal.

CPCS-CI	For CPCS-UNITDATA.invoke, CPCS-CI = 0. The use of this parameter for CPCS-UNITDATA.signal is for further study.

CPCS-UU	This parameter is set to zero by the transmitting entity and is ignored by the receiver.

Figure  2/Q.2110.....[D02]  =  page pleine

# 7	Protocol elements for peer-to-peer communications

## 7.1	SSCOP PDUsXE "SSCOP PDUs"§

The Protocol Data Units (PDUs) are listed and described in Table 2.

TABLE  2/Q.2110

SSCOP PDU names and definitions

| Functionality | PDU name | PDU type field | Description |
| --- | --- | --- | --- |
|  | BGN | 0001 | Request Initialization |
| Establishment | BGAK | 0010 | Request Acknowledgement |
|  | BGREJ | 0111 | Connection Reject |
| Release | END | 0011 | Disconnect Command |
|  | ENDAK | 0100 | Disconnect Acknowledgement |
| Resynchronization | RS | 0101 | Resynchronization Command |
|  | RSAK | 0110 | Resynchronization Acknowledgement |
| Recovery | ER | 1001 | Recovery Command |
|  | ERAK | 1111 | Recovery Acknowledgement |
|  | SD | 1000 | Sequenced Connection-mode Data |
| Assured Data Transfer | POLL | 1010 | Transmitter State Information with request for Receive State Information |
|  | STAT | 1011 | Solicited Receiver State Information |
|  | USTAT | 1100 | Unsolicited Receiver State Information |
| Unacknowledged Data Transfer | UD | 1101 | Unnumbered User Data |
| Management Data Transfer | MD | 1110 | Unnumbered Management Data |

The SSCOP PDU definitions are:

a)	BGN PDU (Begin)

The BGN PDU is used to establish an SSCOP connection between two peer entities. The BGN PDU requests the clearing of the peer’s transmitter and receiver buffers, and the initialization of the peer’s transmitter and receiver state variables.

b)	BGAK PDU (Begin Acknowledge)

The BGAK PDU is used to acknowledge the acceptance of a connection request from the peer.

c)	BGREJ PDU (Begin Reject)

The BGREJ PDU is used to reject the connection request of the peer SSCOP entity.

d)	END PDU (End)

The END PDU is used to release an SSCOP connection between two peer entities.

e)	ENDAK PDU (End Acknowledge)

The ENDAK PDU is used to confirm the release of an SSCOP connection.

f)	RS PDU (Resynchronization)

The RS PDU is used to resynchronize the buffers and data transfer state variables.

g)	RSAK PDU (Resynchronization Acknowledge)

The RSAK PDU is used to acknowledge the acceptance of a resynchronization requested by the peer SSCOP entity.

h)	ER PDU (Error Recovery)

The ER PDU is used to recover from protocol errors.

i)	ERAK PDU (Error Recovery Acknowledge)

The ERAK PDU is used to acknowledge the recovery from protocol error.

j)	SD PDU (Sequenced Data)

The SD PDU is used to transfer, across an SSCOP connection, sequentially numbered PDUs containing information fields provided by the SSCOP user.

k)	POLL PDU (Status Request)

The POLL PDU is used to request, across an SSCOP connection, status information about the peer SSCOP entity.

l)	STAT PDU (Solicited Status Response)

The STAT PDU is used to respond to a status request (POLL PDU) received from a peer SSCOP entity. It contains information regarding the reception status of SD PDUs, credit information for the peer transmitter, and the sequence number [N(PS)] of the POLL PDU to which it is in response.

m)	USTAT PDU (Unsolicited Status Response)

The USTAT PDU is used to respond to a detection of one or more new missing SD PDUs, based on the examination of the sequence number of the SD PDU. It contains information regarding the reception status of SD PDUs and credit information for the peer transmitter.

n)	UD PDU (Unnumbered Data)

The UD PDU is used for unassured data transfer between two SSCOP users. When an SSCOP user requests unacknowledged information transfer, the UD PDU is used to send information to the peer without affecting SSCOP states or variables. UD PDUs do not carry a sequence number and therefore, the UD PDU may be lost without notification.

o)	MD PDU (Management Data)

The MD PDU is used for unassured management data transfer between two management entities. When a management entity requests unacknowledged information transfer, the MD PDU is used to send information to the peer management entity without affecting SSCOP states or variables. MD PDUs do not carry a sequence number and therefore, the MD PDU may be lost without notification.

An invalid PDU is a PDU which:

a)	has an unknown PDU type code, or

b)	is not 32 bit aligned, or

c)	is not the proper length for a PDU of the stated type.

Invalid PDUs shall be discarded without notification to the sender. No additional action is taken as a result of that PDU (length violations from items b) and c) above are reported to layer management).

## 7.2	SSCOP PDU formatsXE "SSCOP PDU formats"§

Figure 3 through Figure 16 illustrate the formats of the SSCOP PDUs. There are 15 defined PDU types listed in 7.1. SSCOP PDU fields are defined in section 7.5.

Figure  3/Q.2110.....[D03]  =  6 cm

Figure  4/Q.2110.....[D04]  =  4.5 cm

Figure  5/Q.2110.....[D05]  =  8 cm

Figure  6/Q.2110.....[D06]  =  6 cm

Figure  7/Q.2110.....[D07]  =  6 cm

Figure  8/Q.2110.....[D08]  =  6 cm

Figure  9/Q.2110.....[D09]  =  6 cm

Figure  10/Q.2110.....[D10]  =  6 cm

Figure  11/Q.2110.....[D11]  =  6 cm

Figure  12/Q.2110.....[D12]  =  4.5 cm

Figure  13/Q.2110.....[D13]  =  6 cm

Figure  14/Q.2110.....[D14]  =  4.5 cm

Figure  15/Q.2110.....[D15]  =  4.5 cm

Figure  16/Q.2110.....[D16]  =  4.5 cm

The following features of these formats are noted.

### 7.2.1	Coding conventions

The coding of the SSCOP PDU conforms to the coding conventions specified in 2.1/I.361 [4].

NOTE – SSCOP is trailer oriented; i.e. the protocol control information is transmitted last.

### 7.2.2	Padding (PAD) fieldXE "Padding (PAD) field"§

a)	SD, MD, and UD PDUs

Between the end of the SD, MD, or UD PDU information field and the trailer, there will be from 0 to 3 unused octets. These unused octets are called the Padding (PAD) field; they are strictly used as filler octets and do not convey any information. Any coding is acceptable. This padding field complements the PDU to an integral number of 4 octets.

The Pad Length (PL) field in each PDU indicates the number of PAD octets present in the PDU. It can take on any integer value from 0 to 3.

b)	BGN, BGAK, BGREJ, END, and RS PDUs

The SSCOP may convey a variable length User-to-User information field (SSCOP-UU) in a BGN, BGAK, BGREJ, END, or RS PDU. If this field is present in a PDU, it is padded with 0 to 3 filler octets in order to complement the PDU length to an integral multiple of 4 octets. These unused octets are called the Padding (PAD) field; they are strictly used as filler octets and do not convey any information. Any coding is acceptable.

The Pad Length (PL) field in each PDU indicates the number of PAD octets present in the PDU. It can take on any integer value from 0 to 3. If no SSCOP-UU field is present in the PDU, the PL field is coded as zero.

c)	STAT and USTAT PDUs

In any STAT or USTAT PDU containing list elements, a one octet PAD field precedes the 3 octet sequence number. These unused PAD octets are used to complement each list element field to four octets in length and do not convey any information. Any coding is acceptable.

### 7.2.3	Reserved fieldXE "Reserved field"§

There is a field of reserved bits (R, Rsvd, Reserved) in each PDU. One function of the reserved field is to achieve 32 bit alignment. Other functions are for further study. Where no functions other than 32 bit alignment are defined, this field shall be coded as zero. This field shall be ignored by the receiver.

### 7.2.4	PDU lengthXE "PDU length"§

The maximum length of the information field in SD, UD, and MD PDUs is k octets. The maximum value of k is 65 528 octets. The value of k is established as part of size negotiation procedures outside SSCOP, upon bilateral agreement, may be specified by an SSCF Recommendation utilizing SSCOP, or may be derived from the maximum length PDU size for protocols using SSCOP. The minimum value of k is 0 octets.

The maximum length of a variable length SSCOP-UU field is j octets. The maximum value of j is 65 524 octets. The value of j is established upon bilateral agreement, may be specified by an SSCF Recommendation utilizing SSCOP, or may be derived from requirements of protocols utilizing SSCOP. The minimum value of j is 0 octets.

### 7.2.5	STAT and USTAT PDU codingsXE "STAT and USTAT PDU codings"§

USTAT PDUs contain two list elements. STAT PDUs contain zero or more list elements. Transmitted STAT messages may be segmented into more than one STAT PDU.

The processing of a STAT PDU does not rely on information in other STAT PDUs. This is true even for the case when multiple STAT PDUs are generated in response to a single POLL PDU, and one or more of these STAT PDUs is lost.

The span list items in the STAT and USTAT PDUs are odd or even elements of a list used for selective retransmission requests. Every odd element represents the first PDU of a missing gap, and every even element represents the first PDU of a received sequence, except possibly the last one. Appendix II provides examples on how to code the span lists.

## 7.3	States of SSCOP protocol entityXE "States of SSCOP protocol entity"§

This subclause describes the states of an SSCOP entity. These states are used in the specification of the peer-to-peer protocol. The states are conceptual and reflect general conditions of the SSCOP entity in the sequences of signals and PDU exchanges with its user and peer, respectively. In addition, other conditions are used in the description, in order to avoid identification of additional states, as detailed in the SDLs. The basic states are:

–	State 1 – Idle

Each SSCOP entity is conceptually initiated in the Idle stateXE "Idle state"§ (State 1) and returns to this state upon the release of a connection.

–	State 2 – Outgoing Connection Pending

An SSCOP entity requesting a connection with its peer is in the Outgoing Connection Pending stateXE "Outgoing Connection Pending state"§ (State 2) until it receives acknowledgement from its peer.

–	State 3 – Incoming Connection Pending

An SSCOP entity that has received a connection request from its peer and is waiting for its user’s response is in the Incoming Connection Pending stateXE "Incoming Connection Pending state"§ (State 3).

–	State 4 – Outgoing Disconnection Pending

An SSCOP entity requesting release of the peer-to-peer connection goes to the Outgoing Disconnection Pending stateXE "Outgoing Disconnection Pending state"§ (State 4) until it receives confirmation that the peer entity has released and transitioned to the Idle state (State 1), after which it does the same.

–	State 5 – Outgoing Resynchronization Pending

An SSCOP entity requesting resynchronization of the connection with its peer is in the Outgoing Resynchronization Pending stateXE "Outgoing Resynchronization Pending state"§ (State 5).

–	State 6 – Incoming Resynchronization Pending

An SSCOP entity that has received a resynchronization request from its peer and is waiting for its user’s response is in the Incoming Resynchronization Pending stateXE "Incoming Resynchronization Pending state"§ (State 6).

–	State 7 – Outgoing Recovery Pending

An SSCOP entity requesting recovery with its peer of an existing connection is in the Outgoing Recovery Pending stateXE "Outgoing Recovery Pending state"§ (State 7).

–	State 8 – Recovery Response Pending

An SSCOP entity which has completed recovery, notified its user, and is awaiting response is in the Recovery Response Pending stateXE "Recovery Response Pending state"§ (State 8).

–	State 9 – Incoming Recovery Pending

An SSCOP entity that has received a recovery request from its peer and is waiting for its user’s response is in the Incoming Recovery Pending stateXE "Incoming Recovery Pending state"§ (State 9).

–	State 10 – Data Transfer Ready

Upon successful completion of the connection establishment, resynchronization, or error recovery procedures, both peer SSCOP entities will be in Data Transfer Ready stateXE "Data Transfer Ready state"§ (State 10) and assured data transfer can take place.

## 7.4	SSCOP state variablesXE "SSCOP state variables"§

This subclause describes the state variables used in the specification of the peer-to-peer protocol.

SD and POLL PDUs are sequentially and independently numbered and may have the value 0 through n minus 1 (where n is the modulus of the sequence numbers). The modulus equals 224 and the sequence numbers cycle through the entire range, 0 through 224 – 1. All arithmetic operations on the following state variables and sequence numbers contained in this Recommendation are affected by the modulus: VT(S), VT(PS), VT(A), VT(PA), VT(MS), VR(R), VR(H), and VR(MR). When performing arithmetic comparisons of transmitter variables, VT(A) is assumed to be the base. When performing arithmetic comparisons of receiver variables, VR(R) is assumed to be the base. In addition, the state variables VT(SQ) and VR(SQ) use modulo 256 arithmetic. The SSCOP maintains the following state variables at the transmitter.

a)	VT(S) – Send state variable

The sequence number of the next SD PDU to be transmitted for the first time (i.e. excluding retransmissions). Incremented after transmission of a SD PDU for the first time (i.e. excluding retransmissions).

b)	VT(PS) – Poll Send state variable

The current value of the poll sequence number. Incremented before transmission of the next POLL PDU.

c)	VT(A) – Acknowledge state variable

The sequence number of the next in-sequence SD PDU expected to be acknowledged, which forms the lower edge of the window of acceptable acknowledgements. VT(A) is updated upon acknowledgement of in-sequence SD PDUs.

d)	VT(PA) – Poll acknowledge state variable

The poll sequence number of the next STAT PDU expected to be received, which forms the lower edge of the acceptable N(PS) window for STAT PDUs. If a STAT PDU is received containing an invalid N(PS), a recovery is initiated or release is performed. If a STAT PDU is accepted, VT(PA) is set to STAT.N(PS).

e)	VT(MS) – Maximum Send state variable

The sequence number of the first SD PDU not allowed by the peer receiver [i.e. the receiver will allow up to VT(MS) – 1]. This value represents the upper edge of the transmit window. The transmitter shall not transmit a new SD PDU if VT(S) ³ VT(MS). VT(MS) is updated based on receipt of a USTAT PDU, STAT PDU, BGN PDU, BGAK PDU, RS PDU, RSAK PDU, ER PDU, or ERAK PDU.

f)	VT(PD) – Poll Data state variable

When acknowledgements are outstanding, this state variable represents the number of SD PDUs transmitted between transmission of a POLL PDU, or the number of SD PDUs transmitted before the transmission of the first POLL PDU after Timer_POLL became active. VT(PD) is incremented upon transmission of an SD PDU, and reset to zero upon transmission of a POLL PDU.

g)	VT(CC) – Connection Control state variable

The number of unacknowledged BGN, END, ER, or RS PDUs. VT(CC) is incremented upon transmission of a BGN, END, ER, or RS PDU. If an END PDU is transmitted in response to a protocol error, SSCOP does not wait for an ENDAK PDU [i.e. SSCOP moves directly to state 1 (Idle)] and VT(CC) is not incremented.

h)	VT(SQ) – Transmitter Connection Sequence state variable

This state variable is used to allow the receiver to identify retransmitted BGN, ER, and RS PDUs. This state variable is initialized to 0 upon creation of the SSCOP process and incremented and then mapped into the N(SQ) field before the initial transmission of either a BGN, RS, or ER PDU.

The SSCOP maintains the following state variables at the receiver:

a)	VR(R) – Receive state variable

The sequence number of the next in-sequence SD PDU expected to be received. Incremented upon receipt of the next in-sequence SD PDU.

b)	VR(H) – Highest expected state variable

The sequence number of the next highest expected SD PDU. Updated in two ways:

1)	reception of a new SD PDU; and

2)	reception of a POLL PDU.

c)	VR(MR) – Maximum acceptable Receive state variable

The sequence number of the first SD PDU not allowed by the receiver [i.e. the receiver will allow up to VR(MR) – 1]. The receiver shall discard SD PDUs with N(S) ³ VR(MR), (in one case, such an SD PDU may cause the transmission of a USTAT). Updating VR(MR) is implementation dependent, but VR(MR) should not be set to a value < VR(H). An example of how VR(MR) may be determined is included in Appendix IV.

d)	VR(SQ) – Receiver Connection Sequence state variable

This state variable is used to identify retransmitted BGN, ER, and RS PDUs. Upon reception of a BGN, ER, or RS PDU, this state variable is compared to the value of N(SQ) and then assigned the value of N(SQ). If the values are different, the PDU is processed and VR(SQ) is set to N(SQ). If they are equal, the PDU is identified as a retransmission.

## 7.5	SSCOP PDU parametersXE "SSCOP PDU parameters"§

a)	N(S)

VT(S) is mapped to N(S) whenever a new SD or POLL PDU is generated.

b)	Information field

The information field of an SD, MD, or UD PDU is mapped from the “Message unit” parameter of an AA-DATA, MAA-UNITDATA, or AA-UNITDATA request, respectively. It is mapped to a “Message unit” parameter of an AA-DATA, MAA-UNITDATA, or AA-UNITDATA indication, respectively.

c)	N(PS)

VT(PS) (after VT(PS) has been incremented) is mapped to N(PS) whenever a POLL PDU is generated. The receiver of a POLL PDU maps the received POLL.N(PS) into the field STAT.N(PS). In addition, to facilitate error recovery procedures, the current value of VT(PS) is mapped into N(PS) and stored in the transmitter buffer with the corresponding SD PDU whenever a SD PDU is sent.

d)	N(R)

VR(R) is mapped to N(R) whenever a STAT or USTAT PDU is generated.

e)	N(MR)

VR(MR) is mapped to N(MR) whenever a STAT, USTAT, RS, RSAK, ER, ERAK, BGN, or BGAK PDU is generated. This is the basis for credit granting by the receiver.

f)	SSCOP-UU

The SSCOP-UU in a BGN, BGAK, BGREJ, END or RS PDU is mapped to and from the “SSCOP-UU” parameter of the corresponding SSCOP signal.

g)	Source (S) bit

In an END PDU this bit conveys whether the originator of the release was the SSCOP or the SSCOP user. When the transmission of an END PDU is stimulated by the user, this bit is set to 0. When the transmission of an END PDU is stimulated by the SSCOP, this bit is set to 1. This bit is mapped into the “Source” field of an AA-RELEASE.indication.

h)	N(SQ)

This field carries the connection sequence value. VT(SQ) is mapped to N(SQ) whenever a new BGN, RS, or ER PDU is transmitted. This field is used by the receiver together with VR(SQ) to identify retransmitted BGN, RS, and ER PDUs.

i)	PDU Type field

The type field codings are listed in Table 2.

## 7.6	SSCOP timersXE "SSCOP timers"§

With the timers, an SSCOP connection is partitioned into phases.

a)	The active phase

In this phase, a Timer_POLLXE "Timer_POLL"§ is running to assure that the peer receiver is polled often enough (via POLL PDUs) to return its status (STAT PDU); this in turn is needed for advancing the credit window and efficient recovery from transmission errors.

The SSCOP transmitter is always in the active phase if there are SD PDUs to be transmitted or if there are any acknowledgements outstanding.

The POLL and STAT PDUs are subject to transmission errors. SSCOP provides for an uninterrupted flow of information by not insisting on a reply to every POLL PDU. In order to detect a broken connection, a Timer_NO RESPONSEXE "Timer_NO RESPONSE"§ is running parallel to Timer_POLL; this timer indicates the maximum time interval during which at least one STAT PDU needs to be received. Failing this, the SSCOP connection is aborted.

Timer_POLL is optimized to maintain the flow of information and may be shorter or longer than the round-trip delay. Timer_NO-RESPONSE must be at least the sum of Timer_KEEP-ALIVEXE "Timer_KEEP-ALIVE"§ and one round-trip delay.

b)	The transient phase

When Timer_POLL expires (and the POLL PDU has been sent) and there are no outstanding acknowledgements or new data pending receipt of credit, the transient phase is entered. Instead of Timer_POLL the Timer_KEEP-ALIVE is started. Also in this phase, POLL or STAT PDUs may be lost. Such loss is protected by Timer_NO RESPONSE determining the maximum time interval during which at least one STAT PDU needs to be received.

The transient phase reverts back to the active phase whenever a new SD PDU is transmitted or if at expiry of Timer_KEEP-ALIVE, new data is pending receipt of credit.

The Timer_KEEP-ALIVE is generally greater than Timer_POLL and greater than a round-trip delay; POLL PDUs are transmitted less often.

c)	The idle phase

When upon receipt of a STAT PDU the Timer_KEEP-ALIVE is still running, both Timer_KEEP-ALIVE and Timer_NO RESPONSE are stopped and Timer_IDLE is started. In this phase, no POLL PDUs are sent. At the expiry of Timer_IDLE, the transient phase is entered again.

NOTES

1	Timer_KEEP-ALIVE only expires when a POLL or STAT PDU is lost due to transmission errors.

2	The absolute maximum tolerated interval between reception of STAT PDUs is the sum of Timer_IDLEXE "Timer_IDLE"§ and Timer_NO RESPONSE.

The idle phase reverts back to the active phase whenever a new SD PDU is transmitted or whenever new data is pending receipt of credit.

Timer_IDLE may be considerably greater than Timer_KEEP-ALIVE.

d)	Connection control phase

During establishment and release of a connection and during resynchronization or recovery, transmission of PDUs is protected by the Timer_CC. It determines the time between transmission of BGN, END, RS, and ER PDUs as long as an acknowledgement to these PDUs has not been received.

At entry into the Data Transfer state, the active phase is entered.

Timer_CC should be somewhat greater than a round-trip delay.

The values of the SSCOP protocol timers are application specific and may be defined in the appropriate SSCF Recommendation which references this Recommendation. The tolerance of protocol timers is not addressed in this Recommendation. These timers should be configurable for different operational environments (e.g. signalling vs. data transfer environments, or environments including satellite links).

## 7.7	SSCOP parametersXE "SSCOP parameters"§

The value of each SSCOP protocol parameter is application specific and may be defined in the appropriate SSCF Recommendation which references this Recommendation.

a)	MaxCCXE "MaxCC"§

Maximum value for the state variable VT(CC), corresponding to the maximum number of transmissions of a BGN, END, ER, or RS PDU.

b)	MaxPDXE "MaxPD"§

Maximum acceptable value for the state variable VT(PD) before sending a POLL PDU and resetting VT(PD) to zero. This parameter is an upper limit for counter VT(PD) that sends a POLL PDU after every (MaxPD) SD PDUs.

c)	MaxSTATXE "MaxSTAT"§

Maximum number of list elements placed in a STAT PDU. When the number of list items exceeds MaxSTAT, the STAT message shall be segmented. All of the PDUs carrying the segmented STAT message, except possibly the last one, contain MaxSTAT list items. This parameter is not used by the receiver of a STAT PDU for length checking, but is only used by the sender of the STAT message for segmentation purposes. This parameter should be an odd integer greater than or equal to 3.

The default value of MaxSTAT is 67. This parameter can be changed on an implementation basis.

NOTE – The default value causes the STAT PDU to fill 6 ATM cells using AAL type 5 common part. In addition, the total length of a STAT PDU should not exceed the maximum length of an SD PDU.

d)	Clear-buffersXE "Clear-buffers"§

This parameter is set upon connection establishment. It holds one of two values: “Yes” or “No”. If this parameter is set to “Yes”, SSCOP can release its transmission buffer and transmission queue upon connection release. If this parameter is set to “No”, SSCOP can not release its transmission buffer and transmission queue  upon connection release. Additionally, if this parameter is set to No, SSCOP cannot release selectively acknowledged messages from its transmission buffer if older ones are still outstanding.

e)	CreditXE "Credit"§

This parameter is used to coordinate credit notifications to layer management. When SSCOP is blocked from transmitting a new SD PDU due to insufficient credit, “Credit” is assigned the value “No”. When SSCOP is permitted to transmit a new SD PDU, “Credit” is assigned the value of “Yes”. Credit is initially assigned “Yes”.

## 7.8	SSCOP credit and flow controlXE "SSCOP credit and flow control"§

### 7.8.1	Credit and peer-to-peer flow controlXE "Credit and peer-to-peer flow control"§

Credit is granted by the SSCOP receiver to allow the peer SSCOP transmitter to transmit new SD PDUs. The process by which a receiver entity determines credit is not subject to standardization, but is related to the buffer availability and the bandwidth/delay of the connection. The credit value is conveyed to the transmitter in the N(MR) field of each BGN, BGAK, RS, RSAK, ER, ERAK, STAT and USTAT PDU sent by the receiver. N(MR) is mapped to the variable VT(MS) at the transmitter. The credit value sent to the transmitter is the sequence number of the first SD PDU that the receiver will not accept. The transmitter does not transmit any SD PDUs that exceed the credit allowed. The receiver discards any SD PDUs that exceed the credit allowed, (in one case, such an SD PDU may cause the transmission of a USTAT PDU).

Previously granted credit can be reduced in order for the receiver to perform flow control, but the receiver credit variable VR(MR) cannot be reduced below the value VR(H). In other words, if a receiver has accepted and acknowledged the receipt of the SD PDU numbered VR(H) – 1, the credit value VR(MR) must be greater than or equal to VR(H).

The operating window of the protocols at the transmitter is lower bounded by VT(A) and upper bounded by the credit available [VT(MS) – 1]. The modulus of the protocol limits the operating window to 224 – 1. Therefore, at the receiver, the credit granted, using modulo arithmetic, must be a value between VR(H) and VR(R) – 1. If VR(MR) = VR(R) = VR(H), the operating window is zero. If VR(MR) = VR(R) – 1, the operating window is maximum.

The SSCOP receiver allocates a buffer to support each connection. In principle, the receiver buffer available should match or exceed the credit granted to the transmitter, to avoid the discard of successfully transmitted data. However, if limited buffers are available for a connection, it is possible to grant credit in excess of the available buffer capacity. This method may obtain a higher throughput than can be achieved by limiting the credit to the available buffer, with the possibility that data may need to be discarded if errors occur. The receiver cannot discard previously received and acknowledged, but not yet delivered, SD PDUs. The receiver must also allocate sufficient buffer capacity to receive and deliver the SD PDU numbered VR(R), at all times unless VR(R) = VR(H) = VR(MR). The granting of credit in excess of buffer capacity should only be performed if limited buffers to support the connection are available and if the SSCOP receiver can still maintain the quality of service (QoS) required for the connection through this method.

### 7.8.2	Local flow control

SSCOP events, such as reception of PDUs and external and internal signals, are normally processed in the order in which they occurred. However, events pertaining to the exchange of SSCOP connection status information have priority over data transfer.

An implementation may detect congestion (for example, a long queuing delay) in its lower protocol layers. If so, data transfer should be temporarily suspended in order to give priority to connection control messages. The means by which an SSCOP entity decides whether or not it is congested depends on the protocol environment, including protocol timer values, and is not subject to standardization.

If an SSCOP entity detects local congestion (“lower layer busy” in the SDL specification), it can elect to suspend the servicing of AA-DATA.request signals, AA-UNITDATA.request signals, and MAA-UNITDATA.request signals. It can also suspend the retransmission of requested SD PDUs. The data transfer procedures allow this to occur without causing protocol errors.

Therefore, in terms of transmitting PDUs to the peer receiver, all types of PDUs except SD PDU, MD PDU, and UD PDU are given highest priority. The SD PDUs, MD PDUs, and UD PDUs have equal priority. Among the SD PDUs, retransmissions have priority over new transmission if both types are pending. These priorities are only internal to SSCOP.

The SSCOP’s local flow control at its user’s interface is implementation dependent.

# 8	Specification of SSCOPXE "Specification of SSCOP"§

This clause provides a set of SDL diagrams defining the procedures of the Service Specific Connection Oriented Protocol. These SDL diagrams are the definitive description of the procedures and in case of conflict with the text, the SDL diagrams take precedence.

## 8.1	Overview

Figure 17 gives an overview over the states of SSCOP and the major transitions between them. The states allow SSCOP a number of connection control services and their relation.

### 8.1.1	Idle

In this state (State 1: Idle), no connection is established. Only unassured and management data may be communicated.

### 8.1.2	Establishment and release

The states in this connection control service assist SSCF in establishing and releasing connections for the assured data transfer service. Establishment and release takes precedence over all of the other connection control services (resynchronization and recovery). The following states are defined:

–	State 2 – Outgoing Connection Pending

In this state, the local SSCF instructed SSCOP to establish a new connection with its peer and awaits the peer’s response.

–	State 3 – Incoming Connection Pending

In this state, SSCOP received the indication that its peer wants to establish a new connection and has notified the SSCF. It is awaiting the response of the SSCF.

–	State 4 – Outgoing Disconnection Pending

In this state, the SSCF has instructed the SSCOP to, or the SSCOP itself has initiated, release the current connection. It awaits confirmation from its peer.

### 8.1.3	Bidirectional resynchronization

The states in this connection control service assist the SSCOP in a resynchronization of both data transfer directions. The bidirectional resynchronization takes precedence over the recovery service. The following states are defined:

–	State 5 – Outgoing Resynchronization Pending

In this state, the local SSCF initiated a resynchronization. SSCOP’s peer has been informed and its response is awaited.

–	State 6 – Incoming Resynchronization Pending

In this state, the peer SSCOP has requested a resynchronization. The SSCF has been notified and its response is awaited.

Figure  17/Q.2110.....[D17]  =  page pleine

### 8.1.4	Recovery

The states in this connection control service assist SSCOP in recovering from protocol errors relating to the assured data transfer (sequence number problems). The following states are defined:

–	State 7 – Outgoing Recovery Pending

In this state, the SSCOP has detected a sequence number problem and has instructed its peer to recover. The confirmation from its peer is awaited.

–	State 8 – Recovery Response Pending

In this state, the SSCOP that detected the sequence number problem received the confirmation from its peer, informed its SSCF and is awaiting the response from its SSCF.

–	State 9 – Incoming Recovery Pending

In this state, the peer SSCOP detected a sequence number problem and informed this SSCOP who in turn informed its SSCF. The response of the SSCF is awaited.

### 8.1.5	Data transfer

This state permits assured data transfer. Connection establishment, release, resynchronization, and recovery procedures will take the state machine out of this state.

–	State 10 – Data Transfer Ready

In this state, assured data transfer takes place.

## 8.2	SDL diagrams

The SDL diagrams are represented in Figures 18 to 20. An equivalent software version of the SDLs has been produced using the SDT tool for SDL. This version may be purchased for verification purposes and is available in electronic form only from the ITU. If the alternate version conflicts with this Recommendation, this Recommendation takes precedence.

Figure  18/Q.2110.....[D18]  =  page pleine

Figure  19/Q.2110.....[D19]  =  page pleine

Figure  20/Q.2110.....[D20]  =  page pleine

Figure  20/Q.2110.....[D21]  =  page pleine

Figure  20/Q.2110.....[D22]  =  page pleine

Figure  20/Q.2110.....[D23]  =  page pleine

Figure  20/Q.2110.....[D24]  =  page pleine

Figure  20/Q.2110.....[D25]  =  page pleine

Figure  20/Q.2110.....[D26]  =  page pleine

Figure  20/Q.2110.....[D27]  =  page pleine

Figure  20/Q.2110.....[D28]  =  page pleine

Figure  20/Q.2110.....[D29]  =  page pleine

Figure  20/Q.2110.....[D30]  =  page pleine

Figure  20/Q.2110.....[D31]  =  page pleine

Figure  20/Q.2110.....[D32]  =  page pleine

Figure  20/Q.2110.....[D33]  =  page pleine

FIGURE 20/Q.2110.(feuillet 15 de 51)    [D34] = PAGE À L'ITALIENNE

Recommendation Q.2110     (07/94)

FIGURE 20/Q.2110.(feuillet 16 de 51)...[D35] = PAGE PLEINE

FIGURE 20/Q.2110.(feuillet 17 de 51)...[D36] = (17 OF 51) PAGE À L'ITALIENNE

FIGURE 20/Q.2110.(feuillet 18 de 51)...[D37] = PAGE PLEINE

FIGURE 20/Q.2110.(feuillet 19 de 51)...[D38] = PAGE PLEINE

FIGURE 20/Q.2110...[D39] = (20 OF 51) PAGE PLEINE

FIGURE 20/Q.2110.(feuillet 21 de 51)...[D40] = PAGE À L'ITALIENNE

FIGURE 20/Q.2110.(feuillet 22 de 51)...[D41] = PAGE À L'ITALIENNE

FIGURE 20/Q.2110.(feuillet 23 de 51)...[D42] = PAGE À L'ITALIENNE

FIGURE 20/Q.2110.(feuillet 24 de 51)...[D43] = PAGE À L'ITALIENNE

FIGURE 20/Q.2110.(feuillet 25 de 51)...[D44] = PAGE PLEINE

FIGURE 20/Q.2110.(feuillet 26 de 51)...[D45] = PAGE PLEINE

FIGURE 20/Q.2110.(feuillet 27 de 51)...[D46] = PAGE À L'ITALIENNE

FIGURE 20/Q.2110.(feuillet 28 de 51)...[D47] = PAGE À L'ITALIENNE

FIGURE 20/Q.2110.(feuillet 29 de 51)...[D48] = PAGE À L'ITALIENNE

FIGURE 20/Q.2110.(feuillet 30 de 51)...[D49] = PAGE PLEINE

FIGURE 20/Q.2110.(feuillet 31 de 51)...[D50] = PAGE À L'ITALIENNE

FIGURE 20/Q.2110.(feuillet 33 de 51)...[D51] = PAGE PLEINE

FIGURE 20/Q.2110.(feuillet 33 de 51)...[D52] = PAGE PLEINE

FIGURE 20/Q.2110.(feuillet 34 de 51)...[D53] = PAGE PLEINE

FIGURE 20/Q.2110.(feuillet 35 de 51)...[D54] = PAGE PLEINE

FIGURE 20/Q.2110.(feuillet 36 de 51)...[D55] = PAGE PLEINE

FIGURE 20/Q.2110.(feuillet 37 de 51)...[D56] = PAGE PLEINE

FIGURE 20/Q.2110.(feuillet 38 de 51)...[D57] = PAGE PLEINE

FIGURE 20/Q.2110.(feuillet 39 de 51)...[D58] = PAGE PLEINE

FIGURE 20/Q.2110.(feuillet 40 de 51)...[D59] = PAGE PLEINE

FIGURE 20/Q.2110.(feuillet 41 de 51)...[D60] = PAGE PLEINE

FIGURE 20/Q.2110.(feuillet 42 de 51)...[D61] = PAGE PLEINE

FIGURE 20/Q.2110.(feuillet 43 de 51)...[D62] = PAGE PLEINE

FIGURE 20/Q.2110.(feuillet 44 de 51)...[D63] = PAGE PLEINE

FIGURE 20/Q.2110.(feuillet 45 de 51)...[D64] = PAGE PLEINE

FIGURE 20/Q.2110.(feuillet 46 de 51)...[D65] = PAGE PLEINE

FIGURE 20/Q.2110.(feuillet 47 de 51)...[D66] = PAGE À L'ITALIENNE

FIGURE 20/Q.2110.(feuillet 48 de 51)...[D67] = PAGE PLEINE

FIGURE 20/Q.2110.(feuillet 49 de 51)...[D68] = PAGE À L'ITALIENNE

FIGURE 20/Q.2110.(feuillet 50 de 51)...[D69] = PAGE À L'ITALIENNE

FIGURE 20/Q.2110.(feuillet 51 de 51)...[D70] = PAGE PLEINE

Annex  A

Management error indications

(This annex forms an integral part of this Recommendation)

A number of events will cause errors to be indicated to the layer management entity. The associated error parameter contains the error code that describes the specific error conditions.

The column entitled “Error condition” together with the “Affected states” describes specific protocol error events and the basic state of the SSCOP entity at the point that the MAA-ERROR indication primitive is generated (see Table A.1).

TABLE  A.1/Q.2110

| Error Type | Error Code | Error Condition | Affected States |
| --- | --- | --- | --- |
|  | A | SD PDU | 1, 3, 6, 9 |
|  | B | BGN PDU | 6, 7, 8, 9 |
|  | C | BGAK PDU | 1, 3, 6, 7, 8, 9 |
|  | D | BGREJ PDU | 1, 3, 5, 6, 7, 8, 9, 10 |
|  | E | END PDU | None |
| Receipt of unsolicited or | F | ENDAK PDU | 3, 5, 6, 7, 8, 9, 10 |
| inappropriate PDU | G | POLL PDU | 1, 3, 6, 9 |
|  | H | STAT PDU | 1, 3, 6, 8, 9 |
|  | I | USTAT PDU | 1, 3, 6, 8, 9 |
|  | J | RS | 1, 3, 6, 7, 8, 9 |
|  | K | RSAK PDU | 1, 3, 6, 7, 8, 9 |
|  | L | ER | 1, 3, 6, 7, 8, 9 |
|  | M | ERAK | 1, 3, 6, 9 |
| Unsuccessful | O | VT(CC) >= MaxCC | 2, 4, 5, 7 |
| retransmission | P | Timer_NO_RESPONSE expiry | 10 |
|  | Q | SD or POLL, N(S) error | 10 |
|  | R | STAT N(PS) error | 10 |
|  | S | STAT N(R) or list elements error | 10 |
| Other list elements error type | T | USTAT N(R) or list elements error | 10 |
|  | U | PDU length violation | ALL |
| SD loss | V | SD PDUs must be retransmitted | 10 |
| Credit Condition | W | Lack of credit | 10 |
|  | X | Credit obtained | 10 |

Annex  B

Protocol implementation conformance statement (PICS)
proforma to Recommendation Q.2110XE "Protocol implementation conformance statement (PICS)
proforma to Recommendation Q.2110"§1)

(This annex forms an integral part of this Recommendation)

## B.1	General

The supplier of a protocol implementation claiming to conform to this Recommendation, shall complete the following Protocol Implementation Conformance Statement (PICS) proforma and accompany it by the information necessary to identify fully both the supplier and the implementation. This PICS proforma applies to the B-ISDN interfaces.

The PICS is a document specifying the capabilities and options which have been implemented, and any features which have been omitted, so that the implementation can be tested for conformance against relevant requirements, and against those requirements only.

This PICS has several uses, the most important are the static conformance review and test case selection in order to identify which conformance tests are applicable to this product.

The PICS proforma is a document, in the form of a questionnaire, normally designed by the protocol specifier or conformance test suite specifier which, when completed for an implementation or system, becomes the PICS.

Subclause B.5 covers the SSCOP Q.2110 Protocol Capabilities, Protocol Data Units, and System Parameters.

NOTE – The SSCOP can be combined with different Service Specific Coordination Functions (SSCFs) to offer different AAL services. As a result, the SSCOP specification defines mandatory functions for a general protocol. Some of these functions may not be needed by a particular service and may be masked from the AAL user by an SSCF. It is possible for an implementation not to implement a certain SSCOP function and still meet the mandatory requirements of certain AAL services (e.g. the SSCOP local data retrieval function is not needed to support B-ISDN UNI signalling). Implementors can refer to the PICS proforma for a particular SSCF, if they are only concerned about providing that service. However, the absence of a mandatory SSCOP function may preclude the possibility of combining the SSCOP implementation with other SSCFs to offer different AAL services.

## B.2	Abbreviations and special symbols

CPE	Customer Premises Equipment

IUT	Implementation Under Test

M	Mandatory

N/A	Not Applicable

O	Optional

O.<n>	Optional, but, if chosen, support is required for either at least one or only one of the options in the group labelled by the same numeral <n>

P	Prohibited

PC	Prefix for the Index number of the Protocol Capabilities Group

PD	Prefix for the Index number of the Protocol Data Units Group

PICS	Protocol Implementation Conformance Statement

PIXIT	Protocol Implementation Extra Information for Testing

S.<i>	Supplementary Information number i

SP	Prefix for the Index number of the System Parameter Group

X.<i>	Exceptional Information number i

## B.3	Instructions for completing the PICS proforma

The main part of the PICS proforma is a fixed-format questionnaire, divided into three sections. Answers to the questionnaire are to be provided in the right most column, either by simply marking an answer to indicate a restricted choice (such as Yes or No), or by entering a value or a set or range of values.

A supplier may also provide additional information, categorized as either Exceptional Information or Supplementary Information (other than PIXIT). When present, each kind of additional information is to be provided as items labelled X.<i> or S.<i> respectively for cross-reference purposes, where <i> is any unambiguous identification for the item. An exception item should contain the appropriate rationale. The Supplementary Information is not mandatory and the PICS is complete without such information. The presence of optional supplementary or exceptional information should not affect test execution, and will in no way affect static conformance verification.

NOTE – Where an implementation is capable of being configured in more than one way, a single PICS may be able to describe all such configurations. However, the supplier has the choice of providing more than one PICS, each covering some subset of the implementation’s configuration capabilities, in case this makes for easier or clearer presentation of the information.

## B.4	Global statement of conformance

Global statement: The implementation specified in this PICS meets all the mandatory requirements of the referenced standards:

Yes/No

NOTE – Answering “No” to this question indicates non-conformance to this Recommendation. Non-supported mandatory capabilities are to be listed in the PICS below, with an explanation for the abnormal status of the implementation.

The supplier will have fully complied with the requirements for a statement of conformance by completing the statement contained in this subclause. However, the supplier may find it helpful to continue to complete the detailed tabulations in the subclauses which follow.

## B.5	SSCOP – Q.2110

### B.5.1	Protocol Capabilities (PC) – SSCOPXE "Protocol Capabilities (PC) – SSCOP"§

See Table B.1.

TABLE  B.1/Q.2110

| ITEM # | Protocol Feature | Status | Reference | Support |
| --- | --- | --- | --- | --- |
| PC1 | Does IUT support Keep Alive function? | M | 5.0 e) | Yes:__No:__X:__ |
| PC2 | Does IUT support the Local Data Retrieve function? | M | 5.0 f) | Yes:__No:__X:__ |
| PC3 | Does the IUT support SSCOP initiated error recovery due to protocol error? | M | 5.0 i) | Yes:__No:__X:__ |
| PC4 | Does the IUT recognize all of the Messages regardless of state? | M | Table 2 | Yes:__No:__X:__ |
| PC5.1 | In the absence of protocol error, does the IUT support assured data transfer with sequence integrity? | M | 5.0 a) h); 7.1 j) | Yes:__No:__X:__ |
| PC5.2 | Does IUT support the sending of the Unassured Data PDU? | M | 5.0 h); 7.1 n) | Yes:__No:__X:__ |
| PC5.3 | Does IUT support the sending of the Management Data PDU? | M | 7.1 o) | Yes:__No:__X:__ |
| PC6 | Does IUT support user invoked re-synchronization procedures? | M | 5.0 g) | Yes:__No:__X:__ |
| PC7 | Does IUT support the establishment procedures for an SSCOP connection? | M | 5.0 g) | Yes:__No:__X:__ |
| PC8 | Does IUT support release procedures for an SSCOP connection? | M | 5.0 g) | Yes:__No:__X:__ |
| PC9 | Does IUT support polling after retransmission? | O | SDL | Yes:__No:__X:__ |
| PC10 | Does IUT support the segmenting of STAT PDUs? | M | 7.2.5 | Yes:__No:__X:__ |
| PC11 | Can the IUT initiate SSCOP connection? | M | 5.0 g) | Yes:__No:__X:__ |
| PC12 | Can the IUT reject (BGREJ) the establishment of an SSCOP connection from its peer? | M | SDL | Yes:__No:__X:__ |
| PC13 | Does IUT support error reporting to layer management? | M | 5.0 d) | Yes:__No:__X:__ |
| PC14 | Does IUT support the Protocol error detection function? | M | 5.0 i) | Yes:__No:__X:__ |
| PC15 | When no SSCOP connection exists, is a connection established only upon receipt of a BGN or a request from the SSCOP user? | M | SDL | Yes:__No:__X:__ |
| PC16 | Does SSCOP permit the conveyance of SSCOP User-to-User Information between users of the SSCOP? | M | 5.0 g); 6.1.2 b) | Yes:__No:__X:__ |

### B.5.2	SSCOP PDUs – Protocol Data UnitsXE "SSCOP PDUs – Protocol Data Units"§ (PD)

See Table B.2.

TABLE  B.2/Q.2110

| ITEM # | Protocol Feature | Status | Reference | Support |
| --- | --- | --- | --- | --- |
| Order of Octet Transmission |  |  |  |  |
| PD1 | Ascending numerical order | M | 7.2.1 | Yes:__No:__X:__ |
| Field Mapping Convention |  |  |  |  |
| PD2 | Lowest bit number = Lowest order value | M | 7.2.1 | Yes:__No:__X:__ |
| PD3 | Are PDU formats 32 bit aligned? | M | 7.2 | Yes:__No:__X:__ |
| PD4 | Are all reserved bits coded as zeros? | M | 7.2.3 | Yes:__No:__X:__ |

### B.5.3	SSCOP System ParametersXE "SSCOP System Parameters"§ (SP)

See Table B.3.

TABLE  B.3/Q.2110

| ITEM # | Protocol Feature | Status | Reference | Support |
| --- | --- | --- | --- | --- |
| SP1 | Maximum number of transmissions of a BGN, END, ER, or RS PDU (MaxCC) | M | 7.7 a) | Yes:__No:__X:__Value:_ |
| SP2 | Maximum number of SD PDUs before transmission of a POLL PDU (MaxPD) | M | 7.7 b) | Yes:__No:__X:__Value:_ |
| SP3 | Maximum number of List Elements in a STAT (MaxSTAT)? | M | 7.7 c) | Yes:__No:__X:__Value:_ |
| SP4 | Maximum PDU size | M | 7.2.4 | Yes:__No:__X:__Value:_ |
| SP5 | Timer_POLL | M | 7.6 a) | Yes:__No:__X:__Value:_ |
| SP6 | Timer_KEEP-ALIVE | M | 7.6 b) | Yes:__No:__X:__Value:_ |
| SP7 | Timer_NO-RESPONSE | M | 7.6 c) | Yes:__No:__X:__Value:_ |
| SP8 | Timer_IDLE | M | 7.6 c) | Yes:__No:__X:__Value:_ |
| SP9 | Timer_CC | M | 7.6 d) | Yes:__No:__X:__Value:_ |
| SP10 | If PC16 is supported, what is the maximum size of the SSCOP-UU? | M | 6.1.2 b); 7.2.4 | Yes:__No:__X:__Value:_ |

Appendix  I

Concepts and terminology

(This appendix does not form an integral part of this Recommendation)

The basic structuring technique in the OSI reference model is layering. According to this technique, communication among application processes is viewed as being logically partitioned into an ordered set of layers represented in a vertical sequence as shown in Figure I.1.

FIGURE I.1/Q.2110...[D71] = 9.5 CM

An SAAL Service Access Point (SAP) is the point at which the SAAL provides services to layer 3. Associated with each SAAL SAP is one or more SAAL connection endpoints. See Figure I.2.

Entities exist in each layer. Entities in the same layer, but in different systems which must exchange information to achieve a common objective are called “peer entities”. Entities in adjacent layers interact through their common boundary. The services provided by the SAAL are the combination of the services and functions provided by the SAAL SSCS, SAAL common part, and the ATM layer.

Cooperation between SAAL entities is governed by a peer-to-peer protocol specific to the layer.

SAAL signaling data units (SDUs) are conveyed between SAAL entities by means of a physical connection.

Layer 3 requests service from the SAAL via service primitives. The same applies for the interaction between the SAAL and the ATM layer. The primitives represent, in an abstract way, the logical exchange of information and control between the SAAL and the adjacent layers and between the SAAL and the SAAL common part. They do not specify or constrain implementation.

FIGURE I.2/Q.2110...[D72] = 8 CM

The primitives that are exchanged between the SAAL and adjacent layers are of the following types (see Figure I.3):

a)	request;

b)	indication;

c)	response;

d)	confirm.

FIGURE I.3/Q.2110...[D73] = 9 CM

The request primitive typeXE "request primitive type"§ is used when a higher layer is requesting a service from the next lower layer.

The indication primitive typeXE "indication primitive type"§ is used by a layer providing a service to notify the next higher layer of any specific activity which is service related. The indication primitive may be the result of an activity of the lower layer related to the primitive type request at the peer entity.

The response primitive typeXE "response primitive type"§ is used by a layer to acknowledge receipt from a lower layer of the primitive type indication.

The confirm primitive typeXE "confirm primitive type"§ is used by the layer providing the requested service to confirm that the activity has been completed.

Information is transferred, in various types of SDUs, between peer entities and between entities in adjacent layers (and sublayers) that are attached to a specific SAP.

Appendix  II

Examples of SSCOP operationXE "Examples of SSCOP operation"§

(This appendix does not form an integral part of this Recommendation)

Table II.1 demonstrates the semantics of the list elements within a STAT and USTAT PDU.

TABLE  II.1/Q.2110

Examples of STAT and USTAT PDU semantics

| RECEIVED SD PDUs | Received POLL PDU | Responding PDU |
| --- | --- | --- |
| 1,x,x,4 | In case of USTAT | USTAT(N(R)=2,{2,4}) |
| 1,x,x,4 | POLL(N(S)=5) | STAT(N(R)=2,{2,4,5}) |
| 1,x,x,x | POLL(N(S)=5) | STAT(N(R)=2,{2,5}) |
| 1,x,x,4,5 | POLL(N(S)=6) | STAT(N(R)=2,{2,4,6}) |
| 1,x,x,4,5,x,x | POLL(N(S)=8) | STAT(N(R)=2,{2,4,6,8}) |
| 1,x,x,4,5,x,x,8,9 | POLL(N(S)=10) | STAT(N(R)=2,{2,4,6,8,10}) |
| NOTES 1 Elements in “{ }” are STAT list elements. 2 Only concerned fields are shown. 3 x represents PDUs lost during transmission. |  |  |

Figure II.1 through Figure II.3 show time flow diagrams of connection-establishment, data transfer, connection-release and resynchronization. All four time flow diagrams depict error-free operation, and are intended to provide a high-level appreciation of the protocol operation.

The following examples demonstrate the operation of the protocol under different scenarios. Each example illustrates a specific aspect of the protocol operation. The following conventions are used in the figures.

–	The numbers shown at the transmitter represent SD sequence numbers, N(S), and the associated poll sequence numbers, N(PS), enclosed in parentheses.

–	The numbers shown at the receiver represent the received SD PDU sequence numbers. An “X” in the delivered column indicates that an SD PDU has been delivered to the upper layer. An “X” in the Rx column represents a missing SD PDU.

–	POLL PDU is represented as: POLL(N(S), N(PS)).

–	STAT PDU is represented as: STAT(N(R), N(PS), N(MR), list elements).

–	USTAT PDU is represented as: USTAT(N(R), N(MR), list elements).

FIGURE II.1/Q.2110...[D74] = 6.5 CM

FIGURE II.2/Q.2110...[D75] = 9 CM

FIGURE II.3/Q.2110...[D76] = 8.5 CM

FIGURE II.4/Q.2110...[D77] = 7.5 CM

Figure II.5 shows the operation of the protocol in the error-free case. SD PDUs are received in-sequence and delivered to the upper layer. Each SD PDU is not acknowledged separately, but a group of SD PDUs is acknowledged via a STAT PDU in response to a POLL PDU.

FIGURE II.5/Q.2110...[D78] = 5.5 CM

Figure II.6 shows error recovery via the USTAT PDU. Upon receiving SD PDU 3 and concluding that SD PDU 2 is missing, the receiver sends a USTAT PDU requesting retransmission of 2. Upon receiving this USTAT PDU, the transmitter retransmits SD PDU 2. Note that a subsequent STAT PDU is received, unnecessary retransmission is avoided due to the N(PS) comparison. The N(PS) associated with SD PDU 2 is 1, and it is not less than the N(PS) in the received STAT PDU(1), hence, 2 is not retransmitted. Upon receiving 2, the receiver delivers it and all subsequent in-sequence saved SD PDUs.

FIGURE II.6/Q.2110...[D79] = 7 CM

Figure II.7 shows error recovery via STAT PDU. It demonstrates the case where an USTAT PDU is lost, however, recovery is still achieved via the STAT PDU.

FIGURE II.7/Q.2110...[D80] = 8 CM

Figure II.8 shows error recovery via STAT PDU of the last transmitted SD PDUs. It demonstrates the case where the last transmitted SD PDUs are all lost. In this case, an USTAT PDU can not be generated, since the receiver has no knowledge that those SD PDUs have been transmitted and lost. However, error recovery is still achieved via the STAT PDU in response to the POLL PDU. Note that when SD PDU 5 is later on received, it does not cause a generation of a USTAT PDU.

FIGURE II.8/Q.2110...[D81] = 8 CM

Figure II.9 shows error recovery via STAT and USTAT PDU of the last transmitted SD PDUs. The STAT is generated similar to the example in Figure II.8. However, subsequent SD PDUs not reported in this STAT and detected as missing when SD PDU 7 is received later on, are recovered via the USTAT.

FIGURE II.9/Q.2110...[D82] = 10 CM

Figure II.10 is similar to Figure II.9, but in this case the STAT PDU is lost. The example shows that the subsequent STAT PDU completes the error recovery.

FIGURE II.10/Q.2110...[D83] = 12.5 CM

Figure II.11 is similar to Figure II.8, however, in this case two span lists are included in the STAT PDU.

FIGURE II.11/Q.2110...[D84] = 8.5 CM

Figure II.12 is similar to Figure II.6, however, in this case two span lists are included in the STAT PDU.

FIGURE II.12/Q.2110...[D85] = 10 CM

Appendix  III

Summary of buffer and state variable management

(This appendix does not form an integral part of this Recommendation)

Table III.1 shows the status of the various buffers and the state variables at the time when a particular state is entered.

TABLE  III.1/Q.2110

Buffer and state variable management

|  | Idle | Outgoing Connection Pending | Incoming Connection Pending | Outgoing Disconnection Pending | Outgoing Resynchronization Pending | Incoming Resynchronization Pending | Outgoing Recovery Pending | Recovery Response Pending | Incoming Recovery Pending | Data Transfer Ready |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 |
| Clear Transmission Queue | C | U | C | C | U | C | C | C | C | A |
| Clear Transmission Buffer | C | U | C | C | U | C | C | C | C | U |
| Clear Retransmission Queue | U | U | U | U | U | U | U | U | U | (Note 1) |
| Clear Receiver Buffer | U | U | U | U | U | U |  | D | D | (Note 2) |
| Reset Receiver State Variables |  |  |  |  |  |  |  |  |  | R |
| Reset Transmitter State Variables |  |  |  |  |  |  |  |  |  | R |
| Data Retrieval Allowed | Y |  | Y | Y |  | Y |  | Y | Y |  |
| U Buffer/Queue is empty unconditionally upon entry into the state. C Buffer/Queue is empty conditionally, i.e. if “Clear-Buffer = NO”, upon entry into the state. A The buffer is cleared unconditionally upon entry into the state, unless when entering from state 8 or 9 and if “Clear-Buffer = NO” (the data is subsequently transmitted). D If “Clear-Buffer = NO”, the content of the buffer is delivered with possible sequence gaps; if “Clear-Buffer = YES”, the buffer is cleared upon entry into this state. R Data transfer state variables are reset upon entry into the state. Y Data retrieval allowed. NOTES 1 Data can be inserted into the Retransmission queue only in state 10, “Data Transfer Ready”. As this buffer is cleared unconditionally upon entering any other state, by default, it is empty upon entry into state 10. 2 Data can be inserted into the Receiver buffer only in state 10, “Data Transfer Ready”. As this buffer is cleared unconditionally before state 10 is entered from any other possible state, the Receiver buffer is empty unconditionally when entering state 10, by default, also. |  |  |  |  |  |  |  |  |  |  |

Appendix  IV

Default window size for SSCOPXE "Default window size for SSCOP"§

(This appendix does not form an integral part of this Recommendation)

This appendix may be used to set the window size parameter (conveyed in the SSCOP N(MR) field) of SSCOP. The following formula can be used to calculate a window size that is sufficient to keep the transmitter active. Alternatively, the window can be optimized to the particular connection or implementation. For example, the window size may be reduced to perform flow control or buffer management. During a connection, the window size can change dynamically based on local requirements.

k = 2 + (2 * Timer_POLL + 6 * Ttd) * Ru/(8 * Ld)

where

k	is the window size

Ttd	is the end-to-end transit delay (seconds)

Timer_POLL	is the POLL timer value for the peer entity (seconds)

Ru	is the SSCOP throughput (bits/s)

Ld	is the data frame size in octets

Information regarding the end-to-end transit delay, throughput, and frame size should be available at the SSCOP endpoints, or may be derived from signaling messages. The Timer_POLL used by the peer can be identified based on the received POLL PDU frequency; alternatively, the selected Timer_POLL value used at the local transmitter can be used.

The values of the Timer_POLL and the round trip delay have implications on the size of buffers needed to support the connection. If the window size implies that excessive buffers are needed for the connection, an implementation may consider shortening the Timer_POLL value at the transmitter, or decoupling the receiver buffer from the offered window at the receiver.

The window passed to the transmitter is conveyed by a sequence number in the N(MR) field of certain SSCOP PDUs. The difference between this sequence number (VR(MR)) and the next in-sequence to be received (VR(R)) is the window at the receiver.

1	) 	Copyright release for PICS proformas:

Users of this Recommendation may freely reproduce the PICS proforma in this Recommendation, so that it may be used for its intended purpose and may further publish the completed PICS.