|  | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

ITU-T	Q.2610

TELECOMMUNICATION	(02/95)
STANDARDIZATION  SECTOR
OF  ITU

COMMON  ASPECTS  OF  B-ISDN  APPLICATION
PROTOCOLS  FOR  ACCESS  SIGNALLING
AND  NETWORK  SIGNALLING
AND  INTERWORKING

BROADBAND  INTEGRATED  SERVICES
DIGITAL  NETWORK  (B-ISDN)  –
USAGE  OF  CAUSE  AND  LOCATION
IN  B-ISDN  USER  PART  AND  DSS  2

ITU-T  Recommendation  Q.2610

(Previously “CCITT  Recommendation”)

# FOREWORD

The ITU-T (Telecommunication Standardization Sector) is a permanent organ of the International Telecommunication Union (ITU). The ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Conference (WTSC), which meets every four years, establishes the topics for study by the ITU-T Study Groups which, in their turn, produce Recommendations on these topics.

The approval of Recommendations by the Members of the ITU-T is covered by the procedure laid down in WTSC Resolution No. 1 (Helsinki, March 1-12, 1993).

ITU-T Recommendation Q.2610 was prepared by ITU-T Study Group 11 (1993-1996) and was approved under the WTSC Resolution No. 1 procedure on the 7th of February 1995.

___________________

NOTE

In this Recommendation, the expression “Administration” is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

ã  ITU  1995

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

CONTENTS

Recommendation Q.2610     (02/95)

Page

1	Scope		1

2	References		1

3	Cause		1

3.1	Format		1

3.2	Cause value		3

3.3	Diagnostics		3

4	General rules for handling of the location field		4

5	Handling of cause and location at the international interface		4

# SUMMARY

This Recommendation describes the interworking between the DSS 2 access interface protocol and the Broadband ISDN User Part protocol. It is part of a set of interlocking B-ISDN service and signalling Recommendations that comprise Broadband signalling Capability Set 1. This Recommendation describes the mapping tables and diagrams which support interworking between the protocols for basic call set-up and clear down.

Recommendation Q.2610

Recommendation Q.2610     (02/95)

Broadband  Integrated  Services  Digital  Network  (B-ISDN)  –
Usage  of  Cause  and  Location  in  B-ISDN
User  Part  and  DSS  2

(Geneva, 1995)

# 1	Scope

This Recommendation defines the format, encoding and semantics of the Cause information element/Cause indicator parameter and the usage of the location field, in the broadband UNI and NNI signalling systems.

# 2	References

The following ITU-T Recommendations contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication the editions indicated were valid. As all Recommendations and other references are subject to revision, users of this Recommendation are encouraged to investigate the possibility of applying the most recent editions of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

[1]	ITU-T Recommendation Q.850 (1993), Use of cause and location in the digital subscriber Signalling System No. 1 and the Signalling System No. 7 ISDN user Part.

[2]	ITU-T Recommendation Q.2931 (1995), Broadband Integrated Services Digital Network (B-ISDN) – Digital Subscriber Signalling No. 2 (DSS 2) – User-network interface layer 3 specification for basic call/connection control.

[3]	ITU-T Recommendation Q.2764 (1995), Broadband Integrated Services Digital Network (B-ISDN) – Signalling System No. 7 B-ISDN user part (B-ISUP) – Basic call procedures.

# 3	Cause

## 3.1	Format

The format of the DSS 2 Cause information elementXE "DSS 2 Cause information element"§ and B-ISDN User Part Cause indicator parameterXE "B-ISDN User Part Cause indicator parameter"§ is shown in Figures 1/Q.2610 and 2/Q.2610 respectively.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | Bit/Octet |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | Cause information element identifier |  |  |  |  |  |  | 1 |
| 1 ext | Coding standard |  | Information element instruction field |  |  |  |  | 2 |
|  |  | Length of cause information contents |  |  |  |  |  | 3 4 |
| 1 ext | 0 | 0 | 0 | Location |  |  |  | 5 |
|  | Spare |  |  |  |  |  |  |  |
| 1 ext | Cause value |  |  |  |  |  |  | 6 |
| Diagnostic(s) (if any) |  |  |  |  |  |  |  | 7* 7n* |
| * Optional octets |  |  |  |  |  |  |  |  |

FIGURE  1/Q.2610

Detailed formatting scheme of the DSS 2 Cause information elementXE "Detailed formatting scheme of the DSS 2 Cause information element"§

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | Bit/Octet |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Parameter name |  |  |  |  |  |  |  | 1 |
| Length of cause information contents |  |  |  |  |  |  |  | 2 3 |
| ext | Parameter compatibility information |  |  |  |  |  |  | 4 |
| ext |  |  |  |  |  |  |  | 4a |
| ext | Coding standard |  | Spare | Location |  |  |  | 5 |
| ext | Cause value |  |  |  |  |  |  | 6 |
| Diagnostic(s) (if any) |  |  |  |  |  |  |  | 7* 7n* |
| * Optional octets |  |  |  |  |  |  |  |  |

FIGURE  2/Q.2610

Detailed formatting scheme of the B-ISDN User Part Cause indicator parameterXE "Detailed formatting scheme of the B-ISDN User Part Cause indicator parameter"§

## 3.2	Cause value

The cause values defined in 2.2.5/Q.850 [1] and 2.2.7/Q.850 [1] are applicable. In addition the following cause values are applicable:

| Cause No. | Definition | Diagnostics | Application | Reference |
| --- | --- | --- | --- | --- |
| 35 | Requested VPCI/VCI not available | (Not applicable) | DSS 2 | 5.2.3.1/Q.2931 [2] 5.2.3.2/Q.2931 [2] |
| 36 | VPCI/VCI assignment failure | (Not applicable) | DSS 2/B-ISDN User Part | 5.1.2/Q.2931 [2] 5.2.3/Q.2931 [2] 2.1.2.3/Q.2764 [3] |
| 37 | User cell rate not available | Identified subfield identifier | DSS 2/B-ISDN User Part | 5.1.3/Q.2931 [2] 2.3.1/Q.2764 [3] 2.3.2/Q.2764 [3] |
| 45 | No VPCI/VCI available | (Not applicable) | DSS 2/B-ISDN User Part | 5.1.2.1.2/Q.2931 [2] 5.2.3.1/Q.2931 [2] 5.2.3.2/Q.2931 [2] 2.3.1/Q.2764 [3] 2.3.2/Q.2764 [3] |
| 93 | AAL parameters cannot be supported | (Not applicable) | DSS 2 | Annex F/Q.2931 [2] |

Additional definitions:

–	Cause No. 35 – Requested VPCI/VCI not availableXE "Requested VPCI/VCI not available"§

This cause is returned when the VPCI/VCI indicated by the requesting entity cannot be provided by the other side of the interface.

–	Cause No. 36 – VPCI/VCI assignment failureXE "VPCI/VCI assignment failure"§

This cause indicates that assignment of VPCI/VCI values is not functioning correctly and that maintenance should be informed.

–	Cause No. 37 – User cell rate not availableXE "User cell rate not available"§

This cause is returned when the user cell rate requested by the user cannot be provided by the network.

–	Cause No. 45 – No VPCI/VCI availableXE "No VPCI/VCI available"§

This cause indicates that there is no appropriate VPCI/VCI presently available to handle the call.

–	Cause No. 93 – AAL parameters cannot be supportedXE "AAL parameters cannot be supported"§

This cause is used to indicate that the requested AAL parameters cannot be provided.

## 3.3	Diagnostics

The diagnostics defined in 2.2.6/Q.850 [1] are applicable. In addition the diagnostics applicable to Cause No. 82, when generated within the B-ISDN, is modified as described below.

### 3.3.1	Cause No. 82

The channel identity diagnosticXE "channel identity diagnostic"§ is encoded as follows:

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | Octet |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| VPCI |  |  |  |  |  |  |  | 7 |
| VPCI (continued) |  |  |  |  |  |  |  | 7a |
| VCI |  |  |  |  |  |  |  | 7b (Note) |
| VCI (continued) |  |  |  |  |  |  |  | 7c (Note) |
| NOTE – Octets 7b and 7c are present if a VCI is to be indicated with the VPCI. If only the VPCI is to be indicated octets 7b and 7c are not present. |  |  |  |  |  |  |  |  |

### 3.3.2	Identified subfield identifierXE "Identified subfield identifier"§

The identified subfield identifier diagnostic is encoded as follows:

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | Octet |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ATM user cell rate subfield identifier |  |  |  |  |  |  |  | 7 |
| NOTE – Octet 7 may be repeated to report multiple ATM user cell rate subfield identifiers. |  |  |  |  |  |  |  |  |

# 4	General rules for handling of the location field

The location field shall be handled as described in clause 3/Q.850 [1].

# 5	Handling of cause and location at the international interface

Clause 4/Q.850 [1] is applicable.