|  | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

ITU-T	Q.2763

TELECOMMUNICATION	(02/95)
STANDARDIZATION  SECTOR
OF  ITU

B-ISDN  APPLICATION  PROTOCOLS
OF  THE  NETWORK

BROADBAND  INTEGRATED  SERVICES
DIGITAL  NETWORK  (B-ISDN)  – 
SIGNALLING  SYSTEM  No.  7  B-ISDN
USER  PART  (B-ISUP)  –  FORMATS
AND  CODES

ITU-T  Recommendation  Q.2763

(Previously “CCITT  Recommendation”)

# FOREWORD

The ITU-T (Telecommunication Standardization Sector) is a permanent organ of the International Telecommunication Union (ITU). The ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Conference (WTSC), which meets every four years, establishes the topics for study by the ITU-T Study Groups which, in their turn, produce Recommendations on these topics.

The approval of Recommendations by the Members of the ITU-T is covered by the procedure laid down in WTSC Resolution No. 1 (Helsinki, March 1-12, 1993).

ITU-T Recommendation Q.2763 was prepared by ITU-T Study Group 11 (1993-1996) and was approved under the WTSC Resolution No. 1 procedure on the 7th of February 1995.

___________________

NOTE

In this Recommendation, the expression “Administration” is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

ã  ITU  1995

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

CONTENTS

Recommendation Q.2763     (02/95)

Page

1	Scope		1

2	References		1

3	Terms and definitions		1

4	Abbreviations		1

5	General encoding principles		1

5.1	Routing label		2

5.2	Message compatibility information		2

5.3	Message type code		2

5.4	Message length		2

5.5	Formatting principles		3

5.6	Encoding principles for subfields		3

5.7	Order of transmission		3

5.8	Coding of spare bits		3

5.9	National message types and parameters		3

5.10	Rules for the allocation of message and parameter name codes		3

5.11	Formats and codes of parameters originated from the access protocol		4

6	Parameter formats and codes		4

6.1	Message type codes		4

6.2	Coding of the message length indicator		5

6.3	Coding of the parameter length indicator		5

6.4	Coding of the message compatibility information		6

6.5	Coding of the parameter compatibility information		7

7	B-ISDN User Part parameters		8

7.1	Parameter names		8

7.2	AAL parameters		10

7.3	Access delivery information		10

7.4	Additional calling party number		10

7.5	Additional connected number		12

7.6	ATM cell rate		12

7.7	Automatic congestion level		13

7.8	Backward narrow-band interworking indicator		14

7.9	Broadband bearer capability		14

7.10	Broadband high layer information		14

7.11	Broadband low layer information		15

7.12	Call diversion information		16

7.13	Call diversion may occur		17

7.14	Call history information		17

7.15	Called party number		18

7.16	Called party sub-address		19

7.17	Called party’s indicators		20

7.18	Calling party number		21

7.19	Calling party sub-address		22

7.20	Calling party’s category		22

7.21	Cause indicators		23

7.22	Charge indicator		24

Page

7.23	Closed User Group information		24

7.24	Connected line identity request		25

7.25	Connected number		25

7.26	Connected sub-address		26

7.27	Connection element identifier		27

7.28	Consistency check result information		27

7.29	Destination signalling identifier		28

7.30	Echo control information		28

7.31	Forward narrow-band interworking indicator		29

7.32	In-band information indicator		30

7.33	Location number		30

7.34	Maximum end-to-end transit delay		32

7.35	MLPP precedence		32

7.36	MLPP user information		33

7.37	Narrow-band bearer capability		34

7.38	Narrow-band high layer compatibility		34

7.39	Narrow-band low layer compatibility		35

7.40	National/international call indicator		36

7.41	Notification		37

7.42	OAM traffic descriptor		38

7.43	Original called number		38

7.44	Origination ISC point code		39

7.45	Origination signalling identifier		40

7.46	Progress indicator		40

7.47	Propagation delay counter		40

7.48	Redirecting number		41

7.49	Redirection information		41

7.50	Redirection number		42

7.51	Redirection number restriction		42

7.52	Resource identifier		43

7.53	Segmentation indicator (national use)		44

7.54	Subsequent number		44

7.55	Suspend/resume indicators		44

7.56	Transit network selection (national use)		45

7.57	User-to-user indicators		46

7.58	User-to-user information		48

8	B-ISDN User Part messages and codes		49

Appendix  I – B-ISUP Messages and Parameters in ASN.1		58

I.1	General		58

SUMMARY

This Recommendation is one of a set of Recommendations that describe the Broadband ISDN User Part. It specifies the formats and codes of the B-ISDN User Part messages and parameters required to support basic bearer services and supplementary services for Capability Set 1 B-ISDN applications.

The scope of the B-ISDN User Part covers international B-ISDN networks. However, the B-ISDN User Part is suitable for national applications. Most messages and parameters specified for international use are also required in typical national applications.

Other ITU-T Recommendations in this group include:

–	ITU-T Recommendation Q.2761 (1995), Broadband Integrated Services Digital Network (B-ISDN) – Functional description of the B-ISDN User Part (B-ISUP) of Signalling System No. 7.

–	ITU-T Recommendation Q.2762 (1995), Broadband Integrated Services Digital Network (B-ISDN) – General functions of messages and signals of the B-ISDN User Part (B-ISUP) of Signalling System No. 7.

–	ITU-T Recommendation Q.2764 (1995), Broadband Integrated Services Digital Network (B-ISDN) – Signalling System No. 7 B-ISDN User Part (B-ISUP) – Basic call procedures.

–	ITU-T Recommendation Q.2730 (1995), Broadband Integrated Services Digital Network (B-ISDN) – Signalling System No. 7 B-ISDN User Part (B-ISUP) – Supplementary services.

–	ITU-T Recommendation Q.2610 (1995), Broadband Integrated Services Digital Network (B-ISDN) – Usage of cause and localtion in B-ISDN User Part and DSS 2.

–	ITU-T Recommendation Q.2650 (1995), Broadband ISDN, interworking between Signalling System No. 7 Broadband ISDN User Part (B-ISUP) and Digital subscriber Signalling System No. 2 (DSS 2).

–	ITU-T Recommendation Q.2660 (1995), Broadband Integrated Services Digital Network (B-ISDN) – Interworking between Signalling System No. 7 – Broadband ISDN User Part (B-ISUP) and Narrow-band ISDN User Part (N-ISUP).

Recommendation Q.2763

Recommendation Q.2763     (02/95)

broadband  integrated  services  digital  network  (B-isdn)  –
signalling  system  no.  7  b-isdn  user  part  (b-isup)  –
Formats  and  codes

(Geneva, 1995)

# 1	Scope

This Recommendation specifies the formats and codes of the B-ISDN User Part messages and parametersXE "B-ISDN User Part messages and parameters"§ required to support basic bearer services and supplementary services for Capability Set 1 B-ISDN applicationsXE "Capability Set 1 B-ISDN applications"§.

The B-ISDN User PartXE "B-ISDN User Part"§ is applicable to international B-ISDN networks. Furthermore, the B-ISDN User Part is suitable for national applications. Most messages and parameters specified for international use are also required in typical national applications. Moreover, coding space has been reserved in order to allow national Administrations and recognized operating agencies to introduce network specific signalling messages and parameters within the internationally standardized protocol structure.

# 2	References

See Recommendation Q.2761.

# 3	Terms and definitions

See Recommendation Q.2761.

# 4	Abbreviations

For the purpose of this Recommendation, the following abbreviations are used:

ATM	Asynchronous Transfer Mode

DPC		Destination Point Code

ECT		Explicit Call Transfer (supplementary service)

FPH		Freephone (supplementary service)

NI		Network Identity

OPC		Origination Point Code

S-AAL	Signalling ATM Adaptation Layer

SLS		Signalling Link Selection

STM	Synchronous Transfer Mode

For further abbreviations, see Recommendation Q.2761.

# 5	General encoding principles

B-ISDN user part messages are carried on the ATM signalling link by means of S-AAL service data units, the format of which is described in 6.2/Q.2110.

As a national option B-ISDN user part messages can be carried on the STM signalling link by means of signal units, the format of which is described in 2.2/Q.703.

The format of and the codes used in the service information octet are described in 14.2/Q.704. The service indicator for the B-ISDN user part is coded 1001.

The signalling information field of each message signal unit containing an B-ISDN user part message consists of an integral number of octets and encompasses the following parts (see Figure 1):

a)	routing label;

b)	message type code;

c)	message length;

d)	message compatibility information;

e)	message content.

A description of the various message parts is given in the following subclauses.

FIGURE  1/Q.2763

Routing label

Message type code

Message length

Message compatibility information

Message content

B-ISDN user part message format

## 5.1	Routing labelXE "Routing label"§

The format and codes used for the routing label are described in 2.2/Q.704. For each ATM virtual connection the same routing label must be used for each message that is transmitted for that connection.

The SLS value is allocated on a cyclic basis per signalling association, e.g. values of a cyclic 4-bit counter incremented for each new signalling association. A counter is required for each allocated OPC/DPC/NI combination.

## 5.2	Message compatibility informationXE "Message compatibility information"§

The message compatibility information consists of a one octet field and is mandatory for all messages. The message compatibility information defines the behaviour of a switch if the message is not understood.

## 5.3	Message type codeXE "Message type code"§

The message type code consists of a one octet field and is mandatory for all messages. The message type code uniquely defines the function and format of each B-ISDN user part message. The allocation with reference to the appropriate descriptive section of this Recommendation is summarized in Table 1.

## 5.4	Message lengthXE "Message length"§

The message length consists of a fixed length field and is mandatory for all messages. The message length uniquely indicates the number of octets included in the B-ISDN user part message content and the message compatibility information fields (see Figure 1).

## 5.5	Formatting principles

The message content of each message contains a number of parameter listed and described in clause 6. Each parameter has a NAME which is coded as a single octet (see Table 2). The length of a parameter may be fixed or variable, and a length indicator of fixed length for each parameter is included. Every parameter contains parameter compatibility information (see Figure 2).

FIGURE  2/Q.2763

Parameter name

Length indicator

Parameter compatibility information

Parameter content

B-ISDN user part parameter format

The detailed format of the length indicator and the parameter compatibility information is described in clause 6.

The detailed format of the parameter content is uniquely defined for each parameter as described in clause 7.

Between parameters there should be no un-used (i.e. dummy) octets.

## 5.6	Encoding principles for subfields

The content of each parameter contains a number of subfields. Within each subfield, the least significant bit of the field is represented by the lowest-numbered bit and the most significant bit is represented by the highest-numbered bit. In case a subfield exceeds one octet, the significance of the bits, if applicable, is indicated in clause 7.

## 5.7	Order of transmission

Since all the fields consist of an integral number of octets, the formats are presented as a stack of octets. The first octet transmitted to the underlying layer/level is the one shown at the top of the stack and the last is the one at the bottom.

Unless otherwise indicated, within each octet and subfield the bits are transmitted to the underlying layer/level with the least significant bit first.

## 5.8	Coding of spare bits

Spare bits are coded “0” unless indicated otherwise.

## 5.9	National message types and parameters

If messages type codes and parameters are required for national uses not included in this Recommendation, the codes chosen should be from the highest code downwards, that is starting with at code 11111110. Message type codes in the range 11111110 to 11100000 and parameter name codes in the range 11111110 to 11000001 are reserved exclusively for this purpose.

## 5.10	Rules for the allocation of message and parameter name codes

a)	Existing N-ISUP message and parameter name codes used in B-ISUP should have the same name codes in B-ISUP.

b)	Existing N-ISUP message and parameter name codes not used in B-ISUP should be marked reserved in B-ISUP.

c)	B-ISUP message and parameter name codes not used in N-ISUP should be marked reserved in N-ISUP.

d)	The code point 11111111 for both message and parameter name codes is reserved in  B-ISUP to indicate extension of the name code:

–	a second parameter name octet shall appear as the first octet in the parameter content (see Figure 2);

–	a second message name octet shall be encoded in a new parameter.

## 5.11	Formats and codes of parameters originated from the access protocol

The formats and codes of the parameters which are originated from the access protocol are the same as specified in Recommendation Q.2931. The coding standard subfield for such parameters shall be coded as specified in Recommendation Q.2931.

# 6	Parameter formats and codes

## 6.1	Message type codes

The encoding of the message type is shown in Table 1.

TABLE  1/Q.2763

| Message type | Reference (Q.2763) (Table) | Code |
| --- | --- | --- |
| Address complete | 03 | 0000 0110 |
| Answer | 04 | 0000 1001 |
| Blocking | 14 | 0001 0011 |
| Blocking acknowledgement | 15 | 0001 0101 |
| Call progress | 05 | 0010 1100 |
| Confusion | 06 | 0010 1111 |
| Consistency check end | 24 | 0001 0111 |
| Consistency check end acknowledgement | 25 | 0001 1000 |
| Consistency check request | 22 | 0000 0101 |
| Consistency check request acknowledgement | 23 | 0001 0001 |
| Forward transfer | 12 | 0000 1000 |
| IAM acknowledgement | 20 | 0000 1010 |
| IAM reject | 21 | 0000 1011 |
| Initial address | 07 | 0000 0001 |
| Network resource management | 18 | 0011 0010 |
| Release | 08 | 0000 1100 |
| Release complete | 09 | 0001 0000 |
| Reset | 14 | 0001 0010 |
| Reset acknowledgement | 15 | 0000 1111 |
| Resume | 13 | 0000 1110 |
| Segmentation (national use) | 19 | 0011 1000 |
| Subsequent address | 10 | 0000 0010 |
| Suspend | 13 | 0000 1101 |
| Unblocking | 14 | 0001 0100 |
| Unblocking acknowledgement | 15 | 0001 0110 |
| User part available | 17 | 0011 0101 |
| User part test | 16 | 0011 0100 |
| User-to-user information | 11 | 0010 1101 |

TABLE  1/Q.2763 (cont.)

| Message type | Reference (Q.2763) (Table) | Code |
| --- | --- | --- |
| Reserved for ECT |  | 0011 1001 |
| Reserved, used in N-ISUP |  | 0000 0011 |
| Reserved, used in N-ISUP |  | 0000 0100 |
| Reserved, used in N-ISUP |  | 0000 0111 |
| Reserved, used in N-ISUP |  | 0001 1001 |
| " " " " |  | to |
| Reserved, used in N-ISUP |  | 0010 1011 |
| Reserved, used in N-ISUP |  | 0010 1110 |
| Reserved, used in N-ISUP |  | 0011 0000 |
| Reserved, used in N-ISUP |  | 0011 0001 |
| Reserved, used in N-ISUP |  | 0011 0011 |
| Reserved, used in N-ISUP |  | 0011 0110 |
| Reserved, used in N-ISUP |  | 0011 0111 |
| Reserved for extension of name code |  | 1111 1111 |

## 6.2	Coding of the message length indicatorXE "Coding of the message length indicator"§

The format of the message length indicator field is shown in Figure 3.

The message length indicator field is binary coded to indicate the number of octets in the message content field and the message compatibility information field (see Figure 1). The length indicated does not include the routing label octets, the message type code octets or the message length indicator octets.

FIGURE 3/Q.2763 [D01] =  CM

The following codes are used in the message length indicator subfield:

a)	Length indicator

A code expressing in pure binary representation the number of octets (see Note). Bit 8 of octet 1 is the most significant and bit 1 of octet 2 is the least significant respectively.

NOTE – The actual maximum message length is given by the lower levels.

## 6.3	Coding of the parameter length indicatorXE "Coding of the parameter length indicator"§

The format of the parameter length indicator subfield corresponds to the format shown in Figure 3.

The parameter length indicator subfield is binary coded to indicate the number of octets in the parameter content and parameter compatibility information fields (see Figure 2). The length indicated does not include the parameter name octet or the parameter length indicator octets.

## 6.4	Coding of the message compatibility informationXE "Coding of the message compatibility information"§

The format of the message compatibility information is shown in Figure 4.

FIGURE 4/Q.2763 [D02] =  CM

The following codes are used in the subfields of the message compatibility information field.

a)	Extension indicator (ext.)

0	Octet continues through the next octet (e.g. octet 1 to 1a).

1	Last octet.

b)	Broadband/narrow-band interworking indicator (octet 1)

00	Pass on.

01	Discard message.

10	Release call.

11	Reserved, assume “00”.

c)	Pass on not possible indicator (octet 1)

0	Release call.

1	Discard information.

d)	Discard message indicator (octet 1)

0	Do not discard message (pass on) (see Note 1).

1	Discard message.

NOTE 1 – If “pass on” is set (discard message indicator = 0) but not possible, then the send notification indicator and the pass on not possible indicator are checked.

e)	Send notification indicator (octet 1) (see Note 2)

0	Do not send notification.

1	Send notification.

NOTE 2 – Notification by Confusion message.

f)	Release call indicator (octet 1)

0	Do not release call.

1	Release call.

g)	Transit at intermediate exchange indicator (octet 1)

0	Transit interpretation.

1	End node interpretation.

## 6.5	Coding of the parameter compatibility informationXE "Coding of the parameter compatibility information"§

The format of the parameter compatibility information subfield is shown in Figure 5.

FIGURE 5/Q.2763 [D03] =  CM

The following codes are used in the parameter compatibility information subfield:

a)	Extension indicator (ext.)

0	Octet continues through the next octet (e.g. octet 1 to 1a).

1	Last octet.

b)	Pass on not possible indicator (octet 1)

00	Release call.

01	Discard message.

10	Discard parameter.

11	Reserved, assume “00”.

c)	Discard parameter indicator (octet 1)

0	Do not discard parameter (pass on) (see Note 1).

1	Discard parameter.

NOTE 1 – If “pass on” is set (discard parameter indicator = 0) but not possible, then the send notification indicator and the pass on not possible indicator are checked.

d)	Discard message indicator (octet 1)

0	Do not discard message (pass on) (see Note 2).

1	Discard message.

NOTE 2 – If “pass on” is set (discard message indicator = 0) but not possible, then the send notification indicator and the pass on not possible indicator are checked.

e)	Send notification indicator (octet 1) (see Note 3)

0	Do not send notification.

1	Send notification.

NOTE 3 – Notification by Confusion or Release Complete message.

f)	Release call indicator (octet 1)

0	Do not release call.

1	Release call.

g)	Transit at intermediate exchange indicator (octet 1)

0	Transit interpretation.

1	End node interpretation.

h)	Broadband/narrow-band interworking indicator (octet 1a)

00	Pass on.

01	Discard message.

10	Release call.

11	Discard parameter.

# 7	B-ISDN User Part parametersXE "B-ISDN User Part parameters"§

## 7.1	Parameter names

The parameter name codes are given in Table 2 together with references to the subclauses in which they are described.

TABLE  2/Q.2763

| Parameter name | Reference (subclause) | Code |
| --- | --- | --- |
| AAL parameters | 7.2 | 0100 0111 |
| Access delivery information | 7.3 | 0010 1110 |
| Additional calling party number | 7.4 | 0001 0000 |
| Additional connected number | 7.5 | 0001 0001 |
| ATM cell rate | 7.6 | 0000 1000 |
| Automatic Congestion level | 7.7 | 0010 0111 |
| Backward narrow-band interworking indicator | 7.8 | 0001 0100 |
| Broadband bearer capability | 7.9 | 0101 0000 |
| Broadband high layer information | 7.10 | 0100 0110 |
| Broadband low layer information | 7.11 | 0100 1111 |
| Call diversion information | 7.12 | 0011 0110 |
| Call diversion may occur | 7.13 | 0010 0110 |
| Call history information | 7.14 | 0010 1101 |
| Called party number | 7.15 | 0000 0100 |
| Called party sub-address | 7.16 | 0001 0101 |
| Called party’s indicators | 7.17 | 0001 0111 |
| Calling party number | 7.18 | 0000 1010 |
| Calling party sub-address | 7.19 | 0001 0110 |
| Calling party’s category | 7.20 | 0000 1001 |
| Cause indicators | 7.21 | 0001 0010 |
| Charge indicator | 7.22 | 0001 1001 |
| Closed user group information | 7.23 | 0001 1010 |
| Connected line ID request | 7.24 | 0001 1011 |
| Connected number | 7.25 | 0010 0001 |
| Connected sub-address | 7.26 | 0010 0100 |
| Connection element identifier | 7.27 | 0000 0110 |
| Consistency check result information | 7.28 | 0100 1010 |
| Destination signalling identifier | 7.29 | 0000 0011 |
| Echo control information | 7.30 | 0011 0111 |
| Forward narrow-band interworking indicator | 7.31 | 0001 1100 |
| In-band information indicator | 7.32 | 0001 1111 |
| Location number | 7.33 | 0011 1111 |
| Maximum end-to-end transit delay | 7.34 | 0000 0111 |

TABLE  2/Q.2763 (cont.)

| Parameter name | Reference (subclause) | Code |
| --- | --- | --- |
| MLPP precedence | 7.35 | 0011 1010 |
| MLPP user information | 7.36 | 0100 1001 |
| Narrow-band bearer capability | 7.37 | 0001 1101 |
| Narrow-band high layer compatibility | 7.38 | 0011 0100 |
| Narrow-band low layer compatibility | 7.39 | 0010 0101 |
| National/international call indicator | 7.40 | 0010 1001 |
| Notification | 7.41 | 0010 1100 |
| OAM traffic descriptor | 7.42 | 0100 1000 |
| Original called number | 7.43 | 0010 1000 |
| Origination ISC point code | 7.44 | 0010 1011 |
| Origination signalling identifier | 7.45 | 0000 0010 |
| Progress indicator | 7.46 | 0011 0101 |
| Propagation delay counter | 7.47 | 0011 0001 |
| Redirecting number | 7.48 | 0000 1011 |
| Redirection information | 7.49 | 0001 0011 |
| Redirection number | 7.50 | 0000 1100 |
| Redirection number restriction | 7.51 | 0100 0000 |
| Resource identifier | 7.52 | 0011 1001 |
| Segmentation indicator (national use) | 7.53 | 0011 1110 |
| Subsequent number | 7.54 | 0000 0101 |
| Suspend/resume indicators | 7.55 | 0010 0010 |
| Transit network selection (national use) | 7.56 | 0010 0011 |
| User-to-user indicators | 7.57 | 0010 1010 |
| User-to-user information | 7.58 | 0010 0000 |
| Reserved for FPH |  | 0101 0001 |
| Reserved, used in N-ISUP |  | 0000 0000 |
| Reserved, used in N-ISUP |  | 0000 0001 |
| Reserved, used in N-ISUP |  | 0000 1101 |
| " " " " |  | to |
| Reserved, used in N-ISUP |  | 0000 1111 |
| Reserved, used in N-ISUP |  | 0001 1000 |
| Reserved, used in N-ISUP |  | 0001 1110 |
| Reserved, used in N-ISUP |  | 0010 1111 |
| Reserved, used in N-ISUP |  | 0011 0000 |
| Reserved, used in N-ISUP |  | 0011 0010 |
| Reserved, used in N-ISUP |  | 0011 0011 |
| Reserved, used in N-ISUP |  | 0011 1000 |
| Reserved, used in N-ISUP |  | 0011 1011 |
| " " " " |  | to |
| Reserved, used in N-ISUP |  | 0011 1101 |
| Reserved, used in N-ISUP |  | 0100 0001 |
| Reserved, used in N-ISUP |  | 0100 0010 |
| Reserved, used in N-ISUP |  | 0100 0011 |
| Reserved, used in N-ISUP |  | 0100 0100 |
| Reserved, used in N-ISUP |  | 0100 0101 |
| Reserved, used in N-ISUP |  | 0100 1011 |
| " " " " |  | to |
| Reserved, used in N-ISUP |  | 0100 1110 |
| Reserved, used in N-ISUP |  | 1100 0000 |
| Reserved for extension of name code |  | 1111 1111 |

## 7.2	AAL parametersXE "AAL parameters"§

The format of the AAL parameters parameter field is shown in Figure 6.

FIGURE 6/Q.2763 [D04] =  CM

The codes to be used in the subfields of the AAL parameters parameter field are defined in the AAL parameters information element in Recommendation Q.2931.

## 7.3	Access delivery informationXE "Access delivery information"§

The format of the access delivery information parameter field is shown in Figure 7.

FIGURE 7/Q.2763 [D05] =  CM

The following codes are used in the subfields of the access delivery information parameter field:

a)	Access delivery indicator

0	Set-up message generated.

1	No set-up message generated.

## 7.4	Additional calling party numberXE "Additional calling party number"§

The format of the additional calling party number parameter field is shown in Figure 8.

FIGURE 8/Q.2763 [D06] =  CM

The following codes are used in the subfields of the additional calling party number parameter field:

a)	Odd/even indicator

See 7.15 a).

b)	Nature of address indicator

0000000		Spare.

0000001		Subscriber number.

0000010		Unknown.

0000011		National (significant) number.

0000100		International number.

Spare.

Reserved for national use.

1111111		Spare.

c)	Numbering plan indicator

000		Spare.

001		ISDN (telephony) numbering plan (see Recommendation E.164).

010		Spare.

011		Reserved for national use.

100		Reserved for national use.

101		Private numbering plan.

110		Reserved for national use.

111		Spare.

d)	Presentation indicator

00	Presentation allowed.

01	Presentation restricted.

10	Address not available.

11	Spare.

e)	Screening indicator

00	User provided, not verified.

01	User provided, verified and passed.

10	User provided, verified and failed.

11	Network provided.

f)	Address signal

0000	Digit 0.

0001	Digit 1.

0010	Digit 2.

0011	Digit 3.

0100	Digit 4.

0101	Digit 5.

0110	Digit 6.

0111	Digit 7.

1000	Digit 8.

1001	Digit 9.

Spare.

g)	Filler

See 7.15 f).

## 7.5	Additional connected numberXE "Additional connected number"§

The format of the additional connected number parameter field corresponds to the format shown in Figure 8.

The encoding is identical to the encoding of the additional calling party number parameter field shown in 7.4.

## 7.6	ATM cell rateXE "ATM cell rate"§

The format of the ATM cell rate parameter field is shown in Figure 9.

The following codes are used in the subfields of the ATM cell rate parameter field:

a)	Extension indicator (ext.)

0	Octet continues through the next octet (e.g. octet 1 to 1a).

1	Last octet.

b)	Cell rate identifier (octet i)

0000000		Not used.

0000010		Forward peak cell rate for Cell Loss Priority = 0.

0000011		Backward peak cell rate for Cell Loss Priority = 0.

0000100		Forward peak cell rate for Cell Loss Priority = 0 + 1.

0000101		Backward peak cell rate for Cell Loss Priority = 0 + 1.

NOTE – For details on the use of Cell Loss Priority, see Recommendation I.371.

c)	Cell rate (octets i + 1 to i + 3)

A code expressing in pure 3 octet integer representation the number of cells per second. Bit 8 of the 1st octet (e.g. i + 1) is the most significant and bit 1 of the 3rd octet (e.g. i + 3) is the least significant respectively.

FIGURE 9/Q.2763 [D07] =  CM

## 7.7	Automatic congestion levelXE "Automatic congestion level"§

The format of the automatic congestion level parameter field is shown in Figure 10.

FIGURE 10/Q.2763 [D08] =  CM

The following codes are used in the automatic congestion level parameter field:

a)	Automatic congestion level

00000000	Spare.

00000001	Congestion level 1 exceeded.

00000010	Congestion level 2 exceeded.

Spare.

## 7.8	Backward narrow-band interworking indicatorXE "Backward narrow-band interworking indicator"§

The format of the backward narrow-band interworking indicator parameter field is shown in Figure 11.

FIGURE 11/Q.2763 [D09] =  CM

The following codes are used in the subfields of the backward narrow-band interworking indicator parameter field:

a)	ISDN access indicator

0	Terminating access non-ISDN.

1	Terminating access ISDN.

b)	ISDN user part indicator

0	ISDN user part not used all the way.

1	ISDN user part used all the way.

c)	Interworking indicator

0	No interworking encountered (No. 7 signalling all the way).

1	Interworking encountered.

## 7.9	Broadband bearer capabilityXE "Broadband bearer capability"§

The format of the broadband bearer capability parameter field is shown in Figure 12.

The codes to be used in the subfields of the broadband bearer capability parameter field are defined in the broadband bearer capability information element in Recommendation Q.2931.

## 7.10	Broadband high layer informationXE "Broadband high layer information"§

The format of the broadband high layer information parameter field is shown in Figure 13.

The codes to be used in the subfields of the broadband high layer information parameter field are defined in the broadband high layer information information elements in Recommendation Q.2931.

FIGURE 12/Q.2763 [D10] =  CM

FIGURE 13/Q.2763 [D11] =  CM

## 7.11	Broadband low layer informationXE "Broadband low layer information"§

The format of the broadband low layer information parameter field is shown in Figure14.

FIGURE 14/Q.2763 [D12] =  CM

The following codes are used in the subfields of the broadband low layer information parameter field.

a)	Repeat indicator (octet 1)

0	Information element not repeated.

1	Information element repeated.

b)	Priority (octet 1)

0000	No prioritized order.

0001	Prioritized list for selecting one possibility: ascending order.

0010	Prioritized list for selecting one possibility: descending order.

Reserved.

c)	Broadband low layer information information elements (octets 2 to n)

This field contains all broadband low layer information information elements as received from DSS 2. The format of the broadband low layer information elements is the same as specified in Recommendation Q.2931. The order of the information elements is not changed.

## 7.12	Call diversion informationXE "Call diversion information"§

The format of the call diversion information parameter field is shown in Figure 15.

FIGURE 15/Q.2763 [D13] =  CM

The following codes are used in the call diversion information parameter field:

a)	Redirection reason

0000	Unknown.

0001	User busy.

0010	No reply.

0011	Unconditional.

0100	Deflection during alerting.

0101	Deflection immediate response.

0110	Mobile subscriber not reachable.

Spare.

b)	Notification subscription options

000		Unknown.

001		Presentation not allowed.

010		Presentation allowed with redirection number.

011		Presentation allowed without redirection number.

Spare.

## 7.13	Call diversion may occurXE "Call diversion may occur"§

The format of the call diversion may occur parameter field is shown in Figure 16.

FIGURE 16/Q.2763 [D14] =  CM

The following codes are used in the subfields of the call diversion may occur parameter field:

a)	Call diversion may occur indicator

0	No indication.

1	Call diversion may occur.

## 7.14	Call history informationXE "Call history information"§

The format of the call history information parameter field is shown in Figure 17.

FIGURE 17/Q.2763 [D15] =  CM

The following codes are used in the subfields of the call history information parameter field:

a)	Delay value (octets 1 to 2)

The delay value expresses in pure binary representation the propagation delay value of a call in ms. Bit 8 of octet 1 is the most significant and bit 1 of octet 2 is the least significant respectively.

## 7.15	Called party numberXE "Called party number"§

The format of the called party number parameter field is shown in Figure 18.

FIGURE 18/Q.2763 [D16] =  CM

The following codes are used in the subfields of the called party number parameter field:

a)	Odd/even indicator (octet 1)

0	Even number of address signals.

1	Odd number of address signals.

b)	Nature of address indicator (octet 1)

0000000		Spare.

0000001		Subscriber number (national use).

0000010		Unknown (national use).

0000011		National (significant) number.

0000100		International number.

Spare.

Reserved for national use.

1111111		Spare.

c)	Internal network number indicator (octet 2)

0	Routing to internal network number allowed.

1	Routing to internal network number not allowed.

d)	Numbering plan indicator (octet 2)

000		Spare.

001		ISDN (telephony) numbering plan (see Recommendation E.164).

010		Spare.

011		Reserved for national use.

100		Reserved for national use.

101		Reserved for national use.

110		Reserved for national use.

111		Spare.

e)	Address signal (octets 3 to n)

0000	Digit 0.

0001	Digit 1.

0010	Digit 2.

0011	Digit 3.

0100	Digit 4.

0101	Digit 5.

0110	Digit 6.

0111	Digit 7.

1000	Digit 8.

1001	Digit 9.

1010	Spare.

1011	Code 11.

1100	Code 12.

1101	Spare.

1110	Spare.

1111	ST.

The most significant address signal is sent first. Subsequent address signals are sent in successive 4-bit fields.

f)	Filler (octet n)

In case of an odd number of address signals, the filler code 0000 is inserted after the last address signal.

## 7.16	Called party sub-addressXE "Called party sub-address"§

The format of the called party sub-address parameter field is shown in Figure 19.

FIGURE 19/Q.2763 [D17] =  CM

The codes to be used in the subfields of the called party sub-address parameter field are defined in the called party sub-address information element in Recommendation Q.2931.

## 7.17	Called party’s indicatorsXE "Called party’s indicators"§

The format of the called party’s indicators parameter field is shown in Figure 20.

FIGURE 20/Q.2763 [D18] =  CM

The following codes are used in the subfields of the called party’s indicators parameter field:

a)	Called party’s category indicator

00	No indication.

01	Ordinary subscriber.

10	Payphone.

11	Spare.

b)	Called party’s status indicator

0	No indication.

1	Alerting.

## 7.18	Calling party numberXE "Calling party number"§

The format of the calling party number parameter field is shown in Figure 21.

FIGURE 21/Q.2763 [D19] =  CM

NOTE 1 – When the address presentation restricted indicator indicates address not available, octets 3 to n are omitted.

The following codes are used in the calling party number parameter field:

a)	Odd/even indicator (octet 1)

See 7.15 a).

b)	Nature of address indicator (octet 1)

0000000		Spare.

0000001		Subscriber number (national use).

0000010		Unknown (national use).

0000011		National (significant) number (national use).

0000100		International number.

Spare.

Reserved for national use.

1111111		Spare.

c)	Calling party number incomplete indicator (octet 2) (national use)

0	Complete.

1	Incomplete (national use).

d)	Numbering plan indicator (octet 2)

See 7.15 d).

e)	Address presentation restricted indicator (octet 2)

00	Presentation allowed.

01	Presentation restricted.

10	Address not available (see Note 2) (national use).

11	Spare.

NOTE 2 – When the address is unavailable, the subfields in items a), b), c) and d) are coded with 0’s.

f)	Screening indicator (octet 2)

00	Reserved (see Note 3).

01	User provided, verified and passed.

10	Reserved (see Note 3).

11	Network provided.

NOTE 3 – Code 00 and 10 are reserved for “user provided, not verified” and “user provided, verified and failed” respectively. Codes 00 and 10 are for national use.

g)	Address signal (octets 3 to n)

0000	Digit 0.

0001	Digit 1.

0010	Digit 2.

0011	Digit 3.

0100	Digit 4.

0101	Digit 5.

0110	Digit 6.

0111	Digit 7.

1000	Digit 8.

1001	Digit 9.

1010	Spare.

1011	Code 11.

1100	Code 12.

Spare.

h)	Filler (octet n)

See 7.15 f).

## 7.19	Calling party sub-addressXE "Calling party sub-address"§

The format of the calling party sub-address parameter field corresponds to the format shown in Figure 19.

The codes to be used in the subfields of the calling party sub-address parameter field are defined in the calling party sub-address information element in Recommendation Q.2931.

## 7.20	Calling party’s categoryXE "Calling party’s category"§

The format of the calling party’s category parameter field is shown in Figure 22.

FIGURE 22/Q.2763 [D20] =  CM

The following codes are used in the calling party’s category parameter field:

a)	Calling party’s category

| 00000000 | Calling party’s category unknown at this time (national use). |  |
| --- | --- | --- |
| 00000001 | Operator, language French. |  |
| 00000010 | Operator, language English. |  |
| 00000011 | Operator, language German. |  |
| 00000100 | Operator, language Russian. |  |
| 00000101 | Operator, language Spanish. |  |
|  |  | Available to Administrations for selecting a particular language by mutual agreement. |
| 00001001 | Reserved (see Recommendation Q.104) (see Note) (national use). |  |
| 00001010 | Ordinary calling subscriber. |  |
| 00001011 | Calling subscriber with priority. |  |
| 00001100 | Data call (voice band data). |  |
| 00001101 | Test call. |  |
| 00001110 | Spare. |  |
| 00001111 | Payphone. |  |
|  |  | Spare. |
|  |  | Reserved for national use. |
| 11111111 | Spare. |  |

NOTE – In national networks, code 00001001 may be used to indicate that the calling party is a national operator.

## 7.21	Cause indicatorsXE "Cause indicators"§

The encoding of the cause indicator parameter field is shown in Recommendation Q.2610.

## 7.22	Charge indicatorXE "Charge indicator"§

The format of the charge indicator parameter field is shown in Figure 23.

FIGURE 23/Q.2763 [D21] =  CM

The following codes are used in the charge indicator parameter field:

a)	Charge indicator

0	No charge.

1	Charge.

NOTE – The interpretation of these bits depends only on the charging exchange.

## 7.23	Closed User Group informationXE "Closed User Group information"§

The format of the Closed User Group (CUG) information parameter field is shown in Figure 24.

FIGURE 24/Q.2763 [D22] =  CM

The following codes are used in the subfields of the Closed User Group information parameter field:

a)	CUG call indicator (octet 1)

0	Closed User Group call, outgoing access allowed.

1	Closed User Group call, outgoing access not allowed.

b)	Network identity (NI) (octets 2 and 3)

Each digit is coded in binary coded decimal representation from 0 to 9.

If the first digit of this field is coded 0 or 9, the TCC (Telephony Country Code) follows in the second to fourth NI digits (the most significant TCC digit is in the 2nd NI digit). If the TCC is one or two digits long, the excess digit(s) is inserted with the code for ROA or network identification, if necessary. If octet 3 is not required, it is coded all zeros.

Coding of the first digit as 1 or 8 is excluded.

If the first digit is not 0, 9, 1 or 8, this field contains a DNIC (Data Network Identification Code) as defined in Recommendation X.121.

c)	Binary code (octets 4 and 5)

A code allocated to a Closed User Group administered by a particular ISDN or data network. Bit 8 of octet 4 is the most significant and bit 1 of octet 5 is the least significant respectively.

NOTE – Octets 2 to 5 constitute the Closed User Group interlock code.

## 7.24	Connected line identity requestXE "Connected line identity request"§

The format of the connected line identity request parameter field is shown in Figure 25.

FIGURE 25/Q.2763 [D23] =  CM

The following codes are used in the connected line identity request parameter field:

a)	Connected line identity request indicator

0	Not requested.

1	Requested.

## 7.25	Connected numberXE "Connected number"§

The format of the connected number parameter field corresponds to the format shown in Figure 26.

FIGURE 26/Q.2763 [D24] =  CM

NOTE 1 – When the address presentation restricted indicator indicates address not available, octets 3 to n are omitted, and the screening indicator is set to 11 network provided.

The following codes are used in the subfields of the connected number parameter field:

a)	Odd/even indicator (octet 1)

See 7.15 a).

b)	Nature of address indicator (octet 1)

See 7.15 b).

c)	Numbering plan indicator (octet 2)

See 7.15 d).

d)	Address presentation restricted indicator (octet 2)

00	Presentation allowed.

01	Presentation restricted.

10	Address not available (see Note 2).

11	Spare.

NOTE 2 – When the address is unavailable, the subfields in items a), b), and c) are coded with 0’s.

e)	Screening indicator (octet 2)

See 7.18 f).

f)	Address signal (octets 2 to n)

See 7.18 g).

g)	Filler (octet n)

See 7.15 f).

## 7.26	Connected sub-addressXE "Connected sub-aaddress"§

The format of the connected sub-address parameter field corresponds to the format shown in Figure 19.

The codes to be used in the subfields of the connected sub-address parameter field are defined in the connected sub-address information element in Recommendation Q.2951.

## 7.27	Connection element identifierXE "Connection element identifier"§

The format of the connection element identifier parameter field is shown in Figure 27.

FIGURE 27/Q.2763 [D25] =  CM

The following codes are used in the subfields of the connection element identifier parameter field:

a)	Virtual Path Connection IdentifierXE "Virtual Path Connection Identifier"§ (octets 1 and 2)

A code expressing in pure binary representation the identifier of the virtual path connection (see Note 1). Bit 8 of octet 1 is the most significant and bit 1 of octet 2 is the least significant respectively.

NOTE 1 – The VPCI is an identifier that represents a VPI on a given interface. The value of this field is not identical to the value used in the VPI field of the corresponding ATM cell header.

b)	Virtual Channel IdentifierXE "Virtual Channel Identifier"§ (octets 3 and 4)

A code expressing in pure binary representation the identifier of the virtual channel (see Notes 2 and 3). Bit 8 of octet 3 is the most significant and bit 1 of octet 4 is the least significant respectively.

NOTE 2 – The value of this field is the same as the value used in the VCI field of the corresponding ATM cell header.

NOTE 3 – Octets 3 and 4 are omitted if the entire virtual path connection is addressed.

NOTE 4 – Consistency with Recommendation I.361 for pre-assigned VPI/VCI values should be assured.

## 7.28	Consistency check result informationXE "Consistency check result information"§

The format of the consistency check result information parameter field is shown in Figure 28.

The following codes are used in the subfields of the consistency check result information parameter field:

a)	VPCI check result indicator

0	Virtual Path Connection Identifier check NOT successful.

1	Virtual Path Connection Identifier check successful.

2	Virtual Path Connection Identifier check not performed.

3	Spare.

FIGURE 28/Q.2763 [D26] =  CM

## 7.29	Destination signalling identifierXE "Destination signalling identifier"§

The format of the destination signalling identifier parameter field is shown in Figure 29.

FIGURE 29/Q.2763 [D27] =  CM

The following codes are used in the subfields of the destination signalling identifier parameter field:

a)	Control ID

A bit string representing the identification allocated to the signalling association.

## 7.30	Echo control informationXE "Echo control information"§

The format of the echo control information parameter field is shown in Figure 30.

FIGURE 30/Q.2763 [D28] =  CM

The following codes are used in the echo control information parameter field:

a)	Incoming half echo control device request indicator

00	No information.

01	Incoming half echo control device activation request.

10	Incoming half echo control device deactivation request.

11	Spare.

b)	Outgoing half echo control device request indicator

00	No information.

01	Outgoing half echo control device activation request.

10	Outgoing half echo control device deactivation request.

11	Spare.

c)	Incoming half echo control device indicator

00	No information.

01	Incoming half echo control device not included.

10	Incoming half echo control device included.

11	Spare.

d)	Outgoing half echo control device indicator

00	No information.

01	Outgoing half echo control device not included.

10	Outgoing half echo control device included.

11	Spare.

## 7.31	Forward narrow-band interworking indicatorXE "Forward narrow-band interworking indicator"§

The format of the forward narrow-band interworking indicator parameter field is shown in Figure 31.

FIGURE 31/Q.2763 [D29] =  CM

The following codes are used in the forward narrow-band interworking indicator parameter field:

a)	ISDN user part preference indicator

00	ISDN user part preferred all the way.

01	ISDN user part not required all the way.

10	ISDN user part required all the way.

11	Spare.

b)	ISDN access indicator

0	Originating access non-ISDN.

1	Originating access ISDN.

c)	ISDN user part indicator

0	ISDN user part not used all the way.

1	ISDN user part used all the way.

d)	Interworking indicator

0	No interworking encountered (No. 7 signalling all the way).

1	Interworking encountered.

## 7.32	In-band information indicatorXE "In-band information indicator"§

The format of the in-band information indicator parameter field is shown in Figure 32.

FIGURE 32/Q.2763 [D30] =  CM

The following codes are used in the in-band information indicator parameter field:

a)	In-band information indicator

0	No indication.

1	In-band information or an appropriate pattern is now available.

## 7.33	Location numberXE "Location number"§

The format of the Location number field is shown in Figure 33.

The following codes are used in the subfields of the location number parameter field:

a)	Odd/even indicator (octet 1)

See 7.15 a).

b)	Nature of address indicator (octet 1)

0000000		Spare.

0000001		Reserved for subscriber number (national use).

0000010		Reserved for unknown (national use).

0000011		National (significant) number (national use).

0000100		International number.

Spare.

Reserved for national use.

1111111		Spare.

FIGURE 33/Q.2763 [D31] =  CM

c)	Internal network number indicator (octet 2)

0	Routing to internal number allowed.

1	Routing to internal number not allowed.

NOTE 1 – Default set to 1.

d)	Numbering plan indicator (octet 2)

000		Spare.

001		ISDN (telephony) numbering plan (see Recommendation E.164).

010		Spare.

011		Reserved for national use.

100		Reserved for national use.

101		Private numbering plan.

110		Reserved for national use.

111		Spare.

NOTE 2 – Default set to 001.

e)	Address presentation restricted indicator (octet 2)

00	Presentation allowed.

01	Presentation restricted.

10	Address not available (national use).

11	Spare.

NOTE 3 – Default set to 01.

f)	Screening indicator (octet 2)

00	Reserved.

01	User provided, verified and passed.

10	Reserved.

11	Network provided.

NOTE 4 – Default set to 11.

g)	Address signals (octets 3 to n)

0000	Digit 0.

0001	Digit 1.

0010	Digit 2.

0011	Digit 3.

0100	Digit 4.

0101	Digit 5.

0110	Digit 6.

0111	Digit 7.

1000	Digit 8.

1001	Digit 9.

Spare.

h)	Filler (octet n)

See 7.15 f).

## 7.34	Maximum end-to-end transit delayXE "Maximum end-to-end transit delay"§

Recommendation Q.2763     (02/95)

The format of the maximum end-to-end transit delay parameter field corresponds to the format shown in Figure 17.

The encoding is identical to the encoding of the call history information parameter field shown in 7.14.

## 7.35	MLPP precedenceXE "MLPP precedence"§

The format of the Multi-level Precedence and Preemption (MLPP) precedence parameter field is shown in Figure 34.

FIGURE 34/Q.2763...[D32] = 3 CM  (118%)

The following codes are used in the subfields of the MLPP precedence parameter field.

Recommendation Q.2763     (02/95)

a)	Look-ahead For Busy (LFB) indicator

00	LFB allowed.

01	Path reserved (national use).

10	LFB not allowed.

11	Spare.

b)	Precedence level

0000	Flash override.

0001	Flash.

0010	Immediate.

0011	Priority.

0100	Routine.

c)	Network identity NI (octets 2 and 3)

Each digit is coded in binary coded decimal representation from 0 to 9.

The first digit of this field is coded 0. The TCC (Telephony Country Code) follows in the second to fourth NI digits (the most significant TCC digit is in the 2nd NI digit). If the TCC is one or two digits long, the excess digit(s) is(are) inserted with the code for ROA or network identification, if necessary. If octet 3 is not required, it is coded all zeros.

d)	MLPP service domain (octets 4, 5 and 6)

A pure binary code allocated to a MLPP service domain administered by a particular ISDN. Bit 8 of octet 4 is the most significant and bit 1 of octet 6 is the least significant respectively.

## 7.36	MLPP user informationXE "MLPP user information"§

The format of the Multi-level Precedence and Preemption (MLPP) user information parameter field is shown in Figure 35.

FIGURE 35/Q.2763...[D33] = 3 CM  (118%)

The following codes are used in the subfields of the MLPP user information parameter field.

a)	MLPP user indicator

0	No indication.

1	MLPP user.

## 7.37	Narrow-band bearer capabilityXE "Narrow-band bearer capability"§

The format of the narrow-band bearer capability parameter field is shown in Figure 36.

FIGURE 36/Q.2763...[D34] = 3 CM  (118%)

The following codes are used in the subfields of the narrow-band bearer capability parameter field.

a)	Repeat indicator (octet 1)

0	Information element not repeated.

1	Information element repeated.

b)	Priority (octet 1)

0000	No prioritized order.

0001	Prioritized list for selecting one possibility: ascending order.

0010	Prioritized list for selecting one possibility: descending order.

c)	Narrow-band bearer capability information elements (octets 2 to n)

This field contains all narrow-band bearer capability information elements as received from DSS 2 or as obtained in case of interworking with N-ISUP. The format of the narrow-band bearer capability elements is the same as specified in Recommendation Q.2931. The order of the information elements is not changed.

## 7.38	Narrow-band high layer compatibilityXE "Narrow-band high layer compatibility"§

The format of the narrow-band high layer compatibility parameter field is shown in Figure 37.

The following codes are used in the subfields of the narrow-band high layer compatibility parameter field.

a)	Repeat indicator (octet 1)

0	Information element not repeated.

1	Information element repeated.

b)	Priority (octet 1)

0000	No prioritized order.

0001	Prioritized list for selecting one possibility: ascending order.

0010	Prioritized list for selecting one possibility: descending order

c)	Narrow-band high layer compatibility information elements (octets 2 to n)

This field contains all narrow-band high layer compatibility information elements as received from DSS 2 or as obtained in case of interworking with N-ISUP. The format of the narrow-band high layer compatibility elements is the same as specified in Recommendation Q.2931. The order of the information elements is not changed.

FIGURE 37/Q.2763...[D35] = 3 CM  (118%)

## 7.39	Narrow-band low layer compatibilityXE "Narrow-band low layer compatibility"§

The format of the narrow-band low layer compatibility parameter field is shown in Figure 38.

The following codes are used in the subfields of the narrow-band low layer compatibility parameter field.

a)	Repeat indicator (octet 1)

0	Information element not repeated.

1	Information element repeated.

b)	Priority (octet 1)

0000	No prioritized order.

0001	Prioritized list for selecting one possibility: ascending order.

0010	Prioritized list for selecting one possibility: descending order.

c)	Narrow-band low layer compatibility information elements (octets 2 to n)XE "Narrow-band low layer compatibility information elements (octet 2 to n)"§

This field contains all narrow-band low layer compatibility information elements as received from DSS 2 or as obtained in case of interworking with N-ISUP. The format of the narrow-band low layer compatibility elements is the same as specified in Recommendation Q.2931. The order of the information elements is not changed.

FIGURE 38/Q.2763...[D36] = 3 CM  (118%)

## 7.40	National/international call indicatorXE "National/international call indicator"§

The format of the national/international call indicator parameter field is shown in Figure 39.

FIGURE 39/Q.2763...[D37] = 3 CM  (118%)

The following codes are used in the national/international call indicator parameter field:

a)	National/international call indicator

0	Call to be treated as national call.

1	Call to be treated as international call.

## 7.41	NotificationXE "Notification"§

The format of the notification parameter field is shown in Figure 40.

FIGURE 40/Q.2763...[D38] = 3 CM  (118%)

The following codes are used in the notification indicator parameter field:

a)	Extension indicator (ext.)

0	Information continues in the next octet (e.g. octet 1 to 1a).

1	Last octet.

b)	Notification indicator

0000100	Call completion delay.

1000010	Conference established.

1000011	Conference disconnected.

1000100	Other party added.

1000101	Isolated.

1000110	Reattached.

1000111	Other party isolated.

1001000	Other party reattached.

1001001	Other party split.

1001010	Other party disconnected.

1001011	Conference floating.

1100000	Call is a waiting call.

1101000	Diversion activated (used in DSS 1).

1101001	Call transfer, alerting.

1101010	Call transfer, active.

1111001	Remote hold.

1111010	Remote retrieval.

1111011	Call is diverting.

All other values are currently reserved for further extensions.

## 7.42	OAM traffic descriptorXE "OAM traffic descriptor"§

The format of the OAM traffic descriptor parameter field is shown in Figure 41.

FIGURE 41/Q.2763...[D39] = 3 CM  (118%)

The codes to be used in the subfields of the OAM traffic descriptor parameter field are defined in the OAM traffic descriptor information element in Recommendation Q.2931.

## 7.43	Original called numberXE "Original called number"§

The format of the original called number parameter field corresponds to the format shown in Figure 42.

The following codes are used in the subfields of the original called number parameter field:

a)	Odd/even indicator (octet 1)

See 7.15 a).

b)	Nature of address indicator (octet 1)

See 7.15 b).

c)	Numbering plan indicator (octet 2)

See 7.15 d).

d)	Address presentation restricted indicator (octet 2)

00	Presentation allowed.

01	Presentation restricted.

10	Spare.

11	Spare.

e)	Address signal (octets 3 to n)

See 7.18 g).

f)	Filler (octet n)

See 7.15 f).

FIGURE 42/Q.2763...[D40] = 3 CM  (118%)

## 7.44	Origination ISC point codeXE "Origination ISC point code"§

The format of the origination ISC point code parameter field is shown in Figure 43.

FIGURE 43/Q.2763...[D41] = 3 CM  (118%)

The following codes are used in the subfields of the origination ISC point code parameter field:

a)	Signalling point codeXE "Signalling point code"§ (octets 1 and 2)

The signalling point code is a pure binary representation of the code allocated to a node in the signalling network. Bit 6 of octet 2 is the most significant and bit 1 of octet 1 is the least significant respectively.

## 7.45	Origination signalling identifierXE "Origination signalling identifier"§

The format of the origination signalling identifier parameter field corresponds to the format shown in Figure 29.

The encoding is identical to the encoding of the destination signalling identifier parameter field shown in 7.29.

## 7.46	Progress indicatorXE "Progress indicator"§

The format of the progress indicator parameter field is shown in Figure 44.

FIGURE 44/Q.2763...[D42] = 3 CM  (118%)

The following codes are used in the subfields of the progress indicator parameter field.

a)	Repeat indicatorXE "Repeat indicator"§ (octet 1)

0	Information element not repeated.

1	Information element repeated.

b)	Priority (octet 1)

0000	No prioritized order.

0001	Prioritized list for selecting one possibility: ascending order.

0010	Prioritized list for selecting one possibility: descending order

c)	Progress indicator information elementsXE "Progress indicator information elements"§ (octets 2 to n)

This field contains all progress indicator information elements as received from DSS 2 or as obtained in case of interworking with N-ISUP. The format of the progress indicator elements is the same as specified in Recommendation Q.2931. The order of the information elements is not changed.

## 7.47	Propagation delay counterXE "Propagation delay counter"§

The format of the propagation delay counter parameter field corresponds to the format shown in Figure 17.

The encoding is identical to the encoding of the call history information parameter field shown in 7.14.

## 7.48	Redirecting numberXE "Redirecting number"§

The format of the redirecting number parameter field corresponds to the format shown in Figure 45.

The following codes are used in the subfields of the redirecting number parameter field:

a)	Odd/even indicator (octet 1)

See 7.15 a).

b)	Nature of address indicator (octet 1)

See 7.15 b).

c)	Numbering plan indicator (octet 2)

See 7.15 d).

d)	Address presentation restricted indicator (octet 2)

See 7.43 d).

e)	Address signal (octets 3 to n)

See 7.18 g).

f)	Filler (octet n)

See 7.15 f).

## 7.49	Redirection informationXE "Redirection information"§

The format of the redirection information parameter field is shown in Figure 45.

FIGURE 45/Q.2763...[D43] = 3 CM  (118%)

NOTE – Octet 2 is omitted if the redirection counter is coded 001.

The following codes are used in the redirection information parameter field:

a)	Original redirection reasonXE "Original redirection reason"§ (octet 1)

0000	Unknown/not available.

0001	User busy (national use).

0010	No reply (national use).

0011	Unconditional (national use).

b)	Redirecting indicatorXE "Redirecting indicator"§ (octet 1)

000	No redirection (national use).

001	Call re-routed (national use).

010	Call re-routed, all redirection information presentation restricted (national use).

011	Call diversion.

100	Call diversion, all redirection information presentation restricted.

101	Call re-routed, redirection number presentation restricted (national use).

110	Call diversion, redirection number presentation restricted.

111	Spare.

c)	Redirecting reasonXE "Redirecting reason"§ (octet 2)

0000	Unknown/not available.

0001	User busy.

0010	No reply.

0011	Unconditional.

0100	Deflection during alerting.

0101	Deflection immediate response.

0110	Mobile subscriber not reachable.

d)	Redirection counterXE "Redirection counter"§ (octet 2)

Number of redirections the call has undergone expressed as a binary number between 1 and 5.

## 7.50	Redirection numberXE "Redirection number"§

The format of the redirection number parameter field corresponds to the format shown in Figure 18.

The following codes are used in the subfields of the redirection number parameter field:

a)	Odd/even indicatorXE "Odd/even indicator"§ (octet 1)

See 7.15 a).

b)	Nature of address indicatorXE "Nature of address indicator"§ (octet 1)

See 7.15 b).

c)	Internal network number indicatorXE "Internal network number indicator"§ (octet 2)

See 7.15 c).

d)	Numbering plan indicatorXE "Numbering plan indicator"§ (octet 2)

See 7.15 d).

e)	Address signalXE "Address signal"§ (octets 3  to n)

See 7.18 g).

f)	FillerXE "Filler"§ (octet n)

See 7.15 f).

## 7.51	Redirection number restrictionXE "Redirection number restriction"§

The format of the redirection number restriction parameter field is shown in Figure 46.

The following codes are used in the redirection number restriction parameter field:

a)	Presentation restricted indicator

00	Presentation allowed.

01	Presentation restricted.

10	Spare.

11	Spare.

FIGURE 46/Q.2763...[D44] = 3 CM  (118%)

## 7.52	Resource identifierXE "Resource identifier"§

The format of the resource identifier parameter field is shown in Figure 47.

FIGURE 47/Q.2763...[D45] = 3 CM  (118%)

The following codes are used in the subfields of the resource identifier parameter field:

a)	Extension indicatorXE "Extension indicator"§ (ext.) (octet 1)

0	Octet continues through the next octet (e.g. octet 1 to 1a).

1	Last octet.

b)	Resource indicatorXE "Resource indicator"§

000	Local signalling identifier.

001	Remote  signalling identifier.

010	Connection element identifier: VPCI/VCI.

011	Connection element identifier: VPCI.

c)	Resource valueXE "Resource value"§ (octets 2 to n)

A code representing the identifier of the resource to be reset.

NOTES

1	This value will be equal to a signalling identifier control ID value if the resource indicator is coded “000” or “001”.

2	This value will be equal to a VPCI/VCI value if the resource indicator is coded “010”.

3	This value will be equal to a VPCI value if the resource indicator is coded “011”.

4	The maximum length of the resource value is 4 octets.

## 7.53	Segmentation indicatorXE "Segmentation indicator"§ (national use)

The format of the segmentation indicator parameter field is shown in Figure 48.

FIGURE 48/Q.2763...[D46] = 3 CM  (118%)

The following codes are used in the subfields of the segmentation indicator parameter field:

a)	Simple segmentation indicatorXE "Simple segmentation indicator"§

0	No additional information will be sent.

1	Additional information will be sent in a segmentation message.

## 7.54	Subsequent numberXE "Subsequent number"§

The format of the subsequent number parameter field is shown in Figure 49.

The following codes are used in the subfields of the subsequent number parameter field:

a)	Odd/even indicator (octet 1)

See 7.15 a).

b)	Address signal (octets 2 to n)

See 7.15 e).

c)	Filler (octet n)

See 7.15 f).

## 7.55	Suspend/resume indicatorsXE "Suspend/resume indicators"§

The format of the suspend/resume indicators parameter field is shown in Figure 50.

FIGURE 49/Q.2763...[D47] = 3 CM  (118%)

FIGURE 50/Q.2763...[D48] = 3 CM  (118%)

The following codes are used in the suspend/resume indicators parameter field:

a)	Suspend/resume indicator

0	ISDN subscriber initiated.XE "ISDN subscriber initiated."§

1	Network initiatedXE "Network initiated"§.

## 7.56	Transit network selectionXE "Transit network selection"§ (national use)

The format of the transit network selection parameter field is shown in Figure 51.

The following codes are used in the subfields of the transit network selection parameter field:

a)	Odd/even indicator (octet 1)

0	Even number of digits.

1	Odd number of digits.

b)	Type of network identification (octet 1)

000	ITU-T (CCITT) – standardized identification.

010	National network identification.

Other	Spare.

c)	Network identification plan (octet 1)

i)	For ITU-T (CCITT) – standardized identification:

0000	Unknown.

0011	Public Data Network Identification Code (DNIC), Recommendation X.121.

0110	Public land Mobile Network Identification Code (MNIC), Recommendation E.212.

Other	Spare.

ii)	For national network identification:

This information is coded according to national specifications.

d)	Network identification (octets 2 to n)

This information is organized according to the network identification plan and the coding principle given in 7.15 e).

FIGURE 51/Q.2763...[D49] = 3 CM  (118%)

## 7.57	User-to-user indicatorsXE "User-to-user indicators"§

The format of the user-to-user indicators parameter field is shown in Figure 52.

The following codes are used in the user-to-user indicators parameter field:

1)	If type (bit 1) equals 0 (request):

a)	Network discard indicatorXE "Network discard indicator"§

Not used for request, set to “0”.

b)	Service 3

00	No information.

01	Spare.

10	Request, not essential.

11	Request, essential.

c)	Service 2

00	No information.

01	Spare.

10	Request, not essential.

11	Request, essential.

d)	Service 1

00	No information.

01	Spare.

10	Request, not essential.

11	Request, essential.

2)	If type (bit 1) equals 1 (response):

a)	Network discard indicator

0	No information.

1	User-to-user information discarded by the network.

b)	Service 3

00	No information.

01	Not provided.

10	Provided.

11	Spare.

c)	Service 2

00	No information.

01	Not provided.

10	Provided.

11	Spare.

d)	Service 1

00	No information.

01	Not provided.

10	Provided.

11	Spare.

FIGURE 52/Q.2763...[D50] = 3 CM  (118%)

## 7.58	User-to-user informationXE "User-to-user information"§

The format of the user-to-user information parameter field is shown in Figure 53.

FIGURE 53/Q.2763...[D51] = 3 CM  (118%)

The following codes are used in the subfields of the user-to-user information parameter field:

a)	Extension indicator (ext.)

0	Octet continues through the next octet (e.g. octet 1 to 1a).

1	Last octet.

b)	More data identifierXE "More data identifier"§ (octet 1)

0	No more data following.

1	More data following.

c)	Coding standardXE "Coding standard"§ (octet 2)

The codes to be used in the coding standard subfield are defined in the user-to-user information element in clause 1/Q.2957.

d)	User-to-user information (octets 3 to n)

The codes to be used in the user-to-user information subfield are defined in the user-to-user information element in clause 1/Q.2957.

# 8	B-ISDN User Part messages and codesXE "B-ISDN User Part messages and codes"§

In the following tables, the format and coding of B-ISDN user part messages is specified. For each message, a list of the relevant parameters is given and for each parameter:

–	a reference to where the formatting and coding of the parameter content is specified;

–	the length of the parameter. The value in the table includes the length, in octets, of the parameter name, length indicator, parameter compatibility information and parameter content. For variable length parameters the minimum and maximum length is indicated (see Note).

NOTE – The length given for Q.2931 information elements is only informative.

For each message type, the parameters need not be sent in the order specified in the tables.

The routing label field, the message type field, the message length field and the message compatibility field belong to the general message format and are not shown explicitly in Tables 3 to 25.

TABLE 3/Q.2763

Message type: Address completeXE "Address complete"§

| Parameter | Reference (subclause) | Length (octets) |
| --- | --- | --- |
| Access delivery information | 7.3 | 5-6 |
| Backward narrow-band interworking indicator | 7.8 | 5-6 |
| Call diversion information | 7.12 | 5-6 |
| Call diversion may occur | 7.13 | 5-6 |
| Called party’s indicators | 7.17 | 5-6 |
| Cause indicators | 7.21 | 6-? |
| Charge indicator | 7.22 | 5-6 |
| Destination signalling identifier | 7.29 | 8-9 |
| Echo control information | 7.30 | 5-6 |
| In-band information indicator | 7.32 | 5-6 |
| MLPP user information | 7.36 | 5-6 |
| Narrow-band bearer capability | 7.37 | 11-? |
| Narrow-band high layer compatibility | 7.38 | 11-? |
| Notification (Note) | 7.41 | 5-6 |
| Progress indicator | 7.46 | 11-? |
| Redirection number | 7.50 | 7-15 |
| Redirection number restriction | 7.51 | 5-6 |
| Segmentation indicator (national use) | 7.53 | 5-6 |
| User-to-user indicators | 7.57 | 5-6 |
| User-to-user information | 7.58 | 7-136 |
| NOTE – This parameter may be repeated. |  |  |

TABLE 4/Q.2763

Message type: AnswerXE "Answer"§

| Parameter | Reference (subclause) | Length (octets) |
| --- | --- | --- |
| AAL parameters | 7.2 | ?-22 |
| Access delivery information | 7.3 | 5-6 |
| Additional connected number | 7.5 | 6-15 |
| Backward narrow-band interworking indicator | 7.8 | 5-6 |
| Broadband low layer information | 7.11 | 10-? |
| Call history information | 7.14 | 6-7 |
| Charge indicator | 7.22 | 5-6 |
| Connected number | 7.25 | 6-15 |
| Connected sub-address | 7.26 | 7-27 |
| Destination signalling identifier | 7.29 | 8-9 |
| Echo control information | 7.30 | 5-6 |
| In-band information indicator | 7.32 | 5-6 |
| Narrow-band bearer capability | 7.37 | 11-? |
| Narrow-band high layer compatibility | 7.38 | 11-? |
| Narrow-band low layer compatibility | 7.39 | 11-? |
| Notification (Note) | 7.41 | 5-6 |
| OAM traffic descriptor | 7.42 | 6-7 |
| Progress indicator | 7.46 | 11-? |
| Redirection number restriction | 7.51 | 5-6 |
| Segmentation indicator (national use) | 7.53 | 5-6 |
| User-to-user indicators | 7.57 | 5-6 |
| User-to-user information | 7.58 | 7-136 |
| NOTE – This parameter may be repeated. |  |  |

TABLE 5/Q.2763

Message type: Call progressXE "Call progress"§

| Parameter | Reference (subclause) | Length (octets) |
| --- | --- | --- |
| Access delivery information | 7.3 | 5-6 |
| Backward narrow-band interworking indicator | 7.8 | 5-6 |
| Call diversion information | 7.12 | 5-6 |
| Call diversion may occur | 7.13 | 5-6 |
| Called party’s indicators | 7.17 | 5-6 |
| Cause indicators | 7.21 | 6-? |
| Charge indicator | 7.22 | 5-6 |
| Destination signalling identifier | 7.29 | 8-9 |
| In-band information indicator | 7.32 | 5-6 |
| Narrow-band bearer capability | 7.37 | 11-? |
| Narrow-band high layer compatibility | 7.38 | 11-? |
| Notification (Note) | 7.41 | 5-6 |
| Progress indicator | 7.46 | 11-? |
| Redirection number | 7.50 | 7-15 |
| Redirection number restriction | 7.51 | 5-6 |
| Segmentation indicator (national use) | 7.53 | 5-6 |
| User-to-user indicators | 7.57 | 5-6 |
| User-to-user information | 7.58 | 7-136 |
| NOTE – This parameter may be repeated. |  |  |

TABLE 6/Q.2763

Message type: ConfusionXE "Confusion"§

| Parameter | Reference (subclause) | Length (octets) |
| --- | --- | --- |
| Cause indicators | 7.21 | 7-? |
| Destination signalling identifier | 7.29 | 8-9 |

TABLE 7/Q.2763

Message type: Initial addressXE "Initial address"§

| Parameter | Reference (subclause) | Length (octets) |
| --- | --- | --- |
| AAL parameters | 7.2 | ?-22 |
| Additional calling party number | 7.4 | 6-15 |
| ATM cell rate | 7.6 | 8-21 |
| Broadband bearer capability | 7.9 | 7-11 |
| Broadband high layer information | 7.10 | ?-17 |
| Broadband low layer information | 7.11 | 10-? |
| Called party number | 7.15 | 7-15 |
| Called party sub-address | 7.16 | 7-27 |
| Calling party number | 7.18 | 6-15 |
| Calling party sub-address | 7.19 | 7-27 |
| Calling party’s category | 7.20 | 5-6 |
| Closed user group information | 7.23 | 9-10 |
| Connected line ID request | 7.24 | 5-6 |
| Connection element identifier | 7.27 | 8-9 |
| Echo control information | 7.30 | 5-6 |
| Forward narrow-band interworking indicator | 7.31 | 5-6 |
| Location number | 7.33 | 7-15 |
| Maximum end-to-end transit delay | 7.34 | 6-7 |
| MLPP precedence | 7.35 | 10-11 |
| Narrow-band bearer capability | 7.37 | 11-? |
| Narrow-band high layer compatibility | 7.38 | 11-? |
| Narrow-band low layer compatibility | 7.39 | 11-? |
| National/international call indicator | 7.40 | 5-6 |
| Notification (Note) | 7.41 | 5-6 |
| OAM traffic descriptor | 7.42 | 6-7 |
| Original called number | 7.43 | 6-15 |
| Origination ISC point code | 7.44 | 6-7 |
| Origination signalling identifier | 7.45 | 8-9 |
| Progress indicator | 7.46 | 11-? |
| Propagation delay counter | 7.47 | 6-7 |
| Redirecting number | 7.48 | 6-15 |
| Redirection information | 7.49 | 5-7 |
| Segmentation indicator (national use) | 7.53 | 5-6 |
| Transit network selection (national use) | 7.56 | 6-? |
| User-to-user indicators | 7.57 | 5-6 |
| User-to-user information | 7.58 | 7-136 |
| NOTE – This parameter may be repeated. |  |  |

TABLE 8/Q.2763

Message type: ReleaseXE "Release"§

| Parameter | Reference (subclause) | Length (octets) |
| --- | --- | --- |
| Access delivery | 7.3 | 5-6 |
| Automatic congestion level | 7.7 | 5-6 |
| Cause indicators | 7.21 | 6-? |
| Destination signalling identifier | 7.29 | 8-9 |
| Notification (Note) | 7.41 | 5-6 |
| Progress indicator | 7.46 | 11-? |
| Redirection information (national use) | 7.49 | 5-7 |
| Redirection number (national use) | 7.50 | 7-15 |
| Redirection number restriction (national use) | 7.51 | 5-6 |
| Segmentation indicator (national use) | 7.53 | 5-6 |
| User-to-user indicators | 7.57 | 5-6 |
| User-to-user information | 7.58 | 7-136 |
| NOTE – This parameter may be repeated. |  |  |

TABLE 9/Q.2763

Message type: Release completeXE "Release complete"§

| Parameter | Reference (subclause) | Length (octets) |
| --- | --- | --- |
| Cause indicators | 7.21 | 7-? |
| Destination signalling identifier | 7.29 | 8-9 |

TABLE 10/Q.2763

Message type: Subsequent addressXE "Subsequent address"§

| Parameter | Reference (subclause) | Length (octets) |
| --- | --- | --- |
| Destination signalling identifier | 7.29 | 8-9 |
| Subsequent number | 7.54 | 6-14 |

TABLE 11/Q.2763

Message type: User-to-user informationXE "User-to-user information"§

| Parameter | Reference (subclause) | Length (octets) |
| --- | --- | --- |
| Destination signalling identifier | 7.29 | 8-9 |
| User-to-user indicators | 7.57 | 5-6 |
| User-to-user information | 7.58 | 7-136 |

TABLE 12/Q.2763

Message type: Forward transferXE "Forward transfer"§

| Parameter | Reference (subclause) | Length (octets) |
| --- | --- | --- |
| Destination signalling identifier | 7.29 | 8-9 |

TABLE 13/Q.2763

Message type: Suspend resumeXE "Suspend resume"§

| Parameter | Reference (subclause) | Length (octets) |
| --- | --- | --- |
| Destination signalling identifier | 7.29 | 8-9 |
| Suspend/Resume indicators | 7.55 | 5-6 |

TABLE 14/Q.2763

Message type: Blocking reset unblockingXE "Blocking reset unblocking"§

| Parameter | Reference (subclause) | Length (octets) |
| --- | --- | --- |
| Origination signalling identifier | 7.45 | 8-9 |
| Resource identifier | 7.52 | 7-10 |

TABLE 15/Q.2763

Message type: Blocking acknowledgementXE "Blocking acknowledgement"§
Reset acknowledgementXE "Reset acknowledgement"§
Unblocking acknowledgementXE "Unblocking acknowledgement"§

| Parameter | Reference (subclause) | Length (octets) |
| --- | --- | --- |
| Destination signalling identifier | 7.29 | 8-9 |

TABLE 16/Q.2763

Message type: User part testXE "User part test"§

| Parameter | Reference (subclause) | Length (octets) |
| --- | --- | --- |
| Origination signalling identifier | 7.45 | 8-9 |

TABLE 17/Q.2763

Message type: User part availableXE "User part available"§

| Parameter | Reference (subclause) | Length (octets) |
| --- | --- | --- |
| Destination signalling identifier | 7.45 | 8-9 |

TABLE 18/Q.2763

Message type: Network resource managementXE " Network resource management"§

| Parameter | Reference (subclause) | Length (octets) |
| --- | --- | --- |
| Destination signalling identifier | 7.29 | 8-9 |
| Echo control information | 7.30 | 5-6 |

TABLE 19/Q.2763

Message type: SegmentationXE "Segmentation"§ (national use)

| Parameter | Reference (subclause) | Length (octets) |
| --- | --- | --- |
| Additional calling party number | 7.4 | 6-15 |
| Additional connected number | 7.5 | 6-15 |
| Broadband high layer information | 7.10 | ?-17 |
| Broadband low layer information | 7.11 | 10-? |
| Called party sub-address | 7.16 | 7-27 |
| Calling party sub-address | 7.19 | 7-27 |
| Connected sub-address | 7.26 | 7-27 |
| Destination signalling identifier | 7.29 | 8-9 |
| Narrow-band high layer compatibility | 7.38 | 11-? |
| Narrow-band low layer compatibility | 7.39 | 11-? |
| Notification (Note) | 7.41 | 5-6 |
| Progress indicator | 7.46 | 11-? |
| User-to-user information | 7.58 | 7-136 |
| NOTE – This parameter may be repeated. |  |  |

TABLE 20/Q.2763

Message type: IAM acknowledgementXE "IAM acknowledgement"§

| Parameter | Reference (subclause) | Length (octets) |
| --- | --- | --- |
| Connection element identifier | 7.27 | 8-9 |
| Destination signalling identifier | 7.29 | 8-9 |
| Origination signalling identifier | 7.45 | 8-9 |

TABLE 21/Q.2763

Message type: IAM rejectXE "IAM reject"§

| Parameter | Reference (subclause) | Length (octets) |
| --- | --- | --- |
| Automatic congestion level | 7.7 | 5-6 |
| Cause indicators | 7.21 | 6-? |
| Destination signalling identifier | 7.29 | 8-9 |

TABLE 22/Q.2763

Message type: Consistency check request

| Parameter | Reference (subclause) | Length (octets) |
| --- | --- | --- |
| Origination signalling identifier | 7.45 | 8-9 |
| Resource identifier | 7.52 | 6-10 |

TABLE 23/Q.2763

Message type: Consistency check request acknowledgementXE "Consistency check request acknowledgement"§

| Parameter | Reference (subclause) | Length (octets) |
| --- | --- | --- |
| Destination signalling identifier | 7.29 | 8-9 |
| Origination signalling identifier | 7.45 | 8-9 |

TABLE 24/Q.2763

Message type: Consistency check endXE "Consistency check end"§

| Parameter | Reference (subclause) | Length (octets) |
| --- | --- | --- |
| Destination signalling identifier | 7.29 | 8-9 |

TABLE 25/Q.2763

Message type: Consistency check end acknowledgementXE "Consistency check end acknowledgement"§

| Parameter | Reference (subclause) | Length (octets) |
| --- | --- | --- |
| Destination signalling identifier | 7.29 | 8-9 |
| Consistency check result information | 7.28 | 5-6 |

Appendix  I

B-ISUP Messages and Parameters in ASN.1XE "B-ISUP Messages and Parameters in ASN.1"§

(This appendix does not form an integral part of this Recommendation)

## I.1	General

This appendix describes the B-ISDN User Part formats and codes in the Abstract Syntax Notation No.1 (ASN.1) as described in Recommendation X.208.

The description given in this appendix is only for informative purposes; the leading notation of the B-ISDN formats and codes is the tabular form as described in clauses 5 and 8.

The encoding rules to be used for the encoding of this ASN.1 description are given by the tabular form. The Basic Encoding Rules (BER) as described in Recommendation X.209 do not apply.

B-ISUPMessages	{...}  DEFINITIONS IMPLICIT TAGS

::=  BEGIN

IMPORTS	MessageInstructionIndicators,

ParameterInstructionIndicators

FROM B-ISUPInformationElementHeaders {...}

AALParameters,

AccessDeliveryInformation,

AdditionalCallingPartyNumber,

AdditionalConnectedNumber,

ATMCellRate,

AutomaticCongestionLevel,

BackwardNarrowbandIWInd,

BroadbandBearerCapability,

BroadbandHighLayerInfo,

BroadbandLowLayerInfo,

CallDiversionInformation,

CallDiversionMayOccur,

CalledPartyNumber,

CalledPartysIndicators,

CalledPartySubaddress,

CallHistoryInformation,

CallingPartyNumber,

CallingPartysCategory,

CallingPartySubaddress,

CauseIndicators,

ChargeInd,

ClosedUserGroupInformation,

ConnectedLineIDRequest,

ConnectedNumber,

ConnectedSubaddress,

ConnectionElementIdentifier,

ConsistencyCheckResultInfo,

DestinationSignallingID,

EchoControlInformation,

ForwardNarrowbandIWInd,

InBandInformationInd,

LocationNumber,

MaxEndToEndTransitDelay,

MLPPPrecedence,

MLPPUserInformation,

NarrowbandBearerCapability,

NarrowbandHighLayerCompa,

NarrowbandLowLayerCompa,

NationalInternationalCallInd,

Notification,

OAMTrafficDescriptor,

OriginalCalledNumber,

OriginationISCPointCode,

OriginationSignallingID,

ProgressIndicator,

PropagationDelayCounter,

RedirectingNumber,

RedirectionInformation,

RedirectionNumber,

RedirectionNumberRestriction,

ResourceID,

SegmentationIndicator,

SubsequentNumber,

SuspendResumeIndicators,

TransitNetworkSelection,

UserToUserIndicators,

UserToUserInformation

FROM B-ISUPParameters {...}

;

B-ISUPMessage ::= CHOICE	--	B-ISUP message type definitions

{

acm	[6]	ACM,	--	address complete

anm	[9]	ANM,	--	answer

bla	[21]	BLA,	--	blocking acknowledgement

blo	[19]	BLO,	--	blocking

cce	[23]	CCE,	--	consistency check end

ccea	[24]	CCEA,	--	consistency check end acknowledgement

ccr	[5]	CR,	--	consistency check request

ccra	[17]	CCRA,	--	consistency check request acknowledgement

cfn	[47]	CFN,	--	confusion

cpg	[44]	CPG,	--	call progress

fot	[8]	FOT,	--	forward transfer

iaa	[10]	IAA,	--	initial address acknowledgement

iam	[1]	IAM,	--	initial address

iar	[11]	IAR,	--	initial address reject

nrm	[50]	NRM,	--	network resource management

ram	[15]	RAM,	--	reset acknowledgement

rel	[12]	REL,	--	release

res	[14]	RES,	--	resume

rlc	[16]	RLC,	--	release complete

rsm	[18]	RSM,	--	reset

sam	[2]	SAM,	--	subsequent address

sgm	[56]	SGM,	--	segmentation (for national use)

sus	[13]	SUS,	--	suspend

uba	[22]	UBA,	--	unblocking acknowledgement

ubl	[20]	UBL,	--	unblocking

upa	[53]	UPA,	--	user part available

upt	[52]	UPT,	--	user part test

usr	[45]	USR	--	user-to-user information

}

ACM ::= SEQUENCE

{

messageInstructionIndicators	MessageInstructionIndicators,

aCM-MessageContent	ACM-MessageContent

}

ACM-MessageContent ::= SET

{

accessDeliveryInformation	[46]	AccessDeliveryInformation  OPTIONAL,

backwardNarrowbandIWInd	[20]	BackwardNarrowbandIWInd  OPTIONAL,

callDiversionInformation	[54]	CallDiversionInformation  OPTIONAL,

callDiversionMayOccur	[38]	CallDiversionMayOccur  OPTIONAL,

calledPartysIndicators	[23]	CalledPartysIndicators  OPTIONAL,

causeIndicators	[18]	CauseIndicators  OPTIONAL,

chargeInd	[25]	ChargeInd  OPTIONAL,

destinationSignallingID	[3]	DestinationSignallingID  OPTIONAL,

echoControlInformation	[55]	EchoControlInformation  OPTIONAL,

inBandInformationInd	[31]	InBandInformationInd  OPTIONAL,

mLPPUserInformation	[73]	MLPPUserInformation  OPTIONAL,

narrowbandBearerCapability	[29]	NarrowbandBearerCapability  OPTIONAL,

narrowbandHighLayerCompa	[52]	NarrowbandHighLayerCompa  OPTIONAL,

notification	[44]	SEQUENCE OF Notification  OPTIONAL,

progressIndicator	[53]	ProgressIndicator  OPTIONAL,

redirectionNumber	[12]	RedirectionNumber  OPTIONAL,

redirectionNumberRestriction	[64]	RedirectionNumberRestriction  OPTIONAL,

segmentationIndicator	[58]	SegmentationIndicator  OPTIONAL,

--	for national use

userToUserIndicators	[42]	UserToUserIndicators  OPTIONAL,

userToUserInformation	[32]	UserToUserInformation  OPTIONAL

}

ANM ::= SEQUENCE

{

messageInstructionIndicators	MessageInstructionIndicators,

aNM-MessageContent	ANM-MessageContent

}

ANM-MessageContent ::= SET

{

aALParameters	[71]	AALParameters  OPTIONAL,

accessDeliveryInformation	[46]	AccessDeliveryInformation  OPTIONAL,

additionalConnectedNumber	[17]	AdditionalConnectedNumber  OPTIONAL,

backwardNarrowbandIWInd	[20]	BackwardNarrowbandIWInd  OPTIONAL,

broadbandLowLayerInfo	[68]	BroadbandLowLayerInfo  OPTIONAL,

callHistoryInformation	[45]	CallHistoryInformation  OPTIONAL,

chargeInd	[25]	ChargeInd  OPTIONAL,

connectedNumber	[33]	ConnectedNumber  OPTIONAL,

connectedSubaddress	[36]	ConnectedSubaddress  OPTIONAL,

destinationSignallingID	[3]	DestinationSignallingID  OPTIONAL,

echoControlInformation	[55]	EchoControlInformation  OPTIONAL,

inBandInformationInd	[31]	InBandInformationInd  OPTIONAL,

narrowbandBearerCapability	[29]	NarrowbandBearerCapability  OPTIONAL,

narrowbandHighLayerCompa	[52]	NarrowbandHighLayerCompa  OPTIONAL,

narrowbandLowLayerCompa	[37]	NarrowbandLowLayerCompa  OPTIONAL,

notification	[44]	SEQUENCE OF Notification  OPTIONAL,

oAMTrafficDescriptor	[72]	OAMTrafficDescriptor  OPTIONAL,

progressIndicator	[53]	ProgressIndicator  OPTIONAL,

redirectionNumberRestriction	[64]	RedirectionNumberRestriction  OPTIONAL,

segmentationIndicator	[58]	SegmentationIndicator  OPTIONAL,

--	for national use

userToUserIndicators	[42]	UserToUserIndicators  OPTIONAL,

userToUserInformation	[32]	UserToUserInformation  OPTIONAL

}

BLA ::= SEQUENCE

{

messageInstructionIndicators	MessageInstructionIndicators,

bLA-MessageContent	BLA-MessageContent

}

BLA-MessageContent ::= SET

{

destinationSignallingID	[3]	DestinationSignallingID  OPTIONAL

}

BLO ::= SEQUENCE

{

messageInstructionIndicators	MessageInstructionIndicators,

bLO-MessageContent	BLO-MessageContent

}

BLO-MessageContent ::= SET

{

originationSignallingID	[2]	OriginationSignallingID  OPTIONAL,

resourceID	[49]	ResourceID  OPTIONAL

}

CCE ::= SEQUENCE

{

messageInstructionIndicators	MessageInstructionIndicators,

cCE-MessageContent	CCE-MessageContent

}

CCE-MessageContent ::= SET

{

destinationSignallingID	[3]	DestinationSignallingID  OPTIONAL

}

CCEA ::= SEQUENCE

{

messageInstructionIndicators	MessageInstructionIndicators,

cCEA-MessageContent	CCEA-MessageContent

}

CCEA-MessageContent ::= SET

{

destinationSignallingID	[3]	DestinationSignallingID  OPTIONAL,

consistencyCheckResultInfo	[74]	ConsistencyCheckResultInfo  OPTIONAL

}

CCR ::= SEQUENCE

{

messageInstructionIndicators	MessageInstructionIndicators,

cCR-MessageContent	CCR-MessageContent

}

CCR-MessageContent ::= SET

{

originationSignallingID	[2]	OriginationSignallingID  OPTIONAL,

resourceID	[49]	ResourceID  OPTIONAL

}

CCRA ::= SEQUENCE

{

messageInstructionIndicators	MessageInstructionIndicators,

cCRA-MessageContent	CCRA-MessageContent

}

CCRA-MessageContent ::= SET

{

destinationSignallingID	[3]	DestinationSignallingID  OPTIONAL,

originationSignallingID	[2]	OriginationSignallingID  OPTIONAL

}

CFN ::= SEQUENCE

{

messageInstructionIndicators	MessageInstructionIndicators,

cFN-MessageContent	CFN-MessageContent

}

CFN-MessageContent ::= SET

{

causeIndicators	[18]	CauseIndicators  OPTIONAL,

destinationSignallingID	[3]	DestinationSignallingID  OPTIONAL

}

CPG ::= SEQUENCE

{

messageInstructionIndicators	MessageInstructionIndicators,

cPG-MessageContent	CPG-MessageContent

}

CPG-MessageContent ::= SET

{

accessDeliveryInformation	[46]	AccessDeliveryInformation  OPTIONAL,

backwardNarrowbandIWInd	[20]	BackwardNarrowbandIWInd  OPTIONAL,

callDiversionInformation	[54]	CallDiversionInformation  OPTIONAL,

callDiversionMayOccur	[38]	CallDiversionMayOccur  OPTIONAL,

calledPartysIndicators	[23]	CalledPartysIndicators  OPTIONAL,

causeIndicators	[18]	CauseIndicators  OPTIONAL,

chargeInd	[25]	ChargeInd  OPTIONAL,

destinationSignallingID	[3]	DestinationSignallingID  OPTIONAL,

inBandInformationInd	[31]	InBandInformationInd  OPTIONAL,

narrowbandBearerCapability	[29]	NarrowbandBearerCapability  OPTIONAL,

narrowbandHighLayerCompa	[52]	NarrowbandHighLayerCompa  OPTIONAL,

notification	[44]	SEQUENCE OF Notification  OPTIONAL,

progressIndicator	[53]	ProgressIndicator  OPTIONAL,

redirectionNumber	[12]	RedirectionNumber  OPTIONAL,

redirectionNumberRestriction	[64]	RedirectionNumberRestriction  OPTIONAL,

segmentationIndicator	[58]	SegmentationIndicator  OPTIONAL,

--	for national use

userToUserIndicators	[42]	UserToUserIndicators  OPTIONAL,

userToUserInformation	[32]	UserToUserInformation  OPTIONAL

}

FOT ::= SEQUENCE

{

messageInstructionIndicators	MessageInstructionIndicators,

fOT-MessageContent	FOT-MessageContent

}

FOT-MessageContent ::= SET

{

destinationSignallingID	[3]	DestinationSignallingID  OPTIONAL

}

IAA ::= SEQUENCE

{

messageInstructionIndicators	MessageInstructionIndicators,

iAA-MessageContent	IAA-MessageContent

}

IAA-MessageContent ::= SET

{

connectionElementIdentifier	[6]	ConnectionElementIdentifier  OPTIONAL,

destinationSignallingID	[3]	DestinationSignallingID  OPTIONAL,

originationSignallingID	[2]	OriginationSignallingID  OPTIONAL

}

IAM ::= SEQUENCE

{

messageInstructionIndicators	MessageInstructionIndicators,

iAM-MessageContent	IAM-MessageContent

}

IAM-MessageContent ::= SET

{

aALParameters	[71]	AALParameters  OPTIONAL,

additionalCallingPartyNumber	[16]	AdditionalCallingPartyNumber  OPTIONAL,

aTMCellRate	[8]	ATMCellRate  OPTIONAL,

broadbandBearerCapability	[67]	BroadbandBearerCapability  OPTIONAL,

broadbandHighLayerInfo	[72]	BroadbandHighLayerInfo  OPTIONAL,

broadbandLowLayerInfo	[68]	BroadbandLowLayerInfo  OPTIONAL,

calledPartyNumber	[4]	CalledPartyNumber  OPTIONAL,

calledPartySubaddress	[21]	CalledPartySubaddress  OPTIONAL,

callingPartyNumber	[10]	CallingPartyNumber  OPTIONAL,

callingPartySubaddress	[22]	CallingPartySubaddress  OPTIONAL,

callingPartysCategory	[9]	CallingPartysCategory  OPTIONAL,

closedUserGroupInformation	[26]	ClosedUserGroupInformation  OPTIONAL,

connectedLineIDRequest	[27]	ConnectedLineIDRequest  OPTIONAL,

connectionElementIdentifier	[6]	ConnectionElementIdentifier  OPTIONAL,

echoControlInformation	[55]	EchoControlInformation  OPTIONAL,

forwardNarrowbandIWInd	[28]	ForwardNarrowbandIWInd  OPTIONAL,

locationNumber	[63]	LocationNumber  OPTIONAL,

maxEndToEndTransitDelay	[7]	MaxEndToEndTransitDelay  OPTIONAL,

mLPPPrecedence	[48]	MLPPPrecedence  OPTIONAL,

narrowbandBearerCapability	[29]	NarrowbandBearerCapability  OPTIONAL,

narrowbandHighLayerCompa	[52]	NarrowbandHighLayerCompa  OPTIONAL,

narrowbandLowLayerCompa	[22]	NarrowbandLowLayerCompa  OPTIONAL,

nationalInternationalCallInd	[41]	NationalInternationalCallInd  OPTIONAL,

notification	[44]	SEQUENCE OF Notification  OPTIONAL,

oAMTrafficDescriptor	[72]	OAMTrafficDescriptor  OPTIONAL,

originalCalledNumber	[40]	OriginalCalledNumber  OPTIONAL,

originationISCPointCode	[43]	OriginationISCPointCode  OPTIONAL,

originationSignallingID	[2]	OriginationSignallingID  OPTIONAL,

progressIndicator	[53]	ProgressIndicator  OPTIONAL,

propagationDelayCounter	[46]	PropagationDelayCounter  OPTIONAL,

redirectingNumber	[31]	RedirectingNumber  OPTIONAL,

redirectionInformation	[19]	RedirectionInformation  OPTIONAL,

segmentationIndicator	[58]	SegmentationIndicator  OPTIONAL,

--	for national use

transitNetworkSelection	[35]	TransitNetworkSelection  OPTIONAL,

--	for national use

userToUserIndicators	[42]	UserToUserIndicators  OPTIONAL,

userToUserInformation	[32]	UserToUserInformation  OPTIONAL

}

IAR ::= SEQUENCE

{

messageInstructionIndicators	MessageInstructionIndicators,

iAR-MessageContent	IAR-MessageContent

}

IAR-MessageContent ::= SET

{

automaticCongestionLevel	[39]	AutomaticCongestionLevel  OPTIONAL,

causeIndicators	[18]	CauseIndicators  OPTIONAL,

destinationSignallingID	[3]	DestinationSignallingID  OPTIONAL

}

NRM ::= SEQUENCE

{

messageInstructionIndicators	MessageInstructionIndicators,

nRM-MessageContent	NRM-MessageContent

}

NRM-MessageContent ::= SET

{

destinationSignallingID	[3]	DestinationSignallingID  OPTIONAL,

echoControlInformation	[55]	EchoControlInformation  OPTIONAL

}

RAM ::= SEQUENCE

{

messageInstructionIndicators	MessageInstructionIndicators,

rAM-MessageContent	RAM-MessageContent

}

RAM-MessageContent ::= SET

{

destinationSignallingID	[3]	DestinationSignallingID  OPTIONAL

}

REL ::= SEQUENCE

{

messageInstructionIndicators	MessageInstructionIndicators,

rEL-MessageContent	REL-MessageContent

}

REL-MessageContent ::= SET

{

accessDeliveryInformation	[46]	AccessDeliveryInformation  OPTIONAL,

automaticCongestionLevel	[39]	AutomaticCongestionLevel  OPTIONAL,

causeIndicators	[18]	CauseIndicators  OPTIONAL,

destinationSignallingID	[3]	DestinationSignallingID  OPTIONAL,

notification	[44]	SEQUENCE OF Notification  OPTIONAL,

progressIndicator	[53]	ProgressIndicator  OPTIONAL,

redirectionInformation	[19]	RedirectionInformation  OPTIONAL,

--	for national use

redirectionNumber	[12]	RedirectionNumber  OPTIONAL,

--	for national use

redirectionNumberRestriction	[64]	RedirectionNumberRestriction  OPTIONAL,

--	for national use

segmentationIndicator	[58]	SegmentationIndicator  OPTIONAL,

--	for national use

userToUserIndicators	[42]	UserToUserIndicators  OPTIONAL,

userToUserInformation	[32]	UserToUserInformation  OPTIONAL

}

RES ::= SEQUENCE

{

messageInstructionIndicators	MessageInstructionIndicators,

rES-MessageContent	RES-MessageContent

}

RES-MessageContent ::= SET

{

destinationSignallingID	[3]	DestinationSignallingID  OPTIONAL,

suspendResumeIndicators	[34]	SuspendResumeIndicators  OPTIONAL

}

RLC ::= SEQUENCE

{

messageInstructionIndicators	MessageInstructionIndicators,

rLC-MessageContent	RLC-MessageContent

}

RLC-MessageContent ::= SET

{

causeIndicators	[18]	CauseIndicators  OPTIONAL,

destinationSignallingID	[3]	DestinationSignallingID  OPTIONAL

}

RSM ::= SEQUENCE

{

messageInstructionIndicators	MessageInstructionIndicators,

rSM-MessageContent	RSM-MessageContent

}

RSM-MessageContent ::= SET

{

originationSignallingID	[2]	OriginationSignallingID  OPTIONAL,

resourceID	[49]	ResourceID  OPTIONAL

}

SAM ::= SEQUENCE

{

messageInstructionIndicators	MessageInstructionIndicators,

sAM-MessageContent	SAM-MessageContent

}

SAM-MessageContent ::= SET

{

destinationSignallingID	[3]	DestinationSignallingID  OPTIONAL,

subsequentNumber	[5]	SubsequentNumber  OPTIONAL

}

SGM ::= SEQUENCE	--	for national use

{

messageInstructionIndicators	MessageInstructionIndicators,

sGM-MessageContent	SGM-MessageContent

}

SGM-MessageContent ::= SET

{

additionalCallingPartyNumber	[16]	AdditionalCallingPartyNumber  OPTIONAL,

additionalConnectedNumber	[17]	AdditionalConnectedNumber  OPTIONAL,

broadbandHighLayerInfo	[72]	BroadbandHighLayerInfo  OPTIONAL,

broadbandLowLayerInfo 	[68]	BroadbandLowLayerInfo  OPTIONAL,

calledPartySubaddress	[21]	CalledPartySubaddress  OPTIONAL,

callingPartySubaddress	[22]	CallingPartySubaddress  OPTIONAL,

connectedSubaddress	[36]	ConnectedSubaddress  OPTIONAL,

destinationSignallingID	[3]	DestinationSignallingID  OPTIONAL,

narrowbandHighLayerCompa	[52]	NarrowbandHighLayerCompa  OPTIONAL,

narrowbandLowLayerCompa	[10]	NarrowbandLowLayerCompa OPTIONAL,

notification	[44]	SEQUENCE OF Notification  OPTIONAL,

progressIndicator	[53]	ProgressIndicator  OPTIONAL,

userToUserInformation	[32]	UserToUserInformation  OPTIONAL

}

SUS ::= SEQUENCE

{

messageInstructionIndicators	MessageInstructionIndicators,

sUS-MessageContent	SUS-MessageContent

}

SUS-MessageContent ::= SET

{

destinationSignallingID	[3]	DestinationSignallingID  OPTIONAL,

suspendResumeIndicators	[34]	SuspendResumeIndicators  OPTIONAL

}

UBA ::= SEQUENCE

{

messageInstructionIndicators	MessageInstructionIndicators,

uBA-MessageContent	UBA-MessageContent

}

UBA-MessageContent ::= SET

{

destinationSignallingID	[3]	DestinationSignallingID  OPTIONAL

}

UBL ::= SEQUENCE

{

messageInstructionIndicators	MessageInstructionIndicators,

uBL-MessageContent	UBL-MessageContent

}

UBL-MessageContent ::= SET

{

originationSignallingID	[2]	OriginationSignallingID  OPTIONAL,

resourceID	[49]	ResourceID  OPTIONAL

}

UPA ::= SEQUENCE

{

messageInstructionIndicators	MessageInstructionIndicators,

uPA-MessageContent	UPA-MessageContent

}

UPA-MessageContent ::= SET

{

destinationSignallingID	[3]	DestinationSignallingID  OPTIONAL

}

UPT ::= SEQUENCE

{

messageInstructionIndicators 	MessageInstructionIndicators,

uPT-MessageContent	UPT-MessageContent

}

UPT-MessageContent ::= SET

{

originationSignallingID	[2]	OriginationSignallingID  OPTIONAL

}

USR ::= SEQUENCE

{

messageInstructionIndicators	MessageInstructionIndicators,

uSR-MessageContent	USR-MessageContent

}

USR-MessageContent ::= SET

{

destinationSignallingID	[3]	DestinationSignallingID  OPTIONAL,

userToUserIndicators	[42]	UserToUserIndicators  OPTIONAL,

userToUserInformation	[32]	UserToUserInformation  OPTIONAL

}

END

B-ISUPParameters {...}  DEFINITIONS  IMPLICIT TAGS

::=  BEGIN

Recommendation Q.2763     (02/95)

IMPORTS	MessageInstructionIndicators,

ParameterInstructionIndicators,

FROM B-ISUPInformationElementHeaders {...}

ATMCellRateField,

CalledPartyAddress,

CallingPartyAddress,

ClosedUserGroupInterlockCode,

ConnectedAddress,

ConnectionElementID,

LocationAddress,

NetworkIdentification,

NetworkIdentificationPlan,

NetworkIdentity,

ResourceValue,

SubsequentAddress,

UserToUserIndicatorsParameter

FROM B-ISUPParameterConstructions {...}

AALParametersField,

AddressSignal,

BinaryCode,

BroadbandBearerCapabilityField,

BroadbandHighLayerInformationField,

BroadbandLowLayerInformationField,

CalledAddressSignal,

CalledPartySubaddressField,

CallingAddressSignal,

CallingPartySubaddressField,

CellRateID,

ConnectedSubaddressField,

ControlID,

DelayValue,

MLPPServiceDomain,

NarrowbandBearerCapabilityField,

NarrowbandHighLayerCompaField,

NarrowbandLowLayerCompaField,

NIdigit,

OAMTrafficDescriptorField,

ProgressIndicatorField,

RedirectionCounter,

SignallingPointCode,

UserToUserInformationField,

VirtualChannelID,

VirtualPathConnectionID

FROM B-ISUPParameterElements {...}

AccessDeliveryIndicator,

AddressPresentationIndicator,

AutomaticCongestionLevelInd,

CallDiversionMayOccurIndicator,

CalledPartysCategoryInd,

CalledPartysStatusInd,

CallingPartysCategoryIndicators,

CCITTNetworkIdenPlanIndicator,

CellRateIdentifier,

ChargeIndicator,

ClosedUserGroupCallIndicator,

ConnectedLineIDRequestIndicator,

InBandInformationIndicator

IncomingHECDIndicator,

IncomingHECDRequestIndicator,

InternalNetworkNumberIndicator,

InterworkingIndicator,

ISDNAccessIndicator,

ISDNUserPartIndicator,

ISDNUserPartPreferenceInd,

LookAheadForBusyIndicator,

MLPPUserInformation,

MoreDataIndicator,

NationalNetworkIdenPlanIndicator,

NationalInternationalCallIndicator,

NatureOfAddressIndicator,

NetworkDiscardIndicator,

NotificationIndicators,

NotificationSubscriptionOption,

NumberIncompleteIndicator,

NumberingPlanIndicator,

OriginalRedirectionReasonIndicator,

OutgoingHECDIndicator,

OutgoingHECDRequestIndicator,

PrecedenceLevelIndicator,

Priority,

RedirectingIndicator,

RedirectingReasonIndicator,

RedirectionNumberRestrictionInd,

RepeatIndicator,

ResourceIndicator,

ScreeningIndicator,

SimpleSegmentationIndicator,

SuspendResumeIndicator,

Type,

TypeOfNetworkIdentification,

UserToUserServiceRequest,

UserToUserServiceResponse,

VPCICheckResultIndicator

FROM B-ISUPParameterIndicators {...}

Location,

CodingStandard,

CauseValue,

Diagnostics

FROM RecommendationQ.2610 {...}

;

ParameterType ::= CHOICE	--	B-ISUP parameter type definitions

{

aALParameters	[71]	AALParameters,

accessDeliveryInformation	[46]	AccessDeliveryInformation,

additionalCallingPartyNumber 	[16]	AdditionalCallingPartyNumber,

additionalConnectedNumber 	[17]	AdditionalConnectedNumber,

aTMCellRate	[8]	ATMCellRate,

automaticCongestionLevel	[39]	AutomaticCongestionLevel,

backwardNarrowbandIWInd	[20]	BackwardNarrowbandIWInd,

broadbandBearerCapability	[80]	BroadbandBearerCapability,

broadbandHighLayerInfo	[72]	BroadbandHighLayerInfo,

broadbandLowLayerInfo	[79]	BroadbandLowLayerInfo,

callDiversionInformation	[54]	CallDiversionInformation,

callDiversionMayOccur	[38]	CallDiversionMayOccur,

calledPartyNumber	[4]	CalledPartyNumber,

calledPartysIndicators	[23]	CalledPartysIndicators,

calledPartySubaddress	[21]	CalledPartySubaddress,

callHistoryInformation	[45]	CallHistoryInformation,

callingPartyNumber	[10]	CallingPartyNumber,

callingPartysCategory	[9]	CallingPartysCategory,

callingPartySubaddress	[22]	CallingPartySubaddress,

causeIndicators	[18]	CauseIndicators,

chargeInd	[25]	ChargeInd,

closedUserGroupInformation	[26]	ClosedUserGroupInformation,

connectedLineIDRequest	[27]	ConnectedLineIDRequest,

connectedNumber	[33]	ConnectedNumber,

connectedSubaddress	[36]	ConnectedSubaddress,

connectionElementIdentifier	[6]	ConnectionElementIdentifier,

consistencyCheckResultInfo	[74]	ConsistencyCheckResultInfo,

destinationSignallingID	[3]	DestinationSignallingID,

echoControlInformation	[55]	EchoControlInformation,

forwardNarrowbandIWInd	[28]	ForwardNarrowbandIWInd,

inBandInformationInd	[31]	InBandInformationInd,

locationNumber	[63]	LocationNumber,

maxEndToEndTransitDelay	[7]	MaxEndToEndTransitDelay,

mLPPPrecedence	[48]	MLPPPrecedence,

mLPPUserInformation	[73]	MLPPUserInformation,

narrowbandBearerCapability	[29]	NarrowbandBearerCapability,

narrowbandHighLayerCompa	[52]	NarrowbandHighLayerCompa,

narrowbandLowLayerCompa	[37]	NarrowbandLowLayerCompa,

nationalInternationalCallInd	[41]	NationalInternationalCallInd,

notification	[44]	Notification,

oAMTrafficDescriptor	[72]	OAMTrafficDescriptor,

originalCalledNumber	[40]	OriginalCalledNumber,

originationISCPointCode	[43]	OriginationISCPointCode,

originationSignallingID	[2]	OriginationSignallingID,

progressIndicator	[53]	ProgressIndicator,

propagationDelayCounter	[46]	PropagationDelayCounter,

redirectingNumber	[11]	RedirectingNumber,

redirectionInformation	[19]	RedirectionInformation,

redirectionNumber	[12]	RedirectionNumber,

redirectionNumberRestriction	[64]	RedirectionNumberRestriction,

resourceID	[49]	ResourceID,

segmentationIndicator	[58]	SegmentationIndicator,

--	for national use

subsequentNumber	[5]	SubsequentNumber,

suspendResumeIndicators	[34]	SuspendResumeIndicators,

transitNetworkSelection 	[35]	TransitNetworkSelection,

--	for national use

userToUserIndicators	[42]	UserToUserIndicators,

userToUserInformation	[32]	UserToUserInformation

}

AALParameters ::= SEQUENCE

{

parameterInstructionIndicators	ParameterInstructionIndicators,

codingStandard	CodingStandard,

aALParametersField	AALParametersField

}

AccessDeliveryInformation ::= SEQUENCE

{

parameterInstructionIndicators	ParameterInstructionIndicators,

accessDeliveryIndicator	AccessDeliveryIndicator

}

AdditionalCallingPartyNumber ::= SEQUENCE

{

parameterInstructionIndicators	ParameterInstructionIndicators,

natureOfAddressIndicator	NatureOfAddressIndicator,

screeningIndicator	ScreeningIndicator,

addressPresentationIndicator	AddressPresentationIndicator,

numberingPlanIndicator	NumberingPlanIndicator,

callingPartyAddress	[1]	CallingPartyAddress  OPTIONAL

}

AdditionalConnectedNumber ::= SEQUENCE

{

parameterInstructionIndicators	ParameterInstructionIndicators,

natureOfAddressIndicator	NatureOfAddressIndicator,

screeningIndicator	ScreeningIndicator,

addressPresentationIndicator	AddressPresentationIndicator,

numberingPlanIndicator	NumberingPlanIndicator,

connectedAddress	[1]	ConnectedAddress  OPTIONAL

}

ATMCellRate ::= SEQUENCE

{

parameterInstructionIndicators	ParameterInstructionIndicators,

aTMCellRateField	ATMCellRateField,

aTMCellRateField	[1]	ATMCellRateField  OPTIONAL

}

AutomaticCongestionLevel ::= SEQUENCE

{

parameterInstructionIndicators	ParameterInstructionIndicators,

automaticCongestionLevelInd	AutomaticCongestionLevelInd

}

BackwardNarrowbandIWInd ::= SEQUENCE

{

parameterInstructionIndicators	ParameterInstructionIndicators,

interworkingIndicator	InterworkingIndicator,

iSDNUserPartIndicator	ISDNUserPartIndicator,

iSDNAccessIndicator	ISDNAccessIndicator

}

BroadbandBearerCapability ::= SEQUENCE

{

parameterInstructionIndicators	ParameterInstructionIndicators,

codingStandard	CodingStandard,

broadbandBearerCapabilityField	BroadbandBearerCapabilityField

}

BroadbandHighLayerInfo ::= SEQUENCE

{

parameterInstructionIndicators	ParameterInstructionIndicators,

codingStandard	CodingStandard,

broadbandHighLayerInformationField	BroadbandHighLayerInformationField

}

BroadbandLowLayerInfo ::= SEQUENCE

{

parameterInstructionIndicators	ParameterInstructionIndicators,

priority	Priority,

repeatIndicator	RepeatIndicator,

broadbandLowLayerInformationField	SEQUENCE OF

BroadbandLowLayerInformationField

}

CallDiversionInformation ::= SEQUENCE

{

parameterInstructionIndicators	ParameterInstructionIndicators,

notificationSubscriptionOption	NotificationSubscriptionOption,

redirectionReasonIndicator	RedirectingReasonIndicator

}

CallDiversionMayOccur ::= SEQUENCE

{

parameterInstructionIndicators	ParameterInstructionIndicators,

callDiversionMayOccurIndicator	CallDiversionMayOccurIndicator

}

CalledPartyNumber ::= SEQUENCE

{

parameterInstructionIndicators	ParameterInstructionIndicators,

natureOfAddressIndicator	NatureOfAddressIndicator,

numberingPlanIndicator	NumberingPlanIndicator,

internalNetworkNumberIndicator	InternalNetworkNumberIndicator,

calledPartyAddress	CalledPartyAddress

}

CalledPartysIndicators ::= SEQUENCE

{

parameterInstructionIndicators	ParameterInstructionIndicators,

calledPartysStatusInd	CalledPartysStatusInd,

calledPartysCategoryInd	CalledPartysCategoryInd

}

CalledPartySubaddress ::= SEQUENCE

{

parameterInstructionIndicators	ParameterInstructionIndicators,

codingStandard	CodingStandard,

calledPartySubaddressField	CalledPartySubaddressField

}

CallHistoryInformation ::= SEQUENCE

{

parameterInstructionIndicators	ParameterInstructionIndicators,

delayValue	DelayValue

}

CallingPartyNumber ::= SEQUENCE

{

parameterInstructionIndicators	ParameterInstructionIndicators,

natureOfAddressIndicator	NatureOfAddressIndicator,

screeningIndicator	ScreeningIndicator,

addressPresentationIndicator	AddressPresentationIndicator,

numberingPlanIndicator	NumberingPlanIndicator,

numberIncompleteIndicator	NumberIncompleteIndicator,

callingPartyAddress	[1]	CallingPartyAddress  OPTIONAL

}

CallingPartysCategory ::= SEQUENCE

{

parameterInstructionIndicators	ParameterInstructionIndicators,

callingPartysCategoryIndicators	CallingPartysCategoryIndicators

}

CallingPartySubaddress ::= SEQUENCE

{

parameterInstructionIndicators	ParameterInstructionIndicators,

codingStandard	CodingStandard,

callingPartySubaddressField	CallingPartySubaddressField

}

CauseIndicators ::= SEQUENCE

{

parameterInstructionIndicators	ParameterInstructionIndicators,

location	Location,

codingStandard	CodingStandard,

causeValue	CauseValue,

diagnostics	[1]	Diagnostics  OPTIONAL

}

ChargeInd ::= SEQUENCE

{

parameterInstructionIndicators	ParameterInstructionIndicators,

chargeIndicator	ChargeIndicator

}

ClosedUserGroupInformation ::= SEQUENCE

{

parameterInstructionIndicators	ParameterInstructionIndicators,

closedUserGroupCallIndicator	ClosedUserGroupCallIndicator,

closedUserGroupInterlockCode	ClosedUserGroupInterlockCode

}

ConnectedLineIDRequest ::= SEQUENCE

{

parameterInstructionIndicators	ParameterInstructionIndicators,

connectedLineIDRequestIndicator 	ConnectedLineIDRequestIndicator

}

ConnectedNumber ::= SEQUENCE

{

parameterInstructionIndicators	ParameterInstructionIndicators,

natureOfAddressIndicator	NatureOfAddressIndicator,

screeningIndicator	ScreeningIndicator,

addressPresentationIndicator	AddressPresentationIndicator,

numberingPlanIndicator	NumberingPlanIndicator,

connectedAddress	[1]	ConnectedAddress  OPTIONAL

}

ConnectedSubaddress ::= SEQUENCE

{

parameterInstructionIndicators	ParameterInstructionIndicators,

codingStandard	CodingStandard,

connectedSubaddressField	ConnectedSubaddressField

}

ConnectionElementIdentifier ::=  SEQUENCE

{

parameterInstructionIndicators	ParameterInstructionIndicators,

connectionElementID	ConnectionElementID

}

ConsistencyCheckResultInfo ::=  SEQUENCE

{

parameterInstructionIndicators	ParameterInstructionIndicators,

vPCICheckResultIndicator	VPCICheckResultIndicator

}

DestinationSignallingID ::=  SEQUENCE

{

parameterInstructionIndicators	ParameterInstructionIndicators,

controlID	ControlID

}

EchoControlInformation ::=  SEQUENCE

{

parameterInstructionIndicators	ParameterInstructionIndicators,

outgoingHECDIndicator	OutgoingHECDIndicator,

incomingHECDIndicator	IncomingHECDIndicator,

outgoingHECDRequestIndicator	OutgoingHECDRequestIndicator,

incomingHECDRequestIndicator	IncomingHECDRequestIndicator

}

ForwardNarrowbandIWInd ::=  SEQUENCE

{

parameterInstructionIndicators	ParameterInstructionIndicators,

interworkingIndicator	InterworkingIndicator,

iSDNUserPartIndicator	ISDNUserPartIndicator,

iSDNUserPartPreferenceInd	ISDNUserPartPreferenceInd,

iSDNAccessIndicator	ISDNAccessIndicator

}

InBandInformationInd ::=  SEQUENCE

{

parameterInstructionIndicators	ParameterInstructionIndicators,

inBandInformationterworkingIndicator	InBandInformationterworkingIndicator

}

LocationNumber ::= SEQUENCE

{

parameterInstructionIndicators	ParameterInstructionIndicators,

natureOfAddressIndicator	NatureOfAddressIndicator,

screeningIndicator	ScreeningIndicator,

addressPresentationIndicator	AddressPresentationIndicator,

numberingPlanIndicator	NumberingPlanIndicator,

internalNetworkNumberIndicator	InternalNetworkNumberIndicator,

locationAddress	[1]	LocationAddress  OPTIONAL

}

MaxEndToEndTransitDelay ::= SEQUENCE

{

parameterInstructionIndicators	ParameterInstructionIndicators,

delayValue	DelayValue

}

MLPPPrecedence ::= SEQUENCE

{

parameterInstructionIndicators	ParameterInstructionIndicators,

precedenceLevelIndicator	PrecedenceLevelIndicator,

lookAheadForBusyIndicator	LookAheadForBusyIndicator,

networkIdentity	NetworkIdentity,

mLPPServiceDomain	MLPPServiceDomain

}

MLPPUserInformation ::= SEQUENCE

{

parameterInstructionIndicators	ParameterInstructionIndicators,

mLPPUserIndicator	MLPPUserIndicator

}

NarrowbandBearerCapability ::= SEQUENCE

{

parameterInstructionIndicators	ParameterInstructionIndicators,

priority	Priority,

repeatIndicator	RepeatIndicator,

narrowbandBearerCapabilityField	SEQUENCE OF

NarrowbandBearerCapabilityField

}

NarrowbandHighLayerCompa ::=  SEQUENCE

{

parameterInstructionIndicators	ParameterInstructionIndicators,

priority	Priority,

repeatIndicator	RepeatIndicator,

narrowbandHighLayerCompaField	SEQUENCE OF

NarrowbandHighLayerCompaField

}

NarrowbandLowLayerCompa ::=  SEQUENCE

{

parameterInstructionIndicators	ParameterInstructionIndicators,

priority	Priority,

repeatIndicator	RepeatIndicator,

narrowbandLowLayerCompaField	SEQUENCE OF

NarrowbandLowLayerCompaField

}

NationalInternationalCallInd ::=  SEQUENCE

{

parameterInstructionIndicators	ParameterInstructionIndicators,

nationalInternationalCallIndicator 	NationalInternationalCallIndicator

}

Notification ::= SEQUENCE

{

parameterInstructionIndicators	ParameterInstructionIndicators,

notificationIndicators	NotificationIndicators

}

OAMTrafficDescriptor ::= SEQUENCE

{

parameterInstructionIndicators	ParameterInstructionIndicators,

codingStandard	CodingStandard,

oAMTrafficDescriptorField	OAMTrafficDescriptorField

}

OriginalCalledNumber ::= SEQUENCE

{

parameterInstructionIndicators	ParameterInstructionIndicators,

natureOfAddressIndicator	NatureOfAddressIndicator,

addressPresentationIndicator	AddressPresentationIndicator,

numberingPlanIndicator	NumberingPlanIndicator,

callingPartyAddress	[1]	CallingPartyAddress  OPTIONAL

}

OriginationISCPointCode ::= SEQUENCE

{

parameterInstructionIndicators	ParameterInstructionIndicators,

signallingPointCode	SignallingPointCode

}

OriginationSignallingID ::= SEQUENCE

{

parameterInstructionIndicators	ParameterInstructionIndicators,

controlID	ControlID

}

ProgressIndicator ::= SEQUENCE

{

parameterInstructionIndicators	ParameterInstructionIndicators,

priority	Priority,

repeatIndicator	RepeatIndicator,

progressIndicatorField	SEQUENCE OF

ProgressIndicatorField

}

PropagationDelayCounter ::= SEQUENCE

{

parameterInstructionIndicators	ParameterInstructionIndicators,

delayValue	DelayValue

}

RedirectingNumber ::= SEQUENCE

{

parameterInstructionIndicators	ParameterInstructionIndicators,

natureOfAddressIndicator	NatureOfAddressIndicator,

addressPresentationIndicator	AddressPresentationIndicator,

numberingPlanIndicator	NumberingPlanIndicator,

callingPartyAddress	[1]	CallingPartyAddress  OPTIONAL

}

RedirectionInformation ::= SEQUENCE

{

parameterInstructionIndicators	ParameterInstructionIndicators,

redirectingIndicator	RedirectingIndicator,

originalRedirectionReasonIndicator	OriginalRedirectionReasonIndicator,

redirectionCounter	RedirectionCounter,

redirectingReasonIndicator	RedirectingReasonIndicator

}

RedirectionNumber ::= SEQUENCE

{

parameterInstructionIndicators	ParameterInstructionIndicators,

natureOfAddressIndicator	NatureOfAddressIndicator,

numberingPlanIndicator	NumberingPlanIndicator,

internalNetworkNumberIndicator	InternalNetworkNumberIndicator,

callingPartyAddress	[1]	CallingPartyAddress  OPTIONAL

}

RedirectionNumberRestriction ::= SEQUENCE

{

parameterInstructionIndicators	ParameterInstructionIndicators,

redirectionNumberRestrictionInd	RedirectionNumberRestrictionInd

}

ResourceID ::= SEQUENCE

{

parameterInstructionIndicators	ParameterInstructionIndicators,

resourceIndicator	ResourceIndicator,

resourceValue	ResourceValue

}

SegmentationIndicator ::= SEQUENCE	--	for national use

{

parameterInstructionIndicators	ParameterInstructionIndicators,

simpleSegmentationIndicator	SimpleSegmentationIndicator

}

SubsequentNumber ::= SEQUENCE

{

parameterInstructionIndicators	ParameterInstructionIndicators,

subsequentAddress	SubsequentAddress

}

SuspendResumeIndicators ::= SEQUENCE

{

parameterInstructionIndicators	ParameterInstructionIndicators,

suspendResumeIndicator	SuspendResumeIndicator

}

TransitNetworkSelection ::= SEQUENCE	--	for national use

{

parameterInstructionIndicators	ParameterInstructionIndicators,

networkIdentificationPlan	NetworkIdentificationPlan,

typesOfNetworkIdentification	TypesOfNetworkIdentification,

networkIdentification	NetworkIdentification

}

UserToUserIndicators ::= SEQUENCE

{

parameterInstructionIndicators	ParameterInstructionIndicators,

userToUserIndicatorsParameter	UserToUserIndicatorsParameter

}

UserToUserInformation ::= SEQUENCE

{

parameterInstructionIndicators	ParameterInstructionIndicators,

moreDataIndicator	MoreDataIndicator,

userToUserInformationField	UserToUserInformationField

}

END

B-ISUPInformationElementHeaders {...}  DEFINITIONS  IMPLICIT TAGS

::=  BEGIN

MessageInstructionIndicators ::= SEQUENCE

{

sharedCompatibilityInformation	SharedCompatibilityInformation,

passOnNotPossibleMessageInd	PassOnNotPossibleMessageInd,

broadbandNarrowbandMessageIWInd	BroadbandNarrowbandMessageIWInd

}

ParameterInstructionIndicators ::= SEQUENCE

{

sharedCompatibilityInformation	SharedCompatibilityInformation,

discardParameterIndicator	DiscardParameterIndicator,

passOnNotPossibleParameterInd	PassOnNotPossibleParameterInd,

broadbandNarrowbandParameterIWInd	BroadbandNarrowbandParameterIWInd

}

SharedCompatibilityInformation ::= SEQUENCE

{

transitAtIntermediateExchangeInd	TransitAtIntermediateExchangeInd,

releaseCallIndicator	ReleaseCallIndicator,

sendNotificationIndicator	SendNotificationIndicator,

discardMessageIndicator	DiscardMessageIndicator

}

TransitAtIntermediateExchangeInd ::= ENUMERATED

{

transitInterpretation(0),

endNodeInterpretation(1)

}

ReleaseCallIndicator ::= ENUMERATED

{

doNotReleaseCall(0),

releaseCall(1)

}

SendNotificationIndicator ::= ENUMERATED

{

doNotSendNotification(0),

sendNotification(1)

}

DiscardMessageIndicator ::= ENUMERATED

{

doNotDiscardMessage(0),

discardMessage(1)

}

PassOnNotPossibleMessageInd ::= ENUMERATED

{

releaseCall(0),

discardMessage(1)

}

BroadbandNarrowbandMessageIWInd ::= ENUMERATED

{

passOn(0),

discardMessage(1),

releaseCall(2)

--  value 3 is reserved

}

DiscardParameterIndicator ::= ENUMERATED

{

doNotDiscardParameter(0),

discardParameter(1)

}

PassOnNotPossibleParameterInd ::= ENUMERATED

{

releaseCall(0),

discardMessage(1),

discardParameter(2)

--  value 3 is reserved

}

BroadbandNarrowbandParameterIWInd ::= ENUMERATED

{

passOn(0),

discardMessage(1),

releaseCall(2),

discardParameter(3)

}

END

B-ISUPParameterConstructions {...}  DEFINITIONS  IMPLICIT TAGS

::=  BEGIN

IMPORTS	AALParametersField,

AddressSignal,

BinaryCode,

BroadbandBearerCapabilityField,

BroadbandHighLayerInformationField,

BroadbandLowLayerInformationField,

CalledAddressSignal,

CalledPartySubaddressField,

CallingAddressSignal,

CallingPartySubaddressField,

CellRateID,

ConnectedSubaddressField,

ControlID,

DelayValue,

MLPPServiceDomain,

NarrowbandBearerCapabilityField,

NarrowbandHighLayerCompaField,

NarrowbandLowLayerCompaField,

NIdigit,

OAMTrafficDescriptorField,

ProgressIndicatorField,

RedirectionCounter,

SignallingPointCode,

UserToUserInformationField,

VirtualChannelID,

VirtualPathConnectionID

FROM B-ISUPParameterElements {...}

AccessDeliveryIndicator,

AddressPresentationIndicator,

AutomaticCongestionLevelInd,

CallDiversionMayOccurIndicator,

CalledPartysCategoryInd,

CalledPartysStatusInd,

CallingPartysCategoryIndicators,

CCITTNetworkIdenPlanIndicator,

CellRateIdentifier,

ChargeIndicator,

ClosedUserGroupCallIndicator,

ConnectedLineIDRequestIndicator,

InBandInformationIndicator

IncomingHECDIndicator,

IncomingHECDRequestIndicator,

InternalNetworkNumberIndicator,

InterworkingIndicator,

ISDNAccessIndicator,

ISDNUserPartIndicator,

ISDNUserPartPreferenceInd,

LookAheadForBusyIndicator,

MLPPUserInformation,

MoreDataIndicator,

NationalNetworkIdenPlanIndicator,

NationalInternationalCallIndicator,

NatureOfAddressIndicator,

NetworkDiscardIndicator,

NotificationIndicators,

NotificationSubscriptionOption,

NumberIncompleteIndicator,

NumberingPlanIndicator,

OriginalRedirectionReasonIndicator,

OutgoingHECDIndicator,

OutgoingHECDRequestIndicator,

PrecedenceLevelIndicator,

Priority,

RedirectingIndicator,

RedirectingReasonIndicator,

RedirectionNumberRestrictionInd,

RepeatIndicator,

ResourceIndicator,

ScreeningIndicator,

SimpleSegmentationIndicator,

SuspendResumeIndicator,

Type,

TypeOfNetworkIdentification,

UserToUserServiceRequest,

UserToUserServiceResponse,

VPCICheckResultIndicator

FROM B-ISUPParameterIndicators {...}

;

ATMCellRateField ::=  SEQUENCE

{

cellRateIdentifier 	CellRateIdentifier,

cellRateID	CellRateID

}

CalledPartyAddress ::= SEQUENCE SIZE (0..16) OF CalledAddressSignal

CallingPartyAddress ::= SEQUENCE SIZE (0..16) OF CallingAddressSignal

ClosedUserGroupInterlockCode ::= SEQUENCE

{

networkIdentity	NetworkIdentity,

binaryCode	BinaryCode

}

ConnectedAddress ::= SEQUENCE SIZE (0..16) OF CallingAddressSignal

ConnectionElementID ::= SEQUENCE

{

virtualPathConnectionID	VirtualPathConnectionID,

virtualChannelID	[1]	VirtualChannelID  OPTIONAL

}

LocationAddress ::= SEQUENCE SIZE (0..16) OF AddressSignal

NetworkIdentification ::= SEQUENCE SIZE (0..16) OF CalledAddressSignal

NetworkIdentificationPlan ::= CHOICE

{

cCITTNetworkIdenPlanIndicator	CCITTNetworkIdenPlanIndicator,

nationalNetworkIdenPlanIndicator	NationalNetworkIdenPlanIndicator

}

NetworkIdentity ::= SEQUENCE SIZE (4) OF NIdigit

ResourceValue ::= CHOICE

{

controlID	ControlID,

connectionElementID	ConnectionElementID,

virtualPathConnectionID	VirtualPathConnectionID

}

SubsequentAddress ::= SEQUENCE SIZE (0..16) OF CalledAddressSignal

UserToUserIndicatorsParameter ::= CHOICE

{

userToUserRequest	[0]	UserToUserRequest,

userToUserResponse	[1]	UserToUserResponse

}

UserToUserRequest ::= SEQUENCE

{

service1	UserToUserServiceRequest,

service2	UserToUserServiceRequest,

service3	UserToUserServiceRequest

}

UserToUserResponse ::= SEQUENCE

{

service1	UserToUserServiceResponse,

service2	UserToUserServiceResponse,

service3	UserToUserServiceResponse,

userToUserNetworkDiscardIndicator	UserToUserNetworkDiscardIndicator

}

END

B-ISUPParameterElements {...}  DEFINITIONS  IMPLICIT TAGS

::=  BEGIN

AALParametersField  :: OCTET STRING (SIZE (?..13)) ...

--	coding as AAL parameters info element Q.2931 starting with octet 5

AddressSignal ::= ENUMERATED

{

digit0(0),

digit1(1),

digit2(2),

digit3(3),

digit4(4),

digit5(5),

digit6(6),

digit7(7),

digit8(8),

digit9(9),

sTsignal(15)

}

BinaryCode ::= INTEGER (0..65535)	--	maximum value 216 – 1

BroadbandBearerCapabilityField ::= OCTET STRING (SIZE (2..11)) ...

--	coding as Broadband BC info element in Q.2931 starting with octet 5

BroadbandHighLayerInformationField ::= OCTET STRING (SIZE (2..3)) ...

--	coding as Broadband HLC info element in Q.2931 starting with octet 5

BroadbandLowLayerInformationField ::= OCTET STRING (SIZE (2..14)) ...

--	coding as Broadband LLC info element in Q.2931 including the header

CalledAddressSignal ::= ENUMERATED

{

digit0(0),

digit1(1),

digit2(2),

digit3(3),

digit4(4),

digit5(5),

digit6(6),

digit7(7),

digit8(8),

digit9(9),

code11(11),

code12(12),

sTsignal(15)

}

CalledPartySubaddressField ::= OCTET STRING (SIZE (1..20)) ...

--	coding as SUB info element in Q.2931 starting with octet 5

CallingAddressSignal ::= ENUMERATED

{

digit(0),

digit(1),

digit(2),

digit(3),

digit(4),

digit(5),

digit(6),

digit(7),

digit(8),

digit(9),

code11(11),

code12(12)

}

CallingPartySubaddressField ::= OCTET STRING (SIZE (1..20)) ...

--	coding as SUB info element in Q.2931 starting with octet 5

CellRateID ::= INTEGER (0..16777215)	--	maximum value 224 – 1

ConnectedSubaddressField ::= OCTET STRING (SIZE (1..20)) ...

--	coding as SUB info element in Q.2951 starting with octet 5

ControlID ::= INTEGER (0..4294967295)	--	maximum value 232 – 1

MLPPServiceDomain ::= INTEGER (0..16777215)	--	maximum value 224 – 1

NarrowbandBearerCapabilityField ::= OCTET STRING (SIZE (2..11)) ...

--	coding as Narrowband BC info element in Q.2931 including the header

NarrowbandHighLayerCompaField ::= OCTET STRING (SIZE (2..3)) ...

--	coding as Narrowband HLC info element in Q.2931 including the header

NarrowbandLowLayerCompaField ::= OCTET STRING (SIZE (2..14)) ...

--	coding as Narrowband LLC info element in Q.2931 including the header

NIdigit ::=  INTEGER (0..9)

OAMTrafficDescriptorField  :: OCTET STRING (SIZE (2..10)) ...

--	coding as OAM traffic descriptor IE in Q.2931 starting with octet 5

ProgressIndicatorField ::= OCTET STRING (SIZE (2)) ...

--	coding as PROGRESS info element in Q.2931 including the header

DelayValue ::= INTEGER (0..65535)	--	maximum value 216 – 1

RedirectionCounter ::= INTEGER (0..5)

SignallingPointCode ::= INTEGER (0..4095)	--	maximum value 212 – 1

UserToUserInformationField ::= OCTET STRING (SIZE (1..128)) ...

--	coding as UUI info element in clause 1/Q.2957 starting with octet 5

VirtualChannelID ::= INTEGER (1..65535)	--	maximum value 216 – 1

VirtualPathConnectionID ::= INTEGER (1..65535)	--	maximum value 216 – 1

END

B-ISUPParameterIndicators {...}  DEFINITIONS  IMPLICIT TAGS

::=  BEGIN

AccessDeliveryIndicator ::= ENUMERATED

{

setUpMessageGenerated(0),

noSetUpMessageGenerated(1)

}

AddressPresentationIndicator ::= ENUMERATED

{

presentationAllowed(0),

presentationRestricted(1),

addressNotAvailable(2)	--	for national use

}

AutomaticCongestionLevelInd ::= ENUMERATED

{

congestionLevel1exceeded(1),

congestionLevel2exceeded(2)

}

CallDiversionMayOccurIndicator ::= ENUMERATED

{

noIndication(0),

callDiversionMayOccur(1)

}

CalledPartysCategoryInd ::= ENUMERATED

{

noIndication(0),

ordinarySubscriber(1),

payphone(2)

}

CalledPartysStatusInd ::= ENUMERATED

{

noIndication(0),

alerting(1),

}

CallingPartysCategoryIndicators ::= ENUMERATED

{

callingPartysCategoryUnknownAtThisMoment(0),	--	for national use

operatorLanguageFrench(1),

operatorLanguageEnglish(2),

operatorLanguageGerman(3),

operatorLanguageRussian(4),

operatorLanguageSpanish(5),

nationalOperator(9),	--	for national use

ordinaryCallingSubscriber(10),

callingSubscriberWithPriority(11),

dataCall(12),

testCall(13),

payphone(15)

--	values 6..8, 14, 224..254 are reserved

}

CCITTNetworkIdenPlanIndicator ::= ENUMERATED

{

unknown(0),

publicDataNetworkIdentificationCodeX.121(1),

publicLandMobileNetworkIdentificationCodeE.212(2)

}

CellRateIdentifier ::= ENUMERATED

{

forwardPeakCellRateForCellLossPriority0(2),

backwardPeakCellRateForCellLossPriority0(3),

forwardPeakCellRateForCellLossPriority0and1(4),

backwardPeakCellRateForCellLossPriority0and1(5)

}

ChargeIndicator ::= ENUMERATED

{

noCharge(0),

charge(1)

}

ClosedUserGroupCallIndicator ::= ENUMERATED

{

cUGcallOutgoingAccessAllowed(0),

cUGcallOutgoingAccessNotAllowed(1)

}

ConnectedLineIDRequestIndicator ::= ENUMERATED

{

notRequested(0),

requested(1)

}

InBandInformationIndicator ::= ENUMERATED

{

noIndication(0),

inBandInfoOrPatternAvailable(1)

}

IncomingHECDIndicator ::= ENUMERATED

{

noInformation(0),

incomingHECDNotIncluded(1),

incomingHECDIncluded(2)

}

IncomingHECDRequestIndicator ::= ENUMERATED

{

noInformation(0),

incomingHECDActivationRequest(1),

incomingHECDDeactivationRequest(2)

}

InternalNetworkNumberIndicator ::= ENUMERATED

{

routingToInternalNetworkNumberAllowed(0),

routingToInternalNetworkNumberNotAllowed(1)

}

InterworkingIndicator ::= ENUMERATED

{

noInterworkingEncountered(0),

interworkingEncountered(1)

}

ISDNAccessIndicator ::= ENUMERATED

{

originatingOrTerminatingAccessNonISDN(0),

originatingOrTerminatingAccessISDN(1)

}

ISDNUserPartIndicator ::= ENUMERATED

{

iSDNUserPartNotUsedAllTheWay(0),

iSDNUserPartUsedAllTheWay(1)

}

ISDNUserPartPreferenceInd ::= ENUMERATED

{

iSDNUserPartPreferredAllTheWay(0),

iSDNUserPartNotRequiredAllTheWay(1),

iSDNUserPartRequiredAllTheWay(2)

}

LookAheadForBusyIndicator ::= ENUMERATED

{

lookAheadForBusyAllowed(0),

pathReserved(1),	--	for national use

lookAheadForBusyNotAllowed(2)

}

MLPPUserIndicator ::= ENUMERATED

{

noIndication(0),

mLPPUser(1)

}

MoreDataIndicator ::= ENUMERATED

{

noMoreData(0),

moreData(1)

}

NationalNetworkIdenPlanIndicator ::= ENUMERATED

{

...	--	for national use

}

NationalInternationalCallIndicator ::= ENUMERATED

{

callToBeTreatedAsNationalCall(0),

callToBeTreatedAsInternationalCall(1)

}

NatureOfAddressIndicator ::= ENUMERATED

{

subscriberNumber(1),	--	for national use

unknown(2),	--	for national use

nationalSignificantNumber(3),

internationalNumber(4)

--	values 112..126 are reserved

}

NetworkDiscardIndicator ::= ENUMERATED

{

noInformation(0),

userToUserInformationDiscardedByTheNetwork(1)

}

NotificationIndicators ::= ENUMERATED

{

userSuspended(0),	--	used in DSS 1

userResumed(1),	--	used in DSS 1

bearerServiceChanged(2),	--	used in DSS 1

discriminatorForExtensionToASN1(3),	--	used in DSS 1

callCompletionDelay(4),

conferenceEstablished(66),

conferenceDisconnected(67),

otherPartyAdded(68),

isolated(69),

reattached(70),

otherPartyIsolated(71),

otherPartyReattached(72),

otherPartySplit(73),

otherPartyDisconnected(74),

conferenceFloating(75),

callIsAWaitingCall(96),

diversionActivated(104),	--	used in DSS 1

callTransferAlerting(105),

callTransferActive(106),

remoteHold(121),

remoteRetrieval(122),

callIsDiverting(123)

--	all other values are reserved

}

NotificationSubscriptionOption ::= ENUMERATED

{

unknown(0),

presentationNotAllowed(1),

presentationAllowedWithRedirectionNumber(2),

presentationAllowedWithoutRedirectionNumber(3)

}

NumberIncompleteIndicator ::= ENUMERATED

{

complete(0),

incomplete(1)	--	for national use

}

NumberingPlanIndicator ::= ENUMERATED

{

iSDNTelephonyNumberingPlanE.164(1),

dataNumberingPlanX.121(3),	--	for national use

privateNumberingPlan(5)	--	depending on the type of number

--  values 3, 4 and 6 are reserved for national use

}

OriginalRedirectionReasonIndicator ::= ENUMERATED

{

unknownNotAvailable(0),

userBusy(1),	--	for national use

noReply(2),	--	for national use

unconditional(3)	--	for national use

}

OutgoingHECDIndicator ::= ENUMERATED

{

noInformation(0),

outgoingHECDNotIncluded(1),

outgoingHECDIncluded(2)

}

OutgoingHECDRequestIndicator ::= ENUMERATED

{

noInformation(0),

outgoingHECDActivationRequest(1),

outgoingHECDDeactivationRequest(2)

}

PrecedenceLevelIndicator ::= ENUMERATED

{

flashOverride(0),

flash(1),

immediate(2),

priority(3),

routine(4)

}

Priority ::= ENUMERATED

{

noPrioritizedOrder(0),

prioritizedListInAscendingOrder(1),

prioritizedListInDecendingOrder(2)

--	values 3..15 are reserved

}

RedirectingIndicator ::= ENUMERATED

{

noRedirection(0),	--	for national use

callRerouted(1),	--	for national use

callReroutedAllRedirectionInfoPresentationRestricted(2),	--	for national use

callDiversion(3),

callDiversionAllRedirectionInfoPresentationRestricted(4),

callReroutedRedirectionNumberPresentationRestricted(5),	--	for national use

callDiversionRedirectionNumberPresentationRestricted(6)

}

RedirectingReasonIndicator ::= ENUMERATED

{

unknownnotAvailable(0),

userBusy(1),

noReply(2),

unconditional(3),

deflectionDuringAlerting(4),

deflectionImmediateResponse(5),

mobileSubscriberNotReachable(6)

}

RedirectionNumberRestrictionInd ::= ENUMERATED

{

presentationAllowed(0),

presentationRestricted(1)

}

RepeatIndicator ::= ENUMERATED

{

informationElementNotRepeated(0),

informationElementRepeated(1)

}

ResourceIndicator ::= ENUMERATED

{

localSignallingID(0),

remoteSignallingID(1),

connectionElementIDVPCIVCI(2),

connectionElementIDVPCI(3)

}

ScreeningIndicator ::= ENUMERATED

{

userProvidedNotVerified(0),	--	for national use

userProvidedVerifiedAndPassed(1),

userProvidedVerifiedAndFailed(2),	--	for national use

networkProvided(3)

}

SimpleSegmentationIndicator ::= ENUMERATED	--	for national use

{

noAdditionalInformationWillBeSent(0),

additionalInformationWillBeSentInSegmentationMessage(1)

}

SuspendResumeIndicator ::= ENUMERATED

{

iSDNSubscriberInitiated(0),

networkInitiated(1)

}

Type ::= ENUMERATED

{

request(0),

response(1)

}

TypeOfNetworkIdentification ::= ENUMERATED

{

cCITTstandardizedIdentification(0),

nationalNetworkIdentification(1)

}

UserToUserServiceRequest ::= ENUMERATED

{

noInformation(0),

requestNotEssential(2)

--	value 3 is reserved

}

UserToUserServiceResponse ::= ENUMERATED

{

noInformation(0),

notProvided(1),

provided(2)

}

VPCICheckResultIndicator ::= ENUMERATED

{

virtualPathConnectionIdentifierCheckNotSuccessful(0),

virtualPathConnectionIdentifierCheckSuccessful(1),

virtualPathConnectionIdentifierCheckNotPerformed(2)

}

END