|  | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

ITU-T	R.4

TELECOMMUNICATION

STANDARDIZATION SECTOR

OF  ITU

TELEGRAPHY

TELEGRAPH  TRANSMISSION

METHODS  FOR  THE  SEPARATE  MEASUREMENTS  OF  THE  DEGREES  
OF  VARIOUS  TYPES  OF  TELEGRAPH  DISTORTION

ITU-T  Recommendation  R.4

(Extract from the Blue Book)

# NOTES

1	ITU-T Recommendation R.4 was published in Fascicle VII.1 of the Blue Book. This file is an extract from the Blue Book. While the presentation and layout of the text might be slightly different from the Blue Book version, the contents of the file are identical to the Blue Book version and copyright conditions remain unchanged (see below).

2	In this Recommendation, the expression “Administration” is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

ã  ITU  1988, 1993

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

Recommendation R.4

Fascicle VII.1 - Rec. R.4

METHODS FOR THE SEPARATE MEASUREMENTS OF THE DEGREES

OF VARIOUS TYPES OF TELEGRAPH DISTORTION

(New Delhi, 1960; amended at Geneva, 1980)

For separate measurements of the degrees of characteristic distortion, bias distortion and fortuitous distortion affecting a telegraph modulation or restitution, the following is recommended where circuits and voice-frequency telegraph (VFT) channels are used to carry information employing International Telegraph Alphabet No. 2, without regeneration;

1	Measure the degree of overall distortion (at the actual mean modulation rate) on text, for instance the QKS text specified in Recommendation R.51 bis. Let D be the measurement obtained.

2	Measure the degree of distortion on reversals at the modulation rate used in the measurement of § (1) above. Let D1 be the measurement obtained. D1 is the sum of the bias and fortuitous distortions.

3	By using a compensator fitted to the distortion-measuring equipment, for example a compensating winding on the distortion meter relay, reduce the degree of distortion reading obtained to its minimum value. Let this figure be d. For practical purposes d is the fortuitous distortion. D1 – d is, for practical purposes, the bias distortion.

4	Keep the distortion meter adjusted as for the measurement of d. Measure the degree of distortion at the actual mean modulation rate on text (QKS text, for instance). Let D¢ be the reading. D¢ – d is, for practical purposes, the characteristic distortion.

Note 1 – This method gives approximate results, it is possible that the equation D1 + D' – d = D may not be exactly satisfied.

Note 2 – The method can be applied by using either an isochronous distortion-measuring set or a start-stop distortion-measuring set.

Note 3 – The fact that the separate measurement of degrees of different types of distortion is said to be possible and that a method is recommended for such a measurement does not mean that separate measurements of the degrees of different types of distortion are to be recommended when international routine maintenance measurements are carried out.