|  | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

ITU-T	R.36

TELECOMMUNICATION

STANDARDIZATION SECTOR

OF  ITU

TELEGRAPHY

TELEGRAPH  TRANSMISSION

COEXISTENCE  OF  
50-BAUD/120-Hz  CHANNELS, 
100-BAUD/240-Hz  CHANNELS, 
200-BAUD/360-Hz  OR  480-Hz  CHANNELS
ON  THE  SAME  VOICE - FREQUENCY  TELEGRAPH  SYSTEM

ITU-T  Recommendation  R.36

(Extract from the Blue Book)

# NOTES

1	ITU-T Recommendation R.36 was published in Fascicle VII.1 of the Blue Book. This file is an extract from the Blue Book. While the presentation and layout of the text might be slightly different from the Blue Book version, the contents of the file are identical to the Blue Book version and copyright conditions remain unchanged (see below).

2	In this Recommendation, the expression “Administration” is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

ã  ITU  1988, 1993

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

Recommendation R.36

Fascicle VII.1 - Rec. R.36

COEXISTENCE OF 50-BAUD/120-Hz CHANNELS,

100-BAUD/240-Hz CHANNELS, 200-BAUD/360-Hz OR 480-Hz CHANNELS

ON THE SAME VOICE-FREQUENCY TELEGRAPH SYSTEM

(New Delhi. 1960; amended at Geneva, 1964 and 1980)

1	Common views

1.1	Channels with higher modulation rates (100 or 200 bauds) must be capable of being inserted in systems of amplitude-modulated 50-baud/120-Hz channels conforming to Recommendations concerning them respectively as well as in systems of frequency-modulated 50-baud/120-Hz channels (conforming to Recommendation R.35). However, it is preferable that these high-speed channels should, as far as possible, be placed in a frequency-modulated system (conforming to Recommendation R.35). However, 200-baud/360-Hz channels can be set up only on systems established on bearer circuits having a spacing of 3 kHz.

1.2	If there are 50-baud channels on a mixed system, the distortion limits for the 50-baud channels on homogeneous 50-baud channel systems will have to be respected; hence, 100-baud and 200-baud channel equipment will have to be designed to this end. If this is not possible, the power levels on the 100-baud and 200-baud channels will have to be reduced.

1.3	The 100- and 200-baud channels should have performances comparable to those that could be obtained in a homogeneous system, as specified in Recommendations R.37, R.38 A, R.38 B, provided that the condition indicated under § 1.2 above is respected. They should, in particular, satisfy § 13a) of Recommendations R.37, R.38 A, or R.38 B respectively.

1.4	The mean power transmitted to line at a point of zero relative level is normally dependent on the transmission characteristics of the bearer circuit as follows:

a)	50 µW total for FMVFT aggregates carried on circuits complying with the limits specified in Annex A to Recommendation R.35;

b)	135 µW total for other circuits and for AMVFT.

The mean normal power for each channel should not exceed the values specified in Table 1/R.36, for cases a) and b) above.

TABLE 1/R.36

VFT channel power levels

| VFT channel power level (µW) |  | Relevant | VFT channel characteristics |  |  |
| --- | --- | --- | --- | --- | --- |
| Bearer case a) | Bearer case b) | Recommendation | Modulation rate (bauds) | Bandwidth (Hz) | Type of modulation |
| – 2.0 4.0 a) – 8.0 a) | 9 5.6 10.8 a) 19.2 a) 21.6 a) | R.31 R.35 R.37 R.38B R.38A | 50 50 100 200 200 | 120 120 240 360 480 | AM FM FM FM FM |

a)	Provided that the condition mentioned under § 1.2 is respected.

2	Combined use of channels with 240-Hz spacing and channels with 120-Hz spacing

2.1	Channels with 240-Hz spacing should be installed in the following preferred order: 12 (if possible), 11, 10, 9, 8,
7, … The channel numbers1)  are in accordance with Recommendation R.37 (100-baud channels with 240-Hz spacing).

3	Combined use of 200-baud channels with 360-Hz spacing and channels with 120-Hz or 240-Hz spacing

3.1	The characteristics of these channels with high modulation rates are defined in Recommendations R.37 on 
100-baud channels with 240-Hz spacing and R.38 B on 200-baud channels with 360-Hz spacing.

3.2	The 200-baud/360-Hz channels should be installed in the following preferred order: 5, 4, 6, 3, 2, 1 instead of the corresponding 50-baud channels. The channel numbers1) are in accordance with Recommendation R.38 B.

3.3	In combined systems using channels with three different modulation rates, the order indicated in § 3.2 above should be used in preference to that indicated in § 2.1 above.

4	Combined use of 200-baud channels with 480-Hz spacing and channels with 120-Hz or 240-Hz spacing

4.1	For a combination of channels with 240-Hz spacing and channels with 480-Hz spacing, the channels with 
480-Hz spacing should be installed in the following preferential order: 4, 3, 5, 2, 62).

4.2	For a combination of channels with 120-Hz spacing and channels with 480-Hz spacing, the order indicated in § 4.1 above is applicable.

Note – In cooperation with a system using 6-channel group modulation, the preferred order would be: 4, 3, 6 (if possible), 12).

4.3	In combined systems using channels with three different modulation rates, the order indicated in § 4.1 above should be used in preference to that indicated in § 2.1 above.

1)	For the numbering of channels that has been adopted in the international services see Recommendation R.70 bis.

2)	For the numbering of channels that has been adopted in the international services see Recommendation R. 70 bis.