|  | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

ITU-T	R.37

TELECOMMUNICATION

STANDARDIZATION SECTOR

OF  ITU

TELEGRAPHY

TELEGRAPH  TRANSMISSION

STANDARDIZATION  OF  FMVFT  SYSTEMS  FOR  A  MODULATION  RATE  OF  100 BAUDS

ITU-T  Recommendation  R.37

(Extract from the Blue Book)

# NOTES

1	ITU-T Recommendation R.37 was published in Fascicle VII.1 of the Blue Book. This file is an extract from the Blue Book. While the presentation and layout of the text might be slightly different from the Blue Book version, the contents of the file are identical to the Blue Book version and copyright conditions remain unchanged (see below).

2	In this Recommendation, the expression “Administration” is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

ã  ITU  1988, 1993

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

Recommendation R.37

Fascicle VII.1 - Rec. R.37

STANDARDIZATION OF FMVFT SYSTEMS FOR A MODULATION RATE OF 100 BAUDS

(Geneva, 1964; amended at Mar del Plata, 1968, Geneva, 1972, 1976, 1980,

Malaga-Torremolinos, 1984 and at Melbourne, 1988)

Note – In this Recommendation frequency-modulated voice-frequency telegraph (FMVFT) equipment with and without crystal control are distinguished. In order to improve the quality of transmission and to minimize maintenance costs. the application of equipment with crystal control is recommended.

1	The nominal modulation rate is standardized at 100 bauds.

2	The nominal mean frequencies are 480 + (n – 1) 240 Hz, n being the channel position number. The mean frequency is defined as half the sum of the characteristic frequencies corresponding to the start polarity and stop polarity. For the numbering of channels that has been adopted in the international service see Recommendation R.70 bis.

3	The mean frequencies at the sending end should not deviate from their nominal value by more than:

a)	for equipment without crystal control 3 Hz;

b)	for equipment with crystal control 0.5 Hz1).

4	The difference between the two characteristic frequencies in the same channel is fixed at 120 Hz.

5	The maximum tolerance on this difference should be ± 4 Hz.

6	The unbalance due to the modulation process    should not exceed 2%,

where

are the two characteristic frequencies measured over a period of 10 s;

is the mean static frequency measured  ;

is the mean dynamic frequency measured with 1:1 rectangular signals during 10 s.

Measurement should be made applying to the input of the transmitter 1:1 rectangular signals with the build-up and hangover time below 1 ms and with the unbalance below 0.1%. In the event that in service the transmitter is controlled by an electromechanical relay (with a certain transit time), the measurement should also be made with that type of relay inserted between the 1:1 signal generator and the input to the transmitter. Both forms of measurement need not necessarily be included in the maintenance procedure but should be included in laboratory type tests.

Note – To determine the unbalance due to the modulation process by the method indicated above, it is necessary to measure the frequencies  ,   and   and to calculate the mean frequency   and the unbalance

A more rapid method for checking whether or not the unbalance is less than the limit fixed is to measure:

-	the mean dynamic frequency   with 1:1 signals during 10 s;

-	the mean dynamic frequency   with 2:2 signals during 10 s;

=

or to subtract:

The absolute value of the difference between the two frequencies measured, Fl, and Fm, must be less than 0.9 Hz.

7	The total average power transmitted to the telephone-type circuit is normally dependent on the transmission characteristics of the circuit as follows:

a)	For circuits with characteristics not exceeding the limits (nominal values) given in Annex A to Recommendation R.35, the mean power per channel at a point of relative zero level should not be more than 4.0 mW (–24.0 dBm0). The pilot channel, where employed, should have a level of not more than 2.0 mW (–27.0 dBm0);

b)	for other circuits, the mean power per channel at a point of relative zero level should not be more than 
10.8 mW (– 19.7 dBm0). The pilot channel, where employed, should have a level of not more than 5.4 mW (–22.7 dBm0).

8	In service, the levels of signals corresponding to continuous condition Z and continuous condition A should not differ by more than 1.7 dB in the same channel. Both of these levels must lie between ± 1.7 dB with reference to the level in § 7 above.

9	The frequency for the transmitted condition corresponding to the condition A is the higher of the two characteristic frequencies and that corresponding to the condition Z is the lower.

10	In the absence of a channel-modulator control telegraph current, a frequency shall be transmitted that shall be within ± 10 Hz of the frequency normally transmitted for the condition A. It is not necessary for this transmission to take place immediately after the control current has been cut.

11	The frequency spectrum of the emitted signal, when transmitting 1:1 reversals (Definition 31.401, Recommendation R. 140) at the modulation rate of 2  (  = frequency of modulation), should be in accordance with the limits specified in Figure 1/R.37, which shows the levels of the spectra of different components with respect to the amplitude of the non-modulated carrier as ordinates and the frequencies as abscissae.

12	The receiving equipment should operate satisfactorily when the receiving level falls to 17.4 dB below the nominal level. The receiving equipment should have been restored to condition A when the receiving level has fallen to 23.5 dB below the nominal level. The alarm-control level is left to the choice of each Administration.

13	On delivery by the manufacturer of 100-baud FMVFT equipment, the following values must not be exceeded for the degree of distortion on a telegraph channel. These values correspond to closed circuit measurements, made with the audio-frequency line terminals of the sending and receiving equipments connected together through an artificial line. Before the series of measurements taken in accordance with Recommendation R.51, the levels are adjusted to their normal values, the mean frequencies are checked to see whether they are within:

1)	for equipment without crystal control ± 3 Hz;

2)	for equipment with crystal control ± 0.1 Hz2),

of their nominal value (see § 3 above) and the difference between the two characteristic frequencies is within the permitted tolerance of less than 4 Hz (see § 5 above). Bias distortion is eliminated by adjustment in the channel receivers. The other channels of the system are modulated with unrelated signals when the effect of interchannel interference is to be included in the measurement. These “unrelated signals” can conveniently be 1:1 signals from different generators at approximately 100 bauds but not synchronous to each other or to the signal on the channel under test.

a)	The transmission levels being normal, the artificial line introducing no frequency drift, but the measured channel being subject to fortuitous distortion due to interchannel interference: 5% for the degree of inherent isochronous distortion.

b)	The level being maintained constant, but at a value different from the normal level, for all constant levels between 8.7 dB above the normal reception level and 17.4 dB below the normal reception level, the other conditions being the original measurement conditions: 7% for the degree of inherent isochronous distortion.

c)	In the presence of interference by a single sine-wave frequency equal first to one and then to the other characteristic frequency, with a level of 20 dB below the signal level, the other conditions for the start of measurements being maintained: 12% for the degree of inherent isochronous distortion (i.e. total distortion including the increase due to the interfering frequency, not distortion due to the interfering frequency alone).

1)	The tightening of this tolerance is for further study.

2)	The tightening of this tolerance is for further study.