|  | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

ITU-T	R.40

TELECOMMUNICATION

STANDARDIZATION SECTOR

OF  ITU

TELEGRAPHY

TELEGRAPH  TRANSMISSION

COEXISTENCE  IN  THE  SAME  CABLE  
OF  TELEPHONY  AND  SUPER - TELEPHONE  TELEGRAPHY

ITU-T  Recommendation  R.40

(Extract from the Blue Book)

# NOTES

1	ITU-T Recommendation R.40 was published in Fascicle VII.1 of the Blue Book. This file is an extract from the Blue Book. While the presentation and layout of the text might be slightly different from the Blue Book version, the contents of the file are identical to the Blue Book version and copyright conditions remain unchanged (see below).

2	In this Recommendation, the expression “Administration” is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

ã  ITU  1988, 1993

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

Recommendation R.40

Fascicle VII.1 - Rec. R.40

COEXISTENCE IN THE SAME CABLE OF TELEPHONY

AND SUPER-TELEPHONE TELEGRAPHY

(former CCIT Recommendation B.17, Brussels, 1948;

amended at Geneva, 1951 and at Melbourne, 1988)

The CCITT,

considering

(a)	that this process provides only one telegraph channel, in addition to the telephone channel, and that it can be applied only in comparatively few cases (lightly loaded or unloaded circuits, which cannot be used for multi-channel carrier telephony);

(b)	that in such cases, the Administrations and private operating agencies concerned could in most cases by common agreement contemplate the possibility of making use of some other more suitable process that would provide, in addition to the audio telephone channel, more than one telegraph channel;

unanimously declares the view

that the use of super-telephone telegraphy (Definition 02.25, Recommendation R.140) should not prejudice the quality of transmission over the adjacent telephone channel and that, in particular, it should not limit the band of frequencies necessary for good speech reproduction (300 to 3400 Hz at least).