|  | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

ITU-T	R.90

TELECOMMUNICATION

STANDARDIZATION SECTOR

OF  ITU

TELEGRAPHY

TELEGRAPH  TRANSMISSION

ORGANIZATION  FOR  LOCATING  AND  CLEARING  FAULTS  IN  INTERNATIONAL  TELEGRAPH  SWITCHED  NETWORKS

ITU-T  Recommendation  R.90

(Extract from the Blue Book)

# NOTES

1	ITU-T Recommendation R.90 was published in Fascicle VII.1 of the Blue Book. This file is an extract from the Blue Book. While the presentation and layout of the text might be slightly different from the Blue Book version, the contents of the file are identical to the Blue Book version and copyright conditions remain unchanged (see below).

2	In this Recommendation, the expression “Administration” is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

ã  ITU  1988, 1993

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

Recommendation R.90

Fascicle VII.1 - Rec. R.90

ORGANIZATION FOR LOCATING AND CLEARING FAULTS

IN INTERNATIONAL TELEGRAPH SWITCHED NETWORKS

(former CCIT Recommendation B.55, Geneva, 1956; amended at New Delhi, 1960

and Malaga Torremolinos, 1984)

The CCITT,

considering

(a)	that it is desirable that faults affecting communication between stations on international switching networks (e.g. telex and gentex service) should be reported and cleared as quickly as possible;

(b)	that it is necessary to unify the essential action to be taken and methods to be employed for locating and clearing faults;

(c)	that, for this purpose, it is necessary to determine the essential testing equipment that is to be provided at the switching centres responsible for locating and clearing faults,

unanimously declares the view

1	That it is necessary to set up switching and testing centres (STCs), defined as switching centres equipped with measuring equipment for testing telex subscribers' and public station lines and equipment and also telegraph channels.

2	Each telex subscriber and each public station in the general switching service should have access to an STC for the purpose of reporting faults and cooperating in tests.

3	International switching and testing centres (ISTCs) are the STCs that are also international line-head offices.

4	All STCs should be subscribers to the telex network, both for the purpose of receiving fault reports and for communication for maintenance purposes. They should also be provided with a telephone subscriber’s line.

5	Each STC should be responsible for coordinating action in locating and clearing faults on all station lines connected to the exchange and on all trunk circuits for which it is nominated as the controlling office. It should also cooperate with other STCs in locating faults on connections established through two or more exchanges.

5.1	It should carry out a preliminary location of faults by finding out whether they affect channels, switching gear or apparatus. The faults are then accurately located by the engineers responsible for each part of the circuit and the STC cooperates with them for this purpose. It may assume the direction of the fault-locating procedure should there be disagreement between these services. Internationally, it is responsible to the STCs of other countries with which it has telex connections.

5.2	The organization of the liaison between the STC and the different technical services is shown in Figure 1/R.90. The STCs must check that the performance given by the equipment involved in the switching service, i.e. VF channels, switching equipment and apparatus, is satisfactory.

6	The staff employed at STCs should be selected with a view to avoiding language difficulties and should be conversant with all types of telegraph equipment used in the switching network, i.e. automatic or manual switching equipment, VFT equipment, telegraph machines and regenerative repeaters. The staff need not necessarily be fully competent to maintain all these items of equipment, but should have sufficient knowledge of them to be able to form an appreciation of the effect that faults on any of them may have on a switched connection. In addition, the staff of ISTCs should have some general knowledge of the types of equipment used in the countries to which they are connected, particularly of the signalling conditions that will be encountered.

7	Each STC should be provided with the following measuring equipment:

a)	50-baud start-stop distortion meter;

b)	test transmitter for generating undistorted 50-baud start-stop signals;

c)	apparatus to measure the modulation rate of teleprinters at a distance;

d)	apparatus for measuring the speed and pulse ratio of dials, where appropriate;

e)	apparatus for measurement of the condition of direct current lines; for example, continuity, resistance, insulation.

7.1	The arrangements for access to established connections for making test measurements should be such as not to cause interruptions or reduce the quality of transmission.

7.2	Considering that some Administrations have found it desirable to have available at the STC other items of apparatus to expedite the clearing of faults, all Administrations are invited to consider the utility of these devices, namely:

a)	apparatus for measuring teleprinter margin;

b)	recording distortion meters for testing established connections;

c)	apparatus for measuring continuously, periodically and automatically, the distortion on subscribers’ lines and apparatus.

8	The following procedure for reporting, locating and clearing faults should be adopted.

8.1	Faults should be reported to the STC concerned by the subscribers or operators who have experienced difficulty in operation. In the same way, it would be useful, in order to give the STCs a full picture of the situation, that the maintenance engineers should inform them of faults noted during the periodic maintenance operations. Faults should preferably be signalled by teleprinter, if their nature does not preclude this procedure.

8.2	A reference number should be given by the STC to the subscriber or service notifying the fault. This number can then be quoted in any subsequent inquiries as to the progress of fault clearance.

8.3	On account of the difficulties that may arise in the detection of faults on the international section of a communication (due to lack of knowledge of languages, etc.), care should be taken in each country to see that the national sections of the communication, including subscribers’ lines and apparatus, are not involved before approaching the STC of the corresponding country.

8.4	Complete holding of a connection that is reported to be faulty should be avoided.

8.5	The STC notified of a fault should therefore begin by ascertaining that it is not located in the national section of the communication and for this purpose should, if necessary, approach the other STCs of its country concerned in the circuit. The STC of the distant country is then advised and, in turn, checks the national section routed over its network. The international section of the communication is not checked until the terminal national sections of telegraph circuits have been definitely exonerated. The STCs in different countries will communicate with one another, either directly or via their ISTCs, as determined by the Administrations concerned.

8.6	If the tests of the two local ends fail to reveal any fault conditions, the STC should report the fault to its ISTC, which will decide what further action, if any, is necessary. As a rule, isolated fault reports would not justify a test of all trunk circuits on a route, and it would be assumed that the condition giving rise to the fault would be cleared on the next routine adjustment. If however, several fault reports were received, some of which might have been due to a faulty circuit on a particular route, then a special routine test of all the circuits on the route might be justified.

8.7	In general, it is considered that the procedure will be broadly the same for manual, semi-automatic and automatic systems.

9	The abbreviations annexed below should be used in calls exchanged between services responsible for the maintenance of telegraph equipment.

ANNEX A

(to Recommendation R.90)

List of service abbreviations for maintenance of telegraph circuits

No.		Abbreviation			Meaning

30 bis		BL				Holding

30		BL . . . SVP			Please hold . . .

2		BR TR . . .			Bad transmission on . . .

39 bis		CCT . . . IN			I have restored circuit No.  . . .

39		CCT . . . IN SVP		Please restore circuit No.  . . .

38 bis		CCT . . . OUT		I have taken circuit No.  . . .  out of service

38		CCT . . . OUT SVP		Please take circuit No.  . . .  out of service

43		CRD . . .			The connection is released after selection on circuit No.  . . .

37 bis		CSR				I am receiving your calling signal

8		DER CCT			Circuit faulty

51		DER REG			Register does not operate

52		DER TAPE			Your perforated tape is faulty

33		DER VF . . .			Fault on voice-frequency system . . .

7		DERA			Machine faulty

9		DERPS			Position equipment faulty

10		DERR			Fault now cleared

64		DEVD			Deviation of distributor speed at your end

23		DEVS . . .			Speed deviation is . . . %

16		. . . DIS . . .			The distortion on is . . . %

62		DS . . .			Distribution switched over to . . .

25		EDIS . . .			The transmitter distortion is . . . %

1		ICI . . .			Here is . . .

53 bis		LOOP . . .			I have looped circuit . . .

53		LOOP . . . SVP		Please loop circuit . . .

24		MAR . . .			The margin is . . . %

18		MEET . . .			Meet me on circuit No.  . . .

50		N IND			I am not receiving your answer-back code

40		N PER A			I am not receiving your permanent start polarity signal

41		N PER Z			I am not receiving your permanent stop polarity signal

66		NARQ . . .	Multiplex  . . .  unprotected; please re-establish automatic request for repetition (ARQ)

31 bis		NBL . . .			Clearing

31		NBL . . . SVP			Please clear . . .

27		NCFM . . .			No call-confirmation signal on . . .

26		NCS . . .			No call-connected signal from . . .

11		NDER			No fault found

42		NPS				I am not receiving your proceed-to-select signal

28		OCC OCC. . .		Permanent busy signal from . . .

65		OPH . . .			Out of phase on system . . .

46		PER A . . .			Permanent start polarity on . . .

48		PER A . . . SVP		Please send permanent start polarity on . . .

47		PER Z . . .			Permanent stop polarity on . . .

49		PER Z . . . SVP		Please send permanent stop polarity on . . .

29		PERC . . .			Permanent call on . . .

63		PH . . .			Please phase system . . .

34 bis		Q DIS A			Is there bias distortion (prolonged start polarity) on the received signals?

35 bis		Q DIS Z			Is there bias distortion (prolonged stop polarity) on the received signals?

13		QDIS . . .			Please measure distortion on . . . and report result

37		QRCS			Are you receiving my calling signal?

3		QREF			Please give reference number

4		QRES			Please report result

15		RAP . . . MNS		I shall recall you in . . . minutes

14		RAP . . . MNS SVP		Please call me in . . . minutes

5		REF . . .			Reference number is . . .

6		RES . . .			Here is result of test on . . .

55		RFC . . .			I am receiving errors in 5-unit code. Please check channel No.  . . .

70		RMUT . . .	I am receiving garbled signals on multiplex channel . . . please check your 7-unit code

54		RQFS . . .	Your repetition cycle transmission contains 7-unit code faults. Please check channel No.  . . .

59		RS . . .			Reception switched over to . . .

44		SIG 1/1 SVP			Please send 1:1 signals

45		SIG 2/2 SVP			Please send 2:2 signals

61		SS . . .			Storage switched over to . . .

12		TESTD . . . SVP		Please send test message with  . . .  % distortion on . . .

67		TRAS . . .			Please send alpha signal on multiplex channel . . .

68		TRBS . . .			Please send beta signal on multiplex channel . . .

60		TRS . . .			Transmission switched over to . . .

21		VERED			Please check the transmitter distortion

22		VERM			Please check the margin

20		VERS			Please check the speed

19		VERX . . .			Please check subscriber No.  . . .

34		ZKWA . . .			The received signals have . . . % bias (start polarity prolonged)

35		ZKWZ . . .			The received signals have . . . % bias (stop polarity prolonged)

32		ZOK				I am receiving correctly

17		ZSU				Your signals are unreadable

71		ZYA				Cease traffic on all channels; send As on A channel for line-up

69		ZYC				Your transmitter is sending permanent ARQ

56		ZYK . . .			Your keying on channel  . . .  is affected; please check

57		ZYM			Change from single printer to multiplex

36		ZYN				Reduce the bias

58		ZYP				Change from multiplex to single printer