|  | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

ITU-T	R.120

TELECOMMUNICATION

STANDARDIZATION SECTOR

OF  ITU

TELEGRAPHY

TELEGRAPH  TRANSMISSION

TOLERABLE  LIMITS  FOR  THE  DEGREE  
OF  ISOCHRONOUS  DISTORTION  OF  
CODE - INDEPENDENT  TELEGRAPH  CIRCUITS  OPERATING  AT  MODULATION  RATES  
OF  75,  100  AND  200 BAUDS

ITU-T  Recommendation  R.120

(Extract from the Blue Book)

# NOTES

1	ITU-T Recommendation R.120 was published in Fascicle VII.1 of the Blue Book. This file is an extract from the Blue Book. While the presentation and layout of the text might be slightly different from the Blue Book version, the contents of the file are identical to the Blue Book version and copyright conditions remain unchanged (see below).

2	In this Recommendation, the expression “Administration” is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

ã  ITU  1988, 1993

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

Recommendation R.120

Fascicle VII.1 - Rec. R.120

TOLERABLE LIMITS FOR THE DEGREE OF ISOCHRONOUS DISTORTION

OF CODE-INDEPENDENT TELEGRAPH CIRCUITS

OPERATING AT MODULATION RATES OF

75, 100 AND 200 BAUDS

(Geneva, 1976; amended at Geneva, 1980)

The CCITT,

considering

(a)	that, to facilitate the study of plans for the establishment of international telegraph circuits, it is convenient to set limits for the degree of isochronous distortion of telegraph circuits and channels;

(b)	that, for whatever purposes normally used, these circuits should be capable of use with start-stop equipment;

(c)	that, until detailed transmission planning standards are established for the trunk sections of international telegraph circuits operating at modulation rates of 75, 100 and 200 bauds, the distortion limits mentioned below should be regarded as provisional standards;

(d)	that the limits laid down are those that should be evident in service conditions on telegraph circuits, excluding the local lines and terminal equipment,

unanimously declares the view

(1)	that circuits (excluding local lines and terminal equipment) should be established and maintained in such a manner that the degree of isochronous distortion will not exceed the limits shown in Table 1/R.120, irrespective of whether any form of regeneration is provided in the circuit or not;

TABLE 1/R.120

| Modulation rate (bauds) | Maximum degree of isochronous distortion permitted |
| --- | --- |
| 75 100 200 | 28 % 24 % 32 % |

(2)	that the degree of isochronous distortion of each channel that may form part of a circuit should be as small as possible, and should not in any case exceed 10%.