|  | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

ITU-T	T.2

TELECOMMUNICATION

STANDARDIZATION SECTOR

OF  ITU

TERMINAL  EQUIPMENT  AND  PROTOCOLS  FOR  TELEMATIC  SERVICES

STANDARDIZATION  OF  GROUP 1  
FACSIMILE  APPARATUS  FOR  
DOCUMENT  TRANSMISSION

ITU-T  Recommendation  T.2

(Extract from the Blue Book)

# NOTES

1	ITU-T Recommendation T.2 was published in Fascicle VII.3 of the Blue Book. This file is an extract from the Blue Book. While the presentation and layout of the text might be slightly different from the Blue Book version, the contents of the file are identical to the Blue Book version and copyright conditions remain unchanged (see below).

2	In this Recommendation, the expression “Administration” is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

ã  ITU  1988, 1993

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

Recommendation T.2

Fascicle VII.3 - Rec. T.2

STANDARDIZATION OF GROUP 1 FACSIMILE APPARATUS

FOR DOCUMENT TRANSMISSION

(Mar del Plata, 1968; amended at Geneva, 1972 and 1976)

The CCITT,

considering

(a)	that there is a requirement for Group 1 facsimile apparatus which enables an ISO A4 document to be transmitted over a telephone-type circuit in approximately six minutes;

(b)	that document facsimile transmission may be requested alternately with telephone conversation or when either or both stations are unattended; in both cases the facsimile operation should conform to Recommendation T.30,

unanimously declares the view

that Group 1 facsimile apparatus for use on the general switched telephone network and international leased circuits should be designed and operated according to the following standards:

1	Scanning track

The message area should be scanned in the same direction in the transmitter and receiver. Viewing the message area in a vertical plane, the scanning direction should be from left to right, and subsequent scans should be adjacent and below the previous scan.

2	Index of cooperation

The nominal index of cooperation is 264. In cases where a lower vertical resolution is acceptable, and by agreement between the users, an optional index of cooperation of 176 may be used.

These values should be observed with a nominal tolerance of ± 1% for each equipment.

3	Dimensions of apparatus

3.1	The apparatus should accept documents up to a minimum of ISO A4 size (nominally 210 mm ´ 297 mm).

3.2	The total scanning line length (active sector plus dead sector) should be nominally 215 mm. Nominally 200 mm should be available for scanning or recording, the remainder being the dead sector.

3.3	For any one document the nominal number of scans should be 1144 for an index of cooperation of 264 (762 scans for an index of 176). The receiver should be capable of recording nominally 1144 scans per document for an index of cooperation of 264 (or 762 scans for an index of 176).

3.4	Apparatus with other dimensions may be used provided that the index of cooperation is respected, that the total scanning line length lies between 210 and 250 mm and the usable recording line length retains the same ratio to the total scanning line length.

4	Scanning density

Scanning density is normally 3.85 lines per mm.

5	Scanning line frequency

In the subscriber-to-subscriber service via the general switched telephone network, the scanning line frequency should be 180 lines per minute (see Note).

For leased circuits operation the best line frequency, which may be higher or lower than 180 lines per minute, may be chosen according to the circuit characteristics.

The scanning line frequency during the transmission should be kept within ± 10 parts in 106 of the nominal value.

Note – With manual control at the two ends of connection set up over the general switched telephone network, another scanning line frequency (e.g. 240 per minute) may be chosen by agreement between the two operators.

6	Phasing

The duration of the phasing signal for transmitters should be 15 ± 1 seconds.

In a preferred method of phasing (see Note 1), the transmitter sends a series of alternating white and black signals in such a way that the white pulse (phasing pulse) is 4 to 6% of the total scanning line length and the leading edge is 2 to 3 % in advance of the middle of the dead sector.

Receiving apparatus should synchronize the middle of its dead sector 0.5 to 4.5% lagging the leading edge of the received phasing pulse (see Note 2).

Note 1 – In a permitted method of phasing for present generation machines, the transmitter sends a series of white and black signals in such a way that the white pulse is 2 to 12% of the total scanning line length and the leading edge is 2 to 3 % in advance of the middle of the dead sector.

Note 2 – Maximum reduction of recorded scanning line length due to synchronizing misalignment should not exceed 3% of total scanning line length. Maximum reduction of recorded scanning line length due to the combined effect of deviations of the transmitter and receiver scanning line frequencies should not exceed 4% of total scanning line length. The effect of these reductions of recorded scanning line length may cause it to be less than the nominal 200 mm.

7	Modulation and demodulation equipments

7.1	Amplitude modulation (for leased circuits only)

The facsimile signal level is higher for black and lower for white.

The carrier frequency should range between 1300 and 1900 Hz and will depend upon the characteristics of the circuits used.

7.2	Frequency modulation (for leased circuits and for switched connections)

The frequency corresponding to black will normally be f0 + 400 Hz and the frequency corresponding to white will normally be f0 – 400 Hz (see Note).

For switched connections f0 = 1700 Hz (provisional). For leased circuits f0 should range between 1300 and 1900 Hz: the choice of the centre frequency f0 will depend upon the circuit characteristics. However, if the user, in some cases, wishes to use the apparatus on switched connections, f0 = 1700 Hz.

The stability of the transmitter must be such that the significant frequencies do not vary by more than 32 Hz from their nominal value in a period of 15 minutes.

Note – Attention is drawn to the fact that there are some equipments currently in operation for which black and white elements are represented in the opposite sense, but the preferred standard for new equipment is as indicated above.

7.3	Power at the transmitter output

When amplitude modulation is used, the power of black at the transmitter output must be able to be adjusted between –7 dBm and 0 dBm. The white level must be approximately 15 dB below the black level.

In frequency-modulated systems, the level at the output of the facsimile apparatus must be able to be adjusted between –15 dBm and 0 dBm.

The equipment should be so designed that there is no possibility of this adjustment being tampered with by an operator.

7.4	Power at the receiver input

The facsimile receiver must be so designed that it functions correctly when the input power ranges between 0 dBm and –40 dBm, the latter value being considered provisional. In the case of amplitude modulation, this concerns the power of the black signal. No control of receiver sensitivity should be provided for operator use.