|  | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

ITU-T	T.3

TELECOMMUNICATION

STANDARDIZATION SECTOR

OF  ITU

TERMINAL  EQUIPMENT  AND  PROTOCOLS  FOR  TELEMATIC  SERVICES

STANDARDIZATION  OF  GROUP  2  
FACSIMILE  APPARATUS  FOR  
DOCUMENT  TRANSMISSION

ITU-T  Recommendation  T.3

(Extract from the Blue Book)

# NOTES

1	ITU-T Recommendation T.3 was published in Fascicle VII.3 of the Blue Book. This file is an extract from the Blue Book. While the presentation and layout of the text might be slightly different from the Blue Book version, the contents of the file are identical to the Blue Book version and copyright conditions remain unchanged (see below).

2	In this Recommendation, the expression “Administration” is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

ã  ITU  1988, 1993

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

Recommendation T.3

Fascicle VII.3 - Rec. T.3

STANDARDIZATION OF GROUP 2 FACSIMILE APPARATUS FOR DOCUMENT TRANSMISSION

(Geneva, 1976; amended at Geneva, 1980)

The CCITT,

considering

(a)	that Recommendation T.2 refers to Group 1 type apparatus for ISO A4 document transmission in approximately six minutes;

(b)	that there is a demand for Group 2 apparatus which enables an ISO A4 document to be transmitted over a telephone-type circuit in approximately three minutes;

(c)	that the Group 2 apparatus reproduces document quality similar to Group 1 apparatus;

(d)	that such a service may be requested either alternatively with telephone conversation, or when either or both stations are not attended; in both cases, the facsimile operation will follow Recommendation T.30;

(e)	interconnection between two machines of different designs, both conforming to Recommendation T.3 as published in the Orange Book may give a lower guaranteed reproducible area in certain cases;

unanimously declares the view

that Group 2 facsimile apparatus for use on the general switched telephone network and international leased circuits shall, in future, be designed and operated in accordance with this Recommendation. Apparatus conforming to Recommendation T.3 of the Orange Book may continue in service.

1	Scanning track

The message area should be scanned in the same direction in the transmitter and receiver. Viewing the message area in a vertical plane, the scanning direction should be from left to right and subsequent scans should be adjacent and below the previous scan.

In order to avoid loss of information, users should insert documents so that the scanning direction is from the wider to the narrower margin.

2	Dimensions of apparatus

The following dimensions are recommended but apparatus with other dimensions may be used provided that the factor of cooperation is respected and that the total scanning line length lies between 215 and 222 mm.

Factor of cooperation (FOC)	829 ± 1%

Total scanning line-length (TLL)	215 mm

Usable scanning line-length (ULL)	205 mm minimum

The end of ULL shall lie between 0 and 1 mm within the nominal position of the right hand edge of an ISO A4 size document.

Input document size 	Up to a minimum of ISO A4 
		size

(nominally 210 ´ 297 mm)

The foregoing dimensions give rise to the following approximate secondary dimensions.

Index of cooperation (IOC)	264

Scanning density	3.85 line/mm

Number of scanning lines in a document 297 mm long	1145

3	Scanning line frequency

3.1	The scanning line frequency should be 360 lines per minute.

Note – Another scanning line frequency (e.g. 300 lines per minute) may be chosen by agreement between the two stations.

3.2	The scanning line frequency during the transmission should be kept within ± 5 parts in 106 of the nominal value.

4	Phasing

For phasing prior to transmission of document information the transmitter sends a signal as shown in Figure 1/T.3. The start of the carrier indicates the end of the lost time. The accuracy of the adjustment at the transmitter of the end of lost time to the start of carrier should be ± 0.5% of TLL. The receiver should phase its reference point with an accuracy of ± 1% of TLL to this start of the carrier. The reference point should be 209 mm before the end of ULL (see Figure 2/T.3).

Note – In the case of the multiple page transmission the phasing procedure may need to be repeated between pages.

µ §

5	Document transmission

5.1	During transmission of document information the transmitter should transmit full amplitude carrier during the lost time for at least 4% of the transmission time for TLL. The phase of the carrier may be reversed at the end of this signal.

Following the lost time the information transmitted to the receiver is the one contained in ULL. The value of ULL is defined in § 2.

The receiver must be able to reproduce at least ULL on the reproduction media after the end of the lost time.

5.2	This signal may be used at the receiver as an amplitude reference independent of the document signal and also to indicate to the receiver that document transmission is still in progress.

6	Paper alignment

All types of apparatus should be designed to ensure that paper is positioned within ± 1.5 mm of nominal in the horizontal direction. In the vertical direction it must be ensured that paper is positioned in such a way that scanning and recording start not more than 4 mm down the document from the top edge.

7	Modulation and demodulation

7.1	Equivalent modulated waveforms with vestigial sideband amplitude modulation – phase modulation characteristics should be used for Group 2 apparatus operated on leased circuits and on the general switched telephone network. The carrier frequency should be 2100 ± 10 Hz (see Note). A white signal should be represented by maximum carrier and a black signal by minimum (at least 26 dB below white) or no carrier. The phase of the carrier representing white may be reversed after each transition through black.

Note – It should be noted that there are equipments in service using, inter alia, a carrier frequency of 2048 Hz.

7.2	The facsimile transmission could contain a limited range of half-tones represented by a carrier amplitude less than that sent during the lost time.

7.3	A vestigial sideband filter symmetrical about the 2100-Hz carrier frequency should be provided at the transmitter. The response of this filter should conform to Figure 3/T.3. The accuracy of this filter is not critical but the relative transmittance at the carrier frequency should be 0.5 with a tolerance of ± 0.05 and the characteristic should be sensibly symmetrical about the carrier frequency.

7.4	The receiving apparatus must be capable of operating correctly when the drift of carrier frequency does not exceed its nominal value by more than ± 16 Hz.

7.5	A fixed compromise equalizer, an automatic equalizer or an adaptive equalizer may be provided in the receiver but this should be a matter for further study.

8	Power at the transmitter output

The power of the white signal should be adjustable from – 15 dBm to 0 dBm but the equipment should be so designed that there is no possibility of this adjustment being tampered with by an operator. The black level should be at least 26 dB below the white level.

9	Power at the receiver input

The receiving apparatus should be capable of functioning correctly when the received signal level for white is within the range 0 dBm to –40 dBm, the latter value being considered provisional. No control of receiver sensitivity should be provided for operator use.

APPENDIX I

(to Recommendation T.3)

Guaranteed reproducible area for Group 2 machines

conforming to Recommendation T.3

I.1	Horizontal loss

TAble I-1/T/3

Results

| Horizontal losses |  |  |  |  |  |  | Left margin (mm) |  |  |  |  | Right margin (mm) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 205 |  |  |  | 210 |  |  | 205 |  |  | 210 |
| Phasing | Transmitter | a |  | ± 0.5% |  |  | 1.11 |  |  | 1.11 |  |  | 1.11 |  | 1.11 |  |  |
|  | Receiver | b |  | ± 1.0% |  |  | 2.22 |  |  | 2.22 |  |  | 2.22 |  | 2.22 |  |  |
| Skew |  |  | c |  |  | ± 5 ´ 10– 6 |  |  | 2.46 | 2.46 |  |  |  | 2.46 |  | 2.46 |  |
| Enlarging |  |  | d |  |  |  |  |  | – | – |  |  |  | 6.46 |  | 6.62 |  |
| Paper insertion |  |  | f |  |  |  |  |  | 1.50 | 1.50 |  |  |  | 1.50 |  | 1.50 |  |
| Lost time | Min | x |  |  |  |  | – |  |  | – |  |  | 1.00 |  | 1.00 |  |  |
|  | Max | x |  |  |  |  | 5.00 |  |  | – |  |  | – |  | – |  |  |
|  |  |  |  |  | Total |  |  | 12.29 |  | 7.29 |  |  |  | 14.75 |  | 14.91 |  |
| TTL: 215 to 222 mm ULL: 205 to 210 mm |  |  |  |  | Shifted value |  |  | 16.29 |  | 3.29 |  |  |  | 10.75 |  | 10.91 |  |

I.2	 Vertical loss

TABLE I-2/T.3

|  |  | Top (mm) | Bottom (mm) |
| --- | --- | --- | --- |
| Gripping loss | g | 4.0 | 4.0 |
| Paper insertion | h |  |  |
| Enlarging + FOC | i | – | 15.15 |
|  | Total | 4.0 | 19.15 |