|  | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

ITU-T	Amendment  1

TELECOMMUNICATION	T.433
STANDARDIZATION  SECTOR	
OF  ITU	(08/95)

TERMINALS  FOR  TELEMATIC  SERVICES

DOCUMENT  TRANSFER,  ACCESS  AND
MANIPULATION  (DTAM)  –  SERVICES  AND  PROTOCOLS  –  PROTOCOL  SPECIFICATION

AMENDMENT  1:  REVISIONS  OF  T.433  TO
SUPPORT  G4  COLOUR  AND  FILE  TRANSFER

Amendment  1  to

ITU-T  Recommendation  T.433

(Previously “CCITT  Recommendation”)

# FOREWORD

The ITU-T (Telecommunication Standardization Sector) is a permanent organ of the International Telecommunication Union (ITU). The ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Conference (WTSC), which meets every four years, establishes the topics for study by the ITU-T Study Groups which, in their turn, produce Recommendations on these topics.

The approval of Recommendations by the Members of the ITU-T is covered by the procedure laid down in WTSC Resolution No. 1 (Helsinki, March 1-12, 1993).

Amendment 1 to ITU-T Recommendation T.433, was prepared by ITU-T Study Group 8 (1993-1996) and was approved under the WTSC Resolution No. 1 procedure on the 11th of August 1995.

___________________

NOTE

In this Recommendation, the expression “Administration” is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

ã  ITU  1995

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

CONTENTS

Recommendation T.433 / Amd.1     (08/95)

Page

1	Introduction		1

2	Modification of Recommendation T.433		1

2.1	The second paragraph of 5.3.2 in Recommendation T.433 should be replaced by the following new paragraph		1

2.2	The following new subclause should be added after 6.2.4.3.1.5 in Recommendation T.433		1

2.3	The following new subclause should replace 6.2.4.3.2 in Recommendation T.433		2

2.4	Insert new subclause 6.2.4.3.3		2

2.5	The following new subclause should be added after 6.5.4.1.1.5 in Recommendation T.433		2

2.6	The following new subclauses should replace 6.5.4.1.2 in Recommendation T.433		2

2.7	The following new paragraph should be inserted after the title 6.6.3.2 in Recommendation T.433		2

2.8	Subclause 7.3.5.2 in Recommendation T.433 should be replaced by the following new subclause		3

2.9	Figure 3 should be replaced with  following new figure		3

2.10	Modules to be imported in DTAM APDU defined in 8.1 should be replaced by the following new modules		4

2.11	Abstract syntax definition of ApplicationCapabilities used in DTAM normal mode should be replaced by the following new definition		4

2.12	Subclause 8.2 should be replaced by the following new subclause		4

8.2	Abstract syntax definition of APDUs for use of the session service in Transparent Mode		4

SUMMARY

Recommendation T.433 currently defines the document bulk transfer of Group 4 Telefax documents as the DTAM transparent mode and of arbitrary documents and binary files in the DTAM normal mode. Amendment 1 provides for the transfer of binary files (based upon Recommendation T.434), and other formats, such as DTM, JPEG, EDIFACT and also Colour facsimile documents in the DTAM transparent mode. This amendment mainly concentrates on the amendments for the association establishment, where the relevant formats need to be recognized, and on the transfer phase, where segmenting must be extended to information that is not structured page by page.

Amendment 1 to Recommendation T.433

Recommendation T.433 / Amd.1     (08/95)

REVISIONS  OF  T.433  TO  SUPPORT  G4
COLOUR  AND  FILE  TRANSFER

(Geneva, 1995)

# 1	Introduction

This contribution contains the revision of existing Recommendations T.433 in order to support colour and file transfer extension for G4 facsimile application.

# 2	Modification of Recommendation T.433

## 2.1	The second paragraph of 5.3.2 in Recommendation T.433 should be replaced by the following new paragraph

### 5.3.2	Functions of DTAM service-providerXE "Functions of DTAM service-provider"§

....

2)	DTAM capabilityXE "DTAM capability"§

The DTAM capability is defined by sets of parameters in order to specify the communication features:

ODA application capabilitiesXE "ODA application capabilities"§

a)	document application profile;

b)	operational application profile;

c)	non-basic document characteristics;

d)	non-basic structural characteristics; and

e)	additional document characteristics, etc.

File transfer capabilities

a)	BFT capabilities;

b)	Transparent Data Capabilities;

c)	DTM capabilities;

d)	EDIFACT capabilities.

Private capabilities

......

## 2.2	The following new subclause should be added after 6.2.4.3.1.5 in Recommendation T.433

###### 6.2.4.3.1.6	Additional document characteristics

The value of this parameter is any combination of additional document characteristics defined in Recommendation T.414.

## 2.3	The following new subclause should replace 6.2.4.3.2 in Recommendation T.433

##### 6.2.4.3.2	 File transfer capabilities

This parameter specifies file transfer capabilities which are available to the sender of this parameter as receiving capabilities. The parameter is present in both, the Transparent Mode and Normal Mode.

–	BFT (Binary File Transfer) capabilitiesXE "BFT (Binary File Transfer) capabilities"§

This value indicates the capability to receive files encoded to the specification of Recommendation T.434.

–	Transparent data capabilitiesXE "Transparent data capabilities"§

This value indicates that the sender is ready to accept (when acting as a receiver) data that are transferred completely transparently.

–	DTM (Document Transfer Mode) capabilitiesXE "DTM (Document Transfer Mode) capabilities"§

This value indicates the capability to receive BFT files (defined in Recommendation T.434) encoded to the DTM encoding rules defined in Recommendation T.563.

–	EDIFACT (electronic data exchange) capabilitiesXE "EDIFACT (electronic data exchange) capabilities"§

This value indicates the capability to receive files encoded to the EDIFACT formats defined in ISO/IEC 9735.

## 2.4	Insert new subclause 6.2.4.3.3

##### 6.2.4.3.3	 Private capabilitiesXE "Private capabilities"§

This parameter  indicates capabilities for private values. DTAM-PM does not care the contents of this parameter. This parameter is only present in the Transparent Mode.

## 2.5	The following new subclause should be added after 6.5.4.1.1.5 in Recommendation T.433

###### 6.5.4.1.1.6	Additional document characteristics

The value of this parameter is any combination of additional document characteristics defined in Recommendation T.414.

## 2.6	The following new subclauses should replace 6.5.4.1.2 in Recommendation T.433

##### 6.5.4.1.2	 File transfer capabilitiesXE "File transfer capabilities"§

This parameter specifies file transfer capabilities which are available to the sender of this parameter as receiving capabilities. The parameter may be present in the Transparent Mode and Normal Mode.

##### 6.5.4.1.3	 Private capabilities

See 6.2.4.3.3.

## 2.7	The following new paragraph should be inserted after the title 6.6.3.2 in Recommendation T.433

The following procedure is applied to the transfer of G4 facsimile documents (according to Recommendation T.503) and to the transfer of files. The interchange data elements are to be interpreted in relationship with the T.503 document structure. In the case of file transfer, segmenting of the file will be carried out according to the length of  the segment and the transfer procedure has to be applied accordingly to the segments of files as specified below for the interchange data elements.

## 2.8	Subclause 7.3.5.2 in Recommendation T.433 should be replaced by the following new subclause

#### 7.3.5.2	Use of S-DATA request and indication primitive parametersXE "Use of S-DATA request and indication primitive parameters"§

The document information is divided into segments such that the segment boundaries coincide with the minor synchronization points. Each segment consists of an integral number of interchange data elements, or in the case of file transfer, consists of the data which is subdivided into the segment of size indicated by the user. The interchange data elements of each segment are encoded using the Basic Encoding Rules defined in Recommendation X.209. The encoded interchange data elements of each segment are concatenated, forming an encoded segment.

## 2.9	Figure 3 should be replaced with  following new figure

S-ACTIVITY-START-user-data ::= CHOICE { [4] IMPLICT DocumentCharacteristics } DocumentCharacteristics ::= SET { documentApplicationProfile ::= CHOICE { [0] IMPLICIT OCTET STRING, -- '01' H Non-DocumentApplicationProfile -- '02' H DocumentApplicationProfile T.503 -- '03' H DocumentApplicationProfile T.501 -- Other values are assigned by Recommendation T.563 [4] IMPLICIT SET OF OBJECT IDENTIFIER }, documentArchitectureClass [1] IMPLICIT OCTET STRING OPTIONAL, -- '00' H means FDA nonBasicDocumentCharacteristics [2] IMPLICIT NonBasicDocumentCharacteristics OPTIONAL, nonBasicStructuralCharacteristics [3] IMPLICIT NonBasicStructuralCharacteristics OPTIONAL, fileTransferCapabilities [10] IMPLICIT FileTransferCapabilities OPTIONAL } NonBasicDocumentCharacteristics ::= SET { commentsCharacterSets [1] IMPLICIT OCTET STRING OPTIONAL, -- string of escape sequences pageDimensions [2] IMPLICIT SET OF Dimension-Pair OPTIONAL, ra-gr-coding-attributes [3] IMPLICIT SET OF Ra-Gr-Coding-Attribute OPTIONAL, -- Ra-Gr-Coding-Attribute is defined in -- Recommendation T.415 ra-gr-presentation-features [4] IMPLICIT SET OF Ra-Gr-Presentation-Features OPTIONAL -- Ra-Gr-Presentation-Features is defined in -- Recommendation T.415 NonBasicStructuralCharacteristics ::= SET { numberOfObjectPerPage [0] IMPLICIT INTEGER OPTIONAL }

FIGURE 3/T.433

User data in S-ACTIVITY-START/RESUME

## 2.10	Modules to be imported in DTAM APDU defined in 8.1 should be replaced by the following new modules

IMPORTS

-- ODA --

DocumentProfileDescriptor, LayoutClassDescriptor, TextUnit,

LogicalClassDescriptor, LogicalObjectDescriptor, LayoutStyleDescriptor,

DocumentApplicationProfile, NonBasicDocCharacteristics,

NonBasicStrucCharacteristics, AdditionalDocCharacteristics

FROM Interchange-Data-Elements{ joint-iso-ccitt 8 1 5 5 }

## 2.11	Abstract syntax definition of ApplicationCapabilities used in DTAM normal mode should be replaced by the following new definition

ApplicationCapabilities		::= CHOICE {

oDAApplicationCapabilities	[0] IMPLICIT SET {

oDADocumentApplicationProfile	[0] IMPLICIT OBJECT IDENTIFIER,

nonBasicODADocumentCharacteristics	[1] NonBasicDocCharacteristics OPTIONAL,

nonBasicODAStructuralCharacteristics	[2] NonBasicStrucCharacteristics OPTIONAL,

operationalApplicationProfile	[3] IMPLICIT SET OF OBJECT IDENTIFIER
					OPTIONAL

additionalDocCharacteristics	[4] IMPLICIT AdditionalDocCharacteristics
					OPTIONAL,
					}

fileTransferCapabilities		[1] IMPLICIT INTEGER {

bftCapabilities	(0),

transparentDataCapabilities	(1),

dtmCapabilities	(2),

ediCapabilities	(3)

}

}

## 2.12	Subclause 8.2 should be replaced by the following new subclause

## 8.2	Abstract syntax definition of APDUs for use of the session service in Transparent ModeXE "Abstract syntax definition of APDUs for use of the session service in Transparent Mode"§

This abstract syntax is described by the notation of ASN.1 defined in the Recommendation X.208.

[1] D-INITIATE-REQ ::= CHOICE {

[4] IMPLICIT ApplicationCapabilities }

ApplicationCapabilities		::= SET {

documentApplicationProfileT73	[0] IMPLICIT OCTET STRING OPTIONAL,

-- '01' H Non-DocumentApplicationProfile

-- '02' H DocumentApplicationProfile T.503

-- '03' H DocumentApplicationProfile T.501

-- Other values are assigned by Recommendation T.563

documentArchitectureClass	[1] IMPLICIT OCTET STRING OPTIONAL }

-- '00'H means FDA

[2] D-INITIATE-RES ::= CHOICE {

[4] IMPLICIT ApplicationCapabilities }

ApplicationCapabilities	::= SET {

documentApplicationProfileT73	[0] IMPLICIT OCTET STRING OPTIONAL,

-- '01' H Non-DocumentApplicationProfile

-- '02' H DocumentApplicationProfile T.503

-- '03' H DocumentApplicationProfile T.501

-- Other values are assigned by Recommendation T.563

documentArchitectureClass	[1] IMPLICIT OCTET STRING OPTIONAL }

-- '00'H means FDA

[3] D-CAPABILITY-REQ ::= CHOICE {

[4] IMPLICIT ApplicationCapabilities }

ApplicationCapabilities ::= SET {

documentApplicationProfileT73	[0] IMPLICIT OCTET STRING OPTIONAL,

documentArchitectureClass	[1] IMPLICIT OCTET STRING OPTIONAL,

nonBasicDocCharacteristics	[2] IMPLICIT NonBasicDocCharacteristics 
							OPTIONAL,

nonBasicStrucCharacteristics	[3] IMPLICIT NonBasicStrucCharacteristics
							OPTIONAL,

additionalDocCharacteristics	[9] IMPLICIT AdditionalDocCharacteristics
							OPTIONAL,

fileTransferCapabilities	[10] IMPLICIT SET OF FileTransferCapabilities
							OPTIONAL,

privateCapabilities		[11] IMPLICIT OCTET STRING OPTIONAL }

-- "NonBasicDocCharacteristics", "NonBasicStrucCharacteristics" and

-- "AdditionalDocCharacteristics" are imported from Recommendation T.415

[4] D-CAPABILITY-RES ::= CHOICE {

[4] IMPLICIT ApplicationCapabilities }

ApplicationCapabilities	::= SET {

documentApplicationProfileT73	[0] IMPLICIT OCTET STRING OPTIONAL,

documentArchitectureClass	[1] IMPLICIT OCTET STRING OPTIONAL,

nonBasicDocCharacteristics	[2] IMPLICIT NonBasicDocCharacteristics
							OPTIONAL,

nonBasicStrucCharacteristics	[3] IMPLICIT NonBasicStrucCharacteristics
							OPTIONAL,

additionalDocCharacteristics	[9] IMPLICIT AdditionalDocCharacteristics
							OPTIONAL,

fileTransferCapabilities	[10] IMPLICIT SET OF FileTransferCapabilities
							OPTIONAL,

privateCapabilities		[11] IMPLICIT OCTET STRING OPTIONAL }

FileTransferCapabilities ::= INTEGER {

bftCapabilities		(0),

transparentDataCapability	(1),

dtmCapabilities		(2),

ediCapabilites 		(3) }