|  | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

ITU-T	T.521

TELECOMMUNICATION	(11/94)
STANDARDIZATION  SECTOR
OF  ITU

TERMINALS  FOR  TELEMATIC  SERVICES

COMMUNICATION  APPLICATION
PROFILE  BT0  FOR  DOCUMENT
BULK  TRANSFER  BASED  ON
THE  SESSION  SERVICE

ITU-T  Recommendation  T.521

(Previously “CCITT  Recommendation”)

# FOREWORD

The ITU-T (Telecommunication Standardization Sector) is a permanent organ of the International Telecommunication Union (ITU). The ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Conference (WTSC), which meets every four years, establishes the topics for study by the ITU-T Study Groups which, in their turn, produce Recommendations on these topics.

The approval of Recommendations by the Members of the ITU-T is covered by the procedure laid down in WTSC Resolution No. 1 (Helsinki, March 1-12, 1993).

ITU-T Recommendation T.521 was revised by ITU-T Study Group 8 (1993-1996) and was approved under the WTSC Resolution No. 1 procedure on the 11th of November 1994.

___________________

NOTE

this Recommendation, the expression “Administration” is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

ã  ITU  1995

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

CONTENTS

Recommendation T.521     (11/94)

Page

1	Scope and field of application		1

2	References		1

3	Definitions		1

4	Abbreviations and conventions		1

5	Definition of communication application profile BT0		1

5.1	Overview of BT0		1

5.2	DTAM functional units		2

5.3	DTAM service primitives and lower layer mapping		2

6	Document interchange data structures		9

7	Document transfer		9

7.1	Synchronization		9

7.2	Document recovery		9

Appendix I – Overall Protocol Sequence		9

Appendix II – Guidance for mapping of DTAM to Recommendation T.62		13

Introduction		13

II.1	Items to be considered when using DTAM on top of Recommendation T.62		13

SUMMARY

Recommendation this defines communication application profile for G4 facsimile apparatus. The definition of «additional document characteristics» in «application capabilities» for colour and other extension is added as an option in this revision.

Recommendation T.521

Recommendation T.521     (11/94)

communication  application  profile  bt0  for  document
bulk  transfer  based  on  the  session  service

(Geneva, 1992; revised in Geneva 1994)

# 1	Scope and field of application

This Recommendation defines the communication application profile for the document bulk transfer using the session service defined in Recommendation T.62 bis in terms of:

a)	DTAM functional units used;

b)	DTAM service primitives and parameters used;

c)	lower layer X.215 session service mapping according to the rules of Recommendation T.62 bis.

# 2	References

The following references are required in order to implement the communication profile defined in this Recommendation.

–	CCITT Recommendation T.431 (1992), Document Transfer and Manipulation (DTAM) – Services and Protocol – Introduction and General Principles.

–	CCITT Recommendation T.432 (1992), Document Transfer and Manipulation (DTAM) – Services and Protocols – Service Definition.

–	CCITT Recommendation T.433 (1992), Document Transfer and Manipulation (DTAM) – Services and Protocols – Protocol Specification.

–	Recommendation T.62 (1993), Control procedures for teletex and group 4 facsimile services.

–	Recommendation T.62 bis (1993), Control procedures for teletex and G4 facsimile services based on Recommendations X.215 and X.225.

–	Recommendation X.215 (1994), Information technology – Open Systems Interconnection – Session service definition.

# 3	Definitions

The definitions in the T.400-Series Recommendations and Recommendation T.62 bis also apply to this Recommendation.

# 4	Abbreviations and conventions

The abbreviations and conventions defined in the T.400-Series Recommendations and Recommendation T.62 bis also apply to this Recommendation.

# 5	Definition of communication application profile BT0XE "communication application profile BT0"§

## 5.1	Overview of BT0

This Recommendation defines functional units and communication support functions in accordance with Recommendation T.431. BT0 uses the document bulk transfer in transparent mode (mapping to session service as defined in the T.430-Series Recommendations).

## 5.2	DTAM functional unitsXE "DTAM functional units"§

The following functional units defined in Recommendation T.432 are used for BT0:

–	association use control (Kernel);

–	capability;

–	document bulk transfer;

–	token control.

NOTE – The capability functional unit must be supported in this profile, but it is not required to use this service prior to every document transfer (D-TRANSFER).

## 5.3	DTAM service primitives and lower layer mappingXE "DTAM service primitives and lower layer mapping"§

### 5.3.1	DTAM service primitives and parametersXE "DTAM service primitives and parameters"§

The general DTAM service definition and parameters are defined in Recommendation T.432. This subclause specifies the parameters of the DTAM service for BT0.

#### 5.3.1.1	D-INITIATE service parametersXE "D-INITIATE service parameters"§

The following parameters of this service are used as follows:

–	transparent mode;

–	telematic requirements;

–	application capabilities;

–	result.

Table 1-A lists the D-INITIATE service parameters.

1)	Transparent mode

This parameter shall be present in the D-INITIATE request service primitive.

2)	Telematic requirements

The following functional units defined in Recommendation T.432 are used for BT0 as mandatory functional units:

–	association use control (Kernel);

–	capability;

–	document bulk transfer;

–	token control.

3)	Application capabilities

This “Application capabilities” parameter is defined in Recommendation T.432 and the following sub-parameter is used:

ODA Application capabilities:

a)	Document application profile

The value of this parameter indicates the document application profile being used. Its value is specified in Recommendations that define terminal characteristics for particular Telematic services.

b)	Document architecture class

The value of this parameter indicates the document architecture class used in the entire association. The use of this parameter and its possible value is specified in the Recommendations that define terminal characteristics for particular Telematic services.

4)	Result

This result field can take the values defined in Recommendation T.432.

TABLE  1-A/T.521

D-INITIATE service parameters

|  | D-INITIATE request | D-INITIATE indication | D-INITIATE response | D-INITIATE confirm |
| --- | --- | --- | --- | --- |

| Transparent mode | M |  |  |  |
| --- | --- | --- | --- | --- |

| - Telematic requirements | M | M(=) | Ca) | C(=) |
| --- | --- | --- | --- | --- |

| ODA Application capabilities - ¦ Document application profile ¦ ¦ Document architecture class | M M M | M(=) M(=) M(=) | Ca) Ca) Ca) | C(=) C(=) C(=) |
| --- | --- | --- | --- | --- |

| - Result |  |  | M | M(=) |
| --- | --- | --- | --- | --- |

a) This parameter is mandatory in case the responder returns the “accepted” result parameter to the proposed requirements.

#### 5.3.1.2	D-TERMINATE service parametersXE "TERMINATE service parameters"§

This service has no parameter for BT0. Only the initiator can issue D-TERMINATE service. In addition, the initiator can issue D-TERMINATE service only if he has the data token.

#### 5.3.1.3	D-U-ABORT service parameters

This service has no parameter.

#### 5.3.1.4	D-P-ABORT service parameter

This service has no parameter.

#### 5.3.1.5	D-CAPABILITY service parametersXE "D-CAPABILITY service parameters"§

This service has the sub-parameter “ODA Application capabilities” which consists of the following parameters:

–	document application profile;

–	document architecture class;

–	non-basic document characteristics;

–	additional document characteristics.

Table 1-B lists the D-CAPABILITY service parameters.

1)	Application capabilities

See 5.3.1.1.

ODA Application capabilities:

a)	Document application profile

See 5.3.1.1.

b)	Document architecture class

See 5.3.1.1.

c)	Non-basic document characteristics

This is the parameter “Non-basic document characteristics” defined in Recommendation T.432.

d)	Additional document characteristics

This is the parameter “Additional document characteristics” defined in Recommendation T414.

TABLE  1-B/T.521

D-CAPABILITY service parameters

|  | D-CAPABILITY request | D-CAPABILITY indication | D-CAPABILITY response | D-CAPABILITY confirm |
| --- | --- | --- | --- | --- |

| ODA Application capabilities - ¦ Document application profile ¦ ¦ Document architecture class ¦ ¦ Non-basic document characteristics ¦ ¦ Additional document characteristics | M M U U | M(=) M(=) C(=) C(=) | M M U U | M(=) M(=) C(=) U |
| --- | --- | --- | --- | --- |

#### 5.3.1.6	D-TRANSFER service parameters

This service has the following parameters:

1)	Document information

This parameter consists of the interchange data elements representing the document. The “Document characteristics” from the document profile are transferred using S-ACT-START service.

All interchange data elements, except the document profile descriptor, are transferred using S-DATA services. The document profile is reconstructed by the received DTAM-PM, on the basis of the “Document characteristics” transferred in the S-ACT-START service.

2)	Document information type

This parameter has the value “transfer of a document from its beginning” in the request primitive (see Recommendation T.432). For the indication and confirm primitive, the value is either “transfer completed” or “transfer not completed”.

3)	Document reference information

The value of this parameter is to be provided by the DTAM user in accordance with the rules specified in Recommendation T.432.

4)	Result

This parameter has one of the values “document information transferred” or “document information not completely transferred”, as defined in Recommendation T.432.

NOTE – The parameter value “document-information-continuation not possible” is for further study.

5)	Checkpoint value

This parameter is used as defined in Recommendation T.432. The number of IDE per segment is integer 2. However, the document profile and the document root are not counted (see Figure 1).

FIGURE 1/T.521...[D01] = PAGE PLAINE

The following restriction is applied to the transfer syntax coding rules defined in Recommendation X.209 for the interchange of the document application profile defined in Recommendation T.503:

–	Length fields longer than three octets shall not be used. A length field of three octets allows for the representation of a length of up to 65 535. A data element with a length exceeding 65 535 shall have a length field of the indefinite form.

Table 1-C lists the D-TRANSFER service parameters.

TABLE  1-C/T.521

D-TRANSFER service parameters

| Parameter | D-TRANSFER request | D-TRANSFER indication | D-TRANSFER confirm |
| --- | --- | --- | --- |

| - Document information | M | M(=) |  |
| --- | --- | --- | --- |

| - Document information type | M | M | M |
| --- | --- | --- | --- |

| - Document reference information | M | M(=) | M(=) |
| --- | --- | --- | --- |

| - Result |  |  | M |
| --- | --- | --- | --- |

| - Checkpoint value | M |  |  |
| --- | --- | --- | --- |

NOTE – The document information is transferred using the type of normal document.

#### 5.3.1.7	D-CONTROL GIVE service parameters

The D-CONTROL GIVE service surrenders all available tokens and has no parameter.

#### 5.3.1.8	D-TOKEN PLEASE service parameters

The D-TOKEN PLEASE service is used to request the data token and has no parameter.

### 5.3.2	Use of session service and parameter mappingXE "Use of session service and parameter mapping"§

#### 5.3.2.1	DTAM protocol mapping to X.215 session serviceXE "DTAM protocol mapping to X.215 session service"§

This mapping rule is defined in 7.3/T.433.

NOTE – D-TRANSFER confirm is implicitly formed by receipt of S-ACT END confirm, S-ACT INT confirm and S-ACT DCAD confirm.

#### 5.3.2.2	DTAM parameter mapping to session service parameters

Table 2-A shows the mapping rule between DTAM service parameters and basic and additional session service parameters.

The category of parameters is defined as follows:

–	1	parameters are generated by DTAM user;

–	2	parameters are generated by DTAM provider;

–	3	parameters provided by other means than DTAM-user or DTAM-provider.

Appendix I illustrates the example of protocol sequences for BT0.

TABLE  2-A/T.521

D-INITIATEXE "D-INITIATE"§

| DTAM service parameters | Basic and additional session service parameters | Presence in session protocol | Category |
| --- | --- | --- | --- |
| Telematic requirements | Session requirements | m (defaultable) | 1 |
| ODA – Application capabilities ¦ Document application profile ¦ ¦ Document architecture class | Session user data | m | 1 |

| - None | Session reference Window size Service identifier Inactivity timer | m m m nm | 2 |
| --- | --- | --- | --- |

| - None | Private use Non-standardized capabilities | nm nm | 3 |
| --- | --- | --- | --- |

m Mandatory nm No mandatory

TABLE  2-B/T.521

D-TERMINATEXE "D-TERMINATE"§

| DTAM service parameters | Basic and additional session service parameters | Presence in session protocol | Category |
| --- | --- | --- | --- |
| None | Session termination parameter | nm | 3 |

TABLE  2-C/T.521

D-U-ABORTXE "D-U-ABORT"§

| DTAM service parameters | Basic and additional session service parameters | Presence in session protocol | Category |
| --- | --- | --- | --- |
| None | Session termination parameter (Transport disconnect) | m | 3 |

TABLE  2-D/T.521

D-CAPABILITYXE "D-CAPABILITY"§

| DTAM service parameters | Basic and additional session service parameters | Presence in session protocol | Category |
| --- | --- | --- | --- |
| ODA Application capabilities ¦ Document application profile ¦ ¦ Document architecture class ¦ ¦ Non-basic document ¦ characteristics | Session user data | m - m m nm | 1 |

| - None | Inactivity timer Storage capacity | - nm nm | 2 |
| --- | --- | --- | --- |

| - None | Private use (Note) Non-standardized capabilities | - nm nm | 3 |
| --- | --- | --- | --- |

NOTE – In case of file transfer, this parameter is used in accordance with the rules defined in Recommendation T.571. The use for facsimile Group 4 is for further study.

TABLE  2-E/T.521

D-TRANSFERXE "D-TRANSFER"§

| DTAM service parameters | Basic and additional session service parameters | Presence in session protocol | Category |
| --- | --- | --- | --- |
| Document information | Session user data | (Note) | 1 |
| Document reference information | Document reference number | (Note) | 1 |
| NOTE – There is no direct mapping of the D-TRANSFER primitive to one single session primitive. |  |  |  |

TABLE  2-F/T.521

D-TOKEN PLEASEXE "D-TOKEN PLEASE"§

| DTAM service parameters | Basic and additional session service parameters | Presence in session protocol | Category |
| --- | --- | --- | --- |
| None | Token | m | 2 |

# 6	Document interchange data structuresXE "Document interchange data structures"§

The interchange representation of a document has to be defined in accordance with the Recommendation hich specifies the relevant document application profile.

# 7	Document transfer

## 7.1	Synchronization

The document information is divided into segments, in accordance with 7.3.5/T.433, such that each segment contains a one page descriptor and the associated content portion. A minor synchronization point is associated with each segment.

## 7.2	Document recovery

For further study.

Appendix  I
(to Recommendation T.521)

Overall Protocol Sequence

(This appendix does not form an integral part of this Recommendation)

This is an informative appendix for:

–	normal procedure;

–	token control procedure;

–	abort procedure;

–	exception report procedure.

FIGURE 1-1/T.521...[D02] = PAGE PLAINE

FIGURE 1-2/T.521...[D03] = 8 CM

FIGURE 1-3/T.521...[D04] =  10 CM

FIGURE 1-4/T.521...[D05] =  PAGE PLAINE

Appendix  II
(to Recommendation T.521)

Guidance for mapping of DTAM to Recommendation T.62XE "Guidance for mapping of DTAM to Recommendation T.62"§

(This appendix does not form an integral part of this Recommendation)

# Introduction

This appendix provides an implementor with a guideline when a mapping of a facsimile Group 4 application based on DTAM to Recommendation T.62 is intended.

The main part of Recommendation T.521 defines the basic mapping of DTAM to the session service, described in Recommendation X.215 and Recommendation X.225 in conjunction with Recommendation T.62 bis.

For the mapping of DTAM onto the T.62 control procedures, further clarifications and explanations are needed.

The clarifications and explanations are considered within two parts.

Part 1 considering the DTAM to session service mapping, whereas part 2 describes the mapping of DTAM to Recommendation T.62 in terms of using the T.62 session commands and responses.

Figure II.1 depicts the scope of this appendix.

FIGURE II.1/T.521...[D06] = 9 CM

## II.1	Items to be considered when using DTAM on top of Recommendation T.62

### II.1.1	Mapping of DTAM to the session serviceXE "Mapping of DTAM to the session service"§

The mapping of DTAM to the session service is described as the “Transparent Mode” within Recommendations T.432 and T.433.

The specific session service parameter mapping is defined in this Recommendation.

With the exception of two cases both are equivalent, the mapping of DTAM to T.62 and the mapping of DTAM to X.215 and X.225 (according to Recommendation T.62 bis).

The two exceptions are the S-U-ABORT and the S-CONTROL-GIVE service. Both services are confirmed services within Recommendation T.62, but unconfirmed service according to Recommendation X.215. The terminology of the session service primitives are aligned with the session service according to Recommendation X.215.

In order to guarantee a correct mapping at the DTAM and T.62 interface, an “Adaptation Management Function (AMF)” is introduced to perform this task.

#### II.1.1.1	U-ABORT serviceXE "U-ABORT service"§

The situation envisaged is as follows.

FIGURE /T.521...[D07] = 4 CM

The DTAM-PM neither knows an S-U-ABORT confirm primitive nor does it issue an S-U-ABORT response primitive. Therefore, the “Adaptation Management Function (AMF)” is responsible for the mapping.

##### II.1.1.1.1	  Mapping rules

An S-U-ABORT request from the DTAM-PM is transparently transferred through the AMF to the T.62-PM.

An S-U-ABORT indication generated by the T.62-PM is transparently transferred through the AMF to the DTAM-PM.

An S-U-ABORT response will not be generated by the DTAM-PM. Therefore, the AMF is in charge of generating an S-U-ABORT response primitive to the T.62-PM in the adequate situation (i.e. after the receipt of an S-U-ABORT indication).

An S-U-ABORT confirm is not known by the DTAM-PM. Therefore, the AMF is in charge of ignoring the S-U-ABORT confirm primitive which will be generated by the T.62-PM (i.e. after the receipt of an ABORT ACCEPT (RSAP) SPDU).

### II.1.1.2	S-CONTROL-GIVE serviceXE "S-CONTROL-GIVE service"§

The situation envisaged is as follows.

FIGURE /T.521...[D08] = 4 CM

The DTAM-PM neither knows an S-CONTROL-GIVE confirm primitive nor does it issue an S-CONTROL-GIVE response primitive. Therefore, the “Adaptation Management Function (AMF)” is responsible for the mapping.

##### II.1.1.2.1	  Mapping rules

An S-CONTROL-GIVE request from the DTAM-PM is transparently transferred through the AMF to the T.62-PM.

An S-CONTROL-GIVE indication generated by the T.62-PM is transparently transferred through the AMF to the DTAM-PM.

An S-CONTROL-GIVE confirm is not known by the DTAM-PM. Therefore, the AMF is in charge of ignoring the S-CONTROL-GIVE. confirm primitive which will be generated by the T.62-PM (i.e. after the receipt of an S-CONTROL-GIVE indication).

An S-CONTROL-GIVE confirm is not known by the DTAM-PM. Therefore, the AMF is in charge of ignoring the S-CONTROL-GIVE confirm primitive which will be generated by the T.62-PM (i.e. after the receipt of a GIVE TOKENS ACK (RSCCP) SPDU).

### II.1.2	Use of T.62 commands and responses

This part of the appendix is related to T.62 session commands and responses in order to point out:

–	in which session parameter the application dependent information is transferred;

–	references to Recommendation T.433 where the application relevant information can be found;

–	the use of Recommendation T.62 in relation to multifunctional equipment (e.g. TTX and G4 facsimile);

–	important communication phases.

#### II.1.2.1	Session establishment (CSS/RSSP)XE "Session establishment (CSS/RSSP)"§

For CSS/RSSP the application capabilities within the Session User Data (SUD) parameter shall indicate the document architecture class and the document application profile T.73 which is available as receiving capability of the sender of the command/response.

#### II.1.2.2	Capability negotiation (CDLC/RDCLP)XE "Capability negotiation (CDLC/RDCLP)"§

For CDCL, the application capabilities indicated within the SUD, should, beside the document architecture class and document application profile T.73, include a list of optional receiving capabilities, such as non-basic document characteristics and/or non-basic structural characteristics, that may be needed at the receiver by the sender of the command.

For RDCLP, the optional application capabilities available should be indicated.

Multifunctional terminals which may support the Teletex and facsimile Group 4 application shall:

–	negotiate options of the basic Teletex document transfer using the non-basic Teletex capabilities PGI and not the SUD PGI;

–	not use the SUD PGI and the non-basic Teletex capabilities PGI together in a single exchange of CDCL/RDCLP (CSS/RSSP).

#### II.1.2.3	Document transfer (CDS/CDC)

For CDS/CDC, the application capabilities indicated within the SUD shall include the application capabilities which are required for the document.

Multifunctional terminals which may support Teletex and facsimile Group 4 application shall use the CDS/CDC command as follows:

–	when a facsimile Group 4 document is to be sent, the SUD parameter is always contained in the CDS/CDC of the document;

–	when a basic Teletex document is to be sent, the SUD is absent from the CDS/CDC of the document.

The coding of the application capabilities attributes conveyed in the SUD of CSS/RSSP, CDCL/RDCLP are specified in 8.2/T.433; and for the SUD of CDS/CDC it is defined in Figure 3/T.433.