|  | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

ITU-T	U.70

TELECOMMUNICATION

STANDARDIZATION SECTOR

OF  ITU

TELEGRAPH  SWITCHING

INTERWORKING  BETWEEN  NEW  INFORMATION  SERVICES  AND  TELEX

TELEX  SERVICE  SIGNALS  FOR  TELEX  
TO  TELETEX  INTERWORKING

ITU-T  Recommendation  U.70

(Extract from the Blue Book)

# NOTES

1	ITU-T Recommendation U.70 was published in Fascicle VII.2 of the Blue Book. This file is an extract from the Blue Book. While the presentation and layout of the text might be slightly different from the Blue Book version, the contents of the file are identical to the Blue Book version and copyright conditions remain unchanged (see below).

2	In this Recommendation, the expression “Administration” is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

ã  ITU  1988, 1993

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

Recommendation U.70

Fascicle VII.2 - Rec. U.70

TELEX SERVICE SIGNALS FOR

TELEX TO TELETEX INTERWORKING

(Malaga-Torremolinos, 1984)

The CCITT,

considering

(a)	that a basic interworking on international connections will be at 50 bauds using an international telex network and that any telex subscriber can call conversion facilities in various countries;

(b)	that Recommendation F.201 specifies:

-	that a validation of a called Teletex terminal is mandatory.  The validation is performed either by a direct validation call or by a data base access;

-	that if a message delivery to a Teletex terminal fails in the case of interworking with two-stage selection procedure, the conversion facility should send a non-delivery notification to a telex terminal;

(c)	that Recommendation S.90 specifies Teletex requirements for interworking with the telex service and Recommendations S.62 and S.70 specify control procedures and a network-independent basic transport service for a Teletex;

(d) that Recommendation X.96 specifies call progress signals in a public data network,

unanimously declares the following view

1	Scope

This Recommendation defines service signals which are to be sent back to the telex terminal in the event of unsuccessful validation for called Teletex terminal addresses, and service signals of the last delivery attempt to the Teletex terminal which are to be sent to the telex terminal as a part of non-delivery notification.

TABLE 1/U.70

Service signals for unsuccessful validation call and non-delivery notification

| Abnormal conditions |  |  |  |  |  |  |  | Service signals |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Call-progress signals |  |  |  |  |
|  |  |  |  |  | No connection |  |  | NC |
|  |  |  |  |  | Selection signal transmisssion error |  |  | NC |
|  |  |  |  |  | Local procedure error |  |  | NC |
|  |  |  |  |  | Network congestion |  |  | NC |
|  |  |  |  |  | Invalide facility request |  |  | NC |
|  |  |  |  |  | Change number |  |  | NCH |
|  |  |  |  |  | Not obtainable |  |  | NP |
|  |  |  |  |  | Access barred |  |  | NA |
| Lower |  | PSDN and |  |  | Incompatible user class of service |  |  | NA |
| Level |  | CSDN |  |  | Incompatible destination |  |  | NC |
| Protocols |  |  |  |  | Out of order |  |  | DER |
|  |  |  |  |  | Network fault in local loop |  |  | DER |
|  |  |  |  |  | DCE power off |  |  | ABS |
|  |  |  |  |  | Uncontrolled not ready |  |  | DER |
|  |  |  |  |  | Controlled not ready |  |  | ABS |
|  |  |  |  |  | Number busy |  |  | OCC |
|  |  |  |  |  | Call information service |  |  | INF |
|  |  |  |  |  | Remote procedure error |  |  | DER |
|  |  |  |  |  | Long-term network congestion |  |  | NC |
|  |  |  |  |  | DTE originated |  |  | DER |
|  |  |  |  | Carrier-off |  |  |  | NC |
|  |  |  |  | No response (time-out) |  |  |  | DER |
|  |  |  |  | Carrier off |  |  |  | NC |
|  |  |  |  | Ring-back tone (No reply) a) |  |  |  | ABS |
|  |  |  |  | No dial tone a) |  |  |  | NC |
| Lower |  | PSTN |  | Busy a) |  |  |  | OCC |
| Level |  |  |  | Equipment engaged a) |  |  |  | NC |
|  |  |  |  | Number unobtainable a) |  |  |  | NP |
|  |  |  |  | No tone a) |  |  |  | NP |
|  |  |  |  | Transport Connection Clear (TCC) receipt |  |  |  |  |
|  |  |  |  |  | Reason not specified |  |  | DER |
|  |  |  |  |  | Terminal occupied |  |  | OCC |
|  |  |  |  |  | Terminal out of order |  |  | DER |
|  |  | Transport |  |  | Address unknown |  |  | NP |
|  |  | Layer |  | Transport Block Reject (TBR) receipt |  |  |  |  |
|  |  |  |  |  | Reason not specified |  |  | DER |
|  |  |  |  |  | Function not implemented |  |  | DER |
|  |  |  |  |  | Invalid block |  |  | DER |
|  |  |  |  |  | Invalid parrameter |  |  | DER |
|  |  |  |  | Illegal command/response receipt |  |  |  | DER |
|  |  |  |  | No response |  |  |  | DER |
|  |  |  |  | Incorrect terminal identifier |  |  |  |  |
|  |  |  |  | in Response Session Start Positive (RSSP) |  |  |  | NP |
|  |  |  |  | Response Session Start Negative (RSSN) receipt |  |  |  |  |
| Higher |  | Session |  |  |  |  | No specific reason stated | DER |
| Level |  | Layer |  |  |  |  | Memory full or out of recording paper | DER |
| Protocols |  |  |  |  |  |  | Explicit text message | DER |
|  |  |  |  | Command Session Abort (CSA) receipt |  |  |  |  |
|  |  |  |  |  |  |  | Local terminal error | DER |
|  |  |  |  |  |  |  | Unrecoverable procedural error | DER |
|  |  |  |  |  |  |  | Reason not defined | DER |
|  |  |  |  | Illegal command/response receipt |  |  |  | DER |
|  |  |  |  | No response |  |  |  | DER |
|  |  |  |  | Response Document Page Boundary Negative (RDPBN) receipt |  |  |  |  |
|  |  |  |  |  |  | No specific reason stated |  | DER |
|  |  |  |  |  |  | Memory full or out of recording paper |  | DER |
|  | Document |  |  |  |  | Sequence error |  | DER |
|  | Layer b) |  |  |  |  | Local terminal error |  | DER |
|  |  |  |  |  |  | Unrecoverable procedural error |  | DER |
|  |  |  |  | Response Document General Reject (RDGR) receipt |  |  |  | DER |
|  |  |  |  | Illegal command//reponse receipt |  |  |  | DER |
|  |  |  |  | No response |  |  |  | DER |

a) It is recognized that these abnormal conditions can be detected if the automatic calling procedures incoroporate detection facilites.

b) Optional (if negotiation is not carried out).

PSDN		Packed switched data network

CSDN		Circuit switched data network

PSTN		Public switched telephone network

2	Principles

The following principles should he taken into consideration:

2.1	Call progress signals arriving from the Teletex network will be converted without changing their original meaning as far as possible.

2.2	A faulty condition between the Teletex network and the conversion facility will be regarded as a faulty condition within the network.

2.3	A faulty condition between the Teletex network and the Teletex terminal will be regarded as a fault of the terminal.

3	Telex service signals

In general, telex service signals specified in Recommendation F.60 should be used.  However, the telex service signals listed below will be used in the following circumstances:

3.1	If the format of the Teletex address part is incorrect, the service signal NP will be returned.

3.2	If the direct validation call is unsuccessful, the conversion facility will transmit the service signals specified in Table 1/U.70 to the telex terminal.

3.3	If the validation by data base access is unsuccessful, the conversion facility will transmit the service signals specified in Table 2/U.70.

3.4	If the message delivery to the Teletex terminal is unsuccessful, the conversion facility will transmit the service signals of the last delivery attempt which are specified in Table 1/U.70, as a part of non-delivery notification to the telex terminal.

TABLE 2/U.70

Service signals for unsuccessful

validation by data base access

| Abnormal conditions | Service signal |
| --- | --- |
| Unsuccessful results | NP |

Note – In addition to the case “the number does not exist”, other cases, for example, “the number is being temporarily prohibited” or “the number is out of order”, might be considered as abnormal conditions. But at this moment such conditions and the service signals corresponding to them are for further study.