|  | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

ITU-T	V.31

TELECOMMUNICATION

STANDARDIZATION SECTOR

OF  ITU

DATA  COMMUNICATION  
OVER  THE  TELEPHONE  NETWORK

ELECTRICAL  CHARACTERISTICS  FOR  SINGLE-CURRENT  INTERCHANGE  CIRCUITS  CONTROLLED  BY  CONTACT  CLOSURE

ITU-T  Recommendation  V.31

(Extract from the Blue Book)

# NOTES

1	ITU-T Recommendation V.31 was published in Fascicle VIII.1 of the Blue Book. This file is an extract from the Blue Book. While the presentation and layout of the text might be slightly different from the Blue Book version, the contents of the file are identical to the Blue Book version and copyright conditions remain unchanged (see below).

2	In this Recommendation, the expression “Administration” is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

ã  ITU  1988, 1993

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

Recommendation V.31

Fascicle VIII.1 - Rec. V.31

ELECTRICAL CHARACTERISTICS FOR SINGLE-CURRENT INTERCHANGE

CIRCUITS CONTROLLED BY CONTACT CLOSURE

(Geneva, 1972)

# 1	General

In general, the electrical characteristics specified in this Recommendation apply to interchange circuits operating at data signalling rates up to 75 bit/s.

Each interchange circuit consists of two conductors (go and return leads) which are electrically insulated from each other and from all other interchange circuits. A common return lead can be assigned to several interchange circuits of a group.

# 2	Equivalent circuit of interface

Figure 1/V.31 shows the equivalent interchange circuit, together with the electrical characteristics laid down in this Recommendation. Some electrical characteristics vary depending upon whether the signal receive side is located in the data circuit-terminating equipment or in the data side is located in the data circuit-terminating equipment or in the data terminal equipment. This fact is specially referred to below.

# 3	Signal source

The signal source must be isolated from ground or earth irrespective of whether it is located within the data circuit-terminating equipment or within the data terminal equipment.

If the signal receive side is in the data circuit-terminating equipment, the open-contact insulation resistance measured from either leg to ground or to any other interchange circuit shall not fall below 5 megohms and the capacitance measured between the same points shall not exceed 1000 picofarads.

Irrespective of the above, the following specifications apply to the signal source.

3.1	Internal resistance of signal source R1, R0

The d.c. resistance of the closed contact R1, including the resistance of the interface cable, measured at the interface (see Figure 1/V.31), should not exceed 10 ohms within the current and voltage ranges of the signal receive side.

The d.c. resistance of the open contact R0 including the insulation resistance of the interface cable should not fall below 250 kilohms when measured at the interface (see Figure 1/V.31) within the voltage range of the signal receive side.

3.2	Capacitance of signal source Cg

The capacitance of the signal source Cg including that of the interface cable, measured at the interface (see Figure 1/V.31), should not exceed 2500 picofarads.

R1	=	internal resistance of the signal source in the closed contact condition

R0	=	internal resistance of the signal source in the open contact condition

Cg	=	capacitance of signal source

Cr		=	capacitance of signal receive side

Vr		=	open circuit voltage of signal receive side

IS		=	current in interchange circuit

Rr		=	internal resistance of signal receive side

RE	=	insulation resistance of signal source if the latter is in the data terminal equipment

CE	=	ground capacitance of signal source if the latter is in the data terminal equipment

FIGURE 1/V.31

Equivalent circuit of interface

4	Signal receive side

4.1	Signal receive side in the data circuit-terminating equipment

The signal receive side in the data circuit-terminating equipment can be floating or connected to ground at  any single point.

4.1.1	Open circuit voltage of the signal receive side Vr

The open circuit voltage Vr on the signal receive side of the data-circuit terminating equipment, measured  at the interface (see Figure 1/V.31), should not fall below 3 volts and should not exceed 12 volts.

4.1.2	Current at interface IS

The current IS supplied by the signal receive side in the data circuit-terminating equipment should not fall below 0.1 milliamp and should not exceed 15 milliamps, when measured at the interface (see Figure 1/V.31) in the closed contact condition, i.e. with an internal resistance of the signal source of R1 £ 10 ohms.

Note - Irrespective of the current IS, in the closed contact conditions, i.e. with an internal resistance of the signal source of R1 £ 10 ohms, the voltage at the interface should not exceed 150 millivolts, when measured between go and return leads.

4.1.3	Internal resistance of signal receive side Rr

The internal resistance Rr of the signal receive side of the data circuit-terminating equipment results from the limits for the open circuit voltage Vr of the signal receive side and the current IS, at the interface, which are specified under §§ 4.1.1 and 4.1.2 above.

Even if Rr has an inductive component, the voltage at the interface should not exceed the maximum of 12 volts specified under § 4.1.1 above.

Note - This item is subject to further study.

4.1.4	Capacitance of signal receive side Cr

The capacitance of Cr of the signal receive side in the data circuit-terminating equipment, including the capacitance of the cable up to the interface (see Figure 1/V.31), is not specified. It is only necessary to ensure that the signal receive side works satisfactorily, allowing for the capacitance of the signal source Cg.

4.2	Signal receive side in the data terminal equipment

The signal receive side in the data terminal equipment can be connected to ground at any single point.

4.2.1	Open circuit voltage of the signal receive side Vr

The open circuit voltage Vr of the signal receive side of the data terminal equipment, measured at the interface (see Figure 1/V.3 1), should not fall below 3 volts and should not exceed 52.8 volts.

4.2.2	Current at the interface IS

The current IS, supplied by the signal receive side in the data terminal equipment, should not fall below 10 milliamps and not exceed 50 milliamps, when measured at the interface (see Figure 1/V.3 1) in the closed contact condition, i.e. with an internal resistance of the signal source of R1 £ 10 ohms.

4.2.3	Internal resistance of signal receive side Rr

The internal resistance Rr of the signal receive side in the data terminal equipment is obtained from the limits for the open circuit voltage Vr of the signal receive side and the current IS at the interface, which are specified under §§ 4.2.1 and 4.2.2 above.

Even if Rr has an inductive component, the voltage at the interface should not exceed the maximum of 52.8 volts, specified under § 4.2.1.

Note - This item is subject to further study.

4.2.4	Capacitance of signal receive side Cr

The capacitance of Cr of the signal receive side in the data terminal equipment including the capacitance of the cable is not specified. It is only necessary to ensure that the signal receive side works satisfactorily, allowing for the capacitance of the signal source Cg.

5	Signal allocation

Table 1/V.31 shows allocations of digital signals for data, control and timing circuits.

TABLE 1/V.31

|  | Closed contact R1 £ 10 W | Open contact R0 ³ 250 kW |
| --- | --- | --- |
| Data circuits | "1" condition | "0" condition |
| Control and timing circuits | ON condition | OFF condition |