|  | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

ITU-T	V.31 bis

TELECOMMUNICATION

STANDARDIZATION SECTOR

OF  ITU

DATA  COMMUNICATION

OVER  THE  TELEPHONE  NETWORK

ELECTRICAL CHARACTERISTICS  FOR  SINGLE-CURRENT  INTERCHANGE  CIRCUITS  USING  OPTOCOUPLERS

ITU-T  Recommendation  V.31 bis

(Extract from the Blue Book)

# NOTES

1	ITU-T Recommendation V.31 bis was published in Fascicle VIII.1 of the Blue Book. This file is an extract from the Blue Book. While the presentation and layout of the text might be slightly different from the Blue Book version, the contents of the file are identical to the Blue Book version and copyright conditions remain unchanged (see below).

2	In this Recommendation, the expression “Administration” is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

ã  ITU  1988, 1993

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

Recommendation V.31 bis

Fascicle VIII.1 - Rec. V.31 bis

ELECTRICAL CHARACTERISTICS FOR SINGLE-CURRENT

INTERCHANGE CIRCUITS USING OPTOCOUPLERS

(Malaga-Torremolinos, 1984)

# 1	General

In general, the electrical characteristics specified in this Recommendation apply to interchange circuits operating at data signalling rates up to 75 bit/s, which have been covered in Recommendation V.31, and also to interchange circuits operating at data signalling rates up to 1200 bit/s.

The electrical characteristics are specified to provide compatibility with existing equipment according to Recommendation V.31 which meet the voltage and current values given in Table 1/V.31 bis. The resistance values defined in Recommendation V.31 are converted into current and voltage values to meet the optocouplers requirements.

Each interchange circuit consists of two conductors (go and return leads) which are electrically insulated from each other and from all other interchange circuits. A common return lead can be assigned to several interchange circuits of a group.

# 2	Equivalent circuit of interface

Figure 1/V.31 bis shows the equivalent interchange circuit, together with the electrical characteristics laid down in this Recommendation. Some electrical characteristics vary depending upon whether the signal receive side is located in the data circuit-terminating equipment (DCE) or in the data terminal equipment (DTE). This fact is specially referred to below.

# 3	Signal source

The signal source must be isolated from ground or earth irrespective of whether it is located within the data circuit-terminating equipment or within the data terminal equipment.

If the signal receive side is in the data circuit-terminating equipment, the insulation resistance in the ON or OFF condition measured from either leg to ground or to any other interchange circuit shall not fall below 5 megohms and the capacitance measured between the same points shall not exceed 1000 picofarads.

Irrespective of the above, the following specifications apply to the signal source.

3.1	Internal resistance of signal source R1, R0

The d.c. resistance of the signal source in the ON (or 1) condition R1 depends on Vs , Vr and I1 (see Figure 1/V.31 bis). The d.c. resistance of the signal source in the OFF (or 0) condition R0 depends on Vr and I0 (see Figure 1/V.31 bis).

3.2	Current at the interface I0

The current I0, which represents the reverse current of the optocouplers in the OFF condition, should not exceed       10 mA (see Figure 1/V.31 bis).

3.3	Capacitance of signal source

The capacitance of the signal source Cg including that of the interface cable, measured at the interface (see

Figure 1/V.31 bis), should not exceed 2500 pF.

# 4	Signal receive side

4.1	Signal receive side in the DCE

The signal receive side in the DCE can be floating or connected to ground at any single point.

4.1.1	Open circuit voltage of the signal receive side Vr

The open circuit voltage Vr on the signal receive side of the DCE, measured at the interface (see Figure 1/V.31 bis), should not fall below 3 V and should not exceed 25 V. The polarity of Vr has to be chosen for the current direction DTE to DCE in the go lead and DCE to DTE in the return lead.

4.1.2	Current at the interface I1

The current I1 supplied by the receive side in the DCE should not fall below 0.1 mA and should not exceed 5 mA, when measured at the interface in the ON (or 1) condition.

4.1.3	Voltage at the interface Vs

The voltage at the interface Vs, measured between the go and return leads in the ON (or 1) condition, should not exceed 1 V.

4.1.4	Internal resistance of signal receive side Rr

The internal resistance Rr of the signal receive side of the DCE results from the limits for the open circuit voltage Vr of the signal receive side and the current I1 at the interface, which are specified under §§ 4.1.1 and 4.1.2 above.

Even if Rr has an inductive component, the voltage at the interface should not exceed the maximum of 25 V specified under § 4.1.1 above.

4.1.5	Capacitance of signal receive side Cr

The capacitance of Cr , of the signal receive side in the DCE, including the capacitance of the cable up to the interface (see Figure 1/V.31 bis), is not specified. It is only necessary to ensure that the signal receive side works satisfactorily, allowing for the capacitance of the signal source Cg.

4.2	Signal receive side in the DTE

The signal receive side in the DTE can be connected to ground at any single point.

4.2.1	Open circuit voltage of the signal receive side Vr

The open circuit voltage Vr of the signal receive side of the DTE, measured at the interface (see Figure 1/V.31 bis), should not fall below 3 V and should not exceed 25 V. The polarity of Vr has to be chosen for the current direction DTE to DCE in the go lead and DCE to DTE in the return lead.

4.2.2	Current at the interface I1

The current I1, supplied by the signal receive side in the DTE, should not fall below 0.1 mA and should not exceed 15 mA, when measured at the interface (see Figure 1/V.31 bis), in the ON (or 1) condition.

4.2.3	Voltage at the interface Vs

The voltage at the interface Vs, measured between the go and return leads in the ON (or 1) condition, should not exceed 1.5 V.

4.2.4	Internal resistance of signal receive side Rr

The internal resistance Rr of the signal receive side of the DTE results from the limits for the open circuit voltage Vr of the signal receive side and the current I1 at the interface, which are specified under §§ 4.2.1 and 4.2.2 above.

4.2.5	Capacitance of signal receive side Cr

The capacitance of Cr of the signal receive side in the DTE including the capacitance of the cable is not specified. It is only necessary to ensure that the signal receive side works satisfactorily, allowing for the capacitance of the signal source Cg.

# 5	Signal allocation

Table 1/V.31 bis shows allocations of digital signals for data, control and timing circuits.

TABLE 1/V.31 bis

|  | 0.1 mA £ I1 £ 5 mA (15 mA) Vs £ 1 V (1.5 V) | I0£10 mA |
| --- | --- | --- |
| Data circuits | 1 condition | 0 condition |
| Control and timing circuits | ON condition | OFF condition |