|  | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

ITU-T	V.58

TELECOMMUNICATION	(09/94)
STANDARDIZATION  SECTOR
OF  ITU

DATA  COMMUNICATION  OVER
THE  TELEPHONE  NETWORK

MANAGEMENT  INFORMATION  MODEL
FOR  V-SERIES  DCE’S

ITU-T  Recommendation  V.58

(Previously “CCITT  Recommendation”)

# FOREWORD

The ITU-T (Telecommunication Standardization Sector) is a permanent organ of the International Telecommunication Union (ITU). The ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Conference (WTSC), which meets every four years, establishes the topics for study by the ITU-T Study Groups which, in their turn, produce Recommendations on these topics.

The approval of Recommendations by the Members of the ITU-T is covered by the procedure laid down in WTSC Resolution No. 1 (Helsinki, March 1-12, 1993).

ITU-T Recommendation V.58 was prepared by ITU-T Study Group 14 (1993-1996) and was approved under the WTSC Resolution No. 1 procedure on the 20th of September 1994.

___________________

NOTE

In this Recommendation, the expression “Administration” is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

ã  ITU  1994

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

CONTENTS

Recommendation V.58     (09/94)

Page

1	Introduction		1

1.1	Scope		1

1.2	Compliance Requirements		1

1.3	Structure of this Recommendation		2

2	Information Model Overview		2

3	Object Classes		2

3.1	V-Series Call Control Managed Object		2

3.1.1	Managed Object Template		2

3.1.2	Attributes		3

3.1.3	Actions			6

3.1.4	Notifications		7

3.2	V-Series Data Compression Managed Object		7

3.2.1 Managed Object Template		7

3.2.2	Attributes		8

3.2.3	Actions	1		10

3.2.4	Notifications		10

3.3	V-Series DCE Managed Object		10

3.3.1	Managed Object Template		10

3.3.2	Attributes		11

3.3.3	Actions			13

3.3.4	Notifications		15

3.4	V-Series DTE Interface Managed Object		15

3.4.1	Managed Object Template		15

3.4.2	Attributes		16

3.4.3	Actions			24

3.4.4	Notifications		24

3.5	V-Series Error Control Managed Object		24

3.5.1	Managed Object Template		24

3.5.2	Attributes		25

3.5.3	Actions			29

3.5.4	Notifications		29

Page

3.6	V-Series Line Interface Managed Object		30

3.6.1	Managed Object Template		30

3.6.2	Attributes		30

3.6.3	Actions			34

3.6.4	Notifications		34

3.7	V-Series Signal Convertor Managed Object		35

3.7.1	Managed Object Template		35

3.7.2	Attributes		36

3.7.3	Actions			44

3.7.4	Notifications		45

3.8	V-Series Test Function Managed Object		46

3.8.1	Managed Object Template		46

3.8.2	Attributes		46

3.8.3	Actions			48

3.8.4	Notifications		50

3.9	Local defined types		50

4	Relationships between managed object classes		51

Appendix I – Some additional background information		53

Appendix II – List of Attributes, Actions and Notifications		54

Appendix III – References		60

Recommendation V.58

Recommendation V.58     (09/94)

MANAGEMENT  INFORMATION  MODEL  FOR  V-SERIES  DCE’S

(Geneva, 1994)

The ITU-T,

considering

(a)	that V-Series Recommendations form a set of specifications for DCEs including the definition of external interfaces, basic and special functions and testing facilities;

(b)	that it is expected that further Recommendation will be published specifying newer DCEs, which offer enhanced management capabilities;

(c)	that Recommendation M.3010 defines the principles for a Telecommunications Management Network (TMN);

(d)	that Recommendation G.773 defines the protocol suites for Q-interfaces;

(e)	that Recommendation M.3100 defines a Generic Network Information Model for the exchange of management information;

recommends

that the management of V-Series DCEs be carried out using the information model defined in accordance with the details contained in this Recommendation.

# 1	Introduction

## 1.1	Scope

This Recommendation provides an information model for the V-Series DCEs. It identifies the Telecommunications Management (TMN) object classes required for the management of Network Elements (NEs) defined by the Recommendations of the V-Series. These objects are relevant to information exchanged across standardized interfaces defined in Recommendation M.3010 TMN architecture. The managed object classes in this Recommendation are specialized from the generic managed object classes defined in Recommendation M.3100 Generic Network Information model.

This Recommendation does not define:

–	the means by which management information may be exchanged among network elements;

–	the network level management process.

The information model defined in this Recommendation is concerned with the management of network elements, the equipment by which they are implemented and the functions contained within them. More precisely, it applies to the Equipment Domain visible at the Element Manager to Element interface, and is only concerned with the information available within that domain. Information proper to the domain of a Network Level Management Process is not included within this model.

## 1.2	Compliance Requirements

The Recommendation does not require DCE behaviour that is inconsistent with other V-Series Recommendations, or with national regulatory requirements, and shall be interpreted accordingly.

In order to be compliant with this Recommendation an implementation must:

–	support at least the vSeriesDce Managed Object,

–	if a Managed Object is supported, then at least the associated mandatory package shall be supported;

–	if a package is supported, then all attributes, actions and notifications within the package shall be supported. If any of these relate to functions that are not implemented within the DCE, then any attempt to use the attribute or action will result in no change to the state of the DCE.

## 1.3	Structure of this Recommendation

Clause 2 provides an overview of the information model in this Recommendation. Clause 3 describes the information model using the notation mechanisms in Recommendation X.722 Guidelines for the Definition of Managed Objects. The relationships between managed object classes contained in this Recommendation are defined in clause 4.  Appendix I contains some background information.

# 2	Information Model Overview

The V-Series DCEs information model is based on the Generic Network Information model of Recommen-dation M.3100. The information exchanged at a management interface is modelled using design principles outlined in Recommendation X.720 Management Information Model. Resources are modelled as objects, and the management view of a resource is a managed object. Objects with similar attributes may be grouped into Object classes. An object is characterized by its object class and object instance, and may posses multiple attribute types and associated values. The terms “managed object class” and “managed object instance” apply specifically to objects that are being managed. This Recommendation specifies the properties of the resource visible for management.

An object class may be a subclass of another Class. A subclass inherits attribute types, packages and behaviour of the super class, in addition to possessing its own specific attributes and properties. The object classes specific to the V-Series DCEs are all derived from super classes in the Generic Network Information Model Recommendation M.3100.

# 3	Object Classes

Object classes and attribute types are defined only for the purpose of communicating network management messages between systems, and need not be related to the structure of data within those systems.

NOTES

1	Where BIT STRING is used to identify options or selections a 0 indicates non-availability or disabled, a 1 indicates availability or enabled.

2	The request and confirm arguments of an Action are named actionNameReq and actionNameConf respectively.

3	The CallProgress, DceBitrate, DceBitrateRange, DteBitrate, EventPriority and PhoneNumber types are defined in 3.9.

## 3.1	V-Series Call Control Managed Object

### 3.1.1	Managed Object Template

vSeriesCallControl MANAGED OBJECT CLASS

-- Source ITU-T SG14 Q4

-- Status final draft 6 June 1994

DERIVED FROM "Recommendation M.3100: 1992":managedElement

CHARACTERIZED BY

callControlPkg	PACKAGE

ATTRIBUTES
audioMonitor	GET-REPLACE,
autoAnswerEnable	GET-REPLACE,
autoCallEnable	GET-REPLACE,
autoCallModeSelect	GET-REPLACE,
autoCallModesSupported	GET,
autoRestoralEnable	GET-REPLACE,
availableNumberCapacity	GET,
blackListingActive	GET,
callProgressState	GET,
dceModeActive	GET,
dceModeSelect	GET-REPLACE,
dceModesSupported	GET,
defaultCallAttemptsTimer	GET-REPLACE,
dialBackupEnable	GET-REPLACE,

directCallNumber	GET-REPLACE,
displayCallProgressMessages	GET-REPLACE,
pauseDuringDialTime	GET-REPLACE,
ringsBeforeAnswer	GET-REPLACE,
telephoneNumbers	GET-REPLACE,
v25bisMode	GET-REPLACE;
ACTIONS
dialConnect,	
dialDisconnect,	
NOTIFICATIONS
callProgressEvents,	
revertedToLeasedLine,

switchedToDialBackup;

REGISTERED AS

{ccitt(0), recommendation(0), v(22), v58(58), vSeriesCallControl(0)};

### 3.1.2	Attributes

audioMonitor XE "audioMonitor "§::= SEQUENCE {

monitorEnable

ENUMERATED {

off		(0),

alwaysOn		(1),

monitorDial		(2),

monitorCallSetup	(3)

},

monitorVolume	ENUMERATED {

low	(0),

normal	(1)

}

}

Description:	This attribute controls the operation of the audio monitor (typically a loudspeaker) which provides audible indication of modem dialling operations.

Operations:	GET-REPLACE

Behaviour:	Specification not required

Applications:	Configuration Management

Registered as:	{vSeriesCallControl audioMonitor (0)}

autoAnswerEnable XE "autoAnswerEnable "§::= BOOLEAN

Description:	This attribute is used to enable or disable auto answer or dial backup answer mode.

Operations:	GET-REPLACE

Behaviour:	Specification not required

Applications:	Configuration Management

Registered as:	{vSeriesCallControl autoAnswerEnable (1)}

autoCallEnable XE "autoCallEnable "§::= BOOLEAN

Description:	This attribute is used to enable or disable autocall or dial backup call mode.

Operations:	GET-REPLACE

Behaviour:	Specification not required

Applications:

Configuration Management

Registered as:	{vSeriesCallControl autoCallEnable (2)}

autoCallModeSelect XE "autoCallModeSelect "§::= ENUMERATED {

none	(0),
	v-25	(1),
	v-25bis	(2),
	v-at	(3),
	cct108-1	(4)
	}

Description:	Indicates which automatic call mode is selected in the DCE.

Operations:	GET-REPLACE

Behaviour:	Must be one of autoCallModesSupported

Applications:	Configuration Management

Registered as:	{vSeriesCallControl autoCallModeSelect (3)}

autoCallModesSupported XE "autoCallModesSupported "§::= BIT STRING {

none	(0),
	v-25	(1),
	v-25bis	(2),
	v-at	(3),
	cct108-1	(4)
	}

Description:	Indicates which automatic call modes are implemented in the DCE.

Operations:	GET

Behaviour:	Specification not required

Applications:	Configuration Management

Registered as:	{vSeriesCallControl autoCallModesSupported (4)}

autoRestoralEnable XE "autoRestoralEnable "§::= BOOLEAN

Description:	This attribute is used to enable or disable automatic restoral to leased line.

Operations:	GET-REPLACE

Behaviour:	See dceModeActive

Applications:	Fault Management

Registered as:	{vSeriesCallControl autoRestoralEnable (5)}

availableNumberCapacity XE "availableNumberCapacity "§::= INTEGER (0..255)

Description:	Number of storage locations for maximum length telephone numbers remaining.

Operations:	GET

Behaviour:	Limits range of directCallNumber

Applications:	Configuration Management

Registered as:	{vSeriesCallControl availableNumberCapacity (6)}

blackListingActive XE "blackListingActive "§::= BOOLEAN

Description:	Indicates whether or not blacklisting is active in the DCE.

Operations:	GET

Behaviour:	Specification not required

Applications:	Fault Management

Registered as:	{vSeriesCallControl blackListingActive (7)}

callProgressState XE "callProgressState "§::= CallProgress

Description:	Indicates the state in the calling process the DCE is in.

Operations:	GET

Behaviour:	Specification not required

Applications:	Configuration Management

Registered as:	{vSeriesCallControl callProgressState (8)}

dceModeActive XE "dceModeActive "§::= ENUMERATED {

leasedPointToPoint		(0),
	leasedMultipointControl		(1),
	leasedMultipointTributary	(2),
	dialAutocall			(3),
	dialAutoanswer		(4),
	dialBackupCall			(5),
	dialBackupAnsw		(6)
	}

Description:	If DCE is capable of operating in leased line or GSTN mode, this parameter indicates the current mode of operation.

Operations:	GET

Behaviour:	See dceModeSelect

Applications:	Configuration Management

Registered as:	{vSeriesCallControl dceModeActive (9)}

dceModeSelect XE "dceModeSelect "§::= BIT STRING  {

leasedPointToPoint	(0),
	leasedMultipointControl	(1),
	leasedMultipointTributary	(2),
	dialAutocall	(3),
	dialAutoanswer	(4),
	dialBackupCall	(5),
	dialBackupAnsw	(6)
	}

Description:	If DCE is capable of operating in leased line or GSTN mode, this parameter specifies the selected mode(s) of operation.

Operations:	GET-REPLACE

Behaviour:	More than one mode can be selected, if dialBackupEnable or autoRestoralEnable is TRUE.

Applications:	Configuration Management

Registered as:	{vSeriesCallControl dceModeSelect (10)}

dceModesSupported XE "dceModesSupported "§::= BIT STRING {

leasedPointToPoint	(0),
	leasedMultipointControl	(1),
	leasedMultipointTributary	(2),
	dialAutocall	(3),
	dialAutoanswer	(4),
	dialBackupCall	(5),
	dialBackupAnsw	(6)
	}

Description:	Indicates the modes the DCE can support.

Operations:	GET

Behaviour:	See dceModeSelect

Applications:	Configuration Management

Registered as:	{vSeriesCallControl dceModesSupported (11)}

defaultCallAttemptsTimer XE "defaultCallAttemptsTimer "§::= INTEGER (0..255)

Description:	This attribute defines the time in 10 second increments between call attempts.

Operations:	GET-REPLACE

Behaviour:	Specification not required

Applications:	Fault Management

Registered as:	{vSeriesCallControl defaultCallAttemptsTimer (12)}

dialBackupEnable XE "dialBackupEnable "§::= BOOLEAN

Description:	Enables the DCE to dial backup.

Operations:	GET-REPLACE

Behaviour:	See dceModeSelect

Applications:	Fault Management

Registered as:	{vSeriesCallControl dialBackupEnable (13)}

directCallNumber XE "directCallNumber "§::= SEQUENCE OF PhoneNumber

Description:	In GSTN mode this determines the default call number. In Leased Line mode this determines the dial backup number.

Operations:	GET-REPLACE

Behaviour:	This is a list of stored telephone numbers. In GSTN mode these will be tried in order. In Leased Line Mode when using double dial backup the numbers will be regarded as pairs, the first as the transmit direction, second as receive direction, etc. The maximum number of phone numbers remaining is given by availableNumberCapacity.

Applications:	Configuration Management

Registered as:	{vSeriesCallControl directCallNumber (14)}

displayCallProgressMessages XE "displayCallProgressMessages "§::= BOOLEAN

Description:	Determines whether or not call progress messages are presented to the DTE.

Operations:	GET-REPLACE

Behaviour:	e.g. CFI in V.25 bis or "ringing” in draft Recommendation V.at (V.25 ter)

Applications:	Configuration Management

Registered as:	{vSeriesCallControl displayCallProgressMessages (15)}

pauseDuringDialTime XE "pauseDuringDialTime "§::= INTEGER (1..8)

Description:	Sets the amount of time, in seconds the DCE pauses upon encountering the "pause during dialling” character (e.g. comma).

Operations:	GET-REPLACE

Behaviour:	Specification not required

Applications:	Configuration Management

Registered as:	{vSeriesCallControl pauseDuringDialTime (16)}

ringsBeforeAnswer XE "ringsBeforeAnswer "§::= INTEGER (0..15)

Description:	Determines the number of rings the DCE must wait before answering a call.

Operations:	GET-REPLACE

Behaviour:	Specification not required

Applications:	Configuration Management

Registered as:	{vSeriesCallControl ringsBeforeAnswer (17)}

telephoneNumbers XE "telephoneNumbers "§::= SEQUENCE OF

SEQUENCE {

numberRef	INTEGER (0..255),
	number	PhoneNumber,
	blacklisted	BOOLEAN
	}

Description:	Presents the list of phone numbers stored in the DCE.

Operations:	GET-REPLACE

Behaviour:	Replacing the blacklisted parameter may not be allowed

Applications:	Configuration Management

Registered as:	{vSeriesCallControl telephoneNumbers (18)}

v25bisMode XE "v25bisMode "§::= ENUMERATED {

startStop	(0),
	hdlc	(1),
	syncCharacter	(2)
	}

Description:	Indicates the V.25 bis mode used at the  DTE and DCE interface.

Operations:	GET-REPLACE

Behaviour:	Specification not required

Applications:	Configuration Management

Registered as:	{vSeriesCallControl v25bisMode (19)}

### 3.1.3	Actions

Name:	dialConnectXE "dialConnect"§

dialConnectReq ::= CHOICE {

useProgrammedNumber		[0]	BOOLEAN,
	useNumberRef 			[1]	INTEGER (0..255),
	number 			[2]	PhoneNumber,
	doubleDialNumbers 		[3]
		SEQUENCE {
		number1		PhoneNumber,
		number2		PhoneNumber
		}
	}

dialConnectConf ::= CallProgress

Description:	This action instructs the DCE to establish a dial up connection.

Operations:	Multiple Response Action

Behaviour:	If the DCE is in dial up mode this action instructs it to establish a connection, i.e. to autocall.

If the DCE is in leased line mode this action instructs it to establish a dial backup connection. If useProgrammedNumber is present, then the DCE shall dial the programmed number.

If the DCE is already connected this action instructs it to disconnect and redial.

Applications:	Configuration and Fault Management

Registered as:	{vSeriesCallControl dialConnect (20)}

Name:	dialDisconnectXE "dialDisconnect"§

dialDisconnectReq  ::= NULL

dialDisconnectConf ::= CallProgress

Description:	This action is used to disconnect the DCE when in dial up or dial backup mode.

Operations:	ACTION

Behaviour:	If the DCE is in dial up mode, this action disconnects any calls in progress and returns the DCE to the enabled unlocked state.

If the DCE is in dial backup, i.e. leased line mode, this action disconnects the DCE and attempts to re-establish communications over the leased line.

Applications:	Configuration and Fault Management

Registered as:	{vSeriesCallControl dialDisconnect (21)}

### 3.1.4	Notifications

callProgressEvents XE "callProgressEvents "§::= SEQUENCE {

priority EventPriority,
	callEvent	ENUMERATED {
			ringing	(0),
			busy	(1),
			numberUnavail	(2),
			noDialTone	(3),
			connected	(4)
			}
	}

Description:	Indicates state transitions during calling.

Operations:	NOTIFICATION

Behaviour:	Specification not required

Applications:	Configuration Management

Registered as:	{vSeriesCallControl callProgressEvents (22)}

revertedToLeasedLine XE "revertedToLeasedLine "§::= EventPriority

Description:	Indicates that the DCE has reverted to leased line from dial backup.

Operations:	NOTIFICATION

Behaviour:	Specification not required

Applications:	Fault Management

Registered as:	{vSeriesCallControl revertedToLeasedLine (23)}

switchedToDialBackup XE "switchedToDialBackup "§::= EventPriority

Description:	Indicates that the DCE has switched to dial back up from leased line.

Operations:	NOTIFICATION

Behaviour:	Specification not required

Applications:	Fault Management

Registered as:	{vSeriesCallControl switchedToDialBackup (24)}

## 3.2	V-Series Data Compression Managed Object

### 3.2.1 Managed Object Template

vSeriesDataCompression MANAGED OBJECT CLASS

-- Source ITU T SG14 Q4

-- Status final draft 6 June 1994

DERIVED FROM "Recommendation M.3100: 1992":managedElement

CHARACTERIZED BY

dataCompressionPkg	PACKAGE
	ATTRIBUTES
	compressionActive		GET,
	compressionEfficiency		GET,
	compressionSelect		GET-REPLACE,
	compressionSupported		GET;

CONDITIONAL PACKAGES

v42bisPkg	PACKAGE
	ATTRIBUTES
	v42bisCompressionActive		GET,
	v42bisCompressionSelect		GET-REPLACE,
	v42bisDictionarySizeActive	GET,
	v42bisDictionarySizeSelect	GET-REPLACE,
	v42bisDictionarySizeSupported	GET,
	v42bisMaximumStringLengthActive	GET,
	v42bisMaximumStringLengthSelect	GET-REPLACE,
	v42bisMaximumStringLengthSupported	GET;
	NOTIFICATIONS
	lossOfSynchronization;
	PRESENT IF "compressionEnable is v42bisonly";

REGISTERED AS

{ccitt(0), recommendation(0), v(22), v58(58),  vSeriesDataCompression(1)};

### 3.2.2	Attributes

compressionActive XE "compressionActive "§::= ENUMERATED {

none	(0),
	v42bisActive	(1),
	reserved	(2),
	vendorSpecificActive	(3)
	}

Description:	Provides the current state of the data compression function.

Operations:	GET

Behaviour:	Specification not required

Applications:	Configuration Management

Registered as:	{vSeriesDataCompression compressionActive (0)}

compressionEfficiency XE "compressionEfficiency "§::= INTEGER (0..65535)

Description:	Number of bytes transferred into the encoder divided by the number of bytes transferred out of the encoder for either the current or last call expressed as a percentage.

Operations:	GET

Behaviour:	Value is reset at call establishment.

Applications:	Performance Management

Registered as:	{vSeriesDataCompression compressionEfficiency (1)}

compressionSelect XE "compressionSelect "§::= ENUMERATED {

disabled	(0),
	v42bisonly	(1),
	reserved	(2),
	vendorSpecific	(3)
	}

Description:	Enables or disables the data compression function.

Operations:	GET-REPLACE

Behaviour:	Specification not required

Applications:	Configuration Management

Registered as:	{vSeriesDataCompression compressionSelect (2)}

compressionSupported XE "compressionSupported "§::= BIT STRING {

none	(0),
	v42bis	(1),
	reserved	(2),
	vendorSpecific 	(3)
	}

Description:	Indicates the data compression alternatives that are supported by the DCE.

Operations:	GET

Behaviour:	Specification not required

Applications:	Configuration Management

Registered as:	{vSeriesDataCompression compressionSupported (3)}

v42bisCompressionActive XE "v42bisCompressionActive "§::= ENUMERATED {

none	(0),
	txOnly	(1),
	rxOnly	(2),
	both	(3)
	}

Description:	Contains the value of the V.42 bis data compression request parameter (P0) established for the current or last call.

Operations:	GET

Behaviour:	If Recommendation V.42 bis is not enabled for the current or past call, then the value will be set to None.

Applications:	Configuration Management

Registered as:	{vSeriesDataCompression v42bisCompressionActive (4)}

v42bisCompressionSelect XE "v42bisCompressionSelect "§::= ENUMERATED {

none	(0),
	txOnly	(1),
	rxOnly	(2),
	both	(3)
	}

Description:	V.42 bis compression mode to be established at the next call.

Operations:	GET-REPLACE

Behaviour:	Sets the value of the V.42 bis data compression request parameter (P0) to be used for subsequent calls.

Applications:	Configuration Management

Registered as:	{vSeriesDataCompression v42bisCompressionSelect (5)}

v42bisDictionarySizeActive XE "v42bisDictionarySizeActive "§::= INTEGER (512..65535)

Description:	Number of dictionary code words established for the current or previous call.

Operations:	GET

Behaviour:	Value which has been set by V.42 bis parameter (P1).

Applications:	Configuration Management

Registered as:	{vSeriesDataCompression v42bisDictionarySizeActive (6)}

v42bisDictionarySizeSelect XE "v42bisDictionarySizeSelect "§::= INTEGER (512..65535)

Description:	Number of Dictionary code words.

Operations:	GET-REPLACE

Behaviour:	Sets the value of the V.42 bis number of dictionary code words parameter (P1) to be used for subsequent calls.

Applications:	Configuration Management

Registered as:	{vSeriesDataCompression v42bisDictionarySizeSelect (7)}

v42bisDictionarySizeSupported XE "v42bisDictionarySizeSupported "§::= INTEGER (512..65535)

Description:	Number of Dictionary code words.

Operations:	GET

Behaviour:	Indicates the value of the V.42 bis number of dictionary code words parameter (P1) supported by the DCE.

Applications:	Configuration Management

Registered as:	{vSeriesDataCompression v42bisDictionarySizeSupported (8)}

v42bisMaximumStringLengthActive XE "v42bisMaximumStringLengthActive "§::= INTEGER (6..250)

Description:	String length in characters established for the current or previous call.

Operations:	GET

Behaviour:	Value which has been set by the V.42 bis parameter (P2).

Applications:	Configuration Management

Registered as:	{vSeriesDataCompression v42bisMaximumStringLengthActive (9)}

v42bisMaximumStringLengthSelect XE "v42bisMaximumStringLengthSelect "§::= INTEGER (6..250)

Description:	Preferred maximum string length in characters.

Operations:	GET-REPLACE

Behaviour:	Sets the value of the V.42 bis maximum string length parameter (P2) to be used for subsequent calls.

Applications:	Configuration Management

Registered as:	{vSeriesDataCompression v42bisMaximumStringLengthSelect (10)}

v42bisMaximumStringLengthSupported XE "v42bisMaximumStringLengthSupported "§::= INTEGER (6..250)

Description:	Maximum string length in characters supported by the DCE.

Operations:	GET-REPLACE

Behaviour:	See v42bisMaximumStringLengthSelect.

Applications:	Configuration Management

Registered as:	{vSeriesDataCompression v42bisMaximumStringLengthSupported (11)}

### 3.2.3	Actions

None.

### 3.2.4	Notifications

lossOfSynchronization XE "lossOfSynchronization "§::= EventPriority

Description:	This notification indicates that an error (for example a procedural error) or a loss of synchronization has been detected by the data compression function.

Operations:	NOTIFICATION

Behaviour:	In most cases the call will be disconnected.

Applications:	Fault Management

Registered as:	{vSeriesDataCompression lossOfSynchronization (12)}

## 3.3	V-Series DCE Managed Object

### 3.3.1	Managed Object Template

vSeriesDce MANAGED OBJECT CLASS

-- Source ITU T SG14 Q4

-- Status final draft 6 June 1994

DERIVED FROM "Recommendation M.3100: 1992":equipment;

CHARACTERIZED BY

vSeriesDcePkg	PACKAGE
	ATTRIBUTES
	equipmentType		GET,
	eventThreshold		GET-REPLACE,
	manufacturerID		GET;
	ACTIONS
	selfTest;
	NOTIFICATIONS
	equipmentFailure,
	powerOn,
	powerOnFailure,
	resetNotification;

CONDITIONAL PACKAGES

internationalPkg	PACKAGE
	ATTRIBUTES
	countryOfInstallationSelect		GET-REPLACE,
	countryOfInstallationSupported		GET;
	PRESENT IF "multi-country product"

downloadableConfigPkg	PACKAGE
	ATTRIBUTES
	disconnectConfiguration		GET-REPLACE,
	powerOnConfiguration		GET-REPLACE,
	presetConfigurationRange		GET,
	userConfigurationRange		GET;
	ACTIONS
	invokeConfiguration,
	loadConfiguration,
	storeConfiguration,
	viewConfiguration;
	PRESENT IF "DCE supports downloadable configurations"

backupDcePkg		PACKAGE
	ATTRIBUTES
	backedUpStatus		GET,
	backUpObjectInstance		GET-REPLACE,
	PRESENT IF "the DCE has a backup relationship with another DCE",

frontPanelLockoutPkg	PACKAGE
	ATTRIBUTES
	frontPanelAccessEnable		GET-REPLACE;
	PRESENT IF "the DCE has a front panel which can be disabled";

REGISTERED AS

{ccitt(0), recommendation(0), v(22), v58(58), vSeriesDce(2)};

### 3.3.2	Attributes

backedUpStatus XE "backedUpStatus "§::= BOOLEAN

Description:	This attribute indicates if the V-Series DCE has been backed up after failure.

Operations:	GET

Behaviour:	Set to TRUE if DCE has failed and been backed up by a standby unit. Set to FALSE in all other cases.

Applications:	Fault Management

Registered as:	{vSeriesDce backedUpStatus (0)}

backUpObjectInstance XE "backUpObjectInstance "§::= OBJECT IDENTIFIER

Description:	This attribute identifies a managed object which has a backup relationship with the V-Series DCE, for example a standby unit.

Operations:	GET-REPLACE

Behaviour:	To be defined

Applications:	Configuration and Fault Management

Registered as:	{vSeriesDce backUpObjectInstance (1)}

countryOfInstallationSelect XE "countryOfInstallationSelect "§::= IA5 String

Description:	Specifies the country in which the DCE is installed to control behaviours within national limits.

Operations:	GET-REPLACE

Behaviour:	Specifies the country of installation as an IA5 value (see countryOfInstallationSupported for format), according to the values specified in ITU-T Recommendation T.35. If set to null, indicates that the country of installation is unknown or irrelevant.

Applications:	Configuration Management

Registered as:	{vSeriesDce countryOfInstallationSelect (2)}

countryOfInstallationSupported XE "countryOfInstallationSupported "§::= IA5 String

Description:	Allows the Network Management System to determine the country codes which may be written to the countryOfInstallationSelect attribute.

Operations:	GET

Behaviour:	Lists the T.35 country codes that may be selected as values of the countryOfInstallationSelect for the DCE. Each value is represented in the form of an IA5 representation of the T.35 hexadecimal codes followed by a comma (for example “3A, 49, D2,”).

Applications:	Configuration Management

Registered as:	{vSeriesDce countryOfInstallationSupported (3)}

disconnectConfiguration XE "disconnectConfiguration "§::= SEQUENCE {

configurationType	ENUMERATED {
			none	(0),
			user	(1),
			preset	(2)
			},
	configurationRef	INTEGER (1..255)
	}

Description:	Specifies which of the stored configurations, if any, will be copied into the active configuration when a call is disconnected.

Operations:	GET-REPLACE

Behaviour:	The indicated reference must be within the relevant range supported or an error results. configurationRef should always be set to 1 if the “none” is specified (which means that the active configuration is unchanged upon disconnection).

Applications:	Configuration Management

Registered as:	{vSeriesDce disconnectConfiguration (4)}

equipmentType XE "equipmentType "§::= SEQUENCE {

vSeriesDceTypes	BIT STRING {
		dialModem		(0),
		leasedLineModem	(1),
		},
	lineInterface	ENUMERATED {
		analogue		(0),
		digital				(1)
		},
	dceImplementation	ENUMERATED {
		rackMount		(0),
		standAlone		(1),
		internal			(2),
		builtIn				(3)
		},
	identification 	OBJECT IDENTIFIER
		}

Description:	Specific type of V-Series DCE, for example “multiport modem”.

Operations:	GET

Behaviour:	Specification not required

Applications:	Configuration and Fault Management

Registered as:	{vSeriesDce equipmentType (5)}

eventThreshold XE "eventThreshold "§::=  EventPriority

Description:	This attribute sets the priority below which events will not be reported.

Operations:	GET-REPLACE

Behaviour:	Controls which events are reported from this and all subordinate managed objects.

Applications:	Fault and Performance Management

Registered as:	{vSeriesDce eventThreshold (6)}

frontPanelAccessEnable XE "frontPanelAccessEnable "§::= BOOLEAN

Description:	This attribute controls access to the DCE via front panel controls.

Operations:	GET-REPLACE

Behaviour:	To be defined

Applications:	Configuration and Security Management

Registered as:	{vSeriesDce frontPanelAccessEnable (7)}

manufacturerID XE "manufacturerID "§::= SEQUENCE {

manufacturerOI	OBJECT IDENTIFIER,
	productDetails	IA5String
	}

Description:	The Manufacturer ID provides a unique reference to the manufacturer of the DCE. The productDetails component may typically be structured into fields for Manufacturer name, product name or ID, software/hardware issue number, product serial number.

Operations:	GET

Behaviour:	Specification not required

Applications:	Manufacturer specific option support

Registered as:	{vSeriesDce manufacturerID (8)}

powerOnConfiguration XE "powerOnConfiguration "§::= SEQUENCE {

configurationType	ENUMERATED {
			user	(0),
			preset	(1)
			},
	configurationRef	INTEGER (1..255)
	}

Description:	Specifies which of the stored configurations will be copied into the active configuration when the DCE is powered on or reset.

Operations:	GET-REPLACE

Behaviour:	The indicated reference must be within the relevant range supported (userConfigurationRange or presetConfigurationRange).

Applications:	Configuration Management

Registered as:	{vSeriesDce powerOnConfiguration (9)}

presetConfigurationRange XE "presetConfigurationRange "§::= INTEGER (0..255)

Description:	Allows the Network Management System to determine the number of factory preset configuration profiles supported by the DCE.

Operations:	GET

Behaviour:	Specification not required

Applications:	Configuration Management

Registered as:	{vSeriesDce presetConfigurationRange (10)}

userConfigurationRange XE "userConfigurationRange "§::= INTEGER (0..255)

Description:	This attribute is used to indicate the number of stored configurations within the DCE. Typically these configurations would affect a number of attributes, including choice of modulation, DTE transfer speed, character format, use of error correction, etc.

Operations:	GET

Behaviour:	Limits range of invoke, load, store and view Configuration actions.

Applications:	Configuration Management

Registered as:	{vSeriesDce userConfigurationRange (11)}

### 3.3.3	Actions

Name:	invokeConfigurationXE "invokeConfiguration"§

invokeConfigurationReq ::= SEQUENCE {

configurationType	ENUMERATED {
			user	(0),
			preset	(1)
			},

configurationRef	INTEGER (1..255)
	}

invokeConfigurationConf  ::= NULL

Description:	This action is used to invoke one of a number of configurations stored within the DCE.

Operations:	ACTION

Behaviour:	This action can result in the modification of one or more attributes within the DCE, both in this managed object and its subordinates. Default values of attributes are invoked as configuration 0.

Applications:	Configuration Management

Registered as:	{vSeriesDce invokeConfiguration (12)}

Name:	loadConfigurationXE "loadConfiguration"§

loadConfigurationReq ::= SEQUENCE {

configurationType	ENUMERATED {
				user	(0),
				active	(1)
				},
	configurationRef	INTEGER (1..255),
	attributes	SEQUENCE OF
			SEQUENCE {
		attributeName 	OBJECT IDENTIFIER,
		attributeValue 	ANY
			}
	}

loadConfigurationConf ::= CHOICE {

noError	[0]	NULL,
	firstError	[1]	SEQUENCE {
		attributeName 	OBJECT IDENTIFIER,
		attributeValue 	ANY
	 		}
	}

Description:	Causes the DCE to update the specified profile to include the values specified for all of the attributes included in the request argument.

Operations:	ACTION

Behaviour:	1 should be used for the Active configuration. All of the attributes are validated before being stored, so that if any are in error the profile remains unchanged. Any attributes that exist in the profile that are not included in the request retain their previous values.

If unsuccessful, the selected profile remains unchanged. The negative confirmation will contain the attribute and value of the first invalid entry.

Applications:	Configuration Management

Registered as:	{vSeriesDce loadConfiguration (13)}

Name:	selfTestXE "selfTest"§

selfTestReq ::= ENUMERATED {

intrusiveFullTest	(0),
	safePartialTest	(1)
	}

selfTestConf ::= SEQUENCE {

testImplemented	BOOLEAN,
	pass	BOOLEAN,
	resultCode	INTEGER
	}

Description:	This Action invokes a self test of the DCE. The nature of this test is not specified, however shall include checks on the operation of hardware components and memory. It is assumed that the test duration is short (typically no longer than 5 seconds) and that a single response on completion of the test is adequate. More specific tests are defined in the Test Function managed object. A full test of DCE functions is assumed to be intrusive, i.e. would interfere with normal operation; a partial test is assumed to be non-intrusive but only able to provide a “health” check.

Operations:	ACTION

Behaviour:	The Intrusive Full Test terminates calls in progress and disables the DCE.

The Safe Partial Test provides a limited test of the DCE and does not terminate calls in progress.

Applications:	Fault Management

Registered as:	{vSeriesDce selfTest (14)}

Name:	storeConfigurationXE "storeConfiguration"§

storeConfigurationReq ::= INTEGER (0..255)

storeConfigurationConf ::= NULL

Description:	This action is used to store the current user configuration within the DCE.

Operations:	ACTION

Behaviour:	This action can result in the modification of one or more attributes within the DCE, both in this managed object and its subordinates. Default values of attributes are invoked as configuration 0. A request argument value outside UserConfigurationRange shall be regarded as invalid.

Applications:	Configuration Management

Registered as:	{vSeriesDce storeConfiguration (15)}

Name:	viewConfigurationXE "viewConfiguration"§

viewConfigurationReq ::= SEQUENCE {

configurationType	ENUMERATED {
			user	(0),
			preset	(1),
			active	(2)
			},
	configurationRef	INTEGER (1..255)
	}

viewConfigurationConf ::= SEQUENCE OF

SEQUENCE {
			attributeName	OBJECT IDENTIFIER,
			attributeValue 	ANY
			}

Description:	Causes the DCE to send the current values of all of the attributes in the specified configuration profile.

Operations:	ACTION

Behaviour:	configurationRef = 1 should be used for the active configuration.

Applications:	Configuration Management

Registered as:	{vSeriesDce viewConfiguration (16)}

### 3.3.4	Notifications

equipmentFailure XE "equipmentFailure "§::= SEQUENCE {

priority EventPriority,
	faultType	ENUMERATED {
			memoryFault	(0),
			deviceFault	(1)
			}
	}

Description:	This Event is used to indicate to the management entity that the DCE has detected an equipment fault condition.

Operations:	NOTIFICATION

Behaviour:	Behaviour of the DCE after this notification is not defined.

Applications:	Fault Management

Registered as:	{vSeriesDce equipmentFailure (17)}

powerOn XE "powerOn "§::= EventPriority

Description:	This Event is used to indicate to the management entity that the DCE is entering service.

Operations:	NOTIFICATION

Behaviour:	Requires that DCE has powered on and passed internal self test.

Applications:	Configuration Management

Registered as:	{vSeriesDce powerOn (18)}

powerOnFailure XE "powerOnFailure "§::= SEQUENCE {

priority EventPriority,
	problemType	IA5String
	}

Description:	This Event is used to indicate to the management entity that the DCE has powered on but is unable to enter service.

Operations:	NOTIFICATION

Behaviour:	Behaviour of the DCE after this notification is not defined.

Applications:	Fault Management

Registered as:	{vSeriesDce powerOnFailure (19)}

resetNotification XE "resetNotification "§::= SEQUENCE {

priority Eventpriority,
	resetType	ENUMERATED {
			userInitiated	(0),
			autonomous	(1)
			}
	}

Description:	This Event is used to indicate to the management entity that the DCE has performed a reset.

Operations:	NOTIFICATION

Behaviour:	Specification not required

Applications:	Fault Management

Registered as:	{vSeriesDce resetNotification (20)}

## 3.4	V-Series DTE Interface Managed Object

### 3.4.1	Managed Object Template

vSeriesDteInterface MANAGED OBJECT CLASS

-- Source ITU T SG14 Q4

-- Status final draft 6 June 1994

DERIVED FROM "Recommendation M.3100: 1992":managedElement

CHARACTERIZED BY

dteInterfacePkg	PACKAGE
	ATTRIBUTES
	antiStreamingTimer		GET-REPLACE,
	bufferedDataDeliveryTimeoutSelect	GET-REPLACE,
	bufferedDataDeliveryTimeoutSupported	GET,
	cct105Mode		GET-REPLACE,
	cct105to106Delay		GET-REPLACE,
	cct106Mode		GET-REPLACE,
	cct107Mode		GET-REPLACE,
	cct108Mode		GET-REPLACE,
	cct109Mode		GET-REPLACE,
	cct109TurnOffDelay		GET-REPLACE,
	cct109TurnOnDelay		GET-REPLACE,
	cct116Mode		GET-REPLACE,
	dteAttached		GET,
	dteInterfaceStatus,		GET,
	dteModeActive		GET-REPLACE,
	dteModesSupported		GET,
	inactivityTimerSelect		GET-REPLACE
	inactivityTimerSupported		GET,
	v13ModeSelect		GET-REPLACE,
	NOTIFICATIONS
	streamingDetected;

CONDITIONAL PACKAGES

startStopAttributes	PACKAGE
	ATTRIBUTES
	autoDetectCharacterFormat		GET	
	cct133ToXonXoffTranslation		GET-REPLACE,
	characterFormatSelect		GET-REPLACE,
	characterFormatSupported		GET,
	echoControlMessages		GET-REPLACE,
	echoUserData		GET-REPLACE,
	flowControlSelect		GET-REPLACE,
	flowControlSupported		GET,
	longSpaceDisconnectSelect		GET-REPLACE,
	longSpaceDisconnectSupported		GET,
	parityBits		GET-REPLACE,
	responseModeSelect		GET-REPLACE,
	responseModeSupported		GET,
	startStopDteInterfaceSpeed		GET-REPLACE,
	startStopDteInterfaceSpeedAdaptation	GET-REPLACE,
	v14SignallingRate		GET-REPLACE;
	PRESENT IF "dteModesSupported includes startStopV14 or startStopErrorControl or startStopBuffered or 	v25bisMode is startStop";

REGISTERED AS

{ccitt(0), recommendation(0), v(22), v58(58), vSeriesDteInterface(3)};

### 3.4.2	Attributes

antiStreamingTimer XE "antiStreamingTimer "§::= INTEGER  (0..255)

Description:	Maximum continuous duration in seconds that a multipoint DCE is allowed to transmit.

Operations:	GET-REPLACE

Behaviour:	The timer is initiated at the OFF/ON transition of circuit 105. If circuit 105 remains on for more than the duration of the timer, then the DCE’s line signal will be turned off. When circuit 105 is turned off normal operation resumes – i.e. line signal follows an OFF/ON transition of circuit 105. A value of 0 indicates that the timer is inhibited. When the timer expires, a streamingDetected notification is transmitted.

Applications:	Fault Management

Registered as:	{vSeriesDteInterface antiStreamingTimer (0)}

autoDetectCharacterFormat XE "autoDetectCharacterFormat "§::= SEQUENCE {

selectFormat 	ENUMERATED {
					adf8Data2Stop		(1),
					adf8Data1Parity1Stop	(2),
					adf8Data1Stop		(3),
					adf7Data2Stop		(4),
					adf7Data1Parity1Stop	(5),
					adf7Data1Stop		(6),
					adf6Data2Stop		(7),
					adf6Data1Parity1Stop	(8),
					adf6Data1Stop		(9),
					adf5Data2Stop		(10),
					adf5Data1Parity1Stop	(11)
					},
		parityBitValue 	ENUMERATED {
					odd	(0),
					even	(1),
					mark	(2),
					space	(3)
			}

Description:	Active start stop character format.

Operations:	GET

Behaviour:	Only valid if selectAutoDetect is TRUE.

Applications:	Configuration Management

Registered as:	{vSeriesDteInterface autoDetectCharacterFormat (1)}

bufferedDataDeliveryTimeoutSelect XE "bufferedDataDeliveryTimeoutSelect "§::= INTEGER (0..255)

Description:	Specifies the time-out in seconds for delivery of buffered data.

Operations:	GET-REPLACE

Behaviour:	Applies to buffered start/stop and error control operation. Specifies the maximum amount of time, in seconds, the DCE will wait to deliver buffered received data to the DTE when carrier is lost or a link disconnect is received; circuit 109 is held high until all data is delivered. Also controls the amount of time the DCE will continue sending (and receiving acknowledgement in error control mode) data after a DTE-initiated request for disconnection [e.g. on-to-off transition of circuit 108 or ATH command in draft Recommendation V.at (V.25 ter)]. When the timer expires (or if the timer is set to 0), all buffered data is discarded. If the value is set to 255, the DCE will wait indefinitely and not disconnect until all data is delivered.

Applications:	Configuration Management

Registered as:	{vSeriesDteInterface bufferedDataDeliveryTimeoutSelect (2)}

bufferedDataDeliveryTimeoutSupported XE "bufferedDataDeliveryTimeoutSupported "§::= INTEGER (0..255)

Description:	Allows Network Management System to determine to supported range of values for time-out in seconds of buffered data delivery.

Operations:	GET

Behaviour:	Specifies the maximum value which can be specified for bufferedDataDeliveryTimeoutSelect.

Applications:	Configuration Management

Registered as:	{vSeriesDteInterface bufferedDataDeliveryTimeoutSupported (3)}

cct105Mode XE "cct105Mode "§::= ENUMERATED {

normal		(0),
		on		(1),
		off		(2)
		}

Description:	Sets the operating mode for circuit 105.

Operations:	GET-REPLACE

Behaviour:	“on” and “off” indicate that the DCE will act as if circuit 105 is permanently ON or OFF, respectively.

Applications:	Configuration Management

Registered as:	{vSeriesDteInterface cct105Mode (4)}

cct105to106Delay XE "cct105to106Delay "§::= INTEGER (0..1023)

Description:	Additional circuit 105 to circuit 106 delay in milliseconds for switched carrier operation, and total circuit 105 to circuit 106 delay for continuous carrier operation.

Operations:	GET-REPLACE

Behaviour:	If circuit 105 mode is switched carrier, then this delay is in addition to the normal training delay. If circuit 105 mode is continuous carrier, then this is the only circuit 105 to circuit 106 delay. This attribute is only valid if circuit106Mode is normal.

Applications:	Configuration Management

Registered as:	{vSeriesDteInterface cct105to106Delay (5)}

cct106Mode XE "cct106Mode "§::= ENUMERATED {

normal		(0),
		on		(1),
		off		(2)
		}

Description	Provides the capability to explicitly set the state of circuit 106.

Operations:	GET-REPLACE

Behaviour:	In normal mode circuit 106 is not explicitly forced ON or OFF. In the case of a V.34 secondary channel that shares the same physical interface as the primary channel, this attribute may also be used to control the operation of circuit 121.

Applications:	Configuration Management

Registered as:	{vSeriesDteInterface cct106Mode (6)}

cct107Mode XE "cct107Mode "§::= ENUMERATED {

normal		(0),
		followsCct108		(1),
		on		(2),
		off		(3)
		}

Description:	Provides the capability to modify circuit 107 operation.

Operations:	GET-REPLACE

Behaviour:	Normal operation is per V.24 and all other relevant V-Series Recommendations.

Applications:	Configuration Management

Registered as:	{vSeriesDteInterface cct107Mode (7)}

cct108Mode XE "cct108Mode"§::= ENUMERATED {

cct108-1			(0),
		cct108-2-GotoCommandState	(1),
		cct108-2-Hangup			(2),
		cct108-2-Reset			(3),
		on			(4),
		off			(5)
		}

Description:	Provides the capability to modify circuit 108 operation.

Operations:	GET-REPLACE

Behaviour:	“on” and “off” indicate that the DCE will act as if circuit 108 is permanently ON or OFF, respectively.

Applications:	Configuration Management

Registered as:	{vSeriesDteInterface cct108Mode (8)}

cct109Mode XE "cct109Mode "§::= ENUMERATED {

normal		(0),
		on		(1),
		off		(2)
		}

Description:	Modifies circuit 109 operation.

Operations:	GET-REPLACE

Behaviour:	In normal mode circuit 109 is not explicitly forced ON or OFF. In the case of a V.34 secondary channel that shares the same physical interface as the primary channel, this attribute may also be used to control the operation of circuit 122.

Applications:	Configuration Management

Registered as:	{vSeriesDteInterface cct109Mode (9)}

cct109TurnOffDelay XE "cct109TurnOffDelay "§::= INTEGER  (0..255)

Description:	Provides the capability to modify the circuit 109 turn off delay in increments of 10 milliseconds.

Operations:	GET-REPLACE

Behaviour:	Delays the turn off of circuit 109 with regard to the loss of line signal.  In the case of a V.34 secondary channel that shares the same physical interface as the primary channel, this attribute may also be used to control the turn off delay of circuit 122.

Applications:	Configuration Management

Registered as:	{vSeriesDteInterface cct109TurnOffDelay (10)}

cct109TurnOnDelay XE "cct109TurnOnDelay "§::= INTEGER  (0..255)

Description:	Provides the capability to modify the circuit 109 turn on delay in increments of 10 milliseconds.

Operations:	GET-REPLACE

Behaviour:	Delays the turn on of circuit 109 with regard to the detection of line signal. In the case of a V.34 secondary channel that shares the same physical interface as the primary channel, this attribute may also be used to control the turn on delay of circuit 122.

Applications:	Configuration Management

Registered as:	{vSeriesDteInterface cct109TurnOnDelay (11)}

cct116Mode XE "cct116Mode "§::= ENUMERATED {

off		(0),
		cct116-1Operation	(1),
		cct116-2Operation	(2)
		}

Description:	Modifies circuit 116 operation.

Operations:	GET-REPLACE

Behaviour:	“Off” indicates that the DCE will act as if circuit 116 is permanently OFF.

Applications:	Configuration Management

Registered as:	{vSeriesDteInterface cct116Mode (12)}

cct133ToXonXoffTranslation XE "cct133ToXonXoffTranslation "§::= BOOLEAN

Description:	Enables the translation of circuit 133 transitions to Xon/Xoff characters.

Operations:	GET-REPLACE

Behaviour:	This attribute is only applicable when circuit 133 is being used as the flow control mechanism. Upon an on to off transition of circuit 133, the DCE transmits an Xoff character to the remote DCE. Upon an off to on transition of circuit 133, the DCE transmits an Xon character to the remote DCE.

Applications:	Configuration Management

Registered as:	{vSeriesDteInterface cct133ToXonXoffTranslation (13)}

characterFormatSelect XE "characterFormatSelect "§::= SEQUENCE {

selectAutoDetect			BOOLEAN,
		selectFormat	ENUMERATED {
					none			(0),
					cf8Data2Stop		(1),
					cf8Data1Parity1Stop	(2),
					cf8Data1Stop		(3),
					cf7Data2Stop		(4),
					cf7Data1Parity1Stop	(5),
					cf7Data1Stop		(6),
					cf6Data2Stop		(7),
					cf6Data1Parity1Stop	(8),
					cf6Data1Stop		(9),
					cf5Data2Stop		(10),
					cf5Data1Parity1Stop	(11)
				}
		}

Description:	Selects start/stop character format.

Operations:	GET-REPLACE

Behaviour:	selectAutoDetect shall only be set to TRUE if characterFormatSupported Includes “automatic”. selectFormat shall be set to “none” if selectAutodetect is TRUE.

Applications:	Configuration Management

Registered as:	{vSeriesDteInterface characterFormatSelect (14)}

characterFormatSupported XE "characterFormatSupported "§::= BIT STRING {

automatic	(0),
		cf8Data2Stop	(1),
		cf8Data1Parity1Stop	(2),
		cf8Data1Stop	(3),
		cf7Data2Stop	(4),
		cf7Data1Parity1Stop	(5),
		cf7Data1Stop	(6),
		cf6Data2Stop	(7),
		cf6Data1Parity1Stop	(8),
		cf6Data1Stop	(9),
		cf5Data2Stop	(10),
		cf5Data1Parity1Stop	(11)
		}

Description:	DCE Implemented start/stop character formats.

Operations:	GET

Behaviour:	Supported range may depend on the value of dteModeActive.

Applications:	Configuration Management

Registered as:	{vSeriesDteInterface characterFormatSupported (15)}

dteAttached XE "dteAttached "§::= ENUMERATED {

dteOff	(0),
		dteOn	(1),
		unknown	(2)
		}

Description:	The DTE is attached and powered on.

Operations:	GET

Behaviour:	The method that the DCE utilizes to make this determination is not part of this Recommendation.

Applications:	Configuration Management

Registered as:	{vSeriesDteInterface dteAttached (16)}

dteInterfaceStatus XE "dteInterfaceStatus "§::= SEQUENCE {

cctTransition	BIT STRING {
				cct103	(0),
				cct104	(1),
				cct111	(2),
				cct113	(3),
				cct114	(4),
				cct115	(5),
				cct125	(6),
				cct142	(7)
			},
		cctState	BIT STRING {
				cct105-133	(0),
				cct106	(1),
				cct107	(2),
				cct108	(3),
				cct109	(4),
				cct116	(5),
				cct140	(6),
				cct141	(7)
				}
		}

Description:	Provides the status of the DTE/DCE interface circuits.

Operations:	GET

Behaviour:	The circuit transition bitstring entry will be a binary one if a transition has occurred on the corresponding circuit since the last GET.

Applications:	Configuration Management

Registered as:	{vSeriesDteInterface dteInterfaceStatus (17)}

dteModeActive XE "dteModeActive "§::= ENUMERATED {

startStopV14	(0),
		startStopErrorControl	(1),
		startStopBuffered	(2),
		synchronous	(3),
		syncAfterDial	(4)
		}

Description:	Type of DTE (sync or start stop async) mode for which the DCE has been configured. For start stop operation, there are three possible operating modes.

Operations:	GET-REPLACE

Behaviour:	Applies only during data transfer.

Applications:	Configuration Management

Registered as:	{vSeriesDteInterface dteModeActive (18)}

dteModesSupported XE "dteModesSupported "§::= BIT STRING {

startStopV14	(0),
		startStopErrorControl	(1),
		startStopBuffered	(2),
		synchronous	(3),
		syncAfterDial	(4)
		}

Description:	DTE/DCE transmission modes (synchronous or asynchronous) which the DCE is capable of supporting. There are three possible start stop asynchronous operating modes.

Operations:	GET

Behaviour:	Specification not required

Applications:	Configuration Management

Registered as:	{vSeriesDteInterface dteModesSupported (19)}

echoControlMessages XE "echoControlMessages "§::= BOOLEAN

Description:	Enables the function in which V.25 bis and draft Recommendation V.at (V.25 ter) control messages appearing on circuit 103 are echoed by the DCE on circuit 104.

Operations:	GET-REPLACE

Behaviour:	Specification not required

Applications:	Configuration Management

Registered as:	{vSeriesDteInterface echoControlMessages (20)}

echoUserData XE "echoUserData "§::= BOOLEAN

Description:	Enables the function in which user data appearing on circuit 103 is echoed by the DCE on circuit 104.

Operations:	GET-REPLACE

Behaviour:	Specification not required

Applications:	Configuration Management

Registered as:	{vSeriesDteInterface echoUserData (21)}

flowControlSelect XE "flowControlSelect "§::= SEQUENCE {

dceByDte	ENUMERATED {
				none		(0),
				xonXoff		(1),
				xonXoffWithPassThrough	(2),
				cct133		(3)
				},
		dteByDce	ENUMERATED {
				none	(0),
				xonXoff	(1),
				cct106	(2)
				}
		}

Description:	Selects the flow control methodology which will be utilized by the DCE.

Operations:	GET-REPLACE

Behaviour:	Some DCEs support different options for DTE by DCE and DCE by DTE flow control. If asymmetric flow control is not available, as indicated by the flowControlSupported attribute, only symmetric values of the dteByDce and dceByDte attributes will be considered valid.

Only dteByDce flow control is supported for a V.34 secondary channel. In the case of a V.34 secondary channel that shares the same physical interface as the primary channel, this attribute may also be used to control the operation of circuit 121.

Applications:	Configuration Management

Registered as:	{vSeriesDteInterface flowControlSelect (22)}

flowControlSupported XE "flowControlSupported "§::= SEQUENCE {

dceByDte	BIT STRING {
				none		(0),
				xonXoff		(1),
				xonXoffWithPassThrough	(2),
				cct133		(3)
				},
		dteByDce	BIT STRING {
				none	(0),
				xonXoff	(1),
				cct106	(2)
				},
		asymmetricFlowControlSupported	BOOLEAN
		}

Description:	The flow control methodology which the DCE is capable of providing.

Operations:	GET

Behaviour:	xonXoffWithPassThrough indicates that Xon/Xoff flow control characters received by the local DCE from the local DTE are transmitted to the remote DCE.

Applications:	Configuration Management

Registered as:	{vSeriesDteInterface flowControlSupported (23)}

inactivityTimerSelect XE "inactivityTimerSelect "§::= INTEGER (0..255)

Description:	Sets the inactivity timeout in minutes.

Operations:	GET-REPLACE

Behaviour:	Applies to start/stop switched operation only. When a call is connected and no data is transferred (continuous marking condition) on both circuits 103 and 104 for the specified time, the DCE disconnects the call. The function is disabled if the value is 0.

Applications:	Configuration Management

Registered as:	{vSeriesDteInterface inactivityTimerSelect (24)}

inactivityTimerSupported XE "inactivityTimerSupported "§::= INTEGER (0..255)

Description:	Allows the network management system to determine whether or not the DCE supports an inactivity timer, and the maximum value (in minutes) supported.

Operations:	GET

Behaviour:	Value 0 indicates that the timer is not supported.

Applications:	Configuration Management

Registered as:	{vSeriesDteInterface inactivityTimerSupported (25)}

longSpaceDisconnectSelect XE "longSpaceDisconnectSelect "§::= ENUMERATED {

disabled	(0),
		disconnectWhenReceived	(1),
		sendToDisconnect	(2),
		both	(3)
		}

Description:	Enables Long Space Disconnect.

Operations:	GET-REPLACE

Behaviour:	Effective only when the DCE is in start/stop non-error-control operation. If disconnectWhenReceived or Both are selected, and the DCE receives a continuous spacing condition in excess of 1.6 seconds, the call is disconnect. In sendToDisconnect mode, the DCE sends at least 4 seconds of long space upon determining that a disconnect is required.

Applications:	Configuration Management

Registered as:	{vSeriesDteInterface longSpaceDisconnectSelect (26)}

longSpaceDisconnectSupported XE "longSpaceDisconnectSupported "§::= BIT STRING {

disabled	(0),
		disconnectWhenReceived	(1),
		sendToDisconnect	(2)
		}

Description:	Allows the Network Management System to determine which modes of long space disconnect are supported by the DCE.

Operations:	GET

Behaviour:	Specification not required

Applications:	Configuration Management

Registered as:	{vSeriesDteInterface longSpaceDisconnectSupported (27)}

parityBits XE "parityBits "§::= ENUMERATED {

odd	(0),
		even	(1),
		mark	(2),
		space	(3)
		}

Description:	Determines the value of the parity bit.

Operations:	GET-REPLACE

Behaviour:	Only applies if selectFormat includes parity. Does not apply if selectAutodetect is TRUE. Refer to characterFormatSelect.

Applications:	Configuration Management

Registered as:	{vSeriesDteInterface parityBits (28)}

responseModeSelect XE "responseModeSelect "§::= ENUMERATED {

disabled	(0),
		shortResponses	(1),
		verboseResponses	(2)
		}

Description:	Specifies the response mode to use.

Operations:	GET-REPLACE

Behaviour:	When disabled, the DCE shall issue no “result codes” of any kind to the DTE either in response to unsolicited events or commands. When shortResponses are selected, the DCE issues responses in their most concise form [e.g. numeric codes in draft Recommendation V.at (V.25 ter)]. When verboseResponses are selected, the DCE issues responses in their long or normal form.

Applications:	Configuration Management

Registered as:	{vSeriesDteInterface responseModeSelect (29)}

responseModeSupported XE "responseModeSupported "§::= BIT STRING {

disabled	(0),
		shortResponses	(1),
		verboseResponses	(2)
		}

Description:	Allows the network management system to determine the V.25 bis and draft Recommendation V.at (V.25 ter) response modes supported by the DCE.

Operations:	GET

Behaviour:	See responseModeSelect.

Applications:	Configuration Management

Registered as:	{vSeriesDteInterface responseModeSupported (30)}

startStopDteInterfaceSpeed XE "startStopDteInterfaceSpeed "§::= SEQUENCE {

toDte	DteBitrate,
		fromDte	DteBitrate
		}

Description:	Sets the speed at the DTE/DCE interface for start stop operation.

Operations:	GET-REPLACE

Behaviour:	It is advisable to use this attribute only if startStopDte-InterfaceSpeed-Adaptation is disabled.

Applications:	Configuration Management

Registered as:	{vSeriesDteInterface startStopDteInterfaceSpeed (31)}

startStopDteInterfaceSpeedAdaptation XE "startStopDteInterfaceSpeedAdaptation "§::= ENUMERATED {

disabled	(0),
		autoDetectDteSpeed	(1),
		sameAsLineRate	(2)
		}

Description:	Sets the method by which DTE interface speed adaptation is accomplished.

Operations:	GET-REPLACE

Behaviour:	Used in conjunction with the startStop-DteInterface-Speed attribute. If dteModeActive is startStopV14, then startStop-DteInterface-SpeedAdaptation should be disabled.

Applications:	Configuration Management

Registered as:	{vSeriesDteInterface startStopDteInterfaceSpeedAdaptation (32)}

v13ModeSelect XE "v13ModeSelect "§::= ENUMERATED {

none	(0),
		txOnly	(1),
		rxOnly	(2),
		bothTxRx	(3)
		}

Description:	Sets the operating mode for the V.13 circuits 105/109 emulation capability.

Operations:	GET-REPLACE

Behaviour:	txOnly and rxOnly indicate V.13 simulated carrier switching is active only in the transmit or receive direction respectively.

Applications:	Configuration Management

Registered as:	{vSeriesDteInterface v13ModeSelect (33)}

v14SignallingRate XE "v14SignallingRate "§::= ENUMERATED {

basicRange	(0),
		extendedRange	(1)
		}

Description:	Selects between V.14 basic and extended signalling rate ranges.

Operations:	GET-REPLACE

Behaviour:	Applies to both transmitter and receiver.

Applications:	Configuration Management

Registered as:	{vSeriesDteInterface v14SignallingRate (34)}

### 3.4.3	Actions

None.

### 3.4.4	Notifications

streamingDetected XE "streamingDetected "§::= EventPriority

Description:	Generated when the anti-streaming timer expires.

Operations:	NOTIFICATION

Behaviour:	Specification not required

Applications:	Configuration Management

Registered as:	{vSeriesDteInterface streamingDetected (34)}

## 3.5	V-Series Error Control Managed Object

### 3.5.1	Managed Object Template

vSeriesErrorControl MANAGED OBJECT CLASS

-- Source ITU T SG14 Q4

-- Status final draft 6 June 1994

DERIVED FROM "Recommendation M.3100: 1992":managedElement

CHARACTERIZED BY

errorControlPkg	PACKAGE
	ATTRIBUTES
	errorControlActive		GET,	
	errorControlSelect		GET-REPLACE,	
	linkState		GET-REPLACE,	
	maximumFrameLengthActive		GET,	
	maximumFrameLengthSelect		GET-REPLACE,	
	maximumFrameLengthSupported		GET,	
	maxRetries		GET-REPLACE,	
	testFrameOptionActive		GET,	
	testFrameOptionSelect		GET-REPLACE,	
	v42BreakOptions		GET-REPLACE,	
	v42CrcActive		GET,	
	v42CrcSelect		GET-REPLACE,	
	v42CrcSupported		GET,	
	v42DetectionPhaseEnable		GET-REPLACE,	
	v42FallbackSelect		GET-REPLACE,	
	v42FallbackSupported		GET,	
	v42RejectOptionsActive		GET,	
	v42RejectOptionsSelect		GET-REPLACE,

v42RejectOptionsSupported		GET,	
	v42Statistics		GET-REPLACE WITH DEFAULT,	
	v42TimedBreakSupported		GET,	
	v42UntimedBreakDuration		GET,	
	windowSizeActive		GET,	
	windowSizeSelect		GET-REPLACE,	
	windowSizeSupported		GET;

REGISTERED AS

{ccitt(0), recommendation(0), v(22), v58(58), vSeriesErrorControl(4)};

### 3.5.2	Attributes

errorControlActive XE "errorControlActive "§::= ENUMERATED {

disable	(0),
		lapm	(1),
		altProtocol	(2)
		}

Description:	Indicates the error control method used during the current or previous call.

Operations:	GET

Behaviour:	Specification not required

Applications:	Configuration Management

Registered as:	{vSeriesErrorControl errorControlActive (0)}

errorControlSelect XE "errorControlSelect "§::= ENUMERATED {

disable	(0),
		lapm	(1),
		forceLAPM	(2),
		forceAltProtocol	(3)
		}

Description:	Enables error control method that can be used during the next call.

Operations:	GET-REPLACE

Behaviour:	Specification not required

Applications:	Configuration Management

Registered as:	{vSeriesErrorControl errorControlSelect (1)}

linkState XE "linkState "§::= ENUMERATED {

linkDisconnected	(0),
		linkConnecting	(1),
		linkIdle	(2),
		linkActive	(3),
		linkDisconnecting	(4)
		}

Description:	Indicates the state of the link layer.

Operations:	GET

Behaviour:	Specification not required

Applications:	Configuration Management

Registered as:	{vSeriesErrorControl linkState (2)}

maximumFrameLengthActive XE "maximumFrameLengthActive "§::= SEQUENCE {

transmitFrameAct	INTEGER (1..65535),
		receiveFrameAct	INTEGER (1..65535)
		}

Description:	Indicates the maximum frame length (in octets) negotiated by the DCE.

Operations:	GET

Behaviour:	Specification not required

Applications:	Configuration Management

Registered as:	{vSeriesErrorControl maximumFrameLengthActive (3)}

maximumFrameLengthSelect XE "maximumFrameLengthSelect "§::= SEQUENCE {

transmitFrameSel	INTEGER (1..65535),
		receiveFrameSel	INTEGER (1..65535)
		}

Description:	Indicates the maximum frame length  (in octets) selected.

Operations:	GET-REPLACE

Behaviour:	Shall be less than or equal to maximumFrameLengthSupported.

Applications:	Configuration Management

Registered as:	{vSeriesErrorControl (4), maximumFrameLengthSelect (4)}

maximumFrameLengthSupported XE "maximumFrameLengthSupported "§::= SEQUENCE {

transmitFrameSup	INTEGER (1..65535),
		receiveFrameSup	INTEGER (1..65535)
		}

Description:	Indicates the maximum frame length  (in octets) the DCE can support.

Operations:	GET

Behaviour:	See maximumFrameLengthSelect.

Applications:	Configuration Management

Registered as:	{vSeriesErrorControl maximumFrameLengthSupported (5)}

maxRetries XE "maxRetries "§::= INTEGER (1..255)

Description:	Determines the maximum number of retries before disconnection (N400).

Operations:	GET-REPLACE

Behaviour:	See also the callCleared notification; clearCause 93.

Applications:	Performance Management

Registered as:	{vSeriesErrorControl maxRetries (6)}

testFrameOptionActive XE "testFrameOptionActive "§::= BOOLEAN

Description:	Indicates whether or not the testframe option is negotiated with the remote DCE.

Operations:	GET

Behaviour:	Specification not required

Applications:	Configuration Management

Registered as:	{vSeriesErrorControl testFrameOptionActive (7)}

testFrameOptionSelect XE "testFrameOptionSelect "§::= BOOLEAN

Description:	Enables the testframe option to be used if implemented and negotiated with the remote DCE.

Operations:	GET-REPLACE

Behaviour:	Specification not required

Applications:	Configuration Management

Registered as:	{vSeriesErrorControl testFrameOptionSelect (8)}

v42BreakOptions XE "v42BreakOptions "§::= ENUMERATED {

nonDesNonExp	(0),
		nonDesExp	(1),
		desExp	(2),
		desNonExp	(3),
		doNotSendBreak	(4)
		}

Description:	Indicates which V.42 break option is to be used.

Operations:	GET-REPLACE

Behaviour:	Specification not required

Applications:	Configuration Management

Registered as:	{vSeriesErrorControl v42BreakOptions (9)}

v42CrcActive XE "v42CrcActive "§::= ENUMERATED {

crc16bit(0),
		crc32bit(1)
		}

Description:	CRC established for the current or previous call.

Operations:	GET

Behaviour:	Specification not required

Applications:	Configuration Management

Registered as:	{vSeriesErrorControl v42CrcActive (10)}

v42CrcSelect XE "v42CrcSelect "§::= ENUMERATED {

crc16bit(0),
		crc32bit(1)
		}

Description:	Indicates the preferred CRC option to be negotiated for the next call.

Operations:	GET-REPLACE

Behaviour:	Specification not required

Applications:	Configuration Management

Registered as:	{vSeriesErrorControl v42CrcSelect (11)}

v42CrcSupported XE "v42CrcSupported "§::= BIT STRING {

crc16bit(0),
		crc32bit(1)
		}

Description:	Indicates the CRC options supported by the DCE.

Operations:	GET

Behaviour:	Specification not required

Applications:	Configuration Management

Registered as:	{vSeriesErrorControl v42CrcSupported (12)}

v42DetectionPhaseEnable XE "v42DetectionPhaseEnable "§::= ENUMERATED {

disable	(0),
		useV42DetPhase	(1),
		useV8ifAvailable	(2)
		}

Description:	Indicates whether the error control detection phase shall be used.

Operations:	GET-REPLACE

Behaviour:	Specification not required

Applications:	Configuration Management

Registered as:	{vSeriesErrorControl v42DetectionPhaseEnable (13)}

v42FallbackSelect XE "v42FallbackSelect "§::= ENUMERATED {

disconnect	(0),
		buffered	(1),
		unbufferedV14	(2)
		}

Description:	Selects what mode the DCE should enter if it is unable to establish an error control protocol (LAPM or Alternative).

Operations:	GET-REPLACE

Behaviour:	If the DCE exhausts all of the attempts specified to establish the enabled error control protocol(s), it takes the action specified by this parameter: disconnect the call, enter buffered mode (with flow control), or enter unbuffered V.14 operation without flow control (adjusting the DTE interface speed to match the line speed). The selected method must be one of v42FallbackSupported.

Applications:	Configuration Management

Registered as:	{vSeriesErrorControl v42FallbackSelect (14)}

v42FallbackSupported XE "v42FallbackSupported "§::= BIT STRING {

disconnect	(0),
		buffered	(1),
		unbufferedV14	(2)
		}

Description:	Allows the Network Management System to determine which fallback modes are supported by the DCE.

Operations:	GET

Behaviour:	See v42FallbackSelect

Applications:	Configuration Management

Registered as:	{vSeriesErrorControl v42FallbackSupported (15)}

v42RejectOptionsActive XE "v42RejectOptionsActive "§::= ENUMERATED {

useGoBackN	(0),
		useSREJ	(1)
		}

Description:	Indicates the negotiated reject option.

Operations:	GET

Behaviour:	See v42RejectOptionsSelect

Applications:	Configuration Management

Registered as:	{vSeriesErrorControl v42RejectOptionsActive (16)}

v42RejectOptionsSelect XE "v42RejectOptionsSelect "§::= ENUMERATED {

useGoBackN	(0),
		useSREJ	(1)
		}

Description:	Indicates the preferred Reject option.

Operations:	GET-REPLACE

Behaviour:	Shall be one of v42RejectOptionsSupported

Applications:	Configuration Management

Registered as:	{vSeriesErrorControl v42RejectOptionsSelect (17)}

v42RejectOptionsSupported XE "v42RejectOptionsSupported "§::= BIT STRING {

useGoBackN	(0),
		useSREJ	(1)
		}

Description:	Indicates the supported reject options.

Operations:	GET

Behaviour:	See v42RejectOptionsSelect

Applications:	Configuration Management

Registered as:	{vSeriesErrorControl v42RejectOptionsSupported (18)}

v42Statistics XE "v42Statistics "§::= SEQUENCE {

framesSentAcknowledged		COUNTER (0..65535),
		framesRetransmitted		COUNTER (0..65535),
		framesReceivedAcknowledged	COUNTER (0..65535),
		framesReceivedDiscarded	COUNTER (0..65535)
		}

Description:	Reports on V.42 performance.

Operations:	GET

Behaviour:	framesSentAcknowledged is the number of frames containing user data transmitted by the DCE for which acknowledgement has been received.

framesRetransmitted is the number of frames containing user data retransmitted by the DCE.

framesReceivedAcknowledged is the number of frames containing user data received by the DCE for which acknowledgement has been sent.

framesReceivedDiscarded is the number of frames discarded.

Default value is 0. Counters do not reset to zero if maximum count reached. Counters reset to zero at reconnection.

Applications:	Performance Management

Registered as:	{vSeriesErrorControl v42Statistics (19)}

v42TimedBreakSupported XE "v42TimedBreakSupported "§::= BOOLEAN

Description:	Allows the Network Management System to determine whether or not the DCE supports the transmission of timed breaks.

Operations:	GET

Behaviour:	Specification not required

Applications:	Configuration Management

Registered as:	{vSeriesErrorControl v42TimedBreakSupported (20)}

v42UntimedBreakDuration XE "v42UntimedBreakDuration "§::= INTEGER (1..255)

Description:	Specifies the duration in 10 millisecond increments of the break to be delivered to the DTE when the DCE receives an untimed break from the remote DCE.

Operations:	GET-REPLACE

Behaviour:	Specification not required

Applications:	Configuration Management

Registered as:	{vSeriesErrorControl v42UntimedBreakDuration (21)}

windowSizeActive XE "windowSizeActive "§::= SEQUENCE {

transmitWindowAct	INTEGER (1..128),
		receiveWindowAct	INTEGER (1..128)
		}

Description:	Indicates the window size negotiated by the DCE.

Operations:	GET

Behaviour:	See windowSizeSelect

Applications:	Configuration Management

Registered as:	{vSeriesErrorControl windowSizeActive (22)}

windowSizeSelect XE "windowSizeSelect "§::= SEQUENCE {

transmitWindowSel	INTEGER (1..128),
		receiveWindowSel	INTEGER (1..128)
		}

Description:	Indicates the window size to be negotiated by the DCE.

Operations:	GET-REPLACE

Behaviour:	Must be less than or equal to windowSizeSupported.

Applications:	Configuration Management

Registered as:	{vSeriesErrorControl windowSizeSelect (23)}

windowSizeSupported XE "windowSizeSupported "§::= SEQUENCE {

transmitWindowSup	INTEGER (1..128),
		receiveWindowSup	INTEGER (1..128)
		}

Description:	Indicates the maximum window size supported by the DCE.

Operations:	GET

Behaviour:	See windowSizeSelect

Applications:	Configuration Management

Registered as:	{vSeriesErrorControl windowSizeSupported (24)}

### 3.5.3	Actions

None.

### 3.5.4	Notifications

None.

## 3.6	V-Series Line Interface Managed Object

### 3.6.1	Managed Object Template

vSeriesLineInterface MANAGED OBJECT CLASS

-- Source ITU T SG14 Q4

-- Status final draft 6 June 1994

DERIVED FROM "Recommendation M.3100: 1992":managedElement

CHARACTERIZED BY

vSeriesLineInterfacePkg	PACKAGE
	ATTRIBUTES
	lineTypeActive			GET,
	lineTypeSelect			GET-REPLACE,
	lineTypeSupported		GET,
	transmitLevelActive		GET,
	transmitLevelAdjustable		GET,
	transmitLevelSelect		GET-REPLACE;

CONDITIONAL PACKAGES

gstnPkg	PACKAGE
	ATTRIBUTES
	lineSignalFailDisconnectTimer		GET-REPLACE WITH DEFAULT;
	NOTIFICATIONS
	callCleared	
	callEstablished	
	ringIndication;	
	PRESENT IF "the lineTypeSelect is GSTN 2 or 4 wire",

	lineAutocallPkg	PACKAGE
	ATTRIBUTES
	answerToneSelect		GET-REPLACE,
	answerToneSupported		GET,
	busyDetection		GET-REPLACE,
	callingToneSelect		GET-REPLACE,
	callingToneSupported		GET,
	callSetupFailTimer		GET-REPLACE,
	dialSignalling		GET-REPLACE,
	dialToneDetection		GET-REPLACE,
	dtmfToneDuration		GET-REPLACE,
	pulseDialModeSelect		GET-REPLACE,
	pulseDialModeSupported		GET;
	PRESENT IF "the V-Series DCE has an autocall capability";

REGISTERED AS

{ccitt(0), recommendation(0), v(22), v58(58), vSeriesLineInterface(5)};

### 3.6.2	Attributes

answerToneSelect XE "answerToneSelect "§::= ENUMERATED {

disabled	(0),
		enabled	(1)
		}

Description:	Selects the type of answer tone to be transmitted, if any.

Operations:	GET-REPLACE

Behaviour:	When the DCE is in switched operation and answers a call, and disabled is not selected, answer tone is transmitted in accordance with the capabilities of the DCE and the appropriate Recommendations.

Applications:	Configuration Management

Registered as:	{vSeriesLineInterface answerToneSelect (0)}

answerToneSupported XE "answerToneSupported "§::= BIT STRING {

none	(0),
		v25	(1),
		v8	(2)
		}

Description:	Allows Network Management System to determine which Answer Tone modes are supported by the DCE.

Operations:	GET

Behaviour:	See answerToneSelect

Applications:	Configuration Management

Registered as:	{vSeriesLineInterface answerToneSupported (1)}

busyDetection XE "busyDetection "§::= BOOLEAN

Description:	Specifies whether or not the DCE listens for busy signals (engaged tones) while placing calls.

Operations:	GET-REPLACE

Behaviour:	If TRUE, the DCE listens for engaged tone after dialling the phone number; if the signal is heard, the call attempt is abandoned. If FALSE, the DCE does not listen for busy signal.

Applications:	Configuration Management

Registered as:	{vSeriesLineInterface busyDetection (2)}

callingToneSelect XE "callingToneSelect "§::= ENUMERATED {

none	(0),
		v25-1100Hz	(1),
		v25-1300Hz	(2),
		dceBinaryOne	(3),
		v8-CallingIndicator	(4),
		automatic	(5)
		}

Description:	Selects the calling tone or indicator that will be transmitted by the DCE.

Operations:	GET-REPLACE

Behaviour:	The automatic option permits the DCE to select the type of calling tone automatically.

Applications:	Configuration Management

Registered as:	{vSeriesLineInterface callingToneSelect (3)}

callingToneSupported XE "callingToneSupported "§::= BIT STRING {

none	(0),
		v25-1100Hz	(1),
		v25-1300Hz	(2),
		dceBinaryOne	(3),
		v8-CallingIndicator	(4)
		}

Description:	Calling tones or indications that the DCE is capable of transmitting.

Operations:	GET

Behaviour:	Specification not required

Applications:	Configuration Management

Registered as:	{vSeriesLineInterface callingToneSupported (4)}

callSetupFailTimer XE "callSetupFailTimer "§::= INTEGER (0..255)

Description:	If the connection has not been established within the duration of this (seconds) timer, the call attempt will be abandoned and the DCE will be disconnected.

Operations:	GET-REPLACE

Behaviour:	A value of 0 indicates that the call set-up fail timer is disabled.

Applications:	Fault Management

Registered as:	{vSeriesLineInterface callSetupFailTimer (5)}

dialSignalling XE "dialSignalling "§::= ENUMERATED {

dTMF	(0),
		pulse	(1)
		}

Description:	Signalling method utilized by the DCE for initial call establishment.

Operations:	GET-REPLACE

Behaviour:	Specification not required

Applications:	Configuration Management

Registered as:	{vSeriesLineInterface dialSignalling (6)}

dialToneDetection XE "dialToneDetection "§::= SEQUENCE {

dialToneRequired	BOOLEAN,
		pause	INTEGER (0..16),
		giveUpTimer	INTEGER (0..255)
		}

Description:	Specifies whether the DCE should listen for dial tone before dialling, or the amount of time, in seconds, to delay before “blind dialling” (whether or not a dial tone is present).

Operations:	GET-REPLACE

Behaviour:	If dialToneRequired is TRUE, the DCE must detect dial tone before it begins dialling a call; if no dial tone is detected, the call attempt is abandoned. If dialToneRequired is FALSE, the DCE does not detect dial tone, but instead simply pauses the specified number of seconds after going off hook before beginning dialling if so instructed.

Applications:	Configuration Management

Registered as:	{vSeriesLineInterface dialToneDetection (7)}

dtmfToneDuration XE "dtmfToneDuration "§::= INTEGER (50..255)

Description:	Duration of time in milliseconds that a particular DTMF tone is transmitted.

Operations:	GET-REPLACE

Behaviour:	Specification not required

Applications:	Configuration Management

Registered as:	{vSeriesLineInterface dtmfToneDuration (8)}

lineSignalFailDisconnectTimer XE "lineSignalFailDisconnectTimer "§::= INTEGER (1..255)

Description:	If line signal is lost for the entire duration (in increments of 100 milliseconds) of this period, the call is presumed to have failed and the DCE is disconnected from the line.

Operations:	GET-REPLACE

Behaviour:	A value of 255 indicates that the timer is disabled.

Applications:	Fault Management

Registered as:	{vSeriesLineInterface lineSignalFailDisconnectTimer (9)}

lineTypeActive XE "lineTypeActive "§::=  SEQUENCE {

lineType	ENUMERATED {
				leasedP-P4Wire	(0),
				leasedMultipoint4Wire	(1),
				leasedP-P2Wire	(2),
				leasedMultipoint2Wire	(3),
				gSTN4Wire	(4),
				gSTN2Wire	(5)
				},
		cellularLinkLocal		BOOLEAN,
		cellularLinkRemote	BOOLEAN,
		cmePresent			BOOLEAN
		}

Description:	Indicates the current line type, whether or not one or more cellular links are present, and whether the connection is passing through circuit multiplexing equipment that is performing a demod/remod function.

Operations:	GET

Behaviour:	See Recommendation V.8 for details on the indication of cellular links and CME.

Applications:	Configuration Management

Registered as:	{vSeriesLineInterface lineTypeActive (10)}

lineTypeSelect XE "lineTypeSelect "§::=  SEQUENCE {

lineType	ENUMERATED {
				leasedP-P4Wire	(0),
				leasedMultipoint4Wire	(1),
				leasedP-P2Wire	(2),
				leasedMultipoint2Wire	(3),
				gSTN4Wire	(4),
				gSTN2Wire	(5)
				},
		cellularLinkLocal	BOOLEAN
		}

Description:	Selects the line type for which the DCE is configured to operate, and whether the local connection is a cellular link.

Operations:	GET-REPLACE

Behaviour:	See Recommendation V.8 for details on the indication of cellular links.

Applications:	Configuration Management

Registered as:	{vSeriesLineInterface lineTypeSelect (11)}

lineTypeSupported XE "lineTypeSupported "§::=  SEQUENCE {

lineType	BIT STRING {
				leasedP-P4Wire	(0),
				leasedMultipoint4Wire	(1),
				leasedP-P2Wire	(2),
				leasedMultipoint2Wire	(3),
				gSTN4Wire	(4),
				gSTN2Wire	(5)
				},
		cellularLink		BOOLEAN
		}

Description:	Indicates the possible line types for which the DCE can be configured, and whether or not the DCE can be configured to indicate the use of a cellular link.

Operations:	GET

Behaviour:	Specification not required

Applications:	Configuration Management

Registered as:	{vSeriesLineInterface lineTypeSupported (12)}

pulseDialModeSelect XE "pulseDialModeSelect "§::= SEQUENCE {

pulseDialRate	ENUMERATED {
				rate10PPS	(0),
				rate20PPS	(1)
				},
		pulseDialRatio	ENUMERATED {
				m33B67	(0),
				m38B62	(1),
				m40M60	(2)
				}
		}

Description:	Selects the pulse dial configuration.

Operations:	GET-REPLACE

Behaviour:	When the DCE is using pulse dialling, it uses the rate and make/break ratio specified.

Applications:	Configuration Management

Registered as:	{vSeriesLineInterface pulseDialModeSelect (13)}

pulseDialModeSupported XE "pulseDialModeSupported "§::= SEQUENCE {

pulseDialRate	BIT STRING {
				rate10PPS	(0),
				rate20PPS 	(1)
				},
		pulseDialRatio	BIT STRING {
				m33B67	(0),
				m38B62	(1),
				m40M60	(2)
				}
		}

Description:	Allows the Network Management System to determine the configurability of the DCE’s pulse dial function.

Operations:	GET

Behaviour:	See pulseDialModeSelect

Applications:	Configuration Management

Registered as:	{vSeriesLineInterface pulseDialModeSupported (14)}

transmitLevelActive XE "transmitLevelActive "§::= INTEGER (-20..6)

Description:	Indicates the current transmit level in dBm of the DCE at its line interface.

Operations:	GET

Behaviour:	For a V.34 modem this may be lower than the transmit level selected.

Applications:	Configuration Management

Registered as:	{vSeriesLineInterface transmitLevelActive (15)}

transmitLevelAdjustable XE "transmitLevelAdjustable "§::= SEQUENCE {

externallyAdjustable	BOOLEAN,
		internallyAdjustable	BOOLEAN
		}

Description:	This attribute indicates whether the transmit level is adjustable.

Operations:	GET

Behaviour:	The FALSE condition of these parameters indicates that the Transmit Level are not programmable.

Applications:	Configuration Management

Registered as:	{vSeriesLineInterface transmitLevelAdjustable (16)}

transmitLevelSelect XE "transmitLevelSelect "§::= INTEGER (-20..6)

Description:	This parameter sets the nominal transmit level in dBm of the DCE at its line interface.

Operations:	GET-REPLACE

Behaviour:	If the transmit level can be adjusted externally, this value can be modified only if transmitLevelAdjustable.internallyAdjustable is TRUE.  The transmit level of a V.34 modem may be adjusted internally to a level below the nominal transmit power selected above.

Applications:	Configuration Management

Registered as:	{vSeriesLineInterface transmitLevelSelect (17)}

### 3.6.3	Actions

None.

### 3.6.4	Notifications

callCleared XE "callCleared "§::= SEQUENCE {

priority	EventPriority,
		clearCause	ENUMERATED {
				causeUnidentified	(0),

-- Network Management System

nMSinitiatedDialCall	(10),
			nMSinitiatedLeasedLineRestoral	(11),
			nMSinitiatedRedial	(12),
			nMSinitiatedDialDisconnect	(13),

-- DCE

powerLoss	(20),
			equipmentFailure	(21),
			frontPanelDisconnectRequested	(22),
			frontPanelLeasedLineRestoral	(23),
			automaticLeasedLineRestoral	(24),
			inactivityTimerExpired	(25),

-- DTE Interface

cct116RestoralRequest	(30),
			cct108isOffInhibitsDial	(31),
			cct108turnedOff	(32),

-- Line Interface

noNumberProvided	(40),
			blacklistedNumber	(41),
			callAttemptsLimitExceeded	(42),
			extensionPhoneOffHook	(43),
			callSetupFailTimerExpired	(44),
			incomingCallDetected	(45),
			loopCurrentInterrupted	(46),
			noDialTone	(47),
			voiceDetected	(48),
			reorderTone	(49),
			sitTone	(50),
			engagedTone	(51),
			longSpaceDisconnect	(52),

-- Signal Converter

carrierLost	(60),
			trainingFailed	(61),
			noModulationinCommon	(62),
			retrainFailed	(63),
			retrainAttemptCountExceeded	(64),
			gstnCleardownReceived	(65),
			faxDetected	(66),

-- Test

inTestMode	(70),
			intrusiveSelfTestInitiated	(71),

-- Call Control

anyKeyAbort	(80),
			dteHangupCommand	(81),
			dteResetCommand	(82),

-- Error Control

frameReject	(90),
			noErrorControlEstablished	(91),
			protocolViolation	(92),
			n400exceeded	(93),
			negotiationFailed	(94),
			disconnectFrameReceived	(95),
			sabmeFrameReceived	(96),

-- Data Compression

lossOfSynchronization	(100)
			}
		}

Description:	Indicates that the DCE has gone on hook and that the previously existing network connection has been cleared.

Operations:	NOTIFICATION

Behaviour:	Specification not required

Applications:	Configuration Management

Registered as:	{vSeriesLineInterface callCleared (18)}

callEstablished XE "callEstablished "§::= SEQUENCE {

priority	EventPriority,
	speed	DceBitRate,
	errorControl	BOOLEAN,
	compression	BOOLEAN
	}

Description:	Indicates that the DCE has gone off hook and that a network connection has been established to the remote DCE.

Operations:	NOTIFICATION

Behaviour:	Specification not required

Applications:	Configuration Management

Registered as:	{vSeriesLineInterface callEstablished (19)}

ringIndication  XE "ringIndication  "§::= EventPriority

Description:	Indicates that the DCE has detected a ringing signal.

Operations:	NOTIFICATION

Behaviour:	Specification not required

Applications:	Configuration Management

Registered as:	{vSeriesLineInterface ringIndication (20)}

## 3.7	V-Series Signal Convertor Managed Object

### 3.7.1	Managed Object Template

vSeriesSignalConvertor MANAGED OBJECT CLASS

-- Source ITU T SG14 Q4

-- Status final draft 6 June 1994

DERIVED FROM "Recommendation M.3100: 1992":managedElement

CHARACTERISED BY

signalConvertorPkg	PACKAGE
	ATTRIBUTES
	gstnCallMode		GET-REPLACE,
	gstnModulationSchemeActive		GET,
	gstnModulationSchemesSelect		GET-REPLACE,
	gstnModulationSchemesSupported	GET,
	leasedCallMode		GET-REPLACE,
	leasedModulationSchemeSelect		GET-REPLACE,

leasedModulationSchemesSupported	GET,
	transmissionSignallingRateActive		GET,
	transmissionSignallingRatesSelect		GET-REPLACE,
	transmissionSignallingRatesSupported	GET,
	transmitClockSource		GET-REPLACE,
	v22V22bisGuardToneEnable		GET-REPLACE;

CONDITIONAL PACKAGES

extendedSignalConvertorPkg	PACKAGE
	ATTRIBUTES
	equalizationSelect		GET-REPLACE,
	equalizationSupported		GET,
	fallBackSignalQualityThreshold		GET-REPLACE,
	fallForwardSignalQualityThreshold	GET-REPLACE,
	rateRenegotiationInitiation		GET-REPLACE,
	receiveLevel		GET,
	redialSignalQualityThreshold		GET-REPLACE,
	retrainInitiateCounter		GET-REPLACE WITH DEFAULT,
	retrainRequestsCounter		GET-REPLACE WITH DEFAULT,
	retrainSignalQualityThreshold		GET-REPLACE,
	signalQualityEstimate		GET,
	v32TrellisActive		GET,
	v32TrellisSelect		GET-REPLACE;
	ACTIONS
	fallBackRequest,
	fallForwardRequest,
	retrain;
	NOTIFICATIONS
	fallBackNotification,
	fallForwardNotification,
	goodSignalQuality,
	lossOfCarrier,
	poorSignalQuality;
	PRESENT IF "implemented";

v34SignalConvertorPkg	PACKAGE
	ATTRIBUTES
	v34FeaturesActive		GET,
	v34FeaturesSelect		GET-REPLACE,
	v34FeaturesSupported		GET,
	v34PreemphasisFilterActive		GET,
	v34SymbolRateCarrierFrequenciesActive	GET,
	v34SymbolRateCarrierFrequenciesSelect	GET-REPLACE,
	v34SymbolRateCarrierFrequenciesSupported	GET;
	PRESENT IF "V.34 implemented";

REGISTERED AS

{ccitt(0), recommendation(0), v(22), v58(58), vSeriesSignalConvertor(6)};

NOTE – Analogue Parameters are meant to be informative, implementations may differ.

### 3.7.2	Attributes

equalizationSelect XE "equalizationSelect "§::= INTEGER (0..255)

Description:	Selects a stored equalizer compromise configuration.

Operations:	GET-REPLACE

Behaviour:	A value of 0 indicates that no compromise equalizer is enabled. A value in excess of equalizationSupported shall be considered invalid.

Applications:	Configuration Management

Registered as:	{vSeriesSignalConvertor equalizationSelect (0)}

equalizationSupported XE "equalizationSupported "§::= INTEGER (0..255)

Description:	Indicates a maximum number of stored compromise equalizer settings.

Operations:	GET

Behaviour:	A value of 0 indicates the DCE does not contain a compromise equalizer function.

Applications:	Configuration Management

Registered as:	{vSeriesSignalConvertor equalizationSupported (1)}

fallBackSignalQualityThreshold XE "fallBackSignalQualityThreshold "§::= ENUMERATED {

disabled	(0),
		high	(1),
		normal	(2),
		low	(3)
		}

Description:	Determines the quality threshold that causes the DCE to fall back.

Operations:	GET-REPLACE

Behaviour:	“High” threshold indicates that the signal quality must be worse than the “normal” or “low” threshold setting to cause fall back.

Applications:	Performance Management

Registered as:	{vSeriesSignalConvertor fallBackSignalQualityThreshold (2)}

fallForwardSignalQualityThreshold XE "fallForwardSignalQualityThreshold "§::= ENUMERATED {

disabled	(0),
		high	(1),
		normal	(2),
		low	(3)
		}

Description:	Determines the quality threshold that causes the DCE to fall forward.

Operations:	GET-REPLACE

Behaviour:	“High” threshold indicates that the signal quality must be better than the “normal” or “low” threshold setting to cause fall forward.

Applications:	Performance Management

Registered as:	{vSeriesSignalConvertor fallForwardSignalQualityThreshold (3)}

gstnCallMode XE "gstnCallMode "§::= ENUMERATED {

normal	(0),
		answerMode	(1),
		callMode	(2)
		}

Description:	Defines whether the DCE is an Answer or Call device in GSTN operation.

Operations:	GET-REPLACE

Behaviour:	Valid only when DCE is in GSTN mode. Normal is call/answer mode dependent on call originate. answerMode is answer mode regardless of call originate. callMode is call mode regardless of call originate.

Applications:	Configuration Management

Registered as:	{vSeriesSignalConvertor gstnCallMode (4)}

gstnModulationSchemeActive XE "gstnModulationSchemeActive "§::= ENUMERATED {

v17	(0),
		v21	(1),
		v22	(2),
		v22bis	(3),
		v23CC	(4),
		v23SC	(5),
		v26bis	(6),
		v26ter	(7),
		v27ter	(8),
		v29HD	(9),
		v32	(10),
		v32bis	(11),
		v34	(12),
		v34HD	(13),
		reserved	(14)
		}

Description:	Indicates the current modulation scheme.

Operations:	GET

Behaviour:	CC = Continuous Carrier, SC = Switched Carrier, HD = Half Duplex. The use of V17 and V29HD for facsimile applications is defined in Recommendation T.30.

Applications:	Configuration Management

Registered as:	{vSeriesSignalConvertor gstnModulationSchemeActive (5)}

gstnModulationSchemesSelect XE "gstnModulationSchemesSelect "§::= BIT STRING {

v17	(0),
		v21	(1),
		v22	(2),
		v22bis	(3),
		v23CC	(4),
		v23SC	(5),
		v26bis	(6),
		v26ter	(7),
		v27ter	(8),
		v29HD	(9),
		v32	(10),
		v32bis	(11),
		v34	(12),
		v34HD	(13),
		reserved	(14)
		}

Description:	Enables one or more modulation schemes. Enabling more than one scheme provides the possibility to specify a range of modulation schemes the DCE can select from (“Auto-moding”).

Operations:	GET-REPLACE

Behaviour:	Must be included in gstnModulationSchemesSupported.

Applications:	Configuration Management

Registered as:	{vSeriesSignalConvertor gstnModulationSchemesSelect (6)}

gstnModulationSchemesSupported XE "gstnModulationSchemesSupported "§::= BIT STRING {

v17	(0),
		v21	(1),
		v22	(2),
		v22bis	(3),
		v23CC	(4),
		v23SC	(5),
		v26bis	(6),
		v26ter	(7),
		v27ter	(8),
		v29HD	(9),
		v32	(10),
		v32bis	(11),
		v34	(12),
		v34HD	(13),
		reserved	(14)
		}

Description:	Modulation schemes implemented in the DCE.

Operations:	GET

Behaviour:	Specified in gstnModulationSchemesSelect

Applications:	Configuration Management

Registered as:	{vSeriesSignalConvertor gstnModulationSchemesSupported (7)}

leasedCallMode XE "leasedCallMode "§::= ENUMERATED {

answerMode	(0),
		callMode	(1)
		}

Description:	Defines whether the DCE is an Answer or Call device in leased line operation.

Operations:	GET-REPLACE

Behaviour:	Valid only when DCE is in Leased Line mode.

Applications:	Configuration Management

Registered as:	{vSeriesSignalConvertor leasedCallMode (8)}

leasedModulationSchemeSelect XE "leasedModulationSchemeSelect "§::= ENUMERATED {

v21	(0),
		v22	(1),
		v22bis	(2),
		v23	(3),
		v26CC	(4),
		v26SC	(5),

v26ter	(6),
		v27CC	(7),
		v27SC	(8),
		v27bisCC	(9),
		v27bisSC	(10),
		v29	(11),
		v32	(12),
		v32bis	(13),
		v33	(14),
		v34	(15),
		v34HD	(16),
		reserved	(17),
		vendorSpecific	(18)
		}

Description:	Selects the modulation scheme for leased line applications.

Operations:	GET-REPLACE

Behaviour:	CC = Continuous Carrier, SC = Switched Carrier, HD = Half Duplex. Must be one of leasedModulationSchemesSupported.

Applications:	Configuration Management

Registered as:	{vSeriesSignalConvertor leasedModulationSchemeSelect (9)}

leasedModulationSchemesSupported XE "leasedModulationSchemesSupported "§::= BIT STRING {

v21	(0),
		v22	(1),
		v22bis	(2),
		v23	(3),
		v26CC	(4),
		v26SC	(5),
		v26ter	(6),
		v27CC	(7),
		v27SC	(8),
		v27bisCC	(9),
		v27bisSC	(10),
		v29	(11),
		v32	(12),
		v32bis	(13),
		v33	(14),
		v34	(15),
		v34HD	(16),
		reserved	(17),
		vendorSpecific	(18)
		}

Description:	Modulation schemes implemented in the DCE.

Operations:	GET

Behaviour:	Specification not required

Applications:	Configuration Management

Registered as:	{vSeriesSignalConvertor leasedModulationSchemesSupported (10)}

rateRenegotiationInitiation XE "rateRenegotiationInitiation "§::= BOOLEAN

Description:	Enables automatic initiation of V.32 bis or V.34 rate renegotiation.

Operations:	GET-REPLACE

Behaviour:	Specification not required

Applications:	Configuration Management

Registered as:	{vSeriesSignalConvertor rateRenegotiationInitiation (11)}

receiveLevel XE "receiveLevel "§::= INTEGER (-60..0)

Description:	Reports receive signal level in dBm at the line interface of the DCE.

Operations:	GET

Behaviour:	Specification not required

Applications:	Configuration Management

Registered as:	{vSeriesSignalConvertor receiveLevel (12)}

redialSignalQualityThreshold XE "redialSignalQualityThreshold "§::= ENUMERATED {

disabled	(0),
		high	(1),
		normal	(2),
		low	(3)
		}

Description:	Determines the quality threshold that causes the DCE to re-establish the connection.

Operations:	GET-REPLACE

Behaviour:	“High” threshold indicates that the signal quality must be worse than the “normal” or “low” threshold setting to cause re-establishment.

Applications:	Performance Management

Registered as:	{vSeriesSignalConvertor redialSignalQualityThreshold (13)}

retrainInitiateCounter XE "retrainInitiateCounter "§::= INTEGER (0..255)

Description:	Counts attempted retrains initiated by the DCE.

Operations:	GET-REPLACE WITH DEFAULT

Behaviour:	Default 0; does not reset to zero if maximum count reached; resets to zero at reconnection.

Applications:	Performance Management

Registered as:	{vSeriesSignalConvertor retrainInitiateCounter (14)}

retrainRequestsCounter XE "retrainRequestsCounter "§::= INTEGER (0..255)

Description:	Counts retrain requests received by the DCE.

Operations:	GET-REPLACE WITH DEFAULT

Behaviour:	Default 0; does not reset to zero if maximum count reached; resets to zero at reconnection.

Applications:	Performance Management

Registered as:	{vSeriesSignalConvertor retrainRequestsCounter (15)}

retrainSignalQualityThreshold XE "retrainSignalQualityThreshold "§::= ENUMERATED {

disabled	(0),
		high	(1),
		normal	(2),
		low	(3)
		}

Description:	Determines the quality threshold that causes the DCE to retrain.

Operations:	GET-REPLACE

Behaviour:	“High” threshold indicates that the signal quality must be worse than the “normal” or “low” threshold setting to cause retrain.

Applications:	Performance Management

Registered as:	{vSeriesSignalConvertor retrainSignalQualityThreshold (16)}

signalQualityEstimate XE "signalQualityEstimate "§::= ENUMERATED {

good	(0),
		average	(1),
		poor	(2)
		}

Description:	Reports an estimate of the signal quality.

Operations:	GET

Behaviour:	Specification not required

Applications:	Performance Management

Registered as:	{vSeriesSignalConvertor signalQualityEstimate (17)}

transmissionSignallingRateActive XE "transmissionSignallingRateActive "§::= SEQUENCE {

transmit	DceBitrate,
		receive	DceBitrate
		}

Description:	Indicates the current DCE signalling rates.

Operations:	GET

Behaviour:	See transmissionSignalling-RatesSupported and transmissionSignalling-RatesSelect

Applications:	Configuration Management

Registered as:	{vSeriesSignalConvertor transmissionSignallingRateActive (18)}

transmissionSignallingRatesSelect XE "transmissionSignallingRatesSelect "§::= SEQUENCE {

transmit	DceBitrateRange,
		receive	DceBitrateRange
		}

Description:	Indicates the range of bitrates enabled.

Operations:	GET-REPLACE

Behaviour:	More than one signalling rate can be enabled. The enabled signalling rates must be contained in transmissionSignallingRatesSupported. The actual selected signalling rate is contained in transmissionSignallingRateActive. Some DCE specifications may not allow “Transmit” and “Receive” to be different.

Applications:	Configuration Management

Registered as:	{vSeriesSignalConvertor transmissionSignallingRatesSelect (19)}

transmissionSignallingRatesSupported XE "transmissionSignallingRatesSupported "§::= SEQUENCE {

transmit	DceBitrateRange,
		receive	DceBitrateRange
		}

Description:	Signalling rate supported by the DCE.

Operations:	GET

Behaviour:	One or more than one signalling rate can be enabled with transmissionSignallingRatesSelect. The actual selected signalling rate is contained in transmissionSignallingRateActive. Some DCE specifications may not allow “Transmit” and “Receive” to be different.

Applications:	Configuration Management

Registered as:	{vSeriesSignalConvertor transmissionSignallingRatesSupported (20)}

transmitClockSource XE "transmitClockSource "§::= ENUMERATED {

internal	(0),
		external	(1),
		slaveToReceiveClock	(2)
		}

Description:	Selects the source of the DCE’s transmit timing.

Operations:	GET-REPLACE

Behaviour:	This attribute applies only to DCEs which use synchronous modulation. For Asynchronous DTE-DCE interfaces, external timing sources are not used.

Applications:	Configuration Management

Registered as:	{vSeriesSignalConvertor transmitClockSource (21)}

v22V22bisGuardToneEnable XE "v22V22bisGuardToneEnable "§::= BOOLEAN

Description:	Enables guard tone in V.22(bis) DCEs.

Operations:	GET-REPLACE

Behaviour:	Specification not required

Applications:	Configuration Management

Registered as:	{vSeriesSignalConvertor v22V22bisGuardToneEnable (22)}

v32TrellisActive XE "v32TrellisActive "§::= BOOLEAN

Description:	Indicates whether Trellis coding is active in the V.32 DCE.

Operations:	GET

Behaviour:	See v32TrellisSelect

Applications:	Configuration Management

Registered as:	{vSeriesSignalConvertor v32TrellisActive (23)}

v32TrellisSelect XE "v32TrellisSelect "§::= BOOLEAN

Description:	Enables Trellis Coding in a V.32 DCE.

Operations:	GET-REPLACE

Behaviour:	In case the value of this attribute is TRUE, this does not prevent the DCE from handshaking to uncoded modulation.

Applications:	Configuration Management

Registered as:	{vSeriesSignalConvertor v32TrellisSelect (24)}

v34FeaturesActive XE "v34FeaturesActive "§::= BIT STRING {

adaptivePreemphasis		(0),
		auxiliaryChannel			(1),
		constellationShaping		(2),
		nonlinearEncoding		(3),
		powerControl			(4),

precoding			(5),
		secondaryChannel		(6),
		trellisEncoding-16state	(7),
		trellisEncoding-32state	(8),
		trellisEncoding-64state	(9)
		}

Description:	Indicates the V.34 features that are currently active.

Operations:	GET

Behaviour:	Specification not required

Applications:	Configuration Management

Registered as:	{vSeriesSignalConvertor v34FeaturesActive (25)}

v34FeaturesSelect XE "v34FeaturesSelect "§::= BIT STRING {

adaptivePreemphasis		(0),
		auxiliaryChannel			(1),
		constellationShaping		(2),
		nonlinearEncoding		(3),
		powerControl			(4),
		precoding			(5),
		secondaryChannel		(6),
		trellisEncoding-16state	(7),
		trellisEncoding-32state	(8),
		trellisEncoding-64state	(9)
		}

Description:	Enables one or more V.34 features.

Operations:	GET-REPLACE

Behaviour:	Specification not required

Applications:	Configuration Management

Registered as:	{vSeriesSignalConvertor v34FeaturesSelect (26)}

v34FeaturesSupported XE "v34FeaturesSupported "§::= BIT STRING {

adaptivePreemphasis		(0),
		auxiliaryChannel			(1),
		constellationShaping		(2),
		nonlinearEncoding		(3),
		powerControl			(4),
		precoding			(5),
		secondaryChannel		(6),
		trellisEncoding-16state	(7),
		trellisEncoding-32state	(8),
		trellisEncoding-64state	(9)
		}

Description:	Features implemented in the DCE’s V.34 receiver.

Operations:	GET

Behaviour:	Specification not required

Applications:	Configuration Management

Registered as:	{vSeriesSignalConvertor v34FeaturesSupported (27)}

v34PreemphasisFilterActive XE "v34PreemphasisFilterActive "§::= INTEGER (0..10)

Description:	Indicates the current transmit preemphasis filter number.

Operations:	GET

Behaviour:	Specification not required

Applications:	Configuration Management

Registered as:	{vSeriesSignalConvertor v34PreemphasisFilterActive (28)}

v34SymbolRateCarrierFrequenciesActive XE "v34SymbolRateCarrierFrequenciesActive "§::= SEQUENCE {

transmitSymbolRateCarrierFrequency	ENUMERATED
			{
			symbol2400-Carrier1600	(0),
			symbol2400-Carrier1800	(1),
			symbol2743-Carrier1646	(2),
			symbol2743-Carrier1829	(3),
			symbol2800-Carrier1680	(4),
			symbol2800-Carrier1867	(5),
			symbol3000-Carrier1800	(6),
			symbol3000-Carrier2000	(7),

symbol3200-Carrier1829	(8),
			symbol3200-Carrier1920	(9),
			symbol3429-Carrier1959	(10)
			},
		receiveSymbolRateCarrierFrequency	ENUMERATED
			{
			symbol2400-Carrier1600	(0),
			symbol2400-Carrier1800	(1),
			symbol2743-Carrier1646	(2),
			symbol2743-Carrier1829	(3),
			symbol2800-Carrier1680	(4),
			symbol2800-Carrier1867	(5),
			symbol3000-Carrier1800	(6),
			symbol3000-Carrier2000	(7),
			symbol3200-Carrier1829	(8),
			symbol3200-Carrier1920	(9),
			symbol3429-Carrier1959	(10)
			}
			}

Description:	Indicates the current symbol rate and carrier frequency for both the transmitter and receiver.

Operations:	GET

Behaviour:	Specification not required

Applications:	Configuration Management

Registered as:	{vSeriesSignalConvertor v34SymbolRateCarrierFrequenciesActive (29)}

v34SymbolRateCarrierFrequenciesSelect XE "v34SymbolRateCarrierFrequenciesSelect "§::= SEQUENCE {

symbolRateCarrierFrequency	BIT STRING {
			symbol2400-Carrier1600	(0),
			symbol2400-Carrier1800	(1),
			symbol2743-Carrier1646	(2),
			symbol2743-Carrier1829	(3),
			symbol2800-Carrier1680	(4),
			symbol2800-Carrier1867	(5),
			symbol3000-Carrier1800	(6),
			symbol3000-Carrier2000	(7),
			symbol3200-Carrier1829	(8),
			symbol3200-Carrier1920	(9),
			symbol3429-Carrier1959	(10)
			},
		symbolRateDifference	INTEGER (0..5)
		}

Description:	Enables one or more symbol rate carrier frequency combinations in the DCE, as well as selecting the maximum symbol rate difference allowed.

Operations:	GET

Behaviour:	Some combinations of symbol rate and carrier frequency may be disabled due to regulatory spectrum requirements.

Applications:	Configuration Management

Registered as:	{vSeriesSignalConvertor v34SymbolRateCarrierFrequenciesSelect (30)}

v34SymbolRateCarrierFrequenciesSupported XE "v34SymbolRateCarrierFrequenciesSupported "§::= SEQUENCE {

symbolRateCarrierFrequency	BIT STRING {
				symbol2400-Carrier1600		(0),
				symbol2400-Carrier1800		(1),
				symbol2743-Carrier1646		(2),
				symbol2743-Carrier1829		(3),
				symbol2800-Carrier1680		(4),
				symbol2800-Carrier1867		(5),
				symbol3000-Carrier1800		(6),
				symbol3000-Carrier2000		(7),
				symbol3200-Carrier1829		(8),
				symbol3200-Carrier1920		(9),
				symbol3429-Carrier1959		(10)
				},
			symbolRateDifference	INTEGER (0..5)
		}

Description:	Symbol rates and carrier frequencies implemented in the DCE.

Operations:	GET

Behaviour:	V.34 mandates some symbol rate carrier frequency combinations, and therefore these should always be indicated as supported. A value of 0 for symbolRateDifference means that asymmetric symbol rate operation is not supported.

Applications:	Configuration Management

Registered as:	{vSeriesSignalConvertor v34SymbolRateCarrierFrequenciesSupported (31)}

### 3.7.3	Actions

Name:	fallBackRequestXE "fallBackRequest"§

fallBackRequestReq ::=  ENUMERATED {
		localTransmitter	(0),
		remoteTransmitter	(1)
			}

fallBackRequestConf ::= BOOLEAN

Description:	Forces the DCE to attempt to fall back to the next lower signalling rate enabled in transmissionSignallingRatesSelect and supported by gstnModulation-SchemesSupported or leasedModulation-SchemesSupported.

Operations:	ACTION

Behaviour:	It is advisable to issue this action only if fallBackSignal-QualityThreshold and fallForward-SignalQualityThreshold are disabled. For some modulation schemes this action will initiate a fallback in both directions of transmission. The confirm argument indicates if a reduction in signalling rate occurred.

Applications:	Performance Management

Registered as:	{vSeriesSignalConvertor fallBackRequest (32)}

Name:	fallForwardRequestXE "fallForwardRequest"§

fallForwardRequestReq ::=  ENUMERATED {

localTransmitter	(0),
		remoteTransmitter	(1)
			}

fallForwardRequestConf ::= BOOLEAN

Description:	Forces the DCE to attempt to fall forward to the next higher signalling rate enabled in transmissionSignallingRatesSelect and supported by gstnModulation-SchemesSupported or leasedModulation-SchemesSupported.

Operations:	ACTION

Behaviour:	It is advisable to issue this action only if fallBackSignal-QualityThreshold and fallForwardSignal-QualityThreshold are disabled. For some modulation schemes this action will initiate a fall forward in both directions of transmission. The confirm argument indicates if an increase in signalling rate occurred.

Applications:	Performance Management

Registered as:	{vSeriesSignalConvertor fallForwardRequest (33)}

Name:	retrainXE "retrain"§

retrainReq  ::= NULL

retrainConf ::= NULL

Description:	Forces the DCE to initiate a retrain.

Operations:	ACTION

Behaviour:	Does not trigger a fallBackNotification or fallForwardNotification.

Applications:	Performance Management

Registered as:	{vSeriesSignalConvertor retrain (34)}

### 3.7.4	Notifications

fallBackNotification  XE "fallBackNotification  "§::= SEQUENCE {

priority	EventPriority,
		direction 	ENUMERATED {
				transmit		(0),
				receive		(1),
				both			(2)
				},
		initiator		ENUMERATED {
				local		(0),
				remote	(1)
				}
		}

Description:	This notification is issued to indicate to the management entity that the DCE has fallen back to a lower signalling rate.

Operations:	NOTIFICATION

Behaviour:	Only issued after automatic fall back or fall back initiated from the remote DCE and only if the signalling rate is actually changed.

Applications:	Fault Management

Registered as:	{vSeriesSignalConvertor fallBackNotification (35)}

fallForwardNotification XE "fallForwardNotification "§::= SEQUENCE {

priority		EventPriority,
		direction 	ENUMERATED {
				transmit		(0),
				receive		(1),
				both		(2)
				},
		initiator		ENUMERATED {
				local	(0),
				remote	(1)
				}
		}

Description:	This notification is issued to indicate to the management entity that the DCE has fallen forward to a higher signalling rate.

Operations:	NOTIFICATION

Behaviour:	Only issued after automatic fall forward or fall forward initiated from the remote DCE and only if the signalling rate is actually changed.

Applications:	Fault Management

Registered as:	{vSeriesSignalConvertor fallForwardNotification (36)}

goodSignalQuality XE "goodSignalQuality "§::= EventPriority

Description:	This notification is issued if the signal quality changes to good.

Operations:	NOTIFICATION

Behaviour:	Not issued if automatic fall forward results

Applications:	Fault Management

Registered as:	{vSeriesSignalConvertor goodSignalQuality (37)}

lossOfCarrier XE "lossOfCarrier "§::= EventPriority

Description:	This notification indicates that an unexpected loss of carrier has been detected by the DCE.

Operations:	NOTIFICATION

Behaviour:	Not issued if call cleared

Applications:	Fault Management

Registered as:	{vSeriesSignalConvertor lossOfCarrier (38)}

poorSignalQuality XE "poorSignalQuality "§::= EventPriority

Description:	This notification is issued if the signal quality changes to poor.

Operations:	NOTIFICATION

Behaviour:	Not issued if automatic fall back results

Applications:	Fault Management

Registered as:	{vSeriesSignalConvertor poorSignalQuality (39)}

## 3.8	V-Series Test Function Managed Object

### 3.8.1	Managed Object Template

vSeriesTestFunction MANAGED OBJECT CLASS

-- Source ITU T SG14 Q4

-- Status final draft 6 June 1994

DERIVED FROM "Recommendation M.3100: 1992":managedElement

CHARACTERIZED BY

testFunctionPkg	PACKAGE
	ATTRIBUTES
	cct140Enable		GET-REPLACE,
	cct141Enable		GET-REPLACE,
	erroredBitsReceived		GET,
	erroredBlocksReceived		GET,
	loop2Local		GET,
	loop2LocalEnable		GET-REPLACE,
	loop2Remote		GET,
	loop2RemoteEnable		GET-REPLACE,
	loop3		GET,
	loop3Enable		GET-REPLACE,
	v54Address		GET REPLACE,
	v54Mode		GET REPLACE;
	ACTIONS
	invokeErrorRateTest,
	invokeLoop2Local,
	invokeLoop2Remote,
	invokeLoop3,
	stopErrorRateTest;
	NOTIFICATIONS
	loop2InvokedByRemoteDce;

REGISTERED AS

{ccitt(0), recommendation(0), v(22), v58(58), vSeriesTestFunction(7)};

NOTE – The Test Object only relates to single port DCEs, its use with multiple port DCEs is for further study.

### 3.8.2	Attributes

cct140Enable XE "cct140Enable "§::= BOOLEAN

Description:	If FALSE, signals on circuit 140 are ignored.

Operations:	GET-REPLACE

Behaviour:	Specification not required

Applications:	Fault Management

Registered as:	{vSeriesTestFunction cct140Enable (0)}

cct141Enable XE "cct141Enable "§::= BOOLEAN

Description:	If FALSE, signals on circuit 141 are ignored.

Operations:	GET-REPLACE

Behaviour:	Specification not required

Applications:	Fault Management

Registered as:	{vSeriesTestFunction cct141Enable (1)}

erroredBitsReceived XE "erroredBitsReceived "§::= INTEGER (0..65535)

Description:	Contains the number of errored bits received during the last or current bit error rate test.

Operations:	GET

Behaviour:	Reset to zero by InvokeErrorRateTest. Does not reset to zero if maximum count reached.

Applications:	Fault Management

Registered as:	{vSeriesTestFunction erroredBitsReceived (2)}

erroredBlocksReceived XE "erroredBlocksReceived "§::= INTEGER (0..65535)

Description:	Contains number of error blocks received during the last or current block error rates test.

Operations:	GET

Behaviour:	Reset to zero by InvokeErrorRateTest. Does not reset to zero if maximum count reached.

Applications:	Fault Management

Registered as:	{vSeriesTestFunction erroredBlocksReceived (3)}

loop2Local XE "loop2Local "§::= ENUMERATED {

disabled				(0),
		enabledInactive				(1),
		frontPanelInvoked			(2),
		networkManagementSystemInvoked	(3),
		remoteInvoked				(4)
		}

Description:	Contains the status of the digital loop (V.54 loop 2) in the addressed DCE.

Operations:	GET

Behaviour:	Specification not required

Applications:	Fault Management

Registered as:	{vSeriesTestFunction loop2Local (4)}

loop2LocalEnable XE "loop2LocalEnable "§::= BOOLEAN

Description:	If TRUE, the digital loop (V.54 loop 2) of the addressed DCE can be controlled via V.54 remote control and/or the front panel.

Operations:	GET-REPLACE

Behaviour:	Specification not required

Applications:	Fault Management

Registered as:	{vSeriesTestFunction loop2LocalEnable (5)}

loop2Remote XE "loop2Remote "§::= ENUMERATED {

disabled				(0),
		enabledInactive				(1),
		cct140Invoked				(2),
		frontPanelInvoked			(3),
		networkManagementSystemInvoked	(4)
		}

Description:	Contains the status of the digital loop (V.54 loop 2) in the remote DCE.

Operations:	GET

Behaviour:	Specification not required

Applications:	Fault Management

Registered as:	{vSeriesTestFunction loop2Remote (6)}

loop2RemoteEnable XE "loop2RemoteEnable "§::= BOOLEAN

Description:	If FALSE, manual control of loop 2 in the remote DCE from the front panel of the local DCE is disabled.

Operations:	GET-REPLACE

Behaviour:	Specification not required

Applications:	Fault Management

Registered as:	{vSeriesTestFunction loop2RemoteEnable (7)}

loop3 XE "loop3 "§::= ENUMERATED {

inactive				(0),
		cct141Invoked				(1),
		frontPanelInvoked			(2),
		networkManagementSystemInvoked	(3)
		}

Description:	Describes the status of the local analog loop (V.54 loop3).

Operations:	GET

Behaviour:	Specification not required

Applications:	Fault Management

Registered as:	{vSeriesTestFunction loop3 (8)}

loop3Enable XE "loop3Enable "§::= BOOLEAN

Description:	If TRUE, the local analog loop (V.54 loop 3) can be controlled via the front panel.

Operations:	GET-REPLACE

Behaviour:	Specification not required

Applications:	Configuration Management

Registered as:	{vSeriesTestFunction loop3Enable (9)}

v54Address XE "v54Address "§::= CHOICE {

shortAddress 	[0] 	INTEGER (0..255),
		longAddress 	[1] 	INTEGER (0..65535)
		}

Description:	Sets the V.54 address of the addressed DCE.

Operations:	GET-REPLACE

Behaviour:	Operates both for loop 2 and tandem loop 3.

Applications:	Fault Management

Registered as:	{vSeriesTestFunction v54Address (10)}

v54Mode XE "v54Mode "§::=ENUMERATED {

pointToPoint			(0),
		multipointOrTandem		(1)
		}

Description:	Choose between V.54 modes.

Operations:	GET-REPLACE

Behaviour:	Specification not required

Applications:	Fault Management

Registered as:	{vSeriesTestFunction v54Mode (11)}

### 3.8.3	Actions

Name:	invokeErrorRateTestXE "invokeErrorRateTest"§

invokeErrorRateTestReq ::= SEQUENCE {

testType 	ENUMERATED {
				bitErrorRateTest		(0),
				blockErrorRateTest		(1),
				both		(2)
				},
		blockLength		INTEGER 	(1..65535),
		numberOfBlocks		INTEGER 	(1..65535),
		testPattern	ENUMERATED {
				test63		(0),
				test511		(1),
				test2047		(2),
				binaryOnes		(3),
				alternatingOnesZeroes	(4)
				}
		}

invokeErrorRateTestConf ::= ENUMERATED {

testInitiated	(0),
		testRestarted	(1),
		testNotSupported	(2),
		noLoopActive	(3)
		}

Description:	Invokes bit and block error rate tests. Test includes generation of the test pattern, reception of test pattern, error counting.

Operations:	ACTION

Behaviour:	A suitable loop shall be set up first.

Applications:	Fault Management

Registered as:	{vSeriesTestFunction invokeErrorRateTest (12)}

Name:	invokeLoop2LocalXE "invokeLoop2Local"§

invokeLoop2LocalReq ::= ENUMERATED {

invoke	(0),
		revoke	(1)
		}

invokeLoop2LocalConf ::= ENUMERATED {

loopNowActive	(0),
		loopNowInactice	(1)
		}

Description:	Invokes/revokes the digital loop (V.54 loop 2) in the addressed DCE.

Operations:	ACTION

Behaviour:	Specification not required

Applications:	Fault Management

Registered as:	{vSeriesTestFunction invokeLoop2Local (13)}

Name:	invokeLoop2RemoteXE "invokeLoop2Remote"§

invokeLoop2RemoteReq ::= SEQUENCE {

invokeRevoke	ENUMERATED {
				invoke	(0),
				revoke	(1)
				},
		optAddress	ENUMERATED {
				v54Address	(0),
				noAddress	(1)
				}
		}

invokeLoop2RemoteConf ::= ENUMERATED {

confirmationReceived	(0),
		noConfirmationReceived	(1)
		}

Description:	Invokes/revokes the digital loop (V.54 loop 2) in the remote DCE by means of V.54 control from the local DCE.

Operations:	ACTION

Behaviour:	optAddress is v54Address if invokeRevoke is Invoke and v54Mode is multipointOrTandem.

Applications:	Fault Management

Registered as:	{vSeriesTestFunction invokeLoop2Remote (14)}

Name:	invokeLoop3XE "invokeLoop3"§

invokeLoop3Req ::= ENUMERATED {

invoke	(0),
		revoke	(1)
		}

invokeLoop3Conf ::= ENUMERATED {

loopNowActive		(0),
		loopNowInactice		(1)
		}

Description:	Invokes/revokes the local analog loop (V.54 loop 3).

Operations:	ACTION

Behaviour:	Specification not required

Applications:	Fault Management

Registered as:	{vSeriesTestFunction invokeLoop3 (15)}

Name:	stopErrorRateTestXE "stopErrorRateTest"§

stopErrorRateTestReq  ::= NULL

stopErrorRateTestConf ::= SEQUENCE {

numberOfBlocksSent	INTEGER (1..65535),
		erroredBlocksReceived	INTEGER,
		erroredBitsReceived		INTEGER
		}

Description:	Aborts bit or block error rate test.

Operations:	ACTION

Behaviour:	Test results are only defined if the relevant test has been conducted using invokeErrorRateTest.

Applications:	Fault Management

Registered as:	{vSeriesTestFunction stopErrorRateTest (16)}

### 3.8.4	Notifications

loop2InvokedByRemoteDce XE "loop2InvokedByRemoteDce "§::= EventPriority

Description:	This event is used to indicate to the management entity that the DCE has entered a loop 2 condition invoked by the remote DCE.

Operations:	NOTIFICATION

Behaviour:	If loop2LocalEnable is FALSE this notification will not be generated.

Applications:	Fault Management

Registered as:	{vSeriesTestFunction loop2InvokedByRemoteDceloop2 (17)}

## 3.9	Local defined types

Description:	CallProgress identifies the states that the DCE line interface may have.

CallProgressXE "CallProgress"§	::= ENUMERATED {

onHook	(0),
	waitingForDialTone	(1),
	dialling	(2),
	waitingForRinging	(3),
	ringing	(4),
	answerTone	(5),
	connected	(6),
	failed	(7)
	}

Description:	DceBitrate is used to select or indicate a specific bitrate for the DCE’s GSTN interface.

DceBitrateXE "DceBitrate"§	::= ENUMERATED {

br75	(0),
	br110	(1),
	br150	(2),
	br300	(3),
	br600	(4),
	br1200	(5),
	br2400	(6),
	br4800	(7),
	br7200	(8),
	br9600	(9),
	br12000	(10),
	br14400	(11),
	br16800	(12),
	br19200	(13),
	br21600	(14),
	br24000	(15),
	br26400	(16),
	br28800	(17),
	br31200	(18),
	br32000	(19),
	br33600	(20),
	br36000	(21),
	br38400	(22),
	br48000	(23),
	br56000	(24),
	br57600	(25),
	br64000	(26)
	}

Description:	DceBitrateRange is used by the DCE to indicate the range of bit rates that it can support over the GSTN interface.

DceBitrateRangeXE "DceBitrateRange"§	::= BIT STRING {

br75	(0),
	br110	(1),
	br150	(2),

br300	(3),
	br600	(4),
	br1200	(5),
	br2400	(6),
	br4800	(7),
	br7200	(8),
	br9600	(9),
	br12000	(10),
	br14400	(11),
	br16800	(12),
	br19200	(13),
	br21600	(14),
	br24000	(15),
	br26400	(16),
	br28800	(17),
	br31200	(18),
	br32000	(19),
	br33600	(20),
	br36000	(21),
	br38400	(22),
	br48000	(23),
	br56000	(24),
	br57600	(25),
	br64000	(26)
	}

Description:	DteBitrate gives the DTE/DCE interface speed in increments of 5 bit/s.

DteBitrateXE "DteBitrate"§	::= INTEGER (1..65535)

Description:	EventPriority is used to assign a priority to notifications to support subsequent processing.

EventPriority XE "EventPriority "§::= ENUMERATED {

noEvents		(0),
		majorFault		(1),
		allFault		(2),
		faultAndProgress		(3),
		allEvents		(4)
		}

Description:	PhoneNumber is used to represent a telephone number and associated characters, as defined in draft Recommendation V.at (V.25 ter).

PhoneNumber  XE "PhoneNumber  "§::= IA5String

# 4	Relationships between managed object classes

Figure 1 depicts the relations between the managed objects classes specified in this Recommendation.

FIGURE 1/V.58...[D01] = 9.5 CM  (118%)

Appendix I

Some additional background information

This Recommendation defines  a set of “building block” objects which  permit a range of V-Series DCEs to be defined. These “building block” objects are:

·	V-Series DCE;

·	V-Series Line Interface;

·	V-Series DTE Interface;

·	V-Series Signal Converter;

·	V-Series Call Control;

·	V-Series Error Control;

·	V-Series Data Compression;

·	V-Series Test Function.

These require the use of the objects Equipment and managed Element, which are defined in Recommendation M.3100.

For each Managed Object the Attributes, Actions and Notifications that define its management behaviour are specified.

An Attribute is a parameter, which may be a single value or a set of values. It may be read-only, write-only, or read-write. It may be mandatory or optional.

An Action is a complex activity that the Managed Object is requested to perform, i.e. one that requires more than the change of a single attribute.

A Notification is an unsolicited message from the Managed Object, for example an alarm indication. Notifications may be filtered within a DCE or some intermediate system, hence management systems would not (necessarily) be flooded with “Ring Indication” events.

Objects are related using the containment tree principle. It is the intent of the methodology set out in this Recommendation that most practical V-Series DCE (at least modem) configurations should be representable. The object definitions defined in this Recommendation do not support multiplexing.

Appendix II

List of Attributes, Actions and Notifications

Attribute / Action / Notification

µanswerToneSelect, 30

answerToneSupported, 31

antiStreamingTimer, 16

audioMonitor, 3

autoAnswerEnable, 3

autoCallEnable, 3

autoCallModeSelect, 3

autoCallModesSupported, 4

autoDetectCharacterFormat, 17

autoRestoralEnable, 4

availableNumberCapacity, 4

backedUpStatus, 11

backUpObjectInstance, 11

blackListingActive, 4

bufferedDataDeliveryTimeoutSelect, 17

bufferedDataDeliveryTimeoutSupported, 17

busyDetection, 31

callCleared, 34

callEstablished, 35

callingToneSelect, 31

callingToneSupported, 31

CallProgress, 50

callProgressEvents, 7

callProgressState, 4

callSetupFailTimer, 31

cct105Mode, 17

cct105to106Delay, 18

cct106Mode, 18

cct107Mode, 18

cct108Mode, 18

cct109Mode, 18

cct109TurnOffDelay, 19

cct109TurnOnDelay, 19

cct116Mode, 19

cct133ToXonXoffTranslation, 19

cct140Enable, 46

cct141Enable, 46

characterFormatSelect, 19

characterFormatSupported, 20

compressionActive, 8

compressionEfficiency, 8

compressionSelect, 8

compressionSupported, 8

countryOfInstallationSelect, 11

countryOfInstallationSupported, 11

DceBitrate, 50

DceBitrateRange, 50

dceModeActive, 4

dceModeSelect, 5

dceModesSupported, 5

defaultCallAttemptsTimer, 5

dialBackupEnable, 5

dialConnect, 6

dialDisconnect, 6

dialSignalling, 31

dialToneDetection, 32

directCallNumber, 5

disconnectConfiguration, 11

displayCallProgressMessages, 5

dteAttached, 20

DteBitrate, 51

dteInterfaceStatus, 20

dteModeActive, 21

dteModesSupported, 21

dtmfToneDuration, 32

echoControlMessages, 21

echoUserData, 21

equalizationSelect, 36

equalizationSupported, 36

equipmentFailure, 15

equipmentType, 12

errorControlActive, 25

errorControlSelect, 25

erroredBitsReceived, 46

erroredBlocksReceived, 47

EventPriority, 51

eventThreshold, 12

fallBackNotification, 45

fallBackRequest, 44

fallBackSignalQualityThreshold, 37

fallForwardNotification, 45

fallForwardRequest, 44

fallForwardSignalQualityThreshold, 37

flowControlSelect, 21

flowControlSupported, 22

frontPanelAccessEnable, 12

goodSignalQuality, 45

gstnCallMode, 37

gstnModulationSchemeActive, 37

gstnModulationSchemesSelect, 38

gstnModulationSchemesSupported, 38

inactivityTimerSelect, 22

inactivityTimerSupported, 22

invokeConfiguration, 13

invokeErrorRateTest, 48

invokeLoop2Local, 48

invokeLoop2Remote, 49

invokeLoop3, 49

leasedCallMode, 38

leasedModulationSchemeSelect, 38

leasedModulationSchemesSupported, 39

lineSignalFailDisconnectTimer, 32

lineTypeActive, 32

lineTypeSelect, 32

lineTypeSupported, 33

linkState, 26

loadConfiguration, 13

longSpaceDisconnectSelect, 22

longSpaceDisconnectSupported, 23

loop2InvokedByRemoteDce, 50

loop2Local, 47

loop2LocalEnable, 47

loop2Remote, 47

loop2RemoteEnable, 47

loop3, 47

loop3Enable, 48

lossOfCarrier, 45

lossOfSynchronization, 10

manufacturerID, 12

maximumFrameLengthActive, 26

maximumFrameLengthSelect, 26

maximumFrameLengthSupported, 26

maxRetries, 26

parityBits, 23

pauseDuringDialTime, 6

PhoneNumber, 51

poorSignalQuality, 45

powerOn, 15

powerOnConfiguration, 12

powerOnFailure, 15

presetConfigurationRange, 13

pulseDialModeSelect, 33

pulseDialModeSupported, 33

rateRenegotiationInitiation, 39

receiveLevel, 39

redialSignalQualityThreshold, 40

resetNotification, 15

responseModeSelect, 23

responseModeSupported, 23

retrain, 44

retrainInitiateCounter, 40

retrainRequestsCounter, 40

retrainSignalQualityThreshold, 40

revertedToLeasedLine, 7

ringIndication, 35

ringsBeforeAnswer, 6

selfTest, 14

signalQualityEstimate, 40

startStopDteInterfaceSpeed, 23

startStopDteInterfaceSpeedAdaptation, 24

stopErrorRateTest, 49

storeConfiguration, 14

streamingDetected, 24

switchedToDialBackup, 7

telephoneNumbers, 6

testFrameOptionActive, 26

testFrameOptionSelect, 26

transmissionSignallingRateActive, 40

transmissionSignallingRatesSelect, 41

transmissionSignallingRatesSupported, 41

transmitClockSource, 41

transmitLevelActive, 33

transmitLevelAdjustable, 34

transmitLevelSelect, 34

userConfigurationRange, 13

v13ModeSelect, 24

v14SignallingRate, 24

v22V22bisGuardToneEnable, 41

v25bisMode, 6

v32TrellisActive, 41

v32TrellisSelect, 41

v34FeaturesActive, 41

v34FeaturesSelect, 42

v34FeaturesSupported, 42

v34PreemphasisFilterActive, 42

v34SymbolRateCarrierFrequenciesActive, 42

v34SymbolRateCarrierFrequenciesSelect, 43

v34SymbolRateCarrierFrequenciesSupported, 43

v42bisCompressionActive, 9

v42bisCompressionSelect, 9

v42bisDictionarySizeActive, 9

v42bisDictionarySizeSelect, 9

v42bisDictionarySizeSupported, 9

v42bisMaximumStringLengthActive, 9

v42bisMaximumStringLengthSelect, 9

v42bisMaximumStringLengthSupported, 10

v42BreakOptions, 27

v42CrcActive, 27

v42CrcSelect, 27

v42CrcSupported, 27

v42DetectionPhaseEnable, 27

v42FallbackSelect, 27

v42FallbackSupported, 28

v42RejectOptionsActive, 28

v42RejectOptionsSelect, 28

v42RejectOptionsSupported, 28

v42Statistics, 28

v42TimedBreakSupported, 29

v42UntimedBreakDuration, 29

v54Address, 48

v54Mode, 48

viewConfiguration, 14

windowSizeActive, 29

windowSizeSelect, 29

windowSizeSupported, 29

Appendix III

References

## References

[1]	CCITT Recommendation M.3100 (1992), Generic Network Information Model.

[2]	CCITT Recommendation M.3010 (1992), Principles for a Telecommunications Management Network.

[3]	CCITT Recommendation X.720 (1992), Information technology – Open Systems Interconnection – Structure of Management Information:  Management Information Model.

[4]	CCITT Recommendation X.721 (1992), Information technology – Open Systems Interconnection – Structure of Management Information:  Definition of Management Information.

[5]	CCITT Recommendation X.722 (1992), Information technology – Open Systems Interconnection – Structure of Management Information:  Guidelines for Definition of Managed Objects.

[6]	CCITT Recommendation X.208 (1989), Specification of Abstract Syntax Notation One (ASN.1).