|  | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

ITU-T	X.3

TELECOMMUNICATION	(03/93)
STANDARDIZATION  SECTOR
OF  ITU

PUBLIC  DATA  NETWORKS:
SERVICES  AND  FACILITIES

PACKET  ASSEMBLY/DISASSEMBLY
FACILITY  (PAD)
IN  A  PUBLIC  DATA  NETWORK

ITU-T  Recommendation  X.3

(Previously “CCITT  Recommendation”)

# FOREWORD

The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of the International Telecommunication Union. The ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Conference (WTSC), which meets every four years, established the topics for study by the ITU-T Study Groups which, in their turn, produce Recommendations on these topics.

ITU-T Recommendation X.3 was revised by the ITU-T Study Group VII (1988-1993) and was approved by the WTSC (Helsinki, March 1-12, 1993).

___________________

NOTES

1	As a consequence of a reform process within the International Telecommunication Union (ITU), the CCITT ceased to exist as of 28 February 1993. In its place, the ITU Telecommunication Standardization Sector (ITU-T) was created as of 1 March 1993. Similarly, in this reform process, the CCIR and the IFRB have been replaced by the Radiocommunication Sector.

In order not to delay publication of this Recommendation, no change has been made in the text to references containing the acronyms “CCITT, CCIR, or IFRB” or their associated entities such as Plenary Assembly, Secretariat, etc. Future editions of this Recommendation will contain the proper terminology related to the new ITU structure.

2	In this Recommendation, the expression “Administration” is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

ã  ITU  1993

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

Recommendation X.3

Recommendation X.3     (03-93)

PACKET  ASSEMBLY/DISASSEMBLY  FACILITY  (PAD)
IN  A  PUBLIC  DATA  NETWORKXE "PACKET  ASSEMBLY/DISASSEMBLY  FACILITY  (PAD)
IN  A  PUBLIC  DATA  NETWORK"§

(Provisional, Geneva, 1977; amended at Geneva, 1980,
Malaga-Torremolinos, 1984 , Melbourne 1988 and Helsinki, 1993)

# Preface

The establishment in various countries of public data networks providing packet switched data transmission services creates a need to produce standards to facilitate access from the public telephone network, circuit switched public data networks and leased circuits.

The CCITT,

considering

(a)	that Recommendations X.1 and X.2 define the user classes of service and user facilities in public data networks, Recommendation X.96 defines call progress signals, Recommendation X.29 defines the procedures between a packet assembly/disassembly facility (PAD) and a packet mode DTE or another PAD, Recommendation X.28 defines the DTE/DCE interface for a start-stop mode DTE accessing the PAD;

(b)	that the logical control links for packet switched data transmission services are defined in Recommendation X.92, and that in particular Recommendation X.92 allows for the incorporation of a PAD;

(c)	the urgent need to allow interworking between a start-stop mode DTE on a public switched telephone network, a public switched data network or leased circuit, and a packet mode DTE or another start-stop mode DTE using the virtual call facility of the packet switched data service;

(d)	that DTEs operating in the start-stop mode will send and receive network control information and user information in the form of characters or the break signals;

(e)	that DTEs operating in the packet mode will send and receive network control information and user information in the form of packets in accordance with Recommendation X.25;

(f)	that the packet mode DTE shall not be obliged to use the control procedures for PAD functions, but that some packet mode DTEs may wish to control specific functions of the PAD,

unanimously declares

1)	that the functions performed by, and operational characteristics of, the PAD for the start-stop mode DTE are described below in clause 1, Description of the basic functions and user selectable functions of the PAD;

2)	that the operation of the PAD for the start-stop mode DTE should depend on the possible values of internal variables known as PAD parameters which are described below in clause 2, Characteristics of PAD parameters;

3)	that the PAD parameters for the start-stop mode DTE and their possible values should be those which are listed below in clause 3, List of PAD parameters and possible values;

4)	that the PAD features described in clauses 1, 2 and 3 could be expanded by future studies to allow interworking with non packet mode DTEs other than start-stop mode DTEs.

# 1	Description of the basic functions and user selectable functions of the PADXE "Description of the basic functions and user selectable functions of the PAD"§

1.1	The PAD performs a number of functions and exhibits operational characteristics. Some of the functions allow either or both the start-stop mode DTE and the packet mode DTE (or remote PAD) to configure the PAD so that its operation is adapted to the start-stop mode DTE characteristics, and possible to the application.

1.2	The operation of the PAD depends on the value of the set of internal variables called PAD parameters. This set of parameters exists for each start-stop mode DTE independently. The current value of each PAD parameter defines the operational characteristics of its related function.

## 1.3	Functions of the PADXE "Functions of the PAD"§

1.3.1	Basic functions include:

–	assembly of characters into packets;

–	disassembly of the user data field of packets;

–	handling of virtual call set-up and clearing, resetting and interrupt procedures;

–	generation of service signals;

–	a mechanism for forwarding packets when the proper conditions exist, e.g., when a packet is full or an idle timer expires;

–	a mechanism for transmitting data characters, including start, stop and parity elements as appropriate to the start-stop mode DTE;

–	a mechanism for handling a break signal from the start-stop mode DTE;

–	editing of PAD command signals;

–	a mechanism for setting and reading the current value of PAD parameters.

1.3.2	Optional functions include:

–	a mechanism for the selection of a standard profile;

–	automatic detection of data rate, code, parity and operational characteristics;

–	a mechanism for the remote DTE to request a virtual call between the start-stop mode DTE and another DTE.

## 1.4	User selectable functions which may be provided by the PADXE "User selectable functions which may be provided by the PAD"§

A number of packet-switched data network facilities may be available either on a subscription basis or on a per call basis, to start-stop mode DTEs as described in Recommendation X.2 for user classes of service 20 to 22. In addition, the following features may be available on a subscription basis:

–	selection of an initial profile;

–	modem type, speed, code and parity to be used by the PAD;

–	other operational characteristics of the DTE.

As defined in this Recommendation, parameters provide for functions which concern:

–	management of the procedure between the start-stop mode DTE and the PAD;

–	management of the assembly and disassembly of packets;

–	a number of additional functions related to the operational characteristics of the start-stop mode DTE;

–	a number of additional functions related to videotex applications.

The method for the control of these functions is specified in Recommendation X.28 for the start-stop mode DTE and in Recommendation X.29 for the packet mode DTE or for another PAD.

Table 1 shows details of the valid values and combination of values of PAD parameters standardized by CCITT. Other values and combinations of values are for further study.

### 1.4.1	PAD recall using a characterXE "PAD recall using a character"§

This function allows the start-stop mode DTE to initiate an escape from the data transfer state or the connection in progress state in order to send PAD command signals.

### 1.4.2	EchoXE "Echo"§

This function provides for characters received from the start-stop mode DTE to be transmitted back to the start-stop mode DTE as well as being interpreted by the PAD.

### 1.4.3	Selection of the data forwarding charactersXE "Selection of the data forwarding characters"§

This function allows the selection of defined sets of character(s) received from the start-stop mode DTE to be recognized by the PAD as an indication to complete the assembly and forward a complete packet sequence as defined in Recommendation X.25.

### 1.4.4	Selection of idle timer delayXE "Selection of idle timer delay"§

This function allows the selection of the duration of an interval between successive characters received from the start-stop mode DTE which, when exceeded, will cause the PAD to terminate the assembly of a packet and to forward it as defined in Recommendation X.255.

### 1.4.5	Ancillary device controlXE "Ancillary device control"§

This function allows for flow control between the PAD and the start-stop mode DTE. The PAD indicates whether it is ready or not to accept characters from the start-stop mode DTE by transmitting special characters. These characters are those which in International Alphabet No. 5 (IA5) are used to switch an ancillary transmitting device on and off.

### 1.4.6	Control of PAD service signalsXE "Control of PAD service signals"§

This function provides the start-stop mode DTE with the ability to decide whether or not and in what format PAD service signals are transmitted.

### 1.4.7	Selection of operation of the PAD on receipt of the break signalXE "Selection of operation of the PAD on receipt of the break signal"§

This function allows the selection of the operation of the PAD after the receipt of a break signal from the start-stop mode DTE.

### 1.4.8	Discard outputXE "Discard output"§

This function provides for a PAD to discard the content of user sequences in packets upon request rather than disassembling and transmitting these to the start-stop mode DTE.

### 1.4.9	Padding after carriage returnXE "Padding after carriage return"§

This function provides for the automatic insertion by the PAD of padding characters in the character stream transmitted to the start-stop mode DTE after the occurrence of a carriage return character. This allows for the printing mechanism for the start-stop mode DTE to perform the carriage return function correctly.

### 1.4.10	Line foldingXE "Line folding"§

This function provides for the automatic insertion by the PAD of appropriate format effectors in the character stream transmitted to the start-stop mode DTE. The predetermined maximum number of graphic characters per line may be set.

### 1.4.11	Flow control of the PAD by the start-stop mode DTEXE "Flow control of the PAD by the start-stop mode DTE"§

This function allows for flow control between the start-stop mode DTE and the PAD. The start-stop mode DTE indicates whether it is ready or not to accept characters from the PAD by transmitting special characters. These characters are those which in IA5 are used to switch an ancillary transmitting device on and off.

### 1.4.12	Linefeed insertion after carriage returnXE "Linefeed insertion after carriage return"§

This function provides for the automatic insertion by the PAD of a linefeed character in the character stream to or from the start-stop mode DTE or after each of each carriage return character. This function applies only in the data transfer state.

### 1.4.13	Padding after linefeedXE "Padding after linefeed"§

This function provides for the automatic insertion by the PAD of padding characters in the character stream transmitted to the start-stop DTE after the occurrence of a linefeed character. This allows for the printing mechanism of the start-stop mode DTE to perform the linefeed operation correctly. This function applies only in the data transfer state.

### 1.4.14	EditingXE "Editing"§

This function provides for character delete, line delete and line display editing capabilities in the PAD command state and the data transfer state for the start-stop mode DTE. During the PAD command state the editing function is always available.

### 1.4.15	Editing PAD service signalsXE "Editing PAD service signals"§

This function provides the start-stop mode DTE with the ability to decide whether or not editing PAD service signals are transmitted and which format should be used.

### 1.4.16	Echo maskXE "Echo mask"§

When echo is enabled (see 1.4.2), this function allows selected defined sets of character(s) received from the start-stop mode DTE not to be transmitted back to the start-stop mode DTE.

### 1.4.17	Parity treatmentXE "Parity treatment"§

This function allows the PAD to check parity in the data stream from the start-stop mode DTE and/or generate parity in the data stream to the start-stop mode DTE.

### 1.4.18	Page waitXE "Page wait"§

This function allows the PAD to suspend transmission of additional characters to the start-stop mode DTE after a specified number of linefeed characters have been transmitted by the PAD.

### 1.4.19	Videotex selectable functionsXE "Videotex selectable functions"§

The following functions have been defined to meet the requirements of Videotex applications.

#### 1.4.19.1  Size of input field

This function allows the definition of the maximum length of an input field which may be smaller or greater than the data packet size. When this input field has been filled up by characters received from the start-stop mode DTE, the PAD should complete the assembly and forward a complete packet sequence as defined in Recommendation X.25.

If this function is not supported or if the parameter reference 23 value is set to 0, only the data forwarding signals defined by parameter references 3 and 4 (and 25 when supported) have to be used.

When this function is supported and when the parameter reference 23 value is not 0, the data forwarding signals defined by parameter references 3 and 4 (and 25 when supported) have to be used.

The size of the input field is defined in graphic characters (see Note in 3.23).

NOTE – This function will be handled by using the editing buffer with the specific rules described in 3.6.1.3/X.28.

#### 1.4.19.2.  End-of-frame signals

This function indicates to the PAD when it can start the echoplex procedure. Upon receipt of the end-of-frame signal, the PAD echoes characters received from the start-stop mode DTE (see 4.19.3/X.28).

When this function is not supported or when the parameter reference 24 value is set to 0, the echoplex procedures should be in accordance with the interleaving rules in 4.19/X.28, 4.19.1/X.28 and 4.19.2/X.28.

#### 1.4.19.3  Selection of extended data forwarding signals

This function allows the selection of defined sets of character(s) or sequences of characters received from the start-stop mode DTE to be recognized by the PAD as an indication to complete the assembly and forward a complete packet sequence as defined in Recommendation X.25.

When this function is not supported or when the parameter reference 25 value is set to 0, data forwarding signals have to be in accordance with 4.4/X.28.

### 1.4.19.4  Display interrupt

This function allows the selection of a character received from the start-stop mode DTE indicating to the PAD to discard the contents of the user sequences received from the X.25 DTE rather than disassembling and transmitting these to the start-stop mode DTE. In addition, characters previously received from the start-stop mode DTE which have not already been sent to the X.25 DTE are also discarded.

### 1.4.19.5  Confirmation of display interrupt

This function allows the selection of the character to be sent by the PAD to the start-stop mode DTE after having performed the display interrupt as described by parameter reference 26.

### 1.4.19.6  Diacritic character editingXE "Diacritic character editing"§

This function allows the X.25 DTE to indicate to the PAD the way of in which diacritic characters are handled, especially when Character delete, Line delete or Size of input field functions are to be performed. This function allows the PAD to know the type of coding used for diacritic characters in order to correctly perform editing functions.

When this function is not supported or if the parameter reference 28 value is set to 0, the basic character set is assumed.

When this function is supported and when the parameter reference 28 value is not 0, the parameter value indicates the diacritic character coding in use.

#### 1.4.19.7  Extended echo maskXE "Extended echo mask"§

When echo is enabled (i.e. parameter 2 is not equal to 0), this function allows a selected defined set of character(s) received from the start-stop mode DTE either not to be transmitted back to the start-stop DTE or to be transmitted back to the start-stop mode DTE.

# 2	Characteristics of PAD parametersXE "Characteristics of PAD parameters"§

2.1	In this Recommendation parameters are identified by decimal reference numbers.

2.2	In this Recommendation the possible values of the parameters are represented by decimal numbers.

2.3	Specific procedures, described in Recommendations X.28 and X.29 are available for initializing, reading and changing values of PAD parameters.

## 2.4	Determination of the values of PAD parametersXE "Determination of the values of PAD parameters"§

### 2.4.1	Initial values of PAD parameters

On initialization, the initial value of each PAD parameter is set according to a predetermined set of values called an initial standard profile. Table 1/X.28 gives details of the initial values of parameters for transparent and simple standard profiles which have been agreed by CCITT.

Networks may offer other standard profiles that provide different, predetermined sets of PAD parameter values.

### 2.4.2	Current values of PAD parameters

The current values of PAD parameters are the values resulting from possible modifications by the PAD, the start-stop mode DTE and/or the packet mode DTE (or remote PAD).

# 3	List of PAD parameters and possible valuesXE "List of PAD parameters and possible values"§

Restrictions on the permissible relationships between the values of the various parameters is a subject for further study.

## 3.1	PAD recall using a characterXE "PAD recall using a character"§

Reference 1

The parameter will have the following selectable values:

not possible		–  represented by decimal 0;

possible by character 1/0 (DLE)	–  represented by decimal 1;

possible by graphic character defined by the user	–  represented by decimal 32 to 126.

A graphic character, defined by the user to escape from the data transfer state and to recall the PAD, is the binary representation of the decimal value in accordance with Recommendation T.50.

## 3.2 	EchoXE "Echo"§

Reference 2

The parameter will have the following selectable values:

no echo			–  represented by decimal 0;

echo (see Note 1)	–  represented by decimal 1;

echo all characters except the data
forwarding sequence defined by
parameter 25 (see Note 3)	–  represented by decimal 2;

scrambled echo character (see Note 2)	–  represented by decimal 32 to 126.

NOTES

1	If parameter 20 is implemented, the selection of the characters to be echoed is dependent on the value of parameter 20.

2	The decimal value represents the character to be transmitted back to the start-stop mode DTE.

3	If parameter 29 is implemented, the selection of the characters to be echoed is dependent on the value of parameter 29.

## 3.3	Selection of data forwarding character(s)XE "Selection of data forwarding character(s)"§

Reference 3

This parameter is represented by the following encoding of basic functions, each having a decimal value as shown below:

no data forwarding character	–  represented by decimal 0;

alphanumeric characters (A to Z, a to z, 0 to 9)	–  represented by decimal 1;

character CR		–  represented by decimal 2;

characters ESC, BEL, ENQ, ACK	–  represented by decimal 4;

characters DEL, CAN, DC2	–  represented by decimal 8;

characters EXT, EOT	–  represented by decimal 16;

characters HT, LF, VT, FF	–  represented by decimal 32;

all other characters in columns 0 and 1 of IA5
not included in above	–  represented by decimal 64.

NOTE – The decimal representation of individual values of this parameter allows coding to represent a single function or combination of functions, see Table 1.

## 3.4	Selection of idle timer delayXE "Selection of idle timer delay"§

Reference 4

The parameter will have the following selectable values:

any number from 0 to 255	–  represented by the respective decimal number.

The value 0 will indicate that no data forwarding on time-out is required; a value between 1 and 255 will indicate the value of the delay in twentieths of a second.

NOTES

1	Some PAD implementations may not offer all possible values of idle timer delay within the selectable range. In such cases where the value selected is not available, the PAD will assume the next higher available value in the range.

2	The effect of the idle timer delay on data forwarding may be subject to flow control constraints.

## 3.5	Ancillary device controlXE "Ancillary device control"§

Reference 5

The parameter will have the following selectable values:

no use of X-ON (DC1) and X-OFF (DC3)	–  represented by decimal 0;

use of X-ON and X-OFF (data transfer)	–  represented by decimal 1;

use of X-ON and X-OFF (data transfer
	and command)	–  represented by decimal 2.

## 3.6	Control of PAD service signals and PAD command signalsXE "Control of PAD service signals and PAD command signals"§

Reference 6

The parameter is represented by the following encoding of basic functions, each having a decimal value as shown below:

no service signals are transmitted to the start-stop
	mode DTE		–  represented by decimal 0;

service signals other than the prompt PAD service
	signal are transmitted in the standard format	–  represented by decimal 1;

editing PAD service signals only are transmitted in
	the format specified by parameter 19	–  represented by decimal 2;

prompt PAD service signal is transmitted in the
	standard format	–  represented by decimal 4;

PAD service signals are transmitted in a
	network-dependent format	–  represented by decimal 8 to 15.

PAD service signals and PAD command signals in the extended dialogue mode format:

extended dialogue mode handling, with PAD 
	service signals in English	–  represented by decimal 16;

extended dialogue mode handling, with PAD 
	service signals in French	–  represented by decimal 32;

extended dialogue mode handling, with PAD 
	service signals in Spanish	–  represented by decimal 48.

NOTES

1	The decimal representation of individual values of this parameter allows coding to represent a single function or combination of functions, see Table 1. The transmission of service signals is controlled by the values 0, 1, 2, 4 and 5. In addition, the actual text of the service signal is controlled by the values 16 and above.

2	Values of 64 to 240 in multiples of 16 represent additional languages provided on a network dependent basis.

3	Values of 16 to 240 in multiples of 16 may be combined with values 8 to 15 to provide a network-dependent extended dialogue mode (e.g. a value of 41 is equivalent to 32 + 9, network-dependent French).

4	To enable or disable service signals in the extended dialogue mode, the values 16 to 240 in multiples of 16 can be combined with: 0 for no service signals, 1 for service signals other than the prompt, 2 for editing service signals alone and 5 (4 + 1) for service signals and the prompt.

## 3.7	Selection of operation of PAD on receipt of break signal from the start-stop mode DTEXE "Selection of operation of PAD on receipt of break signal from the start-stop mode DTE"§

Reference 7

This parameter is represented by the following encoding of basic functions, each having a decimal value as shown below:

nothing			–  represented by decimal 0;

send to packet mode DTE
	or other PAD an interrupt packet	–  represented by decimal 1;

reset				–  represented by decimal 2;

send to packet mode DTE or other PAD an
	indication of break PAD message	–  represented by decimal 4;

escape from data transfer state	–  represented by decimal 8;

discard output to start-stop mode DTE	–  represented by decimal 16.

NOTE – The decimal representation of individual values of this parameter allows coding to represent a single function or combination of functions, see Table 1.

## 3.8	Discard outputXE "Discard output"§

Reference 8

The parameter will have the following selectable values:

normal data delivery to the start-stop mode DTE	–  represented by decimal 0;

discard output to start-stop mode DTE	–  represented by decimal 1.

## 3.9	Padding after carriage returnXE "Padding after carriage return"§

Reference 9

The parameter will have the following selectable values:

any number from 0 to 255	–  represented by the respective decimal number.

A value between 0 and 255 will indicate the number of padding characters to be generated by the PAD after a carriage return character is transmitted to the start-stop mode DTE.

When parameter 9 is 0, there will be no padding except that PAD service signals will contain a number of padding characters according to the data rate of the start-stop mode DTE.

## 3.10	Line foldingXE "Line folding"§

Reference 10

The parameter will have the following selectable values:

no line folding	–  represented by decimal 0;

any value between 1 and 255	–  represented by the respective decimal number.

A value between 1 and 255 will indicate the number of graphic characters per line that will be transmitted by the PAD without inserting appropriate format effectors.

## 3.11	Binary speedXE "Binary speed"§

This parameter is a read-only parameter and cannot be changed by either of the DTEs. It enables the packet-mode DTE to access a characteristic of the start-stop mode DTE which is known by the PAD.

Reference 11

The parameter will have the following values:

50 bit/s	–	represented by decimal 10;

75 bit/s	–	represented by decimal  5;

100 bit/s	–	represented by decimal  9;

110 bit/s	–	represented by decimal  0;

134.5 	bit/s	–	represented by decimal  1;

150 bit/s	–	represented by decimal  6;

200 bit/s	–	represented by decimal  8;

300 bit/s	–	represented by decimal  2;

600 bit/s	–	represented by decimal  4;

1200 	bit/s	–	represented by decimal  3;

1800	 bit/s	–	represented by decimal  7;

75/1200 bit/s	–	represented by decimal 11;

2400 	bit/s	–	represented by decimal 12;

4800	 bit/s	–	represented by decimal 13;

9600	 bit/s	–	represented by decimal 14;

19 200 	bit/s	–	represented by decimal 15;

48 000	 bit/s	–	represented by decimal 16;

56 000	 bit/s	–	represented by decimal 17;

64 000	 bit/s	–	represented by decimal 18;

14 400	 bit/s	–––	represented by decimal 19;

NOTE – The values implemented in individual PADs depend on the range of DTE data transmission rates which are supported. The allocation of decimal values to all known rates is to avoid revision of the Recommendation in the future.

## 3.12	Flow control of the PAD by the start-stop mode DTEXE "Flow control of the PAD by the start-stop mode DTE"§

Reference 12

The parameter will have the following selectable values:

no use of X-ON (DC1) and X-OFF (DC3)
for flow control	–  represented by decimal 0;

use of X-ON and XOFF for flow control	–  represented by decimal 1.

## 3.13	Linefeed insertion after carriage returnXE "Linefeed insertion after carriage return"§

Reference 13

This parameter is represented by the following encoding of basic functions, each having a decimal value as shown below:

no linefeed insertion	–  represented by decimal 0;

insert linefeed after each carriage return in the
	data stream to the start-stop DTE	–  represented by decimal 1;

insert linefeed after each carriage return in the
	data stream from the start-stop mode DTE	–  represented by decimal 2;

insert linefeed after each carriage return in the
	echo stream to the start-stop mode DTE	–  represented by decimal 4.

NOTES

1	The decimal representation of individual values of this parameter allows coding to represent a single function or combination of functions, see Table 1.

2	This function applies only in the data transfer state.

## 3.14	Linefeed paddingXE "Linefeed padding"§

Reference 14

The parameter will have the following selectable values:

any number from 0 to 255	–  represented by the respective decimal number.

A value between 0 and 255 will indicate the number of padding characters to be generated by the PAD after a linefeed character is transmitted to the start-stop mode DTE during the data transfer state.

## 3.15	EditingXE "Editing"§

Reference 15

The parameter will have the following selectable values:

no use of editing in the data transfer state	–  represented by decimal 0;

use of editing in the data transfer state	–  represented by decimal 1.

The use of value 1 suspends the following operations of the PAD:

a)	data forwarding on full packet until the editing buffer is full;

b)	data forwarding on idle timer period expiry.

NOTE – The value of parameter 4 remains unchanged.

## 3.16	Character deleteXE "Character delete"§

Reference 16

The parameter will have the following selectable values:

one character from IA5	–  represented by decimal 0 to 127.

The character defined by the user for character delete is the binary representation of the decimal value in accordance with Recommendation T.50.

the sequence of characters 1/3 4/7	–  represented by decimal 128;

the sequence of characters 1/3 1/3	–  represented by decimal 129;

the sequence of characters 2/10 2/10	–  represented by decimal 130.

## 3.17	Line deleteXE "Line delete"§

Reference 17

The parameter will have the following selectable values:

one character from IA5	–  represented by decimal 0 to 127.

The character defined by the user for line delete is the binary representation of the decimal value in accordance with Recommendation T.50.

## 3.18	Line displayXE "Line display"§

Reference 18

The parameter will have the following selectable values:

one character from IA5	–  represented by decimal 0 to 127.

The character defined by the user for line display is the binary representation of the decimal value in accordance with Recommendation T.50.

## 3.19	Editing PAD service signalsXE "Editing PAD service signals"§

Reference 19

The parameter will have the following selectable values:

no editing PAD service signals	–  represented by decimal 0;

editing PAD service signals for printing terminals	–  represented by decimal 1;

editing PAD service signals for display terminals	–  represented by decimal 2;

editing PAD service signals using one character
	from the range of IA5	–  represented by decimal 8 and 32 to 126.

NOTE – This parameter does not apply if the value of parameter 6 is set to 0.

## 3.20	Echo maskXE "Echo mask"§

Reference 20

This parameter is represented by the following encoding of basic functions, each having a decimal value as shown below:

no echo mask (all characters echoed)	–  represented by decimal 0;

no echo of character CR	–  represented by decimal 1;

no echo of character LF	–  represented by decimal 2;

no echo of characters VT, HT, FF	–  represented by decimal 4;

no echo of characters BEL, BS	–  represented by decimal 8;

no echo of characters ESC, ENQ	– represented by decimal 16;

no echo of characters ACK, NAK, STX, SOH,
	EOT, ETB, ETX	–  represented by decimal 32;

no echo of editing characters as designated by
	parameters 16, 17 and 18	–  represented by decimal 64;

no echo of all other characters in columns 0
	and 1 of IA5 not mentioned above, and the
	character DEL	–  represented by decimal 128.

NOTES

1	The decimal representation of individual values of this parameter allows coding to represent a single function or combination of functions, see Table 1.

2	If parameter 5, 12 or 22 is set to a non-zero value, then X-ON and X-OFF are not echoed.

3	The PAD recall character specified by the value of parameter 1 is not echoed.

4	A character need only be specified by a single value of parameter 20 in order for it not to be echoed.

5	This parameter applies only when parameter 2 is set to 1.

6	Parameter 20 value 64 (in combination) does not apply if editing is not enabled.

## 3.21	Parity treatmentXE "Parity treatment"§

Reference 21

This parameter will have the following selectable values:

no parity checking or generation	–  represented by decimal 0;

parity checking	–  represented by decimal 1;

parity generation	–  represented by decimal 2;

no parity-transparent bit 8	–  represented by decimal 4.

NOTE

1	The decimal representation of individual values of this parameter allows coding to represent a single function or a combination of functions, see Table 1.

2	Characters generated by the PAD itself (e.g. PAD service signals) have even parity when parameter 21 is set to zero unless the parity is determined by alternate means.

## 3.22	Page waitXE "Page wait"§

Reference 22

This parameter will have the following selectable values:

page wait disabled	–  represented by decimal 0;

page wait condition after n linefeed characters
	are sent by the PAD to the start-stop mode DTE	–  represented by decimal 1 to 255.

## 3.23	Size of input fieldXE "Size of input field"§

Reference 23

The parameter will have the following selectable values:

no input field length		–  represented by decimal 0;

size of input field (in graphic characters)	–  represented by decimal 1 to 255.

NOTE – Implementors should take into account that a graphic character may be coded with several bytes. The definition of a graphic character is given in Annexes A/T.101, B/T.101 and C/T.101.

## 3.24	End-of-frameXE "End-of-frame"§

Reference 24

The parameter will have the following selectable values:

no end-of-frame signals	–  represented by decimal 0;

the end-of-frame condition is fulfilled upon
	receipt of this character	–  represented by decimal 1 to 31;

the end-of-frame condition is fulfilled upon
	receipt of a complete packet sequence	–  represented by decimal 32;

the end-of-frame condition is fulfilled upon
	detection of the end of a timer	–  represented by decimal 64.

The value of the timer is service dependent and shall be in the range of 1 to 5 seconds with a typical value of 3 seconds. The value of this timer cannot be changed by the X.25 DTE nor the start-stop mode DTE.

NOTE – The decimal representation of each individual value allows the coding to represent a single function or combination of functions (see Table 1).

The PAD shall consider receipt of an X.25 Reset packet, X.25 Clear indication packet or any X.29 PAD message as an "End-of-frame" signal.

## 3.25	Selection of extended data forwarding signalsXE "Selection of extended data forwarding signals"§

Reference 25

This parameter is represented by the following encoding of basic functions, each having a decimal value as shown below:

no extended data forwarding condition	–  represented by decimal 0;

sequence of two characters  starting with 1/3	–  represented by decimal 1;

character 1/10		–  represented by decimal 2;

character 1/12		–  represented by decimal 4;

character 2/3		–  represented by decimal 8;

character 5/15		–  represented by decimal 16;

sequence of the two characters 1/11 4/10
	or sequence of the two characters 1/11 4/11	–  represented by decimal 32;

sequence of three characters starting with
	characters 2/10 3/0	–  represented by decimal 64;

either character 1/3 or character 2/10	–  represented by decimal 128.

NOTE – The decimal representation of individual value of this parameter allows coding to represent a single function or combination of functions (see Table 1).

## 3.26	Display interruptXE "Display interrupt"§

## Reference 26

The parameter will have the following selectable value:

display interrupt disabled	–  represented by decimal 0;

display interrupt condition occurs upon receipt
	of this character	–  represented by decimal 1 to 127;

display interrupt condition occurs upon receipt
	of any character	–  represented by decimal 128;

display interrupt condition occurs upon receipt
	of any one of 2/10, 5/15, 3/0 to 3/9 inclusive	–  represented by decimal 129;

display interrupt condition occurs upon receipt
	of any one of 1/3, 1/12 or 3/0 to 3/9 inclusive	–  represented by decimal 130.

## 3.27	Confirmation of display interruptXE "Confirmation of display interrupt"§

Reference 27

The parameter will have the following selectable value:

confirmation prompt of display interrupt disabled	–  represented by decimal 0;

the confirmation prompt of display interrupt is
	character from IA5	–  represented by decimal 1 to 127.

## 3.28	Diacritic character editingXE "Diacritic character editing"§

Reference 28

The parameter will have the following selectable value:

basic character set
	primary code table only, normally ASCII	–  represented by decimal 0;

composite coded diacritic character
	set - rule 1 (SS2)	–  represented by decimal 1;

Diacritic and supplementary characters consisting of several bytes created using SS2 code extensions are deleted as a single character.

composite coded diacritic character
	set - rule 2 (G1)	–  represented by decimal 2;

Diacritic characters consisting of several bytes created using the primary table in G0 in G-Left and supplementary in G-Right are deleted as a single character.

direct coded diacritic character rule	–  represented by decimal 3;

Diacritic and supplementary characters consisting of several bytes created using SS2 code extension are deleted as a single character.

coding of SS2		–  represented by decimal 4;

The coding of SS2 is by default the C0 code 1/9 ; this parameter reference value indicates that coding of SS2 uses the alternate code 8/14.

NOTE – This value may be combined with one of the three previous ones.

multi-byte character sets	–  this is for further study.

The support of multi-byte character sets is for further study.

## 3.29	Extended echo maskXE "Extended echo mask"§

Reference 29

The parameter will have the following selectable values:

no extended echo mask
	(the parameter reference 20 may be active)	–  represented by decimal 0;

1/10 is echoed as 2/3	–  represented by decimal 1;

1/3 is echoed as 2/10	–  represented by decimal 2;

no echo of 1/15	–  represented by decimal 4;

no echo of all characters in columns 8
	and 9 (Note 4)		–  represented by decimal 8;

no echo of all characters in columns 8 and 9
	except 9/11 (Note 4)	–  represented by decimal 16;

no echo of the sequence of two characters 1/3 x/y
	(x between 2 and 7, y between 0 and 7)	–  represented by decimal 32.

NOTES

1	The decimal representation of individual values of this parameter allows coding to represent a single function or a combination of functions.

2	This parameter does not apply if parameter 2 is set to 0.

3	If parameter 20 is set to 128, 1/3, 1/10 and 1/15 are not echoed.

4	This value is applicable only in an 8-bit environment.

TABLE  1/X.3

Possible values and combination of values of PAD parameters (Note 1)

| Parameter reference | Parameter | Selectable possible values |  | PAD parameter | Remarks |
| --- | --- | --- | --- | --- | --- |
| number | description | Mandatory | Optional (Note 2) | meaning |  |
| 1 | PAD recall using a character (E) | 0 1 | 32 to 126 | Not possible Character DLE Possible; using one graphic character defined by user |  |
| 2 | Echo (E) | 0 1 | 2 32 to 126 | No echo Echo Echo all characters except the data forwarding sequence defined by parameter 25 Scrambled echo character |  |
| 3 | Selection of data forwarding character(s) (E) | 0 2 126 | 1 4 6 8 16 18 32 64 | No data forwarding character(s) Alphanumeric characters (A to Z, a to z, 0 to 9) Character CR Characters ESC, BEL, ENQ, ACK Characters CR, ESC, BEL, ENQ, ACK Characters DEL, CAN, DC2 Characters ETX, EOT Characters CR, EOT, ETX Characters HT, LF, VT, FF All characters in columns 0 and 1 of IA5 except those shown above for values 2, 4, 8, 16, 32 Values 3, 5 to 7, 9 to 15, 17 to 31, 33 to 63, 65 to 125 127 may be formed by combinations of values 1, 2, 4, 8, 16, 31, 64 All characters in columns 0 and 1 and character DEL | Value formed by combination (2 + 4) Value formed by combination (2 + 16) Value formed by combination (2 + 4 + 8 + 16 + 32 + 64) |
| 4 | Selection of idle timer delay (E) | 0 20 255 | 1 to 19 21 to 254 | Value of idle timer in twentieths of a second | (Note 3) |

TABLE  1/X.3 (continued)

Possible values and combination of values of PAD parameters (Note 1)

| Parameter reference | Parameter | Selectable possible values |  | PAD parameter | Remarks |
| --- | --- | --- | --- | --- | --- |
| number | description | Mandatory | Optional (Note 2) | meaning |  |
| 5 | Ancillary device control (E) | 0 1 | 2 | No use of X-ON (DC1) and X-OFF (DC3) Use of X-ON and X-OFF (data transfer) Use of X-ON and X-OFF (data transfer and command) |  |
| 6 | Control of PAD service signals and PAD command signals (E) | 0 1 | 2 5 8 to 15 16 32 48 | No PAD service signals are transmitted to the start-stop mode DTE PAD service signals are transmitted in the standard format PAD service signals are transmitted in the format specified by parameter 19 PAD service signals and the prompt PAD service signal are transmitted in the standard format PAD service signals are transmitted in a network dependent format Extended dialog mode, PAD service signals are in English Extended dialog mode, PAD service signals are in French Extended dialog mode, PAD service signals are in Spanish | Value formed by combination (1 + 4) |
| 7 | Section of operation of the PAD on receipt of break signal from the start-stop mode DTE (E) | 0 2 8 21 | 1 4 5 16 | Nothing Interrupt Reset Send to DTE an indication of break PAD message Interrupt and indication of break Escape from data transfer state Discard output, to start-stop mode DTE Discard output, interrupt and indication of break Values 3, 4, 6, 7, 9 to 20, 22 to 31 may be formed by combination of values 1, 2, 4, 8, 16 | Value formed by combination (1 + 4) Value formed by combination (1 + 4 + 16) |

TABLE  1/X.3 (continued)

Possible values and combination of values of PAD parameters (Note 1)

| Parameter reference | Parameter | Selectable possible values |  | PAD parameter | Remarks |
| --- | --- | --- | --- | --- | --- |
| number | description | Mandatory | Optional (Note 2) | meaning |  |
| 08 | Discard output (E) | 0 1 |  | Normal data delivery Discard output |  |
| 09 | Padding after carriage return (CR) (E) | 0 1 to 70 | 8 to 255 | No padding after CR (Note 4) Number of padding characters inserted after CR |  |
| 10 | Line folding (E) | 00 1 to 255 |  | No line folding Number of graphic characters per line |  |
| 11 (read only) | Binary speed of start-stop mode DTE (E) | 0 2 | 1 3 4 5 6 7 8 9 10 11 12 13 14 15 16 17 18 19 | 75/1110 bit/s /1.134,5 bit/s 75/1300 bit/s /1 1200 bit/s 75/1600 bit/s 752/175 bit/s 752/150 bit/s 75/1800 bit/s 75/1200 bit/s 75/1100 bit/s 75/2150 bit/s 75/1200 bit/s 71.2400 bit/s 71.4800 bit/s 71.9600 bit/s 719 200 bit/s 748 000 bit/s 756 000 bit/s 764 000 bit/s 714 400 bit/s | The values imple- mented in individ- ual PADs depend on the range of DTE data trans- mission rates which are supported. The allocation of decimal values of all known rates is to avoid revision of the Recommenda- tion in the future |
| 12 | Flow control of the PAD (E) | 0 1 |  | No use of X-ON (DC1) and X-OFF (DC3) for flow control Use of X-ON (DC1) and X-OFF (DC3) for flow control |  |

TABLE  1/X.3 (continued)

Possible values and combination of values of PAD parameters (Note 1)

| Parameter reference | Parameter | Selectable possible values |  | PAD parameter | Remarks |
| --- | --- | --- | --- | --- | --- |
| number | description | Mandatory | Optional (Note 2) | meaning |  |
| 13 | Linefeed insertion after carriage return (A) | 0 1 4 5 6 7 | 2 | No linefeed insertion Insert linefeed after transmission of CR to the start-stop mode DTE Insert linefeed after each carriage return in the data stream from the start-stop mode DTE Insert linefeed after echo of CR to start-stop mode DTE Insert linefeed after trans- mission to the start-stop mode DTE and after echo of CR Insert linefeed in data stream after CR from the start-stop mode DTE and after echo of a CR to the start-stop mode DTE Insert linefeed in the data stream to and from the start-stop mode DTE and after echo of a CR to the start-stop mode DTE | Combination (1 + 4) Combination (2 + 4) Combination (1 + 2 + 4) Note – Applies only to data transfer state |
| 14 | Padding after linefeed (A) | 0 1 to 7 | 8 to 255 | No padding after linefeed Number of padding characters inserted after linefeed | Note – Applies only to data transfer state |
| 15 (Note 5) | Editing (A) | 0 1 |  | No use of editing in the data transfer state Use of editing in the data transfer state |  |
| 16 (Note 5) | Character Delete (A) | 1270 | 0 to 126 128 129 130 | One character from IA5 Character 7/15 (DEL) Sequence of characters 1/3 4/7 Sequence of characters 1/3 1/3 Sequence of characters 2/10 2/10 |  |
| 17 (Note 5) | Line delete (A) | 24 | 0 to 230 25 to 127 | One character from range of IA5 Character 1/8 (CAN) One character from range of IA5 |  |

TABLE  1/X.3 (continued)

Possible values and combination of values of PAD parameters (Note 1)

| Parameter reference | Parameter | Selectable possible values |  | PAD parameter | Remarks |
| --- | --- | --- | --- | --- | --- |
| number | description | Mandatory | Optional (Note 2) | meaning |  |
| 18 (Note 5) | Line display (A) | 18 | 0 to 17 19 to 127 | One character from IA5 Character 1/2 (DC2) One character from IA5 |  |
| 19 (Note 5) | Editing PAD service signals (A) | 1 | 0 2 8 32 to 126 | No editing PAD service signals Editing PAD service signals for printing terminals Editing PAD service signals for display terminals Editing PAD service signals using one character from the range of IA5 |  |
| 20 (Notes 6 and 7) | Echo mask (A) | 0 | 1 2 4 8 16 32 64 128 | No echo mask (all characters echoed) No echo of CR No echo of LF No echo of VT, HT, FF No echo of BEL, BS No echo of ESC, ENQ No echo of ACK, NAK, STX, SOH, EOT, ETB, ETX No echo of editing characters as designated by parameters 16, 17, 18 (Note 8) No echo of all other characters in columns 0 and 1 not mentioned above and DEL | Values may be formed by combi- nation of basic values |
| 21 (Note 9) | Parity treatment (A) | 00 | 1 2 3 4 | No parity checking or generation Parity checking Parity generation Parity checking and parity generation No parity-transparent bit 8 | Value formed by combination (1 + 2) |
| 22 | Page wait (A) | 0 23 | 1 to 22 24 to 255 | Page wait disabled Number of line feed characters considered by the PAD for the page wait function |  |

TABLE  1/X.3 (continued)

Possible values and combination of values of PAD parameters (Note 1)

| Parameter reference | Parameter | Selectable possible values |  | PAD parameter | Remarks |
| --- | --- | --- | --- | --- | --- |
| number | description | Mandatory | Optional (Note 2) | meaning |  |
| 23 | Size of input field (A) | 0 | 1 to 255 | Undefined size Length of input field (number of graphic characters) |  |
| 24 | End-of-frame signals (A) | 0 32 | 1 to 31 64 | No end-of-frame signal Character representing the end of a frame A frame is represented by a complete packet sequence At the end of a timer, an end of frame condition may be assumed | Values may be formed by combi-nation of basic values |
| 25 | Extended data for- warding signals (A) | 0 | 1 2 4 8 16 32 64 128 | No extended data forwarding signal Sequence of two charac-ters 1/3 x/y Character 1/10 Character 1/12 Character 2/3 Character 5/15 Sequence of the two characters 1/11 4/10 or sequence of the two characters 1/11 4/11 Sequence of three characters 2/10 3/0 x/y Either character 1/3 or character 2/10 | Values may be formed by combi- nation of basic values |
| 26 | Display interrupt (A) | 0 | 1 to 127 128 129 130 | No display interrupt One character from IA5 Any character Characters 2/10, 5/15, or 3/0 to 3/9 inclusive Characters 1/3, 1/12, or 3/0 to 3/9 inclusive |  |
| 27 | Display interrupt confirmation (A) | 0 | 1 to 127 | No display interrupt confirmation One character from IA5 |  |
| 28 | Diacritic character coding (A) | 0 | 1 2 3 4 | Basic coding Composit coded diacritic rule 1 (SS2) Composite coded diacritic rule 2 (G1) Direct coded diacritic character rule Coding of SS2 | Value 4 may be conbined with one among 1 to 3 |

TABLE  1/X.3 (end)

Possible values and combination of values of PAD parameters (Note 1)

| Parameter reference | Parameter | Selectable possible values |  | PAD parameter | Remarks |
| --- | --- | --- | --- | --- | --- |
| number | description | Mandatory | Optional (Note 2) | meaning |  |
| 29 | Extended echo mask (A) | 0 | 1 2 4 8 16 32 | No extended echo mask Character 1/10 is echoed as 2/3 Character 1/3 is echoed as 2/10 No echo of character 1/15 No echo of all characters in columns 8 and 9 No echo of all characters in columns 8 and 9 except 9/11 No echo of the sequence of two characters 1/3 x/y (x between 2 and 7, y between 0 and 7) |  |
| E An essential parameter to be made available internationally. A An additional parameter which may be available on certain data networks and may also be available internationally. NOTES 1 Other values and possible combination of values are for further study. 2 These parameter values provide additional user facilities which are not necessarily provided in all PADs. 3 Some PAD implementations may not offer all possible values of idle timer delay within the selectable range. In such cases where the value selected is not available, the PAD will assume the next higher value available. 4 There is no padding after CR except that PAD service signals will contain a number of padding characters according to the data signalling rate of the start-stop mode DTE. 5 When parameter 15 is implemented, the values of parameters 16, 17, 18 and 19 are either default values or are selectable from the optional range shown. The editing function is provided during the PAD command state whether parameter 15 is implemented or not. If parameters 16, 17, 18 and 19 are implemented, the editing characters and editing PAD service signals during the PAD command state are defined by the appropriate values of these parameters. If parameters 16, 17, 18 and 19 are not implemented, the default values for the functions of these parameters are applicable to the PAD command state. 6 This parameter does not apply if parameter 2 is set to zero. 7 If parameter 5, 12 or 22 is set to a non-zero value, then the X-ON and X-OFF characters are not echoed. 8 Parameter 20 value 64 (in combination) does not apply if editing is not enabled. 9 Characters generated by the PAD itself (e.g. PAD service signals) have even parity when parameter 21 is set to zero unless the parity is determined by alternate means. |  |  |  |  |  |