|  | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

ITU-T	X.57

TELECOMMUNICATION

STANDARDIZATION SECTOR

OF  ITU

PUBLIC  DATA  NETWORKS

TRANSMISSION,  SIGNALLING  AND  SWITCHING

METHOD  OF  TRANSMITTING  A  SINGLE  LOWER  SPEED  DATA  CHANNEL  ON  A  
64 kbit/s  DATA  STREAM

ITU-T  Recommendation  X.57

(Extract from the Blue Book)

# NOTES

1	ITU-T Recommendation X.57 was published in Fascicle VIII.3 of the Blue Book. This file is an extract from the Blue Book. While the presentation and layout of the text might be slightly different from the Blue Book version, the contents of the file are identical to the Blue Book version and copyright conditions remain unchanged (see below).

2	In this Recommendation, the expression “Administration” is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

ã  ITU  1988, 1993

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

Recommendation X.57

Fascicle VIII.3 - Rec. X.57

METHOD OF TRANSMITTING A SINGLE LOWER SPEED DATA

CHANNEL ON A 64 kbit/s DATA STREAM

(Malaga-Torremolinos, 1984)

The CCITT,

considering

(a)	that the Recommendation X.1 defines the synchronous user classes of services to be provided by a public data network;

(b)	that Recommendation X.50 defines some envelope data formats which can be used in public data networks;

(c)	that the use of a full 64 kbit/s data stream for the transmission of a single lower speed synchronous data channel may be economically attractive in some applications, when a low cost 64 kbit/s channel is available;

(d)	that for customer access to circuit switched data services in the ISDN, a method for adaption of synchronous user data rates to a 64 kbit/s bearer rate is defined in Recommendation X.30, but that in certain applications as mentioned under (c) a simpler method is preferred,

unanimously declares

that for networks using a 6 + 2 envelope structure, a single low speed synchronous data channel (i.e. 600, 2400, 4800, 9600 bit/s) shall be transmitted through a 64 kbit/s data stream by using the following method:

The 6 + 2 envelopes related to the low speed channel shall be repeated as many times as required to reach the 64 kbit/s speed.

The receive end can recover the original data signal by taking out of the received 64 kbit/s stream an envelope for every envelope period of the chosen data rate.

In the networks which transmit the 8 kHz envelope timing, no alignment circuit is required in the receiver; in the networks not transmitting the 8 kHz timing, the alignment is obtained by transmitting an alignment pattern in the framing bit position of each envelope. When the framing bit is not used for alignment, it is available to transmit housekeeping information or, when this is not required, it is set to zero. Further study is needed for the case of 8 + 2 envelope structure.