|  | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

ITU-T	X.92

TELECOMMUNICATION

STANDARDIZATION SECTOR

OF  ITU

PUBLIC  DATA  NETWORKS

NETWORK  ASPECTS

HYPOTHETICAL  REFERENCE  CONNECTIONS  FOR  PUBLIC  SYNCHRONOUS  DATA  NETWORKS

ITU-T  Recommendation  X.92

(Extract from the Blue Book)

NOTES

1	ITU-T Recommendation X.92 was published in Fascicle VIII.3 of the Blue Book. This file is an extract from the Blue Book. While the presentation and layout of the text might be slightly different from the Blue Book version, the contents of the file are identical to the Blue Book version and copyright conditions remain unchanged (see below).

2	In this Recommendation, the expression “Administration” is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

ã  ITU  1988, 1993

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

Recommendation X.92

HYPOTHETICAL  REFERENCE  CONNECTIONS  FOR  PUBLIC

SYNCHRONOUS  DATA  NETWORKS

(Geneva, 1976; amended at Malaga-Torremolinos, 1984)

The CCITT,

bearing in mind

(a)	the international user classes of service indicated in Recommendation X.1;

(b)	the overall user-to-user performance objectives;

(c)	the need to standardize the procedures for use over public synchronous data networks;

(d)	in the case of packet switching, the need to standardize several procedural levels,

unanimously recommends

the use of the five hypothetical reference connections contained in this Recommendation.

1	The five hypothetical reference connections set down in the present Recommendation (see Figure 1/X.92) are intended for assessing the overall customer-to-customer performance objectives, for determining some data characteristics requirements of the various items in the connections and for setting limits to the impairments these items may introduce.

These hypothetical reference connections should be used for circuit-switched services, packet-switched services and leased line services in public synchronous data networks.

Other hypothetical reference connections may be set up in the future after experience of the design of synchronous public data networks has been gained.

2	The hypothetical reference connections of Figure 1/X.92 are intended for the user data signalling rates as recommended in Recommendation X.1.

Between points Y and Z, transmission takes place over 64 kbit/s digital paths. Such paths may include digital sections using modems over analogue facilities.

It should be assumed that the signalling for the circuit-switched data call control follows the same route as the data connection.

3	The logical links to be considered in the case of packet switching are indicated in Figure 1/X.92 by the dotted lines.

To allow for the incorporation of packet assembly/disassembly facilities, the variants to logical Link D of Figure 1/X.92, which are shown in Figure 2/X.92, are recognized.

4	It would be permissible to include a satellite in the transmission path of the national or local link. To allow for this, the variants of logical Links A and D of Figure 1/X.92, which are shown in Figures 3/X.92 and 4/X.92 respectively, are recognized.

In any connection, the maximum number of links via satellite should not exceed three (see Note).

Note - The maximum number of links via satellite allowable in a connection requires further study, in the light of signalling time-outs and quality of service considerations.