|  | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

CCITT	X.121

THE  INTERNATIONAL	(09/92)
TELEGRAPH  AND  TELEPHONE
CONSULTATIVE  COMMITTEE

DATA  COMMUNICATION  NETWORKS

NETWORK  ASPECTS

INTERNATIONAL  NUMBERING  PLAN
FOR  PUBLIC  DATA  NETWORKS

|  | Recommendation X.121 |
| --- | --- |

# FOREWORD

The CCITT (the International Telegraph and Telephone Consultative Committee) is a permanent organ of the International Telecommunication Union (ITU). CCITT is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The Plenary Assembly of CCITT which meets every four years, establishes the topics for study and approves Recommendations prepared by its Study Groups. The approval of Recommendations by the members of CCITT between Plenary Assemblies is covered by the procedure laid down in CCITT Resolution No. 2 (Melbourne, 1988).

Recommendation X.121 was revised by Study Group VII and was approved under the Resolution No. 2 procedure on the 10th of September 1992.

___________________

# CCITT  NOTES

1)	In this Recommendation, the expression “Administration” is used for conciseness to indicate both a telecommunication administration and a recognized private operating agency.

2)	A list of abbreviations used in this Recommendation can be found in Annex F.

ã  ITU  1993

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

Recommendation X.121

INTERNATIONAL  NUMBERING  PLAN  FOR  PUBLIC  DATA  NETWORKSXE "INTERNATIONAL  NUMBERING  PLAN  FOR  PUBLIC  DATA  NETWORKS"§1)

(provisional, Geneva, 1978; amended, Geneva, 1980, 
Malaga-Torremolinos, 1984; Melbourne, 1988 and Geneva, 1992)

Recommendation X.121     (09/92)

The CCITT,

considering

(a)	that the purpose of an International Numbering Plan for Public Data Networks is to facilitate the introduction of public data networks and provide for their interworking on a worldwide basis;

(b)	that there could be a number of public data networks in a country;

(c)	that the International Numbering Plan should permit the identification of a country as well as a specific public data network in that country;

(d)	that the International Numbering Plan should provide means for interworking with other numbering plans;

(e)	that Recommendation E.164 describes the Numbering Plan for the ISDN Era;

(f)	that Recommendation F.69 describes the Plan for Telex Destination Codes;

(g)	that the International Numbering Plan for Data Networks should provide for substantial spare capacity to accommodate future requirements,

unanimously declares

that the International Numbering Plan for Public Data Networks should be as defined in this Recommendation.

# 1	Design considerations

The design considerations that form the basis of this Numbering Plan are as follows:

1.1	The international data number is to determine only the specific data terminal equipment/data circuit-terminating equipment (DTE/DCE) interface and, in particular, to identify a country, and a network, if several data networks exist in the same country.

1.2	Where a number of public data networks are to be established in a country, it should not be mandatory to integrate the numbering plans of the various networks.

1.3	The number of digits comprising the code used to identify a country and a specific public data network in that country should be the same for all countries.

1.4	A national data number assigned to a DTE/DCE interfacexe " DTE/DCE interface"§ should be unique within a particular national network. This national data number should form part of the international data number which should also be unique on a worldwide basis.

1.5	The number of digits to be used in an international data number should be governed by national and international requirements but a reasonable limit on the overall number of digits should be imposed.

1.6	The Numbering Plan should make provision for the interworking of data terminals on public data networks with data terminals on public telephone and telex networks and on integrated services digital networks (ISDNs).

Note – The term “telex” employed in this Recommendation includes TWX networksxe " TWX networks"§.

1.7	The Numbering Plan should not preclude the possibility of a single national network providing an integrated telecommunications system for services of all kinds.

1.8	Where multiple RPOA facilities exist providing service to or within the same country, provision for the selection of a specific RPOA facility should be allowed for in the facility request part of the selection signals.

Note – The term RPOA in this Recommendation refers to recognized private operating agency.

# 2	Characteristics and application of the Numbering PlanXE "Numbering Plan"§

## 2.1	Number system

2.1.1	The 10-digit numeric character set 0-9 should be used for numbers (or addresses) assigned to DTE/DCE interfaces on public data networks. This principle should apply to both national and international data numbers.

2.1.2	Use of the above number system will make it possible for data terminals on public data networks to interwork with data terminals on public telephone and telex networks and on integrated services digital networks (ISDNs).

## 2.2	Data network identification codes and data country codes

2.2.1	A data network identification code (DNIC) could be assigned as follows:

2.2.1.1	To each public data network (PDN) within a country;

2.2.1.2	To non-zoned service, such as the public mobile satellite system (see § 2.2.10);

2.2.1.3	To a public switched telephone network (PSTN) or to an ISDN for the purpose of making calls from DTEs connected to a PDN to DTEs connected to that PSTN or ISDN;

Note – In order to facilitate the interworking of telex networks with data networks, some countries have allocated DNIC to telex networks.

2.2.1.4	To a group of PDNs within a country, when permitted by national regulations;

2.2.1.5	To a group of private data networks connected to PDNs within a country, where permitted by national regulations.

Note – For administrative purposes, including charging, a group of networks which have been assigned a single DNIC, will, in the international context, be considered as a single entity.

2.2.2	In the system of data network identification codes, the first digit of such codes should be in accordance with Table 1/X.121.

Note 1 – The allocation of codes for non-zoned services, other than the mobile satellite systems, is for further study.

Note 2 – Digits 8, 9 and 0 are used as escape codes, not being part of the DNIC. They are defined in § 2.6.

TABLE  1/X.121

First digit of data network identification code

| 1 2 3 4 5 6 7 | Reserved (see also § 2.2.10) For data network identification codes (DNICs) |
| --- | --- |

2.2.3	All data network identification codes (DNICs) should consist of four digits. The first three digits should always identify a country and could be regarded as a data country code (DCC). The fourth, or network digitxe " network digit"§, should identify a specific data network in the country.

2.2.4	Each country should be assigned at least one 3-digit data country code (DCC). The data country code (DCC) in conjunction with the fourth digit can identify up to 10 public data networks. The format for data network identification codes (DNIC) should be as indicated in Figure 1/X.121.

2.2.5	The system of data network identification codes (DNIC) indicated in §§ 2.2.2 and 2.2.4 will provide for 600 data country codes (DCC) and a theoretical maximum of 6000 DNIC.

2.2.6	In the case where a country requires more than 10 DNICs, additional data country codes (DCCs) could be assigned to the country (see § 2.2.8).

2.2.7	A list of data country codes (DCC) to be used in the development of data network identification codes (DNIC) is given in Annex D to this Recommendation. This list was prepared in accordance with the requirement that the first digit of a DNIC, which is also the first digit of the embedded data country code (DCC), should be restricted to the digits 2-7 inclusive (see § 2.2.2). As first digits of data country codes (DCC), the digits 2-7 are arranged to represent world zones.

2.2.8	The assignment of data country codes (DCC) is to be administered by the CCITT. The assignment of network digits will be made nationally and the CCITT Secretariat notified.

The Member countries of the International Telecommunication Union not mentioned in this list who wish to take part in the international data service or those Members who require an additional data country code(s) (DCC) should ask the Director of the CCITT for the assignment of an available 3-digit data country code(s) (DCC). In their request, they may indicate the available 3-digit code(s) preferred.

Assignments by the Director of the CCITT of data country codes (DCC) as well as assignments by countries of the network digits will be published in the Operational Bulletin of the International Telecommunication Union.

2.2.9	Examples indicating how data network identification codes (DNICs) could be developed, are given in Annex A.

### 2.2.10	International data number for stations in the public mobile satellite systemsXE "International data number for stations in the public mobile satellite systems"§

The DNICs allocated to public mobile satellite systems are 111S where the digit S indicates the ocean area. The digit S has the values as shown in Annex C.

The mobile station is identified by a unique mobile earth station number (INMARSAT mobile number) common for telephony, telex, data transmission and other services as defined in Recommendation E.215/F.125. The first digit of the mobile earth station number (INMARSAT mobile number) is the digit “T” defined in Recommendation E.215/F.125 and is used for discrimination between different public mobile satellite systems (such as the INMARSAT Standard A, B and C and aeronautical systems).

The complete international data number for mobile earth stations is composed as follows:

111S + mobile earth station number + X

where X is an optional digit which, if present, designates a particular DTE associated with the mobile earth station.

Note 1 – In the INMARSAT mobile satellite systems, the use of the S digit for indicating the ocean area in which the mobile earth station is located at the time of the call is considered a temporary arrangement. It is recognized that such an arrangement should be avoided in the future, if possible, since it requires the calling user to know the exact area of a destination mobile earth station at the time of the call, and such an area may change from time to time for the mobile earth station.

Note 2 – Digit “X” requires further studies regarding aeronautical and land mobile earth stations.

## 2.3	International data numberXE "International data number"§

2.3.1	A data terminal on a public data network when called from another country should be addressed by the international data number assigned to its DTE/DCE interface. The international data number should consist of the data network identification code (DNIC) of the called public data network, followed by the network terminal number (NTN) of the called DTE/DCE interface, or, for example, where an integrated numbering scheme exists within a country, the data country code (DCC) followed by the national number (NN) of the called DTE/DCE interface, i.e.:

International data number = DNIC + NTN, or, DCC + NN

2.3.2	The network terminal number (NTN) should consist of the full address that is used when calling the data terminal from within its serving public data network. The national number (NN) should consist of the full address used when calling the data terminal from another terminal within the national integrated numbering scheme. These numbers should consist of all the digits necessary to uniquely identify the corresponding DTE/DCE interface within the serving network and should not include any prefix (or access code) that might be employed for such calling.

Note 1 – Network terminal numbers (NTN) or national numbers (NN) may be assigned by a PDN to DTEs connected to other public networks, when interworking capabilities are provided with that PDN.

Note 2 – An example of the development of NTNs where a DNIC is assigned to a group of public or private data networks connected to PDNs within a country, is shown in Annex B.

## 2.4	Number of digits

2.4.1	International data numbers could be of different lengths but should consist of at least 5 digits but not more than 14 digits.

With the data network identification code (DNIC)xe " data network identification code (DNIC)"§ fixed at 4 digits and the data country code (DCC)xe " data country code (DCC)"§ fixed at 3 digits, it would, therefore, be possible to have a network terminal number (NTN) of 10 digits maximum, or, a national number (NN) of 11 digits maximum.

Note 1 – The limit of 14 digits specified above applies exclusively to the international data number information. Adequate register capacity should be made available at data switching exchanges to accommodate the above digits as well as any additional digits that might be introduced for signalling, or other purposes.

Note 2 – After time “T” (see Recommendation E.165) the maximum number of digits of the international ISDN number will be 15. The need of extending the maximum capacity of the X.121 data number is for further study.

## 2.5	PrefixesXE "Prefixes"§

2.5.1	A prefix is an indicator consisting of one or more digits, allowing the selection of different types of address formats. Prefixes are not part of the international X.121 format and are not signalled over internetwork or international boundaries.

2.5.2	To distinguish between different address formats within a public data network (e.g. national data number and international data number formats), a prefix would generally be required. Any such prefix does not form a part of the data number. Pending further study, the use and composition of such a prefix is a national matter. However, the possible need to accommodate such a prefix with regard to digit register capacity should be noted. It is also a national matter to decide on evaluation of prefixes, escape code, parts of the international data number of incoming path of entry for routing or other purposes.

Note – In the case of X.25 access, a prefix indicating international data number format can only be one digit.

## 2.6	Escape codesXE "Escape codes"§

An escape code is an indicator consisting of one digit. It indicates that the following digits are a number from a different numbering plan.

An escape code when required has to be carried forward through the originating network and can be carried across internetwork and international boundaries.

Digits used for escape codes are the digits 8, 9 and 0. The allocation and their purpose are shown in Table 2/X.121. The escape codes are not part of the international data number but are part of the “international X.121 format” (see Figure 2/X.121).

## 2.7	Number analysis – International calls between public data networksXE "Number analysis – International calls between public data networks"§

2.7.1	In the case of international calls between public data networks, provision should be made in originating countries to interpret the first three digits of the international data number. These digits constitute the data country code (DCC) component of the data network identification code (DNIC) and identify the terminal country. This information is required in the originating country for routing purposes.

TABLE  2/X.121

Allocation of escape codes

| 8 9 0 | Indicates that the digits which follow are from the F.69 Numbering Plan Indicates that the digits which follow are from the E.164 Numbering Plan (Notes 2, 3 and 4) Indicates that the digits which follow are from the E.164 Numbering Plan (Notes 1, 3 and 4) |
| --- | --- |

Note 1 – In this case, 0 is to indicate that a digital interface between the PDN and the destination network (ISDN or integrated ISDN/PSTN) is requested.Note 2 – In this case, 9 is to indicate that an analogue interface on the destination network (PSTN or integrated ISDN/PSTN) is requested.Note 3 – In the case of calls from a packet switched public data network (PSPDN) to an integrated ISDN/PSTN which does not require a distinction between digital and analogue interfaces, only a single escape code (e.g. 9 or 0) may be required. However all PSPDNs interworking with ISDNs, PSTNs and integrated ISDN/PSTNs should also support both 9 and 0 escape codes when acting as an originating, transit or destination network.Note 4 – The E.163 Numbering Plan is fully incorporated into Recommendation E.164.Note 5 – Escape codes may be replaced by signalling means after time “T” (for the definition of time “T”, see Recommendation E.165).

2.7.2	In originating countries, it might also be necessary to interpret the fourth, or network digit of a DNIC and, if the originating network requires it, the first digit after the DNIC. Such interpretation would provide the identity of a specific network in a country where several public data networks are in service. This information might be required for billing purposes or for the selection of specific routes to called networks. An example of the requirement for interpretation of the fifth digit is the use of this digit in the mobile satellite systems for selection of a particular mobile system (digit “T”, see § 2.2.10).

Note 1 – With regard to number analysis and routing in the case of interworking with PSTN and ISDN, see Recommendations X.110 and X.122.

Note 2 – With regard to RPOA selection, see § 1.8.

2.7.3	Countries receiving international calls for public data networks should receive the complete international data number. However, where a country of destination indicates that it does not wish to receive the data country code (DCC) component of the DNIC, arrangements should be made to suppress the DCC.

2.7.4	For destination countries with more than ten public data networks, interpretation of the first three digits of the DNIC [i.e., the data country code (DCC)] would identify the group of networks within which the called network is included. Interpretation of the fourth, or network, digit of the DNIC would identify the called network in that group. Interpretation of the first three digits would also make it possible to verify that an incoming call has in fact reached the correct country.

2.7.5	In the case of destination countries where there are fewer than ten public data networks, the first three digits of the DNIC could provide the verification indicated in § 2.7.4. Interpretation of the fourth, or network, digit of the DNIC would identify the specific network being called.

2.7.6	In transit countries the complete international data number must be received. Interpretation of the first three digits would identify the called country. Interpretation of the fourth or network digit would identify a specific data network in the called country. Interpretation of the fourth digit might be required for billing purposes or for route selection beyond the transit country. It might also be necessary in the transit network to analyse the fifth digit to allow selection of a particular public mobile system (e.g. digit “T”, see § 2.2.10).

2.7.7	Where a data call is to be routed beyond a transit country through a second transit country, the complete international data number should always be sent to the second transit country. Where the data call is to be routed by a transit country to the country of destination, the arrangements indicated in § 2.7.3 should apply.

## 2.8	Numbering plan interworkingXE "Numbering plan interworking"§

Details on numbering plan interworking are outlined in Recommendation X.122/E.166 (see also Recommendations E.165, X.301 and I.330).

Transit cases are considered in these Recommendations. For routing aspects see also Recommen-dation X.110.

## 2.9	Directories and letterheadsXE "Directories and letterheads"§

2.9.1	Directories for public data networks should include information on the procedures to be followed for making international data calls. A diagram, such as that of Figure 2/X.121, could assist the customer in these procedures.

2.9.2	With regard to the prefix shown in Figure 2/X.121, it should be noted that the same prefix (designated P) could be used for all four types of calls. The choice of prefix is, however, a national matter.

2.9.3	With regard to RPOA selection (see § 1.8), it should be noted that an RPOA facility request designatorxe " RPOA facility request designator"§ would be used either in international data calls or within certain countries. Provision of this facility as well as the designation of the RPOA facility selection designator is a national matter in the originating country.

2.9.4	With regard to the publication of international data numbers on letterheads or other written material, it is recommended that the network terminal number (NTN) or national number (NN) should be easily distinguished within the international number, i.e. that there be a space between the 4-digit DNIC and the network terminal number (NTN) or, between the 3-digit data country code (DCC) and the national number (NN), where the fourth digit of the DNIC is included in the national number (NN).

ANNEX  A

(to Recommendation X.121)

Development of data network identification codesXE "Development of data network identification codes"§

Example 1

In this example, it is assumed for illustrative purposes only, that the Netherlands has established its first public data network. To develop the data network identification code (DNIC) for this network, it would be necessary for the Netherlands to assign to it a network digit to follow the listed data country code (DCC) 204 (see Annex D). Assuming that the Netherlands selected the digit 0 as the network digit, the data network identification code (DNIC) for this initial network would be 2040.

Example 2

In this example, it is assumed for illustrative purposes only, that five public data networks have been established in Canada. To develop the data network identification codes for these networks, it would be necessary for Canada to assign to each of these networks a network digit to follow the listed data country code (DCC) 302 (See Annex D). Assuming that Canada assigned the network digits 0-4 to the five networks, the resulting data network identification codes (DNIC) would be 3020, 3021, 3022, 3023 and 3024.

Example 3

In this example, it is assumed for illustrative purposes only, that eight public data networks have been established in the United States of America. It is also assumed that network digits 0-7 would be assigned by the United States of America to follow the listed data country code (DCC) 310 (see Annex D). The data network identification codes (DNIC) thus formed for these eight networks would be 3100, 3101, 3102, 3103, 3104, 3105, 3106 and 3107.

If, some time later, four additional public data networks were to be introduced in the United States of America, two of the four new networks could be assigned network digits 8 and 9 in association with data country code (DCC) 310, to produce the data network identification codes (DNIC) 3108 and 3109.

For the remaining two public data networks, the United States of America would have to ask the CCITT for an additional data country code (DCC). A request for a code next in sequence, i.e. 311, could be made if this code appeared to be spare. If code 311 could be made available it would be assigned to the United States of America. If it was not available, a spare code in the “300” series of data country codes (DCC) would be assigned. Assuming data country code (DCC) 311 was available and issued to the United States of America, the two remaining public data networks could be assigned network digits 0 and 1 in association with data country code (DCC) 311, to produce the data network identification codes (DNIC) 3110 and 3111.

The data network identification codes (DNIC) for the 12 public data networks would then be 3100, 3101, 3102, 3103, 3104, 3105, 3106, 3107, 3108, 3109, 3110 and 3111.

Example 4

In this example, it is assumed for illustrative purposes only, that a public data network is to be established in each of two Caribbean islands that are part of the group of islands known as the French Antilles. The islands concerned are Guadeloupe and Martinique.

To develop the data network identification codes (DNIC) for these public data networks, it is assumed that the French Administration would assign network digit 0 to the network in Guadeloupe and network digit 1 to the network in Martinique and associate these network digits with the listed data country code (DCC) 340 for the French Antilles (see Annex D). The data network identification codes (DNIC) thus formed would be 3400 for Guadeloupe and 3401 for Martinique.

This example indicates that the system of data network identification codes (DNIC) is appropriate for application to groups of islands or regions of a country since one data country code (DCC) could provide for up to ten public data networks dispersed over several islands or regions. At the same time such island or regional networks would be distinguishable from each other.

ANNEX  B

(to Recommendation X.121)

Development of network terminal numbers where a data network
identification code is assigned to a group of public data networks
or to a group of private data networks connected
to public data networks within a country

The following is a guideline for allocating within a country data numbers for DTE/DCE interfaces on private data networks which are in turn connected to public data networks where permitted by national regulations.

Note – In the context of this annex, a private data network identification code (PNIC) may also be used to identify a specific public data network in a group of public data networks that share a common DNIC.

B.1	A private data network identification code (PNIC)xe " private data network identification code (PNIC)"§ is assigned to each private data network contained within a group of private data networks identified by a specific DNIC. The private data network identification code (PNIC)-digits are the first digits of the NTN.

B.2	All private data network identification codes (PNICs) consist of up to six digits. The format for the private data network identification codes (PNICs) is as follows:

ZXXXXX private data network identification code (PNIC)

Z denotes any digit from 2 through 9 as indicated in § B.3 (see Note in Table B-1/X.121).

X denotes any digit 0 through 9.

B.3	In the system of private data network identification codes (PNICs), the first digit of such codes is in accordance with Table B-1/X.121.

TABLE  B-1/X.121

First digit of private data network identification code

Note – The use of 0 or 1 depends on the national use of 0 or 1.

| 0 1 | See Note |
| --- | --- |
| 2 3 4 5 6 7 8 9 | For private data network identification codes (PNICs) |

B.4	If a country has more private data networks than can be grouped under one DNIC or, if the public data networks within a country are not all interconnected, another DNIC may be allocated for each new group of private data networks.

B.5	If a private data network requires more numbers for DTE/DCE interfaces than can be grouped under one PNIC, multiple PNICs may be allocated to a single private data network.

B.6	The assignment of private data network identification codes (PNICs) is administered nationally.

ANNEX  C

(to Recommendation X.121)

List of DNICs for non-zoned systems
public mobile satelite sistemsXE "List of DNICs for non-zoned systems
public mobile satelite sistems"§

Code	Area	System

1110	Spare

1111	Atlantic Ocean	INMARSAT mobile satellite data transmission system

1112	Pacific Ocean	INMARSAT mobile satellite data transmission system

1113	Indian Ocean	INMARSAT mobile satellite data transmission system

1114	Atlantic Ocean-West	INMARSAT mobile satellite data transmission system

1115	Spare

1116	Spare

1117	Spare

1118	Spare

1119	Spare

ANNEX  D

(to Recommendation X.121)

List of data country or geographical area codesXE "List of data country or geographical area codes"§

Note – The countries or geographical areas shown in this annex include those that already have code assignments in the case of other public telecommunication networks.

# Zone 2

Code	Country or geographical area

202	Greece

204	Netherlands (Kingdom of the)

205	Netherlands (Kingdom of the)

206	Belgium

208	France

209	France

212	Monaco

214	Spain

216	Hungarian Republic of)

220	Yugoslavia (Federal Republic of)

222	Italy

225	Vatican City State

226	Romania (Socialist Republic of)

228	Switzerland (Confederation of)

230	Czech and Slovak Federal Republic

232	Austria

234	United Kingdom of Great Britain and Northern Ireland

235	United Kingdom of Great Britain and Northern Ireland

236	United Kingdom of Great Britain and Northern Ireland

237	United Kingdom of Great Britain and Northern Ireland

238	Denmark

240	Sweden

242	Norway

244	Finland

246	Lithuania (Republic of)

247	Latvia (Republic of)

248	Estonia (Republic of)

250	Russian Federation

251	Russian Federation

255	Ukraine

257	Belarus (Republic of)

259	Moldova Republic of)

260	Poland (Republic of)

262	Germany (Federal Republic of)

263	Germany (Federal Republic of)

264	Germany( Federal  Republic of)

265	Germany( Federal  Republic of)

266	Gibraltar

268	Portugal

270	Luxembourg

272	Ireland

274	Iceland

276	Albania (Republic of)

278	Malta (Republic of)

# Zone 2 (cont.)

Code	Country or geographical area

280	Cyprus (Republic of)

282	Gerogia (Republic of)

283	Armenia (Republic of)

284	Bulgaria (Republic of)

286	Turkey

288	Faroe Islands

290	Greenland

292	San Marino (Republic of)

___________

Zone 2, Spare Codes: 48

# Zone 3

Code	Country or Geographical Area

302	Canada

303	Canada

308	St. Pierre and Miquelon

310	United States of America

311	United States of America

312	United States of America

313	United States of America

314	United States of America

315	United States of America

316	United States of America

330	Puerto Rico

332	Virgin Islands (USA)

334	Mexico

338	Jamaica

340	French Antilles

342	Barbados

344	Antigua and Barbuda

346	Cayman Islands

348	British Virgin Islands

350	Bermuda

352	Grenada

354	Montserrat

356	St. Kitts

358	St. Lucia

360	St. Vincent and the Grenadines

362	Netherlands Antilles

364	Bahamas (Commonwealth of the)

366	Dominica

368	Cuba

370	Dominican Republic

372	Haiti (Republic of)

374	Trinidad and Tobago

376	Turks and Calcos Islands

____________

Zone 3, Spare Codes: 67

# Zone 4

Code	Country or Geographical Area

400	Azerbaijani Republic

401	Kazakhstan (Republic of)

404	India (Republic of)

410	Pakistan (Islamic Republic of)

412	Afghanistan (Islamic State of)

413	Sri Lanka (Democratic Socialist Republic of)

414	Myanmar (Union of)

415	Lebanon

416	Jordan (Hashemite Kingdom of)

417	Syrian Arab Republic

418	Iraq (Republic of)

419	Kuwait (State of)

420	Saudi Arabia (Kingdom of)

421	Yemen (Republic of)

422	Oman (Sultanate of)

423	Yemen (Republic of)

424	United Arab Emirates

425	Israel (State of)

426	Bahrain (State of)

427	Qatar (State of)

428	Mongolia

429	Nepal

430	United Arab Emirates (Abu Dhabi)

431	United Arab Emirates (Dubai)

432	Iran (Islamic Republic of)

434	Uzbekistan (Republic of)

436	Tajikistan (Republic of)

437	Kyrgyztan (Republic of)

438	Turkmenistan (Republic of)

440	Japan

441	Japan

442	Japan

443	Japan

450	Korea (Republic of)

452	Viet Nam (Socialist Republic of)

453	Hongkong

454	Hong Kong

455	Macao

456	Cambodia

457	Lao People's Democratic Republic

460	China (People's Republic of)

466	Taiwan, China

467	Democratic People's Republic of Korea

470	Bangladesh (People's Republic of)

472	Maldives (Republic of)

480	Korea (Republic of)

481	Korea (Republic of)

___________

Zone 4, Spare Codes: 54

# Zone 5

Code	Country or Geographical Area

502	Malaysia

505	Australia

510	Indonesia (Republic of)

515	Philippines (Republic of the)

520	Thailand

525	Singapore (Republic of)

528	Brunei Darussalam

530	New Zealand

534	Commonwealth of the Northern Marianas (USA)

535	Guam

536	Nauru (Republic of)

537	Papua New Guinea

539	Tonga (Kingdom of)

540	Solomon Islands

541	Vanuatu (Republic of)

542	Fiji

543	Wallis and Futuna Islands

544	American Samoa

545	Kiribati (Republic of)

546	New Caledonia and Dependencies

547	French Polynesia

548	Cook Islands

549	Western Samoa

550	Micronesia (Federated States of)

___________

Zone 5, Spare Codes: 76

# Zone 6

Code	Country or Geographical Area

602	Egypt (Arab Republic of)

603	Algeria (People's Democratic Republic of)

604	Morocco (Kingdom of)

605	Tunisia

606	Libya (Socialist People's Libyan Arab Jamahiriya)

607	Gambia (Republic of the)

608	Senegal (Republic of)

609	Mauritania (Islamic Republic of)

610	Mali (Republic of)

611	Guinea (Republic of)

612	Côte d'Ivoire (Republic of)

613	Burkina Faso

614	Niger (Republic of the)

615	Togolese Republic

616	Benin (Republic of)

617	Mauritius (Republic of)

618	Liberia (Republic of)

619	Sierra Leone

620	Ghana

621	Nigeria (Federal Republic of)

622	Chad (Republic of the)

623	Central African Republic

# Zone 6 (cont.)

Code	Country or geographical area

624	Cameroon (Republic of)

625	Cape Verde (Republic of)

626	Sao Tome and Principe (Democratic Republic of)

627	Equatorial Guinea (Republic of)

628	Gabonese Republic

629	Congo (Republic of the)

630	Zaire (Republic of)

631	Angola (People's Republic of)

632	Guinea-Bissau (Republic of)

633	Seychelles

634	Sudan (Republic of the)

635	Rwandese (Republic of)

636	Ethiopia

637	Somali Democratic Republic

638	Djibouti (Republic of)

639	Kenya (Republic of)

640	Tanzania (United Republic of)

641	Uganda (Republic of)

642	Burundi (Republic of)

643	Mozambique (Republic of)

645	Zambia (Republic of)

646	Madagascar (Democratic Republic of)

647	Reunion (French Department of)

648	Zimbabwe (Republic of)

649	Namibia (Republic of)

650	Malawi

651	Lesotho (Kingdom of)

652	Botswana (Republic of)

653	Swaziland (Kingdom of)

654	Comoros (Islamic Federal Republic of the)

655	South Africa (Republic of)

___________

Zone 6, Spare Codes: 47

# Zone 7

Code	Country or Geographical Area

702	Belize

704	Guatemala (Republic of)

706	El Salvador (Republic of)

708	Honduras (Republic of)

710	Nicaragua

712	Costa Rica

714	Panama (Republic of)

716	Peru

722	Argentine Republic

724	Brazil (Federative Republic of)

730	Chile

732	Colombia (Republic of)

734	Venezuela (Republic of)

736	Bolivia (Republic of)

# Zone 7 (cont.)

Code	Country or geographical area

738	Guyana

740	Ecuador

742	Guiana (French Department of)

744	Paraguay (Republic of)

746	Suriname (Republic of)

748	Uruguay (Eastern Republic of)

______________

Zone 7, Spare Codes: 80

ANNEX  E

(to Recommendation X.121)

Definitions regarding the international numbering plan
for public data networks

## E.1	data country code

In the context of the international numbering plan for public data networks, a component of the international X.121 format consisting of three digits allocated by CCITT and published in Recommendation X.121.

## E.2	data network identification code

In the context of the international numbering plan for public data networks, a component of the international X.121 format consisting of four digits. The first three digits are regarded as the data country code, the fourth digit identifies a network in that country according to Recommendation X.121.

Note 1 – The digit allocated by countries to establish, together with the data country code the data network identification code, should be notified to the CCITT Secretariat.

Note 2 – CCITT publishes a list of data network identification codes.

## E.3	escape code

In the context of the international numbering plan for public data networks, an indicator consisting of one digit which indicates that the following digits are a number from a different numbering plan.

Note – An escape code is part of the international X.121 format.

## E.4	international data number

In the context of the international numbering plan for public data networks, the address information comprising the data country code and the national number, or the data network identification code and the (national) network terminal number, according to Recommendation X.121.

## E.5	international data number format

In the context of the international numbering plan for public data networks, a numbering plan format comprising of the digits of the international data number, according to Recommendation X.121.

## E.6	international X.121 format

In the context of the international numbering plan for public data networks, a format consisting of digits which are to be transferred across international boundaries, according to Recommendation X.121.

Note 1 – See also “international data number format”.

Note 2 – Escape codes, if required, are part of the international X.121 format and are followed by digits of another international numbering plan.

Note 3 – Prefixes do not belong to the international X.121 format.

## E.7	numbering plan

In the context of the international numbering plan for public data networks, the specification given in Recommendation X.121.

Note – Other international numbering plans are contained in Recommendations E.164 and F.69.

## E.8	numbering plan interworking

In the context of the international numbering plan for public data networks, the methods to establish interworking between networks applying different international numbering plans.

Note – Examples of numbering plan interworking are given in Recommendations X.122, E.166 and I.332.

## E.9	prefix

In the context of the international numbering plan for public data networks, an indicator consisting of one or more digits, allowing the selection of different numbering formats. Prefixes are not part of the international X.121 format.

Note – Prefixes are a national matter.

ANNEX  F

(to Recommendation X.121)

Alphabetical list of abbreviations used
in this Recommendation

CC	Country code

DCC	Data country code

DCE	Data circuit-terminating equipment

DNIC	Data network identification code

DTE	Data terminal equipment

ISDN	Integrated services digital network

NN	National number

NTN	Network terminal number

PDN	Public data network

PNIC	Private data network identification code

PSPDN	Packet switched public data network

PSTN	Public switched telephone network

RPOA	Recognized private operating agency

TCC	Telephone country code

TDC	telex destination code

1)	In this Recommendation, the word “country” means country or geographical area.