|  | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

ITU-T	X.281

TELECOMMUNICATION	(11/95)
STANDARDIZATION  SECTOR
OF  ITU

DATA  NETWORKS  AND  OPEN  SYSTEM
COMMUNICATIONS

OPEN  SYSTEMS  INTERCONNECTION  –
LAYER  MANAGED  OBJECTS

INFORMATION  TECHNOLOGY  –
ELEMENTS  OF  MANAGEMENT  INFORMATION  RELATED  TO  THE  OSI  PHYSICAL  LAYER

ITU-T  Recommendation  X.281

(Previously “CCITT  Recommendation”)

FOREWORD

ITU (International Telecommunication Union) is the United Nations Specialized Agency in the field of telecommunications. The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of the ITU. Some 179 member countries, 84 telecom operating entities, 145 scientific and industrial organizations and 38 international organizations participate in ITU-T which is the body which sets world telecommunications standards (Recommendations).

The approval of Recommendations by the Members of ITU-T is covered by the procedure laid down in WTSC Resolution No. 1 (Helsinki, 1993). In addition, the World Telecommunication Standardization Conference (WTSC), which meets every four years, approves Recommendations submitted to it and establishes the study programme for the following period.

In some areas of information technology which fall within ITU-T’s purview, the necessary standards are prepared on a collaborative basis with ISO and IEC. The text of ITU-T Recommendation X.281 was approved on 21st of November 1995. The identical text is also published as ISO/IEC International Standard 13642.

___________________

NOTE

In this Recommendation, the expression “Administration” is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

ã  ITU  1996

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

ITU-T  X-SERIES  RECOMMENDATIONS

DATA  NETWORKS  AND  OPEN  SYSTEM  COMMUNICATIONS

(February 1994)

ORGANIZATION  OF  X-SERIES  RECOMMENDATIONS

| Subject area | Recommendation Series |
| --- | --- |
| PUBLIC DATA NETWORKS |  |
| Services and Facilities | X.1-X.19 |
| Interfaces | X.20-X.49 |
| Transmission, Signalling and Switching | X.50-X.89 |
| Network Aspects | X.90-X.149 |
| Maintenance | X.150-X.179 |
| Administrative Arrangements | X.180-X.199 |
| OPEN SYSTEMS INTERCONNECTION |  |
| Model and Notation | X.200-X.209 |
| Service Definitions | X.210-X.219 |
| Connection-mode Protocol Specifications | X.220-X.229 |
| Connectionless-mode Protocol Specifications | X.230-X.239 |
| PICS Proformas | X.240-X.259 |
| Protocol Identification | X.260-X.269 |
| Security Protocols | X.270-X.279 |
| Layer Managed Objects | X.280-X.289 |
| Conformance Testing | X.290-X.299 |
| INTERWORKING BETWEEN NETWORKS |  |
| General | X.300-X.349 |
| Mobile Data Transmission Systems | X.350-X.369 |
| Management | X.370-X.399 |
| MESSAGE HANDLING SYSTEMS | X.400-X.499 |
| DIRECTORY | X.500-X.599 |
| OSI NETWORKING AND SYSTEM ASPECTS |  |
| Networking | X.600-X.649 |
| Naming, Addressing and Registration | X.650-X.679 |
| Abstract Syntax Notation One (ASN.1) | X.680-X.699 |
| OSI MANAGEMENT | X.700-X.799 |
| SECURITY | X.800-X.849 |
| OSI APPLICATIONS |  |
| Commitment, Concurrency and Recovery | X.850-X.859 |
| Transaction Processing | X.860-X.879 |
| Remote Operations | X.880-X.899 |
| OPEN DISTRIBUTED PROCESSING | X.900-X.999 |

CONTENTS

Page

ITU-T Rec. X.281 (1995 E)

1	Scope		1

2	Normative references		1

2.1	Identical Recommendations | International Standards		1

2.2	Paired Recommendations | International Standards equivalent in technical content		2

3	Definitions		2

3.1	Basic Reference Model definitions		2

3.2	Management framework definitions		2

3.3	Systems management overview definitions		3

3.4	Common management information service definitions		3

3.5	Management information model definitions		3

3.6	GDMO definitions		3

4	Symbols and abbreviations		3

5	Elements of Physical Layer Management Information		4

5.1	Managed Object Hierarchy		4

5.2	Common Physical Layer GDMO definitions		5

5.3	The Physical Layer Subsystem Managed Object		6

5.4	Physical Entity Managed Object		7

5.5	Physical Service Access Point Managed Object		7

5.6	Data-circuit Managed Object		8

5.7	Physical Connection Managed Object		11

6	ASN.1 Module		12

6.1	Object identifier definitions		12

6.2	Other definitions		12

7	Conformance		13

7.1	Conformance requirements to this Recommendation | International Standard		13

Annex  A  –  Allocation of Object Identifiers		14

Annex  B – Shorthand description of Managed Objects		15

Annex  C – Example of the use of relationship attributes		17

Summary

This Recommendation | International Standard provides the specification of management information related to the Physical Layer. This includes the managed objects class definition of Physical Layer managed objects, the relationship of the managed objects and attributes to both the operation of the layer and to other objects and attributes of the layer, and the allowable actions on the attributes of Physical Layer managed objects.

INTERNATIONAL  STANDARD

ISO/IEC 13642 : 1996 (E)

ITU-T Rec. X.281 (1995 E)

ITU-T  RECOMMENDATION

# INFORMATION  TECHNOLOGY  –  ELEMENTS  OF  MANAGEMENT
INFORMATION  RELATED  TO  THE  OSI  PHYSICAL  LAYER

# 1	Scope

This Recommendation | International Standard provides the specification of management information within an Open System related to those operations of the OSI Physical Layer specified by the specifications in this Recommendation | International Standard. Specifics on how Physical Layer management is accomplished is beyond the scope of this Recommendation | International Standard. Physical Layer management is defined by specifying:

–	the managed object class definition of Physical Layer Managed Objects following guidelines put forth by the Structure of Management Information (see ITU-T Recommendations X.720 to X.723 and ISO/IEC 10165);

–	the relationship of the Managed Objects and attributes to both the operation of the layer and to other objects and attributes of the layer; and

–	the action type operations on the attributes of Physical Layer Managed Objects that are available to OSI Systems Management.

# 2	Normative references

The following Recommendations and International Standards contain provisions which, through reference in this text, constitute provisions of this Recommendation | International Standard. At the time of publication, the editions indicated were valid. All Recommendations and Standards are subject to revision, and parties to agreements based on this Recommendation | International Standard are encouraged to investigate the possibility of applying the most recent edition of the Recommendations and Standards listed below. Members of IEC and ISO maintain registers of currently valid International standards. The Telecommunication Standardization Bureau of the ITU maintains a list of currently valid ITU-T Recommendations.

## 2.1	Identical Recommendations | International Standards

–	ITU-T Recommendation X.200 (1994) | ISO/IEC 7498-1:1994, Information technology – Open Systems Interconnection – Basic Reference Model: The Basic Model.

–	ITU-T Recommendation X.211 (1995) | ISO/IEC 10022:1996, Information technology – Open Systems Interconnection – Physical service definition.

–	CCITT Recommendation X.701 (1992) | ISO/IEC 10040:1992, Information technology – Open Systems Interconnection – Systems management overview.

–	CCITT Recommendation X.720 (1992) | ISO/IEC 10165-1:1993, Information technology – Open Systems Interconnection – Structure of management information: Management Information Model.

–	CCITT Recommendation X.721 (1992) | ISO/IEC 10165-2:1992, Information technology – Open Systems Interconnection – Structure of management information: Definition of management information.

–	CCITT Recommendation X.722 (1992) | ISO/IEC 10165-4:1992, Information technology – Open Systems Interconnection – Structure of management information: Guidelines for the definition of managed objects.

–	ITU-T Recommendation X.723 (1993) | ISO/IEC 10165-5:1994, Information technology – Open Systems Interconnection – Structure of management information: Generic management information.

–	CCITT Recommendation X.730 (1992) | ISO/IEC 10164-1:1993, Information technology – Open Systems Interconnection – Systems management: Object management function.

–	CCITT Recommendation X.731 (1992) | ISO/IEC 10164-2:1993, Information technology – Open Systems Interconnection – Systems management: State management function.

–	CCITT Recommendation X.732 (1992) | ISO/IEC 10164-3:1993, Information technology – Open Systems Interconnection – Systems management: Attributes for representing relationships.

–	CCITT Recommendation X.733 (1992) | ISO/IEC 10164-4:1992, Information technology – Open Systems Interconnection – Systems management: Alarm reporting function.

–	CCITT Recommendation X.734 (1992) | ISO/IEC 10164-5:1993, Information technology – Open Systems Interconnection – Systems management: Event report management function.

–	CCITT Recommendation X.735 (1992) | ISO/IEC 10164-6:1993, Information technology – Open Systems Interconnection – Systems management: Log control function.

## 2.2	Paired Recommendations | International Standards equivalent in technical content

–	CCITT Recommendation X.208 (1988), Specification of Abstract Syntax Notation One (ASN.1).

ISO | IEC 8824:1990, Information technology – Open Systems Interconnection – Specification of Abstract Syntax Notation One (ASN.1).

–	CCITT Recommendation X.700 (1992), Management framework for Open Systems Interconnections (OSI) for CCITT applications.

ISO 7498-4:1989, Information processing systems – Open Systems Interconnection – Basic Reference Model – Part 4: Management framework.

–	CCITT Recommendation X.710 (1991), Common management information service definition for CCITT applications.

ISO/IEC 9595:1991, Information technology – Open Systems Interconnection – Common management information service definition.

–	CCITT Recommendation X.711 (1991), Common management information protocol specification for CCITT applications.

ISO/IEC 9596-1:1991, Information technology – Open Systems Interconnection – Common management information protocol – Part 1:  Specification.

# 3	Definitions

For the purposes of this Recommendation | International Standard, the following definitions apply.

## 3.1	Basic Reference Model definitions

This Recommendation | International Standard is based on the concept in the Basic Reference Model for Open Systems Interconnection and makes use of the following terms defined in ITU-T Rec. X.200 | ISO/IEC 7498-1:

a)	data-circuit;

b)	(N)-connection;

c)	(N)-entity;

d)	(N)-layer;

e)	(N)-protocol;

f)	(N)-service-access-point;

g)	open system;

h)	Physical Layer;

i)	systems management.

## 3.2	Management framework definitions

This Recommendation | International Standard makes use of the following term defined in CCITT Rec. X.700 | ISO/IEC 7498-4:

–	managed object.

## 3.3	Systems management overview definitions

This Recommendation | International Standard makes use of the following terms defined in CCITT Rec. X.701 | ISO/IEC 10040:

a)	managed object class;

b)	management information;

c)	notification;

d)	(systems management) operation.

## 3.4	Common management information service definitions

This Recommendation | International Standard makes use of the following term defined in CCITT Rec. X.710 | ISO/IEC 9595:

–	attributes.

## 3.5	Management information model definitions

This Recommendation | International Standard makes use of the following terms defined in CCITT Rec. X.720 | ISO/IEC 10165-1:

a)	action;

b)	attribute group;

c)	attribute Type;

d)	behaviour;

e)	containment;

f)	distinguished name;

g)	inheritance;

h)	name binding;

i)	package;

j)	parameter;

k)	relative distinguished name;

l)	subclass;

m)	superclass.

## 3.6	GDMO definitions

This Recommendation | International Standard makes use of the following terms defined in CCITT Rec. X.722 | ISO/IEC 10165-4:

a)	managed object class definition;

b)	template.

# 4	Symbols and abbreviations

Within the Managed Object definitions and GDMO templates the following abbreviations are used in the standard-name element of a document-identifier when making references to other documents:

DMI	CCITT Rec. X.721 (1992) | ISO/IEC 10165-2:1992

GMI	ITU-T Rec. X.723 (1993) | ISO/IEC 10165-5:1994

This Recommendation | International Standard makes use of the following symbols and abbreviations:

DMI	Definition of Management Information

GDMO	Guidelines for the Definition of Managed Objects

MO	Managed Object(s)

Ph	Physical

PhC	Physical Connection

PHLM	Physical Layer Management

PhSAP	Physical Service Access Point

QOS	Quality of Service

SAP	Service Access Point

# 5	Elements of Physical Layer Management Information

## 5.1	Managed Object Hierarchy

### 5.1.1	Summary of managed objects

The following set of common managed objects are defined in this Recommendation | International Standard for the OSI Physical Layer:

a)	The Physical Subsystem Managed Object (see 5.3)

b)	The Physical Entity Managed Object (see 5.4)

c)	The Physical SAP Managed Object (see 5.5)

d)	The Data-circuit Managed Object (see 5.6)

e)	The Physical Connection Managed Object (see 5.7)

These Managed Objects represent OSI Management's view of those elements of an Open System which support the OSI Physical Service subject to OSI management operations. Other MOs may be defined under Physical Subsystem using these generic specifications.

### 5.1.2	Containment hierarchy

The containment hierarchy is illustrated in Figure 1. These objects are defined in detail in the following subclauses.

The physical subsystem MO is subordinate to the system MO and it represents the whole of the physical layer in an open system. Only one instance of the physical subsystem MO exists in an open system. One or more physical entity MO and physical SAP MO are contained in the physical subsystem MO. The physical entity MO represents the whole of the management aspect of the entity that executes the functions of the physical layer. The plural instances of the physical entity MO can exist in an open system. The data-circuit MO is generic managed object that represents the management aspect of the entity that executes the transmission of bits. The physical connection MO represents the management view of connections established using relaying functions.

FIGURE 1/X.281...[D01] = 10 CM

### 5.1.3	Relationships

#### 5.1.3.1	General

The use of Relationship attributes is illustrated by examples in Annex C. The following describes the individual relationship in more detail.

#### 5.1.3.2	Physical entity

The physical entity MO has a localSapName attribute inherited from GMI:communicationsEntity. This attribute contains the local distinguished name(s) of SAP MO(s), representing the point at which services are provided to the entity.

#### 5.1.3.3	SAP

There is a relationship between the physical SAP MO and the managed objects that represent the Data Link user entities that are using the SAP. This is represented by the userEntityNames attribute inherited from GMI:sap1.

#### 5.1.3.4	Connections

There is a relationship between the physical connection MO and the managed objects that represents the Data Link connections.

### 5.1.4	Minimum Event Filtering Capabilities

The physical layer management definitions embodied in this Recommendation | International Standard imply the frequent, and possibly excessive generation of notifications during regular layer operation. These notifications are especially useful for effective fault management, where they facilitate the tracing and pinpointing of error situations. To avoid the excessive dissemination of these event reports under normal operating conditions, it is advisable for a managed system to have, as a minimum, the capability to perform discrimination based upon:

a)	the source managed object class;

b)	the object identifier values in the probable cause and specific problems field of communication alarms.

### 5.1.5	Use of Optional Fields

Where reference is made in this Recommendation | International Standard to ASN.1 syntax defined in ITU-T Rec. X.723 | ISO/IEC 10165-5 or CCITT Rec. X.721 | ISO/IEC 10165-2, only the following fields shall be employed:

a)	Those which are not OPTIONAL in the ASN.1 syntax.

b)	Those which are OPTIONAL, but whose use is explicitly required by this Recommendation | International Standard.

c)	Those which are OPTIONAL, but whose ASN.1 type is SET OF ManagementExtension.

The use of any other field is prohibited.

## 5.2	Common Physical Layer GDMO definitions

--  Behaviours

commonCreationDeletion-B	BEHAVIOUR

DEFINED AS

Managed object class imports the X.721 | ISO/IEC 10165-2 objectCreation and/or

objectDeletion notifications. Used as follows:

objectCreation - Generated whenever an instance of the managed object

class is created. Implementations may optionally include the

sourceIndicator parameter in the notification.

If creation occurred as a result of internal operation of the

resource, the value 'resourceOperation' is used. If creation occurred

in response to a management operation, the value 'managementOperation'

is used. A value of 'unknown' may be returned if it is not possible

to determine the source of the operation. None of the other optional

parameters are used.

objectDeletion - Generated whenever an instance of the managed object

class is deleted. Implementations may optionally include the

sourceIndicator parameter in the notification.

If deletion occurred as a result of internal operation of the

resource, the value 'resourceOperation' is used. If deletion occurred

in response to a management operation, the value 'managementOperation'

is used. A value of 'unknown' may be returned if it is not possible

to determine the source of the operation. None of the other optional

parameters are used.

commonDeactivateConnection-B	BEHAVIOUR

DEFINED AS

Managed object class imports the X.723 | ISO/IEC 10165-5 deactivate action.

The deactivate action causes the connection to be terminated. The

termination should occur as rapidly as practical, but no particular

time constraints are implied. Typically, this action simulates a

disconnect request received across the service interface. If a more

rapid means for terminating the connection exists, then this should be

used. The termination shall occur in conformance to the protocol

standard. The Managed Object remains in existence after completion of

the deactivate action. It is subsequently deleted when the connection

is terminated, in the same way as if the connection had been terminated

by other means. A deactivate action may fail (with the ProcessingFailure

response) if it is temporarily not possible to terminate the connection;

commonStateChange-B	BEHAVIOUR

DEFINED AS

Managed object class imports the X.721 | ISO/IEC 10165-2 stateChange notification.

Used to report the changes to the operationalState attribute, and where

present, the administrativeState attribute.  A single parameter set is

included in the State change definition field. Only the (mandatory)

attributeId and (optional) newAttributeValue parameters are used;

communicationsAlarm-B	BEHAVIOUR

DEFINED AS

Managed object class imports the X.721 | ISO/IEC 10165-2 communicationsAlarm

notification.  Used to report the alarm principally associated with

procedures and/or processes required to convey information from one

point to another.

The probableCause parameter is set to the value lossOfSignal.

The perceivedSeverity parameter is set to the value Critical.

equipmentAlarm-B	BEHAVIOUR

DEFINED AS

Managed object class imports the X.721 | ISO/IEC 10165-2 equipmentAlarm

notification.  Used to report the alarm principally associated with

an equipment fault.

The probableCause parameter is set to the value datasetOrModemError.

The perceivedSeverity parameter is set to the value Major.

qualityofServiceAlarm-B	BEHAVIOUR

DEFINED AS

Managed object class imports the X.721 | ISO/IEC 10165-2 qualityofServiceAlarm

notification.  Used to report the alarm principally associated with

a degradation in the quality of a service.

The probableCause parameter is set to the value thresholdCrossed.

The perceivedSeverity parameter is set to the value Warning.

## 5.3	The Physical Layer Subsystem Managed Object

--  There shall be exactly one of these managed objects within a system.

--  It exists to provide a container for all managed objects in a system

--  that relate to the operation of the Physical layer.

--  The physicalSubsystem managed object cannot be created or deleted

--  explicitly by management operation. It exists inherently in a system;

--  created and deleted as part of system operation.

physicalSubsystem	MANAGED OBJECT CLASS

DERIVED FROM "GMI":subsystem;

--  which is derived from "DMI":top

CHARACTERIZED BY physicalSubsystem-P  PACKAGE

ATTRIBUTES

"GMI":subsystemId

INITIAL VALUE PhLM.physicalSubsystemMONameValue

GET;;;

REGISTERED AS{PHLM.moi physicalSubsystem(1)};

--  Name Bindings

physicalSubsystem-system 	NAME BINDING

SUBORDINATE OBJECT CLASS physicalSubsystem AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS "DMI":system AND SUBCLASSES;

WITH ATTRIBUTE "GMI":subsystemId;

REGISTERED AS {PHLM.nboi physicalSubsystem-system(1)};

## 5.4	Physical Entity Managed Object

--  There may be multiple instances of these MOs within a system.

--  These managed objects can not be created or deleted explicitly by

--  management operation.  They exist inherently in a system;

--  created and deleted as part of system operation.

physicalEntity	MANAGED OBJECT CLASS

DERIVED FROM "GMI":communicationsEntity;

CHARACTERIZED BY physicalEntity-P  PACKAGE

ATTRIBUTES

physicalEntityTitles REPLACE-WITH-DEFAULT  GET-REPLACE;

ATTRIBUTE GROUPS

"DMI":state,

"DMI":operationalState;

NOTIFICATIONS

"DMI":stateChange;

REGISTERED AS {PHLM.moi physicalEntity(2)};

--  Name Bindings

-- IMPORT"GMI":communicationsEntity-subsystem NAME BINDING

physicalEntity-physicalSubsystem-Management	NAME BINDING

SUBORDINATE OBJECT CLASS physicalEntity AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS physicalSubsystem AND SUBCLASSES;

WITH ATTRIBUTE "GMI":communicationsEntityId;

REGISTERED AS {PHLM.nboi physicalEntity-physicalSubsystem(2)};

-- Attributes

physicalEntityTitles	ATTRIBUTE

WITH ATTRIBUTES SYNTAX PHLM.PhysicalEntityTitle;

MATCHES FOR EQUALITY;

DEFINED AS The set of Physical Entity Titles which unambiguously

identify the Physical Entity in an End or Intermediate System.

The value may be entered by a system management operation or

it may be derived by some local means.

REGISTERED AS {PHLM.aoi physicalEntityTitles (1)};

## 5.5	Physical Service Access Point Managed Object

--  Physical SAP MO class is used to represent a service access point where the

--  interactions between the physical service user and the physical service

--  provider take place.

--  phSAP managed objects are created automatically as part of system

--  operation when a Physical layer user requests and is granted use

--  of the Physical layer services. The mechanism by which this happens

--  is system-specific and not subject to OSI standardization.

physicalSAP MANAGED OBJECT CLASS

DERIVED FROM "GMI":sap1;

REGISTERED AS {PHLM.moi physicalSAP(3)};

--  Name Bindings

--  IMPORT "GMI":sap1-Subsystem NAME BINDING

phSAP-physicalSubsystem	NAME BINDING

SUBORDINATE OBJECT CLASS phSAP AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS physicalSubsystem AND SUBCLASSES;

WITH ATTRIBUTE "GMI":sapId;

BEHAVIOUR  phSAP-physicalSubsystem-Management-B BEHAVIOUR

DEFINED AS

The name binding which applies when a phSAP managed object (or an

instance of a subclass of the phSAP MO class) can be created by

management as a subordinate object of a phEntity managed object

(or subclass), and deleted by management.;;

CREATE;

DELETE ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS {PHLM.nboi phSAP-physicalSubsystem(3)};

## 5.6	Data-circuit Managed Object

--  The data-circuit managed object class is used to represent a communication

--  path in the physical media for OSI between two physical-entities,

--  together with the facilities necessary in the Physical Layer for the

--  transmission of bits on it.

dataCircuit	MANAGED OBJECT CLASS

DERIVED FROM "GMI":coProtocolMachine;

CHARACTERIZED BY dataCircuit-P  PACKAGE

BEHAVIOUR

commonCreationDeletion-B,

commonStateChange-B;

ATTRIBUTES

bitErrorThreshold	GET REPLACE,

dataCircuitType	GET,

physicalMediaNames	GET,

physicalInterfaceType	GET,

physicalInterfaceStandard	GET;

synchronizationMode	GET,

sync, async

transmissionCoding	GET,

coding scheme

transmissionMode	GET,

FDX, HDX, simplex, duplex

transmissionRate	GET,

ATTRIBUTE GROUPS

"DMI":counters

bitErrorsReceived

bitErrorsTransmitted,

"DMI":operationalState,

"DMI":state;

NOTIFICATIONS

"DMI":communicationAlarm

connectionError-PAR

lossOfSignal-PAR

lossOfSynchronization-PAR,

"DMI":communicationInformation

connectionEstablished-PAR,

"DMI":equipmentAlarm,

"DMI":objectCreation,

"DMI":objectDeletion,

"DMI":qualityofServiceAlarm

bitErrorThresholdReached-PAR,

"DMI":stateChange;

;;

REGISTERED AS {PHLM.moi dataCircuit(4)};

-- Name Bindings

dataCircuit-physicalEntity	NAME BINDING

SUBORDINATE OBJECT CLASS dataCircuit AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS physicalEntity AND SUBCLASSES;

WITH ATTRIBUTE "GMI":coProtocolMachineID;

BEHAVIOUR

DEFINED AS

The name binding which applies when a dataCircuit managed object

can be created by management as a subordinate object of  a

physicalEntity managed object and deleted by management.

DELETE ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS {PHLM.nboi dataCircuit-physicalEntity(4)};

-- Attributes

bitErrorsThreshold	ATTRIBUTE

WITH ATTRIBUTE SYNTAX PHLM.bitErrorsThreshold;

MATCHES FOR EQUALITY;

BEHAVIOUR

DEFINED AS

Value of the threshold for bit Error.;;

REGISTERED AS {PHLM.aoi bitErrorsThreshold(2)};

dataCircuitType	ATTRIBUTE

WITH ATTRIBUTE SYNTAX PHLM.dataCircuitType

MATCHES FOR EQUALITY;

BEHAVIOUR

DEFINED AS

Determines the data-circuit supports the relaying functions.;;

REGISTERED AS {PHLM.aoi dataCircuitType(3)};

physicalInterfaceStandard	ATTRIBUTE

WITH ATTRIBUTE SYNTAX PHLM.physicalInterfaceStandard

MATCHES FOR EQUALITY;

BEHAVIOUR

DEFINED AS

Determines the Standard or Recommendation number/name that the

interface comply to.;;

REGISTERED AS {PHLM.aoi physicalInterfaceStandard(4)};

physicalInterfaceType	ATTRIBUTE

WITH ATTRIBUTE SYNTAX PHLM.physicalInterfaceType

MATCHES FOR EQUALITY;

BEHAVIOUR

DEFINED AS

Determines the physical characteristics of the line/cable that

the data-circuit supports. ;;

REGISTERED AS {PHLM.aoi physicalInterfaceType(5)};

physicalMediaNames	ATTRIBUTE

WITH ATTRIBUTE SYNTAX PHLM.physicalMediaNames

MATCHES FOR EQUALITY;

BEHAVIOUR

DEFINED AS  The distinguished names of physical media which the

data-circuit uses.;;

REGISTERED AS {PHLM.aoi physicalMediaNames(6)};

synchronizationMode	ATTRIBUTE

WITH ATTRIBUTE SYNTAX PHLM.SynchronizationMode

MATCHES FOR EQUALITY;

BEHAVIOUR

DEFINED AS

Determines the mode of synchronization, such as synchronous and

asynchronous.;;

REGISTERED AS {PHLM.aoi synchronizationMode(7)};

transmissionCoding	ATTRIBUTE

WITH ATTRIBUTE SYNTAX PHLM.transmissionCoding;

MATCHES FOR EQUALITY;

BEHAVIOUR

DEFINED AS

Coding rule of data transmission.;;

REGISTERED AS {PHLM.aoi transmissionCoding(8)};

transmissionMode	ATTRIBUTE

WITH ATTRIBUTE SYNTAX PHLM.TransmissionMode

MATCHES FOR EQUALITY;

BEHAVIOUR

DEFINED AS

Determines the transmission mode.;;

REGISTERED AS {PHLM.aoi transmissionMode(9)};

transmissionRate	ATTRIBUTE

WITH ATTRIBUTE SYNTAX PHLM.TransmissionRate

MATCHES FOR EQUALITY;

BEHAVIOUR

DEFINED AS

Value of the transmission rate.;;

REGISTERED AS {PHLM.aoi transmissionRate(10)};

--  Parameters

bitErrorThresholdReached	PARAMETER

CONTEXT EVENT-INFO;

WITH SYNTAX PHLM.bitErrorThresholdReached;

BEHAVIOUR

bitErrorThresholdReached-B BEHAVIOUR

DEFINED AS

Indicates that bit error reached to the threshold value.;;

REGISTERED AS {PHLM.proi bitErrorThresholdReached(1)};

connectionError	PARAMETER

CONTEXT EVENT-INFO;

WITH SYNTAX PHLM.connectionError;

BEHAVIOUR

connectionError-B BEHAVIOUR

DEFINED AS

Indicates that connection error occurred.;;

REGISTERED AS {PHLM.proi connectionError(2)};

connectionEstablished	PARAMETER

CONTEXT EVENT-INFO;

WITH SYNTAX PHLM.connectionEstablished;

BEHAVIOUR

connectionEstablished-B BEHAVIOUR

DEFINED AS

Indicates that connection is established.;;

REGISTERED AS {PHLM.proi connectionEstablished(3)};

lossOfSignal	PARAMETER

CONTEXT EVENT-INFO;

WITH SYNTAX PHLM.lossOfSignal;

BEHAVIOUR

losOfSignal-B BEHAVIOUR

DEFINED AS

Indicates that loss of signal error occurred.;;

REGISTERED AS {PHLM.proi lossOfSignal(4)};

lossOfSynchronization	PARAMETER

CONTEXT EVENT-INFO;

WITH SYNTAX PHLM.lossOfSynchronization;

BEHAVIOUR

losOfSynchronization-B BEHAVIOUR

DEFINED AS

Indicates that loss of synchronization error occurred.;;

REGISTERED AS {PHLM.proi lossOfSynchronization(5)};

## 5.7	Physical Connection Managed Object

--  The Physical Connection managed Object class is used to represent an

--  interconnection of data-circuits in the Physical Layer.

--

physicalConnection	MANAGED OBJECT CLASS

DERIVED FROM "GMI":singlePeerConnection;

CHARACTERIZED BY phConnection-P PACKAGE

BEHAVIOUR

commonCreationDeletion-B,

commonDeactivateConnection-B,

commonStateChange-B;

ATTRIBUTES

"GMI":connectionId GET,

endpointIdendifier GET,

portNumber GET,

NOTIFICATIONS

"DMI":objectCreation,

"DMI":objectDeletion,

"DMI":stateChange,

ACTION

"GMI":activate,

"GMI":deactivate;

REGISTERED AS {PHLM.moi physicalConnection(5)};

--  Name Bindings

phConnection-dataCircuit	NAME BINDING

SUBORDINATE OBJECT CLASS phConnection AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS dataCircuit AND SUBCLASSES;

WITH ATTRIBUTE "GMI":connecitonId;

BEHAVIOUR

DEFINED AS

The name bindings which applies when a phConnection managed object

(or an instance of a subclass of the phConnection MO class) can be

changed by management as a subordinate object of phProtocol managed

object (or subclass), and deleted by management.;;

DELETE ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS {PHLM.nboi phConnection-dataCircuit(5)};

--  Attributes

endpointIdentifier	ATTRIBUTE

WITH ATTRIBUTE SYNTAX PHLM.endpointIdentifier;

MATCHES FOR EQUALITY;

BEHAVIOUR

DEFINED AS

The naming attribute for the physical connection managed object.;;

REGISTERED AS {PHLM.aoi endpointIdentifier(11)};

portNumber	ATTRIBUTE

WITH ATTRIBUTE SYNTAX PHLM.portNumber;

MATCHES FOR EQUALITY;

BEHAVIOUR

DEFINED AS  The distinguished names of port which the

connection is established.

REGISTERED AS {PHLM.aoi portNumber(12)};

# 6	ASN.1 Module

PHLM {joint-iso-itu physical-layer(18)management(0)asn1Module(2)0}

DEFINITIONS IMPLICIT TAGS ::= BEGIN

IMPORTS

GroupObjects

FROM Attribute.ASN1Module{joint-iso-itu ms(9)smi(3)part2(2)asn1Module(2)1}

SetInfoStatus,AttributeId,ObjectInstance

FROM CMIP-1{joint-iso-itu ms(9)cmip(1)modules(0)protocol(3)};

## 6.1	Object identifier definitions

physical-layer OBJECT IDENTIFIER ::= {joint-iso-itu physical-layer(18)}

phloi OBJECT IDENTIFIER ::= {physical-layer management(0)}

sseoi OBJECT IDENTIFIER ::= {phloi standardSpecificExtension(0)}

moi OBJECT IDENTIFIER ::= {phloi objectClass(3)}

poi OBJECT IDENTIFIER ::= {phloi package(4)}

proi OBJECT IDENTIFIER ::= {phloi parameter(5)}

nboi OBJECT IDENTIFIER ::= {phloi nameBinding(6)}

aoi OBJECT IDENTIFIER ::= {phloi attribute(7)}

agoi OBJECT IDENTIFIER ::= {phloi attributeGroup(8)}

acoi OBJECT IDENTIFIER ::= {phloi action(9)}

noi OBJECT IDENTIFIER ::= {phloi notification(10)}

## 6.2	Other definitions

physicalSubsystemId-Value Graphic String ::= "physicalSubsystem"

cRCErrorReceived ::= INTEGER

bitErrorThreshold ::= SEQUENCE {

mantissa [1] INTEGER,

exponent [2] INTEGER }

cRCErrorTransmitted ::= INTEGER

dataCircuitType ::= ENUMERATED {

end_system(0),

relay_system(1)}

physicalInterfaceStandard ::= OCTET STRING

physicalInterfaceType ::= OCTET STRING

physicalMediaNames ::= OCTET STRING

synchronizationMode ::= ENUMERATED {

synchronous(0),

asynchronous(1)}

transmissionCoding ::= OCTET STRING

transmissionMode ::= ENUMERATED {

simplex(0),

duplex(1)}

transmissionRate ::= SEQUENCE {

mantissa [1] INTEGER,

exponent [2] INTEGER }

portNumber ::= INTEGER

endpointIdentifier ::= OCTET STRING

connectionError ::= OCTET STRING

connectionEstablished ::= OCTET STRING

bitErrorThresholdReached ::= OCTET STRING

lossOfSignal ::= OCTET STRING

lossOfSynchronization ::= OCTET STRING

END

# 7	Conformance

## 7.1	Conformance requirements to this Recommendation | International Standard

An implementation for which conformance to this Recommendation | International Standard as a managed implementation is claimed shall:

a)	support the physicalSubsystem MO;

b)	for each supported MO, support at least on name binding defined in this Recommendation | International Standard, for which the MO is the subordinate;

c)	for each supported MO, support all elements of management information as defined in GDMO description for the MO.

Annex  A

Allocation of Object Identifiers

(This annex forms an integral part of this Recommendation | International Standard)

The following Object Identifiers have been allocated by the normative body of this standard. Object Identifiers which had been allocated when this Recommendation | International Standard was at the draft stage will not be re-allocated. If any modification, other than a change to the behaviour clause, is made to any template which has been allocated an object identifier, the new template will be allocated a new Object Identifier and the old Object Identifier will not be re-used.

joint-iso-itu

ms (9)

smi (3)

part2 (2)

asn1Module (2)

(1)

physical-layer (18)

management (0)

standardSpecificExtension (0)

asn1Module (2)

(0)

managedObjectClass (3)

physicalSubsystem (1)

physicalEntity (2)

physicalSAP (3)

dataCircuit (4)

physicalConnection (5)

package (4)

parameters (5)

bitErrorThresholdReached (1)

connectionError (2)

connectionEstablished (3)

lossOfSignal (4)

lossOfSynchronization (5)

nameBinding (6)

physicalSubsystem-system (1)

physicalEntity-physicalSubsystem (2)

physicalSAP-physicalSubsystem (3)

dataCircuit-physicalEntity (4)

phConnection-dataCircuit (5)

attribute (7)

physicalEntityTitles (1)

bitErrorsThreshold (2)

dataCircuitType (3)

physicalInterfaceStandard (4)

physicalInterfaceType (5)

physicalMediaNames (6)

synchronizationMode (7)

transmissionCoding (8)

transmissionMode (9)

transmissionRate (10)

endpointIdentifier (11)

portNumber (12)

attributeGroup (8)

action (9)

notification (10)

Annex  B

Shorthand description of Managed Objects

(This annex does not form an integral part of this Recommendation | International Standard)

The information in this annex is intended only to give a broad outline of the Physical Layer Management Specification. While the information contained herein has been derived from the GDMO text in this Recommendation | International Standard, it should be treated with caution, as there may be errors.

The following abbreviations are used to describe the property lists of attributes:

G	Get

R	Replace

RWD	Replace With Default

A	Add

RM	Remove

The following abbreviations are used for external label references:

DMI:	CCITT Rec. X.721 (1992) | ISO/IEC 10165-2:1992

GMI:	ITU-T Rec. X.723 (1993) | ISO/IEC 10165-5:1994

The inheritance hierarchy is illustrated in Figure B.1.

FIGURE 2/X.281...[D02] = 13 CM

MANAGED OBJECT CLASS physicalSubsystem DERIVED FROM (GMI:subsystem) CONTAINED IN (DMI:system)

GMI:subsystemId  ATTRIBUTE  (G)

END MANAGED OBJECT CLASS physicalSubsystem

MANAGED OBJECT CLASS physicalEntity DERIVED FROM (GMI:communicationsEntity) CONTAINED IN (physicalSubsystem)

DMI: stateChange  NOTIFICATION

physicalEntityTitles  ATTRIBUTE  (G, R, RWD)

The distinguished names of physical entity managed objects.

END MANAGED OBJECT CLASS physicalEntity

MANAGED OBJECT CLASS physicalSAP DERIVED FROM (GMI:sap1) CONTAINED IN (physicalSubsystem)

END MANAGED OBJECT CLASS physicalSAP

MANAGED OBJECT CLASS dataCircuit DERIVED FROM (GMI:coProtocolMachine) CONTAINED IN (physicalEntity)

DMI:communicationAlarm NOTIFICATIONS

DMI:communicationInformation  NOTIFICATIONS

DMI:equipmentAlarm  NOTIFICATIONS

DMI:objectCreation  NOTIFICATIONS

DMI:objectDeletion  NOTIFICATIONS

DMI:qualityofServiceAlarm  NOTIFICATIONS

DMI:stateChange  NOTIFICATIONS

bitErrorThreshold  ATTRIBUTE  (G, R)

Value of the threshold for bit error

dataCircuitType  ATTRIBUTE  (G)

Determines the data-circuit supports the relaying functions

physicalInterfaceStandard  ATTRIBUTE  (G)

The name of the Recommendation or International Standard that the interface complies to

physicalInterfaceType  ATTRIBUTE  (G)

The physical characteristics of the line/cable that the data circuit supports

physicalMediaNames  ATTRIBUTE  (G)

The name of the physical media that the data circuit is attached to

synchronizationMode  ATTRIBUTE  (G)

The type of synchronization

transmissionCoding  ATTRIBUTE  (G)

The coding rule of the data transmission

transmissionMode  ATTRIBUTE  (G)

Mode of transmission

transmissionRate  ATTRIBUTE  (G)

Value of the transmission rate

END MANAGED OBJECT CLASS dataCircuit

MANAGED OBJECT CLASS physicalConnection DERIVED FROM (GMI:singlePeerConnection) CONTAINED IN (dataCircuit)

DMI:objectCreation  NOTIFICATION

DMI:objectDeletion  NOTIFICATION

DMI:stateChange  NOTIFICATION

GMI:activate  ACTION

GMI:connectionId  ATTRIBUTE  (G)

GMI:deactivate  ACTION

endpointIdentifier  ATTRIBUTE  (G)

Name of the physical connection

portNumber  ATTRIBUTE  (G)

Name of the port that the connection is established

END MANAGED OBJECT CLASS physicalConnection

Annex  C

Example of the use of relationship attributes

(This annex does not form an integral part of this Recommendation | International Standard)

This annex provides examples of the use of relationship attributes, both within the Physical Layer and also between the adjoining layer. This example is not intended to be exhaustive. Relationships for other protocol combinations may be constructed in a similar manner, and a particular implementation may be capable of supporting multiple protocols simultaneously (see Figure C.1).

FIGURE 3/X.281...[D03] = 13 CM