|  | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

ITU-T	Amendment  1

TELECOMMUNICATION	X.284
STANDARDIZATION  SECTOR	
OF  ITU	(11/95)

DATA  NETWORKS  AND  OPEN  SYSTEM
COMMUNICATIONS

OPEN  SYSTEMS  INTERCONNECTION  –
LAYER  MANAGED  OBJECTS

ELEMENTS  OF  MANAGEMENT  INFORMATION
RELATED  TO  THE  OSI  TRANSPORT  LAYER

AMENDMENT  1:  NETWORK  CONNECTION
MANAGEMENT  SUBPROTOCOL  (NCMS)
MANAGEMENT

Amendment  1  to

ITU-T  Recommendation  X.284

(Previously “CCITT  Recommendation”)

FOREWORD

The ITU-T (Telecommunication Standardization Sector) is a permanent organ of the International Telecommunication Union (ITU). The ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Conference (WTSC), which meets every four years, establishes the topics for study by the ITU-T Study Groups which, in their turn, produce Recommendations on these topics.

The approval of Recommendations by the Members of the ITU-T is covered by the procedure laid down in WTSC Resolution No. 1 (Helsinki, March 1-12, 1993).

Amendment 1 to ITU-T Recommendation X.284, was prepared by ITU-T Study Group 7 (1993-1996) and was approved under the WTSC Resolution No. 1 procedure on the 21st of November 1995.

___________________

NOTE

In this Recommendation, the expression “Administration” is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

ã  ITU  1996

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

ITU-T  X-SERIES  RECOMMENDATIONS

DATA  NETWORKS  AND  OPEN  SYSTEM  COMMUNICATIONS

(February 1994)

ORGANIZATION  OF  X-SERIES  RECOMMENDATIONS

| Subject area | Recommendation Series |
| --- | --- |
| PUBLIC DATA NETWORKS |  |
| Services and Facilities | X.1-X.19 |
| Interfaces | X.20-X.49 |
| Transmission, Signalling and Switching | X.50-X.89 |
| Network Aspects | X.90-X.149 |
| Maintenance | X.150-X.179 |
| Administrative Arrangements | X.180-X.199 |
| OPEN SYSTEMS INTERCONNECTION |  |
| Model and Notation | X.200-X.209 |
| Service Definitions | X.210-X.219 |
| Connection-mode Protocol Specifications | X.220-X.229 |
| Connectionless-mode Protocol Specifications | X.230-X.239 |
| PICS Proformas | X.240-X.259 |
| Protocol Identification | X.260-X.269 |
| Security Protocols | X.270-X.279 |
| Layer Managed Objects | X.280-X.289 |
| Conformance Testing | X.290-X.299 |
| INTERWORKING BETWEEN NETWORKS |  |
| General | X.300-X.349 |
| Mobile Data Transmission Systems | X.350-X.369 |
| Management | X.370-X.399 |
| MESSAGE HANDLING SYSTEMS | X.400-X.499 |
| DIRECTORY | X.500-X.599 |
| OSI NETWORKING AND SYSTEM ASPECTS |  |
| Networking | X.600-X.649 |
| Naming, Addressing and Registration | X.650-X.679 |
| Abstract Syntax Notation One (ASN.1) | X.680-X.699 |
| OSI MANAGEMENT | X.700-X.799 |
| SECURITY | X.800-X.849 |
| OSI APPLICATIONS |  |
| Commitment, Concurrency and Recovery | X.850-X.859 |
| Transaction Processing | X.860-X.879 |
| Remote Operations | X.880-X.899 |
| OPEN DISTRIBUTED PROCESSING | X.900-X.999 |

CONTENTS

Recommendation X.284 / Amd.1     (11/95)

Page

1	Scope		1

2	Normative references		1

3	Definitions		1

4	Symbols and abbreviations		1

5	Elements of transport layer management information		1

6	ASN.1 module		8

Annex A – Allocation of Object Identifiers		8

Annex B – Shorthand description of managed objects		9

Annex C – Examples of the use of relationships		11

SUMMARY

This amendment describes the elements of management information relating to Network Connection Management Subprotocol (NCMS) specified in Annex B/X.224.

INTRODUCTION

Recommendation X.284 specifies the elements of management information related to the OSI Trasnport layer service and protocol described in Recommendations X.214 and X.224. However, the management information relating to the Network Connection Management Subprotocol (NCMS) is not included.

This amendment adds to Recommendation X.284 the elements of management information relating to NCMS specified in Annex B/X.224.

This amendment has a structure which is similar to that of Recommendation X.284 in order to facilite cross reference between the two documents and the eventual incorporation of this amendment into Recommendation X.284. In the following subclauses italic font is used for instructions for incorporating this amendment into Recommendation X.284.

Amendment 1  to Recommendation X.284

Recommendation X.284 / Amd.1     (11/95)

ELEMENTS  OF  MANAGEMENT  INFORMATION  RELATED
TO  THE  OSI  TRANSPORT  LAYER

AMENDMENT  1:  NETWORK  CONNECTION  MANAGEMENT
SUBPROTOCOL  (ncms)  MANAGEMENT1)

(Geneva, 1995)

# 1	Scope

No changes.

# 2	Normative references

No changes.

# 3	Definitions

No changes.

# 4	Symbols and abbreviations

Insert the following symbols and abbreviations after MO:

NC	Network Connection

NCC	Network Connection Control

NCMS	Network Connection Management Subprotocol

# 5	Elements of transport layer management informationXE "Elements of transport layer management information"§

## 5.1	Managed object hierarchyXE "Managed object hierarchy"§

### 5.1.1	Summary of managed objectsXE "Summary of managed objects"§

Add the following new items  after item g):

h)	NCMS Protocol Machine management object(ncmsPM, subclause 5.9);

i)	NetworkConnection Control managed object(ncc, subclause 5.10.1);

j)	NetworkConnection Control Initial Value managed object(nccIV, subclause 5.10.2).

Replace Figure 1 by the following figure:

FIGURE 1/X.284...[D01] = 3 CM  (118%)

Add the following new subclause 5.9:

## 5.9	NCMS Protocol MachineXE "NCMS Protocol Machine"§

ncmsPM MANAGED OBJECT CLASS

DERIVED FROM "DMI":top;

CHARACTERIZED BY ncmsPM-P PACKAGE

BEHAVIOUR

commonCreationDeletion-B,

commonStateChange-B,

ncmsPMPackageImportedNotifications-B,

ncmsPM-B BEHAVIOUR

DEFINED AS

!This managed object class represents the part of transport entity that performs the

NCMS protocol.

Only one instance of this managed object class may exist within a TEMO instance.!

;

;

ATTRIBUTES

ncmsPMId GET,

"DMI":administrativeState GET-REPLACE,

"DMI":operationalState GET;

ACTIONS

"GMI":activate,

"GMI":deactivate;

NOTIFICATIONS

"DMI":communicationsAlarm,

ncmsPMPDUHeader,

ncmsPMSourceAddress,

"DMI":objectCreation,

"DMI":objectDeletion,

"DMI":stateChange

;

;

REGISTERED AS{TLM.moi ncmsPM (8)};

-- Behaviours

ncmsPMPackageImportedNotifications-B BEHAVIOUR

DEFINED AS

!The ncmsPM-P package imports communicationsAlarm from DMI, in order to report the failure of NC sharing. The probableCause is set to

TLM.communicationsProtocolError. The ncmsPMPDUHeader and ncmsPMSourceAddress are reported as parameters in the additionalInformation field of the communicationsAlarm.

The significance subparameter of each item of the additionalInformation shall be set to the value 'False' (i.e. not significant) so that a managing system receiving the event will be less likely to reject it. The perceivedSeverity shall be set to Minor.

A subsequent communicationsAlarm with a perceived Severity value of 'Cleared' shall not be generated. No other fields or parameters shall be used, with the exception of further parameters in the additionalInformation field.!

;

-- Name Bindings

ncmsPM-transportEntity-Management NAME BINDING

SUBORDINATE OBJECT CLASS ncmsPM AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS transportEntity AND SUBCLASSES;

WITH ATTRIBUTE ncmsPMId;

BEHAVIOUR

ncmsPM-transportEntity-B BEHAVIOUR

DEFINED AS

!The name binding that applies when the ncmsPM managed object is explicitly created by management.!

;

;

CREATE;

DELETE ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS{TLM.nboi ncmsPM-transportEntity-Management (13)};

ncmsPM-transportEntity-Automatic NAME BINDING

SUBORDINATE OBJECT CLASS ncmsPM AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS transportEntity AND SUBCLASSES;

WITH ATTRIBUTE ncmsPMId;

BEHAVIOUR

ncmsPM-transportEntity-B BEHAVIOUR

DEFINED AS

!The name binding that applies when the ncmsPM managed object is created.

The name binding that applies when the ncmsPM managed object can not be explicitly created by management.!

;

;

REGISTERED AS{TLM.nboi ncmsPM-transportEntity-Automatic (14)};

-- Attribute

ncmsPMId ATTRIBUTE

WITH ATTRIBUTE SYNTAX ASN1DefinedTypesModule.NameSyntax;

MATCHES FOR EQUALITY;

BEHAVIOUR

ncmsPMId-B BEHAVIOUR

DEFINED AS

!The attribute that is used in naming instances of the ncms Protocol Machine managed object class.!

;

;

REGISTERED AS{TLM.aoi ncmsPMId (67)};

-- Parameters

ncmsPMPHeader PARAMETER

CONTEXT EVENT-INFO;

WITH SYNTAX TLM.PDUHeaderSyntax;

BEHAVIOUR ncmsPMPDUHeader-B BEHAVIOUR

DEFINED AS

!Header of the PDU that causes the failure of NC sharing.

Returned in the problemData field of a communicationsAlarm notification.

;

;

REGISTERD AS{TLM.proi ncmsPMPDUHeader (8)};

ncmsPMSourceAddress PARAMETER

CONTEXT EVENT-INFO;

WITH SYNTAX TLM.SourceAddressSyntax;

BEHAVIOUR ncmsPMSourceAddress-B BEHAVIOUR

DEFINED AS

!Source N-Address.

Returned in the problemData field of a communicationsAlarm notification.

;

;

REGISTERD AS{TLM.proi ncmsPMSourceAddress (9)};

Add the following  new subclause 5.10:

## 5.10	Network connection control MO and IVMOXE "Network connection control MO and IVMO"§

### 5.10.1	Network connection control managed objectXE "Network connection control managed object"§

nccMANAGED OBJECT CLASS

DERIVED FROM "DMI":top;

CHARACTERIZED BY ncc-P PACKAGE

BEHAVIOUR

nccInitialValues-B,

ncc-B BEHAVIOUR

DEFINED AS

!This managed object class represents the management aspect of the information needed to control the network connections by NCMS.

Multiple instances of this managed object class may exist within a NCMSPM MO instance. This MO is created and deleted as a result of NCMS operation.!

;

;

ATTRIBUTES

nccId GET,

nc-COL GET,

nc-REC GET,

nc-REF GET,

nc-PREF GET,

nc-Right GET,

ncRecoveries GET,

ttrNCTime GET,

tpdNCTime GET,

tfrNCTime GET,

sourceOfAllocation GET,

"GMI":underlyingConnectionNames GET;

NOTIFICATIONS

"DMI":objectCreation,

"DMI":objectDeletion;

;

;

REGISTERED AS{TLM.moi ncc (9)};

### 5.10.2	Network connection control initial value managed objectXE "Network connection control initial value managed object"§

nccIVMO MANAGED OBJECT CLASS

DERIVED FROM "DMI":top;

CHARACTERIZED BY nccIVMO-P PACKAGE

BEHAVIOUR

use-of-nccInitialValues-B,

nccIVMO-B BEHAVIOUR

DEFINED AS

!This managed object class represents the set of initial values for NCC MO instances.

Multiple instances of this managed object class may exist within a NCMSPM MO instance.

The relationship between instances of NCC MO and NCCIV MO is not specified in this Recommendation.!

;

;

ATTRIBUTES

nccIVMOId  GET,

nc-COL  REPLACE-WITH-DEFAULT GET-REPLACE,

nc-REC  REPLACE-WITH-DEFAULT GET-REPLACE,

nc-PREF  REPLACE-WITH-DEFAULT GET-REPLACE,

nc-Right  REPLACE-WITH-DEFAULT GET-REPLACE,

ttrNCTime  REPLACE-WITH-DEFAULT GET-REPLACE,

tpdNCTime  REPLACE-WITH-DEFAULT GET-REPLACE,

tfrNCTime   REPLACE-WITH-DEFAULT GET-REPLACE;

;

;

REGISTERED AS{TLM.moi nccIVMO(10)};

-- NCC Initial values behaviour

nccInitialValues-B BEHAVIOUR

DEFINED AS

!When an instance of the NCC MO is created using the ncc-ncmsPM name binding, the initial values for some of the attributes of the NCC MO may be supplied by an instance of the NCC IVMO. The means by which an instance(if any) of the NCC IVMO are identified are a local matter.!

;

-- Use of NCC initial values behaviour

use-of-nccInitialValues-B BEHAVIOUR

DEFINED AS

!The creation of an instance of the NCC MO using the ncc-ncmsPM name binding may reference an instance of NCC IVMO. When this occurs, some of the initial values of the attributes of the instance of NCC MO may be supplied by the values of the attributes in the specified instance of the NCC IVMO.

However any such value may be overridden by a value supplied by local means (for example across an internal interface). Where values are supplied by the IVMO, the initial values of an attribute of NCC MO shall be the value of the corresponding attribute in the NCC IVMO (that is, which has the same attribute template label).!

;

-- Name Binding

ncc-ncmsPM NAME BINDING

SUBORDINATE OBJECT CLASS ncc AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS ncmsPM AND SUBCLASSES;

WITH ATTRIBUTE nccId;

BEHAVIOUR

ncc-ncmsPM-B BEHAVIOUR

DEFINED AS

!The name binding that applies when the ncc managed object is created and deleted.!

;

;

CREATE WITH-REFERENCE-OBJECT;

DELETE;

REGISTERED AS{TLM.nboi ncc-ncmsPM (15)};

nccIVMO-ncmsPM NAME BINDING

SUBORDINATE OBJECT CLASS nccIV AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS ncmsPM AND SUBCLASSES;

WITH ATTRIBUTE nccIVMOId;

BEHAVIOUR

nccIVMO-ncmsPM-B BEHAVIOUR

DEFINED AS

!The name binding that applies when the nccIV managed object is created and deleted.!

;

;

CREATE WITH-REFERENCE-OBJECT;

DELETE;

REGISTERED AS{TLM.nboi nccIV-ncmsPM(16)};

--- Attribute

nccId ATTRIBUTE

WITH ATTRIBUTE SYNTAX ASN1DefinedTypesModule.NameType;

MATCHES FOR EQUALITY;

BEHAVIOUR

nccId-B BEHAVIOUR

DEFINED AS

!The attribute that is used in naming instances of the network connection control managed object class.!

;

;

REGISTERED AS{TLM.aoi nccId (68)};

nccIVMOId ATTRIBUTE

WITH ATTRIBUTE SYNTAX ASN1DefinedTypesModule.NameType;

MATCHES FOR EQUALITY;

BEHAVIOUR

nccIVMOId-B BEHAVIOUR

DEFINED AS

!The attribute that is used in naming instances of the network connection control initial value managed object class.!

;

;

REGISTERED AS{TLM.aoi nccIVMOId (69)};

nc-COL ATTRIBUTE

WITH ATTRIBUTE SYNTAX TLM.NC-COLSyntax;

MATCHES FOR EQUALITY;

BEHAVIOUR

nc-COL-B BEHAVIOUR

DEFINED AS

!The attribute that indicates the collision algorithm as defined in Annex B of ITU-T Rec. X.224 | ISO/IEC 8073. In NCCIV managed object, indicates the collision algorithm to be used. In NCC managed object, indicates the collision algorithm in use.!

;

;

REGISTERED AS{TLM.aoi nc-COL (70)};

nc-PREF ATTRIBUTE

WITH ATTRIBUTE SYNTAX TLM.NC-PREFSyntax;

MATCHES FOR EQUALITY;

BEHAVIOUR

nc-PREF-B BEHAVIOUR

DEFINED AS

!The attribute that indicates the preference the initiator has to keep the network connection as defined in Annex B of ITU-T Rec. X.224 | ISO/IEC 8073. In NCCIV managed object, indicates the preference to be used. In NCC managed object, indicates the preference in use.!

;

;

REGISTERED AS{TLM.aoi nc-PREF (71)};

nc-REC ATTRIBUTE

WITH ATTRIBUTE SYNTAX TLM.NC-RECSyntax;

MATCHES FOR EQUALITY;

BEHAVIOUR

nc-REC-B BEHAVIOUR

DEFINED AS

!The attribute that indicates the recovery optimization option as defined in Annex B of ITU-T Rec. X.224 | ISO/IEC 8073. In NCCIV managed object, indicates the recovery optimization option to be used. In NCC managed object, indicates the recovery optimization option in use.!

;

;

REGISTERED AS{TLM.aoi nc-REC (72)};

nc-REF ATTRIBUTE

WITH ATTRIBUTE SYNTAX TLM.NC-REFSyntax;

MATCHES FOR EQUALITY;

BEHAVIOUR

nc-REF-B BEHAVIOUR

DEFINED AS

!The attribute that indicates the nc-reference as defined in Annex B of ITU-T Rec. X.224 | ISO/IEC 8073.!

;

;

REGISTERED AS{TLM.aoi nc-REF (73)};

ncRecoveries ATTRIBUTE

DERIVED FROM "GMI":nonwrapping64BitCounter;

BEHAVIOUR

ncRecoveries-B BEHAVIOUR

DEFINED AS

!The attribute that indicates the total number of network connection successful recoveries.!

;

;

REGISTERED AS{TLM.aoi ncRecoveries (74)};

nc-Right ATTRIBUTE

WITH ATTRIBUTE SYNTAX TLM.NC-RightSyntax;

MATCHES FOR EQUALITY;

BEHAVIOUR

nc-Right-B BEHAVIOUR

DEFINED AS

!The attribute that indicates the type of right of use as defined in Annex B of ITU-T Rec. X.224 | ISO/IEC 8073. In NCCIV managed object, indicates the type of right of use to be used. Namely, the value "my-side" means "SA", "remote-side" means "RA" and "both-sides" means "RR". In NCC managed object, indicates the type of right of use in use.!

;

;

REGISTERED AS{TLM.aoi networkConnectionRight (75)};

sourceOfAllocation ATTRIBUTE

WITH ATTRIBUTE SYNTAX TLM.SourceOfAllocationSyntax;

MATCHES FOR EQUALITY;

BEHAVIOUR

sourceOfAllocation-B BEHAVIOUR

DEFINED AS

!The attribute that indicates the transport entity that established the network connection at the first time during the life time of an NC reference.!

;

;

REGISTERED AS{TLM.aoi sourceOfAllocation (76)};

tfrNCTime ATTRIBUTE

DERIVED FROM "GMI":timer;

BEHAVIOUR

tfrNCTime-B BEHAVIOUR

DEFINED AS

!Value of the TFR-NC timer as defined in Annex B of ITU-T Rec. X.224 | ISO/IEC 8073.!

;

;

REGISTERED AS{TLM.aoi tfrNCTime (77)};

tpdNCTime ATTRIBUTE

DERIVED FROM "GMI":timer;

BEHAVIOUR

tpdNCTime-B BEHAVIOUR

DEFINED AS

!Value of the TPD-NC timer as defined in Annex B of ITU-T Rec. X.224 | ISO/IEC 8073.!

;

;

REGISTERED AS{TLM.aoi tpdNCTime (78)};

ttrNCTime ATTRIBUTE

DERIVED FROM "GMI":timer;

BEHAVIOUR

ttrNCTime-B BEHAVIOUR

DEFINED AS

!Value of the TTR-NC timer as defined in Annex B of ITU-T Rec. X.224 | ISO/IEC 8073.!

;

;

REGISTERED AS{TLM.aoi ttrNCTime (79)};

# 6	ASN.1 module

## 6.2	Other definitions

Add the following definitions:

NC-COLSyntax::=ENUMERATED{nc-COL0(0)}

NC-PREFSyntax::=ENUMERATED{highest(0),

medium(1),

lowest(3)}

NC-RECSyntax::=ENUMERATED{pleaseDoNotRecover(0),

pleaseRecover(1)}

NC-REFSyntax::=INTEGER

NC-RightSyntax::=ENUMERATED{my-side(1),

remote-side(2),

both-sides(3)}

SourceOfAllocationSyntax:=ENUMERATED{local(0),

remote(1)}

Annex  A

Allocation of Object IdentifiersXE "Allocation of Object Identifiers"§

(This annex forms an integral part of this Recommendation)

Add the following Object Class Identifiers:

ncmsPM (8)

ncc (9)

nccIVMO (10)

Add the following parameter Identifiers:

ncmsPMPDUHeader (8)

ncmsPMSourceAddress(9)

Add the following name binding Identifiers:

ncmsPM-transportEntity-Automatic (14)

ncc-ncmsPM (15)

nccIVMO-ncmsPM (16)

Add the following attribute Identifiers:

ncmsPMId (67)

nccId (68)

nccIVMOId (69)

nc-COL (70)

nc-PREF (71)

nc-REC (72)

nc-REF (73)

ncRecoveries (74)

networkConnectionRight (75)

sourceOfAllocation (76)

tfrNCTime (77)

tpdNCTime (78)

ttrNCTime (79)

Annex  B

Shorthand description of managed objectsXE "Shorthand description of managed objects"§

(This annex does not form an integral part of this Recommendation)

Replace Figure B.1 by the following figure:

FIGURE B.1/X.254...[D02] = 3 CM  (118%)

Add the shorthand description of NCMS PM MO, NCC MO and NCCIV MO:

MANAGED OBJECT CLASS ncmsPM DERIVED FROM (DMI:top)

CONTAINED IN (transportEntity)

ncmsPMId ATTRIBUTE (G)

DMI:administrativeState ATTRIBUTE (G,R)

DMI:operationalState ATTRIBUTE (G)

GMI:activate ACTION

GMI:deactivate ACTION

DMI:communicationsAlarm NOTIFICATION

DMI:objectCreation NOTIFICATION

DMI:objectDeletion NOTIFICATION

DMI:stateChange NOTIFICATION

END MANAGED OBJECT CLASS ncmsPM

MANAGED OBJECT CLASS ncc DERIVED FROM (DMI:top)

CONTAINED IN (ncmsPM)

nccId ATTRIBUTE (G)

nc-COL ATTRIBUTE (G)

nc-REC ATTRIBUTE (G)

nc-REF ATTRIBUTE (G)

nc-PREF ATTRIBUTE (G)

nc-Right ATTRIBUTE (G)

ncRecoveries ATTRIBUTE (G)

ttrNCTime ATTRIBUTE (G)

tpdNCTime ATTRIBUTE (G)

tfrNCTime ATTRIBUTE (G)

sourceOfAllocation ATTRIBUTE (G)

GMI:underlyingConnectionName ATTRIBUTE (G)

DMI:objectCreation NOTIFICATION

DMI:objectDeletion NOTIFICATION

END MANAGED OBJECT CLASS ncc

MANAGED OBJECT CLASS nccIVMO DERIVED FROM (DMI:top)

CONTAINED IN (ncmsPM)

nccIVMOId ATTRIBUTE (G)

nc-COL ATTRIBUTE (G,R,RWD)

nc-REC ATTRIBUTE (G,R,RWD)

nc-REF ATTRIBUTE (G,R,RWD)

nc-PREF ATTRIBUTE (G,R,RWD)

nc-Right ATTRIBUTE (G,R,RWD)

ttrNCTime ATTRIBUTE (G,R,RWD)

tpdNCTime ATTRIBUTE (G,R,RWD)

tfrNCTime ATTRIBUTE (G,R,RWD)

END MANAGED OBJECT CLASS nccIVMO

Annex  C

Examples of the use of relationships

(This annex does not form an integral part of this Recommendation)

Add the following Figure C.3:

FIGURE C.3/X.284...[D03] = 3 CM  (118%)

1	)	This Recommendation was developed collaboratively with ISO/IEC and is technically aligned with ISO/IEC 10737/Amd.1, Information technology – Telecommunications and information exchange between systems – Elements of management information relating to OSI transport layer standards, Amd.1: NCMS management.