|  | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

CCITT	X.721

THE  INTERNATIONAL
TELEGRAPH  AND  TELEPHONE
CONSULTATIVE  COMMITTEE

DATA  COMMUNICATION  NETWORKS

INFORMATION  TECHNOLOGY -
OPEN  SYSTEMS  INTERCONNECTION  -

STRUCTURE  OF  MANAGEMENT  INFORMATION:  DEFINITION  OF  MANAGEMENT  INFORMATION

Recommendation  X.721

|  | Geneva, 1992 |
| --- | --- |

Foreword

“ITU (International Telecommunication Union) is the United Nations Specialized Agency in the field of telecommunications. The CCITT (the International Telegraph and Telephone Consultative Committee) is a permanent organ of the ITU. Some 166 member countries, 68 telecom operating entities, 163 scientific and industrial organizations and 39 international organizations participate in CCITT which is the body which sets world telecommunications standards (Recommendations).

The approval of Recommendations by the members of CCITT is covered by the procedure laid down in CCITT Resolution No. 2 (Melbourne, 1988). In addition, the Plenary Assembly of CCITT, which meets every four years, approves Recommendations submitted to it and establishes the study programme for the following period”.

In some areas of information technology which fall within CCITT’s purview, the necessary standards are prepared on a collaborative basis with ISO and IEC. The text of CCITT Recommendation X.721 was approved on 10th of February 1992. The identical text is also published as ISO/IEC International Standard 10165-2.

___________________

# CCITT NOTE

In this Recommendation, the expression “Administration” is used for conciseness to indicate both a telecommunication Administration and a recognized private operating agency.

ã  ITU  1992

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

Contents

µ1	Scope	1

2	Normative references	1

2.1	Identical CCITT Recommendations | International Standards	1

2.2	Paired CCITT Recommendations | International Standards equivalent in technical content	2

3	Definitions	2

3.1	Event report management function definitions	2

3.2	Management information model definitions	2

4	Abbreviations	3

5	Notation	3

6	Definition of managed object classes	3

6.1	Alarm record	3

6.2	Attribute value change record	4

6.3	Discriminator	5

6.4	Event forwarding discriminator	5

6.5	Event log record	6

6.6	Log	7

6.7	Log record	8

6.8	Object creation record	8

6.9	Object deletion record	8

6.10	Relationship change record	9

6.11	Security alarm report record	9

6.12	State change record	10

6.13	System	10

6.14	Top	11

7	Name binding for managed object classes	11

7.1	Discriminator	11

7.2	Log	11

7.3	Log record	12

8	Definition of packages	12

8.1	Additional information	12

8.2	Additional text	12

8.3	Attribute identifier list	12

8.4	Attribute list	13

8.5	Availability status	13

8.6	Correlated notifications	13

8.7	Notification identifier	13

8.8	Daily scheduling	13

8.9	Duration	14

8.10	External scheduler	14

8.11	Source indicator	14

8.12	Weekly scheduling	14

9	Definition of generic attribute types	14

9.1	Counter	15

9.2	Gauge	16

9.3	Threshold	17

9.3.1	Counter-threshold	17

9.3.2	Gauge-threshold	18

9.4	Tide-mark	18

10	Definition of specific attribute types	19

10.1	Attributes types used for naming	19

10.1.1	Discriminator Id	19

10.1.2	Log Id	20

10.1.3	Log record Id	20

10.1.4	System Id	20

10.1.5	System title	20

10.2	Counter	20

10.3	Counter-threshold	20

10.4	Gauge	21

10.5	Gauge-threshold	21

10.6	Tide-mark	21

10.7	Miscellaneous attribute types	21

10.7.1	Events related	21

10.7.2	States related	26

10.7.3	Relationships related	28

10.7.4	Other attribute types	29

11 Definition of action types	33

12 Definition of parameters	33

13	Definition of notification types	34

13.1	Attribute value change	34

13.2	Communications alarm	34

13.3	Environmental alarm	35

13.4	Equipment alarm	35

13.5	Integrity violation	36

13.6	Object creation	36

13.7	Object deletion	36

13.8	Operational violation	37

13.9	Physical violation	37

13.10	Processing error alarm	37

13.11	Quality of service alarm	38

13.12	Relationship change	38

13.13	Security service or mechanism violation	39

13.14	State change	39

13.15	Time domain violation	39

14	Supporting productions	40

14.1	Managed object class	40

14.2	Attribute types	40

14.3	Notification types	44

14.4	Parameter types	46

15	Conformance and compliance	46

15.1	Conformance	46

15.2	Compliance	46

Annex A  Counters and counter threshold attribute	47

A.1	Counter	47

A.1.1	Corrupted PDUs received counter	47

A.1.2	Incoming connection reject error counter	47

A.1.3	Incoming connection requests counter	47

A.1.4	Incoming disconnect counter	47

A.1.5	Incoming disconnect error counter	47

A.1.6	Incoming protocol error counter	48

A.1.7	Octets received counter	48

A.1.8	Octets retransmitted error counter	48

A.1.9	Octets sent counter	48

A.1.10	Outgoing connection reject error counter	48

A.1.11	Outgoing connection requests counter	48

A.1.12	Outgoing disconnect counter	48

A.1.13	Outgoing disconnect error counter	49

A.1.14	Outgoing protocol error counter	49

A.1.15	PDUs received counter	49

A.1.16	PDUs retransmitted error counter	49

A.1.17	PDUs sent counter	49

A.2	Counter-threshold	49

A.2.1	Corrupted PDUs received threshold	49

A.2.2	Incoming connection reject error threshold	50

A.2.3	Incoming connection requests threshold	50

A.2.4	Incoming disconnect error threshold	50

A.2.5	Incoming protocol error threshold	50

A.2.6	Octets received threshold	50

A.2.7	Octets retransmitted threshold	50

A.2.8	Octets sent threshold	51

A.2.9	Outgoing connection reject error threshold	51

A.2.10	Outgoing connection requests threshold	51

A.2.11	Outgoing disconnect error threshold	51

A.2.12	Outgoing protocol error threshold	51

A.2.13	PDUs received threshold	51

A.2.14	PDUs retransmitted error threshold	52

A.2.15	PDUs sent threshold	52

Annex B  Index of managed object classes	53

Annex C  Index of packages	54

Annex D  Index of generic and specific attribute types	55

Annex E  Index of notification types	57

Annex F  Management information used by Systems management function	58

F.1	Object management function (CCITT Rec. X.730 | ISO/IEC 10164-1)	58

F.2	State management function (CCITT Rec. X.731 | ISO/IEC 10164-2)	58

F.3	Relationship management function (CCITT Rec. X.732 | ISO/IEC 10164-3)	59

F.4	Alarm reporting function (CCITT Rec. X.733 | ISO/IEC 10164-4)	60

F.5	Event report management function (CCITT Rec. X.734 | ISO/IEC 10164-5)	60

F.6	Log control function (CCITT Rec. X.735 | ISO/IEC 10164-6)	61

F.7	Security alarm reporting function (CCITT Rec. X.736 | ISO/IEC 10164-7)	62

Annex G  Syntax imported from Directory, ACSE and CMIP	64

# ISO/IEC 10165-2 : 1992 (E)

# INFORMATION NOTE

The following table gives a list of X.700 Series Recommendations which were developed in collaboration with the ISO/IEC and are identical to the corresponding International Standard. Cross-references to the corresponding ISO/IEC International Standard number and the short title of the Recommendation | International Standard are provided.

| CCITT Recommendation ISO/IEC International Standard | Short Title |
| --- | --- |
| X.700 ¦ 7498-4 (Note) | Management Framework |
| X.701 ¦ 10040 | System Management Overview |
| X.710 ¦ 9595 | Common Management Information Service Definition |
| X.711 ¦ 9596-1 | Common Management Information Protocol Specification |
| X.712 ¦ 9596-2 | CMIP PICS |
| X.720 ¦ 10165-1 | Management Information Model |
| X.721 ¦ 10165-2 | Definition of Management Information |
| X.722 ¦ 10165-4 | Guidelines for the Definition of Managed Objects |
| X.730 ¦ 10164-1 | Object Management Function |
| X.731 ¦ 10164-2 | State Management Function |
| X.732 ¦ 10164-3 | Attributes for Representing Relationships |
| X.733 ¦ 10164-4 | Alarm Reporting Function |
| X.734 ¦ 10164-5 | Event Management Function |
| X.735 ¦ 10164-6 | Log Control Function |
| X.736 ¦ 10164-7 | Security Alarm Reporting Function |
| X.740 ¦ 10164-8 | Security Audit Trail Function |
| Note – This Recommendation and International Standard are not identical, but are technically aligned. |  |

INTERNATIONAL STANDARD

ISO/IEC 10165-2 : 1992 (E)

CCITT Rec. X.721 (1992 E)

CCITT RECOMMENDATION

# INFORMATION TECHNOLOGY — OPEN SYSTEMS INTERCONNECTION —
 STRUCTURE OF MANAGEMENT INFORMATION: DEFINITION OF MANAGEMENT INFORMATION

# 1	Scopetc  \l 1 "1	Scope"§

This Recommendation | International Standard

–	defines the managed object classes, attribute types, name bindings, packages, specific attributes, action types, parameter types and notification types documented in accordance with CCITT Rec. X.722 | ISO/IEC 10165-4;

–	specifies compliance requirements placed on other CCITT Recommendations | International Standards that make use of these definitions.

This Recommendation | International Standard is applicable to the development of OSI managed object class specifications and provides generic definitions that support OSI systems management functions. These definitions may also be used in other Recommendations | International Standards specifying object classes, attributes, notifications and action types.

# 2	Normative referencestc  \l 1 "2	Normative references"§

The following CCITT Recommendations | International Standards contain provisions which, through reference in this text, constitute provisions of this Recommendation | International Standard. At the time of publication, the editions indicated were valid. All Recommendations and Standards are subject to revision, and parties to agreements based on this Recommendation | International Standard are encouraged to investigate the possibility of applying the most recent editions of the Recommendations|Standards listed below. Members of IEC and ISO maintain registers of currently valid International Standards. The CCITT Secretariat maintains a list of the currently valid CCITT Recommendations.

## 2.1	Identical CCITT Recommendations| International Standardstc  \l 2 "2.1	Identical CCITT Recommendations| International Standards"§

–	CCITT Recommendation X.720 (1992) | ISO/IEC 10165-1 : 1992, Information technology  - Open Systems Interconnection - Structure of management information: Management information model.

–	CCITT Recommendation X.722 (1992) | ISO/IEC 10165-4 : 1992, Information technology - Open Systems Interconnection - Structure of management information: Guidelines for definition of managed objects.

–	CCITT Recommendation X.730 (1992) | ISO/IEC 10164-1 : 1992, Information technology - Open Systems Interconnection - Systems Management: Object management function.

–	CCITT Recommendation X.731 (1992) | ISO/IEC 10164-2 : 1992, Information technology - Open System Interconnection - Systems Management: State management function.

–	CCITT Recommendation X.732 (1992) | ISO/IEC 10164-3 : 1992, Information technology - Open Systems Interconnection  - Systems Management: Attributes for representing relationships.

–	CCITT Recommendation X.733 (1992) | ISO/IEC 10164-4 : 1992, Information technology - Open Systems Interconnection - Systems Management: Alarm reporting function.

–	CCITT Recommendation X.734) | ISO/IEC 10164-5 : 1992, Information technology - Open Systems Interconnection - Systems Management  Event report management function.

–	CCITT Recommendation X.7351) | ISO/IEC 10164-6 : 1992, Information technology - Open Systems Interconnection - Systems Management:  Log control function.

–	CCITT Recommendation X.736 (1992) | ISO/IEC 10164-7 : 1992, Information technology - Open Systems Interconnection - Systems Management: Security alarm reporting function.

## 2.2	Paired CCITT Recommendations| International Standards equivalent in technical contenttc  \l 2 "2.2	Paired CCITT Recommendations| International Standards equivalent in technical content"§

–	CCITT Recommendation X.200 (1988), Reference model of Open Systems Interconnection for CCITT applications.

ISO 7498 : 1984, Information processing systems - Open Systems Interconnection - Basic Reference Model.

–	CCITT Recommendation X.208 (1988), Specification of abstract syntax notation one (ASN.1).

ISO/IEC 8824: 1990, Information technology - Open Systems Interconnection - Specification of Abstract Syntax Notation One (ASN.1).

–	CCITT Recommendation X.501 (1988), The Directory  - Models.

ISO/IEC 9594-2 : 1990 Information technology  - Open Systems Interconnection - The Directory - Part 2: The Models.

–	CCITT Recommendation X.710 (1991), Common management information service definition for CCITT applications.

ISO/IEC 9595 : 1991, Information technology - Open Systems Interconnection - Common management information service definition.

–	CCITT Recommendation X.711 (1991), Common management information protocol specification for CCITT applications.

ISO/IEC 9596-1: 1991, Information technology - Open Systems Interconnection - Common management information protocol specification  - Part 1: Specification.

# 3	Definitionstc  \l 1 "3	Definitions"§

For the purposes of this Recommendation | International Standard the following definitions apply.

## 3.1	Event report management function definitionstc  \l 2 "3.1	Event report management function definitions"§

This Recommendation | International Standard makes use of the following terms defined in CCITT Rec. X.734 | ISO/IEC 10164-5:

a)	discriminator;

b)	event forwarding discriminator;

c)	potential event report.

## 3.2	Management information model definitionstc  \l 2 "3.2	Management information model definitions"§

This Recommendation | International Standard makes use of the following terms defined in CCITT Rec. X.720 | ISO/IEC 10165-1:

a)	Attribute type;

b)	Distinguished name;

c)	Relative distinguished name.

# 4	Abbreviationstc  \l 1 "4	Abbreviations"§

ASN.1	Abstract Syntax Notation One

CMIS	Common Management Information Service

CMIP	Common Management Information Protocol

EFD		Event Forwarding Discriminator

Id		Identifier

PDU	Protocol Data Unit

RDN	Relative Distinguished Name

# 5	Notationtc  \l 1 "5	Notation"§

Attribute types and specific attributes are defined in this Recommendation | International Standard using the templates, defined in CCITT Rec. X.722 | ISO/IEC 10165-4.

The behavioural aspects of the specific attributes described here shall be incorporated into the definition of the managed object class importing these specific attributes.

# 6	Definition of managed object classestc  \l 1 "6	Definition of managed object classes"§

This Recommendation | International Standard defines Managed Object classes which are referenced by the System Management Functions of CCITT Recs. in X.730 - X.736 | ISO/IEC 10164 parts 1 to 7, or which are intended to be used as superclasses for the purpose of inheritance in the definition of Managed Object classes in other standards. The syntax of the attributes referenced in the templates are defined in clause 13.

## 6.1	Alarm recordtc  \l 2 "6.1	Alarm record"§

The alarmRecord managed object class is used to define the information stored in the log as a result of receiving alarm notifications or alarm reports. The semantics of the managed object class, namely its attributes and behaviour are derived from the Alarm notification described in CCITT Rec. X.733 | ISO/IEC 10164-4.

alarmRecord	MANAGED OBJECT CLASS

DERIVED FROM	eventLogRecord;

CHARACTERIZED BY

-- The appropriate object identifier values for the eventType attribute, inherited from eventLogRecord

-- managed object class are communicationAlarm, qualityofServiceAlarm, processingErrorAlarm,

-- equipmentAlarm and environmentalAlarm --

alarmRecordPackage	PACKAGE

BEHAVIOUR

alarmRecordBehaviour	BEHAVIOUR

DEFINED AS "This managed object is used to represent logged information that resulted from alarm notifications or event reports";;

ATTRIBUTES

probableCause	GET,

perceivedSeverity	GET;;;

CONDITIONAL PACKAGES

specificProblemsPackage	PACKAGE

ATTRIBUTES

specificProblems	GET;

REGISTERED AS	{smi2Package 1}; PRESENT IF "the Specific problems parameter is present in the alarm notification or event report corresponding to the instance of alarm record",

backedUpStatusPackage	PACKAGE

ATTRIBUTES

backedUpStatus	GET;

REGISTERED AS	{smi2Package 2}; PRESENT IF "the backedUpStatus attribute has a value TRUE and the Backed up status parameter is present in the alarm notification or event report corresponding to the instance of alarm record",

backUpObjectPackage	PACKAGE

ATTRIBUTES

backUpObject	GET;

REGISTERED AS	{smi2Package 3}; PRESENT IF "Backup object parameter is present in the alarm notification or event report corresponding to the instance of alarm record",

trendIndicationPackage	PACKAGE

ATTRIBUTES

trendIndication	GET;

REGISTERED AS	{smi2Package 4}; PRESENT IF "the Trend indication parameter is present in the alarm notification or event report corresponding to the instance of alarm record",

thresholdInfoPackage	PACKAGE

ATTRIBUTES

thresholdInfo	GET;

REGISTERED AS	{smi2Package 5}; PRESENT IF "the value for probableCause attribute is thresholdCrossed",

stateChangeDefinitionPackage	PACKAGE

ATTRIBUTES

stateChangeDefinition	GET;

REGISTERED AS	{smi2Package 6}; PRESENT IF "there is a state transition for the states defined in State Management Function, corresponding to the alarm type specified in the alarm record",

monitoredAttributesPackage	PACKAGE

ATTRIBUTES

monitoredAttributes	GET;

REGISTERED AS	{smi2Package 7}; PRESENT IF "the monitoredAttributes parameter is present in the alarm notification or event report corresponding to the instance of alarm record",

proposedRepairActionsPackage	PACKAGE

ATTRIBUTES

proposedRepairActions GET;

REGISTERED AS	{smi2Package 8}; PRESENT IF "the proposedRepairActions parameter is present in the alarm notification or event report corresponding to the instance of alarm record";

REGISTERED AS	{smi2MObjectClass 1};

## 6.2	Attribute value change recordtc  \l 2 "6.2	Attribute value change record"§

The attributeValueChangeRecord managed object class is used to define the information stored in the log as a result of receiving attribute value change notifications or attribute value change event reports. The semantics of the managed object class, namely its attributes and behaviour are derived from the Attribute Value Change notification described in CCITT Rec. X.730 | ISO/IEC 10164-1.

attributeValueChangeRecord	MANAGED OBJECT CLASS

DERIVED FROM	 eventLogRecord;

CHARACTERIZED BY

-- The appropriate object identifier values for the eventType attribute, inherited from eventLogRecord

-- managed object class, is attributeValueChange

attributeValueChangeRecordPackage	PACKAGE

BEHAVIOUR

attributeValueChangeRecordBehaviour	BEHAVIOUR

DEFINED AS "This managed object is used to represent logged information that resulted from attribute value change notifications or event reports";;

ATTRIBUTES

attributeValueChangeDefinition	GET;;;

CONDITIONAL PACKAGES

sourceIndicatorPackage		PRESENT IF "the sourceIndicator parameter is present in the attributeValueChange notification or event report corresponding to the instance of attribute value change record",

attributeIdentifierListPackage	PRESENT IF "the attributeIdentifierList parameter is present in the attributeValueChange notification or event report corresponding to the instance of attribute value change record";

REGISTERED AS	{smi2MObjectClass 2};

## 6.3	Discriminatortc  \l 2 "6.3	Discriminator"§

The discriminator managed object class is used to define the criteria for controlling management services. The semantics of the managed object class, namely its attributes and behaviour are described in CCITT Rec. X.734 | ISO/IEC 10164-5.

discriminator	MANAGED OBJECT CLASS

DERIVED FROM	 top;

CHARACTERIZED BY

discriminatorPackage	PACKAGE

BEHAVIOUR

discriminatorBehaviour	BEHAVIOUR

DEFINED AS "This managed object is used to represent the criteria for controlling management services.";;

ATTRIBUTES

discriminatorId	GET,

discriminatorConstruct

REPLACE-WITH-DEFAULT

DEFAULT VALUE Attribute-ASN1Module.defaultDiscriminatorConstruct	GET-REPLACE,

administrativeState	GET-REPLACE,

operationalState	GET;

NOTIFICATIONS

stateChange,

attributeValueChange,

objectCreation,

objectDeletion;;;

--  the above events are defined in CCITT Rec. X.731 | ISO/IEC10164- 2, CCITT Rec. X.730 | ISO/IEC10164-1

CONDITIONAL PACKAGES

availabilityStatusPackage	PRESENT IF "any of the scheduling packages, ( duration, weekly scheduling, external) are present",

duration		PRESENT IF "the discriminator function is scheduled to start at a specified time and stop at either a specified time or function continuously ",

dailyScheduling		PRESENT IF "both the weekly scheduling package and external scheduler packages are not present in an instance and daily scheduling is supported by that instance",

weeklyScheduling	PRESENT IF "both the daily scheduling package and external scheduler packages are not present in an instance and weekly scheduling is supported by that instance",

externalScheduler	PRESENT IF "both the daily scheduling package and weekly scheduling packages are not present in an instance and external scheduling is supported by that instance";

-- see CCITT Rec. X.734 | ISO/IEC 10164-5 for the description of this managed object class.

REGISTERED AS	{smi2MObjectClass 3};

## 6.4	Event forwarding discriminatortc  \l 2 "6.4	Event forwarding discriminator"§

The eventForwardingDiscriminator managed object class is used to define the conditions that shall be satisfied by potential event reports before the event report is forwarded to a particular destination.This managed object class is a subclass of discriminator managed object class. The semantics of the managed object class, namely its attributes, management operations and behaviour, are described in CCITT Rec. X. 734 | ISO/IEC 10164-5.

eventForwardingDiscriminator	MANAGED OBJECT CLASS

DERIVED FROM	discriminator;

CHARACTERIZED BY

-- The value for the administrative state if not specified at initiation defaults to the value unlocked.

efdPackage	PACKAGE

BEHAVIOUR

eventForwardingDiscriminatorBehaviour	BEHAVIOUR

DEFINED AS "This managed object is used to represent the criteria that shall be satisfied by potential event reports before the event report is forwarded to a particular destination.";;

ATTRIBUTES

destination	GET-REPLACE;;;

-- discriminatorConstruct attribute is defined using the attributes of a potential event report object

-- described in CCITT Rec. X.734 | ISO/IEC 10164-5.

CONDITIONAL PACKAGES

backUpDestinationListPackage	PACKAGE

ATTRIBUTES

activeDestination	GET,

backUpDestinationList	GET-REPLACE;

REGISTERED AS	{smi2Package 9} ; PRESENT IF "the event forwarding discriminator is required to provide a backup for the destination",

modePackage	PACKAGE

ATTRIBUTES

confirmedMode	GET;

REGISTERED AS	{smi2Package 10}; PRESENT IF "the event forwarding discriminator permits mode for reporting events to be specified by the managing system";

REGISTERED AS	{smi2MObjectClass 4};

## 6.5	Event log recordtc  \l 2 "6.5	Event log record"§

The eventLogRecord managed object class is used to define the information stored in the log as a result of receiving notifications or event reports. This is a superclass from which records for specific event types are derived.

eventLogRecord	MANAGED OBJECT CLASS

DERIVED FROM	 logRecord;

CHARACTERIZED BY

eventLogRecordPackage	PACKAGE

BEHAVIOUR

eventLogRecordBehaviour	BEHAVIOUR

DEFINED AS "This managed object represents the information stored in the log as a result of receiving notifications or incoming event reports.";;

ATTRIBUTES

managedObjectClass	GET,

managedObjectInstance	GET,

eventType		GET;;;

CONDITIONAL PACKAGES

eventTimePackage	PACKAGE

ATTRIBUTES

eventTime	GET;

REGISTERED AS	{smi2Package 11}; PRESENT IF "the event time parameter was present in the received event report",

notificationIdentifierPackage	PRESENT IF "the notification Identifier parameter is present in the notification or event report corresponding to the instance of an event record or an instance of its subclasses",

correlatedNotificationsPackage 	PRESENT IF "the correlatedNotifications parameter is present in the notification or event report corresponding to the instance of an event record or an instance of its subclasses",

additionalTextPackage		PRESENT IF "the Additional text parameter is present in the notification or report corresponding to the instance of event record or an instance of its subclasses",

additionalInformationPackage	PRESENT IF "the Additional information parameter is present in the notification or report corresponding to the instance of event record or an instance of its subclasses";

REGISTERED AS	{smi2MObjectClass 5};

## 6.6	Logtc  \l 2 "6.6	Log"§

The log managed object class is used to define the criteria for controlling the logging of the information in an open system. The semantics of the managed object class, namely its attributes and behaviour are described in CCITT Rec. X.735 | ISO/IEC 10164-6.

log	MANAGED OBJECT CLASS

DERIVED FROM	 top;

CHARACTERIZED BY

-- see CCITT Rec. X.735 | ISO/IEC 10164-6 for the description of this managed object class.

logPackage	PACKAGE

BEHAVIOUR

logBehaviour	BEHAVIOUR

DEFINED AS "This managed object is used to store incoming event reports and local system notifications. Additional details are defined in CCITT Rec. X. 735 | ISO/IEC 10164-6. ";;

ATTRIBUTES

logId	GET,

discriminatorConstruct	GET-REPLACE ,

administrativeState	GET-REPLACE,

operationalState	GET,

availabilityStatus PERMITTED VALUES Attribute-ASN1Module.LogAvailability

REQUIRED VALUES Attribute-ASN1Module.UnscheduledLogAvailability	GET,

logFullAction	GET-REPLACE;

NOTIFICATIONS

objectCreation,

objectDeletion,

attributeValueChange,

stateChange,

processingErrorAlarm;;;

CONDITIONAL PACKAGES

finiteLogSizePackage	PACKAGE

ATTRIBUTES

maxLogSize	GET-REPLACE,

currentLogSize	GET,

numberOfRecords	GET;

REGISTERED AS	{smi2Package 12} ;	PRESENT IF "an instance supports it",

logAlarmPackage	PACKAGE

ATTRIBUTES

capacityAlarmThreshold	GET-REPLACE ADD-REMOVE;

REGISTERED AS	{smi2Package 13} ;	PRESENT IF "a log is of finite size and halts logging when the availability status has the log full value.",

availabilityStatusPackage				PRESENT IF "any of the scheduling packages, (duration, weekly scheduling, external) are present. The presence of this package makes available the off-duty value of the available status attribute to the object.",

duration					PRESENT IF "the logging function is scheduled to start at a specified time and stop at either a specified time or function continuously.",

dailyScheduling					PRESENT IF "both the weekly scheduling package and external scheduler packages are not present in an instance and daily scheduling is supported by that instance.",

weeklyScheduling				PRESENT IF "both the daily scheduling package and external scheduler packages are not present in an instance and weekly scheduling is supported by that instance.",

externalScheduler				PRESENT IF "both the daily scheduling package and weekly scheduling packages are not present in an instance and external scheduling is supported by that instance.";

REGISTERED AS	 {smi2MObjectClass 6};

## 6.7	Log recordtc  \l 2 "6.7	Log record"§

The logRecord managed object class is used to define the records contained in a log managed object. The semantics of the managed object class, namely its attributes and behaviour are described in CCITT Rec. X.735 | ISO/IEC 10164-6.

logRecord	MANAGED OBJECT CLASS

DERIVED FROM	 top;

CHARACTERIZED BY

logRecordPackage	PACKAGE

BEHAVIOUR

logRecordBehaviour	BEHAVIOUR

DEFINED AS "This managed object represents the information stored in the logs";;

ATTRIBUTES

logRecordId	GET,

loggingTime	GET;;;

REGISTERED AS	 {smi2MObjectClass 7};

## 6.8	Object creation recordtc  \l 2 "6.8	Object creation record"§

The objectCreationRecord managed object class is used to define the information stored in the log as a result of receiving object creation notifications or Object Creation event reports. The semantics of the managed object class, namely its attributes and behaviour are derived from the Object Creation notification described in CCITT Rec. X.730 | ISO/IEC 10164-1.

objectCreationRecord	MANAGED OBJECT CLASS

DERIVED FROM	 eventLogRecord;

CHARACTERIZED BY

-- The appropriate object identifier value for the eventType attribute, inherited from eventLogRecord

-- managed object class, is objectCreation

objectCreationRecordPackage	PACKAGE

BEHAVIOUR

objectCreationRecordBehaviour	BEHAVIOUR

DEFINED AS "This managed object is used to represent logged information that resulted from object creation notifications or event reports";;;;

CONDITIONAL PACKAGES

sourceIndicatorPackage	PRESENT IF "the sourceIndicator parameter is present in the object creation notification or event report corresponding to the instance of object creation record",

attributeListPackage	PRESENT IF "the attributeList parameter is present in the object creation notification or event report corresponding to the instance of object creation record";

REGISTERED AS	{smi2MObjectClass 8};

## 6.9	Object deletion recordtc  \l 2 "6.9	Object deletion record"§

The objectDeletionRecord managed object class is used to define the information stored in the log as a result of receiving object deletion notifications or Object deletion event reports. The semantics of the managed object class, namely its attributes and behaviour are derived from the Object Deletion notification described in CCITT Rec. X.730 | ISO/IEC 10164-1.

objectDeletionRecord	MANAGED OBJECT CLASS

DERIVED FROM	 eventLogRecord;

CHARACTERIZED BY

-- The appropriate object identifier value for the eventType attribute, inherited from eventLogRecord

-- managed object class, is objectDeletion

objectDeletionRecordPackage	PACKAGE

BEHAVIOUR

objectDeletionRecordBehaviour	BEHAVIOUR

DEFINED AS "This managed object is used to represent logged information that resulted from object deletion notifications or event reports";;;;

CONDITIONAL PACKAGES

sourceIndicatorPackage	PRESENT IF "the sourceIndicator parameter is present in the object deletion notification or event report corresponding to the instance of object deletion record",

attributeListPackage	PRESENT IF "the attributeList parameter is present in the object deletion notification or event report corresponding to the instance of object deletion record";

REGISTERED AS	{smi2MObjectClass 9};

## 6.10	Relationship change recordtc  \l 2 "6.10	Relationship change record"§

The relationshipChangeRecord managed object class is used to define the information stored in the log as a result of receiving relationship change notifications or relationship change event reports. The semantics of the managed object class, namely its attributes and behaviour are derived from the Relationship Change notification described in CCITT Rec.X.732 | ISO/IEC 10164-3.

relationshipChangeRecord	 MANAGED OBJECT CLASS

DERIVED FROM	 eventLogRecord;

-- The appropriate object identifier value for the eventType attribute, inherited from eventLogRecord

-- managed object class, is relationshipChange

CHARACTERIZED BY

relationshipChangeRecordPackage	PACKAGE

BEHAVIOUR

relationshipChangeRecordBehaviour	BEHAVIOUR

DEFINED AS "This managed object is used to represent logged information that resulted from

relationship change notifications or event reports";;

ATTRIBUTES

relationshipChangeDefinition	GET;;;

CONDITIONAL PACKAGES

sourceIndicatorPackage		PRESENT IF "the Source indicator parameter is present in the relationshipChange notification or event report corresponding to the instance of relationship change record",

attributeIdentifierListPackage	PRESENT IF "the Attribute identifier list parameter is present in the relationshipChange notification or event report corresponding to the instance of relationship change record";

REGISTERED AS	{smi2MObjectClass 10};

## 6.11	Security alarm report recordtc  \l 2 "6.11	Security alarm report record"§

The securityAlarmReportRecord managed object class is used to define the information stored in the log as a result of receiving security alarm notifications or security alarm event reports. The semantics of the managed object class, namely its attributes and behaviour are derived from the Security Alarm notification described in CCITT Rec. X.736 | ISO/IEC 10164-7.

securityAlarmReportRecord	MANAGED OBJECT CLASS

DERIVED FROM	 eventLogRecord;

CHARACTERIZED BY

-- The appropriate object identifier values for the eventType attribute, inherited from eventLogRecord

-- managed object class, are integrityViolation, operationalViolation, physicalViolation,

-- securityServiceOrMechanismViolation and timeDomainViolation --

securityAlarmRecordPackage	PACKAGE

BEHAVIOUR

securityAlarmReportRecordBehaviour	BEHAVIOUR

DEFINED AS "This managed object is used to represent logged information that resulted from security alarm notifications or event reports";;

ATTRIBUTES

securityAlarmCause	GET,

securityAlarmSeverity	GET,

securityAlarmDetector	GET,

serviceUser		GET,

serviceProvider		GET;;;

REGISTERED AS	{smi2MObjectClass 11};

## 6.12	State change recordtc  \l 2 "6.12	State change record"§

The stateChangeRecord managed object class is used to define the information stored in the log as a result of receiving state change notifications or state change event reports. The semantics of the managed object class, namely its attributes and behaviour are derived from the State Change notification described in CCITT Rec. X.731 | ISO/IEC 10164-2.

stateChangeRecord	MANAGED OBJECT CLASS

DERIVED FROM	 eventLogRecord;

-- The appropriate object identifier value for the eventType attribute, inherited from eventLogRecord

-- managed object class, is stateChange

CHARACTERIZED BY

stateChangeRecordPackage	PACKAGE

BEHAVIOUR

stateChangeRecordBehaviour	BEHAVIOUR

DEFINED AS "This managed object is used to represent logged information that resulted from state change notifications or event reports";;

ATTRIBUTES

stateChangeDefinition	GET;;;

CONDITIONAL PACKAGES

sourceIndicatorPackage		PRESENT IF "the Source indicator parameter is present in the stateChange notification or event report corresponding to the instance of state change record",

attributeIdentifierListPackage	PRESENT IF "the Attribute identifierList parameter is present in the stateChange notification or event report corresponding to the instance of state change record";

REGISTERED AS	{smi2MObjectClass 12};

## 6.13	Systemtc  \l 2 "6.13	System"§

The system managed object class is used to represent a set of hardware and software that forms an autonomous whole capable of performing information processing and/or information transfer.

The specification of the sequence of name bindings to be used in constructing the distinguished name for a system managed object is outside the scope of this Recommendation | International Standard . Examples of names for systems are specified in CCITT Rec. X.720 | ISO/IEC 10165-1.

NOTE- This definition does not correspond to real open system but corresponds to real systems in CCITT Rec. X.200 | ISO 7498.

An instance of this managed object class may be used as the superior in naming managed objects representing either information processing and or information transfer resources contained within this instance.

system	MANAGED OBJECT CLASS

DERIVED FROM	top;

CHARACTERIZED BY

systemPackage	PACKAGE

ATTRIBUTES

systemId	GET,

systemTitle	GET,

operationalState	GET	usageState	GET;;;

CONDITIONAL PACKAGES

administrativeStatePackage	PACKAGE

ATTRIBUTES

administrativeState	GET-REPLACE ;

REGISTERED AS	{smi2Package 14};	PRESENT IF "an instance supports it.",

supportedFeaturesPackage	PACKAGE

ATTRIBUTES

supportedFeatures	GET-REPLACE ADD-REMOVE;

REGISTERED AS	{smi2Package 15}; PRESENT IF "an instance supports it.";

REGISTERED AS	{smi2MObjectClass 13};

## 6.14	Toptc  \l 2 "6.14	Top"§

The top managed object class, is that class of which every other managed object class is a subclass.

The semantics of the attributes objectClass,packages,nameBindings, and allomorphs are defined in CCITT Rec. X.720 | ISO/IEC 10165-1.

top	MANAGED OBJECT CLASS

CHARACTERIZED BY

topPackage	PACKAGE

BEHAVIOUR

topBehaviour;

ATTRIBUTES

objectClass GET,

nameBinding	GET ;;;

CONDITIONAL PACKAGES

packagesPackage	PACKAGE

ATTRIBUTES	packages	GET;

REGISTERED AS	{smi2Package 16};

PRESENT IF "any registered package, other than this package has been instantiated",

allomorphicPackage	PACKAGE

ATTRIBUTES

allomorphs	GET;

REGISTERED AS	{smi2Package 17};

PRESENT IF " if an object supports allomorphism" ;

REGISTERED AS	{smi2MObjectClass 14};

topBehaviour	BEHAVIOUR

DEFINED AS "This is the top level of managed object class hierarchy and every other managed object class is a specialization of either this generic class (top) or a specialization of subclass of top. The parameter miscellaneousError is to be used when a processing failure has occurred and the error condition encountered does not match any of object's defined specific error types.";

# 7	Name binding for managed object classestc  \l 1 "7	Name binding for managed object classes"§

This clause specifies name bindings for the managed object classes. Additional name bindings may be defined for these managed object classes and their subclasses.

## 7.1	Discriminatortc  \l 2 "7.1 Discriminator"§

discriminator-system	NAME BINDING

SUBORDINATE OBJECT CLASS	discriminator	AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS system AND SUBCLASSES ;

WITH ATTRIBUTE	discriminatorId;

CREATE

WITH-REFERENCE-OBJECT ,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS	{smi2NameBinding 1};

## 7.2	Logtc  \l 2 "7.2	Log"§

log-system	NAME BINDING

SUBORDINATE OBJECT CLASS	log	AND SUBCLASSES ;

NAMED BY

SUPERIOR OBJECT CLASS system AND SUBCLASSES ;

WITH ATTRIBUTE	logId;

CREATE

WITH-REFERENCE-OBJECT ,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS	{smi2NameBinding 2};

## 7.3	Log recordtc  \l 2 "7.3	Log record"§

logRecord-log	NAME BINDING

SUBORDINATE OBJECT CLASS	logRecord AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS	log AND SUBCLASSES;

WITH ATTRIBUTE

logRecordId;

DELETE

only-if-no-contained-objects;

REGISTERED AS	{smi2NameBinding 3};

# 8	Definition of packagestc  \l 1 "8	Definition of packages"§

This Recommendation | International Standard defines a number of packages which are referenced by the System Management Functions of CCITT Recs.X.730 - 736 | ISO/IEC 10164 parts 1 to 7.

## 8.1	Additional informationtc  \l 2 "8.1	Additional information"§

The additionalInformationPackage package contains the additionalInformation attribute described in CCITT Rec. X.733 | ISO/IEC 10164-4.

additionalInformationPackage	PACKAGE

ATTRIBUTES

additionalInformation	GET;

REGISTERED AS	{smi2Package 18};

## 8.2	Additional texttc  \l 2 "8.2	Additional text"§

The additionalTextPackage package contains the additionalText attribute described in CCITT Rec. X.733 | ISO/IEC 10164-4.

additionalTextPackage	PACKAGE

ATTRIBUTES

additionalText	GET;

REGISTERED AS	{smi2Package 19};

## 8.3	Attribute identifier listtc  \l 2 "8.3	Attribute identifier list"§

The attributeIdentifierListPackage package contains the attributeIdentifierList attribute described in CCITT Rec. X.730 | ISO/IEC 10164-1, CCITT Rec. X.731 | ISO/IEC 10164-2, and CCITT Rec. X.732 | ISO/IEC 10164-3.

attributeIdentifierListPackage	PACKAGE

ATTRIBUTES

attributeIdentifierList	GET;

REGISTERED AS	{smi2Package 20};

## 8.4	Attribute listtc  \l 2 "8.4	Attribute list"§

The attributeListPackage package contains the attributeList attribute described in CCITT Rec. X.730 | ISO/IEC 10164-1.

attributeListPackage	PACKAGE

ATTRIBUTES

attributeList	GET;

REGISTERED AS	{smi2Package 21};

## 8.5	Availability statustc  \l 2 "8.5	Availability status"§

The availabilityStatusPackage package contains the availabilityStatus attribute described in CCITT Rec. X.731 | ISO/IEC 10164-2.

availabilityStatusPackage	PACKAGE

BEHAVIOUR

availabilityStatusBehaviour	BEHAVIOUR

DEFINED AS " This package is described in CCITT Rec. X.734,X.735 | ISO/IEC 10164-5, 10164-6. It is used to indicate the availability of the resource according to a predetermined time schedule." ;;

ATTRIBUTES

availabilityStatus REQUIRED VALUES Attribute-ASN1Module.SchedulingAvailability	GET;

REGISTERED AS	{smi2Package 22};

## 8.6	Correlated notificationstc  \l 2 "8.6	Correlated notifications"§

The correlatedNotificationsPackage package contains the correlatedNotifications attribute described in CCITT Rec. X.733 | ISO/IEC 10164-4.

correlatedNotificationsPackage	PACKAGE

ATTRIBUTES

correlatedNotifications	GET;

REGISTERED AS	{smi2Package 23};

## 8.7	Notification identifiertc  \l 2 "8.7	Notification identifier"§

The notificationIdentifierPackage package contains the notificationIdentifier attribute described in CCITT Rec. X.733 | ISO/IEC 10164-4.

notificationIdentifierPackage	PACKAGE

ATTRIBUTES

notificationIdentifier	GET;

REGISTERED AS	{smi2Package 24};

## 8.8	Daily schedulingtc  \l 2 "8.8	Daily scheduling"§

The semantics of dailyScheduling package are described in CCITT Rec. X.734 | ISO/IEC 10164-5 and CCITT Rec. X.735 | ISO/IEC 10164-6.

dailyScheduling	PACKAGE

ATTRIBUTES

intervalsOfDay	REPLACE-WITH-DEFAULT

DEFAULT VALUE Attribute-ASN1Module.defaultIntervalsOfDay

GET-REPLACE ADD-REMOVE;

REGISTERED AS	{smi2Package 25};

## 8.9	Durationtc  \l 2 "8.9	Duration"§

The semantics of duration package are described in CCITT Rec.X.734 | ISO/IEC 10164-5 and CCITT Rec.X.735 | ISO/IEC 10164-6.

duration	PACKAGE

ATTRIBUTES

startTime	GET-REPLACE,

stopTime	REPLACE-WITH-DEFAULT

DEFAULT VALUE Attribute-ASN1Module.defaultStopTime

GET-REPLACE;

REGISTERED AS	{smi2Package 26};

## 8.10	External schedulertc  \l 2 "8.10	External scheduler"§

The semantics of externalScheduler package are described in CCITT Rec. X.734 | ISO/IEC 10164-5 and CCITT Rec. X.735 | ISO/IEC 10164-6.

externalScheduler		PACKAGE

ATTRIBUTES

schedulerName	GET;

REGISTERED AS	{smi2Package 27};

## 8.11	Source indicatortc  \l 2 "8.11	Source indicator"§

The sourceIndicatorPackage package contains the sourceIndicator attribute described in CCITT Rec. X.730 | ISO/IEC 10164-1, CCITT Rec. X.731 | ISO/IEC 10164-2, and CCITT Rec. X.732 | ISO/IEC 10164-3.

sourceIndicatorPackage	PACKAGE

ATTRIBUTES

sourceIndicator	GET;

REGISTERED AS	{smi2Package 28};

## 8.12	Weekly schedulingtc  \l 2 "8.12	Weekly scheduling"§

The semantics of weeklyScheduling package are described in CCITT Rec.X.734 | ISO/IEC 10164-5 and CCITT Rec.X.735 | ISO/IEC 10164-6.

weeklyScheduling	PACKAGE

ATTRIBUTES

weekMask	REPLACE-WITH-DEFAULT

DEFAULT VALUE Attribute-ASN1Module.defaultWeekMask

GET-REPLACE ADD-REMOVE;

REGISTERED AS	{smi2Package 29};

# 9	Definition of generic attribute typestc  \l 1 "9	Definition of generic attribute types"§

This Recommendation | International Standard defines a number of generic attribute types. These are intended to be used in the definition of specific attributes appearing in the definitions of Managed Object Classes.

Each attribute type definition comprises

–	the structure of the attribute value;

–	the inherent properties of the attribute in terms of its behaviour;

–	the permitted matching operations that may be performed on an attribute having this type;

–	the ways in which specific attributes having this type relate to other attributes.

An attribute type definition does not include an object identifier. This shall form part of the definition of each of the specific attributes of that type. Definitions of specific attributes may also refine the definition given in the type, for example

–	to extend the behaviour definition to relate the behaviour to the operation of the resource represented by the managed object;

–	parameters be associated with the attribute.

The template definitions are specified below for each attribute's type. The ASN.1 supporting productions are defined in 14.2.

## 9.1	Countertc  \l 2 "9.1	Counter"§

The semantics of the counter attribute type are defined below.

Counters are a management abstraction of an underlying counting process. Two types of counters are modelled to meet different needs. The non-settable (or simple) counter is defined to operate autonomously, i.e. not subject to change by Management operation, in order to enable different managing stations to access its information without interference. The settable counter, in contrast, is defined to allow setting or resetting by Management operation, and therefore is more suited to use by a single managing station. Both types are defined as attributes.

A counter is regarded as associated with some internal event, which may be, but is not in general, a defined event represented in management information. The current value is incremented by 1 when this event occurs. It can take any values in its range. When a counter reaches its maximum value, it wraps around and counts upwards from 0; overflow information is not in general retained. An attribute value change notification may be defined for counters with wrap behaviour.

NOTE – The rule that the counter value can increase only in steps of 1 is a descriptive convention that simplifies the description of counter-threshold in 9.3.1. It does not imply that it will always be possible to observe each value in the counter's range, since the events counted may occur in rapid succession.

Not-settable counter-definition

attribute value

–	single value.

inherent properties

–	current value is a non-negative integer;

–	it has a maximum value (see specification properties);

–	the counting direction is up, with increment 1;

–	the current value wraps around when it reaches its maximum;

–	the initial value is 0.

permitted operations

–	Get only.

implicit relations

–	directly related to a single counter-threshold where applied;

–	as an implementation option, may trigger a defined event when it wraps.

specification properties

–	the internal event that is counted;

–	maximum value;

–	"estimated" wrap around period, to indicate necessary reading rate.

Settable counter definition

attribute value

–	single value.

inherent properties

–	current value is a non-negative integer;

–	it has a maximum value (see specification properties);

–	the counting direction is up, with increment 1;

–	the current value wraps around when it reaches its maximum;

–	the initial value is 0.

permitted operations

–	Get;

–	Set to arbitrary value (within range);

–	Set to the default specified in the managed object definition, or to zero otherwise.

implicit relations

–	directly related to a single counter-threshold, where applied;

–	as an implementation option, may trigger a defined event when it wraps or its value is otherwise changed except by the normal counting process.

specification properties

–	the internal event that is counted;

–	maximum value;

–	"estimated" wrap around period, to indicate necessary reading rate.

counter	ATTRIBUTE

WITH ATTRIBUTE SYNTAX Attribute-ASN1Module.Count;

MATCHES FOR EQUALITY,ORDERING ;;

## 9.2	Gaugetc  \l 2 "9.2	Gauge"§

The semantics of the gauge attribute type is defined below.

The gauge is the management abstraction of the value of a dynamic variable, such as the number of connections currently operated by a protocol machine or the rate of change of a traffic counter. There is no restriction on what the dynamic variable may be, within the constraints set out below.

The value of the gauge is subject to change in either direction. The value of the increment or decrement is unconstrained, except that changes that would take the gauge value beyond its maximum or minimum leave the gauge value at its maximum or minimum respectively.

In order to support use by multiple managing stations, gauges are defined to be read-only.

attribute value

–	single value.

inherent properties

–	current value is a non-negative integer or real (see specification properties);

–	it has a maximum and a minimum value (see specification properties);

–	it may increase or decrease by arbitrary amounts;

–	it does not wrap around.

permitted actions

–	Get only.

implicit relations

–	directly related to a tide-mark and or gauge-threshold, where applied;

–	only one minimum and one maximum tide-mark may be applied;

–	only one (possibly multi-level) threshold may be applied;

–	can be used to measure other management information.

specification properties

–	the dynamic variable measured, with its type: integer or real;

–	maximum and minimum value.

gauge	ATTRIBUTE

WITH ATTRIBUTE SYNTAX Attribute-ASN1Module.ObservedValue;

MATCHES FOR EQUALITY, ORDERING ;;

## 9.3	Thresholdtc  \l 2 "9.3	Threshold"§

A threshold is the general mechanism for generating a defined notification from changes in numeric attributes values. Two types of thresholds are defined, for counters and for gauges, because the required behaviour is significantly different in the two cases. Both allow attribute values that are of integer data types. Other types of threshold may be defined in the future.

### 9.3.1	Counter-thresholdtc  \l 3 "9.3.1	Counter-threshold"§

The counter-Threshold is the general mechanism for generating a defined notification from changes in any value of attribute type counter.

The Counter-Threshold is related to a defined notification. It has the basic property that the defined notification is triggered when the value of the count becomes equal to the comparison level of the threshold. The definition also allows for more complex styles of operation when needed: the comparison attribute is in general a set of levels, for example to represent different degrees of severity of a fault condition, and the notification is triggered whenever the count value reaches any of the levels.

In addition, an offset mechanism is also available to allow particular counting intervals to be detected, as follows. If the offset value is not zero, whenever the threshold is triggered by the counter value reaching a comparison level, that comparison level is incremented by the offset value. This is regarded as taking place instantaneously, i.e. before the count is incremented. Thus, for each level, the threshold triggers a notification every time the count increases by an interval equal to the offset value. When the comparison value exceeds the modulus of the counter, it wraps around.

For a counter-threshold, the time at which the threshold offset was last applied, or the time at which the count was last initialized is the arm-time.

The Counter-Threshold is modelled as a set-valued attribute.

value type

–	comparison level, integer;

–	offset value, integer;

–	notifications on/off switch, boolean.

inherent properties

–	the comparison levels are non-negative integers;

–	the offset values are non-negative integers;

–	the notifications switch is on or off.

permitted operations

–	Get, Set, Add, Remove.

implicit relations

–	directly related to a single count;

–	directly related to a defined notification.

specification properties

–	count to which it applies;

–	the defined notification that may be triggered.

Any relationship between a threshold attribute and any underlying mechanism is specified as part of the behaviour of the managed object class containing the threshold.

counter-Threshold	 ATTRIBUTE

WITH ATTRIBUTE SYNTAX

Attribute-ASN1Module.CounterThreshold;

MATCHES FOR EQUALITY ;;

### 9.3.2	Gauge-thresholdtc  \l 3 "9.3.2	Gauge-threshold"§

The gauge-Threshold is the general mechanism for generating one or more defined notifications from changes in any value of attribute type gauge. A hysteresis mechanism is provided to avoid the repeated triggering of event notifications when the gauge makes small oscillations around a threshold value. This capability is provided by specifying threshold values in pairs; one being a high threshold value and the other being a low threshold value. The difference between threshold values is the hysteresis interval.

The Gauge-Threshold is a set-valued attribute used to define threshold levels associated with notifications. As a set valued attribute, it may have zero or more members. Each member consists of a sequence of two submembers, notifyLow and notifyHigh. Each of these submembers in turn has a structure and consists of a sequence of a gauge value and its associated on/off switch.

notifyHigh: This structured submember defines the value that the gauge must reach or exceed in order to optionally generate notification, together with a switch value that controls the generation of the notification.

notifyLow: This structured submember defines the value that the gauge must fall to or below in order to optionally generate notification, together with a switch value that controls the generation of the notification.

Gauge-Thresholds have the constraint that notifyHigh has a gauge value that is greater than or equal to that of notifyLow.

Gauge-Thresholds have the following constraints

–	notify-high's gauge value and notify-low's threshold value are of the same type as the gauge;

–	notify-high's threshold value is greater or equal to notify-low's threshold value;

–	 notify-high's on/off switch and notify-low's on/off switch are of type boolean.

Gauge-Thresholds have the following behaviour

–	initially, if notify-high's on/off switch is true and the gauge value becomes equal to or greater than notify-high's gauge value, in a positive going direction, then the defined event notification is triggered; subsequent crossings of notify-high's gauge value shall not cause further generation of event reports unless the gauge value becomes equal to or less than notify-low's threshold value.

–	initially, if notify-low's on/off switch is true and the gauge value becomes equal to or less than notify-low's gauge value, in a negative going direction, then the defined event notification is triggered; subsequent crossings of notify-low's gauge value shall not cause further generation of event reports unless the gauge value becomes equal to or greater than notify-high's gauge value.

For either case, the time at which the notification is again enabled is defined as the arm-time.

Any relationship between a threshold attribute and any underlying mechanism is specified as part of the behaviour of the managed object class containing the threshold.

gauge-Threshold	 ATTRIBUTE

WITH ATTRIBUTE SYNTAX Attribute-ASN1Module.GaugeThreshold;

MATCHES FOR EQUALITY ;;

9.4	Tide-marktc  \l 2 "9.4	Tide-mark"§

The semantics of the TideMark attribute type is defined below.

The tide-mark is a mechanism that records the maximum or minimum value reached by a gauge during a measurement period. A tide-mark is essentially read-only, except that it may be reset to a desired value which is the current value of the associated gauge. Each individual tide-mark is specified to be either a maximum tide-mark or a minimum tide-mark, and therefore moves (other than when it is reset) only up or down respectively. Thus, a maximum tide-mark changes (increases) only when its gauge increases beyond the current tide-mark value, and a minimum tide-mark changes (decreases) only when its gauge moves below the current tide-mark value.

A tide-mark is defined as a set-valued attribute with three components. Two value components are defined, the current value and the value immediately prior to the last reset, so as to support different measurement requirements. The third component is the last reset time.

attribute value

–	current value of tide-mark;

–	former value of tide-mark, i.e. value immediately before the last reset;

–	last reset time.

inherent properties

–	it is associated with a gauge;

–	it has a direction (maximum or minimum);

–	the current and former values are integer or real depending upon the associated gauge.

permitted operations

–	Get returns the entire attribute value, viz. the current value, the former value and the last reset time;

–	Set to default sets the former value to the current value,the current value to the value of the associated gauge,and the last reset time to the current time.

implicit relations

–	directly related to a gauge;

–	may be directly related to a defined event that is triggered when the current value changes.

specification properties

–	gauge to which it applies;

–	direction (maximum or minimum).

tideMark	 ATTRIBUTE

WITH ATTRIBUTE SYNTAX Attribute-ASN1Module.TideMarkInfo;

MATCHES FOR EQUALITY ;;

# 10	Definition of specific attribute typestc  \l 1 "10	Definition of specific attribute types"§

This Recommendation | International Standard defines a number of attributes which are referenced by the System Management Functions of CCITT Recs. X.730 - 736 | ISO/IEC 10164 Parts 1 to 7, or which are of widespread applicability in Managed Object definitions. Unlike the attribute types defined in clause 9, the definitions here are directly useable in the definitions of Managed Objects without further elaboration. In particular they include an object identifier by which the attribute is identified. It follows that any of these attributes can only appear once in a Managed Object. Uses of these attributes in Managed Objects may also refine the definition, for example

–	to extend the behaviour definition to relate the behaviour to the operation of the resource represented by the Managed Object;

–	to restrict the values which the attribute may take to a subset of those defined by the attribute type;

–	to require support for some values of the type;

–	to define relations (such as mutual constraints on permitted values) with other attributes.

The template definition along with the allocated object identifier are specified below for each attributes type. The ASN.1 supporting productions are defined in 13.2.

## 10.1	Attributes types used for namingtc  \l 2 "10.1	Attributes types used for naming"§

The following attributes can be used as naming attributes in a relative distinguished name.

### 10.1.1	Discriminator Idtc  \l 3 "10.1.1	Discriminator Id"§

The discriminatorId attribute type is used in naming instances of Discriminator managed object class.

discriminatorId	ATTRIBUTE

WITH ATTRIBUTE SYNTAX Attribute-ASN1Module.SimpleNameType;

MATCHES FOR Equality, Substrings, ORDERING;

BEHAVIOUR

rDNIdBehaviour;

REGISTERED AS	{smi2AttributeID 1};

rDNIdBehaviour	BEHAVIOUR

DEFINED AS " If the string choice for the syntax is used matching on the substrings is permitted . If the number choice for the syntax is used then matching on ordering is permitted.";

### 10.1.2	Log Idtc  \l 3 "10.1.2	Log ID"§

The logId attribute type is used in naming instances of Log managed object class.

logId	ATTRIBUTE

WITH ATTRIBUTE SYNTAX Attribute-ASN1Module.SimpleNameType;

MATCHES FOR EQUALITY, SUBSTRINGS;

BEHAVIOUR

rDNIdBehaviour;

REGISTERED AS	{smi2AttributeID 2};

### 10.1.3	Log record Idtc  \l 3 "10.1.3	Log record Id"§

The logRecordId attribute type is used in naming instances of Log Records managed object class.

logRecordId	ATTRIBUTE

WITH ATTRIBUTE SYNTAX Attribute-ASN1Module.LogRecordId;

MATCHES FOR EQUALITY, ORDERING ;

REGISTERED AS	{smi2AttributeID 3};

### 10.1.4	System Idtc  \l 3 "10.1.4	System Id"§

The systemId attribute type may be used in naming instances of System managed object class.

systemId	ATTRIBUTE

WITH ATTRIBUTE SYNTAX Attribute-ASN1Module.SystemId;

MATCHES FOR EQUALITY;

REGISTERED AS	{smi2AttributeID 4};

### 10.1.5	System titletc  \l 3 "10.1.5	System title"§

The systemTitle attribute type may be used in naming instances of System managed object class.

systemTitle	ATTRIBUTE

WITH ATTRIBUTE SYNTAX Attribute-ASN1Module.SystemTitle;

MATCHES FOR EQUALITY;

REGISTERED AS	{smi2AttributeID 5};

## 10.2	Countertc  \l 2 "10.2	Counter"§

This Recommendation | International Standard defines a number of counter types which are referenced by the System Management Functions in CCITT Recs. X.730-736 | ISO/IEC 10164 Parts 1 to 7.

NOTE – There are no counter types currently defined in the system management functions and this subclause is a place holder for future additions. Annex A defines a number of specific counter attributes generally useful to layer management.

## 10.3 Counter-thresholdtc  \l 2 "10.3 Counter-threshold"§

This Recommendation | International Standard defines a number of counter-threshold types which are referenced by the System Management Functions in CCITT Recs. X.730-736 | ISO/IEC 10164 Parts 1 to 7.

NOTE – There are no counter-threshold types currently defined in the system management functions and this subclause is a place holder for future additions. Annex A defines a number of specific counter-threshold attributes generally useful to layer management.

## 10.4	Gaugetc  \l 2 "10.4	Gauge"§

This Recommendation | International Standard defines a number of Gauge types which are referenced by the System Management Functions in CCITT Recs. X.730-736 | ISO/IEC 10164 Parts 1 to 7.

NOTE – There are no Gauge types currently defined in the system management functions and this subclause is a place holder for future additions.

## 10.5	Gauge-thresholdtc  \l 2 "10.5	Gauge-threshold"§

This Recommendation | International Standard defines a number of Gauge-Threshold types which are referenced by the System Management Functions in CCITT Recs. X.730-736 | ISO/IEC 10164 Parts 1 to 7.

NOTE – There are no gauge threshold types currently defined in the system management functions and this subclause is a place holder for future addition.

## 10.6	Tide-marktc  \l 2 "10.6	Tide-mark"§

This Recommendation | International Standard defines a number of Tide-Mark types which are referenced by the System Management Functions of CCITT Recs. X.730-736 | ISO/IEC 10164 Parts 1 to 7.

NOTE – There are no tide-mark types currently defined in the system management functions and this subclause is a place holder for future addition.

## 10.7	Miscellaneous attribute typestc  \l 2 "10.7	Miscellaneous attribute types"§

### 10.7.1	Events relatedtc  \l 3 "10.7.1	Events related"§

#### 10.7.1.1	Additional informationtc  \l 4 "10.7.1.1	Additional information"§

The semantics of the additionalInformation attribute type are specified in the Additional Information parameter in CCITT Rec. X.733 | ISO/IEC 10164-4.

additionalInformation	ATTRIBUTE

WITH ATTRIBUTE SYNTAX Attribute-ASN1Module.AdditionalInformation;

BEHAVIOUR

additionalInformationBehaviour	BEHAVIOUR

DEFINED AS	"This attribute is used to supply additional information in notifications.";;

REGISTERED AS	 {smi2AttributeID 6};

#### 10.7.1.2	Additional texttc  \l 4 "10.7.1.2	Additional text"§

The semantics of the additionalText attribute type are specified in the Additional Text parameter in CCITT Rec. X.733 | ISO/IEC 10164-4.

additionalText	ATTRIBUTE

WITH ATTRIBUTE SYNTAX Attribute-ASN1Module.AdditionalText;

MATCHES FOR EQUALITY, SUBSTRINGS;

BEHAVIOUR

additionalTextBehaviour	BEHAVIOUR

DEFINED AS	"This attribute is used to specify additional textual information in notifications ";;

REGISTERED AS	 {smi2AttributeID 7};

#### 10.7.1.3	Attribute identifier listtc  \l 4 "10.7.1.3	Attribute identifier list"§

The semantics of the attributeIdentifierList attribute type are specified in the Attribute Identifier List parameter in CCITT Rec. X.730 | ISO/IEC 10164-1.

attributeIdentifierList	ATTRIBUTE

WITH ATTRIBUTE SYNTAX Attribute-ASN1Module.AttributeIdentifierList;

MATCHES FOR EQUALITY, SET-COMPARISON, SET-INTERSECTION ;

BEHAVIOUR

attributeIdentifierListBehaviour	BEHAVIOUR

DEFINED AS	"This attribute contains a list of attribute identifiers.";;

REGISTERED AS	 {smi2AttributeID 8};

#### 10.7.1.4	Attribute listtc  \l 4 "10.7.1.4	Attribute list"§

The semantics of the attributeList attribute type are specified in the Attribute List parameter in CCITT Rec. X.730 | ISO/IEC 10164-1.

attributeList	ATTRIBUTE

WITH ATTRIBUTE SYNTAX Attribute-ASN1Module.AttributeList;

MATCHES FOR EQUALITY, SET-COMPARISON, SET-INTERSECTION ;

BEHAVIOUR

attributeListBehaviour	BEHAVIOUR

DEFINED AS	"This attribute contains a list of attribute identifiers and their values.";;

REGISTERED AS	 {smi2AttributeID 9};

#### 10.7.1.5	Attribute value change definitiontc  \l 4 "10.7.1.5	Attribute value change definition"§

The semantics of the attributeValueChangeDefinition attribute type are defined in the Attribute Value Change definition parameter in CCITT Rec. X.730 | ISO/IEC 10164-1.

attributeValueChangeDefinition	ATTRIBUTE

WITH ATTRIBUTE SYNTAX Attribute-ASN1Module.AttributeValueChangeDefinition;

BEHAVIOUR

attributeValueChangeDefinitionBehaviour	BEHAVIOUR

DEFINED AS	"This attribute contains a set of attribute identifiers and their old and new values.";;

REGISTERED AS	{smi2AttributeID 10};

#### 10.7.1.6	Backed-up statustc  \l 4 "10.7.1.6	Backed-up status"§

The semantics of the backedUpStatus attribute type are specified in the Backed-up Status parameter in CCITT Rec. X.733 | ISO/IEC 10164-4.

backedUpStatus	ATTRIBUTE

WITH ATTRIBUTE SYNTAX Attribute-ASN1Module.BackedUpStatus;

MATCHES FOR EQUALITY ;

REGISTERED AS	{smi2AttributeID 11};

#### 10.7.1.7	Correlated notificationstc  \l 4 "10.7.1.7	Correlated notifications"§

The semantics of the correlatedNotifications attribute type are specified in the Correlated Notifications parameter in CCITT Rec. X.733 | ISO/IEC 10164-4.

correlatedNotifications	ATTRIBUTE

WITH ATTRIBUTE SYNTAX Attribute-ASN1Module.CorrelatedNotifications;

BEHAVIOUR

correlatedNotificationsBehaviour	BEHAVIOUR

DEFINED AS	"This attribute contains a set of notification identifiers and, if necessary, their associated managed object instance names, for the notifications that are considered to be correlated to this notification.";;

REGISTERED AS	 {smi2AttributeID 12};

#### 10.7.1.8	Event timetc  \l 4 "10.7.1.8	Event time"§

The semantics of the eventTime attribute type are specified in the Event Time parameter in CCITT Rec. X.710 | ISO/IEC 9595.

eventTime	ATTRIBUTE

WITH ATTRIBUTE SYNTAX Attribute-ASN1Module.EventTime;

MATCHES FOR EQUALITY, ORDERING;

BEHAVIOUR	timeOrdering;

REGISTERED AS	{smi2AttributeID 13};

timeOrdering	BEHAVIOUR

DEFINED AS

"The year, month, day, hour, minute and seconds field are compared in order to determine whether the specified value is greater or less than the value of the attribute. The values for the year, month,day, hour,minute and seconds are determined from the character string representation and the year value is first compared. If equal the month value is compared and this process is continued if the compared fields  are equal";

#### 10.7.1.9	Event type

The semantics of the eventType attribute type are specified in the Event type parameter in CCITT Rec. X.710 | ISO/IEC 9595.

eventType	ATTRIBUTE

WITH ATTRIBUTE SYNTAX Attribute-ASN1Module.EventTypeId;

MATCHES FOR EQUALITY ;

REGISTERED AS	{smi2AttributeID 14};

#### 10.7.1.10	 Monitored attributestc  \l 4 "10.7.1.10	Monitored attributes"§

The semantics of the monitoredAttributes attribute type are specified in the Monitored Attributes parameter in CCITT Rec. X.733 | ISO/IEC 10164-4.

monitoredAttributes	ATTRIBUTE

WITH ATTRIBUTE SYNTAX Attribute-ASN1Module.MonitoredAttributes;

REGISTERED AS	{smi2AttributeID 15};

#### 10.7.1.11 Notification identifiertc  \l 4 "10.7.1.11	Notification identifier"§

The semantics of the notificationIdentifier attribute type are specified in the Notification identifier parameter in CCITT Rec. X.733 | ISO/IEC 10164-4.

notificationIdentifier	ATTRIBUTE

WITH ATTRIBUTE SYNTAX Attribute-ASN1Module.NotificationIdentifier;

MATCHES FOR EQUALITY;

BEHAVIOUR

notificationIdentifierBehaviour	BEHAVIOUR

DEFINED AS	"This attribute contains a unique identifier for the notification, which may be present in the correlated notifications attribute of other notifications.";;

REGISTERED AS	 {smi2AttributeID 16};

#### 10.7.1.12 Perceived severitytc  \l 4 "10.7.1.12	Perceived severity"§

The semantics of the perceivedSeverity attribute type are specified in the Perceived Severity parameter in CCITT Rec. X.733 | ISO/IEC 10164-4.

perceivedSeverity	ATTRIBUTE

WITH ATTRIBUTE SYNTAX Attribute-ASN1Module.PerceivedSeverity;

MATCHES FOR EQUALITY;

REGISTERED AS	{smi2AttributeID 17};

#### 10.7.1.13 Probable causetc  \l 4 "10.7.1.13	Probable cause"§

The semantics of the probableCause attribute type are specified in the Probable Cause parameter in CCITT Rec. X.733 | ISO/IEC 10164-4.

probableCause	ATTRIBUTE

WITH ATTRIBUTE SYNTAX Attribute-ASN1Module.ProbableCause;

MATCHES FOR EQUALITY ;

REGISTERED AS	{smi2AttributeID 18};

#### 10.7.1.14 Proposed repair actionstc  \l 4 "10.7.1.14	Proposed repair actions"§

The semantics of the proposedRepairActions attribute type are specified in the Proposed Repair Action parameter in CCITT Rec. X.733 | ISO/IEC 10164-4.

proposedRepairActions	ATTRIBUTE

WITH ATTRIBUTE SYNTAX Attribute-ASN1Module.ProposedRepairActions;

MATCHES FOR EQUALITY, SET-COMPARISON, SET-INTERSECTION;

REGISTERED AS	{smi2AttributeID 19};

#### 10.7.1.15 Relationship change definitiontc  \l 4 "10.7.1.15	Relationship change definition"§

The semantics of the relationshipChangeDefinition attribute type are defined in the Relationship change definition parameter in CCITT Rec. X.732 | ISO/IEC 10164-3.

relationshipChangeDefinition	ATTRIBUTE

WITH ATTRIBUTE SYNTAX Attribute-ASN1Module.AttributeValueChangeDefinition;

BEHAVIOUR

relationshipChangeDefinitionBehaviour	BEHAVIOUR

DEFINED AS	"This attribute contains a set of relationship attribute identifiers and their old and new values.";;

REGISTERED AS	{smi2AttributeID 20};

#### 10.7.1.16 Security alarm causetc  \l 4 "10.7.1.16	Security alarm cause"§

The semantics of the securityAlarmCause attribute type are specified in the Security alarm cause parameter in CCITT Rec. X.736 | ISO/IEC 10164-7.

securityAlarmCause	ATTRIBUTE

WITH ATTRIBUTE SYNTAX Attribute-ASN1Module.SecurityAlarmCause;

MATCHES FOR EQUALITY ;

BEHAVIOUR

securityAlarmCauseBehaviour	BEHAVIOUR

DEFINED AS	"This attribute specifies the cause of the security alarm";;

REGISTERED AS	{smi2AttributeID 21};

#### 10.7.1.17 Security alarm detectortc  \l 4 "10.7.1.17	Security alarm detector"§

The semantics of the securityAlarmDetector attribute type are specified in the Security alarm Detector parameter in CCITT Rec. X.736 | ISO/IEC 10164-7.

securityAlarmDetector	ATTRIBUTE

WITH ATTRIBUTE SYNTAX Attribute-ASN1Module.SecurityAlarmDetector;

MATCHES FOR EQUALITY;

BEHAVIOUR

securityAlarmDetectorBehaviour	BEHAVIOUR

DEFINED AS	"This attribute identifies the entity that detected the security alarm";;

REGISTERED AS	{smi2AttributeID 22};

#### 10.7.1.18 Security alarm severitytc  \l 4 "10.7.1.18	Security alarm severity"§

The semantics of the securityAlarmSeverity attribute type are specified in the Security alarm severity parameter in CCITT Rec. X.736 | ISO/IEC 10164-7.

securityAlarmSeverity	ATTRIBUTE

WITH ATTRIBUTE SYNTAX Attribute-ASN1Module.SecurityAlarmSeverity;

MATCHES FOR EQUALITY;

BEHAVIOUR

securityAlarmSeverityBehaviour	BEHAVIOUR

DEFINED AS	"This attribute indicates the severity of the security alarm";;

REGISTERED AS	{smi2AttributeID 23};

#### 10.7.1.19	 Service providertc  \l 4 "10.7.1.19	Service provider"§

The semantics of the serviceProvider attribute type are specified in the Service provider parameter in CCITT Rec. X.736 | ISO/IEC 10164-7.

serviceProvider	ATTRIBUTE

WITH ATTRIBUTE SYNTAX Attribute-ASN1Module.ServiceProvider;

MATCHES FOR EQUALITY;

BEHAVIOUR

serviceProviderBehaviour	BEHAVIOUR

DEFINED AS	"This attribute contains information about the service provider associated with the service request that caused the security alarm";;

REGISTERED AS	{smi2AttributeID 24};

#### 10.7.1.20 Service usertc  \l 4 "10.7.1.20	Service user"§

The semantics of the serviceUser attribute type are specified in the Service user parameter in CCITT Rec. X.736 | ISO/IEC 10164-7.

serviceUser	ATTRIBUTE

WITH ATTRIBUTE SYNTAX Attribute-ASN1Module.ServiceUser;

MATCHES FOR EQUALITY ;

BEHAVIOUR

serviceUserBehaviour	BEHAVIOUR

DEFINED AS	"This attribute contains information about the service user associated with the service request that caused the security alarm";;

REGISTERED AS	{smi2AttributeID 25};

#### 10.7.1.21 Source indicatortc  \l 4 "10.7.1.21	Source indicator"§

The semantics of the sourceIndicator attribute type are defined in the Source Indicator parameter in CCITT Rec. X.730 | ISO/IEC 10164-1.

sourceIndicator	ATTRIBUTE

WITH ATTRIBUTE SYNTAX Attribute-ASN1Module.SourceIndicator;

MATCHES FOR EQUALITY;

BEHAVIOUR

sourceIndicatorBehaviour	BEHAVIOUR

DEFINED AS	"This attribute is used in notifications to indicate the source of the operation that resulted in generating the notification ";;

REGISTERED AS	{smi2AttributeID 26};

#### 10.7.1.22 Specific problemstc  \l 4 "10.7.1.22	Specific problems"§

The semantics of the specificProblems attribute type are specified in the Specific Problems parameter in CCITT Rec. X.733 | ISO/IEC 10164-4.

specificProblems	 ATTRIBUTE

WITH ATTRIBUTE SYNTAX Attribute-ASN1Module.SpecificProblems;

MATCHES FOR EQUALITY, SET-COMPARISON, SET-INTERSECTION ;

REGISTERED AS	{smi2AttributeID 27};

#### 10.7.1.23	 State change definitiontc  \l 4 "10.7.1.23	State change definition"§

The semantics of the stateChangeDefinition attribute type are defined in the State change definition parameter in CCITT Rec. X.731 | ISO/IEC 10164-2.

stateChangeDefinition	ATTRIBUTE

WITH ATTRIBUTE SYNTAX Attribute-ASN1Module.AttributeValueChangeDefinition;

BEHAVIOUR

stateChangeDefinitionBehaviour	BEHAVIOUR

DEFINED AS	"This attribute contains a set of state attribute identifiers and their old and new values.";;

REGISTERED AS	{smi2AttributeID 28};

#### 10.7.1.24	 Threshold infotc  \l 4 "10.7.1.24	Threshold info"§

The semantics of the thresholdInfo attribute type are specified in the Threshold Info parameter in CCITT Rec. X.733 | ISO/IEC 10164-4.

thresholdInfo	ATTRIBUTE

WITH ATTRIBUTE SYNTAX Attribute-ASN1Module.ThresholdInfo;

REGISTERED AS	{smi2AttributeID 29};

#### 10.7.1.25 Trend indicationtc  \l 4 "10.7.1.25	Trend indication"§

The semantics of the trendIndication attribute type are specified in the Trend Indication parameter in CCITT Rec. X.733 | ISO/IEC 10164-4.

trendIndication	ATTRIBUTE

WITH ATTRIBUTE SYNTAX Attribute-ASN1Module.TrendIndication;

MATCHES FOR EQUALITY;

REGISTERED AS	{smi2AttributeID 30};

### 10.7.2	States relatedtc  \l 3 "10.7.2	States related"§

#### 10.7.2.1	Administrative statetc  \l 4 "10.7.2.1	Administrative state"§

The semantics of the administrativeState attribute type are specified in the Administrative State attribute in CCITT Rec. X.731 | ISO/IEC 10164-2.

administrativeState	ATTRIBUTE

WITH ATTRIBUTE SYNTAX Attribute-ASN1Module.AdministrativeState;

MATCHES FOR EQUALITY ;

REGISTERED AS	{smi2AttributeID 31};

#### 10.7.2.2	Alarm statustc  \l 4 "10.7.2.2	Alarm status"§

The semantics of the alarmStatus attribute type are specified in the Alarm Status attribute in CCITT Rec. X.731 | ISO/IEC 10164-2.

alarmStatus	ATTRIBUTE

WITH ATTRIBUTE SYNTAX Attribute-ASN1Module.AlarmStatus;

MATCHES FOR EQUALITY, SET-COMPARISON, SET-INTERSECTION ;

REGISTERED AS	{smi2AttributeID 32};

#### 10.7.2.3	Availability statustc  \l 4 "10.7.2.3	Availability status"§

The semantics of the availabilityStatus attribute type are specified in the Availability Status attribute in CCITT Rec. X.731 | ISO/IEC 10164-2.

availabilityStatus	ATTRIBUTE

WITH ATTRIBUTE SYNTAX Attribute-ASN1Module.AvailabilityStatus;

MATCHES FOR EQUALITY, SET-COMPARISON, SET-INTERSECTION ;

REGISTERED AS	{smi2AttributeID 33};

#### 10.7.2.4	Control statustc  \l 4 "10.7.2.4	Control status"§

The semantics of the controlStatus attribute type are specified in the Control Status attribute in CCITT Rec. X.731 | ISO/IEC 10164-2.

controlStatus	ATTRIBUTE

WITH ATTRIBUTE SYNTAX Attribute-ASN1Module.ControlStatus;

MATCHES FOR EQUALITY, SET-COMPARISON, SET-INTERSECTION ;

REGISTERED AS	{smi2AttributeID 34};

##### 4.10.7.2.5 Operational state

The semantics of the operationalState attribute type are specified in the Operational State attribute in CCITT Rec. X.731 | ISO/IEC 10164-2.

operationalState	ATTRIBUTE

WITH ATTRIBUTE SYNTAX Attribute-ASN1Module.OperationalState;

MATCHES FOR EQUALITY ;

REGISTERED AS	{smi2AttributeID 35};

#### 10.7.2.6	Procedural statustc  \l 4 "10.7.2.6	Procedural status"§

The semantics of the proceduralStatus attribute type are specified in the Procedural Status attribute in CCITT Rec. X.731 | ISO/IEC 10164-2.

proceduralStatus	ATTRIBUTE

WITH ATTRIBUTE SYNTAX Attribute-ASN1Module.ProceduralStatus;

MATCHES FOR EQUALITY, SET-COMPARISON, SET-INTERSECTION;

REGISTERED AS	{smi2AttributeID 36};

#### 10.7.2.7	Standby statustc  \l 4 "10.7.2.7	Standby status"§

The semantics of the standbyStatus attribute type are specified in the Standby Status attribute in CCITT Rec. X.731 | ISO/IEC 10164-2.

standbyStatus	ATTRIBUTE

WITH ATTRIBUTE SYNTAX Attribute-ASN1Module.StandbyStatus;

MATCHES FOR EQUALITY;

REGISTERED AS	{smi2AttributeID 37};

#### 10.7.2.8	Unknown statustc  \l 4 "10.7.2.8	Unknown status"§

The semantics of the unknownStatus attribute type are specified in the Unknown Status attribute in CCITT Rec. X.731 | ISO/IEC 10164-2.

unknownStatus	ATTRIBUTE

WITH ATTRIBUTE SYNTAX Attribute-ASN1Module.UnknownStatus;

MATCHES FOR EQUALITY;

REGISTERED AS	{smi2AttributeID 38};

#### 10.7.2.9	Usage statetc  \l 4 "10.7.2.9	Usage state"§

The semantics of the usageState attribute type are specified in the Usage State attribute in CCITT Rec. X.731 | ISO/IEC 10164-2.

usageState	ATTRIBUTE

WITH ATTRIBUTE SYNTAX Attribute-ASN1Module.UsageState;

MATCHES FOR EQUALITY;

REGISTERED AS	{smi2AttributeID 39};

#### 10.7.2.10 Statetc  \l 4 "10.7.2.10	State"§

The semantics of the State group attribute type are specified in the State attribute group in CCITT Rec. X.731 | ISO/IEC 10164-2.

state	ATTRIBUTE GROUP

DESCRIPTION " This is defined as an empty attribute group. The elements of this group are composed of state attributes in the managed object. The state attributes may include those specified in CCITT Rec. X.731|ISO/IEC 10164-2 and others that are specific to the managed object class.";

REGISTERED AS	{smi2AttributeGroup 1};

### 10.7.3	Relationships relatedtc  \l 3 "10.7.3	Relationships related"§

#### 10.7.3.1	Back-up objecttc  \l 4 "10.7.3.1	Back-up object"§

The semantics of the backUpObject attribute type are specified in Back-Up Object relationship attribute in CCITT Rec. X.732 | ISO/IEC 10164-3. This attribute is also used in CCITT Rec. X.733 | ISO/IEC 10164-4.

backUpObject	ATTRIBUTE

WITH ATTRIBUTE SYNTAX Attribute-ASN1Module.BackUpRelationshipObject;

MATCHES FOR EQUALITY;

REGISTERED AS	{smi2AttributeID 40};

#### 10.7.3.2	Backed-up objecttc  \l 4 "10.7.3.2	Backed-up object"§

The semantics of the backedUpObject attribute type are specified in Backed-Up Object relationship attribute in CCITT Rec. X.732 | ISO/IEC 10164-3.

backedUpObject	ATTRIBUTE

WITH ATTRIBUTE SYNTAX Attribute-ASN1Module.BackUpRelationshipObject;

MATCHES FOR EQUALITY;

REGISTERED AS	{smi2AttributeID 41};

#### 10.7.3.3	Membertc  \l 4 "10.7.3.3	Member"§

The semantics of member attribute type are specified in Member relationship attribute in CCITT Rec. X.732 | ISO/IEC 10164-3.

member	ATTRIBUTE

WITH ATTRIBUTE SYNTAX Attribute-ASN1Module.GroupObjects;

MATCHES FOR EQUALITY,SET-COMPARISON, SET-INTERSECTION;

REGISTERED AS	{smi2AttributeID 42};

#### 10.7.3.4	Ownertc  \l 4 "10.7.3.4	Owner"§

The semantics of owner attribute type are specified in Owner relationship attribute in CCITT Rec. X.732 | ISO/IEC 10164-3.

owner	ATTRIBUTE

WITH ATTRIBUTE SYNTAX Attribute-ASN1Module.GroupObjects;

MATCHES FOR EQUALITY, SET-COMPARISON, SET-INTERSECTION;

REGISTERED AS	{smi2AttributeID 43};

#### 10.7.3.5	Peertc  \l 4 "10.7.3.5	Peer"§

The semantics of peer attribute type are specified in Peer relationship attribute in CCITT Rec. X.732 | ISO/IEC 10164-3.

peer	ATTRIBUTE

WITH ATTRIBUTE SYNTAX Attribute-ASN1Module.BackUpRelationshipObject;

MATCHES FOR EQUALITY;

REGISTERED AS	{smi2AttributeID 44};

#### 10.7.3.6	Primarytc  \l 4 "10.7.3.6	Primary"§

The semantics of primary attribute type are specified in Primary relationship attribute in CCITT Rec. X.732 | ISO/IEC 10164-3.

primary	 ATTRIBUTE

WITH ATTRIBUTE SYNTAX Attribute-ASN1Module.PrioritisedObject;

REGISTERED AS	{smi2AttributeID 45};

#### 10.7.3.7	Provider objecttc  \l 4 "10.7.3.7	Provider object"§

The semantics of providerObject attribute type are specified in Provider Object relationship attribute in CCITT Rec. X.732 | ISO/IEC 10164-3.

providerObject	ATTRIBUTE

WITH ATTRIBUTE SYNTAX Attribute-ASN1Module.PrioritisedObject;

REGISTERED AS	{smi2AttributeID 46};

#### 10.7.3.8	Relationshipstc  \l 4 "10.7.3.8	Relationships"§

The semantics of the relationships group attribute type are specified in the Relationships group attribute in CCITT Rec. X.732 | ISO/IEC 10164-3.

relationships	ATTRIBUTE GROUP

DESCRIPTION " This is defined as an empty attribute group. The elements of this group are composed of relationship attributes in the managed object. The relatioship attributes may include those specified in CCITT Rec. X.732 | ISO/IEC 10164-3 and others that are specific to the managed object class.";

REGISTERED AS	{smi2AttributeGroup 2};

#### 10.7.3.9	Secondarytc  \l 4 "10.7.3.9	Secondary"§

The semantics of secondary attribute type are used in defining Secondary relationship attribute in CCITT Rec. X.732 | ISO/IEC 10164-3.

secondary	ATTRIBUTE

WITH ATTRIBUTE SYNTAX Attribute-ASN1Module.PrioritisedObject;

REGISTERED AS	{smi2AttributeID 47};

#### 10.7.3.10 User objecttc  \l 4 "10.7.3.10	User object"§

The semantics of userObject attribute type are used in defining User Object Relationship attribute in CCITT Rec. X.732 | ISO/IEC 10164-3.

userObject	ATTRIBUTE

WITH ATTRIBUTE SYNTAX Attribute-ASN1Module.PrioritisedObject;

REGISTERED AS	{smi2AttributeID 48};

### 10.7.4	Other attribute typestc  \l 3 "10.7.4	Other attribute types"§

#### 10.7.4.1	Active destinationtc  \l 4 "10.7.4.1	Active destination"§

The semantics of the activeDestination attribute type are specified in the Active destination attribute in CCITT Rec. X.734 | ISO/IEC 10164-5.

activeDestination	ATTRIBUTE

WITH ATTRIBUTE SYNTAX Attribute-ASN1Module.ActiveDestination;

MATCHES FOR EQUALITY;

REGISTERED AS	{smi2AttributeID 49};

#### 10.7.4.2	Allomorphstc  \l 4 "10.7.4.2	Allomorphs"§

The allomorphs attribute appears in a managed object if and only if that managed object supports allomorphism.

allomorphs	ATTRIBUTE

WITH ATTRIBUTE SYNTAX Attribute-ASN1Module.Allomorphs;

MATCHES FOR EQUALITY, SET-COMPARISON, SET-INTERSECTION;

REGISTERED AS	{smi2AttributeID 50};

#### 10.7.4.3	Back-up destination listtc  \l 4 "10.7.4.3	Back-up destination list"§

The semantics of the backUpDestinationList attribute type are specified in the Backup destination list attribute in CCITT Rec. X.734 | ISO/IEC 10164-5.

backUpDestinationList	ATTRIBUTE

WITH ATTRIBUTE SYNTAX Attribute-ASN1Module.BackUpDestinationList;

MATCHES FOR EQUALITY ,SET-COMPARISON, SET-INTERSECTION;

REGISTERED AS	{smi2AttributeID 51};

#### 10.7.4.4	Capacity alarm thresholdtc  \l 4 "10.7.4.4	Capacity alarm threshold"§

The semantics of the capacityAlarmThreshold attribute type are specified in the Capacity Alarm Threshold attribute in CCITT Rec. X.735 | ISO/IEC10164-6.

capacityAlarmThreshold	ATTRIBUTE

WITH ATTRIBUTE SYNTAX Attribute-ASN1Module.CapacityAlarmThreshold;

MATCHES FOR EQUALITY, SET-COMPARISON, SET-INTERSECTION;

REGISTERED AS	{smi2AttributeID 52};

#### 10.7.4.5	Confirmed modetc  \l 4 "10.7.4.5	Confirmed mode"§

The semantics of the confirmedMode attribute type are specified in the Confirmed mode attribute in CCITT Rec. X.734 | ISO/IEC10164-5.

confirmedMode	ATTRIBUTE

WITH ATTRIBUTE SYNTAX Attribute-ASN1Module.ConfirmedMode;

MATCHES FOR EQUALITY;

REGISTERED AS	{smi2AttributeID 53};

#### 10.7.4.6	Current log sizetc  \l 4 "10.7.4.6	Current log size"§

The semantics of the currentLogSize attribute type are specified in the Current Log Size attribute in CCITT Rec. X.735 | ISO/IEC 10164-6.

currentLogSize	ATTRIBUTE

WITH ATTRIBUTE SYNTAX Attribute-ASN1Module.CurrentLogSize;

MATCHES FOR EQUALITY, ORDERING;

REGISTERED AS	{smi2AttributeID 54};

#### 10.7.4.7	Destinationtc  \l 4 "10.7.4.7	Destination"§

The semantics of the destination attribute type are specified in the Destination Address attribute in CCITT Rec. X.734 | ISO/IEC 10164-5.

destination	ATTRIBUTE

WITH ATTRIBUTE SYNTAX Attribute-ASN1Module.Destination;

MATCHES FOR EQUALITY ;

REGISTERED AS	{smi2AttributeID 55};

#### 10.7.4.8	Discriminator constructtc  \l 4 "10.7.4.8	Discriminator construct"§

The semantics of the discriminatorConstruct attribute type are specified in the Discriminator Construct attribute in CCITT Rec. X.734 | ISO/IEC 10164-5.

discriminatorConstruct	ATTRIBUTE

WITH ATTRIBUTE SYNTAX Attribute-ASN1Module.DiscriminatorConstruct;

REGISTERED AS	{smi2AttributeID 56};

#### 10.7.4.9	Intervals of daytc  \l 4 "10.7.4.9	Intervals of day"§

The semantics of the intervalsOfDay attribute type are specified in the Daily Scheduling package in CCITT Rec. X.734 | ISO/IEC 10164-5 and CCITT Rec. X.735 | ISO/IEC 10164-6.

intervalsOfDay	ATTRIBUTE

WITH ATTRIBUTE SYNTAX Attribute-ASN1Module.IntervalsOfDay;

REGISTERED AS	{smi2AttributeID 57};

#### 10.7.4.10 Log full actiontc  \l 4 "10.7.4.10	Log full action"§

The semantics of the logFullAction attribute type are specified in the Log Full Action attribute in CCITT Rec. X.735 | ISO/IEC 10164-6.

logFullAction	ATTRIBUTE

WITH ATTRIBUTE SYNTAX Attribute-ASN1Module.LogFullAction;

MATCHES FOR EQUALITY ;

REGISTERED AS	{smi2AttributeID 58};

#### 10.7.4.11 Logging timetc  \l 4 "10.7.4.11	Logging time"§

The semantics of the loggingTime attribute type are specified in the Logging Time attribute in CCITT Rec. X.735 | ISO/IEC 10164-6.

loggingTime	ATTRIBUTE

WITH ATTRIBUTE SYNTAX Attribute-ASN1Module.LoggingTime;

MATCHES FOR EQUALITY, ORDERING;

REGISTERED AS	{smi2AttributeID 59};

#### 10.7.4.12 Managed object classtc  \l 4 "10.7.4.12	Managed object class"§

The managedObjectClass attribute type is specified to allow filtering of the Managed Object Class parameter when events reports are logged as records.

managedObjectClass	ATTRIBUTE

WITH ATTRIBUTE SYNTAX Attribute-ASN1Module.ObjectClass;

MATCHES FOR EQUALITY;

REGISTERED AS	{smi2AttributeID 60};

#### 10.7.4.13 Managed object instancetc  \l 4 "10.7.4.13	Managed object instance"§

The managedObjectInstance attribute type is specified to allow filtering of the Managed Object Instance parameter when events reports are logged as records.

managedObjectInstance	ATTRIBUTE

WITH ATTRIBUTE SYNTAX Attribute-ASN1Module.ObjectInstance;

MATCHES FOR EQUALITY;

REGISTERED AS	{smi2AttributeID 61};

#### 10.7.4.14 Max log sizetc  \l 4 "10.7.4.14	Max log size"§

The semantics of the maxLogSize attribute type are specified in the Max Log Size attribute in CCITT Rec. X.735 | ISO/IEC 10164-6.

maxLogSize	ATTRIBUTE

WITH ATTRIBUTE SYNTAX Attribute-ASN1Module.MaxLogSize;

MATCHES FOR EQUALITY, ORDERING ;

BEHAVIOUR

maxSizeOrderingBehaviour	BEHAVIOUR

DEFINED AS " The ordering is the same as for sequentially increasing postive integers except that a value of zero is largest and denotes infinite size.";;

REGISTERED AS	{smi2AttributeID 62};

#### 10.7.4.15 Name bindingtc  \l 4 "10.7.4.15	Name binding"§

The nameBinding attribute appears in every managed object and identifies the name binding with which the managed object was instantiated. The semantics of this attribute are described in CCITT Rec. X.720 | ISO/IEC 10165-1.

nameBinding	ATTRIBUTE

WITH ATTRIBUTE SYNTAX Attribute-ASN1Module.NameBinding;

MATCHES FOR EQUALITY;

REGISTERED AS	{smi2AttributeID 63};

#### 10.7.4.16	 Number of recordstc  \l 4 "10.7.4.16	Number of records"§

The semantics of the numberOfRecords attribute type are specified in the Number of Records attribute in CCITT Rec. X.735 | ISO/IEC 10164-6.

numberOfRecords	ATTRIBUTE

WITH ATTRIBUTE SYNTAX Attribute-ASN1Module.NumberOfRecords;

MATCHES FOR EQUALITY, ORDERING ;

REGISTERED AS	{smi2AttributeID 64};

#### 10.7.4.17 Object classtc  \l 4 "10.7.4.17	Object class"§

The objectClass attribute appears in every managed object and indicates the managed object class to which the managed object belongs. The semantics of ordering on object classes are described in CCITT Rec. X.720 | ISO/IEC 10165-1.

objectClass	ATTRIBUTE

WITH ATTRIBUTE SYNTAX Attribute-ASN1Module.ObjectClass;

MATCHES FOR EQUALITY;

REGISTERED AS	{smi2AttributeID 65};

#### 10.7.4.18 Packagestc  \l 4 "10.7.4.18	Packages"§

The packages attribute appears in every managed object in which any registered package, other than the packagePackage has been instantiated.

packages	ATTRIBUTE

WITH ATTRIBUTE SYNTAX Attribute-ASN1Module.Packages;

MATCHES FOR EQUALITY, SET-COMPARISON, SET-INTERSECTION;

REGISTERED AS	{smi2AttributeID 66};

#### 10.7.4.19 Scheduler nametc  \l 4 "10.7.4.19	Scheduler name"§

The semantics of schedulerName attribute type are specified in the External Scheduler scheduling package in CCITT Rec. X.734 | ISO/IEC 10164-5 and CCITT Rec. X.735 | ISO/IEC 10164-6.

schedulerName	ATTRIBUTE

WITH ATTRIBUTE SYNTAX Attribute-ASN1Module.ObjectInstance;

MATCHES FOR EQUALITY;

REGISTERED AS	{smi2AttributeID 67};

#### 10.7.4.20 Start timetc  \l 4 "10.7.4.20	Start time"§

The semantics of startTime attribute type are specified in the Weekly Scheduling package in CCITT Rec. X.734 | ISO/IEC 10164-5 and CCITT Rec. X.735 | ISO/IEC 10164-6.

startTime	ATTRIBUTE

WITH ATTRIBUTE SYNTAX Attribute-ASN1Module.StartTime;

MATCHES FOR EQUALITY, ORDERING;

BEHAVIOUR timeOrdering;

REGISTERED AS	{smi2AttributeID 68};

#### 10.7.4.21 Stop timetc  \l 4 "10.7.4.21	Stop time"§

The semantics of stopTime attribute type are specified in the Weekly Scheduling package in CCITT Rec. X.734 | ISO/IEC 10164-5 and CCITT Rec. X.735 | ISO/IEC 10164-6. The matching criterion "Ordering" is applicable only when the ASN.1 type GeneralizedTime choice is selected for StopTime.

stopTime	ATTRIBUTE

WITH ATTRIBUTE SYNTAX Attribute-ASN1Module.StopTime;

MATCHES FOR EQUALITY, ORDERING;

BEHAVIOUR timeOrdering;

REGISTERED AS	{smi2AttributeID 69};

#### 10.7.4.22 Supported Featurestc  \l 4 "10.7.4.22	Supported Features"§

The semantics of the supportedFeatures  attribute type are given below.

supportedFeatures	ATTRIBUTE

WITH ATTRIBUTE SYNTAX Attribute-ASN1Module.SupportedFeatures;

MATCHES FOR SET-COMPARISON, SET-INTERSECTION, EQUALITY;

BEHAVIOUR

supportedFeaturesBehaviour	BEHAVIOUR

DEFINED AS	"This attribute is used to identify features within the system that are capable of being managed. The registration of each feature is performed by the development authority for the feature and will typically identify a specification in which the details of what can be managed are provided";;

REGISTERED AS	{smi2AttributeID 70};

#### 10.7.4.23 Week masktc  \l 4 "10.7.4.23	Week mask"§

The semantics of weekMask attribute type are specified in the Weekly Scheduling package in CCITT Rec. X.734 | ISO/IEC 10164-5 and CCITT Rec. X.735 | ISO/IEC 10164-6.

weekMask	ATTRIBUTE

WITH ATTRIBUTE SYNTAX Attribute-ASN1Module.WeekMask;

MATCHES FOR EQUALITY;

REGISTERED AS	{smi2AttributeID 71};

# 11	Definition of action typestc  \l 1 "11 Definition of action types"§

This Recommendation | International Standard defines a number of action types which are referenced by the System Management Functions in CCITT Recs. X.730-736 | ISO/IEC 10164 Parts 1 to 7.

NOTE – There are no action types currently defined in the system management functions and this section is a place holder for future addition.

# 12	Definition of parameterstc  \l 1 "12 Definition of parameters"§

This Recommendation | International Standard defines a number of parameters which are referenced by the System Management Functions in CCITT Recs. X.730-736 | ISO/IEC 10164 Parts 1 to 7. The definition of parameter template is specified in CCITT Rec. X.722 | ISO/IEC 10165-4.

The miscellaneousError parameter is included in this Recommendation | International Standard as a possible specific error when processing failure is reported using CMIP linked reply mechanism. This may be imported in any managed object class definition .

miscellaneousError	PARAMETER

CONTEXT	SPECIFIC-ERROR;

WITH SYNTAX Parameter-ASN1Module.MiscellaneousError;

BEHAVIOUR

miscellaneousErrorBehaviour	BEHAVIOUR

DEFINED AS " When a processing error failure has occurred and the error condition encountered does not match any of the object's defined specific error types, this value is used.";;

REGISTERED AS	{smi2Parameter 1};

# 13	Definition of notification typestc  \l 1 "13	Definition of notification types"§

This Recommendation | International Standard defines a number of notification types which are applicable to a wide variety of managed object classes.

Each notification type definition comprises

–	the structure of the notification data carried in the management protocol;

–	the behaviour of the notification;

–	the structure of the notification result data carried in the management protocol;

–	assignment of an object identifier value.

The template definition along with the allocated object identifier are specified below for each notification type. The ASN.1 supporting productions are defined in 14.3.

## 13.1	Attribute value changetc  \l 2 "13.1	Attribute value change"§

The semantics of the attributeValueChange notification type are specified in CCITT Rec. X.730 | ISO/IEC 10164-1.

attributeValueChange	NOTIFICATION

BEHAVIOUR	attributeValueChangeBehaviour;

WITH INFORMATION SYNTAX Notification-ASN1Module.AttributeValueChangeInfo

AND ATTRIBUTE IDS

sourceIndicator			sourceIndicator,

attributeIdentifierList		attributeIdentifierList,

attributeValueChangeDefinition	attributeValueChangeDefinition,

notificationIdentifier		notificationIdentifier,

correlatedNotifications		correlatedNotifications,

additionalText			additionalText,

additionalInformation		additionalInformation;

REGISTERED AS	{ smi2Notification 1};

attributeValueChangeBehaviour

BEHAVIOUR

DEFINED AS "This notification type is used to report changes to the attribute such as addition or deletion of members to one or more set valued attributes, replacement of the value of one or more attributes and setting attribute values to their defaults.";

## 13.2	Communications alarmtc  \l 2 "13.2	Communications alarm"§

The semantics of the communicationsAlarm notification type are specified in CCITT Rec. X.733 | ISO/IEC 10164-4.

communicationsAlarm	NOTIFICATION

BEHAVIOUR	communicationsAlarmBehaviour;

WITH INFORMATION SYNTAX Notification-ASN1Module.AlarmInfo

AND ATTRIBUTE IDS

probableCause		probableCause,

specificProblems		specificProblems,

perceivedSeverity	perceivedSeverity,

backedUpStatus		backedUpStatus,

backUpObject		backUpObject,

trendIndication		trendIndication,

thresholdInfo		thresholdInfo,

notificationIdentifier	notificationIdentifier,

correlatedNotifications	correlatedNotifications,

stateChangeDefinition	stateChangeDefinition,

monitoredAttributes	monitoredAttributes,

proposedRepairActions	proposedRepairActions,

additionalText		additionalText,

additionalInformation	additionalInformation;

REGISTERED AS	{smi2Notification 2};

communicationsAlarmBehaviour

BEHAVIOUR

DEFINED AS "This notification type is used to report when the object detects a communications error.";

## 13.3	Environmental alarmtc  \l 2 "13.3	Environmental alarm"§

The semantics of the environmentalAlarm notification type are specified in CCITT Rec. X.733 | ISO/IEC 10164-4.

environmentalAlarm	NOTIFICATION

BEHAVIOUR	environmentalAlarmBehaviour;

WITH INFORMATION SYNTAX Notification-ASN1Module.AlarmInfo

AND ATTRIBUTE IDS

probableCause		probableCause,

specificProblems		specificProblems,

perceivedSeverity	perceivedSeverity,

backedUpStatus		backedUpStatus,

backUpObject		backUpObject,

trendIndication		trendIndication,

thresholdInfo		thresholdInfo,

notificationIdentifier	notificationIdentifier,

correlatedNotifications	correlatedNotifications,

stateChangeDefinition	stateChangeDefinition,

monitoredAttributes	monitoredAttributes,

proposedRepairActions	proposedRepairActions,

additionalText		additionalText,

additionalInformation	additionalInformation;

REGISTERED AS	{smi2Notification 3};

environmentalAlarmBehaviour

BEHAVIOUR

DEFINED AS "This notification type is used to report a problem in the environment.";

## 13.4	Equipment alarmtc  \l 2 "13.4	Equipment alarm"§

The semantics of the equipmentAlarm notification types are specified in CCITT Rec. X.733 | ISO/IEC 10164-4.

equipmentAlarm	NOTIFICATION

BEHAVIOUR	equipmentAlarmBehaviour;

WITH INFORMATION SYNTAX Notification-ASN1Module.AlarmInfo

AND ATTRIBUTE IDS

probableCause		probableCause,

specificProblems		specificProblems,

perceivedSeverity	perceivedSeverity,

backedUpStatus		backedUpStatus,

backUpObject		backUpObject,

trendIndication		trendIndication,

thresholdInfo		thresholdInfo,

notificationIdentifier	notificationIdentifier,

correlatedNotifications	correlatedNotifications,

stateChangeDefinition	stateChangeDefinition,

monitoredAttributes	monitoredAttributes,

proposedRepairActions	proposedRepairActions,

additionalText		additionalText,

additionalInformation	additionalInformation;

REGISTERED AS	{smi2Notification 4};

equipmentAlarmBehaviour

BEHAVIOUR

DEFINED AS "This notification type is used to report a failure in the equipment.";

## 13.5	Integrity violationtc  \l 2 "13.5	Integrity violation"§

The semantics of the integrityViolation notification type are specified in CCITT Rec. X.736 | ISO/IEC 10164-7.

integrityViolation	NOTIFICATION

BEHAVIOUR	integrityViolationBehaviour;

WITH INFORMATION SYNTAX Notification-ASN1Module.SecurityAlarmInfo

AND ATTRIBUTE IDS

securityAlarmCause	securityAlarmCause,

securityAlarmSeverity	securityAlarmSeverity,

securityAlarmDetector	securityAlarmDetector,

serviceUser 		serviceUser,

serviceProvider 		serviceProvider,

notificationIdentifier	notificationIdentifier,

correlatedNotifications	correlatedNotifications,

additionalText		additionalText,

additionalInformation	additionalInformation;

REGISTERED AS	{smi2Notification 5};

integrityViolationBehaviour	BEHAVIOUR

DEFINED AS	"This notification is used to report that a potential interruption in information flow has occurred such that information may have been illegally modified, inserted or deleted.";

## 13.6	Object creationtc  \l 2 "13.6	Object creation"§

The semantics of the objectCreation notification type are specified in CCITT Rec. X.730 | ISO/IEC 10164-1.

objectCreation	NOTIFICATION

BEHAVIOUR	objectCreationBehaviour;

WITH INFORMATION SYNTAX Notification-ASN1Module.ObjectInfo

AND ATTRIBUTE IDS

sourceIndicator		sourceIndicator,

attributeList		attributeList,

notificationIdentifier	notificationIdentifier,

correlatedNotifications	correlatedNotifications,

additionalText		additionalText,

additionalInformation	additionalInformation;

REGISTERED AS	{smi2Notification 6};

objectCreationBehaviour

BEHAVIOUR

DEFINED AS "This notification type is used to report the creation of a managed object to another open system.";

## 13.7	Object deletiontc  \l 2 "13.7	Object deletion"§

The semantics of the objectDeletion notification type are specified in CCITT Rec. X.730 | ISO/IEC 10164-1.

objectDeletion	NOTIFICATION

BEHAVIOUR	objectDeletionBehaviour;

WITH INFORMATION SYNTAX Notification-ASN1Module.ObjectInfo

AND ATTRIBUTE IDS

sourceIndicator		sourceIndicator,

attributeList		attributeList,

notificationIdentifier	notificationIdentifier,

correlatedNotifications	correlatedNotifications,

additionalText		additionalText,

additionalInformation	additionalInformation;

REGISTERED AS	{smi2Notification 7};

objectDeletionBehaviour

BEHAVIOUR

DEFINED AS "This notification type is used to report the deletion of a managed object to another open system.";

## 13.8	Operational violationtc  \l 2 "13.8	Operational violation"§

The semantics of the operationalViolation notification type are specified in CCITT Rec. X.736 | ISO/IEC 10164-7.

operationalViolation	NOTIFICATION

BEHAVIOUR	operationalViolationBehaviour;

WITH INFORMATION SYNTAX Notification-ASN1Module.SecurityAlarmInfo

AND ATTRIBUTE IDS

securityAlarmCause	securityAlarmCause,

securityAlarmSeverity	securityAlarmSeverity,

securityAlarmDetector	securityAlarmDetector ,

serviceUser 		serviceUser,

serviceProvider 		serviceProvider,

notificationIdentifier 	notificationIdentifier,

correlatedNotifications	correlatedNotifications,

additionalText		additionalText,

additionalInformation	additionalInformation;

REGISTERED AS	{smi2Notification 8};

operationalViolationBehaviour	BEHAVIOUR

DEFINED AS	"This notification is used to report that the provision of the requested service was not possible due to the unavailability, malfunction or incorrect invocation of the service.";

## 13.9	Physical violationtc  \l 2 "13.9	Physical violation"§

The semantics of the physicalViolation notification type are specified in CCITT Rec. X.736 | ISO/IEC 10164-7.

physicalViolation	NOTIFICATION

BEHAVIOUR	physicalViolationBehaviour;

WITH INFORMATION SYNTAX Notification-ASN1Module.SecurityAlarmInfo

AND ATTRIBUTE IDS

securityAlarmCause	securityAlarmCause,

securityAlarmSeverity	securityAlarmSeverity,

securityAlarmDetector	securityAlarmDetector ,

serviceUser		serviceUser,

serviceProvider		serviceProvider,

notificationIdentifier	notificationIdentifier,

correlatedNotifications	correlatedNotifications,

additionalText		additionalText,

additionalInformation	additionalInformation;

REGISTERED AS	{ smi2Notification 9};

physicalViolationBehaviour	BEHAVIOUR

DEFINED AS	"This notification is used to report that a physical resource has been violated in a way that indicates a potential security attack.";

## 13.10	Processing error alarmtc  \l 2 "13.10	Processing error alarm"§

The semantics of the processingErrorAlarm notification type are specified in CCITT Rec. X.733 | ISO/IEC 10164-4.

processingErrorAlarm	NOTIFICATION

BEHAVIOUR	 processingErrorAlarmBehaviour;

WITH INFORMATION SYNTAX Notification-ASN1Module.AlarmInfo

AND ATTRIBUTE IDS

probableCause		probableCause,

specificProblems		specificProblems,

perceivedSeverity	perceivedSeverity,

backedUpStatus		backedUpStatus,

backUpObject		backUpObject,

trendIndication		trendIndication,

thresholdInfo		thresholdInfo,

notificationIdentifier	notificationIdentifier,

correlatedNotifications	correlatedNotifications,

stateChangeDefinition	stateChangeDefinition,

monitoredAttributes	monitoredAttributes,

proposedRepairActions	proposedRepairActions,

additionalText		additionalText,

additionalInformation	additionalInformation;

REGISTERED AS	{smi2Notification 10};

processingErrorAlarmBehaviour	BEHAVIOUR

DEFINED AS	"This notification type is used to report processing failure in a managed object.";

## 13.11	Quality of service alarmtc  \l 2 "13.11	Quality of service alarm"§

The semantics of the qualityofServiceAlarm notification type are specified in CCITT Rec. X.733 | ISO/IEC 10164-4.

qualityofServiceAlarm	NOTIFICATION

BEHAVIOUR	qualityofServiceAlarmBehaviour;

WITH INFORMATION SYNTAX Notification-ASN1Module.AlarmInfo

AND ATTRIBUTE IDS

probableCause		probableCause,

specificProblems		specificProblems,

perceivedSeverity	perceivedSeverity,

backedUpStatus		backedUpStatus,

backUpObject		backUpObject,

trendIndication		trendIndication,

thresholdInfo		thresholdInfo,

notificationIdentifier	notificationIdentifier,

correlatedNotifications	correlatedNotifications,

stateChangeDefinition	stateChangeDefinition,

monitoredAttributes	monitoredAttributes,

proposedRepairActions	proposedRepairActions,

additionalText 		additionalText,

additionalInformation	additionalInformation;

REGISTERED AS	{smi2Notification 11};

qualityofServiceAlarmBehaviour	BEHAVIOUR

DEFINED AS	"This notification type is used to report a failure in the quality of service of the managed object.";

## 13.12	Relationship changetc  \l 2 "13.12	Relationship change"§

The semantics of the relationshipChange notification type are specified in CCITT Rec. X.732 | ISO/IEC 10164-3.

relationshipChange	NOTIFICATION

BEHAVIOUR	relationshipChangeBehaviour;

WITH INFORMATION SYNTAX Notification-ASN1Module.RelationshipChangeInfo

AND ATTRIBUTE IDS

sourceIndicator			sourceIndicator,

attributeIdentifierList		attributeIdentifierList,

relationshipChangeDefinition	relationshipChangeDefinition,

notificationIdentifier		notificationIdentifier,

correlatedNotifications		correlatedNotifications,

additionalText			additionalText,

additionalInformation		additionalInformation;

REGISTERED AS	{ smi2Notification 12};

relationshipChangeBehaviour	BEHAVIOUR

DEFINED AS	"This notification type is used to report the change in the value of one or more relationship attributes of a managed object, that result through either internal operation of the managed object or via management operation.";

## 13.13	Security service or mechanism violationtc  \l 2 "13.13	Security service or mechanism violation"§

The semantics of the securityServiceOrMechanismViolation notification type are specified in CCITT Rec. X.736 | ISO/IEC 10164-7.

securityServiceOrMechanismViolation	NOTIFICATION

BEHAVIOUR	securityServiceOrMechanismViolationBehaviour;

WITH INFORMATION SYNTAX Notification-ASN1Module.SecurityAlarmInfo

AND ATTRIBUTE IDS

securityAlarmCause	securityAlarmCause,

securityAlarmSeverity	securityAlarmSeverity,

securityAlarmDetector	securityAlarmDetector,

serviceUser		serviceUser,

serviceProvider		serviceProvider,

notificationIdentifier	notificationIdentifier,

correlatedNotifications	correlatedNotifications,

additionalText		additionalText,

additionalInformation	additionalInformation;

REGISTERED AS	{ smi2Notification 13};

securityServiceOrMechanismViolationBehaviour	BEHAVIOUR

DEFINED AS	"This notification is used to report that a security attack has been detected by a security service or mechanism.";

## 13.14	State changetc  \l 2 "13.14	State change"§

The semantics of the stateChange notification type are specified in CCITT Rec. X.731 | ISO/IEC 10164-2.

stateChange	NOTIFICATION

BEHAVIOUR	stateChangeBehaviour;

WITH INFORMATION SYNTAX Notification-ASN1Module.StateChangeInfo

AND ATTRIBUTE IDS

sourceIndicator		sourceIndicator,

attributeIdentifierList	attributeIdentifierList,

stateChangeDefinition	stateChangeDefinition,

notificationIdentifier	notificationIdentifier,

correlatedNotifications	correlatedNotifications,

additionalText		additionalText,

additionalInformation	additionalInformation;

REGISTERED AS	{ smi2Notification 14};

stateChangeBehaviour	BEHAVIOUR

DEFINED AS	"This notification type is used to report the change in the the value of one or more state attributes of a managed object, that result through either internal operation of the managed object or via management operation.";

## 13.15	Time domain violationtc  \l 2 "13.15	Time domain violation"§

The semantics of the timeDomainViolation notification type are specified in CCITT Rec. X.736 | ISO/IEC 10164-7.

timeDomainViolation	NOTIFICATION

BEHAVIOUR	timeDomainViolationBehaviour;

WITH INFORMATION SYNTAX Notification-ASN1Module.SecurityAlarmInfo

AND ATTRIBUTE IDS

securityAlarmCause	securityAlarmCause,

securityAlarmSeverity	securityAlarmSeverity,

securityAlarmDetector	securityAlarmDetector ,

serviceUser		serviceUser,

serviceProvider		serviceProvider,

notificationIdentifier	notificationIdentifier,

correlatedNotifications	correlatedNotifications,

additionalText		additionalText,

additionalInformation	additionalInformation;

REGISTERED AS	{ smi2Notification 15};

timeDomainViolationBehaviour	BEHAVIOUR

DEFINED AS	"This notification is used to report that an event has occurred at an unexpected or prohibited time.";

# 14	Supporting productionstc  \l 1 "14	Supporting productions"§

## 14.1	Managed object classtc  \l 2 "14.1	Managed object class"§

This clause specifies the required ASN.1 value notation for the value reference used in the MANAGED OBJECT CLASS template.

ManagedObjectClassesDefinitions {joint-iso-ccitt ms(9) smi(3) part2(2) asn1Module(2) 0}

DEFINITIONS ::=

BEGIN

--EXPORTS everything

smi2MObjectClass OBJECT IDENTIFIER ::= {joint-iso-ccitt ms(9) smi(3) part2(2) managedObjectClass(3)}

smi2NameBinding OBJECT IDENTIFIER ::= {joint-iso-ccitt ms(9) smi(3) part2(2) nameBinding(6)}

smi2Package OBJECT IDENTIFIER ::= {joint-iso-ccitt ms(9) smi(3) part2(2) package(4)}

END

## 14.2	Attribute typestc  \l 2 "14.2	Attribute types"§

This clause specifies the ASN.1 syntax for the supporting productions identified in clauses 9 and 10.

Attribute-ASN1Module {joint-iso-ccitt ms(9) smi(3) part2(2) asn1Module(2) 1}

DEFINITIONS IMPLICIT TAGS::=

BEGIN

--EXPORTS everything

IMPORTS

CMISFilter ,Attribute,BaseManagedObjectId,AttributeId,ObjectInstance,ObjectClass, EventTypeId FROM CMIP-1 {joint-iso-ccitt ms(9) cmip(1) modules(0) protocol(3)}

DistinguishedName FROM InformationFramework {joint-iso-ccitt ds(5) modules(1) informationFramework(1) }

AE-title FROM ACSE-1 {joint-iso-ccitt association-control(2) abstract-syntax(1) apdus(0) version(1)};

-- Note that the syntax of AE-title to be used is from CCITT Rec. X.227 | ISO 8650 corrigendum and

-- not "ANY".

smi2AttributeID OBJECT IDENTIFIER ::= { joint-iso-ccitt ms (9) smi(3) part2(2) attribute(7)}

smi2AttributeGroup OBJECT IDENTIFIER ::= { joint-iso-ccitt ms (9) smi(3) part2(2) attributeGroup(8)}

-- The value assignments for the probable cause used in CCITT Rec. X.733 | ISO/IEC 10164-4 are specified

-- below.

-- These are specified in accordance with the corrigendum to ASN.1 ISO/IEC JTC1/SC21 N5901

arfProbableCause OBJECT IDENTIFIER ::= { joint-iso-ccitt ms(9) smi(3) part2(2) standardSpecificExtension(0) arf(0)}

adapterError ProbableCause ::= globalValue : { arfProbableCause 1 }

applicationSubsystemFailure ProbableCause ::= globalValue : { arfProbableCause 2 }

bandwidthReduced ProbableCause ::= globalValue : { arfProbableCause 3 }

callEstablishmentError ProbableCause ::= globalValue : { arfProbableCause 4 }

communicationsProtocolError ProbableCause ::= globalValue : { arfProbableCause 5 }

communicationsSubsystemFailure ProbableCause ::= globalValue : { arfProbableCause 6 }

configurationOrCustomizationError ProbableCause ::= globalValue : { arfProbableCause 7 }

congestion ProbableCause ::= globalValue : { arfProbableCause 8 }

corruptData ProbableCause ::= globalValue : { arfProbableCause 9 }

cpuCyclesLimitExceeded ProbableCause ::= globalValue: { arfProbableCause 10 }

dataSetOrModemError ProbableCause ::= globalValue : { arfProbableCause 11 }

degradedSignal ProbableCause ::= globalValue : { arfProbableCause 12 }

dTE-DCEInterfaceError ProbableCause ::= globalValue : { arfProbableCause 13 }

enclosureDoorOpen ProbableCause ::= globalValue : { arfProbableCause 14 }

equipmentMalfunction ProbableCause ::= globalValue : { arfProbableCause 15 }

excessiveVibration ProbableCause ::= globalValue : { arfProbableCause 16 }

fileError ProbableCause ::= globalValue : { arfProbableCause 17}

fireDetected ProbableCause ::= globalValue : { arfProbableCause 18 }

floodDetected ProbableCause ::= globalValue : { arfProbableCause 19 }

framingError ProbableCause ::= globalValue : { arfProbableCause 20 }

heatingOrVentilationOrCoolingSystemProblem ProbableCause ::= globalValue : { arfProbableCause 21 }

humidityUnacceptable ProbableCause ::= globalValue : { arfProbableCause 22}

inputOutputDeviceError ProbableCause ::= globalValue : { arfProbableCause 23}

inputDeviceError ProbableCause ::= globalValue : { arfProbableCause 24}

lANError ProbableCause ::= globalValue : { arfProbableCause 25 }

leakDetected ProbableCause ::= globalValue : { arfProbableCause 26 }

localNodeTransmissionError ProbableCause ::= globalValue : { arfProbableCause 27 }

lossOfFrame ProbableCause ::= globalValue : { arfProbableCause 28}

lossOfSignal ProbableCause ::= globalValue : { arfProbableCause 29 }

materialSupplyExhausted ProbableCause ::= globalValue : { arfProbableCause 30 }

multiplexerProblem ProbableCause ::= globalValue : { arfProbableCause 31 }

outOfMemory ProbableCause ::= globalValue : { arfProbableCause 32 }

ouputDeviceError ProbableCause ::= globalValue : { arfProbableCause 33 }

performanceDegraded ProbableCause ::= globalValue : { arfProbableCause 34 }

powerProblem ProbableCause ::= globalValue : { arfProbableCause 35 }

pressureUnacceptable ProbableCause ::= globalValue : { arfProbableCause 36 }

processorProblem ProbableCause ::= globalValue : { arfProbableCause 37 }

pumpFailure ProbableCause::= globalValue : { arfProbableCause 38 }

queueSizeExceeded ProbableCause ::= globalValue : { arfProbableCause 39 }

receiveFailure ProbableCause ::= globalValue : { arfProbableCause 40 }

receiverFailure ProbableCause ::= globalValue : { arfProbableCause 41 }

remoteNodeTransmissionError ProbableCause ::= globalValue : { arfProbableCause 42 }

resourceAtOrNearingCapacity ProbableCause ::= globalValue : { arfProbableCause 43 }

responseTimeExcessive ProbableCause ::= globalValue : { arfProbableCause 44 }

retransmissionRateExcessive ProbableCause ::= globalValue : { arfProbableCause 45 }

softwareError ProbableCause ::= globalValue : { arfProbableCause 46 }

softwareProgramAbnormallyTerminated ProbableCause ::= globalValue : { arfProbableCause 47 }

softwareProgramError ProbableCause ::= globalValue : { arfProbableCause 48 }

storageCapacityProblem ProbableCause ::= globalValue : { arfProbableCause 49 }

temperatureUnacceptable ProbableCause ::= globalValue : { arfProbableCause 50 }

thresholdCrossed ProbableCause ::= globalValue : { arfProbableCause 51 }

timingProblem ProbableCause ::= globalValue : { arfProbableCause 52 }

toxicLeakDetected ProbableCause ::= globalValue : { arfProbableCause 53 }

transmitFailure ProbableCause ::= globalValue : { arfProbableCause 54 }

transmitterFailure ProbableCause ::= globalValue : { arfProbableCause 55 }

underlyingResourceUnavailable ProbableCause ::= globalValue : { arfProbableCause 56 }

versionMismatch ProbableCause ::= globalValue : { arfProbableCause 57 }

-- The following applies to CCITT applications only.

-- The use of the following values is subject to 8.1.2.12 of The Alarm Management Function

-- (CCITT Recommendation X.733 | 10164-4).

arfProposedRepairAction OBJECT IDENTIFIER ::= { joint-iso-ccitt ms(9) smi(3) part2(2) standardSpecificExtension(0) arfpra(2)}

noActionRequired OBJECT IDENTIFIER ::= {arfProposedRepairAction 1}

repairActionRequired OBJECT IDENTIFIER ::= {arfProposedRepairAction 2}

-- The value assignments for the security alarm cause used in CCITT Rec. X.736 | ISO/IEC 10164-7 are -- specified below.

securityAlarmCause OBJECT IDENTIFIER ::= { joint-iso-ccitt ms(9) smi(3) part2(2) standardSpecificExtension(0) sarf(1) }

authenticationFailure SecurityAlarmCause ::= {securityAlarmCause 1}

breachOfConfidentiality SecurityAlarmCause ::= {securityAlarmCause 2}

cableTamper SecurityAlarmCause ::= {securityAlarmCause 3}

delayedInformation SecurityAlarmCause ::= {securityAlarmCause 4}

denialOfService SecurityAlarmCause ::= {securityAlarmCause 5}

duplicateInformation SecurityAlarmCause ::= {securityAlarmCause 6}

informationMissing SecurityAlarmCause ::= {securityAlarmCause 7}

informationModificationDetected SecurityAlarmCause ::= {securityAlarmCause 8}

informationOutOfSequence SecurityAlarmCause ::= {securityAlarmCause 9}

intrusionDetection SecurityAlarmCause ::= {securityAlarmCause 10}

keyExpired SecurityAlarmCause ::= {securityAlarmCause 11}

nonRepudiationFailure SecurityAlarmCause ::= {securityAlarmCause 12}

outOfHoursActivity SecurityAlarmCause ::= {securityAlarmCause 13}

outOfService SecurityAlarmCause ::= {securityAlarmCause 14}

proceduralError SecurityAlarmCause ::= {securityAlarmCause 15}

unauthorizedAccessAttempt SecurityAlarmCause ::= {securityAlarmCause 16}

unexpectedInformation SecurityAlarmCause ::= {securityAlarmCause 17}

unspecifiedReason SecurityAlarmCause ::= {securityAlarmCause 18}

-- default value definitions

defaultIntervalsOfDay 	IntervalsOfDay ::= {{	intervalStart	{hour 0, minute 0},

intervalEnd	{hour 23, minute 59}}}

defaultStopTime		StopTime ::= continual : NULL

defaultWeekMask	WeekMask ::= {{	daysOfWeek	'1111111'B,

intervalsOfDay	defaultIntervalsOfDay}}

defaultDiscriminatorConstruct	DiscriminatorConstruct ::= and : {}

-- supporting productions

ActiveDestination ::= Destination (WITH COMPONENTS {single PRESENT, multiple ABSENT})

AdditionalText ::= GraphicString

AdditionalInformation ::= SET OF ManagementExtension

Allomorphs ::= SET OF ObjectClass

AdministrativeState ::= ENUMERATED {locked(0),unlocked(1),shuttingDown(2) }

AttributeIdentifierList ::= SET OF AttributeId

AttributeList ::= SET OF Attribute

AttributeValueChangeDefinition::= SET OF SEQUENCE {

attributeID	AttributeId,

oldAttributeValue	[1] ANY DEFINED BY attributeID OPTIONAL,

newAttributeValue	[2] ANY DEFINED BY attributeID}

AlarmStatus ::= SET OF INTEGER { underRepair(0), critical(1), major(2), minor(3), alarmOutstanding(4)}

AvailabilityStatus ::= SET OF INTEGER { inTest(0), failed(1), powerOff(2), offLine(3), offDuty(4),	 	dependency(5), degraded(6), notInstalled (7) , logFull(8)}

-- logFull is defined in CCITT Rec X.735 | ISO/IEC 10164-6.

BackedUpStatus ::= BOOLEAN

-- True implies backed up

-- False implies not backed up

BackUpDestinationList ::= SEQUENCE OF AE-title

BackUpRelationshipObject ::= CHOICE { objectName ObjectInstance, noObject NULL}

CapacityAlarmThreshold ::= SET OF INTEGER (0..100)

ConfirmedMode ::= BOOLEAN

-- True implies the mode for event report is confirmed

ControlStatus ::= SET OF INTEGER { subjectToTest(0), partOfServicesLocked(1), reservedForTest(2), suspended(3)}

Count ::= INTEGER

CounterThreshold ::= SET OF SEQUENCE {

comparisonLevel	INTEGER,

offsetValue		INTEGER,

notificationOnOff	BOOLEAN}

CorrelatedNotifications ::= SET OF SEQUENCE {

correlatedNotifications	SET OF NotificationIdentifier,

sourceObjectInst		ObjectInstance OPTIONAL }

CurrentLogSize ::= INTEGER

Destination ::= CHOICE {

single		AE-title,

multiple	SET OF AE-title}

-- Note that the syntax of AE-title to be used is from CCITT X.227 | ISO 8650 Technical

-- corrigendum 1 and not "ANY".

DiscriminatorConstruct ::= CMISFilter

EventTime ::= GeneralizedTime

GaugeThreshold ::= SET OF SEQUENCE {

notifyLow	NotifyThreshold,

notifyHigh	NotifyThreshold}

GaugeThresholdValue ::= ObservedValue

-- same choice must be chosen within a sequence

-- for both GenericOldState and GenericNewState.

GroupObjects ::= SET OF ObjectInstance

IntervalsOfDay ::= SET OF SEQUENCE{

intervalStart	Time24,

intervalEnd	Time24}

LogAvailability ::= AvailabilityStatus (WITH COMPONENT ( logFull | offDuty))

LogFullAction ::= ENUMERATED { wrap(0), halt (1)}

LoggingTime ::= GeneralizedTime

LogRecordId ::= SimpleNameType(WITH COMPONENTS {number PRESENT, string ABSENT})

MaxLogSize ::= INTEGER {unlimited(0)} -- size in octets

ManagementExtension ::= SEQUENCE {

identifier	OBJECT IDENTIFIER,

significance	[1] BOOLEAN DEFAULT FALSE,

information	[2] ANY DEFINED BY identifier }

MonitoredAttributes ::= SET OF Attribute

NameBinding ::= OBJECT IDENTIFIER

NotificationIdentifier ::= INTEGER -- reuse only when no requirement exists

-- for correlating prior notification.

NotifyThreshold ::= SEQUENCE {

threshold	ObservedValue,

notifyOnOff	BOOLEAN }

NumberOfRecords ::= INTEGER

ObservedValue ::= CHOICE {

integer	INTEGER,

real		REAL }

OperationalState::= ENUMERATED { disabled(0), enabled(1) }

Packages ::= SET OF OBJECT IDENTIFIER

PerceivedSeverity ::= ENUMERATED{ indeterminate(0),

-- used when it is not possible to assign the following values

critical(1), major(2), minor(3), warning(4), cleared(5) }

PrioritisedObject ::= SET OF SEQUENCE {

object		ObjectInstance,

priority	INTEGER {lowest(0), highest(127)}}

ProbableCause ::= CHOICE {

globalValue	OBJECT IDENTIFIER,

localValue	INTEGER}

-- The values for the probable cause defined in CCITT Rec X.733 | ISO/IEC 10164-4 are specified

-- above.

ProceduralStatus ::= SET OF INTEGER { initializationRequired(0), notInitialized(1), initializing(2), reporting (3), terminating (4) }

ProposedRepairActions ::= SET OF SpecificIdentifier

SchedulingAvailability ::= AvailabilityStatus (WITH COMPONENT (offDuty))

SecurityAlarmCause ::= OBJECT IDENTIFIER

SecurityAlarmSeverity ::= PerceivedSeverity (indeterminate | critical | major | minor | warning)

SecurityAlarmDetector::= CHOICE {	mechanism [0]	OBJECT IDENTIFIER,

object [1]	ObjectInstance,

application	[2] AE-title}

ServiceProvider ::= ServiceUser

ServiceUser ::= SEQUENCE {

identifier	OBJECT IDENTIFIER,

details	ANY DEFINED BY identifier }

SimpleNameType ::= CHOICE {	number		INTEGER,

string		GraphicString}

SpecificIdentifier ::= CHOICE {	OBJECT IDENTIFIER,

INTEGER}

SourceIndicator ::= ENUMERATED { resourceOperation(0), managementOperation(1), unknown(2)}

SpecificProblems ::= SET OF SpecificIdentifier

StandbyStatus ::= INTEGER { hotStandby(0), coldStandby(1), providingService(2)}

StartTime ::= GeneralizedTime

StopTime ::= CHOICE {	specific		GeneralizedTime,

continual	NULL}

SupportedFeatures ::= SET OF SEQUENCE {

featureIdentifier		OBJECT IDENTIFIER,

featureInfo		ANY DEFINED BY featureIdentifier}

SystemId ::= CHOICE {	name		GraphicString,

number		INTEGER,

nothing		NULL}

SystemTitle ::= CHOICE {	distinguishedName	DistinguishedName,

oid			OBJECT IDENTIFIER,

nothing			NULL}

TideMarkInfo ::= SEQUENCE {	currentTideMark		TideMark,

previousTideMark	TideMark,

resetTime		GeneralizedTime}

TideMark ::= CHOICE {		maxTideMar	[0] ObservedValue,

minTideMark	[1] ObservedValue}

Time24 ::= SEQUENCE {		hour		INTEGER (0..23),

minute	INTEGER (0..59) }

ThresholdInfo ::= SEQUENCE {

triggeredThreshold	AttributeId,

observedValue	ObservedValue,

thresholdLevel	[1] ThresholdLevelInd OPTIONAL,

-- Indication needed for multilevel thresholds

armTime	[2] GeneralizedTime OPTIONAL }

ThresholdLevelInd ::= CHOICE {

up	[1] SEQUENCE {	high	ObservedValue,

low	ObservedValue OPTIONAL

-- up is the only permitted choice for counter --},

down	[2] SEQUENCE {	high	ObservedValue,

low	ObservedValue}}

TrendIndication ::= ENUMERATED { lessSevere(0), noChange(1), moreSevere(2) }

UnknownStatus ::= BOOLEAN -- TRUE implies unknown status

UnscheduledLogAvailability ::= AvailabilityStatus (WITH COMPONENT (logFull))

UsageState::= ENUMERATED { idle(0), active(1), busy(2) }

WeekMask ::= SET OF SEQUENCE {

daysOfWeek	BIT STRING {sunday(0),monday(1),tuesday(2),wednesday(3),		 				thursday(4), friday(5), saturday(6)} (SIZE(7) ),

intervalsOfDay		IntervalsOfDay }

END	 -- end of supporting productions

## 14.3	Notification typestc  \l 2 "14.3	Notification types"§

This clause specifies the ASN.1 syntax for the supporting productions identified in clause 13.

Notification-ASN1Module {joint-iso-ccitt ms(9) smi(3) part2(2) asn1Module(2) 2}

DEFINITIONS IMPLICIT TAGS::=

BEGIN

IMPORTS

ProbableCause,   SpecificProblems,   PerceivedSeverity,   BackedUpStatus,   TrendIndication,   ThresholdInfo,   NotificationIdentifier,   CorrelatedNotifications,   MonitoredAttributes,   ProposedRepairActions,   AdditionalText,   AdditionalInformation,   SecurityAlarmCause,   SecurityAlarmSeverity,   SecurityAlarmDetector,   AttributeValueChangeDefinition,   SourceIndicator,   AttributeIdentifierList,   AttributeList,   ServiceUser,   ServiceProvider

FROM	Attribute-ASN1Module

ObjectClass,AttributeId,ObjectInstance FROM CMIP-1 {joint-iso-ccitt ms(9) cmip(1) version1 (1) protocol(3)} ;

--EXPORTS everything

smi2Notification OBJECT IDENTIFIER ::= { joint-iso-ccitt ms (9) smi(3) part2(2) notification(10)}

AlarmInfo ::= SEQUENCE {

probableCause			ProbableCause,

specificProblems		[1]SpecificProblems OPTIONAL,

perceivedSeverity		PerceivedSeverity,

backedUpStatus			BackedUpStatus OPTIONAL,

backUpObject			[2]ObjectInstance OPTIONAL,

trendIndication			[3]TrendIndication OPTIONAL,

thresholdInfo			[4]ThresholdInfo OPTIONAL,

notificationIdentifier		[5]NotificationIdentifier OPTIONAL,

correlatedNotifications		[6]CorrelatedNotifications OPTIONAL,

stateChangeDefinition		[7]AttributeValueChangeDefinition OPTIONAL,

monitoredAttributes		[8]MonitoredAttributes OPTIONAL,

proposedRepairActions		[9]ProposedRepairActions OPTIONAL,

additionalText			AdditionalText OPTIONAL,

additionalInformation		[10]AdditionalInformation OPTIONAL}

AttributeValueChangeInfo ::= SEQUENCE {

sourceIndicator			SourceIndicator OPTIONAL,

attributeIdentifierList		[1]AttributeIdentifierList OPTIONAL,

attributeValueChangeDefinition	AttributeValueChangeDefinition,

notificationIdentifier		NotificationIdentifier OPTIONAL,

correlatedNotifications		[2]CorrelatedNotifications OPTIONAL,

additionalText			AdditionalText OPTIONAL,

additionalInformation		[3]AdditionalInformation OPTIONAL}

ObjectInfo ::= SEQUENCE {

sourceIndicator			SourceIndicator OPTIONAL,

attributeList			AttributeList OPTIONAL,

notificationIdentifier		NotificationIdentifier OPTIONAL,

correlatedNotifications		[1]CorrelatedNotifications OPTIONAL,

additionalText			AdditionalText OPTIONAL,

additionalInformation		[2]AdditionalInformation OPTIONAL}

RelationshipChangeInfo ::= SEQUENCE {

sourceIndicator			SourceIndicator OPTIONAL,

attributeIdentifierList		[1]AttributeIdentifierList OPTIONAL,

relationshipChangeDefinition	AttributeValueChangeDefinition,

notificationIdentifier		NotificationIdentifier OPTIONAL,

correlatedNotifications		[2]CorrelatedNotifications OPTIONAL,

additionalText			AdditionalText OPTIONAL,

additionalInformation		[3]AdditionalInformation OPTIONAL}

StateChangeInfo ::= SEQUENCE {

sourceIndicator			SourceIndicator OPTIONAL,

attributeIdentifierList		[1]AttributeIdentifierList OPTIONAL,

stateChangeDefinition		AttributeValueChangeDefinition,

notificationIdentifier		NotificationIdentifier OPTIONAL,

correlatedNotifications		[2]CorrelatedNotifications OPTIONAL,

additionalText			AdditionalText OPTIONAL,

additionalInformation		[3]AdditionalInformation OPTIONAL}

SecurityAlarmInfo ::= SEQUENCE {

securityAlarmCause		SecurityAlarmCause,

securityAlarmSeverity		SecurityAlarmSeverity,

securityAlarmDetector		SecurityAlarmDetector,

serviceUser			ServiceUser,

serviceProvider			ServiceProvider,

notificationIdentifier		NotificationIdentifier OPTIONAL,

correlatedNotifications		[1]CorrelatedNotifications OPTIONAL,

additionalText			AdditionalText OPTIONAL,

additionalInformation		[2]AdditionalInformation OPTIONAL}

END

## 14.4	Parameter typestc  \l 2 "14.4	Parameter types"§

This clause specifies the ASN.1 syntax for the supporting productions identified in clause 12.

Parameter-ASN1Module {joint-iso-ccitt ms(9) smi(3) part2(2) asn1Module(2) 3}

DEFINITIONS IMPLICIT TAGS::=

BEGIN

smi2Parameter OBJECT IDENTIFIER ::= {joint-iso-ccitt ms(9) smi(3) part2(2) parameter(5)}

MiscellaneousError ::= NULL

END

# 15	Conformance and compliancetc  \l 1 "15	Conformance and compliance"§

## 15.1	Conformancetc  \l 2 "15.1	Conformance"§

This Recommendation | International Standard does not specify any conformance requirements placed on open systems.

## 15.2	Compliancetc  \l 2 "15.2	Compliance"§

Where other CCITT Recommendations | International Standards refine any of the managed object class definitions contained in this Recommendation | International Standard via the refinement and referencing mechanisms defined in CCITT Rec. X.722 | ISO/IEC 10165-4, the managed object classes that refine these definitions shall comply with the behavioural and syntactic aspects of the superclass as defined in this Recommendation | International Standard.

Where other CCITT Recommendations | International Standards import any of the definitions contained in this Recommendation | International Standard into attribute type or managed object class or notification type definitions via the referencing mechanism defined in CCITT Rec. X.722 | ISO/IEC 10165-4, the attribute type or managed object class or notification type definitions that reference these definitions shall comply with the behavioural and syntactic aspects of those types as specified in this Recommendation | International Standard.

Annex A
Counters and counter threshold attribute

tc  \l 1 "Annex A  Counters and counter threshold attribute"§

(This Annex forms an integral part of this Recommendation | International Standard )

## A.1	Countertc  \l 2 "A.1	Counter"§

This Annex defines a number of counter types which are useful for importing in other standards.

### A.1.1	Corrupted PDUs received countertc  \l 3 "A.1.1	Corrupted PDUs received counter"§

The attribute type corruptedPDUsReceivedCounter specifies the total number of corrupted PDUs received.

corruptedPDUsReceivedCounter	ATTRIBUTE

DERIVED FROM	counter;

REGISTERED AS	{ smi2AttributeID 72};

### A.1.2	Incoming connection reject error countertc  \l 3 "A.1.2	Incoming connection reject error counter"§

The attribute type IncomingConnectionRejectErrorCounter specifies the total number of incoming connection requests which were received by the managed object but rejected due to protocol errors.

incomingConnectionRejectErrorCounter	ATTRIBUTE

DERIVED FROM	counter;

REGISTERED AS	{ smi2AttributeID 73};

### A.1.3	Incoming connection requests countertc  \l 3 "A.1.3	Incoming connection requests counter"§

The attribute type incomingConnectionRequestsCounter specifies the total number of incoming connection requests.

incomingConnectionRequestsCounter	ATTRIBUTE

DERIVED FROM	counter;

REGISTERED AS	{ smi2AttributeID 74};

### A.1.4	Incoming disconnect countertc  \l 3 "A.1.4	Incoming disconnect counter"§

The attribute type incomingDisconnectCounter specifies the total number of incoming disconnect requests received by the managed object.

incomingDisconnectCounter	ATTRIBUTE

DERIVED FROM	counter;

REGISTERED AS	{ smi2AttributeID 75};

### A.1.5	Incoming disconnect error countertc  \l 3 "A.1.5	Incoming disconnect error counter"§

The attribute type incomingDisconnectErrorCounter specifies the total number of incoming disconnect requests received by the managed object due to protocol errors.

incomingDisconnectErrorCounter	ATTRIBUTE

DERIVED FROM	counter;

REGISTERED AS	{ smi2AttributeID 76};

### A.1.6	Incoming protocol error countertc  \l 3 "A.1.6	Incoming protocol error counter"§

The attribute type incomingProtocolErrorCounter specifies the total number of error report or reset PDUs which were received by the managed object due to protocol errors.

incomingProtocolErrorCounter	ATTRIBUTE

DERIVED FROM	counter;

REGISTERED AS	{ smi2AttributeID 77};

### A.1.7	Octets received countertc  \l 3 "A.1.7	Octets received counter"§

The attribute type octetsReceivedCounter specifies the total number of user data octets received by the managed object.

octetsReceivedCounter	ATTRIBUTE

DERIVED FROM	counter;

REGISTERED AS	{ smi2AttributeID 78};

### A.1.8	Octets retransmitted error countertc  \l 3 "A.1.8	Octets retransmitted error counter"§

The attribute type octetsRetransmittedErrorCounter specifies the total number of octets retransmitted by the managed object.

octetsRetransmittedErrorCounter	ATTRIBUTE

DERIVED FROM	counter;

REGISTERED AS	{ smi2AttributeID 79};

### A.1.9	Octets sent countertc  \l 3 "A.1.9	Octets sent counter"§

The attribute type octetsSentCounter specifies the total number of user data octets sent by the managed object.

octetsSentCounter	ATTRIBUTE

DERIVED FROM	counter;

REGISTERED AS	{ smi2AttributeID 80};

### A.1.10	Outgoing connection reject error countertc  \l 3 "A.1.10	Outgoing connection reject error counter"§

The attribute type outgoingConnectionRejectErrorCounter specifies the total number of outgoing connection requests which were sent by the managed object but rejected due to protocol errors.

outgoingConnectionRejectErrorCounter	ATTRIBUTE

DERIVED FROM	counter;

REGISTERED AS	{ smi2AttributeID 81};

### A.1.11	Outgoing connection requests countertc  \l 3 "A.1.11	Outgoing connection requests counter"§

The attribute type outgoingConnectionRequestsCounter specifies the total number of outgoing connection requests.

outgoingConnectionRequestsCounter	ATTRIBUTE

DERIVED FROM	counter;

REGISTERED AS	{ smi2AttributeID 82};

### A.1.12	Outgoing disconnect countertc  \l 3 "A.1.12	Outgoing disconnect counter"§

The attribute type outgoingDisconnectCounter specifies the total number of outgoing disconnect requests processed by the managed object.

outgoingDisconnectCounter	 ATTRIBUTE

DERIVED FROM	counter;

REGISTERED AS	{ smi2AttributeID 83};

### A.1.13	Outgoing disconnect error countertc  \l 3 "A.1.13	Outgoing disconnect error counter"§

The attribute type outgoingDisconnectErrorCounter specifies the total number of outgoing disconnect requests sent by the managed object due to protocol errors.

outgoingDisconnectErrorCounter	ATTRIBUTE

DERIVED FROM	counter;

REGISTERED AS	{ smi2AttributeID 84};

### A.1.14	Outgoing protocol error countertc  \l 3 "A.1.14	Outgoing protocol error counter"§

The attribute type outgoingProtocolErrorCounter specifies the total number of error report or reset PDUs which were sent by the managed object due to protocol errors.

outgoingProtocolErrorCounter	ATTRIBUTE

DERIVED FROM	counter;

REGISTERED AS	{ smi2AttributeID 85};

### A.1.15	PDUs received countertc  \l 3 "A.1.15	PDUs received counter"§

The attribute type pdusReceivedCounter specifies the total number of PDUs received by the managed object.

pdusReceivedCounter	ATTRIBUTE

DERIVED FROM	counter;

REGISTERED AS	{ smi2AttributeID 86};

### A.1.16	PDUs retransmitted error countertc  \l 3 "A.1.16	PDUs retransmitted error counter"§

The attribute type pdusRetransmittedErrorCounter specifies the total number of PDUs retransmitted by the managed object.

pdusRetransmittedErrorCounter	 ATTRIBUTE

DERIVED FROM	counter;

REGISTERED AS	{ smi2AttributeID 87};

### A.1.17	PDUs sent countertc  \l 3 "A.1.17	PDUs sent counter"§

The attribute type specifies pdusSentCounter the total number of PDUs sent by the managed object.

pdusSentCounter	ATTRIBUTE

DERIVED FROM	counter;

REGISTERED AS	{ smi2AttributeID 88};

## A.2	Counter-thresholdtc  \l 2 "A.2	Counter-threshold"§

### A.2.1	Corrupted PDUs received thresholdtc  \l 3 "A.2.1	Corrupted PDUs received threshold"§

The attribute type corruptedPDUsReceivedThreshold specifies a counter-threshold which is associated with the PDUs Corrupted PDUs Received counter attribute type.

corruptedPDUsReceivedThreshold	ATTRIBUTE

DERIVED FROM	counter-Threshold;

REGISTERED AS	{ smi2AttributeID 89};

### A.2.2	Incoming connection reject error thresholdtc  \l 3 "A.2.2	Incoming connection reject error threshold"§

The attribute type incomingConnectionRejectErrorThreshold specifies a counter-threshold which is associated with the Incoming ConnectionRejectError counter attribute type.

incomingConnectionRejectErrorThreshold	 ATTRIBUTE

DERIVED FROM	counter-Threshold;

REGISTERED AS	{ smi2AttributeID 90};

### A.2.3	Incoming connection requests thresholdtc  \l 3 "A.2.3	Incoming connection requests threshold"§

The attribute type incomingConnectionRequestsThreshold specifies a counter-threshold which is associated with the Incoming Connection Requests counter attribute type.

incomingConnectionRequestsThreshold

ATTRIBUTE

DERIVED FROM	counter-Threshold;

REGISTERED AS	{ smi2AttributeID 91};

### A.2.4	Incoming disconnect error thresholdtc  \l 3 "A.2.4	Incoming disconnect error threshold"§

The attribute type incomingDisconnectErrorThreshold specifies a counter-threshold which is associated with the Incoming Disconnect Error counter attribute type.

incomingDisconnectErrorThreshold

ATTRIBUTE

DERIVED FROM	counter-Threshold;

REGISTERED AS	{ smi2AttributeID 92};

### A.2.5	Incoming protocol error thresholdtc  \l 3 "A.2.5	Incoming protocol error threshold"§

The attribute type incomingProtocolErrorThreshold specifies a counter-threshold which is associated with the Incoming Protocol Error counter attribute type.

incomingProtocolErrorThreshold	ATTRIBUTE

DERIVED FROM	counter-Threshold;

REGISTERED AS	{ smi2AttributeID 93};

### A.2.6	Octets received thresholdtc  \l 3 "A.2.6	Octets received threshold"§

The attribute type octetsReceivedThreshold specifies a counter-threshold which is associated with the Octets Received counter attribute type.

octetsReceivedThreshold	ATTRIBUTE

DERIVED FROM	counter-Threshold;

REGISTERED AS	{ smi2AttributeID 94};

### A.2.7	Octets retransmitted thresholdtc  \l 3 "A.2.7	Octets retransmitted threshold"§

The attribute type octetsRetransmittedThreshold specifies a counter threshold which can be associated with the octects retransmitted counter attribute type.

octetsRetransmittedThreshold

ATTRIBUTE

DERIVED FROM	counter-Threshold;

REGISTERED AS	{ smi2AttributeID 95};

### A.2.8	Octets sent thresholdtc  \l 3 "A.2.8	Octets sent threshold"§

The attribute type octetsSentThreshold specifies a counter-threshold which is associated with the Octets Sent counter attribute type.

octetsSentThreshold	 ATTRIBUTE

DERIVED FROM	counter-Threshold;

REGISTERED AS	{ smi2AttributeID 96};

### A.2.9	Outgoing connection reject error thresholdtc  \l 3 "A.2.9	Outgoing connection reject error threshold"§

The attribute type outgoingConnectionRejectErrorThreshold specifies a counter-threshold which is associated with the Outgoing Connection Reject Error counter attribute type.

outgoingConnectionRejectErrorThreshold

ATTRIBUTE

DERIVED FROM	counter-Threshold;

REGISTERED AS	{ smi2AttributeID 97};

### A.2.10	Outgoing connection requests thresholdtc  \l 3 "A.2.10	Outgoing connection requests threshold"§

The attribute type outgoingConnectionRequestsThreshold specifies a counter-threshold which is associated with the Outgoing Connection Requests counter attribute type.

outgoingConnectionRequestsThreshold

ATTRIBUTE

DERIVED FROM	counter-Threshold;

REGISTERED AS	{ smi2AttributeID 98};

### A.2.11	Outgoing disconnect error thresholdtc  \l 3 "A.2.11	Outgoing disconnect error threshold"§

The attribute type outgoingDisconnectErrorThreshold specifies a counter-threshold which is associated with the Outgoing Disconnect Error counter attribute type.

outgoingDisconnectErrorThreshold

ATTRIBUTE

DERIVED FROM	counter-Threshold;

REGISTERED AS	{ smi2AttributeID 99};

### A.2.12	Outgoing protocol error thresholdtc  \l 3 "A.2.12	Outgoing protocol error threshold"§

The attribute type outgoingProtocolErrorThreshold specifies a counter-threshold which is associated with the Outgoing Protocol Error counter attribute type.

outgoingProtocolErrorThreshold

ATTRIBUTE

DERIVED FROM	counter-Threshold;

REGISTERED AS	{ smi2AttributeID 100};

### A.2.13	PDUs received thresholdtc  \l 3 "A.2.13	PDUs received threshold"§

The attribute type pdusReceivedThreshold specifies a counter-threshold which is associated with the PDUs Received counter attribute type.

pdusReceivedThreshold	ATTRIBUTE

DERIVED FROM	counter-Threshold;

REGISTERED AS	{ smi2AttributeID 101};

### A.2.14	PDUs retransmitted error thresholdtc  \l 3 "A.2.14	PDUs retransmitted error threshold"§

The attribute type pdusRetransmittedErrorThreshold specifies a counter-threshold which is associated with the PDUs Retransmitted Error counter attribute type.

pdusRetransmittedErrorThreshold

ATTRIBUTE

DERIVED FROM	counter-Threshold;

REGISTERED AS	{ smi2AttributeID 102};

### A.2.15	PDUs sent thresholdtc  \l 3 "A.2.15	PDUs sent threshold"§

The attribute type pdusSentThreshold specifies a counter-threshold which is associated with the PDUs Sent counter attribute type.

pdusSentThreshold	ATTRIBUTE

DERIVED FROM	counter-Threshold;

REGISTERED AS	{ smi2AttributeID 103};

Annex B
Index of managed object classes

tc  \l 1 "Annex B  Index of managed object classes"§

(This Annex does not form an integral part of this Recommendation | International Standard)

Alarm record	6.1

Attribute value change record	6.2

Discriminator	6.3

Event forwarding discriminator	6.4

Event log record	6.5

Log		6.6

Log record	6.7

Object creation record	6.8

Object deletion record	6.9

Relationship change record	6.10

Security alarm report record	6.11

State change record	6.12

System	6.13

Top		6.14

Annex C
Index of packages

tc  \l 1 "Annex C  Index of packages"§

(This Annex does not form an integral part of this Recommendation | International Standard)

Additional information	8.1

Additional text	8.2

Administrative state	6.13

Allomorphic	6.14

Attribute identifier list	8.3

Attribute list	8.4

Availability status	8.5

Back up destination list	6.4

Back up object	6.1

Backed up status	6.1

Capacity alarm threshold	6.6

Correlated notifications	8.6

Daily scheduling	8.8

Duration	8.9

Event time	6.5

External scheduler	8.10

Finite log size	6.6

Mode		6.4

Monitored attributes	6.1

Notification Identifier	8.7

Packages	6.14

Proposed repair actions	6.1

Source indicator	8.11

Specific problems	6.1

Supported features	6.13

Threshold info	6.1

Trend indication	6.1

Weekly scheduling	8.12

Annex D
Index of generic and specific attribute types

tc  \l 1 "Annex D  Index of generic and specific attribute types"§

(This Annex does not form an integral part of this Recommendation | International Standard)

Active destination	10.7.4.1

Additional information.	10.7.1.1

Additional text	10.7.1.2

Administrative state	10.7.2.1

Alarm status	10.7.2.2

Allomorphs	10.7.4.2

Attribute identifier list	10.7.1.3

Attribute list	10.7.1.4

Attribute value change definition	10.7.1.5

Availability status	10.7.2.3

Back-up destination list	10.7.4.3

Back-up object	10.7.3.1

Backed-up object	10.7.3.2

Backed-up status	10.7.1.6

Capacity alarm threshold	10.7.4.4

Confirmed mode	10.7.4.5

Control status	10.7.2.4

Correlated notifications	10.7.1.7

Corrupted PDUs received counter	A.1.1

Corrupted PDUs received threshold	A.2.1

Counter	9.1

Counter-threshold	9.3.1

Current log size	10.7.4.6

Destination	 10.7.4.7

Discriminator construct	10.7.4.8

Discriminator ID	10.1.1

Event time	10.7.1.8

Event type	10.7.1.9

Gauge		9.2

Gauge-threshold	9.3.2

Incoming connection reject error counter	A.1.2

Incoming connection reject error threshold	A.2.2

Incoming connection requests counter	A.1.3

Incoming connection requests threshold	A.2.3

Incoming disconnect counter	A.1.4

Incoming disconnect error counter	A.1.5

Incoming disconnect error threshold	A.2.4

Incoming protocol error counter	A.1.6

Incoming protocol error threshold	A.2.5

Intervals of day	10.7.4.9

Log full action	10.7.4.10

Log ID	10.1.2

Log record ID	10.1.3

Logging time	10.7.4.11

Managed object class	10.7.4.12

Managed object instance	10.7.4.13

Max log size	10.7.4.14

Member	10.7.3.3

Monitored attributes	10.7.1.10

Name binding	10.7.4.15

Notification identifier	10.7.1.11

Number of records	10.7.4.16

Object class	10.7.4.17

Octets received counter	A.1.7

Octets received threshold	A.2.6

Octets retransmitted error counter	A.1.8

Octets retransmitted threshold	A.2.7

Octets sent counter	A.1.9

Octets sent threshold	A.2.8

Operational state	10.7.2.5

Outgoing connection reject error counter	A.1.10

Outgoing connection reject error threshold	A.2.9

Outgoing connection requests counter	A.1.11

Outgoing connection requests threshold	A.2.10

Outgoing disconnect counter	A.1.12

Outgoing disconnect error counter	A.1.13

Outgoing disconnect error threshold	A.2.11

Outgoing protocol error counter	A.1.14

Outgoing protocol error threshold	A.2.12

Owner		10.7.3.4

Packages	10.7.4.18

PDUs received counter	A.1.15

PDUs received threshold	A.2.13

PDUs retransmitted error counter	A.1.16

PDUs retransmitted error threshold	A.2.14

PDUs sent counter	A.1.17

PDUs sent threshold	A.2.15

Peer		10.7.3.5

Perceived severity	10.7.1.12

Primary	10.7.3.6

Probable cause	10.7.1.13

Procedural status	10.7.2.6

Proposed repair actions	10.7.1.14

Provider object	10.7.3.7

Relationship change definition	10.7.1.15

Relationships	10.7.3.8

Scheduler name	10.7.4.19

Secondary	10.7.3.9

Security alarm cause	10.7.1.16

Security alarm detector	10.7.1.17

Security alarm severity	10.7.1.18

Service provider	10.7.1.19

Service user	10.7.1.20

Source indicator	10.7.1.21

Specific problems	10.7.1.22

Standby status	10.7.2.7

Start time	10.7.4.20

State		10.7.2.10

State change definition	10.7.1.23

Stop time	10.7.4.21

Supported features	10.7.4.22

System ID	10.1.4

System title	10.1.5

Threshold info	10.7.1.24

Tide-mark.	9.4

Trend indication	10.7.1.25

Unknown status	10.7.2.8

Usage state	10.7.2.9

User object	10.7.3.10

Week mask	10.7.4.23

Annex E
Index of notification types

tc  \l 1 "Annex E  Index of notification types" §

(This Annex does not form an integral part of this Recommendation | International Standard)

Attribute Value Change	13.1

Communication Alarm	13.2

Environmental Alarm	13.3

Equipment Alarm	13.4

Integrity Violation	13.5

Object Creation	13.6

Object Deletion	13.7

Operational Violation	13.8

Physical Violation	13.9

Processing Error Alarm	13.10

Quality of Service Alarm	13.11

Relationship Change	13.12

Security Service or Mechanism Violation	13.13

State Change	13.14

Time Domain Violation	13.15

Annex F
Management information used by Systems management function

tc  \l 1 "Annex F  Management information used by Systems management function" §

(This Annex does not form an integral part of this Recommendation | International Standard)

## F.1	Object management function (CCITT Rec. X.730 | ISO/IEC 10164-1)tc  \l 2 "F.1	Object management function (CCITT Rec.X.730 | ISO/IEC 10164-1)"§

Managed Object Classes

Attribute value change record	6.2

Object creation record	6.8

Object deletion record	6.9

Packages

Additional information	8.1

Additional text	8.2

Attribute identifier list	8.3

Attribute list	8.4

Correlated notifications	8.6

Event time	6.5

Notification Identifier	8.7

Source indicator	8.11

Attributes

Additional information.	10.7.1.1

Additional text	10.7.1.2

Attribute identifier list	10.7.1.3

Attribute list	10.7.1.4

Attribute value change definition	10.7.1.5

Correlated notifications	10.7.1.7

Event time	10.7.1.8

Event type	10.7.1.9

Managed object class	10.7.4.12

Managed object instance	10.7.4.13

Notification identifier	10.7.1.11

Source indicator	10.7.1.21

Notifications

Attribute Value Change	13.1

Object Creation	13.6

Object Deletion	13.7

## F.2	State management function (CCITT Rec. X.731 | ISO/IEC 10164-2)tc  \l 2 "F.2	State management function (CCITT Rec. X.731 | ISO/IEC 10164-2)"§

Managed Object Classes

State change record	6.12

Packages

Additional information	8.1

Additional text	8.2

Attribute identifier list	8.3

Correlated notifications	8.6

Event time	6.5

Notification Identifier	8.7

Source indicator	8.11

Attributes

Additional information.	10.7.1.1

Additional text	10.7.1.2

Administrative state	10.7.2.1

Alarm status	10.7.2.2

Attribute identifier list	10.7.1.3

Availability status	10.7.2.3

Control status	10.7.2.4

Correlated notifications	10.7.1.7

Event time	10.7.1.8

Event type	10.7.1.9

Managed object class	10.7.4.12

Managed object instance	10.7.4.13

Notification identifier	10.7.1.11

Operational state	10.7.2.5

Procedural status	10.7.2.6

Source indicator	10.7.1.21

Standby status	10.7.2.7

State		10.7.2.10

State change definition	10.7.1.23

Unknown status	10.7.2.8

Usage state	10.7.2.9

Notifications

State Change	13.14

## F.3	Relationship management function (CCITT Rec. X.732 | ISO/IEC 10164-3)tc  \l 2 "F.3	Relationship management function (CCITT Rec. X.732 | ISO/IEC 10164-3)"§

Managed Object Classes

Relationship change record	6.10

Packages

Additional information	8.1

Additional text	8.2

Attribute identifier list	8.3

Correlated notifications	8.6

Event time	6.5

Notification Identifier	8.7

Source indicator	8.11

Attributes

Additional information.	10.7.1.1

Additional text	10.7.1.2

Attribute identifier list	10.7.1.3

Back-up object	10.7.3.1

Backed-up object	10.7.3.2

Correlated notifications	10.7.1.7

Event time	10.7.1.8

Event type	10.7.1.9

Managed object class	10.7.4.12

Managed object instance	10.7.4.13

Member	10.7.3.3

Notification identifier	10.7.1.11

Owner		10.7.3.4

Peer		10.7.3.5

Primary	10.7.3.6

Provider object	10.7.3.7

Relationship change definition	10.7.1.15

Relationships	10.7.3.8

Secondary	10.7.3.9

Source indicator	10.7.1.21

User object	10.7.3.10

Notifications

Relationship Change	13.12

## F.4	Alarm reporting function (CCITT Rec. X.733 | ISO/IEC 10164-4)tc  \l 2 "F.4	Alarm reporting function (CCITT Rec. X.733 | ISO/IEC 10164-4)"§

Managed Object Classes

Alarm record	6.1

Packages

Additional information	8.1

Additional text	8.2

Back up object	6.1

Backed up status	6.1

Correlated notifications	8.6

Event time	6.5

Monitored attributes	6.1

Notification Identifier	8.7

Proposed repair actions	6.1

Specific problems	6.1

Threshold info	6.1

Trend indication	6.1

Attributes

Additional information.	10.7.1.1

Additional text	10.7.1.2

Back-up object	 10.7.3.1

Backed-up status	10.7.1.6

Correlated notifications	10.7.1.7

Event time	10.7.1.8

Event type	10.7.1.9

Managed object class	10.7.4.12

Managed object instance	10.7.4.13

Monitored attributes	10.7.1.10

Notification identifier	10.7.1.11

Perceived severity	10.7.1.12

Probable cause	10.7.1.13

Proposed repair actions	10.7.1.14

Specific problems	10.7.1.22

Threshold info	10.7.1.24

Trend indication	10.7.1.25

Notifications

Communication Alarm	13.2

Environmental Alarm	13.3

Equipment Alarm	13.4

Processing Error Alarm	13.10

Quality of Service Alarm	13.11

## F.5	Event report management function (CCITT Rec. X.734 | ISO/IEC 10164-5)tc  \l 2 "F.5	Event report management function (CCITT Rec. X.734 | ISO/IEC 10164-5)"§

Managed Object Classes

Discriminator	6.3

Event forwarding discriminator	6.4

Packages

Availability status	8.5

Back up destination list	6.4

Daily scheduling	8.8

Duration	8.9

External scheduler	8.10

Mode		6.4

Weekly scheduling	8.12

Attributes

Active destination	10.7.4.1

Administrative state	10.7.2.1

Availability status	10.7.2.3

Back-up destination list	10.7.4.3

Confirmed mode	10.7.4.5

Destination	 10.7.4.7

Discriminator construct	10.7.4.8

Discriminator ID	10.1.1

Intervals of day	10.7.4.9

Operational state	10.7.2.5

Scheduler name	10.7.4.19

Start time	10.7.4.20

Stop time	10.7.4.21

Week mask	10.7.4.23

Notifications

Attribute Value Change	13.1

Object Creation	13.6

Object Deletion	13.7

State Change	13.14

## F.6	Log control function (CCITT Rec. X.735 | ISO/IEC 10164-6)tc  \l 2 "F.6	Log control function (CCITT Rec. X.735 | ISO/IEC 10164-6)"§

Managed Object Classes

Log		6.6

Log record	6.7

Packages

Availability status	8.5

Capacity alarm threshold	6.6

Daily scheduling	8.8

Duration	8.9

External scheduler	8.10

Finite log size	6.6

Weekly scheduling	8.12

Attributes

Administrative state	10.7.2.1

Availability status	10.7.2.3

Capacity alarm threshold	10.7.4.4

Current log size	10.7.4.6

Intervals of day	10.7.4.9

Log full action	10.7.4.10

Log ID	10.1.2

Log record ID	10.1.3

Logging time	10.7.4.11

Max log size	10.7.4.14

Number of records	10.7.4.16

Operational state	10.7.2.5

Scheduler name	10.7.4.19

Start time	10.7.4.20

Stop time	10.7.4.21

Week mask	10.7.4.23

Notifications

Attribute Value Change	13.1

Object Creation	13.6

Object Deletion	13.7

Processing Error Alarm	13.10

State Change	13.14

## F.7	Security alarm reporting function (CCITT Rec. X.736 | ISO/IEC 10164-7)tc  \l 2 "F.7	Security alarm reporting function (CCITT Rec. X.736 | ISO/IEC 10164-7)"§

Managed Object Classes

Security alarm report record	6.11

Packages

Additional information	8.1

Additional text	8.2

Correlated notifications	8.6

Event time	6.5

Notification Identifier	8.7

Attributes

Additional information.	10.7.1.1

Additional text	10.7.1.2

Correlated notifications	10.7.1.7

Event time	10.7.1.8

Event type	10.7.1.9

Managed object class	10.7.4.12

Managed object instance	10.7.4.13

Notification identifier	10.7.1.11

Security alarm cause	10.7.1.16

Security alarm detector	10.7.1.17

Security alarm severity	10.7.1.18

Service provider	10.7.1.19

Service user	10.7.1.20

Notifications

Integrity Violation	13.5

Operational Violation	13.8

Physical Violation	13.9

Security Service or Mechanism Violation	13.13

Time Domain Violation	13.15

Annex G
Syntax imported from Directory, ACSE and CMIP

tc  \l 1 "Annex G  Syntax imported from Directory, ACSE and CMIP" §

(This Annex does not form an integral part of this Recommendation | International Standard)

## G.1	Directory

DistinguishedName ::= RDNSequence

RDNSequence ::= SEQUENCE OF RelativeDistinguishedName

RelativeDistinguishedName ::= SET OF AttributeValueAssertion

AttributeValueAssertion ::= SEQUENCE {AttributeType, AttributeValue}

AttributeType ::= OBJECT IDENTIFIER

AttributeValue ::= ANY

## G.2	CMIP

EventTypeID ::= CHOICE{	globalform	[6] IMPLICIT OBJECT IDENTIFIER,

localForm	[7] IMPLICIT INTEGER}

ObjectClass ::= CHOICE{	globalForm	[0] OBJECT IDENTIFIER,

localForm	[1] INTEGER}

BaseManagedObjectID ::= SEQUENCE {	baseManagedObjectClass	ObjectClass,

baseManagedObjectInstance	ObjectInstance}

ObjectInstance ::= CHOICE {	distinguishedName		[2] IMPLICIT DistinguishedName,

nonSpecificForm		[3] IMPLICIT OCTET STRING,

localDistinguishedName		[4] IMPLICIT RDNSequence}

AttributeID ::= CHOICE{	globalForm	[0] IMPLICIT OBJECT IDENTIFIER,

localForm	[1] IMPLICIT INTEGER}

CMISFilter ::= CHOICE{	item	[8] FilterItem,

and	[9] IMPLICIT SET OF CMISFilter,

or	[10] IMPLICIT SET OF CMISFilter,

not	[11] CMISFilter}

-- For other supporting productions see CCITT Rec. X.711 | ISO/IEC 9596-1.

## G.3	ACSE

AE-title ::= CHOICE {AE-title-form1, AE-title-form2}

AE-title-form1 ::= Name -- imported from Directory with the choice distinguishedName

AE-title-form2 ::= OBJECT IDENTIFIER

)	Presently at state of draft Recommendation.

1