|  | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

| ITU-T | Q.1228 |
| --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR OF ITU | Fascicle 4/5 (09/97) |

SERIES Q: SWITCHING AND SIGNALLING

Intelligent Network

Interface Recommendation for intelligent network Capability Set 2: Part 4

ITU-T  Recommendation  Q.1228  –  Fascicle 4/5

(Previously  CCITT  Recommendation)

ITU-T  Q-SERIES  RECOMMENDATIONS

SWITCHING AND SIGNALLING

| For further details, please refer to ITU-T List of Recommendations. |  |
| --- | --- |
| SIGNALLING IN THE INTERNATIONAL MANUAL SERVICE | Q.1–Q.3 |
| INTERNATIONAL AUTOMATIC AND SEMI-AUTOMATIC WORKING | Q.4–Q.59 |
| FUNCTIONS AND INFORMATION FLOWS FOR SERVICES IN THE ISDN | Q.60–Q.99 |
| CLAUSES APPLICABLE TO ITU-T STANDARD SYSTEMS | Q.100–Q.119 |
| SPECIFICATIONS OF SIGNALLING SYSTEMS No. 4 AND No. 5 | Q.120–Q.249 |
| SPECIFICATIONS OF SIGNALLING SYSTEM No. 6 | Q.250–Q.309 |
| SPECIFICATIONS OF SIGNALLING SYSTEM R1 | Q.310–Q.399 |
| SPECIFICATIONS OF SIGNALLING SYSTEM R2 | Q.400–Q.499 |
| DIGITAL EXCHANGES | Q.500–Q.599 |
| INTERWORKING OF SIGNALLING SYSTEMS | Q.600–Q.699 |
| SPECIFICATIONS OF SIGNALLING SYSTEM No. 7 | Q.700–Q.849 |
| DIGITAL SUBSCRIBER SIGNALLING SYSTEM No. 1 | Q.850–Q.999 |
| PUBLIC LAND MOBILE NETWORK | Q.1000–Q.1099 |
| INTERWORKING WITH SATELLITE MOBILE SYSTEMS | Q.1100–Q.1199 |
| INTELLIGENT NETWORK | Q.1200–Q.1999 |
| BROADBAND ISDN | Q.2000–Q.2999 |

Recommendation Q.1228

Interface recommendation for intelligent
network capability set 2

FASCICLE 4

FOREWORD

ITU (International Telecommunication Union) is the United Nations Specialized Agency in the field of telecommunications. The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of the ITU. The ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Conference (WTSC), which meets every four years, establishes the topics for study by the ITU-T Study Groups which, in their turn, produce Recommendations on these topics.

The approval of Recommendations by the Members of the ITU-T is covered by the procedure laid down in WTSC Resolution No. 1.

In some areas of information technology which fall within ITU-T’s purview, the necessary standards are prepared on a collaborative basis with ISO and IEC.

NOTE

In this Recommendation, the expression "Administration" is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

INTELLECTUAL PROPERTY RIGHTS

The ITU draws attention to the possibility that the practice or implementation of this Recommendation may involve the use of a claimed Intellectual Property Right. The ITU takes no position concerning the evidence, validity or applicability of claimed Intellectual Property Rights, whether asserted by ITU members or others outside of the Recommendation development process.

As of the date of approval of this Recommendation, the ITU had not received notice of intellectual property, protected by patents, which may be required to implement this Recommendation. However, implementors are cautioned that this may not represent the latest information and are therefore strongly urged to consult the TSB patent database.

  ITU  1999

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

CONTENTS

Page

PART 1

1	Introduction		1

2	General		1

2.1	Normative references		1

2.2	Abbreviations and acronyms		3

2.3	Conventions		8

3	Interface recommendation for telecommunication services		8

3.1	General		8

3.1.1	Definition methodology		8

3.1.2	Example physical scenarios		9

3.1.3	INAP protocol architecture		17

3.1.4	INAP addressing		18

3.1.5	Relationship between Recommendation Q.1224 and
this Recommendation		19

3.1.6	Compatibility mechanisms used for INAP		24

3.2	SACF/MACF rules		25

3.2.1	Reflection of TCAP AC		25

3.2.2	Sequential/parallel execution of operations		25

4	Common IN CS-2 Types		25

4.1	Data types		25

4.2	Error types		53

4.3	Operations codes		55

4.4	Error codes		59

4.5	Classes		60

4.6	Object identifiers		68

5	SSF/SCF interface		73

5.1	Operations and arguments		73

5.2	SSF/SCF packages, contracts and Application Contexts		119

5.2.1	Protocol overview		119

5.2.2	SSF/SCF ASN.1 module		138

6	SCF/SRF interface		160

6.1	SCF/SRF operations and arguments		160

6.2	SRF/SCF contracts, packages and Application Contexts		165

Page

6.2.1	Protocol overview		165

6.2.2	SRF/SCF ASN.1 modules		166

7	SCF-SDF interface		169

7.1	Introduction to the reuse of  X.500 for SDF interfaces		169

7.1.1	Alignment between the X.500 concepts and the IN		169

7.1.2	Use of a limited subset of X.500		170

7.1.3	Working assumptions		170

7.2	The SDF Information Model		170

7.2.1	Information framework		170

7.2.2	Basic Access Control		172

7.2.3	Attribute contexts		173

7.2.4	Attribute definitions		174

7.3	The SCF-SDF Interface Protocol		175

7.3.1	Information types and common procedures		175

7.3.2	Operations		177

7.3.3	Errors		180

7.4	Protocol overview		181

7.4.1	Remote Operations		181

7.4.2	Directory ROS-Objects and Contracts		181

7.4.3	DAP Contract and Packages		182

7.5	Directory protocol abstract syntax		183

7.5.1	Abstract syntaxes		183

7.5.2	Directory application contexts		184

7.5.3	Operation codes		185

7.5.4	Error codes		185

7.5.5	Versions and the rules for extensibility		186

7.6	Conformance		187

7.6.1	Conformance by SCFs		187

7.6.2	Conformance by SDFs		188

7.7	ASN.1 modules for the SCF-SDF interface		189

7.7.1	IN-CS2-SDF-InformationFramework module		189

7.7.2	IN-CS2-SDF-BasicAccessControl Module		191

7.7.3	IN-CS2-SCF-SDF-Operations Module		193

7.7.4	IN-CS2-SCF-SDF-Protocol Module		195

8	SDF/SDF interface		198

8.1	Introduction to the IN X.500 DSP and DISP Subset		198

8.2	Working assumptions		198

Page

8.3	The IN X.500 DISP Subset		199

8.3.1	Shadowing agreement specification		199

8.3.2	DSA Shadow Bind		199

8.3.3	IN-DSA Shadow Unbind		199

8.3.4	Coordinate Shadow Update		199

8.3.5	Update Shadow		200

8.3.6	Request Shadow Update		201

8.4	The IN X.500 DSP Subset		202

8.4.1	Information types and common procedures		202

8.4.2	DSA Bind		206

8.4.3	IN DSA Unbind		206

8.4.4	Chained Operations		206

8.4.5	Chained Errors		207

8.5	Protocol overview		207

8.5.1	ROS-Objects and contracts		207

8.5.2	DSP contract and packages		208

8.5.3	DISP contract and packages		209

8.6	Protocol abstract syntax		210

8.6.1	DSP abstract syntax		210

8.6.2	DISP Abstract Syntax		210

8.6.3	Directory System Application Context		211

8.6.4	Directory Shadow Application Context		211

8.6.5	Versions and the rules for extensibility		212

8.7	Conformance		214

8.7.1	Conformance by SDFs		214

8.7.2	Conformance by a shadow supplier		216

8.7.3	Conformance by a shadow consumer		216

8.8	ASN.1 modules for the SDF-SDF interface		217

8.8.1	IN-CS2-SDF-SDF-Protocol Module		217

9	SCF/SCF interface		221

9.1	SCF/SCF operations and arguments		221

9.2	SCF/SCF contracts, packages and Application Contexts		234

9.2.1	Protocol overview		234

9.2.2	ASN.1 modules		238

10	SCF/CUSF interface		243

10.1	Operations and arguments		243

Page

10.2	SCF/CUSF Contracts, Operation Packages, and Application Contexts		249

10.2.1	Protocol overview		249

10.2.2	ASN.1 module		251

PART 2

11	SSF application entity procedures		255

11.1	General		255

11.2	Model and interfaces		255

11.3	Relations between SSF FSM and the CCF and maintenance functions		256

11.4	SSF management finite state model (SSME FSM)		259

11.5	SSF switching state model (SSM) FSM		260

11.5.1	Finite State Model for Call Segment Association (CSA)		263

11.5.2	Finite State Model for Call Segment		268

11.6	Assisting SSF FSM		279

11.6.1	State aa: Idle		279

11.6.2	State ab: Waiting For Instructions		280

11.6.3	State ac: Waiting For End Of User Interaction		281

11.7	Handed-off SSF FSM		282

11.7.1	State ha: Idle		282

11.7.2	State hb: Waiting For Instructions		283

11.7.3	State hc: Waiting For End Of User Interaction		284

11.8	User Service Interaction USI FSM		285

12	SCF application entity procedures		286

12.1	General		286

12.2	Model and interfaces		286

12.3	Relationship between the SCF FSM and the SLPs/maintenance functions		287

12.4	Partial SCF Management Entity (SCME) State Transition Diagram		289

12.4.1	State M1: Status report idle		291

12.4.2	State M2: Waiting for SSF Resource Status Report		291

12.4.3	State M3: Service filtering idle		291

12.4.4	State M4: Waiting for SSF service filtering response		292

12.4.5	State M5: Activity test idle		292

12.4.6	State M6: Waiting for activity test response		292

12.4.7	State M7: ManageTriggerData idle		292

12.4.8	State M8: Waiting for ManageTriggerData activity test response		293

12.4.9	The Resource Control Object		293

Page

12.5	The SCF Call State Model (SCSM)		293

12.5.1	SSF/SRF-related states (SCSM-SSF/SRF)		294

12.5.2	SDF-related states (SCSM-SDF)		330

12.5.3	SCF-related states		331

12.5.4	CUSF-related states (SCSM-CUSF)		342

12.5.5	USI_SCF FSM		346

13	SRF application entity procedures		346

13.1	General		346

13.2	Model and interfaces		347

13.3	Relationship between the SRF FSM and maintenance functions/bearer connection handling		348

13.4	The SRSM		349

13.4.1	 State 1: Idle		351

13.4.2	State 2: Connected		352

13.4.3	State 3: User interaction		353

13.5	Example SRF control procedures		354

13.5.1	SRF connect procedures		355

13.5.2	SRF end user interaction procedures		359

13.5.3	SRF disconnection procedures		361

13.5.4	Examples illustrating Complete User Interaction Sequences		364

14	SDF application entity procedures		371

14.1	General		371

14.2	Model and interfaces		372

14.3	The SDF FSM structure		373

14.4	SDF state transition models		374

14.4.1	SDF state transition model for SCF-related states		374

14.4.2	SDF state transition model for SDF-related states		376

15	CUSF application entity procedures		394

15.1	General		394

15.2	Model and interfaces		394

15.2.1	Background for the modelling and protocol		395

15.2.2	Modelling and protocol		396

15.3	Relations between CUSF FSM and the SSF/CCF and maintenance functions		397

15.4	CUSF management finite state model (CUSME FSM)		398

15.5	CUSF state transition diagram		398

15.5.1	State a: Idle		400

Page

15.5.2	State b: Waiting For Instructions		401

15.5.3	State c: Monitoring		401

16	Error procedures		402

16.1	Operation related error procedures		402

16.1.1	AttributeError		402

16.1.2	Cancelled		404

16.1.3	CancelFailed		405

16.1.4	DSAReferral		406

16.1.5	ETCFailed		407

16.1.6	ExecutionError		408

16.1.7	ImproperCallerResponse		409

16.1.8	MissingCustomerRecord		411

16.1.9	MissingParameter		418

16.1.10	Name Error		428

16.1.11	ParameterOutOfRange		430

16.1.12	Referral		432

16.1.13	RequestedInfoError		433

16.1.14	ScfReferral		433

16.1.15	Security		435

16.1.16	Service		443

16.1.17	Shadow		444

16.1.18	SystemFailure		446

16.1.19	TaskRefused		449

16.1.20	UnavailableResource		452

16.1.21	UnexpectedComponentSequence		453

16.1.22	UnexpectedDataValue		456

16.1.23	UnexpectedParameter		459

16.1.24	UnknownLegID		462

16.1.25	UnknownResource		463

16.1.26	Update		463

16.1.27	ChainingRefused		464

16.1.28	DirectoryBindError		467

16.1.29	ScfBindFailure		469

16.1.30	ScfTaskRefused		471

16.2	Entity related error procedures		472

16.2.1	Expiration of TSSF		472

16.2.2	Expiration of TSRF		472

16.2.3	Expiration of Tcusf		473

Page

PART 3

17	Detailed operation procedures		475

17.1	ActivateServiceFiltering procedure		475

17.1.1	General description		475

17.1.2	Invoking entity (SCF)		478

17.1.3	Responding entity (SSF)		479

17.2	ActivationReceivedAndAuthorized procedure		480

17.2.1	General description		480

17.2.2	Invoking entity (CUSF)		481

17.2.3	Responding entity (SCF)		481

17.3	ActivityTest procedure		481

17.3.1	General description		481

17.3.2	Invoking entity (SCF)		482

17.3.3	Responding entity (SSF)		482

17.3.4	Responding entity (CUSF)		482

17.3.5	Responding entity (controlling SCF or supporting SCF)		483

17.4	AddEntry procedure		483

17.4.1	General description		483

17.4.2	Invoking entity (SCF)		483

17.4.3	Responding entity (SDF)		484

17.5	AnalysedInformation procedure		485

17.5.1	General description		485

17.5.2	Invoking entity (SSF)		488

17.5.3	Responding entity (SCF)		489

17.5.4	Error handling		490

17.6	AnalyseInformation procedure		490

17.6.1	General description		490

17.6.2	Invoking entity (SCF)		491

17.6.3	Responding entity (SSF)		492

17.7	ApplyCharging procedure		494

17.7.1	General description		494

17.7.2	Invoking entity (SCF)		495

17.7.3	Responding entity (SSF)		495

17.8	ApplyChargingReport procedure		496

17.8.1	General description		496

17.8.2	Invoking entity (SSF)		496

Page

17.8.3	Responding entity (SCF)		497

17.9	AssistRequestInstructions procedure		497

17.9.1	General description		497

17.9.2	Invoking entity (SSF/SRF)		497

17.9.3	Responding entity (SCF)		498

17.10	AssociationReleaseRequested procedure		498

17.10.1	General description		498

17.10.2	Invoking entity (CUSF)		499

17.10.3	Responding entity (SCF)		500

17.11	AuthorizeTermination procedure		500

17.11.1	General description		500

17.11.2	Invoking entity (SSF/SRF)		501

17.11.3	Responding entity (SSF)		502

17.12	CallGap procedure		502

17.12.1	General description		502

17.12.2	Invoking entity (SCF)		505

17.12.3	Responding entity (SSF)		505

17.13	CallInformationReport procedure		506

17.13.1	General description		506

17.13.2	Invoking entity (SSF)		507

17.13.3	Responding entity (SCF)		508

17.13.4	Error handling		508

17.14	CallInformationRequest procedure		508

17.14.1	General description		508

17.14.2	Invoking entity (SCF)		509

17.14.3	Responding entity (SSF)		510

17.15	Cancel procedure		510

17.15.1	General description		510

17.15.2	Invoking entity (SCF)		511

17.15.3	Responding entity (SRF)		511

17.15.4	Responding entity (SSF)		512

17.16	CancelStatusReportRequest procedure		512

17.16.1	General description		512

17.16.2	Invoking entity (SCF)		512

17.16.3	Responding entity (SSF)		512

17.17	chainedAddEntry procedure		513

17.17.1	General description		513

17.17.2	Invoking entity (SDF)		513

Page

17.17.3	Responding entity (SDF)		514

17.18	ChainedConfirmedNotificationProvided procedure		514

17.18.1	General description		514

17.18.2	Invoking entity (chaining initiator supporting SCF)		515

17.18.3	Responding entity (chaining terminator supporting SCF)		515

17.19	ChainedConfirmedReportChargingInformation  procedure		516

17.19.1	General description		516

17.19.2	Invoking entity (chaining initiator supporting SCF)		516

17.19.3	Responding entity (chaining terminator supporting SCF)		516

17.20	ChainedEstablishChargingRecord procedure		516

17.20.1	General description		517

17.20.2	Invoking entity (chaining terminator supporting SCF)		517

17.20.3	Responding entity (chaining initiator supporting SCF)		518

17.21	chainedExecute procedure		518

17.21.1	General description		518

17.21.2	Invoking entity (SDF)		518

17.21.3	Responding entity (SDF)		519

17.22	ChainedHandlingInformationRequest procedure		520

17.22.1	General description		520

17.22.2	Invoking entity (chaining initiator supporting SCF)		520

17.22.3	Responding entity (chaining terminator supporting SCF)		521

17.23	ChainedHandlingInformationResult procedure		521

17.23.1	General description		521

17.23.2	Invoking entity (chaining terminator supporting SCF)		522

17.23.3	Responding entity (chaining initiator supporting SCF)		522

17.24	chainedModifyEntry procedure		522

17.24.1	General description		522

17.24.2	Invoking entity (SDF)		523

17.24.3	Responding entity (SDF)		523

17.25	ChainedNetworkCapability procedure		524

17.25.1	General description		524

17.25.2	Invoking entity (chaining terminator supporting SCF)		525

17.25.3	Responding entity (chaining initiator supporting SCF)		525

17.26	ChainedNotificationProvided procedure		525

17.26.1	General description		525

17.26.2	Invoking entity (chaining initiator supporting SCF)		526

17.26.3	Responding entity (chaining terminator supporting SCF)		526

17.27	ChainedReportChargingInformation procedure		527

Page

17.27.1	General description		527

17.27.2	Invoking entity (chaining initiator supporting SCF)		527

17.27.3	Responding entity (chaining terminator supporting SCF)		528

17.28	ChainedProvideUserInformation procedure		528

17.28.1	General description		528

17.28.2	Invoking entity (chaining terminator supporting SCF)		529

17.28.3	Responding entity (chaining initiator supporting SCF)		529

17.29	chainedRemoveEntry procedure		529

17.29.1	General description		529

17.29.2	Invoking entity (SDF)		530

17.29.3	Responding entity (SDF)		530

17.30	ChainedRequestNotification procedure		531

17.30.1	General description		531

17.30.2	Invoking entity (chaining terminator supporting SCF)		532

17.30.3	Responding entity (chaining initiator supporting SCF)		532

17.31	chainedSearch procedure		532

17.31.1	General description		532

17.31.2	Invoking entity (SDF)		532

17.31.3	Responding entity (SDF)		533

17.32	CollectedInformation procedure		534

17.32.1	General description		534

17.32.2	Invoking entity (SSF)		535

17.32.3	Responding entity (SCF)		537

17.33	CollectInformation procedure		538

17.33.1	General description		538

17.33.2	Invoking entity (SCF)		539

17.33.3	Responding entity (SSF)		539

17.34	ComponentReceived procedure		540

17.34.1	General description		540

17.34.2	Invoking entity (CUSF)		541

17.34.3	Responding entity (SCF)		541

17.35	ConfirmedNotificationProvided procedure		542

17.35.1	General description		542

17.35.2	Invoking entity (controlling SCF)		542

17.35.3	Responding entity (supporting SCF)		543

17.36	ConfirmedReportChargingInformation procedure		543

17.36.1	General description		543

17.36.2	Invoking entity (controlling SCF)		544

Page

17.36.3	Responding entity (supporting SCF)		544

17.37	Connect procedure		545

17.37.1	General description		545

17.37.2	Invoking entity (SCF)		547

17.37.3	Responding entity (SSF)		548

17.38	ConnectToResource procedure		549

17.38.1	General description		549

17.38.2	Invoking entity (SCF)		550

17.38.3	Responding entity (SSF)		550

17.39	Continue procedure		551

17.39.1	General description		551

17.39.2	Invoking entity (SCF)		551

17.39.3	Responding entity (SSF)		551

17.40	ContinueWithArgument procedure		552

17.40.1	General description		552

17.40.2	Invoking entity (SCF)		552

17.40.3	Responding entity (SSF)		553

17.41	CoordinateShadowUpdate procedure		553

17.41.1	General description		553

17.41.2	Supplier entity (SDF)		554

17.41.3	Consumer entity (SDF)		555

17.42	CreateCallSegmentAssociation procedure		556

17.42.1	General description		556

17.42.2	Invoking entity (SCF)		556

17.42.3	Responding entity (SSF)		556

17.43	in-directoryBind procedure		557

17.43.1	General description		557

17.43.2	Invoking entity (SCF)		557

17.43.3	Responding entity (SDF)		557

17.44	DirectoryUnbind procedure		558

17.44.1	General description		558

17.44.2	Invoking entity (SCF)		558

17.44.3	Responding entity (SDF)		558

17.45	DisconnectForwardConnection procedure		559

17.45.1	General description		559

17.45.2	Invoking entity (SCF)		559

17.45.3	Responding entity (SSF)		560

17.46	DisconnectForwardConnectionWithArgument procedure		560

Page

17.46.1	General description		560

17.46.2	Invoking entity (SCF)		561

17.46.3	Responding entity (SSF)		561

17.47	DisconnectLeg procedure		562

17.47.1	General description		562

17.47.2	Invoking entity (SCF)		562

17.47.3	Responding entity (SSF)		562

17.48	dSABind procedure		563

17.48.1	General description		563

17.48.2	Invoking entity (SDF)		563

17.48.3	Responding entity (SDF)		564

17.49	DSAShadowBind procedure		564

17.49.1	General description		564

17.49.2	Supplier entity (SDF)		565

17.49.3	Consumer entity (SDF)		567

17.50	in-DSAShadowUnbind procedure		568

17.50.1	General description		568

17.50.2	Supplier entity (SDF)		569

17.50.3	Consumer entity (SDF)		569

17.51	EntityReleased procedure		570

17.51.1	General description		570

17.51.2	Invoking entity (SSF)		571

17.51.3	Responding entity (SCF)		571

17.52	EstablishChargingRecord procedure		572

17.52.1	General description		572

17.52.2	Invoking entity (supporting SCF)		572

17.52.3	Responding entity (controlling SCF)		573

17.53	EstablishTemporaryConnection procedure		573

17.53.1	General description		573

17.53.2	Invoking entity (SCF)		574

17.53.3	Responding entity (SSF)		574

17.54	EventNotificationCharging procedure		575

17.54.1	General description		575

17.54.2	Invoking entity (SSF)		576

17.54.3	Responding entity (SCF)		576

17.55	EventReportBCSM procedure		577

17.55.1	General description		577

17.55.2	Invoking entity (SSF)		579

Page

17.55.3	Responding entity (SCF)		579

17.56	EventReportFacility procedure		580

17.56.1	General description		580

17.56.2	Invoking entity (SSF)		580

17.56.3	Responding entity (SCF)		581

17.57	Execute procedure		581

17.57.1	General description		581

17.57.2	Invoking entity (SCF)		582

17.57.3	Responding entity (SDF)		582

17.58	FacilitySelectedAndAvailable procedure		584

17.58.1	General description		584

17.58.2	Invoking entity (SSF)		585

17.58.3	Responding entity (SCF)		585

17.59	FurnishChargingInformation procedure		586

17.59.1	General description		586

17.59.2	Invoking entity (SCF)		586

17.59.3	Responding entity (SCF)		587

17.60	HandlingInformationRequest procedure		588

17.60.1	General description		588

17.60.2	Invoking entity (controlling SCF)		589

17.60.3	Responding entity (supporting SCF)		590

17.61	HandlingInformationResult procedure		591

17.61.1	General description		591

17.61.2	Invoking entity (supporting SCF)		592

17.61.3	Responding entity (controlling SCF)		592

17.62	HoldCallInNetwork procedure		593

17.62.1	General description		593

17.62.2	Invoking entity (SCF)		593

17.62.3	Responding entity (SSF)		593

17.63	in-DSAUnbind procedure		593

17.63.1	General description		593

17.63.2	Invoking entity (SDF)		594

17.63.3	Responding entity (SDF)		594

17.64	InitialDP procedure		594

17.64.1	General description		594

17.64.2	Invoking entity (SSF)		598

17.64.3	Responding entity (SCF)		599

17.65	InitiateAssociation procedure		599

Page

17.65.1	General description		599

17.65.2	Invoking entity (SCF)		599

17.65.3	Responding entity (CUSF)		600

17.66	InitiateCallAttempt procedure		600

17.66.1	General description		600

17.66.2	Invoking entity (SCF)		601

17.66.3	Responding entity (SSF)		602

17.67	ManageTriggerData procedure		602

17.67.1	General description		602

17.67.2	Invoking entity (SCF)		603

17.67.3	Responding entity (SSF)		603

17.68	MergeCallSegments procedure		604

17.68.1	General description		604

17.68.2	Invoking entity (SCF)		604

17.68.3	Responding entity (SSF)		605

17.69	ModifyEntry procedure		605

17.69.1	General description		605

17.69.2	Invoking entity (SCF)		605

17.69.3	Responding entity (SDF)		606

17.70	MoveCallSegments procedure		607

17.70.1	General description		607

17.70.2	Invoking entity (SCF)		608

17.70.3	Responding entity (SSF)		608

17.71	MoveLeg procedure		608

17.71.1	General description		608

17.71.2	Invoking entity (SCF)		609

17.71.3	Responding entity (SSF)		609

17.72	NetworkCapability procedure		610

17.72.1	General description		610

17.72.2	Invoking entity (supporting SCF)		610

17.72.3	Responding entity (controlling SCF)		611

17.73	NotificationProvided procedure		611

17.73.1	General description		611

17.73.2	Invoking entity (controlling SCF)		612

17.73.3	Responding entity (supporting SCF)		612

17.74	OAbandon procedure		613

17.74.1	General description		613

17.74.2	Invoking entity (SSF)		613

Page

17.74.3	Responding entity (SCF)		613

17.75	OAnswer procedure		614

17.75.1	General description		614

17.75.2	Invoking entity (SSF)		615

17.75.3	Responding entity (SCF)		615

17.76	OCalledPartyBusy procedure		616

17.76.1	General description		616

17.76.2	Invoking entity (SSF)		617

17.76.3	Responding entity (SCF)		618

17.77	ODisconnect procedure		618

17.77.1	General description		618

17.77.2	Invoking entity (SSF)		619

17.77.3	Responding entity (SCF)		620

17.78	OMidCall procedure		620

17.78.1	General description		620

17.78.2	Invoking entity (SSF)		621

17.78.3	Responding entity (SCF)		622

17.79	ONoAnswer procedure		622

17.79.1	General description		622

17.79.2	Invoking entity (SSF)		623

17.79.3	Responding entity (SCF)		624

17.80	OriginationAttempt procedure		625

17.80.1	General description		625

17.80.2	Invoking entity (SSF)		626

17.80.3	Responding entity (SCF)		626

17.81	OriginationAttemptAuthorized procedure		626

17.81.1	General description		626

17.81.2	Invoking entity (SSF)		627

17.81.3	Responding entity (SCF)		628

17.82	OSuspended procedure		628

17.82.1	General description		628

17.82.2	Invoking entity (SSF)		629

17.82.3	Responding entity (SCF)		629

17.83	PlayAnnouncement procedure		630

17.83.1	General description		630

17.83.2	Invoking entity (SCF)		631

17.83.3	Responding entity (SRF)		632

17.84	PromptAndCollectUserInformation procedure		632

Page

17.84.1	General description		632

17.84.2	Invoking entity (SCF)		636

17.84.3	Responding entity (SRF)		637

17.85	PromptAndReceiveMessage procedure		638

17.85.1	General description		638

17.85.2	Invoking entity (SCF)		641

17.85.3	Responding entity (SRF)		641

17.86	ProvideUserInformation procedure		642

17.86.1	General description		642

17.86.2	Invoking entity (supporting SCF)		643

17.86.3	Responding entity (controlling SCF)		643

17.87	Reconnect procedure		644

17.87.1	General description		644

17.87.2	Invoking entity (SCF)		645

17.87.3	Responding entity (SSF)		645

17.88	ReleaseAssociation procedure		645

17.88.1	General description		645

17.88.2	Invoking entity (SCF)		646

17.88.3	Responding entity (CUSF)		646

17.89	ReleaseCall procedure		646

17.89.1	General description		646

17.89.2	Invoking entity (SCF)		647

17.89.3	Responding entity (SSF)		647

17.90	RemoveEntry procedure		648

17.90.1	General description		648

17.90.2	Invoking entity (SCF)		648

17.90.3	Responding entity (SDF)		648

17.91	ReportChargingInformation procedure		649

17.91.1	General description		649

17.91.2	Invoking entity (controlling SCF)		650

17.91.3	Responding entity (supporting SCF)		650

17.92	ReportUTSI procedure		650

17.92.1	General description		650

17.92.2	Invoking entity (SSF)		651

17.92.3	Responding entity (SCF)		651

17.93	RequestCurrentStatusReport procedure		652

17.93.1	General description		652

17.93.2	Invoking entity (SCF)		652

Page

17.93.3	Responding entity (SSF)		652

17.94	RequestEveryStatusChangeReport procedure		653

17.94.1	General description		653

17.94.2	Invoking entity (SCF)		653

17.94.3	Responding entity (SSF)		654

17.95	RequestFirstStatusMatchReport procedure		654

17.95.1	General description		654

17.95.2	Invoking entity (SCF)		655

17.95.3	Responding entity (SSF)		655

17.96	RequestNotification procedure		656

17.96.1	General description		656

17.96.2	Invoking entity (supporting SCF)		656

17.96.3	Responding entity (controlling SCF)		657

17.97	RequestNotificationChargingEvent procedure		657

17.97.1	General description		657

17.97.2	Invoking entity (SCF)		658

17.97.3	Responding entity (SSF)		658

17.98	RequestReportBCSMEvent procedure		659

17.98.1	General description		659

17.98.2	Invoking entity (SCF)		661

17.98.3	Responding entity (SSF)		661

17.99	RequestReportBCUSMEvent procedure		662

17.99.1	General description		662

17.99.2	Parameters		662

17.99.3	Invoking entity (SCF)		662

17.99.4	Responding entity (CUSF)		662

17.100	RequestReportFacilityEvent procedure		663

17.100.1	General description		663

17.100.2	Invoking entity (SCF)		663

17.100.3	Responding entity (SSF)		664

17.101	RequestReportUTSI procedure		664

17.101.1	General description		664

17.101.2	Invoking entity (SCF)		665

17.101.3	Responding entity (SSF)		665

17.102	RequestShadowUpdate procedure		665

17.102.1	General description		665

17.102.2	Supplier entity (SDF)		666

17.102.3	Consumer entity (SDF)		667

Page

17.103	ResetTimer procedure		667

17.103.1	General description		667

17.103.2	Invoking entity (SCF)		668

17.103.3	Responding entity (SSF)		668

17.104	RouteSelectFailure procedure		669

17.104.1	General description		669

17.104.2	Invoking entity (SSF)		669

17.104.3	Responding entity (SCF)		670

17.105	SCFBind procedure		671

17.105.1	General description		671

17.105.2	Responding entity (supporting SCF)		672

17.106	scfBind procedure (in the chaining case)		672

17.106.1	General description		672

17.106.2	Invoking entity (chaining initiator supporting SCF)		673

17.106.3	Responding entity (chaining terminator supporting SCF)		673

17.107	SCFUnBind procedure		673

17.107.1	General description		673

17.107.2	Invoking entity (controlling SCF)		673

17.107.3	Responding entity (supporting SCF)		674

17.108	scfUnBind procedure (in the chaining case)		674

17.108.1	General description		674

17.108.2	Invoking entity (chaining terminator supporting SCF)		674

17.108.3	Responding entity (chaining terminator supporting SCF)		675

17.109	ScriptClose procedure		675

17.109.1	General description		675

17.109.2	Invoking entity (SCF)		675

17.109.3	Responding entity (SRF)		676

17.110	ScriptEvent procedure		676

17.110.1	General Description		676

17.110.2	Invoking entity (SRF)		676

17.110.3	Responding entity (SCF)		677

17.111	ScriptInformation procedure		678

17.111.1	General description		678

17.111.2	Invoking entity (SCF)		678

17.111.3	Responding entity (controlling SRF)		679

17.112	ScriptRun procedure		679

17.112.1	General description		679

17.112.2	Invoking entity (SCF)		680

Page

17.112.3	Responding entity (SRF)		680

17.113	Search procedure		680

17.113.1	General description		680

17.113.2	Invoking entity (SCF)		681

17.113.3	Responding entity (SDF)		681

17.114	SelectFacility procedure		682

17.114.1	General description		682

17.114.2	Invoking entity (SCF)		683

17.114.3	Responding entity (SSF)		684

17.115	SelectRoute procedure		684

17.115.1	General description		684

17.115.2	Invoking entity (SCF)		686

17.115.3	Responding entity (SSF)		686

17.116	SendChargingInformation procedure		688

17.116.1	General description		688

17.116.2	Invoking entity (SCF)		688

17.116.3	Responding entity (SSF)		689

17.117	SendComponent procedure		690

17.117.1	General description		690

17.117.2	Invoking entity (SCF)		691

17.117.3	Responding entity (CUSF)		691

17.118	SendFacilityInformation procedure		691

17.118.1	General description		691

17.118.2	Invoking entity (SCF)		692

17.118.3	Responding entity (SSF)		692

17.119	SendSTUI procedure		693

17.119.1	General description		693

17.119.2	Invoking entity (SCF)		693

17.120	ServiceFilteringResponse procedure		694

17.120.1	General description		694

17.120.2	Invoking entity (SSF)		694

17.120.3	Responding entity (SCF)		695

17.121	SpecializedResourceReport procedure		696

17.121.1	General description		696

17.121.2	Invoking entity (SRF)		696

17.121.3	Responding entity (SCF)		696

17.122	SplitLeg procedure		696

17.122.1	General description		696

Page

17.122.2	Invoking entity (SCF)		697

17.122.3	Responding entity (SSF)		697

17.123	StatusReport procedure		698

17.123.1	General description		698

17.123.2	Invoking entity (SSF)		698

17.123.3	Responding entity (SCF)		699

17.124	TAnswer procedure		699

17.124.1	General description		699

17.124.2	Invoking entity (SSF)		700

17.124.3	Responding entity (SCF)		700

17.125	TBusy procedure		701

17.125.1	General description		701

17.125.2	Invoking entity (SSF)		702

17.125.3	Responding entity (SCF)		702

17.126	TDisconnect procedure		703

17.126.1	General description		703

17.126.2	Invoking entity (SSF)		704

17.126.3	Responding entity (SCF)		704

17.127	TerminationAttempt procedure		705

17.127.1	General description		705

17.127.2	Invoking entity (SSF)		706

17.127.3	Responding entity (SCF)		706

17.128	TermAttemptAuthorized procedure		706

17.128.1	General description		706

17.128.2	Invoking entity (SSF)		707

17.128.3	Responding entity (SCF)		708

17.129	TMidCall procedure		708

17.129.1	General description		708

17.129.2	Invoking entity (SSF)		709

17.129.3	Responding entity (SCF)		710

17.130	TNoAnswer procedure		710

17.130.1	General description		710

17.130.2	Invoking entity (SSF)		711

17.130.3	Responding entity (SCF)		712

17.131	TSuspended procedure		713

17.131.1	General description		713

17.131.2	Invoking entity		713

17.131.3	Responding entity (SCF)		713

Page

17.132	UpdateShadow procedure		714

17.132.1	General description		714

17.132.2	Supplier entity (SDF)		714

17.132.3	Consumer entity (SDF)		716

PART 4

18	Services assumed from Lower Layers		719

18.1	Services assumed from TCAP		719

18.1.1	Common procedures		719

18.1.2	SSF-SCF interface		730

18.1.3	SCF-SRF interface		737

18.1.4	SCF-CUSF interface		738

18.1.5	SCF-SCF interface		740

18.1.6	SCF-SDF interface		743

18.1.7	SDF-SDF interface		746

18.2	Services assumed from SCCP		749

18.2.1	Normal procedures		749

18.2.2	Service functions from SCCP		749

19	IN generic interface security		752

19.1	Interface security requirements		752

19.1.1	Data confidentiality		752

19.1.2	Data integrity and data origin authentication		752

19.1.3	Key management		753

19.2	Procedures and algorithms		753

19.2.1	Authentication procedures		753

19.2.2	SPKM algorithms and negotiation		754

19.2.3	Three-way mutual authentication		755

19.2.4	Assignment of credentials		755

19.3	Mapping of security information flow definitions to tokens		755

19.4	Security FSM definitions		756

19.4.1	Two-way mutual authentication FSMs		756

19.4.2	Three-way mutual authentication FSMs		759

Page

PART 5

Annex A.1 – Introduction to the INAP CS-1 and CS-2 SDL models		763

A.1.1	Introduction		763

A.1.2	Example for the interworking of the SSF/CCF SDL processes		768

A.1.3	Example for the Three-Party Call setup as seen from the environment		770

Annex A.2 – Transition diagrams		773

A.2.1	Call Segment Association transition diagram		773

A.2.2	Call Segment transition diagram		774

Annex A.3 – SDL Specification of CS-1 SSF/CCF		787

PART 6

Annex A.4 – SDL Specification of CS-2 extensions to SSF/CCF		788

PART 7

Annex A.5 – SDL Specification of CS-2 SRF		789

Annex A.6 – SDL Specification of CS-2 Assist/Hand-off SSF		789

Annex A.7 – SDL Specification of CS-2 CUSF		789

Annex A.8 – SDL Specification of CS-2 SCF		789

Appendix I – Expanded ASN.1 source		790

Appendix II – Data modelling		819

II.1	Introduction		819

II.1.1	Purpose and scope		819

II.1.2	Assumptions		819

II.2	Directory Information Tree (DIT) schema		820

II.2.1	X.500 DIT		820

II.2.2	Object classes		822

II.2.3	Attribute types		823

II.2.4	DIT structure definition		824

Appendix III – Examples of SPKM algorithms for IN CS-2		826

III.1	General		826

III.2	Integrity Algorithm (I-ALG)		826

III.2.1	Example–1		826

III.2.2	Example–2		827

III.2.3	Example–3		827

III.2.4	Example–4		827

Page

III.3	Confidentiality Algorithm (C-ALG)		828

III.3.1	Example–1		828

III.4	Key Establishment Algorithm (K-ALG)		828

III.4.1	Example–1		828

III.4.2	Example–2		828

III.4.3	Example–3		828

III.5	One-Way Function (O-ALG) for Subkey Derivation Algorithm		828

III.5.1	Example–1		829

Recommendation Q.1228

Interface recommendation for intelligent
network capability set 2

(Geneva, 1997)

part 4

# 18	Services assumed from Lower Layers

## 18.1	Services assumed from TCAP

The SS7 application layer protocol defined in this Recommendation is a protocol to provide communication between a pair of application processes. In the SS7, environment this is represented as communication between a pair of Application Entities (AEs) using the Transaction Capabilities. The function of an AE is provided by a set of Application Service Elements (ASEs). The interaction between AEs is described in terms of their use of the services provided by the ASEs.

If Application Contexts (ACs) are to be used for FE differentiation within a physical node, then the version of TC used must support the dialogue portion of TC (i.e. White Book, TC).

This requirement applies to all interfaces, not just those used for internetworking.

Table 18-1 defines which versions of TC are the minimum versions required to support the defined IN interfaces:

Table 18-1/Q.1228 – Minimum TC Recommendations
requirements for INAP interfaces

| Interface | IN CS-2 |
| --- | --- |
| SSF-SCF | Blue Book (Note) |
| SCF-SRF | Blue Book (Note) |
| SCF-SDF | White Book (1993) |
| SCF-SCF | White Book (1993) |
| SDF-SDF | White Book (1993) |
| CUSF-SCF | Blue Book (Note) |
| NOTE – If AC negotiation is required, then White Book (1993) is the minimum version required. |  |

### 18.1.1	Common procedures

This subclause defines the procedures and mapping which apply between INAP and TC to be used in the absence of specific procedures and mapping instructions for the specific INAP interfaces as defined in subsequent subclauses.

#### 18.1.1.1	Normal procedures

This subclause describes the procedures and TCAP primitives that shall be used for transmitting messages between AEs under normal operation.

The INAP, as TC-user, uses only the structured dialogue facility provided by TCAP. The following situations can occur when a message is sent between two physical entities:

–	a dialogue shall be established: the TC-user issues a TC-BEGIN request primitive;

–	a dialogue shall be maintained: the TC-user issues a TC-CONTINUE request primitive;

–	a dialogue shall no longer be maintained: the TC-user issues a TC-END request primitive with either basic end or with prearranged end depending on the following conditions:

•	Basic end:

–	In the case the dialogue is established, operations, leading to a termination of the relationship, can be transmitted by the FE with a TC-END request primitive (basic) in case the FE is not interested in the reception of any ERROR or REJECT components for these sent operations. Once the FE dialogue resources have been released, any ERROR or REJECT components received for these operations will be discarded by TC as described in Recommendation Q.774.

–	In the case where a dialogue has been established and the FE receives an operation leading to the termination of the relationship, and there is no interest by the FE to continue the dialogue, and there is no operation to be sent, a TC-END request primitive (basic) with zero components can be sent from the FE.

•	Prearranged end:

In this case, an entity is interested in possible ERROR or REJECT messages on response to sent operations leading to a termination of the relationship, the dialogue is ended with a TC-END request primitive (prearranged end) after the last associated operation timer expires. The receiving entity can end the dialogue with a TC-END request primitive (prearranged end) after successful processing of these operations (i.e. the relationship is terminated).

–	in general, the use of prearranged end shall be limited to the case for both communicating entities clearly recognisable that peer entity applies prearranged end. In all other cases, basic end shall be used;

–	a dialogue shall not be established: for class 2 or 4 operations only the sending TC-user issues a TC-BEGIN request primitive and ends the dialogue locally after operation time-out by means of a prearranged end. Upon reception of the TC-BEGIN indication primitive, the receiving TC-user shall end the dialogue locally.

#### 18.1.1.2	Abnormal procedures

This subclause describes the procedures and TCAP primitives that shall be used for reporting abnormal situations between AEs. The error cases are defined in clause 16.

The following primitives shall be used to report abnormal situations:

–	operation errors, as defined in the INAP, are reported with TC-U-ERROR request primitive;

–	rejection of a TCAP component by the TC-user shall be reported with TC-U-REJECT request primitive;

–	when the FE detecting error or rejecting operation decides the termination of TC dialogue, TC-END request primitive (basic) with error or reject can be used for the termination of TC dialogue;

–	when the SSF, the SRF, or the CUSF detecting error or rejecting operation recognises the possibility to continue dialogue, TC-CONTINUE request primitive with error or reject can be used for the continuation of TC dialogue;

–	a dialogue shall be aborted by the TC-user with a TC-U-ABORT request primitive;

–	on expiration of application timer TSSF or TSRF or TCUSF dialogue shall be terminated by means of a TC-U-ABORT primitive with an Abort reason, regardless of whether TCAP dialogue is established or not.

For abnormal situations detected by TCAP, the same rules shall apply for reception of TC-R-REJECT indication as for transmission of TC-U-REJECT request and for transmission of TC-P-ABORT indication as for transmission of TC-U-ABORT request primitive.

The following rules shall be applied to terminate the TCAP dialogue under abnormal situations:

–	in the case that abort condition is detected and TCAP dialogue is established, TCAP dialogue is terminated by TC-U-ABORT primitive with an Abort reason;

–	in the case that abort condition is detected and TCAP dialogue is not established, TCAP dialogue is locally terminated by TC-U-ABORT primitive (in the case such as application time-out).

In error situations, prearranged end shall not be used to terminate the TCAP dialogue. In case any application entity encounters an error situation the peer entity shall be explicitly notified of the error, if possible. If from any entity's point of view the error encountered requires the relationship to be ended, it shall close the dialogue via a TC-END request primitive with basic end or via a  request primitive, depending on whether any pending ERROR or REJECT component is to be sent or not.

In case an entity receives a TC-END indication primitive and after all components have been considered, the FSM is not in a state to terminate the relationship, an appropriate internal error should be provided.

In cases when a dialogue needs to be closed by the initiating entity before its establishment has been completed (before the first TC indication primitive to the TC-BEGIN request primitive has been received from the responding entity), the TC-user shall issue a TC-END request primitive with prearranged end or a TC-U-ABORT request primitive. The result of these primitives will be only local, any subsequent TC indication received for this dialogue will be handled according to the abnormal procedures as specified in Recommendation Q.774.

#### 18.1.1.3	Dialogue handling

##### 18.1.1.3.1	Dialogue establishment

The establishment of an INAP dialogue involves two application processes as described in 3.1.3, one that is the dialogue-initiator and one that is the dialogue-responder.

Application context negotiation may not be supported in all physical entities and/or all networks.

This procedure is driven by the following signals:

–	a TC-BEGIN request primitive from the dialogue-initiator;

–	a TC-BEGIN indication primitive occurring at the responding side;

–	the first TC-CONTINUE indication primitive occurring at the initiating side or under specific conditions:

•	a TC-END indication primitive occurring at the initiating side;

•	a TC-U-ABORT indication primitive occurring at the initiating side;

•	a TC-P-ABORT indication primitive occurring at the initiating side.

Sending of a TC-BEGIN request

Before issuing a TC-BEGIN request primitive, SACF shall store the AC-name and, if present, the user-information parameter.

SACF shall request the invocation of the associated operations using the TC-INVOKE service. See 18.1.1.4.1 for a description of the invocation procedure.

After processing of the last invocation request, SACF shall issue a TC-BEGIN request primitive.

The initiator SACF then waits for a TC indication primitive and will not issue any other requests, except a TC-U-ABORT request or a TC-END request with the release method parameter set to "prearranged release".

If no TC indication primitive is expected because no dialogue is to be established according to the rules as stated in 18.1.2.1.1 and 18.1.2.1.2, SACF will wait for the last associated TCAP operation timer to expire and issue a TC-END request with the release method parameter set to "prearranged release".

Receipt of a TC-BEGIN indication

On receipt of a TC-BEGIN indication primitive, responder SACF shall:

–	Analyse the application-context-name if included in the primitive. If it is supported, process any other indication primitives received from TC as described in 18.1.1.4.1.

–	If no dialogue is to be established according to the rules as stated in 18.1.2.1.1 and 18.1.2.1.2, SACF will wait for the last indication primitive from TC and issue a TC-END request with the release method parameter set to "prearranged release".

–	If the application-context-name included in the primitive is not supported, issue a TC-U-ABORT request primitive. If an alternative application-context can be offered, its name is included in the TC-U-ABORT request primitive.

It is for further study whether or not the application-context-negotiation is limited only for using the TC-U ABORT primitive.

Receipt of the first TC-CONTINUE indication

On receipt of the first TC-CONTINUE indication primitive for a dialogue, SACF shall check the value of the application-context-name parameter. If this value matches the one used in the TC-BEGIN request primitive, SACF shall process the following TC component handling indication primitives as described in 18.1.1.4.1; otherwise, it shall issue a TC-U-ABORT request primitive.

It is for further study whether or not the application-context-negotiation is limited only for using the TC-U ABORT primitive.

Receipt of a TC-END indication

On receipt of a TC-END indication primitive in the dialogue initiated state, SACF shall check the value of the application-context-name parameter. If this value matches the one used in the TC-BEGIN request primitive, then the SACF shall process the following TC component handling indication primitives as described in 18.1.1.4.1.

Receipt of a TC-U-ABORT indication

Receipt of a TC-U-ABORT indication primitive is described as part of user abort procedure (see 18.1.1.3.4). If the abort reason is application context name not supported, the responding side may propose an alternative application context name in the TC-U-ABORT indication. If an alternative

application context is proposed, the receiving entity shall check this name and if it can be supported a new dialogue may be established.

Receipt of a TC-P-ABORT indication

Receipt of a TC-P-ABORT indication primitive is described as part of provider abort procedure (see 18.1.1.3.5).

##### 18.1.1.3.2	Dialogue continuation

Once established, the dialogue is said to be in a continuation phase.

Both application processes can request the transfer of INAP APDUs until one of them requests the termination of the dialogue.

Sending entity

SACF shall process any component handling request primitives as described in 18.1.1.4.1.

After processing the last component handling request primitive, SACF shall issue a TC-CONTINUE request primitive.

Receiving entity

On receipt of a TC-CONTINUE indication primitive SACF shall accept zero, one or several TC component handling indication primitives and process them as described in 18.1.1.4.1.

##### 18.1.1.3.3	Dialogue termination

Both the dialogue-initiator and the dialogue-responder have the ability to request the termination of a dialogue after it has been established when no dialogue is to be established or when a dialogue is no longer to be maintained according to the rules as stated in 18.1.2.1.1 and 18.1.2.1.2.

The dialogue termination procedure is driven by the following events:

–	a TC-END request primitive;

–	a TC-END indication primitive.

Sending of TC-END request

When the dialogue shall no longer be maintained, SACF shall process any component handling request primitives as described in 18.1.1.4.1.

After processing the last component handling request primitive (if any), SACF shall issue a TC-END request primitive with the release method parameter set to "basic end" or "prearranged release", according to the rules as stated in 18.1.2.1.1 and 18.1.2.1.2.

When no dialogue is to be established, refer to 18.1.1.3.1.

Receipt of a TC-END indication

On receipt of a TC-END indication primitive, the SACF shall accept any component handling indication primitives and process them as described in 18.1.1.4.1.

After processing the last component handling primitive, all dialogue related resources are released.

##### 18.1.1.3.4	User Abort

Both the dialogue-initiator and the dialogue-responder have the ability to abort a dialogue at any time.

The user abort procedure is driven by one of the following events:

–	a TC-U-ABORT request primitive;

–	a TC-U-ABORT indication primitive.

Sending of TC-U-ABORT request

After issuing a TC-U-ABORT request primitive, all dialogue related resources are released.

Receipt of a TC-U-ABORT indication

On receipt of a TC-U-ABORT indication, all dialogue related resources are released.

##### 18.1.1.3.5	Provider Abort

TC has the ability to abort a dialogue at both the dialogue-initiator side and the dialogue-responder side.

The provider abort procedure is driven by the following event:

–	a TC-P-ABORT indication primitive.

Receipt of a TC-P-ABORT indication

On receipt of a TC-P-ABORT indication, all dialogue related resources are released.

##### 18.1.1.3.6	Mapping to TC dialogue primitives

The TC-UNI service is not used by INAP.

The mapping of parameters onto the TC Dialogue services is as follows:

The use of parameters of the TC-BEGIN service is as defined in 18.1.1.3.7 with the following qualifications:

–	The Destination Address parameter of the TC-BEGIN service shall be set to the INAP address of the AE which is to respond to the TC-BEGIN service.

NOTE 1 – The address used in this parameter may be mapped by SCCP address translation to one of a number of alternative AEs.

–	The Application Context Name parameter of the TC-BEGIN service shall be set according to the specific interface being used between the initiating AE and the responding AE.

–	The Originating Address parameter of the TC-BEGIN service shall be set to the unambiguous INAP address of the AE initiating the TC-BEGIN service.

The use of parameters of the TC-CONTINUE service is as defined in 18.1.1.3.7 with the following qualifications:

–	The Application Context Name parameter of the TC-CONTINUE service shall be set to the value of the Application Context Name parameter of the TC-BEGIN service for the same Dialogue ID parameter value.

–	If present, the Originating Address parameter of the TC-CONTINUE service shall be set to the unambiguous INAP address of the AE initiating the TC-CONTINUE service. This parameter is only present in the first TC-CONTINUE service after a TC-BEGIN service with the same Dialogue ID parameter value.

The use of parameters of the TC-END service is as defined in 18.1.1.3.7 with the following qualifications:

–	The Application Context Name parameter of the TC-END service shall be set to the value of the Application Context Name parameter of the TC-BEGIN service for the same Dialogue ID parameter value. This parameter is only present if the TC-END service is used immediately after the TC-BEGIN service.

The use of parameters of the TC-U-ABORT service is as defined in 18.1.1.3.7 with the following qualifications:

–	The Abort Reason parameter of the TC-U-ABORT service shall be used as specified in Recommendation Q.771.

–	The Application Context Name parameter of the TC-U-ABORT service shall be set to either the value used in the TC-BEGIN service or an alternative value which can be used to establish the dialogue between the initiating AE and the responding AE.

NOTE 2 – This parameter is only present if the TC-U-ABORT is the immediate response to a TC-BEGIN indication.

The use of parameters of the TC-P-ABORT service is as defined in 18.1.1.3.7 with the following qualifications:

–	The P-Abort parameter of the TC-P-ABORT service is set by TC to indicate the reason why TC aborted the dialogue. It shall take the values as defined in Recomendation Q.771.

##### 18.1.1.3.7	Default mapping to TC dialogue parameters

Dialogue Id

The value of this parameter is associated with the INAP invocation in an implementation-dependent manner. This parameter uniquely identifies a specific TC dialogue to a remote INAP AE for an INAP AE.

Application-context-name

The application-context-name parameter is set according to the set of operations which need to be supported by the TC dialogue. The defined Application Context Names can be found in clauses 5 to 11.

User information

This parameter may be used by both initiating and responding application processes in a network-operator specific manner.

Component present

This parameter is used by SACF as described in Recommendation Q.771.

Termination

The value of the release method parameter of the TC-END request primitive is set by SACF according to the rules as stated in 18.1.2.1.1 and 18.1.2.1.2.

Quality of service

The quality of service of TC request primitives is set by the SACF to the following value:

–	sequencing requested;

–	return option, this parameter is set by SACF in an implementation-dependent manner.

#### 18.1.1.4	Component handling

##### 18.1.1.4.1	Procedures for INAP operations

This subclause describes the procedures for INAP operations.

Operation invocation

SACF shall build an operation argument from the parameters received and request the invocation of the associated operation using the TC-INVOKE procedure. If a linked ID parameter is inserted in the primitive, this indicates a child operation and implies that the operation is linked to a parent operation.

Operation invocation receipt

On receipt of a TC-INVOKE indication primitive, SACF shall:

–	if the operation code does not correspond to an operation supported by the application-context, request the transfer of a reject component using the TC-U-REJECT request primitive, with the appropriate problem code (unrecognised operation);

–	if a linked ID is included, perform the following checks: If the operation referred to by the linked ID does not allow linked operations or if the operation code does not correspond to a permitted linked operation, or if the parent operation invocation is not active, issue a TC-U-REJECT request primitive with the appropriate problem code (linked response unexpected or unexpected linked operation);

–	if the type of the argument is not the one defined for the operation, request the transfer of a reject component using the TC-U-REJECT request primitive, with the appropriate problem code (mistyped parameter);

–	if the operation cannot be invoked because the INAP related dialogue is about to be released, request the transfer of the reject component using the TC-U-REJECT request primitive with the problem code (Initiating Release);

–	if sufficient INAP related resources are not available to perform the requested operation, request the transfer of a reject component using the TC-U-REJECT request primitive with the problem code (Resource Limitation);

–	otherwise, accept the TC-INVOKE indication primitive. If the operation is to be user-confirmed, SACF waits for the corresponding response.

Operation Response

For user confirmed operations, SACF shall:

–	if no error indication is included in the response to a class 1 or 3 operation, construct a result information element from the parameters received and request its transfer using the TC-RESULT-L service;

–	if an error indication is included in the response to a class 1 or 2 operation, construct an error parameter from the parameters received and request its transfer using the TC-U-ERROR request primitive.

Receipt of a response

On receipt of a TC-RESULT-NL indication, SACF shall:

–	request the transfer of a reject component using the TC-U-REJECT request primitive, with the appropriate problem code (mistyped parameter).

On receipt of a TC-RESULT-L indication, SACF shall:

–	if the type of the result parameter is not the one defined for the result of this operation, request the transfer of a reject component using the TC-U-REJECT request primitive, with the appropriate problem code (mistyped parameter);

–	otherwise, accept the TC-RESULT-L indication primitive.

On receipt of a TC-U-ERROR indication, SACF shall:

–	if the error code is not defined for the SACF or is not one associated with the operation referred to by the invoke identifier, request the transfer of a reject component using the TC-U-REJECT request primitive, with the appropriate problem code (unrecognised error or unexpected error);

–	if the type of the error parameter is not the one defined for this error, request the transfer of a reject component using the TC-U-REJECT request primitive, with the appropriate problem code (mistyped parameter);

–	otherwise, accept the TC-U-ERROR indication primitive.

On receipt of a TC-U-REJECT indication primitive which affects a pending operation, SACF shall:

–	accept the TC-U-REJECT indication primitive.

On receipt of a TC-L-REJECT indicating "return result problem, return error unexpected", SACF shall inform the application process.

On receipt of a TC-L-REJECT indicating "return error problem, return error unexpected", SACF shall inform the application process.

This event occurs when the local TC detects a protocol error in an incoming component which affects an operation.

When the problem code indicates a general problem, it is considered that the event cannot be related to an active operation even if the invoke Id is provided by TC. This is because it is unclear whether the invoke Id refers to a local or remote invocation. The behaviour of SACF in such a case is described below under the heading "Other events".

On receipt of a TC-L-CANCEL indication, the SACF shall:

–	if the associated operation is a class 1 operation, inform the application process;

–	if the associated operation is a class 2 operation and no linked operations are defined for this operation, ignore the primitive;

–	if the associated operation is a class 2 operation and has linked operations but none of them has been invoked, inform the application process;

–	if the associated operation is a class 2 operation and a linked operation invocation has already been received in response to this operation, ignore the primitive;

–	if the associated operation is a class 3 operation, inform the application process;

–	if the associated operation is a class 4 operation, ignore the primitive.

Other events

This subclause describes the behaviour of SACF on receipt of a component handling indication primitive which cannot be related to any operation or which does not affect a pending one.

On receipt of a TC-U-REJECT indication primitive which does not affect an active operation (i.e. indicating a return result or return error problem), it is up to the application process to abort, continue or terminate the dialogue, if not already terminated by the sending application process according to the rules as stated in 18.1.2.1.2. This is also applicable for invoke problems related to a class 4 linked operation.

On receipt of a TC-R-REJECT indication (i.e. when a protocol error has been detected by the peer TC entity) which does not affect an active operation, it is up to the application process to abort, continue or terminate the dialogue, if not already terminated by the sending application process according to the rules as stated in 18.1.2.1.2.

On receipt of a TC-L-REJECT indication primitive (i.e. when a protocol error has been detected by the local TC entity) which cannot be related to an active operation, it is up to the application process to continue, or to terminate the dialogue and implicitly trigger the transmission of the reject component or to abort the dialogue.

On receipt of a TC-NOTICE indication primitive, which informs the SACF that a message cannot be delivered by the Network Layer, it is for the application process to decide whether to terminate the dialogue or retry.

This primitive can only occur if the Return Option has been set (see 18.1.1.3.6).

##### 18.1.1.4.2	Mapping to TC component primitives

The mapping of parameters onto the TC Component services is as follows:

The TC-U-CANCEL service is not used.

The TC-RESULT-NL service is not used.

The use of parameters of the TC-INVOKE service is as defined in 18.1.1.4.3 with the following qualifications:

–	The Operation parameter of the TC-INVOKE service shall contain the operation.&operationCode value of the INAP operation to be invoked. The operation must be one of the valid operations supported by the negotiated Application Context for the TC dialogue and must be invokable by the local AE.

–	The Parameters parameter of the TC-INVOKE service shall contain a value of the operation.&ArgumentType value for the operation being invoked, as specified by the Operation parameter.

The use of parameters of the TC-RESULT-L service is as defined in 18.1.1.4.3 with the following qualifications:

–	The Invoke Id parameter of the TC-RESULT-L service shall be set to the value of the Invoke Id parameter of the TC-INVOKE service from the remote AE to which a result is being sent.

–	The Operation parameter of the TC-RESULT-L service shall be set to the value of the Operation parameter of the TC-INVOKE service from the remote AE which contains the same Invoke Id Parameter value.

–	The Parameters parameter of the TC-RESULT-L service shall contain the operation.&ResultType value for the operation result, as specified by the Operation parameter.

The use of parameters of the TC-U-ERROR service is as defined in 18.1.1.4.3 with the following qualifications:

–	The Invoke Id parameter of the TC-U-ERROR service shall be set to the value of the Invoke Id parameter of the TC-INVOKE service from the remote AE to which an error is being sent.

–	The Error parameter of the TC-U-ERROR service shall be set to the value of the error.&errorCode of the error to be sent. It must be one of the errors which is expected for the invoked operation as defined in the operation.&Errors specification.

–	The Parameters parameter of the TC-U-ERROR service shall be set to the value of the error.&ParameterType of the error to be sent, as identified by the Error parameter.

The use of parameters of the TC-U-REJECT service is as defined in 18.1.1.4.3 with the following qualifications:

–	The Invoke Id parameter of the TC-U-REJECT service shall be set to the Invoke Id Parameter of the TC component service from the remote AE which is being rejected.

The use of parameters of the TC-L-CANCEL service is as defined in 18.1.1.4.3.

##### 18.1.1.4.3	Default mapping to TC component parameters

Invoke Id

This parameter is set by the sending application process. It represents the unique identity of an instance of an operation which is invoked by an AE within a specific TC dialogue. The TC dialogue is identified by the Dialogue Id parameter.

Linked Id

This parameter is set by the sending application process. It represents the Invoke Id of an operation which was received from the remote AE for a specific TC dialogue to which the operation being invoked by the local AE is to be linked. This parameter is only present if the original operation invoked by the remote AE is defined as having linked operations. The type of local operation invoked must be the same type as one of the operations defined as being linked.

Dialogue Id

The value of this parameter is associated with the INAP invocation in an implementation-dependent manner. It represents the identity of the established TC dialogue which will carry the component services between the local AE and the remote AE.

Class

The value of this parameter is set according to the type of the operation to be invoked according to the operation definitions in clauses 5 through 10.

Time-out

The value of this parameter is set according to the type of operation invoked.

Last component

This parameter is used as described in Recommendation Q.771.

Problem code

This parameter is used as described in 18.1.1.4.1.

Abort reason

This parameter is used by SACF, and attributes and coding are specified by network operator.

### 18.1.2	SSF-SCF interface

#### 18.1.2.1	Normal procedures

##### 18.1.2.1.1	SSF-to-SCF messages

This subclause defines the normal procedures for TC messages from the SSF to the SCF.

SSF-FSM related messages

A dialogue shall be established when the SSF-FSM moves from the state Idle to the state Waiting for Instructions. The relevant INAP operation, which can be the InitialDP operation or one of DP-specific operations for TDP-R, shall be transmitted in the same message.

No dialogue shall be established when the SSF-FSM moves from the state Idle and back to the state Idle on the detection of TDP-N. The relevant INAP operation, which can be the InitialDP operation or one of DP-specific operations for TDP-N, shall be sent with a TC-BEGIN request primitive and the dialogue is locally ended by means of TC-END request primitive with prearranged end.

For all other operations sent from the SSF-FSM, the dialogue shall be maintained except for the following cases.

When the SSF-FSM makes a non-error case state transition to the state Idle and there is one or more pending operation and TCAP dialogue is established, TCAP dialogue can be terminated by TC-END primitive with component(s). When the SSF sends the last EventReportBCSM, ApplyChargingReport or CallInformationReport the dialogue may be ended from the SSF by a TC-END request primitive with basic end.

In the case that there is no pending operation and TCAP dialogue is established, TCAP dialogue can be terminated by TC-END primitive with zero component or prearranged end. When the SSF-FSM makes a non-error case state transition to the state Idle and there is no operation to be sent, the dialogue is ended by means of a TC-END request primitive (basic) with zero components, or the dialogue is locally ended by means of a TC-END request primitive with prearranged end.

The SSF can end a dialogue with a TC-END request primitive with zero component or prearranged end depending on that TCAP dialogue is established or not, in the case call release is initiated by any other entity then the SCF and the SSF has no pending call information requests (or pending requests which should be treated in the same way, see Note 1, in 11.5.2) nor any armed EDP to notify the SCF of the call release (for alternative way, see 18.1.2.2).

When the SSF has sent the last EventReportBCSM, ApplyChargingReport or CallInformationReport the dialogue may be ended from the SCF by a TC-END request primitive with basic end.

Assisting/Hand-off SSF-FSM related messages

A dialogue shall be established when the Assisting/Hand-off SSF-FSM moves from the state Idle to the state Waiting For Instructions. The AssistRequestInstructions operation shall be transmitted with a TC-BEGIN request primitive.

For all other operations sent from the Assisting/Hand-off SSF-FSM, the dialogue shall be maintained except for the following cases.

When the SSF-FSM makes a non-error case state transition to the state Idle and there is one or more pending operation and TCAP dialogue is established, TCAP dialogue can be terminated by TC-END primitive with component(s). When the SSF sends the last ApplyChargingReport, the dialogue may be ended from the SSF by a TC-END request primitive with basic end.

In the case that there is no pending operation and TCAP dialogue is established, TCAP dialogue can be terminated by TC-END primitive with zero component or prearranged end. When the SSF-FSM makes a non-error case state transition to the state Idle and there is no operation to be sent, the dialogue is ended by means of a TC-END request primitive (basic) with zero components, or the dialogue is locally ended by means of a TC-END request primitive with prearranged end.

When the SSF has sent the last ApplyChargingReport, the dialogue may be ended from the SCF by a TC-END request primitive with basic end.

SSME-FSM related messages

The following procedures shall be followed:

–	The dialogue shall be maintained when the ActivityTest Return Result is sent.

–	No dialogue shall be established when the ServiceFilteringResponse operation is sent. The operation is sent with a TC-BEGIN request primitive and the dialogue is ended by means of a TC-END request primitive with prearranged end.

–	A dialogue shall no longer be maintained when the Return Result of the ActivateServiceFiltering operation is sent. The dialogue is ended by means of a TC-END request primitive with basic end, the Return Result is transmitted with the same request.

–	The dialogue is locally terminated by means of a TC-END request primitive with prearranged end, upon reception of a TC-BEGIN indication primitive with a CallGap operation.

–	The dialogue shall be maintained when the RequestCurrentStatusReport, RequestEveryStatusChangeReport or RequestFirstStatusMatchReport operation is received inside the call context.

–	The dialogue shall be maintained on sending the following operations inside the call context if the operations are not the final one:

•	Return Result of the RequestCurrentStatusReport operation;

•	Return Result of the RequestFirstStatusMatchReport or RequestEveryStatusChangeReport operation; and

•	StatusReport operation in reply to RequestFirstStatusMatchReport or RequestEveryStatusChangeReport operation.

–	The dialogue shall no longer be maintained on sending the following operations inside the call context if the operations are the final one:

•	Return Result of the RequestCurrentStatusReport operation;

•	Return Result of the RequestFirstStatusMatchReport or RequestEveryStatusChangeReport operation; and

•	StatusReport operation in reply to RequestFirstStatusMatchReport or RequestEveryStatusChangeReport operation.

The dialogue is ended from the SSF by means of TC-END request primitive with basic end, one of the above operations is transmitted with the same request. If monitor duration expires for RequestFirstStatusMatchReport or RequestEveryStatusChangeReport which has been received inside the call context and there is no need to maintain dialogue, the dialogue is ended from the SSF by means of a TC-END request primitive (basic) with zero component.

–	The dialogue shall be established when the RequestCurrentStatusReport, RequestEveryStatusChangeReport or RequestFirstStatusMatchReport operation is received outside the call context.

–	The dialogue shall no longer be maintained when the Return Result of the RequestCurrentStatusReport operation is sent outside the call context. The dialogue is ended from the SSF by means of a TC-END request primitive with basic end, the Return Result is transmitted with the same request.

–	The dialogue shall be maintained when the Return Result of the RequestFirstStatusMatchReport or RequestEveryStatusChangeReport operation is sent outside the call context.

–	The dialogue shall no longer be maintained when the StatusReport operation is sent in reply to the RequestFirstStatusMatchReport operation outside the call context. The dialogue is ended from the SSF by means of a TC-END request primitive with basic end, the StatusReport operation is transmitted with the same request.

–	The dialogue shall be maintained when the StatusReport operation is sent in reply to the RequestEveryStatusChangeReport operation outside the call context.

–	The dialogue shall no longer be maintained when the monitor duration expires for RequestFirstStatusMatchReport or RequestEveryStatusChangeReport which has been received outside the call context. The dialogue is ended from the SSF by means of a TC-END request primitive (basic) with zero component.

##### 18.1.2.1.2	SCF-to-SSF messages

This subclause defines the normal procedures for TC messages from the SCF to the SSF.

SCSM-FSM related messages

No dialogue shall be established when the SCSM-FSM moves from state Idle to state Idle upon receipt of InitialDP operation or one of DP-specific operations for TDP-N. The operation is received with a TC-BEGIN indication primitive and the dialogue is locally terminated by means of a TC-END request primitive with prearranged end.

A dialogue shall be established when the SCSM-FSM moves from state Idle to state Preparing SSF Instructions upon the receipt of InitialDP operation for TDP-R, one of DP-specific operations for TDP-R or AssistRequestInstructions operation.

A dialogue shall be established when the SCSM-FSM sends an InitiateCallAttempt or a CreateCSA from the Idle state.

For subsequent operations sent from the SCSM-FSM, the dialogue shall be maintained except for the following cases, i.e. all other operations are sent after a dialogue was established from the SSF (the SCF has previously received a TC-BEGIN indication primitive with an InitialDP operation, one of DP-specific operations or an AssistRequestInstructions operation).

The dialogue shall no longer be maintained when the prearranged end condition is met in the SCF. When the SCF does not expect any messages other than possibly REJECT or ERROR messages for the operations sent and when the last associated operation timer expires, the dialogue is locally ended by means of a TC-END request primitive with prearranged end.

Alternatively, the sending of operations, leading to the termination of the relationship, by means of a TC-END request primitive (basic end) is possible.

SCME-FSM related messages

The operations sent from the SCME-FSM shall be issued according to the following procedures:

–	The dialogue shall be maintained when the ActivityTest operation is sent.

–	A dialogue shall not be established when a CallGap operation is sent without using a SCSM associated dialogue. The operation is sent using a TC-BEGIN request primitive and the dialogue is terminated with a prearranged end.

–	For sending one or more CallGap operations, the SCME FSM may use an existing SCSM FSM associated dialogue which was initiated by a SSF-FSM (i.e. established for the transmission of the InitialDP operation or one of DP-specific operations). The dialogue shall be maintained and the CallGap operation(s) shall be sent with the first response of the SCSM FSM to the InitialDP operation or one of DP-specific operations.

–	A dialogue shall be established when an ActivateServiceFiltering operation is sent. The operation shall be transmitted with a TC-BEGIN request primitive.

–	The dialogue is locally terminated upon reception of a ServiceFilteringResponse operation using a TC-END request primitive with prearranged end.

–	The dialogue shall be maintained when the RequestCurrentStatusReport, RequestEveryStatusChangeReport or RequestFirstStatusMatchReport operation is sent inside the call context.

–	The dialogue shall be maintained on receiving the following operations inside the call context if the operations are not final one:

•	Return Result of the RequestCurrentStatusReport operation;

•	Return Result of the RequestFirstStatusMatchReport or RequestEveryStatusChangeReport operation; and

•	StatusReport operation in reply to RequestFirstStatusMatchReport or RequestEveryStatusChangeReport operation.

–	The dialogue shall no longer be maintained on receiving the following operations inside the call context if the operations are final one:

•	Return Result of the RequestCurrentStatusReport operation;

•	RequestEveryStatusChangeReport operation; and

•	StatusReport operation in reply to RequestFirstStatusMatchReport or RequestEveryStatusChangeReport operation.

The dialogue is ended from the SSF by means of TC-END indication primitive with basic end, one of above operations is transmitted with the same request. If monitor duration expires for RequestFirstStatusMatchReport or RequestEveryStatusChangeReport which has been received inside the call context and there is no need to maintain dialogue, the dialogue is ended from the SSF by means of a TC-END indication primitive (basic) with zero component.

–	The dialogue shall be established when the RequestCurrentStatusReport, RequestEveryStatusChangeReport or RequestFirstStatusMatchReport operation is sent outside the call context.

–	The dialogue shall no longer be maintained when the Return Result of the RequestCurrentStatusReport operation is received outside the call context. The dialogue is ended from the SSF by means of a TC-END indication primitive with basic end, the Return Result is transmitted with the same indication.

–	The dialogue shall be maintained when the Return Result of the RequestFirstStatusMatchReport or RequestEveryStatusChangeReport operation is received outside the call context.

–	The dialogue shall no longer be maintained when the StatusReport operation is received in reply to the RequestFirstStatusMatchReport operation outside the call context. The dialogue is ended from the SSF by means of a TC-END indication primitive with basic end, the StatusReport operation is received with the same indication.

–	The dialogue shall be maintained when the StatusReport operation is received in reply to the RequestEveryStatusChangeReport operation outside the call context.

–	The dialogue shall no longer be maintained when the monitor duration expires for RequestFirstStatusMatchReport or RequestEveryStatusChangeReport outside the call context. The dialogue is ended from the SSF by means of a TC-END indication primitive (basic) with zero component.

SCF-SSF – Use of dialogue handling services

Dialogue handling services are used to trigger the sending of the APDUs associated with the operations involved in the INAP packages.

Component grouping is performed under the control of the application-process through an appropriate usage of the TC-BEGIN and TC-CONTINUE service.

The TC-END service is solely used to support the dialogue closing procedure (i.e. it is never used to trigger the sending of components).

On receipt of an empty TC-CONTINUE.req primitive, the FE should ignore the primitive.

On receipt of a TC-END.req with an INAP request, the FE should not perform the request and consider the requested TC-END service as a dialogue closing procedure. The dialogue is then terminated.

It is an application-process responsibility to provide in the TC-BEGIN.req primitive a destination address which can be used by the underlying SCCP to route the message to the proper FE if this FE is addressed through the SS7 network.

The prearranged end can be used.

#### 18.1.2.2	Abnormal procedures

The following procedures also apply to the SCF-SRF and SCF-CUSF interfaces.

SCF-to-SSF/SRF/CUSF messages

Considering that SSF, SRF, and CUSF do not have the logic to recover from error cases detected on the SCF-SSF/SRF/CUSF interface, the following shall apply:

–	Operation errors and rejection of TCAP components shall be transmitted to the SSF and, respectively, the SRF, and the CUSF with a TC-END request primitive, basic end.

If, in violation of the above procedure, an ERROR or REJECT component is received with a TC-CONTINUE indication primitive, the SSF and, respectively, the SRF and the CUSF shall abort the dialogue with a TC-U-ABORT request primitive.

In the case of the SSF relay, it is for further study how to map messages to ROSE capability of bearer signalling system between the SSF and the SRF, and what services are assumed from ROSE.

SSF/SRF/CUSF-to-SCF messages

Operation errors and rejection of TCAP components shall be transmitted to the SCF according to the following rules:

–	The dialogue shall be maintained when the preceding message, which contained the erroneous component, indicated that the dialogue shall be maintained, i.e. the error or reject shall be transmitted with a TC-CONTINUE request primitive if the erroneous component was received with a TC-CONTINUE indication primitive.

On receipt of an ERROR or REJECT component, the SCF decides on further processing. It may either continue, explicitly end or abort the dialogue.

–	In all other situations the dialogue shall no longer be maintained, i.e. the error or reject shall be transmitted with a TC-END request primitive, basic end, if the erroneous component was received with a TC-BEGIN indication primitive.

–	on expiration of application timer TSSF or TSRF or TCUSF, dialogue shall be terminated by means of by TC-U-ABORT primitive with an Abort reason, regardless of whether TCAP dialogue is established or not.

If the error processing in the SSF/SRF/CUSF leads to the case where the SSF/SRF/CUSF is not able to process further SCF operations while the dialogue is to be maintained, the SSF/SRF/CUSF aborts the dialogue with a TC-END request primitive with basic end or a TC-U-ABORT request primitive, depending on whether any pending ERROR or REJECT component is to be sent or not.

The SSF can end a dialogue with a TC-U-ABORT request primitive in case call release is initiated by any other entity then the SCF and the SSF has no pending call information requests (or pending requests which should be treated in the same way, i.e. ApplyCharging nor any armed EDP to notify the SCF of the call release (for alternative way, see 18.1.2.1.1).

In the case of the SSF relay, it is for further study how to map messages to ROSE capability of bearer signalling system between the SSF and the SRF, and what services are assumed from ROSE.

The CUSF can end a dialogue with a TC-U-ABORT request primitive in case the association release between the user and the network is initiated by any other entity.

SCF-SSF – Use of dialogue handling services

On receipt of a TC-U-REJECT.ind in the FE, this primitive should be ignored. It is up to the application process to abort, continue or terminate the dialogue, if not already terminated by the sending application process according to the rules as stated in 18.1.2.2. This is also applicable for invoke problems related to a class 4 linked operation.

A TC-U-REJECT.req should be sent followed by a TC-CONTINUE.req.

On receipt of a TC-R-REJECT.ind in the FE, this primitive should be ignored. It is up to the application process to abort, continue or terminate the dialogue, if not already terminated by the sending application process according to the rules as stated in 18.1.2.2. This is also applicable for invoke problems related to a class 4 linked operation. The dialogue should be released with a TC-U-ABORT.req.

On receipt of a TC-L-REJECT indication primitive (i.e. when a protocol error has been detected by the local TC entity) which cannot be related to an active operation, it is up to the application process to continue or to terminate the dialogue and implicitly trigger the transmission of the reject component or to abort the dialogue.

On receipt of a TC-NOTICE indication the SACF is informed that a message cannot be delivered by the Network Layer. It occurs if the Return Option has been set (see 18.1.2.2). It is for the application process to decide whether to terminate the dialogue or retry.

The application-process is the sole user of the TC-P-ABORT service and TC-NOTICE service.

The receipt of a TC-U-ABORT.ind or TC-P-ABORT.ind on a dialogue terminates all request processing.

#### 18.1.2.3	Dialogue handling

##### 18.1.2.3.1	Dialogue establishment

##### 18.1.2.3.2	Dialogue continuation

##### 18.1.2.3.3	Dialogue termination

##### 18.1.2.3.4	User Abort

##### 18.1.2.3.5	Provider Abort

##### 18.1.2.3.6	Mapping to TC dialogue primitives

The SSF-SCF IN services can be mapped onto TC services. This subclause defines the mapping of the SSF-SCF IN services onto the services of the TC dialogue handling services defined in Recommendation Q.771.

a)	The TC-BEGIN service is used to invoke the operations of the scf-ssfConnectionPackage and ssf-scfConnectionPackage.

b)	The TC-CONTINUE service is used to report the success of the operations invoked in a TC-BEGIN service and to invoke or respond to any other operations.

c)	The TC-U-ABORT service is used to report the failure of operations of the scf-ssfConnectionPackage and ssf-scfConnectionPackage.

The mapping of the parameters onto the TC-BEGIN primitive is defined in 18.1.1.3.6 with the following qualifications:

–	The Application Context Name parameter shall take the value of the application-context-name field of the cs2ssf-scfGenericAC, cs2ssf-scfDPSpecificAC, cs2ssf-scfAssistHandoffAC or cs2ssf-scfServiceManagementAC object if the initiating AE is a SSF or the cs2scf-ssfGenericAC, cs2scf-ssfDPSpecificAC, cs2scf-ssfTrafficManagementAC, cs2scf-ssfServiceManagementAC or cs2scf-ssfStatusReportingAC object if the originating AE is a SCF.

The mapping of the parameters onto the TC-CONTINUE primitive is defined in 18.1.1.3.6.

The mapping of the parameters onto the TC-U-ABORT primitive is defined in 18.1.1.3.6 with the following qualifications:

–	The Application-Context-Name parameter shall be used as specified in Recommendation Q.771. When the responding AE refuses a dialogue because the application-context-name it receives is not supported, this parameter shall have the value of the application-context-name field of the cs2ssf-scfGenericAC, cs2ssf-scfDPSpecificAC, cs2ssf-scfAssistHandoffAC, cs2ssf-scfServiceManagementAC or cs2scf-ssfGenericAC object if the responding AE is a SCF or the cs2scf-ssfDPSpecificAC, cs2scf-ssfTrafficManagementAC, cs2scf-ssfServiceManagementAC or cs2scf-ssfStatusReportingAC object if the responding AE is a SSF.

The use of the parameters of the TC-END service is defined in 18.1.1.3.6.

#### 18.1.2.4	Component handling

##### 18.1.2.4.1	Procedures for INAP operations

The INAP ASEs are users of the TC component handling services except for the TC-L-REJECT and TC-L-CANCEL services which are used by the application-process. Receipt of a TC-L-REJECT.ind leads the application-process to abandon the dialogue (i.e. it issues a TC-U-ABORT.request primitive).

The TC-U-CANCEL service is never used.

##### 18.1.2.4.2	Mapping to TC component parameters

The SSF-SCF IN ASE services are mapped onto the TC component handling services. The mapping of operations and errors onto TC services is defined in 18.1.1.4.2 with the following qualification:

The time-out parameter of the TC-INVOKE.req primitives is set according to Table 5-1.

### 18.1.3	SCF-SRF interface

#### 18.1.3.1	Normal procedures

##### SCF-to/from-SRF messages

A dialogue is established when the SRF sends an AssistRequestInstructions operation to the SCF. For all other operations sent to/from the SRF, the dialogue shall be maintained.

In the case that there is no pending operation and TCAP dialogue is established, TCAP dialogue can be terminated by TC-END primitive with zero component. When the SCSM makes a non-error case state transition to end-user interaction and there is no operation to be sent, the dialogue is ended by means of a TC-END request primitive (basic) with zero components.

The dialogue shall no longer be maintained when sending the SpecializedResourceReport operation for PlayAnnouncement with disconnection from the SRF set to TRUE or Return Result of the PromptAndCollectUserInformation with disconnection from the SRF set to TRUE. The dialogue is ended by means of a TC-END request primitive with basic end, and one of the above operations is transmitted with the same request.

Regardless of whether pending operation exists or not, when the SRSM-FSM is informed of the disconnection of bearer connection (in the case of SCF initiated disconnection or call abandon from call party) and dialogue is established, the dialogue is ended by means of a TC-END request primitive (basic) with zero components or TC-END request primitive (prearranged end).

The dialogue shall no longer be maintained when the prearranged end condition is met in the SRF. When the SRSM-FSM is informed the disconnection of bearer connection and TCAP dialogue is not established, TCAP dialogue is locally terminated by TC-END primitive with prearranged end.

When the SCF does not expect any messages other than possibly REJECT or ERROR messages for the operations sent and when the last associated operation timer expires, the dialogue is locally ended by means of a TC-END request primitive with prearranged end. Alternatively, the sending of operations, leading to the termination of the relationship, by means of a TC-END request primitive (basic end) is possible.

In the relay case, the SRF-SCF relationship uses the SSF-SCF TCAP dialogue. This is possible, because begin and end of the SRF-SCF relationship are embedded in the SSF-SCF relationship. SRF-SCF information shall be exchanged with TC-CONTINUE request primitives.

In the case of the SSF relay, it is for further study how to map messages to ROSE capability of bearer signalling system between the SSF and the SRF, and what services are assumed from ROSE.

#### 18.1.3.2	Abnormal procedures

#### 18.1.3.3	Dialogue handling

##### 18.1.3.3.1	Dialogue establishment

##### 18.1.3.3.2	Dialogue continuation

##### 18.1.3.3.3	Dialogue termination

##### 18.1.3.3.4	User Abort

##### 18.1.3.3.5	Provider Abort

##### 18.1.3.3.6	Mapping to TC dialogue primitives

The SCF-SRF IN services can be mapped onto TC services. This subclause defines the mapping of the SCF-SRF IN services onto the services of the TC dialogue handling services defined in Recommendation Q.771.

a)	The TC-BEGIN service is used to invoke the operations of the srf-scfConnectionPackage.

b)	The TC-CONTINUE service is used to report the success of the operations invoked in a TC-BEGIN service and to invoke or respond to any other operations.

c)	The TC-U-ABORT service is used to report the failure of operation of the scf-srfOperationPackage and srf-scfOperationPackage.

The mapping of parameters onto the TC Dialogue services is as defined in 18.1.1.3.6.

The mapping of the parameters onto the TC-BEGIN primitive is defined in 18.1.1.3.6 with the following qualification:

–	The Application Context Name parameter shall take the value of the application-context-name field of the srf-scf-ac object.

#### 18.1.3.4	Component handling

##### 18.1.3.4.1	Procedures for INAP operations

##### 18.1.3.4.2	Mapping to TC component parameters

The mapping of parameters for the TC component services is defined in 18.1.1.4.2 with the following qualification:

The Time-out Parameter of the TC-INVOKE service is set according to Table 6-1.

### 18.1.4	SCF-CUSF interface

#### 18.1.4.1	Normal procedures

18.1.4.1.1	CUSF-to-SCF messages

CUSF-FSM related messages

A dialogue shall be established when the CUSF-FSM moves from the state Idle to the state Waiting for Instructions. The relevant INAP operation, which is one of operations for TDP-R, shall be transmitted in the same message.

No dialogue shall be established when the CUSF-FSM moves from the state Idle and back to the state Idle on the detection of TDP-N. The relevant INAP operation, which is one of operations for TDP-N, shall be sent with a TC-BEGIN request primitive and the dialogue is locally ended by means of TC-END request primitive with prearranged end.

For all other operations sent from the CUSF-FSM, the dialogue shall be maintained except for the following cases.

When the CUSF sends the last event report operation (currently only ComponentReceived is applicable), the dialogue may be ended from the CUSF by a TC-END request primitive with basic end.

In the case that there is no pending operation and TCAP dialogue is established, TCAP dialogue can be terminated by TC-END primitive with zero component or prearranged end. When the CUSF-FSM makes a non-error case state transition to the state Idle and there is no operation to be sent, the dialogue is ended by means of a TC-END request primitive (basic) with zero components, or the dialogue is locally ended by means of a TC-END request primitive with prearranged end. The CUSF can end a dialogue with a TC-END request primitive with zero component or prearranged end depending on whether TCAP dialogue is established or not, in the case association release between the user and the network is initiated by any other entity.

When the CUSF has sent the last event report operation (currently only ComponentReceived is applicable), the dialogue may be ended from the SCF by a TC-END request primitive with basic end.

CUSME-FSM related messages

The dialogue shall be maintained when the ActivityTest Return Result is sent.

##### 18.1.4.1.2	SCF-to-CUSF messages

SCSM-FSM related messages

No dialogue shall be established when the SCSM-FSM moves from state Idle to state Idle upon receipt of one of DP-specific operations for TDP-N. The operation is received with a TC-BEGIN indication primitive and the dialogue is locally terminated by means of a TC-END request primitive with prearranged end.

A dialogue shall be established when the SCSM-FSM moves from state Idle to state Preparing CUSF Instructions upon the receipt of one of operations for TDP-R.

For subsequent operations sent from the SCSM-FSM, the dialogue shall be maintained except for the following cases, i.e. all other operations are sent after a dialogue was established from the CUSF (the SCF has previously received a TC-BEGIN indication primitive with one of the operations for TDP-R).

The dialogue shall no longer be maintained when the prearranged end condition is met in the SCF. When the SCF does not expect any messages other than possibly REJECT or ERROR messages for the operations sent and when the last associated operation timer expires, the dialogue is locally ended by means of a TC-END request primitive with prearranged end.

Alternatively, the sending of operations, leading to the termination of the relationship, by means of a TC-END request primitive (basic end) is possible.

SCME-FSM related messages

The operation(s) sent from the SCME-FSM shall be issued according to the following procedure(s):

–	The dialogue shall be maintained when the ActivityTest operation is sent.

#### 18.1.4.2	Abnormal procedures

#### 18.1.4.3	Dialogue handling

##### 18.1.4.3.1	Dialogue establishment

##### 18.1.4.3.2	Dialogue continuation

##### 18.1.4.3.3	Dialogue termination

##### 18.1.4.3.4	User Abort

##### 18.1.4.3.5	Provider Abort

##### 18.1.4.3.6	Mapping to TC dialogue primitives

The CUSF-SCF IN services can be mapped onto TC services. This subclause defines the mapping of the CUSF-SCF IN services onto the services of the TC dialogue handling services defined in Recommendation Q.771.

a)	The TC-BEGIN service is used to invoke the operations of the cusf-scfConnectionPackage.

b)	The TC-CONTINUE service is used to report the success of the operations invoked in a TC-BEGIN service and to invoke or respond to any other operations.

c)	The TC-U-ABORT service is used to report the failure of operation of the cusf-scfOperationPackage and scf-cusfOperationPackage.

The mapping of parameters onto the TC Dialogue services is as defined in 18.1.1.3.6.

The mapping of the parameters onto the TC-BEGIN primitive is defined in 18.1.1.3.6 with the following qualification:

–	The Application Context Name parameter shall take the value of the application-context-name field of the cusf-scf-ac object if the originating AE is a CUSF or the scf-cusf-ac object if the originating AE is a SCF.

#### 18.1.4.4	Component handling

##### 18.1.4.4.1	Procedures for INAP operations

##### 18.1.4.4.2	Mapping to TC component parameters

The mapping of parameters for the TC component services is defined in 18.1.1.4.2 with the following qualification:

The Time-out Parameter of the TC-INVOKE service is set according to Table 10-1.

### 18.1.5	SCF-SCF interface

#### 18.1.5.1	Normal procedures

Dialogue handling services are used to trigger the sending of the APDUs associated with the operations involved in the INAP packages.

Component grouping is performed under the control of the application-process through an appropriate usage of the TC-BEGIN and TC-CONTINUE service.

The TC-END service is solely used to support the dialogue closing procedure (i.e. it is never used to trigger the sending of components).

On receipt of an empty TC-CONTINUE.ind primitive, the application process should ignore the primitive.

The prearranged end can be used.

It is an application-process responsibility to provide in the TC-BEGIN.req primitive a destination address which can be used by the underlying SCCP to route the message to the proper FE if this FE is addressed through the SS7 network.

#### 18.1.5.2	Abnormal procedures

On receipt of a TC-U-REJECT.ind in the FE, this primitive should be ignored. It is up to the application process to abort, continue or terminate the dialogue, if not already terminated by the sending application process according to the rules as stated in 18.1.1.4.1. This is also applicable for invoke problems related to a class 4 linked operation.

On receipt of a TC-L-REJECT indication primitive (i.e. when a protocol error has been detected by the local TC entity) which cannot be related to an active operation, it is up to the application process to continue or to terminate the dialogue and implicitly trigger the transmission of the reject component or to abort the dialogue.

On receipt of a TC-NOTICE indication the SACF is informed that a message cannot be delivered by the Network Layer. It occurs if the Return Option has been set (see 18.1.1.3.7). It is for the application process to decide whether to terminate the dialogue or retry.

The receipt of a TC-U-ABORT.Ind or TC-P-ABORT.Ind on a dialogue terminates all request processing.

#### 18.1.5.3	Dialogue handling

##### 18.1.5.3.1	Dialogue establishment

##### 18.1.5.3.2	Dialogue continuation

##### 18.1.5.3.3	Dialogue termination

##### 18.1.5.3.4	User Abort

##### 18.1.5.3.5	Provider Abort

If the Security Exchange Service Element (SESE) is included in the application context, and an SE-P-ABORT is required, then the SE-P-ABORT is mapped to the TC-U-ABORT primitive.

##### 18.1.5.3.6	Mapping to TC dialogue primitives

The SCF-SCF IN services can be mapped onto TC services. This subclause defines the mapping of the SCF-SCF IN services onto the services of the TC dialogue handling services defined in Recommendation Q.771.

a)	The TC-BEGIN service is used to invoke the operations of the scf-scfConnectionPackage and the dsspConnectionPackage and SETransfer operations.

b)	The TC-CONTINUE service is used to report the success of the operations invoked in a TC-BEGIN service and to invoke or respond to any other operations. If the SESE is included in the application context, the TC-CONTINUE service is used for the second and third exchanges.

c)	The TC-U-ABORT service is used to report the failure of operation of the scf-scfConnectionPackage and dsspConnectionPackage and SETransfer operations.

The mapping of the parameters onto the TC-service is as follows:

The use of the parameters of the TC-BEGIN service is as defined in 18.1.1.3.6 with the following qualifications:

–	The Application Context Name parameter of the TC-BEGIN service shall take the value of the application-context-name field of the scf-scfOperationsAC or distributedSCFSystemAC object.

–	The use of the User Information parameter of the TC-BEGIN service shall contain an EXTERNAL ASN.1 Type with the direct-reference field, if present, set to id-as-scfBindingAS and the value to be encoded shall be of type SCF-SCFBinding-PDUs.bind.bind-invoke (SCFBindRequestArgument). If the SESE is included in the application context, the User Information parameter of the TC-BEGIN service shall contain a value of type seItem.

The use of the parameters of the TC-CONTINUE service is as defined in 18.1.1.3.6 with the following qualifications:

–	The use of the User Information parameter of the first TC-CONTINUE service shall contain an EXTERNAL ASN.1 Type with the direct-reference field, if present, set to id-as-scfBindingAS and the value to be encoded shall be of type SCF-SCFBinding-PDUs.bind.bind-result (SCFBindResultArgument). If the SESE is included in the application context, the User Information parameter of the TC-CONTINUE service shall contain a value of type seItem.

In subsequent TC-CONTINUE services, the use of the User Information field is network-operator specific.

The use of the parameters of the TC-U-ABORT service is as defined in 18.1.1.3.6 with the following qualifications:

–	The Application-Context-Name parameter of the TC-U-ABORT service shall be used as specified in Recommendation Q.771. When the SCF refuses a dialogue because the application-context-name it receives is not supported, the value of the application-context-name it returns is selected as follows:

•	if the value has the same root as the scf-scfOperationsAC, the scf-scfOperationsAC is used;

•	if the value has the same root as the distributedSCFSystemAC, the distributedSCFSystemAC is used;

•	otherwise the received value is returned.

The use of the parameters of the TC-END service is as defined in 18.1.1.3.6.

#### 18.1.5.4	Component handling

##### 18.1.5.4.1	Procedures for INAP operations

The INAP ASEs are users of the TC component handling services except for the TC-L-REJECT and TC-L-CANCEL services which are used by the application-process. Receipt of a TC-L-REJECT.ind leads the application-process to abandon the dialogue (i.e. it issues a TC-U-ABORT.request primitive).

The TC-U-CANCEL service is never used.

##### 18.1.5.4.2	Mapping to TC component parameters

The SCF-SCF IN ASE services are mapped onto the TC component handling services. The mapping of operations and errors onto TC services is defined in 18.1.1.4.2 with the following qualification:

The time-out parameter of the TC-INVOKE.req primitives is set according to Table 9-1. The SCFBind timer (Tbi) does not map to a TC timer.

### 18.1.6	SCF-SDF interface

All the services provided by the Directory ASEs are contained in a single AE. The Component Handler (CHA) of the Transaction Capabilities (TC) supports the request/reply paradigm of the operation. The Directory ASEs provide the mapping function of the abstract-syntax notation of the directory operation packages onto the services provided by TC. The Dialogue Handler (DHA) of the TC supports the establishment and release of an application-association called "dialogue" between a pair of AEs. Dialogues between a DUA and a DSA may be established only by the DUA.

#### 18.1.6.1	Normal procedures

Dialogue handling services are used to support the DirectoryBind and DirectoryUnbind operations and to trigger the sending of the APDUs associated with the operations involved in the Directory packages.

Component grouping is performed under the control of the application-process through an appropriate usage of the TC-BEGIN and TC-CONTINUE service.

The TC-END service is solely used to support the unbind procedure (i.e. it is never used to trigger the sending of components).

On receipt of an empty TC-CONTINUE.req primitive, the SDF should ignore the primitive.

On receipt of an TC-END.req with a database request, the SDF should not perform the database request and consider the requested TC-END service as an unbind procedure. The dialogue is then terminated.

#### 18.1.6.2	Abnormal procedures

On receipt of a TC-U-REJECT.ind in the SDF, this primitive should be ignored.

On receipt of a TC-R-REJECT.ind in the SDF, the dialogue should be released with a TC-U-ABORT.req.

If reject situations are detected in the SDF, a TC-U-REJECT.req should be sent followed by a TC-CONTINUE.req.

The prearranged termination procedure is never used.

The application-process is the sole user of the TC-P-ABORT service and TC-NOTICE service.

The receipt of a TC-U-ABORT.ind or TC-P-ABORT.ind on a dialogue terminates all request processing. It is the application-process responsibility to confirm if requested modifications occurred.

It is an application-process responsibility to provide in the TC-BEGIN.req primitive a destination address which can be used by the underlying SCCP to route the message to the proper SDF.

#### 18.1.6.3	Dialogue handling

##### 18.1.6.3.1	Dialogue establishment

##### 18.1.6.3.2	Dialogue continuation

##### 18.1.6.3.3	Dialogue termination

##### 18.1.6.3.4	User Abort

If the SESE is included in the application context, and an SE-U-ABORT is required, then the SE-U-ABORT is mapped to the TC-U-ABORT primitive.

##### 18.1.6.3.5	Provider Abort

If the SESE is included in the application context, and an SE-P-ABORT is required, then the SE-P-ABORT is mapped to the TC-U-ABORT primitive.

##### 18.1.6.3.6	Mapping to TC dialogue primitives

This subclause defines the mapping of the DirectoryBind and DirectoryUnbind services onto the services of the TC dialogue handling services defined in Recommendation Q.771.

Bind

The DirectoryBind service is mapped onto TC-services as follows:

a)	The TC-BEGIN service is used to invoke the DirectoryBind operation and SETransfer operations.

b)	The TC-CONTINUE service is used to report the success of the DirectoryBind operation. If the SESE is included in the application context, the TC-CONTINUE service is used for the second and third exchanges.

c)	The TC-U-ABORT service is used to report the failure of the DirectoryBind operation and SETransfer operations.

The use of the parameter of these services is qualified in the following subclauses.

The use of parameters of the TC-BEGIN service is defined in 18.1.1.3.6 with the following qualifications:

–	The Application Context Name parameter of the TC-BEGIN service shall take the value of the application-context-name field of the iNdirectoryAccessAC or inExtendedDirectoryAccessAC object.

–	The Dialogue ID parameter of the TC-CONTINUE service shall be used as specified in 18.1.1.3.7.

The Authorized Relationship Id can be mapped onto the TC Dialogue ID parameter.

–	The User Information parameter of the TC-BEGIN service shall contain an EXTERNAL ASN.1 Type with the direct-reference field, if present, set to id-as-indirectoryBindingAS and the value to be encoded shall be of type DAPBinding-PDUs.bind.bind-invoke (DirectoryBindArgument).

If the SESE is included in the application context, the User Information parameter of the TC-BEGIN service shall contain a value of type seItem.

The use of parameters of the TC-CONTINUE service is defined in 18.1.1.3.6 with the following qualifications:

–	The Dialogue ID parameter of the TC-CONTINUE service shall be used as specified in 18.1.1.3.7.

The Authorized Relationship Id can be mapped onto the TC Dialogue ID parameter.

–	The User Information parameter of the TC-CONTINUE service shall contain an EXTERNAL ASN.1 Type with the direct-reference field, if present, set to id-as-indirectoryBindingAS and the value to be encoded shall be of type DAPBinding-PDUs.bind.bind-result (DirectoryBindResult).

If the SESE is included in the application context, the User Information parameter of the TC-CONTINUE service shall contain a value of type seItem.

The use of parameters of the TC-U-ABORT service is defined in 18.1.1.3.6 with the following qualifications:

–	The Dialogue ID parameter of the TC-CONTINUE service shall be used as specified in 18.1.1.3.7.

The Authorized Relationship Id can be mapped onto the TC Dialogue ID parameter.

–	The Application Context Name parameter of the TC-U-ABORT service shall be set as defined in 18.1.1.3.6 unless the TC-U-ABORT is reporting the failure of the directoryBind operation. In this case it is set to the value of the application-context-name field of the iNdirectoryAccessAC or inExtendedDirectoryAccessAC object.

–	The User Information parameter of the TC-U-ABORT service shall be used as specified in 18.1.1.3.7, unless the TC-U-ABORT is reporting the failure of a directoryBind operation. In this case this parameter shall contain an EXTERNAL ASN.1 Type with the direct-reference field, if present, set to id-as-indirectoryBindingAS and the value to be encoded shall be of type DAPBinding-PDUs.bind.bind-error (DirectoryBindError).

Unbind

The DirectoryUnbind service is mapped onto the TC-END service.

The use of the parameters of the TC-END service is defined in 18.1.1.3.6 with the following qualifications:

–	The Dialogue ID parameter of the TC-CONTINUE service shall be used as specified in 18.1.1.3.7.

The Authorized Relationship Id can be mapped onto the TC Dialogue ID parameter.

#### 18.1.6.4	Component handling

##### 18.1.6.4.1	Procedures for INAP operations

The Directory ASEs are users of the TC component handling services except for the TC-L-REJECT and TC-L-CANCEL services which are used by the application-process. Receipt of a TC-L-REJECT.ind leads the application-process to abandon the dialogue (i.e. it issues a TC-U-ABORT.request primitive).

The TC-U-CANCEL service is never used.

##### 18.1.6.4.2	Mapping to TC component parameters

The Directory ASE services are mapped onto the TC component handling services. The mapping of operations and errors onto TC services is defined in 18.1.1.4.2 with the following qualification:

The time-out parameter of the TC-INVOKE.req primitives is set according to Table 18-2.

Table 18-2/Q.1228 – TC timer values
of DAP operations

| Operation | Time-out |
| --- | --- |
| search | Medium |
| modifyEntry | Medium |
| addEntry | Medium |
| removeEntry | Medium |
| execute | Medium |

### 18.1.7	SDF-SDF interface

#### 18.1.7.1	Normal procedures

#### 18.1.7.2	Abnormal procedures

#### 18.1.7.3	Dialogue handling

##### 18.1.7.3.1	Dialogue establishment

##### 18.1.7.3.2	Dialogue continuation

##### 18.1.7.3.3	Dialogue termination

##### 18.1.7.3.4	User Abort

If the SESE is included in the application context, and an SE-U-ABORT is required, then the SE-U-ABORT is mapped to the TC-U-ABORT primitive.

##### 18.1.7.3.5	Provider Abort

If the SESE is included in the application context, and an SE-P-ABORT is required, then the SE-P-ABORT is mapped to the TC-U-ABORT primitive.

##### 18.1.7.3.6	Mapping to TC dialogue primitives

The DSP and DISP can be mapped onto TC services. This subclause defines the mapping of the DSABind, DSAUnbind, DSAShadowBind and DSAShadowUnbind services onto the services of the TC dialogue handling services defined in Recommendation Q.771.

The DirectoryBind service in mapped onto TC-services as follows:

a)	The TC-BEGIN service is used to invoke the DSAShadowBind and DSABind operations.

b)	The TC-CONTINUE service is used to report the success of the DSAShadowBind and DSABind operations.

c)	The TC-U-ABORT service is used to report the failure of the DSAShadowBind and DSABind operations.

The mapping of parameters onto the TC Dialogue services is as follows:

The use of parameters of the TC-BEGIN service is defined in 18.1.1.3.6 with the following qualifications:

–	The Application Context Name parameter of the TC-BEGIN service shall take the value of the application-context-name field of the inDirectorySystemAC or shadowSupplierInitiatedAC or shadowConsumerInitiatedAC object.

–	The User Information parameter of the TC-BEGIN service shall contain a value of type EXTERNAL which depends on the value of the Application Context being used.

If the Application Context being used is inDirectorySystemAC then the User Information parameter shall contain an EXTERNAL Type with the direct-reference field, if present, set to id-as-indirectoryDSABindingAS and the value to be encoded shall be of type DSABinding-PDUs.bind.bind-invoke.

If the Application Context being used is shadowSupplierInitiatedAC or shadowConsumerInitiatedAC then the User Information parameter shall contain an EXTERNAL Type with the direct-reference field, if present, set to id-as-indsaShadowBindingAS and the value to be encoded shall be of type DISBinding-PDUs.bind.bind-invoke.

The use of parameters of the TC-CONTINUE service is defined in 18.1.1.3.6 with the following qualifications:

–	The Dialogue Id parameter of the TC-CONTINUE service shall be used as specified in Recommendation Q.771. The Authorized Relationship Id can be mapped onto the TCAP Dialogue Id.

–	The User Information parameter of the first TC-CONTINUE service shall contain a value of type EXTERNAL which depends on the value of the Application Context being used.

If the Application Context being used is inDirectorySystemAC then the User Information parameter shall contain an EXTERNAL Type with the direct-reference field, if present, set to id-as-indirectoryDSABindingAS and the value to be encoded shall be of type DSABinding-PDUs.bind.bind-result.

If the Application Context being used is shadowSupplierInitiatedAC or shadowConsumerInitiatedAC then the User Information parameter shall contain an EXTERNAL Type with the direct-reference field, if present, set to id-as-indsaShadowBindingAS and the value to be encoded shall be of type DISPBinding-PDUs.bind.bind-result.

The use of parameters of the TC-U-ABORT service is defined in 18.1.1.3.6 with the following qualifications:

–	The Dialogue Id parameter of the TC-U-ABORT service shall be used as specified in 18.1.1.3.6. The Authorized Relationship Id can be mapped onto the TCAP Dialogue Id.

–	The Application-Context-Name parameter of the TC-U-ABORT service shall be used as specified in Recommendation Q.771. When the SDF refuses a dialogue because the application-context-name it receives is not supported, this parameter shall have the value of the application-context-name field of the inDirectorySystemAC or shadowSupplierInitiatedAC or shadowConsumerInitiatedAC object.

–	When the TC-U-ABORT is reporting the failure of a bind operation, i.e. the abort reason parameter has the value "dialogue-refused", the User Information parameter of the TC-U-ABORT service shall contain a value of type EXTERNAL which depends on the value of the Application Context being used.

If the Application Context being used is inDirectorySystemAC then the User Information parameter shall contain an EXTERNAL Type with the direct-reference field, if present, set to id-as-indirectoryDSABindingAS and the value to be encoded shall be of type DSABinding-PDUs.bind.bind-error.

If the Application Context being used is shadowSupplierInitiatedAC or shadowConsumerInitiatedAC then the User Information parameter shall contain an EXTERNAL Type with the direct-reference field, if present, set to id-as-indsaShadowBindingAS and the value to be encoded shall be of type DISPBinding-PDUs.bind.bind-error.

Otherwise it shall be absent.

The inDSAUnbind and inDSAShadowUnbind services are mapped onto the TC-END service. The use of the parameters of the TC-END service is defined in 18.1.1.3.6.

The SETransfer service is mapped onto TC-services is the same as for the other services except:

–	If the SESE is included in the application context, the User Information parameter of the TC-CONTINUE service shall contain a value of type seItem.

#### 18.1.7.4	Component handling

##### 18.1.7.4.1	Procedures for INAP operations

##### 18.1.7.4.2	Mapping to TC component parameters

The Directory ASE services are mapped onto the TC component handling services. The mapping of operations and errors onto TC services is defined in 18.1.1.4.2 with the following qualification:

The time-out parameter of the TC-INVOKE.req primitives is set according to Table 18-3.

Table 18-3/Q.1228 – TC timer values
of DSP/DISP operations

| Operation | Time-out |
| --- | --- |
| chainedAddEntry | Medium |
| chainedRemoveEntry | Medium |
| chainedModifyEntry | Medium |
| chainedExecute | Medium |
| chainedSearch | Medium |
| updateShadow | Medium |
| coordinateShadowUpdate | Medium |
| requestShadowUpdate | Medium |

## 18.2	Services assumed from SCCP

This subclause describes the services required from the SCCP that may be used by the IN applications for the IN Application Protocol used between the SSF, SCF and SRF, SDF and CUSF.

The services described are those given in the SCCP post White Book Recommendations Q.711-Q.714 (07/96) and Q.715 (SCCP User Guide) should be consulted to identify possible interworking and compatibility issues between the different SCCP versions.

### 18.2.1	Normal procedures

The SCCP forms the link between the TC and the MTP and provides (in conjunction with the MTP) the network services for the IN applications. The network services provided allow the signalling messages sent by the application to the lower layers to be successfully delivered to the peer application.

### 18.2.2	Service functions from SCCP

#### 18.2.2.1	SCCP connectionless services

The following Connectionless services are expected from the SCCP:

a)	Network Addressing to enable signalling connections between SCCP users.

b)	Sequence Control to enable the SCCP users to invoke "sequence guaranteed" or "sequence not guaranteed" options for a given stream of messages to the same destination.

c)	Segmentation/reassembly of large user messages.

d)	Return Option to enable the SCCP users to invoke "discard message on error" or "return message on error" for a given message not able to be delivered by the SCCP to the destination SCCP user, due to routing or segmentation/reassembly failure.

e)	Congestion control.

The primitives used for the above services are given below.

The N-UNITDATA request and N-UNITDATA indication primitives are used to send and receive data. The parameters of these primitives include the Called and Calling Addresses, Sequence Control, Return Option and User Data with the addressing parameters always mandatory.

The N-NOTICE indication primitive is used to return undelivered data if return option is set and a routing/segmentation error occurs.

##### 18.2.2.1.1	INAP addressing

The INAP addressing elements consist of information contained within the Calling and the Called Addresses which are sent by the application to TC for use by SCCP.

The application expects the SCCP to route messages by either:

a)	the use of the Destination Point Code (DPC), the SubSystem Number (SSN) and MTP SAP (Service Access Point) instance; or

b)	the use of the Global Title (GT) plus optionally the SSN, DPC and MTP SAP.

The application also specifies to SCCP whether to use Route on SSN or Route on GT for both the Called and Calling Addresses.

If INAP requires additional addressing information, it must be carried in the GT portion of the address specification regardless of which form of routing is specified.

Method a) above may be used when the application is aware of the destination point code and the destination SSN located at that point code to which the message is to be delivered. Within a national network different SSNs, according to Recommendation Q.713, may be allocated for the different network-specific applications, e.g. a SSN may be allocated for a SCF functionality.

Method b) above may be used when a message is to be delivered to a SCCP-user which can be identified by the combination of the elements within the GT. An example of the use of this method is when messages have to be delivered between different networks. This method may be used since the originating network is unaware of the point code and SSN's allocations within the destination network. The network that determines the end-node to which the message is to be delivered has to perform a Global Title Translation to derive the destination Point Code and the SSN. If optionally the original address contained the SSN, then this may be used as the destination SSN, or the translation may, if required, provide an appropriate new SSN. Where the destination node is in another network (and is not the gateway node) then the application populates the SSN field with either the SSN in use at the destination or zero.

When GT is used for addressing, the IN application expects that the SCCP supports the following elements as defined in Recommendation Q.713:

Global Title Indicator

This indicator specifies the method employed for the formatting of the address information. The format with the indicator value 4 is always used for internetwork connections.

Translation Type

The Translation Types are defined within Recommendation Q.713.

Numbering Plan

1)	For Addresses used on internetwork interfaces the Numbering Plan must be either Generic Numbering, or E.164 as defined in Annex B/Q.713.

2)	For Addresses used on all other interfaces, any of the Numbering plans defined in Recommendation Q.713 may be used if deemed suitable.

Global Title Address Information

This is the actual INAP address information supplied by the application and is encoded as indicated by the encoding scheme.

Encoding scheme

The application should set the value of the encoding scheme according to the format of the GTAI. The allowed values are defined in Recommendation Q.713.

The network provider must ensure that any change of GT value during translation preserves any INAP specific information contained in the initial GT value. The GT translation data in the network must not delete the GT information, if present, from the Address.

This requirement applies to all interfaces, not just those used for internetworking.

If route on SSN is to be supported from the originating node, then a non-zero internationally standardised SSN is required for international internetworking.

In the absence of a standardized non-zero SSN for INAP services, the use of route on GT is mandatory from the origin node to the network containing the destination node.

The version of SCCP used to support INAP operations must be at least White Book, 1993.

##### 18.2.2.1.2	Sequence control

The application will specify whether SCCP protocol class 0 or 1 is required.

Class 0 is used when the in-sequence delivery of messages to a specific called address is not required.

Class 1 is used when the in-sequence delivery of messages to a specific called address is required.

##### 18.2.2.1.3	Return on error

The use of Return on Error mechanism may be required by the IN applications so that the application is aware of messages that have not been delivered to the destination by the SCCP. The return option allows the return of the message that was not delivered due to routing or segmentation/reassembly failure back to the issuing user.

If the return option is invoked by the application and the message is not delivered, then the SCCP specifies the "return reason" as specified in Recommendation Q.711. The N-NOTICE primitive is used to return the undelivered message to the originating user.

##### 18.2.2.1.4	Segmentation/reassembly

The application expects that since the SCCP can send up to 260 octets of user data (including the address information and TC-message) in a UDT message (248 octets in a XUDT message performing segmentation and congestion control), segmentation is available for long user data.

Also the SCCP is expected to perform the reassembly function on received segmented messages and deliver the reassembled user data to the user.

However, it should be noted that even though the theoretical maximum size of SCCP-user data and addresses that can be segmented by the SCCP is 3968 octets, the SCCP-user would limit the length to about 2560 octets to allow for the largest known addresses. Note that the application must also allow for the octets used for the TC-message in the 2560 octets.

The IN application does not expect the SCCP to segment the user data into more than 16 segments.

##### 18.2.2.1.5	Congestion control

To help control of possible congestion that might occur in the lower layers, the application may assign a value to indicate the importance of the message. The use of this parameter requires the use of SCCP [Q.711-Q.714 (07/96) and Q.715 (SCCP User Guide)] Recommendations.

Also there exist other congestion control mechanisms as indicated below in SCCP Management.

#### 18.2.2.2	SCCP Connection-oriented Services

The use by IN applications for the Connection-oriented services is outside the scope of IN CS-2.

#### 18.2.2.3	SCCP management

The subsystems used within the IN scenario expect the SCCP to provide management procedures to maintain network performance by re-routing in the event of failure of a subsystem, and in case of network congestion by use of the congestion handling procedure. These procedures have appropriate interactions with the SCCP user as described in Recommendation Q.711.

To achieve the above, the SCCP is expected to perform the following procedures:

–	Signalling point status management (N-PCSTATE, which includes the signalling point prohibited, signalling point allowed, signalling point congested, and local MTP availability subprocedures).

–	Subsystem status management (N-STATE, which includes the subsystem prohibited, subsystem allowed, and subsystem status test subprocedures).

–	Coordinated state change (N-COORD, which is a procedure which allows a duplicated subsystem to be withdrawn from service without affecting the performance of the network).

# 19	IN generic interface security

Any interface within the IN functional architecture may have the need to apply security functions to the information flows passing across the interface. This clause defines a generic set of security mechanisms and procedures based on the X.509 Authentication Framework and the Simple Public Key GSS API Mechanism (SPKM) to enable any interfaces to provide suitable secured communications. This clause defines a sub-profile of X.509 and SPKM for use of IN CS-2.

For IN CS-2, the provision for security functions is required for the SCF-SDF, SDF-SDF, and SCF-SCF interfaces.

## 19.1	Interface security requirements

The security requirements on an IN system could be divided in the two following main families:

1)	Requirements to offer security features (Network access security requirements): This covers the various aspects relating to the protection of user access to services and of terminals to networks against attacks at the access interface (for instance user impersonation), by means of security features such as: user/terminal authentication (i.e. the result of a process by which a service user proves his or her identity to an IN system), user profile verification (i.e. the verification that a user is authorised to use a functionality), etc.

2)	Requirements on the internetworking interfaces (Internetworking security requirements): This covers the protection of interactions between the various entities and agents involved in the provision of a telecommunication service against attacks at the internal interfaces of the system or even against data stored inside equipment, by means of security features such as: peer entity authentication (i.e. a process which allows a communicating entity to prove its identity to another entity in the network), signalling data or TMN data integrity, non-repudiation, confidentiality, entity profile verification (i.e. the verification that an entity is authorised to use a functionality), etc.

For each of the above problems, the interfaces have to provide processes to enable data confidentiality, data origin authentication and data integrity. These capabilities are generally described in Recommendation X.500.

### 19.1.1	Data confidentiality

Data Confidentiality ensures that only the parties involved can examine the contents of the message. Encryption, which applies an algorithm to transform the message into a protected form, is sufficient to provide data confidentiality. The encryption methods for all communications with the directory shall be conformant to Directory Security Mechanisms or to directory entry methods.

### 19.1.2	Data integrity and data origin authentication

Data Integrity ensures that the contents of the message have not been altered during transmission. Data Origin Authentication ensures that the message came from the expected origin. Digital Signatures, which applies an encryption algorithm to the result of applying a hash function to the contents of message, are sufficient for providing data integrity and data origin authentication. Data integrity methods for all communications with the directory shall be conformant to Directory Security Mechanisms or to directory entry methods.

### 19.1.3	Key management

Key Management refers to the generation, distribution and maintenance of private keys used in the cryptographic system. Key Management methods conformant with X.509 Recommendation shall be used.

## 19.2	Procedures and algorithms

Several assumptions are provided to the IN CS-2 interface security procedures to focus the work, but which also provides acceptable levels of security (see Table 19-1):

•	Generic Security Interface shall be based on SPKM as much as possible.

•	The number of parameters in messages and procedures should be minimized, because each of them adds to the signalling traffic load and to some processing time.

•	Only one type of security shall be applied to the interface. The order of selection will be none, User, Peer:

Table 19-1/Q.1228 – Categories of security levels

|  | Signature | Encryption |
| --- | --- | --- |
| None | None | None |
| User | None | User |
|  | User | None |
|  | User | User |
| Peer | None | Peer |
|  | Peer | None |
|  | Peer | Peer |

¥	If both user and peer signature/encryption are requested, then user signature/encryption should be used.

¥	Only the mechanisms Encryption and Digital Signatures [including Message Authentication Code (MAC) techniques] shall be used.

### 19.2.1	Authentication procedures

The Authentication Procedure defines the process of establishing authentication and shall perform the following functions:

¥	Authenticate the Entity (User/FE) as being allowed to access an FE.

¥	Establish the level of security for all messages between the Entity and the FE.

¥	If required, establish a session key for encryption of messages between the Entity and the FE.

The current IN CS-2 authentication uses possible procedures defined in Recommendation X.509, together with a new procedure, SPKM. The primary purpose of the X.509 Recommendation is to provide authentication for a directory service, which includes mechanisms on Simple Authentication, Strong Authentication and SPKM. This subclause indicates which aspects of the Directory Authentication should be considered and supported by implementors within the scope of IN CS-2.

The current CS-2 authentication procedures are:

1)	Simple Authentication (see 6.2/X.509), using Bind operations with passwords for IN CS-1 (1995) or IN CS-2.

2)	Strong Authentication, (see clause 10/X.509), using Bind operations with strong credentials for IN CS-2 and 1993 Directory; or using SPKM for IN CS-2.

3)	External Procedures, using Bind operations for IN CS-1 (1995) or IN CS-2.

4)	SPKM (Simple Public Key GSS-API Mechanism):

–	The SPKM allows both unilateral and mutual authentication to be accomplished without the use of secure timestamps. This enables environments which do not have access to secure time to nevertheless have access to secure authentication.

–	The SPKM uses Algorithm Identifiers to specify various algorithms (for data confidentiality, data integrity, etc.) to be used by the communicating peers. This allows maximum flexibility for a variety of environments, for future enhancements, and for alternative algorithms.

### 19.2.2	SPKM algorithms and negotiation

Additional information on SPKM is provided in this subclause. For additional information on other authentication mechanisms, including Directory entry methods, please refer to the user guide and relevant Directory specifications.

A number of algorithm types are employed in SPKM. Examples of algorithms are presented in Appendix III. It should be noted that in the SPKM algorithms, the terms "initiator" and "target" are used. For IN CS-2, the term "target" may be referred to as "consumer".

#### 19.2.2.1	Data Confidentiality Algorithm (C-ALG)

The symmetric algorithm is used to generate the encrypted data. The DES-CBC algorithm is an example for data confidentiality.

#### 19.2.2.2	Data Integrity Algorithm (I-ALG)

The purpose of this algorithm is to ensure that a message has not been altered in any way after being constructed by the legitimate sender. Depending on the algorithm used, the application of this algorithm may also provide authenticity and support non-repudiation (including data origin authentication) for the message. Algorithms such as md5WithRSA, DES-MAC, md5-DES-CBC, and sum64-DES-CBC are examples for data integrity.

#### 19.2.2.3	Key Management Algorithm (K-ALG)

The purpose of this algorithm is to establish a symmetric key (called context key) for use by both the initiator and the responder (called target) over the established context. The keys used for C-ALG and any keyed I-ALGs (for example, DES-MAC) are derived from this context key. Key establishment is done within the X.509 authentication exchange and so the resulting shared symmetric key is authenticated. Algorithms such as RSAEncryption and dhKeyAgreement are examples for Key Management.

#### 19.2.2.4	One-Way Function (O-ALG) for Subkey Derivation Algorithm

Having established a context key using the negotiated K-ALG, both initiator and target must be able to derive a set of subkeys for the various C-ALGs and keyed I-ALGs.

#### 19.2.2.5	SPKM negotiation

During context establishment in SPKM, the initiator offers a set of possible confidentiality algorithms and a set of possible integrity algorithms to the target (note that the term "integrity algorithms" includes digital signature algorithms). The confidentiality algorithms selected by the target become those that may be used for C-ALG over the established context, and the integrity algorithms selected by the target become those that may be used for I-ALG over the established context (the target "selects" algorithms by returning, in the same relative order, the subset of each offered list that it supports). Note that any C-ALG and I-ALG may be used for any message over the context and that the first confidentiality algorithm and the first integrity algorithm in the agreed sets become the default algorithms for that context.

The agreed confidentiality and integrity algorithms for a specific context define the valid values of the Quality of Protection (QOP) parameter. If no response is expected from the target in unilateral authentication, then the algorithms offered by the initiator are the ones that may be used over the context (if this is unacceptable to the target, then the association with this context will not be successfully established).

Furthermore, in the first context establishment token the initiator offers a set of possible K-ALGs, along with the key (or key half) corresponding to the first algorithm in the set (its preferred algorithm). If this K-ALG is unacceptable to the target, then the target must choose one of the other K-ALGs in the set and send, this choice along with the key (or key half) corresponding to this choice in its response (otherwise the association with this context will not be established). If necessary (that is, if the target chooses a 2-pass K-ALG such as dhKeyAgreement), the initiator will send its key half in a response to the target.

Finally, in the first context establishment token the initiator offers a set of possible O-ALGs (subkey derivation algorithms) (only a single O-ALG if no response is expected). The (single) O-ALG chosen by the target becomes the subkey derivation algorithm to be used over the context.

### 19.2.3	Three-way mutual authentication

The use of three-way mutual authentication procedures (including SPKM) in the establishment of a security association between two Functional Entities (FEs) implies the use of the Generic Upper Layers Security (GULS) Security Exchange Service Element (SESE) protocol (defined in Recommendations X.831 and X.832) as described in Recommendation X.519.

### 19.2.4	Assignment of credentials

Procedures for assigning credentials to users or FEs are generally defined in Recommendation X.500.

## 19.3	Mapping of security information flow definitions to tokens

In the case of the SCF-SDF interface, the Context Establishment Tokens will be conveyed in Bind procedure and the Per-Message Tokens will be used in DAP operations.

In the case of the SDF-SDF interface, the Context Establishment Tokens will be conveyed in Bind procedure and the Per-Message Tokens will be used in DSP and DISP operations.

In the case of the SCF-SCF interface, the Context Establishment Tokens will be conveyed in the SCF Bind procedure and the Per-Message Tokens will be used in SCF-SCF operations and Distributed SCF System operations.

### 19.4	Security FSM definitions

The Security FSMs described here are generic machines. They illustrate how the security procedures are handled across the interface between two FEs.

### 19.4.1	Two-way mutual authentication FSMs

#### 19.4.1.1	Outgoing FSM

The outgoing FSM controls the originating side of the interface. It is shown in Figure 19-1.

Figure 19-1/Q.1228 – The originating FSM states

##### 19.4.1.1.1	State 1: Idle

The following event is considered in this state:

–	(e1)	AuthRequest: This is an internal event, caused by the need of the service logic to create an association with a remote FE in order to begin accessing a service function. This event causes a transition to the state 2, Wait for subsequent requests.

##### 19.4.1.1.2	State 2: Wait for subsequent requests

In this state, subsequent operations to be sent with the AuthRequest operation (in the same message) to the remote FE are expected. The following two events are considered in this state:

–	(e2)	Request_to_remote_FE: This is an internal event caused by the reception of an operation. The operation is buffered until the reception of a delimiter (or a timer expiration). The Originating FSM remains in the same state; and

–	(e3)	Send_AuthRequest_with_requests: This is an internal event caused by the reception of a delimiter, that indicates the reception of the last operation to be sent. Once the delimiter is received, a message containing the argument of the AuthRequest operation and other operations' arguments, if any, is sent to the remote FE. This event causes a transition out of this state to the state 3, Wait for AuthResult.

##### 19.4.1.1.3	State 3: Wait for AuthResult

In this state, the local FE is waiting for the response from the remote FE. Two events are considered in this state:

–	(E4)	Auth_Error: This is an external event, caused by the reception of an error to the AuthRequest operation previously issued to the remote FE. This event causes a transition out of this state to state 1, Idle; and

–	(E5)	Response_with_AuthGrant: This is an external event, caused by the reception of a AuthRequest result combined with the responses to other operations previously issued to the remote FE (if any). This event causes a transition to state 4, Secured.

##### 19.4.1.1.4	State 4: Secured

In this state, the local FE has established an authenticated access to the remote FE, and is waiting for requests to the remote FE from the service logic or is waiting for responses to the operations previously issued to the remote FE. Three events are considered in this state:

–	(e6)	Request_to_remote_FE: This is an internal event, caused by the need of the service logic to access the remote FE. The Originating FSM remains in the same state;

–	(E7)	Response_from_remote_FE: This is an external event, caused by the reception of responses to the operations previously issued to the remote FE. The Originating FSM remains in the same state; and

–	(e8)	AuthEnd: This is an internal event, caused by the need of the service logic to terminate the authenticated access to the remote FE. This event causes a transition state to state 1, Idle.

#### 19.4.1.2	Incoming FSM

The incoming FSM controls the terminating side of the interface. It is shown in Figure 19-2.

Figure 19-2/Q.1228 – The terminating FSM states

Each state is discussed in one of the following subclauses.

General rules applicable to more than one state are as follows:

In any state, if the dialogue with the Remote FE is terminated, then the Terminating FSM returns to Idle state after ensuring that any resources allocated to the call have been de-allocated.

##### 19.4.1.2.1	State 1: Idle

The only event accepted in this state is:

–	(E1)	AuthRequest: This is an external event caused by the reception of AuthRequest operation from the remote FE. This event causes a transition out of this state to state 2, Grant Pending.

##### 19.4.1.2.2	State 2: Grant Pending

In this state, an Auth Request has been received from the remote FE. The local FE is performing the authentication procedures. Three events are considered in this state:

–	(e2)	AuthError: This is an internal event, caused by the failure of the AuthRequest operation previously issued to the local FE. This event causes a transition out of this state to state 1, Idle and a AuthError is returned to the invoking SCF;

–	(E3)	Request_from_remote_FE: This is an external event, caused by the reception of operations before the result from the AuthRequest operation is determined. The SDSM remains in the same state; and

–	(e4)	AuthRequest_Granted: This is an internal event, caused by the successful completion of the AuthRequest operation previously issued to the local FE. This event causes a transition out of this state to state 3, Secured.

##### 19.4.1.2.3	State 3: Secured

In this state, the access of the remote FE to the local FE was authorized and operations coming from the remote FE are accepted. Besides waiting for requests from the remote FE, the local FE can send responses to previously issued operations. Three events are considered in this state:

–	(E5)	AuthEnd: This is an external event, caused by the reception of the AuthEnd operation from the remote FE. The authorised association is ended and all associated resources are released. This event causes a transition out of this state to state 1, Idle;

–	(e6)	Response_to_remote_FE: This is an internal event, caused by the completion of the operations previously issued by the remote FE. The results or errors of these operations are returned to the remote FE. The Terminating FSM remains in the same state; and

–	(E7)	Request_from_remote_FE: This is an external event, caused by the reception of a request from the remote FE. The Terminating FSM remains in the same state.

### 19.4.2	Three-way mutual authentication FSMs

An outgoing and incoming generic security FSMs for three-way mutual authentication are described in this subclause. The Security FSMs described here are generic machines. They illustrate how the security procedures are handled across the interface between two FEs.

#### 19.4.2.1	Outgoing FSM for three-way mutual authentication

The outgoing FSM controls the originating side of the interface. It is shown in Figure 19-3.

Figure 19-3/Q.1228 – The originating FSM states for three-way mutual authentication

##### 19.4.2.1.1	State 1: Idle

The following event is considered in this state:

–	(e1)	AuthRequest1: This is an internal event, caused by the need of the service logic to create an association with a Remote FE in order to begin accessing a service function. This event causes a transition to the state 2, Wait for AuthResult1.

##### 19.4.2.1.2	State 2: Wait for AuthResult1

In this state, a response to the initial challenge (sent in the AuthRequest1) from the remote FE is expected. The following event is considered in this state:

–	(E2)	AuthGrant1/AuthRequest2: This is an external event caused by the reception of a response to the initial challenge and a subsequent challenge from the remote FE. This event causes a transition out of this state to the state 3, AuthGrant Processing/AuthRequest Processing.

##### 19.4.2.1.3	State 3: AuthGrant Processing/AuthRequest Processing

In this state, the local FE confirms the correct response to the AuthRequest1. In addition, an appropriate response to AuthRequest2 is created. Two events are considered in this state:

–	(e3)	AuthError: This is an internal event, caused by the an error in processing AuthGrant1 received from the remote FE in response to AuthRequest1. This event causes a transition out of this state to state 1, Idle; and

–	(e4)	AuthGrant2: This is an internal event, caused by the processing of an appropriate response to AuthRequest2 which is issued to the remote FE. This event causes a transition to state 4, Secured.

##### 19.4.2.1.4	State 4: Secured

In this state, the local FE has established a unilateral authentication with the remote FE. Once the remote FE also progresses to a Secured state, requests to and from the remote FE may be processed (these events are not shown for clarification). One event is considered in this state:

–	(e5)	AuthEnd: This is an internal event, caused by the need of the service logic to terminate the authenticated access to the remote FE. This event causes a transition state to state 1, Idle.

#### 19.4.2.2	Incoming FSM for three-way mutual authentication

The incoming FSM controls the terminating side of the interface. It is shown in Figure 19-4.

Figure 19-4/Q.1228 – The terminating FSM states for three-way mutual authentication

##### 19.4.2.2.1	State 1: Idle

The only event accepted in this state is:

–	(E1)	AuthRequest1: This is an external event caused by the reception of AuthRequest1 information from the remote FE. This event causes a transition out of this state to state 2, AuthRequest Processing.

##### 19.4.2.2.2	State 2: AuthRequest Processing

In this state, an AuthRequest has been received from the remote FE. The local FE is performing the authentication procedures. One event is considered in this state:

–	(e2)	AuthGrant1/AuthRequest2: This is an internal event, caused by the successful completion of the AuthRequest1 operation previously issued to the local FE. In addition, the local FE authentication challenge (AuthRequest2) is generated. This event causes a transition out of this state to state 3, Wait for AuthResult.

##### 19.4.2.2.3	State 3: Wait for AuthResult

In this state, the local FE awaits confirmation from the remote FE that either the AuthGrant1 response to AuthRequest1 challenge was accepted, or a response (AuthGrant2) to the AuthRequest2 challenge is received. Two external events are considered in this state:

–	(E3)	AuthError: This is an external error event, generated by the remote FE, on receipt of an invalid response to its challenge. This event causes a transition out of this state to state 1, Idle; and

–	(E4)	AuthGrant2: This is an external event, generated by the remote FE, containing the response (AuthGrant2) to the local FE challenge (AuthRequest2). This event causes a transition out of this state to state 4, AuthGrant Processing.

##### 19.4.2.2.4	State 4: AuthGrant Processing

In this state, the local FE confirms the received response (AuthGrant2) to the challenge (AuthRequest2). Two internal events are considered in this state:

–	(e5)	AuthError: This is an internal error event, generated by the local FE, on receipt of an invalid response to its challenge. This event causes a transition out of this state to state 1, Idle; and

–	(e6)	AuthGrant OK: This is an internal event, generated by the local FE, on receipt of a valid response to its challenge. This event causes a transition to state 5, Secured.

##### 19.4.2.2.5	State 5: Secured

In this state, the local FE has established a unilateral authentication with the remote FE. Since the remote FE is also in a Secured state, requests to and from the remote FE may be processed (these events are not shown for clarification). One event is considered in this state:

–	(E7)	AuthEnd: This is an external event, caused by the need of the service logic to terminate the authenticated access to the remote FE. This event causes a transition state to state 1, Idle.

| ITU-T RECOMMENDATIONS SERIES |  |
| --- | --- |
| Series A | Organization of the work of the ITU-T |
| Series B | Means of expression: definitions, symbols, classification |
| Series C | General telecommunication statistics |
| Series D | General tariff principles |
| Series E | Overall network operation, telephone service, service operation and human factors |
| Series F | Non-telephone telecommunication services |
| Series G | Transmission systems and media, digital systems and networks |
| Series H | Audiovisual and multimedia systems |
| Series I | Integrated services digital network |
| Series J | Transmission of television, sound programme and other multimedia signals |
| Series K | Protection against interference |
| Series L | Construction, installation and protection of cables and other elements of outside plant |
| Series M | TMN and network maintenance: international transmission systems, telephone circuits, telegraphy, facsimile and leased circuits |
| Series N | Maintenance: international sound programme and television transmission circuits |
| Series O | Specifications of measuring equipment |
| Series P | Telephone transmission quality, telephone installations, local line networks |
| Series Q | Switching and signalling |
| Series R | Telegraph transmission |
| Series S | Telegraph services terminal equipment |
| Series T | Terminals for telematic services |
| Series U | Telegraph switching |
| Series V | Data communication over the telephone network |
| Series X | Data networks and open system communications |
| Series Y | Global information infrastructure |
| Series Z | Programming languages |