
	[image: image1.wmf]
	INTERNATIONAL  TELECOMMUNICATION  UNION

	
	

	ITU-T
	G.722.2

	TELECOMMUNICATION
STANDARDIZATION  SECTOR
OF  ITU
	Annex C

(01/2002)   

	
	SERIES G: TRANSMISSION SYSTEMS AND MEDIA, DIGITAL SYSTEMS AND NETWORKS

Digital terminal equipments – Coding of analogue signals by methods other than PCM



	
	Wideband coding of speech at around 16 kbit/s using Adaptive Multi-Rate Wideband (AMR-WB)

Annex C: Fixed-point C-code



	
	ITU‑T  Recommendation  G.722.2  –  Annex C




ITU-T G-SERIES  RECOMMENDATIONS

TRANSMISSION SYSTEMS AND MEDIA, DIGITAL SYSTEMS AND NETWORKS

	
	

	INTERNATIONAL TELEPHONE CONNECTIONS AND CIRCUITS
	G.100–G.199

	GENERAL CHARACTERISTICS COMMON TO ALL ANALOGUE CARRIER-TRANSMISSION SYSTEMS
	G.200–G.299

	INDIVIDUAL CHARACTERISTICS OF INTERNATIONAL CARRIER TELEPHONE SYSTEMS ON METALLIC LINES
	G.300–G.399

	GENERAL CHARACTERISTICS OF INTERNATIONAL CARRIER TELEPHONE SYSTEMS ON RADIO-RELAY OR SATELLITE LINKS AND INTERCONNECTION WITH METALLIC LINES
	G.400–G.449

	COORDINATION OF RADIOTELEPHONY AND LINE TELEPHONY
	G.450–G.499

	TESTING EQUIPMENTS
	G.500–G.599

	TRANSMISSION MEDIA CHARACTERISTICS
	G.600–G.699

	DIGITAL TERMINAL EQUIPMENTS
	G.700–G.799

	General
	G.700–G.709

	Coding of analogue signals by pulse code modulation
	G.710–G.719

	Coding of analogue signals by methods other than PCM
	G.720–G.729

	Principal characteristics of primary multiplex equipment
	G.730–G.739

	Principal characteristics of second order multiplex equipment
	G.740–G.749

	Principal characteristics of higher order multiplex equipment
	G.750–G.759

	Principal characteristics of transcoder and digital multiplication equipment
	G.760–G.769

	Operations, administration and maintenance features of transmission equipment
	G.770–G.779

	Principal characteristics of multiplexing equipment for the synchronous digital hierarchy
	G.780–G.789

	Other terminal equipment
	G.790–G.799

	DIGITAL NETWORKS
	G.800–G.899

	DIGITAL SECTIONS AND DIGITAL LINE SYSTEM
	G.900–G.999

	QUALITY OF SERVICE AND PERFORMANCE
	G.1000–G.1999

	TRANSMISSION MEDIA CHARACTERISTICS
	G.6000–G.6999

	General
	G.6000–G.6099

	Symmetric cable pairs
	G.6100–G.6199

	Land coaxial cable pairs
	G.6200–G.6299

	Submarine cables
	G.6300–G.6499

	Optical fibre cables
	G.6500–G.6599

	Characteristics of optical components and subsystems
	G.6600–G.6999

	DIGITAL TERMINAL EQUIPMENTS
	G.7000–G.7999

	General
	G.7000–G.7099

	Coding of analogue signals by pulse code modulation
	G.7100–G.7199

	Coding of analogue signals by methods other than PCM
	G.7200–G.7299

	Principal characteristics of primary multiplex equipment
	G.7300–G.7399

	Principal characteristics of second order multiplex equipment
	G.7400–G.7499

	Principal characteristics of higher order multiplex equipment
	G.7500–G.7599

	Principal characteristics of transcoder and digital multiplication equipment
	G.7600–G.7699

	Operations, administration and maintenance features of transmission equipment
	G.7700–G.7799

	Principal characteristics of multiplexing equipment for the synchronous digital hierarchy
	G.7800–G.7899

	Other terminal equipment
	G.7900–G.7999

	DIGITAL NETWORKS
	G.8000–G.8999

	General aspects
	G.8000–G.8099

	Design objectives for digital networks
	G.8100–G.8199

	Quality and availability targets
	G.8200–G.8299

	Network capabilities and functions
	G.8300–G.8399

	SDH network characteristics
	G.8400–G.8499

	Management of transport network
	G.8500–G.8599

	SDH radio and satellite systems integration
	G.8600–G.8699

	Optical transport networks
	G.8700–G.8799

	
	


For further details, please refer to the list of ITU-T Recommendations.

	ITU-T Recommendation G.722.2

Wideband coding of speech at around 16 kbit/s using
Adaptive Multi-Rate Wideband (AMR-WB)
Annex C

Fixed-point C-code



	Summary

This annex specifies the bit-exact ANSI C-code implementation of the AMR-WB algorithm specified in ITU-T Rec. G.722.2, its Annexes A and B, and its Appendix I (non-normative). 

The C-code specified in this annex was also adopted by 3GPP in 3GPP specification TS 26.173.

This annex includes an electronic attachment containing the C code of the G.722.2 AMR-WB speech  transcoder.


	Source

Annex C to ITU‑T Recommendation G.722.2 was prepared by ITU‑T Study Group 16 (2001‑2004) and approved under the WTSA Resolution 1 procedure on 13 January 2002.


	


FOREWORD

The International Telecommunication Union (ITU) is the United Nations specialized agency in the field of tele​com​mu​ni​ca​tions. The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of ITU. ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Assembly (WTSA), which meets every four years, establishes the topics for study by the ITU‑T study groups which, in turn, produce Recommendations on these topics.

The approval of ITU-T Recommendations is covered by the procedure laid down in WTSA Resolution 1.

In some areas of information technology which fall within ITU-T's purview, the necessary standards are prepared on a collaborative basis with ISO and IEC.

NOTE

In this Recommendation, the expression "Administration" is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

INTELLECTUAL PROPERTY RIGHTS
ITU draws attention to the possibility that the practice or implementation of this Recommendation may involve the use of a claimed Intellectual Property Right. ITU takes no position concerning the evidence, validity or applicability of claimed Intellectual Property Rights, whether asserted by ITU members or others outside of the Recommendation development process.

As of the date of approval of this Recommendation, ITU had received notice of intellectual property, protected by patents, which may be required to implement this Recommendation. However, implementors are cautioned that this may not represent the latest information and are therefore strongly urged to consult the TSB patent database.

(  ITU  2002

All rights reserved. No part of this publication may be reproduced, by any means whatsoever, without the prior written permission of ITU.

CONTENTS


Page
Annex C – Fixed-point C-code

1
C.1
C code structure

1

C.1.1
Contents of the C source code

1
C.1.2
Program execution

1
C.1.3
Code hierarchy

2
C.1.4
Variables, constants and tables

5
C.2
Homing procedure

10
C.3
File formats

10
C.3.1
Speech file (encoder input/decoder output)

10
C.3.2
Mode control file (encoder input)

11
C.3.3
Parameter bitstream file (encoder output/decoder input)

11
Electronic attachment

ITU-T Recommendation G.722.2

Wideband coding of speech at around 16 kbit/s using
Adaptive Multi-Rate Wideband (AMR-WB)

Annex C

Fixed-point C-code

C.1
C code structure

This annex gives an overview of the structure of the bit‑exact C code for the correct implementation of  the G.722.2 main body, its Annex A (Comfort noise aspects), Annex B (Source Controlled Rate operation), and Appendix I (Error concealment of erroneous or lost frames). It provides an overview of the contents and organization of the C code attached to this annex. In case of discrepancy between the description given in the several parts of ITU-T Rec. G.722.2 (including its Annexes A, B and Appendix I) and the ANSI C-source code, the algorithm description of the ANSI C code shall prevail.

The C code has been verified on a number of systems (please see the readme.txt file for the complete list).

ANSI‑C was selected as the programming language because portability was desirable.

C.1.1
Contents of the C source code

The C code distribution has all files in the root level.

The distributed files with suffix "c" contain the source code and the files with suffix "h" are the header files. The ROM data is contained mostly in files with suffix "tab".

The C code distribution also contains one speech coder installation verification data file, "spch_dos.inp". The reference encoder output file is named "spch_dos.cod", the reference decoder input file is named "spch_dos.dec" and the reference decoder output file is named "spch_dos.out". These four files are formatted such that they are correct for an IBM PC/AT compatible computer. The same files with reversed byte order of the 16-bit words are named "spch_unx.inp", "spch_unx.cod", "spch_unx. dec" and "spch_unx.out", respectively.

Final verification of bit-exactness is to be performed using the Adaptive Multi-Rate Wideband test sequences described in Annex D/G.722.2.

Makefiles are provided for the platforms in which the C code has been verified (listed above). Once the software is installed, this directory will have a compiled version of encoder and decoder (the bit-exact C executables of the speech codec) and all the object files.

C.1.2
Program execution

The Adaptive Multi-Rate Wideband codec is implemented in two programs:

–
(encoder)
speech encoder;

–
(decoder)
speech decoder.

The programs should be called like:

–
encoder [encoder options] <speech input file> <parameter file>;

–
decoder <parameter file> <speech output file>.

The speech files contain 16-bit linear encoded PCM speech samples and the parameter files contain encoded speech data and some additional flags.

The encoder and decoder options will be explained by running the applications without input arguments. See the file readme.txt for more information on how to run the encoder and decoder programs.

C.1.3
Code hierarchy

Tables C.1 to C.3 are call graphs that show the functions used in the speech codec, including the functions of VAD, DTX, and comfort noise generation.

Each column represents a call level and each cell a function. The functions contain calls to the functions in rightwards neighbouring cells. The time order in the call graphs is from the top downwards as the processing of a frame advances. All standard C functions: printf(), fwrite(), etc. have been omitted. Also, no basic operations (add(), L_add(), mac(), etc.) or double precision extended operations (e.g. L_Extract()) appear in the graphs. The initialization of the static RAM (i.e. calling the _init functions) is also omitted.

The basic operations are not counted as extending the depth, therefore the deepest level in this software is level 6. 

The encoder call graph is broken down into two separate call graphs, Tables C.1 to C.2.

Table C.1/G.722.2 – Speech encoder call structure

	coder
	Copy
	
	
	
	

	
	Decim_12k8
	Down_samp
	Interpol (function)
	
	

	
	
	Copy
	
	
	

	
	Set_zero
	
	
	
	

	
	HP50_12k8
	
	
	
	

	
	Scale_sig
	
	
	
	

	
	wb_vad
	Filter_bank
	Filter5
	
	

	
	
	
	Filter3
	
	

	
	
	
	Level_calculation
	
	

	
	
	vad_decision
	Ilog2
	
	

	
	
	
	Noise_estimate_update
	update_cntrl
	

	
	
	
	hangover_addition
	
	

	
	
	Estimate_Speech
	
	
	

	
	tx_dtx_handler
	
	
	
	

	
	Parm_serial
	
	
	
	

	
	Autocorr
	
	
	
	

	
	Lag_window
	
	
	
	

	
	Levinson
	
	
	
	

	
	Az_isp
	Chebps2
	
	
	

	
	Int_isp
	Isp_Az
	Get_isp_pol
	
	

	
	Isp_isf
	
	
	
	

	
	Gp_clip_test_isf
	
	
	
	

	
	Weight_a
	
	
	
	

	
	Residu
	
	
	
	

	
	Deemph2
	
	
	
	

	
	LP_Decim2
	
	
	
	

	
	Scale_mem_Hp_wsp
	
	
	
	

	
	Pitch_med_ol
	Hp_wsp
	
	
	

	
	
	Isqrt_n
	
	
	

	
	wb_vad_tone_detection
	
	
	
	

	
	Med_olag
	median5
	
	
	

	
	dtx_buffer
	Copy
	
	
	

	
	dtx_enc
	Find_frame_indices
	
	
	

	
	
	Aver_isf_history
	
	
	

	
	
	Qisf_ns
	Sub_VQ
	
	

	
	
	
	Disf_ns
	Reorder_isf
	

	
	
	Parm_serial
	
	
	

	
	
	Pow2
	
	
	

	
	
	Random
	
	
	

	
	
	Dot_product12
	
	
	

	
	
	Isqrt_n
	
	
	

	
	Isf_isp
	
	
	
	

	
	Isp_Az
	Get_isp_pol
	
	
	

	
	Synthesis
	Copy
	
	
	

	
	
	Syn_filt_32
	
	
	

	
	
	Deemph_32
	
	
	

	
	
	HP50_12k8
	
	
	

	
	
	Random
	
	
	

	
	
	Scale_sig
	
	
	

	
	
	Dot_product12
	
	
	

	
	
	Isqrt_n
	
	
	

	
	
	HP400_12k8
	
	
	

	
	
	Weight_a
	
	
	

	
	
	Syn_filt
	
	
	

	
	
	Filt_6k_7k
	
	
	

	
	Reset_encoder
	Set_zero
	
	
	

	
	
	Init_gp_clip
	
	
	

	
	
	Init_Phase_dispersion
	Set_zero
	
	

	
	Qpisf_2s_36b
	VQ_stage1
	
	
	

	
	
	Sub_VQ
	
	
	

	
	
	Dpisf_2s_36b
	Reorder_isf
	
	

	
	Qpisf_2s_46b
	VQ_stage1
	
	
	

	
	
	Sub_VQ
	
	
	

	
	
	Dpisf_2s_46b
	Reorder_isf
	
	

	
	Syn_filt
	
	
	
	

	
	Preemph2
	
	
	
	

	
	Pitch_fr4
	Norm_Corr
	Convolve
	
	

	
	
	
	Isqrt_n
	
	

	
	
	Interpol_4
	
	
	

	
	Gp_clip
	
	
	
	

	
	Pred_lt4
	
	
	
	

	
	Convolve
	
	
	
	

	
	G_pitch
	Dot_product12
	
	
	

	
	Updt_tar
	
	
	
	

	
	Preemph
	
	
	
	

	
	Pit_shrp
	
	
	
	

	
	Cor_h_x
	
	
	
	

	
	ACELP_2t64_fx
	Dot_product12
	
	
	

	
	
	Isqrt_n
	
	
	

	
	ACELP_4t64_fx
	See Table 2
	
	
	

	
	Q_gain2
	Dot_product12
	
	
	

	
	
	Pow2
	
	
	

	
	Gp_clip_test_gain_pit
	
	
	
	

	
	voice_factor
	Dot_product12
	
	
	


Table C.2/G.722.2 – ACELP_4t64_fx call structure

	ACELP_4t64_fx
	Dot_product12
	
	
	
	

	
	Isqrt_n
	
	
	
	

	
	cor_h_vec
	
	
	
	

	
	search_ixiy
	
	
	
	

	
	quant_1p_N1
	
	
	
	

	
	quant_2p_2N1
	
	
	
	

	
	quant_3p_3N1
	quant_2p_2N1
	
	
	

	
	
	quant_1p_N1
	
	
	

	
	quant_4p_4N
	quant_4p_4N1
	Quant_2p_2N1
	
	

	
	
	quant_1p_N1
	
	
	

	
	
	quant_3p_3N1
	Quant_2p_2N1
	
	

	
	
	
	Quant_1p_N1
	
	

	
	
	quant_2p_2N1
	
	
	

	
	quant_5p_5N
	quant_3p_3N1
	Quant_2p_2N1
	
	

	
	
	
	Quant_1p_N1
	
	

	
	
	quant_2p_2N1
	
	
	

	
	quant_6p_6N_2
	quant_5p_5N
	Quant_3p_3N1
	quant_2p_2N1
	

	
	
	
	
	Quant_1p_N1
	

	
	
	
	quant_2p_2N1
	
	

	
	
	quant_1p_N1
	
	
	

	
	
	quant_4p_4N
	quant_4p_4N1
	quant_2p_2N1
	

	
	
	
	quant_1p_N1
	
	

	
	
	
	quant_3p_3N1
	quant_2p_2N1
	

	
	
	
	
	quant_1p_N1
	

	
	
	
	quant_2p_2N1
	
	

	
	
	quant_2p_2N1
	
	
	

	
	
	quant_3p_3N1
	quant_2p_2N1
	
	

	
	
	
	Quant_1p_N1
	
	


	Table C.3/G.722.2 – Speech decoder call structure

	decoder
	Rx_dtx_handler
	
	
	
	

	
	Dtx_dec
	Copy
	
	
	

	
	
	Disf_ns
	Reorder_isf
	
	

	
	
	Serial_parm
	
	
	

	
	
	Pow2
	
	
	

	
	
	Random
	
	
	

	
	
	Dot_product12
	
	
	

	
	
	Isqrt_n
	
	
	

	
	Serial_parm
	
	
	
	

	
	Isf_isp
	
	
	
	

	
	Isp_Az
	Get_isp_pol
	
	
	

	
	Copy
	
	
	
	

	
	Synthesis
	Copy
	
	
	

	
	
	Syn_filt_32
	
	
	

	
	
	Deemph_32
	
	
	

	
	
	HP50_12k8
	
	
	

	
	
	Oversamp_16k
	Copy
	
	

	
	
	
	Up_samp
	Interpol
	

	
	
	Random
	
	
	

	
	
	Scale_sig
	
	
	

	
	
	Dot_product12
	
	
	

	
	
	Isqrt_n
	
	
	

	
	
	HP400_12k8
	
	
	

	
	
	Isf_Extrapolation
	Isf_isp
	
	

	
	
	Isp_Az
	Get_isp_pol
	
	

	
	
	Weight_a
	
	
	

	
	
	Syn_filt
	
	
	

	
	
	Filt_6k_7k
	Copy
	
	

	
	
	Filt_7k
	Copy
	
	

	
	Reset_decoder
	Set_zero
	
	
	

	
	
	Init_Phase_dispersion
	Set_zero
	
	

	
	Dpisf_2s_36b
	Reorder_isf
	
	
	

	
	Dpisf_2s_46b
	Reorder_isf
	
	
	

	
	Int_isp
	Isp_Az
	Get_isp_pol
	
	

	
	Lagconc
	insertion_sort
	Insert
	
	

	
	
	Random
	
	
	

	
	Pred_lt4
	
	
	
	

	
	Random
	
	
	
	

	
	DEC_ACELP_2t64_fx
	
	
	
	

	
	DEC_ACELP_4t64_fx
	dec_1p_N1
	
	
	

	
	
	add_pulses
	
	
	

	
	
	dec_2p_2N1
	
	
	

	
	
	dec_3p_3N1
	Dec_2p_2N1
	
	

	
	
	
	dec_1p_N1
	
	

	
	
	dec_4p_4N
	dec_4p_4N1
	dec_2p_2N1
	

	
	
	
	dec_1p_N1
	
	

	
	
	
	Dec_3p_3N1
	Dec_2p_2N1
	

	
	
	
	
	Dec_1p_N1
	

	
	
	
	Dec_2p_2N1
	
	

	
	
	dec_5p_5N
	dec_3p_3N1
	Dec_2p_2N1
	

	
	
	
	
	Dec_1p_N1
	

	
	
	
	Dec_2p_2N1
	
	

	
	
	dec_6p_6N_2
	Dec_5p_5N
	dec_3p_3N1
	Dec_2p_2N1

	
	
	
	
	
	Dec_1p_N1

	
	
	
	
	dec_2p_2N1
	

	
	
	
	dec_1p_N1
	
	

	
	
	
	dec_4p_4N
	dec_4p_4N1
	dec_2p_2N1

	
	
	
	
	dec_1p_N1
	

	
	
	
	
	Dec_3p_3N1
	Dec_2p_2N1

	
	
	
	
	
	Dec_1p_N1

	
	
	
	
	Dec_2p_2N1
	

	
	
	
	dec_2p_2N1
	
	

	
	
	
	dec_3p_3N1
	Dec_2p_2N1
	

	
	
	
	
	Dec_1p_N1
	

	
	Preemph
	
	
	
	

	
	Pit_shrp
	
	
	
	

	
	D_gain2
	Dot_product12
	
	
	

	
	
	Isqrt_n
	
	
	

	
	
	Median5
	
	
	

	
	
	Pow2
	
	
	

	
	Scale_sig
	
	
	
	

	
	voice_factor
	Dot_product12
	
	
	

	
	Phase_dispersion
	Set_zero
	
	
	

	
	Agc2
	Isqrt
	Isqrt_n
	
	

	
	Set_zero
	
	
	
	

	
	Dtx_dec_activity_update
	Copy
	
	
	


C.1.4
Variables, constants and tables

The data types of variables and tables used in the fixed point implementation are signed integers in 2's complement representation, defined by:

–
Word16
16-bit variable;

–
Word32
32-bit variable.

C.1.4.1
Description of constants used in the C-code

This subclause contains a listing of all global constants defined in cnst.h. See Table C.4.

Table C.4/G.722.2 – Global constants

	Constant
	Value
	Description

	L_TOTAL
	384
	total size of speech buffer

	L_WINDOW
	384
	window size in LP analysis

	L_NEXT
	64
	Look-ahead size

	L_FRAME
	256
	frame size in 12.8 kHz

	L_FRAME16k
	320
	frame size in 16 kHz

	L_SUBFR
	64
	Subframe size in 12.8 kHz

	L_SUBFR16k
	80
	Subframe size in 16 kHz

	NB_SUBFR
	4
	Number of subframes

	M16k
	20
	order of LP filter in high-band synthesis in 6.60 mode

	M
	16
	order of LP filter

	L_FILT16k
	15
	Delay of down-sampling filter in 16 kHz

	L_FILT
	12
	Delay of down-sampling filter in 12.8 kHz

	GP_CLIP
	15565
	Pitch gain clipping

	PIT_SHARP
	27853
	pitch sharpening factor

	PIT_MIN
	34
	minimum pitch lag (all modes)

	PIT_FR2
	128
	Minimum pitch lag with resolution ½

	PIT_FR1_9b
	160
	Minimum pitch lag with resolution for 9 bit quantization

	PIT_FR1_8b
	92
	Minimum pitch lag with resolution for 8 bit quantization

	PIT_MAX
	231
	maximum pitch lag

	L_INTERPOL
	(16+1)
	length of filter for interpolation

	OPL_DECIM
	2
	Decimation in open-loop pitch analysis

	PREEMPH_FAC
	22282
	preemphasis factor

	GAMMA1
	30147
	Weighting factor (numerator)

	TILT_FAC
	22282
	tilt factor (denominator)

	Q_MAX
	8
	scaling max for signal

	RANDOM_INITSEED
	21845
	random init value

	L_MEANBUF
	3
	Size of ISF buffer

	ONE_PER_MEANBUF
	10923
	Inverse of L_MEANBUF


C.1.4.2
Description of fixed tables used in the C-code

This subclause contains a listing of all fixed tables sorted by source file name and table name. All table data is declared as Word16. See Table C.5.

Table C.5/G.722.2 – Fixed tables

	File
	Table name
	Length
	Description

	C4t64fx.c
	Tipos
	36
	starting points of iterations

	Cod_main.c
	HP_gain
	16
	High band gain table for 23.85 kbit/s mode

	Cod_main.c
	Interpol_frac
	4
	LPC interpolation coefficients

	Cod_main.c
	Isp_init
	16
	isp tables for initialization

	Cod_main.c
	Isf_init
	16
	isf tables for initialization

	D_gain2.c
	cdown_unusable
	7
	attenuation factors for codebook gain in lost frames

	D_gain2.c
	cdown_usable
	7
	attenuation factors for codebook gain in bad frames

	D_gain2.c
	pdown_unusable
	7
	attenuation factors for adaptive codebook gain in lost frames

	D_gain2.c
	pdown_usable
	7
	attenuation factors for adaptive codebook gain in bad frames

	D_gain2.c
	Pred
	4
	algebraic code book gain MA predictor coefficients

	Dec_main.c
	HP_gain
	16
	High band gain table for 23.85 kbit/s mode

	Dec_main.c
	Interpol_frac
	4
	LPC interpolation coefficients

	Dec_main.c
	Isp_init
	16
	isp tables for initialization

	Dec_main.c
	Isf_init
	16
	isf tables for initialization

	Decim54.c
	fir_down
	120
	Downsample FIR filter coefficients

	Decim54.c
	fir_up
	120
	Upsample FIR filter coefficients

	Dtx.c
	en_adjust
	9
	Energy scaling factor for each mode during comfort noise

	Grid100.tab
	grid
	101
	grid points at wich Chebyshev polynomials

	Ham_wind.tab
	Window
	384
	LP analysis window

	Hp400.c
	A
	3
	HP filter coefficients (denominator) in higher band energy estimation

	Hp400.c
	B
	3
	HP filter coefficients (numerator) in higher band energy estimation

	Hp50.c
	A
	3
	HP filter coefficients (denominator) in pre-filtering

	Hp50.c
	B
	3
	HP filter coefficients (numerator) in pre-filtering

	Hp6k.c
	Fir_6k_7k
	31
	Bandpass FIR filter coefficients for higher band generation

	Hp7k.c
	Fir_7k
	31
	Bandpass FIR filter coefficients for higher band in 23.85 kbit/s mode

	Hp_wsp.c
	A
	3
	HP filter coefficients (denominator) in open-loop lag gain computation

	Hp_wsp.c
	B
	3
	HP filter coefficients (numerator) in open-loop lag gain computation

	Isp_isf.tab
	slope
	128
	table to compute cos(x) in Lsf_lsp()

	Isp_isf.tab
	Table
	129
	table to compute acos(x) in Lsp_lsf()

	Lag_wind.tab
	lag_h
	16
	high part of the lag window table

	Lag_wind.tab
	lag_l
	16
	low part of the lag window table

	Lp_dec2.c
	h_fir
	5
	HP FIR filter coefficients in open-loop lag search

	Math_op.c
	table_isqrt
	49
	table used in inverse square root computation

	Math_op.c
	table_pow2
	33
	table used in power of two computation

	P_med_ol.tab
	Corrweight
	199
	weighting of the correlation function in open loop LTP search

	Ph_disp.c
	ph_imp_low
	64
	phase dispersion impulse response

	Ph_disp.c
	ph_imp_mid
	64
	phase dispersion impulse response

	Pitch_fr4.c
	inter4_1
	32
	interpolation filter coefficients

	Pred_lt4.c
	inter4_2
	128
	interpolation filter coefficients

	Q_gain2.c
	pred
	4
	algebraic code book gain MA predictor coefficients

	Q_gain2.tab
	t_qua_gain6b
	2*64
	gain quantization table for 6-bit gain quantization

	Q_gain2.tab
	t_qua_gain7b
	2*128
	gain quantization table for 7-bit gain quantization

	Qisf_ns.tab
	dico1_isf_noise
	2*64
	1st ISF quantizer for comfort noise

	Qisf_ns.tab
	dico2_isf_noise
	3*64
	2nd ISF quantizer for comfort noise

	Qisf_ns.tab
	Dico3_isf_noise
	3*64
	3rd LSF quantizer for comfort noise

	Qisf_ns.tab
	Dico4_isf_noise
	4*32
	4th LSF quantizer for comfort noise

	Qisf_ns.tab
	Dico5_isf_noise
	4*32
	5th LSF quantizer for comfort noise

	Qisf_ns.tab
	mean_isf_noise
	16
	ISF mean for comfort noise

	Qpisf_2s.tab
	dico1_isf
	9*256
	1st ISF quantizer of the 1st stage

	Qpisf_2s.tab
	Dico2_isf
	7*256
	2nd ISF quantizer of the 1st stage

	Qpisf_2s.tab
	Dico21_isf
	3*64
	1st ISF quantizer of the 2nd stage (not the 6.60 kbit/s mode)

	Qpisf_2s.tab
	Dico21_isf_36b
	5*128
	1st ISF quantizer of the 2nd stage (the 6.60 kbit/s mode)

	Qpisf_2s.tab
	Dico22_isf
	3*128
	2nd ISF quantizer of the 2nd stage (not the 6.60 kbit/s mode)

	Qpisf_2s.tab
	Dico22_isf_36b
	4*128
	2nd ISF quantizer of the 2nd stage (the 6.60 kbit/s mode)

	Qpisf_2s.tab
	Dico23_isf
	3*128
	3rd ISF quantizer of the 2nd stage (not the 6.60 kbit/s mode)

	Qpisf_2s.tab
	Dico23_isf_36b
	7*64
	3rd ISF quantizer of the 2nd stage (the 6.60 kbit/s mode)

	Qpisf_2s.tab
	Dico24_isf
	3*32
	4th ISF quantizer of the 2nd stage (not the 6.60 kbit/s mode)

	Qpisf_2s.tab
	Dico25_isf
	4*32
	5th ISF quantizer of the 2nd stage (not the 6.60 kbit/s mode)

	Qpisf_2s.tab
	Mean_isf
	16
	ISF mean


C.1.4.3
Static variables used in the C-code

In this section two tables that specify the static variables for the speech encoder and decoder respectively are shown. All static variables are declared within a C struct. See Tables C.6 and C.7.
	Table C.6/G.722.2 – Speech encoder static variables

	Struct name
	Variable
	Type[Length]
	Description

	Coder_State
	mem_decim
	Word16[30]
	Decimation filter memory

	
	mem_sig_in
	Word16[6]
	Prefilter memory

	
	mem_preemph
	Word16
	Preemphasis filter memory

	
	old_speech
	Word16[128]
	speech buffer

	
	old_wsp
	Word16[115]
	buffer holding spectral weighted speech

	
	old_exc
	Word16[248]
	excitation vector

	
	mem_levinson
	Word16[18]
	Levinson memories

	
	Ispold
	Word16[16]
	Old ISP vector

	
	ispold_q
	Word16[16]
	Old quantized ISP vector

	
	past_isfq
	Word16[16]
	past quantized ISF prediction error

	
	mem_wsp
	Word16
	Open-loop LTP deemphasis filter memory

	
	mem_decim2
	Word16[3]
	Open-loop LTP decimation filter memory

	
	mem_w0
	Word16
	weighting filter memory (applied to error signal)

	
	mem_syn
	Word16[16]
	synthesis filter memory

	
	tilt_code
	Word16
	Preemhasis filter memory

	
	old_wsp_max
	Word16
	Open loop scaling factor

	
	old_wsp_shift
	Word16
	Maximum open loop scaling factor

	
	Q_old
	Word16
	Old scaling factor

	
	Q_max
	Word16[2]
	Maximum scaling factor

	
	gp_clip
	Word16[2]
	memory of pitch clipping

	
	qua_gain
	Word16[4]
	Gain quantization memory

	
	old_T0_med
	Word16
	weighted open loop pitch lag

	
	ol_gain
	Word16
	Open-loop gain

	
	ada_w
	Word16
	weigthing level depeding on open loop pitch gain

	
	ol_wght_flg
	Word16
	switches lag weighting on and off

	
	old_ol_lag
	Word16[5]
	Open loop lag history

	
	hp_wsp_mem
	Word16[9]
	Open-loop lag gain filter memory

	
	old_hp_wsp
	Word16[243]
	Open-loop lag 

	
	vadSt
	VadVars*
	see below in this table

	
	dtx_encSt
	dtx_encState*
	see below in this table

	
	first_frame
	Word16
	First frame indicator

	
	Isfold
	Word16[16]
	Old ISF vector

	
	L_gc_thres
	Word16
	Noise enhancer threshold

	
	mem_syn_hi
	Word16[16]
	synthesis filter memory (most significant word)

	
	mem_syn_lo
	Word16[16]
	synthesis filter memory (least significant word)

	
	mem_deemph
	Word16
	Deemphasis filter memory

	
	mem_sig_out
	Word16[6]
	HP filter memory in the synthesis

	
	mem_hp400
	Word16[6]
	HP filter memory

	
	mem_oversamp
	Word16[2*12]
	Oversampling filter memory

	
	mem_syn_hf
	Word16[16]
	Higher band synthesis filter memory

	
	mem_hf
	Word16[30]
	Estimated BP filter memory (23.85 kbit/s mode)

	
	mem_hf2
	Word16[30]
	Input BP filter memory (23.85 kbit/s mode)

	
	mem_hf3
	Word16[30]
	Input LP filter memory (23.85 kbit/s mode)

	
	seed2
	Word16
	Random generation seed

	
	disp_mem
	Word16[8]
	Phase dispersion memory

	
	vad_hist
	Word16
	VAD history

	
	Gain_alpha
	Word16
	Higher band gain weighting factor (23.85 kbit/s mode)

	dtx_encState
	Isf_hist
	Word16[128]
	LSP history (8 frames)

	
	Log_en_hist
	Word16[8]
	logarithmic frame energy history (8 frames)

	
	Hist_ptr
	Word16
	pointer to the cyclic history vectors

	
	Log_en_index
	Word16
	Index for logarithmic energy

	
	Cng_seed
	Word16
	Comfort noise excitation seed

	
	D
	Word16[28]
	ISF history distance matrix

	
	sumD
	Word16[8]
	Sum of ISF history distances

	
	dtxHangoverCount
	Word16
	is decreased in DTX hangover period

	
	decAnaElapsedCount
	Word16
	counter for elapsed speech frames in DTX

	vadState1
	bckr_est
	Word16[12]
	background noise estimate

	
	ave_level
	Word16[12]
	averaged input components for stationary estimation

	
	old_level
	Word16[12]
	input levels of the previous frame

	
	sub_level
	Word16[12]
	input levels calculated at the end of a frame (lookahead)

	
	a_data5
	Word16[5][2]
	memory for the filter bank

	
	a_data3
	Word16[6]
	memory for the filter bank

	
	burst_count
	Word16
	counts length of a speech burst

	
	Hang_count
	Word16
	hangover counter

	
	Stat_count
	Word16
	stationary counter

	
	Vadreg
	Word16
	15 flags for intermediate VAD decisions

	
	Tone_flag
	Word16
	15 flags for tone detection

	
	sp_est_cnt
	Word16
	Speech level estimation counter

	
	Sp_max
	Word16
	Maximum signal level

	
	sp_max_cnt
	Word16
	Maximum level estimation counter

	
	Speech_level
	Word16
	Speech level

	
	prev_pow_sum
	Word16
	Power of previous frame


	Table C.7/G.7222.2 – Speech decoder static variables

	Struct name
	Variable
	Type[Length]
	Description

	Decoder_State
	old_exc
	Word16[248]
	excitation vector

	
	ispold
	Word16[16]
	Old ISP vector

	
	isfold
	Word16[16]
	Old ISF vector

	
	isf_buf
	Word16[48]
	ISF vector history

	
	past_isfq
	Word16[16]
	past quantized ISF prediction error

	
	tilt_code
	Word16
	Preemhasis filter memory

	
	Q_old
	Word16
	Old scaling factor

	
	Qsubfr
	Word16
	Scaling factor history

	
	L_gc_thres
	Word16
	Noise enhancer threshold

	
	mem_syn_hi
	Word16[16]
	synthesis filter memory (most significant word)

	
	mem_syn_lo
	Word16[16]
	synthesis filter memory (least significant word)

	
	mem_deemph
	Word16
	Deemphasis filter memory

	
	mem_sig_out
	Word16[6]
	HP filter memory in the synthesis

	
	mem_oversamp
	Word16[24]
	Oversampling filter memory

	
	mem_syn_hf
	Word16[20]
	Higher band synthesis filter memory

	
	mem_hf
	Word16[30]
	Estimated BP filter memory (23.85 kbit/s mode)

	
	mem_hf2
	Word16[30]
	Input BP filter memory (23.85 kbit/s mode)

	
	mem_hf3
	Word16[30]
	Input LP filter memory (23.85 kbit/s mode)

	
	seed
	Word16
	Random code generation seed for bad frames

	
	seed2
	Word16
	Random generation seed for higher band

	
	old_T0
	Word16
	Old LTP lag (integer part)

	
	old_T0_frac
	Word16
	Old LTP lag (fraction part)

	
	lag_hist
	Word16[5]
	LTP lag history

	
	dec_gain
	Word16[23]
	Gain decoding memory

	
	seed3
	Word16
	Random LTP lag generation seed for bad frames 

	
	disp_mem
	Word16[8]
	Phase dispersion memory

	
	mem_hp400
	Word16[6]
	HP filter memory

	
	prev_bfi
	Word16
	Previous BFI

	
	state
	Word16
	BGH state machine memory

	
	first_frame
	Word16
	First frame indicator

	
	dtx_decSt
	dtx_decState*
	see below in this table

	
	Vad_hist
	Word16
	VAD history

	dtx_decState
	Since_last_sid
	Word16
	number of frames since last SID frame

	
	true_sid_period_inv
	Word16
	inverse of true SID update rate

	
	log_en
	Word16
	logarithmic frame energy

	
	old_log_en
	Word16
	previous value of log_en

	
	isf
	Word16[16]
	ISF vector

	
	Isf_old
	Word16[16]
	Previous ISF vector

	
	Cng_seed
	Word16
	Comfort noise excitation seed

	
	Isf_hist
	Word16[128]
	ISF vector history (8 frames)

	
	Log_en_hist
	Word16[8]
	logarithmic frame energy history

	
	Hist_ptr
	Word16
	index to beginning of LSF history

	
	dtxHangoverCount
	Word16
	counts down in hangover period

	
	DecAnaElapsedCount
	Word16
	counts elapsed speech frames after DTX

	
	sid_frame
	Word16
	flags SID frames

	
	valid_data
	Word16
	flags SID frames containing valid data

	
	log_en_adjust
	Word16
	mode-dependent frame energy adjustment

	
	dtxHangoverAdded
	Word16
	flags hangover period at end of speech

	
	dtxGlobalState
	Word16
	DTX state flags

	
	data_updated
	Word16
	flags CNI updates


C.2
Homing procedure 

The principles of the homing procedures are described in the main body of this Recommendation. This clause only includes a detailed description of the 9 decoder homing frames. For each AMR-WB codec mode, the corresponding decoder homing frame has a fixed set of parameters. The parameters in serial format are packed into parameters in 15-bit-long format where the first serial bit is inserted into most significant bit in the 15-bit-long format. These 15-bit-long parameters do not represent real speech parameters, but they decrease memory consumption compared to the speech parameters. Table C.8 shows the homing frame in 15-bit-long format for different modes. In the decoder, the received speech parameters in serial format are first converted into 15-bit-long format. Then the obtained parameters are compared against the homing frame table values (Table C.8).

Table C.8/G.722.2 – Table values for the decoder homing frame
in 15-bit-long format for different modes

	Mode
	Value (MSB=b0)

	0
	25351, 4331, 515, 15620, 20992, 0, 0, 0, 0

	1
	25351, 14010, 26489, 30912, 5254, 3459, 0, 0, 0, 0, 0, 0

	2
	25351, 14010, 29177, 18070, 19971, 3968, 32492, 8430, 13280, 0, 0, 0, 0, 0, 0, 0, 0

	3
	25351, 14010, 1912, 16326, 25140, 16384, 502, 15167, 1772, 11512, 0, 0, 0, 0, 0, 0, 0, 0, 0

	4
	25351, 14010, 1912, 30593, 14594, 19990, 864, 4635, 20446, 27456, 21310, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0

	5
	25351, 14010, 19995, 14446, 6159, 7329, 20752, 4228, 19488, 24383, 364, 20124, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0

	6
	25351, 14010, 3567, 560, 32536, 20534, 5139, 16384, 26161, 18755, 20444, 22173,12623, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0

	7
	25351, 14010, 2912, 28827, 15347, 28610, 9853, 1316, 30720, 786, 32259, 13279,14336, 29152, 23302, 20352, 0, 0, 0, 0, 0, 0, 0, 0,0, 0, 0, 0, 0, 0, 0

	8
	025351, 14010, 1601, 16734, 7923, 15017, 5450, 5477, 5760, 2187, 1534, 12142, 30894, 13419, 13141, 2376, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0


C.3
File formats

This clause describes the file formats used by the encoder and decoder programs. The test sequences also use the file formats described here.

C.3.1
Speech file (encoder input/decoder output)

Speech files read by the encoder and written by the decoder consist of 16-bit words where each word contains a 14-bit, left aligned speech sample. The byte order depends on the host architecture (e.g. MSByte first on SUN workstations, LSByte first on PCs, etc.). Both the encoder and the decoder program process complete frames (of 320 samples) only.

This means that the encoder will only process n frames if the length of the input file is n*320 + k words, while the files produced by the decoder will always have a length of n*320 words.

C.3.2
Mode control file (encoder input)

The encoder program can optionally read in a mode control file which specifies the encoding mode for each frame of speech processed. The file is a text file containing one number per speech frame. Each line contains one of the mode numbers 0-8.

C.3.3
Parameter bitstream file (encoder output/decoder input)

The files produced by the speech encoder/expected by the speech decoder contain an arbitrary number of frames in the following format.

	TYPE_OF_FRAME_TYPE
	FRAME_TYPE
	MODE
	B1
	B2
	…
	Bnn


Each box corresponds to one Word16 value in the bitstream file, for a total of 3+nn words or 6+2nn bytes per frame, where nn is the number of encoded bits in the frame. The fields have the following meaning:

TYPE_OF_FRAME_TYPE
transmit frame type, which is one of 


TX_TYPE
(0x6b21)
RX_TYPE
(0x6b20)

If TYPE_OF_FRAME_TYPE is TX_TYPE,

FRAME_TYPE
transmit frame type, which is one of 


TX_SPEECH
(0x0000)
TX_SID_FIRST
(0x0001)
TX_SID_UPDATE
(0x0002)
TX_NO_DATA
(0x0003) 

If TYPE_OF_FRAME_TYPE is RX_TYPE,

FRAME_TYPE
transmit frame type, which is one of 


RX_SPEECH_GOOD
(0x0000)
RX_SPEECH_PROBABLY_DEGRADED
(0x0001)
RX_SPEECH_LOST
(0x0002)
RX_SPEECH_BAD
(0x0003)
RX_SID_FIRST
(0x0004)
RX_SID_UPDATE
(0x0005)
RX_SID_BAD
(0x0006)
RX_NO_DATA
(0x0007)

B0…B2nn
speech encoder parameter bits (i.e. the bitstream itself). Each Bx either has the value 0x0081 (for bit 0) or 0x007F (for bit 1).

MODE_INFO
encoding mode information, which is one of


6.60  kbit/s mode
(0x0000)
8.85  kbit/s mode
(0x0001)
12.65 kbit/s mode
(0x0002)
14.25 kbit/s mode
(0x0003)
15.85 kbit/s mode
(0x0004)
18.25 kbit/s mode
(0x0005)
19.85 kbit/s mode
(0x0006)
23.05 kbit/s mode
(0x0007)
23.85 kbit/s mode
(0x0008)

As indicated in C.3.1 above, the byte order depends on the host architecture.

	SERIES OF ITU-T RECOMMENDATIONS

	Series A
	Organization of the work of ITU-T

	Series B
	Means of expression: definitions, symbols, classification

	Series C
	General telecommunication statistics

	Series D
	General tariff principles

	Series E
	Overall network operation, telephone service, service operation and human factors

	Series F
	Non-telephone telecommunication services

	Series G
	Transmission systems and media, digital systems and networks

	Series H
	Audiovisual and multimedia systems

	Series I
	Integrated services digital network

	Series J
	Cable networks and transmission of television, sound programme and other multimedia signals

	Series K
	Protection against interference

	Series L
	Construction, installation and protection of cables and other elements of outside plant

	Series M
	TMN and network maintenance: international transmission systems, telephone circuits, telegraphy, facsimile and leased circuits

	Series N
	Maintenance: international sound programme and television transmission circuits

	Series O
	Specifications of measuring equipment

	Series P
	Telephone transmission quality, telephone installations, local line networks

	Series Q
	Switching and signalling

	Series R
	Telegraph transmission

	Series S
	Telegraph services terminal equipment

	Series T
	Terminals for telematic services

	Series U
	Telegraph switching

	Series V
	Data communication over the telephone network

	Series X
	Data networks and open system communications

	Series Y
	Global information infrastructure and Internet protocol aspects

	Series Z
	Languages and general software aspects for telecommunication systems

	
	


*22233*

�	This annex includes an electronic attachment containing the C code of the G.722.2 AMR-WB speech transcoder.





