
	[image: image1.wmf]
	INTERNATIONAL  TELECOMMUNICATION  UNION


	ITU-T
	G.729

	TELECOMMUNICATION
STANDARDIZATION  SECTOR
OF  ITU
	Annex G

(02/00)   


SERIES G: TRANSMISSION SYSTEMS AND MEDIA, DIGITAL SYSTEMS AND NETWORKS

Digital transmission systems – Terminal equipments – Coding of analogue signals by methods other than PCM

Coding of speech at 8 kbit/s using Conjugate Structure Algebraic Code-Excited Linear Prediction (CS-ACELP)

Annex G: Reference implementation of G.729 Annex B DTX functionality for Annex E

ITU-T  Recommendation  G.729  –  Annex G

(Previously  CCITT  Recommendation)
ITU-T  G-SERIES  RECOMMENDATIONS

TRANSMISSION SYSTEMS AND MEDIA, DIGITAL SYSTEMS AND NETWORKS

	
	

	INTERNATIONAL TELEPHONE CONNECTIONS AND CIRCUITS
	G.100–G.199

	INTERNATIONAL ANALOGUE CARRIER SYSTEM

	GENERAL CHARACTERISTICS COMMON TO ALL ANALOGUE CARRIER-TRANSMISSION SYSTEMS
	G.200–G.299

	INDIVIDUAL CHARACTERISTICS OF INTERNATIONAL CARRIER TELEPHONE SYSTEMS ON METALLIC LINES
	G.300–G.399

	GENERAL CHARACTERISTICS OF INTERNATIONAL CARRIER TELEPHONE SYSTEMS ON RADIO-RELAY OR SATELLITE LINKS AND INTERCONNECTION WITH METALLIC LINES
	G.400–G.449

	COORDINATION OF RADIOTELEPHONY AND LINE TELEPHONY
	G.450–G.499

	TESTING EQUIPMENTS

	TRANSMISSION MEDIA CHARACTERISTICS
	G.600–G.699

	DIGITAL TRANSMISSION SYSTEMS

	TERMINAL EQUIPMENTS
	G.700–G.799

	General
	G.700–G.709

	Coding of analogue signals by pulse code modulation
	G.710–G.719

	Coding of analogue signals by methods other than PCM
	G.720–G.729

	Principal characteristics of primary multiplex equipment
	G.730–G.739

	Principal characteristics of second order multiplex equipment
	G.740–G.749

	Principal characteristics of higher order multiplex equipment
	G.750–G.759

	Principal characteristics of transcoder and digital multiplication equipment
	G.760–G.769

	Operations, administration and maintenance features of transmission equipment
	G.770–G.779

	Principal characteristics of multiplexing equipment for the synchronous digital hierarchy
	G.780–G.789

	Other terminal equipment
	G.790–G.799

	DIGITAL NETWORKS
	G.800–G.899

	DIGITAL SECTIONS AND DIGITAL LINE SYSTEM
	G.900–G.999

	
	


For further details, please refer to ITU-T List of Recommendations.

	ITU-T  RECOMMENDATION  G.729

coding of speech at 8 kbit/s using conjugate structure algebraic code-excited linear prediction (CS-ACELP)

Annex G

Reference implementation of G.729 Annex B DTX functionality for Annex E


	Summary

This annex provides the DTX functionality for the 11.8 kbit/s CS-ACELP algorithm of G.729 Annex E using the basic algorithm in G.729 Annex B.

This annex includes an electronic attachment containing version 1.1 of reference C code and test vectors fixed-point implementation of CS-ACELP at 8 kbit/s and 11.8 kbit/s with DTX functionality.


	Source
Annex G to ITU-T Recommendation G.729 was prepared by ITU-T Study Group 16 (1997-2000) and was approved under the WTSC Resolution No. 1 procedure on 17 February 2000.


	


FOREWORD

ITU (International Telecommunication Union) is the United Nations Specialized Agency in the field of telecommunications. The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of the ITU. The ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Conference (WTSC), which meets every four years, establishes the topics for study by the ITU‑T Study Groups which, in their turn, produce Recommendations on these topics.

The approval of Recommendations by the Members of the ITU‑T is covered by the procedure laid down in WTSC Resolution No. 1.

In some areas of information technology which fall within ITU-T’s purview, the necessary standards are prepared on a collaborative basis with ISO and IEC.

NOTE

In this Recommendation, the expression "Administration" is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.
INTELLECTUAL PROPERTY RIGHTS
The ITU draws attention to the possibility that the practice or implementation of this Recommendation may involve the use of a claimed Intellectual Property Right. The ITU takes no position concerning the evidence, validity or applicability of claimed Intellectual Property Rights, whether asserted by ITU members or others outside of the Recommendation development process.

As of the date of approval of this Recommendation, the ITU had received notice of intellectual property, protected by patents, which may be required to implement this Recommendation. However, implementors are cautioned that this may not represent the latest information and are therefore strongly urged to consult the TSB patent database.

(  ITU  2000

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

CONTENTS


Page

Annex G – Reference implementation of G.729 Annex B DTX functionality for Annex E

1
G.1
Scope

1
G.2
Normative references

1
G.3
Overview

1
G.4
New functionality

2
G.4.1
Annex B DTX operation with Annex E

2
G.5
Algorithm Description

2
G.5.1
Music detection

2
G.5.2
Update of state variables specific to Annex E during discontinued transmission

6
G.6
Description of C source code

7
G.6.1
Use of the simulation software

7
G.6.2
Organization of the simulation software

7
Electronic attachment:

–
reference C code implementation

–
test vectors

Recommendation G.729

coding of speech at 8 kbit/s using conjugate structure algebraic code-excited linear prediction (CS-ACELP)

Annex G

Reference implementation of G.729 Annex B DTX functionality for Annex E

(Geneva, 2000)

G.1
Scope

This annex provides a description of integrating the G.729 Annexes B and E, hereby defining DTX functionality for Annex E. It presents a standard way of performing this integration and expansion of the functionality thereby guiding the industry and ensuring a standard speech quality and compatibility worldwide. The integration has been performed with focus on several constraints in order to satisfy the need of the industry:

1)
Bit-exactness with the main body and individual annexes.

2)
Minimum additional program code, memory, and complexity usage.

3)
Stringent quality requirements to new functionality inline with quality and application areas of the according standard annexes.

G.2
Normative references

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; all users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

[1]
ITU-T Recommendation G.729 (1996), Coding of speech at 8 kbit/s using conjugate structure algebraic code-excited linear prediction (CS-ACELP).

[2]
ITU-T Recommendation G.729 Annex B (1996), A silence compression scheme for G.729 optimized for terminals conforming to Recommendation V.70.

[3]
ITU-T Recommendation G.729 Annex E (1998), 11.8 kbit/s CS-ACELP speech coding algorithm.

G.3
Overview

Recommendation G.729 main body [1] and Annexes B [2] and E [3] provide a bit-exact, fixed-point specification of a CS-ACELP coder at 8 kbit/s, with DTX functionality and higher bit-rate extension capability at 11.8 kbit/s. Exact details of these specifications are given in bit-exact, fixed-point C code in an electronic file attached to this annex. This annex describes and defines the integration of the G.729 Annexes B and E.

G.4
New functionality
This subclause presents a brief overview of the modifications/additions to the algorithms in order to facilitate the integration of Annexes B and E. Also certain additions have been found necessary in order to accommodate the application area of the different modules.

G.4.1
Annex B DTX operation with Annex E

Integrating Annexes B and E functionality in order to provide DTX operation with Annex E requires certain considerations. Since the DTX operation of Annex B is based on the 10th order LPC analysis, the VAD function of Annex B is performed after the 10th order forward adaptive LPC analysis and before the backward adaptive LPC analysis of Annex E. In case the VAD function detects "non-speech" the LPC mode of Annex E is forced to forward adaptive LPC and the backward adaptive LPC analysis is skipped. Furthermore, it has been found necessary to add a correctional module after the VAD in order to detect music and accommodate the somewhat expanded application area of Annex E – one of the purposes of Annex E is to provide transmission capability of music with a certain quality. Accordingly, during the development of Annex E there were strict requirements to the performance with music signals. On the other hand, for the main body and Annexes B and D, there were no strict requirements to the performance with music signals. In order to guarantee the quality with music signals of Annex E during Annex B DTX operation, the music detection function forces the VAD to "speech" during music segments, hereby ensuring that the music segments are coded with the 11.8 kbit/s of Annex E. The SID coding and the CNG of Annex B are reused without any modifications. Furthermore, care is taken to appropriately update the parameters of the LPC mode selection algorithm of Annex E during discontinued transmission (see G.5.2).

G.5
Algorithm Description

This subclause presents the algorithm description of the necessary additions to the algorithms of the individual annexes in order to facilitate the integration. All remaining modules originate from the main body, Annex B, or E.

G.5.1
Music detection

The music detection is a new function. It is performed immediately following the VAD and forces the VAD to "speech" during music segments.

The music detection algorithm corrects the decision from the Voice Activity Detection (VAD) in the presence of music signals. The music detection is based on the following parameters:

•
Vad_dec: VAD decision of the current frame.

•
Vad_deci: VAD decision of the previous frame.

•
Lpc_mod: flag indicator of either forward or backward adaptive LPC of the previous frame.

•
Rc: reflection coefficients from LPC analysis.

•
Lag_buf: buffer of corrected open loop pitch lags of last 5 frames.

•
Pgain_buf: buffer of closed loop pitch gain of last 5 subframes.

•
Energy: first autocorrelation coefficient [image: image2.wmf])

0

(

R

from LPC analysis.

•
LLenergy: normalized log energy from VAD module.

•
Frm_count: counter of the number of processed signal frames.

•
Rate: selection of speech coder.

The algorithm has two main parts:

1)
Computation of relevant parameters.

2)
Classification based on parameters.

G.5.1.1
Computation of Relevant Parameters

This subclause describes the computation of the parameters used by the decision module.

Partial Normalized Residual Energy



[image: image3.wmf]ú

ú

û

ù

ê

ê

ë

é

-

=

Õ

=

240

)

)

(

1

(

log

10

4

1

2

10

Energy

i

Rc

Lenergy

i


Spectral Difference and Running Mean of Partial Normalized Residual Energy of Background Noise

A spectral difference measure between the current frame reflection coefficients [image: image4.wmf]Rc

and the running mean reflection coefficients of the background noise mRc  is given by:



[image: image5.wmf]å

=

-

=

10

1

2

))

(

)

(

(

i

i

mRc

i

Rc

SD


The running means [image: image6.wmf]mrc

 and [image: image7.wmf]mLenergy

 are updated as follows using the VAD decision Vad_deci that was generated by the VAD module.


[image: image8.wmf]}

1

.

0

9

.

0

    

1

.

0

9

.

0

     

{

_

 

if

Lenergy

mLenergy

mLenergy

rc

mrc

mrc

NOISE

deci

Vad

+

=

+

=

==


Open loop Pitch Lag Correction for Pitch Lag Buffer Update

The open loop pitch lag Top is corrected to prevent pitch doubling or tripling as follows:


[image: image9.wmf]å

=

=

4

1

4

)

(

_

_

i

i

buf

Lag

lag

avg



[image: image10.wmf]op

op

op

op

op

T

)

Lag_buf(

T

buf

Lag

lag

avg

T

abs

T

buf

Lag

lag

avg

T

abs

=

=

ú

ú

û

ù

ê

ê

ë

é

<=

ú

û

ù

ê

ë

é

-

=

ú

ú

û

ù

ê

ê

ë

é

<=

ú

û

ù

ê

ë

é

-

5

   

else

3

)

5

(

_

2

_

3

 

if

 

else

2

)

5

(

_

2

_

2

 

if


It should be noted that the open loop pitch lag Top is not modified and is the same as derived by the open loop analysis.

Pitch Lag Standard Deviation



[image: image11.wmf]4

Var

std

=


where:



[image: image12.wmf]å

=

=

-

=

5

1

2

)

μ

)

(

_

(

i

i

i

buf

Lag

Var

 and [image: image13.wmf]å

=

=

ú

û

ù

ê

ë

é

=

5

1

5

)

(

_

μ

i

i

i

buf

Lag


Running Mean of Pitch Gain

[image: image14.wmf]θ,

2

.

0

8

.

0

+

=

mPgain

mPgain

 where [image: image15.wmf]å

=

=

ú

û

ù

ê

ë

é

=

5

1

5

)

(

_

θ

i

i

i

buf

Pgain


The pitch gain buffer Pgain_buf is updated after the subframe processing with a pitch gain value of 0.5 if Vad_deci = NOISE, and otherwise with the quantized pitch gain.

Pitch Lag Smoothness and Voicing Strength Indicator

A pitch lag smoothness and voicing strength indicator Pflag is generated using the following logical steps:

First, two intermediary logical flags Pflag1 and Pflag2 are obtained as:


[image: image16.wmf]0

 

else

 

1

1

set 

 

)

45

.

0

 

and

 

3

.

1

(

 

if

=

>

<

Pflag

mPgain

std



[image: image17.wmf]63

.

0

 

where

 

0,

 

else

 

1

2

set 

 

)

(

 

if

=

=

>

Thres

Pflag

Thres

mPgain


Finally, Pflag is determined from the following:


[image: image18.wmf]0

 

else

 

1

set 

)

1

2

(

or 

 

))

1

2

or 

 

1

1

(

 

and

 

_

((

 

if

=

==

==

==

==

Pflag

Pflag

Pflag

Pflag

VOICE

dec

PVad


Stationarity Counters

A set of counters are defined and updated as follows:

a)
[image: image19.wmf]rflag

consc

count

_

_

tracks the number of consecutive frames where the 2nd reflection coefficient and the running mean of the pitch gain satisfy the following condition:


[image: image20.wmf]0

_

_

     

else

1

_

_

_

_

     

)

5

.

0

 

and

 

0

)

2

(

 

and

 

45

.

0

)

2

(

(

 

if

=

+

=

<

>

<

rflag

consc

count

rflag

consc

count

rflag

consc

count

mPgain

Rc

Rc


b)
[image: image21.wmf]music

count

_

tracks the number of frames where the previous frame uses backward adaptive LPC and the current frame is "speech" (according to the VAD) within a window of 64 frames.


[image: image22.wmf]1

_

_

   

)

_

 

and

 

1

pc_mod

(

 

if

+

=

==

==

music

count

music

count

VOICE

deci

Vad

L



Every 64 frames, a running mean of [image: image23.wmf]music

count

_

, [image: image24.wmf]music

mcount

_

 is updated and reset to zero as described below:


[image: image25.wmf]}

_

1

.

0

_

9

.

0

_

      

else

_

_

      

)

64

_

(

 

if

  

0){

64)

 

mod

 

_

(

(

 

if

music

count

music

mcount

music

mcount

music

count

music

mcount

count

Frm

count

Frm

+

=

=

==

==


c)
[image: image26.wmf]consc

count

_

 tracks the number of consecutive frames where the [image: image27.wmf]music

count

_

remains zero:


[image: image28.wmf]0

_

   

else

1

   

)

0

_

(

 

if

=

+

=

==

consc

count

c

count_cons

c

count_cons

music

count




[image: image29.wmf]0

_

set 

  

)

150

_

_

or 

 

500

_

(

 

if

=

>

>

music

mcount

rflag

consc

count

consc

count



[image: image30.wmf]music

count

_

 in b) is reset to zero every 64 frames after the update of the relevant counters.


The logic in c) is used to reset the running mean [image: image31.wmf]music

count

_

.

d)
[image: image32.wmf]pflag

count

_

 tracks the number of frames where [image: image33.wmf]1

=

Pflag

, within a window of 64 frames.


[image: image34.wmf]1

_

_

    

)

 

1

(

 

if

+

=

==

pflag

count

pflag

count

Pflag



Every 64 frames, a running mean of [image: image35.wmf]pflag

count

_

, [image: image36.wmf]pflag

mcount

_

, is updated and reset to zero as described below:


[image: image37.wmf]}

}

   

_

1

.

0

_

9

.

0

_

       

else

    

_

05

.

0

_

95

.

0

_

       

)

20

_

(

 

else

    

_

02

.

0

_

98

.

0

_

       

 

)

25

_

 

(

 

if

     

else{

  

_

_

      

)

64

_

(

 

if

  

0){

64)

 

mod

 

_

(

(

 

if

pflag

count

pflag

mcount

pflag

mcount

pflag

count

pflag

mcount

pflag

mcount

pflag

count

pflag

count

pflag

mcount

pflag

mcount

pflag

count

pflag

count

pflag

mcount

count

Frm

count

Frm

+

=

+

=

>

+

=

>

=

==

==


e)
[image: image38.wmf]pflag

consc

count

_

_

 tracks the number of consecutive frames satisfying the following condition:


[image: image39.wmf]0

_

_

   

else

1

_

_

   

)

0

_

(

 

if

=

+

=

==

pflag

consc

count

pflag

c

count_cons

pflag

c

count_cons

pflag

count



[image: image40.wmf]0

_

set 

  

)

150

_

_

or 

 

100

_

_

(

 

if

=

>

>

pflag

mcount

rflag

consc

count

pflag

consc

count



[image: image41.wmf]pflag

count

_

 is reset to zero every 64 frames. The logic in e) is used to reset the running mean of [image: image42.wmf]pflag

count

_

.

G.5.1.2
Classification

Based on the estimation of the above parameters, the VAD decision [image: image43.wmf]deci

Vad

_

from the VAD module is reverted if the following conditions are satisfied:



[image: image44.wmf]}

      

)

7

 

and

      

          

)

64

_

or 

 

0938

.

1

_

or 

 

0

1

_

(

 

(

 

if

 

else

   

_

      

)

50

 

and

 

)

 

4

)

(

or 

 

38

.

0

 

(

 

(

 

if

 

else

  

_

      

)

50

 

and

 

4

)

(

 

and

 

5

1

.

0

 

(

 

if

  

){

729

 

(

 

if

VOICE

Vad_deci

LLenergy

count

Frm

music

mcount

pflag

mcount

VOICE

deci

Vad

LLenergy

mLenergy

Lenergy

SD

VOICE

deci

Vad

LLenergy

mLenergy

Lenergy

SD

E

G

Rate

=

>

<

>=

>=

=

>

>

-

>

=

>

>

-

>

=


Note that the music detection function is called all the time regardless of the operational coding mode in order to keep the memories current. However, the VAD decision [image: image45.wmf]deci

Vad

_

 is altered only if Annex G is operating at 11.8 kbit/s (Annex E). It should be noted that the music detection only has the capability to change the decision from "non-speech" to "speech" and not vice versa.

G.5.2
Update of state variables specific to Annex E during discontinued transmission

G.5.2.1
Update of encoder state variables specific to Annex E

At the encoder in case of inactive frames, the update of state variables is identical to the update performed in G.729 Annex E [3] in case of switch to the nominal 8 kbit/s bit rate. The update procedure is the following: the LP mode is set to 0, the global stationarity indicator is decreased and the high stationarity indicator is reset to 0 (see E.3.2.7.2 of ITU-T G.729 – Annex E [3]), the interpolation factor used to smoothly switch from LP forward filter to backward LP filter is reset to its maximum value (see E.3.2.7.1 of ITU-T G.729 – Annex E [3]).

G.5.2.2
Update of decoder state variables specific to Annex E during discontinued transmission

At the decoder in case of inactive frames, the update of state variables is almost identical to the update performed in G.729 Annex E [3] in case of switch to the forward mode only rates (8 kbit/s) except that the pitch delay stationary indicator is reset to 0 instead of being computed by the pitch tracking procedure (see E.4.4.5 of ITU-T G.729 – Annex E [3]).

G.6
Description of C source code

Annex G of G.729, integrating the G.729 Annexes B and E, is simulated in 16-bit fixed-point ANSI-C code using the same types of fixed-point data and the same set of fixed-point basic operators as in the G.729 software. The ANSI-C code represents the normative specification of this annex. The algorithmic description given by the C code shall take precedence over the texts contained in the main body of Recommendation G.729 and in Annexes B, E and G. The following subclauses summarize the use of this simulation code, and how the software is organized.

G.6.1
Use of the simulation software 

The C code consists of two main programs coderg.c and decoderg.c, which simulate encoder and decoder, respectively. The encoder is run as follows:


coderg inputfile bitstreamfile dtx_option rate_option

The decoder is run as follows:


decoderg bitstreamfile outputfile

The inputfile and outputfile are 8 kHz sampled data files containing 16-bit PCM signals. The bitstreamfile is a binary file containing the bit stream; the mapping table of the encoded bit stream is contained in the simulation software. The two parameters are used for the encoder: dtx_option and rate_option where:

dtx_option
=
1: DTX enabled 0: DTX disabled, the default is 0 (DTX disabled).

rate_option
 =
1 to select the main Recommendation G.729 (8 kbit/s); = 2 is to select the higher rate (11.8 kbit/s) or a file_rate_name: a binary file of 16-bit word containing either 1, 2 to select the rate on a frame‑by-frame basis; the default is 1 (8 kbit/s).

G.6.2
Organization of the simulation software

The files can be classified into four groups:

1)
Files identical to software files of G.729 main body [1], Annex B [2], Annex E [3] listed in Table G.1.

2)
Files adapted from software files of G.729 Annex B or Annex E, listed in Table G.2, some minor modifications have been introduced to cope with the integration.

3)
Files integrating G.729 software files of G.729 Annex B and Annex E, listed in Table G.3.

4)
Files specific to this integrated G.729 (new files), listed in Table G.4.

Table G.1/G.729 – List of software files identical to software files of G.729 
main body and Annex B or E

	File name
	Description
	Identical to

	Basic_op.c
	Basic operators
	Main

	Oper_32b.c
	Extended basic operators
	Main

	Dspfunc.c
	Mathematical functions
	Main

	Gainpred.c
	Gain predictor
	Main

	Lpcfunc.c
	Miscellaneous routines related to LP filter
	Main

	Pre_proc.c
	Pre-processing (HP filtering and scaling)
	Main

	P_parity.c
	Compute pitch parity
	Main

	Pwf.c
	Computation of perceptual weighting coefficients (8 kbit/s)
	Main

	Pred_lt3.c
	Generation of adaptive codebook
	Main


Table G.1/G.729 – List of software files identical to software files of G.729 
main body and Annex B or E (concluded)

	File name
	Description
	Identical to

	Post_pro.c
	Post processing (HP filtering and scaling)
	Main

	Pitch.c
	Pitch search
	Main

	Dec_lag3.c
	Decode adaptive-codebook index
	Main

	Typedef.h
	Data type definition (machine dependent)
	Main

	Basic_op.h
	Basic operators prototypes
	Main

	Oper_32b.h
	Extended basic operators prototypes
	Main



	Acelp_co.c
	ACELP codebook search
	Main

	De_acelp.c
	Decode ACELP codebook
	Main

	Qua_gain.c
	Gain quantizer
	Main

	De_acelp.c
	ACELP decoding
	Main

	Tab_ld8k.c
	ROM tables
	B

	Taming.c
	Pitch instability control
	B

	Qsidgain.c
	SID Gain Quantization
	B

	QsidLSF.c
	SID-LSF Quantization
	B

	Tab_dtx.c
	ROM tables
	B

	Calcexc.c
	CNG Excitation Calculation
	B

	Util.c
	Utility functions
	B

	Ld8k.h
	Function prototypes
	B

	Tab_ld8k.h
	Extern ROM tables declarations
	B

	Dtx.h
	Prototype and Constants
	B

	Sid.h
	Prototype and Constants
	B

	Octet.h
	Octet transmission mode definition
	B

	Tab_dtx.h
	Extern ROM table declarations
	B

	Vad.h
	Prototype and Constants
	B

	Pwfe.c
	Computation of perceptual weighting coefficients
	E

	Filtere.c
	Filter functions
	E

	Lspgetqe.c
	LSP quantizer
	E

	Lspdece.c
	LSP decoding routing
	E

	Qua_lspe.c
	LSP quantizer
	E

	Bwfwfunc.c
	Miscellaneous routines related to backward/forward switch selection
	E

	Ld8e.h
	Function prototypes for G.729, Annex E
	E

	Acelp_e.c
	Search fixed codebook (11.8 kbit/s)
	E

	Deacelpe.c
	Decode algebraic codebook (11.8 kbit/s)
	E

	Decgaine.c
	Decode gains
	E

	Tab_ld8e.c
	ROM tables for G.729 at 11.8 kbit/s
	E

	Tab_ld8e.h
	Extern ROM declarations for G.729 at 11.8 kbit/s
	E

	Track_pi.c
	Pitch tracking
	E


Table G.2/G.729 – List of software files adapted from software files of G.729
main body, Annexes B and E

	File name
	Description
	Adapted from

	Dtxg.c
	DTX Decision
	B

	Vadg.c
	VAD
	B

	Dec_sidf.c
	Decode SID Information
	B

	Bwfwg.c
	Backward/forward switch selection
	E


Table G.3/G.729 – List of software files integrating software files from G.729
 main body, Annexes B and E

	File name
	Description
	Integrated from

	Coderg.c
	Main encoder routine
	B+E

	Cod_ld8g.c
	Encoder routine
	B+E

	Decoderg.c
	Main decoder routine
	B+E

	Dec_ld8g.c
	Decoder routine
	B+E

	Bitsg.c
	Bit manipulation routines
	B+E

	Lpcg.c
	LP analysis
	B+E

	Pstg.c
	Postfilter routines
	B+E

	Ld8g.h
	Constant and Function prototypes for G.729, Annex G 
	B+E


Table G.4/G.729 – List of software files specific to integrated G.729
Annexes B and E

	File name
	Description

	Mus_dtct.c
	Music detection module


	ITU-T  RECOMMENDATIONS  SERIES

	Series A
	Organization of the work of the ITU-T

	Series B
	Means of expression: definitions, symbols, classification

	Series C
	General telecommunication statistics

	Series D
	General tariff principles

	Series E
	Overall network operation, telephone service, service operation and human factors

	Series F
	Non-telephone telecommunication services

	Series G
	Transmission systems and media, digital systems and networks

	Series H
	Audiovisual and multimedia systems

	Series I
	Integrated services digital network

	Series J
	Transmission of television, sound programme and other multimedia signals

	Series K
	Protection against interference

	Series L
	Construction, installation and protection of cables and other elements of outside plant

	Series M
	TMN and network maintenance: international transmission systems, telephone circuits, telegraphy, facsimile and leased circuits

	Series N
	Maintenance: international sound programme and television transmission circuits

	Series O
	Specifications of measuring equipment

	Series P
	Telephone transmission quality, telephone installations, local line networks

	Series Q
	Switching and signalling

	Series R
	Telegraph transmission

	Series S
	Telegraph services terminal equipment

	Series T
	Terminals for telematic services

	Series U
	Telegraph switching

	Series V
	Data communication over the telephone network

	Series X
	Data networks and open system communications

	Series Y
	Global information infrastructure and Internet protocol aspects

	Series Z
	Languages and general software aspects for telecommunication systems


� 	This annex includes an electronic attachment containing version 1.1 of reference C code and test vectors for fixed-point implementation of CS-ACELP at 8 kbit/s and 11.8 kbit/s with DTX functionality.







Recommendation G.729/Annex G    (02/00)
i

