
	[image: image1.wmf]
	INTERNATIONAL  TELECOMMUNICATION  UNION

	
	

	ITU-T
	E.411

	TELECOMMUNICATION
STANDARDIZATION  SECTOR
OF  ITU
	Amendment 1

(03/2001)   

	
	SERIES E: OVERALL NETWORK OPERATION, TELEPHONE SERVICE, SERVICE OPERATION AND HUMAN FACTORS

Network management – International network management



	
	International network management – Operational guidance

Amendment 1



	
	ITU‑T  Recommendation  E.411  –  Amendment 1

(Formerly  CCITT  Recommendation)


ITU-T E-SERIES  RECOMMENDATIONS

OVERALL NETWORK OPERATION, TELEPHONE SERVICE, SERVICE OPERATION AND HUMAN FACTORS

	
	

	INTERNATIONAL OPERATION
	

	Definitions
	E.100–E.103

	General provisions concerning Administrations
	E.104–E.119

	General provisions concerning users
	E.120–E.139

	Operation of international telephone services
	E.140–E.159

	Numbering plan of the international telephone service
	E.160–E.169

	International routing plan
	E.170–E.179

	Tones in national signalling systems
	E.180–E.189

	Numbering plan of the international telephone service
	E.190–E.199

	Maritime mobile service and public land mobile service
	E.200–E.229

	OPERATIONAL PROVISIONS RELATING TO CHARGING AND ACCOUNTING IN THE INTERNATIONAL TELEPHONE SERVICE
	

	Charging in the international telephone service
	E.230–E.249

	Measuring and recording call durations for accounting purposes
	E.260–E.269

	UTILIZATION OF THE INTERNATIONAL TELEPHONE NETWORK FOR NON-TELEPHONY APPLICATIONS
	

	General
	E.300–E.319

	Phototelegraphy
	E.320–E.329

	ISDN PROVISIONS CONCERNING USERS
	E.330–E.349

	INTERNATIONAL ROUTING PLAN
	E.350–E.399

	NETWORK MANAGEMENT
	

	International service statistics
	E.400–E.409

	International network management
	E.410–E.419

	Checking the quality of the international telephone service
	E.420–E.489

	TRAFFIC ENGINEERING
	

	Measurement and recording of traffic
	E.490–E.505

	Forecasting of traffic
	E.506–E.509

	Determination of the number of circuits in manual operation
	E.510–E.519

	Determination of the number of circuits in automatic and semi-automatic operation
	E.520–E.539

	Grade of service
	E.540–E.599

	Definitions
	E.600–E.649

	Traffic engineering for IP-networks
	E.650–E.699

	ISDN traffic engineering
	E.700–E.749

	Mobile network traffic engineering
	E.750–E.799

	QUALITY OF TELECOMMUNICATION SERVICES: CONCEPTS, MODELS, OBJECTIVES AND DEPENDABILITY PLANNING
	

	Terms and definitions related to the quality of telecommunication services
	E.800–E.809

	Models for telecommunication services
	E.810–E.844

	Objectives for quality of service and related concepts of telecommunication services
	E.845–E.859

	Use of quality of service objectives for planning of telecommunication networks
	E.860–E.879

	Field data collection and evaluation on the performance of equipment, networks and services
	E.880–E.899

	
	


For further details, please refer to the list of ITU-T Recommendations.

	ITU-T Recommendation E.411

International network management – Operational guidance

Amendment 1




	Summary

This amendment aims to make adjustments to ABR (answer bid ratio) and ASR (answer seizure ratio) parameters when introducing number portability. ASR and ABR parameters calculated downstream the rerouting exchange/network and towards the donor exchange/network should be adjusted in order not to consider as failed calls the ones to ported number, resulting in release messages.

It is also proposed to add references at the end of ITU-T E.411.


	Source

Amendment 1 to ITU‑T Recommendation E.411 was prepared by ITU‑T Study Group 2 (2001‑2004) and approved under the WTSA Resolution 1 procedure on 17 March 2001.


	


FOREWORD

The International Telecommunication Union (ITU) is the United Nations specialized agency in the field of tele​com​mu​ni​ca​tions. The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of ITU. ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Assembly (WTSA), which meets every four years, establishes the topics for study by the ITU‑T study groups which, in turn, produce Recommendations on these topics.

The approval of ITU-T Recommendations is covered by the procedure laid down in WTSA Resolution 1.

In some areas of information technology which fall within ITU-T's purview, the necessary standards are prepared on a collaborative basis with ISO and IEC.

NOTE

In this Recommendation, the expression "Administration" is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

INTELLECTUAL PROPERTY RIGHTS
ITU draws attention to the possibility that the practice or implementation of this Recommendation may involve the use of a claimed Intellectual Property Right. ITU takes no position concerning the evidence, validity or applicability of claimed Intellectual Property Rights, whether asserted by ITU members or others outside of the Recommendation development process.

As of the date of approval of this Recommendation, ITU had not received notice of intellectual property, protected by patents, which may be required to implement this Recommendation. However, implementors are cautioned that this may not represent the latest information and are therefore strongly urged to consult the TSB patent database.

(  ITU  2001

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from ITU.

ITU-T Recommendation E.411

International network management ( Operational guidance

amendment 1

1)
Subclauses 3.6.3 and 3.6.4

In order to adjust the calculation of ASR and ABR, modify 3.6.3 and 3.6.4. The modifications are shown in italics:

3.6.3
answer seizure ratio (ASR): ASR gives the relationship between the number of seizures that result in an answer signal and the total number of seizures. This is a direct measure of the effectiveness of the service being offered onward from the point of measurement and is usually expressed as a percentage as follows:




[image: image2.wmf]100

seizures

Total

signal

answer

in

resulting

Seizures

ASR

´

=


Measurement of ASR may be made on a circuit group or on a destination basis.

In presence of number portability, performed by the mechanisms of dropback or query on release, the calculus of ASR should be adjusted to subtract from the total number of seizures the number of release messages indicating the request for rerouting. 




[image: image3.wmf]100

)

(rerouting

 

release

–

seizures

Total

signal

answer

in

resulting

Seizures

ASR

´

=


3.6.4
answer bid ratio (ABR): ABR gives the relationship between the number of bids that result in an answer signal and the total number of bids. ABR may be made on a circuit group or on a destination basis.




[image: image4.wmf]100

bids

Total

signal

answer

in

resulting

Bids

ABR

´

=


ABR is expressed as a percentage and is a direct measure of the effectiveness of traffic onward from the point of measurement. It is similar to ASR except that it includes bids that do not result in a seizure.

In presence of number portability, performed by the mechanisms of dropback or query on release, the calculus of ABR should be adjusted to subtract from the total number of bids the number of release messages indicating the request for rerouting. 




[image: image5.wmf]100

)

(rerouting

 

release

–

bids

Total

signal

answer

in

resulting

Bids

ABR

´

=


2)
References

Add the following:

References

The following ITU-T Recommendations and other references contain provisions, which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

(
ITU-T E.411 (2000), International network management – Operational guidance.

(
ITU-T Q.730 (1999), Signalling System No. 7 – B-ISDN user part (B-ISUP) – Supplementary services.

(
ITU-T Q.763 (1999), Signalling System No. 7 – ISDN user part formats and codes.

(
ITU-T Q.764 (1999), Signalling System No. 7 – ISDN user part signalling procedures.

(
ITU-T Q.769.1 (1999), Signalling System No. 7 – ISDN user part enhancements for the support of number portability.

(
ITU-T Supplement 5 to the Q-Series of Recommendations (1999), Number portability – capability set 2 requirements for service provider portability (Query on release and dropback).
	SERIES OF ITU-T RECOMMENDATIONS

	Series A
	Organization of the work of ITU-T

	Series B
	Means of expression: definitions, symbols, classification

	Series C
	General telecommunication statistics

	Series D
	General tariff principles

	Series E
	Overall network operation, telephone service, service operation and human factors

	Series F
	Non-telephone telecommunication services

	Series G
	Transmission systems and media, digital systems and networks

	Series H
	Audiovisual and multimedia systems

	Series I
	Integrated services digital network

	Series J
	Cable networks and transmission of television, sound programme and other multimedia signals

	Series K
	Protection against interference

	Series L
	Construction, installation and protection of cables and other elements of outside plant

	Series M
	TMN and network maintenance: international transmission systems, telephone circuits, telegraphy, facsimile and leased circuits

	Series N
	Maintenance: international sound programme and television transmission circuits

	Series O
	Specifications of measuring equipment

	Series P
	Telephone transmission quality, telephone installations, local line networks

	Series Q
	Switching and signalling

	Series R
	Telegraph transmission

	Series S
	Telegraph services terminal equipment

	Series T
	Terminals for telematic services

	Series U
	Telegraph switching

	Series V
	Data communication over the telephone network

	Series X
	Data networks and open system communications

	Series Y
	Global information infrastructure and Internet protocol aspects

	Series Z
	Languages and general software aspects for telecommunication systems

	
	





_1061619665.unknown

_1061619728.unknown

_1061620306.unknown

_1061619469.unknown

