
	[image: image1.wmf]
	INTERNATIONAL  TELECOMMUNICATION  UNION

	
	

	ITU-T
	E.451

	TELECOMMUNICATION
STANDARDIZATION  SECTOR
OF  ITU
	(02/2001)   

	
	SERIES E: OVERALL NETWORK OPERATION, TELEPHONE SERVICE, SERVICE OPERATION AND HUMAN FACTORS

Network management – Checking the quality of the international telephone service



	
	Facsimile call cut-off performance

	
	ITU‑T  Recommendation  E.451

(Formerly  CCITT  Recommendation)


ITU-T E-SERIES  RECOMMENDATIONS

OVERALL NETWORK OPERATION, TELEPHONE SERVICE, SERVICE OPERATION AND HUMAN FACTORS

	
	

	INTERNATIONAL OPERATION
	

	Definitions
	E.100–E.103

	General provisions concerning Administrations
	E.104–E.119

	General provisions concerning users
	E.120–E.139

	Operation of international telephone services
	E.140–E.159

	Numbering plan of the international telephone service
	E.160–E.169

	International routing plan
	E.170–E.179

	Tones in national signalling systems
	E.180–E.189

	Numbering plan of the international telephone service
	E.190–E.199

	Maritime mobile service and public land mobile service
	E.200–E.229

	OPERATIONAL PROVISIONS RELATING TO CHARGING AND ACCOUNTING IN THE INTERNATIONAL TELEPHONE SERVICE
	

	Charging in the international telephone service
	E.230–E.249

	Measuring and recording call durations for accounting purposes
	E.260–E.269

	UTILIZATION OF THE INTERNATIONAL TELEPHONE NETWORK FOR NON-TELEPHONY APPLICATIONS
	

	General
	E.300–E.319

	Phototelegraphy
	E.320–E.329

	ISDN PROVISIONS CONCERNING USERS
	E.330–E.349

	INTERNATIONAL ROUTING PLAN
	E.350–E.399

	NETWORK MANAGEMENT
	

	International service statistics
	E.400–E.409

	International network management
	E.410–E.419

	Checking the quality of the international telephone service
	E.420–E.489

	TRAFFIC ENGINEERING
	

	Measurement and recording of traffic
	E.490–E.505

	Forecasting of traffic
	E.506–E.509

	Determination of the number of circuits in manual operation
	E.510–E.519

	Determination of the number of circuits in automatic and semi-automatic operation
	E.520–E.539

	Grade of service
	E.540–E.599

	Definitions
	E.600–E.649

	Traffic engineering for IP-networks
	E.650–E.699

	ISDN traffic engineering
	E.700–E.749

	Mobile network traffic engineering
	E.750–E.799

	QUALITY OF TELECOMMUNICATION SERVICES: CONCEPTS, MODELS, OBJECTIVES AND DEPENDABILITY PLANNING
	

	Terms and definitions related to the quality of telecommunication services
	E.800–E.809

	Models for telecommunication services
	E.810–E.844

	Objectives for quality of service and related concepts of telecommunication services
	E.845–E.859

	Use of quality of service objectives for planning of telecommunication networks
	E.860–E.879

	Field data collection and evaluation on the performance of equipment, networks and services
	E.880–E.899

	
	


For further details, please refer to the list of ITU-T Recommendations.

	ITU-T Recommendation E.451

Facsimile call cut-off performance




	Summary

The metrics defined in this Recommendation are intended for use when data is collected using the intrusive test call based method defined in ITU E.457.  Metrics representing call cut-off performance for high speed facsimile, i.e. facsimile using V.34 modulation in accordance with Annex F/T.30, can be found in ITU-T E.460.




	Source

ITU‑T Recommendation E.451 was revised by ITU‑T Study Group 2 (2001‑2004) and approved under the WTSA Resolution 1 procedure on 2 February 2001.


	


FOREWORD

The International Telecommunication Union (ITU) is the United Nations specialized agency in the field of tele​com​mu​ni​ca​tions. The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of ITU. ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Assembly (WTSA), which meets every four years, establishes the topics for study by the ITU‑T study groups which, in turn, produce Recommendations on these topics.

The approval of ITU-T Recommendations is covered by the procedure laid down in WTSA Resolution 1.

In some areas of information technology which fall within ITU-T's purview, the necessary standards are prepared on a collaborative basis with ISO and IEC.

NOTE

In this Recommendation, the expression "Administration" is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

INTELLECTUAL PROPERTY RIGHTS
ITU draws attention to the possibility that the practice or implementation of this Recommendation may involve the use of a claimed Intellectual Property Right. ITU takes no position concerning the evidence, validity or applicability of claimed Intellectual Property Rights, whether asserted by ITU members or others outside of the Recommendation development process.

As of the date of approval of this Recommendation, ITU had not received notice of intellectual property, protected by patents, which may be required to implement this Recommendation. However, implementors are cautioned that this may not represent the latest information and are therefore strongly urged to consult the TSB patent database.

(  ITU  2001

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from ITU.

CONTENTS


Page

1
Introduction

1
2 
References

1
3
Scope

1
4
Definitions

2
5
Remarks

3
ITU-T Recommendation E.451

Facsimile call cut-off performance

1
Introduction

Call cut-off is one of the most important factors that affect the quality of service for facsimile applications on PSTN. Facsimile cut-offs can occur in various phases of a facsimile call and the failure modes can be quite complex. The cut-off ratio may be defined in general, as the percentage of established facsimile calls that are terminated prior to the transmittal of all the pages in the facsimile transaction. Since the failure modes are complex, it is important to define cut-offs precisely so that valid comparisons can be made between cut-off ratio measurements from a variety of sources. The focus of the definitions is on test calls that follow certain rules and the definitions are developed from the perspective of the transmitting terminal. Uniformly, the terminology in ITU-T T.30 has been employed. 

The metrics defined in this Recommendation are intended for use when data is collected using the intrusive test call based method defined in ITU-T E.457. Metrics representing call cut-off performance for high speed facsimile, i.e. facsimile using V.34 modulation in accordance with Annex F/T.30, can be found in ITU-T E.460.

When collecting and assessing Facsimile Call Cut-off data it may be important for ROAs to examine the cause of observed failures. 

2 
References

The following ITU-T Recommendations and other references contain provisions, which, through reference in this text, constitute provisions of this Recommandation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommandations is regularly published.

–
ITU-T E.450 (1998), Facsimile quality of service on public networks – General aspects.

–
ITU-T E.456 (1998), Test transaction for facsimile transmission performance.

–
ITU-T E.457 (1996), Facsimile measurement methodologies.

–
ITU-T E.460 (2000), Measurements and metrics for monitoring the performance of V.34 Group 3 Facsimile.

–
ITU-T T.30 (1999), Procedures for document facsimile transmission in the general switched telephone network.

3
Scope

The definitions in clause 4 below apply to test calls:

a)
that use automatic-to-automatic scenario (method No. 4 in Table 1/T.30) and may be adapted for other methods in the same table;

b)
that use standard test transactions as defined in ITU-T E.456.

c)
where the intended receiving facsimile terminal has responded by going off-hook and CED has been received at the originating terminal [see clause 5e)];

d)
that are sent to a receiving facsimile terminal that is connected to a dedicated line and not shared with voice terminals.

4
Definitions

This Recommendation defines the following terms:

a)
A pre-message phase B failure is considered to have occurred if prior to the transmittal of the first page, either the originating or the terminating facsimile terminal goes on-hook prior to the reception of a CFR message by the originating terminal.


A post-message phase B failure is considered to have occurred if a phase B failure occurs after the transmission of any page.

b)
A phase C/phase D failure at any page is considered to have occurred if a valid post message response (MCF, RTP, RTN, PIN, PIP) is not received in response to either the MPS or the EOP message. If this occurs at mth page, then m – 1 pages are considered to be successfully transmitted but one can consider that there was a failure at the mth page.

c)
When transmitting N-page transactions, the facsimile call cut-off ratio (expressed as a percentage) for N pages is



%CN = (FN / T) ( 100


where FN is the number of transactions where there was either a phase B, C, or D failure, and T is the total number of transactions satisfying the conditions in clause 3.

d)
Based on the definition in item c) above, facsimile call cut-off ratios for m pages (1 ≤ m < N) is



%Cm = (Fm / T) ( 100


where Fm is the number of transactions that had a phase B, C or D failure at the mth page.

e)
The cut-off ratio for transactions for which a pre-message phase B failure occurs is



%C1B = (F1B / T) ( 100


where FlB is the number of transactions with a pre-picture phase B failure.

f)
Given that conditions a) to d) in clause 3 have been satisfied, it is possible to define conditional facsimile success (CFS) ratios which are the complements of cutoff ratios:



%CFSN = 100 – %CN


%CFSm = 100 – %Cm


%CFS1B = 100 – %C1B
g)
End-to-end facsimile success (FS) ratios can be defined as follows:



%FS = PACR ( %CFSN


%FSm = PACR ( %CFSm


%FS1B = PACR ( %CFS1B
The FSs represent the true facsimile success performance observed by the customers. The ratio PACR is the phase A completion ratio and is dependent on several factors. These factors include network blocking, terminal availability, proper operation of the terminating facsimile terminal and correct reception of the CED signal by the originating terminal.

5
Remarks

a)
During test calls, retransmissions of pages may occur. The retransmitted pages are not counted for computing call cut-off ratios. For example, if retransmissions occur prior to the mth page of the original document followed by a failure at the mth page, it is still counted as a cut-off at the mth page.

b)
A test transaction of 3 pages as defined in ITU-T E.456 shall be used.

c)
For manual to automatic test calls (method No. 2 in Table 1/T.30) there are no changes in definitions contained in clause 4 provided that there are no operator errors.

d)
From a measurement point of view, the precise evaluation of cut-off ratios CN requires that the T.30 HDLC protocol messages be monitored and recorded during test transactions. This should not be a problem for specially constructed/configured test vehicles or network based monitoring systems.

e)
If measurements are made between terminals from the same manufacturer that will result in the use of non-standard features, the possible impact of these features on cut-off ratios should be identified and understood.

f)
Machines from some manufacturers disconnect on the receipt of an RTN while others do not. In the presence of network impairments, machines that disconnect on the receipt of an RTN may result in a higher facsimile call cut-off ratio compared to those machines that do not disconnect on the receipt of an RTN. Therefore, it is important to explicitly state the characteristics of machines such as the response to RTNs when comparing the results from the different machines.

	SERIES OF ITU-T RECOMMENDATIONS

	Series A
	Organization of the work of ITU-T

	Series B
	Means of expression: definitions, symbols, classification

	Series C
	General telecommunication statistics

	Series D
	General tariff principles

	Series E
	Overall network operation, telephone service, service operation and human factors

	Series F
	Non-telephone telecommunication services

	Series G
	Transmission systems and media, digital systems and networks

	Series H
	Audiovisual and multimedia systems

	Series I
	Integrated services digital network

	Series J
	Cable networks and transmission of television, sound programme and other multimedia signals

	Series K
	Protection against interference

	Series L
	Construction, installation and protection of cables and other elements of outside plant

	Series M
	TMN and network maintenance: international transmission systems, telephone circuits, telegraphy, facsimile and leased circuits

	Series N
	Maintenance: international sound programme and television transmission circuits

	Series O
	Specifications of measuring equipment

	Series P
	Telephone transmission quality, telephone installations, local line networks

	Series Q
	Switching and signalling

	Series R
	Telegraph transmission

	Series S
	Telegraph services terminal equipment

	Series T
	Terminals for telematic services

	Series U
	Telegraph switching

	Series V
	Data communication over the telephone network

	Series X
	Data networks and open system communications

	Series Y
	Global information infrastructure and Internet protocol aspects

	Series Z
	Languages and general software aspects for telecommunication systems

	
	


