
	[image: image1.wmf]
	INTERNATIONAL  TELECOMMUNICATION  UNION


	ITU-T
	E.671

	TELECOMMUNICATION
STANDARDIZATION  SECTOR
OF  ITU
	(03/2000)   


SERIES E: OVERALL NETWORK OPERATION, TELEPHONE SERVICE, SERVICE OPERATION AND HUMAN FACTORS

Quality of service, network management and traffic engineering – Traffic engineering – Definitions

Post-selection delay in PSTN/ISDN networks using Internet telephony for a portion of the connection

ITU-T  Recommendation  E.671

(Formely  CCITT  Recommendation)
ITU-T  E-SERIES  RECOMMENDATIONS

OVERALL NETWORK OPERATION, TELEPHONE SERVICE, SERVICE OPERATION 
AND HUMAN FACTORS

	
	

	OPERATION, NUMBERING, ROUTING AND MOBILE SERVICES

	INTERNATIONAL OPERATION
	

	Definitions
	E.100–E.103

	General provisions concerning Administrations
	E.104–E.119

	General provisions concerning users
	E.120–E.139

	Operation of international telephone services
	E.140–E.159

	Numbering plan of the international telephone service
	E.160–E.169

	International routing plan
	E.170–E.179

	Tones in national signalling systems
	E.180–E.189

	Numbering plan of the international telephone service
	E.190–E.199

	Maritime mobile service and public land mobile service
	E.200–E.229

	OPERATIONAL PROVISIONS RELATING TO CHARGING AND ACCOUNTING IN THE INTERNATIONAL TELEPHONE SERVICE
	

	Charging in the international telephone service
	E.230–E.249

	Measuring and recording call durations for accounting purposes
	E.260–E.269

	UTILIZATION OF THE INTERNATIONAL TELEPHONE NETWORK FOR NON-TELEPHONY APPLICATIONS
	

	General
	E.300–E.319

	Phototelegraphy
	E.320–E.329

	ISDN PROVISIONS CONCERNING USERS
	

	International routing plan
	E.350–E.399

	QUALITY OF SERVICE, NETWORK MANAGEMENT AND TRAFFIC ENGINEERING

	NETWORK MANAGEMENT
	

	International service statistics
	E.400–E.409

	International network management
	E.410–E.419

	Checking the quality of the international telephone service
	E.420–E.489

	TRAFFIC ENGINEERING
	

	Measurement and recording of traffic
	E.490–E.505

	Forecasting of traffic
	E.506–E.509

	Determination of the number of circuits in manual operation
	E.510–E.519

	Determination of the number of circuits in automatic and semi-automatic operation
	E.520–E.539

	Grade of service
	E.540–E.599

	Definitions
	E.600–E.699

	ISDN traffic engineering
	E.700–E.749

	Mobile network traffic engineering
	E.750–E.799

	QUALITY OF TELECOMMUNICATION SERVICES: CONCEPTS, MODELS, OBJECTIVES AND DEPENDABILITY PLANNING
	

	Terms and definitions related to the quality of telecommunication services
	E.800–E.809

	Models for telecommunication services
	E.810–E.844

	Objectives for quality of service and related concepts of telecommunication services
	E.845–E.859

	Use of quality of service objectives for planning of telecommunication networks
	E.860–E.879

	Field data collection and evaluation on the performance of equipment, networks and services
	E.880–E.899

	
	


For further details, please refer to the list of ITU-T Recommendations.

	ITU-T  RECOMMENDATION  E.671

post-selection delay in PSTN/ISDN networks using internet telephony for a portion of the connection


	Summary

This Recommendation proposes a post-selection delay Grade of Service (GOS) parameter and target values when part of a PSTN/ISDN connection is replaced by Internet Protocol (IP) based network(s). This parameter is defined and their target values specified assuming that the network and the network components are fully operational. Also, only point-to-point connections supporting basic call flows are considered in this release of the Recommendation.


	Source
ITU-T Recommendation E.671 was prepared by ITU-T Study Group 2 (1997-2000) and was approved under the WTSC Resolution No. 1 procedure on 13 March 2000.


	


FOREWORD

ITU (International Telecommunication Union) is the United Nations Specialized Agency in the field of telecommunications. The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of the ITU. The ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Conference (WTSC), which meets every four years, establishes the topics for study by the ITU‑T Study Groups which, in their turn, produce Recommendations on these topics.

The approval of Recommendations by the Members of the ITU‑T is covered by the procedure laid down in WTSC Resolution No. 1.

In some areas of information technology which fall within ITU-T’s purview, the necessary standards are prepared on a collaborative basis with ISO and IEC.

NOTE

In this Recommendation, the expression "Administration" is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.
INTELLECTUAL PROPERTY RIGHTS
The ITU draws attention to the possibility that the practice or implementation of this Recommendation may involve the use of a claimed Intellectual Property Right. The ITU takes no position concerning the evidence, validity or applicability of claimed Intellectual Property Rights, whether asserted by ITU members or others outside of the Recommendation development process.

As of the date of approval of this Recommendation, the ITU had not received notice of intellectual property, protected by patents, which may be required to implement this Recommendation. However, implementors are cautioned that this may not represent the latest information and are therefore strongly urged to consult the TSB patent database.

(  ITU  2000

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

CONTENTS


Page

1
Scope

1
2
References

1
3
Abbreviations

2
4
Post-selection delay GOS

2
4.1
GOS concepts

2
4.2
Definitions

2
4.3
Target values

2
5
Reference connections

3
6
GOS objectives

5
7
History

5
Recommendation E.671

POST-SELECTION DELAY IN PSTN/ISDN networks USING INTERNET TELEPHONY FOR A PORTION OF THE CONNECTION

(Geneva, 2000)

1
Scope

This Recommendation proposes a post-selection delay Grade of Service (GOS) parameter and target values when part of a PSTN/ISDN connection is replaced by Internet Protocol (IP) based network(s). This parameter is defined and their target values specified assuming that the network and the network components are fully operational. Also, only point-to-point connections supporting basic call flows are considered in this release of the Recommendation.

NOTE 1 – Other GOS parameters such as answer-signal delay, call-release delay, and probability of end-to-end blocking, require further study.

NOTE 2 – This release of the Recommendation is concerned only with the case of transparent service, i.e. users will not be aware of the fact that a portion of the circuit-switched connection is being replaced by IP telephony. When this is not the case, IP telephony may be used to facilitate the offering of a set of service classes with different qualities of service. This subject requires further study.

2
References
The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; all users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.
–
ITU-T Recommendation E.500 (1998), Traffic intensity measurement principles.

–
CCITT Recommendation E.540 (1988), Overall grade of service of the international part of an international connection.

–
CCITT Recommendation E.541 (1988), Overall grade of service for international connections (subscriber-to-subscriber).

–
ITU-T Recommendation E.651 (2000), Reference connections for traffic engineering of IP access networks.

–
ITU-T Recommendation E.721 (1999), Network grade of service parameters and target values for circuit-switched services in the evolving ISDN.

–
ITU-T Recommendation E.771 (1996), Network grade of service parameters and target values for circuit-switched land mobile services.

–
ITU-T Draft Recommendation Y.1231, IP access network architecture.

–
ITU-T Draft Recommendation Y.1530, Call processing performance for voice service in hybrid IP networks.

3
Abbreviations

This Recommendation uses the following abbreviations:

GOS
Grade of Service

IP
Internet Protocol

ISDN
Integrated Services Digital Network

PSTN
Public Switched Telephone Network

TE
Terminal Equipment

VOIP
Voice Over Internet Protocol

4
Post-selection delay GOS 
4.1
GOS concepts

GOS parameters were developed for traditional circuit-switched PSTN and standards set in Recommendations E.540 and E.541. GOS concepts were refined and new standards set in the context of circuit-switched services on ISDN. The new standards are given in Recommendations E.720 and E.721. This Recommendation is based on the concepts, parameter definitions, and service types given in Recommendation E.721.

4.2
Definitions

The following two definitions of "post-selection delay", as specified in Recommendation E.721, are reproduced herein for ease of reference.

4.2.1
post-selection delay (overlap sending) is defined as the time interval from the instant the first bit of the INFORMATION message containing the last selection digit is passed by the calling terminal to the access signalling system until the last bit of the first message indicating call disposition is received by the calling terminal (ALERTING message in case of successful call).

4.2.2
post-selection delay (en bloc sending) is defined as the time interval from the instant the first bit of the initial SETUP message containing all the selection digits is passed by the calling terminal to the access signalling system until the last bit of the first message indicating call disposition is received by the calling terminal (ALERTING message in case of successful call).

NOTE – In case of automatic answering terminals, the ALERTING message is replaced by the CONNECT message.

4.3
Target values

When IP-based networks are used to substitute for a portion of the circuit-switched PSTN/ISDN connection, users will expect and network providers should meet comparable end-to-end post‑selection delay as is encountered on connections provided entirely on PSTN/ISDNs. Refinement of the target values which are provisionally specified herein is for further study.

NOTE – If and when IP telephony is used to provide a set of service classes with different qualities of service, the services so provided will not be transparent to the user in the sense of Note 2, in clause 1. In this case, the target values for the respective service classes will necessarily be different from the ones specified in this Recommendation.

5
Reference connections

This Recommendation is concerned with the class of PSTN/ISDN-to-PSTN/ISDN via IP reference connections as specified in 6.2.3/E.651. It is reproduced in Figure 1.

[image: image2.wmf]T0207910-99

TE

TE

IP core

network

VOIP

gateway

VOIP

gateway

PSTN/ISDN

PSTN/ISDN


Figure 1/E.671 – PSTN/ISDN-to-PSTN/ISDN connection via IP

The IP core network "cloud" in the diagram is also commonly referred to as the "IP backbone". It provides wide-area coverage and interconnection between domains. As specified in Draft Recommendation Y.1231, the IP core network may include one or more IP service provider networks.

The Voice Over IP (VOIP) gateway is responsible for the translation and handling of voice calls between PSTN/ISDN and IP core network.

For this class of reference connections, there are three cases of interest for which separate standards for the IP portion of an overall connection can be set. These are:

•
Connection type 1: IP as a national network where the IP-based network replaces a national PSTN/ISDN in a long-distance connection.

•
Connection type 2: IP as an international network where the IP-based network replaces the international portion of an international connection. There are three subcases:

a)
The PSTN/ISDNs at both the originating and terminating ends are not replaced by an IP‑based network;

b)
The PSTN/ISDN at the originating end is replaced by an IP-based network;

c)
The PSTN/ISDN at the terminating end is replaced by an IP-based network.

•
Connection type 3: IP as an end-to-end network where the IP-based network replaces both the national and international portions of an international connection.

These three types of reference connections are given in Figure 2. [For connection type 2, only subcase a) is shown in the figure.]

[image: image3.wmf]T0207920-99

National

PSTN

International

IP Network

Local switch

Local switch

Local switch

Local switch

Local switch

Local switch

VOIP

VOIP

VOIP

VOIP

VOIP

VOIP

National IP Network

National

PSTN

National/International

IP Network

Connection type 1: National IP network

Connection type 2: International IP network

Connection type 3: National/international IP network


Figure 2/E.671 – Reference connections

6
GOS objectives
The target values for the post-selection delay parameter for the above three types of reference connections are given in Table 1.

Table 1/E.671 – Postselection delays for PSTN/ISDN connections with IP telephony

	Reference connection
	Normal load
	High load

	
	Mean
	95%
	Mean
	95%

	National IP network
(equivalent to E.721 toll)
	5.0 s
	8.0 s
	7.5 s
	12.0 s

	International IP network
	FFS
	FFS
	FFS
	FFS

	National/international IP network
(equivalent to E.721 international)
	8.0 s
	11.0 s
	12.0 s
	16.5 s

	FFS
For further study

	NOTE 1 – The "national IP network" connection is considered equivalent to the E.721 toll connection. The "national/international IP network" connection is considered equivalent to the E.721 international connection. Consistent with E.721, except for mean delay at normal load, all other target values are provisional and require further review.

NOTE 2 – The target values are specified at the normal and high loads, with the loading conditions being specified in the same sense as Recommendation E.500. However, the definitions there may need to be adapted for use in IP-based networks. Also for further study is the application of these loading conditions in a geographically distributed network with non-coincident busy hours.

NOTE 3 – International connections are assumed to include one satellite link.

NOTE 4 – Delays for any required database lookup will need to be added to the above target values.


7
History

This is the first issue of Recommendation E.671.

	ITU-T  RECOMMENDATIONS  SERIES

	Series A
	Organization of the work of the ITU-T

	Series B
	Means of expression: definitions, symbols, classification

	Series C
	General telecommunication statistics

	Series D
	General tariff principles

	Series E
	Overall network operation, telephone service, service operation and human factors

	Series F
	Non-telephone telecommunication services

	Series G
	Transmission systems and media, digital systems and networks

	Series H
	Audiovisual and multimedia systems

	Series I
	Integrated services digital network

	Series J
	Transmission of television, sound programme and other multimedia signals

	Series K
	Protection against interference

	Series L
	Construction, installation and protection of cables and other elements of outside plant

	Series M
	TMN and network maintenance: international transmission systems, telephone circuits, telegraphy, facsimile and leased circuits

	Series N
	Maintenance: international sound programme and television transmission circuits

	Series O
	Specifications of measuring equipment

	Series P
	Telephone transmission quality, telephone installations, local line networks

	Series Q
	Switching and signalling

	Series R
	Telegraph transmission

	Series S
	Telegraph services terminal equipment

	Series T
	Terminals for telematic services

	Series U
	Telegraph switching

	Series V
	Data communication over the telephone network

	Series X
	Data networks and open system communications

	Series Y
	Global information infrastructure and Internet protocol aspects

	Series Z
	Languages and general software aspects for telecommunication systems

	
	













Recommendation E.671    (03/2000)
i

