
	[image: image1.wmf]
	INTERNATIONAL  TELECOMMUNICATION  UNION

	
	

	ITU-T
	G.707/Y.1322

	TELECOMMUNICATION
STANDARDIZATION  SECTOR
OF  ITU
	Amendment 2

(08/2002)   

	
	SERIES G: TRANSMISSION SYSTEMS AND MEDIA, DIGITAL SYSTEMS AND NETWORKS

Digital terminal equipments – General

SERIES Y: GLOBAL INFORMATION INFRASTRUCTURE AND INTERNET PROTOCOL ASPECTS

Internet protocol aspects – Transport



	
	Network node interface for the synchronous digital hierarchy (SDH)

Amendment 2

	
	ITU‑T  Recommendation  G.707/Y.1322 (2000)  –  Amendment 2




ITU-T G-SERIES  RECOMMENDATIONS

TRANSMISSION SYSTEMS AND MEDIA, DIGITAL SYSTEMS AND NETWORKS

	
	

	INTERNATIONAL TELEPHONE CONNECTIONS AND CIRCUITS
	G.100–G.199

	GENERAL CHARACTERISTICS COMMON TO ALL ANALOGUE CARRIER-TRANSMISSION SYSTEMS
	G.200–G.299

	INDIVIDUAL CHARACTERISTICS OF INTERNATIONAL CARRIER TELEPHONE SYSTEMS ON METALLIC LINES
	G.300–G.399

	GENERAL CHARACTERISTICS OF INTERNATIONAL CARRIER TELEPHONE SYSTEMS ON RADIO-RELAY OR SATELLITE LINKS AND INTERCONNECTION WITH METALLIC LINES
	G.400–G.449

	COORDINATION OF RADIOTELEPHONY AND LINE TELEPHONY
	G.450–G.499

	TESTING EQUIPMENTS
	G.500–G.599

	TRANSMISSION MEDIA CHARACTERISTICS
	G.600–G.699

	DIGITAL TERMINAL EQUIPMENTS
	G.700–G.799

	General
	G.700–G.709

	Coding of analogue signals by pulse code modulation
	G.710–G.719

	Coding of analogue signals by methods other than PCM
	G.720–G.729

	Principal characteristics of primary multiplex equipment
	G.730–G.739

	Principal characteristics of second order multiplex equipment
	G.740–G.749

	Principal characteristics of higher order multiplex equipment
	G.750–G.759

	Principal characteristics of transcoder and digital multiplication equipment
	G.760–G.769

	Operations, administration and maintenance features of transmission equipment
	G.770–G.779

	Principal characteristics of multiplexing equipment for the synchronous digital hierarchy
	G.780–G.789

	Other terminal equipment
	G.790–G.799

	DIGITAL NETWORKS
	G.800–G.899

	DIGITAL SECTIONS AND DIGITAL LINE SYSTEM
	G.900–G.999

	QUALITY OF SERVICE AND PERFORMANCE
	G.1000–G.1999

	TRANSMISSION MEDIA CHARACTERISTICS
	G.6000–G.6999

	DIGITAL TERMINAL EQUIPMENTS
	G.7000–G.7999

	DIGITAL NETWORKS
	G.8000–G.8999

	
	


For further details, please refer to the list of ITU-T Recommendations.

	ITU-T Recommendation G.707/Y.1322

Network node interface for the synchronous digital hierarchy (SDH)

Amendment 2




	Summary

This amendment contains editorial and technical additions, to ITU-T Rec. G.707/Y.1322 (2000), with the ODUk (k = 1,2) mapping into C-4-Xc (X = 17,68).


	Source

Amendment 2 to ITU‑T Recommendation G.707/Y.1322 (2000) was prepared by ITU‑T Study Group 15 (2001‑2004) and approved under the WTSA Resolution 1 procedure on 6 August 2002.


	


FOREWORD

The International Telecommunication Union (ITU) is the United Nations specialized agency in the field of tele​com​mu​ni​ca​tions. The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of ITU. ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Assembly (WTSA), which meets every four years, establishes the topics for study by the ITU‑T study groups which, in turn, produce Recommendations on these topics.

The approval of ITU-T Recommendations is covered by the procedure laid down in WTSA Resolution 1.

In some areas of information technology which fall within ITU-T's purview, the necessary standards are prepared on a collaborative basis with ISO and IEC.

NOTE

In this Recommendation, the expression "Administration" is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

INTELLECTUAL PROPERTY RIGHTS
ITU draws attention to the possibility that the practice or implementation of this Recommendation may involve the use of a claimed Intellectual Property Right. ITU takes no position concerning the evidence, validity or applicability of claimed Intellectual Property Rights, whether asserted by ITU members or others outside of the Recommendation development process.

As of the date of approval of this Recommendation, ITU had received notice of intellectual property, protected by patents, which may be required to implement this Recommendation. However, implementors are cautioned that this may not represent the latest information and are therefore strongly urged to consult the TSB patent database.

(  ITU  2002

All rights reserved. No part of this publication may be reproduced, by any means whatsoever, without the prior written permission of ITU.

CONTENTS


Page
1)
Clause 2

1
2)
Clause 4

1
3)
Clause 9.3.1.3

1
4)
Clause 10.7

1
5)
New Appendix XI

6
ITU-T Recommendation G.707/Y.1322

Network node interface for the synchronous digital hierarchy (SDH)

Amendment 2

1)
Clause 2

Add the following reference alphanumerically:

–
ITU-T Recommendation G.709/Y.1331 (2001), Interfaces for the Optical Transport Network (OTN).

2)
Clause 4

Add the following abbreviations alphabetically:

ODUk

Optical channel Data Unit-k

OPUk

Optical Payload Unit-k

OTN

Optical Transport Network

OTUk

Optical channel Transport Unit-k

3)
Clause 9.3.1.3

Add the following Signal Label code to Table 9-11 after Hex Code No. 19:

	0 0 1 0
	0 0 0 0
	20
	Asynchronous mapping of ODUk (k = 1,2) into VC‑4‑Xv (X = 17,68)


4)
Clause 10.7

Add a new clause for mapping of ODUk into VC-4-Xv as follows:

10.7
Asynchronous mapping of ODUk into a C-4-Xc transported via a VC-4-Xv

The purpose of this mapping is to provide for the transport of a subset of OTN elements, as defined in ITU-T Rec. G.709/Y.1331, over existing SDH transport networks by means of VC-4 virtual concatenation.

The number of VC-4s required to transport an OTN element by means of virtual concatenation is found by dividing the bit rate of the OTN entity by the payload rate of a VC-4-Xv, the C-4-Xc. These are provided in Table 10-2. As this number is not an integer, it is necessary to provide both a fixed stuff to pad the C-4-Xc payload area, and a means of mapping the client into the remainder of the payload area.

Table 10-2/G.707/Y.1322 – Mapping of OTN elements in SDH virtually concatenated VC-4s

	OTN entity
	Nominal bit rate, kbit/s
ODUk
	VC-4 virtual concatenation order (X)
	Nominal bit rate, kbit/s
C-4-Xv

	ODU1
	239/238 * 2 488 320
((2 498 775.126)
	17
	2 545 920

	ODU2
	239/237 * 9 953 280
((10 037 273.924)
	68
	10 183 680


The ODUk signal is extended with Frame Alignment Overhead (FAS and MFAS bytes) as specified in 15.6.2.1 and 15.6.2.2/G.709/Y.1331 and an all-0's pattern in the OTUk Overhead field (Figure 10-27).

	            Column

Row
	
	
	

	
	1 ········· 7
	8 ········· 14
	15                              ·········                                    3824

	1
	FA Overhead area
	Fixed-stuff (all-0s)
	OPUk area

(4 × 3810 bytes)

	2
	
ODUk Overhead area
	

	3
	
	

	4
	
	


Figure 10-27/G.707/Y.1322 – Extended ODUk frame structure 
(FA OH included, OTUk OH area contains Fixed Stuff)

Before the extended ODUk signal is mapped into the C-4-Xc, it is scrambled using a self‑synchronizing scrambler with polynomial x43+1. The scrambler operates over the whole extended ODUk frame and is not reset per frame.

10.7.1
Asynchronous mapping of ODU1 into a C-4-17c transported via a VC-4-17v

The basic C-4-17c structure is as shown in Figure 10-28. It is comprised of 9 rows by 4420 (i.e. 17 × 260) columns.

	
	 1 

 2  
	·································································································································
	4420

	1
	C-4-17c

(9 × 4 420 bytes)

	2
	

	3
	

	4
	

	5
	

	6
	

	7
	

	8
	

	9
	


Figure 10-28/G.707/Y.1322 – C-4-17c

This C-4-17c frame is transported via a VC-4-17v. Refer to 11.2.

The extended ODU1 signal is asynchronous mapped into this C-4-17c with the following structure:

–
Each of the nine rows is partitioned into 5 blocks, consisting of 884 octets each (Figure 10‑29).

–
Each block is partitioned into 17 subblocks, consisting of 52 octets each.

•
In each subblock, one negative justification opportunity octet (S) and five justification control bits (C) are provided.

•
The first byte of each subblock consists of either:

–
a fixed stuff byte (R); or

–
a justification control byte (J), which consists of seven fixed stuff bits (bits R; bits 1 to 7) and a justification control bit (bit C, bit 8); or

–
a negative justification opportunity byte (S).

•
The last 51 bytes of one subblock consist of data bytes (D).


NOTE – Each block contains a total of (17 × 51) = 867 data bytes.

The sequence of all these bytes is shown in Figure 10-29.

[image: image2.wmf]G.707/Y.1322AMD.2

F10-29

.....................................................................................................................................

1

2

3

4

5

6

7

8

9

1

4420

2

884 octets

884 octets

884 octets

884 octets

884 octets

884 octets

884 octets

884 octets

884 octets

884 octets

884 octets

884 octets

884 octets

884 octets

884 octets

884 octets

884 octets

884 octets

884 octets

884 octets

884 octets

884 octets

884 octets

884 octets

884 octets

884 octets

884 octets

884 octets

884 octets

884 octets

884 octets

884 octets

884 octets

884 octets

884 octets

884 octets

884 octets

884 octets

884 octets

884 octets

884 octets

884 octets

884 octets

884 octets

884 octets

51D

R

51D

J

51D

R

51D

R

51D

J

51D

R

51D

R

51D

J

51D

R

51D

R

51D

J

51D

R

51D

R

51D

J

51D

R

51D

R

51D

S

R

R

R

R

R

R

R

C

17 sub-blocks of 52 octets

1

2

3

4

5

6

7

8

51D   51 Data octets

R       Fixed stuff

C       Justification control


Figure 10-29/G.707/Y.1322 – Block structure for ODU1 mapping into C-4-17c

The set of five justification control bits (C) in every subblock is used to control the corresponding negative justification opportunity byte (S). CCCCC = 00000 indicates that the S byte is an information byte, whereas CCCCC = 11111 indicates that the S byte is a justification byte.

At the synchronizer all five C bits are set to the same value. Majority vote (3 out of 5) should be used to make the justification decision at the desynchronizer for protection against single and double bit errors in the JC bits.

The value contained in the S byte when used as justification byte is all-ZEROs. The receiver is required to ignore the value contained in this byte whenever it is used as a justification byte.

The value contained in the R bits and bytes is all-ZEROs. The receiver is required to ignore the value contained in these bits/bytes.

NOTE – The maximum bit-rate tolerance between C-4-17c and the ODU1 signal clock, which can be accommodated by this mapping scheme, is approximately –720 to +420 ppm. The nominal justification ratio is 75/119, which is approximately equal to 0.630252. Here, the justification ratio is normalized to 1, i.e., it is the long-run average fraction of justification opportunities for which there is a justification.

10.7.2
Asynchronous mapping of ODU2 into a C-4-68c transported via a VC-4-68v

The basic C-4-68c structure is as shown in Figure 10-30. It is comprised of 9 rows by 17 680 (i.e. 68 × 260) columns.

	
	   1 

   2  
	·································································································································
	17 680

	1
	C-4-687c

(9 × 17 680 bytes)

	2
	

	3
	

	4
	

	5
	

	6
	

	7
	

	8
	

	9
	


Figure 10-30/G.707/Y.1322 – C-4-68c

This C-4-68c frame is transported via a VC-4-68v. Refer to 11.2.

The extended ODU2 signal is asynchronous mapped into this C-4-68c with the following structure:

–
Each of the nine rows is partitioned into 20 blocks, consisting of 884 octets each (Figure 10-31).

–
Each block is partitioned into 13 subblocks, consisting of 68 octets each.

•
In each subblock, one negative justification opportunity octet (S) and five justification control bits (C) are provided.

•
The first byte of each subblock consists of either:

–
a fixed stuff byte (R); or

–
a justification control byte (J), which consists of seven fixed stuff bits (bits R; bits 1 to 7) and a justification control bit (bit C, bit 8); or

–
a negative justification opportunity byte (S).

•
The last 67 bytes of one subblock consist of data bytes (D).


NOTE – Each block contains a total of (13 × 67) = 871 data bytes.

The sequence of all these bytes is shown in Figure 10-31.

[image: image3.wmf].........................................................................................................................................

G.707/Y.1322AMD.2

F10-31

1

2

3

4

5

6

7

8

9

1

17 680

2

884 octets

67D

R

13 sub-blocks of 68 octets

67D

R

67D

J

67D

R

67D

J

67D

R

67D

J

67D

R

67D

J

67D

R

67D

J

67D

R

67D

S

884 octets

884 octets

884 octets

884 octets

884 octets

884 octets

884 octets

884 octets

884 octets

884 octets

884 octets

884 octets

884 octets

884 octets

884 octets

884 octets

884 octets

884 octets

884 octets

884 octets

884 octets

884 octets

884 octets

884 octets

884 octets

884 octets

884 octets

884 octets

884 octets

884 octets

884 octets

884 octets

884 octets

884 octets

884 octets

884 octets

884 octets

884 octets

884 octets

884 octets

884 octets

884 octets

884 octets

884 octets

884 octets

884 octets

884 octets

884 octets

884 octets

884 octets

884 octets

884 octets

884 octets

884 octets

884 octets

884 octets

884 octets

884 octets

884 octets

884 octets

884 octets

884 octets

884 octets

884 octets

884 octets

884 octets

884 octets

884 octets

884 octets

884 octets

884 octets

R

R

R

R

R

R

R

C

1

2

3

4

5

6

7

8

67D    67 Data octets

R        Fixed stuff

C        Justification control


Figure 10-31/G.707/Y.1322 – Block structure for ODU2 mapping into C-4-68c

The set of five justification control bits (C) in every subblock is used to control the corresponding negative justification opportunity byte (S). CCCCC = 00000 indicates that the S byte is an information byte, whereas CCCCC = 11111 indicates that the S byte is a justification byte.

At the synchronizer all five C bits are set to the same value. Majority vote (3 out of 5) should be used to make the justification decision at the desynchronizer for protection against single and double bit errors in the JC bits.

The value contained in the S byte, when used as justification byte, is all-ZEROs. The receiver is required to ignore the value contained in this byte whenever it is used as a justification byte.

The value contained in the R bits and bytes is all-ZEROs. The receiver is required to ignore the value contained in these bits/bytes.

NOTE – The maximum bit-rate tolerance between C-4-68c and the ODU2 signal clock, which can be accommodated by this mapping scheme, is approximately –330 to +810 ppm. The nominal justification ratio is 23/79, which is approximately equal to 0.291139. Here, the justification ratio is normalized to 1, i.e., it is the long-run average fraction of justification opportunities for which there is a justification.

5)
New Appendix XI

Add the following new Appendix XI and renumber existing Appendix XI as Appendix XII:

Appendix XI

Nominal justification ratios for asynchronous mapping 
of ODU1 into C-4-17c and ODU2 into C-4-68c

Appendix V/G.709/Y.1331 provides a relation between justification ratio and frequency offset, for the case of either CBRx or ODUk mapped into ODUn (n > k). Following the notation there, let ( be the justification ratio, i.e., the average number of justifications per C-4-Xc frame; also, as is done there, let positive ( correspond to negative justification and negative ( correspond to positive justification. Then 0 ( ( ( 45 for mapping ODU1 into C-4-17c and 0 ( ( ( 180 for mapping ODU2 into C-4-68c. In addition, as is done in Appendix V/G.709/Y.1331, define the following notation:

N
=
number of fixed stuff bytes in the C-4-Xc payload area;

S
=
nominal ODUk client rate (bytes/s);

T
=
nominal C-4-Xc frame period (s);

yc
=
client (ODUk) frequency offset (fraction);

ys
=
server (C-4-Xc) frequency offset (fraction);

Nf
=
average number of client bytes mapped into a C-4-Xc frame, for the particular 


frequency offsets (averaged over a large number of frames).

(Note that in Appendix V/G.709/Y.1331, a quantity p, representing the fraction of the payload area for the client in question, was defined. Here, p = 1 because there is only one client being mapped; i.e., there is no possible multiplexing as there was in Appendix V/G.709/Y.1331.)

Then Nf is given by:




[image: image4.wmf]s

c

f

y

y

ST

N

+

+

=

1

1


(1)
For frequency offsets small compared to 1, this may be approximated




[image: image5.wmf]b

º

-

+

=

ST

y

y

ST

N

s

c

f

)

1

(


(2)

The quantity (-1 is the net frequency offset due to client and server frequency offset.

Now, the average number of client bytes mapped into an C-4-Xc frame is also equal to the total number of bytes in the payload area available to the client for data mapping (i.e., excluding any fixed stuff bytes (N)), plus the average number of bytes stuffed for this client over a very large number of frames. The latter is equal to the justification ratio. The former is equal to:

Number of data bytes in payload area for ODU1 mapped into C-4-17c

(51 data bytes/subblock)(17 subblocks/block)(5 blocks/row)(9 rows/frame) = 39 015 data bytes/frame.

Number of data bytes in payload area for ODU2 mapped into C-4-68c

(67 data bytes/subblock)(13 subblocks/block)(20 blocks/row)(9 rows/frame) = 156 780 data bytes/frame.

Combining the above with Equations (1) and (2) produces:

ODU1 mapped into C-4-17c




[image: image6.wmf]39015

+

a

=

b

ST


(3)

ODU2 mapped into C-4-68c




[image: image7.wmf]156780

+

a

=

b

ST


(4)

The nominal stuff ratio occurs when the frequency offsets are zero, i.e., when ( = 1. We now set ( = 1 and solve for ( for each case.

ODU1 mapped into C-4-17c

The quantity ST is the nominal number of ODU1 bytes in the nominal C-4-17c frame period. The latter is equal to 125 (s. The former is equal to (239/238)(2.48832 ( 109 bits/s)(1 byte/8 bits). Then




[image: image8.wmf](

)

361345

.

28

    

119

3375

    

238

6750

    

238

)

39015

)(

238

(

)

38880

)(

239

(

    

39015

38880

238

239

    

39015

8

10

48832

.

2

238

239

)

10

125

(

9

6

»

=

=

-

=

-

÷

ø

ö

ç

è

æ

=

-

÷

÷

ø

ö

ç

ç

è

æ

´

÷

ø

ö

ç

è

æ

´

=

a

-


(5)

Then, on average, there are approximately 28.361345 negative justifications per C-4-17c frame out of a total possible 45. The long-run average fraction of justification opportunities for which there is a justification is equal to the above divided by 45 (denote this quantity by (()




[image: image9.wmf]40

6302521008

.

0

119

75

)

45

)(

119

(

3375

'

»

=

=

a

 
(6)

ODU2 mapped into C-4-68c

The quantity ST is the nominal number of ODU1 bytes in the nominal C-4-17c frame period. The latter is equal to 125 (s. The former is equal to (239/237)(29.95328 ( 109 bits/s)(1 byte/8 bits). Then




[image: image10.wmf](

)

405063

.

52

    

79

4140

    

237

12420

    

237

)

156780

)(

237

(

)

155520

)(

239

(

    

156780

155520

237

239

    

39015

8

10

95328

.

9

237

239

)

10

125

(

9

6

»

=

=

-

=

-

÷

ø

ö

ç

è

æ

=

-

÷

÷

ø

ö

ç

ç

è

æ

´

÷

ø

ö

ç

è

æ

´

=

a

-


(7)

Then, on average, there are approximately 52.405063 negative justifications per C-4-68c frame out of a total possible 180. The long-run average fraction of justification opportunities for which there is a justification is equal to the above divided by 180 (denote this quantity by (()




[image: image11.wmf]06

2911392405

.

0

79

23

)

180

)(

79

(

4140

'

»

=

=

a

 
(8)

GLOBAL INFORMATION INFRASTRUCTURE AND INTERNET PROTOCOL ASPECTS

	
	

	GLOBAL INFORMATION INFRASTRUCTURE
	

	General
	Y.100–Y.199

	Services, applications and middleware
	Y.200–Y.299

	Network aspects
	Y.300–Y.399

	Interfaces and protocols
	Y.400–Y.499

	Numbering, addressing and naming
	Y.500–Y.599

	Operation, administration and maintenance
	Y.600–Y.699

	Security
	Y.700–Y.799

	Performances
	Y.800–Y.899

	INTERNET PROTOCOL ASPECTS
	

	General
	Y.1000–Y.1099

	Services and applications
	Y.1100–Y.1199

	Architecture, access, network capabilities and resource management
	Y.1200–Y.1299

	Transport
	Y.1300–Y.1399

	Interworking
	Y.1400–Y.1499

	Quality of service and network performance
	Y.1500–Y.1599

	Signalling
	Y.1600–Y.1699

	Operation, administration and maintenance
	Y.1700–Y.1799

	Charging
	Y.1800–Y.1899

	
	


For further details, please refer to the list of ITU-T Recommendations.

	SERIES OF ITU-T RECOMMENDATIONS

	Series A
	Organization of the work of ITU-T

	Series B
	Means of expression: definitions, symbols, classification

	Series C
	General telecommunication statistics

	Series D
	General tariff principles

	Series E
	Overall network operation, telephone service, service operation and human factors

	Series F
	Non-telephone telecommunication services

	Series G
	Transmission systems and media, digital systems and networks

	Series H
	Audiovisual and multimedia systems

	Series I
	Integrated services digital network

	Series J
	Cable networks and transmission of television, sound programme and other multimedia signals

	Series K
	Protection against interference

	Series L
	Construction, installation and protection of cables and other elements of outside plant

	Series M
	TMN and network maintenance: international transmission systems, telephone circuits, telegraphy, facsimile and leased circuits

	Series N
	Maintenance: international sound programme and television transmission circuits

	Series O
	Specifications of measuring equipment

	Series P
	Telephone transmission quality, telephone installations, local line networks

	Series Q
	Switching and signalling

	Series R
	Telegraph transmission

	Series S
	Telegraph services terminal equipment

	Series T
	Terminals for telematic services

	Series U
	Telegraph switching

	Series V
	Data communication over the telephone network

	Series X
	Data networks and open system communications

	Series Y
	Global information infrastructure and Internet protocol aspects

	Series Z
	Languages and general software aspects for telecommunication systems

	
	


_1094564385.unknown

_1094564492.unknown

_1094564495.unknown

_1094564536.unknown

_1094564436.unknown

_1094564173.unknown

_1094564376.unknown

_1094564009.unknown

