��

INTERNATIONAL  TELECOMMUNICATION  UNION��





ITU-T�G.853.2��TELECOMMUNICATION�STANDARDIZATION  SECTOR�OF  ITU�(11/96)   ��









SERIES G: TRANSMISSION SYSTEMS AND MEDIA, DIGITAL SYSTEMS AND NETWORKS

Digital transmission systems – Digital networks – Telecommunications management network





Subnetwork connection management information viewpoint



ITU-T  Recommendation  G.853.2

(Previously  CCITT  Recommendation)



�

ITU-T  G-SERIES  RECOMMENDATIONS

TRANSMISSION SYSTEMS AND MEDIA, DIGITAL SYSTEMS AND NETWORKS



���INTERNATIONAL TELEPHONE CONNECTIONS AND CIRCUITS�G.100–G.199��INTERNATIONAL ANALOGUE CARRIER SYSTEM��GENERAL CHARACTERISTICS COMMON TO ALL ANALOGUE CARRIER-TRANSMISSION SYSTEMS�G.200–G.299��INDIVIDUAL CHARACTERISTICS OF INTERNATIONAL CARRIER TELEPHONE SYSTEMS ON METALLIC LINES�G.300–G.399��GENERAL CHARACTERISTICS OF INTERNATIONAL CARRIER TELEPHONE SYSTEMS ON RADIO-RELAY OR SATELLITE LINKS AND INTERCONNECTION WITH METALLIC LINES�G.400–G.449��COORDINATION OF RADIOTELEPHONY AND LINE TELEPHONY�G.450–G.499��TRANSMISSION MEDIA CHARACTERISTICS��DIGITAL TRANSMISSION SYSTEMS��TERMINAL EQUIPMENTS�G.700–G.799��General�G.700–G.709��Coding of analogue signals by pulse code modulation�G.710–G.719��Coding of analogue signals by methods other than PCM�G.720–G.729��Principal characteristics of primary multiplex equipment�G.730–G.739��Principal characteristics of second order multiplex equipment�G.740–G.749��Principal characteristics of higher order multiplex equipment�G.750–G.759��Principal characteristics of transcoder and digital multiplication equipment�G.760–G.769��Operations, administration and maintenance features of transmission equipment�G.770–G.779��Principal characteristics of multiplexing equipment for the synchronous digital hierarchy�G.780–G.789��Other terminal equipment�G.790–G.799��DIGITAL NETWORKS�G.800–G.899��General aspects�G.800–G.809��Design objectives for digital networks�G.810–G.819��Quality and availability targets�G.820–G.829��Network capabilities and functions�G.830–G.839��SDH network characteristics�G.840–G.849��Telecommunications management network�G.850–G.859��DIGITAL SECTIONS AND DIGITAL LINE SYSTEM�G.900–G.999��General�G.900–G.909��Parameters for optical fibre cable systems�G.910–G.919��Digital sections at hierarchical bit rates based on a bit rate of 2048 kbit/s�G.920–G.929��Digital line transmission systems on cable at non-hierarchical bit rates�G.930–G.939��Digital line systems provided by FDM transmission bearers�G.940–G.949��Digital line systems�G.950–G.959��Digital section and digital transmission systems for customer access to ISDN�G.960–G.969��Optical fibre submarine cable systems�G.970–G.979��Optical line systems for local and access networks�G.980–G.999�����For further details, please refer to ITU-T List of Recommendations.

�ITU-T  RECOMMENDATION  G.853.2



subnetwork connection managemEnT information viewpoint��



Summary

This Recommendation provides the information viewpoint of all the already defined management services relevant to subnetwork connections established across a subnetwork.��



Source

ITU-T Recommendation G.853.2 was prepared by ITU-T Study Group 15 (1993-1996) and was approved under the WTSC Resolution No. 1 procedure on the 8th of November 1996.��





��

�FOREWORD

ITU (International Telecommunication Union) is the United Nations Specialized Agency in the field of telecommunications. The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of the ITU. The ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Conference (WTSC), which meets every four years, establishes the topics for study by the ITU�T Study Groups which, in their turn, produce Recommendations on these topics.

The approval of Recommendations by the Members of the ITU�T is covered by the procedure laid down in WTSC Resolution No. 1.

In some areas of information technology which fall within ITU-T’s purview, the necessary standards are prepared on a collaborative basis with ISO and IEC.











NOTE

In this Recommendation, the expression "Administration" is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.







INTELLECTUAL PROPERTY RIGHTS

The ITU draws attention to the possibility that the practice or implementation of this Recommendation may involve the use of a claimed Intellectual Property Right. The ITU takes no position concerning the evidence, validity or applicability of claimed Intellectual Property Rights, whether asserted by ITU members or others outside of the Recommendation development process.

As of the date of approval of this Recommendation, the ITU had/had not received notice of intellectual property, protected by patents, which may be required to implement this Recommendation. However, implementors are cautioned that this may not represent the latest information and are therefore strongly urged to consult the TSB patent database.











ã  ITU  1997

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

�CONTENTS

	Page

1	Scope		1

2	References		1

3	Definitions		1

4	Abbreviations		1

5	Subnetwork connection management information viewpoint		1

Annex A – Information viewpoint for the simple subnetwork connection management		2

A.1	Diagrams of information object and relationship classes		2

A.2	Label references		2

A.3	Import		2

A.4	Information object class definitions		3

A.4.1	ssccSubnetwork		3

A.4.2	ssccSubnetworkConnection		4

A.4.3	ssccSubnetworkTPBidirectional		4

A.4.4	ssccSubnetworkTPSink		5

A.4.5	ssccSubnetworkTPSource		6

A.4.6	serviceCharacteristics		6

A.5	Information relationship definitions		7

A.5.1	subnetworkConnectionHasTSC		7

A.6	Static schemas		8

A.6.1	ssccNotConnected		8

A.6.2	ssccConnected		9

A.7	Dynamic schemas		10

A.7.1	ssccNotConnected_ssccConnected		10

A.7.2	ssccConnected_ssccNotConnected		10

Annex B – Information viewpoint for the simple failure monitoring		10

B.1	Diagrams of information object and relationship classes		10

B.2	Label references		10

B.3	Import		10

B.4	Information object classes definition		11

B.4.1	monitoredEntity		11

B.5	Information relationships definition		12

�	Page

B.6	Static schemas		12

B.6.1	inReportFailureOff		12

B.6.2	reportFailureOnEnabled		12

B.6.3	reportFailureOnDisabled		12

B.6.4	reportFailureOnEnabledToDisabled:		12

B.6.5	reportFailureOnDisabledToEnabled:		13

B.7	Attributes		13

B.7.1	reportFailureStatus		13

Annex C – Information viewpoint for the simple monitored subnetwork connection		14

C.1	Diagrams of information object and relationship classes		14

C.2	Import		14

C.3	Information object classes definition		15

C.3.1	monitoredSubnetworkConnection		15



�Recommendation G.853.2

Subnetwork Connection Management Information Viewpoint

(Geneva, 1996)

1	Scope

This Recommendation provides the information viewpoint of all the already defined management services relevant to subnetwork connections established across a subnetwork.

2	References

The following ITU-T Recommendations contain provisions which through reference in this text constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations are subject to revision; all users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations. A list of the currently valid ITU-T Recommendations is regularly published.

ITU-T Recommendation G.851.1 (1996), Management of the transport network – Application of the RM-ODP framework.

ITU-T Recommendation G.852.1 (1996), Management of the transport network – Enterprise viewpoint for simple subnetwork connection management.

ITU-T Recommendation G.853.1 (1996), Common elements of the information viewpoint for the management of a transport network.

3	Definitions

To be extended if needed.

4	Abbreviations

To be extended if needed.

5	Subnetwork connection management information viewpoint

This Recommendation provides the information viewpoint of all the already defined management services relevant to subnetwork connections established across a subnetwork.

Each management service information viewpoint is presented in an annex.

This Recommendation presents in:

–	Annex A: the information viewpoint for the simple subnetwork connection management;

–	Annex B: the information viewpoint of the simple failure monitoring service;

–	Annex C: the information viewpoint for the simple monitored subnetwork connection management.

�Annex A

Information viewpoint for the simple subnetwork connection management

A.1	Diagrams of information object and relationship classes

� EMBED Designer.Drawing.7  ���

Inheritance diagram

A.2	Label references



Full label reference�Local label reference��<"Rec. G.853.1",INFORMATION_OBJECT: subnetwork>�<subnetwork>��<"Rec. G.853.1",INFORMATION_OBJECT: subnetworkConnection>�<subnetworkConnection>��<"Rec. G.853.1",INFORMATION_OBJECT: subnetworkTPSink>�<subnetworkTPSink>��<"Rec. G.853.1",INFORMATION_OBJECT: subnetworkTPSource>�<subnetworkTPSource>��A.3	Import



<"Rec. G.853.1",INFORMATION_RELATIONSHIP:subnetworkIsDelimitedBy>



This information concept is related to the following enterprise entities:



<"Rec. G.852.1",COMMUNITY:sscc,PURPOSE>

<"Rec. G.852.1",COMMUNITY:sscc,ACTION:sscc1,OBLIGATION:OBLG_1>





<"Rec. G.853.1",INFORMATION_RELATIONSHIP:

subnetworkConnectionIsTerminatedByPointToPoint>



This information concept is related to the following enterprise entities:



<"Rec. G.852.1",COMMUNITY:sscc,PURPOSE>

<"Rec. G.852.1",COMMUNITY:sscc,ACTION:sscc1,OBLIGATION:OBLG_2>

<"Rec. G.852.1",COMMUNITY:sscc,ACTION:sscc1,OBLIGATION:PROH_1>





<"Rec. G.853.1",INFORMATION_RELATIONSHIP:subnetworkHasSubnetworkConnections>



�This information concept is related to the following enterprise entities:



<"Rec. G.852.1",COMMUNITY:sscc,PURPOSE>

<"Rec. G.852.1",COMMUNITY:sscc,ACTION:sscc1,OBLIGATION:OBLG_3>

<"Rec. G.852.1",COMMUNITY:sscc,ACTION:sscc1,OBLIGATION:OBLG_4>





<"Rec. G.853.1",ATTRIBUTE:userLabel>



This information concept is related to the following enterprise entities:



<"Rec. G.852.1",COMMUNITY:sscc,ACTION:sscc1,OBLIGATION:OBLG_3>

<"Rec. G.852.1",COMMUNITY:sscc,ACTION:sscc1,OBLIGATION:PERM_1>

<"Rec. G.852.1",COMMUNITY:sscc,ACTION:sscc1,OBLIGATION:OBLG_4>

<"Rec. G.852.1",COMMUNITY:sscc,ACTION:sscc2,OBLIGATION:OBLG_2>

A.4	Information object class definitions

A.4.1	ssccSubnetwork

This information concept is related to the following enterprise entity:

<"Rec. G.852.1",COMMUNITY:sscc,ROLE:sn>.

A.4.1.1	Informal description



DEFINITION

	"This object class is derived from subnetwork."

RELATIONSHIP

	"<subnetworkIsDelimitedBy>,

	<subnetworkHasSubnetworkConnections>"

A.4.1.2	Semi-formal description



ssccSubnetwork INFORMATION OBJECT CLASS

	DERIVED FROM G.853-1: subnetwork;

	CHARACTERIZED BY

		ssccSubnetworkPackage PACKAGE

			BEHAVIOUR

			ssccsubnetworkBehaviour BEHAVIOUR

			DEFINED AS

			"<DEFINITION>,

			<subnetworkIsDelimitedBy>

			<subnetworkHasSubnetworkConnections>"

	;;

A.4.1.3	Formal description

	ssccSubNetwork_Static	

ssccSubNetwork : F OBJECT

subNetwork_Static

ssccSubNetwork �SYMBOL 205 \f "Symbol"� subNetwork

	ssccSubNetwork_Dynamic	

�SYMBOL 68 \f "Symbol"� ssccSubNetwork_Static

subNetwork_Dynamic

�A.4.2	ssccSubnetworkConnection

This information concept is related to the following enterprise entity:

<"Rec. G.852.1",COMMUNITY:sscc,ROLE:snc>.

A.4.2.1	Informal description



DEFINITION

	"This object class is derived from subnetworkConnection."

ATTRIBUTE

	userLabel

		"This attribute value is used to identify the ssccSubnetworkConnection."

RELATIONSHIP

	"<subnetworkIsDelimitedBy>,

	<subnetworkConnectionIsTerminatedByPointToPoint>,

	<subnetworkConnectionHasTSC>,

	<subnetworkHasSubnetworkConnections>"

A.4.2.2	Semi-formal description



ssccSubnetworkConnection INFORMATION OBJECT CLASS

	DERIVED FROM G.853-1: subnetworkConnection;

	CHARACTERIZED BY

		ssccSubnetworkConnectionPackagePACKAGE

			BEHAVIOUR

				ssccSubnetworkConnectionBehaviour BEHAVIOUR

				DEFINED AS

					"DEFINITION,

					<subnetworkIsDelimitedBy>,

					<subnetworkConnectionIsTerminatedByPointToPoint>,

					<subnetworkConnectionHasTSC>,

					<subnetworkHasSubnetworkConnections>"

		ATTRIBUTES

			userLabel;

		;;

A.4.2.3	Formal description

	ssccSubNetworkConnection_Static	

ssccSubNetworkConnection : F OBJECT

subNetworkConnection_Static

userLabel_Static

ssccSubNetworkConnection �SYMBOL 205 \f "Symbol"� subNetworkConnection



ssccSubNetworkConnection �SYMBOL 205 \f "Symbol"� dom userLabel

	ssccSubNetworkConnection_Dynamic	

�SYMBOL 68 \f "Symbol"� ssccSubNetworkConnection_Static

subNetworkConnection_Dynamic

userLabel_Dynamic

A.4.3	ssccSubnetworkTPBidirectional

This information concept is related to the following enterprise entity:

<"Rec. G.852.1",COMMUNITY:sscc,ROLE:port>.

�A.4.3.1	Informal description



DEFINITION

	"This object class is derived from ssccSubnetworkTPSink and ssccSubnetworkTPSource."

A.4.3.2	Semi-formal description



ssccSubnetworkTPBidirectional INFORMATION OBJECT CLASS

	DERIVED FROM ssccSubnetworkTPSink, ssccSubnetworkTPSource;

	CHARACTERIZED BY

		ssccSubnetworkTPBidirectional PackagePACKAGE

			BEHAVIOUR

			ssccSubnetworkTPBidirectional Behaviour BEHAVIOUR

			DEFINED AS

			"DEFINITION"

		;;

A.4.3.3	Formal description

	ssccSubNetworkTpBidirectional_Static	

ssccSubNetworkTpBidirectional : F OBJECT

ssccSubNetworkTpSink_Static

ssccSubNetworkTpSource_Static

ssccSubNetworkTpBidirectional �SYMBOL 205 \f "Symbol"� ssccSubNetworkTpSink �SYMBOL 200 \f "Symbol"� ssccSubNetworkTpSource

	ssccSubNetworkTpBidirectional_Dynamic	

�SYMBOL 68 \f "Symbol"� ssccSubNetworkTpBidirectional_Static

ssccSubNetworkTpSink_Dynamic

ssccSubNetworkTpSource_Dynamic

A.4.4	ssccSubnetworkTPSink

This information concept is related to the following enterprise entity:

<"Rec. G.852.1",COMMUNITY:sscc,ROLE:port>.

A.4.4.1	Informal description



DEFINITION

"This object class reflects is derived from subnetworkTPSink."



RELATIONSHIP

"<subnetworkIsDelimitedBy>,

<subnetworkConnectionIsTerminatedByPointToPoint>"

A.4.4.2	Semi-formal description



ssccSubnetworkTPSink INFORMATION OBJECT CLASS

	DERIVED FROM G.853-1: subnetworkTPSink;

	CHARACTERIZED BY

		ssccSubnetworkTPSinkPackagePACKAGE

			BEHAVIOUR

			ssccSubnetworkTPSinkBehaviour BEHAVIOUR

			DEFINED AS

			"DEFINITION,

			<subnetworkIsDelimitedBy>,

			<subnetworkConnectionIsTerminatedByPointToPoint>."

		;;

�A.4.4.3	Formal description

	ssccSubNetworkTpSink_Static	

ssccSubNetworkTpSink : F OBJECT

subNetworkTpSink_Static

ssccSubNetworkTpSink �SYMBOL 205 \f "Symbol"� subNetworkTpSink

	ssccSubNetworkTpSink_Dynamic	

�SYMBOL 68 \f "Symbol"� ssccSubNetworkTpSink_Static

subNetworkTpSink_Dynamic

A.4.5	ssccSubnetworkTPSource

This information concept is related to the following enterprise entity:

<"Rec. G.852.1",COMMUNITY:sscc,ROLE:port>.

A.4.5.1	Informal description



DEFINITION

	"This object class reflects is derived from subnetworkTPSource."

RELATIONSHIP

	"<subnetworkIsDelimitedBy>,

	<subnetworkConnectionIsTerminatedByPointToPoint>"

A.4.5.2	Semi-formal description



ssccSubnetworkTPSource INFORMATION OBJECT CLASS

	DERIVED FROM G.853-1: subnetworkTPSource;

	CHARACTERIZED BY

		ssccSubnetworkTPSourcePackagePACKAGE

			BEHAVIOUR

			ssccSubnetworkTPSourceBehaviour BEHAVIOUR

			DEFINED AS

			"DEFINITION,

			<subnetworkIsDelimitedBy>,

			<subnetworkConnectionIsTerminatedByPointToPoint>"

		;;

A.4.5.3	Formal description

	ssccSubNetworkTpSource_Static	

ssccSubNetworkTpSource : F OBJECT

subNetworkTpSource_Static

ssccSubNetworkTpSource �SYMBOL 205 \f "Symbol"� subNetworkTpSource

	ssccSubNetworkTpSource_Dynamic	

�SYMBOL 68 \f "Symbol"� ssccSubNetworkTpSource_Static

subNetworkTpSource_Dynamic

A.4.6	serviceCharacteristics

This information concept is related to the following enterprise entity:

<"Rec. G.852.1",COMMUNITY:sscc,ACTION:sscc1,PERMISSION:PERM_3>.

�A.4.6.1	Informal description



DEFINITION



	"This object class reflects all the characteristics associated with the requested quality of the transport service relevant to the subnetwork connection establishment.

	This object class will be refined due to the technological dependent characteristics. "

RELATIONSHIP

	"<subnetworkConnectionHasTSC>."

A.4.6.2	Semi-formal description



serviceCharacteristics INFORMATION OBJECT CLASS

	DERIVED FROM networkInformationTop;

	CHARACTERIZED BY

		serviceCharacteristicsPACKAGE

			BEHAVIOUR

			serviceCharacteristicsBehaviour BEHAVIOUR

			DEFINED AS

			"DEFINITION,

			<subnetworkConnectionHasTSC>.";;

A.4.6.3	Formal description

	serviceCharacteristics_Static	

serviceCharacteristics : F OBJECT

networkInformationTop_Static

serviceCharacteristics �SYMBOL 205 \f "Symbol"� networkInformationTop

	serviceCharacteristics_Dynamic	

�SYMBOL 68 \f "Symbol"� serviceCharacteristics_Static

networkInformationTop_Dynamic

A.5	Information relationship definitions

A.5.1	subnetworkConnectionHasTSC

This information concept is related to the following enterprise entity:

<"Rec. G.852.1",COMMUNITY:sscc,ACTION:sscc1,PERMISSION:PERM_3>.

A.5.1.1	Informal description



DEFINITION

	"The subnetworkConnectionHasTSC relationship type describes the association between a subnetwork connection and the related quality of transport service characteristics."

ROLE

	transportQualified

		"Played by instances of the ssccSubnetworkConnection information object class and subclasses."

	transportQualifier

		"Played by an instance of the serviceCharacteristics object class."

INVARIANT

	inv_1

		"Several objects playing the transportQualified role may be involved in the relationship."

	inv_2

		"Only one object playing the transportQualifier role may be involved in the relationship."

�A.5.1.2	Semi-formal description

subnetworkConnectionHasTSC RELATIONSHIP CLASS

	BEHAVIOUR

		subnetworkConnectionHasTSC Behaviour BEHAVIOUR

			DEFINED AS 

			"DEFINITION";;

	ROLE transportQualified

		PERMITTED-ROLE-CARDINALITY-CONSTRAINT (1..N)

		COMPATIBLE WITH ssccSubnetworkConnection and SUBCLASSES;

	ROLE transportQualifier

		PERMITTED-ROLE-CARDINALITY-CONSTRAINT (1..1)

		COMPATIBLE WITH serviceCharacteristics;

A.5.1.3	Formal description

	subnetworkConnectionHasTSC_Static	

subnetworkConnectionHasTSC : F RELATIONSHIP

transportQualified : RELATIONSHIP �SYMBOL 174 \f "Symbol"� F OBJECT

transportQualifier : RELATIONSHIP �SYMBOL 174 \f "Symbol"� F OBJECT

ssccSubNetworkConnection_Static

serviceCharacteristics_Static

subnetworkConnectionHasTSC �SYMBOL 205 \f "Symbol"� dom transportQualified



subnetworkConnectionHasTSC �SYMBOL 205 \f "Symbol"� dom transportQualifier



�SYMBOL 34 \f "Symbol"� R : subnetworkConnectionHasTSC �SYMBOL 183 \f "Symbol"�

	transportQualified(R) �SYMBOL 205 \f "Symbol"� ssccSubNetworkConnection �SYMBOL 217 \f "Symbol"�

	transportQuantifier(R) �SYMBOL 205 \f "Symbol"� serviceCharacteristics



�SYMBOL 34 \f "Symbol"� R : subnetworkConnectionHasTSC �SYMBOL 183 \f "Symbol"� #( transportQualified(R)) �SYMBOL 163 \f "Symbol"� 1

	subnetworkConnectionHasTSC_Dynamic	

�SYMBOL 68 \f "Symbol"� subnetworkConnectionHasTSC_Static

ssccSubNetworkConnection_Dynamic

serviceCharacteristics_Dynamic

�SYMBOL 34 \f "Symbol"� R : RELATIONSHIP | R �SYMBOL 206 \f "Symbol"� subnetworkConnectionHasTSC �SYMBOL 200 \f "Symbol"� subnetworkConnectionHasTSC’ �SYMBOL 183 \f "Symbol"�

	transportQualified’(R) = transportQualified(R) �SYMBOL 217 \f "Symbol"�

	transportQualifier’(R) = transportQualifier(R)

A.6	Static schemas

A.6.1	ssccNotConnected

This information concept is related to the following enterprise entities:

<"Rec. G.852.1",COMMUNITY:sscc,ACTION:sscc1,OBLIGATION:OBLG_2>,

<"Rec. G.852.1",COMMUNITY:sscc,ACTION:sscc1,OBLIGATION:PROH_1>,

<"Rec. G.852.1",COMMUNITY:sscc,ACTION:sscc2,OBLIGATION:OBLG_2>.

A.6.1.1	Informal description



DEFINITION

	"The ssccNotConnected schema defines a schema type with two non-connected subnetworkTPinformation objects subtypes candidates to the point to point connection management service."

�ROLE

	involvedSubnetwork

		"Played by an instance of the ssccSubnetwork information object type or subtype."

	potentialAEnd

		"Played by an instance of the ssccSubnetworkTPSink, ssccSubnetworkTPSource or ssccSubnetworkTTPBidirectional object types or subtypes."

	potentialZEnd

		"Played by an instance of the ssccSubnetworkTPSink, ssccSubnetworkTPSource or ssccSubnetworkTTPBidirectional object types or subtypes."

INVARIANT

	inv_1

		"The objects playing the potentialAEnd and potentialZEnd roles are involved in an instance of the subnetworkIsDelimitedBy relationship type with the object playing the role involvedSubnetwork."

	inv_2

		"The object playing the potentialAEnd role is not involved in any instance of the subnetworkConnectionIsTerminatedByPointToPoint relationship type and subtypes."

	inv_3

		"The object playing the potentialZEnd role is not involved in any instance of the subnetworkConnectionIsTerminatedByPointToPoint relationship type and subtypes."

A.6.2	ssccConnected

This information concept is related to the following enterprise entities:

<"Rec. G.852.1",COMMUNITY:sscc,ACTION:sscc1,PERMISSION:PERM_3>

<"Rec. G.852.1",COMMUNITY:sscc,ACTION:sscc1,PERMISSION:PERM_1>

<"Rec. G.852.1",COMMUNITY:sscc,ACTION:sscc1,OBLIGATION:OBLG_3>

<"Rec. G.852.1",COMMUNITY:sscc,ACTION:sscc1,OBLIGATION:OBLG_4>.

A.6.2.1	Informal description



DEFINITION

	"The ssccConnected schema defines the schema type of two connected ssccSubnetworkTPinformation objects candidates to the point to point connection management service."

ROLE

	involvedPointToPointSubnetworkConnection

		"Played by an instance of the ssccSubnetworkConnection object type or subtype."

	connectedAEnd

		"Played by an instance of the ssccSubnetworkTPSink, ssccSubnetworkTPSource or ssccSubnetworkTTPBidirectional object types or subtypes"

	connectedZEnd

		"Played by an instance of the ssccSubnetworkTPSink, ssccSubnetworkTPSource or ssccSubnetworkTTPBidirectional object types or subtypes"

	involvedServiceCharacteristics

		"Played by an instance of the serviceCharacteristics object type or subtype"

	involvedSubnetwork

		"Played by an instance of the ssccSubnetwork object type or subtype"

INVARIANT

	inv_1

		"The information objects playing respectively the roles involvedSubnetworkConnection, connectedAEnd and connectedZEnd in a given instance of this relationship must simultaneously play respectively the roles transportEntity, A_end and Z_end in an instance of the relationship subnetworkConnectionIsTerminatedByPointToPoint."

	inv_2

		"The information object playing the role involvedSubnetworkConnection in a given instance of this relationship must simultaneously play the role transportQualified in an instance of the relationship subnetworkConnectionHasTSC."

	inv_3

		"The information objects playing respectively the roles involvedSubnetworkConnection and involvedSubnetwork and in a given instance of this relationship must simultaneously play respectively the roles element and container in an instance of the relationship subnetworkHasSubnetworkConnections."

�	inv_4

		"The value of the userLabel of the information object playing the role involvedPointToPointSubnetworkConnection shall be unique and greater than zero in the context of the information object playing the role involvedSubnetwork."

A.7	Dynamic schemas

A.7.1	ssccNotConnected_ssccConnected

This information concept is related to the following enterprise entity:

<"Rec. G.852.1",COMMUNITY:sscc,ACTION:sscc1>.

A.7.1.1	Informal description



DEFINITION

	"This dynamic schema expresses the transition of two non-connected extremities towards two connected extremities."

PRE-CONDITIONS <ssccNotConnected>;



POST-CONDITIONS <ssccConnected>;

A.7.2	ssccConnected_ssccNotConnected

This information concept is related to the following enterprise entity.

<"Rec. G.852.1",COMMUNITY:sscc,ACTION:sscc2>.

A.7.2.1	Informal description



DEFINITION

	"This dynamic schema expresses the transition of two connected extremities towards two non connected extremities."



PRE-CONDITIONS <ssccConnected>;



POST-CONDITIONS <ssccNotConnected>;

Annex B

Information viewpoint for the simple failure monitoring

B.1	Diagrams of information object and relationship classes

None.

B.2	Label references

Full label reference�Local label reference��<"Rec. G.853.1",INFORMATION_OBJECT:networkInformationTop>�<networkInformationTop>��B.3	Import



<"Rec. G.853.1",ATTRIBUTE: operationalState>



This information concept is related to the following enterprise entities:

<"Rec. G.852.1",COMMUNITY:sfm,PURPOSE>

<"Rec. G.852.1",COMMUNITY:sfm,ACTION,sfm1>

<"Rec. G.852.1",COMMUNITY:sfm,ACTION,sfm2>

�B.4	Information object classes definition

B.4.1	monitoredEntity

This information concept is related to the following enterprise entity:

<"Rec. G.852.1",COMMUNITY:sfm,ROLE:monitored_entity>.

B.4.1.1	Informal description



DEFINITION 

	"This object class reflects the ressource for which a monitoring service is defined." 

ATTRIBUTE 

	operationalState

		"This attribute reflects the operability of the monitored ressource in terms of transport aspects."

	reportFailureStatus

		"This attribute reflects the reporting failure status of the monitored ressource."

INVARIANT

	inv_1

		"The value enabled of the operationalState attribute means that the resource is able to perform partially or fully its transport function."

	inv_2

		"The value disabled of the operationalState attribute means that the resource is totally unable to perform its transport function."

B.4.1.2	Semi-formal description



monitoredEntity INFORMATION OBJECT CLASS

	DERIVED FROM networkInformationTop

	CHARACTERIZED BY

		ms_monitoredEntityPACKAGE

			BEHAVIOUR

			monitoredEntityPackageBehaviour BEHAVIOUR

			DEFINED AS

			"DEFINITION";;

			ATTRIBUTES

			operationalState,

			reportFailureStatus;;;

B.4.1.3	Formal description

	monitoredEntity_Static	

monitoredEntity : F OBJECT

networkInformationTop_Static

operationalState_Static

reportFailureStatus_Static

monitoredEntity �SYMBOL 205 \f "Symbol"� networkInformationTop



monitoredEntity �SYMBOL 205 \f "Symbol"� dom operationalState



monitoredEntity �SYMBOL 205 \f "Symbol"� dom reportFailureStatus

	monitoredEntity_Dynamic	

�SYMBOL 68 \f "Symbol"� monitoredEntity_Static

networkInformationTop_Dynamic

operationalState_Dynamic

reportFailureStatus_Dynamic

�B.5	Information relationships definition

None.

B.6	Static schemas

B.6.1	inReportFailureOff

This information concept is related to the following enterprise entity:

<"Rec. G.852.1",COMMUNITY:sfm,ACTION:sfm3, OBLIGATION:OBLG_3>.

B.6.1.1	Informal description



DEFINITION

	"The inReportFailureOff schema defines the schema type which reflects that a monitoredEntity has the value reportFailureOff."

INVARIANT

	inv_1

		"The value of the reportFailureStatus attribute is reportFailureOff."

B.6.2	reportFailureOnEnabled 

This information concept is related to the following enterprise entities:

<"Rec. G.852.1",COMMUNITY:sfm,ACTION,sfm3>,

<"Rec. G.852.1",COMMUNITY:sfm,ACTION,sfm4>.

B.6.2.1	Informal description



DEFINITION

	"The reportFailureOnEnabled schema defines the schema type which reflects that a monitoredEntity is under reportFailureOn and enabled conditions."

INVARIANT

	inv_1

		"The value of the reportFailureStatus attribute is reportFailureOff"

	inv_2

		"The value of the operationalState attribute is enabled"

B.6.3	reportFailureOnDisabled 

This information concept is related to the following enterprise entities:

<"Rec. G.852.1",COMMUNITY:sfm,ACTION,sfm3>,

<"Rec. G.852.1",COMMUNITY:sfm,ACTION,sfm4>.

B.6.3.1	Informal description



DEFINITION

	"The reportFailureOnDisabled schema defines the schema type which reflects that a monitoredEntity is under reportFailureOn and disabled conditions."

INVARIANT

	inv_1

		"The value of the reportFailureStatus attribute is reportFailureOff."

	inv_2

		"The value of the operationalState attribute is enabled."

B.6.3.2	Dynamic schemas

B.6.4	reportFailureOnEnabledToDisabled:

This information concept is related to the following enterprise entity:

"<"Rec. G.852.1",COMMUNITY:sfm,ACTION,sfm1>.

�B.6.4.1	Informal description



DEFINITION

	"This dynamic schema expresses the transition of the monitored entity from an enabled state to a disabled state under a report failure status condition"

PRE-CONDITIONS <reportFailureOnEnabled>;

POST-CONDITIONS <reportFailureOnDisabled>;;

B.6.5	reportFailureOnDisabledToEnabled:

This information concept is related to the following enterprise entity:

<"Rec. G.852.1",COMMUNITY:sfm,ACTION,sfm2>.

B.6.5.1	Informal description



DEFINITION

	"This dynamic schema expresses the transition of the monitored entity from an disabled state to a enabled state under a report failure status condition."



PRE-CONDITIONS <reportFailureOnDisabled>;



POST-CONDITIONS <reportFailureOnEnabled>;;

B.7	Attributes

B.7.1	reportFailureStatus

This information concept is related to the following enterprise entities:

<"Rec. G.852.1",COMMUNITY:sfm,ACTION:sfm3>,

<"Rec. G.852.1",COMMUNITY:sfm,ACTION:sfm4>.

B.7.1.1	Informal description



DEFINITION:

	"The reportFailureStatus attribute reflects the state concerning the report of failure."



STATE

	reportFailureOn

		"Any change in the failure status of the ressource shall be reported."

	reportFailureOff

		"No change in the failure status of the ressource shall be reported."



TRANSITION

	reportFailureOnToReportFailureOff

		"The value of the reportFailureStatus changes from reportFailureOn to reportFailureOff".

	reportFailureOffToReportFailureOn

		"The value of the reportFailureStatus changes from reportFailureOff to reportFailureOn".

B.7.1.2	Semi formal description



reportFailure ATTRIBUTE

	BEHAVIOUR

	DEFINED AS

		"DEFINITION

		STATE

			<reportFailureOn>,

			<reportFailureOff>

�		TRANSITION

			<reportFailureOnToReportFailureOff>

			<reportFailureOffToReportFailureOn>";;

B.7.1.3	Formal description



ReportFailureStatus ::= reportFailureOn | reportFailureOff

	reportFailureStatus_Static	

reportFailureStatus : OBJECT �SYMBOL 174 \f "Symbol"� ReportFailureStatus

	reportFailureStatus_Transition	

�SYMBOL 68 \f "Symbol"� reportFailureStatus_Static

Annex C

Information viewpoint for the simple monitored subnetwork connection

C.1	Diagrams of information object and relationship classes

� EMBED Designer.Drawing.7  ���

Inheritance diagram

C.2	Import



<"Rec. G.853.1",INFORMATION_RELATIONSHIP:subnetworkIsDelimitedBy>

<"Rec. G.853.1",

INFORMATION_RELATIONSHIP:subnetworkConnectionIsTerminatedByPointToPoint>

<"Rec. G.853.1",INFORMATION_RELATIONSHIP:subnetworkHasSubnetworkConnections>

<INFORMATION_OBJECT:ssccSubNetworkTPSink>

<INFORMATION_OBJECT:ssccSubNetworkTPSource>

<INFORMATION_OBJECT:ssccSubNetworkTPBidirectional>

<INFORMATION_OBJECT:ssccSubNetwork> 

<STATIC_SCHEMA:ssccConnected>

<STATIC_SCHEMA:ssccNotConnected>

<DYNAMIC_SCHEMA:ssccConnected_ssccNotConnected>

<DYNAMIC_SCHEMA:ssccNotConnected_ssccConnected>

<STATIC_SCHEMA:inReportFailureOff>

<DYNAMIC_SCHEMA:reportFailureOnEnabledToDisabled :>

<DYNAMIC_SCHEMA:reportFailureOnDisabledToEnabled :>

Those information concepts have to be related with the following enterprise concern:

<"Rec. G.852.1",COMMUNITY:smscc,PURPOSE>

�C.3	Information object classes definition

C.3.1	monitoredSubnetworkConnection

This information concept is related to the following enterprise entity:

<"Rec. G.852.1",COMMUNITY:smscc, ROLE:monitored_snc >.

C.3.1.1	Informal description



DEFINITION

	"This object class inherits from ssccSubnetworkConnection and monitoredEntity object classes."

C.3.1.2	Semi-formal description



monitoredSubnetworkConnection INFORMATION OBJECT CLASS

	DERIVED FROM ssccSubnetworkConnection, monitoredEntity

	CHARACTERIZED BY

		monitoredSubnetworkConnectionPackage PACKAGE

			BEHAVIOUR

			monitoredSubnetworkConnectionBehaviour BEHAVIOUR

			DEFINED AS

			"DEFINITION";;

C.3.1.3	Formal description

	monitoredSubnetworkConnection_Static	

monitoredSubnetworkConnection : F OBJECT

ssccSubNetworkConnection_Static

monitoredEntity_Static

monitoredSubnetworkConnection �SYMBOL 205 \f "Symbol"� ssccSubNetworkConnection



monitoredSubnetworkConnection �SYMBOL 205 \f "Symbol"� monitoredEntity

	monitoredSubnetworkConnection_Dynamic	

�SYMBOL 68 \f "Symbol"� monitoredSubnetworkConnection_Static

ssccSubNetworkConnection_Dynamic

monitoredEntity_Dynamic



�

ITU-T  RECOMMENDATIONS  SERIES��Series A�Organization of the work of the ITU-T��Series B�Means of expression: definitions, symbols, classification��Series C�General telecommunication statistics��Series D�General tariff principles��Series E�Overall network operation, telephone service, service operation and human factors��Series F�Non-telephone telecommunication services��Series G�Transmission systems and media, digital systems and networks��Series H�Audiovisual and multimedia systems��Series I�Integrated services digital network��Series J�Transmission of television, sound programme and other multimedia signals��Series K�Protection against interference��Series L�Construction, installation and protection of cables and other elements of outside plant��Series M�TMN and network maintenance: international transmission systems, telephone circuits, telegraphy, facsimile and leased circuits��Series N�Maintenance: international sound programme and television transmission circuits��Series O�Specifications of measuring equipment��Series P�Telephone transmission quality, telephone installations, local line networks��Series Q�Switching and signalling��Series R�Telegraph transmission��Series S�Telegraph services terminal equipment��Series T�Terminals for telematic services��Series U�Telegraph switching��Series V�Data communication over the telephone network��Series X�Data networks and open system communication��Series Z�Programming languages��

____________________



















� PAGE �ii�	Recommendation G.853.2     (11/96)



�PAGE  �xiv�





		Recommendation G.853.2     (11/96)	� PAGE �iii�











Printed in Switzerland

Geneva, 1997



Printed in Switzerland, Geneva - 1996







Printed in Switzerland, Geneva - 1996








