	[image: image1.wmf]
	INTERNATIONAL  TELECOMMUNICATION  UNION

	
	

	ITU-T
	H.248.18

	TELECOMMUNICATION
STANDARDIZATION  SECTOR
OF  ITU
	(11/2002)   

	
	SERIES H: AUDIOVISUAL AND MULTIMEDIA SYSTEMS

Infrastructure of audiovisual services – Communication procedures



	
	Package for support of multiple profiles

	
	CAUTION !

PREPUBLISHED  RECOMMENDATION

This prepublication is an unedited version of a recently approved Recommendation. It will be replaced by the published version after editing. Therefore, there will be differences between this prepublication and the published version.


FOREWORD

The International Telecommunication Union (ITU) is the United Nations specialized agency in the field of tele​com​mu​ni​ca​tions. The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of ITU. ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Assembly (WTSA), which meets every four years, establishes the topics for study by the ITU‑T study groups which, in turn, produce Recommendations on these topics.

The approval of ITU-T Recommendations is covered by the procedure laid down in WTSA Resolution 1.

In some areas of information technology which fall within ITU-T's purview, the necessary standards are prepared on a collaborative basis with ISO and IEC.

NOTE

In this Recommendation, the expression "Administration" is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

INTELLECTUAL PROPERTY RIGHTS

ITU draws attention to the possibility that the practice or implementation of this Recommendation may involve the use of a claimed Intellectual Property Right. ITU takes no position concerning the evidence, validity or applicability of claimed Intellectual Property Rights, whether asserted by ITU members or others outside of the Recommendation development process.

As of the date of approval of this Recommendation, ITU [had/had not] received notice of intellectual property, protected by patents, which may be required to implement this Recommendation. However, implementors are cautioned that this may not represent the latest information and are therefore strongly urged to consult the TSB patent database.

  ITU  2002

All rights reserved. No part of this publication may be reproduced, by any means whatsoever, without the prior written permission of ITU.
Recommendation H.248.18 Package for Support of Multiple Profiles

___________________

Summary

H.248.18 describes a package to enable the Media Gateway Controller to determine which Profiles are supported on a Media Gateway. It also allows the MGC to set the profiles that it shall use. 

H.248.18

Profile Package

CONTENTS

Summary

H.248.18 describes a package to enable the Media Gateway Controller to determine which Profiles are supported on a Media Gateway. It also allows the MGC to set the profiles that it shall use. 

1 Scope

H.248.18 describes a package to enable the Media Gateway Controller to determine what profiles are supported on a Media Gateway. It also allows the MGC to set the profiles that it shall use. The support of this package is optional. 

2 References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; all users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

‑ ITU-T Recommendation H.248.1 (05/2002), Gateway Control Protocol

3 Definitions

N/A

4 Abbreviations

MG
Media Gateway

MGC 
Media Gateway Controller

5 Profile Package

Package Name: Profile Package

PackageID: prp, 0x0050

Description: 

H.248.18 describes a package to enable the Media Gateway Controller to determine what profiles are supported on a Media Gateway. Profiles are described in H.248.1. The property in this package shall only be implemented on the root termination.

Version: 1

Designed to be extended only: No

Extends: None

5.1. Properties

5.1.1 Property Name: Profiles Supported


PropertyID: Prof_supp, 0x0001


Description: 

This property indicates the profile/s and versions supported by the Media Gateway. This property is for the root termination.


Type: 
sub-list of string  




(Length 1 – 67 characters – 64 for name, 1 for "/" and 2 for version)


Possible Values:


As per profile the name and version registered registered with IANA.

The Name shall be encoded first followed by a slash "/" following by a string encoding of the version number. eg. fred/1

Note: The name "AuditProfiles" is reserved for the MG to indicate to the MGC via a ServiceChange that it should audit the MG. 

The value "AuditProfiles" shall not be returned when this property is audited.

The name "NoProfile" is reserved to indicate that the MG or MGC does not support any profiles.

Defined in: Termination State Descriptor


Characteristics: Read / Write

5.2. Events


N/A

5.3. Signals


N/A

5.4. Statistics


N/A

5.5. Procedures

If the MG wants to inform the MGC that it supports multiple profiles it shall indicate this with ServiceChange Profile = "AuditProfiles". It may indicate this in an initial ServiceChange or at any time after this. 

Once the MGC receives ServiceChange Profile = "AuditProfiles" the MGC should do an Audit Capabilities command on the "Prof_supp" property of the Root termination. The MG shall then return all profiles that can be supported by that MG. 

Figure 1 presents an example signalling sequence:

[image: image2.wmf]MG

MGC

(1)

 ServiceChange

.

req

   (

Ctxt

=all,

   

Tid

=root,

    Profile=Audit_Profiles)

(4)

 AudCap

.reply

(

Ctxt

=all,

Tid

=root,

  

TermStateDes

(

    ( 

Prof

_

supp

 = 

       koala/1,wombat/2)

(3)

 AudCap

.

req

   (

Ctxt

=all,

   

Tid

=root,

    

TermStateDes

 ( 

           

Prof

_

supp

))

(2)

 ServiceChange

.reply


Figure 1/H.248.18 ( Example Profile determination sequence

The MGC may set the Prof_supp property using a Modify command to indicate the profile/s that it will use on the MG. The MGC shall use one or more of the profiles that the MG indicated in an AuditCapability of the Prof_supp property. If the MGC tries to set the Prof_supp property on the MG to a profile value that is unknown by the MG, at the first unknown profile the MG shall reply with error code 459 "Unsupported or Unknown Profile". If the Prof_supp property is not set by the MGC then it shall be assumed that the MGC supports all the profiles supported by MG and that the MGC can use functionalities/capabilities of any of the profiles. 

Note: This aligns with the ServiceChange procedure where by the MGC may indicate which profile it supports.

An Audit value command may be used to determine the profile/s that have been set.

5.6. Error Codes

This package defines a new error code:

#: 459 Name: Unsupported or Unknown Profile

Definition: The Profile Name is not supported by the receiver. The command related to the unknown profile is disregarded.

Package: Profile Package prp, 0x0050

Reference: H.248.18

Error Text in the error Descriptor: The Profile Name is included in the error text in the error descriptor. String Length 1 – 67 characters – 64 for name, 1 for "/" and 2 for version.
Comment: -

____________________
PAGE  

