	[image: image1.wmf]
	INTERNATIONAL  TELECOMMUNICATION  UNION

	
	

	ITU-T
	H.323

	TELECOMMUNICATION
STANDARDIZATION  SECTOR
OF  ITU
	Annex M.1

(11/2000)   

	
	SERIES H: AUDIOVISUAL AND MULTIMEDIA SYSTEMS

Infrastructure of audiovisual services – Systems and terminal equipment for audiovisual services



	
	Packet-Based Multimedia Communications Systems

Annex M.1: Tunnelling of signalling protocols (QSIG) in H.323



	
	CAUTION !

PREPUBLISHED  RECOMMENDATION

This prepublication is an unedited version of a recently approved Recommendation. It will be replaced by the published version after editing. Therefore, there will be differences between this prepublication and the published version


FOREWORD

The International Telecommunication Union (ITU) is the United Nations specialized agency in the field of telecommunications. The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of ITU. ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Assembly (WTSA), which meets every four years, establishes the topics for study by the ITU‑T study groups which, in turn, produce Recommendations on these topics.

The approval of ITU-T Recommendations is covered by the procedure laid down in WTSA Resolution 1.

In some areas of information technology which fall within ITU-T’s purview, the necessary standards are prepared on a collaborative basis with ISO and IEC.

NOTE

In this Recommendation, the expression "Administration" is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

INTELLECTUAL PROPERTY RIGHTS

ITU draws attention to the possibility that the practice or implementation of this Recommendation may involve the use of a claimed Intellectual Property Right. ITU takes no position concerning the evidence, validity or applicability of claimed Intellectual Property Rights, whether asserted by ITU members or others outside of the Recommendation development process.

As of the date of approval of this Recommendation, ITU [had/had not] received notice of intellectual property, protected by patents, which may be required to implement this Recommendation. However, implementors are cautioned that this may not represent the latest information and are therefore strongly urged to consult the TSB patent database.

  ITU  2000

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.
	ITU-T  RECOMMENDATION  H.323 Annex M.1

TUNNELLING OF SIGNALLING PROTOCOLS (qsig) IN H.323


	Summary

The purpose of this annex is to give guidance how the generic tunnelling mechanism described in section 10.4 of H.323 can be used to tunnel QSIG over H.323 networks. 

This Annex requires H.323 and H.225.0 versions 4or later. Version 4 products can be identified by H.225.0 messages containing a protocolIdentifier = {itu-t (0) recommendation (0) h (8) 2250 version (0) 4} and H.245 messages containing a protocolIdentifier = {itu-t (0) recommendation (0) h (8) 245 version (0) 7}.


	Source
ITU-T Recommendation H.323 Annex M.1 was approved under the WTSC Resolution No. 1 procedure on the 17th of November 2000.


Recommendation H.323 Annex M.1

tunnelling of signalling protocols (QSig) in h.323

(11/2000)

1
Scope

The purpose of this annex is to give guidance how the generic tunnelling mechanism described in section 10.4/H.323 can be used to tunnel QSIG over H.323 networks. Other groups such as ISO/IEC are ultimately responsible for the QSIG procedures themselves. Information on QSIG (also known as PSS1) can be found in references  [1] and [2] below.

2
Normative references

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; all users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

 [1]
ISO/IEC 11572 Information technology – Telecommunications and information exchange between systems – Private Integrated Services Network – Circuit mode bearer services – Inter-exchange signalling procedures and protocol
[2]
ISO/IEC 11582 Information technology – Telecommunications and information exchange between systems – Private Integrated Services Network – Generic functional protocol for the support of supplementary services – Inter-exchange signalling procedures and protocol
[3]
ITU-T Recommendation H.323 (2000), Packet-based multimedia communications systems
[4]
ITU-T Recommendation H.225.0 (2000),  Call signalling protocols and media stream packetization for packet-based multimedia communication systems
3
Endpoint procedures

Endpoints supporting tunnelling of QSIG information shall use the procedures of H.323/10.4, with the following OBJECT IDENTIFIER used as the TunnelledProtocol:

· {iso (1) identified-organization (3) icd-ecma (0012) private-isdn-signalling-domain (9)}
H.225.0 messages tunnel the entire QSIG message, unchanged, starting with the Protocol discriminator field, and ending with the other information elements. The binary content of the QSIG messages is encoded as an OCTET STRING in the H323-UU-PDU.tunnelledSignallingMessage.messageContent.  Since the binary encoding of QSIG messages is what is tunnelled, the integrity of the QSIG messages is fully preserved, including any BER encoding of ASN.1 in Facility or Notification indicator information elements.

QSIG messages can, but need not, be tunnelled in the corresponding H.225.0 messages.  For example, the QSIG SETUP message can be tunnelled in a H.225.0 SETUP message, and the QSIG RELEASE COMPLETE message can be tunnelled in an H.225.0 RELEASE COMPLETE message. For other messages, it is possible that there is no corresponding H.225.0 (Q.931) message (e.g., in the case of a QSIG DISCONNECT message) or the corresponding message is not available because it has already been sent. In those cases, the QSIG message may be tunnelled in an H.225.0 FACILITY message. A QSIG CALL PROCEEDING message should be tunnelled in an H.225.0 FACILITY message since the H.225.0 CALL PROCEEDING message does not have end-to-end significance. Also since the NOTIFY and PROGRESS messages are optional, they might not be delivered end-to-end and should be tunnelled in a FACILITY message. QSIG call clearing procedures may be supported by tunnelling the QSIG DISCONNECT and RELEASE messages in the H.225.0 FACILITY message. In the special case where a tunnelled QSIG RELEASE message is interpreted as a tunnelled QSIG RELEASE COMPLETE message (this happens when a QSIG RELEASE message is received when a RELEASE COMPLETE was expected), the H.323 call may be released by the side receiving the QSIG RELEASE message by sending an H.225.0 RELEASE COMPLETE with no tunnelled QSIG message.

A single QSIG call can be tunnelled in a single H.323 call.  The relationship between QSIG call references and H.225.0 call references is outside the scope of this Recommendation.

Table 1 is indicative only and illustrates an example of the mapping between QSIG messages and H.225.0 messages.

Table 1/Annex M.1 – Mapping between QSIG messages and H.225.0 messages

	QSIG message
	H.225.0 message

	SETUP
	SETUP

	ALERTING
	ALERTING

	CONNECT
	CONNECT

	RELEASE COMPLETE
	RELEASE COMPLETE

	CALL PROCEEDING
	FACILITY

	FACILITY
	

	PROGRESS
	

	NOTIFY
	

	DISCONNECT
	

	RELEASE
	

	all other messages…
	


4
Tunnelling of QSIG connection oriented call independent signalling

For QSIG call independent signalling connections, no H.245 control channel and no media channels are required. 

The call signalling procedures of H.225.0 may be used to establish a call independent signalling connection between the peer endpoints, as described in H.323/10.4.  

5
Gatekeeper procedures

A gatekeeper participating in a call where QSIG tunnelling is used between the endpoints should pass along tunnelled QSIG messages unchanged unless it intends to terminate the tunnel. This may be the case when a gatekeeper is offering emulated QSIG services. 
























