
	[image: image1.wmf]
	INTERNATIONAL  TELECOMMUNICATION  UNION

	
	

	ITU-T
	H.323

	TELECOMMUNICATION
STANDARDIZATION  SECTOR
OF  ITU
	Annex Q

(07/2001)   

	
	SERIES H: AUDIOVISUAL AND MULTIMEDIA SYSTEMS

Infrastructure of audiovisual services – Systems and terminal equipment for audiovisual services



	
	Packet-based multimedia communications systems

Annex Q: Far-end camera control and H.281/H.224

	
	ITU‑T  Recommendation  H.323  –  Annex Q

(Formerly  CCITT  Recommendation)


ITU-T H-SERIES  RECOMMENDATIONS

AUDIOVISUAL AND MULTIMEDIA SYSTEMS

	
	

	CHARACTERISTICS OF VISUAL TELEPHONE SYSTEMS
	H.100–H.199

	INFRASTRUCTURE OF AUDIOVISUAL SERVICES
	

	General
	H.200–H.219

	Transmission multiplexing and synchronization
	H.220–H.229

	Systems aspects
	H.230–H.239

	Communication procedures
	H.240–H.259

	Coding of moving video
	H.260–H.279

	Related systems aspects
	H.280–H.299

	SYSTEMS AND TERMINAL EQUIPMENT FOR AUDIOVISUAL SERVICES
	H.300–H.399

	SUPPLEMENTARY SERVICES FOR MULTIMEDIA
	H.450–H.499

	
	


For further details, please refer to the list of ITU-T Recommendations.

	ITU-T Recommendation H.323

Packet-based multimedia communications systems

Annex Q

Far-end camera control and H.281/H.224




	Summary

This annex defines H.281/H.224-based far-end camera control (FECC).   


	Source

Annex Q to ITU‑T Recommendation H.323 was prepared by ITU‑T Study Group 16 (2001‑2004) and approved under the WTSA Resolution 1 procedure on 29 July 2001.


	


FOREWORD

The International Telecommunication Union (ITU) is the United Nations specialized agency in the field of tele​com​mu​ni​ca​tions. The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of ITU. ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Assembly (WTSA), which meets every four years, establishes the topics for study by the ITU‑T study groups which, in turn, produce Recommendations on these topics.

The approval of ITU-T Recommendations is covered by the procedure laid down in WTSA Resolution 1.

In some areas of information technology which fall within ITU-T's purview, the necessary standards are prepared on a collaborative basis with ISO and IEC.

NOTE

In this Recommendation, the expression "Administration" is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

INTELLECTUAL PROPERTY RIGHTS
ITU draws attention to the possibility that the practice or implementation of this Recommendation may involve the use of a claimed Intellectual Property Right. ITU takes no position concerning the evidence, validity or applicability of claimed Intellectual Property Rights, whether asserted by ITU members or others outside of the Recommendation development process.

As of the date of approval of this Recommendation, ITU had received notice of intellectual property, protected by patents, which may be required to implement this Recommendation. However, implementors are cautioned that this may not represent the latest information and are therefore strongly urged to consult the TSB patent database.

(  ITU  2001

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from ITU.

CONTENTS


Page

Q.1
Scope

1
Q.2
References

1
Q.3
Introduction

1
Q.4
Far-end camera control protocol

1
Q.4.1
General

1
Q.4.2
H.320 to H.323 gateways

2
Q.4.3
H.324 to H.323 gateways

2
Q.4.4
H.245 signalling

2
Q.5
RTP header information

3
ITU-T Recommendation H.323

Packet-based multimedia communications system

Annex Q

Far-end camera control and H.281/H.224

Q.1
Scope

The purpose of this annex is to provide far-end camera control protocol based on H.281/H.224. It also permits an H.323 endpoint to run any H.224 application using the IP/UDP/RTP/H.224 protocol defined in this annex.
Q.2
References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

[1]
ITU-T H.224 (2000), A real time control protocol for simplex applications using the H.221 LSD/HSD/MLP channels.

[2]
ITU-T H.281 (1994), A far end camera control protocol for videoconferences using H.224.

[3]
ITU-T T.140 (1998), Protocol for multimedia application text conversation.

Q.3
Introduction

The protocol described in this annex may be used to support far-end camera control (FECC) in ITU‑T H.323 using the stack IP/UDP/RTP/H.224/H.281. This protocol supports both point‑to‑point and multipoint scenarios. 

This method may be used as a "simple" FECC scheme when the more sophisticated features of H.282/H.283 are not needed.

This method shall be used for FECC thru H.320-H.323 and H.324-H.323 gateways when the H.320 or H.324 endpoints do not support ITU-T H.282.

The requirements given below apply only in the case that the protocol described in this annex has been selected, using the normal procedures of ITU-T H.245.

It is allowed to run any H.224 application using the IP/UDP/RTP/H.224 protocol defined in this annex. The only other currently standardized H.224 application is ITU-T T.140.

Q.4
Far-end camera control protocol

Q.4.1
General

This protocol is based on ITU-T H.281 running over ITU-T H.224 in an RTP/UDP channel.

On IP transport networks, the H.224 protocol octet structure shall be the same as Figure 2/H.224 except that the HDLC bit stuffing, HDLC flags and HDLC Frame Check Sequence shall be omitted. The entire remaining content of each frame shall be placed in a single RTP packet.

References in ITU-T H.224 to the LSD channel of ITU-T H.221 shall be interpreted as referring to the H.224 logical channel as described in this annex. The maximum transmission time requirements of ITU-T H.224 shall be met, with the H.224 logical channel considered as operating at 4800 bit/s, regardless of the actual bit rate of the channel.

This protocol shall run over RTP in a unidirectional unreliable H.245 logical channel. The RTP payload value shall be dynamic. The payload descriptor field of H.245 RTPPayloadType shall use the H.224 Object ID.

Terminal numbering according to the procedures in ITU-T H.243 shall be used in order to support the data link layer in multipoint. The MCU/Terminal address pair <M><T> shall be used to uniquely identify each terminal in a conference. The special destination address of <0><0> shall be used as the broadcast address. The special source address <0><0> shall indicate that the sender does not know its address. An address with the terminal number set to 0 indicates the MC. For example, <n><0> indicates MC number n.

In a point‑to‑point call, when only two terminals are involved, then the terminals do not have an <M><T> address. In this case, the <M><T> source and destination addresses shall be always <0><0>.

In a centralized conference, an H.224 channel shall be opened between each terminal and the MC. When a terminal sends an H.224 packet, the MC shall forward the packet to the destination terminal by either retransmitting each packet to all other connected terminals or, by selectively retransmitting each packet only toward the destination terminal. The decision which method to use is up to the MCU manufacturer.

In a decentralized multicast conference, each terminal shall multicast the FECC packet to all other terminals. The MC is not involved in forwarding the packets. Terminal numbers per ITU-T H.243 shall be used to identify the source and destination terminals.

In decentralized multi-unicast conferences, each terminal shall use a separate logical channel to each far-end terminal to which it wants to send H.224 packets.

Q.4.2
H.320 to H.323 gateways

H.320-H.323 gateways shall insert and remove HDLC flags, HDLC bitstuffing, and HDLC Frame Check Sequence(s) as appropriate in each direction, so that the bitstream on the H.320 side shall conform with ITU‑T H.224, and the bitstream on the H.323 side shall conform with the paragraphs above.

Q.4.3
H.324 to H.323 gateways

H.324-H.323 gateways shall insert and remove HDLC flags, HDLC octet-stuffing, and HDLC Frame Check Sequence(s) as appropriate in each direction, so that the bitstream on the H.324 side shall conform with the use of ITU‑T H.224 as described in ITU-T H.324, and the bitstream on the H.323 side shall conform with the clauses above.

Q.4.4
H.245 signalling

The use of this protocol shall be signalled by the GenericCapability part of the DataApplicationCapability sequence in H.245. The Generic Capability for H.224, described in ITU‑T H.224, shall be used. This shall be placed in the receiveAndTransmitDataApplicationCapability part of the Capability choice.

This protocol shall not be signalled in the receiveDataApplicationCapability or transmitDataApplicationCapability parts of the Capability choice.

Q.5
RTP header information

The following fields shall be filled in the RTP header:

V:



2

M:



0 NA

PT:



The same number sent in the OLC dynamicRTPPayloadType field

Sequence number:
Filled, incremented by one for each RTP packet sent

Timestamp:


Filled with 8 kHz clock rate

SSRC:



Filled with the synchronization source

	SERIES OF ITU-T RECOMMENDATIONS

	Series A
	Organization of the work of ITU-T

	Series B
	Means of expression: definitions, symbols, classification

	Series C
	General telecommunication statistics

	Series D
	General tariff principles

	Series E
	Overall network operation, telephone service, service operation and human factors

	Series F
	Non-telephone telecommunication services

	Series G
	Transmission systems and media, digital systems and networks

	Series H
	Audiovisual and multimedia systems

	Series I
	Integrated services digital network

	Series J
	Cable networks and transmission of television, sound programme and other multimedia signals

	Series K
	Protection against interference

	Series L
	Construction, installation and protection of cables and other elements of outside plant

	Series M
	TMN and network maintenance: international transmission systems, telephone circuits, telegraphy, facsimile and leased circuits

	Series N
	Maintenance: international sound programme and television transmission circuits

	Series O
	Specifications of measuring equipment

	Series P
	Telephone transmission quality, telephone installations, local line networks

	Series Q
	Switching and signalling

	Series R
	Telegraph transmission

	Series S
	Telegraph services terminal equipment

	Series T
	Terminals for telematic services

	Series U
	Telegraph switching

	Series V
	Data communication over the telephone network

	Series X
	Data networks and open system communications

	Series Y
	Global information infrastructure and Internet protocol aspects

	Series Z
	Languages and general software aspects for telecommunication systems

	
	





