
	[image: image1.wmf]
	INTERNATIONAL  TELECOMMUNICATION  UNION

	
	

	ITU-T
	I.610

	TELECOMMUNICATION
STANDARDIZATION  SECTOR
OF  ITU
	Amendment 1

(03/2000)   

	
	SERIES I: INTEGRATED SERVICES DIGITAL NETWORK

Maintenance principles



	
	B-ISDN operation and maintenance principles and functions

Amendment 1



	
	ITU‑T  Recommendation  I.610  –  Amendment 1

(Formerly  CCITT  Recommendation)


ITU-T I-SERIES  RECOMMENDATIONS

INTEGRATED SERVICES DIGITAL NETWORK

	
	

	GENERAL STRUCTURE
	

	Terminology
	I.110–I.119

	Description of ISDNs
	I.120–I.129

	General modelling methods
	I.130–I.139

	Telecommunication network and service attributes
	I.140–I.149

	General description of asynchronous transfer mode
	I.150–I.199

	SERVICE CAPABILITIES
	

	Scope
	I.200–I.209

	General aspects of services in ISDN
	I.210–I.219

	Common aspects of services in the ISDN
	I.220–I.229

	Bearer services supported by an ISDN
	I.230–I.239

	Teleservices supported by an ISDN
	I.240–I.249

	Supplementary services in ISDN
	I.250–I.299

	OVERALL NETWORK ASPECTS AND FUNCTIONS
	

	Network functional principles
	I.310–I.319

	Reference models
	I.320–I.329

	Numbering, addressing and routing
	I.330–I.339

	Connection types
	I.340–I.349

	Performance objectives
	I.350–I.359

	Protocol layer requirements
	I.360–I.369

	General network requirements and functions
	I.370–I.399

	ISDN USER-NETWORK INTERFACES
	

	Application of I-series Recommendations to ISDN user-network interfaces
	I.420–I.429

	Layer 1 Recommendations
	I.430–I.439

	Layer 2 Recommendations
	I.440–I.449

	Layer 3 Recommendations
	I.450–I.459

	Multiplexing, rate adaption and support of existing interfaces
	I.460–I.469

	Aspects of ISDN affecting terminal requirements
	I.470–I.499

	INTERNETWORK INTERFACES
	I.500–I.599

	MAINTENANCE PRINCIPLES
	I.600–I.699

	B-ISDN EQUIPMENT ASPECTS
	

	ATM equipment
	I.730–I.739

	Transport functions
	I.740–I.749

	Management of ATM equipment
	I.750–I.759

	Multiplexing aspects
	I.760–I.769

	
	


For further details, please refer to the list of ITU-T Recommendations.

	ITU-T Recommendation I.610

B-ISDN operation and maintenance principles and functions

Amendment 1




	Summary

This amendment contains only editorial enhancements, but no technical changes, to the third revision (02/99) of ITU-T Recommendation I.610.


	Source

Amendment 1 to ITU‑T Recommendation I.610 was prepared by ITU‑T Study Group 13 (1997‑2000) and approved under the WTSC Resolution 1 procedure on 10 March 2000.


	


FOREWORD

The International Telecommunication Union (ITU) is the United Nations specialized agency in the field of tele​com​mu​ni​ca​tions. The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of ITU. ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Conference (WTSC), which meets every four years, establishes the topics for study by the ITU‑T study groups which, in turn, produce Recommendations on these topics.

The approval of ITU-T Recommendations is covered by the procedure laid down in WTSC Resolution 1.

In some areas of information technology which fall within ITU-T’s purview, the necessary standards are prepared on a collaborative basis with ISO and IEC.

NOTE

In this Recommendation, the expression "Administration" is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

INTELLECTUAL PROPERTY RIGHTS
ITU draws attention to the possibility that the practice or implementation of this Recommendation may involve the use of a claimed Intellectual Property Right. ITU takes no position concerning the evidence, validity or applicability of claimed Intellectual Property Rights, whether asserted by ITU members or others outside of the Recommendation development process.

As of the date of approval of this Recommendation, ITU had not received notice of intellectual property, protected by patents, which may be required to implement this Recommendation. However, implementors are cautioned that this may not represent the latest information and are therefore strongly urged to consult the TSB patent database.

(  ITU  2001

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from ITU.

ITU-T Recommendation I.610

B-ISDN operation and maintenance principles and functions

amendment 1

1
Introduction

This amendment contains only editorial enhancements, but no technical changes, to the third revision (02/99) of ITU-T Recommendation I.610.

2
Additions

2.1
History

Add the following document history information:

	Document history

	Issue
	Notes

	02/99
	Third revision of ITU-T Recommendation I.610.

This revision includes editorial modifications to comply with ITU-T Recommendation A.3. Clauses 2 to 7 in the 1995 revision are as a consequence renumbered as clauses 5 to 10. 

Furthermore, this revision includes numerous modifications and extensions to most of the clauses and subclauses. Special attention should be given to:

•
subclause 8.2.1, which refers to the appropriate physical layer Recommendations for the OAM functions supported by the flows F1 to F3;

•
subclause 8.3, which defines additional OAM functions between the transmission path and ATM layer;

•
subclause 9.2 and its subclauses, which define the segment OAM cells "segment AIS" and "segment RDI" and the associated processing (generation and termination), introduces new "end to end APS" and "segment APS" OAM cells for which ITU-T Recommendation I.630 specifies the processing, has enhanced the specification for the use of the performance management OAM cells, introduces the ability to non-intrusively monitor OAM flows at intermediate points within the VPC or VPC segment, introduces an additional performance management mode in which forward performance monitoring is activated only;
•
clause 10 and its subclauses, which introduce the OAM cell detection procedure (10.1), introduce a coding structure for location identifiers (10.1), extend the definition of the segment loopback OAM cell and associate the processing required in connection points and segment endpoints (including an incoming segment loopback cell extraction option), extend the number of PM block sizes, delete one deactivation response code point;
•
nnex A, which is rewritten;

•
Annex C, which is rewritten;

•
Appendices II, III, IV and V, which are new.

	11/95
	Second revision

	1993
	First revision

	1991
	Initial version


2.2
New clause 11

Add a new clause 11 to address the interworking of the new functionality that is introduced in the third revision (02/99) of ITU-T Recommendation I.610 (i.e. segment VP-AIS, segment VC-AIS, segment VP-RDI, segment VC-RDI OAM cells, new loopback type, larger PM block sizes) in networks that also include equipment designed according to previous versions of ITU-T Recommendation I.610 that do not support this new functionality. 

11
Backward compatibility

Equipment compliant with 1995 or earlier versions of ITU-T Recommendation I.610 will not generate segment VP/VC-AIS OAM cells, will not detect the segment VP/VC-AIS cells and associated defect conditions and will not declare the segment VP/VC-AIS state. When there is a situation in which a 1999 version of an I.610 compliant segment endpoint is interworking with equipment compliant with 1995 or earlier versions of I.610 within the segment, it will be necessary to use segment CC OAM to be able to declare a segment VP/VC AIS state.

Equipment compliant with 1995 or earlier versions of ITU-T Recommendation I.610 will not generate segment VP/VC-RDI OAM cells, will not detect the segment VP/VC-RDI cells and associated defect conditions and will not declare the segment VP/VC-RDI state. 

Equipment compliant with 1995 or earlier versions of this ITU-T Recommendation will not recognize the all-0's LLID code in segment LB OAM cells as an MLT code.

Equipment compliant with 1995 or earlier versions of this ITU-T Recommendation will not recognize the codes for PM block sizes above 1024.

2.3
Subclause III.3.1.2 (Appendix III)

Add a Note at the end of the second paragraph in subclause III.3.1.2:

NOTE ( When an unintended branch exists between CPID (c) and a second VPI [VCI] within CPID (d), two seg_LB cells will be returned with LLID is CPID (d). An additional (8th) LB cell will be returned if the CP following CP with CPID (r) has the VPI [VCI] listed as an expected VPI [VCI].

	SERIES OF ITU-T RECOMMENDATIONS

	Series A
	Organization of the work of ITU-T

	Series B
	Means of expression: definitions, symbols, classification

	Series C
	General telecommunication statistics

	Series D
	General tariff principles

	Series E
	Overall network operation, telephone service, service operation and human factors

	Series F
	Non-telephone telecommunication services

	Series G
	Transmission systems and media, digital systems and networks

	Series H
	Audiovisual and multimedia systems

	Series I
	Integrated services digital network

	Series J
	Transmission of television, sound programme and other multimedia signals

	Series K
	Protection against interference

	Series L
	Construction, installation and protection of cables and other elements of outside plant

	Series M
	TMN and network maintenance: international transmission systems, telephone circuits, telegraphy, facsimile and leased circuits

	Series N
	Maintenance: international sound programme and television transmission circuits

	Series O
	Specifications of measuring equipment

	Series P
	Telephone transmission quality, telephone installations, local line networks

	Series Q
	Switching and signalling

	Series R
	Telegraph transmission

	Series S
	Telegraph services terminal equipment

	Series T
	Terminals for telematic services

	Series U
	Telegraph switching

	Series V
	Data communication over the telephone network

	Series X
	Data networks and open system communications

	Series Y
	Global information infrastructure and Internet protocol aspects

	Series Z
	Languages and general software aspects for telecommunication systems

	
	


