
	[image: image1.wmf]
	INTERNATIONAL  TELECOMMUNICATION  UNION

	
	

	ITU-T
	J.175

	TELECOMMUNICATION
STANDARDIZATION  SECTOR
OF  ITU
	(07/2002)   

	
	SERIES J: CABLE NETWORKS AND TRANSMISSION OF TELEVISION, SOUND PROGRAMME AND OTHER MULTIMEDIA SIGNALS

IPCablecom



	
	Audio server protocol

	
	ITU‑T  Recommendation  J.175




ITU-T J-SERIES  RECOMMENDATIONS

CABLE NETWORKS AND TRANSMISSION OF TELEVISION, SOUND PROGRAMME AND OTHER MULTIMEDIA SIGNALS

	
	

	General Recommendations
	J.1–J.9

	General specifications for analogue sound-programme transmission
	J.10–J.19

	Performance characteristics of analogue sound-programme circuits
	J.20–J.29

	Equipment and lines used for analogue sound-programme circuits
	J.30–J.39

	Digital encoders for analogue sound-programme signals
	J.40–J.49

	Digital transmission of sound-programme signals
	J.50–J.59

	Circuits for analogue television transmission
	J.60–J.69

	Analogue television transmission over metallic lines and interconnection with radio-relay links
	J.70–J.79

	Digital transmission of television signals
	J.80–J.89

	Ancillary digital services for television transmission
	J.90–J.99

	Operational requirements and methods for television transmission
	J.100–J.109

	Interactive systems for digital television distribution
	J.110–J.129

	Transport of MPEG-2 signals on packetised networks
	J.130–J.139

	Measurement of the quality of service
	J.140–J.149

	Digital television distribution through local subscriber networks
	J.150–J.159

	IPCablecom
	J.160–J.179

	Miscellaneous
	J.180–J.199

	Application for Interactive Digital Television
	J.200–J.209

	
	


For further details, please refer to the list of ITU-T Recommendations.

	ITU-T Recommendation J.175

Audio server protocol




	Summary

This Recommendation describes the architecture and protocols that are required for playing announcements in voice-over-IP (VoIP) IPCablecom networks. Announcements are typically needed for calls that do not complete or to provide enhanced information services to the caller.


	Source

ITU‑T Recommendation J.175 was prepared by ITU‑T Study Group 9 (2001‑2004) and approved under the WTSA Resolution 1 procedure on 29 July 2002.


	


FOREWORD

The International Telecommunication Union (ITU) is the United Nations specialized agency in the field of tele​com​mu​ni​ca​tions. The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of ITU. ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Assembly (WTSA), which meets every four years, establishes the topics for study by the ITU‑T study groups which, in turn, produce Recommendations on these topics.

The approval of ITU-T Recommendations is covered by the procedure laid down in WTSA Resolution 1.

In some areas of information technology which fall within ITU-T's purview, the necessary standards are prepared on a collaborative basis with ISO and IEC.

NOTE

In this Recommendation, the expression "Administration" is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

INTELLECTUAL PROPERTY RIGHTS
ITU draws attention to the possibility that the practice or implementation of this Recommendation may involve the use of a claimed Intellectual Property Right. ITU takes no position concerning the evidence, validity or applicability of claimed Intellectual Property Rights, whether asserted by ITU members or others outside of the Recommendation development process.

As of the date of approval of this Recommendation, ITU had received notice of intellectual property, protected by patents, which may be required to implement this Recommendation. However, implementors are cautioned that this may not represent the latest information and are therefore strongly urged to consult the TSB patent database.

(  ITU  2003

All rights reserved. No part of this publication may be reproduced, by any means whatsoever, without the prior written permission of ITU.

CONTENTS


Page
1
Scope

1
2
References

1
2.1
Normative references

1
2.2
Informative references

1
3
Terms and conventions

2
3.1
Terms

2
3.2
Conventions

2
4
Abbreviations and Acronyms

3
5
Technical overview

3
5.1
Architectural requirements

4
5.1.1
Call destination

4
5.1.2
Media Formats

4
5.1.3
Security

4
5.1.4
Operational support systems

5
5.2
Announcement definitions

5
5.2.1
Tones

5
5.2.2
Fixed-content announcements

5
5.2.3
Variable-content announcements

5
5.2.4
Interactive announcements

5
5.2.5
Naming conventions for endpoint identifiers

5
5.3
Architectural components

5
5.3.1
Audio Server (AS)

5
5.3.2
Multimedia Terminal Adapter (MTA)

6
5.3.3
Media Gateway (MG)

6
5.3.4
Call Management Server (CMS)

6
5.4
IPCablecom audio server interface descriptions

6
5.4.1
Ann-1 Interface – CMS/MTA and MGC/MG announcement package

7
5.4.2
Ann-2 interface – MPC/MP announcement package

7
5.4.3
Ann-3 interface – CMS/MPC and CMS/MGC

7
5.4.4
Ann-4 interface – MP/MTA

7
5.4.5
Audio Server Physical vs Logical configuration

7
5.5
Interface specifications

9
6
Ann-1 interface: CMS-MTA and MGC-MG

10
6.1
CMS-MTA interface

10
6.1.1
Announcement list

10
6.2
MGC-MG Interface

11

Page

7
Ann-2 interface: MPC-MP

11
7.1
Introduction

11
7.2
Audio package concepts

12
7.2.1
Understanding audio segments

12
7.2.2
Segment identifiers

13
7.2.3
Segment life

13
7.2.4
Nested sets and sequences

13
7.2.5
Sequence example

14
7.2.6
Set example

14
7.2.7
Set with nested sequence example

14
7.3
Base Audio Package

15
7.3.1
Abstract

15
7.3.2
Events

15
7.3.3
Signal interactions

16
7.3.4
Parameters

16
7.3.5
Type-ahead

20
7.3.6
Return parameters

20
7.3.7
Segment descriptors

22
7.3.8
Variable syntax

22
7.3.9
Variable definitions

23
7.3.10
Examples

24
7.4
Advanced Audio package

26
7.4.1
Abstract

26
7.4.2
Sets

26
7.4.3
Selectors

26
7.4.4
Selector encoding

27
7.4.5
Variable order

27
7.4.6
Overrides

27
7.4.7
Parameters

28
7.4.8
Return codes

28
7.4.9
Examples

28
7.5
Formal syntax description

29
Appendix I – Call flow for network announcement

32
Appendix II – Call flow for MTA-stored announcement

46
II.1
Call flow details

47
Appendix III – Bibliography

50
ITU-T Recommendation J.175

Audio server protocol

1
Scope

This Recommendation describes the architecture and protocols that are required for playing announcements in voice-over-IP (VoIP) IPCablecom networks. Announcements are typically needed for calls that do not complete. Additionally, they may be used to provide enhanced information services to the caller. Different carrier service feature sets require different announcement sets and announcement formats. 

Announcements can be as basic as fixed-content announcements (e.g. all circuits busy) or as complex as those provided by intelligent IVR (Interactive Voice Response) systems. The IPCablecom service model requires that all announcements be provisioned and signalled in a standard manner for all supported call features and use case scenarios.

This Recommendation defines a set of signalling protocols that are used to provide announcement services within a cable network. For one of these protocols – the IPCablecom Network Call Signalling (NCS) protocol (ITU-T Rec. J.162) – this Recommendation defines two new event packages:

•
a Base Audio Package;

•
an Advanced Audio Package.

NOTE ( There may be cases where audio server implementations are based on protocols other than that specified in this Recommendation. If other protocols are implemented, those implementations must adhere to IPCablecom specified architectural and functional requirements, such as security and Quality of Service (QoS), and the required features/capabilities to support interoperability. A variety of such protocols exist, including protocols such as INAP, ITU-T Rec. H.248 and others.

2
References

The following ITU-T Recommendations and other references contain provisions, which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published. The reference to a document within this Recommendation doest not give it, as a stand-alone document, the status of a Recommendation.

2.1
Normative references

(
ITU-T Recommendation J.160 (2002), Architectural framework for the delivery of time‑critical services over cable television networks using cable modems.

(
ITU-T Recommendation J.162 (2001), Network call signalling protocol for the delivery of time‑critical services over cable television networks using cable modems.

2.2
Informative references

(
ITU-T Recommendation J.161 (2001), Audio codec requirements for the provision of bidirectional audio service over cable television networks using cable modems.

(
ITU-T Recommendation J.163 (2001), Dynamic quality of service for the provision of real‑time services over cable television networks using cable modems. 

(
ITU-T Recommendation J.170 (2002), IPCablecom security specification.

(
ITU-T Recommendation J.171 (2002), IPCablecom trunking gateway control protocol (TGCP).

(
ISO 639-2:1998, Codes for the representation of names of languages – Part 2: Alpha-3 code.
(
ISO 4217:2001, Codes for the representation of currencies and funds.

(
ISO 8601:2000, Data elements and interchange formats – Information interchange – Representation of dates and times.

3
Terms and conventions

3.1
Terms

This Recommendation defines the following terms:

3.1.1
access node: As used in this Recommendation, an Access Node is a layer two termination device that terminates the network end of the J.112 connection. It is technology specific. In J.112 Annex A it is called the INA, while in Annex B it is the CMTS.

3.1.2
announcement servers: Also known as Audio Servers, Announcement Servers are network components that manage and play informational tones and messages in response to events that occur in the network. Most announcements are media streams that originate from servers in the network. Some simple tones and short announcements can also reside at the MTA and in the MG.

3.2
Conventions

If this Recommendation is implemented, the keywords "MUST" and "SHALL" as well as "REQUIRED" are to be interpreted as indicating a mandatory aspect of this Recommendation.

The keywords indicating a certain level of significance of a particular requirements that are used throughout this Recommendation are summarized below.

"MUST"


This word or the adjective "REQUIRED" means that the item is an absolute requirement of this Recommendation.

"MUST NOT"

This phrase means that the item is an absolute prohibition of this Recommendation.

"SHOULD"


This word or the adjective "RECOMMENDED" means that there may exist valid reasons in particular circumstances to ignore this item, but the full implications should be understood and the case carefully weighed before choosing a different course.

"SHOULD NOT"
This phrase means that there may exist valid reasons in particular circumstances when the listed behavior is acceptable or even useful, but the full implications should be understood and the case carefully weighed before implementing any behavior described with this label.

"MAY"



This word or the adjective "OPTIONAL" means that this item is truly optional. One vendor may choose to include the item because a particular marketplace requires it or because it enhances the product, for example; another vendor may omit the same item.

4
Abbreviations and Acronyms

This Recommendation uses the following abbreviations:

AN

Access Node

ASP

Audio Server Protocol

CMS

Call Management Server

DNS 

Domain Name System

DTMF

Dual Tone Multi-Frequency

E-MTA 
Embedded MTA 
H.248

An ITU-T/IETF protocol for media gateway control. Also known as MEGACO. See www.itu.int

IANA 

Internet Assigned Numbers Authority

IVR 

Interactive Voice Response 

MEGACO 
An IETF/ITU-T protocol for media gateway control. Also known as H.248. See www.ietf.org for details.

MGCP
 
Media Gateway Control Protocol
MIB 

Management Information Base

MP

Media Player
MPC

Media Player Controller
MTA 

Media Terminal Adapter

NCS

Network Call Signalling

PSTN 

Public Switched Telephone Network

RTP 

Real-Time Protocol

SDP 

Session Description Protocol

SID 

Service ID. A 14-bit number assigned by an AN to identify an upstream virtual circuit. Each SID separately requests and is granted the right to use upstream bandwidth

S-MTA 
Stand-alone MTA – a single node which contains an MTA and a non-J.112 MAC (e.g. Ethernet)

SNMP 

Simple Network Management Protocol

TGCP

Trunking Gateway Control Protocol

TN

Telephone Number

URI

Universal Resource Identifier

VoIP 

Voice-over-IP

5
Technical overview

This Recommendation defines a suite of signalling protocols for providing announcement and media services in an IPCablecom network. This clause:

•
defines the architectural requirements for providing IPCablecom announcement and media services;

•
defines and categorizes announcement and media types;

•
defines the components and their roles in the IPCablecom audio server architecture; and

•
describes the signalling and media interfaces.

5.1
Architectural requirements 

The architectural requirements and assumptions for providing audio and media services for an IPCablecom network are listed below. These requirements are based upon the specifications and technical reports that define the IPCablecom architecture. 

The reference architecture for the IPCablecom network is shown in Figure 1.


[image: image2.wmf]J.175_F01

Managed IP Backbone

(QoS Features)

(Headend, Local, Regional)

PSTN

AN

MTA

AN

MTA

Embedded MTA

Cable

Modem

HFC access

network

Call Management

Server

Announcement

Server

Media Gateway

PSTN Gateway

Controller

Signalling Gateway

OSS

Back-office

Embedded MTA

Cable

Modem

Billing

Provisioning

Problem Resolution

DHCP Servers

TFTP Servers

...

Stand-alone

MTA

Cable

Modem

Cable

Modem

MTA

MTA

Stand-alone

MTA

HFC access

network

IPCablecom Network


Figure 1/J.175 ( IPCablecom network component reference model

5.1.1
Call destination

This Recommendation must define how announcements are provided for IPCablecom on‑net to off-net and on-net to on-net calls
.

5.1.2
Media Formats

The required media formats for announcements are specified by the IPCablecom codecs specification, ITU-T Rec. J.161.

5.1.3
Security

Audio must be signalled and played in a secure manner. Security protocols defined in the IPCablecom security specification (ITU-T Rec. J.170) must be supportable in this Recommendation.

5.1.4
Operational support systems

Audio servers may be required to support the IPCablecom billing and event message protocols as defined in ITU-T Rec. J.164.

5.2
Announcement definitions

Announcements can be divided into four distinct categories: tones, fixed-content, variable content, and interactive announcements.

5.2.1
Tones

Includes tones such as reorder, busy, and ringback. 

5.2.2
Fixed-content announcements

Fixed-content announcements consist of audio messages with fixed-content that require no user interaction. For example, "Your call did not go through. Please hang up and try your call again."

5.2.3
Variable-content announcements

Variable-content announcements are messages that contain a customizing parameter(s) yet require no user interaction. For example, "The number you have dialled, 321-9876, has been changed. The new number is 321-6789."

5.2.4
Interactive announcements

Interactive announcements are announcements that require user interaction, DTMF (Dual Tone Multi-Frequency) or IVR. For example, "The number you have dialled, 541-321-9876, has been changed. The new number is 541-321-6789. To be connected to the new number, at a cost of thirty-five cents, please press 1."

5.2.5
Naming conventions for endpoint identifiers

A flat name space for endpoints is used, with audio ports indicated by the prefix aud and the port number, e.g. aud/12@audio-server-3.whatever.net. Wild cards ($, *) may by used in place of the port numbers in accordance with standard NCS rules for wild card use.

Systems that support announcements only (i.e. no digit collection, no recording, and no speech recognition ability) may use the prefix ann instead of aud. 

Some systems may use one additional level in the naming scheme to support the identification of specific cards. In this case the naming would look like aud/<card number>/<port number>@audio-server-3.whatever.net.

5.3
Architectural components

IPCablecom components responsible for providing announcement services are defined below. These components work together to provide the complete set of announcement services available from the IPCablecom network provider. There may be more than one of these components in the network. Figure 2 defines a logical architecture for providing announcement services and only where an interface is exposed is it expected to meet IPCablecom specification requirements.

5.3.1
Audio Server (AS)

An AS is a logical entity composed of a Media Player Controller (MPC) and a Media Player (MP).

5.3.1.1
Media Player Controller (MPC)

The MPC initiates and manages all announcement services provided by the Media Player. The MPC accepts requests from the CMS and arranges for the MP to provide the announcement in the appropriate stream so that the user hears the announcement. The MPC also serves as the termination for certain calls routed to it for IVR services. These might include, for example, calls in which the user dials an 800 number to reach a credit-card calling service operated by the IPCablecom network operator. When the MP collects information from the end-user, the MPC is responsible for interpreting this information and manage the IVR session accordingly. Hence, the MPC will manage call state.

The MPC can be stand-alone, or it can be embedded within the CMS. See Figures 2 and 3 for illustrations of stand-alone and embedded MPC configurations.

5.3.1.2
Media Player (MP)

The Media Player is a media resource server. It is responsible for receiving and interpreting commands from the MPC and for delivering the appropriate announcement(s) to the MTA. The MP provides the media streams with the announcement contents. The MP also is responsible for accepting and reporting user inputs (e.g. DTMF tones). The MP functions under the control of the MPC.

An MP can be stand-alone, or it can be embedded with the MPC in a Media Server. See Figures 2 and 4 for respective illustrations of the stand-alone and embedded MP configurations.

5.3.2
Multimedia Terminal Adapter (MTA)

The MTA has the ability to provide tones and a limited set of fixed-content announcements to the user. The MTA accepts NCS signalling requests from the CMS and plays the appropriate tones and announcements accordingly.

5.3.3
Media Gateway (MG)
The Media Gateway also has the ability to provide fixed-content announcements to PSTN end-users involved in off-net to on-net calls. The MG accepts TGCP request to play announcements from the Media Gateway Controller (MGC) and provides the announcements accordingly.

5.3.4
Call Management Server (CMS)

The CMS determines when announcements should be played at the MTA, when to use the resources of a network MPC/MP complex, and when to play announcements to a PSTN end-user from the MG. This is based on the status of a call in progress. The CMS then signals the appropriate entity: MTA, MPC, or MGC to play tones or announcements to the end-user, accordingly. 

5.4
IPCablecom audio server interface descriptions

The signalling interfaces to support media services are shown in Figure 2 and are summarized in Table 1. 

Table 1/J.175 – Announcement interfaces

	Interface
	Signalling components
	Protocol

	Ann-1
	MTA/CMS, MGC/MG
	NCS/TGCP with announcement packages

	Ann-2
	MPC/MP
	NCS with announcement package

	Ann-3
	CMS/MPC, CMS/MGC
	Undefined. See 5.4.3

	Ann-4
	MP/MTA
	RTP


The remainder of this clause provides an overview of the announcements interfaces introduced above.

5.4.1
Ann-1 Interface – CMS/MTA and MGC/MG announcement package

An announcement package based upon the IPCablecom Network Call Signalling (NCS) protocol has been defined that can be used for both the CMS-MTA and MGC-MG interfaces.

5.4.1.1
CMS/MTA interface

The CMS to MTA interface provides a mechanism for the CMS to signal the MTA to play locally stored announcements. Simple tones and some frequently used fixed-content announcements (e.g. network busy) MAY be stored in the MTA so they can be played to the IPCablecom subscriber without tying up network bandwidth or media resources. Furthermore, storing these announcements in the MTA allows for providing informative progress tones to the end user independently of the network state (e.g. congestion).

5.4.1.2
MGC/MG interface

The MGC to MG interface provides a mechanism for the MG to play fixed-content announcements to PSTN end-users involved in off-net to on-net calls. For example, MG announcements may be used to provide PSTN users call progress information for calls that cannot be completed to the IPCablecom network (all-lines-busy). Simple, fixed-content announcements (e.g. all-lines-busy) MAY be stored at the Media Gateway to provide announcements to PSTN users. 

5.4.2
Ann-2 interface – MPC/MP announcement package

The MPC-to-MP protocol is based upon an NCS announcement package. Less frequently used tones and fixed-content announcements, as well as all variable‑content and interactive announcements are provided by MPC and MP complex. 

When the CMS identifies a need for an AS-based announcement, it sends a request to the MPC over interface Ann-3. Upon receiving a request from the CMS, the MPC opens a session with the Media Player using the NCS package. The MP then interacts with the specified endpoint over interface Ann-4.

5.4.3
Ann-3 interface – CMS/MPC and CMS/MGC

The Ann-3 interface allows the CMS to request the MPC to establish announcement sessions between the MP and another endpoint. It also allows the CMS to request the MGC to have the MG play fixed-content announcements to a PSTN endpoint. This interface is currently undefined. It is expected that this signalling interface will be based upon the IPCablecom CMS/CMS signalling protocol being specified in draft ITU-T Rec. J.cmss. The protocol for the Ann-3 is for further study.

5.4.4
Ann-4 interface – MP/MTA 

The interface Ann-4 defines the media stream format (RTP) for delivery of the announcement from the Announcement Player to the MTA. The specifics of interface Ann-4 are not within the scope of this Recommendation.

5.4.5
Audio Server Physical vs Logical configuration

It should be noted that the MPC and MP are logical components that MAY reside in the same physical entities. When logical components reside in the same physical entity, interfaces between these components are not exposed and become unspecified.

It should also be noted that stand-alone components using the Ann-2 and Ann-3 interfaces specified in this Recommendation MAY be shared by many network entities.

Figure 2 depicts an example of a network where the CMS, MPC, and MP are implemented as separate physical entities communicating over the Ann-2 and Ann-3 interfaces.


[image: image3.wmf]J.175_F02

CMS

MTA

Media Player

Media Player

Ann-3

interface

Ann-3

interface

Media Player

Controller

Media Player

Controller

Ann-1

interface

Ann-2

interface

Ann-2

interface

Ann-2

interface

Ann-4 media

stream


Figure 2/J.175 – Stand-alone components configuration

The MPC MAY be embedded with the CMS, as shown in Figure 3. In this case, interface Ann-3 is not exposed and becomes unspecified.


[image: image4.wmf]J.175_F03

CMS

Media Player

MTA

Ann-4 media stream

Embedded

Media Player

Controller

Media Player

Controller

Ann-1

interface

Ann-2

interface

Ann-2

interface


Figure 3/J.175 – Embedded MPC configuration

Similarly, the MP MAY be embedded with the MPC, as shown in Figure 4, in which case the interface Ann-2 is not exposed and becomes unspecified. 


[image: image5.wmf]J.175_F04

CMS

Embedded

Media Player

MTA

Ann-4 media stream

Media Player

Controller

Ann-1

interface

Ann-3


Figure 4/J.175 – Embedded MP configuration

Finally, the CMS and AS, (MPC and MP) may be embedded in the same physical entity, in which case the Ann-2 and Ann-3 interfaces are not exposed and become unspecified. (See Figure 5.)


[image: image6.wmf]J.175_F05

CMS

MTA

Ann-4 media stream

Ann-1

interface

Embedded

Audio Server

Media Player

Controller

Media Player


Figure 5/J.175 – Embedded AS configuration

5.5
Interface specifications

This Recommendation defines a set of interfaces between the components responsible for providing audio services. Figure 6 illustrates the interfaces between these components. Only where an interface is exposed is it expected to meet IPCablecom specification requirements. 


[image: image7.wmf]J.175_F06

CMS

MTA

Ann-1

Ann-3

Audio Server (AS)

Ann-4

Ann-1

Ann-2

Ann-3

PSTN

Ann-4

Media Gateway

(MG)

Media Gateway

Controller

(MGC)

Media Player

Controller

(MPC)

Media Player

(MP)


Figure 6/J.175 – IPCablecom audio server components and interfaces

6
Ann-1 interface: CMS-MTA and MGC-MG 

The CMS-MTA and MGC-MG announcement interfaces are implemented by the Legacy Audio Package of the NCS/TGCP protocol, which provides the playback of tones and fixed-content announcements to the end-users.

6.1
CMS-MTA interface

Each MTA in the network MAY store a predefined set of simple announcements locally. When an announcement is needed, the CMS will decide if it should instruct the MTA to play a local announcement or set up a connection between the MTA and a Network ANS and have the announcement played over the network. Playing simple announcements from the MTA saves network resources.

The MTA MAY store announcements in either static or dynamic memory. If announcements are stored in dynamic memory, then the announcements will not be available until the MTA has accessed them from the network.

These simple announcements will require only a small amount of storage on the MTA. Table 2 illustrates the storage requirements for such announcements. The example uses an average announcement time of 10 seconds.

Table 2/J.175 – ANS storage

	Number of announcements
	Announcement length (seconds)
	Encoding bytes/second
	Bytes required

	11
	10
	2000 (G.728)
	220 K

	11
	10
	8000 (PCMU/PCMA)
	880 K


MTAs require the ability to be updated dynamically with announcements so that the same MTA can move from service provider to service provider without requiring complete firmware upgrades. This capability is for further study and will need to be worked jointly with the IPCablecom architecture, security, and provisioning teams.

6.1.1
Announcement list

Table 3 lists the text version of the announcements that should be stored locally on the MTA. Each MTA is required to store and play announcements similar to those defined in Table 3. Announcements that address at least the following network conditions may be played using the Announcement Server Package defined in RFC 2705, Media Gateway Control Protocol, using URI (Universal Resource Identifiers) to identify the announcements. Cached versions of all announcement URIs should be refreshed every time the MTA connects to the network. Other methods of propagating new announcements to MTAs, for instance while the MTA remains in service, is an issue for study by the provisioning team and is beyond the scope of this Recommendation.

Table 3/J.175 – Sample announcements

	Sample announcement
	Name

	Your call cannot be completed as dialled. Please check the number and dial again.
	Vacant Code

	You must first dial a one or zero when calling this number. Please hang up and try your call again.
	Dial One or Zero

	You must first dial a one when calling this number. Please hang up and try your call again.
	Dial One First

	It is not necessary to dial a one when calling this number. Please hang up and try your call again.
	No Dial One

	If you'd like to make a call, please hang up and try again. If you need help, hang up and dial the operator.
	No Digits

	Your call cannot be completed as dialled. Please read the instruction card or call your operator for assistance.
	Assisted Dialling

	Your call did not go through. Please try your call again.
	Reorder

	All circuits are busy now. Please try your call later.
	No Circuit

	Due to facility trouble in the area you are calling, your call cannot be completed at this time. Please try your call later.
	Domestic Facility

	The party you are calling has declined to receive this call. Please try your call again with caller ID enabled.
	Unidentified Call Reject

	Thank you for using [carrier's name] 
	Branding


6.2
MGC-MG Interface

The MG announcement interface (Ann-1) allows for the MGC to request the MG play fixed-content announcements to PSTN end-users. The MGC/MG announcement interface package does not specify any standard announcements to be stored locally in the MG. All announcements are provisioned dynamically and are referenced accordingly.

This MG announcement provisioning capability is for further study and will need to be worked jointly with the IPCablecom architecture, security, and provisioning teams.

7
Ann-2 interface: MPC-MP

7.1
Introduction

An MP (Media Player) is a shared resource in the IPCablecom network that can be instructed to provide media services to an end-user or terminal. These services include streaming fixed-content, variable content and interactive announcements to IPCablecom subscribers. For example, the MP is responsible for playing prompts and collecting digits when charging a call to a calling card.

The MP is controlled by an external element, the MPC (Media Player Controller). The MPC-MP interface defines two new NCS announcement packages used to control the Media Player. The base audio package provides a standard set of IVR functions such as Play, PlayCollect, and PlayRecord. The advanced audio package is a superset of the Base Audio Package and provides additional capabilities.

The MP is responsible for managing its own resources. When accepting a request, the MP MUST make sure that the required resources are available before accepting request. When a single session involves multiple requests to the Media Player, the MP may experience a shortage of resources preventing it from accepting one given request belonging to that session. In this case, the MP user (i.e. the MPC) is responsible for re-sending the request or terminating the end-user session elegantly.

7.2
Audio package concepts

The base and advanced audio packages support both simple and complex audio structures. A simple audio structure might be a single announcement such as "Welcome to Bell South's Automated Directory Assistance Service." A more complex audio structure might consist of an announcement followed by voice variable followed by another announcement, for example "There are thirty-seven minutes remaining on your prepaid calling card," where "There are" is a prompt, the number of minutes is a voice variable, and "minutes remaining on your prepaid calling card" is another prompt.

It is also possible to define complex audio structures that are qualified by user-defined selectors such as language, audio file format, gender, accent, customer, or voice talent. For instance, if the above example were qualified by language and accent selectors, it would be possible to play "There are thirty-seven minutes remaining on your prepaid calling card" in English spoken with a southern accent or in English spoken with a mid-western accent, providing that the audio to support this had been provisioned.

There are two methods of specifying complex audio. The first is to directly reference the individual components. This requires a complete description of each component to be specified via the protocol. The second method is to provision the components on the audio server as a single entity and to export a reference to that entity to the call agent. In this case, only the reference (plus any dynamic data required, such as a variable data) is passed via the protocol, and no specification of individual components is necessary.

These packages provide significant functionality most of which is controlled via protocol parameters. Most parameters are optional, and where ever possible default to reasonable values. An audio application that references to complex provisioned audio structures can specify audio events using a minimum of syntax by taking advantage of parameter optionality and parameter defaults.

7.2.1
Understanding audio segments

An audio segment is a reference that resolves to one or more audio recordings. There are four types of audio segments: 

Physical: A physical segment is the simplest type of segment, a single recording. The recording could be a single word, such as "one", or an extended block of speech, such as "Our office is closed at this time. Please call back during normal business hours." Every physical segment is assigned a unique URI (Universal Resource Identifier) which among other things can be a hierarchical name, or a simple name or number.
Sequence: A sequence is a provisioned ordered list of audio segments. Every sequence is assigned a unique URI. A sequence can contain any of the four segment types (physical segments, other sequences, sets, and variables). On playback a sequence identifier is resolved to an ordered list of physical segments which are played in order. 

Set: A set is a provisioned collection of semantically related audio segments and an associated selector. Each set is assigned a unique URI. A set can contain physical segments, sequences, other sets, or variables. At runtime the value of the selector is used to determine which element of the set is played. 

Individual selector types are not defined in the syntax (except for the pre-defined language selector) and are instead defined by the provisioner. A provisioner could define one or more of the following selector types: language, accent, gender, accent, customer, or day of the week. For each selector type, the provisioner must define a range of valid values. The provisioner may also choose to define a default value. At runtime if a selector value is not supplied the default value is used.

Variable: A voice variable represents a single semantic concept (such as date or number) and dynamically produces the appropriate speech based on information supplied at runtime. Each provisioned voice variable is assigned a unique URI. For example, if an application needs to play a date, rather than telling the audio server to play each individual component of the date (e.g. "March" "twenty" "second" "nineteen" "ninety" "nine"), it can specify a voice variable of type date with value "19990322". The variable then assembles and plays the component audio needed to speak the date. Specification of variables is considered in more detail in a later clause of this Recommendation. 

7.2.2
Segment identifiers

Provisioned segments and segments recorded at runtime are identified by URIs as defined in RFC 2396, Uniform Resource Identifiers: Generic Syntax. 

A URI can be a simple name or it can be a URL. Three URL schemes are allowed: the file: scheme, the ftp:scheme, and the http: scheme. The file: scheme is used for audio local to the Audio Server. The ftp:scheme is used for audio remote to the Audio Server. The http: scheme can be used for audio local to the Audio Server using the http://local host convention or audio remote to the Audio Server. All audio references that require parameters encoded in the URL (e.g. set selectors) must use the http: scheme. Table 4 shows some of the possibilities.

Table 4/J.175 ( Example URIs

	Reference to local audio (flat file):

	
S: pa(an=file://welcome)

	Reference to local audio (flat file):

	
S: pa(an=file://12354)

	Reference to local audio:

	
S: pa(a=file://audio/xyztel/welcome)

	Reference to remote audio:

	
S: pa(n=http://audio/xyztel/welcome)


7.2.3
Segment life

Physical segments may be provisioned or they may be recorded during the course of a call. A physical segment recorded during the course of a call can be either transient or persistent. A transient physical segment lasts only for the life of the call during which it was recorded. A persistent physical segment lasts beyond the live of the call during which it was recorded.

7.2.4
Nested sets and sequences

Nested definition of both sets and sequences is allowed, i.e. it is legal to define a set of sets or a sequence of sequences. In addition, audio structures may also be specified by intermixing sets and sequences, and it is possible to specify a set of sequences or a sequence containing one or more set elements. Direct or transitive definition of a set or segment in terms of itself is not allowed.

Nesting of sets and sequences should be restricted to two or three levels.

7.2.5
Sequence example

In the following example, in Figure 7, a provisioner has provisioned one physical segment and two variable segments and has provisioned a sequence, http:/mysegment, which is an ordered list of the three segments. The sequence when played speaks the following: "Today's date is <weekday> <date>."


[image: image8.wmf]J.175_F07

2nd

Physical Segment

Date Variable

http://audio/my

-

sequence

http://audio/my

-

time

-

date

-

intro

http://audio/my

-

weekday

-

variable

http://audio/my

-

date

-

variable

 Week

day Variable

1st

3rd


Figure 7/J.175 ( Sequence example

7.2.6
Set example

To support an application which plays a particular piece of audio in either Arabic, Welsh, or Tibetan, a provisioner could define a set with the pre-defined selector, "lang", and would use define three of the possible values for that selector, "ara", "cym", and "tib". The provisioner would provision three audio segments, one in each language, and would associate the Arabic segment with the "ara" selector value, etc. The provisioner also could define a default value of the selector when no selector value is supplied, "ara" for instance. The entire set would be assigned a unique URI.

At runtime a reference to the set with the selector set to "cym" would result in the Welsh version of the prompt being played. A reference to the set with no selector would result in the Arabic version of the prompt being played since Arabic has been set as the default selector value. (See Figure 8.)


[image: image9.wmf]J.175_F08

CYM

http://audio/my

-multi-lang-segment

http://audio/my

-arabian-segment

http://audio/my

-

welsh-segment

http://audio/my

-

tibetan-segment

ARA

TIB


Figure 8/J.175 ( Set example

7.2.7
Set with nested sequence example

In this example, in Figure 9, the provisioner has provisioned three physical segments, one in Arabic, one in Welsh, and one in Tibetan, and the provisioner has also provisioned three date variables. Using these six segments the provisioner has provisioned three sequences, each consisting of a physical segment followed by a date variable. Finally the provisioner has provisioned a set consisting of the three sequences and with language as the set selector.

At run-time a reference to the set with the selector set to "ara" and a variable value of "20001015" would result in the following being played in Arabic: "Today's date is October 15th, 2000."


[image: image10.wmf]J.175_F09

CYM

http://audio/my

-multi-lang-segment

http://audio/my

-arabian-sequence

http://audio/my

-

welsh-

sequence

http://audio/my

-

tibetan-

sequence

ARA

TIB

1

2

1

2

1

2

http://audio/

ara/

date-is-segment

Date

Variable

http://audio/cym

/

date-is-segment

Date

Variable

http://audio/tib

/

date-is-segment

Date

Variable


Figure 9/J.175 ( Set with nested sequence example

7.3
Base Audio Package

7.3.1
Abstract

This event package provides support for the standard IVR operations of PlayAnnouncement, PlayCollect, and PlayRecord. It supports direct references to simple audio as well as indirect references to simple and complex audio. It provides audio variables, control of audio interruptibility, digit buffer control, special key sequences, and support for reprompting during data collection. 

Package Name: BAU

7.3.2
Events

Table 5/J.175 ( Events
	Symbol
	Definition
	R
	S
	Duration 

	pa(parms)
	PlayAnnouncement
	
	TO
	variable

	pc(parms)
	PlayCollect
	
	TO
	variable

	pr(parms)
	PlayRecord
	
	TO
	variable

	ma(parms)
	ManageAudio
	
	BR
	variable

	oc
	OperationComplete
	x
	
	

	of(parms)
	OperationFailed
	x
	
	


PlayAnnouncement: Plays an announcement in situations where there is no need for interaction with the user. Because there is no need to monitor the incoming media stream this event is an efficient mechanism for treatments, informational announcements, etc.

PlayCollect: Plays a prompt and collects DTMF digits entered by a user. If no digits are entered or an invalid digit pattern is entered, the user may be reprompted and given another chance to enter a correct pattern of digits. The following digits are supported: 0-9, *, and #. By default PlayCollect does not play an initial prompt, makes only one attempt to collect digits, and therefore functions as a simple Collect operation. Various special purpose keys, key sequences, and key sets can be defined for use during the PlayCollect operation.

PlayRecord: Plays a prompt and records user speech. If the user does not speak, the user may be reprompted and given another chance to record. By default PlayRecord does not play an initial prompt, makes only one attempt to record, and therefore functions as a simple Record operation. The call agent may specify a URI to be associated with the recording or the call agent may ask the Audio Server to allocate a URI and return it to the call agent as part of the OperationComplete event. Digits entered by the user during a recording that are not defined as special key sequences are ignored and become part of the recording.

ManageAudio: Performs audio management operations on persistent audio which typically not related to a current interaction with a user, e.g. "delete an audio segment".

OperationComplete: Detected upon the successful completion of a Play, PlayRecord, Play Collect, or ManageAudio signal.

OperationFailed: Detected upon the failure of a Play, PlayRecord, PlayCollect, or ManageAudio signal.

7.3.3
Signal interactions

If an Audio Package signal is active on an endpoint and another signal of the same type is applied, the two signals including parameters and parameter values will compared If the signals are identical, the signal in progress will be allowed to continue and the new signal will be discarded. Because of this behavior the Advanced Audio Package may not interoperate well with some other packages such as the Line and Trunk packages.

7.3.4
Parameters

The PlayAnnouncement, PlayRecord, and PlayCollect events may each be qualified by a string of parameters, most of which are optional. Where appropriate, parameters default to reasonable values. If a required parameter is not supplied, an error is returned to the application.

These parameters are shown in Table 6:

	Table 6/J.175 ( Parameters

	Symbol 
	Definition 
	pa
	pc
	pr
	ma

	an 
	announcement 
	O
	F
	F
	F

	ip 
	initial prompt 
	F
	O
	O
	F

	rp 
	reprompt 
	F
	O
	O
	F

	nd 
	no digits reprompt 
	F
	O
	F
	F

	ns 
	no speech reprompt 
	F
	F
	O
	F

	fa 
	failure announcement 
	F
	O
	O
	F

	sa 
	success announcement 
	F
	O
	O
	F

	off
	offset
	O
	O
	O
	F

	ni 
	non-interruptible play 
	F
	O
	O
	F

	it 
	iterations 
	O
	F
	F
	F

	iv 
	interval 
	O
	F
	F
	F

	du 
	duration 
	O
	F
	F
	F

	sp 
	speed 
	O
	O
	O
	F

	vl 
	volume 
	O
	O
	O
	F

	cb 
	clear digit buffer 
	F
	O
	O
	F

	dm 
	digit map 
	F
	O
	O
	F

	fdt 
	first digit timer 
	F
	O
	F
	F

	idt 
	inter digit timer 
	F
	O
	O
	F

	edt 
	extra digit timer 
	F
	O
	F
	F

	prt 
	prespeech timer 
	F
	F
	O
	F

	pst 
	postspeech timer 
	F
	F
	O
	F

	rlt 
	recording length timer 
	F
	F
	M
	F

	rsk 
	restart key 
	F
	O
	O
	F

	rik 
	reinput key 
	F
	O
	O
	F

	rtk 
	return key 
	F
	O
	O
	F

	na 
	number of attempts 
	F
	O
	O
	F

	ap
	append
	F
	F
	O
	F

	rid
	recording id
	F
	F
	M
	F

	rpa
	record persistent audio
	F
	F
	F
	O

	dpa
	delete persistent audio
	F
	F
	F
	O

	O
Optional

M
Mandatory

F
Forbidden


Announcement: An announcement to be played. Consists of one or more audio segments.

Initial Prompt: The initial announcement prompting the user to either enter DTMF digits or to speak. Consists of one or more audio segments. If not specified (the default), the event immediately begins digit collection or recording.

Reprompt: Played after the user has made an error such as entering an invalid digit pattern or not speaking. Consists of one or more audio segments. Defaults to the Initial Prompt. 

No Digits Reprompt: Played after the user has failed to enter a valid digit pattern during a PlayCollect event. Consists of one or more audio segments. Defaults to the Reprompt.

No Speech Reprompt: Played after the user has failed to speak during a PlayRecord event. Consists of one or more audio segments. Defaults to the Reprompt.

Failure Announcement: Played when all data entry attempts have failed. Consists of one or more audio segments. No default.

Success Announcement: Played when data collection has succeeded. Consists of one or more audio segments. No default.

Offset: Specifies the offset into an announcement to start playing. Offset must only be used with the initial prompt of the PlayCollect or PlayRecord events where the initial prompt is a single physical segment. An offset must be either positive or negative. A positive offset is the offset going forward from the beginning of the prompt. A negative offset is the offset going backwards from the end of the prompt. Offsets are specified in 10-millisecond units. Defaults to 0.

Offsets are useful when digit handling is done by the call agent, e.g. the user hits a DTMF key, the key is sent to the call agent, the call agent decides to ignore the key and tells the Audio Server to resume playing at the point of interrupt. Another application is to allow the user to skip back and forward in a physical segment.

NonInterruptible Play: If set to true, the initial prompt of the PlayCollect or PlayRecord event is not interruptible by either voice or digits. Defaults to false. Valid values are the text strings "true" and "false." Digits entered during a non-interruptible initial prompt are accumulated and are treated as they would if they had been entered during the second (collect or record) phase of the event.

Iterations: The maximum number of times an announcement is to be played. A value of minus one (–1) indicates the announcement is to be repeated forever. Defaults to one (1).

Interval: The interval of silence to be inserted between iterative plays. Specified in units of 100 milliseconds. Defaults to 10 (one second). 

Duration: The maximum amount of time to play and possibly replay an announcement. Takes precedence over iteration and interval. Specified in units of 100 milliseconds. No default.

Speed: The relative playback speed of announcement specifiable as a positive or negative percentage of the original playback speed.

Volume: The relative playback volume of announcement specifiable as a positive or negative decibel variation from the original playback volume.

Clear Digit Buffer: If set to true, clears the digit buffer before playing the initial prompt. Defaults to false. Valid values are the text strings "true" and "false."

Digit Map: A digit map as specified in RFC 2705, Media Gateway Control Protocol (MGCP) Version 1.0, which specifies one or more digit patterns to be collected. Valid digits are 0-9, *, and #.
First Digit Timer: The amount of time allowed for the user to enter the first digit. The first digit starts after the announcements end. Specified in units of 100 milliseconds. Defaults to 50 (five seconds).

Inter Digit Timer: The amount of time allowed for the user to enter each subsequent digit. Specified units of 100 milliseconds seconds. Defaults to 30 (three seconds).

Extra Digit Timer: The amount of time to wait for a user to enter a final digit once the maximum expected amount of digits have been entered. Typically this timer is used to wait for a terminating key in applications where a specific key has been defined to terminate input. Specified in units of 100 milliseconds. If not specified, this timer is not activated. If an extra digit is entered it is returned to the application along with the other collected digits.

The Extra Digit Timer can be used to implement a consistent human interface when collecting a variable number of digits where collection can be terminated by a Return Key, typically the # key. For example, suppose an application has asked for a minimum of three digits and a maximum of six. If the user consistently uses the # key to terminate collection following digit strings are acceptable: xxx#, xxxx#, xxxxx#, and xxxxxx. The inconsistency arises when the user enters six digits. Because the maximum number of digits have been entered the Audio Server returns the digits immediately without waiting for the # key. If the type ahead is allowed (the default Audio Server behavior) and if user then enters the # key, the application has to decide whether the user meant the # key to terminate the six digits already collected or if the user meant to enter the # key to begin the next digit collection. The Extra Digit Timer tells the Audio Server to wait for an additional period of time after the maximum number of digits have been entered to see if the user is going to enter another key. 

Prespeech Timer: The amount of time to wait for the user to initially speak. Specified in units of 100 milliseconds. Defaults to 30 (three seconds).

Postspeech Timer: The amount of silence necessary after the end of the last speech segment for the recording to be considered complete. Specified in units of 100 milliseconds. Defaults to 50 (five seconds).

Recording Length Timer: The maximum allowable length of the recording, not including pre or post speech silence. Specified in units of 100 milliseconds. This parameter is mandatory for the PlayRecord signal. A value of –1 (minus one) means there is no limit to recording length. In this case the recording is open ended, and it is up to the application to manage the storage resources for recordings.

Restart Key: Defines a digit map that, if matched, has the following action: discard any digits collected or recording in progress, replay the prompt, and resume digit collection or recording. No default. 

The use of this key does not constitute an attempt to enter user input (i.e. it does not count against the number of attempts specified by the Number Of Attempts parameter). Restart Keys are handled locally by the Audio Server and are not returned to the call agent. During a recording, all digits except for the restart, reinput, and return keys (if defined) are ignored and become part of the recording.

Reinput Key: Defines a digit map that, if matched, has the following action: discard any digits collected or recordings in progress and resume digit collection or recording. No default. 

The use of this key does not constitute an attempt to enter user input (i.e. it does not count against the number of attempts specified by the Number Of Attempts parameter). Reinput keys are handled locally by the Audio Server and are not returned to the call agent. During a recording, all digits except for the restart, reinput, and return keys (if defined) are ignored and become part of the recording.

Return Key: Defines a digit map that, if matched, has the following action: stop digit collection or recording. If the return key is hit during a PlayCollect event, all keys collected are returned to the call agent. If the return key is hit during a PlayRecord event, the recording is saved, all keys collected are returned, and a Recording ID is returned if appropriate. (See definition of Recording ID for details) No default. 

Number Of Attempts: The number of attempts the user is allowed to enter a valid digit pattern or to make a recording. Defaults to 1. Also used as a return parameter to indicate the number of attempts the user made.

Append: If set to true, the audio recording will append to any existing content in the Recording ID. It may not be used with wildcarded Recording Ids. Valid values are "true" and "false". 

Recording ID: A URI to be assigned to the physical segment which is to be recorded by the PlayRecord event. If this parameter is set to the ANY wildcard, "$", the Audio Server will allocate the URI, associate it with the newly recorded segment, and return it to the call agent with the OperationComplete event.

Record Persistent Audio: If set to true, the recording that is made is persistent instead of temporary. Defaults to false. Valid values are the text strings "true" and "false."

Delete Persistent Audio: Indicates that the specified persistent audio segment is to be deleted. This parameter is carried by the PlayRecord event, although nothing is either played or recorded in this case.

7.3.5
Type-ahead

The Audio Server supports type-ahead by default. Type-ahead is not supported for the Play event because by definition no digit collection is done during this event. Type-ahead can be turned off for all prompts associated with a PlayCollect or PlayRecord event by setting the Clear Digit Buffer parameter.

7.3.6
Return parameters

Each event has an associated set of possible return parameters which are returned with either the OperationComplete or OperationFailed events. These parameters are listed in Table 7:

Table 7/J.175 – Return parameters

	Symbol
	Definition
	pl
	pc
	pr
	ma

	ap 
	amount played
	F
	C
	C
	F

	dc 
	digits collected 
	F
	O
	O
	F

	na 
	number of attempts 
	F
	M
	M
	F

	rc 
	return code 
	O
	O
	O
	O

	rl
	recording length
	F
	F
	M
	F

	rid 
	recording id 
	F
	F
	O
	F

	O
Optional

M
Mandatory

F
Forbidden

C
Conditional (see expanded definition)


Amount Played: The length played of an initial prompt if the prompt was interrupted, in 10‑ms units. This parameter is mandatory if the prompt was interrupted, and forbidden otherwise.

Digits Collected: If returned with an oc event, this parameter contains the DTMF digits that were collected during a PlayCollect operation. If returned with an of event, this parameter contains the DTMF digits that were collected during an unsuccessful PlayCollect or PlayRecord operation up until the point of failure.

Number Of Attempts: The number of attempts the user actually needed to enter a valid digit pattern or to make a recording. Defaults to 1. Also used as an input parameter to specify the number of attempts the user will be allowed to enter a valid digit pattern or make a recording. This parameter is returned only if a na parameter was specified on the PlayCollect or PlayRecord.

Recording Length: The length of the recording, not including pre or post speech silence. Specified in units of 100 milliseconds. This parameter is mandatory for the PlayRecord signal. In the case where the append operation was used, this is the length of the new recording, not the total length. 

Recording ID: A URI assigned to physical segment recorded by the PlayRecord operation. This parameter is returned only if the RecordingID parameter to the PlayRecord event has been set to the ANY wildcard, "$". If this is the case the Audio Server allocates a unique URI, associates it with the newly recorded segment, and returns it to the call agent.

Return Code: A return code giving the final status of the operation (see Table 8): 

Table 8/J.175 ( Return codes

	Return code
	Meaning

	600
	Illegal syntax 

	601
	Unknown segment ID 

	602
	Variable type not supported 

	603
	Variable subtype not supported 

	604
	Invalid variable name 

	605
	Variable value out of range 

	606
	Inconsistent variable specification 

	607
	Extra sequence data 

	608
	Missing sequence data 

	609
	Mismatch between play specification and provisioned data 

	610
	Delete audio error 

	611
	Unable to record temporary audio 

	612
	Unable to delete temporary audio 

	613
	Unable to record persistent audio 

	614
	Unable to delete persistent audio 

	615
	Unable to override non-existent segment id 

	616
	Unable to remove override from non-existent segment id 

	617
	Provisioning error 

	618
	Hardware failure 

	619
	Unspecified failure 

	620
	No digits 

	621
	No speech 

	622
	Spoke too long 

	623
	Digit map not matched 

	624
	Max attempts exceeded 

	625
	No free segment ids 

	626
	Required parameter not set 

	627
	Inconsistent parameter set

	628
	Value out of range

	629
	Invalid offset

	630
	Invalid digit map


Examples:

The PlayAnnouncement event completed successfully. Note that no return code is necessary:

O: BAU/oc

The PlayAnnouncement event failed the parameters supplied were inconsistent:

O: BAU/of(rc=633)

The PlayCollect event completed successfully on the user's second attempt when the user entered the digits 04375182:

O: BAU/oc(na=2 dc=04375182)

The PlayRecord event was successful on the user's first attempt; the id of the recording made by the user is 983:

O: BAU/oc(na=1 ri=983)

The PlayRecord event was successful on the user's first attempt; the id of the recording made by the user is 983 and the duration was 27.5 seconds: 

O: BAU/oc(na=1 ri=983 rl=275)

7.3.7
Segment descriptors

Segment descriptors are used with the an, ip, rp, nd, ns, fa, and sa parameters to define the segments that make up an announcement. Table 9 shows two kinds of segment descriptors: 

Table 9/J.175 ( Segment descriptors

	Symbol
	Definition

	<URI>
	segment identifier

	vb
	variable


Segment Identifier: A URI identifying a provisioned entity, i.e. a physical segment, sequence, or variable.

Variable: Specifies a voice variable by type, subtype, and value, and used when the application specifies a variable on the fly as opposed to referencing a provisioned variable. Does not apply to provisioned variables. Variables are more completely defined in a subsequent clause of this Recommendation.

7.3.8
Variable syntax

The syntax supports two kinds of variables. Embedded variables are variables that have been provisioned as part of an audio segment. At run‑time the call agent references the segment and specifies a value for the variable. Typically embedded variables are provisioned along with recorded speech, e.g. "A representative will be with you in approximately 5 minutes. If you would prefer to leave a voice message, press 1 now." where the variable is the number of minutes. Stand‑alone variables are variables that are not provisioned and therefore must be completely specified on the fly by the call agent. Variables are specified by the following parameters: type, subtype, and value. Variable types include Date, Money, Number, Time, etc. Subtype is a refinement of type. For example the variable type Money might have an associated range of subtypes such as Dollar, Rupee, Dinar, etc. Not all variables require a subtype, and for these variables the subtype parameter should be set to null.

For embedded variables, the type and subtype must be provisioned. The value may be provisioned. If it is not provisioned it must be specified as part of the variable reference. In a list of segments, an embedded variable value specification applies only to the segment that directly precedes it. If a segment has multiple embedded variables, the values must be given in the order in which the variables are encountered when the segment is played. Some examples:

Standalone variable: S: pa(an=vb(mny,usd,1153))

Embedded variable: S: pa(an=file://ann1<1153>)

Not all variables, such as the date variable shown in the next example, require a subtype. In that case, the subtype is encoded with the value "null": 

S: pa(an=vb(dat,null,101598))

In some cases it may be desirable to play an announcement that contains an embedded variable without playing the variable itself. To do this a single "null" is provided for the value: 

S: pa(an=file://ann1<null>)

7.3.9
Variable definitions

Variable types and subtypes are specified in Table 10.

Table 10/J.175 ( Variable types and subtypes

	Type
	Subtype
	 Definition 

	dat
	mdy, dmy, etc.
	Date 

	
	mdy
	Month-Day-Year

	
	dym
	Day-Year-Month

	dig
	gen, ndn
	Digits 

	
	gen 
	Generic 

	
	ndn 
	North American DN 

	dur
	 
	Duration 

	mth
	
	Month 

	mny
	<ISO 4217 three-letter codes> 
	Money 

	num
	crd, ord 
	Number 

	
	crd 
	Cardinal 

	
	ord 
	Ordinal 

	sil
	
	Silence 

	str
	 
	String 

	tme
	t12, t24
	Time 

	
	t12 
	Twelve-hour format 

	
	t24 
	Twenty-four-hour format 

	wkd
	
	Weekday


Date: Speaks a date specified as YYYYMMDD (per ISO 8601, Data elements and interchange formats – Information interchange – Representation of dates and times). If the subtype is Month-Date-Year "20001015", for example would be spoken as "October Fifteenth Two Thousand." If the subtype is Date-Month_Year the same date would be spoken as "Fifteen October Two Thousand." Date subtypes may be extended as needed as long as they are patterned after the existing subtypes (i.e. they must be a three-letter combination of the letters m, d and y).

Digits: Speaks a string of digits one at a time. If the subtype is North American DN, the format of which is NPA-NXX-XXXX, the digits are spoken with appropriate pauses between the NPA and NXX and between the NXX and XXXX. If the subtype is generic, the digits are spoken with no pauses.

Duration: Duration is specified in seconds and is spoken in one or more units of time as appropriate, e.g. "3661" is spoken as "One hour, one minute, and one second", "3360" is spoken as "One hour and one minute", and "3600" is spoken as "One hour."

Money: Money is specified in the smallest units of a given currency and is spoken in one or more units of currency as appropriate, e.g. "110" in U.S. Dollars would be spoken "one dollar and ten cents." The three-letter codes defined in ISO 4217, codes for the representation of currencies and funds are used to specify the currency subtype. A small excerpt from ISO 4217 is given in Table 11:

Table 11/J.175 ( Sample currency codes

	Code 
	Currency 
	Entity 

	GQE 
	Ekwele 
	Equatorial Guinea 

	GRD 
	Drachma 
	Greece 

	GTQ 
	Quetzal 
	Guatemala 


Money can be specified in negative or positive units of currency. In the above example "–110" would be spoken as "minus one dollar and ten cents."

Month: Speaks the specified month, e.g. "10" is spoken as "October." Specification is in MM format with "01" denoting January, "02" denoting February, etc.

Number: Speaks a number in cardinal form or in ordinal form. For example, "100" is spoken as "one hundred" in cardinal form and "one hundredth" in ordinal form. Cardinal numbers can be specified as negative or positive.

Silence: Plays a specified period of silence. Specification is in 100 millisecond units.

String: Speaks each character of a string, e.g. "a34bc" is spoken "A, three, four, b, c." Valid characters are a-z, A-Z, 0-9, #, and *.

Time: Speaks a time in either twelve hour format or twenty four hour format depending on the specified subtype. For example "1700" is spoken as "Five p.m." in twelve hour format or as "Seventeen hundred hours" in twenty four hour format. Specification is in HHMM format per ISO 8601, Data elements and interchange formats ( Information interchange ( Representation of dates and times.

Weekday: Speaks the day of the week, e.g. "Monday." Weekdays are specified as single digits, with "1" denoting Sunday, "2" denoting Monday, etc.

7.3.10
Examples

This clause presents a number of syntax examples. Play an announcement that consists of a single segment:

S: pa(an=file://12333)

Play an announcement that consists of multiple segments:

S: pa(an=file://ann798,file://ann300,file://ann4747)

Play an announcement that consists of a recording followed by three seconds of silence followed by a stand‑alone voice variable:

S:pa(an=file://ann357,vb(sil,null,30),vb(my,usd,3999))

Play an announcement with an embedded variable. If the separate segments of the previous announcement were provisioned as a sequence with a segment id of ann43321, the following would be exactly equivalent to the previous example:

S: pa(an=file://ann43321<3999>)

Play an announcement with two embedded variables:

S: pa(an=http://jackstraw/audio/xyztel/hello

 <3999,10151998>)

Play a prompt and collect a single digit. If need be, play a reprompt, a no digits prompt, and a success or failure announcement. Give the user three attempts to enter a digit:

S: pc(ip=file://ann27 rp=file://ann19 nd=file://ann102

 fa=file://ann8 sa=file://ann777 na=file://ann31 

 dm=x)

Play a prompt and collect a single digit. If the user does not enter a digit replay the initial prompt. Give the user three attempts to enter a digit:

S: pc(ip=file://audio/ann77775 na=3 dm=x)

Play a prompt and record voice. If the user does not speak, play a no speech prompt. Give the user two attempts to record:

S: pr(ip=http://brenda/audio/ann070500

 ns=http:/althea/audio/no-speech na=2)

Play an announcement at ninety percent of its original speed and five decibels softer than its original volume. Play the announcement three times with two seconds of silence between plays.

S: pa(an=file://ann276 sp=90 vl=-5 it=3 iv=20)

Give the user two attempts to enter a three-digit pattern. Clear the digit buffer before playing the prompt. 

S: pc(ip=file://438975 cb=true dm=xxx na=2)

Give the user three attempts to enter a three-digit pattern. If the user enters one digit or two digits on the first or second attempts, a reprompt is played. If the user enters no digits on the first or second attempts, a no digits reprompt is played. If all three attempts fail, a failure announcement is played. If one of the attempts is successful, a success announcement is played and the collected digits are returned to the call agent. 

S: pc(ip=file://ann493 rp=5 nd=409 fa=file://ann923

 sa=file://ann18337 dm=xxx)

Give the user three chances to enter an 11-digit number that begins with 0 or 1. If the user makes a mistake while entering digits, he can press the * key to discard any digits already collected, replay the prompt, and resume collection.

S: pc(ip=http://stella/blue/audio/ann5684

 dm=0xxxxxxxxxx|1xxxxxxxxxx rsk=* na=3)

Give the user two chances to make a recording. After playing the prompt, wait 5 seconds for the user to speak, otherwise replay the initial prompt and try again. If the user does speak, wait for seven seconds after speech stops to make sure the user is finished. If the recording is successful, return a reference to the recording to the call agent.

S: pr(ip=file://ann432 prt=50 pst=70 na=2)

7.4
Advanced Audio package

7.4.1
Abstract

The Advanced Audio package extends the Base Audio package by adding the set capability which the user can use to create an arbitrary number of user-defined qualifiers to be used in resolving complex audio structures. For example, the user could define qualifiers for any or all of the following: language, accent, audio file format, gender, speaker, or customer.

Package Name: AAU

7.4.2
Sets

A set is a provisioned collection of semantically related audio segments with an associated selector. Each set is assigned a unique URI. A set can contain physical segments, sequences, other sets, or variables. At run‑time the value of the selector is used to determine which element of the set is played. 

Individual selector types are not defined in the syntax (except for the pre-defined language selector, "lang") and are instead defined by the provisioner. A provisioner could define one or more of the following selector types: language, accent, gender, accent, customer, or day of the week. For each selector type, the provisioner must define a range of valid values. The provisioner may also choose to define a default value. At run‑time, if a selector value is not supplied, the default value is used.

7.4.3
Selectors 

Selector types, except for the predefined "lang" (language) selector, are defined by the user. For each selector type, the user must define a range of values that the selector can assume. 

Selectors apply to individual audio segments. If an event specifies multiple segments, each segment may have its own set of selectors. If selectors are not specified for an audio segment, provisioned defaults are used.

For example, if the user defines a selector of type "phaseofthemoon", he might also define the legal values for that selector to be "new", "half", "full", "harvest", and "blue". For the selector to actually work at run‑time, audio associated with each of the selector values must be provisioned.

The three-letter codes defined in ISO standard 639-2, Code For The Representation Of Names Of Languages, must be used as values for user-defined language selectors. For languages that have both a bibliographic and a terminology code, both codes should be supported. A small excerpt from ISO 639-2 follows in Table 12:

Table 12/J.175 ( Sample language codes

	Code
	Language 

	cze
	Czech

	cym
	Welsh

	dan
	Danish


Selectors are applied to variables only after the variable has been resolved. For instance if a date variable resolved to "October 15th, 1998" the voice with which the variable is spoken could resolve to either male or female if a gender selector had been defined.

Selectors are encoded as parameters to the URI segment id. If the URI refers to a physical segment on a node other than the Audio Server, to fetch the audio from the remote node the URI must contain the information necessary for this node to resolve the URI to a specific physical segment. This does not imply that the remote node needs the same capability as the Audio Server to resolve complex audio references. The remote node could for example use a simple scheme such as encoding the hierarchical directory path to the physical in the URI.

7.4.4
Selector encoding

Provisioned segments and segments recorded at run‑time are identified by URIs as defined in RFC 2396, Uniform Resource Identifiers: Generic Syntax. 

A URI can be a simple name or it can be a URL. If a URL refers to audio stored on a node other than the Audio Server, it must contain all the information necessary to resolve the URL to a physical segment. If the URL refers to a set, the selector types and values necessary to resolve the URL to a physical segment must be encoded in the query field of the URL. URLs for audio local to the Audio Server should use the file: scheme. URLs for audio remote to the Audio Server should use the http: scheme. Table 13 shows some of the possibilities.

Table 13/J.175 ( Example URIs

	Reference to local audio (set):

	
S: pa(an=http://localhost/audio/xyztel/welcome?lang=eng&gender=female) 

	Reference to remote audio (set):

	
S: pa(an=http://audio/xyztel/welcome?lang=eng&gender=female)


7.4.5
Variable order

When a provisioned segment containing more than one variable is referenced at run-time, the variable values must be supplied in the order in which they occur in the provisioned segment. This principle extends to sets. If the elements of a set contain more than one variable, then for all elements of the set the variables must occur in the same order. Sets with elements containing variables that do not appear in the same order are not supported.

7.4.6
Overrides

A provisioned physical segment may be replaced (or overridden) by a persistent physical segment. The URI of the provisioned physical segment will then resolve to the persistent physical segment. The overriding persistent audio can subsequently be deleted and the original provisioned audio can be restored.

A provisioned physical segment may be overridden more than once. In this case, the URI of the provisioned physical segment refers to the latest overriding physical segment. When the overriding physical segment is deleted, the original provisioned physical segment is restored, even if the segment has been overridden multiple times.

Segment override could be used for a feature where a standard greeting is played to all customers calling a retail store. Occasionally the store manager may want to call a special number and record a temporary greeting that overrides the standard greeting, for instance a greeting that announces a sale or maybe a seasonal greeting of some kind. When the greeting is no longer wanted, the manager can call the special number, cancel the temporary greeting, and restore the standard greeting.

7.4.7
Parameters

See Table 14.

Table 14/J.175 ( Parameters

	Symbol 
	Definition 
	pa 
	 pc 
	 pr 
	ma

	oa
	override persistent audio
	F
	F
	F
	O

	ra
	restore persistent audio 
	F
	F
	F
	O

	O
Optional

M
Mandatory

F
Forbidden


Override Persistent Audio: The id of the segment to be overridden and the id of the overriding segment.

Restore Persistent Audio: The id of the segment to be restored.

7.4.8
Return codes

The following return codes are defined for the Advanced Audio Package:

Table 15/J.175 – Return Codes

	Return code
	Meaning

	650
	Bad selector type 

	651
	Bad selector value

	652
	Missing selector

	653
	Missing selector value

	654
	Wrong number of selector

	655
	Remove override error 

	656
	Override error 

	657
	Unable to override non-existent segment id

	658
	Unable to remove override from non-existent segment id


7.4.9
Examples

This clause presents a number of examples of how sets and selectors are used.

Play an announcement in English. 

S: pa(an=file://audio/xyztel/hello?lang=eng) 

Play an announcement in a Danish, female voice with a Cajun accent.

S: pa(an=file://audio/xyztel/hello?lang=dan&

 gender=female&accent=cajun)

Play the first part of an announcement in English, the second part in the default language, and the third part in French.

S: pa(an=file://ann1?lang=eng,file://ann2,

 file://ann2?lang=fra)

Play an announcement with an embedded variable in English (the embedded variable is also played in English):

S: pa(an=file://ann4?lang=eng<101599>)

7.5
Formal syntax description

This description uses ABNF (RFC 2234) to formally describe the syntax of the Basic Audio Package and the Advanced Audio Package. The two packages have the same syntax except for the encoding of selector types and selector values in the query field of the URI and the persistent audio override capabilities. See RFC 2396 for the syntax of encoding parameter value pairs in the query field of the URL. 

AudPkgEvent = PlayAnnouncement / PlayCollect / PlayRecord / ManageAudio / OperationComplete / OperationFailed 

PlayAnnouncement = [ AudioPkgToken SLASH ] PlayAnnToken

LPAREN PlayAnnParmList RPAREN 

PlayCollect = [ AudioPkgToken SLASH ] PlayColToken

LPAREN [ PlayColParmList ] RPAREN 

PlayRecord = [ AudioPkgToken SLASH ] PlayRecToken

LPAREN [ PlayRecParmList ] RPAREN

ManageAudio = [AudioPkgToken SLASH] ManageAudToken LPAREN ManageAudParmList RPAREN

OperationComplete = [ AudioPkgToken SLASH ] OpCompleteToken

LPAREN [OpCompleteParmList ] RPAREN

OperationFailed = [ AudioPkgToken SLASH ] OpFailedToken

LPAREN ReturnCodeParm RPAREN

PlayAnnParmList = PlayAnnParm *( WSP PlayAnnParm )

PlayColParmList = PlayColParm *( WSP PlayColParm )

PlayRecParmList = PlayRecParm *( WSP PlayRecParm )

ManageAudParmList = ManageAudParm *( WSP ManageAudParm )

OpCompleteParmList = OpCompleteParm *( WSP OpCompleteParm )

PlayAnnParm = ( AnnouncementParm / IterationsParm / IntervalParm /

DurationParm / SpeedParm / VolumeParm )

PlayColParm = ( InitPromptParm / RepromptParm / NoDigitsParm / FailAnnParm /

SuccessAnnParm / NoInterruptParm / SpeedParm / VolumeParm /

ClearBufferParm / DigitMapParm / FirstDigitParm / InterDigitParm /

ExtraDigitParm / RestartKeyParm / ReinputKeyParm /

ReturnKeyParm / NumAttemptsParm )

PlayRecParm = ( InitPromptParm / RepromptParm / NoSpeechParm / FailAnnParm /

SuccessAnnParm / NoInterruptParm / SpeedParm / VolumeParm /

ClearBufferParm / PreSpeechParm / PostSpeechParm /

RecordLenTimerParm / RestartKeyParm / ReinputKeyParm /

ReturnKeyParm / NumAttemptsParm )

ManageAudParm = (RecPersistParm / DeletePersistParm / OverrideAudioParm / RestoreAudioParm)

OpCompleteParm = ( NumAttemptsParm / AmtPlayedParm / DigitsColParm 

RecordingIdParm / ReturnCodeParm / RecordLenParm) 

AnnouncementParm = AnParmToken EQUALS Segmentlist

InitPromptParm = IpParmToken EQUALS Segmentlist

RepromptParm = RpParmToken EQUALS Segmentlist

NoDigitsParm = NdParmToken EQUALS Segmentlist

NoSpeechParm = NsParmToken EQUALS Segmentlist

FailAnnParm = FaParmToken EQUALS Segmentlist

SuccessAnnParm = SaParmToken EQUALS Segmentlist

OffsetParm = OffParmToken EQUALS OPTSIGNEDINT

DurationParm = DuParmToken EQUALS NUMBER

IterationsParm = ItParmToken EQUALS ( NUMBER / MINUSONE )

IntervalParm = IvParmToken EQUALS NUMBER

SpeedParm = SpParmToken EQUALS SIGNEDINT

VolumeParm = VlParmToken EQUALS SIGNEDINT

NoInterruptParm = NiParmToken EQUALS BOOLSTR

ClearBufferParm = CbParmToken EQUALS BOOLSTR

DigitMapParm = DmParmToken EQUALS DigitMap 

DigitMap = <defined in RFC 2705>

FirstDigitParm = FdtParmToken EQUALS NUMBER

InterDigitParm = IdtParmToken EQUALS NUMBER

ExtraDigitParm = EdtParmToken EQUALS NUMBER

PreSpeechParm = PrtParmToken EQUALS NUMBER

PostSpeechParm = PstParmToken EQUALS NUMBER

RecordLenParm = RlParmToken EQUALS NUMBER

RecordLenTimerParm = RltParmToken EQUALS NUMBER

RestartKeyParm = RskParmToken EQUALS DigitMap

ReinputKeyParm = RikParmToken EQUALS DigitMap

ReturnKeyParm = RtkParmToken EQUALS DigitMap

RecPersistParm = RpaParmToken EQUALS BOOLSTR

DeletePersistParm = DpaParmToken EQUALS SegmentId

OverrideAudioParm = OaParmToken EQUALS OverridenSegId OverridingSegId

OverridenSegId = SegmentId

OverridingSegId = SegmentId

RestoreAudioParm = RaParmToken EQUALS SegmentId
NumAttemptsParm = NaParmToken EQUALS NUMBER

AmtPlayedParm = ApParmToken EQUALS NUMBER

DigitsColParm = DcParmToken EQUALS KeySequence

RecordingIdParm = RidParmToken EQUALS UniversalResourceIdentifier

ReturnCodeParm = RcParmToken EQUALS 3*3(DIGIT)

KeyPadKey = "0" / "1" / "2" / "3" / "4" / "5" / "6" / "7" / "8" / "9" / "*" / "#"

KeySequence = 1*64(KeyPadKey)

KeySet = 1*11(KeyPadKey)

Segmentlist = SegmentDescriptor *( COMMA SegmentDescriptor )

SegmentDescriptor = SegmentId [ EmbedVarList ] / VariableSeg 

SegmentId = UniversalResourceIdentifier

UniversalResourceIdentifier = <defined in RFC 2396>

VariableSeg = VariableSegToken LPAREN FullSpecVar RPAREN

EmbedVarList = LANGLE NAME *( COMMA NAME ) RANGLE

FullSpecVar = ( DateVariable / DigitsVariable / DurationVariable /

MonthVariable / MoneyVariable / NumberVariable /

SilenceVariable / StringVariable / TextVariable /

TimeVariable / WeekdayVariable )

DateVariable = DateVarToken COMMA NullStrToken COMMA Date

Date = 8*8(DIGIT)

DigitsVariable = DigitsVarToken COMMA (NorthAmericanDnToken /

GenericDigitsToken) COMMA NUMBER

DurationVariable = DurationVarToken COMMA NullStrToken COMMA NUMBER

MoneyVariable = MoneyVarToken COMMA 3*3(ALPHA) COMMA OPTSIGNEDINT

MonthVariable = MonthVarToken COMMA NullStrToken COMMA Month

Month = "01" / "02" / "03" / "04" / "05" / "06" / "07" / "08" / "09" / "10" / "11" / "12"

NumberVariable = 

(NumberVarToken COMMA CardinalNumberToken COMMA OPTSIGNEDINT) / 

(NumberVarToken COMMA OrdinalNumberToken COMMA NUMBER)

SilenceVariable = SilenceVarToken COMMA NullStrToken COMMA NUMBER

StringVariable = StringVarToken COMMA NullStrToken COMMA *(KeyPadKey)

SilenceVariable = SilenceVarToken COMMA NullStrToken COMMA NUMBER

StringVariable = StringVarToken COMMA NullStrToken COMMA *(KeyPadKey)

TimeVariable = TimeVarToken COMMA (TwelveHourFormatToken /

TwentyFourHourFormatToken) COMMA 4*4(DIGIT)

WeekdayVariable = WeekdayVarToken COMMA NullStrToken COMMA NAME

AudioPkgToken = BaseAudPkgToken / AdvAudPkgToken

BaseAudPkgToken = "BAU"

AdvAudPkgToken = "AAU"

PlayAnnToken = "pa"

PlayColToken = "pc"

PlayRecToken = "pr"

ManageAudToken = "ma"

OpCompleteToken = "oc"

OpFailedToken = "of"

VariableSegToken = "vb"

AnParmToken = "an"

IpParmToken = "ip"

RpParmToken = "rp"

NdParmToken = "nd"

NsParmToken = "ns"

FaParmToken = "fa"

SaParmToken = "sa"

OffParmToken = "off"

NiParmToken = "ni"

ItParmToken = "it"

IvParmToken = "iv"

DuParmToken = "du"

SpParmToken = "sp"

VlParmToken = "vl"

CbParmToken = "cb"

DmParmToken = "dm"

FdtParmToken = "fdt"

IdtParmToken = "idt"

EdtParmToken = "edt"

PrtParmToken = "prt"

PstParmToken = "pst"

RltParmToken = "rlt"

RlParmToken = "rl"

RskParmToken = "rsk"

RikParmToken = "rik"

RtkParmToken = "rtk"

RpaParmToken = "rpa"

DpaParmToken = "dpa"

OaParmToken = "oa"

RaParmToken = "ra"

ApParmToken = "ap"

DcParmToken = "dc"

NaParmToken = "na"

RcParmToken = "rc"

RidParmToken = "rid"

DateVarToken = "dat"

DigitsVarToken = "dig"

DurationVarToken = "dur"

DayYrMonthToken = "dym"

MonthDayYrToken = "mdy"

MoneyVarToken = "mny"

MonthVarToken = "mth"

NumberVarToken = "num"

SilenceVarToken = "sil"

StringVarToken = "str"

TimeVarToken = "tme"

GenericDigitsToken = "gen"

NorthAmericanDnSToken = "ndn"

CardinalNumberToken = "crd"

OrdinalNumberToken = "ord"

TwelveHourFormatToken = "t12"

TwentyFourHourFormatToken = "t24"

WeekdayVarToken = "wkd"

NullStrToken = "null"

BOOLSTR = "true" / "false"

NAMECHAR = ALPHA / DIGIT / "_" / "-"

NAME = 1*64(NAMECHAR)

NUMBER = DIGIT *31(DIGIT)

SIGNEDINT = ("+" / "-") DIGIT *31(DIGIT)

OPTSIGNEDINT = ["+" / "-"] DIGIT *31(DIGIT)

MINUSONE = "-1"

EQUALS = "="

COMMA = ","

LSQUARE = "["

RSQUARE = "]"

LANGLE = "<"

RANGLE = ">"

LPAREN = "("

RPAREN = ")"

SLASH = "/"

WSP = SP / HTAB

Appendix I

Call flow for network announcement

This appendix provides an example call flow (in Figure I.1) (in where a caller (MTAo) invokes the "Last Number Redial" feature to determine the phone number of the dialling party (MTAt). An Audio Server is used to play an announcement to the caller containing the previous caller's number and to present the option to the caller for completing a return call to MTAt. It should be noted that this call flow, although a valid one, is merely an example that may or may not be used in practice.


[image: image11.wmf]J.175_F10

{3a} 200 OK + SDP profile

{2} NTFY (offhook, digits)

{1} RQNT

{1a} 200 OK

MTA

o

AN

CMS

MTA

t

Time passes.  Eventually phone goes

offhook and user dials digits

{3} 200 OK +

 CRCX(inactive)

A

{6} CRCX + SDP profile

{6a} 200 OK + SDP profile

{9} MDCX (snrcresv) + SDP profile

{8a} GSAck

{9a} 100 pending

{4} SvcInst(LCR)

{5} call setup (LCR, #) + SDP profile

{7} SDP profile

{4a} Ack

{8} Gate

-

Set

{10} DSA

-

REQ

{9b} 200 OK

{9c} 000

{12} progress call

{14} RQNT(play, collect)

{14a} 200 OK

{11} Backbone provisioning

{15} RTP announcement

{13a} GOAck

{10a} DSA

-

RSP

{10b} DSA

-

ACK

{13} Gate

-

Open

{16} RTP in-

band "1"

{17} NTFY ("1")

{18} Reroute to #

{20} MDCX (inactive)

{21a} GSAck

{19a} 250 + stats

{20a} 200 OK

{22a} 100 pending + SDP 

{19} 200 OK +

DLCX

{21} Gate

-

Se

t

{22} CRCX(ring) +

SDP profile of MTA

o

{22c} 200 OK + SDP profile

{24a} 200 OK

{24} MDCX (rb)+ SDP profile of MTA

t

{23} DSA

-

REQ

{23a} DSA

-

RSP

{23b} DSA

-

ACK

Call state is that of an on

-

net to on

-

net call that is ringing. Call flow continues from this state as a normal on

-

net to on

-

net call.

MPC

MP

RKS


Figure I.1/J.175 ( On-net to on-net last number redial call flow

	Flow
	Flow description

	1 
<NCS>
	CMS sends MTAo a NotificationRequest instructing MTA to look for an off-hook event, and to report it. 

RQNT 1201 aaln/1@ec-1.mso.net MGCP 1.0 NCS 1.0

N: ca@ca1.mso.net:5678

X: 0123456789AB

R: hd(A, E(R(hu, [0-9# *T](D)),S(dl)))

D: (0T | 00T | 303[2-9]xxxxxx | 720[2-9]xxxxxx | 1[2-9]xxxxxxxxx | [3469]11 | 

 0[2-9]xxxxxxxxx | 01[2-9]xxxxxxxxxxxxxxT | 011xxxxxxxxxxxxxxxT)

	1a
<NCS>
	MTA sends CMS an ACK in response to the command, repeating in the response the transaction id that the Call Agent attached to the query and providing a return code indicating success:

200 1201 OK

	2 
<NCS>
	MTA sends CMS a Notification message indicating that an off-hook was observed and that the user requested the phone number of the Last Call Received (LCR).

NTFY 2001 aaln/1@ec-1.mso.net MGCP 1.0 NCS 1.0

N: ca@ca1.mso.net:5678

X: 0123456789AB

O: *,6,9

	3
<NCS>
	CMS sends MTA an acknowledgement of the notification. Piggybacked on the acknowledgment, the CMS sends MTAo a Create connection message. The connection is created in inactive mode. Packetization parameters are passed in the CRCX message.

200 2001 OK 

.

CRCX 1202 aaln/1@ec-1.mso.net MGCP 1.0 NCS 1.0

C: A3C47F21456789F0

L: p:10, a:PCMU, sc-rtp: 00/51; 62/51, sc-rtcp: 02/03; 01/03 sc-st: base64:

 pV6BIIHWt+0gDkpgnuxgTfROxYAemhYJTHWgHNt1crTtEUKFatJfSdEFVQuueo==

M: inactive

N: ca@ca1.mso.net:5678

X: 0123456789AC

R: hu

	4
<Event Messages>
	CMS creates the BillingCorrelationID for this transaction.

CMS sends RKS a Svcinst(LCR) Message.

RADIUS Message Header:

<Code = Accounting-Request(1 octet, value = 4)>

<Identifier (1 octet, value = 10)>

<Length (2 octets, min value = 20, max value = 4096)>

<Authenticator (16 octets, value = 0)>

IPCablecom Event Message Header VSA:

<Type = vendor specific (1 octet, value = 26)>

<Length (1 octet, value = ???)>

<vendor-ID = CableLabs (4 octets, value = 4491)>

<Vendor Attribute Type = Event Message Header (1 octet, value = 1)>

<Vendor Attribute Length (1 octet, value = 56)>

<Vendor Attribute Value =

<Version ID = IPCablecom 1.0 (2 octets, value = 1)>

<Billing Correlation ID (16 octets, value = TTTTXXXXXCMSCCCC)> 

<Event Message Type = Call_Signaling_Start (2 octets, value = 1)>

<Element Type = CMS (2 octets, value = 1)>

<Element ID (8 octets, value = xxxxxCMS)>

<Sequence ID (4 octets, value = AA05)>

<Event Message Time and Date (17 octets, value = yyyymmddhhmmss.mm)>

<Message Status = no known errors, message from trusted element (4 octets, value = ????)>

<Message Priority = user-defined (1 octet, value = any)>

<Attribute Count (2 octets, value = 4)>

<Event Object = reserved (1 octet, value = 0)>

>

	3a
<NCS>
	MTA sends CMS an acknowledgement of the CRCX, adding its own SDP profile.

200 1202 OK

I: FDE234C8

v=0

o=- 25678 753849 IN IP4 128.96.41.1

s=- 

c=IN IP4 128.96.41.1

t=0 0

m=audio 3456 RTP/AVP 0

a=X-pc-csuites-rtp: 62/51

a=X-pc-csuites-rtcp: 02/03 01/03

a=X-pc-spi-rtcp: A7843B2

a=X-pc-secret: base64: pV6BIIHWt+0gDkpgnuxgTfROxYAemhYJTHWgHNt1crTtEUKFatJfSdEFVQuueo==

	4a
<EM>
	RKS sends CMS a RADIUS ACK in response to Service Instance message – Svcinst(LCR). 

RADIUS Message Header:

<Code = Accounting-Response (1 octet, value = 5)>

<Identifier (1 octet, value = 10)>

<Length (2 octets, min value = 20, max value = 4096)>

<Authenticator (16 octets, value = 0)>

	5
<proprietary>
	CMS sends MPC all call setup information (LCR, #) including MTAo's SDP profile. [proprietary]

	6
<ASP>
	MPC sends MP a CreateConnection request in send-receive mode.

CRCX 5050 ds/12/1@ec-2.mso.net MGCP 1.0 NCS 1.0

N:ca@ca2.mso.net:5678

C: A3C47F21456789F0

L: p:10, a:PCMU, dg-gi: 1273 sc-rtp: 62/51, sc-rtcp: 02/03; 01/03 sc-st: base64:

 pV6BIIHWt+0gDkpgnuxgTfROxYAemhYJTHWgHNt1crTtEUKFatJfSdEFVQuueo==

M: sendrcv

X: 0123456789B0

R: hd

v=0

o=- 25678 753849 IN IP4 128.96.41.1

s=- 

c=IN IP4 128.96.41.1

t=0 0

m=audio 3456 RTP/AVP 0

a=X-pc-csuites-rtp: 62/51

a=X-pc-csuites-rtcp: 02/03 01/03

a=X-pc-spi-rtcp: A7843B2

a=X-pc-secret: base64: pV6BIIHWt+0gDkpgnuxgTfROxYAemhYJTHWgHNt1crTtEUKFatJfSdEFVQuueo==

	6a
<ASP>
	MP sends MPC an acknowledgement of receipt of Create Connection message. 

200 5050 OK

K:

I: 32F345E2

DQ-RI:D32B8593

v=0

o=- 25678 753849 IN IP4 128.96.41.1

s=- 

c=IN IP4 128.96.63.25

t=0 0

m=audio 1296 RTP/AVP 0

a=X-pc-csuites-rtp: 62/51

a=X-pc-csuites-rtcp: 02/03

a=X-pc-spi-rtcp: 453A78F1

a=X-pc-secret: base64: pV6BIIHWt+0gDkpgnuxgTfROxYAemhYJTHWgHNt1crTtEUKFatJfSdEFVQuueo==

	7
<proprietary>
	MPC sends CMS MP's SDP profile. [proprietary]

	8
<DQoS>
	CMS sends AN a Gate-Set message including a local ID for use with gate coordination. 

Transaction ID – 3177

Subscriber - MTA

Remote Gate Info –

 CMS address – 128.96.22.15

 CMS Port - 2562

 Authentication Algorithm=0x64

 Security Key=FourScoreAndSevenYearsAgo

 Remote Gate ID – 8096

GateSpec

Direction upstream

Protocol UDP

SourceAddress 129.96.41.1 (MTA-o)

DestinationAddress ???.???.???.??? (MG) 

SourcePort 0

Destination Port 6540

b 120

r 12000

p 12000

m 120

M 120

R 12000

S 0

GateSpec

Direction downstream

Protocol UDP

SourceAddress ???.???.???.??? (MG)

DestinationAddress 129.96.41.1 (MTA-o) 

SourcePort 0

Destination Port 3456

b 120

r 12000

p 12000

m 120

M 120

R 12000

S 0

Flag = Auto commit

Billing Info – 

 Billing Correlation ID – TTTTXXXXXCMSCCCC

 RKS_Primary - 128.96.60.110, 5000

 RKS_Secondary - 128.96.60.210, 5001

 Real_time_Flag - 0 (false)

	8a
<DQoS>
	AN sends CMS an acknowledgment of the GateSet

Transaction ID – 3177

Subscriber - MTA

Gate ID - 37125

Activity Count - 2

	9
<NCS>
	CMS sends MTAo an MDCX message. This message indicates that the MTA should go into send-receive mode. This message also contains the session description of the Media Player.

MDCX 1203 aaln/1@ec-1.mso.net MGCP 1.0 NCS 1.0

N:ca@ca1.mso.net:5678

C: A3C47F21456789F0

I: FDE234C8

M: sendrecv

X: 0123456789AE

R: hu

L: dq-qi:37125

v=0

o=- 25678 753849 IN IP4 128.96.41.1

s=- 

c=IN IP4 128.96.63.25

t=0 0

m=audio 1296 RTP/AVP 0

a=X-pc-csuites-rtp: 62/51

a=X-pc-csuites-rtcp: 02/03

a=X-pc-spi-rtcp: 453A78F1

a=X-pc-secret: base64: pV6BIIHWt+0gDkpgnuxgTfROxYAemhYJTHWgHNt1crTtEUKFatJfSdEFVQuueo==

	9a
<NCS>
	MTAo sends CMS an acknowledgement of the MDCX message.

100 1203 PENDING

	10
<J.112>
	MTAo sends AN a DSA request asking for bandwidth commitment in the access network.

DSAREQ 

TransactionID 1

Upstream Service Flow

Service Flow Reference 1

QoSParameterSetType Admitted(2)

TimeoutAdmitted 200

ServiceFlowScheduling UGS(6)

NominalGrantInterval 10ms

ToleratedGrantJitter 2ms

GrantsPerInterval 1

UnsolicitedGrantSize 111

AuthBlock 37125

DownStreamServiceFlow

Service Flow Reference 2

QoSParameterSetType Admitted(2)

TimeoutAdmitted 200

TrafficePriority 5

MaximumSustaninedRate 12,000

AuthBlock 37125

UpstreamPacketClassification

ServiceFlowReference 1

PacketClassifierReference 1

ClassifierPriority 150

ClassifierActivationState Inactive (0)

IPSourceAddress 128.96.41.1 (MTA)

IPSourcePort 3456

IPDestinationAddress ???.???.???.??? (MG)

IPDestinationPort 6540

IPProtocol UDP(17)

DownstreamPacketClassification

ServiceFlowReference 2

PacketClassifierReference 2

ClassifierPriority 150

ClassifierActivationState Inactive (0)

IPSourceAddress ???.???.???.??? (MG)

IPDestinationAddress 128.96.41.1 (MTA)

IPDestinationPort 3456

IPProtocol UDP(17)

	10a
<J.112>
	AN sends MTAo a DSA response indicating that the DSA request has been granted.

DSARSP 

TransactionID 1

ConfirmationCode Success(0)

Upstream Service Flow

ServiceFlowReference 1

ServiceFlowID 1001

QoSParameterSetType Admitted(2)

TimeoutAdmitted 200

ServiceFlowScheduling UGS(6)

NominalGrantInterval 10ms

ToleratedGrantJitter 2ms

GrantsPerInterval 1

UnsolicitedGrantSize 111

AuthBlock 31001

DownStreamServiceFlow

ServiceFlowReference 2

ServiceFlowID 2001

QoSParameterSetType Admitted+Active(6)

TimeoutAdmitted 200

TrafficePriority 5

MaximumSustaninedRate 12,000

AuthBlock 32001

UpstreamPacketClassification

ServiceFlowReference 1

PacketClassifierReference 1

ClassifierID 3001

ClassifierPriority 150

CalssifierActivationState Inactive (0)

IPSourceAddress 128.96.41.1 (MTA)

IPSourcePort 3456

IPDestinationAddress 128.96.63.25 (MG)

IPDestinationPort 1296

IPProtocol UDP(17)

DownstreamPacketClassification

ServiceFlowReference 2

PacketClassifierReference 2

ClassifierID 3002

ClassifierPriority 150

ClassifierActivationState Active (1)

IPSourceAddress 128.96.63.25 (MG)

IPDestinationAddress 128.96.41.1 (MTA)

IPDestinationPort 3456

IPProtocol UDP(17)

	10b
<J.112>
	MTAo sends AN an acknowledgement of the DSARSP.

DSA-ACK

TransactionID 1

ConfirmationCode Success(0)

	11 
<proprietary>
	Any backbone provisioning that is required is performed

	9b
<NCS>
	MTA sends CMS a confirmation of transaction complete for MDCX.

200 1203 OK

K:

	9c
<NCS>
	CMS sends MTA an acknowledgement of the completion of the MDCX transaction.

000 1203

	12 
<proprietary>
	CMS notifies the MPC to progress the call [proprietary]

	13
<D-QoS>
	CMS sends a GATE-OPEN message to AN

GateOpen

TransactionID – 81

Gate-ID - 37125

	13a
<D-QoS>
	AN responds to GateOpen message

GateOpenAck

TransactionID - 81

	14
<ASP>
	MPC sends MP a RQNT message to play the appropriate announcement and prompt for digit collection.

RQNT 5051 aaln/1@ec-1.mso.net MGCP 1.0 NCS 1.0

N: ca@ca1.mso.net:5678

X: 0123456789AB

R: oc, of

S: AAU/pc(ip=file://12345<5145551234>,file://34548 dm=x)

	14a
<ASP>
	MP acknowledges receipt of RQNT from MPC

200 5051 OK

	15
<ASP>
	MP plays announcement to MTAo via RTP media stream

	16
<ASP>
	In response to callers touch-tone, MTAo sends MP a DTMF "1" via in-band signalling

	17
<ASP>
	MP sends MPC a Notification message indicating that a DTMF "1" was received.

NTFY 7070 aaln/1@ec-1.mso.net MGCP 1.0 NCS 1.0

N: ca@ca1.mso.net:5678

X: 0123456789AB

O: oc(dc=1 na=1)

	18
<proprietary>
	MPC notifies CMS to re-route the call to the LCR #

	19
<ASP>
	MPC sends MP an acknowledgement of the NTFY and includes a piggybacked delete connection message.

200 7070 OK

.

DLCX 5052 aaln/1@ec-2.mso.net MGCP 1.0 NCS 1.0

C: A3C47F21456789F0

I: 32F345E2

	19a
<ASP>
	MP sends MPC an acknowledgement of the DLCX and includes the call statistics collected by the MP.

250 5052 OK

P: PS=1245, OS=62345, PR=780, OR=45123, PL=10, JI=27, LA=48

	20
<NCS>
	CMS sends MTAo an MDCX message de-activating the connection.

MDCX 1204 aaln/1@ec-1.mso.net MGCP 1.0 NCS 1.0

N:ca@ca1.mso.net:5678

C: A3C47F21456789F0

I: FDE234C8

M: inactive

X: 0123456789AF

R: hu

	20a
<NCS>
	MTAo sends CMS an acknowledgement of the MDCX message.

200 1204 OK

	21
<DQoS>
	CMS sends AN a Gate-Set message including the local ID for use with gate coordination. 

Transaction ID – 3177

Subscriber - MTA

Remote Gate Info –

 CMS address – 128.96.22.15

 CMS Port - 2562

 Authentication Algorithm=0x64

 Security Key=FourScoreAndSevenYearsAgo

 Remote Gate ID – 8096

GateSpec

Direction upstream

Protocol UDP

SourceAddress 129.96.41.1 (MTA-o)

DestinationAddress ???.???.???.??? (MG) 

SourcePort 0

Destination Port 6540

b 120

r 12000

p 12000

m 120

M 120

R 12000

S 0

GateSpec

Direction downstream

Protocol UDP

SourceAddress ???.???.???.??? (MG)

DestinationAddress 129.96.41.1 (MTA-o) 

SourcePort 0

Destination Port 3456

b 120

r 12000

p 12000

m 120

M 120

R 12000

S 0

Flag = Auto commit

Billing Info – 

 Billing Correlation ID – TTTTXXXXXCMSCCCC

 RKS_Primary - 128.96.60.110, 5000

 RKS_Secondary - 128.96.60.210, 5001

 Real_time_Flag - 0 (false)

	21a
<DQoS>
	AN sends CMS an acknowledgment of the GateSet

Transaction ID – 3177

Subscriber - MTA

Gate ID - 37125

Activity Count - 2

	22
<NCS>
	CMS sends MTAt a create connection message asking MTAt to ring the phone. CRCX Includes the SDP profile of MTAo.

CRCX 1301 aaln/1@ec-2.mso.net MGCP 1.0 NCS 1.0

C: A3C47F21456789F0

L: p:10, a:PCMU, sc-rtp: 00/51; 62/51, sc-rtcp: 02/03; 01/03 sc-st: base64:

 pV6BIIHWt+0gDkpgnuxgTfROxYAemhYJTHWgHNt1crTtEUKFatJfSdEFVQuueo==

M: inactive

N: ca@ca1.mso.net:5678

X: 0123456789AC

R: hu

S: rg

v=0

o=- 25678 753849 IN IP4 128.96.41.1

s=- 

c=IN IP4 128.96.41.1

t=0 0

m=audio 3456 RTP/AVP 0

a=X-pc-csuites-rtp: 62/51

a=X-pc-csuites-rtcp: 02/03

a=X-pc-spi-rtcp: A7843B2

a=X-pc-secret: base64: pV6BIIHWt+0gDkpgnuxgTfROxYAemhYJTHWgHNt1crTtEUKFatJfSdEFVQuueo==

	22a
<NCS>
	MTAt sends CMS a confirmation of transaction complete for CRCX and its SDP profile.

100 1301 pending

v=0

o=- 25678 753849 IN IP4 128.96.41.1

s=- 

c=IN IP4 128.96.10.10

t=0 0

m=audio 6789 RTP/AVP 0

a=X-pc-csuites-rtp: 62/51

a=X-pc-csuites-rtcp: 02/03 01/03

a=X-pc-spi-rtcp: A7843B2

a=X-pc-secret: base64: pV6BIIHWt+0gDkpgnuxgTfROxYAemhYJTHWgHNt1crTtEUKFatJfSdEFVQuueo==

	23
<J.112>
	MTAt sends AN a DSA request asking for bandwidth commitment in the access network.

DSAREQ 

TransactionID 1

Upstream Service Flow

Service Flow Reference 1

QoSParameterSetType Admitted(2)

TimeoutAdmitted 200

ServiceFlowScheduling UGS(6)

NominalGrantInterval 10ms

ToleratedGrantJitter 2ms

GrantsPerInterval 1

UnsolicitedGrantSize 111

AuthBlock 37125

DownStreamServiceFlow

Service Flow Reference 2

QoSParameterSetType Admitted(2)

TimeoutAdmitted 200

TrafficePriority 5

MaximumSustaninedRate 12,000

AuthBlock 37125

UpstreamPacketClassification

ServiceFlowReference 1

PacketClassifierReference 1

ClassifierPriority 150

CalssifierActivationState Inactive (0)

IPSourceAddress 128.96.41.1 (MTA)

IPSourcePort 3456

IPDestinationAddress ???.???.???.??? (MG)

IPDestinationPort 6540

IPProtocol UDP(17)

DownstreamPacketClassification

ServiceFlowReference 2

PacketClassifierReference 2

ClassifierPriority 150

ClassifierActivationState Inactive (0)

IPSourceAddress ???.???.???.??? (MG)

IPDestinationAddress 128.96.41.1 (MTA)

IPDestinationPort 3456

IPProtocol UDP(17)

	23a
<J.112>
	AN sends MTAt a DSA response indicating that the DSA request has been granted.

DSARSP 

TransactionID 1

ConfirmationCode Success(0)

Upstream Service Flow

ServiceFlowReference 1

ServiceFlowID 1001

QoSParameterSetType Admitted(2)

TimeoutAdmitted 200

ServiceFlowScheduling UGS(6)

NominalGrantInterval 10ms

ToleratedGrantJitter 2ms

GrantsPerInterval 1

UnsolicitedGrantSize 111

AuthBlock 31001

DownStreamServiceFlow

ServiceFlowReference 2

ServiceFlowID 2001

QoSParameterSetType Admitted+Active(6)

TimeoutAdmitted 200

TrafficePriority 5

MaximumSustaninedRate 12,000

AuthBlock 32001

UpstreamPacketClassification

ServiceFlowReference 1

PacketClassifierReference 1

ClassifierID 3001

ClassifierPriority 150

CalssifierActivationState Inactive (0)

IPSourceAddress 128.96.41.1 (MTA)

IPSourcePort 3456

IPDestinationAddress 128.96.63.25 (MG)

IPDestinationPort 1296

IPProtocol UDP(17)

DownstreamPacketClassification

ServiceFlowReference 2

PacketClassifierReference 2

ClassifierID 3002

ClassifierPriority 150

ClassifierActivationState Active (1)

IPSourceAddress 128.96.63.25 (MG)

IPDestinationAddress 128.96.41.1 (MTA)

IPDestinationPort 3456

IPProtocol UDP(17)

	23b
<J.112>
	MTAt sends AN an acknowledgement of the DSARSP.

DSA-ACK

TransactionID 1

ConfirmationCode Success(0)

	22c
<NCS>
	MTAt sends CMS a 200 OK and its SDP profile.

200 1301 OK

v=0

c=IN IP4 128.96.63.25

m=audio 1296 RTP/AVP 0

a=X-pc-csuites-rtp: 62/51

a=X-pc-csuites-rtcp: 02/03

	24
<NCS>
	CMS sends MTAo an MDCX message indicating ringback and the SDP profile of MTAt.

MDCX 1205 aaln/1@ec-1.mso.net MGCP 1.0 NCS 1.0

N:ca@ca1.mso.net:5678

C: A3C47F21456789F0

I: FDE234C8

M: sendrecv

X: 0123456789AF

R: hu

S: rb

v=0

o=- 25678 753849 IN IP4 128.96.41.1

s=- 

c=IN IP4 128.96.10.10

t=0 0

m=audio 6789 RTP/AVP 0

a=X-pc-csuites-rtp: 62/51

a=X-pc-csuites-rtcp: 02/03 01/03

a=X-pc-spi-rtcp: A7843B2

a=X-pc-secret: base64: pV6BIIHWt+0gDkpgnuxgTfROxYAemhYJTHWgHNt1crTtEUKFatJfSdEFVQuueo==

	24a
<NCS>
	MTAo sends CMS an acknowledgement of the MDCX transaction.

200 1205 OK

	Call State is a ringing on-net to on-net call between MTAo and MTAt. The call proceeds as a standard on-net to on-net IPCablecom call.


Appendix II

Call flow for MTA-stored announcement

This appendix provides an example call flow (in Figure II.1) where a User-1 attempts to call User-2. Due to facility problems on the terminating side, the call can not be completed. The MTA associated with User-1 is instructed to play a local announcement. It should be noted, that this call flow, although a valid one, is merely an example that may or may not be used in practice.


[image: image12.wmf]J.175_F11

MTA

o

AN

o

AN

t

MTA

t

CMS

o

RKS

{4a} 200 OK

{2a} 200 OK

{5} SigStart

{3} Routing DIP

{6} NTFY (onhook)

{7} SigStart

{7a} SSAck

{3a} DIP Resp

{2} NTFY (offhook, digits)

{4} NTFY (onhook, ann)

{6a} 200 OK + RQNT(offhook)

{5a} SSAck

Time passes.  Eventually phone goes

offhook, dial tone is persented and

user dials digits

Announcement is played to the user.

Eventually the phone goes onhook

{1} RQNT (offhook)

{1a} 200 OK


Figure II.1/J.175 – MTA announcement basic call flow

II.1
Call flow details

	Flow
	Flow description
	Depends upon these completion of these flows:
	Triggers start of these flows:

	Initialization

	1 
<NCS>
	CMS sends MTAo a NotificationRequest instructing MTAo to look for an off-hook event, and to report it. 

RQNT 1201 aaln/1@ec-1.whatever.net MGCP 1.0 NCS 1.X

N: ca@ca1.whatever.net:5678

X: 0123456789AB

R: hd(E (R([0-9#*T](D), hu(N)), S(dl), ;))

D: (0T | 00T | [2-9]xxxxxx | 
1[2-9]xxxxxxxxx | 011xx.T)
	
	1a

	1a 
<NCS>
	MTAo sends CMS an ACK in response to the command, repeating in the response the transaction id that the Call Agent attached to the query and providing a return code indicating success:

200 1201 OK
	1
	

	Service Request

	2 
<NCS>
	MTAo sends CMS a Notification message indicating that an off-hook was observed.

NTFY 2001 aaln/1@ec-1.whatever.net MGCP 1.0 NCS 1.X

N: ca@ca1.whatever.net:5678

X: 0123456789AB

O: hd, 3, 0, 3, 5, 5, 5, 1, 2, 1, 2
	1, user stimulus
	2a, 3, 4, 5

	2a 
<NCS>
	CMS sends MTAo an acknowledgement of the notification.

200 2001 OK
	2
	

	3 
<??>
	CMS contacts the routing database requesting a mapping of the dialled number to a routable destination in the network.
	2
	3a

	3a 
<??>
	The routing database server responds to the CMS with the routing information.
	3
	4, 8

	4 
<NCS>
	CMS sends MTAo a notification request message. The connection is created in inactive mode. Packetization parameters are passed in the CRCX message.

RQNT 1202 aaln/1@ec-1.whatever.net MGCP 1.0 NCS 1.X

N: ca@ca1.whatever.net:5678

X: 0123456789AC

R: hu, oc, of

S: A/ann(file://audio/23945)
	2, 3a
	4a, 5

	4a 
<NCS>
	MTAo sends CMS an acknowledgement of the RQNT, adding its own SDP profile.

200 1202 OK
	4
	6, 8

	Announcement is being played

	5 
<EM>
	CMS creates the BillingCorrelationID for this transaction.

CMS sends RKS a Call_Signaling_Start Event Message.

The message contents include: Event_Message_Header(Version_ID, BillingCorrelationID, "Call_Signaling_Start Event Message", Element_Type, Element_ID, Element_Seq_Num, Message_Timestamp, Message_Status, Message_Priority, Attribute_Count, Event_Object

),

Event_Time, MTA_Port_ID, Calling_Party_Number, Called_Party_Number

The message format is:

<insert example coded message>
	2
	5a

	5a 
<EM>
	RKS sends CMS a RADIUS ACK in response to Call_Signaling_Start

ACK

The message format is:

<insert example coded message>
	5
	

	User is listening to the announcement and hangs up

	11
<NCS>
	MTAo sends CMS a notification that the attached device has gone on-hook.

NTFY 2002 aaln/1@ec-2.whatever.net MGCP 1.0 NCS 1.X
X: 0123456789AF

O: hu
	
	12, 13, 14

	12
<NCS>
	CMS sends MTAo an acknowledgement of the NTFY and includes a piggybacked delete connection message.

200 2002 OK

.

RQNT 1207 aaln/1@ec-2.whatever.net MGCP 1.0 NCS 1.X
X: 0123456789B2

N: ca@ca1.whatever.net:5678

R: hd (E (dl:hu, D/[0-9# *T] (D) ;))

D: (0T | 00T | [2-9]xxxxxx | 1[2-9]xxxxxxxxx | 011xx.T)
	11
	12a, 15

	12a
<NCS>
	MTAo sends CMS an acknowledgement of the DLCX and includes the call statistics collected by the MTA.

250 12?? OK
	12
	22, 25

	14
<EM>
	CMS sends RKS a Media_Connection_Stop Event Message.

The message contents include: Event_Message_Header(Version_ID, BillingCorrelationID, "Media_Connection_Stop Event Message", Element_Type, Element_ID, Element_Seq_Num, Message_Timestamp, Message_Status, Message_Priority, Attribute_Count, Event_Object

),

Event_Time, Call_Termination_Cause 

The message format is:

<insert example coded message>
	11
	14a

	14a
<EM>
	RKS sends CMS a RADIUS ACK in response to Media_Connection_Stop

ACK

The message format is:

<insert example coded message>
	14
	


Appendix III

Bibliography
(
BRADNER (S.): Key words for use in RFCs to Indicate Requirement Levels, IETF RFC 2119, BCP 14, March 1997. www.ietf.org
(
ARANGO (M.), DUGAN (A.), ELLIOTT (I.), HUITEMA (C.), PICKETT (S.): Media Gateway Control Protocol (MGCP) Version 1.0, IETF RFC 2705, October 1999. www.ietf.org 

(
CROMWELL (D.): Proposal for an MGCP Advanced Audio Package, IETF RFC 2897, August 2000.

(
CROMWELL (D.), DURLING (M.): Suggested Requirements For Control Of A IVR Function, IETF Internet Draft, April 1999.

(
BERNERS-LEE (T.), FIELDING (R.) MASINTER (L.): Uniform Resource Identifiers (URI): Generic Syntax, IETF RFC 2396, August 1998. www.ietf.org 

(
CROCKER (D.), OVERELL (P.), Augmented BNF for Syntax Specifications: ABNF, IETF RFC 2234, November 1997. www.ietf.org 

	SERIES OF ITU-T RECOMMENDATIONS

	Series A
	Organization of the work of ITU-T

	Series B
	Means of expression: definitions, symbols, classification

	Series C
	General telecommunication statistics

	Series D
	General tariff principles

	Series E
	Overall network operation, telephone service, service operation and human factors

	Series F
	Non-telephone telecommunication services

	Series G
	Transmission systems and media, digital systems and networks

	Series H
	Audiovisual and multimedia systems

	Series I
	Integrated services digital network

	Series J
	Cable networks and transmission of television, sound programme and other multimedia signals

	Series K
	Protection against interference

	Series L
	Construction, installation and protection of cables and other elements of outside plant

	Series M
	TMN and network maintenance: international transmission systems, telephone circuits, telegraphy, facsimile and leased circuits

	Series N
	Maintenance: international sound programme and television transmission circuits

	Series O
	Specifications of measuring equipment

	Series P
	Telephone transmission quality, telephone installations, local line networks

	Series Q
	Switching and signalling

	Series R
	Telegraph transmission

	Series S
	Telegraph services terminal equipment

	Series T
	Terminals for telematic services

	Series U
	Telegraph switching

	Series V
	Data communication over the telephone network

	Series X
	Data networks and open system communications

	Series Y
	Global information infrastructure and Internet protocol aspects

	Series Z
	Languages and general software aspects for telecommunication systems

	
	








� 	Announcements for off-net to on-net calls will usually be handled by the PSTN as a result of SS7 clearing messages. However when appropriate, they also may be played from the IPCablecom Media Gateway (MG).








_1101196216.unknown

_1101196327.unknown

_1101196447.unknown

_1101197192.unknown

_1101539135.unknown

_1101196487.unknown

_1101196407.unknown

_1101196281.unknown

_1101196078.unknown

_1101196130.unknown

_1101195950.unknown

