

��

INTERNATIONAL  TELECOMMUNICATION  UNION��





ITU-T�Series J��TELECOMMUNICATION�STANDARDIZATION  SECTOR�OF  ITU�Supplement 1

(11/98)   ��









SERIES J: TRANSMISSION OF TELEVISION, SOUND PROGRAMME AND OTHER MULTIMEDIA SIGNALS





Example of linking options between annexes of ITU�T Recommendation J.112 and annexes of ITU�T Recommendation J.83



ITU-T  J-series  Recommendations  –  Supplement  1

(Previously  CCITT  Recommendations)



�

ITU-T  J-SERIES  RECOMMENDATIONS

TRANSMISSION OF TELEVISION, SOUND PROGRAMME AND OTHER MULTIMEDIA SIGNALS



���General Recommendations�J.1–J.9��General specifications for analogue sound-programme transmission�J.10–J.19��Performance characteristics of analogue sound-programme circuits�J.20–J.29��Equipment and lines used for analogue sound-programme circuits�J.30–J.39��Digital encoders for analogue sound-programme signals�J.40–J.49��Digital transmission of sound-programme signals�J.50–J.59��Circuits for analogue television transmission�J.60–J.69��Analogue television transmission over metallic lines and interconnection with radio-relay links�J.70–J.79��Digital transmission of television signals�J.80–J.89��Ancillary digital services for television transmission�J.90–J.99��Operational requirements and methods for television transmission�J.100–J.109��Interactive systems for digital television distribution�J.110–J.129��Transport of MPEG-2 signals on packetised networks�J.130–J.139��Measurement of the quality of service�J.140–J.149��Digital television distribution through local subscriber networks�J.150–J.159�����For further details, please refer to ITU-T List of Recommendations.

�SUPPLEMENT  1  TO  ITU-T  J-SERIES  RECOMMENDATIONS

EXAMPLE  OF  LINKING  OPTIONS  BETWEEN  ANNEXES�OF  ITU-T  RECOMMENDATION  J.112  AND  ANNEXES�OF  ITU-T  RECOMMENDATION  J.83





Source

Supplement 1 to ITU-T J-series Recommendations was prepared by ITU-T Study Group 9 (1997-2000) and was approved under the WTSC Resolution No. 5 procedure on the 19th of November 1998.























































�

FOREWORD

ITU (International Telecommunication Union) is the United Nations Specialized Agency in the field of tele�communi�cations. The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of the ITU. The ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to stan�dard�izing telecommunications on a worldwide basis.

The World Telecommunication Standardization Conference (WTSC), which meets every four years, establishes the topics for study by the ITU�T Study Groups which, in their turn, produce Recommendations on these topics.

The approval of Recommendations by the Members of the ITU�T is covered by the procedure laid down in WTSC Resolution No. 1.

In some areas of information technology which fall within ITU-T’s purview, the necessary standards are prepared on a collaborative basis with ISO and IEC.

NOTE

In this Recommendation the term recognized operating agency (ROA) includes any individual, company, corporation or governmental organization that operates a public correspondence service. The terms Administration, ROA and public correspondence are defined in the Constitution of the ITU (Geneva, 1992).

INTELLECTUAL  PROPERTY  RIGHTS

The ITU draws attention to the possibility that the practice or implementation of this Recommendation may involve the use of a claimed Intellectual Property Right. The ITU takes no position concerning the evidence, validity or applicability of claimed Intellectual Property Rights, whether asserted by ITU members or others outside of the Recommendation development process.

As of the date of approval of this Recommendation, the ITU had not received notice of intellectual property, protected by patents, which may be required to implement this Recommendation. However, implementors are cautioned that this may not represent the latest information and are therefore strongly urged to consult the TSB patent database.

ã  ITU  1999

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

�



INDEX

Page

� TOC \o "1-2" �1	Introduction		� GOTOBUTTON _Toc449497988  � PAGEREF _Toc449497988 �1��

2	Alternative global parameters for Annex B/J.112 to achieve linking		� GOTOBUTTON _Toc449497989  � PAGEREF _Toc449497989 �1��

3	Alternative parameters for B.4.3/J.112 to achieve linking (the numbering of the subclauses is the same as in Recommendation J.112):		� GOTOBUTTON _Toc449497990  � PAGEREF _Toc449497990 �1��

�



























































�Supplement 1 to J-series Recommendations

J series – Supplement 1     (11/98)

EXAMPLE  OF  LINKING  OPTIONS  BETWEEN  ANNEXES�OF  ITU�T  RECOMMENDATION  J.112  AND  ANNEXES�OF  ITU-T  RECOMMENDATION  J.83

(Geneva, 1998)

1	Introduction

Recommendation J.112 includes the statement in its clause 1 Scope, "There is therefore the possibility that, in application, the features chosen for use with the transmission system described in a particular annex of Recommen�dation J.83 are not limited to only the corresponding annex of this Recommendation." In particular, this Supplement defines an example of how to use Annex B/J.112 in conjunction with Annex A/J.83, by indicating the alternative parameters required for B.4.3/J.112 to achieve linking.

2	Alternative global parameters for Annex B/J.112 to achieve linking

In this example: in all cases, mention of Upstream Bandwidth "5 to 42 MHz" in Annex B/J.112 shall be read as "5 to 65 MHz"; and all mention of Downstream Bandwidth "95 to 857 MHz" shall be read as "100 to 860 MHz" throughout. In the case of Annex A/J.83, all mention of "6 MHz" shall be read as "8 MHz" throughout.

3	Alternative parameters for B.4.3/J.112 to achieve linking (the numbering of the subclauses is the same as in Recommendation J.112):

B.4.3	Downstream

B.4.3.1	Downstream protocol

The downstream PMD layer MUST conform to Annex A/J.83.

B.4.3.2	Scaleable interleaving to support low latency

Not required.

B.4.3.3	Downstream frequency plan

Shall meet national requirements.

B.4.3.4	CMTS output electrical

The CMTS MUST output an RF modulated signal with the following characteristics:

CMTS electrical output (Downstream)

Parameter�Value��Centre Frequency (fc)�100 to 860 MHz ± 30 kHz��Level�Adjustable over the range 50 to 61 dBmV��Modulation Type�64 QAM and 256 QAM��Symbol Rate (nominal)�	64 QAM�	256 QAM��6.952 Msym/s�6.952 Msym/s��Connector�F connector per IPS-SP-406��

�Byte/symbol mapping, differential coding and baseband filter characteristics are as in Annex A/J.83.

B.4.3.5	Downstream electrical input to CM

The CM MUST accept an RF modulated signal with the following characteristics:

Electrical input to CM

Parameter�Value��Centre Frequency�100 to 860 MHz ± 30 kHz��Level Range (one channel)�–15 dBmV to +15 dBmV��Modulation Type�64 QAM and 256 QAM��Symbol Rate (nominal)�6.952 Msym/s (64 QAM) and�6.952 Msym/s (256 QAM)��Total Input Power (40-900 MHz)�< 30 dBmV��Input (load) Impedance�75 ohms��Input Return Loss�> 6 dB (100�860 MHz)��Connector�F connector per IPS-SP-406 (common with the output)��

B.4.3.6	CM BER performance

The bit-error-rate performance of a CM MUST be as described in this subclause.

B.4.3.6.1	64 QAM

B.4.3.6.1.1	64 QAM CM BER performance

Implementation loss of the CM MUST be such that the CM achieves a post-FEC BER less than or equal to 10–8 when operating at a carrier to noise ratio (Es/No)� of 24.5 dB or greater.

B.4.3.6.1.2	64 QAM image rejection performance

Performance as described in B.4.3.6.1.1 MUST be met with analogue or digital signal at +10 dBc in any portion of the RF band other than the adjacent channels.

B.4.3.6.1.3	64 QAM adjacent channel performance

Performance as described in B.4.3.6.1.1 MUST be met with digital signal at 0 dBc in the adjacent channels.

Performance as described in B.4.3.6.1.1 MUST be met with analogue signal at +10 dBc in the adjacent channels.

Performance as described in B.4.3.6.1.1, with an additional 0.2 dB allowance, MUST be met with digital signal at +10 dBc in the adjacent channels.

�B.4.3.6.2	256 QAM

B.4.3.6.2.1	256 QAM CM BER performance

Implementation loss of the CM MUST be such that the CM achieves a post-FEC BER less than or equal to 10–8 when operating at a carrier to noise ratio (Es/No) of 31 dB or greater, with input signals greater than –6 dBmV.

B.4.3.6.2.2	256 QAM image rejection performance

Performance as described in B.4.3.6.2.1 MUST be met with analogue or digital signal at +10 dBc in any portion of the RF band other than the adjacent channels.

B.4.3.6.2.3	256 QAM adjacent channel performance

Performance as described in B.4.3.6.2.1 MUST be met with analogue or digital signal at 0 dBc in the adjacent channels.

Performance as described in B.4.3.6.2.1, with an additional 0.5 dB allowance, MUST be met with analogue signal at +10 dBc in the adjacent channels.

Performance as described in B.4.3.6.2.1, with an additional 1.0 dB allowance, MUST be met with digital signal at +10 dBc in the adjacent channels.













































�

ITU-T  RECOMMENDATIONS  SERIES��Series A�Organization of the work of the ITU-T��Series B�Means of expression: definitions, symbols, classification��Series C�General telecommunication statistics��Series D�General tariff principles��Series E�Overall network operation, telephone service, service operation and human factors��Series F�Non-telephone telecommunication services��Series G�Transmission systems and media, digital systems and networks��Series H�Audiovisual and multimedia systems��Series I�Integrated services digital network��Series J�Transmission of television, sound programme and other multimedia signals��Series K�Protection against interference��Series L�Construction, installation and protection of cables and other elements of outside plant��Series M�TMN and network maintenance: international transmission systems, telephone circuits, telegraphy, facsimile and leased circuits��Series N�Maintenance: international sound programme and television transmission circuits��Series O�Specifications of measuring equipment��Series P�Telephone transmission quality, telephone installations, local line networks��Series Q�Switching and signalling��Series R�Telegraph transmission��Series S�Telegraph services terminal equipment��Series T�Terminals for telematic services��Series U�Telegraph switching��Series V�Data communication over the telephone network��Series X�Data networks and open system communications��Series Y�Global information infrastructure��Series Z�Languages and general software aspects for telecommunication systems��



_______________











�PAGE�ii�	�styleref head_foot�J series – Supplement 1     (11/98)�



		�styleref head_foot�J series – Supplement 1     (11/98)�	�PAGE�iii�



�PAGE�2�	�styleref head_foot�J series – Supplement 1     (11/98)�



		�styleref head_foot�J series – Supplement 1     (11/98)�	�PAGE�3�





























�PAGE \# "'Page: '#'�'"  �Page: 2���A: carrier to noise ratio (Es/No)








