
	[image: image1.wmf]
	INTERNATIONAL  TELECOMMUNICATION  UNION

	
	

	ITU-T
	Q.1248.7

	TELECOMMUNICATION
STANDARDIZATION  SECTOR
OF  ITU
	(07/2001)   

	
	SERIES Q: SWITCHING AND SIGNALLING

Intelligent Network



	
	Interface Recommendation for Intelligent Network capability set 4: SCF-CUSF Interface

	
	ITU‑T  Recommendation  Q.1248.7




ITU-T Q-SERIES  RECOMMENDATIONS

SWITCHING AND SIGNALLING

	
	

	SIGNALLING IN THE INTERNATIONAL MANUAL SERVICE
	Q.1–Q.3

	INTERNATIONAL AUTOMATIC AND SEMI-AUTOMATIC WORKING
	Q.4–Q.59

	FUNCTIONS AND INFORMATION FLOWS FOR SERVICES IN THE ISDN
	Q.60–Q.99

	CLAUSES APPLICABLE TO ITU-T STANDARD SYSTEMS
	Q.100–Q.119

	SPECIFICATIONS OF SIGNALLING SYSTEMS No. 4 AND No. 5
	Q.120–Q.249

	SPECIFICATIONS OF SIGNALLING SYSTEM No. 6
	Q.250–Q.309

	SPECIFICATIONS OF SIGNALLING SYSTEM R1
	Q.310–Q.399

	SPECIFICATIONS OF SIGNALLING SYSTEM R2
	Q.400–Q.499

	DIGITAL EXCHANGES
	Q.500–Q.599

	INTERWORKING OF SIGNALLING SYSTEMS
	Q.600–Q.699

	SPECIFICATIONS OF SIGNALLING SYSTEM No. 7
	Q.700–Q.799

	Q3 INTERFACE
	Q.800–Q.849

	DIGITAL SUBSCRIBER SIGNALLING SYSTEM No. 1
	Q.850–Q.999

	PUBLIC LAND MOBILE NETWORK
	Q.1000–Q.1099

	INTERWORKING WITH SATELLITE MOBILE SYSTEMS
	Q.1100–Q.1199

	INTELLIGENT NETWORK
	Q.1200–Q.1699

	SIGNALLING REQUIREMENTS AND PROTOCOLS FOR IMT-2000
	Q.1700–Q.1799

	SPECIFICATIONS OF SIGNALLING RELATED TO BEARER INDEPENDENT CALL CONTROL (BICC)
	Q.1900–Q.1999

	BROADBAND ISDN
	Q.2000–Q.2999

	
	


For further details, please refer to the list of ITU-T Recommendations.

	ITU-T Recommendation Q.1248.7

Interface Recommendation for Intelligent Network
capability set 4: SCF-CUSF Interface



	Summary

The Q.1248.x family of Recommendations defines the Intelligent Network (IN) Application Protocol (INAP) for IN capability set 4 (IN CS-4). This family of Recommendation defines the INAP for IN CS-4 based upon IN CS‑3 ITU-T Q.1238 (2000), and the general rules for INAP provided in ITU-T Q.1208, and is consistent with the scope of IN CS-4 as defined in ITU-T Q.1241.

Within the Q.124x series of Recommendations, the ITU-T Q.1248.x family describes the protocol realizing the Q.1244 Distributed Functional Plane in a service and vendor implementation independent manner, as constrained by the capabilities of the embedded base of network technology. This provides the flexibility to allocate distributed functionality into multiple physical network configurations and to evolve IN from IN CS-4 to some future IN CS-N.

This Recommendation belongs to ITU-T Q.1248.x family of Recommendation for IN capability set 4. It covers the CUSF-SCF interface including the description of the aspects of the CUSF and SCF Functional Entities which are relevant to this interface.

This Recommendation includes an electronic attachment containing Annex A, ASN.1 definitions.


	Source

ITU‑T Recommendation Q.1248.7 was prepared by ITU‑T Study Group 11 (2001‑2004) and approved under the WTSA Resolution 1 procedure on 13 July 2001.


	Keywords

Intelligent Network (IN), Intelligent Network Application Protocol (INAP), IN Capability Set 4 (IN CS-4), Service Control Function (SCF), Call Unrelated Service Function (CUSF).




FOREWORD

The International Telecommunication Union (ITU) is the United Nations specialized agency in the field of tele​com​mu​ni​ca​tions. The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of ITU. ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Assembly (WTSA), which meets every four years, establishes the topics for study by the ITU‑T study groups which, in turn, produce Recommendations on these topics.

The approval of ITU-T Recommendations is covered by the procedure laid down in WTSA Resolution 1.

In some areas of information technology which fall within ITU-T's purview, the necessary standards are prepared on a collaborative basis with ISO and IEC.

NOTE

In this Recommendation, the expression "Administration" is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

INTELLECTUAL PROPERTY RIGHTS
ITU draws attention to the possibility that the practice or implementation of this Recommendation may involve the use of a claimed Intellectual Property Right. ITU takes no position concerning the evidence, validity or applicability of claimed Intellectual Property Rights, whether asserted by ITU members or others outside of the Recommendation development process.

As of the date of approval of this Recommendation, ITU had not received notice of intellectual property, protected by patents, which may be required to implement this Recommendation. However, implementors are cautioned that this may not represent the latest information and are therefore strongly urged to consult the TSB patent database.

(  ITU  2002

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from ITU.

CONTENTS


Page

1
Scope

1
2
References

1
3
Abbreviations and acronyms

1
4
Relationships

1
4.1
SCF-CCF/CUSF relationship

1
4.2
CUSF-SSF relationship

1
5
CCF/CUSF model

1
5.1
General

1
5.2
CCF/CUSF components

2
5.2.1
Basic Non‑Call Manager (BNCM)

2
5.2.2
IN – Non-Switching Manager (IN-NSM)

2
5.2.3
Feature Interaction Manager/Non-Call Manager (FIM/NCM)

2
5.3
Relationship of CCF/CUSF model components

2
5.3.1
BNCM relationship to IN-NSM

2
5.3.2
BNCM and IN-NSM relationship to FIM/NCM

2
5.4
Basic Call Unrelated State Model (BCUSM)

2
5.4.1
BCUSM description

4
5.4.2
BCUSM DP criteria

8
6
SCF model

9
7
FSM for CUSF

9
7.1
Relations between CUSF FSM and the SSF/CCF and maintenance functions

9
7.2
CUSF state transition diagram

11
7.2.1
State a: "Idle"

12
7.2.2
State b: "Waiting for Instructions"

13
7.2.3
State c: "Monitoring"

14
8
FSM for SCF

14
8.1
Overview

14
8.2
SCME FSM

15
8.3
CUSF-related states (SCSM-CUSF)

15
8.3.1
State N1: "Idle"

16
8.3.2
State N2: "Preparing CUSF Instructions"

16
9
FSM for USI

19
9.1
Overview

19
9.2
SCF USI FSM for CUSF

21

Page

9.3
CUSF USI FSM

21
10
Operation procedures

21
10.1
ActivationReceivedAndAuthorized procedure

22
10.1.1
General description

22
10.1.2
Parameters

22
10.1.3
Invoking entity (CUSF)

23
10.1.4
Responding entity (SCF)

23
10.2
ActivityTest procedure

24
10.2.1
General description

24
10.2.2
Parameters

24
10.2.3
Invoking entity (SCF)

24
10.2.4
Responding entity (CUSF)

24
10.3
AssociationReleaseRequested procedure

25
10.3.1
General description

25
10.3.2
Parameters

25
10.3.3
Invoking entity (CUSF)

26
10.3.4
Responding entity (SCF)

26
10.4
ComponentReceived procedure

26
10.4.1
General description

26
10.4.2
Parameters

27
10.4.3
Invoking entity (CUSF)

27
10.4.4
Responding entity (SCF)

27
10.5
ConnectAssociation procedure

28
10.5.1
General description

28
10.5.2
Parameters

28
10.5.3
Invoking entity (SCF)

28
10.5.4
Responding entity (CUSF)

29
10.6
ContinueAssociation procedure

29
10.6.1
General description

29
10.6.2
Parameters

29
10.6.3
Invoking entity (SCF)

29
10.6.4
Responding entity (CUSF)

30
10.7
EventReportBCUSM procedure

30
10.7.1
General description

30
10.7.2
Parameters

31
10.7.3
Invoking entity (CUSF)

31
10.7.4
Responding entity (SCF)

32
10.8
InitialAssociationDP procedure

32
10.8.1
General description

32

Page

10.8.2
Parameters

32
10.8.3
Invoking entity (CUSF)

33
10.8.4
Responding entity (SCF)

34
10.9
InitiateAssociation procedure

34
10.9.1
General description

34
10.9.2
Parameters

34
10.9.3
Invoking entity (SCF)

34
10.9.4
Responding entity (CUSF)

35
10.10
ReleaseAssociation procedure

35
10.10.1
General description

35
10.10.2
Parameters

35
10.10.3
Invoking entity (SCF)

35
10.10.4
Responding entity (CUSF)

35
10.11
ReportUTSI procedure

36
10.11.1
General description

36
10.11.2
Parameters

36
10.11.3
Invoking entity (CUSF)

36
10.11.4
Responding entity (SCF)

36
10.12
RequestReportBCUSMEvent procedure

37
10.12.1
General description

37
10.12.2
Parameters

37
10.12.3
Invoking entity (SCF)

37
10.12.4
Responding entity (CUSF)

38
10.13
RequestReportUTSI procedure

38
10.13.1
General description

38
10.13.2
Parameters

38
10.13.3
Invoking entity (SCF)

38
10.13.4
Responding entity (CUSF)

39
10.14
SendComponent procedure

39
10.14.1
General description

39
10.14.2
Parameters

39
10.14.3
Invoking entity (SCF)

39

10.14.4
Responding entity (CUSF)

40
10.15
SendSTUI procedure

40
10.15.1
General description

40
10.15.2
Parameters

40
10.15.3
Invoking entity (SCF)

40
10.15.4
Responding entity (CUSF)

41
10.16
Operation timers

41

Page

11
Parameters

42
11.1
Address

42
11.2
BCUSMEvents

42
11.3
BearerCapability

42
11.4
CalledPartyNumber

42
11.5
CallingPartyNumber

43
11.6
CallingPartySubaddress

43
11.7
CallUnrelatedDpSpecificCommonParameters

43
11.8
Cause

44
11.9
Component

44
11.10
ComponentCorrelationID

44
11.11
ComponentType

44
11.12
ComponentTypes

44
11.13
CUApplicationInd

44
11.14
CUDPCriteria

45
11.15
EventSpecificInformationBCUSM

45
11.16
EventTypeBCUSM

45
11.17
GenericNumbers

45
11.18
HighLayerCompatibility

45
11.19
LegID

46
11.20
Message

46
11.21
MiscCallInfo

46
11.22
MonitorDuration

46
11.23
RequestedUTSIList

47
11.24
ServiceKey

47
11.25
USIInformation

47
11.26
USIServiceIndicator

47
12
Error procedures

47
12.1
Operation-related error procedures

47
12.1.1
MissingCustomerRecord

48
12.1.2
MissingParameter

49
12.1.3
ParameterOutOfRange

50
12.1.4
SystemFailure

50
12.1.5
TaskRefused

50
12.1.6
UnexpectedComponentSequence

50
12.1.7
UnexpectedDataValue

50

Page

12.1.8
UnexpectedParameter

50
12.1.9
UnknownLegID

50
12.2
Entity-related error procedures

50
12.2.1
Expiration of TCUSF

50
13
Service assumed by TCAP

51
13.1
Normal procedures

51
13.1.1
CUSF-to-SCF messages

51
13.1.2
SCF-to-CUSF messages

52
13.2
Abnormal procedures

52
13.2.1
Dialogue handling

52
13.2.2
Component handling

53
Annex A – ASN.1 definitions (electronic attachment)

53
ITU-T Recommendation Q.1248.7

Interface Recommendation for Intelligent Network
capability set 4: SCF-CUSF Interface

1
Scope

This Recommendation belongs to the Q.1248.x family of Recommendations for IN capability set 4. It specifies the protocol on the CUSF‑SCF interface and provides a description of the aspects of the CUSF and SCF Functional Entities which are involved in the realization of this interface.

2
References

All ITU-T Recommendations and other references are identified in ITU-T Q.1248.1.

3
Abbreviations and acronyms

All abbreviations and acronyms used in this text are defined in ITU-T Q.1248.1.

4
Relationships
4.1
SCF-CCF/CUSF relationship

The SCF-CUSF relationship is used for messages between an SCF and a CUSF in the public network. This relationship provides a service, or service features, which need call-unrelated user interaction. This provides, for example, user location registration, user authentication, supplementary service activation or de-activation. Details of service drivers can be found in ITU-T Q.1241.
This relationship is used for three cases, and respective cases are exclusive for the others.

Case 1)
for sending/receiving ROSE APDUs to/from a user.

Case 2)
for sending/receiving USI informations to/from a user.

Case 3)
for relaying some informations for call unrelated association establishment from a supplementary service (e.g. CCBS) in CCF (BCUSM).

4.2
CUSF-SSF relationship

It is considered that the CUSF and the SSF may have a relationship, but the relationship is not defined in IN CS-4, and is not mandatory. This relationship may be used to influence the basic call processing in the SSF/CCF (e.g. activation or de-activation of Call Forwarding) via call-unrelated interaction.
5
CCF/CUSF model

5.1
General

The CCF/CUSF model shown in Figure 1 is for a single‑ended service logic instance related to a user. The purpose of this model is to provide a framework for the understanding of call unrelated modelling with respect to the CCF/CUSF.

5.2
CCF/CUSF components

The CCF/CUSF model consists of several entities. Each of the entities is briefly described below. It is noted that this shows a conceptual model of CCF/CUSF and is not intended to imply an actual implementation of the CCF/CUSF.

5.2.1
Basic Non‑Call Manager (BNCM)

This entity provides an abstraction of an association and proceedings for call unrelated interactions between a user and a network. It detects basic call unrelated events that can lead to the invocation of IN service logic instances or those that should be reported to active IN service logic instances. It also manages resources required to support basic call unrelated control. The BNCM interacts with the FIM/NCM as described in the FIM/NCM description below. The BNCM also manages the BCUSM and the DP processing logic.

5.2.2
IN – Non-Switching Manager (IN-NSM)

This entity interacts with the SCF via SCF Access Manager in the course of providing IN services/features to users. It provides the SCF with an observable view of a CUSF call unrelated processing activities, and provides the SCF with access to CUSF capabilities, such as managing of an association and invoked operations. It also detects non-call processing events that should be reported to active IN service logic instances or can lead to the invocation of IN service logic instances. It manages CUSF resources required to support IN service logic instances, if any.

5.2.3
Feature Interaction Manager/Non-Call Manager (FIM/NCM)

This entity provides mechanisms to prevent invocation of multiple instances of IN and non-IN service logic instances on a single association. The ability of the FIM/NCM to arbitrate IN/non-IN call associated/call unrelated associated services/features and IN call unrelated associated services/features is outside the scope of this capability set. These two types of arbitration need an implicit relationship. The FIM/NCM integrates these interactions mechanisms with the BNCM and IN-NSM to provide the CUSF with a unified view of call unrelated processing internal to the CUSF for an association.
5.3
Relationship of CCF/CUSF model components

5.3.1
BNCM relationship to IN-NSM

This is the relationship that encompasses the interaction between the BNCM and the IN-NSM, through the FIM/NCM. The message related to this interaction is not externally visible and is not for standardization. However, an understanding of this subject is required to identify how basic call unrelated processing and IN call unrelated processing interact.

5.3.2
BNCM and IN-NSM relationship to FIM/NCM

This is the relationship that encompasses the interaction between the BNCM and the FIM/NCM, and the interaction between the IN-NSM and the FIM/NCM. The messages related to these interactions are not externally visible and are not for standardization. However, an understanding of this subject is required in order to unify the BNCM, IN-NSM and FIM/NCM.
5.4
Basic Call Unrelated State Model (BCUSM)

This model relates to the call unrelated associated connection-oriented interaction between the user and the IN service logic using existing ISDN user-to-network signalling procedures.

The main concept of the BCUSM is similar to the BCSM. The BCUSM is a high-level description of the CCF activities required to establish and maintain an association between users and service processing, to manage invoked operations. For this, it identifies a set of basic call unrelated associated activities in the CCF and shows how these activities are joined together to process a basic call unrelated associated services/features (e.g. establish and maintain an association between a user and a network, manage this and all operation invocations).

Many aspects of the BCUSM are not externally visible to IN service logic instances. However, aspects of the BCUSM that are reflected upward to the IN-NSM and FIM/NCM are visible to IN service logic instance. Only these aspects of the BCUSM will be the subjects of standardization. Therefore, the BCUSM is primarily an explanatory tool for providing a representation of CCF activities that can be analysed to determine which aspects of the BCUSM will be visible to IN service logic instances, if any, and what level of abstraction and granularity is appropriate for this visibility.

The BCUSM identifies points when IN service logic instances are permitted to interact with basic call unrelated interaction processing (for the time being it is reasonable to assume ROSE procedures). In particular, it provides a framework for describing basic call unrelated events that can lead to the invocation of IN service logic instances or reporting of the events to the active logic, for describing those points in association and operation processing when the transfer of control can occur.

Figure 2 shows the components that have been identified to describe a BCUSM. Points in association (PIA), detection points (DPs).
 Transitions and events are not depicted in the figure, but they are self-explanatory. Figure 2 also shows the BCUSM. PIAs represent the states of an association for the call unrelated interaction. The concepts of DPs are same as the BCSM, however an IN service logic instance can influence a call unrelated interaction processing with specifying a ROSE APDU (invoke, return result, return error, reject) which should be delivered via appropriate signalling messages between the user and the network (e.g. REGISTER, FACILITY, REL, and REL COMP on DSS1).

[image: image2.wmf]T11108180-00

SLPIA

SCF

CUSF

SSF

SRF

CCAF

SCUAF

CCAF

CCF

SMF

<INAP operations>

FE Access Manager

FE Access Manager

Switching Manager

Basic Call Manager

<BCSM>

Bearer Control

Interaction which may be

<BCUSM>

<xxx> Subjects Identified

Non-

Switching Manager

IN-

IN-

vender-specific

<INAP operations>

IN

-Non

-Switching

State Model 

Instance

<IN-

NSSM>

<IN-

NSSM events

 processing >

IN Local Resource

Data Manager

IN Local Resource

Data

Non-

IN Feature

Manager

Feature Interaction Manager/

Call Manager

Feature Interaction

Manager/

Non-

Call Manager

Basic Non-

Call Manager

<DP Processing

>

<Basic non-

call triggers

 processing>

<Basic non-

call events

 processing>

Basic Call

Resource Data

Manager

Basic Call

Resource Data

Functional 

Groupin

g/Entity

Data to be 

managed

Functional Grouping

Not addressed in 

this capability set


Figure 1/Q.1248.7 – Internal CUSF structure and the relationship between FEs
5.4.1
BCUSM description 

In the following descriptions, the PIAs are related at a high level to the procedure taking place in an entity that supports ROSE. To aid understanding, Q.931 and Q.932 messages with information elements are attached to the description: however, this should not be understood as limiting the scope of BCUSM only to DSS1.

The following is the BCUSM description for this capability set.

[image: image3.wmf]T11108190-00

Idle and Authorize Activation/

Association request

Activation Received and Authorized

Active

Release

Component Received

Activation Failed

Released

Association Release Requested

Detection Point

Point in Activation


Figure 2/Q.1248.7 – BCUSM
5.4.1.1
BCUSM model for ROSE APDU

5.4.1.1.1
Idle and Authorize Activation/Association request

Entry Event:

–
All previously invoked operations between a user and a network are completed (event: Released), or the (last) invocation of an operation was rejected by the user or the network (event: Activation Failed). There is no relationship between the user and the network; e.g. no call reference exists between the user and the network.

Functions:

–
Logical association is idled (e.g. no call reference, no data link connection).

–
When the indication to establish an association and optionally invoke an operation is received (e.g. Q.932 REGISTER with FACILITY IE), the authority/ability of the user invoking the particular operation is verified. For example, some restrictions may be checked that are assigned to the used access, such as invoking User Registration is prohibited, if any.

Exit Event:

–
An indication to establish an association and optionally invoke an operation is received (e.g. Q.932 REGISTER with FACILITY IE), the authority/ability of the user invoking the particular operation was verified (DP Activation Received and Authorized).

5.4.1.1.2
Active

Entry Event:

–
An indication to establish an association and optionally invoke an operation is received (e.g. Q.932 REGISTER with FACILITY IE), the authority/ability of the user invoking particular operation was verified (DP Activation Received and Authorized), or a response/invocation from the user is received (DP Component Received).

Functions:

–
Logical association is established (i.e. a call reference is assigned).

–
The received operation is processed and an appropriate response is sent to the user, or the response/invocation from the user is monitored.

Exit Event:

–
A response/invocation from the user is received (DP Component Received).

–
Activation is failed (Activation Failed).

–
An association release request is received from the user or the network (e.g. Q.931 REL). (DP Association Release Requested).

5.4.1.1.3
Release

Entry Event:

–
An association release request is received from the user or the network (e.g. Q.931 REL). This request optionally contains response/invocation to the user or from the network (DP Association Release Requested).

Functions:

–
The association is being released with appropriate procedures (a pending response may be delivered with an association release message, e.g. Q.931 RELComp). This procedure may deliver final invocation/response to the user or to the network.

Exit Event:

–
The association is completely released (Released).

5.4.1.1.4
Transition for BCUSM

Table 1/Q.1248.7 – Transition for BCUSM

	From
	To

	Activation Received and Authorized
	Idle and Authorize Activation/Association request

	Activation Received and Authorized
	Active

	Component Received
	Idle and Authorize Activation/Association request

	Component Received
	Active

	Association Release Requested
	Idle and Authorize Activation/Association request

	NOTE – These transitions are depicted here as they are beyond the normal BCUSM transitions.


5.4.1.1.5
Modelling for ROSE APDU handling

The current BCUSM models only the association handling parts and detection of the ROSE APDU as it is received. While it may be possible to model the analysis of the received ROSE APDU and invoke ID, it is not done since all ROSE ADPU are specific to a service. This would mean having to model every service. Therefore, only the general parts (component handling and association handling) are modelled. 

The modelling is similar to the BCSM in the sense of how far the interaction portion of the state model depicts the details of the activities in the physical entity, however the interaction is different from the circuit mode switched bearer services (represented by the BCSM) with the following:

a)
the variation of the service triggering points (corresponding to TDP) is limited to the association establishment/release phases or the reception of the ROSE APDU;

b)
the ROSE APDU is received during the association establishment/release phase or within the established association;

c)
the modelling of the analysis of a received ROSE APDU may be not necessary, because it can be well modelled as the TDP criteria check;

d)
the procedures with the interaction vary from service to service and have many variations, so each supplementary service has different states to handle the ROSE APDUs (but it may be possible to model reject or failure of the invocation).

The DP indicates the place of:

a)
the event of the association establishment request or release;

b)
the detection of the ROSE APDU reception (Figure 3 shows what part is service dependent and what part is general for this type of interaction).
[image: image4.wmf]T1188710-97

SLPI

SCP

INAP header

APDU        header

ISDN

CPE

SW

e.g. REGISTER message

CUSF/SSF/CCF

(Local switch)

TDP message of INAP

INAP APDU

On UNI

service depend, and has various internal structures

general parts (invoke id, component type, etc.)

General parameters are

separately mapped to the

defined INAP parameters

(e.g. "componentType", etc.)

Service dependent APDU is

dealt with as a whole

(mapped to the "component" 

parameter)

SLPI is responsible for

the internal structure of 

the APDU, therefore the

INAP protocol is not 

affected by the APDU

internal structure


Figure 3/Q.1248.7 – ROSE APDU treatment scheme

5.4.1.2
BCUSM model for association establishment from a service ASE in CUSF

5.4.1.2.1
Idle and Authorize Activation/Association request

Entry Event:

–
A previously invoked call unrelated association establishment is completed (event: Released), or the (last) association establishment was failed (event: Activation Failed). There is no association for a service ASE in CUSF.

Functions:

–
Logical association is idled.

–
When the request to establish a call unrelated association from a service ASE in CUSF is received, the authority/ability of the invocation of the association establishment is verified.

Exit Event:

–
The authority/ability of invoking the association establishment was verified (DP Activation Received and Authorized).

5.4.1.2.2
Active

Entry Event:

–
The authority/ability of invoking the association establishment was verified (DP Activation Received and Authorized); or
–
a response for the call unrelated association is received (DP Component Received).

Functions:

–
Logical association is established.

–
The response is processed and a further response is monitored.

Exit Event:

–
A response for the call unrelated association establishment is received (DP: Component Received).

–
The association establishment is failed (Activation Failed).

–
The association is released (DP: Association Release Requested).

5.4.1.2.3
Release

Entry Event:

–
The association is released (DP: Association Release Requested).

Functions:

–
The association is being released with appropriate procedures.
Exit Event:

–
The association is completely released (Released).
5.4.1.2.4
Transition for BCUSM

Table 2/Q.1248.7 – Transition for BCUSM

	From
	To

	Activation Received and Authorized
	Idle and Authorize Activation/Association request

	Activation Received and Authorized
	Active

	Component Received
	Idle and Authorize Activation/Association request

	Component Received
	Active

	Association Release Requested
	Idle and Authorize Activation/Association request

	Association Release Requested
	Active

	NOTE – These transitions are depicted here as they are beyond the normal BCUSM transitions.


5.4.2
BCUSM DP criteria

As for CS-4 BCUSM, the DP criteria means to check the specific out-channel interaction activation at the Activation Received and Authorized, or Component Received DP against the specified pattern, for example an operation code for ROSE like interaction. The formal description is not defined for IN CS-4.

Table 3 denotes the applicability of DP criteria to Dps of BCUSM. The entries in the table can be (these are the same as the BCSM):

•
Line-Based (Subscriber line or Trunk line);

•
Group-Based (including Private Facility Group);

•
Office-Based.
Table 3/Q.1248.7 – BCUSM DP criteria
	
	DPs

	DP criteria
	ActivationReceivedAndAuthorized
	ComponentReceived

	Trigger Assigned
	X
	X

	ITU-T ISDN supplementary services' operation codes 
	X
	O

	Regional/Network operator specific ISDN supplementary services' operation codes (Note)
	X
	O

	USIServiceIndicator
	X
	O

	X
Applicable

O
Optional

(The meaning of "X" and "O" is the same as for the BCSM.)

NOTE – The total number of the codes is regional or network operator specific.


6
SCF model

Refer to ITU-T Q.1248.1.

7
FSM for CUSF
7.1
Relations between CUSF FSM and the SSF/CCF and maintenance functions

The primitive interface between the CUSF FSM and the CCF/SSF/maintenance functions is an internal interface and is not subject to standardization in IN CS-4. Nevertheless, this interface should be in line with the BCUSM defined in 5.4. 

The relationship between the BCUSM and the CUSF FSM may be described as follows for the case of a call unrelated association/operation attempt from an end user or IN service logic:

–
When a call unrelated associated association/operation attempt is initiated by an end user and processed at an exchange, an instance of a BCUSM is created. As the BCUSM proceeds, it encounters detection points (DPs, see 5.4). If a DP is armed as a Trigger DP (TDP)
, an instance of a CUSF FSM is created. 

–
If an InitiateAssociation is received from the SCF, an instance of a BCUSM is created, as well as an instance of a CUSF FSM.

The CUSF logic should:

–
perform the DP processing actions, including if DP criteria are met;
–
check for SCF accessibility; and

–
handle service feature interactions in conjunction with the SSF.


NOTE – This processing is optional and network specific, and there are no more descriptions for these interactions.
The CUSF hands control back to the CCF at least in the following cases:

–
if a trigger (TDP) criteria match is not found (e.g. insufficient information to proceed): the CUSF logic returns supplementary service control processing to the CCF;
–
if the association is abandoned: the CUSF logic returns supplementary service control processing to the CCF;
–
if the destination SCF is not accessible: the CUSF logic will apply final treatment to the end user for the TDP‑R case, or return supplementary service control processing to the CCF for the TDP‑N case.

The management functions related to the execution of operations received from the SCF are executed by the CUSF Management Entity (CUSME)
-control. The CUSME‑control interfaces the different CUSF FSMs and the Functional Entity Access Manager (FEAM)
. Figure 4 shows the CUSF Interfaces.
The function of FEAM is defined in ITU-T Q.1248.1.

The CUSME‑control maintains the dialogues with the SCF on behalf of all instances of the CUSF Finite State Model (FSM)
. These instances of the CUSF FSM occur concurrently and asynchronously as associations occur, which explains the need for a single entity that performs the task of creation, invocation, and maintenance of the CUSF FSMs. In particular, the CUSME‑control performs the following tasks:

1)
interprets the input messages from other FEs and translates them into corresponding CUSF FSM events;

2)
translates the CUSF FSM outputs into corresponding messages to other FEs;

3)
captures asynchronous (with processing association and/or operation request from the end user) activities related to management or supervisory functions in the CUSF;

4)
checks the existence of a CUSF-SCF relationship by receiving an ActivityTest operation from the SCF and returns the result to the SCF.

The CUSF FSM passes component handling instructions to the related instances of the BCUSM as needed. DPs may be dynamically armed as EDPs, requiring the CUSF FSM to remain active. At some point, further interaction with the SCF is not needed, and the CUSF FSM may be terminated while the BCUSM continues to handle the association as needed. 

[image: image5.wmf] 

 

 

 

 

 

 

 

 

T11108200-00

SCUAF

FEAM

SCF

CUSF

FSM

CUSF

FSM

CUSME-

Control


Figure 4/Q.1248.7 – CUSF interfaces

7.2
CUSF state transition diagram

Figure 5 shows the state diagram of the CUSF part of the SSP, CUSP, SN (see ITU-T Q.1205) during the processing of an IN association request from the user or IN service logic/operation attempt.

Each state is discussed in the following subclauses. General rules applicable to more than one state are addressed here.

One or a sequence of components received in one or more TCAP messages may include a single operation or multiple operations, and is processed as follows:

–
Process the operations in the order in which they are received.

–
Each operation causes a state transition independent of whether a single operation or multiple operations are received in a message.

–
The CUSF examines subsequent operations in the sequence. As long as sequential execution of these operations leave the FSM in the same state, it will execute them. If a subsequent operation causes a transition out of the state then the following operations should be buffered until the current operation has been executed. In all other cases, wait for an event that would cause a transition out of the current state (such an event would be the completion of operation being executed, or reception of an external event). An example of this is as follows:


The CUSF receives the operations SendComponent, ReleaseAssociation in a component sequence inside a single TCAP message. Upon receipt of this message, these operations are executed up to and including SendComponent while the CUSF is in the Waiting for Instructions state. However, if the SendComponent specifies to deliver the component with an association release message to the user, the actual transmission of the component should be deferred until the next operation, ReleaseAssociation. The operation will be executed when the FSM transits to the Idle state. 

–
If there is an error in processing one of the operations in the sequence, the CUSF FSM processes the error (see below) and discards all remaining operations in the sequence.

–
If an operation is not understood, or is out of context (i.e. violates the SACF rules defined by the CUSF FSM) as described above, the CUSF aborts the TC dialogue. 

In any state, if there is an error in a received operation, the maintenance functions are informed and the CUSF FSM remains in the same state as when it received the erroneous operation. Depending on the class of the operation, the error could be reported by the CUSF to the SCF using the appropriate component (ITU-T Q.774).

In any state (except Idle), if the user requesting the association abandons before it is established (i.e. before the Active PIA in the BCUSM), then the CUSF should clear the association and ensure that any CUSF and CCF resources allocated to the association have been de‑allocated. The CUSF FSM then moves to the Idle state. 
In any state (except Idle), if the user releases a stable association (i.e. from the Active PIA in the BCUSM), then the CUSF FSM should move to the Idle state.

The CUSF has an application timer, TCUSF, whose purpose is to prevent excessive association processing suspension time and to guard the association between the CUSF and the SCF.

Timer TCUSF is set in the following cases:

–
when the CUSF sends a TDP-R/EDP-R reporting operation (refer to 7.2 State b: Waiting for Instructions);
–
when the CUSF receives an InitiateAssociation operation from the SCF;
–
when the CUSF FSM enters the Waiting for Instructions state under any other conditions than the ones listed above. If the timer is already active, the timer is restarted.

On expiration of TCUSF the CUSF FSM transits to the Idle state and aborts the TC dialogue with the SCF, and the CUSF proceeds with the BCUSM if possible. 

The CUSF state diagram contains the following transitions (events):

er1
TDP‑R encountered;

er2
Idle return from Waiting for Instructions;

er3
Request to send a component received (if no EDP armed) or monitoring instruction received;

er4
TDP‑N encountered;

er5
Request to send a component received (if EDP(s) armed);

er6
EDP‑N not last encountered;

er7
EDP‑N last encountered or monitor duration for last EDP expired;
er8
EDP‑R encountered;

er10
Initiate association received.

The CUSF state diagram contains the following states:

State a

Idle;

State b

Waiting for Instructions;

State c

Monitoring.

[image: image6.wmf]T1188730-97

er4

State a

Idle

State b

Waiting For

Instructions

State c

Monitoring

er1

er10

er2

er7

er5

er8

er3

er6


Figure 5/Q.1248.7 – CUSF FSM

7.2.1
State a: "Idle"

The CUSF FSM enters the Idle state when sending or receiving an ABORT TCAP primitive due to abnormal conditions in any state. 

The CUSF FSM enters the Idle state when one of the following occurs:

–
when the association is released by the end user request in the Waiting for Instructions (transition er2) or in the Monitoring (transition er7);

–
when a ReleaseAssociation, a ConnectAssociation or a ContinueAssociation (without monitor requested) operation is processed in the Waiting for Instructions (transition er2);

–
when a last EDP‑N is reported in the Monitoring (transition er7);

–
when the application timer TCUSF expires in the Waiting for Instructions state (transition er2).

When transiting to the Idle state, if there is a component to be delivered with an association release message to the user, the CUSF sends the component with the specified association release message to the SCUAF before returning to Idle. 

During this state, the following call unrelated associated event can occur:

–
an armed TDP is encountered related to a possible IN call unrelated attempt, the CUSF FSM acts as described below:

•
if the DP is a TDP-N, send a DP‑specific operation or an InitialAssociationDP to the SCF, as determined from DP processing; there is no resulting transition to a different state (transition er4);

•
if the DP is a TDP‑R, send a DP‑specific operation or an InitialAssociationDP to the SCF, as determined from DP processing, and transit to the Waiting for Instructions state (transition er1); 

–
a message related to a new transaction containing an InitiateAssociation operation is received from the SCF: in this case the CUSF sends a return result for the operation and moves to the state Waiting for Instructions (transition er10).

Any other operation received from the SCF while the CUSF is in Idle state should be treated as an error. The event should be reported to the maintenance functions and the transaction should be aborted according to the procedure specified in TCAP (ITU-T Q.774).

NOTE – DP specific operations are the following: ActivationReceivedAndAuthorized, ComponentReceived and AssociationReleaseRequested.

7.2.2
State b: "Waiting for Instructions"

This state is entered from the Idle state on sending an InitialAssociationDP or a DP Specific operation (transition er1) or on receiving an InitiateAssociation (transition er10), or from the Monitoring state on detection of an EDP-R (transition er8). 

In this state, the CUSF FSM is waiting for an instruction from the SCF; association handling/supplementary service processing is suspended and an application timer (TCUSF) should be set on entering this state.

During this state, the following events can occur:

–
The user releases the association. This should be processed in accordance with the general rules in 7.2. 

–
The application timer TCUSF expires: the CUSF FSM moves to the Idle state, the CUSF processes the invocation if possible, the TCUSF expiration is reported to the maintenance functions and the transaction is aborted.

–
An operation is received from the SCF: The CUSF FSM acts according to the operation received, as described below.

The following operations may be received from the SCF, and processed by the CUSF, with no resulting transition to a different state (transition er3):

SendComponent (if no EDP is armed),

RequestReportBCUSMEvent.

The following operations may be received from the SCF and processed by the CUSF, causing a state transition to the Monitoring state (transition er5):

SendComponent (if EDP(s) is(are) armed),
ConnectAssociation (if EDP(s) is(are) armed),
ContinueAssociation (if EDP(s) is(are) armed).
ReleaseAssociation operation may be received from the SCF. In this case, the CUSF FSM transits to the Idle state (transition er2). In addition, the CUSF should release the association to the user and ensure that any CUSF resources allocated to the association have been de‑allocated.
ContinueAssociation or ConnectAssociation operation may be received from the SCF. In this case, the CUSF FSM transits to the Idle state (transition er2).  In addition, the CUSF returns the service control to the suspended association handling/supplementary service processing in the CUSF if no DP is armed.

Any other operations received in this state should be processed in accordance with the general rules in 7.2. 

7.2.3
State c: "Monitoring"

The CUSF enters this state from the Waiting for Instructions state (transition er5) upon receiving a SendComponent, ContinueAssociation or ConnectAssociation if EDP(s) is (are) armed.

In this state, the timer TCUSF is not used.

During this state the following events can occur:
–
An EDP‑N should be reported to the SCF by sending a DP specific operation or an EventReportBCUSM operation; the CUSF FSM should remain in the Monitoring state (transition er6) if one or more EDPs are armed. The CUSF FSM should move to the Idle state (transition er7) if there are no remaining EDPs armed.
–
An EDP‑R should be reported to the SCF by sending a DP specific operation or an EventReportBCUSM operation; the CUSF FSM should move to the Waiting for Instructions state (transition er8).

–
The receipt of an END or ABORT primitive from TCAP handling the INAP has no effect on the association; the association may continue or be completed with the information available. In this case, the CUSF FSM transits to the Idle state (transition er7), disassociating the CUSF FSM from the association. 

–
The user abandons or releases the association. This should be processed in accordance with the general rules in 7.2.
8
FSM for SCF

8.1
Overview

Figure 6 is an SCF FSM structure. Related parts with this clause are:
–
SCME FSM interacting with CUSME-control in CUSF; and

–
SCSM-CUSF interacting with CUSF FSM in CUSF,
and are highlighted in the figure.
[image: image7.wmf] 

 

 

 

 

 

 

 

…

 

 

…

 

…

…

 

…

 

T11108210-00

FEAM

SSF

SRF

SDF

SMF

CUSF

SCF

SCSM-

SSF/SRF

SCSM-

SCF

SCSM-

SDF

SCME-

FSM

SCSM-

CUSF

SCF

SCME

-Control


Figure 6/Q.1248.7 – SCF FSM structure

8.2
SCME FSM
The SCME handles an ActivityTest operation in a CUSF-SCF relationship.

The ActivityTest related SCME FSM is the same FSM as defined in ITU-T Q.1248.2 by replacing "SSF" with "CUSF".

8.3
CUSF-related states (SCSM-CUSF)

Figure 7 shows State Diagram of the SCSM as relevant to the procedures concerning the FSM for CUSF part of the SCP/AD/SN (see ITU-T Q.1205) during the processing of an IN call. Each state is discussed in the following subclauses.

An instance of FSM for CUSF is created on reception of an instruction from the CUSF indicating a TDP, and an instruction from the SCF. The instance is released when the state of the instance of FSM for CUSF transits to the state "Idle".
The following operation is a class 1 and requires reception of a Return Result. Reception of the Return Result for the operation does not result in a state transition in the SCSM-CUSF. Furthermore, this Return Result may be received in any state of the SCSM-CUSF.

–
InitiateAssociation.

The letter "N" is added to the head of the number of each state and event in FSM for CUSF to distinguish the states and events in FSM for CUSF from those in other FSMs in the SCSM.
Note that "Ne" is an event from SCF (internal event), and "NE" is an event from CUSF (external event).

8.3.1
State N1: "Idle"

The following events are considered in this state:
•
(Ne1) CUSF_Initiate_Control_Requested: This is an internal event caused by the service logic's need to have a new control relationship with CUSF. The FSM for CUSF requests to transmit the InitiateAssociation operation to the CUSF. This event causes a transition to the state N2, Preparing CUSF Instructions. 

•
(NE2) Query_from_CUSF: This is an external event, caused by a reception of one of the following operations:

–
ActivationReceivedAndAuthorized (for TDP-R),
–
ComponentReceived (for TDP-R),
–
AssociationReleaseRequested (for TDP-R),
–
InitalAssociationDP (for TDP-R).


This event causes a transition to State N2, Preparing CUSF Instructions.
•
(NE3) Notification_from_CUSF: This is an external event, caused by a reception of one of the following operations:

–
ActivationReceivedAndAuthorized (for TDP-N),
–
ComponentReceived(for TDP-N),
–
AssociationReleaseRequested (for TDP-N),
–
InitalAssociationDP (for TDP-N).


This event causes a transition back to the same state.
8.3.2
State N2: "Preparing CUSF Instructions"

In this state, the SCF prepares appropriate instructions to the CUSF.

The following events are considered in this state:

•
(Ne4) Processing_completed: This is an internal event. In this case, the SCF has completed the processing of the instructions to the CUSF. This event causes the following operation to be sent to the CUSF and a transition to State N1, Idle:
–
ReleaseAssociation,
–
ConnectAssociation (no DP armed),

–
ContinueAssociation (no DP armed).

To further describe the procedures relevant to this state, the state is divided into two sub‑states which are described in the following two clauses (this subdivision is illustrated in Figure 8).

8.3.2.1
State N2.1: "Preparing CUSF Instructions"

In this state, the SCF determines whether the BCUSM processing will be resumed and deals with an EDP related processing.

The following events are considered in this state:

•
(Ne2.1) Event_Request: this is an internal event caused by the service logic when there is a need to send one or more RequestReportBCUSMEvent operations to be issued to the CUSF. This event causes a transition back to State N2.1 Preparing CUSF Instructions.

•
(Ne2.2) Request_Send_Component (monitor not required): this is an internal event caused by the service logic when there is no armed EDP but a need to send one or more SendComponent operations to be issued to the CUSF. This event causes a transition back to State N2.1 Preparing CUSF Instructions.

•
(Ne2.3) Request_Send_Component (monitor required): this is an internal event caused by the service logic when there is (are) an armed EDP(s) and a need to send a SendComponent operation to the CUSF. This resumes the call unrelated processing in the CUSF. This event causes a transition to State N2.2 Waiting for Notification or Request.

•
(Ne2.8) Request_Processing_Resume (monitor required): this is an internal event caused by the service logic when there is one or more armed EDPs and a need to send a ContinueAssociation or a ConnectAssociation operation to the CUSF. This also resumes the call unrelated processing in the CUSF. This event causes a transition to State N2.2 Waiting for Notification or Request.

•
(Ne2.4) Request_Release_Association: this is an internal event caused by the service logic when it needs to release the association between the user and the network by sending a ReleaseAssociation operation to the CUSF. This event maps into the FSM for CUSF in the SCSM event (Ne4). 

8.3.2.2
State N2.2: "Waiting for Notification or Request"

In this state, FSM for CUSF waits for a notification or a request from the CUSF. 

The following events are considered in this state:

•
(NE2.5) EDP-R: this is an external event caused by the reception of one of the following operations:

–
ComponentReceived (for EDP-R),
–
AssociationReleaseRequested (for EDP-R), and

–
EventReportBCUSM (for EDP-R).


This event causes a transition to State N2.1 Preparing CUSF Instructions. 

•
(NE2.6) Not_Last_EDP-N: this is an external event caused by the reception of one of the following operations:

–
ComponentReceived (for EDP-N), 

–
AssociationReleaseRequested (for EDP-N), and

–
EventReportBCUSM (for EDP-N). 

In this case, there is still an outstanding armed EDP. This event causes a transition back to State N2.2 Waiting for Notification or Request. 

•
(NE2.7) Last_EDP-N: this is an external event caused by the reception of one of the following operations:

–
ComponentReceived (for EDP-N),
–
AssociationReleaseRequested (for EDP-N), and

–
EventReportBCUSM (for EDP-N).


This event maps into the FSM for CUSF in the SCSM event (Ne4).

[image: image8.wmf]T11108220-00

(NE3) Notification_from_CUSF

N1 Idle

(Ne4) Processing_Completed

N2 Preparing CUSF

instructions

(Ne1) CUSF Initiate_Control_Request

(NE2) Query_from_CUSF


Figure 7/Q.1248.7 – SCSM: FSM for CUSF

[image: image9.wmf]T11108230-00

(Ne2.2)

(Ne2.3)

(NE2.5) EDP-R

(Ne4)

N2 Preparing CUSF instructions

(Ne2.4) Request_

Release_Association

N2.1 Preparing CUSF

Instructions

Request_to_CUSF

(Ne2.1) Event_

Request

Send_Component_

to_CUSF

Response to

CUSF

N2.2 Waiting for

Notification or Request

(NE2.6)

Not_Last_EDP-N

NOTE  –  (Ne2.2) Request_Send_Component (monitor not required)

 (Ne2.3) Request_Send_Component (monitor required)

(Ne2.8) Request_

Processing_Resume

Ne1 and NE2

(NE2.7) Last_EDP-N


Figure 8/Q.1248.7 – SCSM: Sub‑states of State N2

9
FSM for USI
9.1
Overview

A generic transport mechanism (transparent at the CUSF level) for the exchange of information between the User and the Service Logic is supported based on INAP operations, respectively SendSTUI in the "SCF-to-User" direction and ReportUTSI in the "User-to-SCF" direction. This user information (USI) is composed of two elements (parameters) which are USIServiceIndicator and USIInformation. The first parameter identifies the IN Service Logic invoked while the second carries useful information between the User/Service Logic and the Service Logic. 

Refer to ITU-T Q.1248.1 for a definition of the term "User" in the context of the Out-Channel Call-Unrelated User Interaction (OCCUUI)
 mechanism. A User to Service Information (USI)
 refers to either a UTSI or an STUI.

In the "SCF-to-User" direction, once the CUSF receives an STUI from the SCF within the SendSTUI operation, the CUSF forwards it to the appropriate user application in the network, e.g. an ISDN user. This User is clearly and easily defined by the Association Reference (indicated by the down-lower protocols) and the legID parameter (indicated by the INAP protocol).

In the "User-to-SCF" direction, the CCF/CUSF decides whether it forwards the USI information received from a User to the succeeding/preceding exchange or it passes it to a specific IN service. The received USI information is either of the following two cases:

Case 1)
The USI information is considered as a "notification event" previously requested by the SCF. In this case, there is already a CUSF-SCF relationship.

Case 2)
The USI information is an additional information. In this case, it is only an optional information.

The CUSF addresses the appropriate SCF based upon the ServiceIndicator parameter which is received within the USI. The ServiceIndicator value is indicated by the User (e.g. ISDN User) or by the Service Logic:

–
If the SCF has initiated the USI dialogue, the User sets the ServiceIndicator value of the USI to the ServiceIndicator value as requested by the SCF. This scenario corresponds to the first case.

–
If the user sends a USI without having previously received an STUI from the SCF, then it initializes the ServiceIndicator value of the USI with a predefined value. This scenario corresponds to the second case.

9.1.1.1
Case 1
In this case, the CUSF communicates with the SCF during an already existing SCF-CUSF relationship; the SCF initiates the "Out-Channel" dialogue with the User sending an STUI within the SendSTUI operation. Both SCF and CUSF behaviour can be described as above:

–
Independently from the BCUSM processing, the SCF instructs the CUSF with the RequestReportUTSI operation to report to it all the USI information with a given ServiceIndicator value.

–
Then, once it receives a USI, the CCF/CUSF compares the ServiceIndicator value received with the previously indicated ServiceIndicator value. If they coincide, then the CUSF reports the USI to the SCF with the ReportUTSI operation.

In this case, the required ServiceIndicator value is explicitly indicated by the SCF and stored at the CUSF level; this data is in a table associated with the CUSF USI FSM.

NOTE – The STUI/UTSI is conveyed in the appropriate signalling message (Basic Call Unrelated Control signalling message or Facility message) depending on the phase of the association on the dedicated leg (e.g. on the BCUSM processing).

9.1.1.2
Case 2
The USI information does not impact the usual DP processing. The USI is only an optional parameter that the CUSF introduces in the following operations after analysing the corresponding ServiceIndicator value to make sure that the received USI is targeted to this particular Service Logic.

–
InitialAssociationDP.
–
DP Specific operations.

The CUSF checks if the ServiceIndicator value of the USI coincides with the ServiceIndicator value contained within the Service related data table (trigger table).
NOTE – The STUI/UTSI is conveyed in the appropriate signalling message (Basic Call Unrelated Control signalling message or Facility message) depending on the phase of the call on the dedicated leg (e.g. on the BCUSM processing). 

9.1.1.3
Synthesis

Table 4/Q.1248.7 – Synthesis of OCCUUI conditions
	Scenario
	USI FSM
	Service Indicator indicated by the User
	Service Indicator value of reference
	Operation sent to the SCF

	USI dialogue
	Monitoring USI 
	Indicated in the STUI
Dynamic
	Stored in the data table associated with the CUSF USI FSM
	ReportUTSI

	USI = additional information
	Idle
	Pre-defined
	Explicitly indicated in the IN-related data table of the IN service

Static in the CUSF
	– InitialAssociationDP

– DP specific operations


[image: image10.wmf]T11108240-00

From the User

USI

No

Yes

Scenario 1

Normal DP processing

(e.g. scenario 2)

"Monitoring 

USI"


Figure 9/Q.1248.7 – SDL for OCCUUI processing in CUSF

9.2
SCF USI FSM for CUSF

The SCF USI FSM in this clause specifies USI related state transitions in SCF in a CUSF-SCF relationship. The SCF USI FSM instance exists in any state except when the FSM instance for CUSF in SCSM-CUSF (refer to 8.3) is in the Idle state.

The SCF USI FSM is depicted in Figure 10.

[image: image11.wmf]T11108250-00

(Neo3)

(Neo2)

(Neo1)

Idle

Monitoring 

USI


Figure 10/Q.1248.7 – SCF USI FSM

The SCF USI FSM transitions are defined as follows:

–
(Neo1): the SCF requests the CUSF to monitor the receipt of a USI with a given USIServiceIndicator value by sending a RequestReportUTSI operation.

–
(Neo2): the SCF is no longer interested in the receipt of a USI with the given USIServiceIndicator value. The SCF sends a RequestReportUTSI operation with the uSImonitorMode parameter set to "monitoringInactive".

–
(Neo3): the SCF sends a SendSTUI operation to the User and/or receives a ReportUTSI operation from the User with the given USIServiceIndicator value.
9.3
CUSF USI FSM

The CUSF USI FSM specifies USI related state transitions in CUSF in a CUSF-SCF relationship. The CUSF USI FSM instance exists only in any state except when the FSM instance for CUSF (refer to 7.2) is in the Idle state.

The CUSF USI FSM is the same FSM as defined in ITU-T Q.1248.2 (SSF USI FSM) by replacing "SSF" with "CUSF".
10
Operation procedures

All the parameters in clause 10.1.2 "Parameters", are defined in clause 11.

10.1
ActivationReceivedAndAuthorized procedure

10.1.1
General description

This operation is sent by the CUSF to the SCF after detecting a valid trigger condition at the BCUSM ActivationReceivedAndAuthorized DP. This operation can be available in the following three cases.

Case 1)
TDP is detected by receiving a FACILITY information element (a ROSE APDU) from the user (refer to 5.4 for modelling). In this case, component related parameters (componentType, component, and componentCorrelationID) are sent with this operation. The service ASE is located in the SCF, and the CUSF acts as a relay function between the user and the SCF. The SCF may send ROSE APDU information and receive further ROSE APDU information.
Case 2)
TDP is detected by receiving a USI information element. In this case, USI related parameters (uSIServiceIndicator and uSIInformation) are sent with this operation. The service ASE is located in the SCF, and the CUSF acts as a relay function between the user and the SCF. The SCF may send USI information and receive further USI information.
Case 3)
TDP is detected when a service ASE located in the CCF (BCUSM) needs further information to establish a bearer independent association. In this case, cUApplicationInd parameter and other connection related parameters (calledPartyNumber, genericNumbers, etc.) are sent with this operation. The CUSF acts as a relay function between the service ASE and the SCF. The SCF provides additional information for the association processing.
10.1.2
Parameters

10.1.2.1
Argument parameters

The operation argument consists of the following parameters. These parameters are defined in clause 11.

•
callUnrelatedDPSpecificCommonParameters

–
serviceAddressInformation
•
serviceKey

•
miscCallInfo

–
messageType



When the messageType is "request", the TDP or EDP is reported as a request which means that the call unrelated processing is suspended for the SCF instructions. In EDP case, the message type for the EDP is requested by the RequestReportBCUSMEvent operation with monitorMode set to "interrupted".



When the messageType is "notification", the TDP or EDP is reported as a notification which means that the call unrelated processing is not suspended for the SCF instructions. In EDP case, the message type for the EDP is requested by the RequestReportBCUSMEvent operation with monitorMode set to "notifyAndContinue".

–
dPAssignment



This sub-parameter is used only for TDP.

•
triggerType
–
callingPartyNumber
–
locationNumber
–
terminalType
–
extensions

–
uSIServiceIndicator

–
uSIInformation

–
cUApplicationInd

–
calledPartyNumber

–
callingPartySubadress

–
highLayerCompatibility

–
bearerCapability.



This parameter consists of the following alternatives:

•
bearerCap

or

•
tmr

–
genericNumbers

•
componentType


This indicates the type of ROSE component that is received from a user and reported to the SCF. The value of "invoke" can only be available.

•
componentCorrelationID
•
extensions

•
component
10.1.3
Invoking entity (CUSF)

10.1.3.1
Normal procedure

CUSF Preconditions (TDP):

1)
CUSF-FSM is in State a: Idle.

2)
An event fulfilling the criteria for the TDP has been detected. The event is one of three cases in 10.1.

CUSF Postconditions:

1)
CUSF-FSM moves to State b: Waiting for Instructions if TDP-R.
2)
CUSF-FSM remains in State a: Idle if TDP-N.

10.1.3.2
Error handling

Generic error handling for the operation-related errors is described in clause 12, and the TC services which are used for reporting operation errors are described in clause 13.

10.1.4
Responding entity (SCF)

10.1.4.1
Normal procedure

SCF Precondition (TDP):

1)
FSM for CUSF within the SCF is in the State N1: Idle.

SCF Postconditions (TDP-R):

1)
FSM for CUSF within the SCF moves to State N2: Preparing CUSF Instructions.

2)
Waiting for the request from the SLPI and CUSF instructions is being prepared. 

SCF Postcondition (TDP-N):

1)
FSM for CUSF within the SCF remains in State a: Idle.

10.1.4.2
Error handling

Generic error handling for the operation-related errors is described in clause 12, and the TC services which are used for reporting operation errors are described in clause 13.

10.2
ActivityTest procedure

10.2.1
General description

This operation is used to check for the continued existence of a relationship between the SCF and the CUSF. If the relationship is still in existence, then the CUSF will respond. If no reply is received within a given time period, then the SCF which sent this operation will assume that the CUSF has failed in some way and will take the appropriate action.

10.2.2
Parameters

10.2.2.1
Argument parameters

None.
10.2.2.2
Result parameters

None.
10.2.3
Invoking entity (SCF)

10.2.3.1
Normal procedure

SCF Preconditions:

1)
A relationship exists between the SCF and the CUSF.

2)
The activity test timer (Tati) expires, after which the "ActivityTest" operation is sent to the remote entity.

3)
The SCME is in state "Activity Test Idle".

SCF Postcondition:

1)
The SCME is in the state "Waiting for Activity Test Response". If a Return Result "ActivityTest" is received, the SCME resets the activity test timer, returns to state "Activity Test Idle", and takes no further action.

10.2.3.2
Error handling

Refer to the error handling of the ActivityTest operation procedure in ITU-T Q.1248.2.

10.2.4
Responding entity (CUSF)

10.2.4.1
Normal procedure

CUSF Precondition:

1)
A relationship exists between the SCF and the CUSF.
CUSF Postconditions:

1)
If the dialogue ID is active and if there is an FSM for CUSF using the dialogue, the CUSME-Control sends a Return Result "ActivityTest" to the SCF; or

2)
If the dialogue ID is not active, the TCAP in the CUSF will issue a P-Abort; the CUSME‑Control will in that case never receive the "ActivityTest" invocation and thus will not be able to reply.

10.2.4.2
Error handling

Operation-related error handling is not applicable, due to class 3 operation.

10.3
AssociationReleaseRequested procedure

10.3.1
General description

This operation is sent by the CUSF to the SCF after detecting a valid trigger condition at the BCUSM AssociationReleaseRequested DP (reported as TDP) or after detecting a previously requested event with RequestReportBCUSMEvent operation (reported as EDP). This operation can be available in the following three cases.

Case 1)
TDP/EDP is detected by receiving a FACILITY information element (a ROSE APDU) from the user (refer to 5.4 for modelling). In this case, component related parameters (componentType, component, and componentCorrelationID) are sent with this operation. The service ASE is located in the SCF, and the CUSF acts as a relay function between the user and the SCF. The SCF may send ROSE APDU informations and receive further ROSE APDU informations.
Case 2)
TDP is detected by receiving a USI information element. In this case, USI related parameters (uSIServiceIndicator and uSIInformation) are sent with this operation. The service ASE is located in the SCF, and the CUSF acts as a relay function between the user and the SCF. The SCF may send USI information and receive further USI information.
Case 3)
TDP/EDP is detected by requesting from a service ASE located in the CCF (BCUSM) for needing further information to establish a bearer independent association. In this case, cUApplicationInd parameter and other connection related parameters (calledPartyNumber, genericNumbers, etc.) are sent with this operation. The CUSF acts as a relay function between the service ASE and the SCF. The SCF provides additional information for the association processing.
10.3.2
Parameters
10.3.2.1
Argument parameters

The operation argument consists of the following parameters. These parameters are defined in clause 11.

–
callUnrelatedDPSpecificCommonParameters

Refer to ActivationReceivedAndAuthorized procedure for the sub-parameters usage in this parameter.

–
componentType 


This parameter indicates the type of ROSE component that is received from a user and reported to the SCF. For example, the "invoke" is used to report the invocation of an operation from the user, and the "returnResult" is used to report the return result to the previously issued operation to the user by using a SendComponent operation from the SCF.
–
componentCorrelationID
–
extensions
–
component
–
legID
The alternative "receivingSideID" is only used in this operation. If this parameter is absent, the default value is assumed based on how the association has been initiated. In case of an SCF initiated association by InitiateAssociation operation and an EDP is reported, legID = 2, otherwise, legID = 1.

10.3.3
Invoking entity (CUSF)

10.3.3.1
Normal procedure

CUSF Preconditions (TDP):

1)
CUSF-FSM is in the State a: Idle.

2)
The association establishment request is received from the user, and the component portion of the FACILITY IE meets the triggering criteria (optional).

CUSF Preconditions (EDP):

1)
CUSF-FSM is in the State c: Monitoring.

2)
The operation invocation request is received from the user, and the component portion of the FACILITY IE meets the event report criteria (optional).

CUSF Post conditions [1) or 2)]:

1)
CUSF-FSM moves to State b: Waiting for Instructions (TDP-R or EDP-R).

2)
CUSF-FSM remains in or moves to State a: idle (TDP-N or last EDP-N).

3)
CUSF-FSM remains in State c: Monitoring (not last EDP-N).

10.3.3.2
Error handling

Generic error handling for the operation‑related errors is described in clause 12, and the TC services which are used for reporting operation errors are described in clause 13.

10.3.4
Responding entity (SCF)

10.3.4.1
Normal procedure

SCF Precondition (TDP):

1)
FSM for CUSF (SCSM-CUSF) is in State N1: Idle.

SCF Precondition (EDP):

1)
FSM for CUSF (SCSM-CUSF) is in State N2.2: Waiting for Notification of Request.

SCF Postconditions:

1)
FSM for CUSF (SCSM-CUSF) moves to State N2: Preparing CUSF Instructions (TDP‑R or EDP-R).

2)
FSM for CUSF (SCSM-CUSF) remains in or moves to State a: Idle (TDP-N or last EDP‑N).

3)
FSM for CUSF (SCSM-CUSF) remains in State c: Waiting for Notification or Request (not last EDP-N).

4)
Waiting for the request from the SLPI and CUSF instructions is being prepared.

10.3.4.2
Error handling

Generic error handling for the operation-related errors is described in clause 12, and the TC services which are used for reporting operation errors are described in clause 13.
10.4
ComponentReceived procedure

10.4.1
General description

This operation is sent by the CUSF to the SCF after detecting valid trigger conditions at the BCUSM ComponentReceived DP (reported as TDP) or after detecting a previously requested event with RequestReportBCUSMEvent operation (reported as EDP). This operation can be available in the three cases described in 10.3.

10.4.2
Parameters
10.4.2.1
Argument parameters

The operation argument consists of the following parameters. These parameters are defined in clause 11.

–
callUnrelatedDPSpecificCommonParameters


Refer to ActivationReceivedAndAuthorized procedure for the sub-parameters usage in this parameter.

–
componentType

Refer to AssociationReleaseRequested operation procedure for the usage of this parameter.

–
componentCorrelationID
–
extensions
–
component
–
legID

Refer to AssociationReleaseRequested operation procedure for the usage of this parameter.

10.4.3
Invoking entity (CUSF)

10.4.3.1
Normal procedure

CUSF Preconditions (TDP):

1)
CUSF-FSM is in State a: Idle.

2)
The association has been established between the user and the network.

3)
The operation invocation request is received from the user, and the component portion of the FACILITY IE meets the triggering criteria (optional).

CUSF Preconditions (EDP):

1)
CUSF-FSM is in State c: Monitoring.

2)
The operation invocation request is received from the user, and the component portion of the FACILITY IE meets the event report criteria (optional).

CUSF Postconditions [1) or 2)]:

1)
CUSF-FSM moves to State b: Waiting for Instructions (TDP-R or EDP-R).

2)
CUSF-FSM remains in or moves to State a: Idle (TDP-N or last EDP-N).

3)
CUSF-FSM remains in State c: Monitoring (not last EDP-N).

10.4.3.2
Error handling

Generic error handling for the operation-related errors is described in clause 12, and the TC services which are used for reporting operation errors are described in clause 13.

10.4.4
Responding entity (SCF)

10.4.4.1
Normal procedure

SCF Precondition (TDP):

1)
FSM for CUSF (SCSM-CUSF) is in State N1: Idle.

SCF Precondition (EDP):

1)
FSM for CUSF (SCSM-CUSF) is in State N2.2: Waiting for Notification of Request.

SCF Postconditions:

1)
FSM for CUSF (SCSM-CUSF) moves to State N2.1: Preparing CUSF Instructions (TDP/EDP-R).

2)
FSM for CUSF (SCSM-CUSF) remains in or moves to State N1: Idle (TDP-N or last EDP‑N).

3)
FSM for CUSF (SCSM-CUSF) remains in State N2.2: Waiting for Notification or Request (not last EDP-N).

10.4.4.2
Error handling

Generic error handling for the operation-related errors is described in clause 12, and the TC services which are used for reporting operation errors are described in clause 13. 

10.5
ConnectAssociation procedure

10.5.1
General description

This operation is used to request the CUSF to proceed with processing. Additional information which shall be used in further association establishment is provided by the SCF.

CUSF continues association establishment (connection-oriented bearer independent transport) to the specified destination using the address information received from the SCF. A two-party association between the originating user/network application and the terminating user/network application is established. The communication path toward each of the user/network applications is identified by leg ID. If the release request event is received from the terminating user/network, then a follow-on association establishment is allowed.

10.5.2
Parameters

10.5.2.1
Argument parameters

The operation argument consists of the following parameters. These parameters are defined in clause 11.

–
address

–
genericNumbers
–
extensions
10.5.3
Invoking entity (SCF)

10.5.3.1
Normal procedure

SCF Preconditions:

1)
FSM for CUSF (SCSM-CUSF) is in State N2: Preparing CUSF Instructions. 

2)
The SLPI has determined that a "ConnectAssociation" has to be sent by the SCF.

SCF Postcondition:

1)
FSM for CUSF (SCSM-CUSF) moves to State N1: Idle, if monitoring is not required, or moves to State N2.2: Waiting for Notification or Request, if monitoring is required.

NOTE – The information provided by the SCF depends on the service ASE located in the CUSF.

10.5.3.2
Error handling

Generic error handling for the operation‑related errors is described in clause 12, and the TC services which are used for reporting operation errors are described in clause 13.

10.5.4
Responding entity (CUSF)

10.5.4.1
Normal procedure

CUSF Preconditions:

1)
Bearer unrelated association processing has been suspended at a DP.

2)
CUSF-FSM is in the State b: Waiting for Instructions.

CUSF Postconditions:

1)
CUSF continues association establishment (connection-oriented bearer independent transport) to the specified destination using additional information from the SCF. A two‑party association is established.

2)
CUSF-FSM moves to State a: Idle, if monitoring is not required, or to State c: Monitoring, if monitoring of BCUSM events was requested in a previous operation.
3)
CUSF-FSM processing is resumed.
On receipt of this operation in the CUSF FSM State b: Waiting for Instructions, the CUSF performs following actions:

–
The CUSF cancels TCUSF. 

10.5.4.2
Error handling

Generic error handling for the operation‑related errors is described in clause 12, and the TC services which are used for reporting operation errors are described in clause 13. 
10.6
ContinueAssociation procedure

10.6.1
General description

This operation is used to request the CUSF to proceed with processing. Additional information which is not related to further association establishment may be provided by the SCF. The SCF shall send either a SendComponent or a ContinueAssociation operation to request the CUSF to continue the call unrelated processing from the suspended DP.

CUSF continues association establishment (connection‑oriented bearer independent transport) for the implied destination using any address information available in the BCUSM. A two-party or a one‑party association between an originating user/network application and a terminating user/network application is established depending on the location of the termination point for the concerned service application. For example, a one-party association may be established between the SCF representing the terminating ASE and a user/application in the network representing the originating ASE. The communication path toward user/network application is identified by leg ID.
10.6.2
Parameters

10.6.2.1 Argument parameters

The operation argument consists of the following parameter.

–
extensions
10.6.3
Invoking entity (SCF)

10.6.3.1
Normal procedure

SCF Preconditions:

1)
FSM for CUSF (SCSM-CUSF) is in State N2: Preparing CUSF Instructions. 

2)
The SLPI has determined that a "ContinueAssociation" has to be sent by the SCF.

SCF Postconditions:

1)
FSM for CUSF (SCSM-CUSF) moves to State N1: Idle, if monitoring is not required, or moves to State N2.2: Waiting for Notification or Request, if monitoring is required.

2)
FSM for CUSF (SCSM-CUSF) resumes processing from the current DP where processing was suspended. 

3)
If address information is available in the BCUSM to be used in further association establishment, a two-party association is set up, otherwise a one party association set up applies.
NOTE – The information provided by the SCF depends on the service ASE located in the CCF (BCUSM).

10.6.3.2
Error handling

Generic error handling for the operation‑related errors is described in clause 12, and the TC services which are used for reporting operation errors are described in clause 13.

10.6.4
Responding entity (CUSF)

10.6.4.1
Normal procedure

CUSF Preconditions:

1)
Bearer unrelated association processing has been suspended at a DP.

2)
CUSF-FSM is in State b: Waiting for Instructions.

CUSF Postconditions:

1)
CUSF continues association establishment (connection-oriented bearer independent transport).

2)
CUSF-FSM moves to the State a: Idle, if monitoring is not required, or to State c: Monitoring, if monitoring of BCUSM events was requested in a previous operation. 

On receipt of this operation in the CUSF FSM State b: Waiting for Instructions, the CUSF performs following actions:

–
The CUSF cancels TCUSF.

10.6.4.2
Error handling

Generic error handling for the operation‑related errors is described in clause 12, and the TC services which are used for reporting operation errors are described in clause 13.

10.7
EventReportBCUSM procedure

10.7.1
General description

This operation is used to notify the SCF of a call unrelated event previously requested by the SCF in an "RequestReportBCUSMEvent" operation. The monitoring of more than one event could be requested with a "RequestReportBCUSMEvent" operation, but each of these requested events is reported in a separate "EventReportBCUSM" operation. This operation may be available in the following two cases.

Case 1)
EDP is detected by receiving a FACILITY information element (a ROSE APDU) from the user (refer to 5.4 for modelling). In this case, component related parameters (componentType, component, and componentCorrelationID) are sent with this operation. The service ASE is located in the SCF, and the CUSF acts as a relay function between the user and the SCF. The SCF may send ROSE APDU information and receive further ROSE APDU information.
Case 2)
EDP is detected by receiving a request from a service ASE located in the CCF (BCUSM) indiating that it need further information to establish a bearer independent association. The CUSF acts as a relay function between the service ASE and the SCF. The SCF provides additional information for the association processing.
10.7.2
Parameters

10.7.2.1
Argument parameters

The operation argument consists of the following parameters. These parameters are defined in clause 11.

–
eventTypeBCUSM

–
eventSpecificInformationBCUSM

This parameter consists of the following alternatives:

•
bearerCap

or

•
tmr

–
miscCallInfo

The sub-parameter "dpAssignment" is not used in this operation.

•
messageType



When the messageType is "request", the EDP is reported as a request which means that the call unrelated processing is suspended for the SCF instructions. The message type for the EDP is requested by the RequestReportBCUSMEvent operation with monitorMode set to "interrupted".



When the messageType is "notification", the EDP is reported as a notification which means that the call unrelated processing is not suspended for the SCF instructions. The message type for the EDP is requested by the RequestReportBCUSMEvent operation with monitorMode set to "notifyAndContinue".

–
cUApplicationInd

–
legID

The alternative "receivingSideID" is only used in this operation. If this parameter is absent, the default value is assumed based on how the association has been initiated. In case of an SCF initiated association by InitiateAssociation operation, legID = 2, otherwise, legID = 1.

–
componentType

Refer to AssociationReleaseRequested operation procedure for the usage of this parameter.

–
componentCorrelationID
–
component
–
extensions

10.7.3
Invoking entity (CUSF)

10.7.3.1
Normal procedure

CUSF Preconditions:

1)
The CUSF FSM is in State c: Monitoring.

2)
The BCUSM proceeds to an EDP that is armed.

CUSF Postconditions:

1)
The CUSF FSM stays in State c: Monitoring, if the message type was notification and there are still EDPs armed.

2)
The CUSF FSM moves to State a: Idle, if the message type was notification and there are no more EDPs armed.

3)
The CUSF FSM moves to State b: Waiting for Instructions, if the message type was request. Bearer independent association processing is interrupted.

10.7.3.2
Error handling

In case the message type is request, on expiration of TCUSF before receiving any operation, the CUSF aborts the interaction with the SCF.

Operation-related error handling is not applicable, due to class 4 operation.

10.7.4
Responding entity (SCF)

10.7.4.1
Normal procedure

SCF Precondition:

1)
FSM for CUSF (SCSM-CUSF) is in State N2: Preparing CUSF Instructions, sub-state N2.2: Waiting for Notification or Request.

SCF Postconditions:

1)
FSM for CUSF (SCSM-CUSF) remains in the sub-state N2.2: Waiting for Notification or Request, if the message type was notification and there are still EDPs armed; or


FSM for CUSF (SCSM-CUSF) moves to the State N1: Idle, if the message type was notification and there are no more EDPs armed; or


FSM for CUSF (SCSM-CUSF) moves to sub-State N2.1: Preparing CUSF Instructions, if the message type was request.

2)
The event is reported to a SLPI, based on the dialogue ID. The SCF will prepare CUSF instructions in accordance with the SLPI.

10.7.4.2
Error handling

Operation-related error handling is not applicable, due to class 4 operation.

10.8
InitialAssociationDP procedure

10.8.1
General description

This operation is sent by the CUSF to the SCF after detecting a valid trigger condition at a BCUSM DP (reported as TDP). This operation may be available in the three cases described in 10.1.

10.8.2
Parameters

10.8.2.1
Argument parameters

The operation argument consists of the following parameters. These parameters are defined in clause 11.

–
serviceKey

–
cUApplicationInd

If this parameter exists, both uSIServiceIndicator and uSIInformation parameters shall not be used.
–
miscCallInfo
•
messageType

When the messageType is "request", the TDP is reported as a request which means that the call unrelated processing is suspended for the SCF instructions. 


When the messageType is "notification", the TDP is reported as a notification which means that the call unrelated processing is not suspended for the SCF instructions.

–
dPAssignment–eventTypeBCUSM
–
calledPartyNumber

–
callingPartyNumber

–
callingPartySubaddress

–
highlayerCompatibility

–
bearerCapability

–
uSIServiceIndicator

If this parameter exists, the cUApplicationInd parameter shall not be used.
–
uSIInformation

If this parameter exists, the cUApplicationInd parameter shall not be used.

–
genericNumbers

–
componentType

Refer to ActivationReceivedAndAuthorized procedure, when the ActivationReceived-AndAuthorized TDP is detected, and AssociationReleaseRequested operation procedure, when the other TDP is detected for the usage of this parameter.

–
componentCorrelationID
–
component
–
extensions

10.8.3
Invoking entity (CUSF)

10.8.3.1
Normal procedure

CUSF Preconditions:

1)
CUSF-FSM is in State a: Idle.

2)
The association has been established between the user and the network.

3)
An event fulfilling the criteria for the DP being executed has been detected.

CUSF Postconditions:

1)
CUSF-FSM moves to State b: Waiting for Instructions (TDP-R).

2)
CUSF-FSM remains in or moves to the State a: Idle (TDP-N).

NOTE – The information provided to the SCF depends on the service ASE located in the CUSF (case 1). 

If the DP was armed as a TDP-R, a control relationship is established to the SCF. The CUSF application timer TCUSF is set when the CUSF sends "InitialAssociationDP" for requesting instructions from the SCF. It is used to prevent excessive call suspension time. 

10.8.3.2
Error handling

Generic error handling for the operation‑related errors is described in clause 12, and the TC services which are used for reporting operation errors are described in clause 13.

10.8.4
Responding entity (SCF)

10.8.4.1
Normal procedure

SCF Precondition:

1)
FSM for CUSF (SCSM-CUSF) is in State N1: Idle.

SCF Postconditions:

1)
FSM for CUSF (SCSM-CUSF) moves to State N2.1: Preparing CUSF Instructions (TDP-R)

2)
FSM for CUSF (SCSM-CUSF) remains in State N1: Idle (TDP-N).

10.8.4.2
Error handling

Generic error handling for the operation‑related errors is described in clause 12, and the TC services which are used for reporting operation errors are described in clause 13.

10.9
InitiateAssociation procedure

10.9.1
General description

This operation is used to allow the SCF to initiate a call unrelated association with the user.

10.9.2
Parameters

10.9.2.1
Argument parameters

The operation argument consists of the following parameters. These parameters are defined in clause 11.

–
calledPartyNumber
–
extensions

–
uSIServiceIndicator
–
uSIInformation
–
genericNumbers

10.9.2.2
Result parameters

None.

10.9.3
Invoking entity (SCF)

10.9.3.1
Normal procedure

SCF Preconditions:

1)
FSM for CUSF (SCSM-CUSF) is in State N1: Idle.

2)
SLPI requests to initiate a call unrelated association with the user.

SCF Postcondition:

1)
FSM for CUSF (SCSM-CUSF) prepares to send a component or USI information to the user and goes to State N2: Preparing CUSF Instructions.
The SLPI shall send either a "SendComponent" or a "ContinueAssociation" operation to request the CCF/CUSF to continue the call unrelated processing from the suspended DP.
10.9.3.2
Error handling

Generic error handling for the operation‑related errors is described in clause 12, and the TC services which are used for reporting operation errors are described in clause 13.

10.9.4
Responding entity (CUSF)

10.9.4.1
Normal procedure

CUSF Precondition:

1)
CUSF-FSM is in State a: Idle.

CUSF Postconditions:

1)
CUSF-FSM goes to State b: Waiting for Instructions.
2)
A Return Result is sent.
The BCUSM is instantiated and suspended at the ActivationReceivedAndAuthorized DP. The CUSF is waiting for subsequent instructions from the SCF.

10.9.4.2
Error handling

Generic error handling for the operation‑related errors is described in clause 12, and the TC services which are used for reporting operation errors are described in clause 13.

10.10
ReleaseAssociation procedure

10.10.1
General description

This operation is used by the SCF to release an existing association.

10.10.2
Parameters

10.10.2.1
Argument parameters

The operation argument consists of the following parameter. This parameter is defined in clause 11.

–
cause

This parameter indicates the reason why the SCF releases the association and may be used by CUSF to fill in the "cause" in the association releasing message.
10.10.3
Invoking entity (SCF)

10.10.3.1
Normal procedure

SCF Preconditions:

1)
FSM for CUSF (SCSM-CUSF) is in State N2: Preparing CUSF Instructions.

2)
SLPI requests to release the association between the user and the network.

SCF Postcondition:

1)
FSM for CUSF (SCSM-CUSF) moves to State N1: Idle.

10.10.3.2
Error handling

Operation-related error handling is not applicable, due to class 4 operation.

10.10.4
Responding entity (CUSF)

10.10.4.1
Normal procedure

CUSF Precondition:

1)
CUSF-FSM is in State b: Waiting for Instructions.

CUSF Postconditions:

1)
CUSF releases the association with association release message which may contain previously specified component (by SendComponent) to the user.

2)
CUSF-FSM moves to State a: Idle.

10.10.4.2
Error handling

Operation-related error handling is not applicable, due to class 4 operation.

10.11
ReportUTSI procedure

10.11.1
General description

This operation is used to notify the SCF of a USI previously requested by the SCF in a RequestReportUTSI operation.

NOTE – Refer to ITU-T Q.1248.1 for a definition of the term "User" in the context of the OCCUUI mechanism.

A User to Service Information (USI) refers to either a UTSI or an STUI. 

10.11.2
Parameters

10.11.2.1
Argument parameters

The operation argument consists of the following parameters. These parameters are defined in clause 11.

–
uSIServiceIndicator
–
legID
–
uSIInformation

–
extensions
10.11.3
Invoking entity (CUSF)

10.11.3.1
Normal procedure

CUSF preconditions:

1)
The CUSF FSM is in any state except "Idle".

2)
The CUSF_USI FSM is in the state "Monitoring USI IE".

CUSF Postconditions:

1)
The CUSF FSM remains in the same state.

2)
The CUSF_USI FSM remains in the same state.

10.11.3.2
Error handling

Operation-related error handling is not applicable, due to class 4 operation.

10.11.4
Responding entity (SCF)

10.11.4.1
Normal procedure

The procedure is the same as that of the corresponding operation procedure in ITU-T Q.1248.2 by replacing the "SSF" with the "CUSF".
10.11.4.2
Error handling

Operation-related error handling is not applicable, due to class 4 operation.
10.12
RequestReportBCUSMEvent procedure

10.12.1
General description

This operation requests the CUSF to report the reception of the ROSE APDU from the user (optionally a ROSE APDU information pattern, e.g. FACILITY IE pattern, is specified as a DP criteria) or requests the CUSF to monitor for call unrelated events matching the requested criteria.
Table 5/Q.1248.7 – DP arming table for BCUSM
	BCUSM
	Controlling
leg
	Passive
leg

	Activation_Received_And_Authorized DP
	–
	–

	Component_Received DP 
	X
	X

	Association_Release_Requested DP 
	X
	X

	X
Arming applicable
–
Not Applicable
NOTE – Only applicable as TDP, because first DP that can be encountered cannot be armed as EDP.


10.12.2
Parameters

10.12.2.1
Argument parameters

The operation argument consists of the following parameters. These parameters are defined in clause 11.

–
bCUSMEvents
–
componentTypes


This parameter indicates the one or more types of component that will be monitored. In case a specific componentType is provided, this operation is used to request the CUSF to report the reception of invocation of an operation or a result from the side of the association, which is indicated by the legID sub-parameter, to the SCF. The requesting event(s) can be either an invocation, the return result, return error, or reject from the indicated side. For example, "return result" is used to request the CUSF reporting the result of the previous invocation of an operation from the SCF.
–
componentCorrelationID
–
monitorDuration
–
extensions

–
cUDPCriteria
–
legID
The alternative "sendingSideID" is only used in this operation. If this parameter is absent, the default value is assumed based on how the association has been initiated. In case of an SCF initiated association by InitiateAssociation operation, legID = 2, otherwise, legID = 1.

10.12.3
Invoking entity (SCF)

10.12.3.1
Normal procedure

SCF Precondition:

1)
FSM for CUSF (SCSM-CUSF) is in State N2.1: preparing CUSF instructions.

2)
SLPI requests to monitor the reception of a component form the user or requests to monitor for call unrelated events matching the requested criteria.

SCF Postconditions:

1)
FSM for CUSF (SCSM-CUSF) remains in the same State N2.1: preparing CUSF instructions.

10.12.3.2
Error handling

Generic error handling for the operation‑related errors is described in clause 12, and the TC services which are used for reporting operation errors are described in clause 13.

10.12.4
Responding entity (CUSF)

10.12.4.1
Normal procedure

CUSF Precondition:

1)
CUSF-FSM is in State b: Waiting for Instructions.

CUSF Postconditions:

1)
CUSF starts the monitoring process for the specified event(s) or clears the armed EDP(s).

2)
CUSF-FSM remains in the same state.
3)
Requested events are monitored until the EDPs are detected or until the monitor duration is elapsed.
10.12.4.2
Error handling

Generic error handling for the operation‑related errors is described in clause 12, and the TC services which are used for reporting operation errors are described in clause 13.

10.13
RequestReportUTSI procedure

10.13.1
General description

This operation is used to request the CUSF to monitor for the receipt of a USI IE with a given ServiceIndicator value, then send this USI IE back to the SCF when this IE is received.

NOTE – Refer to ITU-T Q.1248.1 for a definition of the term "User" in the context of the OCCUUI mechanism.

A User to Service Information (USI) IE refers to either a UTSI IE or an STUI IE. 

10.13.2
Parameters

10.13.2.1
Argument parameters

The operation argument consists of the following parameters. These parameters are defined in clause 11.

–
requestedUTSIList

–
extensions

–
legID

10.13.3
Invoking entity (SCF)

10.13.3.1
Normal procedure

The procedure is the same as that of the corresponding operation procedure in ITU-T Q.1248.2 by replacing the "SSF" with the "CUSF".
10.13.3.2
Error handling

Generic error handling for the operation-related errors is described in clause 12, and the TC services which are used for reporting operation errors are described in clause 13.

10.13.4
Responding entity (CUSF)

10.13.4.1
Normal procedure

CUSF Preconditions:

1)
The CUSF FSM is any state except "Idle".

2)
The CUSF_USI FSM is in any state.

CUSF Postconditions:

1)
The CUSF FSM remains in the same state.

2)
The CUSF_USI FSM moves to the state "Monitoring USI" (if the USIMonitorMode is "monitoringActive") or to the state "Idle" (if the USIMonitorMode is "monitoringInactive").

10.13.4.2
Error handling

Generic error handling for the operation-related errors is described in clause 12, and the TC services which are used for reporting operation errors are described in clause 13.

10.14
SendComponent procedure

10.14.1
General description

This operation requests the CUSF to send the specified FACILITY information to the user.

10.14.2
Parameters

10.14.2.1
Argument parameters

The operation argument consists of the following parameters. These parameters are defined in clause 11.

–
componentType

This parameter indicates the type of component that will be delivered to the user. For example, "return result" is used to report the result of the previous invocation of an operation from a user.

–
componentCorrelationID
–
message
–
monitorDuration
–
extensions

–
component

This parameter indicates the component that will be delivered to the user.

10.14.3
Invoking entity (SCF)

10.14.3.1
Normal Procedure

SCF Preconditions:

1)
FSM for CUSF (SCSM-CUSF) is in State N2: Preparing CUSF Instructions.

2)
SLPI requests to send the component to the user.

SCF Postcondition:

1)
FSM for CUSF (SCSM-CUSF) remains in State N2: Preparing CUSF Instructions if monitoring is not required or moves to State N2.2: Waiting for Notification or Request if monitoring is required.

10.14.3.2
Error handling

Generic error handling for the operation-related errors is described in clause 12, and the TC services which are used for reporting operation errors are described in clause 13.

10.14.4
Responding entity (CUSF)

10.14.4.1
Normal procedure

CUSF Precondition:

1)
CUSF-FSM is in State b: Waiting for Instructions.

CUSF Postconditions:

1)
CUSF send specified component to the user with appropriate message, but if the association release message is specified to send the component, it will be cued until the association release request received.

2)
CUSF-FSM remains in State b: Waiting for Instructions if an EDP is armed or moves to the State c: Monitoring.
10.14.4.2
Error handling

Generic error handling for the operation‑related errors is described in clause 12, and the TC services which are used for reporting operation errors are described in clause 13.
10.15
SendSTUI procedure

10.15.1
General description

This operation is used to request the CUSF to forward an STUI with a given ServiceIndicator value to the User (indicated by leg ID).
NOTE – Refer to ITU-T Q.1248.1 for a definition of the term "User" in the context of the OCCUUI mechanism.

10.15.2
Parameters

10.15.2.1
Argument parameters

The operation argument consists of the following parameters. These parameters are defined in clause 11.

–
uSIServiceIndicator

–
legID
–
uSIInformation

–
extensions

10.15.3
Invoking entity (SCF)

10.15.3.1
Normal procedure

The procedure is the same as that of the corresponding operation procedure in ITU-T Q.1248.2 by replacing the "SSF" with the "CUSF".
10.15.3.2
Error handling

Generic error handling for the operation-related errors is described in clause 12, and the TC services which are used for reporting operation errors are described in clause 13.
10.15.4
Responding entity (CUSF)

10.15.4.1
Normal procedure

CUSF Preconditions:

1)
The CUSF FSM is any state except "Idle".

2)
The CUSF_USI FSM is in any state.

CUSF Postconditions:

1)
The CUSF FSM remains in the same state.

2)
The CUSF_USI FSM remains in the same state.

On receipt of this operation, the CUSF will forward the STUI IE to the User (identified by the LegID).

10.15.4.2
Error handling

Generic error handling for the operation-related errors is described in clause 12, and the TC services which are used for reporting operation errors are described in clause 13.
10.16
Operation timers 
Table 6 lists all operation timers invoked in SCF-CUSF relationship and the value range for each timer. The definitive value for each operation timer may be network specific and has to be defined by the network operator.

The following value range does apply for operation specific timers:

short:
1-10 seconds

Table 6/Q.1248.7 – Operation timers and their value range

	Operation Name
	Timer
	value range

	ActivityTest
	Tat
	short

	activationReceivedAndAuthorized
	Tara
	short

	associationReleaseRequested
	Tarr
	short

	ComponentReceived
	Tcre
	short

	ConnectAssociation
	Tcoa
	short

	continueAssociation
	Tcona
	short

	EventReportBCUSM
	Terbce
	short

	initialAssociationDP
	Tiadp
	short

	InitiateAssociation
	Tia
	short

	ReleaseAssociation
	Trel
	short

	requestReportBCUSMEvent
	Trrbce
	short

	RequestReportUTSI
	Trru
	short

	ReportUTSI
	Tru
	short

	SendSTUI
	Tss
	short

	SendComponent
	Tsdc
	short


11
Parameters 
This clause defines the parameters used in the operations procedures as specified in clause 10.

11.1
Address
This parameter contains the called party number to be used in the further association establishment (connection-oriented bearer independent transport). Refer to ITU-T Q.762 called party number for the actual definition of this parameter.
11.2
BCUSMEvents

This parameter specifies a list of (1 to n) requested BCUSM events and indicates how and which DP should be reported (DP name and report mode). Each BCUSM event comprises the following sub-parameters:

–
eventTypeBCUSM


This sub-parameter is identical to the EventTypeBCUSM parameter.

–
monitorMode


This sub-parameter is defined in ITU-T Q.1248.2.

11.3
BearerCapability

This parameter indicates the type of the bearer capability required for connection or the transmission medium requirements to the user. It comprises the following alternative sub‑parameters and is a network option to select one of them to be used.

–
bearerCap

This alternative specifies the value of the DSS1 Bearer Capability parameter (ITU-T Q.931) in case the CUSF is at local exchange level or the value received in a TC message.


The parameter "bearerCapability" is included in an "InitialAssociationDP" or a DP specific operation in case the Bearer Capability parameter is available.


If two values for bearer capability are available at the CUSF or if User Service Information and User Service Information Prime are available at the CUSF, the "bearerCap" shall contain the value of the preferred bearer capability or the value of the User Service Information Prime parameter respectively.

or

–
tmr

For a definition of this sub-parameter, refer to ITU-T Q.1248.2.
Depending on the parameters of the feature specific operation, the BearerCapability can be retrieved from the operation, which builds up the association (in case of CCBS the BearerCapability is a mandatory part of the CCBS request).

11.4
CalledPartyNumber

This parameter contains the number used to identify the called party in the forward direction. Refer to ITU-T Q.762 for the actual definition of this parameter.

The CalledPartyNumber is available in the DSS1 IE (local exchange only) or can be retrieved from the feature specific operation, which builds up the association (e.g. in case of CCBS CalledNumber is a mandatory part of the CCBS request).

11.5
CallingPartyNumber

This parameter contains the address of the calling party. Refer to ITU-T Q.762 for the actual definition of this parameter.

The CallingPartyNumber is available in the DSS1 IE (local exchange only) or can be retrieved from the feature specific operation, which builds up the association (e.g. in case of CCBS CallingPartyNumber is an optional part of the CCBS request).
11.6
CallingPartySubaddress

This parameter contains the calling party subaddress provided either as a result of DSS1 or in a bearer unrelated (TC) signalling. Refer to ITU-T Q.931 for the actual definition of this parameter.

The CallingPartySubaddress is available in the DSS1 IE (local exchange only) or can be retrieved from the feature specific operation, which builds up the association (e.g. in case of CCBS CallingPartySubaddress is part of the optional parameter AccessTransport of the CCBS request).

This parameter contains information that may not have been checked by the network for coding errors.

11.7
CallUnrelatedDpSpecificCommonParameters

This parameter indicates the call unrelated DP specific information. It comprises the following sub‑parameters:

–
serviceAddressInformation

This sub-parameter indicates the trigger related information. It comprises the following sub‑parameters:
•
serviceKey

This sub-parameter is identical to the ServiceKey parameter.

•
miscCallInfo

This sub-parameter is identical to the MiscCallInfo parameter.

•
triggerType


This sub-parameter indicates to the SCF the particular event which caused the detection of a valid trigger condition. For a definition of this sub-parameter, refer to ITU-T Q.1248.2.
–
callingPartyNumber 

This sub-parameter is identical to the CallingPartyNumber parameter.

–
locationNumber 

This sub-parameter indicates the Location Number for the calling party. Refer to ITU-T Q.762 for the actual definition of this parameter

–
terminalType

This is defined in ITU-T Q.1248.2. The absence of this parameter means that the terminal type is "ISDN".

–
uSIServiceIndicator

This sub-parameter is identical to the USIServiceIndicator parameter.

–
uSIInformation

This sub-parameter is identical to the USIInformation parameter.

–
cUApplicationInd


This sub-parameter is identical to the CUApplicationInd parameter.

–
calledPartyNumber


This sub-parameter is identical to the CalledPartyNumber parameter.

–
callingPartySubaddress


This sub-parameter is identical to the CallingPartySubaddress parameter.

–
highLayerCompatibility


This sub-parameter is identical to the HighLayerCompatibility parameter.

–
bearerCapability


This sub-parameter is identical to the BearerCapability parameter.

–
genericNumbers


This sub-parameter is identical to the GenericNumbers parameter.

11.8
Cause

This parameter indicates the reason of releasing the specific association. Refer to ITU-T Q.762 for the actual definition of this parameter.

11.9
Component

This parameter is derived from the UNI APDU or mapped to the UNI APDU depending on the transferring direction of this parameter (CUSF to SCF for the former and SCF to CUSF for the latter). The following alternatives are possible according to the data type, and one of them should be chosen.

Case 1)
component data type is an OCTETSTRING. It contains the operation value (object identifier) error value, etc. within the UNI APDU, and in addition also contains the parameter set/sequence for the operation invocation/return result or return error/reject on UNI. Refer to ITU-T Q.932 for the actual definition of this parameter. 

Case 2)
component data type is an EMBEDDED-PDV. In this case, componentType and componentCorrelationID shall not be used. 

11.10
ComponentCorrelationID

This parameter links the invoke ID that is used by the CUSF and the SCF with the invoke ID assigned between the user and the network locally. The value indicated in this parameter is used to correlate the response from the SCF (which will be done by SendComponent) within the CUSF.

11.11
ComponentType

This parameter indicates the type of ROSE component (invoke, return result, return error, and reject). This is derived from/to the UNI APDU or used for indicating the monitor condition.
11.12
ComponentTypes

This parameter specifies a list of (1 to n) requested component types. The component type is identical to the ComponentType parameter.

11.13
CUApplicationInd

This parameter identifies the triggered application (case service ASE located in the CCF (BCUSM)). It is derived CUSF internally. This parameter shall indicate the operation code of the triggered application specific operation. Two types of values shall be supported: global values for standardized applications and local values for non-standardized applications.
11.14
CUDPCriteria
This parameter identifies the EDP criteria for the requested DP. This EDP criteria may correspond to the parameter component or cUApplicationInd in the "InitialAssociationDP" or DP specific operations.
When this parameter is provided, ComponentType and ComponentCorrelationID may not be used.
11.15
EventSpecificInformationBCUSM

This parameter indicates the call unrelated information according to the detected EDP. It comprises the following alternative sub-parameters:

–
componentReceivedSpecificInfo


This sub-parameter specifies the Component_Received EDP specific information. It comprises the following sub-parameter:

•
componentReceivedInfo


This sub-parameter indicates additional information about the component, but not the component itself.
or

–
associationReleaseRequestedSpecificInfo


This sub-parameter specifies the Association_Release_Requested EDP specific information. It comprises the following sub-parameters:

•
associationReleaseInfo


This sub-parameter indicates additional information about the component, but not the component itself.
•
releaseCause


This sub-parameter indicates the reason why the association is released. Refer to ITU-T Q.762 for the actual definition of this parameter.

11.16
EventTypeBCUSM

This parameter specifies the type of BCUSM DP event that is reported. 

11.17
GenericNumbers

This parameter indicates one or more additional numbers and specifies a list of (1 to n) generic numbers. Refer to ITU-T Q.762 for the actual definition of each generic number.

Depending on the parameters of the feature specific operation the GenericNumber can be retrieved from the operation, which builds up the association.

11.18
HighLayerCompatibility

This parameter indicates the type of the high layer compatibility, which will be used to determine the ISDN – teleservice of a connected ISDN terminal. Refer to ITU-T Q.931 for the actual definition of this parameter.
The HighLayerCompatibility is available in the DSS1 IE (local exchange only) or can be retrieved from the feature specific operation, which builds up the association (e.g. in case of CCBS the HighLayerCompatibility is part of the optional parameter AccessTransport of the CCBS request).

11.19
LegID
This parameter indicates the side of the association for which an event is being reported or is to be monitored. It comprises the following alternative sub-parameters:

–
sendingSideID


This alternative indicates the leg ID assigned by the side which sends the operation.

or

–
receivingSideID


This alternative indicates the leg ID assigned by the side which receives the operation.

The CUSF uses the "receivingSideID" alternative only. The SCF uses the "sendingSideID" alternative only. 

The leg numbering is based on the following principles:
–
LegID = 1 is the controlling leg

–
LegID = 2 is the passive leg

When a TDP is detected, the controlling leg corresponds to the initiating side of the association and the passive leg corresponds to the responding side. The outgoing signalling dialogue with the responding side is initiated by a ConnectAssociation or ContinueAssociation operation.

When an InitiateAssociation operation is used, the passive leg represents the responding side of the association. There is no controlling leg.

When the LegID parameter is absent from one of the following operations:
–
AssociationReleaseRequested

–
ComponentReceived

–
EventReportBCUSM

–
RequestReportBCUSMEvent

A default value is assumed based on how the association has been initiated. In case of an SCF initiated association by InitiateAssociation operation, legID = 2 is assumed, otherwise, legID = 1 is assumed.
11.20
Message

This parameter specifies the message (release, release complete, or facility) for delivering the component to the user.
11.21
MiscCallInfo

This parameter indicates the DP related information. It comprises the following sub-parameters:

–
messageType 


This sub-parameter indicates whether the message is a "request" or a "notification".
–
dPAssignment

For a definition of this sub-parameter, refer to ITU-T Q.1248.2.

11.22
MonitorDuration
This parameter indicates how long the CUSF should monitor the component transmission result.
11.23
RequestedUTSIList

For a definition of this parameter, refer to ITU-T Q.1248.2. 

11.24
ServiceKey

For a definition of this parameter, refer to ITU-T Q.1248.2.

11.25
USIInformation

For a definition of this parameter, refer to ITU-T Q.1248.2. It is transparent at the CCF/CUSF level.
11.26
USIServiceIndicator

For a definition of this parameter, refer to ITU-T Q.1248.2. It is used as a monitoring criterion at the CCF/CUSF level.
12
Error procedures
This clause defines the error procedures for the CUSF-SCF interface. Error descriptions are provided in ITU-T 1248.1 and the following subclauses provide operation-related error procedures and, when relevant, error procedures related to error conditions which are not directly related to the failure of an operation.

12.1
Operation-related error procedures

The following subclauses define the generic error handling for the operation-related error procedures on the CUSF-SCF interface. The errors are defined as operation errors in the ASN.1 operations related description. The TC services which are used for reporting operations errors are described in clause 13.

Table 7 provides the list of operations which may return each of the errors used on the CUSF-SCF interface.
Table 7/Q.1248.7 – Available errors for each operation

	Errors
	SCF(CUSF
	
	CUSF(SCF

	
	InitiateAssociation
	RequestReportBCUSMEvent
	SendComponent
	ConnectAssociation
	ContinueAssociation
	RequestReportUTSI
	SendSTUi
	
	ActivationReceivedAndAuthorized
	AssociationReleaseRequested
	ComponentReceived
	InitialAssociationDP

	MissingCustomerRecord
	
	
	
	
	
	
	
	
	X
	X
	X
	X

	MissingParameter
	X
	X
	X
	X
	X
	X
	X
	
	X
	X
	X
	X

	ParameterOutOfRange
	X
	X
	X
	X
	X
	X
	X
	
	X
	X
	X
	X

	SystemFailure
	X
	X
	X
	X
	X
	X
	X
	
	X
	X
	X
	X

	TaskRefused
	X
	X
	X
	X
	X
	X
	X
	
	X
	X
	X
	X

	UnexpectedComponentSequence
	X
	X
	X
	X
	X
	X
	X
	
	X
	X
	X
	X

	UnexpectedDataValue
	X
	X
	X
	X
	X
	X
	X
	
	X
	X
	X
	X

	UnexpectedParameter
	X
	X
	X
	X
	X
	X
	X
	
	X
	X
	X
	X

	UnknownLegID
	
	X
	
	
	
	
	X
	
	
	X
	X
	

	NOTE – If the inconsistency exists, ASN.1 definition takes precedence.


12.1.1
MissingCustomerRecord

The MissingCustomerRecord error is defined in ITU-T Q.1248.1.

12.1.1.1
Operations CUSF(SCF

This clause describes the procedure when the error for an operation invoked from the CUSF occurs in the SCF.

Relevant operations are described in Table 7.

12.1.1.1.1
Procedures at the invoking entity (CUSF)

a)
Sending Operation

Precondition:
refer to the relevant "Operation procedures CUSF – precondition" clause

Postcondition:
refer to the relevant "Operation procedures CUSF – postcondition" clause
b)
Receiving Error

Precondition:
refer to the relevant "Operation procedures CUSF – postcondition" clause

Postcondition:
CUSF FSM State a: Idle
The CUSF continues to handle the association or terminate the association with default procedures (network operator specific).
12.1.1.1.2
Procedures at the responding entity (SCF)

Precondition:
refer to the relevant "Operation procedures SCF – precondition" clause
Postcondition:
FSM for CUSF State N1: Idle 
The FSM for CUSF detects that the required Service Logic Program does not exist. This is the same situation as for the SSF-SCF case.

12.1.2
MissingParameter

The MissingParameter error is defined in ITU-T Q.1248.1.

12.1.2.1
Operations SCF(CUSF

This clause describes the procedure when the error for an operation invoked from the SCF occurs in the CUSF.

Relevant operations are described in Table 7.

12.1.2.1.1
Procedures at the invoking entity (SCF)

a)
Sending Operation

Precondition:
refer to the relevant "Operation procedures SCF – precondition" clause

Postcondition:
refer to the relevant "Operation procedures SCF – postcondition" clause
b)
Receiving Error

Precondition:
refer to the relevant "Operation procedures SCF – postcondition" clause
Postcondition:
remain in the same state as the operation was sent
The Service Logic and maintenance functions are informed. Further treatment of the call unrelated processing is dependent on Service Logic.

12.1.2.1.2
Procedures at the responding entity (CUSF)

Precondition:
refer to the relevant "Operation procedures CUSF – precondition" clause

Postcondition:
remain in the same state
The CUSF FSM detects the error in the received operation. 

12.1.2.2
Operations CUSF(SCF 
This clause describes the procedure when the error for an operation invoked from the CUSF occurs in the SCF.

Relevant operations are described in Table 7.

12.1.2.2.1
Procedures at the invoking entity (CUSF)
a)
Sending Operation

Precondition:
refer to the relevant "Operation procedures CUSF – precondition" clause

Postcondition:
refer to the relevant "Operation procedures CUSF – postcondition" clause
b)
Receiving Error

Precondition:
refer to the relevant "Operation procedures CUSF – postcondition" clause 
Postcondition:
CUSF FSM State a: Idle
The CUSF terminates the association if necessary. If the supplementary service is already active and ready for responding, the CUSF may maintain the association and continue service processing. The choice between these two options is network operator specific.

12.1.2.2.2
Procedures at the responding entity (SCF)

Precondition:
refer to the relevant "Operation procedures SCF – precondition" clause

Postcondition:
FSM for CUSF State N1: Idle 
The FSM for CUSF detects the erroneous situation. The Service Logic and maintenance functions are informed.
12.1.3
ParameterOutOfRange

The ParameterOutOfRange error is defined in ITU-T Q.1248.1.
This error procedure is the same as the MissingParameter error procedure.
12.1.4
SystemFailure

The SystemFailure error is defined in ITU-T Q.1248.1.
This error procedure is the same as the MissingParameter error procedure.
12.1.5
TaskRefused

The TaskRefused error is defined in ITU-T Q.1248.1.
This error procedure is the same as the MissingParameter error procedure.
12.1.6
UnexpectedComponentSequence

The UnexpectedComponentSequence error is defined in ITU-T Q.1248.1.
This error procedure is the same as the MissingParameter error procedure.
12.1.7
UnexpectedDataValue

The UnexpectedDataValue error is defined in ITU-T Q.1248.1.
This error procedure is the same as the MissingParameter error procedure.
12.1.8
UnexpectedParameter

The UnexpectedParameter error is defined in ITU-T Q.1248.1.
This error procedure is the same as the MissingParameter error procedure.
12.1.9
UnknownLegID

The UnknownLegID error is defined in ITU-T Q.1248.1.
This error procedure is the same as the MissingParameter error procedure.
12.2
Entity-related error procedures

The following subclauses define the error handling for the entity-related errors. Since the error situations are not originated by the reception of an operation, the invoking entity is denoted here as the entity at which the error situation is detected. The responding entity is the entity which receives the error report.

The TCAP services used for reporting errors are described in ITU-T Q.1248.1.

12.2.1
Expiration of TCUSF
12.2.1.1
Error description

This error occurs in the CUSF because of an expiration of an application timer TCUSF.

12.2.1.2
Procedures at the invoking entity (CUSF)

Timeout occurs in CUSF for TCUSF.

Precondition:
CUSF FSM State b: Waiting for Instructions.

Postcondition:
CUSF FSM State a: Idle.

The CUSF FSM aborts the dialogue and moves to the Idle state, and the CUSF terminates the association if necessary (e.g. default exception handling). The abort is reported to the maintenance functions.

12.2.1.3
Procedures at the responding entity (SCF)

SCF receives a dialogue abort.

Precondition:
Any state.

Postcondition:
FSM for CUSF State N1: Idle.

The SCF releases all allocated resources and reports the abort to the maintenance functions.

13
Service assumed by TCAP

13.1
Normal procedures

13.1.1
CUSF-to-SCF messages

13.1.1.1
CUSF-FSM-related messages

A dialogue shall be established when the CUSF-FSM moves from the state Idle to the state Waiting for Instructions. The relevant INAP operation, which is a DP specific operation or an InitialAssociationDP for TDP-R or an InitiateAssociation operation, shall be transmitted in the same message.

No dialogue shall be established when the CUSF-FSM moves from the state Idle and back to the state Idle on the detection of TDP-N. The relevant INAP operation, which is a DP specific operation or an InitialAssociationDP for TDP-N, shall be sent with a TC-BEGIN request primitive and the dialogue is locally ended by means of TC-END request primitive with prearranged end.

For all other operations sent from the CUSF-FSM, the dialogue shall be maintained except for the following cases.

When the CUSF sends the last event report operation, the dialogue may be ended from the CUSF by a TC-END request primitive with basic end.
In the case that there is no pending operation and TCAP dialogue is established, TCAP dialogue can be terminated by TC-END primitive with zero component or prearranged end. When the CUSF‑FSM makes a non-error case state transition to the state Idle and there is no operation to be sent, the dialogue is ended by means of a TC-END request primitive (basic) with zero components, or the dialogue is locally ended by means of a TC-END request primitive with prearranged end. The CUSF can end a dialogue with a TC-END request primitive with zero component or prearranged end depending on that TCAP dialogue is established or not, in the case association release between the user and the network is initiated by any other entity. 

When the CUSF has sent the last event report operation, the dialogue may be ended from the SCF by a TC-END request primitive with basic end.
13.1.1.2
CUSME-related messages

The dialogue shall be maintained when the ActivityTest Return Result is sent.

13.1.2
SCF-to-CUSF messages

13.1.2.1
SCSM-FSM-related messages

No dialogue shall be established when the SCSM-FSM moves from state Idle to state Idle upon receipt of a DP specific operation or an InitialAssociationDP operation for TDP-N. The operation is received with a TC-BEGIN indication primitive and the dialogue is locally terminated by means of a TC-END request primitive with prearranged end.

A dialogue shall be established when the SCSM-FSM moves from state Idle to state Preparing CUSF Instructions upon the receipt of a DP specific operation or an InitialAssociationDP operation for TDP-R, or upon the sending of an InitiateAssociation operation.

For subsequent operations sent from the SCSM-FSM, the dialogue shall be maintained except for the following cases, i.e. all other operations are sent after a dialogue was established from the CUSF (the SCF has previously received a TC-BEGIN indication primitive with one of operations for TDP‑R).

The dialogue shall no longer be maintained when the prearranged end condition is met in the SCF. When the SCF does not expect any messages other than possibly REJECT or ERROR messages for the operations sent and when the last associated operation timer expires, the dialogue is locally ended by means of a TC-END request primitive with prearranged end.

Alternatively, the sending of operations, leading to the termination of the relationship, by means of a TC-END request primitive (basic end) is possible. 

13.1.2.2
SCME-FSM related messages

The operation(s) sent from the SCME-FSM shall be issued according to the following procedure(s):

–
The dialogue shall be maintained when the ActivityTest operation is sent.

13.2
Abnormal procedures
The procedures for SCF-CUSF are the same as that for SCF-SSF. Refer to ITU-T Q.1248.2.

13.2.1
Dialogue handling
Refer to the general descriptions in ITU-T Q.1248.1.

13.2.1.1
Dialogue establishment
Refer to the general descriptions in ITU-T Q.1248.1.

13.2.1.2
Dialogue continuation
Refer to the general descriptions in ITU-T Q.1248.1.

13.2.1.3
Dialogue termination
Refer to the general descriptions in ITU-T Q.1248.1.

13.2.1.4
User abort
Refer to the general descriptions in ITU-T Q.1248.1.

13.2.1.5
Provider abort
Refer to the general descriptions in ITU-T Q.1248.1.

13.2.1.6
Mapping to TC dialogue primitives

The CUSF-SCF IN services can be mapped onto TC services. This clause defines the mapping of the CUSF-SCF IN services onto the services of the TC dialogue handling services defined in ITU-T Q.771.

The mapping of the parameters onto the TC-BEGIN primitive is defined in ITU-T Q.1248.1 with the following qualifications:

–
The Application Context Name parameter shall take the value of the application-context-name field of the cusfscfDPSpecificAC or cusfscfGenericAC object if the originating AE is a CUSF, or the scfcusfDPSpecificAC or scfcusfGenericAC object if the originating AE is an SCF.

13.2.2
Component handling

13.2.2.1
Procedures for INAP operations
Refer to the general descriptions in ITU-T Q.1248.1.

13.2.2.2
Mapping to TC component parameters

The mapping of parameters for the TC component services is defined in ITU-T Q.1248.1 with the following qualifications.

The Timeout Parameter of the TC-INVOKE service is set according to the requirements set out in the SCF-CUSF interface section.

ANNEX A
ASN.1 definitions

Annex A is available as an electronic attachment.
	SERIES OF ITU-T RECOMMENDATIONS

	Series A
	Organization of the work of ITU-T

	Series B
	Means of expression: definitions, symbols, classification

	Series C
	General telecommunication statistics

	Series D
	General tariff principles

	Series E
	Overall network operation, telephone service, service operation and human factors

	Series F
	Non-telephone telecommunication services

	Series G
	Transmission systems and media, digital systems and networks

	Series H
	Audiovisual and multimedia systems

	Series I
	Integrated services digital network

	Series J
	Cable networks and transmission of television, sound programme and other multimedia signals

	Series K
	Protection against interference

	Series L
	Construction, installation and protection of cables and other elements of outside plant

	Series M
	TMN and network maintenance: international transmission systems, telephone circuits, telegraphy, facsimile and leased circuits

	Series N
	Maintenance: international sound programme and television transmission circuits

	Series O
	Specifications of measuring equipment

	Series P
	Telephone transmission quality, telephone installations, local line networks

	Series Q
	Switching and signalling

	Series R
	Telegraph transmission

	Series S
	Telegraph services terminal equipment

	Series T
	Terminals for telematic services

	Series U
	Telegraph switching

	Series V
	Data communication over the telephone network

	Series X
	Data networks and open system communications

	Series Y
	Global information infrastructure and Internet protocol aspects

	Series Z
	Languages and general software aspects for telecommunication systems

	
	


� 	This Recommendation includes an electronic attachment containing Annex A, ASN.1 definitions.





�PAGE \# "'Page: '#'�'"  �Page: 2���A:	Basic Non�Call Manager (BNCM)


�PAGE \# "'Page: '#'�'"  �Page: 2���A:	IN – Non-Switching Manager (IN-NSM)


�PAGE \# "'Page: '#'�'"  �Page: 2���A:	Feature Interaction Manager/Non-Call Manager (FIM/NCM)


�PAGE \# "'Page: '#'�'"  �Page: 3���A:	Basic Call Unrelated State Model (BCUSM)


�PAGE \# "'Page: '#'�'"  �Page: 3���A:	Points in association (PIA), detection points (DPs).


�PAGE \# "'Page: '#'�'"  �Page: 9���A:	Trigger DP (TDP)


�PAGE \# "'Page: '#'�'"  �Page: 10���A:	CUSF Management Entity (CUSME)


�PAGE \# "'Page: '#'�'"  �Page: 10���A:	Functional Entity Access Manager (FEAM)


�PAGE \# "'Page: '#'�'"  �Page: 10���A:	CUSF Finite State Model (FSM)


�PAGE \# "'Page: '#'�'"  �Page: 19���A:	Out-Channel Call-Unrelated User Interaction (OCCUUI)


�PAGE \# "'Page: '#'�'"  �Page: 19���A:	User to Service Information (USI)





