
	[image: image1.wmf]
	INTERNATIONAL  TELECOMMUNICATION  UNION

	
	

	ITU-T
	Q.1912.3

	TELECOMMUNICATION
STANDARDIZATION  SECTOR
OF  ITU
	(07/2001)   

	
	SERIES Q: SWITCHING AND SIGNALLING

Specifications of signalling related to Bearer Independent Call Control (BICC)



	
	Interworking between H.323 and the Bearer Independent Call Control protocol

	
	ITU‑T  Recommendation  Q.1912.3

(Formerly  CCITT  Recommendation)


ITU-T Q-SERIES  RECOMMENDATIONS

SWITCHING AND SIGNALLING

	
	

	SIGNALLING IN THE INTERNATIONAL MANUAL SERVICE
	Q.1–Q.3

	INTERNATIONAL AUTOMATIC AND SEMI-AUTOMATIC WORKING
	Q.4–Q.59

	FUNCTIONS AND INFORMATION FLOWS FOR SERVICES IN THE ISDN
	Q.60–Q.99

	CLAUSES APPLICABLE TO ITU-T STANDARD SYSTEMS
	Q.100–Q.119

	SPECIFICATIONS OF SIGNALLING SYSTEMS No. 4 AND No. 5
	Q.120–Q.249

	SPECIFICATIONS OF SIGNALLING SYSTEM No. 6
	Q.250–Q.309

	SPECIFICATIONS OF SIGNALLING SYSTEM R1
	Q.310–Q.399

	SPECIFICATIONS OF SIGNALLING SYSTEM R2
	Q.400–Q.499

	DIGITAL EXCHANGES
	Q.500–Q.599

	INTERWORKING OF SIGNALLING SYSTEMS
	Q.600–Q.699

	SPECIFICATIONS OF SIGNALLING SYSTEM No. 7
	Q.700–Q.799

	Q3 INTERFACE
	Q.800–Q.849

	DIGITAL SUBSCRIBER SIGNALLING SYSTEM No. 1
	Q.850–Q.999

	PUBLIC LAND MOBILE NETWORK
	Q.1000–Q.1099

	INTERWORKING WITH SATELLITE MOBILE SYSTEMS
	Q.1100–Q.1199

	INTELLIGENT NETWORK
	Q.1200–Q.1699

	SIGNALLING REQUIREMENTS AND PROTOCOLS FOR IMT-2000
	Q.1700–Q.1799

	SPECIFICATIONS OF SIGNALLING RELATED TO BEARER INDEPENDENT CALL CONTROL (BICC)
	Q.1900–Q.1999

	BROADBAND ISDN
	Q.2000–Q.2999


For further details, please refer to the list of ITU-T Recommendations.

	ITU-T Recommendation Q.1912.3

Interworking between H.323 and the Bearer Independent Call Control protocol




	Summary

This Recommendation defines the signalling interworking between the Bearer Independent Call Control (BICC) protocol and H.323. It defines in particular the signalling interworking between the BICC protocol and the H.225.0 Multimedia Call Control protocol based on the interworking between H.225.0 and the ISDN User Part (ISUP) protocol of Signalling System No. 7.


	Source

ITU‑T Recommendation Q.1912.3 was prepared by ITU‑T Study Group 11 (2001‑2004) and approved under the WTSA Resolution 1 procedure on 2 July 2001.


	


FOREWORD

The International Telecommunication Union (ITU) is the United Nations specialized agency in the field of tele​com​mu​ni​ca​tions. The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of ITU. ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Assembly (WTSA), which meets every four years, establishes the topics for study by the ITU‑T study groups, which, in turn, produce Recommendations on these topics.

The approval of ITU-T Recommendations is covered by the procedure laid down in WTSA Resolution 1.

In some areas of information technology which fall within ITU-T's purview, the necessary standards are prepared on a collaborative basis with ISO and IEC.

NOTE

In this Recommendation, the expression "Administration" is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

INTELLECTUAL PROPERTY RIGHTS
ITU draws attention to the possibility that the practice or implementation of this Recommendation may involve the use of a claimed Intellectual Property Right. ITU takes no position concerning the evidence, validity or applicability of claimed Intellectual Property Rights, whether asserted by ITU members or others outside of the Recommendation development process.

As of the date of approval of this Recommendation, ITU had not received notice of intellectual property, protected by patents, which may be required to implement this Recommendation. However, implementors are cautioned that this may not represent the latest information and are therefore strongly urged to consult the TSB patent database.

(  ITU  2002

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from ITU.

CONTENTS


Page
1
Scope

1
2
References

2
3
Definitions

3
4
Abbreviations

3
5
General considerations

3
6
Interworking considerations

4
6.1
General

4
6.2
Prevention of speech clipping

4
6.3
Codec negotiation and out-of-band transport of DTMF and tone information

4
6.4
Interworking of ISDN supplementary services

4
Appendix I ( Guidelines BICC/H.323 interworking for end-to-end codec negotiation

5
I.1
Introduction

5
I.2
H.323 to BICC interworking

5
I.2.1
If the H.245 procedures are used (with a separate H.245 control channel or using the H.245 encapsulation method)

5
I.2.2
If Fast Connect procedure is used

6
I.3
BICC to H.323 interworking

6
I.3.1
If H.245 procedures are used (with a separate H.245 control channel or using the H.245 encapsulation method)

7
I.3.2
If Fast Connect procedure is used

7
ITU-T Recommendation Q.1912.3

Interworking between H.323 and the Bearer Independent Call Control protocol

1
Scope

This Recommendation defines the signalling interworking between the Bearer Independent Call Control (BICC) protocol and H.323.

It defines in particular the signalling interworking between the BICC protocol and the H.225.0 Multimedia Call Control protocol in an H.323 network based on the interworking between H.225.0 and the ISDN User Part (ISUP) protocol of Signalling System No. 7.

The interworking between H.245 and BICC is for further study (some aspects are addressed in 6.3 and Appendix I).

BICC is the protocol defined in ITU-T Recommendations Q.1902.1 to Q.1902.4 [5]. The Multimedia Call Control protocol is defined in ITU-T H.225.0 [1] for a network in accordance with ITU-T  H.323 [3]. ISUP is the protocol defined in ITU-T Recommendations Q.761 to Q.764 [4].

The description of the signalling interworking between the H.225.0 Multimedia Call Control protocol and the BICC protocol in this Recommendation builds upon:

(
The signalling interworking between the H.225.0 Multimedia Call Control protocol and the ISDN User Part (ISUP) protocol of Signalling System No. 7 as defined in Annex C/H.246 [2].

(
The signalling interworking between the Bearer Independent Call Control (BICC) protocol and the ISDN User Part (ISUP) protocol of Signalling System No. 7 as defined in ITU-T Q.1912.1 [6].

The scope of this Recommendation with regard to the signalling interworking between the H.225.0 Multimedia Call Control protocol and the BICC protocol is as shown in Figure 1.

[image: image2.wmf]T11111930-01

CSF-N

ISN

BICC

BCF-N

BIWF

BICC

H.225.0

H.245

Bearer control

Scope of 

Annex C/H.246

Scope of this

Recommendation

Scope of 

ITU-T 

Q.1912.1

H.323

call/bearer 

control

N-ISUP 

call/bearer 

control

BICC network using separate

call and bearer signalling

H.323

 network


Figure 1/Q.1912.3 ( Scope of this Recommendation for BICC/H.225.0 interworking

2
References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

[1]
ITU-T H.225.0 (2000), Call signalling protocols and media stream packetization for packet-based multimedia communication systems.
[2]
ITU-T H.246 Annex C (2000), ISDN user part function – H.225.0 Interworking.
[3]
ITU-T H.323 (2000), Packet-based multimedia communications systems.
[4]
ITU-T Q.761 to Q.764 (1999), Specifications of Signalling System No. 7( ISDN user part (ISUP).
[5]
ITU-T Q.1902.1 to Q.1902.4 (2001), Specifications of the Bearer Independent Call Control protocol (BICC).
[6]
ITU-T Q.1912.1 (2001), Interworking between Signalling System No. 7 ISDN user part (ISUP) and the Bearer Independent Call Control protocol.
[7]
ITU-T H.450.x-family (1998 onwards), Procedures and the signalling protocols between H.323 entities for the control of supplementary services.
[8]
ITU-T Implementors Guide (2001) for H.323, H.225.0, H.245, H.246, H.283, H.235, H.450‑series and H.341 Recommendations.

[9]
ITU-T H.245 (2001), Control protocol for multimedia communication.
3
Definitions

For BICC specific terminology, the reader is referred to ITU-T Q.1902.1 [5].

4
Abbreviations

This Recommendation uses the following abbreviations:

APM

Application Transport Message

BCF-N

Bearer Control Nodal Function

BICC

Bearer Independent Call Control

BIWF

Bearer InterWorking Function

CSF-N

Call Service Nodal Function

DTMF

Dual Tone Multi Frequency

ISDN

Integrated Services Digital Network

ISN

Interface Serving Node

ISUP

ISDN User Part

SCN

Switched Circuit Network

5
General considerations

This clause describes the interworking principle between H.225.0 and BICC based on the interworking between H.225.0 and ISUP concatenated with the interworking between ISUP and BICC. This refers to the necessary mapping of signalling information to achieve connectivity and functionality between an H.323 network and a BICC network. Figure 2 provides the functional grouping of the interworking functions involved following the half call model description technique for BICC.


[image: image3.wmf]T11111940-01

CSF-N 

BICC

Q.1912.1

H.225.0

Half call

Half call

Annex C/H.246 


Figure 2/Q.1912.3 ( Functional description of the interworking configuration

6
Interworking considerations

6.1
General

The interworking between H.225.0 and BICC shall act according to the interworking between H.225.0 and ISUP in Annex C/H.246 [3], and the additional interworking statements in the Implementors Guide for H.323, H.225.0, H.245, H.246, H.283, H.235, H.450‑series and H.341 Recommendations in ITU-T Implementors Guide [8], concatenated with the interworking between ISUP and BICC in ITU-T Q.1912.1 [6].

6.2
Prevention of speech clipping

For an incoming H.323 call for which the incoming bearer is not established yet in the H.323 network, the Continuity Indicator in the Nature of Connection Indicators parameter in the IAM in BICC shall be set to indicate "COT to be expected" if the ISN is aware that the call establishment procedures in ITU-T H.245 [9] can take place in the H.323 network without waiting until an ACM message is received in BICC (Note).

The Continuity message, with the Continuity Indicators parameter set to "Continuity" shall be sent in BICC upon successful completion of the incoming H.323 bearer set-up procedures.

NOTE – In some H.323 implementations, the H.245 call establishment procedures cannot be initiated before an ACM is received from the BICC network (because the H.245 procedures are not handled until a CONNECT is received or because a CALL PROCEEDING message is not generated before an ACM is received). In such cases, speech clipping cannot be prevented. H.323 networks using such implementations should not be interconnected to BICC networks. The H.245 call establishment procedures shall be initiated as soon as possible on the H.323 side.

If the Fast Connect procedure is used on the H.323 side, the ISN shall first check that the bearer establishment can be successfully completed according to the proposals received in the fastStart element of the SETUP message received from the H.323 side, before sending the IAM in BICC.

6.3
Codec negotiation and out-of-band transport of DTMF and tone information

Interworking procedures related to codec negotiation (including mid-call codec modification and mid-call codec negotiation procedures) and out-of-band transport of DTMF and tone information have also to be considered in case of interworking between an H.323 network and a BICC network. These procedures require the definition of interactions between the BICC protocol and some parts of the H.245 protocol. The accurate definition of these procedures is for further study. Appendix I provides guidelines for the implementation of the end-to-end codec negotiation procedures through a BICC and H.323 network.

6.4
Interworking of ISDN supplementary services

Please note that the interworking of ISDN supplementary services (ITU-T H.450.x-series [7]) is for further study.

Appendix I

Guidelines BICC/H.323 interworking for end-to-end codec negotiation

I.1
Introduction 

The H.245 capability exchange procedures are more complex than the BICC codec negotiation procedures since H.323 nodes have the possibility to separately indicate receive and transmit capabilities, as well as various combination of modes in which the node can operate simultaneously. A different codec can be selected for each unidirectional media channel in H.323.

As the same codec is used for transmission and reception in a BICC network, only the codecs which are proposed by the H.323 network for reception and transmission will be included in the initial BICC Supported Codecs list used to perform the BICC codec negotiation. This principle is generally applied in the procedures defined below although it is not explicitly mentioned.

I.2
 USERADDRESS  \* MERGEFORMAT H.323 to BICC interworking

	H.323
network
	BICC
network

	Node A
	ISN B
	Node C


I.2.1
If the H.245 procedures are used (with a separate H.245 control channel or using the H.245 encapsulation method)

1)
If the capabilities supported by the preceding H.323 node (node A) are received by the ISN (in an H.245 TerminalCapabilitySet message) before the IAM is sent on the outgoing side, the Supported Codec List sent in the IAM is derived from the capability structures contained in the received TerminalCapabilitySet message by deleting the codecs that cannot be used for the call (not offered by the ISN) and adding (with a lower priority) the codecs that can be used for the call (proposed by the ISN) but were not indicated in the received capabilities from the preceding H.323 node (node A). 


The ISN shall wait until an APM with the selected codec is received before indicating its capabilities to the preceding H.323 node (node A). 

(
If the Selected Codec received in the APM was contained in the received TerminalCapabilitySet message from the preceding H.323 node (node A), the H.245 TerminalCapabilitySet message sent by the ISN shall only contain the selected codec received in the BICC APM. The end-to-end codec negotiation procedures are successfully completed. The codec selected by the ISN on the H.323 side (sent in the OpenLogicalChannel message) is the codec resulting from the BICC negotiation procedure.

(
If the Selected Codec received in the APM does not belong to the capabilities indicating by the preceding H.323 node (node A), a transcoder shall be used.

2)
If the capabilities supported by the preceding H.323 node (node A) are not received by the ISN (in an H.245 TerminalCapabilitySet message) before the IAM is sent on the outgoing side, the Supported Codec List is constructed in the ISN as described in ITU-T Q.1902.4 (SN initiating codec negotiation). The ISN waits until the codec negotiation procedure is completed on the BICC side before indicating its capabilities to the preceding node (node A).


When the APM with the selected codec is received by the ISN:

(
The ISN sends a TerminalCapabilitySet message containing the codecs that can be used for the call (codecs supported by the ISN), allocating the highest priority to the BICC selected codec and the other codecs of the Available codec List.

(
If the codec selected by the preceding H.323 node (node A) (received in the H.245 OpenLogicalChannel message) is the BICC Selected codec, end-to-end negotiation procedures are successfully completed. The codec selected by the ISN on the H.323 side (sent in the OpenLogicalChannel message) is the same as the codec selected by the preceding H.323 node.

(
If the codec selected by the preceding H.323 node (node A) (received in the H.245 OpenLogicalChannel message) is not the BICC Selected codec but belongs to the BICC Available codec List, the BICC Modification procedure shall be applied on the BICC side: the newly selected codec is the codec selected by node A and the new Available codec List is the old one from which the codecs not supported by node A are deleted. The codec selected by the ISN on the H.323 side (sent in the OpenLogicalChannel message) is the same as the codec selected by the preceding H.323 node.

(
If the codec selected by the preceding H.323 node (node A) does not belong to the BICC Available codec List, a transcoder shall be used.

I.2.2
If Fast Connect procedure is used

The Supported Codec List sent in the IAM is derived from the OpenLogicalChannel proposals received in the fastStart element of the Setup message by deleting the codecs which are not supported for the call (the codecs are listed in the same priority order as received in the fastStart element) and adding (with a lower priority) the codecs that can be used for the call but were not proposed in the received fastStart element.

The backward APM with the Selected codec is awaited before the fastStart element is sent back by the ISN.

(
If the Selected Codec received in the APM was proposed in the received fastStart element, the ISN sends a fastStart element selecting amongst the OpenLogicalChannel proposals containing the Selected codec received in the APM. The end-to-end codec negotiation is successfully completed.

(
If the Selected Codec received in the APM was not proposed in the received fastStart element, a transcoder shall be used.

I.3
 USERADDRESS  \* MERGEFORMAT BICC to H.323 interworking

	BICC
network
	H.323
network

	Node A
	ISN B
	Node C


I.3.1
If H.245 procedures are used (with a separate H.245 control channel or using the H.245 encapsulation method)

It may not be possible to wait until the capabilities of node C are received before selecting the codec on the BICC side: H.245 procedures may only begin in the H.323 network when a H.225.0 CONNECT message is sent (when an answer is received from the called party) whereas the BICC codec negotiation procedures must be completed before the ACM is sent. It is therefore preferable to complete the BICC codec negotiation procedures independently of the H.323 procedures on the outgoing side.

(
The ISN waits until a TerminalCapabilitySet message is received from the succeeding H.323 node.

(
If the BICC Selected codec is supported by the succeeding H.323 node (node C), end‑to‑end negotiation procedures are successfully completed. The ISN sends a TerminalCapabilitySet containing the BICC selected codec only. The OpenLogicalChannel message which will be sent by the ISN shall contain this same BICC selected codec. The end-to-end codec negotiation procedures are successfully completed.

(
If the BICC selected codec does not belong to the list of codecs supported by the succeeding H.323 node, the ISN selects the codec with the highest priority in the BICC Available Codecs List which is also supported by the succeeding H.323 node. The ISN sends a TerminalCapabilitySet containing the BICC selected codec only. The OpenLogicalChannel message which will be sent by the ISN shall contain this same BICC selected codec. The BICC modification procedure is applied on the BICC side: the newly selected codec is the codec selected by the ISN and the new Available codec List is the old one from which the codecs not supported by the succeeding H.323 (node C) are deleted.

(
If no codec of the BICC Available Codecs List is supported by the succeeding H.323 node, a transcoder shall be used.
I.3.2
If Fast Connect procedure is used

The OpenLogicalChannel proposals sent in the fastStart element of the SETUP message contains the codecs that can be used for the call (supported by the ISN). The BICC Selected codec and the codecs of the BICC Available codecs List resulting from the BICC negotiation procedure are proposed with the highest priority.

(
If the codec selected by the succeeding H.323 node (node C) (received in the selected OpenLogicalChannel indicated in the fastStart element) is the BICC Selected codec, end‑to-end negotiation procedures are successfully completed.

(
If the codec selected by the succeeding H.323 node (node C) (received in the OpenLogicalChannel contained in the fastStart element) is not the BICC Selected codec but belongs to the BICC Available codec List, the BICC Modification procedure shall be applied on the BICC side: the newly selected codec is the codec selected by the succeeding H.323 node (node C) and the new Available codec List is the old one from which the codecs which were proposed with a higher priority than the H.323 Selected codec (codecs not supported by the succeeding H.323 node (node C)) are deleted.

(
If the codec selected by the succeeding H.323 node (node C) does not belong to the BICC Available Codecs List, a transcoder shall be used.

	SERIES OF ITU-T RECOMMENDATIONS

	Series A
	Organization of the work of ITU-T

	Series B
	Means of expression: definitions, symbols, classification

	Series C
	General telecommunication statistics

	Series D
	General tariff principles

	Series E
	Overall network operation, telephone service, service operation and human factors

	Series F
	Non-telephone telecommunication services

	Series G
	Transmission systems and media, digital systems and networks

	Series H
	Audiovisual and multimedia systems

	Series I
	Integrated services digital network

	Series J
	Cable networks and transmission of television, sound programme and other multimedia signals

	Series K
	Protection against interference

	Series L
	Construction, installation and protection of cables and other elements of outside plant

	Series M
	TMN and network maintenance: international transmission systems, telephone circuits, telegraphy, facsimile and leased circuits

	Series N
	Maintenance: international sound programme and television transmission circuits

	Series O
	Specifications of measuring equipment

	Series P
	Telephone transmission quality, telephone installations, local line networks

	Series Q
	Switching and signalling

	Series R
	Telegraph transmission

	Series S
	Telegraph services terminal equipment

	Series T
	Terminals for telematic services

	Series U
	Telegraph switching

	Series V
	Data communication over the telephone network

	Series X
	Data networks and open system communications

	Series Y
	Global information infrastructure and Internet protocol aspects

	Series Z
	Languages and general software aspects for telecommunication systems

	
	
























































































































































































































































































































































































































































































































































































































































































































































_1079350696.unknown

