
	[image: image1.wmf]
	INTERNATIONAL  TELECOMMUNICATION  UNION

	
	

	ITU-T
	Q.2150.0

	TELECOMMUNICATION
STANDARDIZATION  SECTOR
OF  ITU
	(05/2001)   

	
	SERIES Q: SWITCHING AND SIGNALLING

Broadband ISDN – Signalling ATM adaptation layer (SAAL)



	
	Generic Signalling Transport Service

	
	ITU‑T  Recommendation  Q.2150.0

(Formerly  CCITT  Recommendation)


ITU-T Q-SERIES  RECOMMENDATIONS

SWITCHING AND SIGNALLING

	
	

	SIGNALLING IN THE INTERNATIONAL MANUAL SERVICE
	Q.1–Q.3

	INTERNATIONAL AUTOMATIC AND SEMI-AUTOMATIC WORKING
	Q.4–Q.59

	FUNCTIONS AND INFORMATION FLOWS FOR SERVICES IN THE ISDN
	Q.60–Q.99

	CLAUSES APPLICABLE TO ITU-T STANDARD SYSTEMS
	Q.100–Q.119

	SPECIFICATIONS OF SIGNALLING SYSTEMS No. 4 AND No. 5
	Q.120–Q.249

	SPECIFICATIONS OF SIGNALLING SYSTEM No. 6
	Q.250–Q.309

	SPECIFICATIONS OF SIGNALLING SYSTEM R1
	Q.310–Q.399

	SPECIFICATIONS OF SIGNALLING SYSTEM R2
	Q.400–Q.499

	DIGITAL EXCHANGES
	Q.500–Q.599

	INTERWORKING OF SIGNALLING SYSTEMS
	Q.600–Q.699

	SPECIFICATIONS OF SIGNALLING SYSTEM No. 7
	Q.700–Q.799

	Q3 INTERFACE
	Q.800–Q.849

	DIGITAL SUBSCRIBER SIGNALLING SYSTEM No. 1
	Q.850–Q.999

	PUBLIC LAND MOBILE NETWORK
	Q.1000–Q.1099

	INTERWORKING WITH SATELLITE MOBILE SYSTEMS
	Q.1100–Q.1199

	INTELLIGENT NETWORK
	Q.1200–Q.1699

	SIGNALLING REQUIREMENTS AND PROTOCOLS FOR IMT-2000
	Q.1700–Q.1799

	SPECIFICATIONS OF SIGNALLING RELATED TO BEARER INDEPENDENT CALL CONTROL (BICC)
	Q.1900–Q.1999

	BROADBAND ISDN
	Q.2000–Q.2999

	General aspects
	Q.2000–Q.2099

	Signalling ATM adaptation layer (SAAL)
	Q.2100–Q.2199

	Signalling network protocols
	Q.2200–Q.2299

	Common aspects of B-ISDN application protocols for access signalling and network signalling and interworking
	Q.2600–Q.2699

	B-ISDN application protocols for the network signalling
	Q.2700–Q.2899

	B-ISDN application protocols for access signalling
	Q.2900–Q.2999

	
	


For further details, please refer to the list of ITU-T Recommendations.

	ITU-T Recommendation Q.2150.0

Generic Signalling Transport Service




	Summary

This Recommendation specifies the Generic Signalling Transport Service that allows the development of signalling specifications without considerations of the idiosyncrasies of the underlying signalling transport mechanisms.


	Source

ITU‑T Recommendation Q.2150.0 was prepared by ITU‑T Study Group 11 (2001‑2004) and approved under the WTSA Resolution 1 procedure on 15 May 2001.


	Keywords

Asynchronous Transfer Mode (ATM), ATM Adaptation Layer (AAL), Bearer Independent Call Control (BICC), Broadband Integrated Services Network (B-ISDN), Message Transfer Part (MTP), Signalling AAL (SAAL), Signalling Transport Converter (STC).


FOREWORD

The International Telecommunication Union (ITU) is the United Nations specialized agency in the field of tele​com​mu​ni​ca​tions. The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of ITU. ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Assembly (WTSA), which meets every four years, establishes the topics for study by the ITU‑T study groups which, in turn, produce Recommendations on these topics.

The approval of ITU-T Recommendations is covered by the procedure laid down in WTSA Resolution 1.

In some areas of information technology which fall within ITU-T's purview, the necessary standards are prepared on a collaborative basis with ISO and IEC.

NOTE

In this Recommendation, the expression "Administration" is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

INTELLECTUAL PROPERTY RIGHTS
ITU draws attention to the possibility that the practice or implementation of this Recommendation may involve the use of a claimed Intellectual Property Right. ITU takes no position concerning the evidence, validity or applicability of claimed Intellectual Property Rights, whether asserted by ITU members or others outside of the Recommendation development process.

As of the date of approval of this Recommendation, ITU had not received notice of intellectual property, protected by patents, which may be required to implement this Recommendation. However, implementors are cautioned that this may not represent the latest information and are therefore strongly urged to consult the TSB patent database.

(  ITU  2001

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from ITU.

CONTENTS


Page

1
Scope

1
2
References

1
2.1
Normative references

1
2.2
Bibliography

1
3
Definitions

2
4
Abbreviations

2
5
Framework of the Generic Signalling Transport Service

3
5.1
General framework

3
5.2
Conventions

4
6
Service definition

4
6.1
Primitives between signalling entities and the Generic Signalling Transport

4
6.2
Parameters

5
6.3
Establishment

6
6.4
State transition diagram for sequences of primitives of the Generic Signalling Transport service

6
ITU-T Recommendation Q.2150.0

Generic Signalling Transport Service

1
Scope

This Recommendation describes the Generic Signalling Transport Service. It allows the definition of signalling specifications without considerations of the idiosyncrasies of the underlying signalling transport mechanisms.

This Recommendation also describes the interface states and the definition of the service by a number of primitives. The Generic Signalling Transport Service can be deployed by means of Signalling Transport Converters over a range of signalling transport protocol stacks.

NOTE – Signalling Transport Converters are defined, for example, in ITU-T Q.2150.1 [6], ITU‑T Q.2150.2 [7], or ITU-T Q.2150.3 [8].

This Recommendation is based on the requirements defined in ITU-T Q-series Supplement 8 (1999), Technical Report TRQ.2400 [3] "Transport control signalling requirements ( Signalling requirements for AAL type 2 link control capability set 1", in ITU-T Q-series Supplement 33 (2000), Technical Report TRQ.2401 [4] "Signalling requirements for Q.AAL2 capability set 2", and in ITU‑T Q-series Supplement 38 (2001), Technical Report TRQ.2600 [5] "BICC signalling transport requirements, capability set 1".

2
References

2.1
Normative references

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of currently valid ITU-T Recommendations is regularly published.

[1]
ITU-T X.200 (1994), Information technology ( Open Systems Interconnection ( Basic Reference Model: The basic model.

[2]
ITU-T X.210 (1993), Information technology ( Open Systems Interconnection ( Basic Reference Model: Conventions for the definition of OSI services.

2.2
Bibliography

The following ITU-T Recommendations and other documents contain information that may be useful to understanding the usage of this Recommendation. There are no additional provisions of this Recommendation derived from these documents.

[3]
ITU-T Q-series Supplement 8 (1999), Technical Report TRQ.2400: Transport control signalling requirements (  Signalling requirements for AAL type 2 link control capability set 1.

[4]
ITU-T Q-series Supplement 33 (2000), Technical Report TRQ.2401: Signalling requirements for Q.AAL2 capability set 2.

[5]
ITU-T Q-series Supplement 38 (2001), Technical Report TRQ.2600: BICC signalling requirements, capability set 1.

[6]
ITU-T Q.2150.1 (2001), Signalling transport converter on MTP3 and MTP3b.

[7]
ITU-T Q.2150.2 (2001), Signalling transport converter on SSCOP and SSCOPMCE.

[8]
ITU-T Q.2150.3 (2001), Signalling transport converter on SCTP.

[9]
ITU-T Q.704 (1996), Signalling network functions and messages.

[10]
ITU-T Q.2210 (1996), Message transfer part level 3 functions and messages using the services of ITU-T Recommendation Q.2140.

[11]
ITU-T Q.2110 (1994), B-ISDN ATM adaptation layer (  Service specific connection oriented protocol (SSCOP).

[12]
ITU-T Q.2111 (1999), B-ISDN ATM adaptation layer (   Service specific connection oriented protocol in a multi-link and connectionless environment (SSCOPMCE).

[13]
IETF RFC 2960 (2000), Stream Control Transmission Protocol.
3
Definitions

This Recommendation is based upon the concepts developed in ITU-T X.200 [1] and ITU‑T X.210 [2].

In addition, for the purpose of this Recommendation, the following definitions apply:

3.1
generic signalling transport service: The function that enables a signalling entity to communicate with a peer signal​ling entity independently of the underlying signalling transport.

3.2
signalling transport: A signalling link or network that connects two signalling entities.

3.3
signalling transport converter: A function that converts the services provided by a particular Signalling Transport to the services required by the Generic Signalling Transport Service.

4
Abbreviations

This Recommendation uses the following abbreviations:

AAL
ATM Adaptation Layer

GST
Generic Signalling Transport

LM
Layer Management

MTP3
Message Transfer Part level 3

MTP3b
Message Transfer Part level 3 using Q.2140

SAAL
Signalling ATM Adaptation Layer

SAP
Service Access Point

SAR
Segmentation and Reassembly (Sublayer)

SCTP
Stream Control Transmission Protocol

SSCOP
Service Specific Connection Oriented Protocol

SSCOPMCE
Service Specific Connection Oriented Protocol in a Multilink and Connectionless Environment

SSCS
Service Specific Convergence Sublayer

STC
Signalling Transport Converter

SVC
Switched Virtual Channel

TED
Transmission Error Detection

VCC
Virtual Channel Connection

VPC
Virtual Path Connection

5
Framework of the Generic Signalling Transport Service

5.1
General framework

A signalling protocol entity makes use of the Generic Signalling Transport service which is provided by the Signalling Transport Converter (for example, see ITU-T Q.2150.1 [6], ITU-T Q.2150.2 [7], or ITU-T Q.2150.3 [8]). The Generic Signalling Transport enables a signalling entity to communicate with a peer signalling entity independently of the underlying signalling transports.

This framework is illustrated in Figure 5-1.


[image: image2.wmf]T11112720-01

SAP

SAP

Generic Signalling

Transport Service

Signalling Application

Primitives

Specific Signalling

Transport Service

Signalling Transport Converter

Specific Signalling Transport

(this Recommendation)

Primitives


Figure 5-1/Q.2150.0 – Framework for the Generic Signalling Transport Service

Signalling protocols can be deployed over a range of signalling transport protocol stacks. Two peer signalling entities rely on the Generic Signalling Transport service to provide assured data transfer between them and service availability indications; i.e., signalling messages are exchanged between peer protocol entities using the Generic Signalling Transport Service.

Example protocol stacks are shown in Figure 5-2.


[image: image3.wmf]T11112730-01

MTP3

Q.704

MTP3b

Q.2210

SSCOP

Q.2110

SSCOPMCE

Q.2111

SCTP

RFC 2960

Signalling Transport Converter

on MTP3 and MTP3b

Q.2150.1

Primitives

Generic Signalling

Transport Service

Primitives

Primitives

Primitives

Primitives

Primitives

Primitives

Primitives

STC

on SCTP

Q.2150.3

Signalling Transport Converter

on SSCOP

Q.2150.2


Figure 5-2/Q.2150.0 – Example protocol stacks for the Generic Signalling Transport Service

5.2
Conventions

This Recommendation specifies the information flow across the Signalling Transport Converter ( signalling entity boundary. Conceptually, there exists one Signalling Transport Converter entity per signalling association. A signalling entity transfers or receives signalling messages on a particular signalling association by utilizing a particular SAP.
6
Service definition

6.1
Primitives between signalling entities and the Generic Signalling Transport

The services are provided through the transfer of primitives that are summarized in Table 6‑1, and are defined as follows:

a)
IN-SERVICE.indication

This primitive indicates that the signalling transport is able to exchange signalling messages with the peer entity. This indication shall be provided without the signalling entity requesting any service across the SAP.

b)
OUT-OF-SERVICE.indication

This primitive indicates that the signalling transport is unable to exchange signalling messages with the peer entity. This indication shall be provided without the signalling entity requesting any service across the SAP.

c)
TRANSFER.request

This primitive is used by the signalling entity to convey a signalling message to its peer entity.

d)
TRANSFER.indication

This primitive provides a signalling message from the peer entity to the signalling entity.

e)
CONGESTION.indication

This primitive is used to convey information concerning signalling network congestion.


NOTE – Some Signalling Transport services may not issue the CONGESTION.indication primitive.

f)
START-INFO.indication

This primitive indicates at start-up to the signalling entity the maximum length of an SDU that the Signalling Transport Converter can transfer and whether this signalling entity is the controlling node of the call association.

Table 6-1/Q.2150.0 – Primitives and parameters of the Generic Signalling Transport sublayer

	Primitive
Generic Name
	Type

	
	Request
	Indication
	Response
	Confirm

	START-INFO
	–
	Max_Length
CIC_Control
	–
	–

	IN-SERVICE
	–
	Level
	–
	–

	OUT-OF-SERVICE
	–
	(Note 1)
	–
	–

	CONGESTION
	–
	Level
	–
	–

	TRANSFER
	Sequence Control
STC User Data
Priority (Note 2)
	
STC User Data
Priority (Note 2)
	–
	–

	(
This primitive is not defined.

NOTE 1 – This primitive has no parameters.

NOTE 2 – This parameter is a national option (and the use of this parameter is not supported by all signalling transports).


6.2
Parameters

a)
CIC_Control


This parameter indicates to the signalling entity whether it serves as the controlling entity on this signalling association.

b)
Level


This parameter indicates the level of congestion. This variable can hold the values from a level indicating "no congestion" through to a level indicating "maximum congestion" in increments.


NOTE – The values for indication "no congestion" and "maximum congestion" as well as the number of steps of congestion level and/or amount of increase/decrease are considered to be network implementation dependent.

c)
Max_Length


This parameter indicates the maximum length of signalling messages that can be transported on this signalling association.


NOTE 1 – The length indicated is a characterization of the underlying signalling transport's length limitations and ( in case of MTP transport ( includes the MTP header (see ITU-T Q.704 [9] and ITU‑T Q.2210 [10]). In particular:

•
the value "272" is used to indicate interworking with MTP3 signalling transports (see ITU-T Q.704 [9]);

•
the value "4096" is used to indicate interworking with MTP3b signalling transports (see ITU-T Q.2210 [10]); and

•
the value "65 328" is used to indicate interworking with SSCOP signalling transports (see ITU-T Q.2110 [11] and ITU-T Q.2111 [12]).


NOTE 2 – The signalling transport based on the Stream Control Transmission Protocol (SCTP, see RFC 2960 [13]) has no length limitations.

d)
Priority


This parameter indicates the priority of the signalling message.


NOTE 3 – This parameter is a national option.

e)
Sequence Control


This parameter indicates to the Signalling Transport Converter a value that can be used by the underlying signalling transport for load sharing and/or in-sequence delivery. Signalling messages accompanied by the same Sequence Control value shall be delivered in-sequence.

f)
STC User Data


This parameter contains a complete signalling message; it represents the STC SDU.

6.3
Establishment

On the establishment of a signalling transport and the associated Signalling Transport Converter entity, for example at power-up, the initial conditions are the same as if an OUT‑OF‑SERVICE.indication had been conveyed across this SAP. Also at this time the START‑INFO.indication is sent to the signalling entity.

6.4
State transition diagram for sequences of primitives of the Generic Signalling Transport service

This clause defines the constraints on the sequences in which the primitives may occur at the layer boundaries of the Generic Signalling Transport service. The sequences are related to the states at one Generic Signalling Transport endpoint between the Generic Signalling Transport service provider and its user. The possible overall sequences of primitives are shown in the state transition diagram, Figure 6-1.


[image: image4.wmf]T11112740-01

1

2

TRANSFER.request

IN-SERVICE.indication

OUT-OF-SERVICE.indication

TRANSFER.indication

TRANSFER.request

Out of

Service

In

Service

TRANSFER.indication

CONGESTION.indication

Start

START-INFO.indication


Figure 6-1/Q.2150.0 – State transition diagram for sequences of primitives

	SERIES OF ITU-T RECOMMENDATIONS

	Series A
	Organization of the work of ITU-T

	Series B
	Means of expression: definitions, symbols, classification

	Series C
	General telecommunication statistics

	Series D
	General tariff principles

	Series E
	Overall network operation, telephone service, service operation and human factors

	Series F
	Non-telephone telecommunication services

	Series G
	Transmission systems and media, digital systems and networks

	Series H
	Audiovisual and multimedia systems

	Series I
	Integrated services digital network

	Series J
	Cable networks and transmission of television, sound programme and other multimedia signals

	Series K
	Protection against interference

	Series L
	Construction, installation and protection of cables and other elements of outside plant

	Series M
	TMN and network maintenance: international transmission systems, telephone circuits, telegraphy, facsimile and leased circuits

	Series N
	Maintenance: international sound programme and television transmission circuits

	Series O
	Specifications of measuring equipment

	Series P
	Telephone transmission quality, telephone installations, local line networks

	Series Q
	Switching and signalling

	Series R
	Telegraph transmission

	Series S
	Telegraph services terminal equipment

	Series T
	Terminals for telematic services

	Series U
	Telegraph switching

	Series V
	Data communication over the telephone network

	Series X
	Data networks and open system communications

	Series Y
	Global information infrastructure and Internet protocol aspects

	Series Z
	Languages and general software aspects for telecommunication systems

	
	











_1065519936.unknown

_1067427774.unknown

_1065519797.unknown

