
	[image: image1.wmf]
	INTERNATIONAL  TELECOMMUNICATION  UNION

	
	

	ITU-T
	Q.2722.1

	TELECOMMUNICATION
STANDARDIZATION  SECTOR
OF  ITU
	Amendment 1

(06/2000)   

	
	SERIES Q: SWITCHING AND SIGNALLING

Broadband ISDN – B-ISDN application protocols for the network signalling



	
	B‑ISDN User Part ( Network node interface specification for point-to-multipoint call/connection control

Amendment 1



	
	ITU‑T  Recommendation  Q.2722.1  –  Amendment 1

(Formerly  CCITT  Recommendation)


ITU-T Q-SERIES  RECOMMENDATIONS

SWITCHING AND SIGNALLING

	
	

	SIGNALLING IN THE INTERNATIONAL MANUAL SERVICE
	Q.1–Q.3

	INTERNATIONAL AUTOMATIC AND SEMI-AUTOMATIC WORKING
	Q.4–Q.59

	FUNCTIONS AND INFORMATION FLOWS FOR SERVICES IN THE ISDN
	Q.60–Q.99

	CLAUSES APPLICABLE TO ITU-T STANDARD SYSTEMS
	Q.100–Q.119

	SPECIFICATIONS OF SIGNALLING SYSTEMS No. 4 AND No. 5
	Q.120–Q.249

	SPECIFICATIONS OF SIGNALLING SYSTEM No. 6
	Q.250–Q.309

	SPECIFICATIONS OF SIGNALLING SYSTEM R1
	Q.310–Q.399

	SPECIFICATIONS OF SIGNALLING SYSTEM R2
	Q.400–Q.499

	DIGITAL EXCHANGES
	Q.500–Q.599

	INTERWORKING OF SIGNALLING SYSTEMS
	Q.600–Q.699

	SPECIFICATIONS OF SIGNALLING SYSTEM No. 7
	Q.700–Q.799

	Q3 INTERFACE
	Q.800–Q.849

	DIGITAL SUBSCRIBER SIGNALLING SYSTEM No. 1
	Q.850–Q.999

	PUBLIC LAND MOBILE NETWORK
	Q.1000–Q.1099

	INTERWORKING WITH SATELLITE MOBILE SYSTEMS
	Q.1100–Q.1199

	INTELLIGENT NETWORK
	Q.1200–Q.1699

	SIGNALLING REQUIREMENTS AND PROTOCOLS FOR IMT-2000
	Q.1700–Q.1799

	BROADBAND ISDN
	Q.2000–Q.2999

	General aspects
	Q.2000–Q.2099

	Signalling ATM adaptation layer (SAAL)
	Q.2100–Q.2199

	Signalling network protocols
	Q.2200–Q.2299

	Common aspects of B-ISDN application protocols for access signalling and network signalling and interworking
	Q.2600–Q.2699

	B-ISDN application protocols for the network signalling
	Q.2700–Q.2899

	B-ISDN application protocols for access signalling
	Q.2900–Q.2999

	
	


For further details, please refer to the list of ITU-T Recommendations.

	ITU-T Recommendation Q.2722.1

B‑ISDN User Part ( Network node interface specification
for point-to-multipoint call/connection control

Amendment 1




	Summary

ITU-T Q.2722.1 specifies the extensions to the Broadband ISDN User Part (B‑ISUP) to support point-to-multipoint call/connection control. This Amendment 1 to ITU-T Q.2722.1 has been prepared to enable the use of the modified B‑ISUP assignment procedures together with the point‑to‑multipoint call/connection control. This amendment includes the additional procedures required to support the modified B‑ISUP assignment procedures. In addition it contains corrections of editorial errors identified in ITU-T Q.2722.1 since its first publication in 1996.


	Source

Amendment 1 to ITU‑T Recommendation Q.2722.1 was prepared by ITU‑T Study Group 11 (1997‑2000) and approved under the WTSC Resolution 1 procedure on 15 June 2000.


	


FOREWORD

The International Telecommunication Union (ITU) is the United Nations specialized agency in the field of tele​com​mu​ni​ca​tions. The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of ITU. ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Conference (WTSC), which meets every four years, establishes the topics for study by the ITU‑T study groups which, in turn, produce Recommendations on these topics.

The approval of ITU-T Recommendations is covered by the procedure laid down in WTSC Resolution 1.

In some areas of information technology which fall within ITU-T’s purview, the necessary standards are prepared on a collaborative basis with ISO and IEC.

NOTE

In this Recommendation, the expression "Administration" is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

INTELLECTUAL PROPERTY RIGHTS
ITU draws attention to the possibility that the practice or implementation of this Recommendation may involve the use of a claimed Intellectual Property Right. ITU takes no position concerning the evidence, validity or applicability of claimed Intellectual Property Rights, whether asserted by ITU members or others outside of the Recommendation development process.

As of the date of approval of this Recommendation, ITU had not received notice of intellectual property, protected by patents, which may be required to implement this Recommendation. However, implementors are cautioned that this may not represent the latest information and are therefore strongly urged to consult the TSB patent database.

(  ITU  2001

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from ITU.

Introduction

This Amendment 1 to ITU-T Q.2722.1 has been prepared to enable the use of the modified B‑ISUP assignment procedures together with the point-to-multipoint call/connection control.

This amendment includes the additional procedures required to support the modified B‑ISUP assignment procedures. In addition it contains corrections of editorial errors identified in ITU‑T Q.2722.1 since its first publication in 1996.

ITU-T Recommendation Q.2722.1

B‑ISDN User Part ( Network node interface specification
for point-to-multipoint call/connection control

Amendment 1

1)
Replacements and additions

a)
Clause 1.2

Replace 1.2 with the following:
1.2
References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; all users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

–
ITU-T I.610 (1995), B-ISDN operation and maintenance principles and functions.

–
ITU-T Q.2650 (1999), Interworking between Signalling System No. 7 broadband ISDN user part (B-ISUP) and Digital Subscriber Signalling System No. 2 (DSS2).

–
ITU-T Q.2721.1 (1996), B-ISDN user part ( Overview of the B-ISDN network node interface signalling capability set 2, step 1. 

NOTE ( The content of this Recommendation was integrated into the "core" set of B‑ISDN user part Recommendations (Q.2761 through Q.2764).

–
ITU-T Q.2761 (1999), Functional description of the B‑ISDN user part (B‑ISUP) of Signalling System No. 7.
–
ITU-T Q.2762 (1999), General functions of messages and signals of the B-ISDN user part (B-ISUP) of Signalling System No. 7.

–
ITU-T Q.2763 (1999), Signalling System No. 7 B-ISDN user part (B‑ISUP) ( Formats and codes.

–
ITU-T Q.2764 (1999), Signalling System No. 7 B-ISDN user part (B‑ISUP) ( Basic call procedures.

–
ITU-T Q.2931 (1995), Digital Subscriber Signalling System No. 2 (  User‑Network Interface (UNI) layer 3 specification for basic call/connection control.

–
ITU-T Q.2971 (1995), Digital Subscriber Signalling System No. 2 (  User‑network interface layer 3 specification for point-to-multipoint call/connection control.

b)
Clause 2.2.1.1.1, item e)

Replace item e) with the following:

e)
Set_Up request primitive – issued by the non-assigning exchange


Option 1

With regard to the issuing of the Set_Up request primitive by the non-assigning exchange, the same procedures as for the assigning exchange are applicable, with the exception that the connection element identifier shall not be included in the Set_Up request primitive.


Option 2

With regard to the issuing of the Set_Up request primitive by the non-assigning exchange, the same procedures as for the assigning exchange are applicable, with the exception that the Exclusive connection element identifier parameter shall be included in the Set_Up request primitive instead of the Connection element identifier parameter.

c)
Clause 2.2.1.1.2.1

Replace 2.2.1.1.2.1 with the following:

2.2.1.1.2.1
Incoming side of the exchange

a)
Assigning exchange


Option 1

After having received a Set_Up indication primitive, an incoming connection link object instance shall be created. An assigning exchange shall perform the assignment procedure for VPCI/VCI and bandwidth as described in 2.1.2.2 b)/Q.2764. If this is successful the Incoming_Resources_ Accepted request primitive shall be issued immediately. The Incoming_Resources_Accepted request primitive shall include the mandatory parameters listed in Table 2-4, including the origination connection link identifier and connection element identifier parameters.


Option 2

After having received a Set_Up indication primitive, an incoming connection link object instance shall be created. An assigning exchange shall perform the assignment procedure as described in 2.1.2.2 b)/Q.2764. If this is successful the Incoming_Resources_Accepted request primitive shall be issued immediately. The Incoming_Resources_Accepted request primitive shall include the mandatory parameters listed in Table 2-4, including the origination connection link identifier and connection element identifier parameters.

b)
Non-assigning exchange


Option 1

After having received a Set_Up Indication primitive an incoming connection link object instance shall be created. A non-assigning exchange shall issue the Incoming_Resources_Accepted request primitive immediately. The Incoming_Resources_Accepted request primitive shall include the mandatory parameters listed in Table 2-4, without the connection element identifier parameter but containing the assigned origination connection link identifier parameter.


Option 2

After having received a Set_Up Indication primitive an incoming connection link object instance shall be created. A non-assigning exchange shall perform the assignment procedure as described in 2.1.2.2 a)/Q.2764. If this is successful the Incoming_Resources_Accepted request primitive shall be issued immediately. The Incoming_Resources_Accepted request primitive shall include the mandatory parameters listed in Table 2-4, without the connection element identifier parameter but containing the assigned origination connection link identifier parameter.

d)
Clause 2.2.1.1.2.2, item c)

Replace item c) with the following:

c)
Set_Up request primitive – issued by the non-assigning exchange


Option 1

With regard to the issuing of the Set_Up request primitive by the non-assigning exchange, the same procedures as for the assigning exchange are applicable, with the exception that the connection element identifier shall not be included in the Set_Up request primitive.


Option 2

With regard to the issuing of the Set_Up request primitive by the non-assigning exchange, the same procedures as for the assigning exchange are applicable, with the exception that the Exclusive connection element identifier parameter shall be included in the Set_Up request primitive instead of the Connection element identifier parameter.

e)
Clause 2.2.1.2.1, item e)

Replace item e) with the following:

e)
Set_Up request primitive – issued by the non-assigning exchange


Option 1

With regard to issuing the Set_Up request primitive by the non-assigning exchange, the same procedures as for the assigning exchange are applicable, with the exception that the connection element identifier shall not be included in the Set_Up request primitive.


Option 2

With regard to the issuing of the Set_Up request primitive by the non-assigning exchange, the same procedures as for the assigning exchange are applicable, with the exception that the Exclusive connection element identifier parameter shall be included in the Set_Up request primitive instead of the Connection element identifier parameter.

f)
Additions to Table 2-2

Add the following to Table 2-2:

	Set_Up request/indication

	Parameter
	Mandatory/Optional

	(
	(

	Exclusive connection element identifier
	O

	(
	(


2)
Error corrections

a)
Correct 2.4.5.1, item d) as follows:

d)
Collision of release primitives

i)
Outgoing side of an exchange


After sending the release request primitive with the destination connection link identifier parameter, a release indication primitive may be received. In this case, the exchange shall return a release response primitive towards the B-ISUP AEI from which the concerned release indication primitive was received. The release response primitive shall be issued when the transmission path is disconnected.

ii)
Incoming side of an exchange


After sending a release request primitive a release indication primitive with a destination connection link identifier parameter may be received. In this case, the exchange shall continue as described in 2.4.5.1 b).

	SERIES OF ITU-T RECOMMENDATIONS

	Series A
	Organization of the work of ITU-T

	Series B
	Means of expression: definitions, symbols, classification

	Series C
	General telecommunication statistics

	Series D
	General tariff principles

	Series E
	Overall network operation, telephone service, service operation and human factors

	Series F
	Non-telephone telecommunication services

	Series G
	Transmission systems and media, digital systems and networks

	Series H
	Audiovisual and multimedia systems

	Series I
	Integrated services digital network

	Series J
	Transmission of television, sound programme and other multimedia signals

	Series K
	Protection against interference

	Series L
	Construction, installation and protection of cables and other elements of outside plant

	Series M
	TMN and network maintenance: international transmission systems, telephone circuits, telegraphy, facsimile and leased circuits

	Series N
	Maintenance: international sound programme and television transmission circuits

	Series O
	Specifications of measuring equipment

	Series P
	Telephone transmission quality, telephone installations, local line networks

	Series Q
	Switching and signalling

	Series R
	Telegraph transmission

	Series S
	Telegraph services terminal equipment

	Series T
	Terminals for telematic services

	Series U
	Telegraph switching

	Series V
	Data communication over the telephone network

	Series X
	Data networks and open system communications

	Series Y
	Global information infrastructure and Internet protocol aspects

	Series Z
	Languages and general software aspects for telecommunication systems

	
	





