
	[image: image1.wmf]
	INTERNATIONAL  TELECOMMUNICATION  UNION


	ITU-T
	Q.2931

	TELECOMMUNICATION
STANDARDIZATION  SECTOR
OF  ITU
	Amendment 2

(03/99)   


SERIES Q: SWITCHING AND SIGNALLING

Broadband ISDN – B-ISDN application protocols for access signalling

Digital subscriber signalling system No. 2 – User‑network interface (UNI) layer 3 specification for basic call/connection control

Amendment 2

ITU-T  Recommendation  Q.2931  –  Amendment 2

(Previously  CCITT  Recommendation)
ITU-T  Q-SERIES  RECOMMENDATIONS

SWITCHING AND SIGNALLING

	
	

	SIGNALLING IN THE INTERNATIONAL MANUAL SERVICE
	Q.1–Q.3

	INTERNATIONAL AUTOMATIC AND SEMI-AUTOMATIC WORKING
	Q.4–Q.59

	FUNCTIONS AND INFORMATION FLOWS FOR SERVICES IN THE ISDN
	Q.60–Q.99

	CLAUSES APPLICABLE TO ITU-T STANDARD SYSTEMS
	Q.100–Q.119

	SPECIFICATIONS OF SIGNALLING SYSTEMS No. 4 AND No. 5
	Q.120–Q.249

	SPECIFICATIONS OF SIGNALLING SYSTEM No. 6
	Q.250–Q.309

	SPECIFICATIONS OF SIGNALLING SYSTEM R1
	Q.310–Q.399

	SPECIFICATIONS OF SIGNALLING SYSTEM R2
	Q.400–Q.499

	DIGITAL EXCHANGES
	Q.500–Q.599

	INTERWORKING OF SIGNALLING SYSTEMS
	Q.600–Q.699

	SPECIFICATIONS OF SIGNALLING SYSTEM No. 7
	Q.700–Q.849

	DIGITAL SUBSCRIBER SIGNALLING SYSTEM No. 1
	Q.850–Q.999

	PUBLIC LAND MOBILE NETWORK
	Q.1000–Q.1099

	INTERWORKING WITH SATELLITE MOBILE SYSTEMS
	Q.1100–Q.1199

	INTELLIGENT NETWORK
	Q.1200–Q.1699

	SIGNALLING REQUIREMENTS AND PROTOCOLS FOR IMT-2000
	Q.1700–Q.1799

	BROADBAND ISDN
	Q.2000–Q.2999

	General aspects
	Q.2000–Q.2099

	Signalling ATM adaptation layer (SAAL)
	Q.2100–Q.2199

	Signalling network protocols
	Q.2200–Q.2299

	Common aspects of B-ISDN application protocols for access signalling and network signalling and interworking
	Q.2600–Q.2699

	B-ISDN application protocols for the network signalling
	Q.2700–Q.2899

	B-ISDN application protocols for access signalling
	Q.2900–Q.2999

	
	


For further details, please refer to ITU-T List of Recommendations.

	ITU-T  RECOMMENDATION  Q.2931

digital subscriber signalling system no. 2 – User-network INTERFACE (UNI) layer 3 specification for basic call/connection control

AMENDMENT 2


	Summary

Recommendation Q.2931 provides basic call and connection control for point-to-point connections in a B-ISDN. Amendment 2 to Recommendation Q.2931 has been prepared to allow signalling of AAL parameters associated with ATM Adaptation Layers specified in the I-series Recommendations since the original publication of Recommendation Q.2931 and is designed to be compatible with implementations conforming to the first edition of Recommendation Q.2931 (1995).

The changes from Recommendation Q.2931 (1995) specify B-ISDN call control to support advances made in the I-series Recommendations concerned with ATM Adaptation Layer protocols that have been introduced since the original publication of Q.2931 (1995). Specifically, this amendment includes support for the following Recommendations:

–
ITU-T Recommendation I.363.1 (1996), B-ISDN ATM Adaptation Layer specification: Type 1 AAL.

–
ITU-T Recommendation I.363.2 (1997), B-ISDN ATM Adaptation Layer specification: Type 2 AAL.

–
ITU-T Recommendation I.363.3 (1996), B-ISDN ATM Adaptation Layer specification: Type 3/4 AAL.

–
ITU-T Recommendation I.363.5 (1996), B-ISDN ATM Adaptation Layer specification: Type 5 AAL.

–
ITU-T Recommendation I.365.1 (1993), B-ISDN ATM adaptation layer sublayers: Frame relaying service specific convergence sublayer (FR-SSCS).

–
ITU-T Recommendation I.365.2 (1995), B-ISDN ATM adaptation layer sublayers: Service specific coordination function to provide the connection-oriented network service.

–
ITU-T Recommendation I.365.3 (1995), B-ISDN ATM adaptation layer sublayers: Service specific coordination function to provide the connection-oriented transport service.

–
ITU-T Recommendation I.366.1 (1998), Segmentation and Reassembly Service Specific Convergence Sublayer for the AAL Type 2.

–
ITU-T Recommendation I.366.2 (1999), AAL Type 2 SSCS for trunking.

–
ITU-T Recommendation Q.2110 (1994), B-ISDN ATM adaptation layer – Service specific connection oriented protocol (SSCOP).


	Source
Amendment 2 to ITU-T Recommendation Q.2931 was prepared by ITU-T Study Group 11 (1997‑2000) and was approved under the WTSC Resolution No. 1 procedure on 15 March 1999.


	


FOREWORD

ITU (International Telecommunication Union) is the United Nations Specialized Agency in the field of telecommunications. The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of the ITU. The ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Conference (WTSC), which meets every four years, establishes the topics for study by the ITU‑T Study Groups which, in their turn, produce Recommendations on these topics.

The approval of Recommendations by the Members of the ITU‑T is covered by the procedure laid down in WTSC Resolution No. 1.

In some areas of information technology which fall within ITU-T’s purview, the necessary standards are prepared on a collaborative basis with ISO and IEC.

NOTE

In this Recommendation the term recognized operating agency (ROA) includes any individual, company, corporation or governmental organization that operates a public correspondence service. The terms Administration, ROA and public correspondence are defined in the Constitution of the ITU (Geneva, 1992).

INTELLECTUAL PROPERTY RIGHTS
The ITU draws attention to the possibility that the practice or implementation of this Recommendation may involve the use of a claimed Intellectual Property Right. The ITU takes no position concerning the evidence, validity or applicability of claimed Intellectual Property Rights, whether asserted by ITU members or others outside of the Recommendation development process.

As of the date of approval of this Recommendation, the ITU had not received notice of intellectual property, protected by patents, which may be required to implement this Recommendation. However, implementors are cautioned that this may not represent the latest information and are therefore strongly urged to consult the TSB patent database.

  ITU  1999

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

CONTENTS


Page

Annex F – ATM adaptation layer parameters indication and negotiation

12
F.1
General

12
F.2
ATM adaptation layer parameter indication in the SETUP message

12
F.3
Maximum CPCS-SDU size negotiation

13
F.4
MID range negotiation

13
F.5
Use of forward and backward maximum CPCS-SDU size by the AAL 3/4 or 5 entity in the user plane

14
F.6
Use of maximum CPS-SDU size by the AAL 2 entity in the user plane

14

Introduction

This amendment to Recommendation Q.2931 has been prepared to allow compatibility with implementations conforming to the first edition of Recommendation Q.2931 (1995).

The changes from Recommendation Q.2931 (1995) specify B-ISDN call control to support advances made in the I-series Recommendations concerned with ATM Adaptation Layer protocols that have been introduced since the original publication of Q.2931 (1995). Specifically, this amendment includes support for the following Recommendations:

–
ITU-T Recommendation I.363.1 (1996), B-ISDN ATM Adaptation Layer: Type 1 AAL specification.

–
ITU-T Recommendation I.363.2 (1997), B-ISDN ATM Adaptation Layer: Type 2 AAL specification.

–
ITU-T Recommendation I.363.3 (1996), B-ISDN ATM Adaptation Layer: Type 3/4 AAL specification.

–
ITU-T Recommendation I.363.5 (1996), B-ISDN ATM Adaptation Layer: Type 5 AAL specification.

–
ITU-T Recommendation I.365.1 (1993), B-ISDN ATM adaptation layer sublayers: Frame relaying service specific convergence sublayer (FR-SSCS).

–
ITU-T Recommendation I.365.2 (1995), B-ISDN ATM adaptation layer sublayers: Service specific coordination function to provide the connection-oriented network service.

–
ITU-T Recommendation I.365.3 (1995), B-ISDN ATM adaptation layer sublayers: Service specific coordination function to provide the connection-oriented transport service.
–
ITU-T Recommendation I.366.1 (1998), Segmentation and Reassembly Service Specific Convergence Sublayer for the AAL Type 2.

–
ITU-T Recommendation I.366.2 (1999), AAL Type 2 SSCS for trunking.

–
ITU-T Recommendation Q.2110 (1994), B-ISDN ATM adaptation layer – Service specific connection oriented protocol (SSCOP).
NOTE – This amendment to Recommendation Q.2931 does not include an explicit modification of the reference list in Annex J/Q.2931. It is expected that a future amendment or revision of Recommendation Q.2931 will include the following additions to the list of references:

–
ITU-T Recommendation I.363.1 (1996), B-ISDN ATM Adaptation Layer: Type 1 AAL specification.

–
ITU-T Recommendation I.363.2 (1997), B-ISDN ATM Adaptation Layer: Type 2 AAL specification.

–
ITU-T Recommendation I.363.3 (1996), B-ISDN ATM Adaptation Layer: Type 3/4 AAL specification.

–
ITU-T Recommendation I.363.5 (1996), B-ISDN ATM Adaptation Layer: Type 5 AAL specification.

–
ITU-T Recommendation I.365.1 (1993), B-ISDN ATM adaptation layer sublayers: Frame relaying service specific convergence sublayer (FR-SSCS).

–
ITU-T Recommendation I.365.2 (1995), B-ISDN ATM adaptation layer sublayers: Service specific coordination function to provide the connection-oriented network service.

–
ITU-T Recommendation I.365.3 (1995), B-ISDN ATM adaptation layer sublayers: Service specific coordination function to provide the connection-oriented transport service.

–
ITU-T Recommendation I.366.1 (1998), Segmentation and Reassembly Service Specific Convergence Sublayer for the AAL Type 2.

–
ITU-T Recommendation I.366.2 (1999), AAL Type 2 SSCS for trunking.

–
CCITT Recommendation Q.320 (1988), (Signalling System R1) Signal code for register signalling.

–
CCITT Recommendation Q.322 (1988), (Signalling System R1) Multifrequency signal sender.

–
CCITT Recommendation Q.323 (1988), (Signalling System R1) Multifrequency signal receiving equipment.

–
CCITT Recommendation Q.441 (1988), (Signalling System R2 – Interregister signalling) Signalling code.

–
IEEE Standard 802-1990, IEEE Standards for Local and Metropolitan Area Networks: Overview and Architecture.

Recommendation Q.2931

digital subscriber signalling system no. 2 – user-network interface (UNI) layer 3 specification for basic call/connection control

amendment 2

(Geneva, 1999)

Replace 4.5.5 (AAL parameters parameter) and Annex F (ATM adaptation parameters indication and negotiation) with the text presented in this amendment.

1)
Subclause 4.5.5

Replace subclause 4.5.5 with the following text:
4.5.5
ATM adaptation layer parameters

The purpose of the ATM adaptation layer (AAL) parameters information element is to indicate the requested AAL parameter values (end-to-end significance) for the ATM adaptation layer elements of procedures to be used for the call. It contains the parameters selectable by the user for all AAL sublayers.

The contents of this information element is transparent for the network, except for the case of interworking.

The maximum length of this information element is 24 octets.

The AAL parameters information element is coded as shown in Figure 4-12 and Table 4-6.


[image: image2.wmf]8

7

6

5

4

3

2

1

1

2

3

4

5

T1162660-94

0

1

1

0

1

0

0

0

Octets

ext.

1 

Coding

standard

Flag

IE instruction field

Res.

IE action ind.

Length of AAL parameter contents

Further contents depending on AAL type

(see below)

AAL type

AAL prameters i

nformation element identifier


Figure 4-12/Q.2931 – AAL parameters information element (part 1 of 8)


[image: image3.wmf]8

7

6

5

4

3

2

1

6

6.1

7

7.1

8*

T1162670-94

1

0

0

0

0

0

1

1

1

0

0

1

0

0

1

0

1

0

0

1

0

0

1

1

8.1*

8.2*

9*

9.1*

10*

10.1*

11*

1

0

0

0

0

0

1

0

1

0

0

0

0

0

1

1

1

0

1

1

0

0

11.1*

11.2*

0

0

12*

12.1*

1

0

0

0

0

1

1

1

Octets

Subtype identifier

Subtype

CBR rate identifier

CBR rate 

Multiplier identifier

Multiplier

Source clock frequency recovery method identifier

Source clock frequency recovery method

Error correction method identifier

Error correction method

Structured data transfer block size

Partially filled cells method identifier

Partially filled cells method

Structured data transfer block size identifier

(Note)

(Note)

(Note)

NOTE – These octets are only present if octet 7.1 indicates "n × 64 kbit/s or n × 8 kbit/s".

(further contents for AAL type 1)


Figure 4-12/Q.2931 – AAL parameters information element (part 2 of 8)


[image: image4.wmf]8

7

6

5

4

3

2

1

6*

6.1*

T1162680-94

1

0

0

0

0

1

1

0

6.2*

7*

1

0

0

0

0

1

0

0

7.1*

7.2*

8*

1

0

0

0

0

1

0

0

8.1*

8.2*

8.3*

8.4*

9*

1

0

0

0

0

1

0

0

9.1*

(further contents for AAL type 3/4)

Octets

Forward maximum CPCS-SDU size identifier

Forward maximum CPCS-SDU size

Backward maximum CPCS-SDU size identifier

Backward maximum CPCS-SDU size

MID range identifier

MID range (lowest MID value)

MID range (highest MID value)

SSCS-type identifier

SSCS-type

NOTE – The indication of values for octet groups 6-8 for use in the CONNECT message 

is specified in Annex F.


Figure 4-12/Q.2931 – AAL parameters information element (part 3 of 8)


[image: image5.wmf]8

7

6

5

4

3

2

1

6*

6.1*

T1162690-94

1

0

0

0

0

1

1

0

6.2*

7*

1

0

0

0

0

1

0

0

7.1*

7.2*

8*

1

0

0

0

0

1

0

0

8.1*

(further contents for AAL type 5)

Octets

Forward maximum CPCS-SDU size identifier

Backward maximum CPCS-SDU size identifier

Forward maximum CPCS-SDU size 

Backward maximum CPCS-SDU size 

SSCS-type identifier

SSCS-type

NOTE – The indication of values for octet groups 6-7 for use in the CONNECT message 

is specified in Annex F.


Figure 4-12/Q.2931 – AAL parameters information element (part 4 of 8)

[image: image6.wmf]8

7

6

5

4

3

2

1

5.1*

5.2*

T1162700-94

5.3*

5.4*

(further contents for User-defined AAL)

User defined AAL information

User defined AAL information

User defined AAL information

User defined AAL information

Octets


Figure 4-12/Q.2931 – AAL parameters information element (part 5 of 8)

(further contents for AAL type 2)

	8
	7
	6
	5
	4
	3
	2
	1
	Octets

	Maximum CPS-SDU size identifier
	

	1
	0
	1
	0
	0
	0
	0
	1
	6*

	Maximum CPS-SDU size
	6.1*

	Maximum number of multiplexed channels identifier
	

	1
	0
	1
	0
	0
	0
	1
	0
	7*

	Maximum number of multiplexed channels
	7.1*

	SSCS type identifier
	

	1
	0
	0
	0
	0
	1
	0
	0
	8*

	SSCS type
	8.1*


Further contents depending on SSCS type (see below)

Figure 4-12/Q.2931 – AAL parameters information element (part 6 of 8)
(further contents for SSCS type = SAR, defined in Recommendation I.366.1)

	8
	7
	6
	5
	4
	3
	2
	1
	Octets

	SSCS type for SAR parameters identifier
	

	1
	0
	1
	0
	0
	1
	0
	1
	9*

	Assured data
	Error detect
	Reserved
	9.1*

	Forward maximum SSSAR-SDU size identifier
	

	1
	0
	1
	0
	1
	0
	0
	0
	10*

	Forward maximum SSSAR-SDU size
	10.1*

	
	
	
	
	
	
	
	
	10.2*

	
	
	
	
	
	
	
	
	10.3*

	Backward maximum SSSAR-SDU size identifier
	

	1
	0
	1
	0
	1
	0
	0
	1
	11*

	Backward maximum SSSAR-SDU size
	11.1*

	
	
	
	
	
	
	
	
	11.2*

	
	
	
	
	
	
	
	
	11.3*


Figure 4-12/Q.2931 – AAL parameters information element (part 7 of 8)

(further contents for SSCS for trunking, defined in Recommendation I.366.2)

	8
	7
	6
	5
	4
	3
	2
	1
	Octets

	SSCS type for trunking parameters identifier
	

	1
	0
	1
	0
	0
	1
	1
	0
	9*

	Service category
	CMD
	FMD
	Reserved
	9.1*

	Reserved
	Fax
	CAS
	DTMF
	MF-R1
	MF-R2
	PCM encoding
	9.2*

	Reserved
	Multiplier
	9.3*

	Maximum length of frame mode data unit identifier
	

	1
	0
	1
	0
	1
	0
	1
	0
	10*

	Maximum length of frame mode data unit
	10.1*

	
	
	
	
	
	
	
	
	10.2*

	Profile identification identifier
	

	1
	0
	1
	0
	1
	0
	1
	1
	11*

	Profile source
	Reserved
	11.1*

	Predefined profile
	11.2*

	
	
	
	
	
	
	
	
	11.3*

	IEEE Organizationally Unique Identifier (OUI)
	11.4*

	
	
	
	
	
	
	
	
	11.5*


Figure 4-12/Q.2931 – AAL parameters information element (part 8 of 8)

Table 4-6/Q.2931 – AAL parameters information element

	AAL type (octet 5)

	Bits

	
	8
	7
	6
	5
	4
	3
	2
	1
	

	
	0
	0
	0
	0
	0
	0
	0
	0
	AAL for voice (Notes 1 and 2)

	
	0
	0
	0
	0
	0
	0
	0
	1
	AAL type 1

	
	0
	0
	0
	0
	0
	0
	1
	0
	AAL type 2

	
	0
	0
	0
	0
	0
	0
	1
	1
	AAL type 3/4

	
	0
	0
	0
	0
	0
	1
	0
	1
	AAL type 5

	
	0
	0
	0
	1
	0
	0
	0
	0
	User defined AAL

	All other values are reserved.

	Subtype (octet 6.1 for AAL type 1)

	Bits

	
	8
	7
	6
	5
	4
	3
	2
	1
	

	
	0
	0
	0
	0
	0
	0
	0
	0
	Null

	
	0
	0
	0
	0
	0
	0
	0
	1
	Voiceband signal transport based on 64 kbit/s (see Recommendation G.711/G.722) (see ITU-T Recommendation I.363.1)

	
	0
	0
	0
	0
	0
	0
	1
	0
	Circuit transport (see ITU-T Recommendation I.363.1)

	
	0
	0
	0
	0
	0
	1
	0
	0
	High-quality audio signal transport (see ITU-T Recommendation I.363.1)

	
	0
	0
	0
	0
	0
	1
	0
	1
	Video signal transport (see ITU-T Recommendation I.363.1)

	All other values are reserved.


Table 4-6/Q.2931 – AAL parameters information element (continued)
	CBR rate (octet 7.1 for AAL type 1)

	Bits

	
	8
	7
	6
	5
	4
	3
	2
	1
	

	
	0
	0
	0
	0
	0
	0
	0
	1
	
64 kbit/s

	
	0
	0
	0
	0
	0
	1
	0
	0
	
1 544 kbit/s

	
	0
	0
	0
	0
	0
	1
	0
	1
	
6 312 kbit/s

	
	0
	0
	0
	0
	0
	1
	1
	0
	
32 064 kbit/s

	
	0
	0
	0
	0
	0
	1
	1
	1
	
44 736 kbit/s

	
	0
	0
	0
	0
	1
	0
	0
	0
	
97 728 kbit/s

	
	0
	0
	0
	1
	0
	0
	0
	0
	
2 048 kbit/s

	
	0
	0
	0
	1
	0
	0
	0
	1
	
8 448 kbit/s

	
	0
	0
	0
	1
	0
	0
	1
	0
	
34 368 kbit/s

	
	0
	0
	0
	1
	0
	0
	1
	1
	
139 264 kbit/s

	
	0
	1
	0
	0
	0
	0
	0
	0
	n ( 64 kbit/s

	
	0
	1
	0
	0
	0
	0
	0
	1
	n ( 8 kbit/s

	All other values are reserved.

	Multiplier (octets 8.1 and 8.2 for AAL type 1 and n ( 64 kbit/s or n ( 8 kbit/s indication in octet 7.1 for AAL type 1) (octet 10b, bits 5-1, when octet 8.1 = 00010001 and octet 9.3, bits 5-1 = 00011 for AAL type 2)

	Integer representation of multiplier values between 2 and 216 ( 1 for n ( 64 kbit/s in AAL type 1 channels.

	Integer representation of multiplier values between 1 and 7 for n ( 8 kbit/s in AAL type 1 channels.

	Integer representation of multiplier values between 1 and 31 for n ( 64 kbits/s in AAL type 2 channels.

	Source clock frequency recovery method (octet 9.1 for AAL type 1)

	Bits

	
	8
	7
	6
	5
	4
	3
	2
	1
	

	
	0
	0
	0
	0
	0
	0
	0
	0
	Null (synchronous circuit transport)

	
	0
	0
	0
	0
	0
	0
	0
	1
	Synchronous Residual Time Stamp (SRTS) method (asynchronous circuit transport) (see ITU-T Recommendation I.363.1)

	
	0
	0
	0
	0
	0
	0
	1
	0
	Adaptive clock method (see ITU-T Recommendation I.363.1)

	All other values are reserved.

	Error correction method (octet 10.1 for AAL type 1)

	Bits

	
	8
	7
	6
	5
	4
	3
	2
	1
	

	
	0
	0
	0
	0
	0
	0
	0
	0
	Null (no error correction is provided)

	
	0
	0
	0
	0
	0
	0
	0
	1
	A forward error correction method for loss sensitive signal transport (see ITU-T Recommendation I.363.1)

	
	0
	0
	0
	0
	0
	0
	1
	0
	A forward error correction method for delay sensitive signal transport (see ITU-T Recommendation I.363.1)

	All other values are reserved.

	Structured data transfer block size (octet 11.1 and 11.2 for AAL type 1)

	16-bit integer representation of values between 1 and 65 535, i.e. 216 ( 1. This parameter represents the block size of SDT CBR service. (Note 3)


Table 4-6/Q.2931 – AAL parameters information element (continued)
	Partially filled cells method (octet 12.1 for AAL type 1)

	Bits

	
	8
	7
	6
	5
	4
	3
	2
	1
	

	
	0
	0
	0
	0
	0
	0
	0
	0
	Partially filled cells method is not used, i.e. cells are filled completely; the indication of this codepoint is equivalent to the absence of the subfield for partially filled cells method.

	
	0
	0
	0
	0
	0
	0
	0
	1
	Integer representation of the number of leading octets of SAR-PDU payload in use (values between 1 and 47) (see ITU-T Recommendation I.363.1).

	
	
	
	
	to
	
	
	
	

	
	0
	0
	1
	0
	1
	1
	1
	1
	

	Forward maximum CPCS-SDU size (octets 6.1 and 6.2 for AAL types 3/4 and 5)

	16-bit integer representation of the values between 0 and 65 535, i.e. 216 ( 1. This parameter refers to the forward direction (calling user to called user, see definition in Annex J).

	Backward maximum CPCS-SDU size (octets 7.1 and 7.2 for AAL types 3/4 and 5)

	16-bit integer representation of the values between 0 and 65 535, i.e. 216 ( 1. This parameter refers to the backward direction (called user to calling user, see definition in Annex J).

	MID range (octets 8.1, 8.2, 8.3 and 8.4 for AAL type 3/4)

	Integer representation of the lowest MID value (octets 8.1 and 8.2) and of the highest MID value (octets 8.3 and 8.4) of the MID range, only values between 0 and 1023.

	User defined AAL information (octets 5.1 to 5.4 for user defined AAL)

	Bits

	
	8
	7
	6
	5
	4
	3
	2
	1
	

	The contents of this field are user-specified.

	Maximum CPS-SDU size (octet 6.1 for AAL type 2)

	8-bit integer representation of the value 45 or 64.

	NOTE – some applications may limit the CPS-SDU size assignment to 45 octets.

	Maximum number of multiplexed channels (octet 7.1 for AAL type 2)

	The value ranges from 1 through 255.

	SSCS type (octet 9.1 for AAL type 3/4; octet 8.1 for AAL types 2 and 5)

	Bits

	
	8
	7
	6
	5
	4
	3
	2
	1
	

	
	0
	0
	0
	0
	0
	0
	0
	0
	Null

	
	0
	0
	0
	0
	0
	0
	0
	1
	ITU-T Recommendation Q.2110 (SSCOP assured operation) with unspecified SSCS

	
	0
	0
	0
	0
	0
	0
	1
	0
	ITU-T Recommendation Q.2110 (SSCOP non-assured operation) with unspecified SSCS

	
	0
	0
	0
	0
	0
	1
	0
	0
	ITU-T Recommendation I.365.1 (SSCS to provide frame relay services)

	
	0
	0
	0
	0
	1
	0
	0
	0
	ITU-T Recommendation I.365.2 [SSCF to provide CONS and Q.2110 (SSCOP)] (only for AAL types 2, 3/4 and 5)

	
	0
	0
	0
	0
	1
	0
	0
	1
	ITU-T Recommendation I.365.3 [SSCF to provide COTS and Q.2110 (SSCOP)] (only for AAL types 2, 3/4 and 5)

	
	0
	0
	0
	1
	0
	0
	0
	0
	ITU-T Recommendation I.366.1 (SSCS to provide segmentation and re-assembly) (only for AAL type 2)

	
	0
	0
	0
	1
	0
	0
	0
	1
	ITU-T Recommendation I.366.2 (SSCS to provide trunking) (only for AAL type 2)

	All other values are reserved.


Table 4-6/Q.2931 – AAL parameters information element (continued)
	Assured data (octet 9.1, bit 8, when octet 8.1 = 00010000 for AAL type 2)

	Bit

	
	8
	
	

	
	0
	
	Assured data transfer mechanism not used

	
	1
	
	Assured data transfer mechanism is used (Notes 4 and 5)

	Error detect (octet 9.1, bit 7, when octet 8.1 = 0001000 for AAL type 2)

	Bit

	
	7
	
	

	
	0
	
	Transmission error detection mechanism is not selected

	
	1
	
	Transmission error detection mechanism is selected

	Forward Maximum SSSAR-SDU (octets 10.1, 10.2 and 10.3, when octet 8.1 = 00010000 for AAL type 2)

	This field contains the binary coded value for the maximum length of an SSSAR-SDU that may be transmitted in the forward direction. (The value 00000000 is reserved.)

	Backward maximum SSSAR-SDU size (octets 11.1, 11.2 and 11.3, when octet 8.1 = 00010000 for AAL type 2)

	This field contains the binary coded value for the maximum length of an SSSAR-SDU that may be transmitted in the backward direction. (The value 00000000 is reserved.)

	Service category (octet 9.1, bits 8-5, when octet 8.1 = 00010001 for AAL type 2)

	Bits

	
	8
	7
	6
	5
	

	
	0
	0
	0
	0
	Audio service

	
	0
	0
	0
	1
	Multirate service

	All other values are reserved.

	CMD (octet 9.1, bit 4, when octet 8.1 = 00010001 and octet 9.1, bits 8-5 = 0000 or 0001 for AAL type 2)

	Bit

	
	4
	
	

	
	0
	
	Transport of circuit mode data disabled (Note 6)

	
	1
	
	Transport of circuit mode data enabled (Note 7)

	FMD (octet 9.1, bit 3, when octet 8.1 = 00010001 and octet 9.1, bits 8-5 = 0000 or 0001 for AAL type 2)

	Bit

	
	3
	
	

	
	0
	
	Transport of frame mode data disabled

	
	1
	
	Transport of frame mode data enabled

	FAX (octet 9.2, bit 6, when octet 8.1 = 00010001 and octet 9.1, bits 8-5 = 0000 for AAL type 2)

	Bit

	
	6
	
	

	
	0
	
	Transport of demodulated facsimile data disabled (Note 6)

	
	1
	
	Transport of demodulated facsimile data enabled

	CAS (octet 9.2, bit 5, when octet 8.1 = 00010001 and octet 9.1, bits 8-5 = 0000 for AAL type 2)

	Bit

	
	5
	
	

	
	0
	
	Transport of channel associated signalling bits disabled

	
	1
	
	Transport of channel associated signalling bits enabled


Table 4-6/Q.2931 – AAL parameters information element (continued)
	DTMF (octet 9.2, bit 4, when octet 8.1 = 00010001 and octet 9.1, bits 8-5 = 0000 for AAL type 2)

	Bit

	
	4
	
	

	
	0
	
	Transport of DTMF dialled digits disabled

	
	1
	
	Transport of DTMF dialled digits enabled

	MF-R1 (octet 9.2, bit 3, when octet 8.1 = 00010001 and octet 9.1, bits 8-5 = 0000 for AAL type 2)

	Bit

	
	3
	
	

	
	0
	
	Transport of R1 dialled digits disabled

	
	1
	
	Transport of R1 dialled digits enabled (see ITU-T Recommendations Q.320, Q.322, and Q.323)

	MF-R2 (octet 9.2, bit 2, when octet 8.1 = 00010001 and octet 9.1, bits 8-5 = 0000 for AAL type 2)

	Bit

	
	2
	
	

	
	0
	
	Transport of R2 dialled digits disabled

	
	1
	
	Transport of R2 dialled digits enabled (see ITU-T Recommendation Q.441)

	PCM Encoding (octet 9.2, bit 1, when octet 8.1 = 00010001 and octet 9.1 bits 8-5 = 0000)

	Bit

	
	1
	
	

	
	0
	
	A-law

	
	1
	
	(-law

	Maximum length of a frame mode data unit (octets 10.1 & 10.2, when octet 8.1 = 00010001 and octet 9.1, bit 3 = 1 for AAL type 2)

	This field contains the binary coded value for the maximum length of a frame mode data unit that may be transmitted. (The value 00000000 is reserved.)

	Profile Source (octet 11.1, bits 8-7 when octet 8.1 = 00010001 for AAL type 2)

	Bits

	
	8
	7
	
	
	

	
	0
	0
	ITU-T predefined profile used (See Annex P/I.366.2.)

Other predefined profile used

	
	0
	0
	

	All other values reserved.

	Predefined profile (octet 11.2, when octet 8.1 = 00010001 and octet 9.1, bits 8-5 = 0000 for AAL type 2)

	This field contains the binary coded value for the profile identifier of the profile which is administered by the organization identified by the OUI in the following octets (if octet 11.1, bits 8-7 = 01) or which is administered by the ITU-T in ITU-T Recommendation I.366.2 (if octet 11.1, bits 8-7 = 00).

	IEEE Organizationally Unique Identifier (OUI) (octets 11.3, 11.4 and 11.5, when octet 8.1 = 00010001; octet 9, bits 8-5 = 0000 and octet 11.1, bits 8-7 = 01

	When the "other predefined profile " is used, octets 11.3, 11.4, and 11.5 contain a globally administered Organizationally Unique Identifier (OUI) (as specified in IEEE Standard 802-1990, section 5.1); octet 0 of the OUI is mapped to octet 11.3 of the AAL parameters information element, and so on; the LSB of the OUI is mapped to bit 8 of the AAL parameters information element, the MSB of the OUI is mapped to bit 1 of the AAL parameters information element.


Table 4-6/Q.2931 – AAL parameters information element (end)
	NOTE 1 – The default AAL for voice is the AAL specified in Recommendation I.363.1 for voiceband signal transport based on 64 kbit/s (see Recommendation G.711/G.722).

NOTE 2 – For AAL for voice, no further parameters are specified beyond the ones given in part 1 of 8 of Figure 4-12.

NOTE 3 – When provisioning ATM connections that support AAL type 1 SDT service, the SDT protocol may distinguish between SDT block sizes with a value of "1" and SDT block sizes ranging from 2 to 216 ( 1. The special case using a block size of "1" is under study; see Recommendation I.363.1.

NOTE 4 – I.366.1 indicates that the parameters for the assured data transfer mechanism are specified in Q.2110.

NOTE 5 – The assured data transfer mechanism can only be selected if the transmission error detection mechanism is also detected.

NOTE 6 – If the value of this parameter is disabled, the user shall not change to Circuit Mode Data or Facsimile Demodulation operation.

NOTE 7 – If octet 9, bits 8-5 = 0001 (Multirate service), transport of Circuit Mode Data must be enabled.

NOTE 8 – In case of the absence of AAL parameter subfields, the following default values will apply:

–
Subtype: no default (must be signalled for AAL type 1).

–
CBR Rate: no default (must be signalled for AAL type 1).

–
Multiplier: no default (must be signalled for CBR Rate n = 64 kbit/s and n = 8 kbit/s).

–
Clock Frequency Recovery: default = null.

–
Error Correction: default = null.

–
SDT Block Size: default = no SDT is used.

–
Partially Filled Cells: default = partially filled cells method is not used, i.e. cells are filled completely.

–
Forward max. CPCS-SDU size: default = 65 535 octets.

–
Backward max. CPCS-SDU size: default = 65 535 octets.

–
MID range: default = 0-0 (no multiplexing via MID field).

–
SSCS-Type: default = null.

–
Maximum CPS-SDU size: default = 45.

–
Maximum number of multiplexed channels: default = 255.

–
Assured data: default = mechanism not used.

–
Error detect: default = not selected.

–
Forward maximum SSSAR-SDU size: default = 65 535 octets.

–
Backward maximum SSSAR-SDU size: default = 65 535 octets.

–
Service category: default = audio service.

–
PCM encoding: default = A-law.

–
Profile source: default = ITU-T and predefined profile = 1.

–
Fax, CAS, DTMF, MF-R1, MF-R2, CMD, FMD: default = disabled.

–
Maximum length of a frame mode data unit (for AAL type 2) = 65 535 octets.


2)
Annex F

Replace Annex F with the following text:
Annex F

ATM adaptation layer parameters indication and negotiation

This annex describes procedures for the use of the ATM adaptation layer parameters information element by endpoint equipment.

F.1
General

The purpose of the ATM adaptation layer parameters information element is to provide a means which may be used for conveying information related to the ATM adaptation layer between endpoints. The ATM adaptation layer parameters information element is transferred transparently between ATM endpoints by the network.

F.2
ATM adaptation layer parameter indication in the SETUP message

When the calling endpoint wishes to indicate to the called endpoint the AAL common part parameters and service specific part to be used during the call, the calling endpoint shall include an ATM adaptation layer parameters information element in the SETUP message. This information element is conveyed by the network and delivered to the called user.

The ATM adaptation layer parameters information element may include the following parameters for different AAL connection types:

a)
For AAL connection type 1

–
Subtype;

–
CBR rate;

–
Source clock frequency recovery method;

–
Error correction;

–
Structured data transfer blocksize;

–
Partially filled cells indicator.

b)
For AAL connection type 2

–
Maximum CPS-SDU size;

–
SSCS type and related parameters.

c)
For AAL connection type 3/4

–
Forward maximum CPCS-SDU size;

–
Backward maximum CPCS-SDU size;

–
MID range;

–
SSCS type and related parameters.

d)
For AAL connection type 5

–
Forward maximum CPCS-SDU size;

–
Backward maximum CPCS-SDU size;

–
SSCS type and related parameters.

NOTE – For AAL connection types 3/4 and 5, the Forward maximum CPCS-SDU size and Backward maximum CPCS-SDU size parameters shall either both be present or both be absent in the ATM adaptation layer parameters information element. For unidirectional ATM virtual connections, the Backward maximum CPCS-SDU size shall be set to "0".

If the called endpoint receives an ATM adaptation layer parameters information element in the SETUP message which contains the forward or backward maximum CPCS-SDU size but not both, the called endpoint should clear the call with cause No. 100, "invalid information element contents".

F.3
Maximum CPCS-SDU size negotiation

When the called user has received an ATM adaptation layer parameters information element in a SETUP message and the AAL type is either AAL 3/4 or AAL 5, the ATM adaptation layer parameters information element shall be included in the CONNECT message. The ATM adaptation layer parameters information element shall include the forward maximum CPCS-SDU size, indicating the size of the largest CPCS-SDU that the called user is able to receive, and the backward maximum CPCS-SDU size, indicating the size of the largest CPCS-SDU that it will transmit. The values for the forward and backward maximum CPCS-SDU size indicated in the CONNECT message shall not be greater than the values indicated by the calling user in the SETUP message. The ATM adaptation layer parameters information element will be conveyed to the calling user.

NOTE – For unidirectional ATM virtual connections, the backward maximum CPCS-SDU size shall be set to "0".

If the called user does not include the CPCS-SDU size in the CONNECT message, the calling user shall assume that the called user accepts the values of the forward and backward maximum CPCS‑SDU size indicated by the calling user in the SETUP message.

If the calling party cannot use the forward or backward maximum CPCS-SDU size indicated in the CONNECT message (i.e. because the value negotiated by the called party is unacceptably small), then the call shall be cleared with cause No. 93, "AAL parameters cannot be supported".

If the calling endpoint receives an ATM adaptation layer parameters information element in the CONNECT message which:

a)
contains octet groups other than the forward and backward maximum CPCS-SDU size and/or MID range;

b)
contains a maximum CPCS-SDU size which is greater than the maximum CPCS-SDU size which was sent in the SETUP message; or

c)
is missing the forward or backward maximum CPCS-SDU size;

the calling endpoint should clear the call with cause No. 100, "invalid information element contents".

F.4
MID range negotiation

When the called user receives the ATM adaptation layer parameters information element in the SETUP message which indicates AAL type 3/4, the called user shall check the MID range value. If the called user cannot support the indicated MID range but it can support a smaller range, the called user includes an ATM adaptation layer parameters information element in the CONNECT message containing the MID range that it can support.

The calling user will either accept the MID range contained in the CONNECT message or will clear the call with cause No. 93, "AAL parameters cannot be supported".

If the called user does not include the MID range in the CONNECT message, the calling user shall assume that the called user accepts the MID range indicated by the calling user in the SETUP message.

If the calling endpoint receives an ATM adaptation layer parameters information element in the CONNECT message which:

a)
contains octet groups other than the forward and backward maximum CPCS-SDU size and/or MID range; or

b)
contains a MID range which is greater than the MID range which was sent in the SETUP message;

the calling endpoint should clear the call with cause No. 100, "invalid information element contents".

F.5
Use of forward and backward maximum CPCS-SDU size by the AAL 3/4 or 5 entity in the user plane

The values of forward and backward maximum CPCS-SDU size resulting from AAL parameters negotiation shall be used by the AAL entities in the user plane. The AAL entity in the calling user equipment shall not send a CPCS-SDU size larger than the indicated value specified in the forward maximum CPCS-SDU size parameter, and may allocate its internal resources based on the value indicated in the backward maximum CPCS-SDU size parameter. Similarly, the AAL entity in the called user equipment shall not send a CPCS-SDU size larger than the indicated value specified in the backward maximum CPCS-SDU size parameter, and may allocate its internal resources based on the value indicated in the forward maximum CPCS-SDU size parameter.

F.6
Use of maximum CPS-SDU size by the AAL 2 entity in the user plane

The value for the maximum CPS-SDU size is either 45 octets (the default) or 64 octets and is not negotiated.

If the called user does not include the CPS-SDU size in the CONNECT message, the calling user shall assume that the called user accepts the default value of 45 octets for both the forward and backward maximum CPS-SDU size indicated by the calling user in the SETUP message.

If the calling party cannot use the forward or backward maximum CPS-SDU size indicated in the CONNECT message (i.e. because the value in the calling user SETUP message indicated 64 octets), then the call shall be cleared with cause No. 93, "AAL parameters cannot be supported".

	ITU-T  RECOMMENDATIONS  SERIES

	Series A
	Organization of the work of the ITU-T

	Series B
	Means of expression: definitions, symbols, classification

	Series C
	General telecommunication statistics

	Series D
	General tariff principles

	Series E
	Overall network operation, telephone service, service operation and human factors

	Series F
	Non-telephone telecommunication services

	Series G
	Transmission systems and media, digital systems and networks

	Series H
	Audiovisual and multimedia systems

	Series I
	Integrated services digital network

	Series J
	Transmission of television, sound programme and other multimedia signals

	Series K
	Protection against interference

	Series L
	Construction, installation and protection of cables and other elements of outside plant

	Series M
	TMN and network maintenance: international transmission systems, telephone circuits, telegraphy, facsimile and leased circuits

	Series N
	Maintenance: international sound programme and television transmission circuits

	Series O
	Specifications of measuring equipment

	Series P
	Telephone transmission quality, telephone installations, local line networks

	Series Q
	Switching and signalling

	Series R
	Telegraph transmission

	Series S
	Telegraph services terminal equipment

	Series T
	Terminals for telematic services

	Series U
	Telegraph switching

	Series V
	Data communication over the telephone network

	Series X
	Data networks and open system communications

	Series Y
	Global information infrastructure and Internet protocol aspects

	Series Z
	Languages and general software aspects for telecommunication systems










Recommendation Q.2931/Amd.2    (03/99)
iii

_1006154725.unknown

_1006154978.unknown

_1006170795.unknown

_1006154840.unknown

_1006152553.unknown

