

��

INTERNATIONAL  TELECOMMUNICATION  UNION��





ITU-T�Q.2962��TELECOMMUNICATION�STANDARDIZATION  SECTOR�OF  ITU�(05/98)   ��









SERIES Q: SWITCHING AND SIGNALLING

Broadband ISDN – B-ISDN application protocols for access signalling





Digital Subscriber Signalling System No. 2 – Connection characteristics negotiation during call/connection establishment phase



ITU-T  Recommendation  Q.2962

(Previously  CCITT  Recommendation)



�

ITU-T  Q-SERIES  RECOMMENDATIONS

SWITCHING AND SIGNALLING



���SIGNALLING IN THE INTERNATIONAL MANUAL SERVICE�Q.1–Q.3��INTERNATIONAL AUTOMATIC AND SEMI-AUTOMATIC WORKING�Q.4–Q.59��FUNCTIONS AND INFORMATION FLOWS FOR SERVICES IN THE ISDN�Q.60–Q.99��CLAUSES APPLICABLE TO ITU-T STANDARD SYSTEMS�Q.100–Q.119��SPECIFICATIONS OF SIGNALLING SYSTEMS No. 4 AND No. 5�Q.120–Q.249��SPECIFICATIONS OF SIGNALLING SYSTEM No. 6�Q.250–Q.309��SPECIFICATIONS OF SIGNALLING SYSTEM R1�Q.310–Q.399��SPECIFICATIONS OF SIGNALLING SYSTEM R2�Q.400–Q.499��DIGITAL EXCHANGES�Q.500–Q.599��INTERWORKING OF SIGNALLING SYSTEMS�Q.600–Q.699��SPECIFICATIONS OF SIGNALLING SYSTEM No. 7�Q.700–Q.849��DIGITAL SUBSCRIBER SIGNALLING SYSTEM No. 1�Q.850–Q.999��PUBLIC LAND MOBILE NETWORK�Q.1000–Q.1099��INTERWORKING WITH SATELLITE MOBILE SYSTEMS�Q.1100–Q.1199��INTELLIGENT NETWORK�Q.1200–Q.1999��BROADBAND ISDN�Q.2000–Q.2999��General aspects�Q.2000–Q.2099��Signalling ATM adaptation layer (SAAL)�Q.2100–Q.2199��Signalling network protocols�Q.2200–Q.2299��Common aspects of B-ISDN application protocols for access signalling and network signalling and interworking�Q.2600–Q.2699��B-ISDN application protocols for the network signalling�Q.2700–Q.2899��B-ISDN application protocols for access signalling�Q.2900–Q.2999�����For further details, please refer to ITU-T List of Recommendations.



�ITU-T  RECOMMENDATION  Q.2962

DIGITAL  SUBSCRIBER  SIGNALLING  SYSTEM  No. 2  –  CONNECTION  CHARACTERISTICS  NEGOTIATION  DURING  CALL/CONNECTION  ESTABLISHMENT  PHASE





Summary

This Recommendation defines the operation of the Digital Subscriber Signalling System No. 2 (DSS 2) for the handling of the Connection characteristics negotiation during Call/Connection establishment phase, that may be supported, as a network option, at the TB or at the coincident SB and TB reference point of the User-to-Network Interface of the Broadband Integrated Services Digital Network (B-ISDN). The Connection characteristics negotiation capability defined in this Recommendation enables in particular the negotiation of cell rate traffic parameters using an alternative set of values given in an alternative ATM traffic descriptor information element, or negotiation of cell rate traffic parameters by using a minimum acceptable traffic descriptor information element containing minimum acceptable traffic parameter values.

This revised edition extends the scope of ATM traffic parameter negotiation capability in Q.2962 (1996) to include the possible negotiation of SCR and MBS parameters within a range down to minimum acceptable SCR and MBS parameters values.







Source

ITU-T Recommendation Q.2962 was revised by ITU-T Study Group 11 (1997-2000) and was approved under the WTSC Resolution No. 1 procedure on the 15th of May 1998.



























�

FOREWORD

ITU (International Telecommunication Union) is the United Nations Specialized Agency in the field of tele�communi�cations. The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of the ITU. The ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to stan�dard�izing telecommunications on a worldwide basis.

The World Telecommunication Standardization Conference (WTSC), which meets every four years, establishes the topics for study by the ITU�T Study Groups which, in their turn, produce Recommendations on these topics.

The approval of Recommendations by the Members of the ITU�T is covered by the procedure laid down in WTSC Resolution No. 1.

In some areas of information technology which fall within ITU-T’s purview, the necessary standards are prepared on a collaborative basis with ISO and IEC.

NOTE

In this Recommendation, the expression "Administration" is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

INTELLECTUAL  PROPERTY  RIGHTS

The ITU draws attention to the possibility that the practice or implementation of this Recommendation may involve the use of a claimed Intellectual Property Right. The ITU takes no position concerning the evidence, validity or applicability of claimed Intellectual Property Rights, whether asserted by ITU members or others outside of the Recommendation development process.

As of the date of approval of this Recommendation, the ITU had not received notice of intellectual property, protected by patents, which may be required to implement this Recommendation. However, implementors are cautioned that this may not represent the latest information and are therefore strongly urged to consult the TSB patent database.

ã  ITU  1998

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

�



CONTENTS

Page

1	Scope		1

2	References		1

3	Definitions		1

4	Abbreviations		2

5	Description		2

6	Operational requirements		2

6.1	Provision and withdrawal		2

6.2	Requirements on the originating network side		2

6.3	Requirements on the destination network side		2

7	Primitive definitions and state definitions		2

7.1	Primitive definitions		2

7.2	State definitions		2

8	Coding requirements		3

8.1	Messages		3

8.1.1	Modification of messages in Recommendation Q.2962		3

8.2	Information elements		4

8.2.1	Alternative ATM traffic descriptor		4

8.2.2	Minimum acceptable ATM traffic descriptor		4

9	Signalling procedures at the coincident SB and TB reference point		5

9.1	Negotiating the connection characteristics at the origination interface		5

9.1.1	Negotiation request (originating interface)		5

9.1.2	Traffic parameter negotiation procedures (originating interface)		5

9.1.3	Negotiation acceptance (originating interface)		7

9.2	Negotiating the connection characteristics at the destination interface		7

9.2.1	Negotiation request (destination interface)		7

9.2.2	Traffic parameter negotiation procedures (destination interface)		7

9.2.3	Negotiation confirmation (destination interface)		8

10	Procedures at the TB reference point for interworking with private B-ISDNs		8

11	Interworking with other networks		8

12	Interactions with supplementary services		8

13	Parameter values		8

14	Dynamic description (SDLs)		8

Appendix I – Message flow diagram for negotiation of traffic parameters at call/connection setup		9

Appendix II – Guidelines for the use of the instruction indicator		9

















�Recommendation Q.2962

Recommendation Q.2962     (05/98)

DIGITAL  SUBSCRIBER  SIGNALLING  SYSTEM  No. 2  –  CONNECTION  CHARACTERISTICS  NEGOTIATION  DURING  CALL/CONNECTION  ESTABLISHMENT  PHASE

(revised in 1998)

1	Scope

This Recommendation specifies the procedures for negotiation of connection characteristics by means of the Digital Subscriber Signalling System No. 2 (DSS 2) protocol at the TB reference point or coincident SB and TB reference point (as defined in Recommendation I.413) of the Broadband Integrated Services Digital Network (B-ISDN).

This Recommendation specifies the signalling protocol to negotiate the traffic parameters that are specified in Recommendations Q.2931 and Q.2961.1 for basic call/connection control. The capability described in this Recommendation is also applicable for negotiating the connection characteristics for the first party of a point-to-multipoint call/connections as specified in Recommendation Q.2971. The capabilities in this Recommendation are only applicable during the call/connection establishment phase.

This Recommendation is part of the DSS 2 family of ITU-T Recommendations. It specifies extensions to Recommendations Q.2931, Q.2961.1 and Q.2971; it does not repeat states, information elements, messages and procedures contained therein but only specifies extensions related to traffic parameter negotiation.

2	References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; all users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

[1]	ITU-T Recommendation Q.2931 (1995), Digital Subscriber Signalling System No. 2 (DSS 2) – User Network Interface (UNI) layer 3 specification for basic call/connection control.

[2]	ITU-T Recommendation Q.2971 (1995), Broadband Integrated Services Digital Network (B�ISDN) – Digital Subscriber Signalling System No. 2 (DSS 2) – User Network Interface layer 3 specification for point-to-multipoint call/connection control.

[3]	ITU-T Recommendation I.327 (1993), B-ISDN functional architecture.

[4]	ITU-T Recommendation Q.2961.1 (1995), Digital Subscriber Signalling System No. 2 (DSS 2) – Additional traffic parameters: Additional signalling capabilities to support traffic parameters for the tagging option and the sustainable cell rate parameter set.

3	Definitions

The definitions of Annex J/Q.2931 [1] apply.

�4	Abbreviations

This Recommendation uses the following abbreviations:

ATM		Asynchronous Transfer Mode�

B-ISDN		Broadband Integrated Services Digital Network�

CLP			Cell Loss Priority�

DSS 2		Digital Subscriber Signalling System No. 2�

OAM		Operations, Administration and Maintenance�

5	Description

This Recommendation specifies the signalling protocol for negotiation of connection characteristics for point-to-point call/connections and for the first party of point-to-multipoint call/connections. The negotiation capabilities are only applicable during the call/connection establishment phase. In particular, the following capabilities are specified:

–	negotiation of a set of connection characteristics using an alternative traffic descriptor; and

–	negotiation of individual traffic parameters using a minimum traffic descriptor.

In the case of the use of the alternative ATM traffic descriptor information element, the parameters of the information element are handled as a single set whereas the minimum acceptable ATM traffic descriptor information element allows the specification of a range of values for parameters which are then handled independently for the selection of their respective values. The use of both the alternative ATM traffic descriptor information element and the minimum acceptable ATM traffic descriptor information element allows negotiation of any relevant traffic parameters (i.e. peak cell rate, sustainable cell rate and maximum burst size parameters, depending on the ATM transfer capability actually used for the connection).

6	Operational requirements

6.1	Provision and withdrawal

It is a user and a network option to provide the procedures described in this Recommendation. If implemented, the procedures of this Recommendation may be provided as a subscription option to the served user on the origination side.

6.2	Requirements on the originating network side

See 6.1.

6.3	Requirements on the destination network side

See 6.1.

7	Primitive definitions and state definitions

7.1	Primitive definitions

Clause 8/Q.2931 [1] shall apply.

7.2	State definitions

No additional states to those specified in Recommendations Q.2931 [1] and Q.2971 [2] are required.

�8	Coding requirements

8.1	Messages

The messages structure, functional definition and information contents are specified according to the rules given in clause 3/Q.2931 [1].

8.1.1	Modification of messages in Recommendation Q.2962

Table 8-1 lists the existing Q.2931 messages that have had their contents modified to support connection characteristics negotiation during the establishment phase of the call/connection.

Table 8-1/Q.2962 – Modified Q.2931 messages

Message�Reference��SETUP�8.1.1.1��CONNECT�8.1.1.2��

8.1.1.1	SETUP

This message is sent by the calling user to the network and by the network to the called user to initiate B-ISDN call and connection establishment. See Table 8-2 for additions to the structure of this message shown in Table 3-8/Q.2931 [1] and in 8.1/Q.2961.1 [4].

Table 8-2/Q.2962 – SETUP message additional content

Message type:	SETUP

Significance:	Global

Direction:	Both��Information element�Reference�Direction�Type�Length��Alternative ATM traffic descriptor�8.2.1�Both�O (Note)�4-28��Minimum acceptable ATM traffic descriptor�8.2.2�Both�O (Note)�4-28��NOTE – Either the alternative ATM traffic descriptor information element or the minimum acceptable ATM traffic descriptor information element, but not both, shall be included in the SETUP message when traffic parameters are negotiable.��

8.1.1.2	CONNECT

This message is sent by the called user to the network and by the network to the calling user to indicate call/connection acceptance by the called user. See Table 8-3 for additions to the structure of this message shown in Table 3-2/Q.2931 [1] and in 8.1/Q.2961.1 [4].

Table 8-3/Q.2962 – CONNECT message additional content

Message type:	CONNECT

Significance:	Global

Direction:	Both��Information element �Reference�Direction�Type�Length��ATM traffic descriptor�4.5.6/Q.2931�8.2.1/Q.2961.1�Both�O (Note)�4-30��NOTE – Included to specify the traffic parameter values allocated for the call/connection if one or more traffic parameters were negotiable in the SETUP message.��

�8.2	Information elements

8.2.1	Alternative ATM traffic descriptor

The purpose of the alternative ATM traffic descriptor information element is to specify an alternative ATM traffic descriptor for the negotiation of traffic parameters during call/connection setup.

The alternative ATM traffic descriptor information element is coded as shown in Figure 8�1. The maximum length of this information element is 28 octets.



8�7�6�5�4�3�2�1�Octet��Alternative ATM traffic descriptor information element identifier���1�0�0�0�0�0�1�0�1��ext.�Coding standard�Information element instruction field���1���Flag�Reserved�Action indicator�2��Length of alternative ATM traffic descriptor contents�3����������4��Further octets as contents of ATM traffic descriptor (Figure 4-13/Q.2931 and�Figure 1/Q.2961.1) – Note�5*����������16*��	NOTE – The octet group 17 (Traffic management options) in Figure 1/Q.2961.1 [4] is not applicable.

Figure 8-1/Q.2962 – Alternative ATM traffic descriptor information element

The alternative ATM traffic descriptor information element can have any combination of traffic parameters that is allowed for the ATM traffic descriptor information element for the specified ATM transfer capability. Within a single SETUP message, the combination of traffic parameters may be different for these two information elements. The alternative bandwidth requirements must be reduced compared to those originally requested.

8.2.2	Minimum acceptable ATM traffic descriptor

The purpose of the minimum acceptable ATM traffic descriptor information element is to specify the minimum acceptable ATM traffic parameters in the negotiation of traffic parameters during call/connection setup. The minimum acceptable ATM traffic parameters are the lowest values that the user is willing to accept for the call/connection.

The minimum acceptable ATM traffic descriptor information element is coded as shown in Figure 8�2. The maximum length of this information element is 28 octets.

In a SETUP message, a traffic parameter [i.e. forward peak cell rate (CLP = 0), forward peak cell rate (CLP = 0 + 1), backward peak cell rate (CLP = 0), backward peak cell rate (CLP = 0 + 1), forward sustainable cell rate (CLP = 0 + 1), forward sustainable cell rate (CLP = 0), backward sustainable cell rate (CLP = 0 + 1), backward sustainable cell rate (CLP = 0), forward maximum burst size (CLP = 0 + 1), forward maximum burst size (CLP = 0), backward maximum burst size (CLP = 0 + 1) or backward maximum burst size (CLP = 0)] is allowed in the minimum acceptable ATM traffic descriptor information element only if the corresponding traffic parameter is in the ATM traffic descriptor information element of the SETUP message [e.g. if there is no forward peak cell rate (CLP = 0) parameter in the ATM traffic descriptor information element of a SETUP message, then the forward peak cell rate (CLP = 0) is not allowed in the minimum acceptable ATM traffic descriptor information element of the SETUP message].

�

8�7�6�5�4�3�2�1�Octet��1�0�0�0�0�0�0�1�1��Minimum acceptable ATM traffic descriptor information element identifier����������ext.�Coding standard��Information element instruction field�����2��1���Flag�Reserved�Action indicator�����Length of minimum acceptable ATM traffic descriptor contents��������3����������4��Further octets as contents of ATM traffic descriptor (Figure 4-13/Q.2931 and�Figure 1/Q.2961.1) – Note�5*����������16*��	NOTE – The octet group 17 (Traffic management options) in Figure 1/Q.2961.1 [4] is not applicable.

Figure 8-2/Q.2962 – Minimum acceptable ATM traffic descriptor information element

9	Signalling procedures at the coincident SB and TB reference point

9.1	Negotiating the connection characteristics at the origination interface

9.1.1	Negotiation request (originating interface)

The user initiates the negotiation of the connection characteristics by including, in addition to the ATM traffic descriptor information element, either the minimum acceptable ATM traffic descriptor information element or the alternative ATM traffic descriptor information element, but not both, in the SETUP message. In the case of the use of the alternative ATM traffic descriptor information element, the parameters of the information element are handled as a single entity whereas the minimum acceptable ATM traffic descriptor information element allows the specification of a range of values for parameters which are handled independently. If a minimum acceptable ATM traffic descriptor information element is used, the traffic parameter values indicated shall be lesser than the corresponding traffic parameter values specified in the ATM traffic descriptor information element.

If point-to-multipoint procedures are supported, the user may initiate negotiation for the first party of a point-to-multipoint call. If the user initiates negotiation, the user shall send ADD PARTY messages only if the link is in the active state. If an ADD PARTY message is received for a call for which the SETUP message contained either a minimum acceptable ATM traffic descriptor information element or an alternative ATM traffic descriptor information element while in the call delivered state, an ADD PARTY REJECT message with a cause information element indicating cause No. 111, "protocol error, unspecified", shall be sent in response.

9.1.2	Traffic parameter negotiation procedures (originating interface)

When both the minimum acceptable ATM traffic descriptor and the alternative ATM traffic descriptor information elements are present in a SETUP message, the call shall be rejected as specified in 5.4.2/Q.2931 with cause No. 73, "unsupported combination of traffic parameters".

If the parameters of the alternative ATM traffic descriptor information element or minimum acceptable ATM traffic descriptor information element are not according to the allowed combinations as specified in 8.2.1 and 8.2.2 respectively, the network shall handle these information elements as if they were non-mandatory information elements with content error as specified in 5.6.8/Q.2931.

�When the minimum acceptable ATM traffic descriptor information element is included in the SETUP message and the network is able to provide the traffic parameter values specified in the ATM traffic descriptor information element, the network shall progress the connection establishment request with both the ATM traffic descriptor information element and the minimum acceptable ATM traffic descriptor information element.

When the alternative ATM traffic descriptor information element is included in the SETUP message and the network is able to provide the traffic parameter values specified in the ATM traffic descriptor information element and the network is able to provide the traffic parameter values specified in the alternative ATM traffic descriptor information element, the network shall progress the call/connection establishment request with both the ATM traffic descriptor information element and the alternative ATM traffic descriptor information element.

When the alternative ATM traffic descriptor information element is included in the SETUP message and the network is able to provide the traffic parameter values specified in the ATM traffic descriptor information element and the network is not able to provide the traffic parameter values specified in the alternative ATM traffic descriptor information element, the network shall progress the call/connection establishment request with the ATM traffic descriptor information element and without the alternative ATM traffic descriptor information element.

If the network is not able to provide some of the traffic parameter values indicated in the ATM traffic descriptor information element and the minimum acceptable ATM traffic descriptor information element is included, the procedures of 9.1.2.1 shall apply.

If the network is not able to provide some of the traffic parameter values indicated in the ATM traffic descriptor information element and the alternative ATM traffic descriptor information element is included, the procedures of 9.1.2.2 shall apply.

When the OAM traffic descriptor information element is present, the allocation of bandwidth for OAM flow is based on the ATM traffic descriptor agreed in the CONNECT message. Since the OAM flow allocation is bidirectional (see Note 2 of Table 4-22/Q.2931), the available user cell rate in one direction can be affected by negotiation of bandwidth in the other direction.

9.1.2.1	Minimum acceptable ATM traffic parameter negotiation

If the network is not able to provide some of the traffic parameter values indicated in the ATM traffic descriptor information element but able to provide at least their corresponding values in the minimum acceptable ATM traffic descriptor information element, the network shall progress the connection establishment request after adjusting the traffic parameter values indication in the ATM traffic descriptor information element. The adjusted parameter values will support at least the corresponding minimum acceptable values. If some of the parameters in the minimum acceptable ATM traffic descriptor information element are still less than the corresponding parameters in the modified ATM traffic descriptor information element, then the call/connection shall be progressed with the minimum acceptable ATM traffic descriptor information element containing all such parameters, in addition to the modified ATM traffic descriptor information element. Otherwise, the call shall progress with the modified ATM traffic descriptor information element and without the minimum acceptable ATM traffic descriptor information element.

If the network is not able to provide at least the traffic parameter values indicated in the minimum acceptable ATM traffic descriptor information element, the network shall reject the connection establishment request, as specified in 5.4.2/Q.2931 with cause No. 37, "user cell rate unavailable".

9.1.2.2	Alternative traffic parameter negotiation

If the network is not able to provide the ATM traffic descriptor indicated in the ATM traffic descriptor information element but able to provide the ATM traffic descriptor indicated in the alternative ATM traffic descriptor information element, the network shall progress the connection establishment request by using the contents of the alternative ATM traffic descriptor information element as the ATM traffic descriptor.

If the network can provide neither the ATM traffic descriptor indicated in the ATM traffic descriptor information element nor the ATM traffic descriptor indicated in the alternative ATM traffic descriptor information element, the network shall reject the connection establishment request, as specified in 5.4.2/Q.2931 with cause No. 37, "user cell rate unavailable".

�9.1.3	Negotiation acceptance (originating interface)

Upon receiving an indication that the request has been accepted, the network shall send a CONNECT message across the UNI and enter the active state. The message returned to the user shall contain the ATM traffic descriptor information element indicating the traffic parameter values finally allocated to the connection.

If no ATM traffic descriptor information element is included in the CONNECT message, the connection characteristics specified in the ATM traffic descriptor information element sent in the SETUP message shall apply.

9.2	Negotiating the connection characteristics at the destination interface

9.2.1	Negotiation request (destination interface)

The network shall send a SETUP message containing, in addition to the ATM traffic descriptor information element, either the minimum acceptable ATM traffic descriptor information element or the alternative ATM traffic descriptor information element when traffic parameters are negotiable.

9.2.2	Traffic parameter negotiation procedures (destination interface)

If the user is able to provide the connection characteristics specified in the ATM traffic descriptor information element, the user shall progress the connection establishment request.

If the user is not able to provide some of the traffic parameter values indicated in the ATM traffic descriptor information element and the minimum acceptable ATM traffic descriptor information element is included, the procedures of 9.2.2.1 shall apply.

If the user is not able to provide the ATM traffic descriptor indicated by the ATM traffic descriptor information element and the alternative ATM traffic descriptor information element is included, the procedures of 9.2.2.2 shall apply.

When the OAM traffic descriptor information element is present, the allocation of bandwidth for OAM flow is based on the ATM traffic descriptor agreed in the CONNECT message. Since the OAM flow allocation is bidirectional (see Note 2 of Table 4-22/Q.2931), the available user cell rate in one direction can be affected by negotiation of bandwidth in the other direction.

9.2.2.1	Minimum acceptable traffic parameter negotiation

If the user is not able to provide some of the traffic parameter values indicated in the ATM traffic descriptor information element but is able to provide at least their corresponding values in the minimum acceptable ATM traffic descriptor information element, the user shall progress the connection establishment request.

If the user is not able to provide at least the traffic parameter values indicated in the minimum acceptable ATM traffic descriptor information element, the user shall reject the connection establishment request, as specified in 5.4.2/Q.2931 with cause No. 47, "Resources not available, unspecified".

9.2.2.2	Alternative traffic parameter negotiation

If the user is not able to provide the ATM traffic descriptor indicated by the ATM traffic descriptor information element but is able to provide the ATM traffic descriptor indicated by the alternative ATM traffic descriptor information element, the user shall progress the connection establishment request on the basis of the alternative ATM traffic descriptor information element.

If the user can provide neither the ATM traffic descriptor indicated by the ATM traffic descriptor information element nor the ATM traffic descriptor indicated by the alternative ATM traffic descriptor information element, the user shall reject the connection establishment request, as specified in 5.4.2/Q.2931 with cause No. 47, "Resources not available, unspecified".

�9.2.3	Negotiation confirmation (destination interface)

When the user receives a SETUP message and wishes to accept the request, the user responds with a CONNECT message and enters the appropriate connection state. The message returned by the user shall contain the ATM traffic descriptor information element with the accepted connection characteristics. The ATM traffic descriptor information element shall contain the same set of parameters as received in the ATM traffic descriptor information element, if negotiation has proceeded on the basis of the minimum acceptable ATM traffic descriptor information element, or the alternative ATM traffic descriptor information element if this has been used for negotiation.

If no ATM traffic descriptor information element is included in the CONNECT message, the connection characteristics specified in the ATM traffic descriptor information element sent in the SETUP message shall apply.

If the user rejects the request, it will send the appropriate message (RELEASE COMPLETE) and enter the null state.

10	Procedures at the TB reference point for interworking with private B-ISDNs

The procedures of clause 9 apply.

11	Interworking with other networks

Negotiation procedures are terminated at interworking points with other networks. An interworking unit can apply the procedures of 9.2 in order to obtain an acceptable ATM traffic descriptor to allow interworking to proceed.

12	Interactions with supplementary services

There are no interactions with supplementary services.

13	Parameter values

None required.

14	Dynamic description (SDLs)

None required.



















�Appendix  I��Message flow diagram for negotiation of traffic parameters at call/connection setup





� EMBED Word.Picture.6  ���

FIGURE I.1/Q.2962...[D01] = 11.5 CM



Appendix  II��Guidelines for the use of the instruction indicator

It is suggested that the instruction indicator for the alternative ATM traffic descriptor and minimum acceptable ATM traffic descriptor information elements be encoded to indicate "discard, proceed and report status".













�

ITU-T  RECOMMENDATIONS  SERIES��Series A�Organization of the work of the ITU-T��Series B�Means of expression: definitions, symbols, classification��Series C�General telecommunication statistics��Series D�General tariff principles��Series E�Overall network operation, telephone service, service operation and human factors��Series F�Non-telephone telecommunication services��Series G�Transmission systems and media, digital systems and networks��Series H�Audiovisual and multimedia systems��Series I�Integrated services digital network��Series J�Transmission of television, sound programme and other multimedia signals��Series K�Protection against interference��Series L�Construction, installation and protection of cables and other elements of outside plant��Series M�TMN and network maintenance: international transmission systems, telephone circuits, telegraphy, facsimile and leased circuits��Series N�Maintenance: international sound programme and television transmission circuits��Series O�Specifications of measuring equipment��Series P�Telephone transmission quality, telephone installations, local line networks��Series Q�Switching and signalling��Series R�Telegraph transmission��Series S�Telegraph services terminal equipment��Series T�Terminals for telematic services��Series U�Telegraph switching��Series V�Data communication over the telephone network��Series X�Data networks and open system communications��Series Y�Global information infrastructure��Series Z�Programming languages��

_______________











�PAGE�ii�	�styleref head_foot�Recommendation Q.2962     (05/98)�



		�styleref head_foot�Recommendation Q.2962     (05/98)�	�PAGE�iii�



�PAGE�8�	�styleref head_foot�Recommendation Q.2962     (05/98)�



		�styleref head_foot�Recommendation Q.2962     (05/98)�	�PAGE�9�













�PAGE \# "'Page: '#'�'"  �Page: 2���A: ATM		Asynchronous Transfer Mode

�PAGE \# "'Page: '#'�'"  �Page: 2���A: B-ISDN		Broadband Integrated Services Digital Network

�PAGE \# "'Page: '#'�'"  �Page: 2���A: CLP			Cell Loss Priority

�PAGE \# "'Page: '#'�'"  �Page: 2���A: DSS 2		Digital Subscriber Signalling System No. 2

�PAGE \# "'Page: '#'�'"  �Page: 2���A: OAM		Operations, Administration and Maintenance






