
	[image: image1.wmf]
	INTERNATIONAL  TELECOMMUNICATION  UNION

	
	

	ITU-T
	Q.2963.1

	TELECOMMUNICATION
STANDARDIZATION  SECTOR
OF  ITU
	(12/1999)   

	
	SERIES Q: SWITCHING AND SIGNALLING

Broadband ISDN – B-ISDN application protocols for access signalling



	
	Digital subscriber signalling system No. 2 – Connection modification: Peak cell rate modification by the connection owner

	
	ITU‑T  Recommendation  Q.2963.1

(Formerly  CCITT  Recommendation)


ITU-T Q-SERIES  RECOMMENDATIONS

SWITCHING AND SIGNALLING

	
	

	SIGNALLING IN THE INTERNATIONAL MANUAL SERVICE
	Q.1–Q.3

	INTERNATIONAL AUTOMATIC AND SEMI-AUTOMATIC WORKING
	Q.4–Q.59

	FUNCTIONS AND INFORMATION FLOWS FOR SERVICES IN THE ISDN
	Q.60–Q.99

	CLAUSES APPLICABLE TO ITU-T STANDARD SYSTEMS
	Q.100–Q.119

	SPECIFICATIONS OF SIGNALLING SYSTEMS No. 4 AND No. 5
	Q.120–Q.249

	SPECIFICATIONS OF SIGNALLING SYSTEM No. 6
	Q.250–Q.309

	SPECIFICATIONS OF SIGNALLING SYSTEM R1
	Q.310–Q.399

	SPECIFICATIONS OF SIGNALLING SYSTEM R2
	Q.400–Q.499

	DIGITAL EXCHANGES
	Q.500–Q.599

	INTERWORKING OF SIGNALLING SYSTEMS
	Q.600–Q.699

	SPECIFICATIONS OF SIGNALLING SYSTEM No. 7
	Q.700–Q.849

	DIGITAL SUBSCRIBER SIGNALLING SYSTEM No. 1
	Q.850–Q.999

	PUBLIC LAND MOBILE NETWORK
	Q.1000–Q.1099

	INTERWORKING WITH SATELLITE MOBILE SYSTEMS
	Q.1100–Q.1199

	INTELLIGENT NETWORK
	Q.1200–Q.1699

	SIGNALLING REQUIREMENTS AND PROTOCOLS FOR IMT-2000
	Q.1700–Q.1799

	BROADBAND ISDN
	Q.2000–Q.2999

	General aspects
	Q.2000–Q.2099

	Signalling ATM adaptation layer (SAAL)
	Q.2100–Q.2199

	Signalling network protocols
	Q.2200–Q.2299

	Common aspects of B-ISDN application protocols for access signalling and network signalling and interworking
	Q.2600–Q.2699

	B-ISDN application protocols for the network signalling
	Q.2700–Q.2899

	B-ISDN application protocols for access signalling
	Q.2900–Q.2999

	
	


For further details, please refer to the list of ITU-T Recommendations.

	ITU-T Recommendation Q.2963.1

Digital subscriber signalling system No. 2 – Connection modification:
Peak cell rate modification by the connection owner



	Summary

This ITU-T Recommendation defines the operation of the Digital Subscriber Signalling System No. 2 (DSS2) for the handling of the Connection characteristics modification feature that may be supported, as a network option, at the TB or at the coincident SB and TB reference point of the User-to-Network Interface of the Broadband Integrated Services Digital Network (B-ISDN). This edition of ITU-T Recommendation Q.2963.1 has been prepared to align the inconsistencies in ITU‑T Recommendation Q.2963.1 (07/96), and to be consistent with ITU-T Recommendation Q.2931 by which the encoding of the Broadband report type information element is aligned in conjunction with Amendment 4/Q.2931.


	Source

ITU‑T Recommendation Q.2963.1 was revised by ITU‑T Study Group 11 (1997‑2000) and approved under the WTSC Resolution 1 procedure on 3 December 1999.


	


FOREWORD

The International Telecommunication Union (ITU) is the United Nations specialized agency in the field of telecommunications. The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of ITU. ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Conference (WTSC), which meets every four years, establishes the topics for study by the ITU‑T study groups which, in turn, produce Recommendations on these topics.

The approval of ITU-T Recommendations is covered by the procedure laid down in WTSC Resolution 1.

In some areas of information technology which fall within ITU-T’s purview, the necessary standards are prepared on a collaborative basis with ISO and IEC.

NOTE

In this Recommendation, the expression "Administration" is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

INTELLECTUAL PROPERTY RIGHTS
ITU draws attention to the possibility that the practice or implementation of this Recommendation may involve the use of a claimed Intellectual Property Right. ITU takes no position concerning the evidence, validity or applicability of claimed Intellectual Property Rights, whether asserted by ITU members or others outside of the Recommendation development process.

As of the date of approval of this Recommendation, ITU had not received notice of intellectual property, protected by patents, which may be required to implement this Recommendation. However, implementors are cautioned that this may not represent the latest information and are therefore strongly urged to consult the TSB patent database.

(  ITU  2001

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

CONTENTS


Page

1
Scope

1
2
References

1
3
Definitions

1
4
Abbreviations

2
5
Description

2
5.1
Modifiable connections

2
5.2
Modification of a point-to-point connection

2
5.3
Increasing or decreasing the Peak Cell Rate

3
6
Operational requirements

3
6.1
Provision and withdrawal

3
6.2
Requirements on the originating network side

3
6.3
Requirements on the destination network side

3
7
Primitive and state definitions

3
7.1
Primitive definitions

3
7.2
State definitions

3
8
Coding requirements

4
8.1
Messages

4
8.1.1
MODIFY REQUEST

4
8.1.2
MODIFY ACKNOWLEDGE

5
8.1.3
MODIFY REJECT

5
8.2
Coding of specific message types and specific information elements

5
8.2.1
Coding of specific message types

5
8.2.2
Coding of specific information elements

6
9
Signalling procedures at the coincident SB and TB reference point

6
9.1
Modification procedures at the requesting entity

6
9.1.1
Modification request

6
9.1.2
Modification acknowledgement

6
9.1.3
Indication of modification rejection

7
9.1.4
Response to STATUS messages while in the modify request state

7
9.1.5
No response to modification request

8
9.2
Modification procedures at the responding entity

8
9.2.1
Modification indication

8
9.2.2
Modification acceptance

8
9.2.3
Modification confirmation

9
9.2.4
Modification rejection

9

Page

9.2.5
Exceptional procedures

9
9.3
Transit entity conveyance of CONNECTION AVAILABLE messages

10
10
Procedures at the TB reference point for interworking with private B-ISDNs

10
11
Interworking with other networks

10
12
Interworking with supplementary services

10
13
Parameter values

10
13.1
Timers at the requesting entity

10
13.2
Timers at the responding entity

11
13.3
Timers at the transit entity

11
14
Dynamic description SDLs

11
Annex A ( Message flow diagrams for modification

28
Appendix I ( Example configuration of user and  network behaviour uring modification procedures

30
Appendix II ( Guidelines for the use of Instruction indicators

32
ITU-T Recommendation Q.2963.1

Digital subscriber signalling system No. 2 – Connection modification:
Peak cell rate modification by the connection owner

1
Scope

This ITU-T Recommendation specifies the signalling protocol for peak cell rate modification for the Broadband Integrated Services Digital Network (B-ISDN) at the TB reference point or coincident SB and TB reference point (as defined in ITU-T Recommendation I.413 [1]) by means of the Digital Subscriber Signalling System No. 2 (DSS2). It is the initial Recommendation in a family of Recommendations that concern the modification of ATM traffic parameters in B-ISDN connections.

In addition, this ITU-T Recommendation specifies the protocol requirements at the TB reference point where the service is provided to the user via a private B-ISDN.

The capability described in this ITU-T Recommendation enables the connection owner to modify the peak cell rate for call/connections that have already been established.

Peak cell rate modification is applicable to all connection-oriented telecommunication services that are based on single point-to-point calls/connections. The peak cell rate modification for point‑to‑multipoint calls/connections is outside the scope of this ITU-T Recommendation.

This ITU-T Recommendation is applicable to equipment, supporting peak cell rate modification, to be attached at either side of a TB reference point or coincident SB and TB reference point when used as an access to the public B-ISDN.

2
References

The following ITU‑T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision, all users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

[1]
ITU-T Recommendation I.413 (1993), B-ISDN user-network interface.

[2]
ITU-T Recommendation Q.2931 (1995) plus Amd.1 (06/97), Amd.2 (03/99), Amd.3 (03/99), and Amd.4 (12/99), Digital Subscriber Signalling System No. 2 – User-Network Interface (UNI) layer 3 specification for basic call/connection control.

[3]
ITU-T Recommendation Q.2961.1 (1995), Additional signalling capabilities to support traffic parameters for the tagging option and the sustainable cell rate parameter set.
3
Definitions

This ITU-T Recommendation defines the following terms:

3.1
addressed user: The end user involved in the call that is the recipient of the connection modification request.

3.2
connection modification: The alteration of the characteristics of an established connection with regard to its traffic parameters under the control of the user (connection owner).

3.3
connection owner: One who initiates a connection is the connection owner. There is only one connection owner per connection.

3.4
initiating user: The end user involved in the call that requests the modification of the connection.

3.5
terminating entity: An end user that terminates the modification procedure.

3.6
transit entity: An entity that does not terminate the modification procedure.

3.7
requesting entity: An entity that requests modification. A requesting entity may be a terminating entity or a transit entity.

3.8
responding entity: An entity that receives a modifications request. A responding entity may be a terminating entity or a transit entity.

4
Abbreviations

The abbreviations of Annex J/Q.2931 [2] apply. For the purposes of this ITU-T Recommendation, no new abbreviations are required in addition.

5
Description

The basic capabilities supported by this ITU-T Recommendation are applicable for:

1)
In a given direction, all the specified traffic parameters must be requested to be modified either for increase or decrease. Increase of traffic parameters in one direction and decrease of traffic parameters in the other direction is possible within a single modification request.

2)
Increasing the PCR.

3)
Decreasing the PCR.

4)
Modifying a point-to-point connection (Type 1).

5)
Modifications may be initiated only by the connection owner for a call/connection that is already established (i.e. in the active state). Call/connections that are in the process of being established or cleared cannot be modified.

The following subclauses describe each capability in more detail.

5.1
Modifiable connections

Modification can only be requested by the connection owner for connections which are already established. Therefore, connections which are being established, modified, or cleared cannot be modified.

In the case where clearing is requested of a connection which is being modified, the clearing operation has priority. This results in termination of the modification procedure, i.e. no more messages related to the modification procedure are sent across the user-network interface.

Modification shall not be requested for emulated N-ISDN services; however, a transit entity shall not reject any request for this reason.

5.2
Modification of a point-to-point connection

This ITU-T Recommendation only supports the modification of the attributes of a point-to-point connection (Type 1).

5.3
Increasing or decreasing the Peak Cell Rate 

In this ITU-T Recommendation, only the increase or decease of the Peak Cell Rate (PCR) is specified. When increasing or decreasing PCR, the following rule applies:

•
the modification initiating user is prepared to receive based on an ATM traffic descriptor for which the backward parameters are the greater of the existing backward traffic parameters and of the requested modified backward traffic parameters; and

•
the modification initiating user transmits based on an ATM traffic descriptor for which the forward traffic parameters are the lesser of the existing forward traffic parameters and of the requested modified forward traffic parameters.

In a given direction, the PCR for CLP ( 0 and the PCR for CLP ( 0 ( 1 can be modified in the same modification request. In this case, the modified parameters in the given direction shall all be increased or all be decreased. In a given direction, a given parameter can only be modified if that parameter was specified for that direction during call establishment.

During the operation of modifying the ATM traffic parameters of a connection, the service application supported by the affected connection remains active.

NOTE – During the modification the connection remains usable for the application, with the above constraints.

When the OAM traffic descriptor information element is included at call/connection establishment, the allocation of bandwidth for OAM flows is based on the ATM traffic descriptor agreed. Since the OAM F5 flow allocation is bidirectional, the available user cell rate in one direction can be affected by modification of bandwidth in the other direction.

6
Operational requirements

The provision of the connection modification capability is a service provider option.

6.1
Provision and withdrawal

It is a user and a network option to provide the procedures described in this ITU-T Recommendation. If implemented, the procedures of this ITU-T Recommendation may be provided as a subscription option to the served user on the origination side.

6.2
Requirements on the originating network side

See 6.1 above.

6.3
Requirements on the destination network side

See 6.1 above.

7
Primitive and state definitions

7.1
Primitive definitions

Clause 8/Q.2931 [2] shall apply. No additional primitives between DSS2 layer 3 and the Signalling ATM Adaptation Layer are defined for the purpose of this ITU-T Recommendation.

7.2
State definitions

The call/connection states listed in 2.1/Q.2931 [2] shall apply with the following additions:

7.2.1
modify requested (U/N 13): A MODIFY REQUEST message has been sent to the other side of the interface.

7.2.2
modify received (U/N 14): A MODIFY REQUEST message has been received from the other side of the interface.

7.2.3
State values 

The call state values listed in Table 4-11/Q.2931 [2] shall apply with the following additions:

Bit

6 5 4 3 2 1

User state 
Network state

0 0 1 1 0 1

U13-Modify requested
N13-Modify requested

0 0 1 1 1 0

U14-Modify received
N14-Modify received

8
Coding requirements

8.1
Messages

For the establishment/clearing of call/connections, the messages described in ITU‑T Recommendation Q.2931 [2] as modified by ITU-T Recommendation Q.2961.1 [3] remain valid, and do not have to be enhanced. In order to support the modification of the connections, the following messages should be supported:

MODIFY REQUEST

MODIFY ACKNOWLEDGE

MODIFY REJECT

8.1.1
MODIFY REQUEST

This message is sent from the modification requesting entity to the modification responding entity to request modification of a single connection. See Table 8-1.

Table 8-1/Q.2963.1 – MODIFY REQUEST message content

	Message type:
MODIFY REQUEST
Significance:
Global
Direction:

Both

	Information element
	Reference
	Direction
	Type
	Length

	Protocol discriminator
	4.2/Q.2931
	Both
	M
	1

	Call reference
	4.3/Q.2931
	Both
	M
	4

	Message type
	8.2.1/Q.2963.1
	Both
	M
	2

	Message length
	4.4/Q.2931
	Both
	M
	2

	ATM traffic descriptor (Notes 3, 4)
	4.5/Q.2931
	Both
	M 
	8-20 (Note 2)

	Notification indicator
	4.5/Q.2931
	Both
	O (Note 1)
	4-*

	NOTE 1 – This indicator may be present whenever notification is delivered.

NOTE 2 – All four peak cell rate parameters are optional but at least one shall be present.

NOTE 3 – In a given direction, the PCR for CLP ( 0 and the PCR for CLP ( 0 ( 1 can be modified in the same modification request. In this case, the modified parameters in the given direction shall all be increased or all be decreased.

NOTE 4 – In a given direction, a given parameter can only be specified if that parameter was specified for that direction during call establishment.


8.1.2
MODIFY ACKNOWLEDGE

This message is sent by the modification responding entity to indicate that the modify request is accepted. See Table 8-2.

Table 8-2/Q.2963.1 – MODIFY ACKNOWLEDGE message content

	Message type:
MODIFY ACKNOWLEDGE 
Significance:
Global
Direction:

Both

	Information element
	Reference
	Direction
	Type
	Length

	Protocol discriminator
	4.2/Q.2931
	Both
	M
	1

	Call reference
	4.3/Q.2931
	Both
	M
	4

	Message type
	8.2.1/Q.2963.1
	Both
	M
	2

	Message length
	4.4/Q.2931
	Both
	M
	2

	Notification indicator
	4.5/Q.2931
	Both
	O (Note 1)
	4-*

	Broadband report type
	4.5.25/Q.2931
	Both
	O (Note 2)
	5

	NOTE 1 – This indicator may be present whenever notification is delivered.

NOTE 2 – Included when the addressed user requires confirmation of the success of modification in the addressed user to initiating user direction.


8.1.3
MODIFY REJECT

This message is sent by the modification responding entity to indicate that the modify connection request is rejected. See Table 8-3.

Table 8-3/Q.2963.1 – MODIFY REJECT message content

	Message type:
MODIFY REJECT
Significance:
Global
Direction:

Both

	Information element
	Reference
	Direction
	Type
	Length

	Protocol discriminator
	4.2/Q.2931
	Both
	M
	1

	Call reference
	4.3/Q.2931
	Both
	M
	4

	Message type
	8.2.1/Q.2963.1
	Both
	M
	2

	Message length
	4.4/Q.2931
	Both
	M
	2

	Notification indicator
	4.5/Q.2931
	Both
	O (Note)
	4-*

	Cause
	4.5/Q.2931
	Both
	M
	6-34

	NOTE – This indicator may be present whenever notification is delivered.


8.2
Coding of specific message types and specific information elements

8.2.1
Coding of specific message types

Table 8-4 shows message type (octet 1) code points for the messages defined in 8.1. These are in addition to the values shown in Table 4-4/Q.2931 [2].

Table 8-4/Q.2963.1 – Specific connection modification message types

	Message type (octet 1)

Bits

8 7 6 5 4 3 2 1

1 0 0 0 1 0 0 0
MODIFY REQUEST 

1 0 0 0 1 0 0 1
MODIFY ACKNOWLEDGE 

1 0 0 0 1 0 1 0
MODIFY REJECT


8.2.2
Coding of specific information elements 

For the establishment/clearing of call/connections, the information elements described in ITU-T Recommendations Q.2931 [2] and Q.2961.1 [3] remain valid, and do not have to be enhanced. 

9
Signalling procedures at the coincident SB and TB reference point

The procedures for modification of connection characteristics are specified between modification requesting and responding entities. A requesting entity or a responding entity may be a terminating entity (i.e. end user at the coincident SB and TB reference point) or a transit (i.e. network) entity.

These procedures are additional to those specified in ITU-T Recommendation Q.2931 [2].

An illustration of the resource reservation and allocation procedures is given in Appendix I.

9.1
Modification procedures at the requesting entity

9.1.1
Modification request

To request modification of a call/connection in the active state the entity shall:

–
if a terminating entity:

•
reserve corresponding resources if increase of ATM traffic parameters is requested;

•
reduce the transmitting ATM traffic parameters when decrease is requested; 

–
if a transit entity:

•
reserve corresponding resources if increase of ATM traffic parameters is requested;

–
send a MODIFY REQUEST message with the call reference of the active call/connection including an ATM traffic descriptor information element describing the requested connection characteristics;

–
start Timer T360; and

–
enter the modify requested state (U/N 13).

9.1.2
Modification acknowledgement

On reception of a MODIFY ACKNOWLEDGE message in the modify requested state, the requesting entity shall:

–
if a terminating entity:

•
allocate corresponding resources (i.e. the connection defined by the requested ATM traffic descriptor is available for use);

•
send a CONNECTION AVAILABLE message if the MODIFY ACKNOWLEDGE message contains a broadband report type information element indicating that confirmation of modification is required;

–
if a transit entity:

•
allocate corresponding resources;

•
change UPC according to the ATM traffic parameters;

•
forward the modify acknowledgement towards the initiating user;

–
stop Timer T360; and

–
enter the active state.

9.1.3
Indication of modification rejection

An entity receiving a MODIFY REJECT message while in the modify requested state shall:

–
if a terminating entity:

•
cancel the reservation of resources (i.e. the ATM traffic parameters are as applied prior to the modification request);

–
if a transit entity:

•
cancel the reservation of resources;

•
forward the modification rejection to the initiating user;

–
stop Timer T360; and

–
enter the active state.

9.1.4
Response to STATUS messages while in the modify request state

On reception of a STATUS message while in the modify requested state, the requesting entity shall:

–
if the STATUS message indicates that the responding entity is in the active state and includes a cause value No. 97 "message type non-existent or not implemented" or No. 101 "message not compatible with call state" with a diagnostic indicating that the MODIFY REQUEST message was not understood;

•
if a terminating entity:

–
cancel the reservation of resources (i.e. the ATM traffic parameters are as applied prior to the modification request);

•
if a transit entity:

–
cancel the reservation of resources;

–
forward the modification rejection to the initiating user;

•
stop Timer T360;

•
enter the active state;

–
if the STATUS message includes a cause value No. 97 or No. 101 without a diagnostic indicating that the MODIFY REQUEST message was not understood, send a STATUS ENQUIRY message to the responding entity;

–
if a STATUS message is received in response to this STATUS ENQUIRY indicating the modify received state, the entity shall remain in the modify requested state;

–
if a STATUS message is received in response to this STATUS ENQUIRY indicating that the responding entity is in the active state the entity shall:

•
if a terminating entity:

–
cancel the reservation of resources (i.e. the ATM traffic parameters are as applied prior to the modification request);

•
if a transit entity:

–
cancel the reservation of resources;

–
forward the modification rejection to the initiating user;

•
stop Timer T360; and

•
enter the active state.

9.1.5
No response to modification request

On expiry of Timer T360 the call/connection shall be cleared with cause value No. 102 "recover on timer expiry".

9.2
Modification procedures at the responding entity

9.2.1
Modification indication

On receiving a MODIFY REQUEST message in the active state, the responding entity shall:

–
if the responding entity is a transit entity and is able to support the modification request:

•
reserve corresponding resources if increase of ATM traffic parameters is requested;

•
change the forward UPC if decrease of forward ATM traffic parameters is requested;

•
progress the modification towards the remote user; and 

•
enter the modified received (U/N 14) state.

9.2.2
Modification acceptance

If the responding entity is a transit entity, on receiving an indication that the modification has been accepted while in the Modify Received state:

–
if confirmation of modification is not required in the indication, it shall:

•
allocate corresponding resources;

•
change forward UPC if increase of forward ATM traffic parameters is requested;

•
send a MODIFY ACKNOWLEDGE message; and

•
enter the active state;

–
if confirmation of modification is required in the indication, it shall:

•
allocate corresponding resources;

•
change forward UPC if increase of forward ATM traffic parameters is requested;

•
send a MODIFY ACKNOWLEDGE message including a Broadband report type information element requesting confirmation;

•
start Timer T334; and

•
enter the active state.

On reception of a MODIFY REQUEST message in the active state a responding terminating entity shall, if the request for modification is to be accepted:

–
if confirmation of modification is not required:

•
change the ATM traffic parameters as requested;

•
send a MODIFY ACKNOWLEDGE message; and 

•
enter the active state;

–
if confirmation of modification is required:

•
reduce the backward ATM traffic parameters when decrease is requested;

•
change the forward (i.e. receiving) ATM traffic parameters as requested;

•
send a MODIFY ACKNOWLEDGE message including a broadband report type information element requesting confirmation;

•
start Timer T361; and 

•
enter the active state.

9.2.3
Modification confirmation

If a CONNECTION AVAILABLE message is received by a terminating entity while Timer T361 is running, the entity shall:

–
change ATM traffic parameters as requested (i.e. increase backward ATM traffic parameters);

–
stop Timer T361; and

–
remain in the active state.

If Timer T361 expires the entity shall:

–
change ATM traffic parameters as requested (i.e. increase backward ATM traffic parameters); and

–
remain in the active state.

9.2.4
Modification rejection

If the responding entity is a transit entity, on receiving an indication that the modification has been rejected while in the Modify Received state, it shall:

–
cancel the reservation of resources and reinstate the policing policy that applied prior to the modification request;

–
send a MODIFY REJECT message including the cause information element generated by the addressed entity; and 

–
enter the active state.

If the responding entity is a terminating entity, and receives a request for modification of emulated N-ISDN services, it shall:

–
send a MODIFY REJECT message including a cause information element with cause value No. 29 "Facility reject"; and

–
enter the active.

If the responding entity is a terminating entity, and the request of modification is to be rejected while in the modify received state, it shall:

–
send a MODIFY REJECT message including a cause information element with an appropriate cause value; and

–
enter the active state.

9.2.5
Exceptional procedures

If the responding entity receives the MODIFY REQUEST message in the active state with the ATM traffic parameters which are not according to the allowed combinations as specified in 8.1.1, it shall reject the request as specified in 9.2.4 with cause value No. 73 "unsupported combination of traffic parameters".

9.3
Transit entity conveyance of CONNECTION AVAILABLE messages

While Timer T334 is running in the active state, a transit entity receiving a CONNECTION AVAILABLE message shall stop Timer T334, transfer the CONNECTION AVAILABLE message transparently, and remain in the active state. If Timer T334 is not running, the CONNECTION AVAILABLE message shall be ignored.

10
Procedures at the TB reference point for interworking with private B-ISDNs

The procedures of clause 9 shall apply at the TB reference point.

11
Interworking with other networks

No interworking with other networks has been identified.

NOTE – It is anticipated that an interworking unit may perform the role of a responding entity in order to undertake the rejection of any modification request.

12
Interworking with supplementary services

No requirements for interworking with supplementary services have been identified.

13
Parameter values

The description of timers in the following tables provides the default values of the timers and a brief summary of their use. The precise detail is specified in clauses 9 and 10.

13.1
Timers at the requesting entity

The timers specified in Table 13-1 are used at the requesting entity.

Table 13-1/Q.2963.1 – Timers in the requesting entity defined in clauses 9 and 10

	Timer number
	Default time out value
	Requesting
entity state
	Cause for 
start
	Normal
stop
	At the 
first
expiry
	At the
second
expiry
	Implemen​tation

	T360
	20-30 s
(Note)
	Modify requested
	MODIFY REQUEST sent
	MODIFY ACKNOW-LEDGE or MODIFY REJECT received
	Release call
	Timer is not restarted
	Mandatory

	NOTE – It is recommended that a terminating entity use a value of 30 s and that a transit entity use
a value of 20 s.


13.2
Timers at the responding entity

The timers specified in Table 13-2 are used at the responding entity.

Table 13-2/Q.2963.1 – Timers in the responding entity defined in clauses 9 and 10

	Timer number
	Default time out value
	Respond​ing entity state
	Cause for
start
	Normal
stop
	At the
first
expiry
	At the
second
expiry
	Implemen-
tation

	T361
	20 s
	Active
	MODIFY ACKNOW-LEDGE sent with broadband report type indicating confirmation is required
	CONNEC-TION AVAILABLE received
	Commence sending in the user plane at the modified rate
	Timer is not restarted
	Mandatory if modification confirmation is requested (Note)

	NOTE – Only applicable at a terminating entity.


13.3
Timers at the transit entity

An additional cause of start is added to the network Timer T334 (see Table 13-3).

Table 13-3/Q.2963.1 – Timers in the network as a transit entity defined in clauses 9 and 10

	Timer number
	Default time out value
	Respond​ing entity state
	Cause for
start
	Normal
stop
	At the
first
expiry
	At the
second
expiry
	Implemen-
tation

	T334
	1 s
	Active
	MODIFY ACKNOW-LEDGE sent with broadband report type indicating confirmation is required
	CONNEC-TION AVAILABLE received
	No action
	Not applicable
	Mandatory if modification confirmation is requested 


14
Dynamic description SDLs

Detailed Specification and Description Language (SDL) diagrams for the procedures specified in clauses 9 and 10 are contained in this clause. When there is an ambiguity in the narrative text, the SDL diagrams should be used to resolve the conflict. Where the text and the SDL are in disagreement, the text should be used as the prime source.

The terminology and acronyms of Annex A/Q.2931 [2] shall apply in the SDLs in this clause. These SDLs extend the SDLs defined in Annex A/Q.2931 [2] and should be read in conjunction with the SDLs in Annex A/Q.2931 [2].


[image: image2.wmf]T1179510-96

Q.2931 SDL - Network Side

(Q.2963 Extensions)

Signal lists

Signal for B-ISDN Calls

Primitives to/from Application Process

From AP

Modify-req

Modify-resp

Modify-reject-req

Connection-available-req

To AP

Modify-ind

Modify-conf

Modify-reject-ind

Connection-available-ind

Primitives to/from Q.2931-N

Signal Lists

CDtoON

Modify-req

Modify-resp

Modify-reject-req

Connection-available-req

ONtoCD

Modify-ind

Modify-conf

Modify-reject-ind

Connection-available-ind

Message to/from Q.2931-N for B-ISDN calls

Signal Lists

CDtoON

MODIFY-REQUEST

MODIFY-ACKNOWLEDGE

MODIFY-REJECT

CONNECTION-AVAILABLE

ONtoCD

MODIFY-REQUEST

MODIFY-ACKNOWLEDGE

MODIFY-REJECT

CONNECTION AVAILABLE

PROCESS Co-ord-N;


Figure 14-1/Q.2963.1


[image: image3.wmf]T1178830-96

PROCESS Q.2931-N

N10-Active

9.1.1

Modify-req

(#)

MODIFY-

REQUEST

(#)

Set (Now+t360,

timer T360)

The value t360 of Timer T360

at a transit entity will be

set to 20 s

SET

MOD_CK_FLG:=0

N13 -

Modify

Requested

9.2.1

MODIFY-

REQUEST

(#)

Verify

Msg

Result

STATUS

(Cause, CS:=N14)

RAP

OK

RAI

I

CLR

Modify-ind

(#)

N14-

Modify

Received

STATUS

(Cause, CS:=N10)

N10-Active

D


Figure 14-2/Q.2963.1 (sheet 1 of 7)

[image: image4.wmf]T1178840-96

No

Yes

PROCESS Q.2931-N

N10-Active

Connection

available

req (#)

13.3

9.3

CONNECTION

AVAILABLE

(#)

CONNECTION

AVAILABLE

(#)

N10-Active

Verify

Msg

Result

STATUS

(Cause, CS:=N10)

STATUS

(Cause, CS:=N10)

RAP

OK

RAI

I

CLR

Connection-

available-

ind (#)

N10-Active

D

RESET T334

T334 running ?

Time out T334

9.3


Figure 14-2/Q.2963.1 (sheet 2 of 7)

[image: image5.wmf]T1178850-96

N13 -

Modify

 Requested

9.1.2

MODIFY-

ACKNOWLEDGE

(#)

Verify

Msg

Result

RAP

OK

RAI

I

CLR

STATUS

(Cause, CS:=N10)

STATUS

(Cause, CS:=N13)

RESET T360

Modify-conf

(#)

N10-Active

N13-

Modify

Requested

D

PROCESS Q.2931-N


Figure 14-2/Q.2963.1 (sheet 3 of 7)

[image: image6.wmf]T1178860-96

In 16 of 31 of Q.2931 SDLs,

add N13 to exception states

N13-

Modify

Requested

STATUS

(Cause, CS-R)

9.1.4

Verify

Msg

Result

RAP

OK

RAI

I

CLR

STATUS

(Cause, CS:=N13)

STATUS

(Cause, CS:=N13)

N13-

Modify

Requested

D

Release-conf

(Cause)

CS-R=0

CS-R-10

Yes

No

No

Cause=97,101

Diagnostics

presents

Cause=30

RESET T322

MN

SET (Now+t322,

timer T322)

MOD _CK_FLG:=1

STATUS ENQUIRY

(#)

Diagnostics

indicate the

MODIFY-REQUEST

RESET T360

Modify-

reject-

ind (#)

N10-Active

MN

MN

MOD_CK_FLG=1

RESET T322

Cause=30

N13-

Modify

Requested

PROCESS Q.2931-N

Yes

Yes

Yes

Yes

Yes

Yes

Yes

No

No

No

No

No

No


Figure 14-2/Q.2963.1 (sheet 4 of 7)

[image: image7.wmf]T1178870-96

PROCESS Q.2931-N

N13-

Modify

Requested

9.1.5

Timeout T360

Cause:=102

D

MODIFY

REJECT

(#)

9.1.3

Verify

Msg

Result

RAP

OK

RAI

I

CLR

STATUS

(Cause, CS:=N10)

STATUS

(Cause, CS:=N13)

RESET T360

Modify-reject

-ind (#)

N10-Active

N13-

Modify

Requested

D


Figure 14-2/Q.2963.1 (sheet 5 of 7)

[image: image8.wmf]T1178880-96

N13-

Modify

Requested

In 18 of 31 of Q.2931 SDLs,

add N13 to exception states

PROCESS Q.2931-N

Link Release

indication

Link Establish

error

Link Establish

indication

Link Establish

confirm

5.6.9 c)/Q.2931 and 

 5.6.10/Q.2931.

Same procedure applies

as for the calls in the

Active state of ITU-T 

Recommendation Q.2931

Link Establish

request

Release conf

(Cause:=27)

N13-

Modify

Requested

SE

MN

Verify

State

Compatible

State

Yes

Yes

No

No

Recovery

option

Status-ind

(Cause:=101)

D

Recovery

procedure

N13-

Modify

Requested

Implementation

specific


Figure 14-2/Q.2963.1 (sheet 6 of 7)

[image: image9.wmf]T11104300-99

PROCESS Q.2931-N

N14-

Modify

Received

9.2.2

Modify-resp

(#)

MODIFY-

ACKNOWLEDGE

(#)

Modify-

reject-

req (#)

9.2.4

MODIFY-REJECT

(#)

N10-Active

Broadband report

type IE is

included

No

Yes

SET(Now+t334,

timer T334)



[image: image10.wmf]T1178900-96

PROCESS Q.2931-N

N14-

Modify

Received

In 18 of 31 of Q.2931 SDLs,

add N14 to exception states

Link Release

indication

Link Establish

error

Link Establish

indication

Link Establish

confirm

5.6.9 c)/Q.2931 and

 5.6.10/Q.2931.

Same procedure applies

as for the calls  in the

Active state of ITU-T

Recommendation  Q.2931

Link Establish

request

Release conf

(Cause:=27)

N14-

Modify

Received

SE


Figure 14-2/Q.2963.1 (sheet 7 of 7)

[image: image11.wmf]T1178910-96

PROCESS Co-ord-U;

Q.2931 SDL – User Side

(Q.2963 Extensions)

Signal lists

Signal for B-ISDN Calls

Primitives to/from Application Process

From AP

Modify-req

Modify-resp

Modify-reject-req

Connection-available-req

To AP

Modify-ind

Modify-conf

Modify-reject-ind

Connection-available-ind

Primitives to/from Q.2931-U

Signal Lists

CDtoOU

Modify-req

Modify-resp

Modify-reject-req

Connection-available-req

OUtoCD

Modify-ind

Modify-conf

Modify-reject-ind

Connection-available-ind

Message to/from Q.2931-U to B-ISDN calls

Signal Lists

CDtoOU

MODIFY-REQUEST

MODIFY-ACKNOWLEDGE

MODIFY-REJECT

CONNECTION-AVAILABLE

OUtoCD

MODIFY-REQUEST

MODIFY-ACKNOWLEDGE

MODIFY-REJECT

CONNECTION-AVAILABLE


Figure 14-3/Q.2963.1


[image: image12.wmf]T1178920-96

U10-Active

Modify-req

(#)

9.1.1

MODIFY-

REQUEST

(#)

SET (Now+t360,

timer T360)

SET

MOD_CK_FLG:=0

The value t360 of Timer T360 at a

terminating entity will be set to 30 

s.

And the value t360 of Timer

T360 at a transit entity will be set

to 20 s.

MODIFY-

REQUEST

(#)

9.2.1

Verify

Msg

Result

U13-

Modify

Requested

STATUS

(Cause, CS:=U14)

STATUS

(Cause, CS:=U10)

Modify-ind

(#)

U14-

Modify

Received

U10-Active

D

RAP

OK

RAI

I

CLR

PROCESS Q.2931-U


Figure 14-4/Q.2963.1 (sheet 1 of 7)

[image: image13.wmf]T1178930-96

U10-Active

Timeout T361

9.2.3

Connection-

available-

req (#)

9.1.2

CONNECTION-

AVAILABLE

(#)

9.2.3

CONNECTION-

AVAILABLE

(#)

Verify

Msg

U10-Active

Result

STATUS

(Cause, CS:=U10)

STATUS

(Cause, CS:=U10)

RAP

OK

RAI

I

CLR

RESET T361

Connection-

available-

ind  (#)

U10-Active

D

PROCESS Q.2931-U


Figure 14-4/Q.2963.1 (sheet 2 of 7)

[image: image14.wmf]T1178940-96

PROCESS Q.2931-U

U13-

Modify

Requested

MODIFY-

ACKNOWLEDGE

(#)

9.1.2

Verify

Msg

Result

RAP

OK

RAI

I

CLR

STATUS

(Cause, CS:=U10)

STATUS

(Cause, CS:=U13)

RESET T360

Modify-conf

(#)

U10-Active

U13-

Modify

Requested

D


Figure 14-4/Q.2963.1 (sheet 3 of 7)

[image: image15.wmf]T1178950-96

In 15 of 31 of Q.2931 SDLs,

add U13 to exception states

PROCESS Q.2931-U

U13-

Modify

Requested

STATUS

(Cause, CS-R)

9.1.4

Verify

Msg

Result

RAP

OK

RAI

I

CLR

STATUS

(Cause, CS:=U13)

Release-conf

(Cause)

CS-R=0

CS-R=10

Yes

No

Cause=30

No

Yes

RESET T322

MU

STATUS

(Cause, CS:=U13)

U13-

Modify

Requested

D

Cause=97, 101

Yes

No

No

Cause=30

Yes

Diagnostics

presents

No

Yes

Yes

SET (Now+t322,

timer T322)

MOD_CK_FLG:=1

STATUS ENQUIRY

(#)

U13-

Modify

Requested

Diagnostics

indicate the

MODIFY-REQUEST

No

MU

RESET T360

Modify-

reject

ind (#)

U10-Active

RESET T322

MOD_CK_FLG=1

No

Yes

MU

Yes


Figure 14-4/Q.2963.1 (sheet 4 of 7)

[image: image16.wmf]T1178960-96

PROCESS Q.2931-N

U13-

Modify

Requested

9.1.5

Timeout T360

Cause:=102

D

MODIFY

REJECT

(#)

9.1.3

Verify

Msg

Result

RAP

OK

RAI

I

CLR

STATUS

(Cause, CS:=U10)

STATUS

(Cause, CS:=U13)

RESET T360

Modify-reject

-ind (#)

U10-Active

U13-

Modify

Requested

D


Figure 14-4/Q.2963.1 (sheet 5 of 7)

[image: image17.wmf]T1178970-96

PROCESS Q.2931-U

U13-

Modify

Requested

In 17 of 31 of Q.2931 SDLs,

add U13 to exception states

Link Release

indication

Link Establish

error

Link Establish

indication

Link Establish

confirm

5.6.9  c)/Q.2931 and

 5.6.10/Q.2931.

Same procedure applies

as for the calls in the

Active state of ITU-T

Recommendation Q.2931

Link Establish

request

Release conf

(Cause:=27)

U13-

Modify

Requested

SE

MU

Verify

State

Compatible

State

Yes

Recovery

option

No

No

Yes

Status-ind

(Cause:=101)

D

U13-

Modify

Requested

Recovery

procedure

Implementation

specific


Figure 14-4/Q.2963.1 (sheet 6 of 7)

[image: image18.wmf]T1178980-96

PROCESS Q.2931-N

U14-

Modify

Received

9.2.2

Modify-resp

(#)

MODIFY-

ACKNOWLEDGE

(#)

Modify-

reject-

req (#)

9.2.4

MODIFY-REJECT

(#)

N10-Active

Broadband report

type IE is

included

No

Yes

SET (Now+t361,

timer T361)



[image: image19.wmf]T1178990-96

PROCESS Q.2931-U

U14-

Modify

Received

In 17 of 31 of Q.2931 SDLs,

add U14 to exception states

Link Release

indication

Link Establish

error

Link Establish

indication

Link Establish

confirm

5.6.9 c)/Q.2931 and

 5.6.10/Q.2931.

Same procedure applies

as for the calls in the

Active state of ITU-T

Recommendation Q.2931

Link Establish

request

Release conf

(Cause:=27)

U14-

Modify

Received

SE


Figure 14-4/Q.2963.1 (sheet 7 of 7)
Annex A

Message flow diagrams for modification

See Figures A.1 and A.2.


[image: image20.wmf]T1179000-96

B-ISDN

Initiating

user

MODIFY REQUEST

MODIFY REQUEST

MODIFY ACKNOWLEDGE

MODIFY ACKNOWLEDGE

Network

signalling

messages

CONNECTION AVAILABLE 

a)

CONNECTION AVAILABLE 

a)

a) 

This message is sent only if requested by the addressed user by  including a broadband

    report type information element in the MODIFY ACKNOWLEDGE message. 

Addressed

user


Figure A.1/Q.2963.1 – Successful modification


[image: image21.wmf]T1179010-96

B-ISDN

MODIFY REQUEST

MODIFY REQUEST

MODIFY REJECT

 a)

a) 

Shall contain cause IE indicating why modification was unsuccessful.

Network

signalling

messages

Addressed

user

Initiating

user

MODIFY REJECT

 a)


Figure A.2/Q.2963.1 – Unsuccessful modification (sheet 1 of 2)

[image: image22.wmf]T1179020-96

B-ISDN

MODIFY REQUEST

a) 

Shall contain cause IE indicating why modification was unsuccessful.

Network

signalling

messages

Addressed

user

Initiating

user

MODIFY REJECT

 a)


Figure A.2/Q.2963.1 – Unsuccessful modification (sheet 2 of 2)
Appendix I

Example configuration of user and  network behaviour uring modification procedures

These examples (Figures I.1 and I.2) show the configuration in which both the initiating and the addressed users are terminating entities.


[image: image23.wmf]T1179030-96

Network

UNI

UNI

MODIFY REQUEST

MODIFY REQUEST

MODIFY ACKNOWLEDGE

MODIFY ACKNOWLEDGE

R

R

A

A

Forward UPC change

Backward UPC change

Change of applied forward ATM traffic parameters

Change of applied backward ATM traffic parameters

Increase

Decrease

CAC change

R  Reservation of resources

A  Allocation of resources

Initiating

user

Addressed

user


Figure I.1/Q.2963.1 – Procedure of modification without
request of connection acknowledgement


[image: image24.wmf]T1179040-96

Network

UNI

UNI

MODIFY REQUEST

MODIFY REQUEST

R

R

A

A

MODIFY ACKNOWLEDGE

MODIFY ACKNOWLEDGE

CONNECTION AVAILABLE

CONNECTION AVAILABLE

Forward UPC change

Backward UPC change

Change of applied forward ATM traffic parameters

Change of applied backward ATM traffic parameters

Increase

Decrease

CAC change

R  Reservation of resources

A  Allocation of resources

Initiating

user

Addressed

user


Figure I.2/Q.2963.1 – Procedure of modification with
request of connection acknowledgement

Appendix II

Guidelines for the use of Instruction indicators

This appendix provides guidelines for the recommended use of the instruction indicator in the connection modification message (see Table II.1) and the Broadband report type information element (see Table II.2).

The following abbreviations have been used in the tables:

Used
Follow explicit instructions

Not used
Instruction field not significant

N
Network

U
User 

Table II.1/Q.2963.1 – Recommended use of the Message action
indicators for the connection modification messages

	Message
	Flag 
	Origin
	Action indicator

	MODIFY REQUEST
	Not used
	N & U
	Not significant

	MODIFY ACKNOWLEDGEMENT
	Not used
	N & U
	Not significant

	MODIFY REJECT
	Not used
	N & U
	Not significant


Table II.2/Q.2963.1 – Recommended use of the Instruction indicators for the Broadband
report type information element in the MODIFY ACKNOWLEDGE message

	Information element
	Flag 
	Origin
	Action indicator

	Broadband report type
	Used
	N & U
	Discard information element and proceed


	SERIES OF ITU-T RECOMMENDATIONS

	Series A
	Organization of the work of ITU-T

	Series B
	Means of expression: definitions, symbols, classification

	Series C
	General telecommunication statistics

	Series D
	General tariff principles

	Series E
	Overall network operation, telephone service, service operation and human factors

	Series F
	Non-telephone telecommunication services

	Series G
	Transmission systems and media, digital systems and networks

	Series H
	Audiovisual and multimedia systems

	Series I
	Integrated services digital network

	Series J
	Transmission of television, sound programme and other multimedia signals

	Series K
	Protection against interference

	Series L
	Construction, installation and protection of cables and other elements of outside plant

	Series M
	TMN and network maintenance: international transmission systems, telephone circuits, telegraphy, facsimile and leased circuits

	Series N
	Maintenance: international sound programme and television transmission circuits

	Series O
	Specifications of measuring equipment

	Series P
	Telephone transmission quality, telephone installations, local line networks

	Series Q
	Switching and signalling

	Series R
	Telegraph transmission

	Series S
	Telegraph services terminal equipment

	Series T
	Terminals for telematic services

	Series U
	Telegraph switching

	Series V
	Data communication over the telephone network

	Series X
	Data networks and open system communications

	Series Y
	Global information infrastructure and Internet protocol aspects

	Series Z
	Languages and general software aspects for telecommunication systems

	
	











_1040543767.unknown

_1040622859.unknown

_1041421542.unknown

_1042282918.unknown

_1042283074.unknown

_1041421761.unknown

_1040632418.unknown

_1040543945.unknown

_1040555468.unknown

_1040555613.unknown

_1040621917.unknown

_1040555519.unknown

_1040555410.unknown

_1040543842.unknown

_1040543894.unknown

_1040543802.unknown

_1040543308.unknown

_1040543630.unknown

_1040543673.unknown

_1040543586.unknown

_1040543028.unknown

_1040543074.unknown

_1040542868.unknown

